3GPP TSG-RAN WG1 #86	R1-167532

Gothenburg, Sweden, 22nd– 26th August 2016

Agenda item:	8.1.4.1

Source: 	MediaTek Inc.

Title: 	Discussion on LDPC coding scheme of code structure, granularity and HARQ-IR

Document for:	Discussion/Decision

# Introduction

In RAN1 85 meeting, Way Forward on LDPC Coding Schemes for NR [1] has been agreed. Quasi-cyclic like LDPC code is defined as:

The parity check matrix of quasi-cyclic like LDPC code is defined by a matrix H of size (mb×z)×(nb×z), which consists of sub-block matrices of size z×z,  where each sub-block matrix is composed by circularly shifted matrices or zero matrices. Wherein, mb, nb and z are integers larger than 1.

In this contribution, we provide a LDPC code to support flexible code rate, flexible information block size and HARQ-IR.

# Quasi-cyclic LDPC Code Design

A protomatrix serves as a blueprint for constructing protograph LDPC codes with different block sizes. The performance of a protograph LDPC code can be predicted by analyzing the SNR threshold using density evolution or EXIT charts. One protomatrix for LDPC codes is illustrated in Figure 1, where the (i,j)-th element of the protomatrix represents the number of edges between the i-th check node and the j-th variable node. To convert the protomatrix to the parity check matrix of QC-LDPC code, each element in the protomatrix is replaced by a circular matrix with dimension ZxZ, where Z is called lifting factor and can be adjusted according to the specified block size.

For the high rate code, such as CR = 16/17, it is hard to find a single-edge protograph with good SNR threshold. Therefore, we propose to use a multi-edge protograph LDPC code for the high rate code. Let  be the protomatrix. In the lifting process of a multi-edge protograph LDPC code, the parity-check matrix is constructed as follows. Every positive element in the protomatrix, say ,  is replaced by a circular matrix which is the summation of  circular shifted identity matrices. The circular shifted identity matrices are specified by another matrix called the shift-coefficient matrix whose dimension is the same as the protomatrix as shown in Figure 2. The (i,j)-th element of the shift-coefficient matrix contains nonnegative values if and contains -1 if . A non-negative value, say ,  in the shift-coefficient matrix represents an identity matrix cyclically shift to the right bytimes, and the number -1 represents the all-zero matrix. Besides, we also propose to puncture on part of information bits to enhance the performance. Based on the EXIT analysis, this would further improve the SNR threshold by 0.1dB to 0.2dB.

For lower rate codes, we propose to use the raptor-like structure for the parity bit generation to reduce the complexity of the encoder and decoder. In addition, a raptor-like LDPC code results in a good SNR threshold.

The proposed punctured multi-edge and raptor-like LDPC protomatrix can be represented by

,

where  is a punctured multi-edge protomatrix with highest code rate of  , and  is a matrix with elements being either 1 or 0. Based on our SNR threshold analysis and BLER simulation results, one punctured column can already produce excellent performance. The proposed protomatrix is of size 33x49 with N = 2, Npunc=1, M1 = 18 and M2 = 31 as given in Figure 1. In order to support the LTE granularity with smooth performance among different information block sizes, we propose to use 38 different lifting factors of Z from 3 to 384 which will be introduced Chapter 4.

Proposal 1: Multi-edge should be considered in NR channel coding to tradeoff performance and complexity.

Proposal 2: Puncture on information bits should be considered in NR channel coding to tradeoff performance and complexity.

Figure 1: Proposed protomatrix

Figure 2: Proposed Shift Coefficient Matrix of Z=384

# Flexible Code Rate design with HARQ-IR capability

With the protomatrix given in Figure 1, we can already support code rates from 0.33 to 0.94 in the resolution as shown in  Table 1. Since the number of resource elements for transmission is very flexible in the LTE, puncturing on a few parity bits is still required in the rate matching process. However, the number of punctured bits is smaller than the lifting factor of Z which should not result in performance jitter among different CRs as in Figure 3.

Table 1: Code Rate resolution of proposed protomatrix

| K | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| CR | 0.94 | 0.89 | 0.84 | 0.80 | 0.76 | 0.73 | 0.70 | 0.67 | 0.64 | 0.62 | 0.59 | 0.57 | 0.55 | 0.53 | 0.52 | 0.50 |
| N | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 |
| CR | 0.48 | 0.47 | 0.46 | 0.44 | 0.43 | 0.42 | 0.41 | 0.40 | 0.39 | 0.38 | 0.37 | 0.36 | 0.36 | 0.35 | 0.34 | 0.33 |

Figure 3: Performance among different Code Rate

To support further lower CRs and flexible information block sizes, we proposed to perform zero-padding on the information bits. With carefully protomatrix design, for the low rate codes, we could still exploit further coding gain of 0.5 dB as in Figure 4 without much cost increment using zero-padding. The supported combinations of further lower CRs and information block sizes are shown in Table 2. In addition, the power consumption of the decoder can also be reduced, because the variable nodes corresponding to zero-padded information bits can be removed in the LDPC decoder. In Figure 2, a code of CR=14/48 can be constructed which is obtained by padding two zero message blocks from a code of CR=16/48. Note that in our design, the zero-padding blocks are placed starting from the left most position of the information block.

Figure 4: Coding gain for further low CR from 1/3 to 1/5

Table 2: Further supported combination of CR and information block size by zero-padding

| zero padded | K | N | CR | largest info block size |
| --- | --- | --- | --- | --- |
| 0 | 16 | 48 | 0.33 | 6144 |
| 1 | 15 | 47 | 0.32 | 5760 |
| 2 | 14 | 46 | 0.30 | 5376 |
| 3 | 13 | 45 | 0.29 | 4992 |
| 4 | 12 | 44 | 0.27 | 4608 |
| 5 | 11 | 43 | 0.26 | 4224 |
| 6 | 10 | 42 | 0.24 | 3840 |
| 7 | 9 | 41 | 0.22 | 3456 |
| 8 | 8 | 40 | 0.20 | 3072 |

Figure 5: Illustration of zero padding to generate low rate codes

Since different CRs share the same protomatrix and raptor-like structure is used for further parity-bit generation, the incremental redundancy retransmission is trivial. Taking Figure 3 as an example, CR=16/17 is used in the first transmission. If the first transmission is not successfully decoded by the receiver, new parity bits (15 x Z bits in this example) are further generated and transmitted as the second transmission. The effective CR will be decreased to 1/2 after the second transmission. If the soft buffer size is large enough, the code rates for HARQ-IR can range from 16/17 to CR 1/3.

Observation 1: Puncture on parity bits is required to meet NR channel coding requirement.

Observation 2: One protomatrix can already provide good performance which is good for control overhead and IR capability.

Proposal 3: Zero padding on information bits should be considered in NR channel coding to further exploit coding gain without complexity incremental.

Proposal 4: A region of several CRs should share the same protomatrix to make IR retransmission possible. Raptor-like structure should be considered because of easy encoding and good performance.

Figure 6: Example of HARQ-IR

# Flexible information block sizes

In LTE, 188 different information block sizes are supported for turbo coding. For QC-LDPC, adjusting lifting factor Z is a straightforward method to support different information block sizes. But too many different lifting factors make the code descriptions, shift-coefficient designs, and decoder implementations more difficult. Therefore, a reasonable number of different lifting factors should be considered. In our proposal, 38 lifting factors are suggested to support LTE granularity. However, with 38 lifting factors, only limited information block sizes are supported. In order to have a flexible design, zero-padding method which is already introduced in Chapter 3 is used again. Taking the LTE granularity as an example, Table 3 gives the lifting factors, number of padded bits for each of the information block size. To summarize, we will need 38 different lifting factors with zero-padding scheme to support LTE granularity. With carefully optimization, the performance can be smooth among different information block sizes.

Proposal 5: Zero padding on information bits should be considered in NR channel coding to support flexible information block sizes without too much number of lifting factors.

# Conclusion

A LDPC code design is proposed which is capable for flexible CR, flexible information block size and HARQ-IR in the granularity better than LTE. The performance can be found in [2] and area/ power analysis can be found in [3].

The following summarizes the proposals in this contribution.

Proposal 1: Multi-edge should be considered in NR channel coding to tradeoff performance and complexity.

Proposal 2: Puncture on information bits should be considered in NR channel coding to tradeoff performance and complexity.

Observation 1: Puncture on parity bits is required to meet NR channel coding requirement.

Observation 2: One protomatrix can already provide good performance which is good for control overhead and IR capability.

Proposal 3: Zero padding on information bits should be considered in NR channel coding to further exploit coding gain without complexity incremental.

Proposal 4: A region of several CRs should share the same protomatrix to make IR retransmission possible. Raptor-like structure should be considered because of easy encoding and good performance.

Proposal 5: Zero padding on information bits should be considered in NR channel coding to support flexible information block sizes without too much number of lifting factors.

# References

- R1-165643, Way Forward on LDPC Coding Schemes for NR.

- R1-167544, Data sharing on NR Channel Coding Simulation Result.

- R1-167531, Area, Power and Latency Comparison for NR High Throughput Decoder.

Table 3: Parameters to support LTE granularity

| K | Z | # padded bits | # padded blocks |  | K | Z | # padded bits | # padded blocks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 3 | 8 | 2.67 |  | 1120 | 76 | 96 | 1.26 |
| 48 | 3 | 0 | 0.00 |  | 1152 | 76 | 64 | 0.84 |
| 56 | 4 | 8 | 2.00 |  | 1184 | 76 | 32 | 0.42 |
| 64 | 4 | 0 | 0.00 |  | 1216 | 76 | 0 | 0.00 |
| 72 | 5 | 8 | 1.60 |  | 1248 | 78 | 0 | 0.00 |
| 80 | 5 | 0 | 0.00 |  | 1280 | 88 | 128 | 1.45 |
| 88 | 6 | 8 | 1.33 |  | 1312 | 88 | 96 | 1.09 |
| 96 | 6 | 0 | 0.00 |  | 1344 | 88 | 64 | 0.73 |
| 104 | 7 | 8 | 1.14 |  | 1376 | 88 | 32 | 0.36 |
| 112 | 7 | 0 | 0.00 |  | 1408 | 88 | 0 | 0.00 |
| 120 | 8 | 8 | 1.00 |  | 1440 | 102 | 192 | 1.88 |
| 128 | 8 | 0 | 0.00 |  | 1472 | 102 | 160 | 1.57 |
| 136 | 9 | 8 | 0.89 |  | 1504 | 102 | 128 | 1.25 |
| 144 | 9 | 0 | 0.00 |  | 1536 | 102 | 96 | 0.94 |
| 152 | 10 | 8 | 0.80 |  | 1568 | 102 | 64 | 0.63 |
| 160 | 10 | 0 | 0.00 |  | 1600 | 102 | 32 | 0.31 |
| 168 | 12 | 24 | 2.00 |  | 1632 | 102 | 0 | 0.00 |
| 176 | 12 | 16 | 1.33 |  | 1664 | 118 | 224 | 1.90 |
| 184 | 12 | 8 | 0.67 |  | 1696 | 118 | 192 | 1.63 |
| 192 | 12 | 0 | 0.00 |  | 1728 | 118 | 160 | 1.36 |
| 200 | 14 | 24 | 1.71 |  | 1760 | 118 | 128 | 1.08 |
| 208 | 14 | 16 | 1.14 |  | 1792 | 118 | 96 | 0.81 |
| 216 | 14 | 8 | 0.57 |  | 1824 | 118 | 64 | 0.54 |
| 224 | 14 | 0 | 0.00 |  | 1856 | 118 | 32 | 0.27 |
| 232 | 16 | 24 | 1.50 |  | 1888 | 118 | 0 | 0.00 |
| 240 | 16 | 16 | 1.00 |  | 1920 | 136 | 256 | 1.88 |
| 248 | 16 | 8 | 0.50 |  | 1952 | 136 | 224 | 1.65 |
| 256 | 16 | 0 | 0.00 |  | 1984 | 136 | 192 | 1.41 |
| 264 | 18 | 24 | 1.33 |  | 2016 | 136 | 160 | 1.18 |
| 272 | 18 | 16 | 0.89 |  | 2048 | 136 | 128 | 0.94 |
| 280 | 18 | 8 | 0.44 |  | 2112 | 136 | 64 | 0.47 |
| 288 | 18 | 0 | 0.00 |  | 2176 | 136 | 0 | 0.00 |
| 296 | 21 | 40 | 1.90 |  | 2240 | 160 | 320 | 2.00 |
| 304 | 21 | 32 | 1.52 |  | 2304 | 160 | 256 | 1.60 |
| 312 | 21 | 24 | 1.14 |  | 2368 | 160 | 192 | 1.20 |
| 320 | 21 | 16 | 0.76 |  | 2432 | 160 | 128 | 0.80 |
| 328 | 21 | 8 | 0.38 |  | 2496 | 160 | 64 | 0.40 |
| 336 | 21 | 0 | 0.00 |  | 2560 | 160 | 0 | 0.00 |
| 344 | 24 | 40 | 1.67 |  | 2624 | 184 | 320 | 1.74 |
| 352 | 24 | 32 | 1.33 |  | 2688 | 184 | 256 | 1.39 |
| 360 | 24 | 24 | 1.00 |  | 2752 | 184 | 192 | 1.04 |
| 368 | 24 | 16 | 0.67 |  | 2816 | 184 | 128 | 0.70 |
| 376 | 24 | 8 | 0.33 |  | 2880 | 184 | 64 | 0.35 |
| 384 | 24 | 0 | 0.00 |  | 2944 | 184 | 0 | 0.00 |
| 392 | 28 | 56 | 2.00 |  | 3008 | 212 | 384 | 1.81 |
| 400 | 28 | 48 | 1.71 |  | 3072 | 212 | 320 | 1.51 |
| 408 | 28 | 40 | 1.43 |  | 3136 | 212 | 256 | 1.21 |
| 416 | 28 | 32 | 1.14 |  | 3200 | 212 | 192 | 0.91 |
| 424 | 28 | 24 | 0.86 |  | 3264 | 212 | 128 | 0.60 |
| 432 | 28 | 16 | 0.57 |  | 3328 | 212 | 64 | 0.30 |
| 440 | 28 | 8 | 0.29 |  | 3392 | 212 | 0 | 0.00 |
| 448 | 28 | 0 | 0.00 |  | 3456 | 244 | 448 | 1.84 |
| 456 | 32 | 56 | 1.75 |  | 3520 | 244 | 384 | 1.57 |
| 464 | 32 | 48 | 1.50 |  | 3584 | 244 | 320 | 1.31 |
| 472 | 32 | 40 | 1.25 |  | 3648 | 244 | 256 | 1.05 |
| 480 | 32 | 32 | 1.00 |  | 3712 | 244 | 192 | 0.79 |
| 488 | 32 | 24 | 0.75 |  | 3776 | 244 | 128 | 0.52 |
| 496 | 32 | 16 | 0.50 |  | 3840 | 244 | 64 | 0.26 |
| 504 | 32 | 8 | 0.25 |  | 3904 | 244 | 0 | 0.00 |
| 512 | 32 | 0 | 0.00 |  | 3968 | 280 | 512 | 1.83 |
| 528 | 37 | 64 | 1.73 |  | 4032 | 280 | 448 | 1.60 |
| 544 | 37 | 48 | 1.30 |  | 4096 | 280 | 384 | 1.37 |
| 560 | 37 | 32 | 0.86 |  | 4160 | 280 | 320 | 1.14 |
| 576 | 37 | 16 | 0.43 |  | 4224 | 280 | 256 | 0.91 |
| 592 | 37 | 0 | 0.00 |  | 4288 | 280 | 192 | 0.69 |
| 608 | 38 | 0 | 0.00 |  | 4352 | 280 | 128 | 0.46 |
| 624 | 39 | 0 | 0.00 |  | 4416 | 280 | 64 | 0.23 |
| 640 | 43 | 48 | 1.12 |  | 4480 | 280 | 0 | 0.00 |
| 656 | 43 | 32 | 0.74 |  | 4544 | 324 | 640 | 1.98 |
| 672 | 43 | 16 | 0.37 |  | 4608 | 324 | 576 | 1.78 |
| 688 | 43 | 0 | 0.00 |  | 4672 | 324 | 512 | 1.58 |
| 704 | 50 | 96 | 1.92 |  | 4736 | 324 | 448 | 1.38 |
| 720 | 50 | 80 | 1.60 |  | 4800 | 324 | 384 | 1.19 |
| 736 | 50 | 64 | 1.28 |  | 4864 | 324 | 320 | 0.99 |
| 752 | 50 | 48 | 0.96 |  | 4928 | 324 | 256 | 0.79 |
| 768 | 50 | 32 | 0.64 |  | 4992 | 324 | 192 | 0.59 |
| 784 | 50 | 16 | 0.32 |  | 5056 | 324 | 128 | 0.40 |
| 800 | 50 | 0 | 0.00 |  | 5120 | 324 | 64 | 0.20 |
| 816 | 58 | 112 | 1.93 |  | 5184 | 324 | 0 | 0.00 |
| 832 | 58 | 96 | 1.66 |  | 5248 | 372 | 704 | 1.89 |
| 848 | 58 | 80 | 1.38 |  | 5312 | 372 | 640 | 1.72 |
| 864 | 58 | 64 | 1.10 |  | 5376 | 372 | 576 | 1.55 |
| 880 | 58 | 48 | 0.83 |  | 5440 | 372 | 512 | 1.38 |
| 896 | 58 | 32 | 0.55 |  | 5504 | 372 | 448 | 1.20 |
| 912 | 58 | 16 | 0.28 |  | 5568 | 372 | 384 | 1.03 |
| 928 | 58 | 0 | 0.00 |  | 5632 | 372 | 320 | 0.86 |
| 944 | 66 | 112 | 1.70 |  | 5696 | 372 | 256 | 0.69 |
| 960 | 66 | 96 | 1.45 |  | 5760 | 372 | 192 | 0.52 |
| 976 | 66 | 80 | 1.21 |  | 5824 | 372 | 128 | 0.34 |
| 992 | 66 | 64 | 0.97 |  | 5888 | 372 | 64 | 0.17 |
| 1008 | 66 | 48 | 0.73 |  | 5952 | 372 | 0 | 0.00 |
| 1024 | 66 | 32 | 0.48 |  | 6016 | 380 | 64 | 0.17 |
| 1056 | 66 | 0 | 0.00 |  | 6080 | 380 | 0 | 0.00 |
| 1088 | 76 | 128 | 1.68 |  | 6144 | 384 | 0 | 0.00 |