3GPP TSG RAN WG1 Meeting #92	R1-1803386

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	7.7

Source:	Huawei

Title:	Summary of offline discussion for IMT-2020 self evaluation

Document for:	Discussion and Decision

Background

Study on self evaluation towards IMT-2020 submission was planned in RAN WGs according to [1]. RAN WG1 takes responsibility of most of technical performance requirements related to eMBB, mMTC and URLLC. To well progress the work, the offline discussion was organized and the evaluation features and parameters for eMBB evaluation specifically for spectral efficiency and user experienced data rate were discussed.

It was acknowledged that companies can choose the features that are supported by Rel-15 and that can contribute to related KPIs for self evaluation, and it is good to have some common understanding for some basic features.

Based on this understanding, the following proposals are made.

Proposal

The technical features and system configurations that are supported by Rel-15 can be used in self evaluation.

Companies are encouraged to start the evaluation from NR single-band FDD and TDD. Other features can be further considered.

NOTE: According to ITU-R criteria, channel estimation error should be considered in eMBB evaluation, including DMRS, CSI-RS, SRS, PTRS etc.

Detailed technical parameters for system level simulation

DL FDD (FR1)

The following technical parameters as shown in Table 1 are considered as starting point for DL FDD evaluation for FR1. Other features and parameters that are supported by Rel-15 are not precluded.

Table 1 NR parameters for DL FDD evaluation (FR1)

| DL FDD – FR1 |  |  | NR |
| --- | --- | --- | --- |
| Waveform |  |  | OFDM based |
| Multiple access |  |  | OFDMA |
| Modulation |  |  | Up to 256QAM |
| Coding on PDSCH |  |  | LDPC Max code-block size=8448bit [with BP decoding] |
| Numerology | Slot/non-slot |  | 14 OFDM symbol slot |
|  | SCS |  | 15kHz 30 kHz can also be considered |
| Number of RBs |  |  | 52 for 15 kHz SCS, 24 for 30 kHz SCS |
|  |  |  | Larger number of RBs can be considered to reflect the benefit of reduced guard band ratio for larger BW. A scalar can be applied to spectral efficiency results, SE’=SE×(1-gb(NRB))/(1-gb(NRB0)); where gb(N) is the guard band ratio at given number of RB, N. |
| Simulation bandwidth |  |  | 10 MHz |
| Frame structure |  |  | Slot Format 0 (all downlink) for all slots |
| MIMO |  | Transmission mode | Closed loop SU/MU-MIMO adaptation |
|  |  | Codebook | - NR Type II codebook - NR Type II port-selection codebook can be included - NR Type I codebook (single panel) Detailed configuration to be reported |
|  |  | PRB bundling | 4PRB, or allocated PRB; (It impacts DMRS channel estimation, but does not limit the scheduling unit.) |
|  |  | MU dimension | Up to 12 layer (considering up to 12 orthogonal DMRS ports) |
|  |  | SU dimension | Rank adaptation up to 8 layers (depending on UE antenna elements) |
|  |  | Codeword (CW)-to-layer mapping | For 1~4 layers, CW1; For 5 layers or more, two CWs |
|  |  | DMRS transmission for overhead | Up to 2 OFDM symbols per slot according to the number of layers |
|  |  | CSI-RS transmission | Non-precoded/precoded according to CSI measurement (with codebook selection) and interference measurement |
|  |  | CSI feedback | For slot/non-slot: PMI, CQI: every [5] slot; RI: every [5] slot, CRI: every [5] slot Subband based |
|  |  | Interference measurement | Option 1: SU-CQI Option 2 can be considered: SU-CQI and MU-CQI for intra-cell interference measurement ; For intra-cell interference measurement, precoded CSI-RS is used |
| HARQ |  | CBG retransmission | Max CBG=1 in one TB |
|  |  | ACK/NACK delay | According to UE capability#1 (minimum the next available UL slot) |
|  |  | Re-transmission delay | Companies to report the minimum delay assumed in the simulation |
| PDCCH resource sharing |  |  | Resource sharing with PDSCH might be considered in overhead calculation |
| Overhead |  |  | SS/PBCH block ([1 SS/PBCH block in every 20ms]) CSI-RS, DMRS and TRS, CSI-IM (if used) PDCCH Detailed OH value FFS; companies report OH |

DL TDD (FR1)

The following technical parameters as shown in Table 2 are considered as starting point for DL TDD evaluation for FR1. Other features and parameters that are supported by Rel-15 are not precluded.

It is noted that the parameters that share the same value as in Table 1 (NR FDD table) is not shown.

Table 2 NR parameters for DL TDD evaluation (FR1)

| DL TDD – FR1 |  |  | NR |  |
| --- | --- | --- | --- | --- |
| Numerology | Slot/non-slot |  | 14 OFDM symbol slot |  |
|  | SCS |  | 15kHz | 30kHz |
| Number of RBs |  |  | 106 | 51 |
|  |  |  | Larger number of RBs can be considered to reflect the benefit of reduced guard band ratio for larger BW. A scalar can be applied to spectral efficiency results, SE’=SE×(1-gb(NRB))/(1-gb(NRB0)); where gb(N) is the guard band ratio at given number of RB, N. |  |
| Simulation bandwidth |  |  | 20 MHz |  |
| Frame structure |  |  | Companies report DL/UL configuration and slot format |  |
| MIMO |  | Transmission mode | Closed loop SU/MU-MIMO adaptation |  |
|  |  | Downlink CSI measurement | - Option A: Non-PMI feedback: BS transmits precoded CSI-RS using N “eigen-vector”, UE measures on N CSI-RS port(s) to derive equivalent channel, and then derive rank L (L≤N) and corresponding CQI - Option B can also be considered: BS transmits non-precoded / precoded CSI-RS and UE measures on CSI-RS port(s) to derive channel, and use type I/II codebook to derive precoder, and use the precoder to derive rank and CQI. - NOTE: CSI can be further determined at the gNB. |  |
|  |  | SRS transmission | - Non-precoded SRS - 1 slot is configured to up to 6 OFDM symbols for SRS transmissions. - For NTNR UE (N=1/2/4), one SRS resource for N port SRS. - For 1T4R/2T4R UE (SRS switch) - 1T4R: 1 SRS resource with 1 port SRS, totally 4 SRS resources for SRS switch; different SRS resources occupy different OFDM symbols - 2T4R: 1 SRS resource with 2 port SRS, totally 2 SRS resources for SRS switch; different SRS resources occupy different OFDM symbols - For 1T2R UE (SRS switch) - 1 SRS resource with 1 port SRS, totally 2 SRS resources for SRS switch; different SRS resources occupy different OFDM symbols SRS estimation error model shown in R1-145389 can be used as starting point. |  |

DL TDD (FR2)

The following technical parameters as shown in Table 4 are considered as starting point for DL TDD evaluation for FR2. Other features and parameters that are supported by Rel-15 are not precluded.

Table 4 NR parameters for DL TDD evaluation (FR2)

| DL TDD – FR2 | NR |
| --- | --- |
| Transmit mode | - Analog beam selection based |
| UE antenna panel selection for data transmission and UE attachment | The UE panel with the best receive SNR is chosen for transmission and reception Multi-panel combination is not precluded. |
| UE attachment | Beam selection refinement can be considered. |

UL FDD (FR1)

The following technical parameters as shown in Table 5 are considered as starting point for UL FDD evaluation for FR1. Other features and parameters that are supported by Rel-15 are not precluded.

Table 5 NR parameters for UL FDD evaluation (FR1)

| UL FDD – FR1 |  |  | NR |
| --- | --- | --- | --- |
| Waveform |  |  | OFDM based |
| Multiple access |  |  | OFDMA Companies report power back-off model if used. |
| Modulation |  |  | Up to 256QAM |
| Coding on PDSCH |  |  | LDPC Max code-block size=8448bit |
| Numerology |  | Slot/non-slot | 14 OFDM symbol slot |
|  |  | SCS | 15kHz 30 kHz can also be considered |
| Number of RBs |  |  | 52 for 15 kHz SCS, 24 for 30 kHz SCS |
|  |  |  | Larger number of RBs can be considered to reflect the benefit of reduced guard band ratio for larger BW. A scalar can be applied to spectral efficiency results, SE’=SE×(1-gb(NRB))/(1-gb(NRB0)); where gb(N) is the guard band ratio at given number of RB, N. |
| Simulation bandwidth |  |  | 10 MHz |
| Frame structure |  |  | Slot Format 1 (all uplink) for all slots / non-slots |
| MIMO | Transmission scheme |  | UL codebook based SU-MIMO / MU-MIMO |
|  | UL Codebook |  | - NR 2Tx/4Tx codebook |
|  | MU dimension |  | Up to [2/4] layers per UE depending on UE antenna (considering up to 12 orthogonal DMRS ports) Companies report the maximum number of MU layers. |
|  | SU dimension |  | Rank adaptation up to 4 layers depending on UE antenna |
|  | Codeword (CW)-to-layer mapping |  | For 1~4 layers, CW1 |
|  | DMRS transmission for overhead |  | Up to 2 OFDM symbols per slot according to the number of layers |
|  | SRS transmission |  | Non-precoded 1 slot is configured to up to 6 OFDM symbols for SRS transmissions. |
| HARQ | CBG retransmission |  | Max CBG=1 in one TB |
|  | Re-transmission delay |  | Companies to report the minimum delay assumed in the simulation |
| Power control schemes |  |  | Companies to report |
| Overhead |  |  | DMRS and SRS PUCCH DL feedback Detailed OH value FFS; companies report OH |

UL TDD (FR1)

The following technical parameters as shown in Table 6 are considered as starting point for UL TDD evaluation for FR1. Other features and parameters that are supported by Rel-15 are not precluded.

It is noted that the parameters that share the same value as in Table 5 (UL FDD table) is not shown.

Table 6 NR parameters for UL TDD evaluation (FR1)

| UL TDD – FR1 |  | NR |  |
| --- | --- | --- | --- |
| Numerology | Slot/non-slot | 14 OS slot |  |
|  | SCS | 15kHz | 30kHz |
| RB number |  | 106 | 51 |
|  |  | Larger RB number can be considered to reflect the benefit of reduced guard band ratio for larger BW. A scalar can be applied to spectral efficiency results, SE’=SE×(1-gb(NRB))/(1-gb(NRB0)); where gb(N) is the guard band ratio at given number of RB, N. |  |
| Simulation bandwidth |  | 20 MHz |  |
| Frame structure |  | Companies report DL/UL configuration and slot format |  |
| MIMO | Transmission scheme | UL SU-MIMO / MU-MIMO - Non-codebook based - Codebook based can also be considered |  |
|  | SRS transmission | - Precoded SRS for non-codebook based - Non-precoded SRS for codebook based 1 slot is configured to up to 6 OFDM symbols for SRS transmissions. |  |

Detailed configuration parameters

The following configuration parameters are to be reported or used by companies.

| Parameter | Value |
| --- | --- |
| Antenna configuration at BS | are the number of vertical, horizontal antenna elements within a panel, are the number of vertical, horizontal TXRUs within a panel and polarization, and is number of polarizations, is the number of panels in a column, is the number of panels in row. |
| Antenna configuration at UE |  |
| Scheduling | PF (MU-PF for MU-MIMO or SU-PF for SU-MIMO) |
| Receiver | MMSE-IRC Advance receiver is not precluded. |
| Down-tilt | Mechanical tilt (calibration parameters can be used as starting point) Electrical tilt |
| Uplink power control parameters | , P0 |
| Interference over thermal | Companies report (use the “effective IoT” model as in Section A.2.1.8 in TR36.814) |

Other features for further consideration

The technical features listed in Appendix 1 can be further considered for eMBB evaluation.

- References

- RP- 172532, “Work plan for self evaluation towards 3GPP Update Submission to ITU-R”, Huawei, Ericsson, Telecom Italia, December 2017.

- Appendix: Candidate additional features for eMBB evaluation.

TABLE 1

Candidate features for eMBB

| General features | FR1 (below 6 GHz) | FR2 (above 24 GHz) |
| --- | --- | --- |
| NR on single-band | Un-paired spectrum: TDD Paired spectrum: FDD, flexible duplex, etc. | Un-paired spectrum: TDD |
| LTE on single-band | Un-paired spectrum: TDD Paired spectrum: FDD | - |
| NR on multi-band | NR with SUL+TDD band using macro layer (at least for UL user experienced data rate) |  |
| NR/LTE inter-working on multi-band | NR/LTE dual connectivity with two frequency bands, with or without SUL |  |