3GPP TSG-RAN WG2 Meeting #96	R2-168174

Reno, Nevada, USA, 14 – 18 November 2016

Agenda Item:	9.2.2.4

Source:	Samsung

Title:	Consideration to Network Energy Efficiency for NR

Document for:	Discussion

# Introduction

Network energy efficiency is an important aspect to be considered for the study of NR, as it directly influences the cost of the cellular network operation. While LTE has shown improvements in the network energy efficiency compared to previous cellular technologies, some of the design principles of LTE (e.g. reference signal design) do not allow the network to switch to low power state even during sporadic user traffic conditions.

3GPP has included network energy efficiency as one of the KPIs for study of NR [1]. In order to meet the requirements of energy efficiency, RAN2 procedures need to be designed considering network power consumption without compromising user performance.

This paper discusses some aspects for configuration of reference signals for NR. Note that, the reference signal design falls mainly into RAN1 responsibility, and the RAN2 discussion should be initiated only after sufficient details are progress by RAN1. Nevertheless considering huge impact it will have on UE mobility procedures and network power consumption, we think it is beneficial to bring RAN2 interest into this area for further discussion.

# Discussion

The reference signal design has significant impact on the overall energy efficiency of the network. Following downlink reference signals (RS) have been defined for LTE,

- Cell Specific Reference Signals (CRS)

- MBSFN Reference Signals

- UE-specific Reference Signals

- Positioning Reference Signals

- CSI Reference Signals (CSI-RS)

Out of the above RSs, CRS is an always-on type signal while other RSs are only transmitted when required.

CRS in LTE is used for different purposes like RRM, RLM, downlink channel estimation, data demodulation, etc. CRS is transmitted by an LTE eNB in every radio subframe and across the entire bandwidth of the cell. Such continuous transmission of CRS does not allow LTE eNB to implement efficient sleep cycles even during sparse traffic conditions, hence leading to high network power consumption.

- LTE CRS design does not allow LTE eNB to implement efficient sleep mechanism.

CRS transmissions cannot be switched off, to ensure backward compatibility with rel-8 UEs. This results in CRS being transmitted even when/where it is not required. For instance, a UE may not use CRS for demodulation of PDSCH transmissions from antenna ports other than ports 0-3. For such PDSCH transmissions, separate RSs are configured for channel estimation, which use different radio resources than used by CRS transmissions.

Hence, LTE CRS design not only increases the power consumption but also restricts the deployment of different features and services due to its always-on transmission across the entire bandwidth. Impact of this was observed during standardization of enhanced MIMO and beamforming features for LTE. This issue is recognized by different companies, which envision addressing this issue in NR [2].

- LTE CRS design restricts deployment of different features and services due to its always-on transmission across the entire cell bandwidth.

- Generally, RRM and RLM procedures do not impose strict measurement requirements for UE. In LTE, UE performs RLM measurements once every DRX cycle if DRX is configured for the UE and once every radio frame if DRX is not configured. While, RRM is mainly performed during initial cell acquisition and neighbour cell measurements. Hence, the reference signal transmission from the perspective of RRM and RLM can be made lean without compromising UE’s performance.

- Reference signal design for RRM and RLM purposes can be made lean without compromising UE’s performance.

## Flexible configuration of Reference Signals

### Always-On Reference Signals

One reason for always-on transmission of CRS in LTE is to enable UEs to perform RRM and RLM measurements with minimal delay.

In NR, different services and deployment scenarios may have their own set of requirements for UE measurements. For instance, mMTC type devices may not require frequent RRM and RLM measurements, owing to low speed of UEs and low UE complexity design. While, a URLLC or eMBB device operating in high frequency band may need to perform these measurements more frequently in order to maintain reliable connection. Also, considering wide range in supportable UE mobility scenarios, there will be cases where RS transmission will be over-frequent (low mobility), while in other cases it may be too sparse (high mobility) for the UEs. Owing to the given considerations, a flexibly configured RS can efficiently cater to the requirements of NR. This RS requirement of flexible/lean configuration is also supported by many companies [2].

- In RAN1#85 [3], it is agreed to study multiplexing of different numerologies within the same carrier bandwidth. Here, a numerology can be specific to a particular service vertical and may be identified by subcarrier spacing, subframe duration, etc. For such scenarios, each service vertical can have its own set of RSs with resource allocation defined based on user characteristics, while initial acquisition is performed by UEs using a common set of RS including PSS and SSS. Hence, in order to support this, RAN2 needs to define the framework for flexible RS configuration based on different use cases and services.

Figure 1 Service specific RS transmission where different services are multiplexed within same carrier bandwidth

### Aperiodic Reference Signals

CSI-RS and UE specific RSs were introduced to enable enhanced MIMO and beamforming operation. CSI-RS is used for channel state estimation and can be flexibly configured in LTE. While, UE specific reference signals are used by the UEs to derive the channel estimate for demodulating the data and are embedded only in the resource blocks to which the corresponding PDSCH is mapped. In RAN1#86bis [4], agreements were made for CSI-RS design, which included studying the functionality of aperiodic, semi-persistent as well as periodic transmission of CSI-RS.

UE-specific or aperiodic RSs have the potential to reduce the network power consumption especially in low traffic scenarios. Configurability of these RSs also avoids transmission over the entire frequency band, which may facilitate further introduction of new services and physical channels, if required.

RAN2 consider framework for flexible RS configuration (periodic and aperiodic) based on different use cases and services.

- For aperiodic RS transmission, certain enhancements can be considered to support on demand SI framework being discussed in RAN2 and also UE specific RS requirements. For instance, the requirements for downlink RS transmission may be specific to each UE according to its radio conditions, hence UE indication to NR NB about its RS requirements can be considered. For example, a UE experiencing severe signal strength degradation from the serving beam may require frequent RS transmission from neighbor beams for channel measurement, so that beam change procedure can be expedited and be made more reliable. Other use cases include RS periodicity correction based on UE mobility and to prevent of radio link failure by increasing the frequency/power of RS transmission when serving beam/cell signal strength deteriorates.

Figure 2 Aperiodic RS transmission based on UE requirements

# Conclusion

In this contribution we have discussed some aspects of network energy efficiency for NR. Following observations and proposals are made in this contribution.

Observation 1	LTE CRS design does not allow LTE eNB to implement efficient sleep mechanism.

Observation 2	LTE CRS design restricts deployment of different features and services due to its always-on transmission across the entire cell bandwidth.

Observation 3	Reference signal design for RRM and RLM purposes can be made lean without compromising UE’s performance.

Proposal 1	RAN2 consider framework for flexible RS configuration (periodic and aperiodic) based on different use cases and services.

# References

3GPP TR 38.913 V0.3.0 (2016-03), “Study on Scenarios and Requirements for Next Generation Access Technologies (Release 14)”

METIS-II, Deliverable D4.1 v1.0 (2016-05), “Draft air interface harmonization and user plane design”.

R1-166056, “Final Report of 3GPP TSG RAN WG1 #85 v1.0.0”, Nanjing, China

R1-16xxxxx, “Draft Report of 3GPP TSG RAN WG1 #86bisbis v0.1.0”, Lisbon, Portugal