3GPP TSG-RAN WG5 Meeting #58 	R5-130486

# Malta, MT, 28 January – 01 February 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.121-1 | CR |  | rev | - | Current version: | 10.5.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating Chapter 6 receiver test cases for 4C_HSDPA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RANimp-4C_HSDPA_UEConTest |  |  |  |  |  |  | Date: |  | 2013-01-18 |
| Category: |  |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | During RAN5#57 chapter 6 test cases were added for 4C_HSDPA WP, some of the sections of these test cases were incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated incomplete sections of Chapter 6 completing applicability, annexure, test tolerances of the test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Item for 4C-HSDPA will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C, 6.2D, 6.3G, 6.3H, 6.5D, 6.5E, 6.5F, 6.5G, 6.6C, 6.6D, 6.7D, 6.7E. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR .34.121-2.. CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Draft version of 4C-HSDPA(FDD) applicable test cases were added in meeting #57 |  |  |  |  |  |  |  |  |

[Many Sections are skipped here]

## 6.2C	Reference Sensitivity Level for Single band 4C-HSDPA

### 6.2C.1	Definition and applicability

The reference sensitivity level <REFSENS> is the minimum mean power received at the UE antenna port at which the Block Error Ratio (BLER) on each individual cell shall not exceed a specific value.

The requirements and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support Single band 4C-HSDPA.

Single band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.2C.2	Minimum Requirements

For all requirements listed in Table 6.2C.1, corresponding to the specific single band 4C-HSDPA configuration(s) supported by the UE, (see Table 4.0A), the BLER measured on each individual cell shall not exceed 0.1.

NOTE:	The reference sensitivity level <REFSENS> requirement for single band 4C-HSDPA is not applicable for dual uplink operation. However, there might be a substantial Rx de-sensitization for the UE operating in bands which have less than 80 MHz Tx-Rx frequency separation, transmitting on more than one uplink frequency, at maximum power.

Table 6.2C.1: Test parameters for reference sensitivity, additional requirement for single band 4C-HSDPA

| Single band 4C-HSDPA configuration | DL Band | Unit | HS-PDSCH_Ec <REFSENS> | <REFÎor> | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-3 | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
| NOTE 1 For Power class 3, 3bis and 4, this shall be at the maximum output power |  |  |  |  |  |

For the UE which supports DB-DC-HSDPA configuration in Table 6.2C.2, the reference sensitivity level HS-PDSCH_Ec <REFSENS> and corresponding <REFÎor> in Table 6.2C.1 are allowed to be increased by the amount given in Table 6.2C.2 for the applicable bands.

Table 6.2C.2: Allowed de-sensitization relative to reference sensitivity for UE which supports
DB-DC-HSDPA

| DB-DC-HSDPA Configuration | Allowed de-sensitization (dB) | Applicable bands |
| --- | --- | --- |
| 4 | 1 | I |

The normative reference for this requirement is TS 25.101 [1] clause 7.3.4.

### 6.2C.3	Test purpose

To verify that the UE BLER of HS-PDSCH on each individual cell shall not exceed 0,1 for the parameters specified in table 6.2C.4 for the DL reference channel H-Set 1B specified in Annex C.8.1.1.

The lack of the reception sensitivity decreases the HSDPA coverage area at the far side from Node B.

### 6.2C.4	Method of test

6.2C.4.1	Initial conditions

Test environment: normal, TL/VL, TL/VH, TH/VL, TH/VH; see clauses G.2.1 and G.2.2.

Frequencies to be tested: low range, mid range, high range; see clause G.2.4.

1)	Connect the SS to the UE antenna connector as shown in figure A.48.

2)	Channel conditions are initially set up with received CPICH_RSCP >-85 dBm. The relative power level of primary downlink physical channels to Ior are set up according to clause E.5.0.

3)	Switch on the phone.

4)	A call is set up according to the Generic call setup procedure in TS34.108 [3] sub clause 7.3.16.

5)	The RF parameters are set up according to table 6.2C.3 for each individual cells.

Contents of RADIO BEARER SETUP message: AM or UM (4C-HSDPA)

See TS 34.109 [4] for details regarding loopback test.

#### 6.2C.4.2	Procedure

1)	Set and send continuously Up power control commands to the UE until the UE output power shall be maximum level.

2)	Measure the BLER of HS-PDSCH on each individual cell received from the UE at the SS.

### 6.2C.5	Test requirements

The measured BLER, derived in step 2), shall not exceed 0,1 on each individual cell. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.0.

Table 6.2C.3: Test parameters for reference sensitivity, additional requirement for single band 4C-HSDPA.

| Single band 4C-HSDPA configuration | DL Band | Unit | HS-PDSCH_Ec <REFSENS> | <REFÎor> | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-3 | I | dBm/3.84 MHz | -112.3 | -102 | Minimum |
| NOTE 1 For Power class 3, 3bis and 4, this shall be at the maximum output power |  |  |  |  |  |

For the UE which supports DB-DC-HSDPA configuration in Table 6.2C.5, the reference sensitivity level HS-PDSCH_Ec <REFSENS> and corresponding <REFÎor> in Table 6.2C.4 are allowed to be increased by the amount given in Table 6.2C.5 for the applicable bands.

Table 6.2C.4: Allowed de-sensitization relative to reference sensitivity for UE which supports DB-DC-HSDPA.

| DB-DC-HSDPA Configuration | Allowed de-sensitization (dB) | Applicable bands |
| --- | --- | --- |
| 4 | 1 | I |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

## 6.2D	Reference Sensitivity Level for Dual band 4C-HSDPA

### 6.2D.1	Definition and applicability

The reference sensitivity level <REFSENS> is the minimum mean power received at the UE antenna port at which the Block Error Ratio (BLER) on each individual cell shall not exceed a specific value.

The requirements and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support dual band 4C-HSDPA.

Dual band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.2D.2	Minimum Requirements

For all requirements listed in Table 6.2D.1, corresponding to the specific dual band 4C-HSDPA configuration(s) supported by the UE, (see Table 4.0B), the BLER measured on each individual cell shall not exceed 0.1.

Note:	The reference sensitivity level <REFSENS> requirement for dual band 4C-HSDPA is not applicable for dual uplink operation. However, there might be a substantial Rx de-sensitization for the UE operating in bands which have less than 80 MHz Tx-Rx frequency separation, transmitting on more than one uplink frequency, at maximum power.

Table 6.2D.1: Test parameters for reference sensitivity, additional requirement for dual band 4C-HSDPA

| Dual band 4C-HSDPA configuration | DL Band | UL Band | Unit | HS-PDSCH_Ec <REFSENS> | <REFÎor> | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | I | VIII | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 | Minimum |
|  | II | IV | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | I | V | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
| NOTE 1 For Power class 3, 3bis and 4, this shall be at the maximum output power |  |  |  |  |  |  |

The normative reference for this requirement is TS 25.101 [1] clause 7.3.5.

### 6.2D.3	Test purpose

To verify that the UE BLER of HS-PDSCH on each individual cell shall not exceed 0,1 for the parameters specified in table 6.2D.3 for the DL reference channel H-Set 1C specified in Annex C.8.1.1.

The lack of the reception sensitivity decreases the HSDPA coverage area at the far side from Node B.

### 6.2D.4	Method of test

6.2D.4.1	Initial conditions

Test environment: normal, TL/VL, TL/VH, TH/VL, TH/VH; see clauses G.2.1 and G.2.2.

Frequencies to be tested: low range, mid range, high range; see clause G.2.4.

1)	Connect the SS to the UE antenna connector as shown in figure A.48.

2)	Channel conditions are initially set up with received CPICH_RSCP >-85 dBm. The relative power level of primary downlink physical channels to Ior are set up according to clause E.5.0.

3)	Switch on the phone.

4)	A call is set up according to the Generic call setup procedure in TS34.108 [3] sub clause7.3.16.

5)	The RF parameters are set up according to table 6.2D.2 for each individual cells.

See TS 34.109 [4] for details regarding loopback test.

6.2D.4.2	Procedure

1)	Set and send continuously Up power control commands to the UE until the UE output power shall be maximum level.

2)	Measure the BLER of HS-PDSCH on each individual cell received from the UE at the SS.

### 6.2D.5	Test requirements

The measured BLER, derived in step 2), shall not exceed 0,1 on each individual cell. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.0.

Table 6.2D.2: Test parameters for reference sensitivity, additional requirement for dual band 4C-HSDPA

| Dual band 4C-HSDPA configuration | DL Band | UL Band | Unit | HS-PDSCH_Ec <REFSENS> | <REFÎor> | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | dBm/3.84 MHz | -112.3 | -102 | Minimum |
|  | VIII |  | dBm/3.84 MHz | -109.3 | -99 | Minimum |
|  | I | VIII | dBm/3.84 MHz | -112.3 | -102 | Minimum |
|  | VIII |  | dBm/3.84 MHz | -109.3 | -99 | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | dBm/3.84 MHz | -109.3 | -99 | Minimum |
|  | IV |  | dBm/3.84 MHz | -111.3 | -101 | Minimum |
|  | II | IV | dBm/3.84 MHz | -109.3 | -99 | Minimum |
|  | IV |  | dBm/3.84 MHz | -111.3 | -101 | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | dBm/3.84 MHz | -112.3 | -102 | Minimum |
|  | V |  | dBm/3.84 MHz | -110.3 | -100 | Minimum |
|  | I | V | dBm/3.84 MHz | -112.3 | -102 | Minimum |
|  | V |  | dBm/3.84 MHz | -110.3 | -100 | Minimum |
| NOTE 1 For Power class 3, 3bis and 4, this shall be at the maximum output power |  |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

[Many Sections are skipped here]

## 6.3G	Maximum Input Level for 4C-HSDPA Reception (16QAM)

### 6.3G.1	Definition and applicability

Maximum input level for 4C-HSDPA reception is defined as the maximum power received at the UE antenna port, which shall not degrade the specified 4C-HSDPA throughput performance.

The requirements and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support Single band/Dual band 4C-HSDPA with 16QAM.

Single band/Dual band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.3G.2	Minimum requirements

The additional 4C-HSDPA requirements are specified in terms of a minimum information throughput per cell R with the DL reference channel H-Set 1B/1C (16QAM version) specified in Annex C.8.1.1, with the addition of the parameters in Table 6.3G.1, and the downlink physical channel setup according to table E.5.1, applied to all the cells simultaneously. Using this configuration the throughput shall meet or exceed the minimum requirements specified in table 6.3G.2.

Table 6.3G.1 Minimum requirement parameters for 16QAM Maximum Input Level (4C-HSDPA)

| Parameter | Unit | Value |
| --- | --- | --- |
| Phase reference |  | P-CPICH |
| Wanted signal mean power per band (dBm) | dBm/band | -22 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |
| DPCH_Ec/Ior | dB | -13 |
| HS-SCCH_1_Ec/Ior | dB | -13 |
| Redundancy and constellation version |  | 6 |
| Maximum number of HARQ transmissions |  | 1 |
| Note 1: The HS-SCCH and corresponding HS-PDSCH shall be transmitted continuously with constant power but the HS-SCCH shall only use the identity of the UE under test every third TTI Note 2: Wanted signal mean power per band is the sum of measured mean power on each carrier in a band over 3.84 MHz. |  |  |

Table 6.3G.2: Minimum throughput requirement (4C-HSDPA)

| HS-PDSCH (dB) | T-put (kbps) |
| --- | --- |
| -3 | 700 |

The reference for this requirement is TS 25.101 [1] clause 7.4.4.1

### 6.3G.3	Test purpose

To verify that the UE DC-HSDPA throughput meets the minimum requirements specified in table 6.3G.2 for the DL reference channel H-Set 1B/1C specified in Annex C.8.1.1 with the addition of the parameters specified in table 6.3G.4.

An inadequate maximum input level causes loss of 4C-HSDPA coverage near the Node B.

### 6.3G.4	Method of test

6.3G.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

RF parameters are given in tables 6.3G.4 and table E.5.1.

Contents of RADIO BEARER SETUP message: AM or UM (Test Loop Mode1)

Table 6.3G.3: Specific Message Contents for 16QAM Maximum Input Level (4C-HSDPA)

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Uplink DPCH info |  | Rel-6 |
| - Uplink DPCH power control info |  |  |
| - CHOICE mode | FDD |  |
| - Power Control Algorithm | Algorithm2 |  |

6.3G.4.2	Procedure

Connect the SS to the UE antenna connector as shown in figure A.48.

1)	The UE is switched on.

2)	An RRC connection is set-up according to the generic HSDPA set-up procedure specified in TS 34.108 [3] clause 7.3.16 with exceptions for information elements listed in table 6.3G.3

3)	Set the power level of UE according to the table 6.3G.4 and send power control commands to the UE .The UE output power measured by Test System shall be kept at the specified power level with 1dB tolerance.

4)	Measure the HS-PDSCH throughput R received by the UE on each individual cell by counting the number of NACK, ACK and statDTX on the UL HS-DPCCH (Throughput = blocksize*number of blocks acknowledged/time).

5)	The UE is switched off.

### 6.3G.5	Test requirements

The measured throughput, as derived in step 4), shall meet or exceed 700Kbit/second on each individual cell. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.1.

Table 6.3G.4: Test requirement parameters for 16QAM Maximum Input Level (4C-HSDPA)

| Parameter | Unit | Value |
| --- | --- | --- |
| Phase reference |  | P-CPICH |
| Wanted signal mean power per band (dBm) | dBm/band | -22.7 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |
| DPCH_Ec/Ior | dB | -13 |
| HS-SCCH_1_Ec/Ior | dB | -13 |
| Redundancy and constellation version |  | 6 |
| Maximum number of HARQ transmissions |  | 1 |
| Note 1: The HS-SCCH and corresponding HS-PDSCH shall be transmitted continuously with constant power but the HS-SCCH shall only use the identity of the UE under test every third TTI. Note 2: Wanted signal mean power per band is the sum of measured mean power on each carrier in a band over 3.84 MHz. |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

## 6.3H	Maximum Input Level for 4C-HSDPA Reception (64QAM)

### 6.3H.1	Definition and applicability

Maximum input level for DC-HSDPA reception is defined as the maximum power received at the UE antenna port, which shall not degrade the specified DC-HSDPA throughput performance.

The requirements and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support Single band/Dual band 4C-HSDPA with 64QAM.

Single band/Dual band  4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.3H.2	Minimum requirements

The additional 4C-HSDPA requirements are specified in terms of a minimum information throughput per cell R with the DL reference channel H-Set 8B/8C specified in Annex C.8.1.8, with the addition of the parameters in Table 6.3H.1, and the downlink physical channel setup according to table E.5.1, applied to all the cells simultaneously. Using this configuration the throughput shall meet or exceed the minimum requirements specified in table 6.3H.2.

Table 6.3H.1: Minimum requirement parameters for 64QAM Maximum Input Level (4C-HSDPA)

| Parameter | Unit | Value |
| --- | --- | --- |
| Phase reference |  | P-CPICH |
| Wanted signal mean power per band (dBm) | dBm/band | -22 |
| UE transmitted mean power | dBm | 0 |
| DPCH_Ec/Ior | dB | -13 |
| HS-SCCH_1_Ec/Ior | dB | -13 |
| Redundancy and constellation version |  | 6 |
| Maximum number of HARQ transmissions |  | 1 |
| Note 1: The HS-SCCH and corresponding HS-PDSCH shall be transmitted continuously with constant power but the HS-SCCH shall only use the identity of the UE under test every third TTI Note 2: Wanted signal mean power per band is the sum of measured mean power on each carrier in a band over 3.84 MHz. |  |  |

Table 6.3H.2: Minimum throughput requirement (4C-HSDPA)

| HS-PDSCH (dB) | T-put (kbps) |
| --- | --- |
| -2 | 11800 |

The reference for this requirement is TS 25.101 [1] clause 7.4.4.2.

### 6.3H.3	Test purpose

To verify that the UE 4C-HSDPA throughput meets the minimum requirements specified in table 6.3H.2 for the DL reference channel H-Set 8B/8C specified in Annex C.8.1.8 with the addition of the parameters specified in table 6.3H.4.

An inadequate maximum input level causes loss of 4C-HSDPA coverage near the Node B.

### 6.3H.4	Method of test

6.3H.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

RF parameters are given in tables 6.3H.4 and table E.5.1.

Contents of RRC CONNECTION SETUP message: UM (Transition to CELL_DCH)

Table 6.3H.3: Specific Message Contents for 64QAM Maximum Input Level (4C-HSDPA)

| Information Element | Value/remark | Version |
| --- | --- | --- |
| Downlink information for per radio links list |  |  |
| -Downlink information for each radio links |  |  |
| - Downlink DPCH info for each RL |  |  |
| - DL channelisation code |  |  |
| - Code number | 14 |  |

Contents of RADIO BEARER SETUP message: AM or UM (Test Loop Mode1)

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Uplink DPCH info |  | Rel-6 |
| - Uplink DPCH power control info |  |  |
| - CHOICE mode | FDD |  |
| - Power Control Algorithm | Algorithm2 |  |
| Downlink information per radio link list |  |  |
| - Downlink information for each radio link |  |  |
| - Downlink DPCH info for each RL |  |  |
| - DL channelisation code |  |  |
| - Code number | 7 |  |

Contents of RADIO BEARER SETUP message: AM or UM (4C-HSDPA)

| Information Element | Value/remark | Version |
| --- | --- | --- |
| Downlink HS-PDSCH Information |  |  |
| - CHOICE mode | FDD |  |
| - Downlink 64QAM configured | TRUE | Rel-7 |
| Downlink information per radio link list |  |  |
| - Downlink information for each radio link |  |  |
| - Downlink DPCH info for each RL |  |  |
| - DL channelisation code |  |  |
| - Code number | 7 |  |
| Downlink secondary cell info FDD |  | Rel-8 |  |
| - CHOICE Configuration info | New configuration |  |  |
| - Downlink 64QAM configured | TRUE |  |  |

6.3H.4.2	Procedure

Connect the SS to the UE antenna connector as shown in figure A.48.

1)	The UE is switched on.

2)	An RRC connection is set-up according to the generic HSDPA set-up procedure specified in TS 34.108 [3] clause 7.3.16 exceptions for information elements listed in table 6.3H.3

3)	Set the power level of UE according to the table 6.3H.4 and send power control commands to the UE .The UE output power measured by Test System shall be kept at the specified power level with 1dB tolerance.

4)	Measure the HS-PDSCH throughput R received by the UE on each individual cell by counting the number of NACK, ACK and statDTX on the UL HS-DPCCH (Throughput = blocksize*number of blocks acknowledged/time).

5)	The UE is switched off.

### 6.3H.5	Test requirements

The measured throughput, as derived in step 4), shall meet or exceed 11800 Kbit/second on each individual cell. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.1A.

Table 6.3H.4: Test requirement parameters for 64QAM Maximum Input Level (4C-HSDPA)

| Parameter | Unit | Value |
| --- | --- | --- |
| Phase reference |  | P-CPICH |
| Wanted signal mean power per band (dBm) | dBm/band | -22.7 |
| UE transmitted mean power | dBm | 0 |
| DPCH_Ec/Ior | dB | -13 |
| HS-SCCH_1_Ec/Ior | dB | -13 |
| Redundancy and constellation version |  | 6 |
| Maximum number of HARQ transmissions |  | 1 |
| Note 1: The HS-SCCH and corresponding HS-PDSCH shall be transmitted continuously with constant power but the HS-SCCH shall only use the identity of the UE under test every third TTI Note 2: Wanted signal mean power per band is the sum of measured mean power on each carrier in a band over 3.84 MHz. |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

[Many Sections are skipped here]

## 6.5D	Blocking Characteristics for single Uplink Single band 4C-HSDPA

### 6.5D.1	Definition and applicability

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

The requirements in clause 6.5D.2.1 and 6.5D.2.2 and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support single band single uplink 4C-HSDPA.

Single band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.5D.2	Minimum Requirements

#### 6.5D.2.1	Minimum Requirements (In-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.5D.1 and 6.5D.2. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 6.5D.1: Test parameters for in-band blocking, single band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |

NOTE 1:	Iblocking (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	For single band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies.

Table 6.5D.2: In-band blocking requirements, single band 4C-HSDPA, single uplink operation

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- |
| I-3 | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2C.1 for single band 4C-HSDPA.

The normative reference for this requirement is TS 25.101 [1] clause 7.6.1C.1.

#### 6.5D.2.2	Minimum requirements (Out of-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.5D.3 and 6.5D.4. Out-of-band band blocking is defined for an unwanted interfering signal falling more than 15 MHz below or above the UE receive band.

For Table 6.5D.3 in frequency range 1, 2 and 3, up to 24 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 6.6C spurious response are applicable.

For Table 6.5D.4 in frequency range 4, up to 8 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 6.6C spurious response are applicable.

Table 6.5D.3: Test parameters for out of band blocking, single band 4C-HSDPA

| Parameter | Unit | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 |
| --- | --- | --- | --- | --- | --- |
| Iblocking (CW) | dBm | -44 | -30 | -15 | -15 |
| Fuw (Single band 4C-HSDPA Configuration I-3) | MHz | 2050<f <2095 2185<f <2230 | 2025 <f 2050 2230 f <2255 | 1< f 2025 2255f<12750 | - |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |
| Single band 4C-HSDPA Configuration I-3 | For 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause [6.4D.2] and subclause 6.5D.2.1 shall be applied. |  |  |  |  |

Table 6.5D.4: Out of band blocking requirements, single band 4C-HSDPA

| Singe band 4C-HSDPA Configuration | Parameter | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-3 | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB |  |

NOTE:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2C.1.

The normative reference for this requirement is TS 25.101 [1] clause 7.6.2C.

### 6.5D.3	Test purpose

To verify that the UE BLER of HS-PDSCH on each individual cell does not exceed 0.1 for the parameters specified in table 6.5D.6, table 6.5D.7, table 6.5D.8 and table 6.5D.10 for the DL reference channel H-Set 1B specified in Annex C.8.1.1.

The lack of the blocking ability decreases the 4C-HSDPA coverage area when other transmitter exists (except in the adjacent channels and spurious response).

### 6.5D.4	Method of test

#### 6.5D.4.1	Initial conditions

For in-band case:

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clauseG.2.4.

For out-of-band case:

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequency to be tested: 1 arbitrary frequency chosen from the low, mid or high range; see clause G.2.4.

1)	Connect the SS to the UE antenna connector as shown in figure A.49.

2)	RF parameters are set up according to table 6.5D.6, table 6.5D.7, table 6.5D.8 and table 6.5D.9.

3)	A call is set up according to the Generic 4C-HSDPA setup procedure specified in TS34.108[3] sub clause 7.3.16, with exceptions for information elements listed in Table 6.5D.5. With this exception, the Power Control Algorithm for the Uplink is set to algorithm 2.

Contents of RADIO BEARER SETUP message: AM or UM (Test Loop Mode1)

Table 6.5D.5: Specific Message Contents for In-band blocking characteristics for Single band 4C-HSDPA, Single uplink operation

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Uplink DPCH info |  | Rel-6 |
| - Uplink DPCH power control info |  |  |
| - CHOICE mode | FDD |  |
| - Power Control Algorithm | Algorithm2 |  |

#### 6.5D.4.2	Procedure

1)	Set the parameters of the CW generator or the interference signal generator as shown in table 6.5D.6 and table 6.5D.7, for inblocking measurements and table 6.5D.8 and table 6.5D.9 for out of band measurements. For table 6.5D.8 and 6.5D.9, the frequency step size is 1 MHz.

2)	Set the power level of UE according to the table 6.5D.6, table 6.5D.7, table 6.5D.8 and table 6.5D.9, or send the power control commands (1dB step size should be used.) to the UE until UE output power measured by Test System shall be kept at the specified power level with 1dB tolerance.

3)	Measure the BLER of HS-PDSCH received from the UE at the SS by counting the number of NACK, ACK and statDTX on the UL HS-DPCCH (BLER = (NACK + statDTX)/(NACK + statDTX + ACK)).

4)	For table 6.5D.8 and 6.5D.9, record the frequencies for which BLER exceed the test requirements.

### 6.5D.5	Test requirements

For table 6.5D.6 and 6.5D.7, the measured BLER, derived in step 2), shall not exceed 0.1 on each individual cell. For table 6.5D.8 and 6.5D.9, the measured BLER, derived in step 2) shall not exceed 0.1 on each individual cell except for the spurious response frequencies, recorded in step 3). The number of spurious response frequencies, recorded in step 3) shall not exceed 24 for frequency range 1, 2 and 3. The number of spurious response frequencies, recorded in step 3) shall not exceed 8 for frequency range 4. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.0.

Table 6.5D.6: Test parameters for in-band blocking, single band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |

NOTE 1:	Iblocking (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	For single band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies.

Table 6.5D.7: In-band blocking requirements, single band 4C-HSDPA, single uplink operation

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- |
| I-3 | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2C.1 for single band 4C-HSDPA.

Table 6.5D.8: Test parameters for out of band blocking, single band 4C-HSDPA

| Parameter | Unit | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 |
| --- | --- | --- | --- | --- | --- |
| Iblocking (CW) | dBm | -44 | -30 | -15 | -15 |
| Fuw (Single band 4C-HSDPA Configuration I-3) | MHz | 2050<f <2095 2185<f <2230 | 2025 <f 2050 2230 f <2255 | 1< f 2025 2255f<12750 | - |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |
| Single band 4C-HSDPA Configuration I-3 | For 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause [6.4D.2] and subclause 6.5D.2.1 shall be applied. |  |  |  |  |

Table 6.5D.9: Out of band blocking requirements, single band 4C-HSDPA

| Singe band 4C-HSDPA Configuration | Parameter | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-3 | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB |  |

NOTE 4:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2C.1.

NOTE 5:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

## 6.5E	Blocking Characteristics for dual uplink single band 4C-HSDPA

### 6.5E.1	Definition and applicability

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

The requirements in clause 6.5E.2.1 and 6.5E.2.2 and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support single band dual uplink 4C-HSDPA.

Single band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.5E.2	Minimum Requirements

#### 6.5E.2.1	Minimum Requirements (In-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.5E.1 and 6.5E.2. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 6.5E.1: Test parameters for in-band blocking, single band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |

NOTE 1:	Iblocking (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	For single band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies.

Table 6.5E.2: In-band blocking requirements, single band 4C-HSDPA, dual uplink operation

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-3 | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| NOTE 1: For the UE which supports DB-DC-HSDPA configuration in section 4.2 the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 6.1A. NOTE 2: For the UE which supports dual band 4C-HSDPA configuration in Table 4.0B the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 6.1B. |  |  |  |  |  |

The normative reference for this requirement is TS 25.101 [1] clause 7.6.1C.2.

### 6.5E.3	Test purpose

To verify that the UE BLER of HS-PDSCH on each individual cell does not exceed 0.1 for the parameters specified in table 6.5E.4 and table 6.5E.5 for the DL reference channel H-Set 1B specified in Annex C.8.1.1.

The lack of the blocking ability decreases the 4C-HSDPA coverage area when other transmitter exists (except in the adjacent channels and spurious response).

### 6.5E.4	Method of test

#### 6.5E.4.1	Initial conditions

For in-band case:

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clauseG.2.4.

For out-of-band case:

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequency to be tested: 1 arbitrary frequency chosen from the low, mid or high range; see clause G.2.4.

1)	Connect the SS to the UE antenna connector as shown in figureA.49.

2)	RF parameters are set up according to table 6.5E.4 and table 6.5E.5.

3)	A call is set up according to the Generic 4C-HSDPA setup procedure specified in TS34.108[3] sub clause 7.3.16, with exceptions for information elements listed in Table 6.5E.3. With this exception, the Power Control Algorithm for the Uplink is set to algorithm 2.

Contents of RADIO BEARER SETUP message: AM or UM (Test Loop Mode1)

Table 6.5E.3: Specific Message Contents for In-band blocking characteristics for Single band 4C-HSDPA, Single uplink operation

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Uplink DPCH info |  | Rel-6 |
| - Uplink DPCH power control info |  |  |
| - CHOICE mode | FDD |  |
| - Power Control Algorithm | Algorithm2 |  |

#### 6.5E.4.2	Procedure

1)	Set the parameters of the CW generator or the interference signal generator as shown in table 6.5E.4 and table 6.5E.5, for inblocking measurements.

2)	Set the power level of UE according to the table 6.5E.4 and table 6.5E.5, or send the power control commands (1dB step size should be used.) to the UE until UE output power measured by Test System shall be kept at the specified power level with 1dB tolerance.

3)	Measure the BLER of HS-PDSCH received from the UE at the SS by counting the number of NACK, ACK and statDTX on the UL HS-DPCCH (BLER = (NACK + statDTX)/(NACK + statDTX + ACK)).

4)	For table 6.5E.5, record the frequencies for which BLER exceed the test requirements.

### 6.5E.5	Test requirements

For table 6.5E.4 and 6.5E.5, the measured BLER, derived in step 2), shall not exceed 0.1 on each individual cell. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.0.

Table 6.5E.4: Test parameters for in-band blocking, single band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |

NOTE 1:	Iblocking (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	For single band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies.

Table 6.5E.5: In-band blocking requirements, single band 4C-HSDPA, dual uplink operation

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-3 | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| NOTE 1: For the UE which supports DB-DC-HSDPA configuration in section 4.2 the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 6.1A. NOTE 2: For the UE which supports dual band 4C-HSDPA configuration in Table 4.0B the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 6.1B. |  |  |  |  |  |

NOTE 3:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

## 6.5F	Blocking Characteristics for single Uplink dual band 4C-HSDPA

### 6.5F.1	Definition and applicability

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

The requirements in clause 6.5F.2.1 and 6.5F.2.2 and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support dual band single uplink 4C-HSDPA.

Dual band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.5F.2	Minimum Requirements

#### 6.5F.2.1	Minimum Requirements (In-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.5F.1 and 6.5F.2. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 6.5F.1: Test parameters for in-band blocking, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |

NOTE 1:	Iblocking (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies.

Table 6.5F.2: In-band blocking requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | VIII | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | II | IV | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | V | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1

The normative reference for this requirement is TS 25.101 [1] clause 7.6.1D.1.

#### 6.5F.2.2	Minimum requirements (Out of-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.5F.3 and 6.5F.4. Out-of-band band blocking is defined for an unwanted interfering signal falling more than 15 MHz below or above the UE receive band.

For Table 6.5F.3 in frequency range 1, 2 and 3, up to 24 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 6.6D spurious response are applicable.

For Table 6.5F.4 in frequency range 4, up to 8 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 6.6D spurious response are applicable.

Table 6.5F.3: Test parameters for out of band blocking, dual band 4C-HSDPA

| Parameter | Unit | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 |
| --- | --- | --- | --- | --- | --- |
| Iblocking (CW) | dBm | -44 | -30 | -15 | -15 |
| Fuw (Dual band 4C-HSDPA Configuration I-2-VIII-1, I-3-VIII-1) | MHz | 865< f <910 975< f <1020 2050< f <2095 2185< f <2230 | 840< f 865 1020 f <1045 2025< f 2050 2230 f <2255 | 1< f 840 1045 f <2025 2255< f 12750 | - |
| Fuw (Dual band 4C-HSDPA Configuration II-1-IV-2, II-2-IV-1, II-2-IV-2) | MHz | 1870< f <1915 2005< f <2095 2170< f <2215 | 1845< f 1870 2215 f <2240 | 1< f 1845 2240 f <12750 | 1850 f 1910 |
| Fuw (Dual band 4C-HSDPA Configuration I-1-V-2, I-2-V-1, I-2-V-2) | MHz | 809< f <854 909< f <954 2050< f <2095 2185< f <2230 | 784< f 809 954 f < 979 2025< f 2050 2230 f <2255 | 1< f 784 979 f <2025 2255< f 12750 | 824 f 849 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |
| Dual band 4C-HSDPA Configuration I-2-VIII-1, I-3-VIII-1 | For 910f 975 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause [6.4D.2] and subclause 6.5F.2.1 shall be applied. |  |  |  |  |
| Dual band 4C-HSDPA Configuration II-1-IV-2, II-2-IV-1, II-2-IV-2 | For 1915f 2005 MHz and 2095f 2070 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause [6.4D.2] and subclause 6.5F.2.1 shall be applied. |  |  |  |  |
| Dual band 4C-HSDPA Configuration I-1-V-2, I-2-V-1, I-2-V-2 | For 854f909 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause [6.4D.2] and subclause 6.5F.2.1 shall be applied. |  |  |  |  |

Table 6.5F.4: Out of band blocking requirements, dual band 4C-HSDPA

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | Parameter | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | VIII |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | I | VIII | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | VIII |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | IV |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | II | IV | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | IV |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | V |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | I | V | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | V |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |

NOTE:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1.

The normative reference for this requirement is TS 25.101 [1] clause 7.6.2D.

#### 6.5F.2.3	Minimum requirements (Narrow band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.5F.5 and 6.5F.6. This requirement is measure of a receiver’s ability to receive a W-CDMA signal at its assigned channel frequency in the presence of an unwanted narrow band interferer at a frequency, which is less than the nominal channel spacing.

Table 6.5F.5: Test parameters for narrow band blocking characteristics, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Band II, IV, V | Band VIII |
| --- | --- | --- | --- |
| Iblocking (GMSK) | dBm | -57 | -56 |
| Fuw (offset) (NOTE 2) | MHz | 2.7 | 2.8 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |

NOTE 1:	Iblocking (GMSK) is an interfering signal as defined in TS 45.004.

NOTE 2:	For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequency(ies) in each band.

Table 6.5F.6: Narrow band blocking requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | VIII | I | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | VIII | VIII | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | IV |  | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | II | IV | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | IV |  | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | V | I | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | V | V | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1 for dual band 4C-HSDPA.

The normative reference for this requirement is TS 25.101 [1] clause 7.6.3D.1.

### 6.5F.3	Test purpose

To verify that the UE BLER of HS-PDSCH on each individual cell does not exceed 0.1 for the parameters specified in table 6.5F.8, table 6.5F.9, table 6.5F.10, table 6.5F.11, table 6.5F.12 and table 6.5F.13 for the DL reference channel H-Set 1B/1C specified in Annex C.8.1.1.

The lack of the blocking ability decreases the 4C-HSDPA coverage area when other transmitter exists (except in the adjacent channels and spurious response).

### 6.5F.4	Method of test

#### 6.5F.4.1	Initial conditions

For in-band case:

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clauseG.2.4.

For out-of-band case:

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequency to be tested: 1 arbitrary frequency chosen from the low, mid or high range; see clause G.2.4.

1)	Connect the SS to the UE antenna connector as shown in figureA.49.

2)	RF parameters are set up according to table 6.5F.8 through table 6.5F.13.

3)	A call is set up according to the Generic 4C-HSDPA setup procedure specified in TS34.108[3] sub clause 7.3.16, with exceptions for information elements listed in Table 6.5F.5. With this exception, the Power Control Algorithm for the Uplink is set to algorithm 2.

Contents of RADIO BEARER SETUP message: AM or UM (Test Loop Mode1)

Table 6.5F.7: Specific Message Contents for In-band blocking characteristics for Single band 4C-HSDPA, Single uplink operation

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Uplink DPCH info |  | Rel-6 |
| - Uplink DPCH power control info |  |  |
| - CHOICE mode | FDD |  |
| - Power Control Algorithm | Algorithm2 |  |

#### 6.5F.4.2	Procedure

1)	Set the parameters of the CW generator or the interference signal generator as shown in table 6.5F.8 and table 6.5F.9, for inblocking measurements, Table 6.5F.10 and table 6.5F.11 for out of band measurements and Table 6.5F.12 and table 6.5F.13 for narrow band blocking. For table 6.5F.8 and 6.5F.9, the frequency step size is 1 MHz.

2)	Set the power level of UE according to the table 6.5F.8 through  table 6.5F.13, or send the power control commands (1dB step size should be used.) to the UE until UE output power measured by Test System shall be kept at the specified power level with 1dB tolerance.

3)	Measure the BLER of HS-PDSCH received from the UE at the SS by counting the number of NACK, ACK and statDTX on the UL HS-DPCCH (BLER = (NACK + statDTX)/(NACK + statDTX + ACK)).

4)	For table 6.5F.9, table 6.5F.11 and table 6.5F.13, record the frequencies for which BLER exceed the test requirements.

### 6.5F.5	Test requirements

For table 6.5F.8 and 6.5F.9, the measured BLER, derived in step 2), shall not exceed 0.1 on each individual cell. For table 6.5F.10 and 6.5F.11, the measured BLER, derived in step 2) shall not exceed 0.1 on each individual cell except for the spurious response frequencies, recorded in step 3). The number of spurious response frequencies, recorded in step 3) shall not exceed 24 for frequency range 1, 2 and 3. The number of spurious response frequencies, recorded in step 3) shall not exceed 8 for frequency range 4. For table 6.5F.12 and 6.5F.13, the measured BLER, derived in step 2), shall not exceed 0.1 on each individual cell. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.0.

Table 6.5F.8: Test parameters for in-band blocking, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |

NOTE 1:	Iblocking (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies.

Table 6.5F.9: In-band blocking requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | VIII | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | II | IV | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | V | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1.

Table 6.5F.10: Test parameters for out of band blocking, dual band 4C-HSDPA

| Parameter | Unit | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 |
| --- | --- | --- | --- | --- | --- |
| Iblocking (CW) | dBm | -44 | -30 | -15 | -15 |
| Fuw (Dual band 4C-HSDPA Configuration I-2-VIII-1, I-3-VIII-1) | MHz | 865< f <910 975< f <1020 2050< f <2095 2185< f <2230 | 840< f 865 1020 f <1045 2025< f 2050 2230 f <2255 | 1< f 840 1045 f <2025 2255< f 12750 | - |
| Fuw (Dual band 4C-HSDPA Configuration II-1-IV-2, II-2-IV-1, II-2-IV-2) | MHz | 1870< f <1915 2005< f <2095 2170< f <2215 | 1845< f 1870 2215 f <2240 | 1< f 1845 2240 f <12750 | 1850 f 1910 |
| Fuw (Dual band 4C-HSDPA Configuration I-1-V-2, I-2-V-1, I-2-V-2) | MHz | 809< f <854 909< f <954 2050< f <2095 2185< f <2230 | 784< f 809 954 f < 979 2025< f 2050 2230 f <2255 | 1< f 784 979 f <2025 2255< f 12750 | 824 f 849 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |
| Dual band 4C-HSDPA Configuration I-2-VIII-1, I-3-VIII-1 | For 910f 975 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause [6.4D.2] and subclause 6.5F.2.1 shall be applied. |  |  |  |  |
| Dual band 4C-HSDPA Configuration II-1-IV-2, II-2-IV-1, II-2-IV-2 | For 1915f 2005 MHz and 2095f 2070 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause [6.4D.2] and subclause 6.5F.2.1 shall be applied. |  |  |  |  |
| Dual band 4C-HSDPA Configuration I-1-V-2, I-2-V-1, I-2-V-2 | For 854f909 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause [6.4D.2] and subclause 6.5F.2.1 shall be applied. |  |  |  |  |

Table 6.5F.11: Out of band blocking requirements, dual band 4C-HSDPA

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | Parameter | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | VIII |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | I | VIII | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | VIII |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | IV |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | II | IV | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | IV |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | V |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | I | V | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | V |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |

NOTE 4:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1

Table 6.5F.12: Test parameters for narrow band blocking characteristics, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Band II, IV, V | Band VIII |
| --- | --- | --- | --- |
| Iblocking (GMSK) | dBm | -57 | -56 |
| Fuw (offset) (NOTE 2) | MHz | 2.7 | 2.8 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |

NOTE 5:	Iblocking (GMSK) is an interfering signal as defined in TS 45.004

NOTE 6:	For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequency(ies) in each band.

Table 6.5F.13: Narrow band blocking requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | VIII | I | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | VIII | VIII | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | IV |  | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | II | IV | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | IV |  | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | V | I | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | V | V | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |

NOTE 7:	<REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1 for dual band 4C-HSDPA.

NOTE 8:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

## 6.5G	Blocking Characteristics for dual uplink dual band 4C-HSDPA

### 6.5G.1	Definition and applicability

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

The requirements in clause 6.5G.2.1 and 6.5G.2.2 and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support dual band dual uplink 4C-HSDPA.

Dual band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.5G.2	Minimum Requirements

#### 6.5G.2.1	Minimum Requirements (In-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.5G.1 and 6.5G.2. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 6.5G.1: Test parameters for in-band blocking, dual band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |

NOTE 1:	Iblocking (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	For single band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies.

Table 6.5G.2: In-band blocking requirements, dual band 4C-HSDPA, dual uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-IV-2 | II | IV | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-1 | II | II | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-2 | II | II | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | II | IV | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-V-2 | I | V | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.2 | -92.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-1 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -108 | -97.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-2 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -108 | -97.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | I | V | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.2 | -92.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1 for single band 4C-HSDPA.

The normative reference for this requirement is TS 25.101 [1] clause 7.6.1D.2.

#### 6.5G.2.2	Minimum requirements (Narrow band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.5G.3 and Table 6.5G.4. This requirement is measure of a receiver’s ability to receive a W-CDMA signal at its assigned channel frequency in the presence of an unwanted narrow band interferer at a frequency, which is less than the nominal channel spacing.

Table 6.5G.3: Test parameters for narrow band blocking characteristics for dual band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Band II, IV, V | Band VIII |
| --- | --- | --- | --- |
| Iblocking (GMSK) | dBm | -57 | -56 |
| Fuw (offset) (NOTE 2) | MHz | 2.7 | 2.8 |

NOTE 1:	Iblocking (GMSK) is an interfering signal as defined in TS 45.004

NOTE 2:	For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequency(ies) in each band.

Table 6.5G.4: Narrow band blocking requirements, dual band 4C-HSDPA, dual uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | VIII | I | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-IV-2 | II | IV | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-1 | II | II | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-2 | II | II | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | II | IV | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-V-2 | V | V | -99.8 | -89.5 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-1 | V | I | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-2 | V | I | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V | V | -99.8 | -89.5 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |

The normative reference for this requirement is TS 25.101 [1] clause 7.6.3D.2.

### 6.5G.3	Test purpose

To verify that the UE BLER of HS-PDSCH on each individual cell does not exceed 0.1 for the parameters specified in table 6.5G.6, table 6.5G.7, table 6.5G.8 and table 6.5G.9 for the DL reference channel H-Set 1B/1C specified in Annex C.8.1.1.

The lack of the blocking ability decreases the 4C-HSDPA coverage area when other transmitter exists (except in the adjacent channels and spurious response).

### 6.5G.4	Method of test

6.5G.4.1	Initial conditions

For in-band case:

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clauseG.2.4.

For out-of-band case:

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequency to be tested: 1 arbitrary frequency chosen from the low, mid or high range; see clause G.2.4.

1)	Connect the SS to the UE antenna connector as shown in figure A.49.

2)	RF parameters are set up according to table 6.5G.6, table 6.5G.7, table 6.5G.8 and table 6.5G.9.

3)	A call is set up according to the Generic 4C-HSDPA setup procedure specified in TS34.108[3] sub clause 7.3.16, with exceptions for information elements listed in Table 6.5G.5. With this exception, the Power Control Algorithm for the Uplink is set to algorithm 2.

Contents of RADIO BEARER SETUP message: AM or UM (Test Loop Mode1)

Table 6.5G.5: Specific Message Contents for In-band blocking characteristics for Single band 4C-HSDPA, Single uplink operation

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Uplink DPCH info |  | Rel-6 |
| - Uplink DPCH power control info |  |  |
| - CHOICE mode | FDD |  |
| - Power Control Algorithm | Algorithm2 |  |

6.5G.4.2	Procedure

1)	Set the parameters of the CW generator or the interference signal generator as shown in table 6.5G.6 and table 6.5G.7, for inblocking measurements and table 6.5G.8 and table 6.5G.9 for narrow  band measurements. For table 6.5G.8 and 6.5G.9, the frequency step size is 1 MHz.

2)	Set the power level of UE according to the table 6.5G.6, table 6.5G.7, table 6.5G.8 and table 6.5G.9, or send the power control commands (1dB step size should be used.) to the UE until UE output power measured by Test System shall be kept at the specified power level with 1dB tolerance.

3)	Measure the BLER of HS-PDSCH received from the UE at the SS by counting the number of NACK, ACK and statDTX on the UL HS-DPCCH (BLER = (NACK + statDTX)/(NACK + statDTX + ACK)).

4)	For table 6.5G.8 and 6.5G.9, record the frequencies for which BLER exceed the test requirements.

### 6.5G.5	Test requirements

For table 6.5G.6 and 6.5G.7, the measured BLER, derived in step 2), shall not exceed 0.1 on each individual cell. For table 6.5G.8 and 6.5G.9, the measured BLER, derived in step 2) shall not exceed 0.1 on each individual cell except for the spurious response frequencies, recorded in step 3). The number of spurious response frequencies, recorded in step 3) shall not exceed 24 for frequency range 1, 2 and 3. The number of spurious response frequencies, recorded in step 3) shall not exceed 8 for frequency range 4. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.0.

Table 6.5G.6: Test parameters for in-band blocking, dual band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |

NOTE 1:	Iblocking (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	For single band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies.

Table 6.5G.7: In-band blocking requirements, dual band 4C-HSDPA, dual uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-IV-2 | II | IV | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-1 | II | II | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-2 | II | II | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | II | IV | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-V-2 | I | V | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.2 | -92.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-1 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -108 | -97.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-2 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -108 | -97.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | I | V | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.2 | -92.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1 for single band 4C-HSDPA.

Table 6.5G.8: Test parameters for narrow band blocking characteristics for dual band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Band II, IV, V | Band VIII |
| --- | --- | --- | --- |
| Iblocking (GMSK) | dBm | -57 | -56 |
| Fuw (offset) (NOTE 2) | MHz | 2.7 | 2.8 |

NOTE 4:	Iblocking (GMSK) is an interfering signal as defined in TS 45.004.

NOTE 5:	For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequency(ies) in each band.

Table 6.5G.9: Narrow band blocking requirements, dual band 4C-HSDPA, dual uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | VIII | I | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-IV-2 | II | IV | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-1 | II | II | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-2 | II | II | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | II | IV | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-V-2 | V | V | -99.8 | -89.5 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-1 | V | I | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-2 | V | I | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V | V | -99.8 | -89.5 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |

NOTE 6:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

[Many Sections are skipped here]

## 6.6C	Spurious Response for single band 4C-HSDPA

### 6.6C.1	Definition and applicability

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in subclause 6.5D.2.2 is not met.

The requirements and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support single band 4C-HSDPA.

Single band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.6C.2	Minimum Requirements

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.6C.1.

Table 6.6C.1: Test Parameters for Spurious Response single band 4C-HSDPA

| Parameter | Unit | Level |
| --- | --- | --- |
| HS-PDSCH_Ec | dBm/3.84 MHz | <REFSENS> +3 dB |
| Îor | dBm/3.84 MHz | <REFÎor> +3 dB |
| Iblocking (CW) | dBm | -44 |
| Fuw | MHz | Spurious response frequencies |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |

NOTE:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2C.1.

The normative reference for this requirement is TS 25.101 [1] clause 7.7.2.

### 6.6C.3	Test purpose

To verify that the UE BLER of HS-PDSCH does not exceed 0.1 for the parameters specified in table 6.6C.3 for the DL reference channel H-Set 1B specified in Annex C.8.1.1.

The lack of the spurious response ability decreases the 4C-HSDPA coverage area when other unwanted interfering signal exists at any other frequency.

### 6.6C.4	Method of test

6.6C.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequency to be tested: the same frequency as chosen in clause 6.5D.4.1 for Blocking characteristics out-of-band case.

1)	Connect the SS to the UE antenna connector as shown in figureA.50.

2)	RF parameters are set up according to table 6.6C.3.

3)	A call is set up according to the Generic call setup procedure specified in TS 34.108 [3] sub clause 7.3.16, with exceptions for information elements listed in Table 6.6C.2. With this exception, the Power Control Algorithm for the Uplink is set to algorithm 2.

Contents of RADIO BEARER SETUP message: AM or UM (Test Loop Mode1)

Table 6.6C.2: Specific Message Contents for Spurious Response (single band 4C-HSDPA)

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Uplink DPCH info |  | Rel-6 |
| - Uplink DPCH power control info |  |  |
| - CHOICE mode | FDD |  |
| - Power Control Algorithm | Algorithm2 |  |

See TS 34.108 [3] and TS 34.109 [4] for details regarding generic call setup procedure and loopback test.

6.6C.4.2	Procedure

1)	Set the parameter of the CW generator as shown in table 6.6C.3. The spurious response frequencies are determined in step 3) of clause 6.5D.4.2.

2)	Set the power level of UE according to the table 6.6C.3 or send the power control commands (1dB step size should be used.) to the UE until UE output power measured by Test System shall be kept at the specified power level with 1dB tolerance.

3)	Measure the BLER of HS-PDSCH received from the UE at the SS by counting the number of NACK, ACK and statDTX on the UL HS-DPCCH (BLER = (NACK + statDTX)/(NACK + statDTX + ACK)).

### 6.6C.5	Test requirements

The measured BLER, derived in step 3), shall not exceed 0.1 on each individual cell. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.0.

Table 6.6C.3: Test parameters for Spurious Response (single band 4C-HSDPA)

| Parameter | Level | Unit |
| --- | --- | --- |
| HS-PDSCH_Ec | <REFSENS> +3 dB | dBm / 3,84MHz |
| Îor | <REFÎor> +3 dB | dBm / 3,84MHz |
| Iblocking(CW) | 44 | dBm |
| Fuw | Spurious response frequencies | MHz |
| UE transmitted mean power | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | dBm |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

## 6.6D	Spurious Response for dual band 4C-HSDPA

### 6.6D.1	Definition and applicability

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in subclause 6.5B.2.2 is not met.

The requirements and this test apply for Release 10 and later releases to all types of UTRA for the FDD UE that support dual band 4C-HSDPA.

Dual band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.6D.2	Minimum Requirements

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.6D.1.

Table 6.6D.1: Test Parameters for Spurious Response (Dual band 4C-HSDPA)

| Parameter | Unit | Level |
| --- | --- | --- |
| HS-PDSCH_Ec | dBm/3.84 MHz | <REFSENS> +3 dB |
| Îor | dBm/3.84 MHz | <REFÎor> +3 dB |
| Iblocking (CW) | dBm | -44 |
| Fuw | MHz | Spurious response frequencies |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |

NOTE:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1.

The normative reference for this requirement is TS 25.101 [1] clause 7.7.2.

### 6.6D.3	Test purpose

To verify that the UE BLER of HS-PDSCH does not exceed 0.1 for the parameters specified in table 6.6D.3 for the DL reference channel H-Set 1B/1C specified in Annex C.8.1.1.

The lack of the spurious response ability decreases the 4C-HSDPA coverage area when other unwanted interfering signal exists at any other frequency.

### 6.6D.4	Method of test

6.6D.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequency to be tested: the same frequency as chosen in clause 6.5F.4.1 for Blocking characteristics out-of-band case.

1)	Connect the SS to the UE antenna connector as shown in figure A.50.

2)	RF parameters are set up according to table 6.6D.3.

3)	A call is set up according to the Generic call setup procedure specified in TS 34.108 [3] sub clause 7.3.16, with exceptions for information elements listed in Table 6.6D.2. With this exception, the Power Control Algorithm for the Uplink is set to algorithm 2.

Contents of RADIO BEARER SETUP message: AM or UM (Test Loop Mode1)

Table 6.6D.2: Specific Message Contents for Spurious Response (Dual band 4C-HSDPA)

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Uplink DPCH info |  | Rel-6 |
| - Uplink DPCH power control info |  |  |
| - CHOICE mode | FDD |  |
| - Power Control Algorithm | Algorithm2 |  |

See TS 34.108 [3] and TS 34.109 [4] for details regarding generic call setup procedure and loopback test.

6.6D.4.2	Procedure

1)	Set the parameter of the CW generator as shown in table 6.6D.3. The spurious response frequencies are determined in step 3) of clause 6.5F.4.2.

2)	Set the power level of UE according to the table 6.6D.3 or send the power control commands (1dB step size should be used.) to the UE until UE output power measured by Test System shall be kept at the specified power level with 1dB tolerance.

3)	Measure the BLER of HS-PDSCH received from the UE at the SS by counting the number of NACK, ACK and statDTX on the UL HS-DPCCH (BLER = (NACK + statDTX)/(NACK + statDTX + ACK)).

### 6.6D.5	Test requirements

The measured BLER, derived in step 3), shall not exceed 0.1 on each individual cell. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.0.

Table 6.6D.3: Test parameters for Spurious Response (Dual band 4C-HSDPA)

| Parameter | Level | Unit |
| --- | --- | --- |
| HS-PDSCH_Ec | <REFSENS> +3 dB | dBm / 3,84MHz |
| Îor | <REFÎor> +3 dB | dBm / 3,84MHz |
| Iblocking(CW) | 44 | dBm |
| Fuw | Spurious response frequencies | MHz |
| UE transmitted mean power | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | dBm |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

[Many Sections are skipped here]

## 6.7D	Intermodulation Characteristics for single uplink single band 4C-HSDPA

### 6.7D.1	Definition and applicability

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

The test parameters in Tables 6.7D.1 and 6.7D.4 apply for Release 10 and later releases to all types of UTRA for the FDD UE that support single band 4C-HSDPA, single uplink operation.

Single band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.7D.2	Minimum Requirements

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.7D.1 and 6.7D.2.

Table 6.7D.1: Test parameters for receive intermodulation characteristics, single band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |

NOTE 1:	Iouw2 (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	Negative offsets refer to the assigned channel frequency of the lowest carrier frequency used and positive offsets refer to the assigned channel frequency of the highest carrier frequency used.

Table 6.7D.2: Intermodulation requirements, single band 4C-HSDPA, single uplink operation

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- |
| I-3 | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2C.1.

The normative reference for this requirement is TS 25.101 [1] clause 7.8.1C.1.

### 6.7D.3	Test purpose

To verify that the UE BLER of HS-PDSCH does not exceed 0.1 for the parameters specified in table 6.7D.4 and in table 6.7D.5 for the DL reference channel H-Set 1B/1C specified in Annex C.8.1.1.

The lack of the intermodulation response rejection ability decreases the 4C-HSDPA coverage area when two or more interfering signals, which have a specific frequency relationship to the wanted signal, exist.

### 6.7D.4	Method of test

6.7D.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

1)	Connect the SS to the UE antenna connector as shown in figure A.51.

2)	RF parameters are set up according to table 6.7D.4 and table 6.7D.5.

3)	A call is set up according to the Generic 4C-HSDPA setup procedure specified in TS 34.108 [3] sub clause 7.3.16, with  exceptions for information elements listed in Table 6.7D.3. With this exception, the Power Control Algorithm for the Uplink is set to algorithm 2.

Contents of RADIO BEARER SETUP message: AM or UM (Test Loop Mode1)

Table 6.7D.3: Specific Message Contents for Intermodulation Characteristics
(Single Uplink Single band 4C-HSDPA)

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Uplink DPCH info |  | Rel-6 |
| - Uplink DPCH power control info |  |  |
| - CHOICE mode | FDD |  |
| - Power Control Algorithm | Algorithm2 |  |

See TS 34.108 [3] and TS 34.109 [4] for details regarding generic call setup procedure and loopback test.

6.7D.4.2	Procedure

1)	Set the parameters of the CW generator and interference signal generator as shown in table 6.7D.4 and in table 6.7D.5.

2)	Set the power level of UE according to the tables 6.7D.4, and table 6.7D.5 or send the power control commands (1dB step size should be used.) to the UE until UE output power measured by Test System shall be kept at the specified power level with 1dB tolerance.

3)	Measure the BLER of HS-PDSCH received from the UE at the SS by counting the number of NACK, ACK and statDTX on the UL HS-DPCCH (BLER = (NACK + statDTX)/(NACK + statDTX + ACK)).

### 6.7D.5	Test requirements

The measured BLER, derived in step 3), shall not exceed 0.1 on each individual cell. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.0.

Table 6.7D.4: Test parameters for receive intermodulation characteristics, single band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |

NOTE 1:	Iouw2 (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	Negative offsets refer to the assigned channel frequency of the lowest carrier frequency used and positive offsets refer to the assigned channel frequency of the highest carrier frequency used.

Table 6.7D.5: Intermodulation requirements, single band 4C-HSDPA, single uplink operation

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- |
| I-3 | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2C.1.

NOTE 4:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

## 6.7E	Intermodulation Characteristics for single uplink dual band 4C-HSDPA

### 6.7E.1	Definition and applicability

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

The test parameters in Tables 6.7E.1 and 6.7E.4 apply for Release 10 and later releases to all types of UTRA for the FDD UE that support dual band 4C-HSDPA, single uplink operation.

Dual band 4C-HSDPA is designed to operate in configurations specified in clause 4.2.

### 6.7E.2	Minimum Requirements

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.7E.1 and 6.7E.2.

Table 6.7E.1: Test parameters for receive intermodulation characteristics, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |

NOTE 1:	Iouw2 (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	Negative offsets refer to the assigned channel frequency of the lowest carrier frequency used and positive offsets refer to the assigned channel frequency of the highest carrier frequency used.

Table 6.7E.2: Intermodulation requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | VIII | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | II | IV | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | V | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1.

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 6.7E.3 and Table 6.7E.4.

Table 6.7E.3: Test parameters for receive narrow-band intermodulation characteristics, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Band II, IV, V |  | Band VIII |  |
| --- | --- | --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -44 |  | -43 |  |
| Iouw2 (GMSK) | dBm | -44 |  | -43 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 3.5 | -3.5 | 3.6 | -3.6 |
| Fuw2 (offset) (NOTE 2) | MHz | 5.9 | -5.9 | 6.0 | -6.0 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |

NOTE 1:	Iouw2 (GMSK) is an interfering signal as defined in TS 45.004.

NOTE 2:	For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequency(ies) in each band.

Table 6.7E.4: Narrow-band intermodulation requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | VIII | I | <REFSENS>+16.6 dB | <REFÎor>+16.6 dB | Minimum |
|  | VIII | VIII | <REFSENS>+16.6 dB | <REFÎor>+16.6 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |
|  | IV |  | <REFSENS>+18.9 dB | <REFÎor>+18.9 dB | Minimum |
|  | II | IV | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |
|  | IV |  | <REFSENS>+18.9 dB | <REFÎor>+18.9 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | V | I | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |
|  | V | V | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1for dual band 4C-HSDPA.

The normative reference for this requirement is TS 25.101 [1] clause 7.8.1D.1and clause 7.8.2D.1.

### 6.7E.3	Test purpose

To verify that the UE BLER of HS-PDSCH does not exceed 0.1 for the parameters specified in table 6.7E.6 through 6.7E.9 for the DL reference channel H-Set 1B/1C specified in Annex C.8.1.1.

The lack of the intermodulation response rejection ability decreases the 4C-HSDPA coverage area when two or more interfering signals, which have a specific frequency relationship to the wanted signal, exist.

### 6.7E.4	Method of test

6.7E.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

1)	Connect the SS to the UE antenna connector as shown in figure A.51.

2)	RF parameters are set up according to table 6.7E.6 and table 6.7E.8.

3)	A call is set up according to the Generic 4C-HSDPA setup procedure specified in TS 34.108 [3] sub clause 7.3.16, with exceptions for information elements listed in Table 6.7E.5. With this exception, the Power Control Algorithm for the Uplink is set to algorithm 2.

Contents of RADIO BEARER SETUP message: AM or UM (Test Loop Mode1)

Table 6.7E.5: Specific Message Contents for Intermodulation Characteristics (Single Uplink Single band 4C--HSDPA)

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Uplink DPCH info |  | Rel-6 |
| - Uplink DPCH power control info |  |  |
| - CHOICE mode | FDD |  |
| - Power Control Algorithm | Algorithm2 |  |

See TS 34.108 [3] and TS 34.109 [4] for details regarding generic call setup procedure and loopback test.

6.7E.4.2	Procedure

1)	Set the parameters of the CW generator and interference signal generator as shown in table 6.7E.4 and in table 6.7E.5.

2)	Set the power level of UE according to the tables 6.7E.6, 6.7E.7, 6.7E.8 and table 6.7E.9 or send the power control commands (1dB step size should be used.) to the UE until UE output power measured by Test System shall be kept at the specified power level with 1dB tolerance.

3)	Measure the BLER of HS-PDSCH received from the UE at the SS by counting the number of NACK, ACK and statDTX on the UL HS-DPCCH (BLER = (NACK + statDTX)/(NACK + statDTX + ACK)).

### 6.7E.5	Test requirements

The measured BLER, derived in step 3), shall not exceed 0.1 on each individual cell. The minimum number of measurements required for a statistically significant result to this test is clarified in annex F.6.3, Table F.6.3.5.0.

Table 6.7E.6: Test parameters for receive intermodulation characteristics, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |

NOTE 1:	Iouw2 (modulated) consists of the common channels needed for tests as specified in Table E.4.1 and 16 dedicated data channels as specified in Table E.3.6.

NOTE 2:	Negative offsets refer to the assigned channel frequency of the lowest carrier frequency used and positive offsets refer to the assigned channel frequency of the highest carrier frequency used.

Table 6.7E.7: Intermodulation requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | VIII | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | II | IV | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | V | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |

Table 6.7E.8: Test parameters for receive narrow-band intermodulation characteristics, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Band II, IV, V |  | Band VIII |  |
| --- | --- | --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -44 |  | -43 |  |
| Iouw2 (GMSK) | dBm | -44 |  | -43 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 3.5 | -3.5 | 3.6 | -3.6 |
| Fuw2 (offset) (NOTE 2) | MHz | 5.9 | -5.9 | 6.0 | -6.0 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |

NOTE 1:	Iouw2 (GMSK) is an interfering signal as defined in TS 45.004.

NOTE 2:	For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequency(ies) in each band.

Table 6.7E.8: Narrow-band intermodulation requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | VIII | I | <REFSENS>+16.6 dB | <REFÎor>+16.6 dB | Minimum |
|  | VIII | VIII | <REFSENS>+16.6 dB | <REFÎor>+16.6 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |
|  | IV |  | <REFSENS>+18.9 dB | <REFÎor>+18.9 dB | Minimum |
|  | II | IV | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |
|  | IV |  | <REFSENS>+18.9 dB | <REFÎor>+18.9 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | V | I | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |
|  | V | V | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |

NOTE 3:	<REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 6.2D.1for dual band 4C-HSDPA.

NOTE 4:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.