3GPP TSG-RAN WG5 Meeting #58 	R5-130492

# Malta, MT, 28 January – 01 February 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.121-1 | CR | CRNum | rev | - | Current version: | 10.5.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annexure update to test tolerance and statistical testing for 4C-HSDPA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RANimp-4C_HSDPA_UEConTest |  |  |  |  |  |  | Date: |  | 2013-01-18 |
| Category: |  |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Test cases for Chapter 6 and Chapter 9 were added in RAN 5#57, test tolerance and statistical testing related information had to be updated… |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test tolerances and statistical testing information to section F.2.2, F.4.2,Table F.6.3.5.0, Table F.6.3.5.1 and Table F.6.3.5.1A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Item for 4C-HSDPA will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.2.2, F.2.4, Table F.6.3.5.0, Table F.6.3.5.1 and Table F.6.3.5.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 34.121-1... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Draft version of 4C-HSDPA applicable test cases were added in meeting #57 |  |  |  |  |  |  |  |  |

[Beginning of change modification]

# F.2	Test Tolerances (This clause is informative)

The Test Tolerances defined in this clause have been used to relax the Minimum Requirements in the present document to derive the Test Requirements.

The Test Tolerances are derived from Test System uncertainties, regulatory requirements and criticality to system performance. As a result, the Test Tolerances may sometimes be set to zero.

The test tolerances should not be modified for any reason e.g. to take account of commonly known test system errors (such as mismatch, cable loss, etc.).

## F.2.1	Transmitter

Table F.2.1: Test Tolerances for transmitter tests

| Clause | Test Tolerance |
| --- | --- |
| 5.2 Maximum Output Power | 0.7 dB |
| 5.2A Maximum Output Power with HS-DPCCH (Release 5 only) | 0.7 dB |
| 5.2AA Maximum Output Power with HS-DPCCH (Release 6 and later) | 0.7 dB |
| 5.2B Maximum Output Power with HS-DPCCH and E-DCH | 0.7 dB |
| 5.2BA UE Maximum Output Power for DC-HSUPA (QPSK) | 0.7 dB |
| 5.2BB UE Maximum Output Power for DC-HSUPA (16QAM) | 0.7 dB |
| 5.2C UE relative code domain power accuracy | For 0 dB ≥ -10 dB CDP 0.2 dB For -10 dB ≥ -15 dB CDP 0.3 dB For -15 dB ≥ -20 dB CDP 0.4 dB |
| 5.2D UE Relative Code Domain Power Accuracy with HS-DPCCH and E-DCH | For 0 dB ≥ -10 dB CDP 0.2 dB For -10 dB ≥ -15 dB CDP 0.3 dB For -15 dB ≥ -20 dB CDP 0.4 dB |
| 5.2DA UE Relative Code Domain Power Accuracy for DC-HSUPA with QPSK | For 0 dB ≥ -10 dB CDP 0.2 dB For -10 dB ≥ -15 dB CDP 0.3 dB For -15 dB ≥ -20 dB CDP 0.4 dB |
| 5.2E UE Relative Code Domain Power Accuracy for HS-DPCCH and E-DCH with 16QAM | For 0 dB ≥ -10 dB CDP 0.2 dB For -10 dB ≥ -15 dB CDP 0.3 dB For -15 dB ≥ -20 dB CDP 0.4 dB For -20 dB ≥ -30 dB CDP 0.5 dB |
| 5.2 EA UE relative code domain power accuracy for DC-HSUPA using HS-DPCCH and E-DCH with 16QAM | For 0 dB ≥ -10 dB CDP 0.2 dB For -10 dB ≥ -15 dB CDP 0.3 dB For -15 dB ≥ -20 dB CDP 0.4 dB For -20 dB ≥ -30 dB CDP 0.5 dB |
| 5.3 Frequency error | 10 Hz |
| 5.3A Frequency Error for DC-HSUPA | 10 Hz per carrier |
| 5.4.1 Open loop power control in uplink | 1.0 dB |
| 5.4.1A Open Loop Power Control in the Uplink for DC-HSUPA | 1.0 dB per carrier |
| 5.4.2 Inner loop power control in the uplink | 0.1 dB (1 dB and 0 dB range) 0.15 dB (2 dB range) 0.2 dB (3 dB range 0.3 dB (> 3 dB range)) |
| 5.4.2A Inner Loop Power Control in the Uplink for DC-HSUPA | 0.1 dB per carrier (1 dB and 0 dB range) 0.15 dB per carrier (2 dB range) 0.2 dB per carrier (3 dB range 0.3 dB per carrier (> 3 dB range)) |
| 5.4.3 Minimum Output Power | 1.0 dB |
| 5.4.3A Minimum Output Power for DC-HSUPA | 1.0 dB |
| 5.4.4 Out-of-synchronisation handling of output power: | 0.4 dB |
| 5.4.4 Out-of-synchronisation handling of output power: transmit ON/OFF time | 0 ms |
| 5.4.4A Out-of-synchronisation handling of output power for a UE which supports type 1 for DCH: | 0.4 dB |
| 5.4.4 A Out-of-synchronisation handling of output power for a UE which supports type 1 for DCH: transmit ON/OFF time | 0 ms |
| 5.5.1 Transmit OFF power | 1.0 dB |
| 5.5.2 Transmit ON/OFF time mask (dynamic case) | On power +0.7 dB / -1.0 dB Off power TT 1.0 dB |
| 5.6 Change of TFC: power control step size | 0.3 dB |
| 5.7 Power setting in uplink compressed mode:- UE output power | See subset of 5.4.2 |
| 5.7A HS-DPCCH | 0.1 dB (1 dB and 0 dB range) 0.15 dB (2 dB range) 0.2 dB (3 dB range) 0.3 dB (> 3 dB range) |
| 5.8 Occupied Bandwidth | 0 kHz |
| 5.8A Occupied Bandwidth for DC-HSUPA | 0 kHz |
| 5.9 Spectrum emission mask | 1.5 dB (0 dB for additional requirements for Band II, Band IV, Band V and Band X only) |
| 5.9A Spectrum emission mask with HS-DPCCH | 1.5 dB (0 dB for additional requirements for Band II, Band IV, Band V and Band X only) |
| 5.9B Spectrum emission mask with E-DCH | 1.5 dB (0 dB for additional requirements for Band II, Band IV and Band V only) |
| 5.9C Additional Spectrum Emission Mask for DC-HSUPA (QPSK) | 1.5 dB (0 dB for additional requirements for band II, IV, V and X only) |
| 5.9D Additional Spectrum Emission Mask for DC-HSUPA (16QAM) | 1.5 dB (0 dB for additional requirements for band II, IV, V and X only) |
| 5.10 ACLR | 0.8 dB for ratio 0.0 dB for absolute power |
| 5.10A ACLR with HS-DPCCH | 0.8 dB for ratio 0.0 dB for absolute power |
| 5.10B ACLR with E-DCH | 0.8 dB for ratio 0.0 dB for absolute power |
| 5.10C ACLR with E-DCH for DC-HSUPA (QPSK) | 0.8 dB for ratio 0.0 dB for absolute power |
| 5.10D ACLR with E-DCH for DC-HSUPA (16QAM) | 0.8 dB for ratio 0.0 dB for absolute power |
| 5.11 Spurious emissions | 0 dB |
| 5.11A Spurious emissions for DC-HSUPA | 0 dB |
| 5.12 Transmit Intermodulation | 0 dB |
| 5.12A Transmit Intermodulation for DC-HSUPA | 0 dB |
| 5.13.1 Transmit modulation: EVM | 0% |
| 5.13.1A Transmit modulation: EVM with HS-DPCCH | 0% |
| 5.13.1AA Transmit modulation: EVM and phase discontinuity with HS-DPCCH | 0% EVM 6 degress phase discontinuity |
| 5.13.1AAA EVM and IQ origin offset for HS-DPCCH with E-DCH with 16 QAM | ±0.5 dB (for IQ origin offset) |
| 5.13.2 Transmit modulation: peak code domain error | 1.0 dB |
| 5.13.2A Relative Code Domain Error | ±0.5 dB |
| 5.13.2B Relative Code Domain Error with HS-DPCCH and E-DCH | ±0.5 dB |
| 5.13.2B Relative Code Domain Error with HS-DPCCH and E-DCH for DC-HSUPA | ±0.5 dB |
| 5.13.2C Relative Code Domain Error for HS-DPCCH and E-DCH with 16QAM | ±0.5 dB |
| 5.13.3 UE phase discontinuity | 0% for EVM |
|  | 10 Hz for Frequency error |
|  | 6 degree for Phase discontinuity |
| 5.13.4 PRACH preamble quality (EVM) | 0% |
| 5.13.4 PRACH preamble quality (Frequency error) | 10 Hz |
| 5.13.5 In-band emission for DC-HSUPA | 0.8 dB |

## F.2.2	Receiver

Table F.2.2: Test Tolerances for receiver tests

| Clause | Test Tolerance |
| --- | --- |
| 6.2 Reference sensitivity level | 0.7 dB for Ior and Ec |
| 6.2A Reference sensitivity level for DC-HSDPA | 0.7 dB for Ior and Ec (for both DC-HSDPA cells) |
| 6.2B Reference sensitivity level for DB-DC-HSDPA | 0.7 dB for Ior and Ec (for both DB-DC-HSDPA cells) |
| 6.2C Reference sensitivity level for single band 4C-HSDPA | 0.7 dB for Ior and Ec (for all 3C/4C-HSDPA cells) |
| 6.2D Reference sensitivity level for Dual band 4C-HSDPA | 0.7 dB for Ior and Ec (for all 3C/4C-HSDPA cells) |
| 6.3 Maximum input level: | 0.7 dB for Ior |
| 6.3A Maximum Input Level for HS-PDSCH Reception (16QAM) | 0.7 dB for Ior |
| 6.3B Maximum Input Level for HS-PDSCH Reception (64QAM) | 0.7 dB for Ior |
| 6.3C Maximum Input Level for DC-HSDPA Reception (16QAM) | 0.7 dB for Ior (for both DC-HSDPA cells) |
| 6.3D Maximum Input Level for DC-HSDPA Reception (64QAM) | 0.7 dB for Ior (for both DC-HSDPA cells) |
| 6.3E Maximum Input Level for DB-DC-HSDPA Reception (16QAM) | 0.7 dB for Ior (for both DB-DC-HSDPA cells) |
| 6.3F Maximum Input Level for DB-DC-HSDPA Reception (64QAM) | 0.7 dB for Ior (for both DB-DC-HSDPA cells) |
| 6.3G Maximum Input Level for 4C-HSDPA Reception (16QAM) | 0.7 dB for Ior and Ec (for all 3C/4C-HSDPA cells) |
| 6.3H Maximum Input Level for 4C-HSDPA Reception (64QAM) | 0.7 dB for Ior and Ec (for all 3C/4C-HSDPA cells) |
| 6.4 Adjacent channel selectivity (Rel-99 and Rel-4) | 0 dB |
| 6.4A Adjacent channel selectivity (Rel-5 and later releases) | 0 dB |
| 6.4B Adjacent channel selectivity (ACS) for DC-HSDPA | 0 dB |
| 6.4C Adjacent channel selectivity (ACS) for DB-DC-HSDPA | 0 dB |
| 6.5 Blocking characteristics | 0 dB |
| 6.5A Blocking characteristics for DC-HSDPA | 0 dB |
| 6.5B Blocking characteristics for DB-DC-HSDPA | 0 dB |
| 6.5C Blocking characteristics for DC-HSUPA | 0 dB |
| 6.5D Blocking Characteristics for single Uplink Single band 4C-HSDPA | 0 dB |
| 6.5E Blocking Characteristics for dual Uplink Single band 4C-HSDPA | 0 dB |
| 6.5F Blocking Characteristics for single Uplink Dual band 4C-HSDPA | 0 dB |
| 6.5G Blocking Characteristics for dual Uplink Dual band 4C-HSDPA | 0 dB |
| 6.6 Spurious Response | 0 dB |
| 6.6A Spurious Response for DC-HSDPA | 0 dB |
| 6.6B Spurious Response for DB-DC-HSDPA | 0 dB |
| 6.6C Spurious Response for single band 4C-HSDPA | 0 dB |
| 6.6D Spurious Response for dual band 4C-HSDPA | 0 dB |
| 6.7 Intermodulation Characteristics | 0 dB |
| 6.7A Intermodulation Characteristics for DC-HSDPA | 0 dB |
| 6.7B Intermodulation Characteristics for DB-DC-HSDPA | 0 dB |
| 6.7C Intermodulation Characteristics for DC-HSUPA | 0 dB |
| 6.7D Intermodulation Characteristics for single uplink single band 4C-HSDPA | 0 dB |
| 6.7E Intermodulation Characteristics for single uplink dual band 4C-HSDPA | 0 dB |
| 6.8 Spurious emissions | 0 dB |

[Many Sections are skipped here]

# F.4	Derivation of Test Requirements (This clause is informative)

The Test Requirements in the present document have been calculated by relaxing the Minimum Requirements of the core specification using the Test Tolerances defined in clause F.2. When the Test Tolerance is zero, the Test Requirement will be the same as the Minimum Requirement. When the Test Tolerance is non-zero, the Test Requirements will differ from the Minimum Requirements, and the formula used for this relaxation is given in table F.4.

[Many Sections are skipped here]

## F.4.2	Receiver

Table F.4.2: Derivation of Test Requirements (Receiver tests)

| Test | Minimum Requirement in TS 25.101 |  | Test Tolerance (TT) | Test Requirement in TS 34.121 |  |
| --- | --- | --- | --- | --- | --- |
| 6.2 Reference sensitivity level | Îor = -106.7 dBm / 3.84 MHz DPCH_Ec = 117 dBm / 3.84 MHz BER limit = 0.001 |  | 0.7 dB | Formula: Îor Minimum Requirement + TT DPCH_Ec + TT BER limit unchanged Îor = -106 dBm / 3.84 MHz DPCH_Ec = -116.3 dBm / 3.84 MHz |  |
| 6.2A Reference sensitivity level for DC-HSDPA | Îor = -102.7 dBm / 3.84 MHz HS-PDSCH_Ec = 113 dBm / 3.84 MHz BLER limit = 0.1 |  | 0.7 dB | Formula: Îor: Minimum Requirement + TT HS-PDSCH_Ec Minimum Requirement +TT BLER limit unchanged Îor = -102 dBm / 3.84 MHz HS-PDSCH_Ec = -112.3 dBm / 3.84 MHz |  |
| 6.2B Reference sensitivity level for DB-DC-HSDPA | Îor = -102.7 dBm / 3.84 MHz HS-PDSCH_Ec = 113 dBm / 3.84 MHz BLER limit = 0.1 |  | 0.7 dB | Formula: Îor: Minimum Requirement + TT HS-PDSCH_Ec Minimum Requirement +TT BLER limit unchanged Îor = -102 dBm / 3.84 MHz HS-PDSCH_Ec = -112.3 dBm / 3.84 MHz |  |
| 6.2C Reference Sensitivity Level for Single band 4C-HSDPA | Îor = -102.7 dBm / 3.84 MHz HS-PDSCH_Ec = 113 dBm / 3.84 MHz BLER limit = 0.1 |  | 0.7 dB | Formula: Îor: Minimum Requirement + TT HS-PDSCH_Ec Minimum Requirement +TT BLER limit unchanged Îor = -102 dBm / 3.84 MHz HS-PDSCH_Ec = -112.3 dBm / 3.84 MHz |  |
| 6.2D Reference Sensitivity Level for Dual band 4C-HSDPA | Îor = -102.7 dBm / 3.84 MHz HS-PDSCH_Ec = 113 dBm / 3.84 MHz BLER limit = 0.1 |  | 0.7 dB | Formula: Îor: Minimum Requirement + TT HS-PDSCH_Ec Minimum Requirement +TT BLER limit unchanged Îor = -102 dBm / 3.84 MHz HS-PDSCH_Ec = -112.3 dBm / 3.84 MHz |  |
| 6.3 Maximum input level | -25 dBm Ior -19 dBc DPCH_Ec/Ior |  | 0.7 dB | Formula: Ior Minimum Requirement -TT Ior = -25.7 dBm |  |
| 6.3A Maximum Input Level for HS-PDSCH Reception (16QAM) | -25 dBm Ior |  | 0.7 dB | Formula: Minimum Requirement -TT Ior = -25.7 dBm |  |
| 6.3B Maximum Input Level for HS-PDSCH Reception (64QAM) | -25 dBm Ior |  | 0.7 dB | Formula: Minimum Requirement -TT Ior = -25.7 dBm |  |
| 6.3C Maximum Input Level for DC-HSDPA Reception (16QAM) | -25 dBm Ior |  | 0.7 dB | Formula: Minimum Requirement -TT Ior = -25.7 dBm |  |
| 6.3D Maximum Input Level for DC-HSDPA Reception (64QAM) | -25 dBm Ior |  | 0.7 dB | Formula: Minimum Requirement -TT Ior = -25.7 dBm |  |
| 6.3E Maximum Input Level for DB-DC-HSDPA Reception (16QAM) | -25 dBm Ior |  | 0.7 dB | Formula: Minimum Requirement -TT Ior = -25.7 dBm |  |
| 6.3F Maximum Input Level for DB-DC-HSDPA Reception (64QAM) | -25 dBm Ior |  | 0.7 dB | Formula: Minimum Requirement -TT Ior = -25.7 dBm |  |
| 6.3G Maximum Input Level for 4C-HSDPA Reception (16QAM) | -25 dBm Ior |  | 0.7 dB | Formula: Minimum Requirement -TT Ior = -25.7 dBm |  |
| 6.3H Maximum Input Level for 4C-HSDPA Reception (64QAM) | -25 dBm Ior |  | 0.7 dB | Formula: Minimum Requirement -TT Ior = -25.7 dBm |  |
| 6.4 Adjacent Channel Selectivity (Rel-99 and Rel-4) | Îor = -92.7 dBm / 3.84 MHz DPCH_Ec = 103 dBm / 3.84 MHz Ioac (modulated) = -52 dBm/3.84 MHz BER limit = 0.001 |  | 0 dB | Formula: Îor unchanged DPCH_Ec unchanged Ioac Minimum Requirement – TT BER limit unchanged Ioac = -52 dBm/3.84 MHz |  |
| 6.4A Adjacent Channel Selectivity (Rel-5 and later releases) | Case 1: Îor = <REFÎor> + 14 dB / 3.84 MHz DPCH_Ec = <REFSENS> + 14 dB / 3.84 MHz Ioac (modulated) = -52 dBm/3.84 MHz BER limit = 0.001 Case 2: Îor = <REFÎor> + 41 dB / 3.84 MHz DPCH_Ec = <REFSENS> + 41 dB / 3.84 MHz Ioac (modulated) = -25 dBm/3.84 MHz BER limit = 0.001 |  | 0 dB | Formula: Îor unchanged DPCH_Ec unchanged Ioac Minimum Requirement – TT BER limit unchanged Case1: Ioac = -52 dBm/3.84 MHz Case2: Ioac = -25 dBm/3.84 MHz |  |
| 6.4B Adjacent channel selectivity (ACS) for DC-HSDPA | Case 1: Îor = <REFÎor> + 14 dB / 3.84 MHz HS-PDSCH_Ec = <REFSENS> + 14 dB / 3.84 MHz Ioac (modulated) = -52 dBm/3.84 MHz BLER limit = 0.1 Case 2: Îor = <REFÎor> + 41 dB / 3.84 MHz HS-PDSCH_Ec = <REFSENS> + 41 dB / 3.84 MHz Ioac (modulated) = -25 dBm/3.84 MHz BLER limit = 0.1 |  | 0 dB | Formula: Îor unchanged DPCH_Ec unchanged Ioac Minimum Requirement – TT BLER limit unchanged Case1: Ioac = -52 dBm/3.84 MHz Case2: Ioac = -25 dBm/3.84 MHz |  |
| 6.5 Blocking Characteristics | See Table 6.5.1, 6.5.2 and 6.5.3. in TS34.121 BER limit = 0.001 |  | 0 dB | Formula: I blocking (modulated) Minimum Requirement - TT (dBm/3.84MHz) I blocking (CW) Minimum Requirement - TT (dBm) BER limit unchanged |  |
| 6.5A Blocking characteristics for DC-HSDPA | See Table 6.5A.1, 6.5A.2 and 6.5A.3 in TS34.121 BLER limit = 0.1 |  | 0 dB | Formula: I blocking (modulated) Minimum Requirement - TT (dBm/3.84MHz) I blocking (CW) Minimum Requirement - TT (dBm) BLER limit unchanged |  |
| 6.5B Blocking characteristics for DB-DC-HSDPA | See Table 6.5B.1, 6.5B.2 and 6.5B.3 in TS34.121 BLER limit = 0.1 |  | 0 dB | Formula: I blocking (modulated) Minimum Requirement - TT (dBm/3.84MHz) I blocking (CW) Minimum Requirement - TT (dBm) BLER limit unchanged |  |
| 6.5C Blocking characteristics for DC-HSUPA | See Table 6.5C.1 and 6.5C.3 in TS34.121 BLER limit = 0.1 |  | 0 dB | Formula: I blocking (modulated) Minimum Requirement - TT (dBm/3.84MHz) I blocking (CW) Minimum Requirement - TT (dBm) BLER limit unchanged |  |
| 6.5D Blocking Characteristics for single Uplink Single band 4C-HSDPA | See Table 6.5D.1, 6.5D.2 and 6.5D.3 in TS34.121 BLER limit = 0.1 |  | 0 dB | Formula: I blocking (modulated) Minimum Requirement - TT (dBm/3.84MHz) I blocking (CW) Minimum Requirement - TT (dBm) BLER limit unchanged |  |
| 6.5E Blocking Characteristics for dual Uplink Single band 4C-HSDPA | See Table 6.5E.1 and 6.5E.3 in TS34.121 BLER limit = 0.1 |  | 0 dB | Formula: I blocking (modulated) Minimum Requirement - TT (dBm/3.84MHz) I blocking (CW) Minimum Requirement - TT (dBm) BLER limit unchanged |  |
| 6.5F Blocking Characteristics for single Uplink Dual band 4C-HSDPA | See Table 6.5F.1, 6.5F.2 and 6.5F.3 in TS34.121 BLER limit = 0.1 |  | 0 dB | Formula: I blocking (modulated) Minimum Requirement - TT (dBm/3.84MHz) I blocking (CW) Minimum Requirement - TT (dBm) BLER limit unchanged |  |
| 6.5G Blocking Characteristics for dual Uplink Dual band 4C-HSDPA | See Table 6.5G.1 and 6.5G.3 in TS34.121 BLER limit = 0.1 |  | 0 dB | Formula: I blocking (modulated) Minimum Requirement - TT (dBm/3.84MHz) I blocking (CW) Minimum Requirement - TT (dBm) BLER limit unchanged |  |
| 6.6 Spurious Response | Iblocking(CW) –44 dBm Fuw: Spurious response frequencies BER limit = 0.001 |  | 0 dB | Formula: I blocking (CW) Minimum Requirement - TT (dBm) Fuw unchanged BER limit unchanged Iblocking(CW) = -44 dBm |  |
| 6.6A Spurious Response for DC-HSDPA | Iblocking(CW) –44 dBm Fuw: Spurious response frequencies BLER limit = 0.1 |  | 0 dB | Formula: I blocking (CW) Minimum Requirement - TT (dBm) Fuw unchanged BLER limit unchanged Iblocking(CW) = -44 dBm |  |
| 6.6B Spurious Response for DB-DC-HSDPA | Iblocking(CW) –44 dBm Fuw: Spurious response frequencies BLER limit = 0.1 |  | 0 dB | Formula: I blocking (CW) Minimum Requirement - TT (dBm) Fuw unchanged BLER limit unchanged Iblocking(CW) = -44 dBm |  |
| 6.6C Spurious Response for single band 4C-HSDPA | Iblocking(CW) –44 dBm Fuw: Spurious response frequencies BLER limit = 0.1 |  | 0 dB | Formula: I blocking (CW) Minimum Requirement - TT (dBm) Fuw unchanged BLER limit unchanged Iblocking(CW) = -44 dBm |  |
| 6.6D Spurious Response for dual band 4C-HSDPA | Iblocking(CW) –44 dBm Fuw: Spurious response frequencies BLER limit = 0.1 |  | 0 dB | Formula: I blocking (CW) Minimum Requirement - TT (dBm) Fuw unchanged BLER limit unchanged Iblocking(CW) = -44 dBm |  |
| 6.7 Intermodulation Characteristics | Iouw1 (CW) -46 dBm Iouw2 (modulated) –46 dBm / 3.84 MHz Fuw1 (offset) 10 MHz Fuw2 (offset) 20 MHz Ior = -103.7 dBm/3.84 MHz DPCH_Ec = -114 dBm/3.84 MHz BER limit = 0.001 |  | 0 dB | Formula: Ior Minimum Requirement + TT DPCH_Ec + TT Iouw1 level unchanged Iouw2 level unchanged BER limit unchanged. |  |
| 6.7A Intermodulation Characteristics for DC-HSDPA | Iouw1 (CW) -46 dBm Iouw2 (modulated) –46 dBm / 3.84 MHz Fuw1 (offset) 10 MHz Fuw2 (offset) 20 MHz Ior = -99.7 dBm/3.84 MHz HS-PDSCH_Ec = -110 dBm/3.84 MHz BER limit = 0.1 |  | 0 dB | Formula: Ior Minimum Requirement + TT HS-PDSCH_Ec + TT Iouw1 level unchanged Iouw2 level unchanged BLER limit unchanged. |  |
| 6.7B Intermodulation Characteristics for DB-DC-HSDPA | Iouw1 (CW) -46 dBm Iouw2 (modulated) –46 dBm / 3.84 MHz Fuw1 (offset) 10 MHz Fuw2 (offset) 20 MHz Ior = -99.7 dBm/3.84 MHz HS-PDSCH_Ec = -110 dBm/3.84 MHz BER limit = 0.1 |  | 0 dB | Formula: Ior Minimum Requirement + TT HS-PDSCH_Ec + TT Iouw1 level unchanged Iouw2 level unchanged BLER limit unchanged. |  |
| 6.7C Intermodulation Characteristics for DC-HSUPA | Iouw1 (CW) -46 dBm Iouw2 (modulated) –46 dBm / 3.84 MHz Fuw1 (offset) 10 MHz Fuw2 (offset) 20 MHz Ior = -99.7 dBm/3.84 MHz HS-PDSCH_Ec = -110 dBm/3.84 MHz BER limit = 0.1 |  | 0 dB | Formula: Ior Minimum Requirement + TT HS-PDSCH_Ec + TT Iouw1 level unchanged Iouw2 level unchanged BLER limit unchanged. |  |
| 6.7D Intermodulation Characteristics for single uplink single band 4C-HSDPA | Iouw1 (CW) -46 dBm Iouw2 (modulated) –46 dBm / 3.84 MHz Fuw1 (offset) 10 MHz Fuw2 (offset) 20 MHz Ior = -99.7 dBm/3.84 MHz HS-PDSCH_Ec = -110 dBm/3.84 MHz BER limit = 0.1 |  | 0 dB | Formula: Ior Minimum Requirement + TT HS-PDSCH_Ec + TT Iouw1 level unchanged Iouw2 level unchanged BLER limit unchanged. |  |
| 6.7E Intermodulation Characteristics for single uplink dual band 4C-HSDPA | Iouw1 (CW) -46 dBm Iouw2 (modulated) –46 dBm / 3.84 MHz Fuw1 (offset) 10 MHz Fuw2 (offset) 20 MHz Ior = -99.7 dBm/3.84 MHz HS-PDSCH_Ec = -110 dBm/3.84 MHz BER limit = 0.1 |  | 0 dB | Formula: Ior Minimum Requirement + TT HS-PDSCH_Ec + TT Iouw1 level unchanged Iouw2 level unchanged BLER limit unchanged. |  |
| 6.8 Spurious Emissions |  |  |  | Formula: Maximum level + TT Add zero to all the values of Maximum Level in table 6.8.1. |  |
|  | Frequency Band | Maximum level |  | Frequency Band | Maximum level |
|  | 9kHz f 1GHz | -57dBm /100kHz | 0 dB | 9kHz f 1GHz | -57dBm /100kHz |
|  | 1GHz f 12.75GHz | -47dBm /1MHz | 0 dB | 1GHz f 2.2GHz | -47dBm /1MHz |
|  |  |  | 0 dB | 2.2GHz f 4GHz | -47dBm /1MHz |
|  |  |  | 0 dB | 4GHz f 12.75GHz | -47dBm /1MHz |
|  | 1920MHz f 1980MHz | -60dBm /3.84MHz | 0 dB | 1920MHz f 1980MHz | -60dBm /3.84MHz |
|  | 2110MHz f 2170MHz | -60dBm /3.84MHz | 0 dB | 2110MHz f 2170MHz | -60dBm /3.84MHz |

[Many Sections are skipped here]

## F.6.3	Statistical Testing of HSDPA Receiver Performance

### F.6.3.1	Definition

Information Bit Throughput R:

The measured information bit throughput R is defined as the sum (in kilobits) of the information bit payloads (excluding the 24-bit HS-DSCH CRC) successfully received during the test interval, divided by the duration of the test interval (in seconds).

### F.6.3.2	Mapping throughput to block error ratio

a)	In measurement practice the UE indicates successfully received information bit payload by signalling an ACK to the SS.
If payload is received, but damaged and cannot be decoded, the UE signals a NACK.

b)	Only the ACK and NACK signals, not the data bits received, are accessible to the SS.
The number of bits is known in the SS from knowledge of what payload was sent.

c)	For fixed reference channel the number of bits in a TTI is fixed during one test.

d)	The time in the measurement interval is composed of successful TTIs (ACK), unsuccessful TTIs (NACK) and DTX-TTIs.

e)	DTX-TTIs occur regularly according to the H-set. (regDTX).
In real live this is the time when other UEs are served.
regDTX vary from test to test but are fixed within the test.

f)	Additional DTX-TTIs occur statistically when the UE is not responding ACK or NACK where it should. (statDTX)
This may happen when the UE was not expecting data or decided that the data were not intended for it.

The pass / fail decision is done by observing the:

-	number of NACKs

-	number of ACKs and

-	number of statDTXs (regDTX is implicitly known to the SS)

The ratio (NACK + statDTX)/(NACK+ statDTX +ACK)is the Bock Error Ratio BLER. Taking into account the time consumed by the ACK-, NACK-, and DTX-TTIs (regular and statistical), BLER can be mapped unambiguously to throughput for any single FRC test.

### F.6.3.3	Bad DUT factor

NOTE:	Data throughput in a communication system is of statistical nature and must be measured and decided pass or fail. The specified  limit of throughput related to the ideal throughput in different throughput tests is in the range of a few % to near 100%. To make it comparable with BER, we define the complement of the relative throughput:  BLER as defined above. Complementary this is in the range of near 100% down to a few %  For e.g. BLER = 1%, the currently in BER BLER used Bad DUT factor M=1.5 is highly meaningful. For e.g. BLER = 99%, the currently used M=1.5 obviously meaningless.

An appropriate definition of the bad DUT factor is illustrated in figure F.6.3.3: constant and variable Bad DUT factor.

It illustrates how to find the Bad BLER when the nominal BLER is given.

1)	In the range 0%< nominal BLER>10% the Bad DUT factor is constant 1.5

2)	In the range 90%< bad BLER>100% it decreases to 1. (symmetrical to (1))

3)	The range in between is interpolated by an arc section.

The example shows: nominal BLER=35,6%  bad BLER=47.67.5%   M=1.34

(blue mapping)

0      10     20    30      40    50%                             nominal BLER

Bad (1-BLER)

0    10     20     30     40     50     60%                 bad BLER

nominal (1-BLER)

Figure F.6.3.3: constant and variable Bad DUT factor

Formula:

For 0 < BLER<= 0.1 M = 1.5

For 0.1 <BLER <.9

For 0.9 <= BLER < 1 M(BLER)= 2/3BLER + 1/3

With BLER: nominal Block Error Ratio (0<BLER<1)

With r = 2.70415 (Radius of the arc)

#### F.6.3.3.1	Bad DUT factor, range of applicability

Inaccuracy is one practical reason to avoid the grey shaded area of figure F.6.3.3: constant and variable Bad DUT factor.  For BLER near 1 the Bad DUT factor M is near 1. For M=1,exactly, the pass and fail criteria do not intersect. The test never is finalised.

For M near 1 the pass and fail criteria exhibit a very smooth intersection. In addition the binomial distribution and its inverse are of discrete nature. Therefore the test limit and the number of samples is calculable only very ambiguous.

It is proposed to apply the bad DUT factor only in the not shaded area of figure F.6.3.3.

This is done by the following:

BLER mode:

Use BLER as defined above in the range of 0 to 50%, use M >1 as defined above.

The Test Limit  will be > the Test Requirement in the table F.6.3.5. below.

Relative Throughput mode:

If BLER is in the range 50 to 100%, use 1-BLER instead. Use m<1 instead of M.

1-BLER is the relative throughput with respect to the ideal throughput.

As a consequence, the Test Limit  <  Test Requirement

Formula for m:

For 0 < (1-BLER) <= 0.15 m = 1/1.5

For 0.15 <(1-BLER) <.85

In figure F.6.3.3 this is represented by the red mapping.

The tables F.6.3.5... below distinguish between m and M.

### F.6.3.4	Minimum Test time

Same as with BER BLER there is a minimum test time is necessary for multipath fading profiles with the same justification:

Table F.6.3.4 Applicability and characteristics of the Tables F.6.3.5. 0 to F.6.3.5.4. 10

| profile | Minimum Test time |
| --- | --- |
| PA3, PB3 | 164s |
| VA30, Case 8 | 16.4s |
| VA 120 | 4.1s |

The purpose of tables F.6.3.5.0 to F.6.3.5.4.10 is to decide throughput pass or fail.

(the Ior/Ioc levels are only for reference)

Meaning of a decision:

-	A passed DUT is not worse than a Bad DUT with 95% confidence level.

-	A failed DUT is not better than a Limit DUT with 95% confidence level.

The minimum Test Time is

1)	the minimum test time due to statistical reasons

( To ensure the confidence level, the test must be continued until a certain number of samples (NACK+ statDTX +ACK) is reached.)

2)	the minimum test time due to multipath fading.

The longer test time applies. It is marked in  table F.6.3.5. which one applies.

Statistical independence:

If a process works within an incremental redundancy sequence, the samples are not independent. The incremental redundancy sequence for every process must be finalised, successfully or unsuccessfully, on or beyond the minimum test time.

Then the BLER (or 1-BLER) is compared with the Test Limit to decide pass or fail.

Note:	It is FFS, if correlation within groups of retransmissions may influence the confidence level of the test.

Formula:

The theory, to derive  the minimum number  of  samples and the Test Limit, takes into consideration that BLER is in the range of near 0% to near 100%. Hence it is based on the binomial distribution and its inverse cumulative function: qbinom:

For the BLER test mode:

nelow=qbinom(D,ns,M*BLERlimit)             (1)

nehigh=qbinom(1-D,ns,BLERlimit)               (2)

given: 1-D: confidence level= 95%

BLERlimit=Block error ratio at the limit

M: Bad DUT factor >1

Input:   ns: number of samples (NACK+ statDTX + ACK)

Output  ne: number of events (NACK+ statDTX)

The intersection of (1) and (2) is the Test Limit with the coordinates: ns and ne

For the Relative Throughput  test mode:

nelow=qbinom(D,ns,1-BLERlimit)                          (3)

nehigh=qbinom(1-D,ns,m*(1-BLERlimit))               (4)

given: 1-D: confidence level= 95%

1-BLERlimit= Relative Throughput at the limit

m: Bad DUT factor <1

Input:   ns: number of samples (NACK+ statDTX + ACK)

Output  ne: number of events (ACK)

The intersection of (3) and (4) is the Test Limit with the coordinates: ns and ne

Note 1:	In contrast to BER BLER test, this approach does not contain any test time optimisation. (early pass, early fail)

Note 2:	The intersection of (3) and (4) above is a multipoint intersection due to the discrete nature of those curves. Thus, the specific intersection point used for the test limit is a subjective decision about the intersection point. In all cases, the intersection point was chosen such that it fell in the middle of the set of intersections of curves (3) and (4) above.

Nomenclature used in the tables F.6.3.5… below:

-	NACK+ statDTX + ACK is summarised as No of samples

-	NACK+ statDTX is summarised as No of errors

-	ACK is summarised as No of successes

-	In the BLER (BL) test mode  the ratio:              No of errors/ No of samples           is recorded. In this mode a pass is below the test limit

-	In the Relative Throughput (RT) test mode (1-BLER) the ratio: No of successes/ No of samples     is recorded. In this mode a pass is above the test limit

-	The test mode, used, is indicated in the rightmost column with BL or RT

-	The transition from the BL to the RT test mode can also be seen in the column relative test requirement: BLER% (1-BLER%)

-	The generic term for No of errors (BLER mode) or No of successes (Relative Throughput mode)   is   No of events. This is used in the table column Test Limit.

### F.6.3.5	Test conditions for HSDPA Receiver Performance

Table F.6.3.5.0: DC-HSDPA/4C-HSDPA receiver test case 6.2A, 6.2C, 6.2D, 6.4B, 6.5A, 6.5D, 6.5E, 6.5F, 6.5G, 6.6A, 6.6C, 6.6D,  6.7A, 6.7D and 6.7E

| DC-HSDPA Reception | Absolute Test requirement (kbps) | Relative test requirement (normalized to ideal=60 kbps) No of events/No of samples in % | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 12 |  |  |  |  |  |  |
|  | 54 | 10% | 58/467 (M=1.5) | 467 (58) | 0.934s (stat) | BL |

Table F.6.3.5.1: Maximum Input Level test cases 6.3A, 6.3C and 6.3G

| Maximum Input Level for HS-PDSCH Reception (16QAM) | Absolute Test requirement (kbps) | Relative test requirement (normalized to ideal=777 kbps) No of events/No of samples in % | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 1 |  |  |  |  |  |  |
|  | 700 | 10% | 58/467 (M=1.5) | 467 (58) | 2.8s (stat) | BL |

Table F.6.3.5.1A: Maximum Input Level test cases 6.3B, 6.3D and 6.3H

| Maximum Input Level for HS-PDSCH Reception (64QAM) | Absolute Test requirement (kbps) | Relative test requirement (normalized to ideal=13252 kbps) No of events/No of samples in % | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- |
| 64 QAM H-Set 8 |  |  |  |  |  |  |
|  | 11800 | 10.96% | 57/422 (M=1.499) | 422 (57) | 0.844s (stat) | BL |

Table F.6.3.5.2.1: Single link performance for test case 9.2.1A, 9.2.1C 9.2.1F, 9.2.1FA to 9.2.1FD demodulation of HS-DSCH (QPSK, H-Set 1, 2, 3/3A/3B/3C)

| Single link Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=534 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 65 | 87,82% (12.18%) | 60/595 (m = 1 / 1.5) | N.A. | 164s (fading) | RT |
| 2 (= 0 dB) | PB3 | 23 | 95.69% (4.31%) | 64/1796 (m = 1/1.5) | N.A | 164s (fading) | RT |
|  |  | 138 | 74.14% (25.86%) | 58/268 (m = 0.682) | N.A. | 164s(fading) | RT |
| 3 (= 0 dB) | VA30 | 22 | 95.9% (4.1%) | 64/1888 (m=1/1.5) | N.A. | 16.4s(fading) | RT |
|  |  | 142 | 73.4% (26.6%) | 59/264 (m = 0.684) | N.A. | 16.4s(fading) | RT |
| 4 (= 0 dB) | VA120 | 13 | 97.564% (2.436%) | 63/3224 (m = 1/1.5) | 3224 (63) | H-set 1: 19.5s(stat) H-set 2: 13s (stat) H-set 3: 6.5s (stat) | RT |
|  |  | 140 | 73.77% (26.23%) | 59/268 (m = 0.683) | N.A. | 4.1s(fading) | RT |
| 1 (= 10 dB) | PA3 | 309 | 42.1% | 83/171 (M = 1.295) | N.A. | 164s (fading) | BL |
|  |  | 423 | 20.74% | 60/237 (M = 1.445) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 181 | 66.1% (33.9%) | 62/215 (m = 0.703) | N.A | 164s (fading) | RT |
|  |  | 287 | 46.22% (53,78%) | 84/176 (m = 0.77) | N.A. | 164s(fading) | RT |
| 3 (= 10 dB) | VA30 | 190 | 64.4% (35.6%) | 64/211 (m = 0.708) | N.A. | 16.4s(fading) | RT |
|  |  | 295 | 44.72% (55.28%) | 85/173 (m = 0.775) | N.A. | 16.4s(fading) | RT |
| 4 (= 10 dB) | VA120 | 181 | 66.1% (33.9%) | 62/215 (m = 0.703) | N.A. | 4.1s(fading) | RT |
|  |  | 275 | 48.5% (51.5%) | 79/174 (m = 0.761) | N.A. | 4.1s(fading) | RT |

Table F.6.3.5.2.1A: Single link Performance for test case 9.2.1D, 9.2.1E demodulation of HS-DSCH (enhanced requirement type 1, QPSK, H-Set 1, 2, 3) and 9.2.1G, 9.2.1GA, 9.2.1GB, 9.2.1GC, 9.2.1GD demodulation of HS-DSCH (enhanced requirement type 3, QPSK, H-Set 3)

| Single link enhanced requirement type 1 Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=534 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 195 | 63.46% (36.54%) | 64/205 (m = 0.710) | N.A. | 164s (fading) | RT |
|  |  | 329 | 38.35% (61.65%) | 78/175 (M = 1.320) | N.A. | 164s (fading) | BL |
| 2 (= 0 dB) | PB3 | 156 | 70.77% (29.23%) | 59/239 (m = 0.690) | N.A. | 164s (fading) | RT |
|  |  | 263 | 50.72% (49.28%) | 76/176 (m = 0.753) | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 171 | 67.96% (32.04%) | 61/225 (m = 0.697) | N.A. | 16.4s(fading) | RT |
|  |  | 273 | 48.84% (51.16%) | 96/174 (M =1.252) | N.A. | 16.4s(fading) | BL |
| 4 (= 0 dB) | VA120 | 168 | 68.52% (34.48%) | 60/228 (m = 0.696) | N.A. | 4.1s(fading) | RT |
|  |  | 263 | 50.72% (49.28%) | 76/176 (m = 0.753) | N.A. | 4.1s(fading) | RT |
| 1 (= 10 dB) | PA3 | 247 | 53.72% (46.28%) | 72/180 (m = 0.742) | N.A. | 164s (fading) | RT |
|  |  | 379 | 28.95% (71.02%) | 66/193 (M = 1.386) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 195 | 63.46% (36.54%) | 63/204 (m = 0.710) | N.A. | 164s (fading) | RT |
|  |  | 316 | 40.79% (59.21%) | 81/172 (M = 1.303) | N.A. | 164s (fading) | BL |
| 3 (= 10 dB) | VA30 | 212 | 60.27% (39.73%) | 66/194 (m = 0.720) | N.A. | 16.4s(fading) | RT |
|  |  | 329 | 38.35% (61.65%) | 78/175 (M = 1.320) | N.A. | 16.4s(fading) | BL |
| 4 (= 10 dB) | VA120 | 191 | 64.21% (35.79%) | 63/208 (m = 0.708) | N.A. | 4.1s(fading) | RT |
|  |  | 293 | 45.10% (54.90%) | 89/173 (M = 1.275) | N.A. | 4.1s(fading) | BL |

Table F.6.3.5.2.2: Single link performance for test case 9.2.1A and 9.2.1C demodulation of HS-DSCH (16 QAM, H-Set 1, 2, 3)

| Single link Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=777 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 198 | 74.53% (25.47%) | 58/272 (m=0.681) | N.A. | 164s (fading) | RT |
|  |  | 368 | 52.66% (47.34%) | 74/179 m=0.746 | N.A. | 164s(fading) | RT |
| 2 (= 10 dB) | PB3 | 34 | 95.626% (4.374%) | 64/1770 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 219 | 71.83% (28,17%) | 58/240 (m=0.687) | N.A. | 164s (fading) | RT |
| 3 (= 10 dB) | VA30 | 47 | 93.95% (6.05%) | 63/1259 (m=1/1.5) | N.A. | 16.4s (fading) | RT |
|  |  | 214 | 72.47% (27.53%) | 59/255 (m=0.686) | N.A. | 16.4s (fading) | RT |
| 4 (= 10 dB) | VA120 | 28 | 96.4% (3.6%) | 64/2150 (m=1/1.5) | 2150 (64) | 12.9s H-set1 8.6s H-set2 4.3s Hset3 (stat) | RT |
|  |  | 167 | 78.51% (21.49%) | 57/319 (m=0.673) | N.A. | 4.1s (fading) | RT |

Table F.6.3.5.2.2A: Single link Performance for test case 9.2.1D and 9.2.1E demodulation of HS-DSCH (enhanced requirement type 1, 16 QAM, H-Set 1, 2, 3)

| Single link enhanced requirement type 1 Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=777 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 312 | 59.86% (40.14%) | 66/193 (m = 0.722) | N.A. | 164s (fading) | RT |
|  |  | 487 | 37.35% (62.65) | 76/176 (M = 1.327) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 275 | 64.62% (35.38%) | 63/209 (m = 0.707) | N.A. | 164s (fading) | RT |
|  |  | 408 | 47.51% (52.49) | 94/174 (M = 1.260) | N.A. | 164s (fading) | BL |
| 3 (= 10 dB) | VA30 | 296 | 61.92% (38.08%) | 65/199 (m = 0.715) | N.A. | 16.4s (fading) | RT |
|  |  | 430 | 44.68% (55.32%) | 88/173 (M = 1.278) | N.A. | 16.4s (fading) | BL |
| 4 (= 10 dB) | VA120 | 271 | 65.14% (34.86%) | 62/211 (m = 0.705) | N.A. | 4.1s (fading) | RT |
|  |  | 392 | 49.57% (50.43%) | 97/175 | N.A. | 4.1s (fading) | BL |

Table F.6.3.5.2.3: Single link performance for test case 9.2.1B demodulation of HS-DSCH (QPSK H-Set 4)

| Single link Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=534 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 4 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 72 | 86.5% (13.5%) | 59/528 (m=1/1.5) | N.A. | 164s (fading) | RT |
| 2 (= 0 dB) | PB3 | 24 | 95.5% (4.5%) | 63/1695 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 142 | 73.4% (26.6%) | 59/264 (m=0.684) | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 19 | 96.44% (3.56%) | 64/2176 (m=1/1.5) | N.A. | 16.4s (fading) | RT |
|  |  | 148 | 72.27% (27.73%) | 59/253 (m=0.686) | N.A. | 16.4s (fading) | RT |
| 4 (= 0 dB) | VA120 | 11 | 98% (2%) | 65/3746 (m=1/1.5) | 3746 (65) | 22.5s (stat) | RT |
|  |  | 144 | 73% (27%) | 58/256 (m=0.684) | N.A. | 4.1s (fading) | RT |
| 1 (= 10 dB) | PA3 | 340 | 36.29% | 75/177 (M=1.334) | N.A. | 164s (fading) | BL |
|  |  | 439 | 17.74% | 58/266 (M=1.468) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 186 | 65.15% (34.85%) | 62/209 (m=0.705) | N.A. | 164s (fading) | RT |
|  |  | 299 | 44% (56%) | 87/174 (m=0.778) | N.A. | 164s(fading) | RT |
| 3 (= 10 dB) | VA30 | 183 | 65.7% (34.3%) | 63/216 (m=0.704) | N.A. | 16.4s (fading) | RT |
|  |  | 306 | 42.66% | 86/176 (M=1.291) | N.A. | 16.4s (fading) | BL |
| 4 (= 10 dB) | VA120 | 170 | 68,14% (31.86%) | 61/226 (m=697) | N.A. | 4.1s (fading) | RT |
|  |  | 284 | 46.78% (53.22%) | 81/172 (m = 0.767) | N.A. | 4.1s (fading) | RT |

Table F.6.3.5.2.4: Single link performance for test case 9.2.1B demodulation of HS-DSCH (QPSK H-Set 5)

| Single link Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=801 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 5 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 98 | 87.76% (12.24%) | 59/583 (m=1/1.5) | N.A. | 164s (fading) | RT |
| 2 (= 0 dB) | PB3 | 35 | 95.63% (4.37%) | 63/1746 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 207 | 74.14% (25.86%) | 58/268 (m=0.682) | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 33 | 95.88% (4.12%) | 64/1879 (m=1/1.5) | N.A. | 16.4s (fading) | RT |
|  |  | 213 | 73.4% (26.6%) | 59/264% (m=0.684) | N.A. | 16.2s (fading) | RT |
| 4 (= 0 dB) | VA120 | 20 | 97.5% (2.5%) | 64/3101 (m=1/1.5) | 3101 (64) | 12.4s (stat) | RT |
|  |  | 210 | 73.77% (26.23%) | 59/268 (m=0.683) | N.A. | 4.1s (fading) | RT |
| 1 (= 10 dB) | PA3 | 464 | 42% | 84/174 (M=1.295) | N.A. | 164s (fading) | BL |
|  |  | 635 | 20.67% | 59/234 (M=1.446) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 272 | 66.02% (33.98%) | 63/218 (m=0.703) | N.A. | 164s (fading) | RT |
|  |  | 431 | 46.16% (53.84) | 84/176 (m=0.77) | N.A. | 164s(fading) | RT |
| 3 (= 10 dB) | VA30 | 285 | 64.4% (35.6%) | 64/211 (m=0.708) | N.A. | 16.4s (fading) | RT |
|  |  | 443 | 44.7% (55.3%) | 85/173 (m=0.775) | N.A. | 16.4s(fading) | RT |
| 4 (= 10 dB) | VA120 | 272 | 66.02% (33.98%) | 63/218 (m=0.703) | N.A. | 4.1s (fading) | RT |
|  |  | 413 | 48.4% (51.6%) | 81/176 (m=0.761) | N.A. | 4.1s(fading) | RT |

Table F.6.3.5.2.5: Single link Performance for test case 9.2.1C demodulation of HS-DSCH (QPSK H-Set 6)

| Single link Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=3219 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 6 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 1407 | 56.29% (43.71%) | 70/185 | N.A. | 164s (fading) | RT |
|  |  | 2090 | 35.07% (64.93%) | 73/179 | N.A. | 164s (fading) | BL |

Table F.6.3.5.2.5A: Single link Performance for test case 9.2.1E demodulation of HS-DSCH (enhanced requirement type 1, QPSK H-Set 6)

| Single link enhanced requirement type 1 Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=3219 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 6 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 672 | 79.12% (20.88%) | 57/328 (m = 0.672) | N.A. | 164s (fading) | RT |
|  |  | 1305 | 59.46% (40.54%) | 67/193 (m = 0.723) | N.A. | 164s (fading) | RT |

Table F.6.3.5.2.5B: Single link Performance for test case 9.2.1F, 9.2.1FA to9.2.1FD demodulation of HS-DSCH (enhanced requirement type 2, QPSK H-Set 6/6A/6B/6C)

| Single link enhanced requirement type 2 Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=3219 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 6 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 1494 | 53.59% (46.41%) | 72/179 (m = 0.743) | N.A. | 164s (fading) | RT |
|  |  | 2153 | 33.12% (66.88%) | 71/182 (M = 1.356) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 1038 | 67.75% (32.25%) | 61/224 (m = 0.698) | N.A. | 164s (fading) | RT |
|  |  | 1744 | 45.82% (54.18%) | 90/172 (M = 1.271) | N.A. | 164s (fading) | BL |
| 3 (= 10 dB) | VA30 | 1142 | 64.52% (35.48%) | 63/209 (m = 0.707) | N.A. | 16.4s(fading) | RT |
|  |  | 1782 | 44.64% (55.36%) | 88/172 (M = 1.278) | N.A. | 16.4s(fading) | BL |
| 4 (= 10 dB) | VA120 | 909 | 71.76% (28.24%) | 59/248 (m = 0.687) | N.A. | 4.1s(fading) | RT |
|  |  | 1467 | 54.43% (45.57%) | 72/181 (m = 0.740) | N.A. | 4.1s(fading) | RT |

Table F.6.3.5.2.5C: Single link Performance for test case 9.2.1G, 9.2.1GA, 9.2.1GB, 9.2.1GC, 9.2.1GD  demodulation of HS-DSCH (enhanced requirement type 3, QPSK H-Set 6/6A)

| Single link enhanced requirement type 3 Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=3219 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 6 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 1554 | 51.72% (48.28%) | 75/178 (m = 0.749) | N.A. | 164s (fading) | RT |
|  |  | 2495 | 22.49% (77.51%) | 61/226 (M = 1.433) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 1190 | 63.03% (36.94%) | 64/205 (m = 0.712) | N.A. | 164s (fading) | RT |
|  |  | 2098 | 34.82% (65.18%) | 73/180 (M = 1.344) | N.A. | 164s (fading) | BL |
| 3 (= 10 dB) | VA30 | 1299 | 59.65% (40.35%) | 66/192 (m = 0.722) | N.A. | 16.4s(fading) | RT |
|  |  | 2013 | 37.46% (62.54%) | 77/176 (M = 1.326) | N.A. | 16.4s(fading) | BL |
| 4 (= 10 dB) | VA120 | 1060 | 67.07% (39.93%) | 61/221 (m = 0.700) | N.A. | 4.1s(fading) | RT |
|  |  | 1674 | 48.00% (52.00%) | 96/174 (M = 1.252) | N.A. | 4.1s(fading) | BL |
| 5 (= 5 dB) | PB3 | 1248 | 61.23% (38.77%) | 66/198 (m = 0.717) | N.A. | 164s (fading) | RT |
|  |  | 2044 | 36.50% (63.50%) | 75/176 (M = 1.332) | N.A. | 164s (fading) | BL |

Table F.6.3.5.2.6: Single link Performance for test case 9.2.1C demodulation of HS-DSCH (16 QAM H-Set 6)

| Single link Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=4689 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 6 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 887 | 81.08% (18.92%) | 56/362 (m = 0.669) | N.A. | 164s (fading) | RT |
|  |  | 1664 | 64.51% (35.49%) | 63/209 (m = 0.707) | N.A. | 164s (fading) | RT |

Table F.6.3.5.2.6A: Single link Performance for test case 9.2.1E demodulation of HS-DSCH (enhanced requirement type 1, 16 QAM H-Set 6)

| Single link enhanced requirement type 1 Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=4689 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 6 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 912 | 80.55% (19.45%) | 56/352 (m = 0.670) | N.A. | 164s (fading) | RT |
|  |  | 1730 | 63.10% (36.90%) | 64/203 (m = 0.712) | N.A. | 164s (fading) | RT |

Table F.6.3.5.2.6B: Single link Performance for test case 9.2.1F, 9.2.1FA to9.2.1FD demodulation of HS-DSCH (enhanced requirement type 2, 16 QAM H-Set 6/6A/6B/6C)

| Single link enhanced requirement type 2 Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=4689 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 6 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 991 | 78.86% (21.14%) | 57/324 (m = 0.673) | N.A. | 164s (fading) | RT |
|  |  | 1808 | 61.44% (38.56%) | 65/197 (m = 0.717) | N.A. | 164s (fading) | RT |
| 2 (= 10 dB) | PB3 | 465 | 90.08% (9.92%) | 60/740 (m = 1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 1370 | 70.78% (29.22%) | 59/242 (m = 0.690) | N.A. | 164s (fading) | RT |
| 3 (= 10 dB) | VA30 | 587 | 87.48% (12.52%) | 59/573 (m = 1/1.5) | N.A. | 16.4s(fading) | RT |
|  |  | 1488 | 68.26% (31.74%) | 60/226 (m = 0.697) | N.A. | 16.4s(fading) | RT |
| 4 (= 10 dB) | VA120 | 386 | 91.77% (8.23%) | 61/905 (m = 1/1.5) | N.A. | 4.1s(fading) | RT |
|  |  | 1291 | 72.46% (27.54%) | 58/254 | N.A. | 4.1s(fading) | RT |

Table F.6.3.5.2.6C: Single link Performance for test case 9.2.1G, 9.2.1GA, 9.2.1GB, 9.2.1GC, 9.2.1GD  demodulation of HS-DSCH (enhanced requirement type 3, 16 QAM H-Set 6/6A)

| Single link enhanced requirement type 3 Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=4689 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 6 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 1979 | 57.79% (42.21%) | 69/190 (m = 0.728) | N.A. | 164s (fading) | RT |
|  |  | 3032 | 35.34% (64.66%) | 73/178 (M = 1.340) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 1619 | 65.47% (34.53%) | 62/211 (m = 0.704) | N.A. | 164s (fading) | RT |
|  |  | 2464 | 47.45% (52.55%) | 92/171 (M = 1.260) | N.A. | 164s (fading) | BL |
| 3 (= 10 dB) | VA30 | 1710 | 63.53% (36.47%) | 63/204 (m = 0.710) | N.A. | 16.4s(fading) | RT |
|  |  | 2490 | 46.90% (53.10%) | 91/171 (M = 1.264) | N.A. | 16.4s(fading) | BL |
| 4 (= 10 dB) | VA120 | 1437 | 69.35% (30.65%) | 59/231 (m = 0.694) | N.A. | 4.1s(fading) | RT |
|  |  | 2148 | 54.19% (45.81%) | 72/182 (m = 0.740) | N.A. | 4.1s(fading) | RT |
| 5 (= 5 dB) | PB3 | 779 | 83.39% (16.61%) | 57/414 (m = 0.667) | N.A. | 164s (fading) | RT |
|  |  | 1688 | 64.00% (36.00%) | 63/207 (m = 0.709) | N.A. | 164s (fading) | RT |

Table F.6.3.5.2.7: Single link Performance for test case 9.2.1H, 9.2.1HA to- 9.2.1HD demodulation of HS-DSCH (enhanced requirement type 2, 64QAM H-Set 8/8A/8B/8C)

| Single link enhanced requirement type 2 Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=13245 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64 QAM H-Set 8 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 15 and 18 dB) | PA3 | 4507 | 65.97% (34.03%) | 57/324 (m = 0.703) | N.A. | 164s (fading) | RT |
|  |  | 5736 | 56.69% (43.31%) | 70/188 (m = 0.732) | N.A. | 164s (fading) | RT |

Table F.6.3.5.2.7A: Single link Performance for test case 9.2.1I, 9.2.1IA to 9.2.1ID demodulation of HS-DSCH (enhanced requirement type 3, 64QAM H-Set 8/8A/8B/8C)

| Single link enhanced requirement type 3 Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=13245 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64 QAM H-Set 8 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 15 and 18 dB) | PA3 | 6412 | 51.59% (48.41%) | 78/184 (m = 0.750) | N.A. | 164s (fading) | RT |
|  |  | 7638 | 42.33% (57.67%) | 85/175 (M = 1.293) | N.A. | 164s (fading) | BL |

Table F.6.3.5.2.7B: Single link Performance for test case 9.2.1J, 9.2.1JA to 9.2.1JD -Enhanced requirement type 2, QPSK / 16QAM, FRC  H-Set 10/10A/10B/10C)

| Single link enhanced requirement Type2 | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=4860(QPSK) ideal=8774 (16QAM) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK/16QAM H-Set 10 |  |  |  |  |  |  |  |
| Test |  |  |  |  |  |  |  |
| QPSK,(= 4.6 dB) | VA3 | 1397 | 71.255% (28.745%) | 63/259 (m=0.698 ) | N.A. | 164s (fading) | RT |
| 16 QAM, (= 8.6 dB) | VA3 | 1726 | 80.33% (19.67%) | 56/343 (m=0.67 ) | N.A. | 164s (fading) | RT |

Table F.6.3.5.2.7C: Single link Performance for test case 9.2.1K, 9.2.1KA to 9.2.1KD Enhanced requirement type 3, QPSK / 16 QAM FRC H-Set 10/10A/10B/10C)

| Single link enhanced Type 3 | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=4860 (QPSK) ideal=8774 (16QAM) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK/16QAM H-Set 10 |  |  |  |  |  |  |  |
| Test |  |  |  |  |  |  |  |
| QPSK (= 4.6 dB) | VA3 | 2621 | 46.07% (53.93%) | 90/172 (M=1.269) | N.A. | 164s (fading) | BL |
| 16QAM (= 8.6 dB) | VA3 | 3396 | 61.29% (38.71%) | 65/196 (m=0.717 ) | N.A. | 164s (fading) | RT |

Table F.6.3.5.2.7D: Single link Performance for test case 9.2.1L, 9.2.1LA to 9.2.1LDEnhanced requirement type 3i, QPSK,  FRC H-Set 6/6A/6B/6C)

| Single link enhanced Type 3i | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=3219 No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 6/6A |  |  |  |  |  |  |  |
| Test |  |  |  |  |  |  |  |
| QPSK (= 0dB DIP1=-2.75 dB DIP2=-7.64 dB | PB3 | 691 | 78.534% (21.466%) | 57/319 (m=0.673) | N.A. | 164s (fading) | RT |
|  |  | 1359 | 57.782% (42.218%) | 69/189 (m=0.728) | N.A. | 164s (fading) | RT |
|  | VA30 | 661 | 79.466% (20.534%) | 57/334 (m=0.672) | N.A. | 16.4s (fading) | RT |
|  |  | 1327 | 58.776% (41.224%) | 68/191 (m=0.725 ) | N.A. | 16.4s (fading) | RT |

Table F.6.3.5.3.1: Open Loop Diversity Performance for test case 9.2.2A and 9.2.2D demodulation of HS-DSCH (QPSK, H-Set 1, 2, 3)

| Open Loop Diversity Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=534 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 77 | 85.57%(14.43%) | 58/486 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 180 | 66.27%(33.73%) | 62/216 (m=0.702) | N.A. | 164s (fading) | RT |
| 2 (= 0 dB) | PB3 | 20 | 96.25% (3.75%) | 64/2065 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 154 | 71.14% (28,86%) | 59/243 (m=0.689) | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 15 | 97.19% (2.81%) | 64/2758 (m=1/1.5) | H-Set 1: 2758 (64) | H-Set 2,3: 16.4s (fading) H-Set 1: 16.6s(stat.) | RT |
|  |  | 162 | 69.64% (30.36%) | 60/235 (m=0.693) | N.A. | 16.4s (fading) | RT |
| 1 (= 10 dB) | PA3 | 375 | 29.7% | 68/192 (M=1.38) | N.A. | 164s (fading) | BL |
|  |  | 475 | 11% | 58/425 (M=1.499) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 183 | 65.7% (34.3%) | 63/216 (m=0.704) | N.A. | 164s (fading) | RT |
|  |  | 274 | 48.7% (51.3%) | 80/177 (m=0.76) | N.A. | 164s (fading) | RT |
| 3 (= 10 dB) | VA30 | 187 | 65% (35%) | 62/208 (m=0.706) | N.A. | 16.4s (fading) | RT |
|  |  | 284 | 46.8% (53.2%) | 82/174 (m=0.767) | N.A. | 16.4s (fading) | RT |

Table F.6.3.5.3.2: Open Loop Diversity Performance for test case 9.2.2A and 9.2.2D demodulation of HS-DSCH (16 QAM, H-Set 1, 2, 3)

| Open Loop Diversity Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=777 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 295 | 62% (38%) | 66/203 (m=0.715) | N.A. | 164s (fading) | RT |
|  |  | 463 | 40.4% | 82/176 (M=1.306) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 24 | 96.9% (3.1%) | 64/2500 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 243 | 68.7% (31.3%) | 60/227 (m=0.695) | N.A. | 164s (fading) | RT |
| 3 (= 10 dB) | VA30 | 35 | 95.5% (4.5%) | 63/1695 (m=1/1.5) | N.A. | 16.4s (fading) | RT |
|  |  | 251 | 67.7% (32.3%) | 61/223 (m=0.698) | N.A. | 16.4s (fading) | RT |

Table F.6.3.5.3.3: Open Loop Diversity Performance for test case 9.2.2B demodulation of HS-DSCH (QPSK, H-Set 4)

| Open Loop Diversity Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=534 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 4 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 70 | 86.9% (13.1%) | 59/544 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 171 | 68% (32%) | 61/225 (m=0.697) | N.A. | 164s (fading) | RT |
| 2 (= 0 dB) | PB3 | 14 | 97.4% (2.6%) | 64/2982 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 150 | 71.9% (28.1%) | 59/250 (m=0.687) | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 11 | 97.04% (2.06%) | 65/3819 (m=1/1.5) | 3819 (65) | 23s (stat) | RT |
|  |  | 156 | 70.8% (29.2%) | 60/243 (m=0.69) | N.A. | 16.4s (fading) | RT |
| 1 (= 10 dB) | PA3 | 369 | 30.9% | 69/188 (M=1.372) | N.A. | 164s (fading) | BL |
|  |  | 471 | 11.7% | 58/400 (M=1.497) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 180 | 66.3% (33.7%) | 63/220 (m=0.702) | N.A. | 164s (fading) | RT |
|  |  | 276 | 48.3% (51.7%) | 79/173 (m=0.762) | N.A. | 164s (fading) | RT |
| 3 (= 10 dB) | VA30 | 184 | 65.5% (34.5%) | 62/211 (m=0.704) | N.A. | 16.4s (fading) | RT |
|  |  | 285 | 46.6% (53.4%) | 81/171 (m=0.768) | N.A. | 16.4s (fading) | RT |

Table F.6.3.5.3.4: Open Loop Diversity Performance for test case 9.2.2B demodulation of HS-DSCH (QPSK, H-Set 5)

| Open Loop Diversity Performance | Absolute Test requirement (kbps) |  | Relative test requirement, normalized to ideal=801 kbps No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 5 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 116 | 85.5% (14.5%) | 59/492 (m=0.667) | N.A. | 164s (fading) | RT |
|  |  | 270 | 66.27% (33.73%) | 62/216 (m=0.702) | N.A. | 164s (fading) | RT |
| 2 (= 0 dB) | PB3 | 30 | 96.25% (3.75%) | 65/2100 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 231 | 71.14% (28.86%) | 58/243 (m=0.689) | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 23 | 97.13% (2.87%) | 64/2741 (m=1/1.5) | N.A. | 16.4s (fading) | RT |
|  |  | 243 | 69.64% (30.36%) | 60/234 (m=0.693) | N.A. | 16.4s (fading) | RT |
| 1 (= 10 dB) | PA3 | 563 | 29.67% | 68/194 (M=1.381) | N.A. | 164s (fading) | BL |
|  |  | 713 | 10.93% | 58/428 (M=1.499) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 275 | 65.65% (34.35%) | 64/212 (m=0.704) | N.A. | 164s (fading) | RT |
|  |  | 411 | 48.66% (51.34%) | 77/170 (m=0.76) | N.A. | 164s (fading) | RT |
| 3 (= 10 dB) | VA30 | 281 | 64.9% (35.1%) | 63/211 (m=0.706) | N.A. | 16.4s (fading) | RT |
|  |  | 426 | 46.78% (53.22%) | 81/172 (m=0.767) | N.A. | 16.4s (fading) | RT |

Table F.6.3.5.3.5: Open Loop Diversity Performance for test case 9.2.2C demodulation of HS-DSCH (QPSK, H-Set 1, 2, 3) and test case 9.2.2E

| Open Loop Diversity Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=534 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 197 | 63.09% (36.91%) | 64/203 (m = 0.712) | N.A. | 164s (fading) | RT |
|  |  | 330 | 38.16% (61.84%) | 80/181 (M = 1.321) | N.A. | 164s (fading) | BL |
| 2 (= 0 dB) | PB3 | 152 | 71.52% (28.48%) | 59/247 (m = 0.688) | N.A. | 164s (fading) | RT |
|  |  | 251 | 52.97% (47.03%) | 73/179 (m = 0.745 | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 164 | 69.27% (30.73%) | 60/232 (m = 0.694) | N.A. | 16.4s (fading) | RT |
|  |  | 261 | 51.09% (48.91%) | 75/176 (m = 0.751) | N.A. | 16.4s (fading) | RT |
| 1 (= 10 dB) | PA3 | 268 | 49.78% (50.22%) | 101/181 (M = 1.246) | N.A. | 164s (fading) | BL |
|  |  | 407 | 23.74% (76.26%) | 62/217 (M = 1.424) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 183 | 65.71% (34.29%) | 62/213 (m = 0.704) | N.A. | 164s (fading) | RT |
|  |  | 288 | 46.03% (53.97%) | 93/178 (M = 1.269) | N.A. | 164s (fading) | BL |
| 3 (= 10 dB) | VA30 | 197 | 63.09% (36.91%) | 64/203 (m = 0.712) | N.A. | 16.4s (fading) | RT |
|  |  | 307 | 42.47% (57.53%) | 87/178 (M = 1.292) | N.A. | 16.4s (fading) | BL |

Table F.6.3.5.3.6: Open Loop Diversity Performance for test case 9.2.2C demodulation of HS-DSCH (16QAM, H-Set 1, 2, 3) and test case 9.2.2E

| Open Loop Diversity Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=777 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 340 | 56.26% (43.74%) | 70/184 (m = 0.733) | N.A. | 16.4s (fading) | RT |
|  |  | 513 | 34.01% (65.99%) | 72/180 (M = 1.350) | N.A. | 16.4s (fading) | BL |
| 2 (= 10 dB) | PB3 | 251 | 67.71% (32.29%) | 60/222 (m = 0.698) | N.A. | 16.4s (fading) | RT |
|  |  | 374 | 51.89% (48.11%) | 74/177 (m = 0.749) | N.A. | 16.4s (fading) | RT |
| 3 (= 10 dB) | VA30 | 280 | 63.98% (36.02%) | 63/206 (m = 0.709) | N.A. | 16.4s (fading) | RT |
|  |  | 398 | 48.80% (51.20%) | 96/174 (M = 1.252) | N.A. | 16.4s (fading) | BL |

Table F.6.3.5.4.1: Closed Loop Diversity Performance for test case 9.2.3A and 9.2.3D demodulation of HS-DSCH (QPSK, H-Set 1, 2, 3)

| Closed Loop Diversity Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=534 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 118 | 77.89% (22.11%) | 58/315 (m=0.674) | N.A. | 164s (fading) | RT |
|  |  | 225 | 57.84% (42.16%) | 69/189(m=0.728) | N.A. | 164s (fading) | RT |
| 2 (= 0 dB) | PB3 | 50 | 90.63% (9.37%) | 61/787 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 173 | 67.58% (32.42%) | 61/222 (m=0.698) | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 47 | 91.2% (8.8%) | 62/852 (m=1/1.5) | N.A. | 16.4s (fading) | RT |
|  |  | 172 | 67.77% (32.23%) | 61/223 (m=0.698) | N.A. | 16.4s (fading) | RT |
| 1 (= 10 dB) | PA3 | 399 | 25.23% | 63/207 (M=1.413) | N.A. | 164s (fading) | BL |
|  |  | 458 | 14.18% | 57/325 (M=1.487) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 199 | 62.71% (37.29%) | 65/204 (m=0.713) | N.A. | 164s (fading) | RT |
|  |  | 301 | 43.6% | 88/180 (M=1.285) | N.A. | 164s (fading) | BL |
| 3 (= 10 dB) | VA30 | 204 | 61.77% (38.23%) | 65/198 (m=0.716) | N.A. | 16.4s (fading) | RT |
|  |  | 305 | 42.85% | 85/173 (M=1.29) | N.A. | 16.4s (fading) | BL |

Table F.6.3.5.4.1A: Closed Loop Diversity Performance for test case 9.2.3C demodulation of HS-DSCH (QPSK, H-Set 1, 2, 3), type 1 and test case 9.2.3E

| Closed Loop Diversity Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=534 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 242 | 54.7% (45.3%) | 71/180 (m=0.739) | N.A. | 164s (fading) | RT |
|  |  | 369 | 30.9% | 86/239(M=1.327) | N.A. | 164s (fading) | RT |
| 2 (= 0 dB) | PB3 | 170 | 68.2% (31.8%) | 53/199 (m=0.679) | N.A. | 164s (fading) | RT |
|  |  | 272 | 49.1% (50.9%) | 78/174 (m=0.759) | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 172 | 67.8% (32.2%) | 62/227 (m=0.698) | N.A. | 16.4s (fading) | RT |
|  |  | 270 | 49.4% (50.6%) | 78/175 (m=0.758) | N.A. | 16.4s (fading) | RT |
| 1 (= 10 dB) | PA3 | 297 | 44.4.% | 88/173 (M=1.28) | N.A. | 164s (fading) | BL |
|  |  | 410 | 23.2% | 60/213 (M=1.434) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 194 | 63.7% (36.3%) | 63/203 (m=0.71) | N.A. | 164s (fading) | RT |
|  |  | 308 | 42.3% | 84/173 (M=1.293) | N.A. | 164s (fading) | BL |
| 3 (= 10 dB) | VA30 | 204 | 61.77% (38.23%) | 65/198 (m=0.716) | N.A. | 16.4s (fading) | RT |
|  |  | 315 | 41.0% | 80/169 (M=1.302) | N.A. | 16.4s (fading) | BL |

Table F.6.3.5.4.2: Closed Loop Diversity Performance for test case 9.2.3A and 9.2.3D  demodulation of HS-DSCH (16 QAM, H-Set 1, 2, 3)

| Closed Loop Diversity Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=777 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 361 | 53.56% (46.44%) | 73/180 (m=0.743) | N.A. | 164s (fading) | RT |
|  |  | 500 | 35.68% | 74/177 (M=1.338) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 74 | 90.48% (9.52%) | 62/788 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 255 | 67.2% (32.8%) | 61/219 (m=0.7) | N.A. | 164s (fading) | RT |
| 3 (= 10 dB) | VA30 | 84 | 89.2% (10.8%) | 61/683 (m=1/1.5) | N.A. | 16.4s (fading) | RT |
|  |  | 254 | 67.32% (32.68%) | 61/220 (m=0.699) | N.A. | 16.4s (fading) | RT |

Table F.6.3.5.4.2A: Closed Loop Diversity Performance for test case 9.2.3C demodulation of HS-DSCH (16 QAM, H-Set 1, 2, 3), type 1 and test case 9.2.3E

| Closed Loop Diversity Performance | H-SET 1 Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=777 kbps for H-SET 1) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples for H-SET 1, 2, 3 (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 QAM H-Set 1/2/3 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PA3 | 376 | 51.6% (48.4%) | 75/177 (m=0.75) | N.A. | 164s (fading) | RT |
|  |  | 532 | 31.5% | 72/193 (M=1.368) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 267 | 65.6% (34.4%) | 62/212 (m=0.704) | N.A. | 164s (fading) | RT |
|  |  | 393 | 49.4% (50.6%) | 78/175 (m=0.758) | N.A. | 164s (fading) | RT |
| 3 (= 10 dB) | VA30 | 279 | 64.1% (35.9%) | 63/206 (m=0.708) | N.A. | 16.4s (fading) | RT |
|  |  | 404 | 48% (52%) | 79/172 (m=0.763) | N.A. | 16.4s (fading) | RT |

Table F.6.3.5.4.3: Closed Loop Diversity Performance for test case 9.2.3B demodulation of HS-DSCH (QPSK, H-Set 4)

| Closed Loop Diversity Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=534 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 4 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 114 | 78.64% (21.36%) | 58/327 (m=0.673) | N.A. | 164s (fading) | RT |
|  |  | 223 | 58.21% (41.79%) | 69/191 (m=0.727) | N.A. | 164s (fading) | RT |
| 2 (= 0 dB) | PB3 | 43 | 91.94% (8.06%) | 62/930 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 167 | 68.71% (31.29%) | 60/227 (m=0.695) | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 40 | 92.5% (7.5%) | 63/1017 (m=1/1.5) | N.A. | 16.4s (fading) | RT |
|  |  | 170 | 68.14% (31.86%) | 61/226 (m=0.697) | N.A. | 16.4s (fading) | RT |
| 1 (= 10 dB) | PA3 | 398 | 25.42% | 63/206 (M=1.412) | N.A. | 164s (fading) | BL |
|  |  | 457 | 14.37% | 57/321 (M=1.486) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 196 | 63.27 (36.73%) | 64/204 (m=0.711) | N.A. | 164s (fading) | RT |
|  |  | 292 | 45.28% (54.72%) | 85/175 (m=0.773) | N.A. | 164s (fading) | RT |
| 3 (= 10 dB) | VA30 | 199 | 62.71% (37.29%) | 65/204 (m=0.713) | N.A. | 16.4s (fading) | RT |
|  |  | 305 | 42.85% | 85/173 (M=1.29) | N.A. | 16.4s (fading) | BL |

Table F.6.3.5.4.4: Closed Loop Diversity Performance for test case 9.2.3B demodulation of HS-DSCH (QPSK, H-Set 5)

| Closed Loop Diversity Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=801 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 5 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | PA3 | 177 | 77.89% (22.11%) | 58/315 (m=0.674) | N.A. | 164s (fading) | RT |
|  |  | 338 | 57.78% (42.22%) | 68/186 (m=0.728) | N.A. | 164s (fading) | RT |
| 2 (= 0 dB) | PB3 | 75 | 90.63% (9.37%) | 61/787 (m=1/1.5) | N.A. | 164s (fading) | RT |
|  |  | 260 | 67.52% (32.48%) | 62/225 (m=0.699) | N.A. | 164s (fading) | RT |
| 3 (= 0 dB) | VA30 | 71 | 91.13% (8.87%) | 62/846 (m=1/1.5) | N.A. | 16.4s (fading) | RT |
|  |  | 258 | 67.77% (32.23%) | 61/223 (m=0.698) | N.A. | 16.4s (fading) | RT |
| 1 (= 10 dB) | PA3 | 599 | 25.17% | 64/211 (M=1.413) | N.A. | 164s (fading) | BL |
|  |  | 687 | 14.18% | 57/325 (M=1.487) | N.A. | 164s (fading) | BL |
| 2 (= 10 dB) | PB3 | 299 | 62.65% (37.35%) | 64/200 (m=0.713) | N.A. | 164s (fading) | RT |
|  |  | 452 | 43.54% | 87/174 (M=1.285) | N.A. | 164s (fading) | BL |
| 3 (= 10 dB) | VA30 | 306 | 61.77% (38.23%) | 65/198 (m=0.716) | N.A. | 16.4s (fading) | RT |
|  |  | 458 | 42.79% | 86/175 (M=1.29) | N.A. | 16.4s (fading) | BL |

Table F.6.3.5.4.5: Closed Loop Diversity Performance for test case 9.2.3D demodulation of HS-DSCH (QPSK, H-Set 6)

| Closed Loop Diversity Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=3219 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 6 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PB3 | 1536 | 52.28% (47.72%) | 74/178 (m = 0.747) | N.A. | 164s (fading) | RT |

Table F.6.3.5.4.6: Closed Loop Diversity Performance for test case 9.2.3D demodulation of HS-DSCH (16QAM, H-Set 6)

| Closed Loop Diversity Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=4689 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16QAM H-Set 6 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 10 dB) | PB3 | 1154 | 75.39% (24.61%) | 57/280 (m = 0.679) | N.A. | 164s (fading) | RT |

Table F.6.3.5.4.7: HS-SCCH-less demodulation of HS-DSCH for test case 9.5.1 (QPSK, H-Set 7))

| HS-SCCH-less demodulation of HS-DSCH | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=TBD kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 7 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | Case 8 | 19.9 | 47.35% (52.65%) | 91/170 (M = 1.26) | N.A. | 16.4s (fading) | BL |

Table F.6.3.5.4.8: HS-SCCH-less demodulation of HS-DSCH for test case 9.5.1A (Enhanced requirement type 1, QPSK, H-Set 7))

| HS-SCCH-less demodulation of HS-DSCH | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=TBD kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 7 |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 0 dB) | Case 8 | 23.5 | 37.83% (62.17%) | 76/173 (M = 1.32) | N.A. | 16.4s (fading) | BL |

Table F.6.3.5.4.9: HS-DSCH and HS-SCCH reception in CELL-FACH state

| 9.6.1 Single link HS-DSCH demodulation performance in CELL-FACK | Absolute Test requirement SDU ER |  | No of events/No of samples in % Error Ratio(ER) (Success Ratio(SR)) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | ER / SR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK H-Set 3 |  |  |  |  |  |  |  |
| (= 0 dB) | VA30 | 0.82 | 82% (18%) | 57/382 (m = 0.668) | 382 (>=57) | 30.5s | SR |
| 9.6.2 (HS-DCCH) (= 0 dB) | VA30 | 0.01 | 1% (99%) | 65/5247 (M = 1.5) | 5247 (<=65) | 420s | ER |

Table F.6.3.5.4.10: MIMO Performance for test case 9.2.4A, 9.2.4C, 9.2.4E, 9.2.4G (FRC , H-Set 9/9A)

| MIMO Performance | Absolute Test requirement (kbps) | Relative test requirement (normalized to ideal=13510 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- |
| H-Set 9/9A |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |

| 1 (= 10 dB) | PA3 | 5563 | 55.8% (41.2%) | 68/192* (m = 0.725) | N.A. | 164s (fading) | RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 (= 10 dB) | VA3 | 4347 | 67.8% (32.2%) | 61/225* (m = 0.679) | N.A. | 164s (fading) | RT |
| *)nominator and denominator by its own are irrelevant, only the ratio is relevant. |  |  |  |  |  |  |  |
| MIMO Performance H-Set 9/9A Test number | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=8650 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| 3 (= 6 dB) | PA3 | 3933 | 54.5% (45.5%) | 71/180 (m = 0.739) | N.A. | 164s (fading) | RT |
| 4 (= 6 dB) | VA3 | 3011 | 65.2% (34.8%) | 62/210 (m = 0.705) | N.A. | 164s (fading) | RT |

Table F.6.3.5.2.11: MIMO Performance for test case 9.2.4B, 9.2.4D, 9.2.4F, 9.2.4H  (FRC H-Set 11/11A)

| Single link Performance | Absolute Test requirement (kbps) |  | Relative test requirement (normalized to ideal=22074 kbps) No of events/No of samples in % BL (RT) | Test limit expressed as No of events/min No of samples (Bad DUT factor) | Min No of samples (number of events to pass) Mandatory if applicable | Test time in s Mandatory if fading Informative and approx. if statistical | BL / RT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HSET-11/11A 64 QAM |  |  |  |  |  |  |  |
| Test number |  |  |  |  |  |  |  |
| 1 (= 18 dB) | PA3 | 9980 | 54.79% (45.21%) | 71/180 (m = 0.739) | N.A. | 164s (fading) | RT |

[End of  modification]