TSG SA Meeting #66	SP-140771

10 - 12 December 2014, Maui, USA

# Source:	Qualcomm Incorporated, KDDI

# Title:	Updated new Work Item on Extended DRX cycle for Power Consumption Optimization

# Document for:	Approval

Agenda Item:	16

Abstract of the contribution: Updates the SA2 approved study item on eDRX to add another supporting company.

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	Feasibility study on Extended DRX cycle for Power Consumption Optimization

## Acronym: FS_eDRX

## Unique identifier:

## 1	3GPP Work Area

| X | Radio Access |
| --- | --- |
| X | Core Network |
|  | Services |

## 2	Classification of WI and linked work items

### 2.0	Primary classification

This work item is a …

| X | Study Item (go to 2.1) |
| --- | --- |
|  | Feature (go to 2.2) |
|  | Building Block (go to 2.3) |
|  | Work Task (go to 2.4) |

### 2.1	Study Item

| Related Work Item(s) (if any] |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 560322 | Machine-Type and other mobile data applications Communications enhancements-UE Power Consumptions Optimizations (MTCe-UEPCOP) | Rel-12 SA2 work item for UE power consumptions optimizations |
| 680012 | Study of Power Saving for MTC Devices (FS_uPOD) | GERAN study item for UE power saving for MTC Devices |
| 650013 | Study on Small Data Transmission Enhancements for UMTS (FS_UTRA_SDATA) | RAN study item for UTRAN device power enhancements |

Go to §3.

### 2.2	Feature

| Related Study Item or Feature (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.3	Building Block

| Parent Feature (or Study Item) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

This work item is …

|  | Stage 1 (go to 2.3.1) |
| --- | --- |
| X | Stage 2 (go to 2.3.2) |
|  | Stage 3 (go to 2.3.3) |
|  | Test spec (go to 2.3.4) |
|  | Other (go to 2.3.5) |

#### 2.3.1	Stage 1

| Source of external requirements (if any) |  |  |
| --- | --- | --- |
| Organization | Document | Remarks |

Go to §3.

#### 2.3.2	Stage 2

| Corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other source of stage 1 information |  |  |
| --- | --- | --- |
| TS or CR(s) | Clause | Remarks |

If no identified source of stage 1 information, justify:

Go to §3.

#### 2.3.3	Stage 3

| Corresponding stage 2 work item (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Else, corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other justification |  |  |
| --- | --- | --- |
| TS or CR(s) or external document | Clause | Remarks |

If no identified source of stage 2 information, justify:

Go to §3.

#### 2.3.4	Test spec

| Related Work Item(s) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

Go to §3.

#### 2.3.5	Other

| Related Work Item(s) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship | TS / TR |

Go to §3.

### 2.4	Work task

| Parent Building Block |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

## 3	Justification

Power consumption is important for UEs using battery and also for UEs using external power supply its importance increases with the continued growth of device populations and more demanding use cases. The importance can be illustrated by following scenarios, e.g.:

-	For M2M use cases like sensors that run on battery it is a major cost to on site to exchange (or charge) the batteries for a large amount of devices and the battery lifetime may even determine the device’s lifetime if it is not foreseen to charge or replace the battery;

-	Even for scenarios where UEs may consume power from an external power supply it may be desirable to consume less power for energy efficiency purposes.

In Release 12, 3GPP SA2 adopted a solution for power consumption optimization called Power Saving Mode (PSM), under the work item MTCe-UEPCOP. PSM allows the UE to save battery life by negotiating with the network periods for which the UE becomes unreachable, in which case the UE stops access stratum activities. The UE exits this mode when it has mobile originated data pending, or when the periodic TAU/RAU timer expires.

However, PSM has limited applicability for use cases where unscheduled Mobile Terminated data with some requirement on MT data delay tolerance are required to be sent to the UE. In that case, the UE would need to negotiate Periodic TAU/RAU timer equal (or slightly shorter) to maximum allowed delay tolerance for MT data in order to be able to receive the MT within the time required. If the maximum allowed delay tolerance for MT data is not in the order of an hour or more, then the PSM solution introduces an important increase in signalling, due to more frequent periodic TAU/RAU procedures, and also suffers in terms of power consumption performance because of the required "wake ups" to perform this signalling.

During Rel-12 study phase of MTCe-UEPCOP, it was considered to extend the idle and connected mode DRX to reduce power consumption and the system impacts were investigated. Extending the idle and connected mode DRX have advantage for scenarios requiring mobile terminated data since it does not introduce additional signalling load due to MT data availability (i.e. they do not require to set relatively short periodic TAU timer for MT data) and provide larger battery life for cases with required delay tolerance for MT data in the order of minutes to an hour. The DRX cycle can be set depending on the requirements on delay tolerance for MT data.

## 4	Objective

Objective is to study extending the DRX cycle in idle and connected mode for GERAN, UTRAN and E-UTRAN. .

The objective of this work is to continue the work started in Rel-12 in TR 23.887 for Extended Idle and Connected mode DRX (as documented in clause 7.1.3.1 and 7.1.3.6 of TR 23.887), by:

- A study:

  - Identifying which open issues remained from the Rel-12 study for extended Idle and Connected mode DRX.

    - For idle mode eDRX extension address at least the following open issues:

      - On CN:

-	Support NAS-protocol extensions to enable extended DRX cycles:

-	Coordination between UE and MME for use of extended DRX

-	Paging strategy in CN that fits for the needs of the extended DRX cycle and normal DRX cycle.

-	Address the impacts on MT SMS.

      - -	The impacts for MT data need to be addressed in line with ongoing work in HLCom SID.On UE:

-	Support NAS-protocol extensions to enable extended I-DRX cycles.

- For connected mode eDRX extension address the following open issues:

      - On CN:

-	Whether MME/SGW/PGW need to be made aware of use of extended C-DRX.

-	If connected mode DRX periods become longer than NAS and 3GPP feature procedure timers then NAS protocols, 3GPP feature specific protocol (e.g. location query), SMS related protocols, need to be enabled to cope with this.

  - Discussing and concluding on solutions to such open issues.

Solutions already captured in TR 23.887 for extending idle and connected mode DRX will be considered as a starting point in the study.

No connected mode mobility enhancements for extended C-DRX are considered in this work, i.e., if a UE using extended C-DRX moves to a different cell, current connected mode mobility procedures apply.

Work in RAN and co-operation with RAN will be required for UTRAN and E-UTRAN aspects. Work in GERAN and co-operation with GERAN will be required for the GERAN aspects. Feedback from RAN and GERAN for any AS related aspects will be needed before concluding this study.

Feedback from CT groups will be requested if needed.

## 5	Service Aspects

None

## 6	MMI-Aspects

None

## 7	Charging Aspects

None

## 8	Security Aspects

Any necessary security analysis will be undertaken by SA3 with support from SA2.

## 9	Impacts

| Affects: | UICC apps | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

## 10	Expected Output and Time scale

| New specifications [If Study Item, one TR is anticipated] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title | 1st rsp. WG | 2nd rsp. WG(s) | Presented for information at plenary# | Approved at plenary # | Comments |
| 23.xxx | Study on system impacts of extended DRX cycle for power consumption optimization | SA2 |  | SA#67 (March 2015 | SA#68 (June 2015) |  |

| Affected existing specifications [None in the case of Study Items] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Spec No. | CR | Subject of the CR | Approved at plenary# | Comments |

## 11	Work item rapporteur(s)

Qualcomm Inc, Miguel Griot (mgriot@qti.qualcomm.com).

## 12	Work item leadership

SA2

## 13	Supporting Individual Members

Supporting IM name

Qualcomm Incorporated

Ericsson

ZTE Corporation

Sierra Wireless

Verizon

AT&T

China Unicom

InterDigital

Alcatel-Lucent

Orange

Sony

KDDI

2013-10-03 version 1.14.0