3GPP TSG-SA5 Meeting #122	S5-187406

# Spokane,US, 12-16 November 2018 											   Revision of S5-187179

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0511 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PRA Charging Clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AULC-CH, TEI15 |  |  |  |  |  | Date: |  |  | 2018-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The notifications of UE status(es) in PRA(s) occur in CCR-U, including the UE presence status in the PRA(s) and the inactive. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the usage of PRA status(es). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to apply appropriate UE status(es) for multiple PRA(s). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.4.2	Multiple Presence Reporting Areas (PRAs)

UE presence in multiple presence reporting area (s) charging information, as defined in TS 23.401 [208] and TS 23.203 [215], is collected for each UE by S-GWs, P-GWs which are serving that UE.

During IP-CAN session establishment/modification, and independently from the PCRF, the OCS may provide to the P-GW, a list of:

-	 Presence Reporting Area (PRA) Identifiers to be activated for Core Network pre-configured Presence Reporting Areas,

-	Presence Reporting Area (PRA) Identifiers and their elements for UE-dedicated Presence Reporting Areas.

By providing the corresponding above lists in Presence-Reporting-Area-Information AVP(s), the OCS subscribes to the notifications of change of UE presence status in the PRA(s) through the Trigger-Type AVP. UE presence status in the PRA(s) describes whether the UE is entering or leaving Presence Reporting Area (s) and if the corresponding Presence Reporting Area(s) is set to inactive by the serving node.

The OCS may change the notifications of whether the UE is entering or leaving Presence Reporting Area (s) through Trigger-Type AVP in a CCA, with the Presence-Reporting-Area-Information AVP(s) to replace previous provided list.

The OCS may unsubscribe to the change of UE presence in Presence Reporting Area through Trigger-Type AVP in a CCA without the value of CHANGE_OF_UE_PRESENCE_IN_PRESENCE_REPORTING_AREA_REPORT (73), if it has been provided in the previous CCA.

The UE presence initial status in the PRA(s) at the time of the subscription shall be reported by the P-GW to the OCS for online charging in Debit / Reserve Units Request[Update]when the subscription is activated. The UE presence initial status in the PRA(s) at the time of the subscription shall be reported by the S-GW and P-GW for offline charging in Charging Data Request[Start] / [Interim] when the subscription is activated. Subsequently, whether the UE enters or leaves the PRA(s), shall be reported by the P-GW to the OCS for online charging, and by the S-GW and P-GW for offline charging.

When a PRA set i.e. PRA Identifier was subscribed to, the PRA Identifier of the PRA set, along with the individual PRA Identifier(s) belonging to the PRA set and indication(s) of whether the UE is inside or outside the individual Presence Reporting Area(s) are reported to the OCS.

For offline charging, the initial status of UE presence in the PRA(s) shall be captured in current counts and reported on first charging event. For online charging, this initial status shall be reported when received by the PCEF if quota have already been requested for the service usage, otherwise this initial status shall be reported on the first quota request.

For PRA(s) subscribed-to by the OCS, the PRA(s) and their UE presence status(es) which are reported to OCS are captured in offline charging from P-GW, with an indication for each PRA, the PRA was subscribed-to by OCS.

For PRA(s) subscribed-to by the PCRF, the PRA(s) and their UE presence status(es) which are reported to PCRF are captured in offline charging from P-GW, with an indication for each PRA, the PRA was subscribed-to by PCRF.

In the S-GW the PRA(s) and their UE presence status(es) are captured in offline charging without any differentiation between OCS and PCRF subscription, since the S-GW is unaware about this difference.

End of change