SA WG2 Meeting #131 	S2-1902301

# 25 February - 1st March 2019, Santa Cruz de Tenerife, Spain	         (revision of S2-1902232)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0848 | rev | 5 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UL CL/BP controlled by I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The ETSUN study item has concluded on an architecture that supports UL CL/BP controlled by I-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify an architecture that supports UL CL/BP controlled by I-SMF Clarify the nature of the information sent over N16a to support a local UL CL / BP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The way for SMF to control traffic offload via a I-SMF is unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (5.x.d, 5.x.e, 5.x.f) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE all text is new

### 5.x.d	Usage of an UL Classifier for a PDU Session controlled by I-SMF

This clause applies only in case of non-roaming or LBO roaming as control of UL CL/BP in VPLMN is not supported in HR case.

When I-SMF is involved for a PDU Session, it is possible that the UL CL controlled by I-SMF is inserted into the data path of the PDU Session. The usage of an ULCL controlled I-SMF in the data path of a PDU Session is depicted in Figure 5.x.d-1.

Figure 5.x.d-1 User plane Architecture for the Uplink Classifier controlled by I-SMF

The I-SMF determines whether UL CL will be inserted based on information received from SMF, and the I-SMF selects the UPFs acting as UL CL and/or PDU Session Anchor providing local access to the Data Network.

### 5.x.e	Usage of IPv6 multi-homing for a PDU Session controlled by I-SMF

This clause applies only in case of non-roaming or LBO roaming as control of UL CL/BP in VPLMN is not supported in HR case.

When I-SMF is involved for a PDU Session, it is possible that the BP controlled by I-SMF is inserted into the data path of the PDU Session. The usage of a BP controlled I-SMF in the data path of a PDU Session is depicted in Figure 5.x.e-1.

Figure 5.x.e-1: Multi-homed PDU Session: Branching Point controlled by I-SMF

The I-SMF determines whether BP will be inserted based on information received from SMF, and the I-SMF selects the UPFs acting as BP and/or PDU Session Anchor providing local access to the Data Network.

### 5.x.f	Interaction between I-SMF and SMF for the support of traffic offload by UPF controlled by the I-SMF

This clause applies only in case of non-roaming or LBO roaming as control of UL CL/BP in VPLMN is not supported in HR case. It applies for the architectures described in clauses 5.x.d and 5.x.e

When the I-SMF is inserted into a PDU Session, e.g. during PDU Session establishment or due to UE mobility, the I-SMF provides the DNAI list it supports to the SMF. Based on the DNAI list information received from I-SMF, the SMF may provide location information for local traffic steering to the I-SMF e.g. immediately or when a new or updated or removedPCC rule(s) is/are received. The location information for local traffic steering is derived from PCC rules and refers to the DNAI(s) of interest for the PDU Session that are served by the I-SMF.

The I-SMF is responsible for the insertion, modification and removal of UPF(s) to ensure local traffic steering. The SMF does not need to have access to local configuration or NRF output related with UPF(s) controlled by I-SMF. Based on the location information for local traffic steering and UE location the I-SMF may select UPF(s) acting as UL CL/BP and/or PDU Session Anchor based on selected DNAI, and insert these UPF(s) into the data path of the PDU Session.

When a UL CL/BP has been inserted, changed or removed, the I-SMF indicates to the SMF that traffic offload have been  inserted, updated or removed for a DNAI, providing also the IPv6 prefix that has been allocated in case a new IPv6 prefix has been allocated for the PDU Session.

From now on the SMF and I-SMF interactions entail:

- Notifying the UE with the new Prefix (multi-Homing case): the SMF is responsible of Issuing Router advertisement to the UE including the priorities provided to the UE to select among the prefixes related with the PDU Session.

- N4 interactions related with traffic offloading. The SMF may provide N4 information to the I-SMF for how the traffic shall be detected, enforced, monitored in UPF(s) controlled by the I-SMF: the SMF issues requests to the I-SMF containing N4 information to be used for creating / updating /removing PDR, FAR, QER, URR, ….

- Receiving N4 notifications related with traffic usage reporting: the I-SMF forwards to the SMF N4 information corresponding to UPF notifications related with traffic usage reporting; the SMF aggregates and constructs usage reports towards PCF/CHF.

NOTE x: How the SMF decides what traffic steering and enforcement actions are enforced in UPF(s) controlled by I-SMF is left for implementation.

Editor’s Note; it is FFS how the I-SMF maps between N4 Information,  received from SMF on N16a and actual PDR, FAR, QER, URR, sent on N4 to the UPF(s) it controls. For example how the Network instance(if necessary) sent over N4 to the ”local UPF” is determined.

The I-SMF is responsible of the N4 interface towards the local UPF(s) including the usage of  AN Tunnel Info received from the 5G AN via the AMF in order to build PDR and FAR or to control UPF actions when the UP of the PDU Session becomes INACTIVE.

NEXT CHANGE

END OF CHANGES