3GPP TSG-SA5 Meeting #125 	S5-193449

# Newport Beach, USA,  8 – 12 April 2019										     Revision of S5-193266

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0015 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Throughput KPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-ADPM5G |  |  |  |  |  | Date: |  |  | 2019-03-29 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are error in the description in the Throughput DL and UL KPIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The formula has been updated (AMF changed to UPF) and the downstream direction description been updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current specification should be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 6.3.2	Upstream Throughput for Network and Network Slice Instance

a)	Upstream throughput for network and network slice instance.

b)	This KPI describes the upstream throughput of one single network slice instance by computing the packet size for each successfully transmitted UL IP packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance.

c)	This KPI is obtained by upstream throughput provided by N3 interface from NG-RAN to all UPFs which are related to the single network slice instance.

d)	UTSNSI=UPFGTP.InDataOctN3UPF

e)	GTP.InDataOctN3UPF

f)	5GS

g)	Integrity

h)	Kbit/s

i)	CUM

Next Modified Section

### 6.3.3	Downstream Throughput for Single Network Slice Instance

a)	Downstream throughput for network and network slice instance.

b)	This KPI describes the downstream throughput of one single network slice instance by computing the packet size for each successfully transmitted DL IP packet through the network slice instance during each observing granularity period and is used to evaluate integrity performance of the end-to-end network slice instance.

c)	This KPI is obtained by downstream throughput provided by N3 interface from all UPFs to NG-RAN which are related to the single network slice instance.

d)

e)	GTP.OutDataOctN3UPF

f)	5GS

g)	Integrity

h)	Kbit/s

i)	CUM

End of Modified Sections