3GPP TSG-SA WG2 Meeting #133	S2-1905905

Reno, Nevada, USA, May 13 – 17, 2019	(revision of S2-1905431)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1395 | rev | 1 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarificaitons on Reflective QoS for MPTCP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  |  | Date: |  | 2019-05-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the MPTCP is used, the UE and MPTCP proxy establishes TCP subflow and actual traffic is sent using the subflow. The UE uses "link-specific multipath" addresses for each access and the MPTCP proxy uses its own IP address for TCP subflow. In this case, if the NW uses Reflective QoS, it is not clear how the UE generate derived QoS rule. If the UE generate the derived QoS rule based on "link-specific multipath" addresses and MPTCP proxy, it is hard for the NW to manage the QoS rule of the UE because the PCF does not know the "link-specific multipath" addresses and MPTCP proxy address. In addition, because the "link-specific multipath" address is different in each access, the number of derived QoS rule is increased. So it is proposed that the UE generate the derived QoS rule based on IP address of MA PDU and final destination address. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the UE shall use the IP address/prefix of the MA PDU session and the final destination address to generate the derived QoS rule. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the UE generate the derived QoS rule when the MPTCP is used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

5.32.4	QoS Support

The 5G QoS model for the Single-Access PDU Session is also applied to the Multi-Access PDU Session, i.e. the QoS Flow is the finest granularity of QoS differentiation in the Multi-Access PDU Session. One difference compared to the Single-Access PDU Session, is that in a MA PDU Session there can be separate user-plane tunnels between the AN and the PSA, each one associated with a different access. However, the QoS Flow is not associated with specific access, i.e. it is access agnostic, so the same QoS is supported when the traffic is distributed over 3GPP and non-3GPP accesses. The SMF shall provide the same QFI in 3GPP and non-3GPP accesses so that the same QoS is supported in both accesses.

A QoS Flow of the MA PDU Session may be either Non-GBR or GBR depending on its QoS profile. For a Non-GBR QoS Flow, the SMF provides a QoS profile to both access networks, if the UE is in CM-CONNECTED state in both accesses. For a GBR QoS Flow, the SMF shall provide a QoS profile to a single access network as follows:

-	If the PCC rule allows a GBR QoS Flow in a single access, the SMF provides the QoS profile for the GBR QoS Flow to the access network allowed by the PCC rule.

-	If the PCC rule allows a GBR QoS Flow in both accesses, the SMF decides to which access network to provide the QoS profile for the GBR QoS Flow based on its local policy.

For a GBR QoS Flow, traffic splitting is not supported because the QoS profile is provided to a single access network at a given time.

When the MA PDU Session is established, the SMF provides QoS rule(s) to the UE, which are applied by the UE as specified in clause 5.7.1.4. The QoS rule(s) are commonly used for both 3GPP access and non-3GPP access, so the QoS classification is independent of ATSSS rules.

The derived QoS rule generated by Reflective QoS is applied independently of the access on which the RQI was received. When the MPTCP functionality is used in the UE, the UE shall use the IP address/prefix of the MA PDU session and the final destination address to generate the derived QoS rule.

* * * * End of Change * * * *