3GPP TSG-SA WG1 Meeting #66bis on MCPTT and GCSE	S1-142058

Dublin, Ireland, 15-17 July 2014

Title:	Modifications to Audio/Voice Quality Call performance, 4.5.3

Agenda Item:	5 MCPTT: Mission Critical Push To Talk over LTE [SP-130728]

Source:	Huawei, TD-Tech

Contact:	chende@huawei.com,xiaoqian.yang@td-tech.com

Abstract: This contribution proposes clarifications of audio quality to make sure the MCPTT service can provide an acceptable Average MOS LQO when interworking with non-LTE MCPTT systems.

TS 22.179 V0.4.1 (2014-06)

4.5	Audio MCPTT Call performance

4.5.3	Audio / Voice Quality

The MCPTT service shall provide a mechanism to accommodate ongoing Encoder/Decoder (codec) improvements within LTE.

Average MOS LQO shall be greater than or equal to 2.7 measured according to P.862 standard for PESQ (ref. (a)).

This requirement shall also apply when LTE system is interworking with non-LTE MCPTT systems.

Average MOS LQO shall be greater than or equal to 2.7 measured according to P.863 standard for Personal Objective Listening Quality Analysis (POLQA) (ref. (b)).

This requirement shall also apply when LTE system is interworking with non-LTE MCPTT systems.

MOS LQO shall achieve the noise reduction performance of TIA-102.BABG (ref. (c)) Table 3-1.

MOS LQO shall achieve a noise reduction performance for 3GPP adaptive multi-rate wideband (3GPP AMR-WB: See 3GPP TS 26.190 Speech Codec and TS 26.194 VAD) that is equal to or greater than that specified for P25 full rate and half rate voice codecs in TIA-102. BABG.

References included in the above requirements

a. Perceptual evaluation of speech quality (PESQ). s.l. : International Telecommunications Union, February 2001. (NORMATIVE). ITU Recommendation P.862.

b. http://www.itu.int/rec/T-REC-P.863/en  ITU-T Recommendation P.863: Perceptual objective listening quality assessment

c. Enhanced Vocoder Methods of Measurement for Performance. TIA. March 2010. (NORMATIVE).  TIA-102.BABG.