3GPP TSG-SA WG1 Meeting #66bis on MCPTT and GCSE	S1-142108

Dublin, Ireland, 15-17 July 2014	(revision of S1-142057)

Title:	Modifications to Audio MCPTT Access time and Mouth-to-ear latency, 4.5.1

Agenda Item:	5 MCPTT: Mission Critical Push To Talk over LTE [SP-130728]

Source:	Huawei, TD-Tech

Contact:	chende@huawei.com,xiaoqian.yang@td-tech.com

Abstract: This contribution clarifies the application senarios of KPI2, based on the doc S1-141115 presented by Ericsson in SA1#66 meeting. And additional corrections are also added.

TS 22.179 V0.4.1 (2014-06)

4.5	Audio MCPTT Call performance

4.5.1	MCPTT Access time and Mouth-to-ear latency

4.5.1.1	General

For MCPTT subscribers, one of the most important performance criteria is the MCPTT Access time (KPI 1). The MCPTT Access time is defined as the time between when an MCPTT User request to speak (normally by pressing the MCPTT control on the UE) and when this user gets a signal to start speaking. This time does not include confirmations from receiving users.

The MCPTT Access time (KPI 1) does not include any MCPTT Session establishment procedure. This is the common scenario within Public Safety, meaning that MCPTT Sessions are long lived sessions during several working hours. The End-to-end MCPTT Access time (KPI 2) is defined as the time between when an MCPTT User requests to speak (normally by pressing the MCPTT control on the UE) and when this user gets a signal to start speaking, including MCPTT Session establishment and acknowledgement from first receiving user before voice can be transmitted.  A typical case for the End-to-end MCPTT Access time is a one-to-one MCPTT session request where the receiving user’s client accepts the call automatically.

Note: The End-to-end MCPTT Access time (KPI 2) is not applicable for an MCPTT Group transmission call

setup when no acknowledgment is requested by any MCPTT User of that MCPTT Group.

The Mouth-to-ear latency (KPI 3) is the time between an utterance by the transmitting user, and the playback of the utterance at the receiving user’s speaker. Figure 4.5.1.1-1 illustrates the MCPTT Access  time and Mouth-to-ear latency.

Figure 4.5.1.1-1: Illustration of MCPTT Access time and Mouth-to-ear latency

4.5.1.2 	Requirements

The MCPTT service shall provide the MCPTT Access time and Mouth-to-ear latency specified in this section to all members of an MCPTT Session regardless of group size and/or user density.  
Note: This is to ensure the all users experience the same performance regardless of e.g. the audio is transferred over unicast or multicast delivery.

The MCPTT service shall be capable of providing the performance specified herein for all MCPTT Users in the Group Call when there is not a transcoder in the bearer path.

The KPIs defined in this section shall apply in an LTE network under traffic load not exceeding [70%] of each network nodes capacity.

On networks with QOS services, the KPIs defined in this section shall apply when the total sector loading of the serving sector by MCPTT Users with equal or greater priority than the subject MCPTT User is less than [70%].

The KPIs defined in this section shall apply when the transmitting MCPTT User is registered and affiliated with the MCPTT service.

The KPIs defined in this section shall apply when the receiving MCPTT User is registered and affiliated with the MCPTT service.

The KPIs, except KPI 2, defined in this section shall apply when the call under consideration is setup without acknowledgement from the receiving UEs.

When there are transcoding functions in the bearer path of the MCPTT service, the performance provided by the MCPTT service shall be no more than [40] ms greater than the performance specified herein when there are no transcoding functions in the bearer path.

The MCPTT service shall provide an MCPTT Access time (KPI 1) less than [300] ms for [95]% of all MCPTT requests.

For MCPTT Emergency Group Calls and Imminent Peril Calls the MCPTT service shall provide an MCPTT Access time (KPI 1) less than [300] ms for [99]% of all MCPTT requests.

The MCPTT service shall provide an End-to-end MCPTT Access time (KPI2) less than[1000] ms when the MCPTT Session has not been established prior to the initiation of the MCPTT request.

Editor’s Note: Actual KPI values may require reconsideration.

The MCPTT service shall provide a Mouth-to-ear latency (KPI 3) that is less than [300] ms for [95]% of all voice bursts.

There shall be no (0 ms) initial lost audio at receiving user.

Note: In the case of Late Call Entry initial loss of audio is acceptable see section 4.5.2.1.

There shall be no (0 ms) trailing lost audio at the end of the voice burst at receiving user.