3GPP TSG-SA WG1 Meeting #74	S1-161329

Venice, Italy, 9-13 May 2016	(revision of S1-161081)

Title:	FS_eMBB: consolidation of requirements and endorsement eMBB KPIs/values

Agenda Item:	9.4

Source:	Huawei, Hisilicon, Vodafone

Contact:	Laurence Meriau  laurence.meriau@huawei.com

Abstract: This document proposes

- to consolidate eMBB potential requirements in section 7

- and  to confirm some eMBB values currently under brakets (like [100ms]) when they are endorsed by other SDOs like NGMN or ITU.

# Discussion

In the table below, we propose Consolidated Requirements (CPRs) for eMBB TR section 7 to align wording, remove duplications, merge, re-phase text in a requirement style when needed… for all eMBB Potential Requirements (PR):

- The first left column of the table below contains the current text of the potential requirements (extracted from section 5 on use cases of TR agreed during last SA1).

  - Requirement numbering is added

- The column 2 in the middle explains the reason for the consolidation:

  - Merging when there is a similar requirement in another use case, rewording to align with other use cases…

  - Clarify current eMBB non agreed values (identified with brackets [xxx]), in order to allow provision of SA1 views to RAN WG.

  - Check alignment with other SDOs such as NGMN information

- The 3rd  column on the right provides:

  - Numbering for consolidated requirements

  - Consolidated text proposed, in line with comments of column 2

| 5.1 Higher data rate family The section starts with: “For both indoor and outdoor scenario, and non-wide area coverage” that applies to all requirements |  |  |
| --- | --- | --- |
| Current Potential Requirements from current TR use cases | Reason for the consolidation | Changes to the potential requirements and consolidated requirements |
| [PR.5.1.3.1-001] The 3GPP system shall support the user experienced data rate up to [1] Gbps DL and 500 Mbps UL while the user is moving slowly up to 10 km/h. | This requirement has already been consolidated in section 7 during SA1#73, based on Scenario B of NGMN scenario#2 “Indoor Ultra-high Broadband Access” for pedestrian in hotspot (see Annex A). As 1 Gbps value is in line with NGMN in-door requirement, brackets on potential requirement are removed . Just a typo UL/DL is corrected, “indoor” and “ultra-high” added to clarify the use case and CPR number added. | Existing [CPR-7-004] is updated: For indoor users with pedestrian (speed up to 10km/h), in ultra-high connection density scenario (75000/km2) (e.g. in hotspot), the 3GPP system shall support user experienced data rate up to [TBD]1 Gbps UL DL and [1]500 MGbps DLUL, resulting into maximum traffic density of 17 Tbps/km2 (UL+DL). [CPR-7-004] is added in table of Annex A for NGMN#2 |
| No PCR | A consolidated requirement was added during last SA1 meeting to support NGMN#3 “Broadband Access in a Crowd” A CPR number is added, and “in a crowd” is added to clarify the origin of the use case. | Existing [CPR-7-005] is updated: For users withpedestrian (speed up to 10km/h) in a crowded area with very high density scenario (150000/km2 connection density, like in stadium with 30000 users per stadium) the 3GPP system shall provide user experienced data downlink rate of 25 Mbps and uplink rate of 50Mbps, resulting into traffic density of 12 Tbps/km2 (UL+DL). [CPR-7-005] is added in table of Annex A for NGMN#3 |
| [PR.5.1.3.1-002] The 3GPP system shall support the peak data rate per user at [10] Gbps while the user is moving slowly up to 10 km/h. | The requirement has already been consolidated during SA1#73 As 10 Gbps value is the only explicit scenario in ITU-R.M2083, brackets are removed. removing the brackets and addition of CPR numbering | Existing [CPR-7-003] is updated to remove brackets: For users withpedestrian (speed up to 10km/h) in area with connection density lower than 200/km2 (e.g. in office, in residential, streets), the 3GPP system shall support the peak data rate at up to [10] Gbps DL and user experienced data rate up to [TBD] Gbps UL and [TBD] Gbps DL. Add [CPR-7-003] in Annex A use case A |
| [PR.5.1.3.1-003] The 3GPP system shall be able to support user experienced broadcast data rate up to [300 Mbps] (e.g. video streams such as 4k UHD or 8k UHD). | The NGMN scenario #14 clarifies this is for DL. NGMN applies this for user with speed range 0-500 km/h, but the current use case 5.1.3.1 is for pedestrians. It is expected that the density is also the same as previous requirement (200/km2) as this is in the same use case 5.1.3.1. Both points are clarified in consolidated text proposal. Also NGMN scenario #14 proposes 200Mbps instead of 300Mbps but 300Mbps is referenced in [CPR-6-006] that is also for pedestrian but in high density case, so we see no reason to reduce to 200 Mbps for same speed. | New [CPR.7-007] For users with speed up to 10km/h in area with connection density lower than 200/km2 (e.g. in office, streets), the 3GPP system shall be able to support experienced broadcast DL data rate up to 300 Mbps (e.g. video streams such as 4k UHD or 8k UHD). |
| [PR.5.1.3.1-004]The 3GPP system shall be able to support at least [15] broadcast channels of [20Mbps] each simultaneously over the same carrier. NOTE: 4k UHD: 3840 x 2160, 50FPS, HEVC, 20~30 Mbit/s (Medium quality), 4k UHD: 3840 x 2160, 50FPS, HEVC, ~75Mbit/s (High quality), 4k UHD: 3840 x 2160, 50FPS, AVC ~ 150Mbit/s (High quality), 8K 7680*4320, 50FPS, HEVC, ~300Mbps (High quality) | This is a complement to previous requirement in same use case 5.1.3.1: so the same conditions are added at beginning of the CPR. | New [CPR-7-008] as complement to [CPR-7-007]: For users with speed up to 10km/h in area with connection density lower than 200/km2 (e.g. in office, streets), the 3GPP system shall be able to support at least [15] broadcast channels of [20Mbps] each simultaneously over the same carrier. NOTE: 4k UHD: 3840 x 2160, 50FPS, HEVC, 20~30 Mbit/s (Medium quality), 4k UHD: 3840 x 2160, 50FPS, HEVC, ~75Mbit/s (High quality), 4k UHD: 3840 x 2160, 50FPS, AVC ~ 150Mbit/s (High quality), 8K 7680*4320, 50FPS, HEVC, ~300Mbps (High quality) |
| [PR.5.1.3.1-005]The 3GPP system shall be able to provide roundtrip delay including coding/decoding in the magnitude of 10-12ms and be able to provide bandwidth capable of running an 8k 3D video streaming [250Mbps] for uplink and downlink | Clarify it is UE to UE delay, clarify UL is unicast, change roundtrip latency into a UE to UE latency and reduce values accordingly and in line with initially promoted values of the use case. Clarify the usage as e.g. virtual meeting usage. The 250Mbps is modified into 300Mbps to align with the NOTE of [PR.5.1.3.1-004] | New [CPR-7-009] as complement to [CPR-7-008]: For users with speed up to 10km/h in area with connection density lower than 200/km2 (e.g. in office, streets), the 3GPP system shall be able to provide UE to UE latency in the magnitude of 2-4ms and be able to provide bandwidth capable of running an 8k 3D DL video streaming [300Mbps] for uplink (unicast) and downlink, for e.g. virtual meeting usage. |
| [PR.5.1.3.1-006]The 3GPP system shall support residential deployment with high peak [10s of Gbps] and experienced data rates [up to 1 Gbps]. | We see no specific need to separate requirements for residential from requirements of office scenario. As greater requirements are already included in the Section 7 for office scenario, with [CPR-7-003], we propose to generalise it by adding “in residential” | Existing [CPR-7-003] updated: with “(e.g. in office, in residential, streets)” |
| [PR.5.1.3.1-007]The 3GPP system shall support residential deployment with a latency of [10 ms]. | 4G is 10ms. We did not propose a consolidation requirement for it. | None |

| 5.2 Higher density family |  |  |
| --- | --- | --- |
| Current Potential Requirements from current TR use cases | Reason for the consolidation | Changes to the potential requirements and consolidated requirements |
| [PR.5.2.3.1-001]The 3GPP system shall support the aggregate traffic volume in the area at least the level of Tbps/ km2 to support case of traffic for pedestrians (up to 10 km/h) and users in urban vehicle (up to 60 km/h), in 200-2500 /km2 connection density. | Consolidation has already been done during SA1#73 in section 7 in line with NGMN scenario#1 “Broadband access in dense area” but it unfortunately considered the user bitrate while original [PR.5.2.3.1-001] only talks about aggregate Tbps. We replace CPR-006 text by the original text of [PR.5.2.3.1-001] to remove reference to user bitrate. We propose some re-wording in order to align with NGMN “Broadband access in dense area” terminology. | Update existing [CPR-7-006] The 3GPP system shall support the aggregate traffic volume in the area at least the level of Tbps/ km2 to support case of traffic for UEs with speed pedestrian (up to 10 km/h and users in urban vehicle (up to 60 km/h), in 200-2500 /km2 connection density.For pedestrian (speed up to 10km/h) in high density scenario (200-2500 /km2 connection density, like in city centre) or for users moving in public transportation (buses) with speed 0-60km/h, the 3GPP system shall provide user experienced data downlink rate of 300Mbps and uplink rate of 50Mbps, resulting into traffic density of 1 Tbps/km2 (UL+DL). A reference to [CPR-7-006] is added in Annex A NGMN #1 Broadband access in dense area |
| [PR.5.2.3.1-002]The 3GPP system shall provide guaranteed per user experience for mobile broadband like live video in areas with a high UE density which requires user experienced data downlink rate of 300Mbps and uplink rate of 50Mbps in 200-2500 /km2 connection density. | This potential requirement does not make differentiation between pedestrian and urban vehicle. It is in line with NMGN#1 which talks about 300 Mb/s for speed range 0-100 km/h (but in SA1 case it is up to 60 km/h) | New [CPR-7-007]: For users with speed up to 10km/h, in high density scenario (200-2500 /km2 connection density, like in city centre) or for users moving in public transportation (buses) with speed 0-60km/h, the 3GPP system shall provide user experienced data downlink rate of 300Mbps and uplink rate of 50Mbps. A reference to [CPR-7-007] is added in Annex A NGMN #1 Broadband access in dense area |
| [PR.5.2.3.1-003]The 3GPP system shall provide guaranteed user experience for mobile broadband services like live video in areas with a high UE density which requires user experienced data downlink rate of 120Mbps and uplink rate of 50Mbps in 200-2500 /km2 connection density with user velocities in the range of 0-60 km/h. | This requirement is for urban vehicle (speed up to 60km/h). It is already taken into account in [CPR-7-007] in line with NGMN#1, so with 300 Mbps instead of 120Mbps. | same as above [CPR-7-007] |

| 5.3 Deployment and Coverage family |  |  |
| --- | --- | --- |
| Current Potential Requirements from current TR use cases | Reason for the consolidation | Changes to the potential requirements and consolidated requirements |
| [PR.5.3.3.1-001]For wide area coverage, the system shall support a minimum experienced data rate per user for mobile broadband anytime and anywhere, e.g., 100 Mbps, with the user velocity up to 120 km/h. | Values are already in accordance with ITU M.2083 and with NGMN. So no change proposed except add a CPR number and editorial change. Replace” Pedestrian (up to 10 km/h)” by “UEs with speed up to 10 km/h” | Existing [CPR-7-001], editorial change: For wide area coverage, the system shall support a minimum experienced data rate per user for mobile broadband services anytime and anywhere, 100Mbps, for UEs with speed up to 10 km/h and users in urban vehicle (up to 120 km/h), except for very high user density from150000/km2 (e.g. stadium). ## [CPR-7-001] is added in Annex A NGMN#4 |
| [PR.5.3.3.2-001] The 3GPP system shall enable support up to 100% geographic area as part of wide area connectivity (one way shall be via satellite access). Note that the 100% geographic area refers to the entire area allowed to be covered by the spectrum license and could be regional or global. | “Wide area connectivity” = “wide area coverage” of existing [CPR-7-001] This PR.5.3.3.2 is added as a complement just after CPR-7-001, with reuse of the same terminology “coverage” as 001 | New [CPR-7-002] The 3GPP system shall enable support up to 100% geographic area as part of wide area coverage (one way shall be via satellite access). Note that the 100% geographic area refers to the entire area allowed to be covered by the spectrum license and could be regional or global. |
| [PR.5.3.3.2-002]The 3GPP system shall support residential deployment with service limited to a pre-defined geographic area | There is no need to have this requirement as residential KPI requirements are already in [PR5.1.3.1-006] which is consolidated into [CPR-7-003]: For users with speed up to 10km/h in area with connection density lower than 200/km2 (e.g. in office, in residential, streets), the 3GPP system shall support the peak data rate up to 10 Gbps DL. | Already in [CPR-7-003] |

| 5.4 Higher User Mobility family |  |  |
| --- | --- | --- |
| Current Potential Requirements from current TR use cases | Reason for the consolidation | Changes to the potential requirements and consolidated requirements |
| [PR.5.4.3-001]The 3GPP system shall support mobile broadband to users in fast moving road vehicles up to 200 km/h and trains 500 km/h and user experienced data rate up to 50 Mbps per user at DL and 25 Mbps per user at UL. | Values are in accordance with NGMN. So no change proposed except to give a number for the CPR. | New [CPR-7-010] The 3GPP system shall support mobile broadband to users in fast moving road vehicles up to 200 km/h and trains 500 km/h and user experienced data rate up to 50 Mbps per user at DL and 25 Mbps per user at UL. |
| [PR.5.4.3-002]The 3GPP system shall support mobile broadband to base stations/relays in fast moving road vehicles up to 200 km/h and trains 500 km/h and user experienced data rate up to 50 Mbps per user at DL and 25 Mbps per user at UL). | Values are in accordance with NGMN. So no change proposed for the values. “to base stations/relays” is understood to clarify that the KPIs proposed per user are valid whatever the user access directly the network or via a relay hiding all the users in the fast moving vehicle. As NGMN does not make any difference on the way the users data are transferred to the network, previous CPR-7-010 applies but is enhanced. | Enhance the [CPR-7-010] by addition of direct/via relay: “The 3GPP system shall support mobile broadband, via direct connection or via on-board base station relaying, to users in fast moving road vehicles up to 200 km/h and trains 500 km/h and user experienced data rate up to 50 Mbps per user at DL and 25 Mbps per user at UL.” |
| [PR.5.4.3-003]The 3GPP system shall support mobile broadband to users in airplanes (up to 1000 km/h) and user experienced data rate up to 15 Mbps per user at DL and 7.5 Mbps per user at UL). | Values are in accordance with NGMN. So no change proposed Just CPR number | New [CPR-7-011]The 3GPP system shall support mobile broadband to users in airplanes (up to 1000 km/h) and user experienced data rate up to 15 Mbps per user at DL and 7.5 Mbps per user at UL). |
| [PR.5.4.3-004] The 3GPP system shall support enhanced connectivity services to base stations/relays in fast moving airplanes (1000 km/h) with enhanced user experience (user experienced data rate up to 15 Mbps per user at DL and 7.5 Mbps per user at UL). | Values are in accordance with NGMN. So no change proposed for the values. “to base stations/relays” is understood to clarify that the KPIs proposed per user are valid whatever the user access directly t the network or a relay is hiding all the users in the fast moving vehicle as NGMN does not make any difference on the way the users data are transferred to the network. So previous CPR applies but is proposed to be enhanced. | Enhance [CPR-7-011] with “via direct connection or via on-board base station relaying” |
| [PR.5.4.3-005]The 3GPP system shall be able to provide the mobile broadband in fast moving road vehicles and trains (traffic density of up to 100 Gbps / km2 at DL, and 50 Gbps / km2 at UL). | Values are in accordance with NGMN. So no change proposed Just CPR number | New [CPR-7-012]The 3GPP system shall be able to provide the mobile broadband in fast moving road vehicles and trains (traffic density of up to 100 Gbps / km2 at DL, and 50 Gbps / km2 at UL). |
| [PR.5.4.3-006]The 3GPP system shall be able to provide fast moving airplanes connectivity (traffic density of up to 1.2 Gbps / airplane at DL and 600 Mbps / plane at UL). | Values are in accordance with NGMN. So no change proposed Just CPR number | New [CPR-7-013] The 3GPP system shall be able to provide fast moving airplanes connectivity (traffic density of up to 1.2 Gbps / airplane at DL and 600 Mbps / plane at UL). |
| [PR.5.5.3-001]The 3GPP System shall be efficient and flexible for transmission of both small and large amounts of data (e.g., streaming video) from the same device. | No change proposed Just CPR number | New [CPR-7-014] The 3GPP System shall be efficient and flexible for transmission of both small and large amounts of data (e.g., streaming video) from the same device. |
| [PR.5.5.3-002]The 3GPP system shall minimize signalling overhead e.g. for security needed for transmission of small amounts of data without reducing its level of security. | A generic requirement, non-security related, is consolidated in clause 7 [CPR-7-015] A requirement for security aspect may be created in clause 6 | New [CPR-7-015] The 3GPP system shall minimize signalling overhead needed for transmission of small amounts of data. |

Annex A of the TR for reference:

Figure A-1: Comparison of use cases

Table A-1: eMBB use cases and NGMN use cases

| MBB family | Higher Data Rate |  | Higher density | Deployment and Coverage |
| --- | --- | --- | --- | --- |
| Speed | Pedestrian (upto10 km/h) in small areas: offices, stadium |  | Pedestrian or vehicle up to 60 km/h | Pedestrian or vehicle up to 120 km/h |
| Data Rate | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) |  | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) |
|  | - Peak rate: 10 Gbps, experience rate: up to Gbps (value missing) - Missing: experienced rate 1 Gbps DL, total 17 Tbps/km2 (UL+DL) | Experienced rate (stadium) DL 25Mbps UL 50Mbps missing in eMBB TR Total 12 Tbps/km2 (UL+DL) | Guaranteed exper. rate: 300Mbps DL/50Mbps UL (same in NGMN) Total: 1 Tbps/ km2 |  |
| Density | - Missing : <200/km2 connections - 75000/km2 connections | 150000/km2, 30000/stadium missing in eMBB TR | 200-2500/km2 connections (in line in NGMN) | 400 /km2 connections (suburban) 100/km2 connections (rural) |
| NGMN category | - Not in NGMN - Matches with #2 “Indoor Ultra-high Broadband Access” | matches with #3 “Broadband Access in a Crowd “ | #1 “Broadband access in dense area” | #4 “50+ Mbps everywhere” |

# Proposal

Changes are proposed to FS_SMARTER eMBB TR 22.863in line with above discussion:

- Give numbers to potential requirements in line with column 1 of previous Discussion section above

- Update consolidation requirement section 7 in line with column 3 above

- Add reference to consolidated requirmement numbers in table of Annex A.

***** BEGIN OF CHANGE *****

# 7	Consolidation of Potential Requirements

[CPR-7-001]For wide area coverage, the system shall support a minimum experienced data rate per user for mobile broadband services anytime and anywhere, 100Mbps, for pedestrian users (up to [10 km/h]) and users in vehicle (up to 120 km/h), except for very high connection density up to75000/km2).

[CPR-7-002]The 3GPP system shall enable support up to 100% geographic area as part of wide area coverage (one way may be via satellite access). Note that the 100% geographic area refers to the entire area allowed to be covered by the spectrum license and could be regional or global.

[CPR-7-003]For pedestrians users in area with connection density lower than [200/km2] (e.g. in office, in residential, streets), the 3GPP system shall support the peak  data rate up to 10 Gbps DL.

[CPR-7-004]For indoor users, in high connection density scenario (75000/km2) (e.g. in hotspot), the 3GPP system shall support user experienced data rate up to 1 Gbps DL and 500 Mbps UL, resulting into traffic density of 112,5 Tbps/km2 (UL+DL).

[CPR-7-005]For pedestrian users in a crowded area with very high density ([150000]/km2 connection density, like in stadium with [30000]  connected users per stadium) the 3GPP system shall provide user experienced data downlink rate of 25 Mbps and uplink rate of 50Mbps, resulting into traffic density of [12] Tbps/km2 (UL+DL).

[CPR-7-006] The 3GPP system shall support the traffic density in the area at least the level of Tbps/ km2 to support case of traffic for pedestrian users and users in urban vehicle (up to 60 km/h), in 200-2500 /km2 connection density

[CPR-7-007] For pedestrian users in area with connection density lower than 200/km2 [200/km2 is quite low for an office, residential, or even streets] (e.g. in office, residential, streets), the 3GPP system shall be able to support experienced broadcast DL data rate up to 300 Mbps (e.g. video streams such as 4k UHD or 8k UHD).

[CPR-7-008] For pedestrian users in area with connection density lower than 200/km2 (e.g. in office, streets), the 3GPP system shall be able to support at least [15] broadcast channels of [20Mbps] each simultaneously over the same carrier. The current speed limits the service to pedestrians.]

NOTE:	4k UHD: 3840 x 2160, 50FPS, HEVC, 20~30 Mbit/s (Medium quality), 4k UHD: 3840 x 2160, 50FPS, HEVC, ~75Mbit/s (High quality), 4k UHD: 3840 x 2160, 50FPS, AVC ~ 150Mbit/s (High quality), 8K 7680*4320, 50FPS, HEVC, ~300Mbps (High quality)

[CPR-7-009] For pedestrian users in area with connection density lower than 200/km2 (e.g. in office, residential, streets),the 3GPP system shall be able to provide UE to UE latency in the magnitude of 2-4ms and be able to provide bandwidth capable of running an 8k 3D DL video streaming [300Mbps] for uplink (unicast) and downlink, for e.g. virtual meeting usage..

[CPR-7-010]The 3GPP system shall support mobile broadband via direct connection or via on-board base station relaying, to users in fast moving road vehicles up to 200 km/h and trains 500 km/h and user experienced data rate up to 50 Mbps per user at DL and 25 Mbps per user at UL, whatever user access directly to the network or via a base stations relaying data transfer for all users in the fast moving vehicles.

[CPR-7-011]The 3GPP system shall support mobile broadband via direct connection or via on-board base station relaying, to users in airplanes (up to 1000 km/h) and user experienced data rate up to 15 Mbps per user at DL and 7.5 Mbps per user at UL), whatever user access directly to the network or via a base stations relaying data transfer for all users in the airplane.

[CPR-7-012]The 3GPP system shall be able to provide the mobile broadband in fast moving road vehicles and trains (traffic density of up to 100 Gbps / km2  at DL, and 50 Gbps / km2 at UL).

[CPR-7-013]The 3GPP system shall be able to provide fast moving airplanes connectivity (traffic of up to 1.2 Gbps / airplane at DL and 600 Mbps / plane at UL).

[CPR-7-014]The 3GPP System shall be efficient and flexible for transmission of both small and large amounts of data (e.g., streaming video) from the same device.

[CPR-7-015]The 3GPP system shall minimize signalling overhead needed for transmission of small amounts of data.

[NEW CPR-7-MKa 2] The 3GPP system shall support service limited to a pre-defined geographic area for e.g. residential deployment.

Editor’s Note: performances values may have to be modified during normative step.

***** NEXT CHANGE *****

Annex A:
Comparison of use cases

The Figure and tables below provide a comparison of use cases: Use cases from eMBB families 1, 2 and 3 are in “black” text. New values are in “green”,Use cases from NGMN are in “blue” text.

Figure A-1: Comparison of use cases

Table A-1: eMBB use cases and NGMN use cases

| MBB family | Higher Data Rate |  | Higher density | Deployment and Coverage |
| --- | --- | --- | --- | --- |
| Speed | Pedestrian (upto10 km/h) in small areas: offices, stadium |  | Pedestrian or vehicle up to 60 km/h | Pedestrian or vehicle up to 120 km/h |
| Data Rate | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) |  | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) | Everywhere: minimum 100 Mbps (“DL” is missing) (NGMN: 50+ Mbps) |
|  | - Peak rate: 10 Gbps, experience rate: up to Gbps (value missing) - Missing: experienced rate 1 Gbps DL, total 17 Tbps/km2 (UL+DL) | Experienced rate (stadium) DL 25Mbps UL 50Mbps missing in eMBB TR Total 12 Tbps/km2 (UL+DL) | Guaranteed exper. rate: 300Mbps DL/50Mbps UL (same in NGMN) Total: 1 Tbps/ km2 |  |
| Density | - Missing : <200/km2 connections - 75000/km2 connections | 150000/km2, 30000/stadium | 200-2500/km2 connections (in line in NGMN) | 400 /km2 connections (suburban) 100/km2 connections (rural) |
| NGMN category | - Not in NGMN but in ITU-.M2083: [CPR-7-003] - Matches with #2 “Indoor Ultra-high Broadband Access”: [CPR-7-004] | matches with #3 “Broadband Access in a Crowd “: [CPR-7-005] | #1 “Broadband access in dense area”: [CPR-7-006] + [CPR-7-007] | #4 “50+ Mbps everywhere” [CPR-7-001] |

***** END OF CHANGE *****