3GPP TSG-SA WG2 Meeting #130	S2-1900182

Kochi, India, 21 - 25 January, 2019	(revision of S2-19XXXXX)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | CR0891 | rev | - | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on EBI revocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2019-01-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a UE establishes a PDU Session in 3GPP access, the AMF allocates EBI and it is provided to the UE and SMF for the interworking with EPS if network supports N26 interface. The PDU Session may be moved to non-3GPP access and if the UE moves to EPS, the MME does not have EBI information allocated to the non-3GPP PDU Session. As a consequence, the MME may allocated the same EBI which is allocated to the non-3GPP PDU Session when additonal bearer is created in the EPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the AMF detects that the UE is moving from the EPS and there is a duplicated EBIs for the non-3GPP PDU Sessions, the AMF revokes the duplicated EBIs of the non-3GPP PDU Sessions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interwoking with EPS may not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.11.1.4.3	EPS bearer ID revocation

Following procedures are updated to revoke the EPS bearer ID(s) assigned to the QoS flow(s):

-	UE or network requested PDU Session Release for Non-roaming and Roaming with Local Breakout (clause 4.3.4.2).

-	UE or network requested PDU Session Release for Home-routed Roaming (clause 4.3.4.3).

-	UE or network requested PDU Session Modification (non-roaming and roaming with local breakout) (clause 4.3.3.2).

-	UE or network requested PDU Session Modification (home-routed roaming) (clause 4.3.3.3).

-	Registration (clause 4.2.2.2.2)

When the PDU Session is released as described in clauses 4.3.4.2 or 4.3.4.3, and the SMF invokes Nsmf_PDUSession_StatusNotify to notify AMF that the SM context for this PDU Session is released, the AMF releases the association between the SMF ID and the PDU Session ID, and releases the EBIs assigned for this PDU Session. When all the PDU sessions which are allocated with EBIs are relased in the same SMF, the AMF may revoke DNN and PGW-C+SMF FQDN for S5/S8 interface in the UDM using Nudm_UECM_Update service operation.

NOTE 1:	If the PGW-C+SMF in which the PDU sessions support EPS interworking is changed for the same DNN, the AMF can update the DNN and new PGW-C+SMF FQDN for S5/S8 interface in the UDM using Nudm_UECM_Update service operation.

When the UE or AMF initiates a PDU Session Modification as described in clauses 4.3.3.2 or 4.3.3.3, and the SMF needs to release the assigned EBI from a QoS flow (e.g. when the QoS flow is released), the SMF can indicate the Released EBI list in the Nsmf_PDUSession_UpdateSMContext Response to the AMF. The AMF releases the corresponding EBI allocation for this PDU Session.

When the SMF initiates a PDU Session Modification as described in clauses 4.3.3.2 or 4.3.3.3, and the SMF needs to release the assigned EBI from a QoS flow (e.g. when the QoS flow is released), the SMF invokes Namf_Communication_EBIAssignment and indicates the Released EBI list to the AMF. The AMF releases the corresponding EBI allocation for this PDU Session.

When the UE initiates a Registration as described in clause 4.2.2.2.2, and the AMF detects that the UE moves from the EPS and detects that a PDU Session associated with non-3GPP access has overlapping EBI with PDU Session associated with 3GPP access, the AMF release the corresponding EBI for the non-3GPP PDU Session. The AMF invokes Nsmf_PDUSession_UpdateSMContext (EBI(s) to be revoked) to request the related SMF to release the the EBI(s) to be revoked. The SMF sends N1 SM container to the UE to inform the revoked EBI(s).

* * * * End of Changes * * * *