3GPP TSG-SA WG2 Meeting #130 	S2-1900362

# Kochi, India, 21st January - 25th January

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0821 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction to non-public network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction to non-public networks – standalone and non-standalone non-public networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General description for standalone non-public network. New clause describing the identifiers, credentials, network selection process, access to PLMN services via NPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Start of Changes *****

# 4	Architecture model and concepts

### 4.X	Support for non-public network

#### 4.X.1	Stand-alone non-public network

##### 4.X.1.1	General

A stand-alone non-public network shall be able to operate without dependency on the network functions of a PLMN.

Figure 4.X.1.1-1: Architecture reference for Non-public network

##### 4.X.1.2	Identifiers and authentication

A non-public network ID (NPN-ID) is an identifier used to identify a stand-alone non-public network.

A NPN-ID assignment may be:

-	Locally managed at deployment time and chosen in a way that to avoids collisions with other adjacent non-public networks.

-	Universally managed by a central entity to ensure uniqueness.

Editor’s note: Pending feedback from RAN working groups the following text can be included.

To identify a cell as part of non-public network, the cell shall broadcast the PLMN ID consisting of MCC 999 and MNC defined by the 3GPP.

To prevent the UE that do not support access to non-public network from accessing the cell that is part of non-public network, the cell shall broadcast cellReservedForOtherUse indication. The UE that support non-public network shall consider a cell that broadcast the cellReservedForOtherUse indication as not barred.

The UE can access the non-public networks using either 3GPP authentication methods, identities and credentials, or using non-AKA based EAP-compatible authentication methods and credentials. In the latter case the subscriber of the stand-alone non-public network shall use a network-specific identifier in NAI format and the realm part of the NAI shall include the NPN-ID of the stand-alone non-public network.

A UE may be configured with subscriber identifiers and credentials to support multiple stand-alone non-public networks.

##### 4.X.1.3	Network selection

In order to determine with which non-public network to attempt registration, the UE performs non-public network selection. The procedures for non-public network selection follows the same principles as PLMN selection as specified in TS 23.122 [17].

When a UE has identifiers and credentials to register both with a PLMN and a stand-alone non-public network, the trigger for performing network selection for PLMN vs network selection for a stand-alone non-public network is left to UE implementation.

When a UE has identifiers and credentials to register with multiple non-public networks, the priority for network selection is left to to UE implementation.

##### 4.X.1.4	Access to PLMN services via non-public network

A UE may access PLMN services via a stand-alone non-public network applying the same architectural principles as specified in clause 4.2.8, the stand-alone non-public network being in the role of “Untrusted non-3GPP access”.

#### 4.2.X.2	Non-public network deployed as part of PLMN

Editor’s note: This is a placeholder to be filled in after feedback from RAN working groups is received.

***** End Of Changes *****