3GPP TSG-SA WG2 Meeting #134	S2-1907763

Sapporo, Japan, 24 - 28th June 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1654 | rev |  | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on binding ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  |  | Date: |  | 2019-06-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is unclear whether the binding indication is sent to producer or not.The binding indication is useful for the producer to determine whether to updat the binding indication or not. Therefore the consumer and SCP should send the request message together with the binding indication to the producer. In binding for subscription and notification procedure, the binding indication of the consumer is missing. The change of the binding indication of the producer is also missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add binding indication from consumer/SCP to producer. - Add description of binding indication change in subscription and notification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consumer may not know if an binding indication needs to be updated or not. The binding indication for subscription and notification procedure is imcomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.11, 4.17.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.17.11	Indirect Communication without delegated discovery Procedure

This clause provides the call flow for indirect communication model without delegated discovery.

Figure 4.17.11-1: Procedure for Indirect Communication without delegated discovery

The NF/NF service discovery procedure is defined in clauses 4.17.4 and 4.17.5. In a successful discovery the NF service consumer gets the NF profile(s) matching the search criteria provided in the Nnrf_NFDiscovery_Request message.

1.	When the NF Service Consumer needs to send a Service Request and has stored results from the Discovery Procedure, the NF Service Consumer selects an appropriate NF Producer / NF Service Producer instance from the list of NF profiles provided by the NRF. The NF Service Consumer considers the NF and NF service parameters (e.g. TAI, S-NSSAI, locality, priority etc) in the NF profiles. The NF Service consumer requests service from the NF Service producer by sending a service request message to the NF service producer via the SCP and the NF Service Consumer may provide a binding indication if it is provided by NRF in the NF/NF service discovery procedure.

2.	If the binding indication is provided by the NF Service Consumer, SCP (re-)selects as specified in Table 6.3.1.0-1 of TS 23.501 [2] and routes the service requet to target NF Service Consumer together with the binding indication accordingly. If the binding indication is not provided by the NF Service Consumer, then the SCP routes the service request based on routing information available.

3.	The NF Service Producer responds via SCP. The NF Service Producer may provide an updated binding indication in the response message.

4.	SCP forwards the response message.

* * * * Start of 2nd Change * * * *

4.17.12.4	Binding for subscription and notification

Binding can be created as part of the implicit subscription request. In this case, the service request may include a binding indication of  the NF Service Consumer as in 4.17.12.3, and the response to the service request that creates the implicit subscription may include a binding indication of the NF Service Provider as in 4.17.12.2. For subsequent operations on the created implicit subscription, bindings may be used. The description in 4.17.12.1 applies for Figure 4.17.12.4.-1.

If the binding indication of the NF Service Consumer needs to be updated, the NF service consumer may initiate a new request to the NF service producer using the binding indication of the NF Service Provider. The request includes  an updated Notification Correlation ID and an updated binding indication of the NF Service Consumer.

If the binding indication of the NF Service Provider needs to be updated, the NF service provider may initiate a notification to the NF service consumer using the binding indication of the NF Service Consumer. The notification includes an updated Subscription Correlation ID and an updated binding indication of the NF Service Provider.

Figure 4.17.12.4-1: Binding for subscription requests

* * * * End of Changes * * * *