3GPP TSG-SA3 LI Meeting #15 	Tdoc S3LI04_114r1

Povoa de Varzim , 19-20 July 2004

| CR-Form-v7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | CRNum |  rev | - |  Current version: | 6.6.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | HI2 SIP Content clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | Cingular, Siemens |  |  |  |  |  |  |  |  |  |
| Work item code:  | SEC1-LI |  |  |  |  |  |  | Date:  |  | 06/07/2004 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-6 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) |  |
| Reason for change:  |  | National requirements should be included |  |  |  |  |  |  |  |  |
| Summary of change:  |  | If the national regulation forbids the delivery of specific content in the SIP parameter on HI2 this should be mentioned in the standard |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Problems with the regulators in these specific countries |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

**** Changes ****

## 7.2	IRI for IMS

In addition, information on non-transmission related actions of a target constitute IRI and is sent via HI2, e.g. information on subscriber controlled input.

The IRI may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information).

2.	Basic data context information, for standard data transmission between two parties (e.g. SIP-message).

For each event, a Record is sent to the LEMF, if this is required. The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

Table 7.1: Mapping between IMS Events and HI2 Records Type

| Event | IRI Record Type |
| --- | --- |
| SIP-Message | REPORT |

A set of information is used to generate the record. The records used transmit the information from mediation function to LEMF. This set of information can be extended in the CSCF or DF2 MF, if this is necessary in a specific country. The following table gives the mapping between information received per event and information sent in records.

Table 7.2: Mapping between IMS Events Information and IRI Information

| Parameter | Description | HI2 ASN.1 parameter |
| --- | --- | --- |
| Observed SIP URL | Observed SIP URL | partyInformation (sip-url) |
| Observed TEL URL | Observed TEL URL | partyInformation (tel-url) |
| Event type | IMS Event | iMSevent |
| Event date | Date of the event generation in the CSCF | timeStamp |
| Event time | Time of the event generation in the CSCF |  |
| Network identifier | Unique number of the intercepting CSCF | networkIdentifier |
| Correlation number | Unique number for each PDP context delivered to the LEMF, to help the LEA, to have a correlation between each PDP Context and the IRI. | gPRSCorrelationNumber |
| Lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifier |
| SIP message | Whole SIP message | sIPMessage |

NOTE1:	LIID parameter must be present in each record sent to the LEMF.

NOTE2:	Details for the parameter SIP message. If national regulations require only signaling information on HI2, specific content in parameter SIP message like IMS (Immediate Messaging) had to be deleted/filtered.

### 7.2.1	Events and information

This clause describes the information sent from the Delivery Function (DF) to the Law Enforcement Monitoring Facility (LEMF) to support Lawfully Authorized Electronic Surveillance (LAES). The information is described as records and information carried by a record. This focus is on describing the information being transferred to the LEMF.

The IRI events and data are encoded into records as defined in the Table 7-1 Mapping between IMS Events and HI2 Records Type and Annex B.3 Intercept related information (HI2) [1]. IRI is described in terms of a 'causing event' and information associated with that event. Within each IRI Record there is a set of events and associated information elements to support the particular service.

The communication events described in Table 7-1: Mapping between the IMS Event and HI2 Record Type and Table 7-2: Mapping between IMS Events Information and IRI Information convey the basic information for reporting the disposition of a communication. This clause describes those events and supporting information.

Each record described in this clause consists of a set of parameters. Each parameter is either:

mandatory (M)	- required for the record,

conditional (C)	- required in situations where a condition is met (the condition is given in the Description), or

optional (O)	- provided at the discretion of the implementation.

The information to be carried by each parameter is identified. Both optional and conditional parameters are considered to be OPTIONAL syntactically in ASN.1 Stage 3 descriptions. The Stage 2 inclusion takes precedence over Stage 3 syntax.

Table 7.3: SIP-Message REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed SIP-URL | C | SIP URL of the interception target (if available) |
| observed TEL-URL | C | TEL URL of the interception target (if available) |
| event type | M | Provide IMS event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| correlation number | C | If available and not included in the SIP-message |
| SIP message | M | The relevant SIP message |