3GPP TSG-SA WG6 Meeting #17	S6-170745

Prague, Czech Republic, 8th – 12th May 2017	(revision of S6-170687 was S6-170520)

Source:	FirstNet, NEC

Title:	pCR on new key issue and solution for "unstoppable audio"

Spec:	3GPP TR 23.782 ver.0.5.2

Agenda item:	9.2

Document for:	Approval

Contact:	Bill Janky (William dot Janky at firstnet dot gov)

1. Introduction

This pCR introduces a new key issue and solution on "unstoppable audio" which can be triggered by floor control arbitration between the MCPTT and the LMR system.

2. Reason for Change

In the legacy LMR systems, there is a known problem where the transmitting radio sometimes cannot be signalled that the floor has been taken or revoked.  For example, there is no floor control in conventional LMR radio at all (i.e. polite operation).  In the case of simultaneous PTT, sometimes the user on the LMR system is unaware that he/she no longer has the floor and will continue to transmit (i.e. unstoppable audio).  The issue of dealing with the unstoppable audio needs to be addressed.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.782 version 0.5.2.

* * * First Change * * * *

# 5	Key issues

<skipping existing content…>

## 5.4	Media plane

### 5.4.X	Key issue #4-X: Unstoppable audio

#### 5.4.X.1	Description

In many LMR systems still in operation (e.g. analog or digital conventional, P25 Phase 1 FDMA trunking system), a transmitting radio sometimes cannot be signalled that the floor has been taken or revoked.  For example, there is no floor control in conventional LMR radio at all (i.e. polite operation), and in a P25 Phase 1 FDMA system an overridden user cannot be signalled to stop transmitting. Thus, in the case of simultaneous PTT between MCPTT and LMR, the user on the LMR system is sometimes unaware that he/she no longer has the floor and will continue to transmit (i.e. unstoppable audio). The issue of dealing with the unstoppable audio needs to be addressed. In some LMR systems, this audio is referred to as "losing audio" and this audio is often routed to a dispatcher so that the transmission is not lost which is particularly important in emergency group calls.

Within the context of interworking between MCPTT and legacy LMR system, this condition can occur due to simultaneous floor requests where both MCPTT and LMR users are granted the floor by their respective servers (key issue #8-1), or the floor granted to an LMR user is revoked by an MCPTT user (key issue #8-2).

The corresponding stage 1 requirements from 3GPP TS 22.179 [2] are given below:

[R-6.18.3.2-018] Interworking between the MCPTT Service and P25 shall support conveyance of Losing audio from P25 subscriber units and consoles to authorized MCPTT Users.

[R-6.18.3.2-019] The MCPTT Service shall provide a mechanism for an MCPTT Administrator to authorize MCPTT Users to be able to receive Losing audio from P25 subscribers units and consoles.

* * * Next Change * * * *

# 6	Solutions

<skipping existing content…>

## 6.5	Media plane

### 6.5.X	Solution #X-1: Floor override with unstoppable audio

#### 6.5.X.1	Description

Unstoppable audio may be unique to legacy LMR systems, but the issue of how to deal with multiple audio streams during a call is not. MCPTT has similar requirements concerning routing of overridden audio in 3GPP TS 22.179 [2].  The solution proposed is based on the solution defined in 3GPP TS 23.379 [11], section 10.9.1.3.2.2, Floor override without using floor revoked during an MCPTT session.

#### 6.5.X.2		Floor override with unstoppable audio on an interworking group

Figure 6.5.X.2-1 shows the high-level procedure where an MCPTT session is already established between the floor participants (with floor granted to an LMR user represented by the IWF) and the floor control server (with an override based on priority and configured to permit the transmission of overridden floor participant from the IWF). The group is defined in the MCPTT system and the MCPTT Server is the floor control server.  Only two UEs involved in the session are shown for simplicity.

Pre-conditions

-	The MCPTT floor control server has been configured to support override.

-	The override supported in this case permits both the overridden floor participant and the overriding floor participant to be transmitting.

-	An MCPTT session is established between an MCPTT client, the interworked system, and controlling server.

-	Session is ongoing.

Figure 6.5.x.2-1: Floor override (overridden continues to transmit) during an interworking session

1.	It is assumed that the floor participant B (represented by the IWF in the figure) has been given the floor and is transmitting voice media.

2.	Floor participant A having a floor priority which is relatively higher than that of floor participant B wants to send voice media over the session.

3.	Floor participant A sends a floor request message to the floor control server.

4.	The floor control server determines to accept the floor request from floor participant A based on arbitration result e.g., according to the floor priority information that is received in the floor request message.

5a.	Floor control server responds with a floor granted message to floor participant A.

5b.	 Floor control server sends a floor taken message to the other floor participants (via the IWF). Floor participant B continues transmitting the (overridden) voice media transmission.

NOTE 1:	All other floor participants (not shown) that are part of this group call receive a floor taken message, so that the other floor participants learn who the newly granted talker (overriding) is.

6a.	The floor granted causes the user of floor participant A to be notified.

6b.	The user of floor participant B is notified of the status that the floor is now taken by floor participant A.  In this case floor participant B is unable to receive the message and continues transmitting (unstoppable audio).

7.	Floor participant A (overriding) starts sending voice media over the session established beforehand.

NOTE 2:	Floor participant B is still sending voice (overridden). The list of floor participants that receive the overriding, overridden, or both transmissions is based on configuration.

NOTE 3:	When floor participant A stop transmitting, if floor participant B is still sending voice, then the floor is granted back to floor participant B and audio is routed to all current floor particpants.

#### 6.5.X.3	Impacts on existing nodes and functionality

In MCPTT, a new user profile item may be needed to identify users who are authorized to receive overriding, overridden, or both transmissions. An affiliation option may be needed so an authorized user can specify reception of the losing audio in addition to, or instead of, the winning audio.

#### 6.5.X.4	Solution evaluation

The solution proposes a new procedure to support the detection and propagation of the "unstoppable audio" condition and allows distributing this audio to authorized group members within the MCPTT system. This prevents important transmissions from being lost. This solution reuses the existing floor override without using floor revoked procedure defined in 3GPP TS 23.379.

* * * End of Change * * * *