3GPP TSG-CT WG1 Meeting #157	C1-256298

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.558 | CR | 0173 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the Eees_ACREvents Service and EASInfoProvReq |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NBI19 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following issues have been identified in the current specification: - The description for Eees_ACREvents service in PUT/PATCH method can be simplified to avoid subsequent expansion. - The quotation mark for resource name is missing in clause 5.4.2.5.2. - The EASInfoProvReq data type name is incorrect in clauses 6.6.5.1 and 6.6.5.2.2, and not aligned with the OpenAPI file. - Additional editorial and format issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix the above issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.2.2, 5.4.2.3.2, 5.4.2.4.3, 5.4.2.5.2, 6.6.5.1, 6.6.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact on the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.4.2.2.2	EEC subscribing to ACR information from EES using Eees_ACREvents_Subscribe operation

To subscribe to ACR information reporting at the EES, the EEC shall send an HTTP POST message to the EES on the "ACR events subscriptions" resource. The body of the POST message shall include ACREventsSubscription data type as specified in clause 6.4.5.2.2.

Upon receiving the HTTP POST message from the EEC, the EES shall:

a)	process the EEC ACR information subscription request;

b)	verify and check if the EEC is authorized to subscribe ACR information about the requested EAS(s); and

c)	if the EEC is authorized to subscribe for the ACR information notification, then the EES;

1)	shall create a new resource with the Individual ACR events subscription resource as specified in clause 6.4.2.3;

2)	if a list of identifier(s) of ACs is provided by the EEC, the EES shall use it during ACR information notification as specified in clause 5.4.2.3; and

3)	shall send an ACR information subscription response to the EEC. The URI of the created resource (including the subscription identifier) shall be returned in the "Location" HTTP header. The response may include the expiration time, indicating when the subscription will automatically expire.

If the expiration time is provided, then to maintain the subscription information, the EEC shall send an ACR information subscription update request (as described in clause 5.3.2.4) prior to the expiration time. If the ACR information subscription update request is not sent prior to the expiration time, the EES shall treat the EEC as implicitly unsubscribed and remove the corresponding Individual ACR information subscription resource.

# *** Next Change ***

5.4.2.3.2	EES notifying the ACR information to EEC using Eees_ACREvents_Notify operation

The EES determines to notify the EEC with the ACR information on following events:

-	For EEC executed ACR via S-EES scenario, when S-EAS sends the ACR Complete message to the S-EES to confirm that the ACR has completed;

-	For S-EAS decided ACR scenario, when T-EAS selection information received from the S-EAS or when S-EAS informs the S-EES of the complete of ACT;

-	For S-EES executed ACR scenario, when S-EES determines T-EES and T-EAS via the Discover T-EAS procedure or when S-EAS informs the S-EES of the complete of ACT; and

-	For EEC executed ACR via T-EES scenario, when T-EAS sends the ACR Complete message to the T-EES to confirm that the ACR has completed.

To notify the ACR information events, the EES shall:

a)	identify ACs that satisfies trigger conditions for providing ACR information notification if a list of identifier(s) of ACs was provided by the EEC when subscribing to ACR information; and

b)	if the EdgeApp_2 feature is supported and:

1)	if the "expectedLocArea" attribute was provided in the service continuity request, then the S-EES shall detect if the UE has moved to the expected location or geographical service area; and

2)	if the EES received the successful EEC Context Push response from the T-EES that includes T-EES selected ACR scenario list, the same shall be included in the "acrScenarioList" attribute of ACRInfoNotification data type sent towards the EEC.

c)	send an HTTP POST message using the Notification Destination URI received in the subscription request, as specified in clause 6.4.4. The EES shall include "eecCtxtReloc" attribute containing the registration ID and registration expiration time as specified in clause 6.4.5.2.3.

Upon receiving the HTTP POST message, the EEC shall process the ACR information Notification, in case of "acrRes" attribute with in the ACRCompleteEventInfo data type indicates the ACR is successful and:

a)	if the "acrScenarioList" attribute is not included, the EEC may either select ACR scenario considering the supported ACR scenarios of AC, EEC, T-EES and T-EAS or request T-EES to select list of ACR scenarios by triggering EAS Information provisioning procedure as specified in clause 6.6;

b)	if the "eecCtxtReloc" attribute is included that indicates EEC context relocation has failed the EEC can trigger EAS Information provisioning procedure specified in clause 6.6 to perform re-selection of the ACR scenarios.

# *** Next Change ***

5.4.2.4.3	EEC updating ACR information subscription at EES using Eees_ACREvents_UpdateSubscription operation

To update ACR information subscription at the EES, the EEC shall send an HTTP PATCH message (for partial modification) or HTTP PUT message (for fully replacement) to the EES on resource URI identifying the Individual ACR events subscription resource representation, as specified in clause 6.4.2.3.3.3 for an HTTP PATCH message and in clause 6.4.2.3.3.1 for an HTTP PUT message.

The PATCH message includes the parameters within the ACREventsSubscriptionPatch data type that need to be replaced in the existing subscription resource.

The PUT message shall replace all properties of the existing resource within the ACREventsSubscription data type in the request. The values of the "eecId" and "ueId" attributes provided during the subscription creation shall not be changed.

Upon receiving the HTTP PATCH or PUT message from the EEC, the EES:

a)	shall check the update subscription message from the EEC to see if the EEC is authorized to modify the requested subscription resource;

b)	if the EEC is authorized to update the ACR information subscription and the eecId of the requesting EEC and the eecId in the resource match, then the EES;

1)	shall update the resource identified by Resource URI of the ACR information subscription with the updated information received in the HTTP PATCH or PUT request message;

3)	shall return the ACR information subscription response. The EES may send "200 OK" response code which includes the subscription identifier and the expiration time, indicating when the subscription will automatically expire. Otherwise, the EES sends "204 No Content" response code.

If the expiration time is provided, the EEC shall send an ACR information subscription update request prior to the expiration time if the EEC wants to maintain the subscription. If the ACR information subscription update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly unsubscribed and remove the corresponding ACR information subscription resource.

# *** Next Change ***

5.4.2.5.2	EEC unsubscribing to service provisioning subscription from EES using Eees_ACREvents_Unsubscribe operation

To unsubscribe ACR information subscription from the EES, the EEC shall send an HTTP DELETE message to the EES, on the resource URI identifying the "Individual ACR events subscription" resource representation as specified in clause 6.4.2.3.3.2. Upon receiving the HTTP DELETE request, the EES:

a)	shall verify and check if the EEC is authorized to unsubscribe the "Individual ACR events subscription" resource;

b)	if the EEC is authorized to delete the "Individual ACR events subscription" resource, then the EES shall unsubscribe the EEC for the ACR information subscription identified by the "subscriptionId" URI;

c)	shall return the "204 Not Content" message to the EEC, indicating the successful removal of the subscription information.

# *** Next Change ***

6.6.5.1	General

This clause specifies the application data model supported by the Eees_EASInformationProvisioning API.

Table 6.6.5.1-1 specifies the data types defined specifically for the Eees_EASInformationProvisioning API service.

Table 6.6.5.1-1: Eees_EASInformationProvisioning API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EASInfoProvReq | 6.6.5.2.2 | Describes the parameters shared to perform EAS Information Provision related operations. |  |
| EasInfoProvReqType | 6.6.5.3.3 | Represents the type of EAS Information Provisioning Request. |  |
| EasInfoProvResp | 6.6.5.2.3 | Information about the EAS information provisioning response. |  |
| InstantiatedEASInfo | 6.6.5.2.4 | Contains the instantiated EAS information. |  |

Table 6.6.5.1-2 specifies data types re-used by the Eees_EASInformationProvisioning API service from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Eees_EASInformationProvisioning.

Table 6.6.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACRScenario | 3GPP TS 29.558 [4] | To represent the selected ACR scenarios in ACR_SELECTION event. |  |
| Dnai | 3GPP TS 29.571 [5] | Identifies a DNAI. |  |
| EndPoint | 3GPP TS 29.558 [4] | Represents the endpoint information of an EAS. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Used to define the geographic and topological area served by EAS. |  |

# *** Next Change ***

6.6.5.2.2	Type: EASInfoProvReq

Table 6.6.5.2.2-1: Definition of type EASInfoProvReq

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eecId | string | M | 1 | Represents a unique identifier of the EEC. |  |
| acId | string | M | 1 | Identity of the AC. |  |
| selEasIds | array(string) | O | 1..N | The identifier(s) (e.g., FQDN, URI) of the selected EAS (or the selected EAS(s) for EAS bundles) which is either instantiated or instantiable. |  |
| appGrpId | string | O | 0..1 | The application group identifier, identifying a group of UEs using the same EAS. (NOTE 4) |  |
| eesList | array(EESInfo) | O | 1..N | Contains the list of EESs which support the application group identifier used for common EAS announcement. (NOTE 4) |  |
| eesId | string | O | 0..1 | Represents the identifier of the EES. | EdgeApp_3 |
| eesEndPnts | array(EndPoint) | O | 1..N | Represents the endpoint(s) of the EES. | EdgeApp_3 |
| reqType | EasInfoProvReqType | O | 0..1 | Indicates the type of the EAS Information Provisioning Request. |  |
| selAcrScenarios | array(ACRScenario) | O | 1..N | The list of ACR scenarios (or the list of ACR scenarios for EAS bundles) selected by the EEC. (NOTE 1) |  |
| selEasEndPoints | array(EndPoint) | O | 1..N | The endpoint(s) of the selected EAS (or the selected EAS(s) for EAS bundles). |  |
| dnais | array(Dnai) | O | 1..N | Represents list of Data network access identifier for each selected EAS identifier. |  |
| svcArea | array(LocationArea5G) | O | 1..N | Service availability area (geographical and topological) for each selected EAS identifier |  |
| assEesEndPoints | array(EndPoint) | O | 1..N | EES information of the other EES(s) which support the direct bundled EAS, within the same EDN, and associated with the EASID list. |  |
| casInfo | EndPoint | O | 0..1 | Target cloud application server information provided by the AC. |  |
| acProf | ACProfile | O | 0..1 | Profile of AC for which the EEC provides edge enabling services. (NOTE 2, NOTE 3) |  |
| eecSvcContSupp | array(ACRScenario) | O | 1..N | The ACR scenarios supported by the EEC for service continuity. If this attribute is not present, then the EEC does not support service continuity. (NOTE 2, NOTE 5) |  |
| NOTE 1: The attribute may be present only if Selected EASID(s) and Selected EAS Endpoint(s) are present and "reqType" attribute is set to "ACR_SCENARIO_SELECTION_ANNOUNCEMENT". NOTE 2: The attributes are present only if the "reqType" attribute is set to "ACR_SCENARIO_SELECTION_REQUEST" NOTE 3: The attribute is present if AC Profile is not shared to EES previously NOTE 4: This attribute may be present only if the "reqType" attribute is set to "EAS_SELECTION". NOTE 5: The EAS bundle information is not applicable for proxy type of EAS bundle. |  |  |  |  |  |

# *** End of Changes ***