3GPP TSG-CT WG1 Meeting #157	C1-256160

# Sophia Antipolis, France 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.543 | CR | 0102 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving editor's note for the clarifcation in SEALDD enabled bandwidth control - CoAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2, XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is an editor's note for the clarification in SEALDD enabled bandwidth control based on xrAppDevCapability attribute. The clarifications are introduced in the SA6 CR S6-253244. This has to be implemented and the related Editor's note need to be resolved in stage 3 for CoAP coding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Editor's note to define the xrAppDevCapability attribute is resolved based on the stage 2 CR S6-253244 requirements for CoAP coding. The CoAP procedure is clarified and CoAP coding is implemented for xrAppDevCapability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved editor's note, missing stage 3 implementation for the xrAppDevCapability attribute. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.3, A.2.4.2, A.2.4.9, A.2.4.15 (new), A.2.6.5 (new), A.2.6.6(new), A.3.1.5.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 7.2.2.3	SDDM client CoAP procedure

In order to request an SEALDD regular transmission connection establishment to the SDDM-S, the SDDM-C shall send a CoAP POST request message to the SDDM-S according to procedures specified in IETF RFC 7252 [14]. In the CoAP POST request, the SDDM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SDDM-S as specified in clause A.4.1.1 with:

1)	the "apiRoot" set to the SDDM-S URI;

b)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-req";

c)	shall include an "EstablishmentRequest" object:

1)	shall include a "requestorId" attribute set to "sealddclient";

2)	shall include a "sealddFlowId" attribute set to the identity of the SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic;

3)	shall include a "serverId" attribute set to the information of the VAL server;

4)	shall include an "endpointId" attribute set to the information of the endpoint of the selected VAL server to which the SDMM regular transmission connection establishment request has to be sent;

5)	may include a "valServiceId" attribute set to the VAL service identity of the vertical application;

6)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

7)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

8)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic;

9)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic;

10)	may include a "valUserId" attribute set to the identity of the VAL user or the identity of the SDDM-C acting as the VAL UE and performing the request; and

11)	may include a "batPeriodAdaptCap" attribute to indicate a BAT and periodicity adaptation capability or a "transmisAssistInfo" attribute specifying a transmission assistance information. In the "transmisAssistInfo" attribute the SDDM-C:

i)	shall include at least one of the following attributes:

A)	a "bat" attribute specifying the arrival time of the first packet of the data burst; and

B)	a "periodicity" attribute specifying the time period between the start of two bursts;

ii)	if the "bat" attribute is included, may include a "batWindow" attribute containing the acceptable earliest and latest arrival time of the first packet of the data burst; and

iii)	if the "bat", "batWindow" and "periodicity" attributes are included, may include a "periodRange" attribute specifying the periodicity range. In the "periodRange" attribute the SDDM-C shall include:

A)	a "lowerBound" attribute set to the lower bound of the periodicity and an "upperBound" attribute set to the upper bound of the periodicity of the start two bursts; or

B)	a "periodicityValues" attribute set to the acceptable periodicity values;

12)	may include an "l4sFeedbackCapability" element set to the L4S feedback capability (i.e. ECN identification, L4S feedback); and

NOTE 1:	The L4S feedback capability is used for the SEALDD enabled congestion control for VAL applications (see 3GPP TS 23.433 [2] clause 9.8.2.2).

13)	may include an "xrAppDevCap" attribute to indicate an XR application device capability information. In the "xrAppDevCap" attribute, the SDDM-C may include:

i)	a "mediaCodec" attribute to indicate the media codec information (H.264, H.265);

ii)	a "mediaResolution" attribute to indicate the media resolution information in pixels;

iii)	a "mediaFrameRate" attribute to indicate the media frame rate information; and

iv)	a "mediaFov" attribute to indicate the media field of view information in degrees. In the "mediaFov" attribute, the SDDM-C shall include:

A)	a "horizontalFov" attribute to indicate the horizontal field of view information; and

B)	a "verticalFov" attribute to indicate the vertical field of view information; and

NOTE 2:	The XR application device capability is used for the SEALDD enabled bandwidth control for different VAL users (see 3GPP TS 23.433 [2] clause 9.8.2.1).

d)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [7].

Upon receiving a CoAP POST request where the CoAP URI of the CoAP POST request identifies the establishment resource as specified in clause A.3.1.1, and containing:

a)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-req", and

b)	an "EstablishmentRequest" object;

the SDDM-C shall generate a CoAP POST response according to IETF RFC 7252 [14]. In the CoAP POST response message, the SDDM-C:

a)	shall include a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=establishment-res";

b)	shall attempt to create the SDDM regular transmission connection resource pointed at by the CoAP URI with the content of "EstablishmentRequest" object received in the request and:

1)	if successfully created, shall include an "EstablishmentResponse" object in the CoAP POST 2.01 (Created) response message;

i)	shall include a "result" attribute set to "success";

ii)	may include a "userPlaneAddress" attribute specifying the identity of the IP address of the traffic;

iii)	may include a "portNumber" attribute specifying the identity of the port number of the traffic;

iv)	may include a "url" attribute specifying the address of a given unique resource on the Web for the traffic;

v)	may include a "transportLayerProtocol" attribute specifying the transport layer protocol for the traffic; and

vi)	may include a "batPeriodAdaptCap" attribute to indicate a BAT and periodicity adaptation capability or a "transmisAssistInfo" attribute specifying a transmission assistance information. In the "transmisAssistInfo" attribute the SDDM-C:

A)	shall include at least one of the following attributes:

I)	a "bat" attribute specifying the arrival time of the first packet of the data burst; and

II)	a "periodicity" attribute specifying the time period between the start of two bursts;

B)	if the "bat" attribute is included, may include a "batWindow" attribute containing the acceptable earliest and latest arrival time of the first packet of the data burst; and

C)	if the "bat", "batWindow" and "periodicity" attributes are included, may include a "periodRange" attribute specifying the periodicity range. In the "periodRange" attribute the SDDM-C shall include:

I)	a "lowerBound" attribute set to the lower bound of the periodicity and an "upperBound" attribute set to the upper bound of the periodicity of the start two bursts; or

II)	a "periodicityValues" attribute set to the acceptable periodicity values;

2)	otherwise, shall include an "EstablishmentResponse" object with a "result" attribute set to "failure" and a "cause" attribute specifying the cause of the failure of the operation, e.g. VAL client error in the CoAP POST response; and

c)	shall send the CoAP POST response towards the SDDM-S.

# *** Next Change ***

### A.2.4.2	Type: EstablishmentRequest

Table A.2.4.2.1: Definition of type EstablishmentRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | RequestorId | M | 1 | Identity of the requestor of the establishment request (NOTE 1). |  |
| sealddflowId | Uinteger | M | 1 | Identity of SDDM flow used by the SDDM-C and SDDM-S to identify the application traffic. |  |
| endpointId | string | M | 1 | Identity of the endpoint of the selected VAL server to which the establishment request has to be sent. |  |
| sealddCommunicationLifetime | Uinteger | O | 0..1 | Information of the data delivery communication lifetime in milliseconds (NOTE 2). |  |
| valServiceId | string | O | 0..1 | Identity of the VAL service enabled by the SDD regular transmission connection. |  |
| userPlaneAddress | string | O | 0..1 | Identity of the IP address of the traffic. |  |
| portNumber | Uinteger | O | 0..1 | Identity of the port number of the traffic. |  |
| url | string | O | 0..1 | Identity of the address of a given unique resource on the Web for the traffic. |  |
| transportLayer | string | O | 0..1 | Identity of the transport layer protocol for the traffic. |  |
| valTgtUe | ValTargetUe | O | 0..1 | VAL user to whom the establishment request is applied. |  |
| l4sFeedbackCapability | L4sFeedbackCapability | O | 0..1 | Information of the L4S feedback capability (i.e. ECN identification, L4S feedback). |  |
| batPeriodAdaptCap | boolean | O | 0..1 | Indicates a BAT and periodicity adaptation capability of the SEALDD client side. (NOTE 3, NOTE 4) |  |
| transmisAssistInfo | TransmissionAssistInfo | O | 0..1 | Indicates a transmission assistance information for uplink SEALDD traffic. It includes a burst arrival time (BAT), BAT time window, periodicity, and periodicity range. (NOTE 3, NOTE 4) |  |
| xrAppDevCap | XrAppDevCapability | O | 0..1 | Indicates an XR application device capability information. |  |
| NOTE 1: Thie requestorId attribute shall be set to either "SEALDDSERVER" if the requesting entity is the SDDM-S or."SEALDDCLIENT" if the requesting entity is the SDDM-C. NOTE 2: The sealddCommunicationLifetime attribute shall be included when the requesting entity is the SDDM-S. This attribute shall be included when the requesting entity is the SDDM-C. NOTE 3: This attribute may only be included in the SDDM-C initiated request. NOTE 4: The "batPeriodAdaptCap" attribute and "transmisAssistInfo" attribute are mutually exclusive. |  |  |  |  |  |

# *** Next Change ***

### A.2.4.9	Type: XrAppDevCapability

Table A.2.4.9.1: Definition of type XrAppDevCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mediaCodec | MediaCodec | O | 0..1 | Indicates media codec information. |  |
| mediaResolution | MediaResolution | O | 0..1 | Indicates media resolution information in pixels. |  |
| mediaFrameRate | Uinteger | O | 0..1 | Indicates the media frame rate information. |  |
| mediaFov | MediaFov | O | 0..1 | Indicates the media Field of View information in degrees. |  |

# *** Next Change ***

### A.2.4.15	Type: MediaFOV

Table A.2.4.15.1: Definition of type MediaFOV

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| horizontalFov | Uinteger | M | 1 | Indicates the maximum horizontal field-of-view in degrees. |  |
| verticalFov | Uinteger | M | 1 | Indicates the maximum vertical field-of-view in degrees. |  |

# *** Next Change ***

### A.2.6.5	Enumeration: MediaCodec

Table A.2.6.5.1: MediaCodec

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| H_264 | Indicates the H.264 codec information. |  |
| H_265 | Indicates the H.265 codec information. |  |

# *** Next Change ***

### A.2.6.6	Enumeration: MediaResolution

Table A.2.6.6.1: MediaResolution

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| 4320P | Indicates the 4320p media resolution information. |  |
| 2160P | Indicates the 2160p media resolution information. |  |
| 1440P | Indicates the 1440p media resolution information. |  |
| 1080P | Indicates the 1080p media resolution information. |  |
| 720P | Indicates the 720p media resolution information. |  |
| 480P | Indicates the 480p media resolution information. |  |
| 360P | Indicates the 360p media resolution information. |  |
| 240P | Indicates the 240p media resolution information. |  |

# *** Next Change ***

#### A.3.1.5.2	CDDL document

;;; EstablishmentRequest

;;+ Represents a request for establishing an SDDM regular transmission connection.

EstablishmentRequest = {

requestorId: RequestorId

sealddFlowId: Uinteger

serverId: ServerId

endpointId: string

? sealddCommunicationLifetime: Uinteger

? valServiceId: string

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

? transportLayer: string

? valTgtUe: ValTargetUe

? l4sFeedbackCapability: L4sFeedbackCapability

? xrAppDevCap: XrAppDevCapability

}

;;; EstablishmentResponse

;;+ Represents a response of establishing an SDDM regular transmission connection.

EstablishmentResponse = {

result: ResultOp

? cause: Cause

? userPlaneAddress: string

? portNumber: Uinteger

? url: string

.?.batAndPeriodicityCapability: boolean

.?.transmisAssistInfo: TransmissionAssistInfo

? transportLayer: string

}

;;; ReleaseRequest

;;+ Represents a request for releasing an SDDM regular transmission connection.

ReleaseRequest = {

serverId: ServerId

sealddFlowId: Uinteger

}

;;; RequestorId

;;+ Indicates requestor of an operation.

RequestorId = "SEALDDCLIENT" / "SEALDDSERVER"

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

;;; ValTargetUe

;;+ Represents information identifying a VAL user ID or a VAL UE ID.

valUserId = {

valUserId: text                 ; Unique identifier of a VAL user.

}

valUeId = {

valUeId: text                   ; Unique identifier of a VAL UE.

}

ValTargetUe = valUserId / valUeId

;;; ServerId

;;+ Represents information identifying a unique server.

serverId = text

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; Cause

;;+ Represents the cause of failure of an operation.

Cause = "VAL CLIENT ERROR" / "SEALDD POLICY MISMATCH" / "OTHER"

;;; TransmissionAssistInfo

;;+ Indicates a transmission assistance information for uplink SEALDD traffic.

TransmissionAssistInfo = {

? bat: DateTime

? periodicity: Uinteger

? batWindow: TimeWindow

? periodRange: PeriodicityRange

}

;;; TimeWindow

;;+ Indicates the acceptable earliest and latest arrival time of the first packet of the data burst. The start time contains the earliest acceptable arrival time, and the stop time contains the latest acceptable arrival time.

TransmissionAssistInfo = {

? startTime: DateTime

? stopTime: DateTime

}

;;; PeriodicityRange

;;+ Indicates the acceptable time period range between the start of two bursts or the acceptable periodicity value(s).

PeriodicityRange = {

? lowerBound: Uinteger

? upperBound: Uinteger

? periodicityValues: [* Uinteger]

}

;;; L4sFeedbackCapability

;;+ Indicates the L4S feedback capability (i.e. ECN identification, L4S feedback).

L4sFeedbackCapability = {

ecnMarking: EcnMarking

l4sFeeedback: L4sFeedback

}

;;; EcnMarking

;;+ Indicates the ECN marking.

EcnMarking = {

supportsEct0: boolean

? ect0Value: string

supportsEct1: boolean

? ect1Value: string

}

;;; L4sFeedback

;;+ Indicates the L4S feedback.

L4sFeedback = {

perPacketCeReporting: boolean

? feedbackInterval: Uinteger      ; How often feedback is sent.

? maxFeedbackFrequency: Uinteger  ; The fastest rate at which feedback may be sent.

? protocol: string                ; Transport protocol e.g. "RTCP", "in-band".

}

;;; XrAppDevCapability

;;+ Indicates the XR application device capability information.

XrAppDevCapability = {

? mediaCodec: MediaCodec

? mediaResolution: MediaResolution

? mediaFrameRate: Uinteger

? mediaFov: MediaFov

}

;;; MediaCodec

;;+ Represents the media codec information.

MediaCodec = "H_264" / "H_265"

;;; MediaResolution

;;+ Represents the media resolution information.

MediaResolution = "4320P" / "2160P" / "1440P" / "1080P" / "720P" / "480P" / "360P" / "240P"

;;; MediaFov

;;+ Indicates the media field of view information.

MediaFov = {

horizontalFov: Uinteger

verticalFov: Uinteger

}

# *** End of Changes ***