3GPP TSG-CT WG1 Meeting #157	C1-256829

# Sophia-Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.543 | CR | 0105 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding sleeping duration attribute to Connection Status Notification - CoAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Stage-2 a new attribute, sleeping duration, was introduced in S6-253249 to the SEALDD connection status procedure. This attribute needs to be added to 3GPP TS 24.543 to align with Stage-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The sleeping duration attribute is added to the Connection status Notification for CoAP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with Stage-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.21.3, A.3.4.3.2.4, A.3.4.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 7.2.21.3	SDDM client CoAP procedure

Upon reception of a CoAP FETCH request message containing:

a)	an "observe" option set to the value "0" (register);

b)	an Accept option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-notification";

c)	a Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-config-subsc"; and

d)	a "ConnectionStatusConfigurationSubscription" object,

the SDDM-C shall provide an SEALDD connection status reporting notification to the SDDM-S within a CoAP FETCH 2.05 (Content) response (as specified in IETF RFC 8132 [17]). In the CoAP FETCH 2.05 (Content) response the SDDM-C:

a)	shall include Content-Format option set to "application/vnd.3gpp.seal-data-delivery-info+cbor;modeltype=connection-status-notification";

b)	shall include a "ConnectionStatusNotification" object containing:

1)	a "clientConnectionStatus" attribute set to the status of the VAL UE, i.e. reachable, unreachable, or sleeping;

2)	an "accessUsage" attribute indicating which access (3GPP or non-3GPP) is used for the SEALDD-UU data transmission;

3)	a "non3gppAccessMeasurements" attribute to indicate measurement results for the requested non-3GPP access measurement policies; and

4)	if the "clientConnectionStatus" attribute is set to "SLEEPING", may include a "sleepingDuration" attribute to indicate the duration the client connection status of the VAL client is set to sleeping, in units of microseconds; and

c)	shall send CoAP FETCH 2.05 (Content) response towards the SDDM-S.

# *** Next Change ***

##### A.3.4.3.2.4	Type: ConnectionStatusNotification

Table A.3.4.3.2.4.1: Definition of type ConnectionStatusNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientConnectionStatus | ConnectionStatus | M | 1 | Indicates the connection status of the VAL client, i.e. reachable, unreachable, or sleeping. |  |
| accessUsage | AccessUsage | M | 1 | Indicates which access is used for the SEALDD-UU data transmission. |  |
| non3gppAccessMeasurements | array(Non3gppAccessMeasurement) | O | 1..N | Information of the non-3GPP access measurement information report. |  |
| sleepingDuration | Uinteger | O | 0..1 | Indicates the duration the client connection status of the VAL client is set to sleeping, in units of microseconds. (NOTE) |  |
| NOTE: This attribute may only be included if the "clientConnectionStatus" is set to "SLEEPING". |  |  |  |  |  |

# *** Next Change ***

#### A.3.4.5.2	CDDL document

;;; ConnectionStatusConfigurationRequest

;;+ Represents a request for performing SDDM connection status reporting.

ConnectionStatusConfigurationRequest = {

sealddFlowId: Uinteger

? reportingMode: ReportingMode

? reportingInterval: Uinteger

? reportingPriority: ReportingPriority

? non3gppAccessPolicy: Non3gppAccessPolicy

}

;;; ConnectionStatusConfigurationResponse

;;+ Represents the response of a request for performing SDDM connection status reporting.

ConnectionStatusConfigurationResponse = {

result: ResultOp

}

;;; ConnectionStatusConfigurationSubscription

;;+ Represents a request for performing SDDM connection status reporting notification.

ConnectionStatusConfigurationSubscription = {

sealddFlowId: Uinteger

}

;;; ConnectionStatusNotification

;;+ Represents an SDDM connection status reporting notification.

ConnectionStatusNotification = {

clientConnectionStatus: ConnectionStatus

accessUsage: AccessUsage

? non3gppAccessMeasurements: [* Non3gppAccessMeasurement]

? sleepingDuration: Uinteger

}

;;; Non3gppAccessMeasurement

;;+ Information of the non-3GPP access measurement information report.

? measuredAccess: MeasuredNon3gppAccess

signalStrengthValues: [* Uinteger]

;;; MeasuredNon3gppAccess

;;+ Identity of the measured non-3GPP access i.e. WLAN SSID or WLAN BSSID.

? ssId: string

? bssId: string

;;; ResultOp

;;+ Represents the result of an operation.

ResultOp = "SUCCESS" / "FAILURE"

;;; ConnectionStatus

;;+ Represents the VAL client connection status.

ConnectionStatus = "REACHABLE" / "UNREACHABLE" / "SLEEPING"

;;; ReportingPriority

;;+ Indicates the priority of SEALDD client connection status for the requested SEALDD flow ID.

ReportingPriority = "LOW" / "HIGH"

;;; ReportingMode

;;+ Represents the mode of the reporting.

ReportMode = "PERIODIC" / "EVENT_TRIGGERED"

;;; Non3gppAccessPolicy

;;+ Represents the non-3GPP access measurement policy.

Non3gppAccessPolicy = "WLAN SSID" / "WLAN BSSID" / "LOCATION_BASED"

;;; AccessUsage

;;+ Represents the type of the used access.

AccessUsage = "3GPP_ACCESS" / "NON_3GPP_ACCESS"

;;; Uinteger

;;+ Unsigned Integer, i.e. only value 0 and integers above 0 are permissible.

Uinteger = int .ge 0

# *** End of Changes ***