3GPP TSG-CT WG1 Meeting #157	C1-256521

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7031 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on the QoS differentiation of traffic for non-3GPP device(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As S2-2507791, SA2 has agreed updates to TS 23.501 such that how the UE determines which PDU session to use for the non-3GPP device’s traffic can use the existing mechanisms. In current Stage 3 specification, it is defined as: NOTE 2: To support QoS differentiation of traffic for non-3GPP device(s) connecting through the UE, how the UE determines which PDU session to use for the non-3GPP device traffic is up to UE implementation. Hence, it is proposed to align stage 3 specification to udpate the NOTE accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clatify that how the UE determines which PDU session to use for the non-3GPP device’s traffic can use the existing mechanisms |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications for QoS differentiation of traffic for non-3GPP device(s) scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

## 4.30	QoS differentiation of traffic for non-3GPP device identifier

A 5GS can support QoS differentiation of traffic for non-3GPP devices connecting through the UE, as specified in 3GPP TS 23.501 [8]. The UE and the network may support QoS differentiation of traffic for non-3GPP devices.

The UE shall bind a non-3GPP device identifier to a non-3GPP device, for the traffic of the non-3GPP device that requires differentiated QoS (see 3GPP TS 23.501 [8] and 3GPP TS 23.316 [6D]). A non-3GPP device identifier shall be bound to no more than one non-3GPP device simultaneously. This binding enables the 5GS to distinguish between the traffic generated by different non-3GPP devices connected through the same UE.

NOTE 1:	QoS differentiation of traffic for the non-3GPP device identifier also applies to the wireline access when a non-3GPP device is connecting to the 5G-RG.

NOTE 2:	To support QoS differentiation of traffic for non-3GPP device(s) connecting through the UE, how the UE determines which PDU session to use for the non-3GPP device traffic and when to initiate the UE-requested PDU session modification procedure is up to UE implementation.

The UE enables QoS differentiation of traffic from one or more non-3GPP devices associated with non-3GPP device identifiers by providing each non-3GPP device identifier and the corresponding connection information to the SMF in the UE-requested PDU session modification procedure. The UE shall provide the non-3GPP device connection information for each non-3GPP device depending on the PDU session type of the PDU session, as follows:

a)	for Ethernet PDU session type the UE shall include the non-3GPP device identifier and the Ethernet connection information;

b)	for IPv4 PDU session type the UE shall include the non-3GPP device identifier and the IPv4 connection information;

c)	for IPv6 PDU session type the UE shall include the non-3GPP device identifier and the IPv6 connection information; and

d)	for IPv4v6 PDU session type the UE shall include the non-3GPP device identifier and:

1)	the IPv4 connection information;

2)	the IPv6 connection information; or

3)	both 1) and 2).

NOTE 3:	For an IPv4v6 PDU session type, it is up to UE implementation to decide whether to use IPv4 connection information, IPv6 connection information, or a combination of both. This decision can be based on the traffic associated with the non-3GPP device and the IP address allocation by the network for that PDU session.

# *** End of Changes ***