3GPP TSG-CT WG1 Meeting #157	C1-256437

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7050 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the Non-3GPP device identifier connection information IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the IE encoding of Non-3GPP device identifier connection information, several errors exists: 1) The unspecified figure figure 9.11.4.x3.1 should be figure 9.11.4.41.1; 2) The "If no IPv4 address is included in the connection information for the non-3GPP device identifier, the IPv4 address of the PDU session applies" should be moved after the IPv4 address presence indicator since it applies to the indicater set to "No IPv4 address included"; 3) Missing "+" for octet s+10 for IPv4 port range field; 4) Duplicate "octet octet"; 5) Wrong bits specified for IPv6 addess or prefix presence indicator field. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix the above listed errors in Non-3GPP device identifier connection information IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above errors remain in the Non-3GPP device identifier connection information IE which cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.4.41 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.11.4.41	Non-3GPP device identifier connection information

The purpose of the Non-3GPP device identifier connection information information element is to indicate one or more non-3GPP device identifiers and the corresponding connection information.

The Non-3GPP device identifier connection information information element is coded as shown in figure 9.11.4.41.1, figure 9.11.4.41.2, figure 9.11.4.41.3, figure 9.11.4.41.4, figure 9.11.4.41.5, figure 9.11.4.41.6, and table 9.11.4.41.1.

The Non-3GPP device identifier connection information information element is a type 6 information element with minimum length of 8 octets.

| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Non-3GPP device identifier connection information IEI |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of non-3GPP device identifier connection information contents |  |  |  |  |  |  |  |  |  |  |  | octet 2 octet 3 |
| 0 Spare |  | 0 Spare | 0 Spare | 0 Spare |  | 0 Spare |  | PDU session type value |  |  |  | octet 4 |
| Non-3GPP device identifier connection information 1 |  |  |  |  |  |  |  |  |  |  |  | octet 5 octet u |
| Non-3GPP device identifier connection information 2 |  |  |  |  |  |  |  |  |  |  |  | octet (u+1)* octet w* |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |
| Non-3GPP device identifier connection information n |  |  |  |  |  |  |  |  |  |  |  | octet (w+1)* octet m* |

Figure 9.11.4.41.1: Non-3GPP device connection information information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of non-3GPP device identifier connection information |  |  |  |  |  |  |  | octet 5 |
| Length of non-3GPP device identifier |  |  |  |  |  |  |  | octet 6 |
| Non-3GPP device identifier |  |  |  |  |  |  |  | octet 7 octet s |
| Connection information |  |  |  |  |  |  |  | octet (s+1) octet u |

Figure 9.11.4.41.2: Non-3GPP device identifier connection information

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | I4PRPI | I4API | octet (s+1) |
| Spare | Spare | Spare | Spare | Spare | Spare |  |  |  |
| IPv4 address |  |  |  |  |  |  |  | octet (s+2)* octet (s+5)* |
| Number of port ranges for IPv4 |  |  |  |  |  |  |  | octet (s+6)* |
| IPv4 port range 1 |  |  |  |  |  |  |  | octet (s+7)* octet (s+10)* |
| … |  |  |  |  |  |  |  |  |
| IPv4 port range x |  |  |  |  |  |  |  | octet v* octet (v+3)* |

Figure 9.11.4.41.3: Connection information when PDU session type value = "IPv4"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 Spare | I6PRPI | I6ATI | octet (s+1) |
| Spare | Spare | Spare | Spare | Spare |  |  |  |  |
| IPv6 address |  |  |  |  |  |  |  | octet (s+2)* octet (s+17)* |
| IPv6 prefix |  |  |  |  |  |  |  | octet (s+2)* octet (s+18)* |
| Number of port ranges for IPv6 |  |  |  |  |  |  |  | octet a* |
| IPv6 port range 1 |  |  |  |  |  |  |  | octet (a+1)* octet (a+4)* |
| … |  |  |  |  |  |  |  |  |
| IPv6 port range y |  |  |  |  |  |  |  | octet b* octet (b+3)* |

Figure 9.11.4.41.4: Connection information when PDU session type value = "IPv6"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 Spare | I6PRPI | I6APPI |  | I4PRPI | I4API |  | octet (s+1) |
| Spare |  |  |  |  |  |  |  |  |
| IPv4 address |  |  |  |  |  |  |  | octet (s+2)* octet (s+5)* |
| Number of port ranges for IPv4 |  |  |  |  |  |  |  | octet c* |
| IPv4 port range 1 |  |  |  |  |  |  |  | octet (c+1)* octet (c+4)* |
| … |  |  |  |  |  |  |  |  |
| IPv4 port range x |  |  |  |  |  |  |  | octet d* octet (d+3)* |
| IPv6 address |  |  |  |  |  |  |  | octet e* octet (e+15)* |
| IPv6 prefix |  |  |  |  |  |  |  | octet e* octet (e+16)* |
| Number of port ranges for IPv6 |  |  |  |  |  |  |  | octet f* |
| IPv6 port range 1 |  |  |  |  |  |  |  | octet (f+1)* octet (f+4)* |
| … |  |  |  |  |  |  |  |  |
| IPv6 port range y |  |  |  |  |  |  |  | octet g* octet (g+3)* |

Figure 9.11.4.41.5: Connection information when PDU session type value = "IPv4v6"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | VTPI | octet s+1 |
| Spare | Spare | Spare | Spare | Spare | Spare | Spare |  |  |
| MAC address |  |  |  |  |  |  |  | octet s+2 octet s+7 |
| VLAN tag ID |  |  |  |  |  |  |  | octet (s+8)* octet (s+9)* |

Figure 9.11.4.41.6: Connection information when PDU session type value = "Ethernet"

Table 9.11.4.41.1: Non-3GPP device connection information information element

| PDU session type value (octet 4, bits 1 to 3) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |
| 3 | 2 |  | 1 |  |  |  |
| 0 | 0 |  | 1 |  |  | IPv4 |
| 0 | 1 |  | 0 |  |  | IPv6 |
| 0 | 1 |  | 1 |  |  | IPv4v6 |
| 1 | 0 |  | 1 |  |  | Ethernet |
| All other values are reserved. |  |  |  |  |  |  |
| Non-3GPP device identifier (octet 7 to octet s) |  |  |  |  |  |  |
| The non-3GPP device identifier field contains the non-3GPP device identifier associated with a non-3GPP device connecting through the UE. The structure of the non-3GPP device identifier is defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |  |
| Connection information (octet s+1 to octet u) |  |  |  |  |  |  |
| The connection information field contains the connection information details for QoS differentiation of the non-3GPP device identifier indicated according to figures 9.11.4.3, 9.11.4.4, 9.11.4.5 and 9.11.4.6, depending on the PDU session type. |  |  |  |  |  |  |
| For PDU session type value = "IPv4" |  |  |  |  |  |  |
| IPv4 address presence indicator (I4API) (octet s+1, bit 1) |  |  |  |  |  |  |
| The I4API field indicates whether IPv4 address is included in the connection information or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 0 |  |  |  |  | No IPv4 address included |  |
| 1 |  |  |  |  | IPv4 address included |  |
| If no IPv4 address is included in the connection information for the non-3GPP device identifier, the IPv4 address of the PDU session applies. |  |  |  |  |  |  |
| IPv4 port range presence indicator (I4PRPI) (octet s+1, bit 2) |  |  |  |  |  |  |
| The I4PRPI field indicates whether IPv4 port range fields are included in the IE or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 0 |  |  |  |  | No IPv4 port ranges included |  |
| 1 |  |  |  |  | One or more IPv4 port ranges included |  |
| IPv4 address (octet s+2 to octet s+5) |  |  |  |  |  |  |
| The IPv4 address field contains an IPv4 address for the QoS differentiation of the non-3GPP device identifier. |  |  |  |  |  |  |
| Number of port ranges for IPv4 (octet (s+6)) |  |  |  |  |  |  |
| The number of port ranges for IPv4 field indicates the number of IPv4 port ranges for the non-3GPP devices connecting to the UE, and shall be included if the I4PRPI field indicates "One or more IPv4 port ranges included". |  |  |  |  |  |  |
| IPv4 port range (octet s+7 to octet s+10) |  |  |  |  |  |  |
| The IPv4 port range field indicates the port range associated with the traffic for the non-3GPP device identifier. The IPv4 port range field consists of the lowest port number in octets s+7 and s+8, and the highest port number in octets s+9 and s+10. |  |  |  |  |  |  |
| For PDU session type value = "IPv6" |  |  |  |  |  |  |
| IPv6 address type indicator (I6ATI) (octet s+1, bit 1) |  |  |  |  |  |  |
| The I6ATI field indicates whether the IPv6 address field includes an IPv6 address or an IPv6 prefix. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 0 |  |  |  |  | IPv6 address included |  |
| 1 |  |  |  |  | IPv6 prefix included |  |
| IPv6 port range presence indicator (I6PRPI) (octet s+1, bit 2) |  |  |  |  |  |  |
| The I6PRPI field indicates whether the IPv6 port range fields are included in the connection information or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 0 |  |  |  |  | No IPv6 port ranges included |  |
| 1 |  |  |  |  | One or more IPv6 port ranges included |  |
| IPv6 address (octet s+2 to octet s+17) |  |  |  |  |  |  |
| The IPv6 address field contains an IPv6 address. |  |  |  |  |  |  |
| IPv6 prefix (octet s+2 to octet s+18) |  |  |  |  |  |  |
| The IPv6 prefix field contains an IPv6 address in octets s+2 to octet s+17, and a prefix length value in octet s+18. |  |  |  |  |  |  |
| Number of port ranges for IPv6 (octet a) |  |  |  |  |  |  |
| The number of port ranges for IPv6 field indicates the number of IPv6 port ranges for the non-3GPP devices connecting to the UE, and shall be included if the I6PRPI field indicates "One or more IPv6 port ranges included". |  |  |  |  |  |  |
| IPv6 port range (octet (a+1) to octet (a+4)) |  |  |  |  |  |  |
| The IPv6 port range field indicates the port range associated with the traffic for the non-3GPP device identifier. The IPv6 port range field consists of the lowest port number in octets a+1 and a+2, and the highest port number in octets a+3 and a+4. |  |  |  |  |  |  |
| For PDU session type value = "IPv4v6" |  |  |  |  |  |  |
| IPv4 addess presence indicator (I4API) (octet s+1, bits 1 to 2) |  |  |  |  |  |  |
| The I4API field indicates whether a IPv4 addess field or is applicable the connection information or not, and whether an applicable IPv4 addres is included in the connection information or not. |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |
| 2 | 1 |  |  |  |  |  |
| 0 | 0 |  |  |  |  | No IPv4 address applicable |
| 0 | 1 |  |  |  |  | Applicable IPv4 address included in the connection information |
| 1 | 0 |  |  |  |  | Applicable IPv4 address not included in the connection information. PDU session IPv4 address applies. |
| 1 | 1 |  |  |  |  | reserved |
| IPv4 port range presence indicator (I4PRPI) (octet s+1, bit 3) |  |  |  |  |  |  |
| The I4PRPI field indicates whether IPv4 port range fields are included in the IE or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 0 |  |  |  |  | No IPv4 port ranges included |  |
| 1 |  |  |  |  | One or more IPv4 port ranges included |  |
| IPv6 addess or prefix presence indicator (I6APPI) (octet s+1, bits 4 to 5) |  |  |  |  |  |  |
| The I6APPI field indicates whether a IPv6 addess field or IPv6 prefix is included in the IE or not. |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |
| 5 | 4 |  |  |  |  |  |
| 0 | 0 |  |  |  |  | No IPv6 address or IPv6 prefix included |
| 0 | 1 |  |  |  |  | IPv6 address included |
| 1 | 0 |  |  |  |  | IPv6 prefix included |
| 1 | 1 |  |  |  |  | reserved |
| IPv6 port range presence indicator (I6PRPI) (octet s+1, bit 6) |  |  |  |  |  |  |
| The I6PRPI field indicates whether the IPv6 port range fields are included in the IE or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 0 |  |  |  |  | No IPv6 port ranges included |  |
| 1 |  |  |  |  | One or more IPv6 port ranges included |  |
| IPv4 address (octet s+2 to octet s+5) |  |  |  |  |  |  |
| The IPv4 address field contains an IPv4 address applicable for the QoS differentiation of the non-3GPP device identifier. |  |  |  |  |  |  |
| Number of port ranges for IPv4 (octet c) |  |  |  |  |  |  |
| The number of port ranges for IPv4 field indicates the number of IPv4 port ranges for the non-3GPP devices connecting to the UE, and shall be included if the I4PRPI field indicates "One or more IPv4 port ranges included". |  |  |  |  |  |  |
| IPv4 port range (octet c+1 to octet c+4) |  |  |  |  |  |  |
| The IPv4 port range field indicates the port range associated with the traffic for the non-3GPP device identifier. The IPv4 port range field consists of the lowest port number in octets c+1 and c+2, and the highest port number in octets c+3 and c+4. |  |  |  |  |  |  |
| IPv6 address (octet e to octet e+15) |  |  |  |  |  |  |
| The IPv6 address field contains an IPv6 address. |  |  |  |  |  |  |
| IPv6 prefix (octet e to octet e+16) |  |  |  |  |  |  |
| The IPv6 prefix field contains an IPv6 address in octets e to octet e+15, and a prefix length value in octet e+16. |  |  |  |  |  |  |
| Number of port ranges for IPv6 (octet f) |  |  |  |  |  |  |
| The number of port ranges for IPv6 field indicates the number of IPv6 port ranges for the non-3GPP devices connecting to the UE, and shall be included if the I6PRPI field indicates "One or more IPv6 port ranges included". |  |  |  |  |  |  |
| IPv6 port range (octet f+1 to octet f+4) |  |  |  |  |  |  |
| The IPv6 port range field indicates the port range associated with the traffic for the non-3GPP device identifier. The IPv6 port range field consists of the lowest port number in octets f+1 and f+2, and the highest port number in octets f+3 and f+4. |  |  |  |  |  |  |
| For PDU session type value = "Ethernet" |  |  |  |  |  |  |
| VLAN tag ID presence indicator (VTPI) (octet (s+1), bit 1) |  |  |  |  |  |  |
| The VTPI field indicates whether the VLAN tag ID field is included in the IE or not. |  |  |  |  |  |  |
| Bit |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 0 |  |  |  |  | VLAN tag ID not included |  |
| 1 |  |  |  |  | VLAN tag ID included |  |
| MAC address (octet s+2 to octet s+7) |  |  |  |  |  |  |
| The MAC address field indicates the MAC address associated with the traffic for the non-3GPP device identifier. The MAC address is defined as in subclause 8 of IEEE Std 802 [43]. Bit 8 of octet s+2 is the most significant bit and bit 1 of octet s+7 is the least significant bit. |  |  |  |  |  |  |
| VLAN tag ID (octet s+8 to octet s+9) |  |  |  |  |  |  |
| The VLAN tag ID field indicates the VLAN tag ID associated with the traffic for the non-3GPP device identifier. The VLAN tag ID is defined as specified in IEEE Std 802.1Q [74]. Bit 8 of octet s+8 is the most significant bit and bit 1 of octet s+9 is the least significant bit. |  |  |  |  |  |  |

* * * End of Changes * * * *