3GPP TSG-CT1 Meeting #157	C1-256826

# Sophia Antipolis, France,  13th - 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.548 | CR | 0089 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of mobile meta connectivity procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In 3GPP TS 23.434, clause 14.3.9.2.3 (Procedure for establishing communication in Situational Awareness use case) has the procedures mobile meta service connectivity request and procedure mobile meta service connectivity response that have been specified between NRM server and NRM client. These are respectively step 6 and step 7 in Figure 14.3.9.2.3-1 of 3GPP TS 23.434. That Figure is shown below: A Stage 3 Protocol implementation for these procedures is proposed in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SNRM server and SNRM client HTTP Procedures are specified along with appropriate additions to the Application communication requirements info document, to the XML schema for application/vnd.3gpp.seal-app-comm-requirements-info+xml, and to the Data Semantics for Application communication requirements info document |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 Protocol implementation of the new feature related to metaverse study will not be included in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.13.x(new), 7.3.5, 7.4.6, 7.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

##### 6.2.3.13.x	Mobile meta service connectivity procedure

###### 6.2.3.13.x.1	SNRM server HTTP procedure

Upon receieving an HTTP POST request message containing the Content-Type header field set to "application/vnd.3gpp.seal-app-comm-requirements-info+xml", the SNRM-S shall generate an HTTP POST request message in accordance with IETF RFC 9110 [22]. In the HTTP POST request, the SNRM-S:

a)	shall set the Request-URI to the URI corresponding to the identity of the SNRM-C;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.seal-app-comm-requirements-info+xml";

c)	shall include the XML representing the mobile meta service connectivity request in the HTTP POST body. The XML shall be generated as specified in clause 7.4.6 by including root element <seal-app-comm-info> with the <mmeta-service-connectivity-request> element within the <anyExt> element. The <mmeta-service-connectivity-request> element:

1)	shall include a <VAL-service-id> element set to the VAL service identity associated with the request;

2)	shall include <session-info> element and shall include the sub-elements below:

i)	<mmeta-session-id> element set with the identity of the session;

ii)	<mmeta-session-type> element set with the value "UE-TO-NETWORK" or "UE-TO-UE";

iii)	<mmeta-qos-configuration> element that provides configuration of the QoS attributes for the VAL session within the mobile meta service and shall include at least one of the elements below:

A)	<packet-e2e-latency> element set with the end-to-end latency value for the packet transmission; or

B)	<packet-trans-interval> element set with the transmission interval between the consecutive packets; and

iv)	may include <VAL-ue-id-list> sub-element with one or more <VAL-ue-id> child elements set to the identities of the VAL UEs that are selected for the connectivity to be established for the mobile meta service communication;

3)	shall include <service-area> element set with the geographical or topological area where the mobile meta service is deployed;

4)	may include <validity-period> element set with the time for which the connectivity request is valid; and

5)	may include an <app-connectivity-context> element that may include the sub-elements:

i)	<location> element set with the location coordinates information of the VAL UE as specified in clause 7.4.2 of 3GPP TS 24.545 [8];

ii)	<speed> element set with the speed information of the VAL UE; and

iii)	<direction> element set with the direction information of the VAL UE.

###### 6.2.3.13.x.2	SNRM client HTTP procedure

Upon receieving an HTTP POST request message containing the Content-Type header field set to "application/vnd.3gpp.seal-app-comm-requirements-info+xml", the SNRM-C shall generate an HTTP POST request message in accordance with IETF RFC 9110 [22]. In the HTTP POST request, the SNRM-C:

a)	shall process the HTTP POST body carrying the XML specifying the mobile meta service connectivity request;

b)	shall generate an HTTP POST response message in accordance with IETF RFC 9110 [22]. In the HTTP POST request, the SNRM-C:

1)	shall set the Request-URI to the URI corresponding to the identity of the SNRM-S;

2)	shall include the XML specifying the mobile meta service connectivity response in the HTTP POST body. The XML shall be generated as specified in clause 7.4.6 by including root element <seal-app-comm-info> with the <mmeta-service-connectivity-response> element within the <anyExt> element. The <mmeta-service-connectivity-response> element:

i)	shall include <result> set with either "SUCCESS" or "FAILURE";

ii)	may include <cause> set wih a string specifying the cause of the failure if the result indicates failure; and

iii)	may include <val-ue-dynamic-info> sub-element that may include sub-elements:

A)	<val-ue-id> set to a value of the UE id; and

B) shall include a child element <state> set to the value "DISCONNECTED" or "WEAKLY CONNECTED" or "STRONGLY CONNECTED".

* * * * Second Change * * * *

### 7.3.5	Application communication requirements info document

Editor’s note: There is need to check against latest stage-2 requirements and also align within the present specification

The <seal-app-comm-info> element shall be the root element of the Application communication requirements info document.

The <seal-app-comm-info> element shall include one of the followings:

a)	a <app-connectivity-request> element;

b)	a <app-connectivity-notify> element;

c)	a <app-connectivity-context-request> element; and

d)	a <app-connectivity-context-response> element;

e)	may include a <mmeta-service-connectivity-request> element in the <anyExt> element; or

f)	may include a <mmeta-service-connectivity-response> element in the <anyExt> element;

The <app-connectivity-request> element:

a)	shall include an <source-val-ue-id> element;

b)	shall include an <source-ip-address> element;

c)	shall include an <VAL-service-id> element;

d)	shall include an <target-val-ue-id-list> element;

e)	may include an <app-service-requirements> element; or

f)	may include an <app-connectivity-context> element;

The <app-service-requirements> element may include any:

a)	a <packet-size> element;

b)	a <packet-trans-interval> element;

c)	a <packet-e2e-latency> element;

d)	a <packet-error-kpi> element; or

e)	a <bitrate> element;

The <app-connectivity-context> element may include any of the following:

a)	a <location> element;

b)	a <speed> element; or

c)	a <direction> element;

The <app-connectivity-notify> element:

a)	a <session-info> element;

b)	a <VAL-service-id> element;

c)	a <requestor-val-ue-id> element; and

d)	a <target-val-ue-id-list> element;

The <app-connectivity-context-request> element:

a)	a <requestor-val-ue-id> element;

b)	a <VAL-service-id> element; and

c)	a <VAL-specific-context> element.

The <app-connectivity-context-response> element may include and an <app-connectivity-context> element.

The <mmeta-service-connectivity-request> element:

a) 	shall include a <VAL-service-id> element;

b)	shall include <session-info> element;

c)	shall include <service-area> element;

d)	may include <validity-period> element; and

e)	may include a <app-connectivity-context> element.

The <session-info> element shall include:

a)	<mmeta-session-id> element;

b)	<mmeta-session-type> element; and

c)	<mmeta-qos-configuration> element; and may include

d)	<VAL-ue-id-list> element.

The <mmeta-qos-configuration> element shall include the elements below:

a)	<packet-e2e-latency> element; and

b)	<packet-trans-interval> element.

The <app-connectivity-context> sub-element that may include the sub-elements:

a) 	<location> element;

b)	<speed> element; and

c)	<direction> element.

The <mmeta-service-connectivity-response> element:

a)	shall include <result> element;

b)	may include <cause> element; and

c)	may include <val-ue-dynamic-info> element.

The <val-ue-dynamic-info> element may include:

a)	<val-ue-id> element.

The <val-ue-id> element shall include

a)	<state> element.

* * * * Third Change * * * *

### 7.4.6	XML schema for application/vnd.3gpp.seal-app-comm-requirements-info+xml

Editor’s note: There is need to check against latest stage-2 requirements and also align within the present specification

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:ns:sealAppCommunicationInfo:1.0" xmlns:sealappcomminfo="urn:3gpp:ns:sealAppCommunicationInfo:1.0" xmlns:sealloc="urn:3gpp:ns:sealLocationInfo:1.0" elementFormDefault="qualified" attributeFormDefault="unqualified" xmlns:xenc="http://www.w3.org/2001/04/xmlenc#">

<!-- the sealloc.xsd shall be referenced from 3GPP TS 24.545 -->

<xs:import namespace="urn:3gpp:ns:sealLocationInfo:1.0" schemaLocation="sealloc.xsd"/>

<!-- the root element -->

<xs:element name="seal-app-comm-info" type="sealappcomminfo:seal-app-comm-info-Type" id="mbs"/>

<xs:complexType name="seal-app-comm-info-Type">

<xs:sequence>

<xs:element name="app-connectivity-request" type="sealappcomminfo:app-connectivity-requestType" minOccurs="0"/>

<xs:element name="app-connectivity-notify" type="sealappcomminfo:app-connectivity-notifyType" minOccurs="0"/>

<xs:element name="app-connectivity-context-request" type="sealappcomminfo:app-connectivity-context-requestType" minOccurs="0"/>

<xs:element name="app-connectivity-context-response" type="sealappcomminfo:app-connectivity-context-responseType" minOccurs="0"/>

<xs:element name="version" type="xs:integer"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--app-connectivity-reqType-->

<xs:complexType name="app-connectivity-requestType">

<xs:sequence>

<xs:element name="source-val-ue-id" type="xs:string"/>

<xs:element name="source-ip-address" type="xs:string"/>

<xs:element name="VAL-service-id" type="xs:string"/>

<xs:element name="target-val-ue-id-list" type="sealappcomminfo:VAL-ue-id-listType"/>

<xs:element name="app-service-requirements" type="sealappcomminfo:app-service-requirementsType" minOccurs="0"/>

<xs:element name="app-connectivity-context" type="sealappcomminfo:app-connectivity-contextType" minOccurs="0"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--VAL-ue-id-listType-->

<xs:complexType name="VAL-ue-id-listType">

<xs:sequence>

<xs:element name="VAL-ue-id" type="xs:string"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--app-service-requirementsType-->

<xs:complexType name="app-service-requirementsType">

<xs:sequence>

<xs:element name="packet-size" type="xs:integer" minOccurs="0"/>

<xs:element name="packet-trans-interval" type="xs:integer" minOccurs="0"/>

<xs:element name="packet-e2e-latency" type="xs:integer" minOccurs="0"/>

<xs:element name="packet-error-kpi" type="xs:string" minOccurs="0"/>

<xs:element name="bitrate" type="xs:integer" minOccurs="0"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--app-connectivity-contextType-->

<xs:complexType name="app-connectivity-contextType">

<xs:sequence>

<xs:element name="location" type="sealloc:tPointCoordinate" minOccurs="0"/>

<xs:element name="speed" type="xs:integer" minOccurs="0"/>

<xs:element name="direction" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--app-connectivity-notifyType-->

<xs:complexType name="app-connectivity-notifyType">

<xs:sequence>

<xs:element name="session-info" type="xs:string"/>

<xs:element name="VAL-service-id" type="xs:string"/>

<xs:element name="requestor-val-ue-id" type="xs:string"/>

<xs:element name="target-val-ue-id-list" type="sealappcomminfo:VAL-ue-id-listType"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--app-connectivity-context-reqType-->

<xs:complexType name="app-connectivity-context-requestType">

<xs:sequence>

<xs:element name="requestor-val-ue-id" type="xs:string"/>

<xs:element name="VAL-service-id" type="xs:string"/>

<xs:element name="VAL-specific-context" type="xs:string"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--app-connectivity-context-responseType-->

<xs:complexType name="app-connectivity-context-responseType">

<xs:sequence>

<xs:element name="app-connectivity-context" type="sealappcomminfo:app-connectivity-contextType" minOccurs="0"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element of seal-app-comm-info-Type-->

<xs:element name="mmeta-service-connectivity-request" type="sealappcomminfo:mmeta-service-connectivity-requestType"/>

<xs:element name="mmeta-service-connectivity-response" type="sealappcomminfo:mmeta-service-connectivity-responseType"/>

<!-- mmeta-service-connectivity-requestType-->

<xs:complexType name="mmeta-service-connectivity-requestType">

<xs:sequence>

<xs:element name="VAL-service-id" type="xs:string"/>

<xs:element name="session-info" type="sealappcomminfo:session-info-Type"/>

<xs:element name="service-area" type="sealappcomminfo:service-area-Type"/>

<xs:element name="validity-period" type="xs:string" minOccurs="0"/>

<xs:element name="app-connectivity-context" type="sealappcomminfo:app-connectivity-context-Type" minOccurs="0"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

<xs:complexType>

<!-- session-info-Type -->

<xs:complexType name="session-info-Type">

<xs:sequence>

<xs:element name="mmeta-session-id" type="xs:string"/>

<xs:element name="mmeta-session-type" type="sealappcomminfo:mmetaSessionTypeType"/>

<xs:element name="mmeta-qos-configuration" type="sealappcomminfo:mmeta-qos-configuration-Type">

<xs:element name="VAL-ue-id-list" type="sealappcomminfo:tIDsListType" minOccurs="0"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

<xs:complexType>

<!-- mmetaSessionTypeType-->

<xs:simpleType name="mmetaSessionTypeType">

<xs:restriction base="xs:string">

<xs:enumeration value="ue-to-network"/>

<xs:enumeration value="ue-to-ue"/>

</xs:restriction>

</xs:simpleType>

<!-- mmeta-qos-configuration-Type-->

<xs:complexType name="mmeta-qos-configuration-Type">

<xs:sequence>

<xs:sequence>

<xs:element name="packet-e2e-latency" type="xs:positiveInteger"/>

<xs:element name="packet-trans-interval" type="xs:positiveInteger"/>

</xs:sequence>

<xs:sequence>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

<xs:complexType>

<!-- tIDsListType-->

<xs:complexType name="tIDsListType">

<xs:choice maxOccurs="unbounded">

<xs:element name="VAL-user-id" type="sealappcomminfo:contentType" minOccurs="0"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- contentType-->

<xs:complexType name="contentType">

<xs:choice>

<xs:element name="sealURI" type="xs:anyURI"/>

<xs:element name="sealString" type="xs:string"/>

<xs:element name="sealBoolean" type="xs:boolean"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<!-- service-area-Type --->

<xs:complexType name="service-area-Type">

<xs:sequence>

<xs:element name="t-location" type ="sealappcomminfo:tLocationType"

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- tLocationType-->

<xs:complexType name="tLocationType">

<xs:choice minOccurs="1" maxOccurs="1">

<xs:element name="Ncgi" type="sealappcomminfo:tNcgi" minOccurs="0"/>

<xs:element name="SaId" type="sealappcomminfo:tMbmsSaIdentity" minOccurs="0"/>

<xs:element name="MbsfnAreaId" type="sealappcomminfo:tMbsfnAreaIdentity" minOccurs="0"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

<xs:attribute name="type" type="sealappcomminfo:protectionType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- tNcgi-->

<xs:simpleType name="tNcgi">

<xs:restriction base="xs:string">

<xs:pattern value="\d{3}\d{3}[0-1]{28}"/>

</xs:restriction>

</xs:simpleType>

<!—tMbmsSaIdentityFormat-->

<xs:simpleType name="tMbmsSaIdentityFormat">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="65535"/>

</xs:restriction>

</xs:simpleType>

<!—tMbmsSaIdentity-->

<xs:complexType name="tMbmsSaIdentity">

<xs:simpleContent>

<xs:extension base="sealappcomminfo:tMbmsSaIdentityFormat">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<!-- tMbsfnAreaIdentityFormat-->

<xs:simpleType name="tMbsfnAreaIdentityFormat">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="255"/>

</xs:restriction>

</xs:simpleType>

<!-- tMbsfnAreaIdentity-->

<xs:complexType name="tMbsfnAreaIdentity">

<xs:simpleContent>

<xs:extension base="sealappcomminfo:tMbsfnAreaIdentityFormat">

<xs:attribute name="trigger-id" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<!-- protectionType-->

<xs:simpleType name="protectionType">

<xs:restriction base="xs:string">

<xs:enumeration value="Normal"/>

<xs:enumeration value="Encrypted"/>

</xs:restriction>

</xs:simpleType>

<!--app-connectivity-context-Type -->

<xs:complexType name="app-connectivity-context-Type">

<xs:sequence>

<xs:element name="location" type="xs:integer" minOccurs="0"/>

<xs:element name="speed" type="xs:integer" minOccurs="0"/>

<xs:element name="direction" type="xs:integer" minOccurs="0"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- mmeta-service-connectivity-responseType-->

<xs:complexType name="mmeta-service-connectivity-responseType">

<xs:sequence>

<xs:element name="result" type="sealappcomminfo:result-Type"/>

<xs:element name="cause" type="xs:string"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

<xs:complexType>

<!-- result-Type-->

<xs:simpleType name="result-Type">

<xs:restriction base="xs:string">

<xs:enumeration value="success"/>

<xs:enumeration value="failure"/>

</xs:restriction>

</xs:simpleType>

<!-- The following elements are added for extensibility and to be placed in the anyExt element of  mmeta-service-connectivity-responseType -->

<xs:element name="val-ue-dynamic-info" type="sealappcomminfo: val-ue-dynamic-info-Type"/>

<!--val-ue-dynamic-info-Type -->

<xs:complexType name="val-ue-dynamic-info-Type">

<xs:sequence>

<xs:element name="val-ue-id" type="xs:string"/>

<xs:element state="state" type="sealappcomminfo:state-Type"/>

<xs:element name="anyExt" type="sealappcomminfo:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

<xs:complexType>

<!-- state-Type-->

<xs:simpleType name="state-Type">

<xs:restriction base="xs:string">

<xs:enumeration value="disconnected"/>

<xs:enumeration value="weakly-connected"/>

<xs:enumeration value="strongly-connected"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

* * * * Fourth Change * * * *

### 7.5.5	ApplicationCommunicationRequirementsInfo document

Editor’s note: There is need to check against latest stage-2 requirements and also align within the present specification

The recipient of the ApplicationCommunicationRequirementsInfo XML ignores any unknown element and any unknown attribute.

The <seal-app-comm-info> element is the root element of the XML document. The <seal-app-comm-info> element contains anyone of these <app-connectivity-request>, <app-connectivity-notify> element, <app-connectivity-context-request> element,<app-connectivity-context-response> or <anyExt> as sub-element. The <anyExt> element contains <mmeta-service-connectivity-request>, and <mmeta-service-connectivity-response> as sub-elements.

The <app-connectivity-request> element is used to initiate the application coordinated communication. The <app-connectivity-request> element:

a)	shall include a <source-val-ue-id> element contains the identity of the VAL Client or VAL UE initiating the application coordinated communication;

b)	shall include a <source-ip-address> element contains the identity of IP address of the VAL UE performing the request;

c)	shall include a <VAL-service-id> element which contains the VAL service identity of the VAL application;

d)	shall include a <target-val-ue-id-list> element which contain one or more <VAL-ue-id> child elements that store the VAL identity(s) of the target VAL-UE; and

e)	may include an <app-service-requirements> element that shall include any of the following sub-elements:

a)	a <packet-size> element contains an integer to specify the size of the packet;

b)	a <packet-trans-interval> element contains an integer to specify the interval;

c)	a <packet-e2e-latency> element contains an integer to specify the end-to-end latency;

d)	a <packet-error-kpi> element contains an integer to specify the KPI for packet error; or

e)	a <bitrate> element contains an integer to specify the bit rate.

f)	The <app-connectivity-context> element that shall include any of the following sub-elements:

a)	a <location> element contains an location co-ordinate as defined as "tPointCoordinate";

b)	a <speed> element contains an integer to specify the speed; or

c)	a <direction> element contains a string used to indicate the direction.

The <app-connectivity-notify> element shared contains the notification information for the application coordinated communication initiated and shall include:

a)	a <session-info> element contains a string used to identify the application communication session;

b)	a <VAL-service-id> element contains the VAL service identity of the VAL application;

c)	a <requestor-val-ue-id> element contains one <VAL-ue-id> child element that store the VAL identity of the requestor VAL-UE; and

d)	a <target-val-ue-id-list> element which contain one or more <VAL-ue-id> child elements that store the VAL identity(s) of the target VAL-UE for whom the application coordinated communication is accepted by the SNRM-S.

The <app-connectivity-context-request> element:

a)	a <requestor-val-ue-id> element contains one <VAL-ue-id> child element that store the VAL identity of the requestor VAL-UE;

b)	a <VAL-service-id> element contains the VAL service identity of the VAL application; and

c)	a <VAL-specific-context> element contains a string used to identify the context.

The <app-connectivity-context-response> element may include and an <app-connectivity-context> element that shall include any of the following sub-elements:

a)	a <location> element contains a location co-ordinate as defined as "tPointCoordinate";

b)	a <speed> element contains an integer to specify the speed; or

c)	a <direction> element contains a string used to indicate the direction.

The <mmeta-service-connectivity-request> element:

a)	shall include a <VAL-service-id> sub-element set to the VAL service identity associated with the request;

b)	shall include <session-info> sub-element and shall include the sub-elements below:

a)	<mmeta-session-id> set with the identity of the session;

b)	<mmeta-session-type> set with the value "UE-TO-NETWORK" or "UE-TO-UE"; and

c)	<mmeta-qos-configuration> for configuration of the QoS attributes for the VAL session within the mobile meta service and shall include at least one of the elements below:

A)	<packet-e2e-latency> element set with the end-to-end latency value for the packet transmission; and

B)	<packet-trans-interval> element set with the transmission interval between the consecutive packets; and

d)	may include <VAL-ue-id-list> sub-element with one or more <VAL-ue-id> child elements set to the identities of the VAL UEs that are selected for the connectivity to be established for the mobile meta service communication;

c)	shall include <service-area> set with the geographical or topological area where the mobile meta service is deployed;

d)	may include <validity-period> set with the time for which the connectivity request is valid; or

e)	may include may include a <app-connectivity-context> sub-element that may include the sub-elements:

a)	<location> element set with the location coordinates information of the VAL UE as specified in clause 7.4.2 of 3GPP TS 24.545 [8];

b)	<speed> element set with the speed information of the VAL UE; and

c)	<direction> element set with the direction information of the VAL UE.

The <mmeta-service-connectivity-response> element:

a)	shall include <result> set with either "SUCCESS" or "FAILURE";

b) 	may include <cause> set wih a string specifying the cause of the failure if the result indicates failure; or

c)	may include <val-ue-dynamic-info> sub-element that may include sub-elements:

a)	<val-ue-id> set to a value of the UE id and shall include a child element <state> set to the value "DISCONNECTED" or "WEAKLY CONNECTED" or "STRONGLY CONNECTED".