3GPP TSG-CT1 Meeting #157	C1-256606

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7054 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to UE handling of collision between registration and de-registration procedure. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicion |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc19 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In CT1 #156 meeting a CR was submitted in C1-254897 for solving the below issue: When the UE initiates a deregistration request message with type switch off, the other ongoing procedures like mobility registration update and service request procedures are aborted and the deregistration procedure is continued. Similary the NW on receiving a deregistration procedure with type switch off will handle the message and abort ongoing procedure if any. But for a scenario where the UE has a valid security context, if the UE sent the initial NAS signalling messages like REGISTRATION REQUEST message or SERVICE REQUEST message integrity protected but unciphered, it is possible for the NW to activate ciphering and send the REGISTRATION ACCEPT message and SERVICE ACCEPT message ciphered. According to the current specification if the NAS message is ciphered all the subsequent messages needs to be ciphered otherwise the messags can be ignored. For a deregistration request message with type power off, there is no retry mechanism, so if the NW does not handle the unciphered deregistration request then the NW has no way to know that the UE is powered off. The initial solution proposed in C1-254897 was for the NW to handle the deregistration request with power off unciphered when the MRU/SR is ongoing. But several companies commented that it is better not to add NW handling for unciphered message, instead make sure that the UE release the connetion and send the new deregistraiton message as an initial message which will be treated by the network. But as it is a deregistration with power off, some UE implementaion may send the deregistration over the same connetion. So a clarification is added that the UE releases the connetion while aborting the ongoing registration when there is a need to trigger deregistration procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clarification that If the UE needs to trigger a de-registration procedure with power off, when the UE aborts the registration procedure, UE releases the connection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network unable to detect that the UE has switched off leading to wastage of paging resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.5.1.3.7	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Timer T3346 is running.

The UE shall not start the registration procedure for mobility and periodic registration update unless:

1)	the UE is in 5GMM-CONNECTED mode;

2)	the UE received a paging;

3)	the UE receives a NOTIFICATION message over non-3GPP access when the UE is in 5GMM-CONNECTED mode over non-3GPP access and in 5GMM-IDLE mode over 3GPP access;

4)	the UE is a UE configured for high priority access in selected PLMN or SNPN;

5)	the UE has an emergency PDU session established or is establishing an emergency PDU session;

6)	the UE receives a request from the upper layers to perform emergency services fallback;

7)	the UE receives the CONFIGURATION UPDATE COMMAND message as specified in subclause 5.4.4.3;

8)	the UE in NB-N1 mode is requested by the upper layer to transmit user data related to an exceptional event and:

-	the UE is allowed to use exception data reporting (see the ExceptionDataReportingAllowed leaf of the NAS configuration MO in 3GPP TS 24.368 [17] or the USIM file EFNASCONFIG in 3GPP TS 31.102 [22]); and

-	timer T3346 was not started when N1 NAS signalling connection was established with RRC establishment cause set to "mo-ExceptionData";

9)	the MUSIM UE needs to request a new 5G-GUTI assignment as specified in subclause 5.5.1.3.2; or

10)	the UE needs to report unavailability information due to discontinuous coverage.

The UE stays in the current serving cell and applies the normal cell reselection process.

NOTE 1:	It is considered an abnormal case if the UE needs to initiate a registration procedure for mobility and periodic registration update while timer T3346 is running independent on whether timer T3346 was started due to an abnormal case or a non-successful case.

If the registration procedure for mobility and periodic registration update was initiated for an MO MMTEL voice call (i.e. access category 4), for an MO MMTEL video call (i.e. access category 5), for an MO IMS registration related signalling (i.e. access category 9) or for NAS signalling connection recovery during an ongoing MO MMTEL voice call (i.e. access category 4), or during an MO MMTEL video call (i.e. access category 5) or during an ongoing MO IMS registration related signalling (i.e. access category 9), then a notification that the procedure was not initiated due to network congestion shall be provided to upper layers.

b)	The lower layers indicate that the access attempt is barred.

The UE shall not start the registration procedure for mobility and periodic registration update. The UE stays in the current serving cell and applies the normal cell reselection process. Receipt of the access barred indication shall not trigger the selection of a different core network type (EPC or 5GCN).

The registration procedure for mobility and periodic registration update is started, if still needed, when the lower layers indicate that the barring is alleviated for the access category with which the access attempt was associated.

ba)	The lower layers indicate that:

1)	access barring is applicable for all access categories except categories 0 and 2 and the access category with which the access attempt was associated is other than 0 and 2; or

2)	access barring is applicable for all access categories except category 0 and the access category with which the access attempt was associated is other than 0.

If the REGISTRATION REQUEST message has not been sent, the UE shall proceed as specified for case b. If the REGISTRATION REQUEST message has been sent, the UE shall proceed as specified for case e and, additionally, the registration procedure for mobility and periodic registration update is started, if still needed, when the lower layers indicate that the barring is alleviated for the access category with which the access attempt was associated. For additional UE requirements for both cases see subclause 4.5.5.

c)	T3510 timeout.

The UE shall abort the registration update procedure and the N1 NAS signalling connection, if any, shall be released locally.

If the UE has initiated the registration procedure in order to enable performing the service request procedure for emergency services fallback,the UE shall inform the upper layers of the failure of the emergency services fallback (see 3GP P TS 24.229 [14]). Otherwise, the UE shall proceed as described below.

d)	REGISTRATION REJECT message, other 5GMM cause values than those treated in subclause 5.5.1.3.5, and cases of 5GMM cause values #11, #15, #22, #31, #36, #72, #73, #74, #75, #76, #77, #78, #79, #80, #81 and #82, if considered as abnormal cases according to subclause 5.5.1.3.5.

Upon reception of the 5GMM causes #95, #96, #97, #99 and #111 the UE should set the registration attempt counter to 5.

The UE shall proceed as described below.

d1)	Lower layer failure to establish the RRC connection and:

1)	the UE is in a PLMN of its home country or in an EHPLMN (if the EHPLMN list is present) or in its subscribed SNPN;

2)	the registration procedure is neither for initiating an emergency PDU session nor for performing an emergency services fallback procedure;

3)	the UE does not have an emergency PDU session;

4)	the UE supports being configured with CustomLLFailureRetry5G as specified in 3GPP TS 24.368 [15A]); and

5)	CustomLLFailureRetry5G leaf is present as specified in 3GPP TS 24.368 [15A]);

The UE shall increment the UE implementation specific attempt counter for lower layer failure. If the UE implementation specific attempt counter for lower layer failure is less than the value of MaxMinRetry, the UE shall use the duration of MinRetryTimer when starting T3511. If the UE implementation specific attempt counter for lower layer failure is equal to the value of MaxMinRetry, the UE shall use the duration of MaxRetryTimer when starting T3502 and set the registration attempt counter to 5.

The UE shall abort the registration procedure and proceed as described below.

e)	Lower layer failure, release of the NAS signalling connection received from lower layers before the REGISTRATION ACCEPT or REGISTRATION REJECT message is received.

The UE shall abort the registration procedure and proceed as described below.

e1)	Lower layer failure, the lower layers indicate that the RRC connection has been suspended without a cell change before the REGISTRATION ACCEPT or REGISTRATION REJECT message is received.

The registration procedure for mobility and periodic registration update shall be aborted and re-initiated immediately. The UE shall set the 5GS update status to 5U2 NOT UPDATED.

f)	Change in the current TAI.

If the current TAI is changed before the registration procedure for mobility and periodic registration update is completed, the registration procedure for mobility and periodic registration update shall be aborted and re-initiated immediately. The UE shall set the 5GS update status to 5U2 NOT UPDATED.

g)	Registration procedure for mobility and periodic registration update and de-registration procedure collision.

If the UE receives a DEREGISTRATION REQUEST message without 5GMM cause value #11, #12, #13, #15, #36, #62, #74, #75 or #78 before the registration procedure for mobility and periodic registration update has been completed, the registration procedure for mobility and periodic registration update shall be aborted and the de-registration procedure shall be progressed.

If the UE receives a DEREGISTRATION REQUEST message with 5GMM cause value #11, #12, #13, #15, #36, #62, #74, #75 or #78 before the registration procedure for mobility and periodic registration update has been completed, the registration procedure for mobility and periodic registration update shall be progressed and the de-registration procedure shall be aborted.

NOTE 2:	The registration procedure for mobility and periodic registration update shall be aborted only if the DEREGISTRATION REQUEST message indicates in the access type that the access in which the registration procedure for mobility and periodic registration update was attempted shall be de-registered. Otherwise both the procedures shall be progressed.

h)	Void

i)	Transmission failure of REGISTRATION REQUEST message indication from the lower layers or the lower layers indicate that the RRC connection has been suspended with a cell change.

The registration procedure for mobility and periodic registration update shall be aborted and re-initiated immediately. The UE shall set the 5GS update status to 5U2 NOT UPDATED.

j)	Transmission failure of REGISTRATION COMPLETE message indication with change in the current TAI.

If the current TAI is not in the TAI list, the registration procedure for mobility and periodic registration update shall be aborted and re-initiated immediately. The UE shall set the 5GS update status to 5U2 NOT UPDATED.

If the current TAI is still part of the TAI list, it is up to the UE implementation how to re-run the ongoing procedure.

k)	Transmission failure of REGISTRATION COMPLETE message indication without change in the current TAI.

It is up to the UE implementation how to re-run the ongoing procedure.

l)	UE-initiated de-registration required.

If the de-registration procedure is triggered due to removal of USIM or entry update in the "list of subscriber data" or due to switch off:

The registration procedure for mobility and periodic registration update shall be aborted, and the UE initiated de-registration procedure shall be performed. The UE shall populate the 5GS mobile identity IE in the DEREGISTRATION REQUEST message with the same UE identity as used in the REGISTRATION REQUEST message for the aborted mobility and periodic registration update procedure;

NOTE 2a:	When the UE aborts the registration procedure for mobility and periodic registration update as above, as an implementation option, the UE releases the N1 NAS signalling connection locally before performing the de-registration procedure.

Otherwise:

the UE initiated de-registration procedure shall be initiated after successful completion of the registration procedure for mobility and periodic registration update.

m)	Timer T3447 is running

The UE shall not start any registration procedure for mobility and registration update with Uplink data status IE or Follow-on request indicator set to "Follow-on request pending" unless:

1)	the UE received a paging;

2)	the UE is a UE configured for high priority access in selected PLMN;

3)	the UE has an emergency PDU session established or is establishing an emergency PDU session;

4)	the UE receives a request from the upper layers to perform emergency services fallback;

5)	the MUSIM UE needs to request a new 5G-GUTI assignment as specified in subclause 5.5.1.3.2; or

6)	the UE in NB-N1 mode is requested by the upper layer to transmit user data related to an exceptional event and the UE is allowed to use exception data reporting (see the ExceptionDataReportingAllowed leaf of the NAS configuration MO in 3GPP TS 24.368 [17] or the USIM file EFNASCONFIG in 3GPP TS 31.102 [22]).

The UE stays in the current serving cell and applies the normal cell reselection process. The mobility and periodic registration update procedure is started, if still necessary, when timer T3447 expires or timer T3447 is stopped.

n)	Timer T3448 is running

The UE in 5GMM-IDLE mode shall not start any mobility and periodic registration update procedure with Follow-on request indicator set to "Follow-on request pending" unless:

1)	the UE is a UE configured for high priority access in selected PLMN;

2)	the UE which is only using 5GS services with control plane CIoT 5GS optimization received a paging request;

3)	the UE in NB-N1 mode is requested by the upper layer to transmit user data related to an exceptional event and the UE is allowed to use exception data reporting (see the ExceptionDataReportingAllowed leaf of the NAS configuration MO in 3GPP TS 24.368 [17] or the USIM file EFNASCONFIG in 3GPP TS 31.102 [22]);

4)	the UE has an emergency PDU session established or is establishing an emergency PDU session; or

5)	the UE receives a request from the upper layers to perform emergency services fallback.

The UE stays in the current serving cell and applies the normal cell reselection process. The mobility and periodic registration update procedure is started, if still necessary, when timer T3448 expires.

o)	UE is not registered to the access other than the access the REGISTRATION ACCEPT message is received and the 5GS registration result value in the 5GS registration result IE value in the REGISTRATION ACCEPT message is set to "3GPP access and non-3GPP access".

The UE shall consider itself as being registered to only the access where the REGISTRATION ACCEPT message is received.

p)	Access for localized services in current SNPN is no longer allowed.

If the mobility and periodic registration update is not for initiating an emergency PDU session, the registered SNPN is an SNPN selected for localized services in SNPN (see 3GPP TS 23.122 [5]) and:

-	access for localized services in SNPN is disabled; or

-	the validity information for the selected SNPN is no longer met;

the UE shall reset the registration attempt counter, stop T3510, abort the registration procedure for mobility and periodic registration update, locally release the NAS signalling connection, if any, and enter state 5GMM-REGISTERED.LIMITED-SERVICE or 5GMM-REGISTERED.PLMN-SEARCH in order to perform SNPN selection according to 3GPP TS 23.122 [5].

q)	Expiry or stopping of the last running Tsor-cm timer.

If the last running Tsor-cm has expired or was stopped while the registration procedure for mobility and registration update is in progress, the UE initiated de-registration procedure shall be initiated after successful completion of the registration procedure for mobility and periodic registration update.

NOTE 3:	As an implementation option, UE may locally release the N1 NAS signalling connection, locally deregister and perform PLMN selection.

For the cases c, d, d1 and e the UE shall proceed as follows:

Timer T3510 shall be stopped if still running.

For the cases c, d and e, if the registration procedure is neither for initiating an emergency PDU session nor for performing an emergency services fallback procedure, the registration attempt counter shall be incremented, unless it was already set to 5.

If the registration attempt counter is less than 5:

-	if the TAI of the current serving cell is not included in the TAI list or the 5GS update status is different to 5U1 UPDATED or if the registration procedure was triggered due to cases c, g, n, v in subclause 5.5.1.3.2, the UE shall start timer T3511, shall set the 5GS update status to 5U2 NOT UPDATED and change to state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE. When timer T3511 expires, the registration update procedure is triggered again.

-	if the TAI of the current serving cell is included in the TAI list, the 5GS update status is equal to 5U1 UPDATED, and the UE is not performing the registration procedure after an inter-system change from S1 mode to N1 mode, the UE shall keep the 5GS update status to 5U1 UPDATED and enter state 5GMM-REGISTERED.NORMAL-SERVICE or 5GMM-REGISTERED.NON-ALLOWED-SERVICE (as described in subclause 5.3.5.2). The UE shall start timer T3511. If in addition the REGISTRATION REQUEST message did not include the MICO indication IE or the Extended DRX IE, and:

-	the REGISTRATION REQUEST message indicated "periodic registration updating";

-	the registration procedure was initiated to recover the NAS signalling connection due to "RRC Connection failure" from the lower layers;

-	the registration procedure was initiated when the UE in 5GMM-CONNECTED mode over 3GPP access or 5GMM-CONNECTED mode with RRC inactive indication receives a fallback indication from lower layers; or

-	the registration procedure was initiated by the UE in 5GMM-CONNECTED mode with RRC inactive indication entering a cell in the current registration area belonging to an equivalent PLMN of the registered PLMN and not belonging to the registered PLMN,

and none of the other reasons for initiating the registration updating procedure listed in subclause 5.5.1.3.2 was applicable, the timer T3511 may be stopped when the UE either enters 5GMM-CONNECTED mode or performs an inter-system change from N1 mode to S1 mode.

-	if the TAI of the current serving cell is included in the TAI list, the 5GS update status is equal to 5U1 UPDATED and the UE is performing the registration procedure after an inter-system change from S1 mode to N1 mode, the UE shall change the 5GS update status to 5U2 NOT UPDATED and enter state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE. The UE shall start timer T3511.

-	If the procedure is performed via 3GPP access and the UE is operating in single-registration mode, the UE shall in addition handle the EPS update status as specified in 3GPP TS 24.301 [15] for the abnormal cases when a normal or periodic tracking area updating procedure fails and the tracking area attempt counter is less than 5 and the EPS update status is different from EU1 UPDATED.

If the registration attempt counter is equal to 5

-	the UE shall start timer T3502 if the value of the timer as indicated by the network is not zero, shall set the 5GS update status to 5U2 NOT UPDATED.

-	the UE shall delete the list of equivalent PLMNs (if any) or the list of equivalent SNPNs (if any) if the UE is not registering or has not registered to the same PLMN over both 3GPP access and non-3GPP access, and shall change to state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE or optionally to 5GMM-REGISTERED.PLMN-SEARCH in order to perform a PLMN selection, SNPN selection or SNPN selection for onboarding services according to 3GPP TS 23.122 [5].

-	if the value of T3502 as indicated by the network is zero, the UE shall perform the actions defined for the expiry of the timer T3502.

NOTE 4:	For case e) if the lower layer failure is on a cell which was selected due to network slice-based cell reselection (see 3GPP TS 23.501 [8]), the UE can as an implementation option change the S-NSSAI(s) in the requested NSSAI to try and find a suitable NR cell.

-	if the procedure is performed via NG-RAN access over non-satellite NG-RAN cells and the UE is operating in single-registration mode:

-	the UE shall in addition handle the EPS update status as specified in 3GPP TS 24.301 [15] for the abnormal cases when a normal or periodic tracking area updating procedure fails and the tracking area attempt counter is equal to 5; and

-	if the UE does not change to state 5GMM-REGISTERED.PLMN-SEARCH, the UE shall attempt to select E-UTRAN radio access technology. The UE may disable the N1 mode capability as specified in subclause 4.9.

-	if the procedure is performed via satellite NG-RAN access and the UE is operating in single-registration mode:

-	the UE shall in addition handle the EMM parameters EPS update status, EMM state, 4G-GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs and eKSI as specified in 3GPP TS 24.301 [15] for the abnormal cases when a normal or periodic tracking area updating procedure via satellite EUTRA access fails and the tracking area attempt counter is equal to 5; and

-	the UE may attempt to select NG-RAN over non-satellite NG-RAN cells or E-UTRAN radio access technology and proceed with appropriate 5GMM or EMM specific procedures. Additionally, The UE may disable the satellite NG-RAN capability as specified in subclause 4.9.4.

* * * Next Change * * * *

* * * End of Changes * * * *