3GPP TSG-CT WG4 Meeting #131	C4-254275

# Sophia Antipolis, France; 13th – 17th October 2025                                               was4098

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.175 | CR | 0096 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the presence condition of arMediaSpecification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As agreed CR(#1602) of TS23.228, the following information is addressed in the NOTE: NOTE 2: DC Media Specification, AR Media Specification and Avatar Media Specification are optional and they are used depending on the scenario. So, this CR propose to change the condition of arMediaSpecification to O and update the descriptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the condition of arMediaSpecification to O. Update the description for arMediaSpecification and dcMediaSpecification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The presence of arMediaSpecification is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any OpenAPI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Cover page: change the category fro B to A. Update the WIC to NG_RTC. |  |  |  |  |  |  |  |  |

* * *1st Change * * * *

6.2.6.2.3	Type: MediaInstructions

Table 6.2.6.2.3-1: Definition of type MediaInstructions

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mediaId | MediaId | M | 1 | The media ID uniquely identifies this media item within the list and used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the media ID it received from the IMS AS for referencing the same media. This field shall be null for instructions related to originating new media or for data channel media components de-multiplexed from a data channel multiplexing media which do not reuse the media ID of that data channel multiplexing media. |
| associatedMediaId | MediaId | C | 0..1 | The identifier of the original multiplexed media stream which is de-multiplexed. It shall be included only when the mediaId is null, mediaInstruction is set to "ORIGINATE" and the media stream identified by the value of this attribute is de-multiplexed. |
| mediaResourceType | MediaResourceType | M | 1 | The mediaResourceType identify the Media Resource types the Media instruction is intended for. |
| mediaInstruction | MediaInstruction | C | 0..1 | It shall be included if the mediaResourceType is set to "DC". When present, mediaInstruction includes instructions to the producer (IMS AS) for handling the media. |
| dcMediaSpecification | DcMediaSpecification | C | 0..1 | Represents the additional media specifications needed for data channel media stream from application layer. It shall be contained if the mediaResourceType is set to "DC". |
| arMediaSpecification | ArMediaSpecification | O | 0..1 | Represents the additional media specifications needed for AR communication services from application layer. |
| mediaProcessingUrl | Uri | C | 0..1 | The mediaProcessingURL indicates the address where MF receive service-related media instructions. This field shall be contained in the response when originating a new media. |
| audioVideoReNegotiationInd | AudioVideoReNegotiationInd | C | 0..1 | Represents the audio/video media re-negotiation indication information to connect the UE's audio/video media stream to MF which is required by the application data channel. It shall be included if the mediaResourceType is set to "AUDIO" or "VIDEO". |
| avatarMedia | AvatarMedia | O | 0..1 | Represents the additional media specifications needed for avatar communication services. |

* * * End of Changes * * * *