3GPP TSG-CT WG4 Meeting #131	C4-254283

# Sophia Antipolis, France; 13rd – 17th October 2025					(Revision of C4-254205)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.176 | CR | 0046 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the procedure for AR communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The value of "AR" has been removed in the common data type MediaResourceType in TS 29.571 in the pervisous meeting. It is because there is no media type “AR” and with this value, the AR media specification cannot be included in the DC media specification when the data carried in the data channel is used for AR communication. While, in the procedure of media context creation, it is corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the procedure for AR communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will lead to wrong implementation for AR communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any OpenAPI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correct some typos and add the condition that if the media resource type is set to "DC" or "VIDEO". |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.2.2.2.2	Creation of a new media context

The NF service consumer shall request a new context by using HTTP method POST with "{apiRoot}/nmf/<apiVersion>/contexts" as resource URI representing the "Contexts Collection", see clause 6.1.3.2.

Figure 5.2.2.2.2-1 Creation of a media context

1.	The NF Service Consumer shall send a POST request to create an "Individual Context" resource in the MF. The payload body of the POST request shall contain a representation of the individual context resource to be created.

The NF service consumer shall include in the HTTP message body within the MediaContext data type list of termination descriptors in the attribute "terminations". Each termination descriptor shall include list of media stream descriptors in the attribute "medias". Each media stream descriptor shall include:

-	media ID in the attribute "mediaId", i.e. a unique identity of the media stream within the media context instance, which is generated by the NF service consumer (e.g. IMS AS) based on implementation;

-	remote Mb specifications in the attribute "remoteMbEndpoint", i.e. the media stream IP address and port allocated at the remote endpoint, i.e. remote UE, remote network; and

-	media resource description, which includes:

1)	media resource type, e.g. DC, VIDEO, AUDIO;

2)	if media resource type is set to "DC", the DC media specification in the attribute "dcMedia" within the data type MediaInfo shall be included. The DC media specification shall include:

a)	media proxy configuration applicable to the media flow in the attribute "mediaProxyConfig";

b)	data channel mapping and configuration information in the attribute "streams" when originating or terminating data channel media flows on the Mb interface which shall include the SCTP stream Id for the DC, and may include subprotocol, order, maxRetry, maxTime and priority; and

c)	the remote SCTP endpoint and DTLS endpoint information in the attribute "remoteDcEndpoint" which includes the SCTP port for the Data Channel, the security setup of the DTLS connection, the security certificate fingerprint and the transport layer identity;

and may include:

a)	maximum message size in the attribute " maxMessageSize", which represents the maximum size to be expected;

for establishing bootstrap data channel, the DC media specification shall include the remote MDC1 media specification information in the attribute "remoteMdc1Endpoint" within the data type "Mdc1Info" to be applied on the media by the MF and also the replacement HTTP URL for each streamId allocated by the application layer representing the application list (e.g. graphical user interface) offered to the IMS subscriber via the MDC1 interface;

for establishing P2A/A2P application data channel, the DC media specification shall include the remote MDC2 media specification information in the attribute"remoteMdc2Endpoint" within the data type "Mdc2Info" to be applied on the media by the MF;

3)	if media resource type is set to "VIDEO" or "AUDIO", the video or audio media resource description in the attribute "remoteNonDcMedia" within the data type MediaInfo shall be included; and

may include:

- the AR media resource description in the attribute "ArMedia" within the data type MediaInfo if the media resource type is set to "DC" or "VIDEO" and the media is used for AR communication.

2a.	Upon the reception of the HTTP POST request, if the request is accepted and no error occur, the MF shall:

-	create a new media context;

-	assign a media contextId;

-	assign a terminationId for each termination descriptor; and

-	reserve media resources for each mediaId.

The MF shall include a HTTP Location header to provide the location of a newly created resource (MediaContext) together with the status code 201 indicating the requested resource is created in the response message.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.2.3.1-3.

On redirection, the MF shall include 3xx status code, which shall contain a Location header with an URI pointing to the endpoint to another MF (service) instance.

* * * End of Changes * * * *