3GPP TSG-CT WG4 Meeting #131	C4-254193

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.228 | CR | 0702 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Overload Control over Cx/Dx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_RES-CT |  |  |  |  |  | Date: |  |  | 2025-09-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of the conclusions of TR 29.866, solution#10 "Dynamic extension of registration expiration timers by the Registration Management function" should be specified as normative. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define a new abatement algorithm for the Diameter Overload Control mechanism. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution#10 in TR 29.866 is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex H.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *First Change * * * *

# H.X	Extension of the UE registration timer

IETF RFC 7683 [24] defines a default algorithm for traffic abatement ("OLR_DEFAULT_ALGO"), based on the loss algorithm (i.e. the reporting nodes request a percentage reduction in the amount of traffic sent by the reacting node).

The present specification defines an additional algorithm called "Registration Timer Extension" that can optionally be supported by reporting and reacting nodes.

Its support is advertised via the OC-Feature-Vector AVP, as follows:

-	OLR_REG_TIMER_EXT (0x000000000000XXXX)

Editor's Note:	The actual value needs to be allocated by IANA, and documented in the "AAA parameters" repository, found under the following URL: https://www.iana.org/assignments/aaa-parameters/aaa-parameters.xhtml

The behaviour of the reporting and reacting node shall be as follows:

-	Reporting node: For the protocols defined in the present specification (Cx/Dx), the HSS takes the role of reporting node and, when the overload situation starts, it may send an overload report message by including the OC-OLR AVP in the corresponding diameter message towards the I/S-CSCF, as indicated in clause H.2.

In this abatement algorithm, the OC-OLR AVP is extended from the AVP defined in IETF RFC 7683 [24] as described in 3GPP TS 29.229 [5], clause 6.3.61.

The OC-OLR AVP shall include the OC-Ext-Reg-Timer sub-AVP set to "TRUE" (and, similarly, this sub-AVP shall be set to "FALSE" when the overload situation ends).

-	Reacting node: For the protocols defined in the present specification, the S-CSCF or I-CSCF take the role of reacting node and, when receiving an overload report from HSS, it shall act as described in clause H.3 for the abatement of traffic according to the loss algorithm.

In addition, if it supports the "OLR_REG_TIMER_EXT" algorithm, and it receives the OC-OLR AVP with the OC-Ext-Reg-Timer  sub-AVP set to "TRUE", it shall extend the registration timer of the UEs and use a higher implementation-dependent value; when such sub-AVP is received with value "FALSE", the registration timer is re-set to the default value typically used the the I/S-CSCF in normal, non-overload situations.

* * * End of Changes * * * *