3GPP TSG-CT WG4 Meeting #131	C4-254252

# Sophia Antipolis, France; 13th – 17th October 2025	(was C4-254195)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.229 | CR | 0311 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Overload Control over Cx/Dx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_RES-CT |  |  |  |  |  | Date: |  |  | 2025-09-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of the conclusions of TR 29.866, solution#10 "Dynamic extension of registration expiration timers by the Registration Management function" should be specified as normative. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define a new AVP inside OC-OLR AVP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution#10 in TR 29.866 is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.0, 6.3.60, 6.3.61, 6.3.xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *First Change * * * *

### 6.3.0	General

The following table describes the Diameter AVPs defined by 3GPP for the Cx interface protocol, their AVP Code values, types, possible flag values and whether or not the AVP may be encrypted. The Vendor-ID header of all AVPs defined in this specification shall be set to 3GPP (10415) if not otherwise indicated.

Table 6.3.0.1: Diameter Multimedia Application AVPs

|  |  |  |  | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type | Must | May | Should not | Must not | May Encr. |
| Visited-Network-Identifier | 600 | 6.3.1 | OctetString | M, V |  |  |  | No |
| Public-Identity | 601 | 6.3.2 | UTF8String | M, V |  |  |  | No |
| Server-Name | 602 | 6.3.3 | UTF8String | M, V |  |  |  | No |
| Server-Capabilities | 603 | 6.3.4 | Grouped | M, V |  |  |  | No |
| Mandatory-Capability | 604 | 6.3.5 | Unsigned32 | M, V |  |  |  | No |
| Optional-Capability | 605 | 6.3.6 | Unsigned32 | M, V |  |  |  | No |
| User-Data | 606 | 6.3.7 | OctetString | M, V |  |  |  | No |
| SIP-Number-Auth-Items | 607 | 6.3.8 | Unsigned32 | M, V |  |  |  | No |
| SIP-Authentication-Scheme | 608 | 6.3.9 | UTF8String | M, V |  |  |  | No |
| SIP-Authenticate | 609 | 6.3.10 | OctetString | M, V |  |  |  | No |
| SIP-Authorization | 610 | 6.3.11 | OctetString | M, V |  |  |  | No |
| SIP-Authentication-Context | 611 | 6.3.12 | OctetString | M, V |  |  |  | No |
| SIP-Auth-Data-Item | 612 | 6.3.13 | Grouped | M, V |  |  |  | No |
| SIP-Item-Number | 613 | 6.3.14 | Unsigned32 | M, V |  |  |  | No |
| Server-Assignment-Type | 614 | 6.3.15 | Enumerated | M, V |  |  |  | No |
| Deregistration-Reason | 615 | 6.3.16 | Grouped | M, V |  |  |  | No |
| Reason-Code | 616 | 6.3.17 | Enumerated | M, V |  |  |  | No |
| Reason-Info | 617 | 6.3.18 | UTF8String | M, V |  |  |  | No |
| Charging-Information | 618 | 6.3.19 | Grouped | M, V |  |  |  | No |
| Primary-Event-Charging-Function-Name | 619 | 6.3.20 | DiameterURI | M, V |  |  |  | No |
| Secondary-Event-Charging-Function-Name | 620 | 6.3.21 | DiameterURI | M, V |  |  |  | No |
| Primary-Charging-Collection-Function-Name | 621 | 6.3.22 | DiameterURI | M, V |  |  |  | No |
| Secondary-Charging-Collection-Function-Name | 622 | 6.3.23 | DiameterURI | M, V |  |  |  | No |
| User-Authorization-Type | 623 | 6.3.24 | Enumerated | M, V |  |  |  | No |
| User-Data-Already-Available | 624 | 6.3.26 | Enumerated | M, V |  |  |  | No |
| Confidentiality-Key | 625 | 6.3.27 | OctetString | M, V |  |  |  | No |
| Integrity-Key | 626 | 6.3.28 | OctetString | M, V |  |  |  | No |
| Supported-Features | 628 | 6.3.29 | Grouped | V | M |  |  | No |
| Feature-List-ID | 629 | 6.3.30 | Unsigned32 | V |  |  | M | No |
| Feature-List | 630 | 6.3.31 | Unsigned32 | V |  |  | M | No |
| Supported-Applications | 631 | 6.3.32 | Grouped | V |  |  | M | No |
| Associated-Identities | 632 | 6.3.33 | Grouped | V |  |  | M | No |
| Originating-Request | 633 | 6.3.34 | Enumerated | M,V |  |  |  | No |
| Wildcarded-Public-Identity | 634 | 6.3.35 | UTF8String | V |  |  | M | No |
| SIP-Digest-Authenticate | 635 | 6.3.36 | Grouped | V |  |  | M | No |
| Digest-Realm | 104 NOTE 3 | 6.3.37 | UTF8String | M |  |  | V | No |
| Digest-Algorithm | 111 NOTE 3 | 6.3.39 | UTF8String | M |  |  | V | No |
| Digest-QoP | 110 NOTE 3 | 6.3.40 | UTF8String | M |  |  | V | No |
| Digest-HA1 | 121 NOTE 3 | 6.3.41 | UTF8String | M |  |  | V | No |
| UAR-Flags | 637 | 6.3.44 | Unsigned32 | V |  |  | M | No |
| Loose-Route-Indication | 638 | 6.3.45 | Enumerated | V |  |  | M | No |
| SCSCF-Restoration-Info | 639 | 6.3.46 | Grouped | V |  |  | M | No |
| Path | 640 | 6.3.47 | OctetString | V |  |  | M | No |
| Contact | 641 | 6.3.48 | OctetString | V |  |  | M | No |
| Subscription-Info | 642 | 6.3.49 | Grouped | V |  |  | M | No |
| Call-ID-SIP-Header | 643 | 6.3.49.1 | OctetString | V |  |  | M | No |
| From-SIP-Header | 644 | 6.3.49.2 | OctetString | V |  |  | M | No |
| To-SIP-Header | 645 | 6.3.49.3 | OctetString | V |  |  | M | No |
| Record-Route | 646 | 6.3.49.4 | OctetString | V |  |  | M | No |
| Associated-Registered-Identities | 647 | 6.3.50 | Grouped | V |  |  | M | No |
| Multiple-Registration-Indication | 648 | 6.3.51 | Enumerated | V |  |  | M | No |
| Restoration-Info | 649 | 6.3.52 | Grouped | V |  |  | M | No |
| Session-Priority | 650 | 6.3.56 | Enumerated | V |  |  | M | No |
| Identity-with-Emergency-Registration | 651 | 6.3.57 | Grouped | V |  |  | M | No |
| Priviledged-Sender-Indication | 652 | 6.3.58 | Enumerated | V |  |  | M | No |
| LIA-Flags | 653 | 6.3.59 | Unsigned32 | V |  |  | M | No |
| OC-Supported-Features | 621 NOTE 4 | 6.3.60 | Grouped |  |  |  | M, V | No |
| OC-OLR | 623 NOTE 4 | 6.3.61 | Grouped |  |  |  | M, V | No |
| Initial-CSeq-Sequence-Number | 654 | 6.3.62 | Unsigned32 | V |  |  | M | No |
| SAR-Flags | 655 | 6.3.63 | Unsigned32 | V |  |  | M | No |
| Allowed-WAF-WWSF-Identities | 656 | 6.3.64 | Grouped | V |  |  | M | No |
| WebRTC-Authentication-Function-Name | 657 | 6.3.65 | UTF8String | V |  |  | M | No |
| WebRTC-Web-Server-Function-Name | 658 | 6.3.66 | UTF8String | V |  |  | M | No |
| DRMP | 301 NOTE 5 | 6.3.67 | Enumerated |  |  |  | M, V | No |
| Load | NOTE 6 | 6.3.68 | Grouped |  |  |  | M, V | No |
| RTR-Flags | 659 | 6.3.69 | Unsigned32 | V |  |  | M | No |
| P-CSCF-Subscription-Info | 660 | 6.3.70 | Grouped | V |  |  | M | No |
| Registration-Time-Out | 661 | 6.3.71 | Time | V |  |  | M | No |
| Alternate-Digest-Algorithm | 662 NOTE 7 | 6.3.72 | UTF8String | V |  |  | M | No |
| Alternate-Digest-HA1 | 663 NOTE 7 | 6.3.73 | UTF8String | V |  |  | M | No |
| Failed-PCSCF | 664 | 6.3.74 | Grouped | V |  |  | M | No |
| PCSCF-FQDN | 665 | 6.3.75 | DiameterIdentity | V |  |  | M | No |
| PCSCF-IP-Address | 666 | 6.3.76 | Address | V |  |  | M | No |
| OC-Reg-Timer-Ext | xxx | 6.3.xx | Unsigned32 | V |  |  | M | No |
| NOTE 1: The AVP header bit denoted as 'M', indicates whether support of the AVP is required. The AVP header bit denoted as 'V', indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [28]. NOTE 1a: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. NOTE 2: Depending on the concrete command. NOTE 3: The value of these attributes is defined in IETF RFC 4590 [20]. NOTE 4: The value of these attributes is defined in IETF RFC 7683 [23]. NOTE 5: The value of this attribute is defined in IETF RFC 7944 [26]. NOTE 6: The value of this attribute is defined in IEFT RFC 8583 [27]. NOTE 7: The Alternate-Digest-HA1 AVP is defined in the same way as Digest-HA1 AVP in accordance with IETF RFC 7616 [29]. If the Alternate-Digest-HA1 AVP is present, the Digest-HA1 AVP shall also be present and the Digest-HA1 AVP shall contain the MD5 hash. The algorithm of the Alternate-Digest-HA1 AVP shall be determined by the Alternate-Digest-Algorithm AVP. |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

### 6.3.60	OC-Supported-Features

The OC-Supported-Features AVP is of type Grouped and it is defined in IETF RFC 7683 [23]. This AVP is used to support Diameter overload control mechanism.

The reporting and reacting nodes shall support the OLR_DEFAULT_ALGO abatement algorithm and may optionally support the OLR_REG_TIMER_EXT abatement algorithm, as described in 3GPP TS 29.228 [1], Annex H.X.

* * * Next Change * * * *

### 6.3.61	OC-OLR

The OC-OLR AVP is of type Grouped and it is defined in IETF RFC 7683 [23]. This AVP is used to support Diameter overload control mechanism.

This AVP is extended from the format defined in IETF RFC 7683 [23] by adding a new optional sub-AVP (OC-Reg-Timer-Ext).

AVP format

OC-OLR ::= < AVP header: 623 >

< OC-Sequence-Number >

< OC-Report-Type >

[ OC-Reduction-Percentage ]

[ OC-Validity-Duration ]

[ OC-Reg-Timer-Ext ]

* [ AVP ]

* * * Next Change * * * *

### 6.3.xx	OC-Reg-Timer-Ext

The OC-Reg-Timer-Ext AVP is of type Unsigned32 and is used to convey, by the reporting node, the indication to the reacting node about the convenience to extend the registration timer for the UEs.

The following values are defined:

FALSE (0)

This value indicates that the reacting node shall use a default registration timer.

TRUE (1)

This value indicates that the reacting node shall use an extended registration timer (the actual value is determined by the reacting node via implementation-specific means).

* * * End of Changes * * * *