3GPP TSG-CT WG4 Meeting #131	C4-254251

# Sophia Antipolis, France; 13th – 17th October 2025	(was C4-254191)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.500 | CR | 0479 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HTTP header parameter to extend registration timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_RES-CT |  |  |  |  |  | Date: |  |  | 2025-09-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of the conclusions of TR 29.866, solution#10 "Dynamic extension of registration expiration timers by the Registration Management function" should be specified as normative. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define a new parameter called "Extend-Registration-Timer" in the "3gpp-Sbi-Oci" HTTP header. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution#10 in TR 29.866 is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2.9, 6.4.3.4.x (new), D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.3.2.9	3gpp-Sbi-Oci

The header contains a comma-delimited list of Overload Control Information (OCI). See clause 6.4.3.

The encoding of the header follows the ABNF as defined in IETF RFC 9110 [11].

Sbi-Oci-Header = "3gpp-Sbi-Oci:" OWS 1#( timestamp ";" RWS validityPeriod ";" RWS olcMetric ";" RWS olcScope [ ";" RWS olcExtRegTimer ] ) OWS

timestamp = "Timestamp:" RWS DQUOTE date-time DQUOTE

validityPeriod = "Period-of-Validity:" RWS 1*DIGIT "s"

olcMetric = "Overload-Reduction-Metric:" RWS ( DIGIT / %x31-39 DIGIT / "100" ) "%"

olcScope = nfProducerScope / nfConsumerScope / scpScope / seppScope

nfProducerScope = (   ( "NF-Instance:" RWS nfinst )

/ ( "NF-Set:" RWS nfset )

/ ( "NF-Service-Instance:" RWS nfservinst [ ";" RWS "NF-Inst:" RWS nfinst ] )

/ ( "NF-Service-Set:" RWS nfserviceset )

) [ ";" RWS sNssaiList ";" RWS dnnList ]

nfConsumerScope = ( "NF-Instance:" RWS nfinst [ ";" RWS "Service-Name:" RWS servname ] )

/ ( "NF-Set:" RWS nfset [ ";" RWS "Service-Name:" RWS servname ] )

/ ( "NF-Service-Instance:" RWS nfservinst [ ";" RWS "NF-Inst:" RWS nfinst ] )

/ ( "NF-Service-Set:" RWS nfserviceset )

/ ( "Callback-Uri:" RWS DQUOTE URI DQUOTE *( RWS "&" RWS DQUOTE URI DQUOTE ) )

scpScope =	"SCP-FQDN:" RWS fqdn

seppScope =	"SEPP-FQDN:" RWS fqdn

dnnList = "DNN:" RWS 1*tchar *( RWS "&" RWS 1*tchar )

sNssaiList = "S-NSSAI:" RWS snssai *( RWS "&" RWS snssai )

snssai = 1*tchar

olcExtRegTimer	= "Extend-Registration-Timer:" RWS ert-value

ert-value = "true" / "false"

NOTE 1:	Implementations complying with earlier releases of the specification can require the presence of a leading white space after "3gpp-Sbi-Oci:".

"Timestamp" (Mandatory parameter): The date-time type is specified in IETF RFC 5322 [37] and clause 5.6.7 of IETF RFC 9110 [11]. It indicates the timestamp at which the overload control information was generated.

"Period-of-Validity" (Mandatory parameter): Period of validity is a timer that is measured in seconds. Once the timer expires, the OCI becomes invalid.

"Overload-Reduction-Metric" (Mandatory parameter): Overload-Reduction-Metric up to 3 digits long decimal string and the value range shall be from 0 to 100.

The Overload Control Scope is a mandatory header component, and it shall contain one of the parameters: "NF-Instance", "NF-Set", "NF-Service-Instance" or "NF-Service-Set" (for NF consumers or NF producers), "Callback-URI" (for NF consumers), "SCP-FQDN" (for SCP), or "SEPP-FQDN" (for SEPP).

The Extend-Registration-Timer is an optional header component applicable to the IMS network; if present, it shall contain an indication ("true" or "false") requiring the receiving entity (e.g., a "serving" Network Function handling the access control to the network, such as I-CSCF, or S-CSCF) to extend the registration timers for UEs trying to get access to the IMS network. Other NFs may ignore this header parameter.

See clause 6.4.3.4.5. The nfinst, nfset, nfservinst, nfserviceset and servname parameters are defined in clause 5.2.3.2.8. fqdn shall encode an FQDN. URI is defined in clause 3 of IETF RFC 3986 [14].

When signaling overload control information of an NF service instance, the "NF-Inst" parameter shall be present to identify the NF service instance unambiguously. If the "NF-Inst" parameter is absent, the receiving NF should assume the last known NF instance ID of NF service producer or consumer, if available.

NOTE 2:	Implementations complying with earlier versions of the specification can signal overload control information of an NF service instance without including the NF-Inst parameter.

"DNN" (Optional parameter): Used for S-NSSAI/DNN based overload control by SMF, see clause 6.4.3.4.5.2.2, that refers to one or more specific DNN(s). DNN format is defined in 3GPP TS 23.003 [15].

"S-NSSAI" (Optional parameter): Used for S-NSSAI/DNN based overload control by SMF, see clause 6.4.3.4.5.2.2, that refers to one or more specific S-NSSAI(s).

S-NSSAI format is defined in clause 5.4.4.2 of 3GPP TS 29.571 [13]. It shall be encoded as the object format (i.e. not converted to the string pattern defined in clause 5.4.4.2 of 3GPP TS 29.571 [13]).

EXAMPLE 1:	Overload Control Information for an NF Instance:

3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 75s; Overload-Reduction-Metric: 50%; NF-Instance: 54804518-4191-46b3-955c-ac631f953ed8

EXAMPLE 2:	Overload Control Information for an NF Service Set:

3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 120s; Overload-Reduction-Metric: 50%; NF-Service-Set: setxyz.snnsmf-pdusession.nfi54804518-4191-46b3-955c-ac631f953ed8.5gc.mnc012.mcc345

EXAMPLE 3:	Overload Control Information for an SMF instance related to a particular DNN of an S-NSSAI:

3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 600s; Overload-Reduction-Metric: 50%; NF-Instance: 54804518-4191-46b3-955c-ac631f953ed8; S-NSSAI: %7B%22sst%22%3A%201%2C%20%22sd%22%3A%20%22A08923%22%7D; DNN: internet.mnc012.mcc345.gprs

NOTE 3:	The S-NSSAI parameter corresponds to the JSON encoding: {"sst": 1, "sd": "A08923"} (see clause 5.2.3.1)

EXAMPLE 4:	Overload Control Information for an SMF instance related to a particular DNN shared by two S-NSSAIs:

3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 240s; Overload-Reduction-Metric: 50%; NF-Instance: 54804518-4191-46b3-955c-ac631f953ed8; S-NSSAI: %7B%22sst%22%3A%201%2C%20%22sd%22%3A%20%22A08923%22%7D & %7B%22sst%22%3A%201%2C%20%22sd%22%3A%20%22A08924%22%7D; DNN: internet.mnc012.mcc345.gprs

NOTE 4:	The S- NSSAI parameter corresponds to the JSON encoding: {"sst": 1, "sd": "A08923"} & {"sst": 1, "sd": "A08924"} (see clause 5.2.3.1)

EXAMPLE 5:	Overload Control Information sent by a NF service consumer with a scope set to a Callback-Uri:

3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 120s; Overload-Reduction-Metric: 25%; Callback-Uri: "https://pcf12.operator.com/serviceY"

EXAMPLE 6:	Overload Control Information sent by a NF service consumer with a scope set to a specific NF Instance and service:

3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 120s; Overload-Reduction-Metric: 25%; NF-Instance: 54804518-4191-46b3-955c-ac631f953ed8; Service-Name: nsmf-pdusession

EXAMPLE 7:	Overload Control Information sent by an SCP:

3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 120s; Overload-Reduction-Metric: 25%; SCP-FQDN: scp1.example.com

EXAMPLE 8:	Example with two OCI values, one for an SMF Instance and another one for a specific DNN of an S-NSSAI for the same SMF Instance:

3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 75s; Overload-Reduction-Metric: 50%; NF-Instance: 54804518-4191-46b3-955c-ac631f953ed8
3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 600s; Overload-Reduction-Metric: 40%; NF-Instance: 54804518-4191-46b3-955c-ac631f953ed8; S-NSSAI: %7B%22sst%22%3A%201%2C%20%22sd%22%3A%20%22A08923%22%7D; DNN: internet.mnc012.mcc345.gprs

NOTE 5:	The S-NSSAI parameter corresponds to the JSON encoding: {"sst": 1, "sd": "A08923"} (see clause 5.2.3.1)

EXAMPLE 9:	Overload Control Information sent by an SEPP:

3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 120s; Overload-Reduction-Metric: 25%; SEPP-FQDN: sepp1.example.com

NOTE 6:	Example 8 is formatted as two distinct headers (which improves the readability), but it can also be formatted as a single header with two OCI values separated by a comma.

EXAMPLE 10:	Overload Control Information for an NF Service Instance:

3gpp-Sbi-Oci: Timestamp: "Tue, 04 Feb 2020 08:49:37 GMT"; Period-of-Validity: 75s; Overload-Reduction-Metric: 50%; NF-Service-Instance: xyz; NF-Inst: 54804518-4191-46b3-955c-ac631f953ed8

* * * Next Change * * * *

##### 6.4.3.4.x	Extension of registration timer in OCI

The "Extend-Registration-Timer" parameter is a boolean indication (true / false) applicable to the IMS network that may optionally be included by an NF Service Producer (e.g. HSS) to inform the NF Service Consumer (e.g. I-CSCF or S-CSCF) about the need to abate the traffic by extending the registraton timers of the UEs trying to get access to the IMS network; this situation shall be enforced during the time period indicated in the "Period-of-Validity" parameter (see clause 6.4.3.4.4).

* * * Next Change * * * *

# D.2	ABNF definitions (Filename: "TS29500_CustomHeaders.abnf")

(... text not shown for clarity ...)

; ----------------------------------------

;   3GPP TS 29.500

;

;   Version: 19.5.0 (December 2025)

;

;   (c) 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

; ----------------------------------------

(... text not shown for clarity ...)

;

; Header: 3gpp-Sbi-Oci

;

Sbi-Oci-Header  = "3gpp-Sbi-Oci:" OWS oci-element *( OWS "," OWS oci-element ) OWS

oci-element     = timestamp ";" RWS validityPeriod ";" RWS olcMetric ";" RWS olcScope

[ ";" RWS olcExtRegTimer ]

timestamp       = "Timestamp:" RWS DQUOTE date-time DQUOTE

validityPeriod  = "Period-of-Validity:" RWS 1*DIGIT "s"

olcMetric       = "Overload-Reduction-Metric:" RWS ( "100" / %x31-39 DIGIT / DIGIT ) "%"

olcScope        = nfProducerScope / nfConsumerScope / scpScope / seppScope

nfProducerScope = ( ( "NF-Instance:" RWS nfinst )

/ ( "NF-Set:" RWS nfset )

/ ( "NF-Service-Instance:" RWS nfservinst [ ";" RWS "NF-Inst:" RWS nfinst ] )

/ ( "NF-Service-Set:" RWS nfserviceset )

) [ ";" RWS sNssaiList ";" RWS dnnList ]

nfConsumerScope = ( "NFC-Instance:" RWS nfinst [ ";" RWS "Service-Name:" RWS servname ] )

/ ( "NFC-Set:" RWS nfset [ ";" RWS "Service-Name:" RWS servname ] )

/ ( "NFC-Service-Instance:" RWS nfservinst [";" RWS "NF-Inst:" RWS nfinst ] )

/ ( "NFC-Service-Set:" RWS nfserviceset )

/ ( "Callback-Uri:" RWS DQUOTE URI DQUOTE *( RWS "&" RWS DQUOTE URI DQUOTE ) )

servname        = token

scpScope        = "SCP-FQDN:" RWS fqdn

seppScope       = "SEPP-FQDN:" RWS fqdn

fqdn            = token

dnnList         = "DNN:" RWS 1*tchar *( RWS "&" RWS 1*tchar )

sNssaiList      = "S-NSSAI:" RWS snssai *( RWS "&" RWS snssai )

snssai          = 1*tchar

olcExtRegTimer	= "Extend-Registration-Timer:" RWS ert-value

ert-value       = "true" / "false"

(... text not shown for clarity ...)

* * * End of Changes * * * *