3GPP TSG-RAN WG3 Meeting #130	R3-258607

Dallas, USA 17th – 21st Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.420 | CR | 0053 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Evolution of NR duplex operation for 38.420 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Regarding the description of CLI indication function, the “gNB-to-gNB CLI measurements” is not clear whether it is related to measurement configurations or measurement results. Besides, the SRS Resource Indication is actually transferred between NG-RAN nodes to indicate the need for SRS Resource Configuration information. Hence, the description of CLI indication function should be modified where this function allows the transfer of SRS Resource indication between NG-RAN nodes, not SRS resource configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Futher clarify the “gNB-to-gNB CLI measurements” to “gNB-to-gNB CLI measurement result”, and change the “SRS resource configuration” to “SRS Resource indication” in the description of CLI indication function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of CLI indication function is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 5.2.16	CLI indication function

This function allows the transfer of gNB-to-gNB CLI measurement results, gNB-to-gNB CLI mitigation request and SRS Resource indication between NG-RAN nodes.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>