3GPP TSG-RAN WG3 Meeting #130	R3-258714

Dallas, USA 17th – 21th Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0185 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to E1AP for UE performance metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE, Huawei, Ericsson, Nokia, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR enhances the definitions of measurements associated to UE performance metrics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR: - Renames the Average Packet Loss DL IE to Average Packet Drop DL IE - Updates the semantics description of Average Packet Loss UL IE and Average Packet Drop DL IE with a reference to the corresponding clause in TS 28.558. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.11.2, 9.2.1.23, 9.3.1.149, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-0: Capture the agreement in CR R3-257247; Rev-1: Resubmission to RAN3 #130; Rev-2: Revise the WI code in RAN3#130; |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.2.11	Data Collection Reporting Initiation

#### 8.2.11.1	General

This procedure is used by a gNB-CU-CP to request from a gNB-CU-UP the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.2.11.2	Successful Operation

Figure 8.4.11.2-1: Data Collection Reporting Initiation, successful operation

The gNB-CU-CP initiates the procedure by sending the DATA COLLECTION REQUEST message to gNB-CU-UP to start information reporting and stop information reporting. Upon receipt, the gNB-CU-UP:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to “start”; or

-	shall stop all information reporting and terminate the reporting in case the Registration Request for Data Collection IE is set to “stop”.

If the gNB-CU-UP is capable to provide all of the requested information, it shall initiate the information reporting as requested by the gNB-CU-CP and respond with the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic information. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and one or more of the UE performance metrics are requested, the UE Performance Collection Configuration IE shall be included. The gNB-CU-UP shall take the UE Performance Collection Configuration IE into account for the configuration of UE performance collection and reporting. The gNB-CU-UP shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since the bearer context was successfully established is equal to the value of the Collection Time Duration for UE Performance IE;

-	the established bearer context is suspended or released.

Interaction with other procedures

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST indicates the type of objects gNB-CU-UP shall perform measurements on. The gNB-CU-UP shall include in the DATA COLLECTION UPDATE message:

-	the Average Packet Delay IE, if the first bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP;

-	the Average Packet Drop DL IE, if the second bit, "Average Packet Drop DL" of the Report Characteristics for   Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP;

-	the Average Packet Loss UL IE, if the third bit, "Average Packet Loss UL" of the Report Characteristics for   Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP.

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.23	DATA COLLECTION REQUEST

This message is sent by gNB-CU-CP to gNB-CU-UP to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. First Bit = Average Packet Delay, Second Bit = Average Packet Drop DL, Third Bit = Average Packet Loss UL Other bits are ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.149	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | 9.3.1.150 |  |
| Average Packet Drop DL | O |  | INTEGER(0..1000000, …) | Corresponds to DL PDCP SDU Drop Rate in gNB per DRB per UE as specified in TS 28.558 [37], clause 6.3.1.6.1.1. |
| Average Packet Loss UL | O |  | INTEGER(0..1000000, …) | Corresponds to UL PDCP SDU Loss Rate per DRB per UE as specified in TS 28.558 [37], clause 6.3.1.7.1. |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

*** unchanged parts omitted ***

UEPerformance ::= SEQUENCE {

uE-AveragePacketDelay						AveragePacketDelay		OPTIONAL,

uE-AveragePacketDropDL						INTEGER (0..1000000, ...)		OPTIONAL,

uE-AveragePacketLossUL						INTEGER (0..1000000, ...)		OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEPerformance-ExtIEs} } OPTIONAL,

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>