3GPP TSG-RAN WG5 Meeting #109	R5-256049

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3565 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Carrier Leakage for ATG UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC 6.4J.2.2 Carrier Leakage for ATG UE is missing and need to be added as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC 6.4J.2.2 Carrier Leakage for ATG UE has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4J.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.905 CR 1097 TS 38.522 CR 0673 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The related TP analysis CR is R5-256048(CR 1097). The related applicability CR is R5-256055(CR 0673). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.4J.2.2	Carrier leakage for ATG

Editor’s Note: The following aspects are not yet determined:

- MU/TT are FFS for ATG UEs with more than 2 transmit antenna connectors/ transceiver array boundary (TAB) connectors.

- MU/TT already defined are based on preliminary analysis of single TE vendor. Values will be revisited once more detailed analysis, including analysis from other TE vendors, is available.

6.4J.2.2.1	Test purpose

Same test purpose as in 6.4.2.2.1.

6.4J.2.2.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

Support of 256QAM is optional, only applicable if indicating support.

6.4J.2.2.3	Minimum conformance requirements

The requirements for transmit modulation quality defined in clause 6.4.2.2.3 shall apply for ATG UE at each transmit antenna connector for UE with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UE with antenna array, except for the phase continuity requirements for DMRS bundling in 6.4J.2.5. The requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4J.2.

6.4J.2.2.4	Test description

Same test description as in clause 6.4.2.2.4 with the following exceptions:

-	Pmin is defined in clause 6.3J.1.

-	In steps 2, 4, 6 and 8 where Uplink power control window is used, power control shall be applied to the sum of power of all antenna connectors (for ATG UE with omni-directional antennas) or of all TAB connectors (for ATG UE with antenna array).

-	In steps 3, 5,7 and 9 where carrier leakage is measured, measure the carrier leakage and power for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array).

6.4J.2.2.5	Test requirement

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE, carrier leakage measurement requirement shall be waived. Otherwise, each of the n carrier leakage results, derived in Annex E.3.1, for each antenna connector (for ATG UE with omni-directional antennas) or for each TAB connector (for ATG UE with antenna array), shall not exceed the values in table 6.4J.2.2.5-1. Allocated RBs are not under test.

Table 6.4J.2.2.5-1: Test requirements for Relative Carrier Leakage Power

| Parameters UE output power | Relative limit (dBc) |
| --- | --- |
| 10 + MU to 10 + (MU + Uplink power control window size) dBm | -28 + TT |
| 0 + MU to 0 + (MU + Uplink power control window size) dBm | -25 + TT |
| -30 + MU to -30 + (MU + Uplink power control window size) dBm | -20 + TT |
| Pmin + MU to Pmin + (MU + Uplink power control window size) dBm | -10 + TT |
| NOTE 1: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 2: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RBs containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 3: is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 4: MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW. NOTE 5: Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1. NOTE 6: Test tolerance TT = 0.8 dB. NOTE 7: Pmin is the minimum output power according to Table 6.3J.1.5-3. |  |

<< END OF CHANGES >>