3GPP TSG-RAN5 Meeting #109	R5-255674

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5153 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ATSSS Phase 3 TC 10.6.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC 10.6.1.1 UE-requested MA PDU session establishment/ATSSS/UE establishing PDN connection over untrusted non-3GPP access as a user-plane resource of an MA PDU session to be established is needed according to the WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC 10.6.1.1 UE-requested MA PDU session establishment/ATSSS/UE establishing PDN connection over untrusted non-3GPP access as a user-plane resource of an MA PDU session to be established is needed according to the WP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0631 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

10.6	ATSSS with PDN connection as a user-plane resource over untrusted non-3GPP access

10.6.1	UE-requested MA PDU session establishment

10.6.1.1	UE-requested MA PDU session establishment / ATSSS / UE establishing PDN connection over untrusted non-3GPP access as a user-plane resource of an MA PDU session to be established / Success

10.6.1.1.1	Test Purpose (TP)

(1)

with { UE supporting both N1 mode and S1 mode and is switched on without cellular coverage but with good untrusted non-3GPP coverage connected to EPC }

ensure that {

when { UE is configured to request a new MA PDU session }

then { UE sends a IKE_AUTH request message including an indication about Attach Type indicating Initial Attach and ATSSS_REQUEST Notify payload to the ePDG }

}

10.6.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.193, clause 5.3a.2. Unless otherwise stated these are Rel-18 requirements.

[TS 24.193, clause 5.3a.2]

In order to establish a PDN connection over untrusted non-3GPP access network connected to EPC as a user-plane resource of an MA PDU session to be established, the UE shall initiate the IPsec tunnel establishment procedure using the IKEv2 protocol according to 3GPP TS 24.302 [17].

In the IKE_AUTH request message to the ePDG:

a)	the UE shall provide an indication about Attach Type, which indicates Initial Attach as specified in 3GPP TS 24.302 [17]; and

NOTE 1:	According to 3GPP TS 24.302 [17], a newly generated PDU session ID is included in N1_MODE_CAPABILITY Notify payload.

b)	the UE shall include ATSSS_REQUEST Notify payload as specified in clause 8.2.9.20 of 3GPP TS 24.302 [17] and:

1)	if the UE supports ATSSS Low-Layer functionality with any steering mode (i.e., any steering mode allowed for ATSSS Low-Layer functionality) as specified in clause 5.32.6 of 3GPP TS 23.501 [2], the UE shall set the ATSSS request information field of the ATSSS_REQUEST Notify payload to "ATSSS Low-Layer functionality with any steering mode allowed for ATSSS-LL supported";

2)	if the UE supports MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode as specified in clause 5.32.6 of 3GPP TS 23.501 [2], the UE shall set the ATSSS request information field of ATSSS_REQUEST Notify payload to "MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported";

3)	if the UE supports MPTCP functionality with any steering mode and ATSSS-LL functionality with any steering mode (i.e., any steering mode allowed for ATSSS Low-Layer functionality) as specified in clause 5.32.6 of 3GPP TS 23.501 [2], the UE shall set the ATSSS request information field of ATSSS_REQUEST Notify payload to "MPTCP functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported";

4)	if the UE supports MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode as specified in clause 5.32.6 of 3GPP TS 23.501 [2], the UE shall set the ATSSS request information field of ATSSS_REQUEST Notify payload to "MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported";

5)	if the UE supports MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode (i.e., any steering mode allowed for ATSSS Low-Layer functionality) as specified in clause 5.32.6 of 3GPP TS 23.501 [2], the UE shall set the ATSSS request information field of ATSSS_REQUEST Notify payload to "MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported";

6)	if the UE supports MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode, and ATSSS-LL functionality with only active-standby steering mode as specified in clause 5.32.6 of 3GPP TS 23.501 [2], the UE shall set the ATSSS request information field of ATSSS_REQUEST Notify payload to "MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported"; or

7)	if the UE supports MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode, and ATSSS-LL functionality with any steering mode (i.e., any steering mode allowed for ATSSS Low-Layer functionality) as specified in clause 5.32.6 of 3GPP TS 23.501 [2], the UE shall set the ATSSS request information field of ATSSS_REQUEST Notify payload to "MPTCP functionality with any steering mode, MPQUIC functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported".

Upon receipt of IKE_AUTH response message as specified in 3GPP TS 24.302 [17] containing the ATSSS_RESPONSE Notify payload:

a)	the UE shall consider that the MA PDU session is established based on parameters in the IKE_AUTH response message, as follows:

1)	the PDN type shall be mapped to the PDU session type of the MA PDU session as follows:

i)	if the CFG_REPLY contains the INTERNAL_IP4_ADDRESS attribute, the PDU session type is set to "IPv4";

ii)	if the CFG_REPLY contains the INTERNAL_IP6_SUBNET attribute or INTERNAL_IP6_ADDRESS attribute, the PDU session type is set to "IPv6"; or

iii)	if the CFG_REPLY contains the INTERNAL_IP4_ADDRESS attribute and INTERNAL_IP6_SUBNET attributes, or the INTERNAL_IP4_ADDRESS attribute and INTERNAL_IP6_ADDRESS attribute, the PDU session type is set to "IPv4v6";

NOTE 2:	PDN connections with PDN type "Ethernet" or PDN type "non-IP" are not supported over ePDG.

2)	the PDN address shall be mapped to PDU address of the MA PDU session;

3)	the APN of the PDN connection shall be mapped to the DNN of the MA PDU session;

4)	the PDU session identity of the MA PDU session shall be set to the PDU session identity included by the UE in the PDU Session ID field of the N1_MODE_CAPABILITY Notify payload in the IKE_AUTH request message;

5)	the S-NSSAI of the MA PDU session shall be set to the S-NSSAI included by the network in the S-NSSAI Value field of the N1_MODE INFORMATION Notify payload in the IKE_AUTH response message, if the PDN connection is a non-emergency PDN connection;

6)	the SSC mode of the MA PDU session shall be set to "SSC mode 1";

7)	state of the PDU session shall be set to PDU SESSION ACTIVE; and

8)	the Private Notify Message Status Types, if received, shall be mapped to the 5GSM cause of the MA PDU session as follows:

i)	if a PDN_TYPE_IPv4_ONLY_ALLOWED Notify payload is received, the 5GSM cause of the MA PDU session is set to #50 "PDU session type IPv4 only allowed"; or

ii)	if a PDN_TYPE_IPv6_ONLY_ALLOWED Notify payload is received, the 5GSM cause of the MA PDU session is set to #51 "PDU session type IPv6 only allowed";

and that the PDN connection over untrusted non-3GPP access network is established as a user-plane resource of the MA PDU session;

b)	if the network steering functionalities information is included in the ATSSS response information field of the ATSSS_RESPONSE Notify payload, the UE shall use the network steering functionalities information;

c)	if the measurement assistance information is included in ATSSS response information field of the ATSSS_RESPONSE Notify payload, the UE shall use the measurement assistance information; and

d)	if the ATSSS rules are included in ATSSS response information field of the ATSSS_RESPONSE Notify payload, the UE shall use the ATSSS rules.

If the UE receives:

a)	a IKE_AUTH response message including a Notify payload with a Private Notify Message Type as specified in clause 7.2.2.2 of 3GPP TS 24.302 [17]; or

b)	no IKE_AUTH response message including the ATSSS_RESPONSE Notify payload;

the UE shall consider that the MA PDU session is not established and the PDN connection over untrusted non-3GPP access network is not established as a user-plane resource of the MA PDU session.

10.6.1.1.3	Test description

10.6.1.1.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 is configured according to 38.508-1 [4], clause 4.4.1.3.1.

UE:

-	None.

Preamble:

-	The UE is in state 0W-B as defined in TS 38.508-1 [4], subclause 4. 4A.2.

10.6.1.1.3.2	Test procedure sequence

Table 10.6.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | UE associates with the WLAN AP and obtains the local IP address. |  |  |  |  |
| 3 | Cause the UE to request to establish an MA PDU session.(NOTE 1) |  |  |  |  |
| 4 | UE transmits a DNS Query message. | --> | DNS Query |  |  |
| 5 | SS transmits a DNS Response message with the IP address of the ePDG | <-- | DNS Response |  |  |
| 6 | UE transmits an IKE_SA_INIT message to the ePDG | --> | IKE_SA_INIT Request |  |  |
| 7 | SS transmits an IKE_SA_INIT message | <-- | IKE_SA_INIT Response |  |  |
| 8 | Check: Does the UE transmit an IKE_AUTH request message including an indication about Attach Type indicating Initial Attach and ATSSS_REQUEST Notify payload. | --> | IKE_AUTH Request | 1 | P |
| 9 | SS transmits an IKE_AUTH Response message containing the ATSSS_RESPONSE Notify payload. | <-- | IKE_AUTH Response | - | - |
| NOTE 1: The request to establish a MA PDU session may be performed by MMI or AT Command. |  |  |  |  |  |

10.6.1.1.3.3	Specific message contents

Table 10.6.1.1.3.3-1: IKE_AUTH REQUEST (step 8, Table 10.6.1.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.7G-3 |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| IKE Header |  |  |  |
| Next Payload | ‘00101111’B | CP |  |
| Exchange Type | ‘00100011’B | IKE_AUTH |  |
| Configuration Payload |  |  |  |
| Next Payload | ‘00101001’B | Notify |  |
| CFG Type | ‘00000001’B | Request |  |
| Configuration Attribute |  |  |  |
| Attribute Type | ‘00000001’B | INTERNAL_IP4_ADDRESS attribute, no value, length field set to 0, used to indicate Initial Attach | IPv4 |
| Length | ‘0000000000000000’B |  |  |
| Attribute Type | ‘00001000’B | INTERNAL_IP6_ADDRESS attribute, no value, length field set to 0, used to indicate Initial Attach | IPv6 |
| Length | ‘0000000000000000’B |  |  |
| Notify Payload |  |  |  |
| Next Payload | ‘00101001’B | Notify |  |
| Notify Message Type | ‘1100011101000111’B | N1_MODE_CAPABILITY |  |
| Length | Not check |  |  |
| PDU Session ID | A newly generated PDU session ID |  |  |
| Notify Payload |  |  |  |
| Next Payload | ‘00000000’B | No Next Payload |  |
| Notify Message Type | ‘1100110001101011’B | ATSSS_REQUEST |  |
| Length | Not check |  |  |
| ATSSS request information | Any allowed value according to TS 24.193 [45] subclause 6.1.6.2 that UE supports |  |  |
| Padding | Set by the UE | Fields from Encrypted payload |  |
| Pad Length | Set by the UE | Fields from Encrypted payload |  |
| Integrity checksum data | Set by the UE | Fields from Encrypted payload |  |

==============End of change==============