3GPP TSG-RAN5 Meeting #109	R5-255822

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5168 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ATSSS Phase 3 TC 11.9.5 to 11.9.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS 38.508-1 Table 4.4A.2-3, UE state 3W-A is for WLAN connected to 5GC, this is not the case of MA PDU session with WLAN connected to EPC in the ATSSS Phase 3 test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE state 3W-A was replaced by WLAN AP association in Preamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC configuration will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.9.5.3.1, 11.9.6.3.1, 11.9.7.3.1 and 11.9.8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

11.9.5.3	Test description

11.9.5.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 is configured according to 38.508-1 [4], clause 4.4.1.3.1.

-	NR Cell 1 is configured according to TS 38.508-1 [4].

-	E-UTRA Cell 1 is configured to TS 36.508 [7].

-	NR Cell 1, WLAN Cell 27 and E-UTRA Cell 1 belong to the same PLMN and both NR Cell 1 and WLAN Cell 27 are set to '' Serving cell'', E-UTRA Cell 1 is set to "non-suitable cell".

UE:

-	UE is registered over both 3GPP access to 5GC and non-3GPP access to EPC in the same PLMN.

Preamble:

-	UE is brought to state 3N-A with UE test loop mode B active according to TS 38.508-1 [4].

-	UE has associated with the WLAN Cell and obtained the local IP address.

==============Next change==============

11.9.6.3	Test description

11.9.6.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 is configured according to 38.508-1 [4], clause 4.4.1.3.1.

-	NR Cell 1 is configured according to TS 38.508-1 [4].

-	E-UTRA Cell 1 is configured to TS 36.508 [7].

-	NR Cell 1, WLAN Cell 27 and E-UTRA Cell 1 belong to the same PLMN and both NR Cell 1 and WLAN Cell 27 are set to '' Serving cell'', E-UTRA Cell 1 is set to "non-suitable cell".

UE:

-	UE is registered over both 3GPP access to 5GC and non-3GPP access to EPC in the same PLMN.

Preamble:

-	UE is brought to state 3N-A with PDU SESSION INACTIVE according to TS 38.508-1 [4].

-	UE has associated with the WLAN Cell and obtained the local IP address.

==============Next change==============

11.9.7.3	Test description

11.9.7.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 is configured according to 38.508-1 [4], clause 4.4.1.3.1.

-	NR Cell 1 is configured according to TS 38.508-1 [4].

-	E-UTRA Cell 1 is configured to TS 36.508 [7].

-	NR Cell 1, WLAN Cell 27 and E-UTRA Cell 1 belong to the same PLMN and both NR Cell 1 and WLAN Cell 27 are set to '' Serving cell'', E-UTRA Cell 1 is set to "non-suitable cell".

UE:

-	UE is registered over both 3GPP access to 5GC and non-3GPP access to EPC in the same PLMN.

Preamble:

-	UE is brought to state 3N-A with PDU SESSION INACTIVE according to TS 38.508-1 [4].

-	UE has associated with the WLAN Cell and obtained the local IP address.

==============Next change==============

11.9.8.3	Test description

11.9.8.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 is configured according to 38.508-1 [4], clause 4.4.1.3.1.

-	NR Cell 1 is configured according to TS 38.508-1 [4].

-	E-UTRA Cell 1 is configured to TS 36.508 [7].

-	NR Cell 1, WLAN Cell 27 and E-UTRA Cell 1 belong to the same PLMN and both NR Cell 1 and WLAN Cell 27 are set to '' Serving cell'', E-UTRA Cell 1 is set to "non-suitable cell".

UE:

-	UE is registered over both 3GPP access to 5GC and non-3GPP access to EPC in the same PLMN.

Preamble:

-	UE is brought to state 3N-A with PDU SESSION INACTIVE according to TS 38.508-1 [4].

-	UE has associated with the WLAN Cell and obtained the local IP address.

==============End of change==============