3GPP TSG-RAN5 Meeting #109	R5-256993

# Dallas, US, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0121 | rev | 1 | Current version: | 18.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New additional spectrum emission mask test case, Transmit power density and other updates for band n254 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, NR_lic_bands_BW_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Minimum conformance requirements have been clarified for Transmit power density. - Test procedure for Transmit power density test case is incomplete. - Test requirements for Transmit power density test case require alignment with updated minimum conformance requirements. - Missing test case for Additional Spectrum Emission Mask applicable to band n254. - Minimum conformance requirements for band n254 have been clarified for Additional spurious emissions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minimum conformance requirements for Transmit power density updated and associated editor’s note removed. - Test procedure for Transmit power density test case has been completed. - Transmit power density test requriements updated - New test case for Additional Spectrum Emission Mask applicable to band n254 added. - Minimum conformance requirements for band n254 for Additional spurious emissions updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For band n254, Transmit power density test case will remain incomplete, Additional Spectrum Emission Mask test will be missing and Additional spurious emissions will remain ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.1_1, 6.5.2.3, 6.5.3.3.3.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on RAN4 CR R4-2522386. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-256504 and the outcome of 2 revisions |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

### 6.2.1	UE maximum output power

#### 6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

#### 6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

#### 6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1 ms).

Table 6.2.1.3-1: UE Power Class

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | ±2 |
| n254 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

The UE shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.2.1-2 when NS is signaled and when the configured channel overlaps with any portion of the specified frequency range.

Table 6.2.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) |  | Maximum power density |  | Measurement method |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n254 | NS_04N | 5 | 1610 - 1618.25 |  | 27dBm/4kHz |  | Mean spectral limit (note 2) |
|  | NS_05N | 5 | 1618.25 - 1626.5 |  |  |  |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |  |  |  |
|  | NS_11N | 5 | 1610 - 1618.25 |  | 15dBm/4kHz |  | Peak spectral limit (note 2) |
|  | NS_12N | 5 | 1618.25 - 1626.5 |  |  |  |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |  |  |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. NOTE 2: Indicated limit is applied to any frequency of the 1610-1626.5MHz band where a UE can operate. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-5 [11] clause 6.2.1.

#### 6.2.1.4	Test description

##### 6.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table table 6.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.2.1.4.3.

##### 6.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1 ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

##### 6.2.1.4.3	Message contents

6.2.1.4.3.0	General

Message contents are according to TS 38.508-1 [12] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2.1.4.3.0-1: PUSCH-Config

Derivation Path: TS 38.508-1 [12], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.2.1.4.3.1	Void

6.2.1.4.3.2	Void

#### 6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±(2+TT) |
| n255 | 23 | ±(2+TT) |
| n254 | 23 | ±(2+TT) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-2 |  |  |

Table 6.2.1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz |
| --- | --- |
| BW ≤ 30MHz | 0.7 dB |

### 6.2.1_1	UE maximum output power: maximum power density

Editor's Note: This test case is incomplete. The following aspects are either missing or not yet determined:

- MU Analysis, test tolerances and any associated Annex F updates for new power density metric is pending.

#### 6.2.1_1.1	Test purpose

Same as in 6.2.1.1.

#### 6.2.1_1.2	Test applicability

Same as in 6.2.1.2.

#### 6.2.1_1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2.1.3.

#### 6.2.1_1.4	Test description

##### 6.2.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table table 6.2.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1_1.4.1-1: Test Configuration Table for transmit power density measurements

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Low range, Mid range for NS_04N and NS_11N Mid range, High range for NS_05N and NS_12N |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Left |
| 2 |  | DFT-s-OFDM Pi/2 BPSK | Inner 1RB Right |
| 3 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 4 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.2.1_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508-1 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1.

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.2.1_1.4.3.

##### 6.2.1_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1_1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in each 4 kHz interval within the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1 ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

4.	Derive mean or peak values of all 4 kHz intervals within the channel bandwidth (depending on the network signaled value) of the radio access mode. These values will correspond to transmit EIRP mean or peak density values respectively.

5.	Repeat steps 1, 2, 3 and 4 for all applicable network signalled values.

##### 6.2.1_1.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 and 5.4 with additional exceptions described in 6.2.1.4.3 and the ones described in 6.2.3.4.3 associated to each applicable network signalled value used in maximum power density measurements.

##### 6.2.1_1.5	Test requirement

Mean and peak transmit power density values derived in step 4 shall be within the range prescribed by the additional requirements for transmit power density in Table 6.2.1_1.5-1 and tolerances in Table 6.2.1_1.5-2.

Table 6.2.1_1.5-1: Transmit power density test requirement

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) |  | Maximum power density |  | Measurement method |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n254 | NS_04N | 5 | 1610 - 1618.25 |  | 27dBm/4kHz + TT |  | Mean spectral limit (note 2) |
|  | NS_05N | 5 | 1618.25 - 1626.5 |  |  |  |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |  |  |  |
|  | NS_11N | 5 | 1610 - 1618.25 |  | 15dBm/4kHz +TT |  | Peak spectral limit (note 2) |
|  | NS_12N | 5 | 1618.25 - 1626.5 |  |  |  |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |  |  |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. NOTE 2: Indicated limit is applied to any frequency of the 1610-1626.5MHz band where a UE can operate. |  |  |  |  |  |  |  |

Table 6.2.1_1.5-2: Test tolerance (tansmit power density test requirement)

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Test Tolerance |
| --- | --- | --- | --- | --- |
| n254 | NS_04N | 5 | 1610 - 1618.25 | TBD |
|  | NS_05N | 5 | 1618.25 - 1626.5 |  |
|  |  | 10, 15 | 1610 – 1626.5 |  |
|  | NS_11N | 5 | 1610 - 1618.25 | TBD |
|  | NS_12N | 10, 15 | 1610 – 1626.5 |  |

<<< Skip Unchanged Sections >>>

#### 6.5.2.3	Additional spectrum emission mask

Editor's Note: This test case is incomplete. The following aspects are either missing or not yet determined:

- MU Analysis, test tolerances and any associated Annex F updates pending.

- Addition to applicability spec is pending

##### 6.5.2.3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

##### 6.5.2.3.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

##### 6.5.2.3.3	Minimum conformance requirements

##### 6.5.2.3.3.1	Requirements for network signalling value "NS_04N" and “NS_11N”

When "NS_04N" or “NS_11N”is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3.1-1 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5.2.3.3.1-1: Additional requirements for "NS_04N" and “NS_11N”

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| 0-160 | -2 | 30kHz |  |
| 160-2300 | -2 to -26 |  | Average |
| 2300-18500 | -26 |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

##### 6.5.2.3.3.2	Requirements for network signalling value “NS_05N" and “NS_12N”

When "NS_05N" or “NS_12N” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3.2-1 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5.2.3.3.2-1: Additional requirements for "NS_05N" and “NS_12N”

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| 0-160 | -5 | 30kHz |  |
| 160-225 | -5 to -8.5 |  |  |
| 225-650 | -8.5 to -15 |  | Average |
| 650-1365 | -15 |  |  |
| 1365-1800 | -23 to -26 |  |  |
| 1800-16500 | -26 |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

The normative reference for this requirement is TS 38.101-5 [11] clause 6.5.2.3.

##### 6.5.2.3.4	Test description

6.5.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.3.4.1-5 and Table 6.2.3.4.1-6 depending on the network signalling value under test. The details of the uplink reference measurement channels (RMCs) are specified in TS 38.521-1 [2] Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [2] Annex C.2.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.2.3.4.1-5 and Table 6.2.3.4.1-6 depending on the network signalling value under test..

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	 Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508 [12] Table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and Table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in TS 38.508-1 [12] clause 5.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means.

9.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.5.2.3.4.3.

6.5.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.3.4.1-5 and Table 6.2.3.4.1-6 depending on the network signalling value under test.. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control "up" commands to the UE until the UE transmits at PUMAX level. Allow at least 200 ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.2.5-1. The period of the measurement shall be at least the continuous duration of 1 ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the average power of the transmitted signal with a measurement filter of bandwidths according to table 6.5.2.3.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step and ΔfOOB range. The measurement period shall capture the active TSs.

5. Repeat steps 1, 2, 3 and 4 for all applicable network signalled values.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.3.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [12] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 and 5.4 with additional exceptions described in 6.5.2.2.4.3 and the ones described in 6.2.3.4.3 associated to each applicable network signalled value used in additional spectrum emission mask measurements.

##### 6.5.2.3.5	Test requirement

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.5.2.3.5.1	Test Requirements for network signalling value "NS_04N" and “NS_11N”

When "NS_04N" or “NS_11N” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.5.1-1 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5.2.3.5.1-1: Additional spectrum emission mask Test Requirements for "NS_04N" and “NS_11N”

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| 0-160 | -2 + TT | 30kHz |  |
| 160-2300 | -2 to -26 + TT |  | Average |
| 2300-18500 | -26 + TT |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 6.5.2.3.5.1-2: Additional spectrum emission mask Test Tolerances for "NS_04N" and “NS_11N”

| ΔfOOB (kHz) | Test Tolerance (dB) |
| --- | --- |
| 0-160 | FFS |
| 160-2300 | FFS |
| 2300-18500 | FFS |

##### 6.5.2.3.5.2	Requirements for network signalling value “NS_05N" and “NS_12N”

When "NS_05N" or “NS_12N” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.5.2-1 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5.2.3.5.2-1: Additional spectrum emission mask Test Requirements for "NS_05N" and “NS_12N”

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
| 0-160 | -5 | 30kHz |  |
| 160-225 | -5 to -8.5 |  |  |
| 225-650 | -8.5 to -15 |  | Average |
| 650-1365 | -15 |  |  |
| 1365-1800 | -23 to -26 |  |  |
| 1800-16500 | -26 |  |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 6.5.2.3.5.1-2: Additional spectrum emission mask Test Tolerances for "NS_05N" and “NS_12N”

| ΔfOOB (kHz) | Test Tolerance (dB) |
| --- | --- |
| 0-160 | FFS |
| 160-225 | FFS |
| 225-650 | FFS |
| 650-1365 | FFS |
| 1365-1800 | FFS |
| 1800-16500 | FFS |

<<< Skip Unchanged Sections >>>

##### 6.5.3.3.3.3	Minimum conformance requirements (network signalling value "NS_04N", "NS_05N", “NS_11N” and “NS_12N”)

When "NS_04N" or "NS_05N" or “NS_11N” or “NS_12N” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3.3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3.3-1: Additional out-of-band requirements for "NS_04N", "NS_05N", “NS_11N” and “NS_12N”

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | Measurement method |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |  |
| 1559 ≤ f < 1605 | -40 | 1MHz | Average (note 2) l |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f < 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f < 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f < 1646.5 | -30 | 300kHz | Average |
| 1646.5 ≤ f < 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 2: In the sub-band 1573.42MHz to 1580.42 MHz the average measurement time is 20ms. |  |  |  |

<<< END OF CHANGES >>>