3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250456

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3764 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC testcase 8.2.3.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN-3 implementation of test case 8.2.3.3.1 , measurement report is not properly checked for the presence of IE measResultNeighCellListNR-r15 at step 5 and 8. This implementation is not in line with Table 8.2.3.3.1.3.3-6: MeasurementReport (step 5, Table 8.2.3.3.1.3.2-2) and Table 8.2.3.3.1.3.3-7: MeasurementReport (step 8, Table 8.2.3.3.1.3.2-2). Also, this missing implementation can cause 8.2.3.3.1.ENDC to incorrectly pass when it should fail |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test case 8.2.3.3.1 implementation at step 5 and step 8 to properly check for the presence of measResultNeighCellListNR-r15 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case can incorrectly pass when it should fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.3.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_8_2_3_3_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | In the current TTCN-3 implementation of test case 8.2.3.3.1 , measurement report is not properly checked for the presence of IE measResultNeighCellListNR-r15 at step 5 and 8. This implementation is not in line with Table 8.2.3.3.1.3.3-6: MeasurementReport (step 5, Table 8.2.3.3.1.3.2-2) and Table 8.2.3.3.1.3.3-7: MeasurementReport (step 8, Table 8.2.3.3.1.3.2-2). Also, this missing implementation can cause 8.2.3.3.1.ENDC to incorrectly pass when it should fail |
| Summary of change | Updated test case 8.2.3.3.1 implementation at step 5 and step 8 to properly check for the presence of measResultNeighCellListNR-r15 |
| TTCN module | RRCMeasurement_ENDC_EUTRA.ttcn |

Before Change:

function fl_TC_8_2_3_3_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var EUTRA_NR_CoOrd_SysInfo_Type v_EUTRA_NR_SysInfo; var integer v_NR_PhysChId_Cell1; var integer v_NR_ARFCN_f1; var integer v_NR_PhysChId_Cell2; var RS_ConfigSSB_NR_r15.subcarrierSpacingSSB_r15 v_SCS; var FreqBandIndicatorNR_r15 v_FreqBandIndicatorNR_r15; //@sic R5-194875 sic@ var ReportConfigInterRAT.reportAmount v_ReportAmount := infinity_; var FilterCoefficient v_FilterCoefficientRSRP := fc0; var FilterCoefficient v_FilterCoefficientRSRQ := fc0; var FilterCoefficient v_FilterCoefficientSINR := fc0; //@sic R5s190103 sic@ var float v_TimerValue_15s; var float v_TimerValue_10s; var float v_TimerValue_57s; //@sic R5s200411 sic@ var default v_DefaultRef; v_TimerValue_15s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 15.0); v_TimerValue_10s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 10.0); v_TimerValue_57s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 57.0); //@sic R5s200411 sic@ f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1); //Acquire information of NR cell 1 and 2 v_EUTRA_NR_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); v_NR_PhysChId_Cell1 := v_EUTRA_NR_SysInfo.NR[0].PhysCellId; v_NR_PhysChId_Cell2 := v_EUTRA_NR_SysInfo.NR[1].PhysCellId; v_NR_ARFCN_f1 := v_EUTRA_NR_SysInfo.NR[0].Arfcn; v_SCS := f_NR_ConvertIntToSCS(v_EUTRA_NR_SysInfo.NR[0].SCS); v_FreqBandIndicatorNR_r15 := v_EUTRA_NR_SysInfo.NR[0].Band; //@sic R5-194875 sic@ //Acquire information of NR cell 2 // v_SCS := f_NR_ConvertIntToSCS (v_EUTRA_NR_SysInfo.NR[1].SCS); //to be checked (Prose should be changed accordingly) //@siclog "Step 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including measConfig to setup inter RAT measurement and //reporting for NR Cell 1 and 2 and receives an RRCConnectionReconfigurationComplete message to confirm the setup of //inter RAT measurements for NR Cell 1 and 2 f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_NR_Periodical_1Entry(v_RRC_TI, v_NR_ARFCN_f1, v_SCS, v_FreqBandIndicatorNR_r15, //@sic R5-194875 sic@ v_ReportAmount, v_FilterCoefficientRSRP, v_FilterCoefficientRSRQ, v_FilterCoefficientSINR, cs_ReportQuantityNR_r15(true, true, pc_ss_SINR_Meas))); //@sic R5-194850, R5s190103, 5-190621 sic@ //@siclog "Step 3" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T1" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 4" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency else wait for 15 s //to allow change of power levels and UE measurements for NR Cell 1 if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s201023, R5-201632 sic@ //@siclog "Step 5" siclog@ //Check: Does the UE transmit a MeasurementReport message to perform periodical reporting for NR Cell 1? if(pc_ss_SINR_Meas) { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell1, ?, ?, omit, omit))); //@sic R5s190103 sic@ } else { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell1, ?, omit, omit, omit))); }; //@sic R5-194850 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 5"); //Activate the default to receive measurement reports for NR Cell 1 v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //@siclog "Step 6" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T2" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 and NR Cell2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 7" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency //else wait for 15 s to allow change of power levels and UE measurements for NR Cell 2 if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s200411 sic@ deactivate(v_DefaultRef); //@sic R5s190103 sic@ //@siclog "Step 8" siclog@ //Check: Does the UE transmit a MeasurementReport message to perform periodical reporting for NR Cell 2? if(pc_ss_SINR_Meas) { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell2, ?, ?, omit, omit))); //@sic R5s190103 sic@ } else { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell2, ?, omit, omit, omit))); }; //@sic R5-194850 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 8"); //@siclog "Step 9" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T3" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //@sic R5s190103 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 10" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency else wait for 15 s //to allow change of power levels and UE measurements for NR Cell 1 and NR Cell 2. if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s200411 sic@ //@siclog "Step 11-12" siclog@ //The SS transmits an RRCConnectionReconfiguration message to remove inter-RAT measurement and reporting for NR Cell and //receives an RRCConnectionReconfigurationComplete message to confirm the remove of inter-RAT measurement for NR Cell. f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_NR_RemoveMeasId_1Entry(v_RRC_TI, 1)); deactivate(v_DefaultRef); //@siclog "Step 13" siclog@ //Check: Does the UE transmit a MeasurementReport message on EUTRA Cell1 to report periodical reporting for NR Cell 1 and 2 during the next 10s? f_EUTRA_NoMeasurementReport(eutra_Cell1, v_TimerValue_10s, cr_MeasurementReport_CheckAny); }

After Change:

function fl_TC_8_2_3_3_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var EUTRA_NR_CoOrd_SysInfo_Type v_EUTRA_NR_SysInfo; var integer v_NR_PhysChId_Cell1; var integer v_NR_ARFCN_f1; var integer v_NR_PhysChId_Cell2; var RS_ConfigSSB_NR_r15.subcarrierSpacingSSB_r15 v_SCS; var FreqBandIndicatorNR_r15 v_FreqBandIndicatorNR_r15; //@sic R5-194875 sic@ var ReportConfigInterRAT.reportAmount v_ReportAmount := infinity_; var FilterCoefficient v_FilterCoefficientRSRP := fc0; var FilterCoefficient v_FilterCoefficientRSRQ := fc0; var FilterCoefficient v_FilterCoefficientSINR := fc0; //@sic R5s190103 sic@ var float v_TimerValue_15s; var float v_TimerValue_10s; var float v_TimerValue_57s; //@sic R5s200411 sic@ var default v_DefaultRef; var SRB_COMMON_IND v_SRB_IND_measurement; //WA#WI=1461995 v_TimerValue_15s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 15.0); v_TimerValue_10s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 10.0); v_TimerValue_57s := f_EUTRA_SetTimerToleranceMax(eutra_Cell1, nonProtocolTimer, 57.0); //@sic R5s200411 sic@ f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1); //Acquire information of NR cell 1 and 2 v_EUTRA_NR_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(NR); v_NR_PhysChId_Cell1 := v_EUTRA_NR_SysInfo.NR[0].PhysCellId; v_NR_PhysChId_Cell2 := v_EUTRA_NR_SysInfo.NR[1].PhysCellId; v_NR_ARFCN_f1 := v_EUTRA_NR_SysInfo.NR[0].Arfcn; v_SCS := f_NR_ConvertIntToSCS(v_EUTRA_NR_SysInfo.NR[0].SCS); v_FreqBandIndicatorNR_r15 := v_EUTRA_NR_SysInfo.NR[0].Band; //@sic R5-194875 sic@ //Acquire information of NR cell 2 // v_SCS := f_NR_ConvertIntToSCS (v_EUTRA_NR_SysInfo.NR[1].SCS); //to be checked (Prose should be changed accordingly) //@siclog "Step 1-2" siclog@ //The SS transmits an RRCConnectionReconfiguration message including measConfig to setup inter RAT measurement and //reporting for NR Cell 1 and 2 and receives an RRCConnectionReconfigurationComplete message to confirm the setup of //inter RAT measurements for NR Cell 1 and 2 f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_NR_Periodical_1Entry(v_RRC_TI, v_NR_ARFCN_f1, v_SCS, v_FreqBandIndicatorNR_r15, //@sic R5-194875 sic@ v_ReportAmount, v_FilterCoefficientRSRP, v_FilterCoefficientRSRQ, v_FilterCoefficientSINR, cs_ReportQuantityNR_r15(true, true, pc_ss_SINR_Meas))); //@sic R5-194850, R5s190103, 5-190621 sic@ //@siclog "Step 3" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T1" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 4" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency else wait for 15 s //to allow change of power levels and UE measurements for NR Cell 1 if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s201023, R5-201632 sic@ //@siclog "Step 5" siclog@ //Check: Does the UE transmit a MeasurementReport message to perform periodical reporting for NR Cell 1? if(pc_ss_SINR_Meas) { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell1, ?, ?, omit, omit))) -> value v_SRB_IND_measurement; //@sic R5s190103 sic@ //WA#WI=1461995 } else { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell1, ?, omit, omit, omit))) -> value v_SRB_IND_measurement; //WA#WI=1461995 }; //@sic R5-194850 sic@ // Validate that measResultNeighCells is present for test case 8.2.3.3.1 if (not ispresent(v_SRB_IND_measurement.Signalling.Rrc.Dcch.message_.c1.measurementReport.criticalExtensions.c1.measurementReport_r8.measResults.measResultNeighCells)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 5: measResultNeighCells is missing from MeasurementReport for NR Cell 1 - UE compliance issue"); } //WA#WI=1461995 f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 5"); //Activate the default to receive measurement reports for NR Cell 1 v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //@siclog "Step 6" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T2" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 and NR Cell2 f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 7" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency //else wait for 15 s to allow change of power levels and UE measurements for NR Cell 2 if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s200411 sic@ deactivate(v_DefaultRef); //@sic R5s190103 sic@ //@siclog "Step 8" siclog@ //Check: Does the UE transmit a MeasurementReport message to perform periodical reporting for NR Cell 2? if(pc_ss_SINR_Meas) { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell2, ?, ?, omit, omit))) -> value v_SRB_IND_measurement; //@sic R5s190103 sic@ //WA#WI=1461995 } else { SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell1, cr_MeasurementReport_NR_1Entry(1, v_NR_PhysChId_Cell2, ?, omit, omit, omit))) -> value v_SRB_IND_measurement; //WA#WI=1461995 }; //@sic R5-194850 sic@ // Validate that measResultNeighCells is present for test case 8.2.3.3.1 if (not ispresent(v_SRB_IND_measurement.Signalling.Rrc.Dcch.message_.c1.measurementReport.criticalExtensions.c1.measurementReport_r8.measResults.measResultNeighCells)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 8: measResultNeighCells is missing from MeasurementReport for NR Cell 2 - UE compliance issue"); } //WA#WI=1461995 f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.3.3.1 Step 8"); //@siclog "Step 9" siclog@ //The SS changes NR Cell 1 and 2 parameters according to the row "T3" in table 8.2.3.3.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(eutra_Cell1, cr_MeasurementReport_CheckAny)); //@sic R5s190103 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Step 10" siclog@ //Wait and ignore MeasurementReport messages for 57s if NR Cell 1 is on FR2 frequency else wait for 15 s //to allow change of power levels and UE measurements for NR Cell 1 and NR Cell 2. if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_15s, cr_MeasurementReport_CheckAny); } else { f_EUTRA_WaitAndIgnoreMeasurementReport(eutra_Cell1, v_TimerValue_57s, cr_MeasurementReport_CheckAny); } //@sic R5s201420, R5s200411 sic@ //@siclog "Step 11-12" siclog@ //The SS transmits an RRCConnectionReconfiguration message to remove inter-RAT measurement and reporting for NR Cell and //receives an RRCConnectionReconfigurationComplete message to confirm the remove of inter-RAT measurement for NR Cell. f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_NR_RemoveMeasId_1Entry(v_RRC_TI, 1)); deactivate(v_DefaultRef); //@siclog "Step 13" siclog@ //Check: Does the UE transmit a MeasurementReport message on EUTRA Cell1 to report periodical reporting for NR Cell 1 and 2 during the next 10s? f_EUTRA_NoMeasurementReport(eutra_Cell1, v_TimerValue_10s, cr_MeasurementReport_CheckAny); }