3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250463

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3765 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PEI testcase 8.1.1.1a.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Inline with agreed Tdocs R5-204415 and R5-241495, correct TCI-state mapping needs to be performed when SS sends RRC Resume on NR Cell 2 at step 10, as NR Cell 2 has different SSB ID than NR Cell 1. - At step 18, SS transmits PEI and step 19 has a negative check for transmission of RRC Resume Request message which is incorrect, as UE does not transmit RRC Resume Request message based on PEI and without associated Paging message in PO |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added specific message content tables for RRCResume with the PDSCH-config updated to the correct Tci-state and radioLinkMonitoringConfig. - Steps 18, 19 are made Void and updated Step 20 to have Paging Indication field in PEI DCI 2_7 set to 0 (no Paging in PO). NOTE: A Prose CR for these changes shall be raised at the next RAN5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1a.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:		Bhise Parikshit

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_1_1a_2_TestBody() |
| --- | --- |
| Reason for change | - Inline with agreed Tdocs R5-204415 and R5-241495, correct TCI-state mapping needs to be performed when SS sends RRC Resume on NR Cell 2 at step 10, as NR Cell 2 has different SSB ID than NR Cell 1. - At step 18, SS transmits PEI and step 19 has a negative check for transmission of RRC Resume Request message which is incorrect, as UE does not transmit RRC Resume Request message based on PEI and without associated Paging message in PO |
| Summary of change | - Added message content for RRCResume with the PDSCH-config updated to the correct Tci-state and radioLinkMonitoringConfig. - Steps 18, 19 are made Void and updated Step 20 to have Paging Indication field in PEI DCI 2_7 set to 0 (no Paging in PO). NOTE: A Prose CR for these changes shall be raised at the next RAN5 |
| TTCN module | RRC_PEI_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_1_1a_2_TestBody() runs on NR5GC_PTC { //@sic R5s250221 sic@ var I_RNTI_Value v_FullI_RNTI_diff; //@sic R5-254147 sic@ var PCCH_Config v_PcchConfig; var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var template (value) RRCRelease_v1610_IEs v_RRCRelease_v1610_IEs; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); var RNTI_Value_Type v_RNTI_Value_Type := f_NR_CellInfo_GetRNTI(nr_Cell2); timer t_Wait := 10.0; <<SKIPPED CODE>> //@siclog "Step 10" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell2); <<SKIPPED CODE>> //@siclog "Step 18" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(nr_Cell2, -,'11'B, 22); //@siclog "Step 19" siclog@ //Check: Does the UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value within 10s ? t_Wait.start; alt { [] t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); } [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 19"); } } f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_RRCResume(v_NR_PhysicalParameters, v_RNTI_Value_Type, tsc_NR_Msg4GrantHO_Def)); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2, tsc_NR_38508_NextHopChainingCount); //@siclog "Step 20" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging(nr_Cell2, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), v_PcchConfig.defaultPagingCycle); //@siclog "Step 21" siclog@ //The UE transmits an RRCResumeRequest message. f_NR_RRC_ResumeRequest_Def(nr_Cell2); //@siclog "Step 22" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell2); //@siclog "Step 23" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell2); }

After Change

function fl_TC_8_1_1_1a_2_TestBody() runs on NR5GC_PTC { var I_RNTI_Value v_FullI_RNTI_diff; //@sic R5-254147 sic@ var PCCH_Config v_PcchConfig; var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var template (value) RRCRelease_v1610_IEs v_RRCRelease_v1610_IEs; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); var RNTI_Value_Type v_RNTI_Value_Type := f_NR_CellInfo_GetRNTI(nr_Cell2); timer t_Wait := 10.0; //WA# var template (value) CellGroupConfig v_CellGroupConfig; var template (value) PDSCH_Config v_PDSCH_Config; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; var integer v_SSB_IndexCell2; var template (value) PUSCH_Config v_PUSCH_Config; var template (omit) NR_BWP_DownlinkDedicated_RadioLinkMonitoringConfig_Type v_RadioLinkMonitoringConfig := f_NR_CellInfo_GetRadioLinkMonitoringConfig(nr_Cell2); <<SKIPPED CODE>> //@siclog "Step 10" siclog@ //The SS transmits an RRCResume message. //WA# v_SSB_IndexCell2 := f_NR_CellInfo_GetSSB_Index(nr_Cell2); v_NR_DownlinkBWP_Cell2 := f_NR_CellInfo_GetDownlinkBWP(nr_Cell2, tsc_NR_BWP_Id); v_PDSCH_Config := v_NR_DownlinkBWP_Cell2.Pdsch.ConfigDedicated.R15; // Table 6.4.2.1.3.3-6 v_PDSCH_Config.dmrs_DownlinkForPDSCH_MappingTypeA := omit; v_PDSCH_Config.tci_StatesToAddModList[0] := cs_38508_TCI_State(v_SSB_IndexCell2); v_PDSCH_Config.prb_BundlingType := cs_NR_Prb_OmitBundleSize; v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config}; v_RadioLinkMonitoringConfig := cs_38508_RadioLinkMonitoringConfigDef(f_NR_CellInfo_GetSSB_Index(nr_Cell2)); f_NR_CellInfo_SetRadioLinkMonitoringConfig(nr_Cell2, v_RadioLinkMonitoringConfig); v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell2); v_CellGroupConfig.spCellConfig.spCellConfigDedicated.initialDownlinkBWP := cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config, -, v_RadioLinkMonitoringConfig); //@sic R5s241495 sic@ v_PUSCH_Config := cs_38508_PUSCH_Config_FR1(cs_38508_DMRS_UplinkConfig(pos1), v_SSB_IndexCell2); v_CellGroupConfig.spCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config := {setup := v_PUSCH_Config}; // @sic R5s210092 sic@ f_NR_SendRRCResume_Def(nr_Cell2, -, -, v_CellGroupConfig); <<SKIPPED CODE>> f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_RRCResume(v_NR_PhysicalParameters, v_RNTI_Value_Type, tsc_NR_Msg4GrantHO_Def)); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2, tsc_NR_38508_NextHopChainingCount); /*REMOVED @siclog "Step 18" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(nr_Cell2, -,'11'B, 22);*/ /*REMOVED @siclog "Step 19" siclog@ //Check: Does the UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value within 10s ? t_Wait.start; alt { [] t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); } [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 19"); } }*/ //@siclog "Step 20" siclog@ // The SS transmits a PEI with Paging indication field set to 0 and a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Trigger_DCI_2_7(nr_Cell2, -,'10'B, 22, true); //WA# //@siclog "Step 21" siclog@ //The UE transmits an RRCResumeRequest message. f_NR_RRC_ResumeRequest_Def(nr_Cell2); //@siclog "Step 22" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell2); //@siclog "Step 23" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell2); }