3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250465

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3766 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to errors found during TTCN-3 quality review |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2025-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Several errors were found during a quality review of the TTCN-3 implementation: - ‘isvalue’ and ‘valueof’ shall not be used on variables which are not defined as templates. - In the NasEmu_DecodePiggybacked functions, if a list only contains only one element, which is set to omit then the list itself should be set to omit. - There shall be no valueof operations for values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed ‘isvalue’ and ‘valueof’ where required. - Moved the assignment of v_NAS_UL_PduList[0] in the NasEmu_DecodePiggybacked functions to the branch where the decoding is valid. - Removed all appareances of valueof() functions over values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TTCN-3 code quality will not be optimised. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All TTCN-3 Testsuites |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## Change 1

| Function name | fl_TC_8_1_1_3_3_TestBody |
| --- | --- |
| Reason for change | The parameter v_PDUSessionInfo is not defined as a template. |
| Summary of change | Removed the test for isvalue and the valueof Similar change made in fl_TC_8_1_1_4_1_TestBody and f_TC_8_1_5_9_3_NR5GC_TestBody. |
| TTCN module | RRC_ConnRel_NR5GC.ttcn |

After Change

function fl_TC_8_1_1_3_3_TestBody() runs on NR5GC_PTC { //@sic R5-197055 sic@ var ARFCN_ValueNR v_NRf1_SSB_Cell1; var ARFCN_ValueNR v_NRf2_SSB_Cell3; var ARFCN_ValueNR v_NRf3_SSB_Cell6; var FreqPriorityListNR v_FreqPriorityListNR_Step2; var FreqPriorityListNR v_FreqPriorityListNR_Step6; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var PDUSessionInfoList_Type v_PDUSessionInfo; // @sic R5s220423 sic@ var integer v_DRBId; var integer v_WaitTime := 300; var float v_WaitForConnectionMin := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_WaitTime)); ... //@siclog "Step 3G" siclog@ //@sic R5-201136 sic@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); // @sic R5s220423 sic@ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); // @sic R5s220423 sic@ f_NR5GC_CheckDataPath(nr_Cell3, f_NR5GC_GetPdnIndex( v_PDUSessionInfo[0]), dedicatedBearer, v_DRBId, "Step 3G"); //@siclog "Step 3H" siclog@ //@sic R5-201136 sic@ //The SS releases RRC connection. f_NR_RRCRelease(nr_Cell3); //@siclog "Step 4" siclog@ //The SS changes NR Cell 1,NR Cell 3 and NR Cell 6 power level according to the row "T0" in table 8.1.1.3.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Step 5 - 5B" siclog@ //@sic R5-201136 sic@ //Steps 1-3 in TS 38.508-1 [4] Table 4.9.5.2.2-1 are peformed on NR Cell 1. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 5C - 5D" siclog@ //@sic R5-201136 sic@ //The SS transmits a SecurityModeCommand message. //The UE transmits a SecurityModeComplete message. f_NR_RRC_ActivateSecurity (nr_Cell1, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); //@siclog "Step 5E - 5F" siclog@ //@sic R5-201136 sic@ //Steps 4-5 of TS 38.508-1 [4] Table 4.9.5.2.2-1 of the generic procedure are performed. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message containing IE freqPriorityListNR to update the cell reselection priority of NR Cell 6 and the timer T320. f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, -, -, cs_CellReselectionPriorities(-, v_FreqPriorityListNR_Step6, min5))); //@siclog "Step 7" siclog@ //The SS changes NR Cell 1,NR Cell 3 and NR Cell 6 power level according to the row "T2" in table 8.1.1.3.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 8" siclog@ //Check: Does the UE transmit an RRCSetupRequest message within T320 running? f_NR_CheckNoRRCSetupReq(nr_Cell6, v_WaitForConnectionMin, -, "Step 8"); //@sic R5s201387 sic@ //@siclog "Step 9" siclog@ //@sic R5-201136 sic@ //Check: After timer T320 expiry, does the UE perform the Registration procedure for mobility registration update as described in steps 1-3 of Table 4.9.5.2.2-1 on NR Cell 6? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell6, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 10 - 11" siclog@ //@sic R5-201136 sic@ //The SS transmits a SecurityModeCommand message. //The UE transmits a SecurityModeComplete message. f_NR_RRC_ActivateSecurity (nr_Cell6, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); //@siclog "Step 12 - 13" siclog@ //@sic R5-201136 sic@ //Steps 4-5 of TS 38.508-1 [4] Table 4.9.5.2.2-1 of the generic procedure are performed. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell6, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); //@siclog "Step 14 - 15" siclog@ //@sic R5-201136 sic@ //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. //The UE transmits an RRCReconfigurationComplete message. f_NR_RRCReconfigExistingDRBs (nr_Cell6, omit); //@siclog "Step 16" siclog@ //@sic R5-201136 sic@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 6? f_NR5GC_CheckDataPath(nr_Cell6, f_NR5GC_GetPdnIndex( v_PDUSessionInfo[0]), dedicatedBearer, v_DRBId, "Step 16"); // @sic R5s200517, R5s220423 sic@ }

## Change 2

| Function name | f_NasEmu_DecodePiggybacked |
| --- | --- |
| Reason for change | The list variable v_NAS_UL_PduList should remain set to omit if the decoding is invalid. |
| Summary of change | Moved v_NAS_UL_PduList[0] := v_NAS_UL_Pdu; into the else statement above. The same change is also made in f_NG_NasEmu_DecodePiggybacked. |
| TTCN module | NasEmu_Common.ttcn |

After Change

function f_NasEmu_DecodePiggybacked(NAS_UL_Message_Type p_NAS_UL_Message) return template (omit) NAS_UL_PduList_Type { var template (omit) NAS_UL_PduList_Type v_NAS_UL_PduList := omit; var template (omit) NAS_UL_Pdu_Type v_NAS_UL_Pdu := omit; var octetstring v_PiggybackedOctetString := ''O; var NasEmu_DecodedNasPduUL_Type v_DecodedNasPduUL; if (ischosen(p_NAS_UL_Message.ul_NAS_TRANSPORT)) { // v_PiggybackedOctetString := p_NAS_UL_Message.ul_NAS_TRANSPORT.nasMessage.smsMessage; // decoding of smsMessage is not supported (i.e. the test case needs to deal with an octetstring) } else if (ischosen(p_NAS_UL_Message.aTTACH_COMPLETE)) { v_PiggybackedOctetString := p_NAS_UL_Message.aTTACH_COMPLETE.esmMessage.esmPdu; } else if (ischosen(p_NAS_UL_Message.aTTACH_REQUEST)) { v_PiggybackedOctetString := p_NAS_UL_Message.aTTACH_REQUEST.esmMessage.esmPdu; } else if (ischosen(p_NAS_UL_Message.cONTROL_PLANE_SERVICE_REQUEST)) { /* @sic R5s170364: handling of CONTROL_PLANE_SERVICE_REQUEST sic@ */ if (ispresent(p_NAS_UL_Message.cONTROL_PLANE_SERVICE_REQUEST.esmMessage.esmPdu)) { v_PiggybackedOctetString := p_NAS_UL_Message.cONTROL_PLANE_SERVICE_REQUEST.esmMessage.esmPdu; } } else { // NAS message does not contain piggybacked NAS message } if (lengthof(v_PiggybackedOctetString) > 0) { v_DecodedNasPduUL := f_NasEmu_DecodeMsg(v_PiggybackedOctetString); if (ischosen(v_DecodedNasPduUL.Invalid)) { // do nothing i.e. keep v_NAS_UL_Pdu as omit } else { v_NAS_UL_Pdu.Msg := v_DecodedNasPduUL.Msg; // recursive call: check whether piggybacked message contains further piggybacked messages v_NAS_UL_Pdu.PiggybackedPduList := f_NasEmu_DecodePiggybacked(v_DecodedNasPduUL.Msg); v_NAS_UL_PduList[0] := v_NAS_UL_Pdu; // Note: acc. to 24.301 V8.0.0 cl. 9.9.3.15 there is only one PDU piggybacked } } return v_NAS_UL_PduList; }

## Change 3

| Function name | f_UT_USIM_Insert_Or_Preconfigure |
| --- | --- |
| Reason for change | There shall be no valueof operations for values. |
| Summary of change | Removed all appareances of valueof() functions over values. Same change as shown in this clause done in following modules: EUTRA_SecuritySteps NG_NASTemplateFunctions NR_CellInfo NR_CommonFunctions NR5GC_CommonTestcaseFunctions NR5GC_IMSProcedures NR5GC_NASSteps NR5GC_RRCSteps NR5GC_RRCSteps_NRDC WLAN_CommonProcedures MAC_CovEnh_ENDC_NR MAC_ENDC_NR MAC_R16_R17_ENDC_NR PDCP_ENDC_NR NR_Handover_ENDC IMS_Procedures_Conferencing IPCAN_NR5GC HTTP_XCAP_Main ESM_Testcases RRC_MDT_IRAT_BW RSRVCC_EUTRA_UTRAN MCX_SIP_Common MCX_SIP_Registration IpcanModel_IPCAN_Component MCData_SDP_Messages NBIOT_CellInfo SOR_NR5GC MAC_R16_R17_NR5GC PDCP_NR5GC RRC_ConnEst_NR5GC RRC_MDT_IntraNR_RLF_NR5GC SecurityMode_NR5GC Service_Request_NR5GC GSM_NR5GC UAC_NR5GC Emergency_NR5GC MAC_CA_A_NR5GC PDCP_A_NR5GC RRC_Others_SDT_A_NR5GC Idle_PLMNSelection_NR5GC_NR IRAT_Handover_NR5GC_EUTRA IRAT_Handover_NR5GC_NR RRCMeasurement_NEDC_EUTRA RRC_Handover_UPIP_NR5GC_EUTRA EUTRA38_NR5GC_InitialRegistration EUTRA38_NR5GCProcedures NR5GC_IRAT_CommonFunctions NR5GC_PosSIB_Functions UTRAN34_feFACH_CommonFunctions |
| TTCN module | NasEmu_Common.ttcn |

After Change

function f_UT_USIM_Insert_Or_Preconfigure(UT_PTC_MTC_PORT p_Port, charstring p_USIM_ID) { fl_UT_ApplyCommand(p_Port, cas_UT_Req(INSERT_USIM_OR_PRECONFIGURE, CNF_REQUIRED, cs_InsertUsim(p_USIM_ID))); }