3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250468

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3767 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function fl_Check_featureSets_r15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per TS 38.508-2, pc_nrMIMO_CB_PUSCH is independent from the value of its sub-IE maxNumberMIMO_LayersCB_PUSCH. Current TTCN implementation is setting this PICS to true only if UE reports twolayer or fourLayer in the sub-IE maxNumberMIMO_LayersCB_PUSCH which is not aligned with the top-level IE PICS pc_nrMIMO_CB_PUSCH definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | pc_nrMIMO_CB_PUSCH is set to TRUE based upon reporting the top-level IE mimo-CB-PUSCH. The check for reporting a value for maxNumberMIMO_LayersCB_PUSCH is moved inside the top-level IE check to align with the check added for maxNumberSRS-ResourcePerSet. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC, 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function fl_Check_featureSets_r15 |
| --- | --- |
| Reason for change | As per TS 38.508-2, pc_nrMIMO_CB_PUSCH is independent from the value of its sub-IE maxNumberMIMO_LayersCB_PUSCH. Current TTCN implementation is setting this PICS to true only if UE reports twolayer or fourLayer in the sub-IE maxNumberMIMO_LayersCB_PUSCH which is not aligned with the top-level IE PICS pc_nrMIMO_CB_PUSCH definition. |
| Summary of change | pc_nrMIMO_CB_PUSCH is set to TRUE based upon reporting the top-level IE mimo-CB-PUSCH. The check for reporting a value for maxNumberMIMO_LayersCB_PUSCH is moved inside the top-level IE check to align with the check added for maxNumberSRS-ResourcePerSet. |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_Check_featureSets_r15 (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5-250329 R5s221100 R5s221147 R5-227571 R5-227572 R5-227573 R5-231410 R5-241518 R5-242327 R5-246407 R5s250177 R5s250243 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Index := 0; var boolean v_Found_dynamicSwitchSUL := false; var boolean v_Found_twoPUCCH_Group := false; var boolean v_Found_mimo_CB_PUSCH := false; v_Received_NRCapabilityMsg := p_NRCapability; f_Check_maxNumberMIMO_LayersPDSCH (p_NRCapability, p_Configuration_Type); // Check featureSets.featureSetDownlinkPerCC.maxNumberMIMO-LayersPDSCH if(ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink)) { while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink)) and ((not v_Found_dynamicSwitchSUL) or (not v_Found_twoPUCCH_Group))) { // check featureSetsUplink if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink[v_Index].dynamicSwitchSUL)) { v_Found_dynamicSwitchSUL := true; } if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink[v_Index].twoPUCCH_Group)) { v_Found_twoPUCCH_Group := true; } v_Index := v_Index + 1; } } if (not f_NR_CompareCapability_vs_PICS(pc_dynamicSwitch_SUL, v_Found_dynamicSwitchSUL)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_dynamicSwitch_SUL not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_twoPUCCH_group, v_Found_twoPUCCH_Group)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_twoPUCCH_group not correctly set"); } v_Index := 0; if(ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsUplinkPerCC)) { while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplinkPerCC)) and ((not v_Found_mimo_CB_PUSCH))) { // check featureSetsUplinkPerCC if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplinkPerCC[v_Index].mimo_CB_PUSCH) and match(v_Received_NRCapabilityMsg.featureSets.featureSetsUplinkPerCC[v_Index].mimo_CB_PUSCH.maxNumberMIMO_LayersCB_PUSCH, (twoLayers, fourLayers))) { v_Found_mimo_CB_PUSCH := true; if (not Sisvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplinkPerCC[v_Index].mimo_CB_PUSCH.maxNumberSRS_ResourcePerSet)) { f_SetVerdict(fail,__FILE__, __LINE__, "mimo-CB-PUSCH.maxNumberSRS-ResourcePerSet is not signalled by UE in an existing featureSetsUplinkPerCC"); } } v_Index := v_Index + 1; } if (not f_NR_CompareCapability_vs_PICS(pc_nrMIMO_CB_PUSCH, v_Found_mimo_CB_PUSCH)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_nrMIMO_CB_PUSCH not correctly set"); } } }

After Change:

function fl_Check_featureSets_r15 (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5-250329 R5s221100 R5s221147 R5-227571 R5-227572 R5-227573 R5-231410 R5-241518 R5-242327 R5-246407 R5s250177 R5s250243 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Index := 0; var boolean v_Found_dynamicSwitchSUL := false; var boolean v_Found_twoPUCCH_Group := false; var boolean v_Found_mimo_CB_PUSCH := false; v_Received_NRCapabilityMsg := p_NRCapability; f_Check_maxNumberMIMO_LayersPDSCH (p_NRCapability, p_Configuration_Type); // Check featureSets.featureSetDownlinkPerCC.maxNumberMIMO-LayersPDSCH if(ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink)) { while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink)) and ((not v_Found_dynamicSwitchSUL) or (not v_Found_twoPUCCH_Group))) { // check featureSetsUplink if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink[v_Index].dynamicSwitchSUL)) { v_Found_dynamicSwitchSUL := true; } if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink[v_Index].twoPUCCH_Group)) { v_Found_twoPUCCH_Group := true; } v_Index := v_Index + 1; } } if (not f_NR_CompareCapability_vs_PICS(pc_dynamicSwitch_SUL, v_Found_dynamicSwitchSUL)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_dynamicSwitch_SUL not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_twoPUCCH_group, v_Found_twoPUCCH_Group)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_twoPUCCH_group not correctly set"); } v_Index := 0; if(ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsUplinkPerCC)) { while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplinkPerCC)) and ((not v_Found_mimo_CB_PUSCH))) { // check featureSetsUplinkPerCC if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplinkPerCC[v_Index].mimo_CB_PUSCH) ) { v_Found_mimo_CB_PUSCH := true; if (not isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplinkPerCC[v_Index].mimo_CB_PUSCH.maxNumberMIMO_LayersCB_PUSCH)) { f_SetVerdict(fail,__FILE__, __LINE__, "maxNumberMIMO_LayersCB_PUSCH is not signalled by UE in an existing featureSetsUplinkPerCC"); } if (not isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplinkPerCC[v_Index].mimo_CB_PUSCH.maxNumberSRS_ResourcePerSet)) { f_SetVerdict(fail,__FILE__, __LINE__, "mimo-CB-PUSCH.maxNumberSRS-ResourcePerSet is not signalled by UE in an existing featureSetsUplinkPerCC"); } } v_Index := v_Index + 1; } if (not f_NR_CompareCapability_vs_PICS(pc_nrMIMO_CB_PUSCH, v_Found_mimo_CB_PUSCH)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_nrMIMO_CB_PUSCH not correctly set"); } } }