3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250473

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3772 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_EmergencyCallRelease |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN-3 implementation of testcase 11.4.7, the function fl_EmergencyCallAndReleasef_NR5GC_EmergencyCallRelease is called many times. The function f_NR5GC_EmergencyCallRelease is based on 38.508-1 table 4.9.12A. As per the expected sequence in this Table, the MO release of voice call/5GS according to 34.229-5 Annex A.7 should be performed starting of timer of 5s. The current TTCN starts this timer as soon as this function is entered. This is incorrect and needs to be addressed (Similar issue was fixed in R5s250335). Also the testcases 11.4.7 and 11.48 when run manually take much longer than the existing guard timer value of 900s. This also needs to be accommodated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN implementation of function f_NR5GC_EmergencyCallRelease. Please see below. Also increased the guard timers for 11.4.7 and 11.4.8 to 900s. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.7.NR5GC, 11.4.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	6

2.3	Change 3	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_NR5GC_EmergencyCallRelease |
| --- | --- |
| Reason for change | In the current TTCN-3 implementation of testcase 11.4.7, the function fl_EmergencyCallAndReleasef_NR5GC_EmergencyCallRelease is called many times. The function f_NR5GC_EmergencyCallRelease is based on 38.508-1 table 4.9.12A. As per the expected sequence in this Table, the MO release of voice call/5GS according to 34.229-5 Annex A.7 should be performed starting of timer of 5s. The current TTCN starts this timer as soon as this function is entered. This is incorrect and needs to be addressed (Similar issue was fixed in R5s250335). |
| Summary of change | Updated the TTCN implementation of function f_NR5GC_EmergencyCallRelease. Please see below. |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC TF160 comment |  |

Before Change:

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId, boolean p_MOTriggerRequired := true, // set to false if MT or if UT command already sent boolean p_IMSMsgsNotYetSent:= true, // set to false in IPCAN as IMS msgs will have already been sent before f_IMS_CC_ReleaseConnection is called EmergencyCallRelease_PDUSession p_Release_Keep := Release_PDUSession) runs on NR5GC_PTC // set to Keep_PDUSession if you only want to modify the emergency PDU session to remove the voice Qos { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var PDN_Index_Type v_PdnIndex := v_PDUSession.PdnIndex; // @sic R5s220829 sic@ var boolean v_IMSMsgsNotYetSent := p_IMSMsgsNotYetSent; timer t_Wait := 5.0; if (p_MOTriggerRequired) { // @sic R5s220531 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Trigger MO Call Release")); // @sic R5s220039, R5-225362 sic@ } // Step 3A1 t_Wait.start; alt { // @sic R5-216829 sic@ [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp // @sic R5s211524 sic@ { // Step 3Ba1 t_Wait.stop; // Step 3Ba2 if (not f_Check_NG_PDUSessionReleaseReq (v_PDUSession, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request failed"); } if (v_IMSMsgsNotYetSent) { // @sic R5s220789 sic@ // Even though NAS PDU Release Req is received, IMS Msgs might not have been processed IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); // @sic R5s220039, R5-225362 sic@ sic@ } // Step 3Ba3 f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); // PDU session released, so finish function f_IP_RtpRtcpLoopback(IP, v_PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363, R5s220829 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); } [v_IMSMsgsNotYetSent] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")) // @sic R5s220039, R5-225362 sic@ sic@ { // this may be received at anytime v_IMSMsgsNotYetSent := false; repeat; } [] t_Wait.timeout // @sic R5s220039 sic@ { // Step 3Bb1 // Steps 3Bb2a1 - 3Bb2b3 are the same as MT procedure in 4.9.12B f_NR5GC_EmergencyCallReleaseMT (p_NR_CellId, v_IMSMsgsNotYetSent, p_Release_Keep); // @sic R5s210363, R5s220829, R5-225362 sic@ } } }

After Change:

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId, boolean p_MOTriggerRequired := true, // set to false if MT or if UT command already sent boolean p_IMSMsgsNotYetSent:= true, // set to false in IPCAN as IMS msgs will have already been sent before f_IMS_CC_ReleaseConnection is called EmergencyCallRelease_PDUSession p_Release_Keep := Release_PDUSession) runs on NR5GC_PTC // set to Keep_PDUSession if you only want to modify the emergency PDU session to remove the voice Qos { var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var PDN_Index_Type v_PdnIndex := v_PDUSession.PdnIndex; // @sic R5s220829 sic@ var boolean v_IMSMsgsNotYetSent := p_IMSMsgsNotYetSent; var boolean v_ReceivedReleaseComplete := false; //WA#WI1462630 WA#11_4_7 timer t_Wait := 5.0; if (p_MOTriggerRequired) { // @sic R5s220531 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2], cms_IMS_IPCAN_Trigger("Trigger MO Call Release")); // @sic R5s220039, R5-225362 sic@ } // Step 3A1 // COMMENTED OUT t_Wait.start; //WA#WI1462630 WA#11_4_7 alt { // @sic R5-216829 sic@ [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp // @sic R5s211524 sic@ { // Step 3Ba1 t_Wait.stop; // Step 3Ba2 if (not f_Check_NG_PDUSessionReleaseReq (v_PDUSession, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request failed"); } if (v_IMSMsgsNotYetSent) { // @sic R5s220789 sic@ // Even though NAS PDU Release Req is received, IMS Msgs might not have been processed IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); // @sic R5s220039, R5-225362 sic@ sic@ } // Step 3Ba3 f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); v_ReceivedReleaseComplete := true; //WA#WI1462630 WA#11_4_7 // PDU session released, so finish function f_IP_RtpRtcpLoopback(IP, v_PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363, R5s220829 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); } [v_IMSMsgsNotYetSent] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")) // @sic R5s220039, R5-225362 sic@ sic@ { // this may be received at anytime v_IMSMsgsNotYetSent := false; t_Wait.start; //WA#WI1462630 WA#11_4_7 repeat; } [] t_Wait.timeout // @sic R5s220039 sic@ { // Step 3Bb1 // Steps 3Bb2a1 - 3Bb2b3 are the same as MT procedure in 4.9.12B if(not v_ReceivedReleaseComplete) { //WA#WI1462630 WA#11_4_7 f_NR5GC_EmergencyCallReleaseMT (p_NR_CellId, v_IMSMsgsNotYetSent, p_Release_Keep); // @sic R5s210363, R5s220829, R5-225362 sic@ } } } }

## Change 2

| Function name | TC_11_4_7_NR5GC |
| --- | --- |
| Reason for change | The testcase 11.4.7 when run manually take much longer than the existing guard timer value of 900s. This needs to be accommodated. |
| Summary of change | Increased the guard timer to 1800s |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC TF160 comment |  |

Before Change:

testcase TC_11_4_7_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Handling of Local and Extended emergency numbers / Mobility var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(900); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_4_7_NR5GC()); v_IMS1.start(f_TC_11_4_7_NR5GC_IMS1()); v_IMS2.start(f_TC_11_4_7_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change:

testcase TC_11_4_7_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Handling of Local and Extended emergency numbers / Mobility var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(1800); //WA#WI1462630 WA#11_4_7 900-->1800 v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_4_7_NR5GC()); v_IMS1.start(f_TC_11_4_7_NR5GC_IMS1()); v_IMS2.start(f_TC_11_4_7_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

## Change 3

| Function name | TC_11_4_8_NR5GC |
| --- | --- |
| Reason for change | The testcase 11.4.8 when run manually take much longer than the existing guard timer value of 900s. This needs to be accommodated. |
| Summary of change | Increased the guard timer to 1800s |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC TF160 comment |  |

Before Change:

testcase TC_11_4_8_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Handling of Local and Extended emergency numbers / Switch-off and maximum local numbers storage var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(900); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_4_8_NR5GC()); v_IMS1.start(f_TC_11_4_8_NR5GC_IMS1()); v_IMS2.start(f_TC_11_4_8_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change:

testcase TC_11_4_8_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Handling of Local and Extended emergency numbers / Switch-off and maximum local numbers storage var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(1800); //WA#WI1462630 WA#11_4_8 900-->1800 v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_4_8_NR5GC()); v_IMS1.start(f_TC_11_4_8_NR5GC_IMS1()); v_IMS2.start(f_TC_11_4_8_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }