3GPP TSG- RAN4 Meeting #117	R4-2522680

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6085 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BigCR on core maintenance for NonCol_intraB_ENDC_NR_CA_Ph2_RRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Resubmission of endorsed big CR R4-2514871. - On top of endorsed big CR R4-2514871, further updates are made at RAN4#117 meeting. |  |  |  |  |  |  |  |  |
| s |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | The document includes the endorsed big CR R4-2514871, and the updates which are further endorsed during RAN4#117 meeting: T-doc number; Company. R4-2520922; Huawei, HiSilicon. R4-2521081; Apple. R4-2521162; ZTE Corporation, Sanechips. |
| Consequences if not approved: |  | RRM requirements for supporting intra-band non-collocated NR-CA are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.1, 7.5.4, 7.6.2.1, 7.6.4, 8.1.7.2, 8.2.2.2.3, 8.3.2, 9.2.5.3.1, 9.2.5.3.2, 9.3.9.3.1, 9.3.9.3.2, 9.5.6.2; 9.7.4.1, 9.8.6.2, 9.10.2.6, 9.13.6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

#### 7.5.2.1	Minimum requirements for inter-band synchronous EN-DC

The requirements in this clause apply as a reference for inter-band synchronous EN-DC.

The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell for inter-band synchronous EN-DC as shown in table 7.5.2.1-1 1. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD, E-UTRA FDD-NR FDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC.

For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping or partially overlapping DL bands, only synchronized operation is assumed. The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in table 7.5.2.1-1 provided that UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and not provided with nonCollocatedTypeMRDC-r18, and in table 7.5.3-1 provided that UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or interBandMRDC-WithOverlapDL-Bands-r19 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and provided with nonCollocatedTypeMRDC-r18.

For E-UTRA FDD-NR FDD inter-band EN-DC with overlapping DL bands, only synchronized operation is assumed if UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation. The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in table 7.5.2.1-1 provided that UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and not provided with nonCollocatedTypeMRDC-r18, and in table 7.5.3-1 provided that UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or interBandMRDC-WithOverlapDL-Bands-r19 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and provided with nonCollocatedTypeMRDC-r18.For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping or partially overlapping DL bands, or E-UTRA FDD-NR FDD inter-band EN-DC with overlapping DL bands, if UE indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4,

2> if nonCollocatedTypeMRDC-v1900 is present with “type 4”,

3>	the UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in table 7.5.2.1-1.

2> otherwise, if nonCollocatedTypeMRDC-v1900 is present with “type 1” or absent,

3>	the UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in table 7.5.3-1.

- when UE is configured with maxMIMO-Layers with value equal to 2,

2> if nonCollocatedTypeMRDC-r18 is not provided,

3>	the UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in table 7.5.2.1-1.

2> otherwise

3> the UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in table 7.5.3-1.

Table 7.5.2.1-1 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 15 | 30 | 35.21 |
| 15 | 60 | 35.21 |
| 15 | 120 | 35.21 |

<Unchanged Text Skipped>

### 7.5.4	Minimum requirements for NR Carrier Aggregation

The UE shall be capable of handling at least a relative transmission timing difference between slot timing of all pairs of TAGs in FR1 and/or FR2-1 as shown in table 7.5.4-1, provided that the UE is:

-	configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode, or

-	configured with more than one sTAG for inter-band NR carrier aggregation in EN-DC or NE-DC mode.

The UE shall be capable of handling at least a relative transmission timing difference between subframe timing of all pairs of TAGs between FR1 and FR2-2 as shown in table 7.5.4-1, provided that the UE is:

-	configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode.

For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative transmission timing difference as shown in table 7.5.4-1, between slot timing of all FR1 pairs of TAGs provided that UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

For FR1 intra-band non-contiguous NR carrier aggregation and UE supporting intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4,

2> if nonCollocatedTypeNR-CA-v1900 is present with “type 4”,

3>	the UE shall be capable of handling at least a relative transmission timing difference between slot timing of all FR1 pairs of TAGs as shown in table 7.5.4-1.

1> when UE is configured with maxMIMO-Layers with value equal to 2,

2> if nonCollocatedTypeNR-CA-r18 is not provided,

3> the UE shall be capable of handling at least a relative transmission timing difference between slot timing of all FR1 pairs of TAGs as shown in table 7.5.4-1.

Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 Note1 |
| Between FR1 and FR2-1 | 26.1 |
| Between FR1 and FR2-2 | 26.1 |
| NOTE1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

<Unchanged Text Skipped>

#### 7.6.2.1	Minimum requirements for inter-band synchronous EN-DC

The requirements in this clause apply as a reference for inter-band synchronous EN-DC.

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from an E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to SCG at the UE receiver for inter-band synchronous EN-DC as shown in table 7.6.2.1-1. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD, E-UTRA FDD-NR FDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC.

For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping or partially overlapping DL bands, only synchronized operation is assumed. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in table 7.6.2.1-1 provided that UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and not provided with nonCollocatedTypeMRDC-r18, and in table 7.6.3-1 provided that UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16 or interBandMRDC-WithOverlapDL-Bands-r19 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and provided with nonCollocatedTypeMRDC-r18.

For E-UTRA FDD-NR FDD inter-band EN-DC with overlapping DL bands, only synchronized operation is assumed if UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in table 7.6.2.1-1 provided that UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and not provided with nonCollocatedTypeMRDC-r18, and in table 7.6.3-1 provided that UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16 or interBandMRDC-WithOverlapDL-Bands-r19 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and provided with nonCollocatedTypeMRDC-r18.

For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping or partially overlapping DL bands, or E-UTRA FDD-NR FDD inter-band EN-DC with overlapping DL bands, if UE indicates it is capable of interBandMRDC-WithOverlapDL-Bands-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4,

2> if nonCollocatedTypeMRDC-v1900 is present with “type 4”,

3>	the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in table 7.6.2.1-1.

2> otherwise, if nonCollocatedTypeMRDC-v1900 is present with “type 1” or absent,

3>	the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in table 7.6.3-1.

- when UE is configured with maxMIMO-Layers with value equal to 2,

2> if nonCollocatedTypeMRDC-r18 is not provided,

3> the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in table 7.6.2.1-1.

2> otherwise

3>	the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in table 7.6.3-1.

Table 7.6.2.1-1: Maximum receive timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing of E-UTRA cell in MCG (kHz) | DL Sub-carrier spacing of cell in SCG (kHz) (Note1) | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 33 |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |
| NOTE 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

<Unchanged Text Skipped>

### 7.6.4	Minimum requirements for NR Carrier Aggregation

For intra-band contiguous CA, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE not capable of intraBandNR-CA-non-collocated-r18 shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in table 7.6.4-1 below.

For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference as shown in table 7.6.4-2, between slot timing of different FR1 carriers to be aggregated at the UE receiver provided that UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided. Otherwise, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in table 7.6.4-1 below if UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided.

For FR1 intra-band non-contiguous NR carrier aggregation and UE supporting intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4,

2> if nonCollocatedTypeNR-CA-v1900 is present with “type 4”,

3>	the UE shall be capable of handling at least a relative receive timing difference between slot timing of different FR1 carriers to be aggregated at the UE receiver as shown in table 7.6.4-2.

2> otherwise, if nonCollocatedTypeNR-CA-v1900 is present with “type 1” or absent,

3>	the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in table 7.6.4-1.

1> when UE is configured with maxMIMO-Layers with value equal to 2,

2> if nonCollocatedTypeNR-CA-r18 is not provided,

3>	the UE shall be capable of handling at least a relative receive timing difference between slot timing of different FR1 carriers to be aggregated at the UE receiver as shown in table 7.6.4-2.

2> otherwise,

3>	the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in table 7.6.4-1.

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 31 |
| FR2-1 | 0.26 |
| NOTE 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. |  |

For inter-band NR carrier aggregation,

-	the UE shall be capable of handling at least a relative receive timing difference between slot timing of all pairs of carriers in FR1 and FR2-1 to be aggregated at the UE receiver as shown in table 7.6.4-2 below.

-	the UE shall be capable of handling at least a relative receive timing difference between subframe timing of all pairs of carriers in FR1 and FR2-2 to be aggregated at the UE receiver as shown in table 7.6.4-2 below.

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 note1 |
| Between FR1 and FR2-1 | 25 |
| Between FR1 and FR2-2 | 25 |
| NOTE1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

<Unchanged Text Skipped>

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring based on SSB as RLM-RS. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to radio link monitoring based on SSB as RLM -RS.

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for radio link monitoring.

When intra-band carrier aggregation in FR1 is performed, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band if nonCollocatedTypeNR-CA-r18 is not provided. Otherwise, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

When intra-band non-contiguous carrier aggregation is configured for a UE supporting intraBandNR-CA-non-collocated-r19,

1> 	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> 	if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band

2> 	if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions on FR1 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions on FR1 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band

2>else

3> the scheduling restrictions on FR1 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When inter-band FR1-FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to radio link monitoring performed on FR1 serving PCell or PSCell in different bands.

<Unchanged Text Skipped>

8.2.2.2.3	Interruptions during measurements on deactivated SCC

Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK

when the configured measCycleSCell [2] is 640 ms or longer.

-	If the PCell or activated SCell(s) is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

-	If the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band and UE is capable of intraBandNR-CA-non-collocated-r18 on this FR1 band and nonCollocatedTypeNR-CA-r18 is not provided, the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

-	If the PCell or activated SCell(s) is contiguous to the deactivated SCell in the same FR1 band, or if the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

-	If the PCell or activated SCell(s) is contiguous to the deactivated SCell in the same FR1 band, or if the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band and UE is not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

-	If the PCell or activated SCell(s) is non-contiguous to the deactivated SCell in the same FR1 band, and UE is capable of intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA-v1900 is presented with “type 4”

3>	the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

2> if nonCollocatedTypeNR-CA-v1900 is presented with “type 1”

3>	the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

2>else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3>	the UE is only allowed to cause interruptions on PCell or activated SCell(s) immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in table 8.2.2.2.2-1.

2> else

3> the UE is only allowed to cause an interruption on PCell or activated SCell(s) no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by table 8.2.2.2.3-1. The interruption shall not exceed requirements in table 8.2.2.2.3-1.

The interruption requirements in table 8.2.2.2.3-1 are not applicable when a UE is configured with NCSG in the same frequency range as the deactivated SCell unless the SMTC on the deactivated SCC is fully non-overlapped with NCSG.

When UE supporting On-demand SSB operation, during meausrments on deactivated SCC

-    No interruption is allowed on deactivated SCC before OD-SSB is indicated when there is only OD-SSB but not the first SSB transmission.

-    If UE does not receive SCell activation command at the end of FMW, the same interruption of deactivated SCell measurement interruption can be applied, provided that the measCycleSCell equal or longer than 640ms when

-	SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321).

Table 8.2.2.2.3-1: Interruption duration for measurement on deactivated SCC for intra-band CA

|  | NR Slot length (ms) | X (slots) | Interruption length (slots) |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 | 8 + TSMTC_duration * Nslotsubframe,μ |
| 5 | 0.03125 | 16 | 32 + TSMTC_duration * Nslotsubframe,μ |
| 6 | 0.015625 | 32 | 64 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is the longest SMTC duration among all above active serving cells and the deactivated SCell to be measured i; NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]. |  |  |  |  |

For a UE supporting LB-CA via switching and configured with switch pattern, interruptions on PCell due to the measurement on deactivated SDL SCell are allowed with up to Y % probability of missed ACK/NACK. The value of Y is defined as 2*Length_intmeasCycleSCell ;where, Length_int is equal to  (Tuning time from PCell to SCell  + TSMTC_duation + Retuning time from SCell to PCell),

Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell, i.e., LBCA-SwitchingGap-Duration-PCelltoSCell-r19 in slots

Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap-SCelltoPCell-r19 in slots

<Unchanged Text Skipped>

8.3.2	SCell Activation Delay Requirement for Deactivated SCell

<Unchanged Text Skipped>

TSMTC_MAX:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR1, in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	TSMTC_MAX is the SMTC periodicity of SCell being activated

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3>	TSMTC_MAX is the SMTC periodicity of SCell being activated

2>else

3>	TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot

-	In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	TSMTC_MAX is bounded to a minimum value of 10 ms.

TSMTC_MAX, enhanced:

-	In FR1 or FR2-1, a UE supporting shortMeasInterval-r18 if the SMTC for SCell being activated is only configured in measObjectNR, TSMTC_MAX, enhanced is the SSB periodicity of SCell being activated. Otherwise, TSMTC_MAX, enhanced = TSMTC_MAX.

Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There are no requirements if the SSB transmission periodicity is not 5 ms.

Trs, enhanced is the SSB periodicity of the SCell being activated for a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR. Otherwise, Trs, enhanced = Trs

T∆ is the additional time for fine time tracking and acquiring full timing information of the SCell. T∆ = 2*Trs ms for SCell operating with 12 PRB SSB BW. Otherwise, T∆ = 0 ms.

TFirstSSB: is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5 ms if SMTC is not configured, after slot n + THARQ+3msNR slot length.

TFirstSSB_MAX: Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5 ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR1, in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>the first occasion when the SCell being activated is transmitting SSB burst

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3>	the first occasion when the SCell being activated is transmitting SSB burst

2>else

3>	the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

<Unchanged Text Skipped>

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

For UE indicating intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2>else

3> the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

When inter-band FR1+FR1 NR-DC is performed, the scheduling restrictions due to a given serving cell also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which does not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured(in TS 38.331 [2]), the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

If the following conditions are met:

-	The UE has been notified about system information update through paging,

-	The gap between the UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots

The UE shall receive the PDCCH that the UE monitors in the Type0-PDCCH CSS set, and/or the corresponding PDSCH, on SSB symbols to be measured.

When intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

For UE indicating intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2>else

3> the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

<Unchanged Text Skipped>

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1

When UE performs inter-frequency measurements without measurement gaps in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

When UE performs inter-frequency measurements without measurement gaps in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration.

When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to one serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions due to one serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols if nonCollocatedTypeNR-CA-r18 is provided

For UE indicating intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2>else

3> the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UE which do not support simultaneousRxDataSSB-DiffNumerology-Inter-r16 [14] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If UE performs inter-frequency measurements without measurement gaps in a TDD band, UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

-	If UE performs inter-frequency measurements without measurement gaps in a FDD band, UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on the union of restricted serving cell symbols due to measurement of all MOs, where the restricted serving cell symbols due to measurement of MO i include

-	serving cell symbols fully or partially overlap with SSB symbols to be measured on MO i, and △t serving cell symbol before each consecutive SSB symbols to be measured and △t serving cell symbol after each consecutive SSB symbols to be measured within SMTC window duration, if deriveSSB-IndexFromCellInter-r17 is enabled for MO i and UE supporting deriveSSB-IndexFromCellInterNon-NCSG-r17. △t is defined as the minimum integer number of symbols with total duration no smaller than the tolerance specified in clause 7.9, or

-	serving cell symbols fully or partially overlap with SMTC window for MO i and on 1 serving cell symbol before and after the SMTC window, if deriveSSB-IndexFromCellInter-r17 is not enabled for MO i, or UE not supporting deriveSSB-IndexFromCellInterNon-NCSG-r17.

When intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

For UE indicating intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2>else

3> the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

<Unchanged Text Skipped>

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] and do not support rxTimingDiff-r18, the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] but support rxTimingDiff-r18, the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI [on symbols that are overlapping or partially overlapping with the symbols corresponding to] the SSB indexes configured for L1-RSRP measurement.

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on serving cell where L1-RSRP measurement is performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on serving cell where L1-RSRP measurement is performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

For UE indicating intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2>else

3> the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which the serving cell where L1-RSRP measurement is performed is configured.

When inter-band FR1+FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which the serving cell where L1-RSRP measurement is performed is configured.

<Unchanged Text Skipped>

9.7.4.1	Scheduling availability of UE performing measurement on FR1

The following scheduling restriction applies due to CLI measurements.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs CLI measurements, and on 1 data symbol before an OFDM symbol used for CLI measurements for 15 kHz and 30 kHz subcarrier spacing.

-	For the UE which does not support cli-SRS-RSRP-FDM_DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs SRS-RSRP measurements, and on 1 data symbol before an OFDM symbol used for SRS-RSRP measurements for 15 kHz and 30 kHz subcarrier spacing.

-	For the UE which does not support cli-RSSI-FDM-DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs CLI-RSSI measurements, and on 1 data symbol before an OFDM symbol used for CLI-RSSI measurements for 15 kHz and 30 kHz subcarrier spacing.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs CLI measurement, and on 2 data symbols before an OFDM symbol used for CLI measurements for 60 kHz subcarrier spacing.

-	For the UE which does not support cli-SRS-RSRP-FDM_DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs SRS-RSRP measurement, and on 2 data symbols before an OFDM symbol used for SRS-RSRP measurements for 60 kHz subcarrier spacing.

-	For the UE which does not support cli-RSSI-FDM-DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs CLI-RSSI measurement, and on 2 data symbols before an OFDM symbol used for CLI-RSSI measurements for 60 kHz subcarrier spacing.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where CLI measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on serving cell where CLI measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

For UE indicating intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2>else

3> the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

<Unchanged Text Skipped>

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-SINR measurement based on SSB configured for L1-SINR measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-SINR measurement based on SSB configured for L1-SINR measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking /CSI-RS for CQI on SSB symbols to be measured for L1-SINR measurement.

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on serving cell where L1-SINR measurement is performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on serving cell where L1-SINR measurement is performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols if nonCollocatedTypeNR-CA-r18 is provided

For UE indicating intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2>else

3> the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which the serving cell where L1-SINR measurement is performed is configured.

<Unchanged Text Skipped>

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements

UE is required to be capable of measuring without measurement gaps when CSI-RS resources are completely contained in the active BWP of the UE. Note the configured CSI-RS symbol is indicated in firstOFDMSymbolInTimeDomain included in CSI-RS-ResourceConfigMobility for RRM. When UE is required to perform CSI-RS based RRM measurements, and any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note same numerology for intra-frequency CSI-RS and data of serving cell is considered in this release.

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands

When UE performs CSI-RS intra-frequency measurements in a TDD band,

-	UE is not expected to transmit PUCCH/PUSCH/SRS on configured CSI-RS resource symbols, and on 1 OFDM symbol before and after each consecutively configured CSI-RS symbols.

When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band. Ohterwise, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

For UE indicating intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2>else

3> the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2

When the UE performs CSI-RS based intra-frequency measurements for L3 mobility management in FR2, the following restrictions apply.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on the configured CSI-RS symbol within the configured slot as indicated in slotConfig of the corresponding CSI-RS resource to be measured for mobility.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 serving cells in the bands due to CSI-RSRP, CSI-RSRQ or CSI-SINR measurement on an FR2 intra-frequency cell in different bands, provided that UE is capable of independent beam management on this FR2 band pair.

<Unchanged Text Skipped>

9.13.6	Scheduling availability of UE during L1-RSRP measurement

Scheduling availability restrictions described in the following clauses apply when UE is performing L1-RSRP measurement on cell(s) with PCI different from serving cell, and UE is receiving PDCCH/PDSCH from serving cell and/or cell(s) with different PCI.

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to L1-RSRP measurement performed on SSB as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1.

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

If UE does not support [multiDCI-InterCellMultiTRP-TwoTA-r18 or multiDCI-IntraCellMultiTRP-TwoTA-r18 and rxTimingDiff-r18],

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

If UE supports [multiDCI-InterCellMultiTRP-TwoTA-r18 or multiDCI-IntraCellMultiTRP-TwoTA-r18 and rxTimingDiff-r18],

-	For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols which are overlapped or partially overlapped with SSB symbols configured for L1-RSRP measurement

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions apply to cell(s) in the same band on the symbols that fully or partially overlap with restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions apply to cell(s) in the same band on the symbols that fully or partially overlap with restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

For UE indicating intraBandNR-CA-non-collocated-r19,

1>	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2> if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there is no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band.

2>else

3> the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 cells configured in other bands than the bands in which the cell where L1-RSRP measurement is performed is configured.

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions is performed apply to cell(s) in the band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 cells in the bands due to L1-RSRP measurement performed on FR2 cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

If following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE shall receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for L1-RSRP measurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE shall receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for L1-RSRP measurement.

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA

There are no scheduling restrictions on FR1 cell(s) due to L1-RSRP measurement performed on FR2 cell(s).

There are no scheduling restrictions on FR2 cell(s) due to L1-RSRP measurement performed on FR1 cell(s).

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1

When UE performs L1-RSRP measurement on cell with different PCI from serving cell in a TDD band, the following restrictions apply due to L1-RSRP measurement

If UE does not support [multiDCI-InterCellMultiTRP-TwoTA-r18 or multiDCI-IntraCellMultiTRP-TwoTA-r18 and rxTimingDiff-r18],

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on symbols corresponding to the SSB indexes configured for L1-RSRP measurement, where the transmission of PUCCH/PUSCH/SRS may be on serving cell(s) and cell(s) with PCI different from serving cell(s), and restricted symbols may partially or fully overlap with UL symbols

If UE supports [multiDCI-InterCellMultiTRP-TwoTA-r18 or multiDCI-IntraCellMultiTRP-TwoTA-r18 and rxTimingDiff-r18],

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on the symbols configured for L1-RSRP measurement which are overlapped or partially overlapped with SSB symbols configured for L1-RSRP measurement

<Unchanged Text Skipped>