3GPP TSG-RAN4 Meeting #117	R4-2520730

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.161 | CR | 0028 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.161 on NTN OTA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Apple, Keysight, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TRP_TRS_MIMO_OTA_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is CR to introduce NTN OTA into TS 38.161, based on the endorsed draft CR in R4-2513963. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NTN OTA test methodogloy is added in Annex D, as informative clause, including device type, use scenarios, operating bands, performance metric, UE configurations and test procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is not appicable for NTN OTA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.3, new Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.870: “Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)”.

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[5]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone".

[6]	3GPP TS 38.521-3: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[7]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment ".

[8]	3GPP TS 37.544: “Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) Over The Air (OTA) performance; Conformance testing “.

[9]	CTIA Certification™: “CTIA Certification Test Plan for Wireless Device Over-the-Air Performance, CTIA 01.71 Device Setup and Positioning Guidelines”, latest active version available at: https://ctiacertification.org/test-plans/

[10]	CTIA Certification™: “CTIA Certification Test Plan for Wireless Device Over-the-Air Performance, CTIA 01.72: Near-Field Phantoms”, latest active version available at: https://ctiacertification.org/test-plans/

[11]	3GPP TS 38.101-5: "NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[12]	3GPP TS 36.102: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access".

[13]	3GPP TS 38.521-5: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[14]	3GPP TS 36.521-4: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Satellite access Radio Frequency (RF) and performance Conformance Testing".

[15]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Browsing mode usage: This mode corresponds to “data” mode, the device is tested via hand-only phantoms.

Head-mounted mode usage: This mode corresponds to wearable device for head-mounted mode, the device (e.g., XR glass and head mounted display) is tested via specific Head phantoms for head-mounted devices.

Free Space mode: This mode is not specifically limited to “data” mode or “talk” mode, the device is tested via free space mode without phantoms.

Partial Radiated Power: The radiated power in a partial sphere by a device, integrated over partial directions of entire sphere surrounding the device. Partial Radiated Power integrated within different range of sphere will generate different metrics.

Partial Radiated Sensitivity: The minimum received radiated power level required by a device to maintain a specified communication quality (e.g., 95% maximum throughput), averaged over a partial sphere surrounding the device. Partial Radiated sensitivity integrated within different range of sphere will generate different metrics.

Primary mechanical mode: The mode that is most often used for a specific user scenario. Every terminal has at least one primary mechanical mode, if multiple modes are supported, different primary mechanical modes may be applicable for different user scenarios, e.g., different primary mechanical modes for Browsing mode usage and Talk mode usage for the same UE.

Talk mode usage: This mode corresponds to “talk” mode, the device is tested via head&hand phantoms.

Total Radiated Power: The total power radiated by a device, integrated over all directions and over the entire sphere surrounding the device.

Total Radiated Sensitivity: The minimum received radiated power level required by a device to maintain a specified communication quality (e.g., 95% maximum throughput), averaged over all spatial directions in a 3D sphere.

Wrist-worn mode usage: This mode corresponds to wearable device for wrist-worn mode, the device is tested via forearm phantoms.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

TRPaverage	The average measured total radiated power of low, mid and high channel. When hand phantom is involved, the average is performed with low, mid and high channel from both hand left and hand right.

TRSaverage	The average measured total radiated sensitivity of low, mid and high channel. When hand phantom is involved, the average is performed with low, mid and high channel from both hand left and hand right.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Anechoic Chamber

BHH	Beside Head and Hand

BHHL	Beside Head and Hand Left Side (Head and Hand Phantom)

BHHR	Beside Head and Hand Right Side (Head and Hand Phantom)

BS	Base Station

CA	Carrier Aggregation

CBW	Channel Bandwidth

CC	Component Carriers

DUT	Device Under Test

E-UTRA	Evolved UTRA

EIRP	Effective Isotropic Radiated Power

EIS	Effective Isotropic Sensitivity

EUT	Equipment Under Test

FR1	Frequency Range 1

FS	Free Space

HL	Hand Left (Hand Phantom Only)

HR	Hand Right (Hand Phantom Only)

MPR	Allowed maximum power reduction

NR	New Radio

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

OTA	Over The Air

P-MPR	Power Management Maximum Power Reduction

PRP	Partial Radiated Power

PRS	Partial Radiated Sensitivity

QZ	Quiet Zone

RAT	Radio Access Technology

RC	Reverberation Chamber

RedCap	Reduced Capability

RMC	Reference Measurement Channel

SA	Standalone

SCS	Subcarrier spacing

SS	System Simulator

TAA	Time-Averaging Algorithm

TAS	Tx Antenna Switching

TRP	Total Radiated Power

TRS	Total Radiated Sensitivity

TxD	Tx Diversity

UE	User Equipment

UL MIMO	Uplink Multiple Antenna transmission

==============Next change==============

Annex D (informative):
NTN OTA Test methodology

D.1	General

This clause specifies the test methodologies for NTN devices. The following sub-clauses are informative.

D.1.1	NTN Device types

NTN devices includes both NR-NTN and IoT-NTN UEs.

D.1.2	NTN UE use scenarios

For NTN testing of smartphones, the use scenarios are categorized as following which are based on UE declaration:

- 	Usage scenario 1: both head+hand mode and hand only mode

- 	Usage scenario 2: hand only mode targeted for specific UE orientation usage

- 	Usage scenario 3: hand only mode targeted for arbitrary UE orientation usage

Note:	For each supported NR NTN band, declare only one supported scenario. The declaration of scenario applicability shall be consistent with product manual usage scenario. The declared scenario shall be clearly stated in the test report.

D.1.3	NTN operation bands and test parameters

The detailed test parameters for each band for NR-NTN UE are defined in Table D.1.3-1 and Table D.1.3-2.

Table D.1.3-1: FR1 TRP measurement parameters for NR-NTN UE

| NR-NTN Band | CBW (MHz) | SCS (kHz) | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation (LCRB @ RBstart) | DL configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 5 | 15 | DFT-s-OFDM QPSK | Low | 396500 | 1982.5 | 434500 | 2172.5 | 12@6 | N/A |
|  |  |  |  | Mid | 399000 | 1995 | 437000 | 2185 |  |  |
|  |  |  |  | High | 401500 | 2007.5 | 439500 | 2197.5 |  |  |
| n255 | 5 | 15 | DFT-s-OFDM QPSK | Low | 325800 | 1629 | 305500 | 1527.5 | 12@6 | N/A |
|  |  |  |  | Mid | 328700 | 1643.5 | 308400 | 1542 |  |  |
|  |  |  |  | High | 331600 | 1658 | 311300 | 1556.5 |  |  |
| n254 | 5 | 15 | DFT-s-OFDM QPSK | Low | 322500 | 1612.5 | 497200 | 2486 | 12@6 | N/A |
|  |  |  |  | Mid | 323700 | 1618.5 | 498400 | 2492 |  |  |
|  |  |  |  | High | 324800 | 1624 | 499500 | 2497.5 |  |  |

Table D.1.3-2: FR1 TRS measurement parameters for NR-NTN UE

| NR-NTN Band | CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation (LCRB @ RBstart) | DL Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 5 | 15 | CP-OFDM QPSK | DFT-s-OFDM QPSK | Low | 396500 | 1982.5 | 434500 | 2172.5 | 25@0 | 25@0 |
|  |  |  |  |  | Mid | 399000 | 1995 | 437000 | 2185 |  |  |
|  |  |  |  |  | High | 401500 | 2007.5 | 439500 | 2197.5 |  |  |
| n255 | 5 | 15 | CP-OFDM QPSK | DFT-s-OFDM QPSK | Low | 325800 | 1629 | 305500 | 1527.5 | 25@0 | 25@0 |
|  |  |  |  |  | Mid | 328700 | 1643.5 | 308400 | 1542 |  |  |
|  |  |  |  |  | High | 331600 | 1658 | 311300 | 1556.5 |  |  |
| n254 | 5 | 15 | CP-OFDM QPSK | DFT-s-OFDM QPSK | Low | 322500 | 1612.5 | 497200 | 2486 | 25@0 | 25@0 |
|  |  |  |  |  | Mid | 323700 | 1618.5 | 498400 | 2492 |  |  |
|  |  |  |  |  | High | 324800 | 1624 | 499500 | 2497.5 |  |  |

The detailed test parameters for each band for UE category M1 NTN are defined in Table D.1.3-3 and Table D.1.3-4

Table D.1.3-3: TRP measurement parameters for UE category M1 NTN

| NTN Band | CBW (MHz) | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation HD-FDD (LCRB @ RBstart) | DL configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 1.4 | QPSK | Low | 261851 | 1980.7 | 229083 | 2170.7 | 1@0 | N/A |
|  |  |  | Mid | 261994 | 1995 | 229226 | 2185 |  |  |
|  |  |  | High | 262137 | 2009.3 | 229369 | 2199.3 |  |  |
| 255 | 1.4 | QPSK | Low | 261511 | 1627.2 | 228743 | 1525.7 | 1@0 | N/A |
|  |  |  | Mid | 261674 | 1643.5 | 228906 | 1542 |  |  |
|  |  |  | High | 261837 | 1659.8 | 229069 | 1558.3 |  |  |
| 254 | 1.4 | QPSK | Low | 261346 | 1610.7 | 228578 | 2484.2 | 1@0 | N/A |
|  |  |  | Mid | 261422 | 1618.3 | 228654 | 2491.8 |  |  |
|  |  |  | High | 261497 | 1625.8 | 228729 | 2499.3 |  |  |

Table D.1.3-4: TRS measurement parameters for UE category M1 NTN

| NTN Band | CBW (MHz) | DL modulation | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | UL RB Allocation HD-FDD (LCRB @ RBstart) | DL Configuration FDD (LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 1.4 | QPSK | QPSK | Low | 261851 | 1980.7 | 229083 | 2170.7 | 6@0 | 4@0 |
|  |  |  |  | Mid | 261994 | 1995 | 229226 | 2185 |  |  |
|  |  |  |  | High | 262137 | 2009.3 | 229369 | 2199.3 |  |  |
| 255 | 1.4 | QPSK | QPSK | Low | 261511 | 1627.2 | 228743 | 1525.7 | 6@0 | 4@0 |
|  |  |  |  | Mid | 261674 | 1643.5 | 228906 | 1542 |  |  |
|  |  |  |  | High | 261837 | 1659.8 | 229069 | 1558.3 |  |  |
| 254 | 1.4 | QPSK | QPSK | Low | 261346 | 1610.7 | 228578 | 2484.2 | 6@0 | 4@0 |
|  |  |  |  | Mid | 261422 | 1618.3 | 228654 | 2491.8 |  |  |
|  |  |  |  | High | 261497 | 1625.8 | 228729 | 2499.3 |  |  |

The detailed test parameters for each band for NB-IoT NTN are defined in Table D.1.3-5 and Table D.1.3-6.

Table D.1.3-5: TRP measurement parameters for NB-IoT NTN UE

| NTN Band | CBW (MHz) | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | MUL/ MDL | UL Configuration Ntones | DL Configuration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 0.2 | QPSK | Low | 261845 | 1980.1 | 229077 | 2170.1 | 0 | 1@0 | N/A |
|  |  |  | Mid | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  | High | 262143 | 2009.9 | 229375 | 2199.9 | 0 | 1@11 |  |
| 255 | 0.2 | QPSK | Low | 261505 | 1626.6 | 228737 | 1525.1 | 0 | 1@0 | N/A |
|  |  |  | Mid | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  | High | 261843 | 1660.4 | 229075 | 1558.9 | 0 | 1@11 |  |
| 254 | 0.2 | QPSK | Low | 261340 | 1610.1 | 228572 | 2483.6 | 0 | 1@0 | N/A |
|  |  |  | Mid | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  | High | 261503 | 1626.4 | 228735 | 2499.9 | 0 | 1@11 |  |

Table D.1.3-6: TRS measurement parameters for NB-IoT NTN UE

| NTN Band | CBW (MHz) | DL modulation | UL modulation | Range | UL Carrier centre [ARFCN] | UL Carrier Center (MHz) | DL Carrier centre [ARFCN] | DL Carrier Center (MHz) | MUL/ MDL | UL Configuration Ntones | DL Configuration Subcarriers |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 | 0.2 | QPSK | BPSK | Low | 261845 | 1980.1 | 229077 | 2170.1 | 0 | 1@0 | 12 |
|  |  |  |  | Mid | N/A | N/A | N/A | N/A | N/A |  |  |
|  |  |  |  | High | 262143 | 2009.9 | 229375 | 2199.9 | 0 |  |  |
| 255 | 0.2 | QPSK | BPSK | Low | 261505 | 1626.6 | 228737 | 1525.1 | 0 | 1@0 | 12 |
|  |  |  |  | Mid | N/A | N/A | N/A | N/A | N/A |  |  |
|  |  |  |  | High | 261843 | 1660.4 | 229075 | 1558.9 | 0 |  |  |
| 254 | 0.2 | QPSK | BPSK | Low | 261340 | 1610.1 | 228572 | 2483.6 | 0 | 1@0 | 12 |
|  |  |  |  | Mid | N/A | N/A | N/A | N/A | N/A |  |  |
|  |  |  |  | High | 261503 | 1626.4 | 228735 | 2499.9 | 0 |  |  |

D.2	NTN performance metrics

D.2.1	General

This clause specifies the performance metrics for NTN OTA, including total radiated metric (TRP/TRS) and partial radiated metric (PRP/PRS) in Clauses D.2.2 through D.2.5, with the applicability of performance metric defined in table below.

Table D.2.1-1: Applicability of TRP/TRS and PRP/PRS metrics for NTN

| Clause | TX/RX | TRP/TRS or PRP/PRS definition | Applicability |
| --- | --- | --- | --- |
| D.2.2 | TX | TRP | Applicable to NTN scenario 1 and 3 |
| D.2.4.1 | TX | UPRP±60≅12a4cutTX,4+i=03cutTX,i | Applicable to NTN scenario 2 (if required by Table D.3.1-1) |
| D.2.4.2 | TX | UHPRP≅12a6cutTX,6+i=05cutTX,i | Applicable to NTN scenario 2 (if required by Table D.3.1-1) |
| D.2.3 | RX | TRS | Applicable to NTN scenario 1 and 3 |
| D.2.5.1 | RX | UPRS±60=21cutRX,0+cutRX,1+b2cutRX,2 | Applicable to NTN scenario 2 (if required by Table D.3.1-1) |
| D.2.5.2 | RX | UHPRS=21cutRX,0+cutRX,1+cutRX,2+b3cutRX,3 | Applicable to NTN scenario 2 (if required by Table D.3.1-1) |
| where cutTX,i=1Mj=0M-1EiRPθθi,ϕj+EiRPϕθi,ϕj⋅Wθi cutRX,i=1Mj=0M-11EISθ(θi,ϕj)+1EISϕ(θi,ϕj)⋅W(θi)and Wθi are the quadrature weights of either the Clenshaw-Curtis (CC) weights or the traditional sin(q) weights, and ak=1-cosk*15°-i=0k-1W(θi)W(θk) and bk=1-cosk*30°-i=0k-1WθiWθk are the coefficients for the partial segments. |  |  |  |

D.2.2	Total Radiated Power (TRP)

The definition of TRP is identical with that in A.3.5.1.

D.2.3	Total Radiated Sensitivity (TRS)

The definition of TRS is identical with that in A.3.5.2.

D.2.4	Partial Radiated Power (PRP)

D.2.4.1	Upper Partial Radiated Power Integrated over ±60° Sector (UPRP±60)

The UPRP±60 metric is defined as the power radiated with EIRP for the partial surface integrated from q=0° to q=60° in the upper hemisphere. This metric with the partial surface visualized in Figure D.2.4.1-1 can be expressed mathematically as

UPRP±60≅12a4cutTX,4+i=03cutTX,i

with

a4=1-cos60°-i=03W(θi)W(θ4)≈0.5126,  &sin(q) quadrature0.4875,  &Clenshaw-Curtis quadrature

Figure D.2.4.1-1: Visualization of partial surface with ±60° angular width for classical TRP grid.

D.2.4.2	Upper Hemisphere Partial Radiated Power (UHPRP)

The UHPRP metric, which could also be referred to as UPRP±90, is defined as the radiated power for the partial surface in the upper hemisphere. This metric with the partial surface visualized in Figure D.2.4.2-1 can be expressed mathematically as

UHPRP≅12a6cutTX,6+i=05cutTX,iwhere

cutTX,i=1Mj=0M-1EiRPθθi,ϕj+EiRPϕθi,ϕj⋅Wθi

with

a6=1-cos90°-i=05W(θi)W(θ6)≈0.5128,  &sin(q) quadrature0.5000,  &Clenshaw-Curtis quadrature

for both Clenshaw-Curtis or sin(q) quadratures whose weights are expressed using Wθi, defined in Clause A.3.5.1.

Figure D.2.4.2-1: Visualization of upper hemisphere partial surface with ±90° angular width for classical TRP grids.

D.2.5	Partial Radiated Sensitivity (PRS)

D.2.5.1	Upper Partial Radiated Sensitivity Integrated over ±60° Sector (UPRS±60)

The UPRS±60 metric is defined as the sensitivity with EIS for the partial surface integrated from q=0° to q=60° in the upper hemisphere. This metric with the partial surface visualized in Figure D.2.5.1-1 can be expressed mathematically as

UPRS±60=21cutRX,0+cutRX,1+b2cutRX,2

with

b2=1-cos60°-i=01W(θi)W(θ2)≈0.5253,  &sin(q) quadrature0.4757,  &CC quadrature

Figure D.2.5.1-1: Visualization of partial surface with ±60° angular width for classical TRS grid.

D.2.5.2	Upper Hemisphere Partial Radiated Sensitivity (UHPRS)

The UHPRS metric, which could also be referred to as UPRS±90, is defined as radiated sensitivity for the partial surface in the upper hemisphere. This metric with the partial surface visualized in Figure 5.3.1.7-1 can be expressed mathematically as

UHPRS=21cutRX,0+cutRX,1+cutRX,2+b3cutRX,3where

cutRX,i=1Mj=0M-11EISθ(θi,ϕj)+1EISϕ(θi,ϕj)⋅W(θi)

with

b3=1-cos90°-i=02W(θi)W(θ3)≈0.5438,  &sin(q) quadrature0.5000,  &CC quadrature

for both Clenshaw-Curtis or sin(q) quadratures whose weights are expressed using Wθi, defined in Clause A.3.5.2.

Figure 5.3.1.7-1: Visualization of upper hemisphere partial surface with ±90° angular width for classical TRS grids.

D.3	Test Procedure for NTN UE

D.3.1	General

The TRP definition and measurement of NTN is identical with TN UEs specified in A.3.3.2 with exceptions of UE configuration defined in sub-clauses of Annex D.3.

The Partial Radiated Power (PRP) of the DUT is measured by sampling the radiated transmit power of the DUT with partial sphere of three-dimensional scan at various locations surrounding the device, which is defined in Clause D.2. The measurement is performed with a constant sampling step of 15 degrees, in both theta () and phi () axes. All of the measured power values will be integrated to PRP, as defined in Clause D.2. The applicability of different PRP metric is specified in Table D.2.1-1.

The TRS definition and measurement of NTN is identical with TN UEs specified in A.3.3.3 with exceptions of UE configuration defined in sub-clauses of Annex D.3.

The Partial Radiated Sensitivity (PRS) of the DUT is measured by sampling the radiated sensitivity power of the DUT with partial sphere of three-dimensional scan at various locations surrounding the device, which is defined in Clause D.2. The measurement is performed with a constant sampling step of 30 degrees, in both theta () and phi () axes. All of the measured sensitivity power values will be integrated to PRS, as defined in Clause D.2. The applicability of different PRS metric is specified in Table D.2.1-1.

For TRP/PRP and TRS/PRS measurement, the evaluations shall be performed at maximum transmit power.

For TRS and PRS measurement, no specific setting is needed for Rx antennas. By default, the maximum number of Rx antennas supported at each band should be enabled during the TRS and PRS test.

The applicability of NTN usage scenarios and corresponding test procedure is summarized in Table D.3.1-1.

Table D.3.1-1: Applicability of NTN usage scenarios and corresponding test procedure

| NTN Scenario / description | OTA performance metric for NTN | Applicability of mechanical modes for NTN | Additional parameters declared by the OEM for NTN testing |
| --- | --- | --- | --- |
| Scenario 1 in Clause D.1.2 both head+hand mode and hand only mode | Full sphere defined in Table D.3.1-1 TRP/TRS | Talk mode (head+hand phantom) and browsing mode (hand phantom) | None |
| Scenario 2 in Clause D.1.2 hand only mode targeted for specific UE orientation usage | Partial Sphere defined in Table D.3.1-1 UHRP or UPRP, depending on additional parameters declared by the OEM (if applicable) | Browsing mode (hand phantom) | DUT tilt for initial positioning is declared as one of the following values: θT ∈ [-90, -75, -60, -45, -30, -15, 0, 15, 30, 45, 60, 75, 90]o |
|  |  |  | Elevation range: θ ∈ [-θR, θR] o, Option1: θR =90 is the only metric Option 2: θR can be declared as one of the following values: θR ∈ [90, 60]o and single set requirement will be defined. |
| Scenario 3 in Clause D.1.2 hand only mode targeted for arbitrary UE orientation usage | Full sphere defined in Table D.3.1-1 TRP/TRS | Browsing mode (hand phantom) | None |

D.3.2	TRP and PRP test procedure for NR NTN

D.3.2.1	UE configuration

NR-NTN TRP and PRP radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

The NR NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the TRP and PRP test.

For NR NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 38.521-5 [13], section 6.2.1 (UE maximum output power) using the default settings specified in TS 38.521-5 [13] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table D.1.3-1 Clause D.1.3.

D.3.2.2	Test Procedure

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For NR-NTN UE support usage scenario 1 or usage scenario 3 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2.1.4.2 of TS 38.521-5 [13] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the TRP performance metric using the TRP integration approaches outlined in Clause A.3.5.1.

For NR-NTN UE declaring support of usage scenario 2 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the hand only positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2.1.4.2 of TS 38.521-5 [13] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point of upper Hemisphere and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the partial sphere performance metric using the approaches outlined in Clause D.2.4. The proper metric for scenario 2 is selected based on Table D.3.1-1.

D.3.3	TRP and PRP test procedure for IoT-NTN

D.3.3.1	UE configuration

IoT-NTN TRP and PRP radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

The IoT-NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the TRP and TRS test. For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

For cat M1 NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [14], section 6.2.A.1 (UE maximum output power for category M1) using the default settings specified in TS 36.521-4 [14] and TS 36.508 [15] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table D.1.3-3 Clause D.1.

For NB-IoT NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [14], section 6.2.B.1 (UE maximum output power for category NB1 and NB2) using the default settings specified in TS 36.521-4 [14] and TS 36.508 [15] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table D.1.3-3 Clause D.1.3.

D.3.3.2	Test Procedure for cat M1 NTN

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For cat M1 NTN UE support usage scenario 1 or usage scenario 3 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2A.1.4.2 of TS 36.521-4 [14] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the TRP performance metric using the TRP integration approaches outlined in Clause A.3.5.1.

For cat M1 NTN UE declaring support of usage scenario 2 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the hand only positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2A.1.4.2 of TS 36.521-4 [14] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point of upper hemisphere and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the partial sphere performance metric using the approaches outlined in Clause D.2.4. The proper metric for scenario 2 is selected based on Table D.3.1-1.

D.3.3.3	Test Procedure for NB-IoT NTN

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For NB-IoT NTN UE support usage scenario 1 or usage scenario 3 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2B.1.4.2 of TS 36.521-4 [14] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the TRP performance metric using the TRP integration approaches outlined in Clause A.3.5.1.

For NB-IoT NTN UE declaring support of usage scenario 2 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the hand only positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the link antenna following steps 1 and 2 in section 6.2B.1.4.2 of TS 36.521-4 [14] and ensure the DUT transmits with its maximum power.

3)	Measure the power at each measurement point of upper hemisphere and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

4)	Calculate the partial sphere performance metric using the approaches outlined in Clause D.2.4. The proper metric for scenario 2 is selected based on Table D.3.1-1.

D.3.4	TRS and PRS test procedure for NR NTN

D.3.4.1	UE configuration

NR-NTN TRS and PRS radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

The NR NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the OTA test.

For NR NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 38.521-5 [13], section 7.3 (Reference sensitivity) using the default settings specified in TS 38.521-5 [13] and TS 38.508-1 [7] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table D.1.3-2 Clause D.1.3.

D.3.4.2	Test Procedure

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For NR-NTN UE support usage scenario 1 or usage scenario 3 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 7 in section 7.3.2.4.2, TS 38.521-5 [13], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR NTN sensitivity level.

4)	Calculate the TRS performance metric using the TRS integration approaches outlined in Clause A.3.5.2.

For NR-NTN UE declaring support of usage scenario 2 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 7 in section 7.3.2.4.2, TS 38.521-5 [13], with the following exception: determine each EIS at each measurement point of upper hemisphere, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR NTN sensitivity level.

4)  Calculate the partial sphere performance metric using the approaches outlined in Clause D.2.4. The proper metric for scenario 2 is selected based on Table D.3.1-1.

D.3.5	TRS and PRS test procedure for IoT-NTN

D.3.5.1	UE configuration

IoT-NTN TRS and PRS radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions OFF should be based on manufacturer declaration, if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

The IoT-NTN SS should send continuous uplink power control “up” commands in every uplink scheduling information to the DUT to ensure the DUT’s transmitter is at maximum output power during the TRS and PRS test. For NTN OTA testing, the Doppler conditions should be set to zero and delay conditions are set to constant for all types of satellites.

For cat M1 NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [14], section 7.3.A (Reference sensitivity power level for UE category M1) using the default settings specified in TS 36.521-4 [14] and TS 36.508 [15] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table 4.3.3-8 Clause 4.3.

For NB-IoT NTN, the NTN System Simulator (SS) and DUT shall be configured per TS 36.521-4 [14], section 7.3.B (Reference sensitivity power level for UE category NB1 and NB2) using the default settings specified in TS 36.521-4 [14] and TS 36.508 [15] as applicable. The measurement should be carried out based on the detailed test parameters of each band, as defined in Table D.1.3-4 Clause D.1.3.

D.3.5.2	Test Procedure for cat M1 NTN

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For cat M1 NTN UE support usage scenario 1 or usage scenario 3 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 4 in section 7.3A.4.2, TS 36.521-4 [14], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the M1 NTN sensitivity level.

4)	Calculate the TRS performance metric using the TRS integration approaches outlined in Clause A.3.5.2.

For cat M1 NTN UE declaring support of usage scenario 2 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 4 in section 7.3A.4.2, TS 36.521-4 [14], with the following exception: determine each EIS at each measurement point of upper hemisphere, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the M1 NTN sensitivity level.

4)	Calculate the partial sphere performance metric using the approaches outlined in Clause D.2.4. The proper metric for scenario 2 is selected based on Table D.3.1-1.

D.3.5.3	Test Procedure for NB-IoT NTN

It is note that the test procedure and performance metric is same for a handheld UE support GSO and/or NGSO.

For NB-IoT NTN UE support usage scenario 1 or usage scenario 3 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 3 in section 7.3B.4.2, TS 36.521-4 [14], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NB-IoT NTN sensitivity level.

4)	Calculate the TRS performance metric using the TRS integration approaches outlined in Clause A.3.5.2.

For NB-IoT NTN UE declaring support of usage scenario 2 defined in Clause D.1.2, the measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the corresponding positioning guideline defined in Clause B.3.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 3 in section 7.3B.4.2, TS 36.521-4 [14], with the following exception: determine each EIS at each measurement point of upper hemisphere, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NB-IoT NTN sensitivity level.

4)	Calculate the partial sphere performance metric using the approaches outlined in Clause D.2.4. The proper metric for scenario 2 is selected based on Table D.3.1-1.

D.4	NTN MU assessment

D.4.1	General

This Clause define the preliminary MU assessment for NTN OTA.

D.4.2	MU for TRP/PRP test

## The preliminary MU assessment is same as that in Table A.4.2-1 and Table A.4.2-2, Clause A.4.2.

D.4.3	MU for TRS/PRS test

## The preliminary MU assessment is same as that in Table A.4.2-3 and Table A.4.2-4, Clause A.4.2.

==============End of change==============