3GPP TSG-RAN WG4 Meeting #117	R4-2522298

# Dallas, USA,  17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | 0044 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for the simulation assumptions for NTN L-band and S-band power back-off results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-19 introduced supports for high-power classes for NTN, whereupon quite extensive simulations were run for different bands and NS flags to determine final A-MPR values. However, while power back-off results were captured in this TR, the corresponding simulation assumptions were not captured at all or remain uncomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corresponding clarifications are introduced into sub-clauses with simulation assumptions to make it clearer which assumptions and parameters were used while simulating power back-off results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will remain unclear which simulation assumptions were used for the power back-off simulations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.1.1.1, F.2.1.1.2.1, F.2.1.1.5.1, G.2.1.1.1.1, G.2.1.1.4.1, G.4.1.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### F.1.1.1.1.1	Simulation assumptions

The power back-off simulations are performed for the PC3 power class and the 3MHz channel with the following common assumptions:

-	Tx power: PC3 (+23dBm)

-	Calibration: 1dB MPR with 30dBc ACLR and DFT-s-OFDM QPSK 20MHz 100RB0

-	Carrier leakage: -28dBc

-	Image: -28dBc

-	CIM3: -60dBc

-	CIM5: -70dBc

-	LO placement: always in the centre of the carrier

-	Additional requirements: -50dBm/MHz to protect the 2010 ≤ f ≤ 2025MHz frequency range

The power back-off simulations were conducted for the following center frequencies:

-	the right-most channel with the 5MHz offset, as "guard" band, from the upper edge of the band (Fc=2003.5MHz)

-	the upper channel with the 6.5MHz offset from the upper edge (Fc=2002MHz)

-	the upper channel with the 8MHz offset from the upper edge (Fc=2000.5MHz)

********** NEXT CHANGED SECTION **********

#### F.2.1.1.2	Company B (Qualcomm/R4-2511400)

##### F.2.1.1.2.1	Simulation assumptions

The following simulation assumptions are used:

-	Tx power: PC3 (23dBm)

-	PA calibrated to 1dB MPR with 30dBc ACLR with DFT-s-OFDM QPSK 100 RB waveform

-	-28 dBc IQ-image

-	-28 dBc LO-leakage

-	-60 dBc CIM3 and -70 dBc CIM5

##### F.2.1.1.2.2	Simulation results

********** NEXT CHANGED SECTION **********

#### F.2.1.1.5	Company E (R4-2509537/Apple)

##### F.2.1.1.5.1	Simulation assumptions

PC2 power class simulations are conducted with the common assumptions as presented below:

-	Tx power: PC2 (+26dBm)

-	Calibration: 1dB MPR with 31dBc ACLR and DFT-s-OFDM QPSK 20MHz 100RB0

-	Carrier leakage: -28dBc

-	Image: -28dBc

-	CIM3: -60dBc

-	CIM5: -70dBc

-	LO placement: always in the centre of the carrier

-	Regulations: Requirements from NS_24

For the sake of completeness, the channel bandwidts and the corresponding center frequencies are summarised below:

-	5MHz: Fc=2002.5MHz, Fc=1997.5MHz, Fc=1992.5MHz

-	10MHz: Fc=2000MHz, Fc=1995MHz, Fc=1985MHz

-	15MHz: Fc=1997.5MHz, Fc=1987.5MHz

-	20MHz: Fc=1995MHz, Fc=1990MHz

********** NEXT CHANGED SECTION **********

#### G.2.1.1.1	Company A (Qualcomm/R4-2511400)

##### G.2.1.1.1.1	Simulation assumptions

The following simulation assumptions are used:

-	Tx power: PC2 (26dBm)

-	PA calibrated to 1dB MPR with 31dBc ACLR with DFT-s-OFDM QPSK 100 RB waveform

-	-28 dBc IQ-image

-	-28 dBc LO-leakage

-	-60 dBc CIM3 and -70 dBc CIM5

##### G.2.1.1.1.2	Simulation results

********** NEXT CHANGED SECTION **********

#### G.2.1.1.4	Company D (Apple/R4-2509536)

##### G.2.1.1.4.1	Simulation assumptions

Simulation assumptions are as follows:

-	Tx power: PC2 (+26dBm)

-	Calibration: 1dB MPR with 31dBc ACLR and DFT-s-OFDM QPSK 20MHz 100RB0

-	Carrier leakage: -28dBc

-	Image: -28dBc

-	CIM3: -60dBc

-	CIM5: -70dBc

-	LO placement: always in the centre of the carrier

-	Regulations: Requirements from NS_02N

-	10dB filter rejection is assumed

For the sake of brevity, simulations are considered for the 5, 10, 15 and 20MHz channel bandwidths, whereupon the corresponding channel is always located at the lower edge of the n255 band:

-	5MHz: Fc=1629MHz

-	10MHz: Fc=1631.5MHz

-	15MHz: Fc=1634MHz

-	20MHz: Fc=1636.5MHz

********** NEXT CHANGED SECTION **********

#### G.4.1.1.1	Company A (Qualcomm/R4-2511400)

##### G.4.1.1.1.1	Simulation assumptions

The following simulation assumptions are used:

-	Tx power: PC1 (31dBm) with 4TX

-	PA calibrated to 1dB MPR with 34dBc ACLR with DFT-s-OFDM QPSK 100 RB waveform

-	-28 dBc IQ-image

-	-28 dBc LO-leakage

-	-60 dBc CIM3 and -70 dBc CIM5

##### G.4.1.1.1.2	Simulation results