3GPP TSG-WG2 Meeting # 132	R2-2509445

# Dallas, USA, 17th - 21st 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0568 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SSR Provider ID [SSR-ProviderID] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce an SSR Provider ID attribute for SSR assistance data components. RTCM [30] has introduced an SSR Provider ID into their SSR specifications. To introduce an SSR Provider ID would ensure to avoid the incorrect combination of SSR components from different SSR providers. The purpose of the SSR Provider ID is to ensure that data from adjacent regions are not incorrectly combined. The SSR provider ID is a globally unique number assigned by RTCM to identify an SSR provider [30]. This field is 16 bits, i.e Integer value 0 to 65535 SSR Solution ID indicates different SSR services of one SSR provider. This is used to distinguish between different correction solutions or versions from the same provider. This field is 4 bits. Integer value 0 to 15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An IE SSR-ProviderInfo has been added to SSR Assistance Data components to align with RTCM Specification [30]. The IE contains the two fields SSR Provider ID and Solution ID as specified in RTCM Specification [30]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may not understand that data components should not be combined if they have different SSR Provider and thus this can degrage the positioning performance (e.g. positioning accuracy is degraded). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2, 6.5.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Changes

#### 6.5.2.2	GNSS Assistance Data Elements

#### –	GNSS-ReferenceTime

#### <Skip Unmodified Changes>

#### –	GNSS-SSR-CorrectionPoints

The IE GNSS-SSR-CorrectionPoints is used by the location server to provide a list of correction point coordinates or an array of correction points ("grid") for which the GNSS-SSR-GriddedCorrection are valid.

-- ASN1START

GNSS-SSR-CorrectionPoints-r16 ::= SEQUENCE {

correctionPointSetID-r16			INTEGER (0..16383),

correctionPoints-r16				CHOICE {

listOfCorrectionPoints-r16			GNSS-SSR-ListOfCorrectionPoints-r16,

arrayOfCorrectionPoints-r16			GNSS-SSR-ArrayOfCorrectionPoints-r16

},

...,

[[

ssr-ProviderInfo-r19				GNSS-SSR-ProviderInfo-r19			OPTIONAL 	-- Need OR

]]

}

GNSS-SSR-ListOfCorrectionPoints-r16 ::= SEQUENCE {

referencePointLatitude-r16		INTEGER (-16384..16383),

referencePointLongitude-r16		INTEGER (-32768..32767),

relativeLocationsList-r16		SEQUENCE (SIZE (0..63)) OF RelativeLocationElement-r16,

...

}

RelativeLocationElement-r16 ::= SEQUENCE {

deltaLatitude-r16				INTEGER (-512..511),

deltaLongitude-r16				INTEGER (-1024..1023),

...

}

GNSS-SSR-ArrayOfCorrectionPoints-r16 ::=SEQUENCE {

referencePointLatitude-r16			INTEGER (-16384..16383),

referencePointLongitude-r16			INTEGER (-32768..32767),

numberOfStepsLatitude-r16			INTEGER (0..63),

numberOfStepsLongitude-r16			INTEGER (0..63),

stepOfLatitude-r16					INTEGER (1..511),

stepOfLongitude-r16					INTEGER (1..1023),

bitmaskOfGrids-r16					BIT STRING (SIZE(64))				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

GNSS-SSR-CorrectionPoints field descriptions

correctionPointSetID This field provides the ID of the Atmospheric Correction Point set. It is a regionally unique arbitrary number that is used by the UE to ensure that the atmospheric corrections are being applied to the correct set of points.

referencePointLatitude This field specifies the latitude for the reference point, expressed in the range of -90° , +90°, coded as a number between -214 and 214-1, coded in 2's complement binary on 15 bits. The relation between the latitude X in the range [-90°, 90°] and the coded number N is: where denotes the greatest integer less than or equal to x (floor operator). For the listOfCorrectionPoints, the reference point defines the 1st correction point location. For the arrayOfCorrectionPoints, the reference point defines the northwest corner of the correction point array.

referencePointLongitude This field specifies the longitude for the reference point, expressed in the range -180°, +180°, coded as a number between -215 and 215-1, coded in 2's complement binary on 16 bits. The relation between the longitude X in the range [-180°, 180°) and the coded number N is: For the listOfCorrectionPoints, the reference point defines the 1st correction point location. For the arrayOfCorrectionPoints, the reference point defines the northwest corner of the correction point array.

relativeLocationsList This field specifies the 2nd, 3rd, …, 64th correction point location.

deltaLatitude This field specifies the delta value in latitude of this correction point location relative to the previous point on the list or the reference point in the case of the first additional point, defined as "correction point location" minus "previous correction point location" in units of 0.01 degrees.

deltaLongitude This field specifies the delta value in longitude of this correction point location relative to the previous point on the list or the reference point in the case of the first additional point, defined as "correction point location" minus "previous correction point location" in units of 0.01 degrees.

numberOfStepsLatitude, numberOfStepsLongitude These fields specify the number of steps for latitude and longitude direction respectively.

stepOfLatitude, stepOfLongitude These fields specify the spacing of the correction points for latitude and longitude respectively. The unit and scale factor is 0.01 degrees.

bitmaskOfGrids This field specifies the availability of correction data at the correction points in the array. If a specific bit is enabled (set to '1'), the correction is available. Only the first (numberOfStepsLatitude + 1)×(numberOfStepsLongitude + 1) bits are used, the remainder are set to '0'. Starting with the northwest corner of the array (top left on a north oriented map) the correction points are enumerated with row precedence – first row west to east, second row west to east, until last row west to east – ending with the southeast corner of the array. If the field is omitted all correction points are used and none omitted.

ssr-ProviderInfo This field provides the SSR Provider Information.

#### <Skip Unmodified Changes>

#### –	GNSS-SSR-IOD-Update

The IE GNSS-SSR-IOD-Update is used to indicate the Issue of Data (IOD) values of other IEs that are infrequently updated. The IEs with a matching IOD are linked and valid while the GNSS-SSR-IOD-Update is valid, i.e. during the GNSS-SSR-IOD-Update and ssrUpdateInterval and with respect to the iod-ssr.

-- ASN1START

GNSS-SSR-IOD-Update-r18 ::= SEQUENCE {

epochTime-r18						GNSS-SystemTime,

ssrUpdateInterval-r18				INTEGER (0..15),

iod-ssr-r18							INTEGER (0..15),

iod-ssr-PCVResiduals-r18			INTEGER (0..64),

...,

[[

ssr-ProviderInfo-r19				GNSS-SSR-ProviderInfo-r19			OPTIONAL 	-- Need OR

]]

}

-- ASN1STOP

GNSS-SSR-IOD-Update field descriptions

epochTime This field specifies the epoch time of validity of the information elements referenced via IOD values from this IE. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

iod-ssr-PCVResiduals This field specifies the Issue of Data for the SSR Satellite PCV Residuals that is valid while the GNSS-SSR-IOD-Update is valid.

ssr-ProviderInfo This field provides the SSR Provider Information.

#### <Skip Unmodified Changes>

#### –	GNSS-SSR-OrbitCorrections

The IE GNSS-SSR-OrbitCorrections is used by the location server to provide radial, along-track and cross-track orbit corrections together with integrity information. The target device may use the SSR-OrbitCorrectionList to compute a satellite position correction to be combined with the satellite position calculated from broadcast ephemeris.

The parameters provided in IE GNSS-SSR-OrbitCorrections – except for ORBIT-IntegrityParameters and SSR-IntegrityOrbitBounds –  are used as specified for SSR Orbit Messages (e.g., message type 1057 and 1063) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-OrbitCorrections-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

satelliteReferenceDatum-r15			ENUMERATED { itrf, regional, ... },

iod-ssr-r15							INTEGER (0..15),

ssr-OrbitCorrectionList-r15			SSR-OrbitCorrectionList-r15,

...,

[[

orbit-IntegrityParameters-r17	ORBIT-IntegrityParameters-r17	OPTIONAL -- Need OR

]],

[[

ssr-ProviderInfo-r19				GNSS-SSR-ProviderInfo-r19			OPTIONAL 	-- Need OR

]]

}

SSR-OrbitCorrectionList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-OrbitCorrectionSatelliteElement-r15

SSR-OrbitCorrectionSatelliteElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

iod-r15								BIT STRING (SIZE(11)),

delta-radial-r15					INTEGER (-2097152..2097151),

delta-AlongTrack-r15				INTEGER (-524288..524287),

delta-CrossTrack-r15				INTEGER (-524288..524287),

dot-delta-radial-r15				INTEGER (-1048576..1048575)		OPTIONAL, -- Need ON

dot-delta-AlongTrack-r15			INTEGER (-262144..262143)		OPTIONAL, -- Need ON

dot-delta-CrossTrack-r15			INTEGER (-262144..262143)		OPTIONAL, -- Need ON

...,

[[

ssr-IntegrityOrbitBounds-r17	SSR-IntegrityOrbitBounds-r17	OPTIONAL  -- Cond Integrity1

]]

}

ORBIT-IntegrityParameters-r17 ::= SEQUENCE {

probOnsetConstFault-r17					INTEGER (0..255),

meanConstFaultDuration-r17				INTEGER (1..3600),

probOnsetSatFault-r17					INTEGER (0..255),

meanSatFaultDuration-r17				INTEGER (1..3600),

orbitRangeErrorCorrelationTime-r17		INTEGER (0..255)			OPTIONAL, -- Need OR

orbitRangeRateErrorCorrelationTime-r17	INTEGER (0..255)			OPTIONAL, -- Cond Integrity2

...

}

SSR-IntegrityOrbitBounds-r17 ::= SEQUENCE {

meanOrbitError-r17				RAC-OrbitalErrorComponents-r17,

stdDevOrbitError-r17			RAC-OrbitalErrorComponents-r17,

meanOrbitRateError-r17			RAC-OrbitalErrorComponents-r17,

stdDevOrbitRateError-r17		RAC-OrbitalErrorComponents-r17,

...

}

RAC-OrbitalErrorComponents-r17	::= SEQUENCE {

radial-r17			INTEGER (0..255),

alongTrack-r17		INTEGER (0..255),

crossTrack-r17		INTEGER (0..255)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is mandatory present if ORBIT-IntegrityParameters is present; otherwise it is not present. |
| Integrity2 | The field is mandatory present if orbitRangeErrorCorrelationTime is present; otherwise it is not present. |

GNSS-SSR-OrbitCorrections field descriptions

epochTime This field specifies the epoch time of the orbit corrections. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation below. NOTE 1.

satelliteReferenceDatum This field specifies the satellite refence datum for the orbit corrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the satellite for which the orbit corrections are provided.

iod This field specifies the IOD value of the broadcast ephemeris for which the orbit corrections are valid (see IE GNSS-NavigationModel). NOTE 2, NOTE 4.

delta-radial This field specifies the radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.1 mm; range ±209.7151 m.

delta-AlongTrack This field specifies the along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

delta-CrossTrack This field specifies the cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

dot-delta-radial This field specifies the velocity of radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.001 mm/s; range ±1.048575 m/s.

dot-delta-AlongTrack This field specifies the velocity of along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

dot-delta-CrossTrack This field specifies the velocity of cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

probOnsetConstFault This field specifies the Probability of Onset of Constellation Fault per Time Unit where a constellation fault is at least two satellites being faulty simultaneously due to the same event. This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetConstFault and the range is 10-10.2 to 1 per hour.

meanConstFaultDuration This field specifies the Mean Constellation Fault Duration which is the mean duration between when a constellation fault occurs, and the user is alerted by IE GNSS-RealTimeIntegrity (or the integrity violation is over). Scale factor 1 s; range 1-3600 s.

probOnsetSatFault This field specifies the Probability of Onset of Satellite Fault per Time Unit which is the probability of occurrence of satellite error to exceed the residual error bound for more than the Time to Alert (TTA). This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetSatFault and the range is 10-10.2 to 1 per hour.

meanSatFaultDuration This field specifies the Mean Satellite Fault Duration which is the mean duration between when a satellite fault occurs, and the user is alerted by IE GNSS-RealTimeIntegrity (or the integrity violation is over). Scale factor 1 s; range 1-3,600 s.

orbitRangeErrorCorrelationTime This field specifies the Orbit Range Error Correlation Time which is the upper bound of the correlation time of the satellite residual range error due to orbit. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

orbitRangeRateErrorCorrelationTime This field specifies the Orbit Range Rate Error Correlation Time which is the upper bound of the correlation time of the satellite residual range rate error due to orbit. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanOrbitError This field specifies the Mean Orbit Error bound in satellite radial, along-track and cross-track coordinates, which are the mean values for a set of three overbounding models that bound the residual orbit error in satellite radial, along-track and cross-track directions. Each mean is calculated using: μ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

stdDevOrbitError This field specifies the Standard Deviation Orbit Error bound in satellite radial, along-track and cross-track coordinates, which are the standard deviation values for a set of three overbounding models that bound the residual orbit error in satellite radial, along-track and cross-track directions. Each standard deviation is calculated using: σ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

meanOrbitRateError This field specifies the Mean Orbit Rate Error in satellite radial, along-track and cross-track coordinates, which are the mean values for a set of three overbounding models that bound the residual satellite orbit rate error in satellite radial, along-track and cross-track directions. Scale factor 0.001 m/s; range 0-0.255 m/s.

stdDevOrbitRateError This field specifies the Standard Deviation Orbit Rate Error in satellite radial, along-track and cross-track coordinates, which are the standard deviation values for a set of three overbounding models that bound the residual satellite orbit rate error in satellite radial, along-track and cross-track directions. Scale factor 0.001 m/s; range 0-0.255 m/s.

ssr-ProviderInfo This field provides the SSR Provider Information.

NOTE 1:	The update intervals are aligned to the GPS time scale for all GNSSs in order to allow synchronous operation for multiple GNSS services. This means that the update intervals may not be aligned to the beginning of the day for another GNSS. Due to the leap seconds, this is generally the case for GLONASS.

NOTE 2:	In the cases that gnss-ID indicates 'gps', 'qzss', 'bds' or 'navic', the iod refers to the broadcast ephemeris of GPS L1 C/A, QZSS QZS-L1, BDS B1I/B3I or NavIC L5, respectively, in table GNSS to iod Bit String(11) relation in IE GNSS-NavigationModel.

NOTE 3:	The reference time t0 is epochTime + ½ × ssrUpdateInterval. The reference time t0 for ssrUpdateInterval '0' is epochTime.

NOTE 4:	In the cases that the GNSS-SSR-OrbitCorrection IE is contained within a GNSS-SSR-OrbitCorrectionsSet2 IE, then the reference ephemeris is denoted by the refEph field within the GNSS-SSR-OrbitCorrectionsSet2 IE, not the default as per NOTE 2. In the case gnss-ID indicates 'bds' and if refEph is set to b1c, the iod refers to the B-CNAV1 broadcast ephemeris (BDS B1C/B2a in table GNSS to iod Bit String (11) relation in IE GNSS-NavigationModel). In the case gnss-ID indicates 'navic' and if refEph is set to navicL1, the iod refers to the broadcast ephemeris of NavIC L1 in table GNSS to iod Bit String (11) relation in IE GNSS-NavigationModel.

Value of ssrUpdateInterval to SSR Update Interval relation

| Value of ssrUpdateInterval | SSR Update Interval |
| --- | --- |
| 0 | 1 second |
| 1 | 2 seconds |
| 2 | 5 seconds |
| 3 | 10 seconds |
| 4 | 15 seconds |
| 5 | 30 seconds |
| 6 | 60 seconds |
| 7 | 120 seconds |
| 8 | 240 seconds |
| 9 | 300 seconds |
| 10 | 600 seconds |
| 11 | 900 seconds |
| 12 | 1800 seconds |
| 13 | 3600 seconds |
| 14 | 7200 seconds |
| 15 | 10800 seconds |

#### –	GNSS-SSR-ClockCorrections

The IE GNSS-SSR-ClockCorrections is used by the location server to provide clock correction parameters together with integrity information. The target device may use the SSR-ClockCorrectionList to compute a clock correction to be applied to the broadcast satellite clock parameters, identified by iod of corresponding GNSS-SSR-OrbitCorrections.

The parameters provided in IE GNSS-SSR-ClockCorrections – except for CLOCK-IntegrityParameters and SSR-IntegrityClockBounds – are used as specified for SSR Clock Messages (e.g., message type 1058 and 1064) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-ClockCorrections-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

iod-ssr-r15							INTEGER (0..15),

ssr-ClockCorrectionList-r15			SSR-ClockCorrectionList-r15,

...,

[[

clock-IntegrityParameters-r17	CLOCK-IntegrityParameters-r17		OPTIONAL   -- Need OR

]],

[[

ssr-ProviderInfo-r19				GNSS-SSR-ProviderInfo-r19			OPTIONAL 	-- Need OR

]]

}

SSR-ClockCorrectionList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-ClockCorrectionSatelliteElement-r15

SSR-ClockCorrectionSatelliteElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

delta-Clock-C0-r15					INTEGER (-2097152..2097151),

delta-Clock-C1-r15					INTEGER (-1048576..1048575)			OPTIONAL, -- Need ON

delta-Clock-C2-r15					INTEGER (-67108864..67108863)		OPTIONAL, -- Need ON

...,

[[

ssr-IntegrityClockBounds-r17	SSR-IntegrityClockBounds-r17		OPTIONAL  -- Need OR

]]

}

CLOCK-IntegrityParameters-r17 ::= SEQUENCE {

clockRangeErrorCorrelationTime-r17		INTEGER (0..255),

clockRangeRateErrorCorrelationTime-r17	INTEGER (0..255),

...

}

SSR-IntegrityClockBounds-r17 ::= SEQUENCE {

meanClock-r17							INTEGER (0..255),

stdDevClock-r17							INTEGER (0..255),

meanClockRate-r17						INTEGER (0..255),

stdDevClockRate-r17						INTEGER (0..255),

...

}

-- ASN1STOP

GNSS-SSR-ClockCorrections field descriptions

epochTime This field specifies the epoch time of the clock corrections. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the satellite for which the clock corrections are provided.

delta-Clock-C0 This field specifies the C0 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.1 mm; range ±209.7151 m.

delta-Clock-C1 This field specifies the C1 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.001 mm/s; range ±1.048575 m/s.

delta-Clock-C2 This field specifies the C2 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.00002 mm/s2; range ±1.34217726 m/s2.

clockRangeErrorCorrelationTime This field specifies the Clock Range Error Correlation Time which is the upper bound of the correlation time of the satellite residual range error due to clock. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

clockRangeRateErrorCorrelationTime This field specifies the Clock Range Rate Error Correlation Time which is the upper bound of the correlation time of the satellite residual range rate error due to clock. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanClock This field specifies the Mean Clock Error bound which is the mean value for an overbounding model that bounds the residual clock error. The bound is meanClock + K * stdDevClock and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. The mean is calculated using: μ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

stdDevClock This field specifies the Standard Deviation Clock Error bound which is the standard deviation for an overbounding model that bounds the residual clock error. The standard deviation is calculated using: σ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

meanClockRate This field specifies the Mean Clock Rate Error bound which is the mean value for an overbounding model that bounds the residual clock rate error. The bound is meanClockRate + K * stdDevClockRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.001 m/s; range 0.000-0.255 m/s.

stdDevClockRate This field specifies the Standard Deviation Clock Rate Error bound which is the standard deviation for an overbounding model that bounds the residual clock rate error. Scale factor 0.001 m/s; range 0.000-0.255 m/s.

ssr-ProviderInfo This field provides the SSR Provider Information.

NOTE 1:	The reference time t0 is epochTime + ½ × ssrUpdateInterval. The reference time t0 for ssrUpdateInterval '0' is epochTime.

#### –	GNSS-SSR-CodeBias

The IE GNSS-SSR-CodeBias is used by the location server to provide GNSS signal code bias together with integrity information. The target device may add the code bias to the pseudo-range measurement of the corresponding code signal to get corrected pseudo-ranges.

NOTE:	Any code biases transmitted in the broadcast messages (e.g., the GPS group delay differential TGD [4] (NAV-ClockModel)) are not applied at all by the target device.

The parameters provided in IE GNSS-SSR-CodeBias – except for SSR-IntegrityCodeBiasBounds – are used as specified for SSR Code Bias Messages (e.g., message type 1059 and 1065) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-CodeBias-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

iod-ssr-r15							INTEGER (0..15),

ssr-CodeBiasSatList-r15				SSR-CodeBiasSatList-r15,

...,

[[

ssr-ProviderInfo-r19				GNSS-SSR-ProviderInfo-r19			OPTIONAL 	-- Need OR

]]

}

SSR-CodeBiasSatList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-CodeBiasSatElement-r15

SSR-CodeBiasSatElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

ssr-CodeBiasSignalList-r15			SSR-CodeBiasSignalList-r15,

...

}

SSR-CodeBiasSignalList-r15 ::= SEQUENCE (SIZE(1..16)) OF SSR-CodeBiasSignalElement-r15

SSR-CodeBiasSignalElement-r15 ::= SEQUENCE {

signal-and-tracking-mode-ID-r15		GNSS-SignalID,

codeBias-r15						INTEGER (-8192..8191),

...,

[[

ssr-IntegrityCodeBiasBounds-r17		SSR-IntegrityCodeBiasBounds-r17		OPTIONAL	-- Need OR

]]

}

SSR-IntegrityCodeBiasBounds-r17 ::= SEQUENCE {

meanCodeBias-r17					INTEGER (0..255),

stdDevCodeBias-r17					INTEGER (0..255),

meanCodeBiasRate-r17				INTEGER (0..255),

stdDevCodeBiasRate-r17				INTEGER (0..255),

...

}

-- ASN1STOP

GNSS-SSR-CodeBias field descriptions

epochTime This field specifies the epoch time of the code bias data. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the GNSS satellite for which the code biases are provided.

signal-and-tracking-mode-ID This field specifies the GNSS signal for which the code biases are provided.

codeBias This field provides the code bias for the GNSS signal indicated by signal-and-tracking-mode-ID. Scale factor 0.01 m; range ±81.91 m.

meanCodeBias This field specifies the Mean Code Bias Error bound which is the mean value for an overbounding model that bounds the residual code bias error. The bound is meanCodeBias + K * stdDevCodeBias and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.005 m; range 0-1.275 m.

stdDevCodeBias This field specifies the Standard Deviation Code Bias Error bound which is the standard deviation for an overbounding model that bounds the residual code bias error. Scale factor 0.005 m; range 0-1.275 m.

meanCodeBiasRate This field specifies the Mean Code Bias Rate Error bound which is the mean value for an overbounding model that bounds the residual code bias rate error. The bound is meanCodeBiasRate + K * stdDevCodeBiasRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

stdDevCodeBiasRate This field specifies the Standard Deviation Code Bias Rate Error bound which is the standard deviation for an overbounding model that bounds the residual code bias rate error. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

ssr-ProviderInfo This field provides the SSR Provider Information.

#### –	GNSS-SSR-URA

The IE GNSS-SSR-URA is used by the location server to provide quality information for the provided SSR assistance data.

The parameters provided in IE GNSS-SSR-URA are used as specified for the SSR URA Messages (e.g., message type 1061 and 1067) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-URA-r16 ::= SEQUENCE {

epochTime-r16						GNSS-SystemTime,

ssrUpdateInterval-r16				INTEGER (0..15),

iod-ssr-r16							INTEGER (0..15),

ssr-URA-SatList-r16					SSR-URA-SatList-r16,

...,

[[

ssr-ProviderInfo-r19				GNSS-SSR-ProviderInfo-r19			OPTIONAL 	-- Need OR

]]

}

SSR-URA-SatList-r16 ::= SEQUENCE (SIZE(1..64)) OF SSR-URA-SatElement-r16

SSR-URA-SatElement-r16 ::= SEQUENCE {

svID-r16							SV-ID,

ssr-URA-r16							BIT STRING (SIZE (6)),

...

}

-- ASN1STOP

GNSS-SSR-URA field descriptions

epochTime This field specifies the epoch time of the SSR User Range Accuracy (URA). The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the GNSS satellite for which the SSR URA is provided.

ssr-URA This field specifies the User Range Accuracy (URA) (1-sigma) for the range correction provided in the SSR assistance data. The URA is represented by a combination of CLASS and VALUE. The 3 MSB define the CLASS with a range of 0-7 and the 3 LSB define the VALUE with a range of 0-7. The URA is computed by: SSR URA mm≤3CLASS1+VALUE4-1 [mm] See Table 'Relationship between SSR troposphere quality and URA indicator and physical quantity' in IE GNSS-SSR-GriddedCorrection.

ssr-ProviderInfo This field provides the SSR Provider Information.

#### –	GNSS-SSR-PhaseBias

The IE GNSS-SSR-PhaseBias is used by the location server to provide GNSS signal phase bias together with integrity information. The target device may add the phase bias to the phase-range measurement of the corresponding phase signal to get corrected phase-ranges.

The parameters provided in IE GNSS-SSR-PhaseBias – except for SSR-IntegrityPhaseBiasBounds – are used as specified for Compact SSR GNSS Satellite Phase Bias Messages (e.g., message type 4073,5) in [43] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-PhaseBias-r16 ::= SEQUENCE {

epochTime-r16						GNSS-SystemTime,

ssrUpdateInterval-r16				INTEGER (0..15),

iod-ssr-r16							INTEGER (0..15),

ssr-PhaseBiasSatList-r16			SSR-PhaseBiasSatList-r16,

...,

[[

ssr-ProviderInfo-r19				GNSS-SSR-ProviderInfo-r19			OPTIONAL 	-- Need OR

]]

}

SSR-PhaseBiasSatList-r16 ::= SEQUENCE (SIZE(1..64)) OF SSR-PhaseBiasSatElement-r16

SSR-PhaseBiasSatElement-r16 ::= SEQUENCE {

svID-r16							SV-ID,

ssr-PhaseBiasSignalList-r16			SSR-PhaseBiasSignalList-r16,

...

}

SSR-PhaseBiasSignalList-r16 ::= SEQUENCE (SIZE(1..16)) OF SSR-PhaseBiasSignalElement-r16

SSR-PhaseBiasSignalElement-r16 ::= SEQUENCE {

signal-and-tracking-mode-ID-r16		GNSS-SignalID,

phaseBias-r16						INTEGER (-16384..16383),

phaseDiscontinuityIndicator-r16		INTEGER (0..3),

phaseBiasIntegerIndicator-r16		INTEGER (0..3)					OPTIONAL,	-- Need OP

...,

[[

ssr-IntegrityPhaseBiasBounds-r17	SSR-IntegrityPhaseBiasBounds-r17	OPTIONAL	-- Need OR

]]

}

SSR-IntegrityPhaseBiasBounds-r17 ::= SEQUENCE {

meanPhaseBias-r17					INTEGER (0..255),

stdDevPhaseBias-r17					INTEGER (0..255),

meanPhaseBiasRate-r17				INTEGER (0..255),

stdDevPhaseBiasRate-r17				INTEGER (0..255),

...

}

-- ASN1STOP

GNSS-SSR-PhaseBias field descriptions

epochTime This field specifies the epoch time of the phase bias data. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the GNSS satellite for which the phase biases are provided.

signal-and-tracking-mode-ID This field specifies the GNSS signal for which the phase biases are provided.

phaseBias This field provides the phase bias for the GNSS signal indicated by signal-and-tracking-mode-ID. Scale factor 0.001 m; range ±16.383 m.

phaseDiscontinuityIndicator This field provides the phase discontinuity counter for the GNSS signal indicated by signal-and-tracking-mode-ID. This counter is increased for every discontinuity in phase (roll-over from 3 to 0).

phaseBiasIntegerIndicator This field informs whether the phase bias is Undifferenced Integer (Value 0), Widelane Integer (Value 1) or Non-Integer (Value 2): Value 0: The Undifferenced Integer Phase Bias supports PPP-RTK fixed, widelane or float mode. Value 1: The Widelane Integer Phase Bias indicates that after application of the Phase Bias value, this signal can be differenced with any other signal from the same satellite that also has Widelane Integer Phase Bias indicated to form a new combined carrier phase measurement of integer quality, supporting PPP-RTK widelane fixed mode. Value 2: The Non-Integer Phase Bias supports PPP-RTK float mode. Value 3: Reserved. If the phaseBiasIntegerIndicator field is not present then it is interpreted as having Value 0 (Undifferenced Integer).

meanPhaseBias This field specifies the Mean Phase Bias Error bound which is the mean value for an overbounding model that bounds the residual phase bias error. The bound is meanPhaseBias + K * stdDevPhaseBias and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.005 m; range 0-1.275 m.

stdDevPhaseBias This field specifies the Standard Deviation Phase Bias Error bound which is the standard deviation for an overbounding model that bounds the residual phase bias error. Scale factor 0.005 m; range 0-1.275 m.

meanPhaseBiasRate This field specifies the Mean Phase Bias Rate Error bound which is the mean value for an overbounding model that bounds the residual phase bias rate error. The bound is meanPhaseBiasRate + K * stdDevPhaseBiasRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

stdDevPhaseBiasRate This field specifies the Standard Deviation Phase Bias Rate Error bound which is the standard deviation for an overbounding model that bounds the residual phase bias rate error. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

ssr-ProviderInfo This field provides the SSR Provider Information.

#### –	GNSS-SSR-STEC-Correction

The IE GNSS-SSR-STEC-Correction is used by the location server to provide ionosphere slant delay correction together with integrity information. The ionosphere slant delay (STEC) consists of the polynomial part provided in GNSS-SSR-STEC-Correction and the residual part provided in GNSS-SSR-GriddedCorrection.

The parameters provided in IE GNSS-SSR-STEC-Correction – except for STEC-IntegrityParameters and STEC-IntegrityErrorBounds – are used as specified for Compact SSR STEC Correction Messages (e.g., message type 4073,8) in [43] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-STEC-Correction-r16 ::= SEQUENCE {

epochTime-r16						GNSS-SystemTime,

ssrUpdateInterval-r16				INTEGER (0..15),

iod-ssr-r16							INTEGER (0..15),

correctionPointSetID-r16			INTEGER (0..16383),

stec-SatList-r16					STEC-SatList-r16,

...,

[[

stec-IntegrityParameters-r17	STEC-IntegrityParameters-r17		OPTIONAL	-- Need OR

]],

[[

ssr-ProviderInfo-r19				GNSS-SSR-ProviderInfo-r19			OPTIONAL 	-- Need OR

]]

}

STEC-SatList-r16 ::= SEQUENCE (SIZE(1..64)) OF STEC-SatElement-r16

STEC-SatElement-r16 ::= SEQUENCE {

svID-r16							SV-ID,

stecQualityIndicator-r16			BIT STRING (SIZE(6)),

stec-C00-r16						INTEGER (-8192..8191),

stec-C01-r16						INTEGER (-2048..2047)				OPTIONAL, -- Need ON

stec-C10-r16						INTEGER (-2048..2047)				OPTIONAL, -- Need ON

stec-C11-r16						INTEGER (-512..511)					OPTIONAL, -- Need ON

...,

[[

stec-IntegrityErrorBounds-r17	STEC-IntegrityErrorBounds-r17	OPTIONAL  -- Cond Integrity1

]]

}

STEC-IntegrityParameters-r17 ::= SEQUENCE {

probOnsetIonoFault-r17						INTEGER (0..255),

meanIonoFaultDuration-r17					INTEGER (1..256),

ionoRangeErrorCorrelationTime-r17			INTEGER (1..255)		OPTIONAL, -- Need OR

ionoRangeRateErrorCorrelationTime-r17		INTEGER (1..255)		OPTIONAL, -- Cond Integrity2

...

}

STEC-IntegrityErrorBounds-r17 ::= SEQUENCE {

meanIonosphere-r17							INTEGER (0..255),

stdDevIonosphere-r17						INTEGER (0..255),

meanIonosphereRate-r17						INTEGER (0..255),

stdDevIonosphereRate-r17					INTEGER (0..255),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is mandatory present if STEC-IntegrityParameters is present; otherwise it is not present. |
| Integrity2 | The field is mandatory present if ionoRangeErrorCorrelationTime is present; otherwise it is not present. |

GNSS-SSR-STEC-Correction field descriptions

epochTime This field specifies the epoch time of the STEC correction data. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

correctionPointSetID This field provides the ID of the GNSS-SSR-CorrectionPoints set. The reference point used for the STEC calculations (see NOTE below) is the reference point provided in IE GNSS-SSR-CorrectionPoints with the same correctionPointSetID.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the GNSS satellite for which the STEC corrections are provided.

stecQualityIndicator This field specifies SSR STEC quality indicator. The STEC quality indicator is represented by a combination of CLASS and VALUE. The 3 MSB define the CLASS with a range of 0-7 and the 3 LSB define the VALUE with a range of 0-7. See Table 'Relationship between SSR STEC quality indicator and physical quantity' below.

stec-C00 This field provides the polynomial coefficient C00 used to define the STEC. as defined in [43]. NOTE Scale factor 0.05 TECU; range ±409.55 TECU.

stec-C01 This field provides the polynomial coefficient C01 used to define the STEC as defined in [43]. NOTE Scale factor 0.02 TECU/deg; range ±40.94 TECU/deg.

stec-C10 This field provides the polynomial coefficient C10 used to define the STEC as defined in [43]. NOTE Scale factor 0.02 TECU/deg; range ±40.94 TECU/deg.

stec-C11 This field provides the polynomial coefficient C11 used to define the STEC as defined in [43]. NOTE Scale factor 0.02 TECU/deg2; range ±10.22 TECU/deg2.

probOnsetIonoFault This field specifies the Probability of Onset of Ionosphere Fault per Time Unit which is the probability of occurrence of ionosphere error to exceed the residual error bound for more than the Time to Alert (TTA). This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetIonoFault and the range is 10-10.2 to 1 per hour.

meanIonoFaultDuration This field specifies the Mean Ionosphere Fault Duration which is the mean duration between when an ionosphere integrity violation occurs, and the user is alerted through GNSS-Integrity-ServiceAlert (or the integrity violation is over). Scale factor 1 s; range 1-256 s.

ionoRangeErrorCorrelationTime This field specifies the Ionosphere Range Error Correlation Time which is the upper bound of the correlation time of the ionosphere residual range error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

ionoRangeRateErrorCorrelationTime This field specifies the Ionosphere Range Rate Error Correlation Time which is the upper bound of the correlation time of the ionosphere residual range rate error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanIonosphere This field specifies the Mean Ionospherre Error bound which is the mean value for an overbounding model that bounds the residual ionosphere error. The bound is meanIonosphere + K * stdDevIonosphere and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. The mean is calculated using: μ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

stdDevIonosphere This field specifies the Standard Deviation Ionosphere Error bound which is the standard deviation for an overbounding model that bounds the residual ionosphere error. The standard deviation is calculated using: σ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

meanIonosphereRate This field specifies the Mean Ionosphere Rate Error which is the mean value for an overbounding model that bounds the residual ionosphere rate error. The bound is meanIonosphereRate + K * stdDevIonosphereRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

stdDevIonosphereRate This field specifies the Standard Deviation Ionosphere Rate Error which is the standard deviation for an overbounding model that bounds the residual ionosphere rate error. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

ssr-ProviderInfo This field provides the SSR Provider Information.

NOTE:	The polynomial coefficients C00, C01, C10, C11 are used to define the STEC as follows:

(1)	If only C00 is included in STEC-SatElement:

Iai = C00.

(2)	If only C00, C01 and C10 are included in STEC-SatElement:

Iai = C00 + C01( – 0) +  C10( – 0).

(3)	If all of C00, C01, C10 and C11 are included in STEC-SatElement:

Iai = C00 + C01( – 0) + C10( – 0) +C11( – 0) ( – 0).

Other combinations of C00, C01, C10, C11 than (1)-(3) above are undefined in this version of the specification.

The equations above depend on the latitude  and longitude  of an evaluated point and latitude 0 and longitude 0 of the reference point which is defined in IE GNSS-SSR-CorrectionPoints (referencePointLatitude and referencePointLongitude).

Relationship between SSR STEC quality indicator and physical quantity

| CLASS | VALUE | Index | SSR STEC Quality Indicator Q [TECU] |
| --- | --- | --- | --- |
| 7 | 7 | 63 | 33.6664 < Q |
| 7 | 6 | 62 | 30.2992 < Q ≤ 33.6664 |
| 7 | 5 | 61 | 26.9319 < Q ≤ 30.2992 |
| 7 | 4 | 60 | 23.5647 < Q ≤ 26.9319 |
| 7 | 3 | 59 | 20.1974 < Q ≤ 23.5647 |
| 7 | 2 | 58 | 16.8301 < Q ≤ 20.1974 |
| 7 | 1 | 57 | 13.4629 < Q ≤ 16.8301 |
| 7 | 0 | 56 | 12.3405 < Q ≤ 13.4629 |
| 6 | 7 | 55 | 11.2180 < Q ≤ 12.3405 |
| 6 | 6 | 54 | 10.0956 < Q ≤ 11.2180 |
| 6 | 5 | 53 | 8.9732 < Q ≤ 10.0956 |
| 6 | 4 | 52 | 7.8508 < Q ≤ 8.9732 |
| 6 | 3 | 51 | 6.7284 < Q ≤ 7.8508 |
| 6 | 2 | 50 | 5.6059 < Q ≤ 6.7284 |
| 6 | 1 | 49 | 4.4835 < Q ≤ 5.6059 |
| 6 | 0 | 48 | 4.1094 < Q ≤ 4.4835 |
| 5 | 7 | 47 | 3.7352 < Q ≤ 4.1094 |
| 5 | 6 | 46 | 3.3611 < Q ≤ 3.7352 |
| 5 | 5 | 45 | 2.9870 < Q ≤ 3.3611 |
| 5 | 4 | 44 | 2.6128 < Q ≤ 2.9870 |
| 5 | 3 | 43 | 2.2387 < Q ≤ 2.6128 |
| 5 | 2 | 42 | 1.8645 < Q ≤ 2.2387 |
| 5 | 1 | 41 | 1.4904 < Q ≤ 1.8645 |
| 5 | 0 | 40 | 1.3657 < Q ≤ 1.4904 |
| 4 | 7 | 39 | 1.2410 < Q ≤ 1.3657 |
| 4 | 6 | 38 | 1.1163 < Q ≤ 1.2410 |
| 4 | 5 | 37 | 0.9915 < Q ≤ 1.1163 |
| 4 | 4 | 36 | 0.8668 < Q ≤ 0.9915 |
| 4 | 3 | 35 | 0.7421 < Q ≤ 0.8668 |
| 4 | 2 | 34 | 0.6174 < Q ≤ 0.7421 |
| 4 | 1 | 33 | 0.4927 < Q ≤ 0.6174 |
| 4 | 0 | 32 | 0.4511 < Q ≤ 0.4927 |
| 3 | 7 | 31 | 0.4096 < Q ≤ 0.4511 |
| 3 | 6 | 30 | 0.3680 < Q ≤ 0.4096 |
| 3 | 5 | 29 | 0.3264 < Q ≤ 0.3680 |
| 3 | 4 | 28 | 0.2848 < Q ≤ 0.3264 |
| 3 | 3 | 27 | 0.2433 < Q ≤ 0.2848 |
| 3 | 2 | 26 | 0.2017 < Q ≤ 0.2433 |
| 3 | 1 | 25 | 0.1601 < Q ≤ 0.2017 |
| 3 | 0 | 24 | 0.1463 < Q ≤ 0.1601 |
| 2 | 7 | 23 | 0.1324 < Q ≤ 0.1463 |
| 2 | 6 | 22 | 0.1186 < Q ≤ 0.1324 |
| 2 | 5 | 21 | 0.1047 < Q ≤ 0.1186 |
| 2 | 4 | 20 | 0.0908 < Q ≤ 0.1047 |
| 2 | 3 | 19 | 0.0770 < Q ≤ 0.0908 |
| 2 | 2 | 18 | 0.0631 < Q ≤ 0.0770 |
| 2 | 1 | 17 | 0.0493 < Q ≤ 0.0631 |
| 2 | 0 | 16 | 0.0447 < Q ≤ 0.0493 |
| 1 | 7 | 15 | 0.0400 < Q ≤ 0.0447 |
| 1 | 6 | 14 | 0.0354 < Q ≤ 0.0400 |
| 1 | 5 | 13 | 0.0308 < Q ≤ 0.0354 |
| 1 | 4 | 12 | 0.0262 < Q ≤ 0.0308 |
| 1 | 3 | 11 | 0.0216 < Q ≤ 0.0262 |
| 1 | 2 | 10 | 0.0169 < Q ≤ 0.0216 |
| 1 | 1 | 9 | 0.0123 < Q ≤ 0.0169 |
| 1 | 0 | 8 | 0.0108 < Q ≤ 0.0123 |
| 0 | 7 | 7 | 0.0092 < Q ≤ 0.0108 |
| 0 | 6 | 6 | 0.0077 < Q ≤ 0.0092 |
| 0 | 5 | 5 | 0.0062 < Q ≤ 0.0077 |
| 0 | 4 | 4 | 0.0046 < Q ≤ 0.0062 |
| 0 | 3 | 3 | 0.0031 < Q ≤ 0.0046 |
| 0 | 2 | 2 | 0.0015 < Q ≤ 0.0031 |
| 0 | 1 | 1 | Q ≤ 0.0015 |
| 0 | 0 | 0 | undefined/unknown |

#### –	GNSS-SSR-GriddedCorrection

The IE GNSS-SSR-GriddedCorrection is used by the location server to provide troposphere delay correction, together with the residual part of the STEC corrections and integrity information.

The parameters provided in IE GNSS-SSR-GriddedCorrection – except for SSR-GriddedCorrectionIntegrityParameters and TropoDelayIntegrityErrorBounds – are used as specified for Compact SSR Gridded Correction Message (e.g., message type 4073,9) in [43] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-GriddedCorrection-r16 ::= SEQUENCE {

epochTime-r16								GNSS-SystemTime,

ssrUpdateInterval-r16						INTEGER (0..15),

iod-ssr-r16									INTEGER (0..15),

troposphericDelayQualityIndicator-r16		BIT STRING (SIZE(6))		OPTIONAL, -- Cond Tropo

correctionPointSetID-r16					INTEGER (0..16383),

gridList-r16								GridList-r16,

...,

[[

ssr-GriddedCorrectionIntegrityParameters-r17

SSR-GriddedCorrectionIntegrityParameters-r17

OPTIONAL  -- Need OR

]],

[[

ssr-ProviderInfo-r19				GNSS-SSR-ProviderInfo-r19			OPTIONAL 	-- Need OR

]]

}

GridList-r16 ::= SEQUENCE (SIZE(1..64)) OF GridElement-r16

GridElement-r16 ::= SEQUENCE {

tropospericDelayCorrection-r16	TropospericDelayCorrection-r16	OPTIONAL, -- Need ON

stec-ResidualSatList-r16		STEC-ResidualSatList-r16		OPTIONAL, -- Need ON

...

}

TropospericDelayCorrection-r16 ::= SEQUENCE {

tropoHydroStaticVerticalDelay-r16		INTEGER (-256..255),

tropoWetVerticalDelay-r16				INTEGER (-128..127),

...,

[[

tropoDelayIntegrityErrorBounds-r17		TropoDelayIntegrityErrorBounds-r17

OPTIONAL -- Cond Integrity1

]]

}

STEC-ResidualSatList-r16 ::= SEQUENCE (SIZE(1..64)) OF STEC-ResidualSatElement-r16

STEC-ResidualSatElement-r16 ::= SEQUENCE {

svID-r16							SV-ID,

stecResidualCorrection-r16			CHOICE {

b7-r16					INTEGER (-64..63),

b16-r16					INTEGER (-32768..32767)

},

...

}

SSR-GriddedCorrectionIntegrityParameters-r17 ::= SEQUENCE {

probOnsetTroposphereFault-r17					INTEGER (0..255),

meanTroposphereFaultDuration-r17				INTEGER (1..256),

troposphereRangeErrorCorrelationTime-r17		INTEGER (1..255)	OPTIONAL, -- Need OR

troposphereRangeRateErrorCorrelationTime-r17	INTEGER (1..255)	OPTIONAL, -- Cond Integrity2

...

}

TropoDelayIntegrityErrorBounds-r17 ::= SEQUENCE {

meanTroposphereVerticalHydroStaticDelay-r17			INTEGER (0..255),

stdDevTroposphereVerticalHydroStaticDelay-r17		INTEGER (0..255),

meanTroposphereVerticalWetDelay-r17					INTEGER (0..255),

stdDevTroposphereVerticalWetDelay-r17				INTEGER (0..255),

meanTroposphereVerticalHydroStaticDelayRate-r17		INTEGER (0..255),

stdDevTroposphereVerticalHydroStaticDelayRate-r17	INTEGER (0..255),

meanTroposphereVerticalWetDelayRate-r17				INTEGER (0..255),

stdDevTroposphereVerticalWetDelayRate-r17			INTEGER (0..255),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Tropo | The field is mandatory present if tropospericDelayCorrection is included in gridList. Otherwise it is not present. |
| Integrity1 | The field is mandatory present if SSR-GriddedCorrectionIntegrityParameters is present; otherwise it is not present. |
| Integrity2 | The field is mandatory present if troposphereRangeErrorCorrelationTime is present; otherwise it is not present. |

GNSS-SSR-GriddedCorrection field descriptions

epochTime This field specifies the epoch time of the gridded correction data. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

troposphericDelayQualityIndicator This field specifies the quality indicator of the tropospheric delay. The troposphere quality indicator is represented by a combination of CLASS and VALUE. The 3 MSB define the CLASS with a range of 0-7 and the 3 LSB define the VALUE with a range of 0-7. The troposphere quality indicator is computed by: SSR Troposphere Quality [mm] ≤ 3CLASS1+VALUE4-1 [mm] See Table 'Relationship between SSR troposphere quality and URA indicator and physical quantity' below.

correctionPointSetID This field provides the ID of the GNSS-SSR-CorrectionPoints set. The GNSS-SSR-GriddedCorrection are valid for the correction points provided in IE GNSS-SSR-CorrectionPoints with the same correctionPointSetID.

gridList This field provides the troposphere delay correction together with the residual part of the STEC corrections for up to 64 correction points defined in IE GNSS-SSR-CorrectionPoints. If the IE GNSS-SSR-CorrectionPoints, which belongs to the correctionPointSetID, includes the listOfCorrectionPoints, the gridList includes the same number of entries, and listed in the same order, as in the listOfCorrectionPoints. If the IE GNSS-SSR-CorrectionPoints, which belongs to this correctionPointSetID, includes the arrayOfCorrectionPoints the gridList includes the same number of entries, and listed in the same order, as defined by the enabled bits in the bitmaskOfGrids.

tropoHydroStaticVerticalDelay This field specifies the variation in the hydro static troposphere vertical delay relative to nominal value. The target device should add the constant nominal value of 2.3 m to calculate the tropospheric hydro-static vertical delay. Scale factor 0.004 m; range ±1.02 m.

tropoWetVerticalDelay This field specifies the variation in the wet troposphere vertical delay relative to nominal value. The target device should add the constant value of 0.252 m to calculate the tropospheric wet (non hydro-static) vertical delay. Scale factor 0.004 m; range ±0.508 m.

svID This field specifies the GNSS satellite for which the STEC residual corrections are provided.

stecResidualCorrection This field specifies the STEC residual correction. Scale factor 0.04 TECU; range ±2.52 TECU (b7) or ±1310.68 TECU (b16).

probOnsetTroposphereFault This field specifies the Probability of Onset of Troposphere Fault per Time Unit which is the probability of occurrence of troposphere error to exceed the residual error bound for more than the Time to Alert (TTA) This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2) and irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetTroposphereFault and the range is 10-10.2 to 1 per hour.

meanTroposphereFaultDuration This field specifies the Mean Troposphere Fault Duration which is the mean duration between when a troposphere integrity violation occurs, and the user is alerted through GNSS-Integrity-ServiceAlert (or the integrity violation is over). Scale factor 1 s; range 1-256 s.

troposphereRangeErrorCorrelationTime This field specifies the Troposphere Range Error Correlation Time which is the upper bound of the correlation time of the troposphere residual range error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

troposphereRangeRateErrorCorrelationTime This field specifies the Troposphere Range Rate Error Correlation Time which is the upper bound of the correlation time of the troposphere residual range rate error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanTroposphereVerticalHydroStaticDelay This field specifies the Mean Troposphere Vertical Hydro Static Delay Error bound which is the mean value for an overbounding model that bounds the residual troposphere error in the vertical hydro static delay component. The bound is meanTroposphereVerticalHydroStaticDelay + K * stdDevTroposphereVerticalHydroStaticDelay and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.005 m; range 0-1.275 m.

stdDevTroposphereVerticalHydroStaticDelay This field specifies the Standard Deviation Troposphere Vertical Hydro Static Delay Error bound which is the standard deviation for an overbounding model that bounds the residual troposphere error in the vertical hydro static delay component. Scale factor 0.005 m; range 0-1.275 m.

meanTroposphereVerticalWetDelay This field specifies the Mean Troposphere Vertical Wet Static Delay Error bound which is the mean value for an overbounding model that bounds the residual troposphere error in the vertical wet delay component. The bound is meanTroposphereVerticalWetDelay + K * stdDevTroposphereVerticalWetDelay and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.005 m; range 0-1.275 m.

stdDevTroposphereVerticalWetDelay This field specifies the Standard Deviation Troposphere Vertical Wet Static Delay Error bound which is the standard deviation for an overbounding model that bounds the residual troposphere error in the vertical wet delay component. Scale factor 0.005 m; range 0-1.275 m.

meanTroposphereVerticalHydroStaticDelayRate This field specifies the Mean Troposphere Vertical Hydro Static Delay Rate Error bound which is the mean value for an overbounding model that bounds the residual troposphere rate error in the vertical hydro static delay component. The bound is meanTroposphereVerticalHydroStaticDelayRate + K * stdDevTroposphereVerticalHydroStaticDelayRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

stdDevTroposphereVerticalHydroStaticDelayRate This field specifies the Standard Deviation Troposphere Vertical Hydro Static Delay Rate Error bound which is the standard deviation for an overbounding model that bounds the residual troposphere rate error in the vertical hydro static delay component. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

meanTroposphereVerticalWetDelayRate This field specifies the Mean Troposphere Vertical Wet Static Delay Rate Error bound which is the mean value for an overbounding model that bounds the residual troposphere rate error in the vertical wet delay component. The bound is meanTroposphereVerticalWetDelayRate + K * stdDevTroposphereVerticalWetDelayRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

stdDevTroposphereVerticalWetDelayRate This field specifies the Standard Deviation Troposphere Vertical Wet Static Delay Rate Error bound which is the standard deviation for an overbounding model that bounds the residual troposphere rate error in the vertical wet delay component. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

ssr-ProviderInfo This field provides the SSR Provider Information.

Relationship between SSR troposphere quality and URA indicator and physical quantity

| CLASS | VALUE | Index | SSR troposphere quality indicator and SSR URA Q [mm] |
| --- | --- | --- | --- |
| 7 | 7 | 63 | 5466.50 < Q |
| 7 | 6 | 62 | 4919.75 < Q ≤ 5466.50 |
| 7 | 5 | 61 | 4373.75 < Q ≤ 4919.75 |
| 7 | 4 | 60 | 3826.25 < Q ≤ 4373.00 |
| 7 | 3 | 59 | 3279.50 < Q ≤ 3826.25 |
| 7 | 2 | 58 | 2732.75 < Q ≤ 3279.50 |
| 7 | 1 | 57 | 2186.00 < Q ≤ 2732.75 |
| 7 | 0 | 56 | 2003.75 < Q ≤ 2186.00 |
| 6 | 7 | 55 | 1821.50 < Q ≤ 2003.75 |
| 6 | 6 | 54 | 1639.25 < Q ≤ 1821.50 |
| 6 | 5 | 53 | 1457.00 < Q ≤ 1639.25 |
| 6 | 4 | 52 | 1274.75 < Q ≤ 1457.00 |
| 6 | 3 | 51 | 1092.50 < Q ≤ 1274.75 |
| 6 | 2 | 50 | 910.25 < Q ≤ 1092.50 |
| 6 | 1 | 49 | 728.00 < Q ≤ 910.25 |
| 6 | 0 | 48 | 667.25 < Q ≤ 728.00 |
| 5 | 7 | 47 | 606.50 < Q ≤ 667.25 |
| 5 | 6 | 46 | 545.75 < Q ≤ 606.50 |
| 5 | 5 | 45 | 485.00 < Q ≤ 545.75 |
| 5 | 4 | 44 | 424.25 < Q ≤ 485.00 |
| 5 | 3 | 43 | 363.50 < Q ≤ 425.25 |
| 5 | 2 | 42 | 302.75 < Q ≤ 363.50 |
| 5 | 1 | 41 | 242.00 < Q ≤ 302.75 |
| 5 | 0 | 40 | 221.75 < Q ≤ 242.00 |
| 4 | 7 | 39 | 201.50 < Q ≤ 221.75 |
| 4 | 6 | 38 | 181.25 < Q ≤ 201.50 |
| 4 | 5 | 37 | 161.00 < Q ≤ 181.25 |
| 4 | 4 | 36 | 140.75 < Q ≤ 161.00 |
| 4 | 3 | 35 | 120.50 < Q ≤ 140.75 |
| 4 | 2 | 34 | 100.25 < Q ≤ 120.50 |
| 4 | 1 | 33 | 80.00 < Q ≤ 100.25 |
| 4 | 0 | 32 | 73.25 < Q ≤ 80.00 |
| 3 | 7 | 31 | 66.50 < Q ≤ 73.25 |
| 3 | 6 | 30 | 59.75 < Q ≤ 66.50 |
| 3 | 5 | 29 | 53.00 < Q ≤ 59.75 |
| 3 | 4 | 28 | 46.25 < Q ≤ 53.00 |
| 3 | 3 | 27 | 39.50 < Q ≤ 46.25 |
| 3 | 2 | 26 | 32.75 < Q ≤ 39.50 |
| 3 | 1 | 25 | 26.00 < Q ≤ 32.75 |
| 3 | 0 | 24 | 23.75 < Q ≤ 26.00 |
| 2 | 7 | 23 | 21.50 < Q ≤ 23.75 |
| 2 | 6 | 22 | 19.25 < Q ≤ 21.50 |
| 2 | 5 | 21 | 17.00 < Q ≤ 19.25 |
| 2 | 4 | 20 | 14.75 < Q ≤ 17.00 |
| 2 | 3 | 19 | 12.50 < Q ≤ 14.75 |
| 2 | 2 | 18 | 10.25 < Q ≤ 12.50 |
| 2 | 1 | 17 | 8.00 < Q ≤ 10.25 |
| 2 | 0 | 16 | 7.25 < Q ≤ 8.00 |
| 1 | 7 | 15 | 6.50 < Q ≤ 7.25 |
| 1 | 6 | 14 | 5.75 < Q ≤ 6.50 |
| 1 | 5 | 13 | 5.00 < Q ≤ 5.75 |
| 1 | 4 | 12 | 4.25 < Q ≤ 5.00 |
| 1 | 3 | 11 | 3.50 < Q ≤ 4.25 |
| 1 | 2 | 10 | 2.75 < Q ≤ 3.50 |
| 1 | 1 | 9 | 2.00 < Q ≤ 2.75 |
| 1 | 0 | 8 | 1.75 < Q ≤ 2.00 |
| 0 | 7 | 7 | 1.50 < Q ≤ 1.75 |
| 0 | 6 | 6 | 1.25 < Q ≤ 1.50 |
| 0 | 5 | 5 | 1.00 < Q ≤ 1.25 |
| 0 | 4 | 4 | 0.75 < Q ≤ 1.00 |
| 0 | 3 | 3 | 0.50 < Q ≤ 0.75 |
| 0 | 2 | 2 | 0.25 < Q ≤ 0.50 |
| 0 | 1 | 1 | Q ≤ 0.25 |
| 0 | 0 | 0 | undefined/unknown |

#### <Skip Unmodified Changes>

#### –	GNSS-SSR-SatellitePCVResiduals

The IE GNSS-SSR-SatellitePCVResiduals is used by the location server to provide the nadir-angle-dependent phase center variations.

-- ASN1START

GNSS-SSR-SatellitePCVResiduals-r18 ::= SEQUENCE {

iod-ssr-PCVResiduals-r18			INTEGER (0..64),

ssr-SatellitePCV-List-r18			SSR-SatellitePCV-List-r18,

...,

[[

ssr-ProviderInfo-r19				GNSS-SSR-ProviderInfo-r19			OPTIONAL 	-- Need OR

]]

}

SSR-SatellitePCV-List-r18 ::= SEQUENCE (SIZE(1..64)) OF SSR-SatellitePCV-Element-r18

SSR-SatellitePCV-Element-r18 ::= SEQUENCE {

svID-r18							SV-ID,

ssr-SatellitePCV-FrequencyList-r18	SSR-SatellitePCV-FrequencyList-r18,

...

}

SSR-SatellitePCV-FrequencyList-r18 ::= SEQUENCE (SIZE(1..8)) OF

SSR-SatellitePCV-FrequencyElement-r18

SSR-SatellitePCV-FrequencyElement-r18 ::= SEQUENCE {

frequencyID-r18							GNSS-FrequencyID-r15,

phaseCenterVariations-r18				SSR-PhaseCenterVariationList-r18,

...

}

SSR-PhaseCenterVariationList-r18 ::= SEQUENCE (SIZE(1..32)) OF INTEGER(-1024..1023)

-- ASN1STOP

GNSS-SSR-SatellitePCVResiduals field descriptions

iod-ssr-PCVResiduals This field specifies the Issue of Data for the SSR Satellite PCV Residuals that is valid while the GNSS-SSR-IOD-Update is valid.

svID This field specifies the satellite for which the Satellite PCV Residual corrections are provided.

frequencyID This field specifies the satellite carrier frequency to which this correction applies.

phaseCenterVariations This field specifies the residual nadir only variations of the phase center that are not already accounted for within the SSR Phase Bias. The nadir angle is defined to be the angle away from the z-axis. The variations are given in increments of 1 degree steps, the first element is the variation at 1 degree. In units of 1 mm.

ssr-ProviderInfo This field provides the SSR Provider Information.

#### 6.5.2.3	GNSS Assistance Data Request

Next Changes

#### 6.5.2.13	Common GNSS Information Elements

#### –	GNSS-FrequencyID

#### <Skip Unmodified Changes>

#### –	GNSS-SignalIDs

The IE GNSSSignal-IDs is used to indicate several GNSS signals using a bit map. The interpretation of GNSSSignal-IDs depends on the GNSS-ID.

-- ASN1START

GNSS-SignalIDs	::= SEQUENCE {

gnss-SignalIDs		BIT STRING (SIZE(8)),

...,

[[

gnss-SignalIDs-Ext-r15	BIT STRING (SIZE(16))			OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

GNSS-SignalIDs field descriptions

gnss-SignalIDs, gnss-SignalIDs-Ext This field specifies one or several GNSS signals using a bit map. A one-value at the bit position means the particular signal is addressed; a zero-value at the particular bit position means the signal is not addressed. The interpretation of the bit map in gnssSignalIDs and gnss-SignalIDs-Ext depends on the GNSS-ID and is shown in the table below. Unfilled table entries indicate no assignment and shall be set to zero.

Interpretation of the bit map in gnssSignalIDs

| GNSS | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 (LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L1 C/A | L1C | L2C | L5 | L1P | L1 Z | L2 C/A | L2 P |
| SBAS | L1 C/A | L5 I | L5 Q | L5 I+Q |  |  |  |  |
| QZSS | QZS-L1 C/A | QZS-L1C | QZS-L2C | QZS-L5 | LEX S | LEX L | LEX S+L | L2C(M) |
| GLONASS | G1 C/A | G2 C/A | G3 | G1 P | G2 P | G1a(D) | G1a(P) | G1a(D+P) |
| Galileo | E1 | E5a | E5b | E6 | E5a+E5b | E1 C No Data | E1 A | E1 B I/NAV OS/CS/SoL |
| BDS | B1 I | B1 Q | B1 I+Q | B3 I | B3 Q | B3 I+Q | B2 I | B2 Q |
| NavIC | L5 SPS | L1 SPS(D) | L1 SPS(P) | L1 SPS(D+P) |  |  |  |  |

Interpretation of the bit map in gnssSignalIDs-Ext

| GNSS | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L2 Z | L2C(M) | L2C(L) | L2C(M+L) | L5 I | L5 Q | L5 I+Q | L1C(D) |
| SBAS |  |  |  |  |  |  |  |  |
| QZSS | L2C(L) | L2C(M+L) | L5 I | L5 Q | L5 I+Q | L1C(D) | L1C(P) | L1C(D+P) |
| GLONASS | G2a(I) | G2a(P) | G2a(I+P) | G3 I | G3 Q | G3(I+Q) |  |  |
| Galileo | E1 B+C | E1 A+B+C | E6C | E6A | E6B | E6 B+C | E6 A+B+C | E5B I |
| BDS | B2 I+Q | B1C(D) | B1C(P) | B1C(D+P) | B2a(D) | B2a(P) | B2a(D+P) | B2b I |
| NavIC |  |  |  |  |  |  |  |  |

| GNSS | Bit 9 | Bit 10 | Bit 11 | Bit 12 | Bit 13 | Bit 14 | Bit 15 | Bit 16 (LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L1C(P) | L1C(D+P) |  |  |  |  |  |  |
| SBAS |  |  |  |  |  |  |  |  |
| QZSS |  |  |  |  |  |  |  |  |
| GLONASS |  |  |  |  |  |  |  |  |
| Galileo | E5B Q | E5B I+Q | E5(A+B) I | E5(A+B) Q | E5(A+B) I+Q | E5A I | E5A Q | E5A I+Q |
| BDS |  |  |  |  |  |  |  |  |
| NavIC |  |  |  |  |  |  |  |  |

#### –	GNSS-SSR-ProviderInfo

The IE GNSS-SSR-ProviderInfo is used by the location server to provide unique identification of the SSR Provider and to distinguish different SSR services of one SSR provider as defined in [30]. The parameters provided in IE GNSS-SSR-ProviderInfo are used as specified for DF414/DF415 in [30].

-- ASN1START

GNSS-SSR-ProviderInfo-r19 ::= SEQUENCE {

ssr-ProviderID-r19					INTEGER (0..65535),

ssr-SolutionID-r19					INTEGER	(0..15)					OPTIONAL,  -- Need OR

...

}

-- ASN1STOP

GNSS-SSR-ProviderInfo field descriptions

ssr-ProviderID This field indicates the SSR Provider ID. It is unique in the network and represents an SSR provider ID as defined in [30].

ssr-SolutionID This field indicates the different SSR services of one SSR provider as defined in [30].

#### –	GNSS-SubNetworkID

The IE GNSS-SubNetworkID defines the subnetwork of a network identified by GNSS-NetworkID. This IE is used for MAC Network RTK as described in [30].

-- ASN1START

GNSS-SubNetworkID-r15 ::= SEQUENCE {

subNetworkID-r15				INTEGER (0..15),

...

}

-- ASN1STOP

End of Changes