3GPP TSG-SA WG6 Meeting #69	S6-254632

Wuhan, P.R.China, 13th – 17th October 2025	(revision of S6-254045)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.234 | CR | 0401 | rev | 01 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 23.434 for UE ID usage correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2025-10-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As it is metioned in Table 5.3.3-1of 3GPP TR 23.888 and the related content in 3GPP TS 23.682, the feature of PDU connectivity status exposure is defined mainly to support MT terminated communication for MTC. In some MT usecase, there is no uplink triggering message from UE. So the MTC server has to know if there is PDU connection is activiated for the transmission of downlink messages, by retrieving the PDU connection statues information from CN. The solution assumes external UE ID that AF/AS provides should known-able by CN (e.g., HSS) and can be mapped to PLMN subsciber user ID(e.g IMPI/IMPU), however this assumption does not applicable to all the VAL applications,especially widely used OTT. In OTT case, the IP address of UE is known to application server(VAL sever). ******************** Table 5.3.3-1 in 3GPP TR 23.888**************************** ……. The mechanism shall minimize the required messaging transactions by the MTC Server to initiate MT communications;; With always on support no additional message transactions required for MT terminated communication, besides perhaps for some specific situations where triggering may be used e.g. to recover the (always-on) PDP/PDN connection from error cases or to set triggers for devices that are out of coverage. MTC Server can be aware of presence of MTC device and assigned Private IP address as described using techniques as described in 6.29.3 and 6.29.4 For PDP/PDN connections not using always on, triggering should be used similar to any non-always-on solution. Triggering may also be used e.g. to recover the PDP/PDN connection from error cases or to set triggers for devices that are out of coverage.. ***************Content about PDU connectivity status in TS 23.682 ************* 5.6.1.10 Specific Parameters for Monitoring Event: PDN Connectivity Status This monitoring event allows the SCS/AS to be notified of the PDN Connectivity status when PDN Connections are created or deleted for the UE, and the existing PDN Connectivity status at the monitoring event configuration. The PDN Connectivity Status monitoring events report includes for each PDN Connection (that matches the requested APN if an APN has been specified) the IP address(es) allocated for the UE PDN connection(s), the PDN Types and optionally the APNs. This may be used by the SCS/AS to initiate communication with the UE, or to know when communication is no longer possible. Reporting is also done for PDN Connections using T6a/T6b connection towards the SCEF. **************************************************************************************** |
| Summary of change: |  | 1. Align the description of “List of VAL UEs or VAL Group ID” information in 14.3.2.1 and 14.3.2.2a, with the Table 14.3.2.13-1: end-to-end QoS management request. 2. Addressing the case where is UE IP address is provided by VAL server in the procedure description in 14.3.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution is not applicable to VAL application server without CN knownable UE ID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.1 ,14.3.2.2a,14.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********************Begin of the first change***************

14.3.1	General

The procedures related to the network resource management are described in the following subclauses.

14.3.2	Information flows

14.3.2.1	Network resource adaptation request

Table 14.3.2.1-1 describes the information flow network resource adaptation request from the VAL/SEALDD server to the NRM server.

Table 14.3.2.1-1: Network resource adaptation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UEs | O (see NOTE) | List of VAL UEs or single VAL UEs for whom the network resource is required |
| >VAL UE/user ID | O | Identity of the VAL UE |
| > IP address | O | IP address of the VAL UE |
| VAL group ID | O (see NOTE) | The VAL group ID for whom the network resource adaptation occurs. |
| Resource adaptation requirement | M | The resource adaptation requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. bandwidth, resource, VAL service ID). |
| BDT Reference ID | O | Indicates the Background data transfer Reference ID provided by the 3GPP network |
| NOTE: Either of the information elements should be present. |  |  |

14.3.2.2	Network resource adaptation response

Table 14.3.2.2-1 describes the information flow network resource adaptation response from the NRM server to the VAL server.

Table 14.3.2.2-1: Network resource adaptation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource adaptation with the underlying network. The response may also include an updated value for some of the parameters included in the network resource adaptation request (e.g. negotiation of resource offering) |

14.3.2.2a	Network resource adaptation modification request

Table 14.3.2.2a-1 describes the information flow for the network resource adaptation modification request for a VAL UE ID or VAL user ID from VAL server to NRM server for unicast resources.

Table 14.3.2.2a-1: Network resource adaptation modification request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UEs | O (see NOTE) | List of VAL UEs or single VAL UE for whom the network resource is required |
| >VAL UE/user ID | O | Identity of the VAL UE |
| > IP address | O | IP address of the VAL UE |
| VAL group ID | O (see NOTE) | The VAL group ID for whom the network resource adaptation modification occurs. |
| Resource adaptation requirement | O | VAL service requirements for unicast resource (e.g. VAL service ID, Bitrate) |
| BDT Reference ID | O | Indicates the Background data transfer Reference ID provided by the 3GPP network |
| NOTE: Either of the information elements should be present. |  |  |

14.3.2.2b	Network resource adaptation modification response

Table 14.3.2.2b-1 describes the information flow for the network resource adaptation modification response from NRM server to VAL server for unicast resources.

Table 14.3.2.2b-1: Network resource adaptation modification response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the resource modification operation. |

**************************The second change******************************************************

14.3.3.3	Unicast resource management without SIP core

14.3.3.3.1	Network resource adaptation

14.3.3.3.1.1	General

This subclause describes the procedure for network resource adaptation using PCC procedures. This procedure satisfies the requirements for requesting unicast resources and modification to already allocated unicast resources to VAL communications.

14.3.3.3.1.2	Procedure

Figure 14.3.3.3.1.2-1 illustrates the procedure for the network resource adaptation.

Figure 14.3.3.3.1.2-1: Network resource adaptation

1.	The VAL server (or SEALDD server) sends a network resource adaptation request to the NRM server for one or more users belonging to one or more VAL services, and may comprise one or more VAL UEs that will have updated resource requirement. This requirement may be in the form of exact resources /resource pools to be used or indication of bandwidth increase/decrease for the corresponding VAL UEs or group of VAL UEs. The BDT Reference ID may also be included for setting BDT policies.

NOTE:	The SEALDD server can use the NRM netwok resource adaption service as per the procedure defined in 3GPP TS 23.433 clause 9.11 SEALDD Background data transfer.

2.	The NRM server processes the request and applies / enforces the resource adaptation per VAL UE.

3.	The NRM server retrieves UE IP address by using event monitoring capability for PDU session status (or PDN connectivity status) if not provided in step 1 and initiates the PCC procedures for each VAL UE.

4.	The NRM server provides a network resource adaptation response to the VAL server, providing information on the fulfilment of the network resource adaptation request. This will include information either per VAL UE or per set of VAL UEs, as indicated by the request of the VAL server in step 1.

14.3.3.3.2	Request for unicast resources at VAL service communication establishment

14.3.3.3.2.1	General

The procedure defined in this subclause specifies how network resources are requested at VAL service communication establishment. If concurrent sessions are used the NRM server may utilize the capability of resource sharing specified for underlying network policy and charging functions. The request for resources includes application type, bandwidth, priority, application identifier and resource sharing information.

14.3.3.3.2.2	Procedure

The procedure is generic to any type of session establishment that requires requests for network resources.

Procedures in figure 14.3.3.3.2.2-1 are the signalling procedures for the requesting resource at session establishment.

Pre-condition:

-	The VAL client has requested VAL service communication with the VAL server.

Figure 14.3.3.3.2.2-1: Resource request at VAL service communication establishment

1.	The VAL server sends a network resource adaptation request to the NRM server for a VAL UE or a VAL user ID.

2.	The NRM server evaluates the need for network resources and use of resource sharing.

3.	The NRM server retrieves UE IP address by using event monitoring capability for PDU session status (or PDN connectivity status) if not provided in step1, and then PCC procedures are initiated from NRM server.

4.	The NRM server sends a network resource adaptation response to the VAL server.

5.	The VAL service communication is established, and resources have been allocated.

**************************The end of the changes*******************************************