3GPP TS 37.571-1 V18.3.0 (2025-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

User Equipment (UE) conformance specification for UE positioning;

Part 1: Conformance test specification

(Release 18)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	51

Introduction	51

1	Scope	52

2	References	52

3	Definitions, symbols and abbreviations	55

3.1	Definitions	55

3.2	Symbols	55

3.3	Abbreviations	56

4	General test conditions	57

4.1	Introduction	57

4.2	GNSS test conditions	58

4.2.0	General	58

4.2.1	GNSS signals	58

4.2.2	GNSS frequency	58

4.2.3	GNSS static propagation conditions	58

4.2.4	GNSS multi-path conditions	58

4.2.5	UEs supporting multiple satellite signals	59

4.2.6	GNSS multi System Time Offsets	59

4.3	UTRA test conditions	60

4.3.1	UTRA frequency band and frequency range	60

4.3.2	UTRA frequency	60

4.3.3	Sensors	60

4.4	E-UTRA test conditions	60

4.4.1	E-UTRA frequency band and frequency range	60

4.4.2	Groups of bands	60

4.4.3	Sensors	60

4.5	A-GNSS test conditions	61

4.5.1	General	61

4.5.2	UTRAN measurement parameters	61

4.5.2.1	UE based A-GNSS measurement parameters	61

4.5.2.2	UE assisted A-GNSS measurement parameters	61

4.5.2.3	2D position error	61

4.5.2.4	Response time	61

4.5.3	E-UTRAN and NR measurement parameters	61

4.5.3.1	UE based A-GNSS measurement parameters	61

4.5.3.2	UE assisted A-GNSS measurement parameters	61

4.5.3.3	2D Error definition	62

4.5.3.4	Response time	62

4.5.4	Converting A-GNSS UE-assisted measurement reports into position estimates	62

4.6	ECID test conditions	62

4.6.1	Simulated cells	62

4.6.2	Propagation conditions	62

4.6.2.1	Static	62

4.6.2.2	Multi-path fading	62

4.6.3	UE Rx - Tx time difference reporting range	62

4.7	OTDOA test conditions	63

4.7.1	Simulated cells	63

4.7.2	Propagation conditions	64

4.7.2.1	Static	64

4.7.2.2	Multi-path fading	64

4.7.3	Response time	64

4.7.4	RSTD reporting range	64

4.7.5	RSTD Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	65

4.8	MBS test conditions	65

4.8.1	MBS signals	65

4.8.2	Propagation conditions	66

4.8.2.1	Static	66

4.8.2.2	Multi-path fading	66

4.8.3	Response time	66

4.9	WLAN test conditions	66

4.9.1	Simulated WLAN Access Points	66

4.9.2	Propagation conditions	66

4.9.2.1	Static	66

4.9.3	Response time	66

4.9.4	Void	67

4.10	BLE test conditions	67

4.10.1	Simulated BLE	67

4.10.2	Propagation conditions	67

4.10.2.1	Static	67

4.10.3	Response time	67

4.11	NB-IOT test conditions	67

4.11.1	Groups of bands	67

4.11.2	NB-IOT inband mode	67

4.12	NR test conditions	67

4.12.1	NR terminology	67

4.12.2	NR frequency band and frequency range	68

4.12.3	Groups of bands	68

4.12.4	Sensors	68

4.12.5	Default signal conditions for FR2	68

4.12.6	Frequency Bands for Testing	69

4.12.6.1	EN-DC band combination groups	69

4.12.6.2	Applicable EN-DC band combinations for performing A-GNSS Sensitivity Requirements	69

4.12.6.3	Test frequencies for EN-DC band combinations	69

4.14	Multi-RTT test conditions	69

4.14.1	Simulated cells	69

4.14.2	Propagation conditions	69

4.14.3	Measurement Reporting Requirements	70

4.14.3.1	Measurement Reporting Requirements in RRC_CONNECTED state	70

4.14.3.2	Measurement Reporting Requirements in RRC_INACTIVE state	70

4.14.4	Measurement Period Requirements	70

4.14.4.1	Measurement Period Requirements in RRC_CONNECTED state within Measurement Gaps	70

4.14.4.2	Measurement Period Requirements in RRC_CONNECTED state without Measurement Gaps	72

4.14.4.2A	Measurement Period Requirements in RRC_CONNECTED state with Bandwidth Aggregation	72

4.14.4.3	Measurement Period Requirements in RRC_INACTIVE state	72

4.14.4.4	Measurement Period Requirements in RRC_INACTIVE state with Bandwidth Aggregation	72

4.14.5	Measurement Accuracy Requirements	72

4.14.5B	Measurement Accuracy Requirements with Bandwidth Aggregation	84

4.14.6	Reporting mapping	85

4.14.6.1	 Absolute UE Rx-Tx Measurement Report Mapping	85

4.14.6.2	 Differential UE Rx-Tx Measurement Report Mapping	87

4.14.6.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	88

4.14A	Multi-RTT test conditions for Redcap UE	90

4.14A.1	Simulated cells	90

4.14A.2	Propagation conditions	90

4.14A.3	Measurement Reporting Requirements	90

4.14A.3.1	Measurement Reporting Requirements in RRC_CONNECTED state without RX FH with Measurement Gaps	90

4.14A.3.2	Measurement Reporting Requirements in RRC_INACTIVE state	90

4.14A.4	Measurement Period Requirements	91

4.14A.4.1	Measurement Reporting Requirements in RRC_CONNECTED state without RX FH within Measurement Gaps	91

4.14A.4.2	Measurement Reporting Requirements in RRC_CONNECTED state without RX FH without Measurement Gaps	91

4.14A.4.3	Measurement Reporting Requirements in RRC_CONNECTED state without RX FH within both Measurement Gaps and PPW	91

4.14A.4.4	Measurement Reporting Requirements in RRC_CONNECTED state with RX FH	91

4.14A.4.5	Measurement Period Requirements in RRC_INACTIVE state without RX FH	91

4.14A.4.6	Measurement Period Requirements in RRC_INACTIVE state with RX FH	91

4.14A.5	Measurement Accuracy Requirements	91

4.14A.5.1	Measurement Accuracy Requirements for 2RX RedCap UE without RX FH	91

4.14A.5.2	Measurement Accuracy Requirements for 1RX RedCap UE without RX FH	91

4.14A.5.3	Measurement Accuracy Requirements for 2RX RedCap UE with RX FH	91

4.14A.5.4	Measurement Accuracy Requirements for 1RX RedCap UE with RX FH	91

4.14A.6	Reporting mapping	91

4.14A.6.1	Absolute UE Rx-Tx Measurement Report Mapping	91

4.14A.6.2	Differential UE Rx-Tx Measurement Report Mapping	92

4.14A.6.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	92

4.15	DL-TDOA test conditions	92

4.15.1	Simulated cells	92

4.15.2	Propagation conditions	92

4.15.3	Measurement Reporting Requirements	92

4.15.3.1	Measurement Reporting Requirements in RRC_CONNECTED state	92

4.15.3.2	Measurement Reporting Requirements in RRC_INACTIVE state	92

4.15.4	Measurement Period Requirements	92

4.15.4.1	Measurement Period Requirements within Measurement Gaps	92

4.15.4.2	Measurement Period Requirements without Measurement Gaps	95

4.15.4.3	Measurement Period Requirements in RRC_INACTIVE state	95

4.15.4.4	Measurement Period Requirements in RRC_INACTIVE state with Bandwidth Aggregation	95

4.15.5	Measurement Accuracy Requirements	98

4.15.5A	Measurement Accuracy Requirements with Bandwidth Aggregation	106

4.15.6	Reporting mapping	113

4.15.6.1	 Absolute DL RSTD Measurement Reporting	113

4.15.6.2	 Differential Reporting for DL RSTD Measurement	115

4.15.6.3	Additional Path Report Mapping for DL RSTD	117

4.16	DL-AoD test conditions	119

4.16.1	Simulated cells	119

4.16.2	Propagation conditions	119

4.16.3	Measurement Reporting Requirements	119

4.16.3.1	Measurement Reporting Requirements in RRC_CONNECTED state	119

4.16.3.2	Measurement Reporting Requirements in RRC_INACTIVE state	119

4.16.4	PRS-RSRP Measurement Period Requirements	120

4.16.4.1	Measurement Period Requirements within Measurement Gaps	120

4.16.4.2	Measurement Period Requirements without Measurement Gaps	121

4.16.4.3	Measurement Period Requirements in RRC_INACTIVE state	121

4.16.5	PRS-RSRP Measurement Accuracy Requirements	122

4.16.6	PRS-RSRP Reporting mapping	129

4.16.6.1	Absolute PRS-RSRP Measurement Report Mapping	129

4.16.6.2	Differential Report Mapping for PRS-RSRP Measurement	130

4.16.7	PRS-RSRPP Measurement Period Requirements	132

4.16.8	PRS-RSRPP Measurement Accuracy Requirements	132

4.16.9	PRS-RSRPP Reporting mapping	133

4.16.9.1	 Absolute PRS-RSRPP Measurement Report Mapping	133

4.16.9.2	 Differential Report Mapping for PRS-RSRPP Measurement	133

4A	Support of 4 Rx capable UEs	134

4A.0	Introduction	134

4A.1	RAT Independent Tests	135

4A.2	RAT Dependent Tests	135

4A.2.1	Principle of testing	135

4A.2.1.1	Single carrier tests	135

4A.2.1.2	Carrier Aggregation tests	135

4A.2.1.3	Antenna connection for bands where 2RX is supported	135

4A.2.1.4	Antenna connection for bands where 4RX is supported	135

4B	Applicability of tests for types and Categories of UE	135

4B.1	Introduction	135

4B.2	Applicability of requirements and tests	135

5	UTRA A-GPS Minimum Performance requirements	138

5.1	General	138

5.2	Sensitivity	138

5.2.1	Sensitivity Coarse Time Assistance	138

5.2.1.1	Definition and applicability	138

5.2.1.2	Minimum requirements	138

5.2.1.3	Test purpose	138

5.2.1.4	Method of test	139

5.2.1.4.1	Initial conditions	139

5.2.1.4.2	Procedure	139

5.2.1.5	Test Requirements	139

5.2.2	Sensitivity Fine Time Assistance	140

5.2.2.1	Definition and applicability	140

5.2.2.2	Minimum requirements	140

5.2.2.3	Test purpose	141

5.2.2.4	Method of test	141

5.2.2.4.1	Initial conditions	141

5.2.2.4.2	Procedure	141

5.2.2.5	Test Requirements	141

5.3	Nominal Accuracy	142

5.3.1	Definition and applicability	142

5.3.2	Minimum requirements	142

5.3.3	Test purpose	142

5.3.4	Method of test	143

5.3.4.1	Initial conditions	143

5.3.4.2	Procedure	143

5.3.5	Test Requirements	144

5.4	Dynamic Range	144

5.4.1	Definition and applicability	144

5.4.2	Minimum requirements	144

5.4.3	Test purpose	145

5.4.4	Method of test	145

5.4.4.1	Initial conditions	145

5.4.4.2	Procedure	145

5.4.5	Test Requirements	146

5.5	Multi-path Performance	146

5.5.1	Definition and applicability	146

5.5.2	Minimum requirements	146

5.5.3	Test purpose	147

5.5.4	Method of test	147

5.5.4.1	Initial conditions	147

5.5.4.2	Procedure	147

5.5.5	Test Requirements	148

5.6	Moving Scenario and Periodic Update Performance	148

5.6.1	Definition and applicability	148

5.6.2	Minimum requirements	148

5.6.3	Test purpose	149

5.6.4	Method of test	149

5.6.4.1	Initial conditions	149

5.6.4.2	Procedure	150

5.6.5	Test Requirements	151

6	UTRA A-GNSS Minimum Performance requirements	152

6.1	General	152

6.2	Sensitivity	152

6.2.1	Sensitivity Coarse Time Assistance	152

6.2.1.1	Definition and applicability	152

6.2.1.2	Minimum requirements	152

6.2.1.3	Test purpose	153

6.2.1.4	Method of test	153

6.2.1.4.1	Initial conditions	153

6.2.1.4.2	Procedure	154

6.2.1.5	Test Requirements	154

6.2.2	Sensitivity Fine Time Assistance	155

6.2.2.1	Definition and applicability	155

6.2.2.2	Minimum requirements	156

6.2.2.3	Test purpose	156

6.2.2.4	Method of test	157

6.2.2.4.1	Initial conditions	157

6.2.2.4.2	Procedure	157

6.2.2.5	Test Requirements	157

6.3	Nominal Accuracy	158

6.3.1	Definition and applicability	158

6.3.2	Minimum requirements	159

6.3.3	Test purpose	159

6.3.4	Method of test	159

6.3.4.1	Initial conditions	159

6.3.4.2	Procedure	160

6.3.5	Test Requirements	160

6.4	Dynamic Range	161

6.4.1	Definition and applicability	161

6.4.2	Minimum requirements	162

6.4.3	Test purpose	163

6.4.4	Method of test	163

6.4.4.1	Initial conditions	163

6.4.4.2	Procedure	163

6.4.5	Test Requirements	164

6.5	Multi-path Performance	164

6.5.1	Definition and applicability	164

6.5.2	Minimum requirements	165

6.5.3	Test purpose	165

6.5.4	Method of test	166

6.5.4.1	Initial conditions	166

6.5.4.2	Procedure	166

6.5.5	Test Requirements	167

6.6	Moving Scenario and Periodic Update Performance	168

6.6.1	Definition and applicability	168

6.6.2	Minimum requirements	168

6.6.3	Test purpose	169

6.6.4	Method of test	169

6.6.4.1	Initial conditions	169

6.6.4.2	Procedure	170

6.6.5	Test Requirements	170

7	E-UTRA A-GNSS minimum performance requirements	172

7.0	General	172

7.1	Sensitivity	172

7.1.1	Sensitivity Coarse time assistance	172

7.1.1.1	Sub-tests	172

7.1.1.2	Test purpose	172

7.1.1.3	Test applicability	172

7.1.1.4	Minimum conformance requirements	172

7.1.1.5	Test description	174

7.1.1.5.1	Initial conditions	174

7.1.1.5.2	Test procedure	174

7.1.1.5.3	Message contents	175

7.1.1.6	Test requirement	176

7.1.2	Sensitivity Fine time assistance	177

7.1.2.1	Sub-tests	177

7.1.2.2	Test purpose	178

7.1.2.3	Test applicability	178

7.1.2.4	Minimum conformance requirements	178

7.1.2.5	Test description	179

7.1.2.5.1	Initial conditions	179

7.1.2.5.2	Test procedure	179

7.1.2.5.3	Message contents	180

7.1.2.6	Test requirement	181

7.2	Nominal Accuracy	182

7.2.1	Sub-tests	182

7.2.2	Test purpose	183

7.2.3	Test applicability	183

7.2.4	Minimum conformance requirements	183

7.2.5	Test description	184

7.2.5.1	Initial conditions	184

7.2.5.2	Test procedure	184

7.2.5.3	Message contents	185

7.2.6	Test requirement	186

7.3	Dynamic Range	187

7.3.1	Sub-tests	187

7.3.2	Test purpose	188

7.3.3	Test applicability	188

7.3.4	Minimum conformance requirements	188

7.3.5	Test description	189

7.3.5.1	Initial conditions	189

7.3.5.2	Test procedure	190

7.3.5.3	Message contents	191

7.3.6	Test requirement	191

7.4	Multi-Path scenario	193

7.4.1	Sub-tests	193

7.4.2	Test purpose	193

7.4.3	Test applicability	193

7.4.4	Minimum conformance requirements	193

7.4.5	Test description	194

7.4.5.1	Initial conditions	194

7.4.5.2	Test procedure	195

7.4.5.3	Message contents	196

7.4.6	Test requirement	196

7.5	Moving scenario and periodic update (Rel-9 to Rel-13)	198

7.5.1	Sub-tests	198

7.5.2	Test purpose	198

7.5.3	Test applicability	198

7.5.4	Minimum conformance requirements	198

7.5.5	Test description	199

7.5.5.1	Initial conditions	199

7.5.5.2	Test procedure	200

7.5.5.3	Message contents	201

7.5.6	Test requirement	202

7.5A	Moving scenario and periodic update (Rel-14 onwards)	203

7.5A.1	Sub-tests	203

7.5A.2	Test purpose	203

7.5A.3	Test applicability	204

7.5A.4	Minimum conformance requirements	204

7.5A.5	Test description	204

7.5A.6	Test requirement	204

8	E-UTRA ECID measurement requirements	204

8.0	General	204

8.1	UE Rx – Tx Time Difference	204

8.1.1	E-UTRAN FDD UE Rx – Tx time difference case (Rel-9 to Rel-11)	204

8.1.1.1	Test purpose	204

8.1.1.2	Test applicability	204

8.1.1.3	Minimum conformance requirements	204

8.1.1.4	Test description	205

8.1.1.4.1	Initial conditions	205

8.1.1.4.2	Test procedure	206

8.1.1.4.3	Message contents	206

8.1.1.5	Test requirement	210

8.1.1A	E-UTRAN FDD UE Rx – Tx time difference case (Rel-12 onwards)	211

8.1.1A.1	Test purpose	211

8.1.1A.2	Test applicability	211

8.1.1A.3	Minimum conformance requirements	211

8.1.1A.4	Test description	212

8.1.1A.4.1	Initial conditions	212

8.1.1A.4.2	Test procedure	212

8.1.1A.4.3	Message contents	212

8.1.1A.5	Test requirement	212

8.1.1B	E-UTRAN FDD UE Rx – Tx time difference case for UE Category 1bis	213

8.1.1B.1	Test purpose	213

8.1.1B.2	Test applicability	213

8.1.1B.3	Minimum conformance requirements	213

8.1.1B.4	Test description	213

8.1.1B.4.1	Initial conditions	213

8.1.1B.4.2	Test procedure	213

8.1.1B.4.3	Message contents	213

8.1.1B.5	Test requirement	213

8.1.2	E-UTRAN TDD UE Rx – Tx time difference case (Rel-9 to Rel-11)	213

8.1.2.1	Test purpose	213

8.1.2.2	Test applicability	213

8.1.2.3	Minimum conformance requirements	213

8.1.2.4	Test description	214

8.1.2.4.1	Initial conditions	214

8.1.2.4.2	Test procedure	214

8.1.2.4.3	Message contents	215

8.1.2.5	Test requirement	219

8.1.2A	E-UTRAN TDD UE Rx – Tx time difference case (Rel-12 onwards)	221

8.1.2A.1	Test purpose	221

8.1.2A.2	Test applicability	221

8.1.2A.3	Minimum conformance requirements	221

8.1.2A.4	Test description	221

8.1.2A.4.1	Initial conditions	222

8.1.2A.4.2	Test procedure	222

8.1.2A.4.3	Message contents	222

8.1.2A.5	Test requirement	222

8.1.2B	E-UTRAN TDD UE Rx – Tx time difference case for UE Category 1bis	222

8.1.2B.1	Test purpose	222

8.1.2B.2	Test applicability	222

8.1.2B.3	Minimum conformance requirements	222

8.1.2B.4	Test description	222

8.1.2B.4.1	Initial conditions	222

8.1.2B.4.2	Test procedure	222

8.1.2B.4.3	Message contents	222

8.1.2B.5	Test requirement	223

8.1.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC)	223

8.1.3.1	Test purpose	223

8.1.3.2	Test applicability	223

8.1.3.3	Minimum conformance requirements	223

8.1.3.4	Test description	224

8.1.3.4.1	Initial conditions	224

8.1.3.4.2	Test procedure	225

8.1.3.4.3	Message contents	226

8.1.3.5	Test requirement	229

8.1.4	E-UTRAN TDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC)	231

8.1.4.1	Test purpose	231

8.1.4.2	Test applicability	231

8.1.4.3	Minimum conformance requirements	231

8.1.4.4	Test description	232

8.1.4.4.1	Initial conditions	233

8.1.4.4.2	Test procedure	233

8.1.4.4.3	Message contents	234

8.1.4.5	Test requirement	238

8.1.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC)	240

8.1.5.1	Test purpose	240

8.1.5.2	Test applicability	240

8.1.5.3	Minimum conformance requirements	240

8.1.5.4	Test description	242

8.1.5.4.1	Initial conditions	242

8.1.5.4.2	Test procedure	243

8.1.5.4.3	Message contents	244

8.1.5.5	Test requirement	249

8.1.6	E-UTRAN TDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC)	251

8.1.6.1	Test purpose	251

8.1.6.2	Test applicability	251

8.1.6.3	Minimum conformance requirements	251

8.1.6.4	Test description	253

8.1.6.4.1	Initial conditions	253

8.1.6.4.2	Test procedure	254

8.1.6.4.3	Message contents	255

8.1.6.5	Test requirement	260

8.1.7	E-UTRAN FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA	262

8.1.7.1	Test purpose	262

8.1.7.2	Test applicability	262

8.1.7.3	Minimum conformance requirements	262

8.1.7.4	Test description	263

8.1.7.4.1	Initial conditions	263

8.1.7.4.2	Test procedure	264

8.1.7.4.3	Message contents	264

8.1.7.5	Test requirement	266

8.1.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA	268

8.1.8.1	Test purpose	268

8.1.8.2	Test applicability	268

8.1.8.3	Minimum conformance requirements	268

8.1.8.4	Test description	268

8.1.8.4.1	Initial conditions	268

8.1.8.4.2	Test procedure	268

8.1.8.4.3	Message contents	268

8.1.8.5	Test requirement	268

8.1.9	E-UTRAN TDD UE Rx-Tx time difference case for Category M1/M2 UE in CEModeA	270

8.1.9.1	Test purpose	270

8.1.9.2	Test applicability	270

8.1.9.3	Minimum conformance requirements	270

8.1.9.4	Test description	270

8.1.9.4.1	Initial conditions	270

8.1.9.4.2	Test procedure	270

8.1.9.4.3	Message contents	270

8.1.9.5	Test requirement	271

9	E-UTRA OTDOA measurement requirements	273

9.0	General	273

9.1	RSTD Intra-Frequency Measurements	273

9.1.1	FDD RSTD Measurement Reporting Delay	273

9.1.1.1	Test purpose	273

9.1.1.2	Test applicability	273

9.1.1.3	Minimum conformance requirements	273

9.1.1.4	Test description	275

9.1.1.4.1	Initial conditions	275

9.1.1.4.2	Test procedure	278

9.1.1.4.3	Message contents	279

9.1.1.5	Test requirement	282

9.1.1A	FDD RSTD Measurement Reporting Delay for UE Category 1bis	285

9.1.1A.1	Test purpose	285

9.1.1A.2	Test applicability	285

9.1.1A.3	Minimum conformance requirements	285

9.1.1A.4	Test description	285

9.1.1A.4.1	Initial conditions	285

9.1.1A.4.2	Test procedure	288

9.1.1A.4.3	Message contents	288

9.1.1A.5	Test requirement	288

9.1.2	TDD RSTD Measurement Reporting Delay	289

9.1.2.1	Test purpose	289

9.1.2.2	Test applicability	289

9.1.2.3	Minimum conformance requirements	289

9.1.2.4	Test description	290

9.1.2.4.1	Initial conditions	290

9.1.2.4.2	Test procedure	293

9.1.2.4.3	Message contents	294

9.1.2.5	Test requirement	297

9.1.2A	TDD RSTD Measurement Reporting Delay for UE Category 1bis	300

9.1.2A.1	Test purpose	300

9.1.2A.2	Test applicability	300

9.1.2A.3	Minimum conformance requirements	300

9.1.2A.4	Test description	300

9.1.2A.4.1	Initial conditions	300

9.1.2A.4.2	Test procedure	303

9.1.2A.4.3	Message contents	303

9.1.2A.5	Test requirement	303

9.1.3	FDD RSTD Measurement Accuracy	304

9.1.3.1	Test purpose	304

9.1.3.2	Test applicability	304

9.1.3.3	Minimum conformance requirements	304

9.1.3.4	Test description	306

9.1.3.4.1	Initial conditions	306

9.1.3.4.2	Test procedure	309

9.1.3.4.3	Message contents	310

9.1.3.5	Test requirement	314

9.1.3A	FDD RSTD Measurement Accuracy for UE Category 1bis	315

9.1.3A.1	Test purpose	315

9.1.3A.2	Test applicability	315

9.1.3A.3	Minimum conformance requirements	315

9.1.3A.4	Test description	315

9.1.3A.4.1	Initial conditions	315

9.1.3A.4.2	Test procedure	318

9.1.3A.4.3	Message contents	318

9.1.3A.5	Test requirement	318

9.1.4	TDD RSTD Measurement Accuracy	319

9.1.4.1	Test purpose	319

9.1.4.2	Test applicability	319

9.1.4.3	Minimum conformance requirements	319

9.1.4.4	Test description	319

9.1.4.4.1	Initial conditions	319

9.1.4.4.2	Test procedure	322

9.1.4.4.3	Message contents	323

9.1.4.5	Test requirement	327

9.1.4A	TDD RSTD Measurement Accuracy for UE Category 1bis	328

9.1.4A.1	Test purpose	328

9.1.4A.2	Test applicability	328

9.1.4A.3	Minimum conformance requirements	328

9.1.4A.4	Test description	328

9.1.4A.4.1	Initial conditions	328

9.1.4A.4.2	Test procedure	331

9.1.4A.4.3	Message contents	331

9.1.4A.5	Test requirement	331

9.2	RSTD Inter-Frequency Measurements	332

9.2.1	FDD-FDD inter-frequency RSTD measurement reporting delay	332

9.2.1.1	Test purpose	332

9.2.1.2	Test applicability	332

9.2.1.3	Minimum conformance requirements	332

9.2.1.4	Test description	333

9.2.1.4.1	Initial conditions	333

9.2.1.4.2	Test procedure	336

9.2.1.4.3	Message contents	337

9.2.1.5	Test requirement	341

9.2.1A	FDD-FDD inter-frequency RSTD Measurement Reporting Delay for UE Category 1bis	344

9.2.1A.1	Test purpose	344

9.2.1A.2	Test applicability	344

9.2.1A.3	Minimum conformance requirements	344

9.2.1A.4	Test description	344

9.2.1A.4.1	Initial conditions	344

9.2.1A.4.2	Test procedure	347

9.2.1A.4.3	Message contents	347

9.2.1A.5	Test requirement	348

9.2.2	 TDD-TDD inter-frequency RSTD measurement reporting delay	348

9.2.2.1	Test purpose	348

9.2.2.2	Test applicability	348

9.2.2.3	Minimum conformance requirements	349

9.2.2.4	Test description	350

9.2.2.4.1	Initial conditions	350

9.2.2.4.2	Test procedure	353

9.2.2.4.3	Message contents	354

9.2.2.5	Test requirement	358

9.2.2A	TDD-TDD inter-frequency RSTD Measurement Reporting Delay for UE Category 1bis	361

9.2.2A.1	Test purpose	361

9.2.2A.2	Test applicability	361

9.2.2A.3	Minimum conformance requirements	361

9.2.2A.4	Test description	361

9.2.2A.4.1	Initial conditions	361

9.2.2A.4.2	Test procedure	364

9.2.2A.4.3	Message contents	364

9.2.2A.5	Test requirement	365

9.2.3	Void	365

9.2.4	FDD-FDD inter-frequency RSTD Accuracy	365

9.2.4.1	Test purpose	365

9.2.4.2	Test applicability	365

9.2.4.3	Minimum conformance requirements	366

9.2.4.4	Test description	368

9.2.4.4.1	Initial conditions	368

9.2.4.4.2	Test procedure	370

9.2.4.4.3	Message contents	370

9.2.4.5	Test requirement	375

9.2.4A	FDD-FDD inter-frequency RSTD Accuracy for UE Category 1bis	376

9.2.4A.1	Test purpose	376

9.2.4A.2	Test applicability	376

9.2.4A.3	Minimum conformance requirements	376

9.2.4A.4	Test description	376

9.2.4A.4.1	Initial conditions	376

9.2.4A.4.2	Test procedure	378

9.2.4A.4.3	Message contents	378

9.2.4A.5	Test requirement	378

9.2.5	TDD-TDD inter-frequency RSTD Accuracy	378

9.2.5.1	Test purpose	378

9.2.5.2	Test applicability	378

9.2.5.3	Minimum conformance requirements	378

9.2.5.4	Test description	379

9.2.5.4.1	Initial conditions	379

9.2.5.4.2	Test procedure	381

9.2.5.4.3	Message contents	381

9.2.5.5	Test requirement	386

9.2.5A	TDD-TDD inter-frequency RSTD Accuracy for UE Category 1bis	388

9.2.5A.1	Test purpose	388

9.2.5A.2	Test applicability	388

9.2.5A.3	Minimum conformance requirements	388

9.2.5A.4	Test description	388

9.2.5A.4.1	Initial conditions	388

9.2.5A.4.2	Test procedure	390

9.2.5A.4.3	Message contents	390

9.2.5A.5	Test requirement	390

9.3	RSTD Intra-Frequency Measurements for UE Category M1/M2	391

9.3.1	FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A	391

9.3.1.1	FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1	391

9.3.1.1.1	Test purpose	391

9.3.1.1.2	Test applicability	391

9.3.1.1.3	Minimum conformance requirements	391

9.3.1.1.4	Test description	393

9.3.1.1.4.1	Initial conditions	393

9.3.1.1.4.2	Test procedure	395

9.3.1.1.4.3	Message contents	396

9.3.1.1.5	Test requirement	398

9.3.1.2	FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2	401

9.3.1.2.1	Test purpose	401

9.3.1.2.2	Test applicability	401

9.3.1.2.3	Minimum conformance requirements	401

9.3.1.2.4	Test description	401

9.3.1.2.4.1	Initial conditions	401

9.3.1.2.4.2	Test procedure	403

9.3.1.2.4.3	Message contents	403

9.3.1.2.5	Test requirement	403

9.3.2	HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A	403

9.3.2.1	HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1	403

9.3.2.1.1	Test purpose	403

9.3.2.1.2	Test applicability	403

9.3.2.1.3	Minimum conformance requirements	403

9.3.2.1.4	Test description	404

9.3.2.1.4.1	Initial conditions	404

9.3.2.1.4.2	Test procedure	406

9.3.2.1.4.3	Message contents	406

9.3.2.1.5	Test requirement	406

9.3.2.2	HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2	406

9.3.2.2.1	Test purpose	406

9.3.2.2.2	Test applicability	406

9.3.2.2.3	Minimum conformance requirements	406

9.3.2.2.4	Test description	406

9.3.2.2.4.1	Initial conditions	406

9.3.2.2.4.2	Test procedure	408

9.3.2.2.4.3	Message contents	408

9.3.2.2.5	Test requirement	408

9.3.3	TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A	408

9.3.3.1	TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1	408

9.3.3.1.1	Test purpose	408

9.3.3.1.2	Test applicability	408

9.3.3.1.3	Minimum conformance requirements	408

9.3.3.1.4	Test description	409

9.3.3.1.4.1	Initial conditions	409

9.3.3.1.4.2	Test procedure	411

9.3.3.1.4.3	Message contents	411

9.3.3.1.5	Test requirement	411

9.3.3.2	TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2	414

9.3.3.2.1	Test purpose	414

9.3.3.2.2	Test applicability	414

9.3.3.2.3	Minimum conformance requirements	414

9.3.3.2.4	Test description	414

9.3.3.2.4.1	Initial conditions	414

9.3.3.2.4.2	Test procedure	416

9.3.3.2.4.3	Message contents	416

9.3.3.2.5	Test requirement	416

9.3.4	FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B	416

9.3.4.1	FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1	416

9.3.4.1.1	Test purpose	416

9.3.4.1.2	Test applicability	417

9.3.4.1.3	Minimum conformance requirements	417

9.3.4.1.4	Test description	417

9.3.4.1.4.1	Initial conditions	417

9.3.4.1.4.2	Test procedure	419

9.3.4.1.4.3	Message contents	419

9.3.4.1.5	Test requirement	419

9.3.4.2	FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2	422

9.3.4.2.1	Test purpose	422

9.3.4.2.2	Test applicability	422

9.3.4.2.3	Minimum conformance requirements	422

9.3.4.2.4	Test description	422

9.3.4.2.4.1	Initial conditions	422

9.3.4.2.4.2	Test procedure	424

9.3.4.2.4.3	Message contents	424

9.3.4.2.5	Test requirement	424

9.3.5	HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B	425

9.3.5.1	HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1	425

9.3.5.1.1	Test purpose	425

9.3.5.1.2	Test applicability	425

9.3.5.1.3	Minimum conformance requirements	425

9.3.5.1.4	Test description	425

9.3.5.1.4.1	Initial conditions	425

9.3.5.1.4.2	Test procedure	427

9.3.5.1.4.3	Message contents	427

9.3.5.1.5	Test requirement	427

9.3.5.2	HD-FDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2	427

9.3.5.2.1	Test purpose	427

9.3.5.2.2	Test applicability	427

9.3.5.2.3	Minimum conformance requirements	427

9.3.5.2.4	Test description	427

9.3.5.2.4.1	Initial conditions	427

9.3.5.2.4.2	Test procedure	429

9.3.5.2.4.3	Message contents	429

9.3.5.2.5	Test requirement	429

9.3.6	TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B	429

9.3.6.1	TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1	429

9.3.6.1.1	Test purpose	429

9.3.6.1.2	Test applicability	429

9.3.6.1.3	Minimum conformance requirements	429

9.3.6.1.4	Test description	430

9.3.6.1.4.1	Initial conditions	430

9.3.6.1.4.2	Test procedure	432

9.3.6.1.4.3	Message contents	432

9.3.6.1.5	Test requirement	432

9.3.6.2	TDD intra-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2	435

9.3.6.2.1	Test purpose	435

9.3.6.2.2	Test applicability	435

9.3.6.2.3	Minimum conformance requirements	435

9.3.6.2.4	Test description	435

9.3.6.2.4.1	Initial conditions	435

9.3.6.2.4.2	Test procedure	437

9.3.6.2.4.3	Message contents	437

9.3.6.2.5	Test requirement	437

9.3.7	FDD intra-frequency RSTD Measurement Accuracy in CE Mode A	438

9.3.7.1	FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1	438

9.3.7.1.1	Test purpose	438

9.3.7.1.2	Test applicability	438

9.3.7.1.3	Minimum conformance requirements	438

9.3.7.1.4	Test description	439

9.3.7.1.4.1	Initial conditions	439

9.3.7.1.4.2	Test procedure	442

9.3.7.1.4.3	Message contents	443

9.3.7.1.5	Test requirement	445

9.3.7.2	FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2	446

9.3.7.2.1	Test purpose	446

9.3.7.2.2	Test applicability	446

9.3.7.2.3	Minimum conformance requirements	447

9.3.7.2.4	Test description	448

9.3.7.2.4.1	Initial conditions	448

9.3.7.2.4.2	Test procedure	451

9.3.7.2.4.3	Message contents	451

9.3.7.2.5	Test requirement	451

9.3.8	HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A	451

9.3.8.1	HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1	451

9.3.8.1.1	Test purpose	451

9.3.8.1.2	Test applicability	451

9.3.8.1.3	Minimum conformance requirements	452

9.3.8.1.4	Test description	452

9.3.8.1.4.1	Initial conditions	452

9.3.8.1.4.2	Test procedure	454

9.3.8.1.4.3	Message contents	454

9.3.8.1.5	Test requirement	454

9.3.8.2	HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2	454

9.3.8.2.1	Test purpose	454

9.3.8.2.2	Test applicability	454

9.3.8.2.3	Minimum conformance requirements	454

9.3.8.2.4	Test description	454

9.3.8.2.4.1	Initial conditions	454

9.3.8.2.4.2	Test procedure	456

9.3.8.2.4.3	Message contents	456

9.3.8.2.5	Test requirement	456

9.3.9	TDD intra-frequency RSTD Measurement Accuracy in CE Mode A	456

9.3.9.1	TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M1	456

9.3.9.1.1	Test purpose	456

9.3.9.1.2	Test applicability	456

9.3.9.1.3	Minimum conformance requirements	456

9.3.9.1.4	Test description	457

9.3.9.1.4.1	Initial conditions	457

9.3.9.1.4.2	Test procedure	459

9.3.9.1.4.3	Message contents	459

9.3.9.1.5	Test requirement	459

9.3.9.2	TDD intra-frequency RSTD Measurement Accuracy in CE Mode A for Category M2	459

9.3.9.2.1	Test purpose	459

9.3.9.2.2	Test applicability	459

9.3.9.2.3	Minimum conformance requirements	459

9.3.9.2.4	Test description	460

9.3.9.2.4.1	Initial conditions	460

9.3.9.2.4.2	Test procedure	462

9.3.9.2.4.3	Message contents	462

9.3.9.2.5	Test requirement	462

9.3.10	FDD intra-frequency RSTD Measurement Accuracy in CE Mode B	462

9.3.10.1	FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1	462

9.3.10.1.1	Test purpose	462

9.3.10.1.2	Test applicability	463

9.3.10.1.3	Minimum conformance requirements	463

9.3.10.1.4	Test description	464

9.3.10.1.4.1	Initial conditions	464

9.3.10.1.4.2	Test procedure	466

9.3.10.1.4.3	Message contents	466

9.3.10.1.5	Test requirement	466

9.3.10.2	FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2	467

9.3.10.2.1	Test purpose	467

9.3.10.2.2	Test applicability	467

9.3.10.2.3	Minimum conformance requirements	468

9.3.10.2.4	Test description	469

9.3.10.2.4.1	Initial conditions	469

9.3.10.2.4.2	Test procedure	472

9.3.10.2.4.3	Message contents	472

9.3.10.2.5	Test requirement	473

9.3.11	HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B	473

9.3.11.1	HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1	473

9.3.11.1.1	Test purpose	473

9.3.11.1.2	Test applicability	473

9.3.11.1.3	Minimum conformance requirements	473

9.3.11.1.4	Test description	473

9.3.11.1.4.1	Initial conditions	473

9.3.11.1.4.2	Test procedure	475

9.3.11.1.4.3	Message contents	475

9.3.11.1.5	Test requirement	475

9.3.11.2	HD-FDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2	475

9.3.11.2.1	Test purpose	475

9.3.11.2.2	Test applicability	475

9.3.11.2.3	Minimum conformance requirements	475

9.3.11.2.4	Test description	475

9.3.11.2.4.1	Initial conditions	475

9.3.11.2.4.2	Test procedure	477

9.3.11.2.4.3	Message contents	477

9.3.11.2.5	Test requirement	477

9.3.12	TDD intra-frequency RSTD Measurement Accuracy in CE Mode B	477

9.3.12.1	TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M1	477

9.3.12.1.1	Test purpose	477

9.3.12.1.2	Test applicability	477

9.3.12.1.3	Minimum conformance requirements	477

9.3.12.1.4	Test description	478

9.3.12.1.4.1	Initial conditions	478

9.3.12.1.4.2	Test procedure	480

9.3.12.1.4.3	Message contents	480

9.3.12.1.5	Test requirement	480

9.3.12.2	TDD intra-frequency RSTD Measurement Accuracy in CE Mode B for Category M2	480

9.3.12.2.1	Test purpose	480

9.3.12.2.2	Test applicability	480

9.3.12.2.3	Minimum conformance requirements	480

9.3.12.2.4	Test description	481

9.3.12.2.4.1	Initial conditions	481

9.3.12.2.4.2	Test procedure	483

9.3.12.2.4.3	Message contents	483

9.3.12.2.5	Test requirement	483

9.3.13	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	483

9.3.13.1	Test purpose	483

9.3.13.2	Test applicability	484

9.3.13.3	Minimum conformance requirements	484

9.3.13.4	Test description	484

9.3.13.4.1	Initial conditions	484

9.3.13.4.2	Test procedure	486

9.3.13.4.3	Message contents	486

9.3.13.5	Test requirement	487

9.3.14	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	487

9.3.14.1	Test purpose	487

9.3.14.2	Test applicability	487

9.3.14.3	Minimum conformance requirements	488

9.3.14.4	Test description	488

9.3.14.4.1	Initial conditions	488

9.3.14.4.2	Test procedure	489

9.3.14.4.3	Message contents	489

9.3.14.5	Test requirement	490

9.3.15	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	490

9.3.15.1	Test purpose	490

9.3.15.2	Test applicability	490

9.3.15.3	Minimum conformance requirements	490

9.3.15.4	Test description	490

9.3.15.4.1	Initial conditions	490

9.3.15.4.2	Test procedure	492

9.3.15.4.3	Message contents	492

9.3.15.5	Test requirement	493

9.3.16	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	493

9.3.16.1	Test purpose	493

9.3.16.2	Test applicability	493

9.3.16.3	Minimum conformance requirements	493

9.3.16.4	Test description	493

9.3.16.4.1	Initial conditions	493

9.3.16.4.2	Test procedure	495

9.3.16.4.3	Message contents	495

9.3.16.5	Test requirement	495

9.3.17	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	496

9.3.17.1	Test purpose	496

9.3.17.2	Test applicability	496

9.3.17.3	Minimum conformance requirements	496

9.3.17.4	Test description	496

9.3.17.4.1	Initial conditions	496

9.3.17.4.2	Test procedure	498

9.3.17.4.3	Message contents	498

9.3.17.5	Test requirement	498

9.3.18	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	499

9.3.18.1	Test purpose	499

9.3.18.2	Test applicability	499

9.3.18.3	Minimum conformance requirements	499

9.3.18.4	Test description	499

9.3.18.4.1	Initial conditions	499

9.3.18.4.2	Test procedure	501

9.3.18.4.3	Message contents	501

9.3.18.5	Test requirement	502

9.4	RSTD Inter-Frequency Measurements for UE Category M1/M2	503

9.4.1	FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A	503

9.4.1.1	FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1	503

9.4.1.1.1	Test purpose	503

9.4.1.1.2	Test applicability	503

9.4.1.1.3	Minimum conformance requirements	503

9.4.1.1.4	Test description	504

9.4.1.1.4.1	Initial conditions	504

9.4.1.1.4.2	Test procedure	507

9.4.1.1.4.3	Message contents	508

9.4.1.1.5	Test requirement	509

9.4.1.2	FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2	512

9.4.1.2.1	Test purpose	512

9.4.1.2.2	Test applicability	512

9.4.1.2.3	Minimum conformance requirements	512

9.4.1.2.4	Test description	512

9.4.1.2.4.1	Initial conditions	512

9.4.1.2.4.2	Test procedure	514

9.4.1.2.4.3	Message contents	514

9.4.1.2.5	Test requirement	515

9.4.2	HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A	515

9.4.2.1	HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1	515

9.4.2.1.1	Test purpose	515

9.4.2.1.2	Test applicability	516

9.4.2.1.3	Minimum conformance requirements	516

9.4.2.1.4	Test description	516

9.4.2.1.4.1	Initial conditions	516

9.4.2.1.4.2	Test procedure	518

9.4.2.1.4.3	Message contents	518

9.4.2.1.5	Test requirement	518

9.4.2.2	HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2	518

9.4.2.2.1	Test purpose	518

9.4.2.2.2	Test applicability	518

9.4.2.2.3	Minimum conformance requirements	519

9.4.2.2.4	Test description	519

9.4.2.2.4.1	Initial conditions	519

9.4.2.2.4.2	Test procedure	521

9.4.2.2.4.3	Message contents	521

9.4.2.2.5	Test requirement	521

9.4.3	TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A	522

9.4.3.1	TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M1	522

9.4.3.1.1	Test purpose	522

9.4.3.1.2	Test applicability	522

9.4.3.1.3	Minimum conformance requirements	522

9.4.3.1.4	Test description	522

9.4.3.1.4.1	Initial conditions	522

9.4.3.1.4.2	Test procedure	524

9.4.3.1.4.3	Message contents	524

9.4.3.1.5	Test requirement	524

9.4.3.2	TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode A for Category M2	527

9.4.3.2.1	Test purpose	527

9.4.3.2.2	Test applicability	527

9.4.3.2.3	Minimum conformance requirements	527

9.4.3.2.4	Test description	527

9.4.3.2.4.1	Initial conditions	527

9.4.3.2.4.2	Test procedure	529

9.4.3.2.4.3	Message contents	529

9.4.3.2.5	Test requirement	529

9.4.4	FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B	530

9.4.4.1	FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1	530

9.4.4.1.1	Test purpose	530

9.4.4.1.2	Test applicability	530

9.4.4.1.3	Minimum conformance requirements	530

9.4.4.1.4	Test description	530

9.4.4.1.4.1	Initial conditions	530

9.4.4.1.4.2	Test procedure	532

9.4.4.1.4.3	Message contents	532

9.4.4.1.5	Test requirement	532

9.4.4.2	FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2	535

9.4.4.2.1	Test purpose	535

9.4.4.2.2	Test applicability	535

9.4.4.2.3	Minimum conformance requirements	535

9.4.4.2.4	Test description	535

9.4.4.2.4.1	Initial conditions	535

9.4.4.2.4.2	Test procedure	537

9.4.4.2.4.3	Message contents	537

9.4.4.2.5	Test requirement	538

9.4.5	HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B	538

9.4.5.1	HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1	538

9.4.5.1.1	Test purpose	538

9.4.5.1.2	Test applicability	538

9.4.5.1.3	Minimum conformance requirements	538

9.4.5.1.4	Test description	538

9.4.5.1.4.1	Initial conditions	538

9.4.5.1.4.2	Test procedure	540

9.4.5.1.4.3	Message contents	540

9.4.5.1.5	Test requirement	540

9.4.5.2	HD-FDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2	540

9.4.5.2.1	Test purpose	540

9.4.5.2.2	Test applicability	540

9.4.5.2.3	Minimum conformance requirements	540

9.4.5.2.4	Test description	540

9.4.5.2.4.1	Initial conditions	540

9.4.5.2.4.2	Test procedure	542

9.4.5.2.4.3	Message contents	542

9.4.5.2.5	Test requirement	542

9.4.6	TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B	543

9.4.6.1	TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M1	543

9.4.6.1.1	Test purpose	543

9.4.6.1.2	Test applicability	543

9.4.6.1.3	Minimum conformance requirements	543

9.4.6.1.4	Test description	543

9.4.6.1.4.1	Initial conditions	543

9.4.6.1.4.2	Test procedure	545

9.4.6.1.4.3	Message contents	545

9.4.6.1.5	Test requirement	545

9.4.6.2	TDD inter-frequency RSTD Measurement Reporting Delay in CE Mode B for Category M2	548

9.4.6.2.1	Test purpose	548

9.4.6.2.2	Test applicability	548

9.4.6.2.3	Minimum conformance requirements	548

9.4.6.2.4	Test description	548

9.4.6.2.4.1	Initial conditions	548

9.4.6.2.4.2	Test procedure	550

9.4.6.2.4.3	Message contents	550

9.4.6.2.5	Test requirement	550

9.4.7	FDD inter-frequency RSTD Measurement Accuracy in CE Mode A	551

9.4.7.1	FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1	551

9.4.7.1.1	Test purpose	551

9.4.7.1.2	Test applicability	551

9.4.7.1.3	Minimum conformance requirements	551

9.4.7.1.4	Test description	552

9.4.7.1.4.1	Initial conditions	552

9.4.7.1.4.2	Test procedure	555

9.4.7.1.4.3	Message contents	556

9.4.7.1.5	Test requirement	556

9.4.7.2	FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2	557

9.4.7.2.1	Test purpose	557

9.4.7.2.2	Test applicability	558

9.4.7.2.3	Minimum conformance requirements	558

9.4.7.2.4	Test description	559

9.4.7.2.4.1	Initial conditions	559

9.4.7.2.4.2	Test procedure	562

9.4.7.2.4.3	Message contents	562

9.4.7.2.5	Test requirement	562

9.4.8	HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A	563

9.4.8.1	HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1	563

9.4.8.1.1	Test purpose	563

9.4.8.1.2	Test applicability	563

9.4.8.1.3	Minimum conformance requirements	563

9.4.8.1.4	Test description	563

9.4.8.1.4.1	Initial conditions	563

9.4.8.1.4.2	Test procedure	565

9.4.8.1.4.3	Message contents	565

9.4.8.1.5	Test requirement	565

9.4.8.2	HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2	565

9.4.8.2.1	Test purpose	565

9.4.8.2.2	Test applicability	565

9.4.8.2.3	Minimum conformance requirements	565

9.4.8.2.4	Test description	565

9.4.8.2.4.1	Initial conditions	565

9.4.8.2.4.2	Test procedure	567

9.4.8.2.4.3	Message contents	567

9.4.8.2.5	Test requirement	567

9.4.9	TDD inter-frequency RSTD Measurement Accuracy in CE Mode A	567

9.4.9.1	TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M1	567

9.4.9.1.1	Test purpose	567

9.4.9.1.2	Test applicability	567

9.4.9.1.3	Minimum conformance requirements	567

9.4.9.1.4	Test description	567

9.4.9.1.4.1	Initial conditions	567

9.4.9.1.4.2	Test procedure	569

9.4.9.1.4.3	Message contents	569

9.4.9.1.5	Test requirement	569

9.4.9.2	TDD inter-frequency RSTD Measurement Accuracy in CE Mode A for Category M2	570

9.4.9.2.1	Test purpose	570

9.4.9.2.2	Test applicability	571

9.4.9.2.3	Minimum conformance requirements	571

9.4.9.2.4	Test description	571

9.4.9.2.4.1	Initial conditions	571

9.4.9.2.4.2	Test procedure	573

9.4.9.2.4.3	Message contents	573

9.4.9.2.5	Test requirement	573

9.4.10	FDD inter-frequency RSTD Measurement Accuracy in CE Mode B	573

9.4.10.1	FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1	573

9.4.10.1.1	Test purpose	573

9.4.10.1.2	Test applicability	573

9.4.10.1.3	Minimum conformance requirements	574

9.4.10.1.4	Test description	575

9.4.10.1.4.1	Initial conditions	575

9.4.10.1.4.2	Test procedure	577

9.4.10.1.4.3	Message contents	577

9.4.10.1.5	Test requirement	577

9.4.10.2	FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2	578

9.4.10.2.1	Test purpose	578

9.4.10.2.2	Test applicability	579

9.4.10.2.3	Minimum conformance requirements	579

9.4.10.2.4	Test description	581

9.4.10.2.4.1	Initial conditions	581

9.4.10.2.4.2	Test procedure	583

9.4.10.2.4.3	Message contents	583

9.4.10.2.5	Test requirement	583

9.4.11	HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B	584

9.4.11.1	HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1	584

9.4.11.1.1	Test purpose	584

9.4.11.1.2	Test applicability	584

9.4.11.1.3	Minimum conformance requirements	584

9.4.11.1.4	Test description	584

9.4.11.1.4.1	Initial conditions	584

9.4.11.1.4.2	Test procedure	586

9.4.11.1.4.3	Message contents	586

9.4.11.1.5	Test requirement	586

9.4.11.2	HD-FDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2	586

9.4.11.2.1	Test purpose	586

9.4.11.2.2	Test applicability	586

9.4.11.2.3	Minimum conformance requirements	586

9.4.11.2.4	Test description	586

9.4.11.2.4.1	Initial conditions	586

9.4.11.2.4.2	Test procedure	588

9.4.11.2.4.3	Message contents	588

9.4.11.2.5	Test requirement	588

9.4.12	TDD inter-frequency RSTD Measurement Accuracy in CE Mode B	588

9.4.12.1	TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M1	588

9.4.12.1.1	Test purpose	588

9.4.12.1.2	Test applicability	588

9.4.12.1.3	Minimum conformance requirements	588

9.4.12.1.4	Test description	589

9.4.12.1.4.1	Initial conditions	589

9.4.12.1.4.2	Test procedure	591

9.4.12.1.4.3	Message contents	591

9.4.12.1.5	Test requirement	591

9.4.12.2	TDD inter-frequency RSTD Measurement Accuracy in CE Mode B for Category M2	592

9.4.12.2.1	Test purpose	592

9.4.12.2.2	Test applicability	593

9.4.12.2.3	Minimum conformance requirements	593

9.4.12.2.4	Test description	593

9.4.12.2.4.1	Initial conditions	593

9.4.12.2.4.2	Test procedure	595

9.4.12.2.4.3	Message contents	595

9.4.12.2.5	Test requirement	595

9.4.13	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	595

9.4.13.1	Test purpose	595

9.4.13.2	Test applicability	595

9.4.13.3	Minimum conformance requirements	595

9.4.13.4	Test description	596

9.4.13.4.1	Initial conditions	596

9.4.13.4.2	Test procedure	599

9.4.13.4.3	Message contents	599

9.4.13.5	Test requirement	599

9.4.14	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	600

9.4.14.1	Test purpose	600

9.4.14.2	Test applicability	600

9.4.14.3	Minimum conformance requirements	600

9.4.14.4	Test description	600

9.4.14.4.1	Initial conditions	600

9.4.14.4.2	Test procedure	602

9.4.14.4.3	Message contents	602

9.4.14.5	Test requirement	603

9.4.15	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	603

9.4.15.1	Test purpose	603

9.4.15.2	Test applicability	603

9.4.15.3	Minimum conformance requirements	603

9.4.15.4	Test description	603

9.4.15.4.1	Initial conditions	603

9.4.15.4.2	Test procedure	606

9.4.15.4.3	Message contents	606

9.4.15.5	Test requirement	606

9.4.16	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	606

9.4.16.1	Test purpose	606

9.4.16.2	Test applicability	606

9.4.16.3	Minimum conformance requirements	606

9.4.16.4	Test description	606

9.4.16.4.1	Initial conditions	606

9.4.16.4.2	Test procedure	608

9.4.16.4.3	Message contents	609

9.4.16.5	Test requirement	609

9.4.17	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	609

9.4.17.1	Test purpose	609

9.4.17.2	Test applicability	609

9.4.17.3	Minimum conformance requirements	609

9.4.17.4	Test description	609

9.4.17.4.1	Initial conditions	609

9.4.17.4.2	Test procedure	611

9.4.17.4.3	Message contents	612

9.4.17.5	Test requirement	612

9.4.18	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	612

9.4.18.1	Test purpose	612

9.4.18.2	Test applicability	612

9.4.18.3	Minimum conformance requirements	612

9.4.18.4	Test description	612

9.4.18.4.1	Initial conditions	612

9.4.18.4.2	Test procedure	615

9.4.18.4.3	Message contents	615

9.4.18.5	Test requirement	615

9.5	HD-FDD RSTD Intra-Frequency Measurements for NB-IOT	616

9.5.1	HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage	616

9.5.1.1	Test purpose	616

9.5.1.2	Test applicability	616

9.5.1.3	Minimum conformance requirements	616

9.5.1.4	Test description	618

9.5.1.4.1	Initial conditions	618

9.5.1.4.2	Test procedure	619

9.5.1.4.3	Message contents	620

9.5.1.5	Test requirement	622

9.5.2	HD-FDD Intra frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage	624

9.5.2.1	Test purpose	624

9.5.2.2	Test applicability	625

9.5.2.3	Minimum conformance requirements	625

9.5.2.4	Test description	625

9.5.2.4.1	Initial conditions	625

9.5.2.4.2	Test procedure	627

9.5.2.4.3	Message contents	628

9.5.2.5	Test requirement	628

9.5.3	HD-FDD Intra frequency RSTD Measurement Reporting Delay for NB-IOT Standalone Mode in enhanced coverage	630

9.5.3.1	Test purpose	630

9.5.3.2	Test applicability	631

9.5.3.3	Minimum conformance requirements	631

9.5.3.4	Test description	632

9.5.3.4.1	Initial conditions	632

9.5.3.4.2	Test procedure	635

9.5.3.4.3	Message contents	636

9.5.3.5	Test requirement	636

9.5.4	Void	639

9.6	HD-FDD RSTD Inter-Frequency Measurements for NB-IOT	639

9.6.1	HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage	639

9.6.1.1	Test purpose	639

9.6.1.2	Test applicability	639

9.6.1.3	Minimum conformance requirements	639

9.6.1.4	Test description	641

9.6.1.4.1	Initial conditions	641

9.6.1.4.2	Test procedure	643

9.6.1.4.3	Message contents	644

9.6.1.5	Test requirement	645

9.6.2	HD-FDD Inter-Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage	647

9.6.2.1	Test purpose	647

9.6.2.2	Test applicability	648

9.6.2.3	Minimum conformance requirements	648

9.6.2.4	Test description	648

9.6.2.4.1	Initial conditions	648

9.6.2.4.2	Test procedure	650

9.6.2.4.3	Message contents	651

9.6.2.5	Test requirement	651

9.6.3	HD-FDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Standalone Mode in enhanced coverage	653

9.6.3.1	Test purpose	653

9.6.3.2	Test applicability	654

9.6.3.3	Minimum conformance requirements	654

9.6.3.4	Test description	655

9.6.3.4.1	Initial conditions	655

9.6.3.4.2	Test procedure	658

9.6.3.4.3	Message contents	659

9.6.3.5	Test requirement	659

9.6.4	Void	662

9.7	TDD RSTD Intra-Frequency Measurements for NB-IOT	662

9.7.1	TDD Intra Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage	662

9.7.2	TDD Intra Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage	667

9.7.3	TDD Intra Frequency RSTD Measurement Reporting Delay for NB-IOT Standalone Mode in enhanced coverage	673

9.8	TDD RSTD Inter-Frequency Measurements for NB-IOT	681

9.8.1	TDD Inter Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in normal coverage	681

9.8.2	TDD Inter Frequency RSTD Measurement Accuracy for NB-IOT Inband Mode in enhanced coverage	687

10	E-UTRA OTDOA measurement requirements for Carrier Aggregation	700

10.0	General	700

10.1	FDD RSTD Measurement Reporting Delay for Carrier Aggregation	700

10.1.1	Test purpose	700

10.1.2	Test applicability	700

10.1.3	Minimum conformance requirements	701

10.1.3.1	Measurements on the secondary component carrier	701

10.1.3.2	Measurements on both primary component carrier and secondary component carrier	701

10.1.4	Test description	701

10.1.4.1	Initial conditions	701

10.1.4.2	Test procedure	704

10.1.4.3	Message contents	706

10.1.5	Test requirement	710

10.1A	FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz Bandwidth	712

10.1A.1	Test purpose	712

10.1A.2	Test applicability	712

10.1A.3	Minimum conformance requirements	712

10.1A.4	Test description	712

10.1A.4.1	Initial conditions	712

10.1A.4.2	Test procedure	713

10.1A.4.3	Message contents	713

10.1A.5	Test requirement	713

10.1B	FDD RSTD Measurement Reporting Delay Carrier Aggregation for 5 MHz + 5 MHz Bandwidth	714

10.1B.1	Test purpose	714

10.1B.2	Test applicability	714

10.1B.3	Minimum conformance requirements	714

10.1B.4	Test description	714

10.1B.4.1	Initial conditions	714

10.1B.4.2	Test procedure	714

10.1B.4.3	Message contents	714

10.1B.5	Test requirement	715

10.1C	FDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz+5 MHz Bandwidth	715

10.1C.1	Test purpose	715

10.1C.2	Test applicability	715

10.1C.3	Minimum conformance requirements	715

10.1C.4	Test description	716

10.1C.4.1	Initial conditions	716

10.1C.4.2	Test procedure	716

10.1C.4.3	Message contents	716

10.1C.5	Test requirement	716

10.2	TDD RSTD Measurement Reporting Delay for Carrier Aggregation	717

10.2.1	Test purpose	717

10.2.2	Test applicability	717

10.2.3	Minimum conformance requirements	717

10.2.3.1	Measurements on the secondary component carrier	717

10.2.3.2	Measurements on both primary component carrier and secondary component carrier	717

10.2.4	Test description	718

10.2.4.1	Initial conditions	718

10.2.4.2	Test procedure	721

10.2.4.3	Message contents	723

10.2.5	Test requirement	727

10.2A	TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz Bandwidth	729

10.2A.1	Test purpose	729

10.2A.2	Test applicability	729

10.2A.3	Minimum conformance requirements	729

10.2A.4	Test description	729

10.2A.4.1	Initial conditions	729

10.2A.4.2	Test procedure	730

10.2A.4.3	Message contents	730

10.2A.5	Test requirement	730

10.2B	TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 5 MHz + 5 MHz Bandwidth	731

10.2B.1	Test purpose	731

10.2B.2	Test applicability	731

10.2B.3	Minimum conformance requirements	731

10.2B.4	Test description	731

10.2B.4.1	Initial conditions	731

10.2B.4.2	Test procedure	731

10.2B.4.3	Message contents	731

10.2B.5	Test requirement	732

10.2C	TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 10 MHz + 5 MHz Bandwidth	732

10.2C.1	Test purpose	732

10.2C.2	Test applicability	732

10.2C.3	Minimum conformance requirements	732

10.2C.4	Test description	733

10.2C.4.1	Initial conditions	733

10.2C.4.2	Test procedure	733

10.2C.4.3	Message contents	733

10.2C.5	Test requirement	733

10.2D	TDD RSTD Measurement Reporting Delay for Carrier Aggregation for 20 MHz +10 MHz Bandwidth	734

10.2D.1	Test purpose	734

10.2D.2	Test applicability	734

10.2D.3	Minimum conformance requirements	734

10.2D.4	Test description	734

10.2D.4.1	Initial conditions	734

10.2D.4.2	Test procedure	735

10.2D.4.3	Message contents	735

10.2D.5	Test requirement	735

10.3	FDD RSTD Measurement Accuracy for Carrier Aggregation	736

10.3.1	Test purpose	736

10.3.2	Test applicability	736

10.3.3	Minimum conformance requirements	736

10.3.4	Test description	736

10.3.4.1	Initial conditions	736

10.3.4.2	Test procedure	738

10.3.4.3	Message contents	739

10.3.5	Test requirement	742

10.3A	FDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz Bandwidth (Rel-10 and Rel-11)	743

10.3A.1	Test purpose	743

10.3A.2	Test applicability	743

10.3A.3	Minimum conformance requirements	744

10.3A.4	Test description	744

10.3A.4.1	Initial conditions	744

10.3A.4.2	Test procedure	744

10.3A.4.3	Message contents	744

10.3A.5	Test requirement	744

10.3A_1	FDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz Bandwidth (Rel-12 onwards)	745

10.3A_1.1	Test purpose	745

10.3A_1.2	Test applicability	745

10.3A_1.3	Minimum conformance requirements	745

10.3A_1.4	Test description	745

10.3A_1.4.1	Initial conditions	745

10.3A_1.4.2	Test procedure	745

10.3A_1.4.3	Message contents	745

10.3A_1.5	Test requirement	745

10.3B	FDD RSTD Measurement Accuracy for Carrier Aggregation for 5 MHz + 5 MHz Bandwidth	745

10.3B.1	Test purpose	745

10.3B.2	Test applicability	745

10.3B.3	Minimum conformance requirements	746

10.3B.4	Test description	746

10.3B.4.1	Initial conditions	746

10.3B.4.2	Test procedure	746

10.3B.4.3	Message contents	746

10.3B.5	Test requirement	746

10.3C	FDD RSTD Measurement Accuracy for Carrier Aggregation for 10 MHz + 5 MHz Bandwidth	747

10.3C.1	Test purpose	747

10.3C.2	Test applicability	747

10.3C.3	Minimum conformance requirements	747

10.3C.4	Test description	747

10.3C.4.1	Initial conditions	747

10.3C.4.2	Test procedure	748

10.3C.4.3	Message contents	748

10.3C.5	Test requirement	748

10.4	TDD RSTD Measurement Accuracy for Carrier Aggregation	749

10.4.1	Test purpose	749

10.4.2	Test applicability	749

10.4.3	Minimum conformance requirements	749

10.4.4	Test description	749

10.4.4.1	Initial conditions	749

10.4.4.2	Test procedure	752

10.4.4.3	Message contents	753

10.4.5	Test requirement	756

10.4A	TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz Bandwidth (Rel-10 and Rel-11)	757

10.4A.1	Test purpose	757

10.4A.2	Test applicability	757

10.4A.3	Minimum conformance requirements	758

10.4A.4	Test description	758

10.4A.4.1	Initial conditions	758

10.4A.4.2	Test procedure	758

10.4A.4.3	Message contents	758

10.4A.5	Test requirement	758

10.4A_1	TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz Bandwidth (Rel-12 onwards)	759

10.4A_1.1	Test purpose	759

10.4A_1.2	Test applicability	759

10.4A_1.3	Minimum conformance requirements	759

10.4A_1.4	Test description	759

10.4A_1.4.1	Initial conditions	759

10.4A_1.4.2	Test procedure	759

10.4A_1.4.3	Message contents	759

10.4A_1.5	Test requirement	759

10.4B	TDD RSTD Measurement Accuracy for Carrier Aggregation for 5 MHz + 5 MHz bandwidth	759

10.4B.1	Test purpose	759

10.4B.2	Test applicability	759

10.4B.3	Minimum conformance requirements	760

10.4B.4	Test description	760

10.4B.4.1	Initial conditions	760

10.4B.4.2	Test procedure	760

10.4B.4.3	Message contents	760

10.4B.5	Test requirement	760

10.4C	TDD RSTD Measurement Accuracy for Carrier Aggregation for 10 MHz + 5 MHz Bandwidth	761

10.4C.1	Test purpose	761

10.4C.2	Test applicability	761

10.4C.3	Minimum conformance requirements	761

10.4C.4	Test description	761

10.4C.4.1	Initial conditions	761

10.4C.4.2	Test procedure	762

10.4C.4.3	Message contents	762

10.4C.5	Test requirement	762

10.4D	TDD RSTD Measurement Accuracy for Carrier Aggregation for 20 MHz+10 MHz Bandwidth	763

10.4D.1	Test purpose	763

10.4D.2	Test applicability	763

10.4D.3	Minimum conformance requirements	763

10.4D.4	Test description	763

10.4D.4.1	Initial conditions	763

10.4D.4.2	Test procedure	763

10.4D.4.3	Message contents	763

10.4D.5	Test requirement	764

10.5	FDD 3 DL CA RSTD Measurement Reporting Delay	764

10.5.1	Test Purpose	764

10.5.2	Test applicability	764

10.5.3	Minimum conformance requirements	764

10.5.3.1	Measurements on the secondary component carrier	764

10.5.3.2	Measurements on both primary component carrier and secondary component carrier	764

10.5.3.3	Measurements on different secondary component carriers	765

10.5.4	Test description	765

10.5.4.1	Initial conditions	765

10.5.4.2	Test procedure	770

10.5.4.3	Message contents	771

10.5.5	Test Requirements	776

10.6	TDD 3 DL CA RSTD Measurement Reporting Delay	778

10.6.1	Test Purpose	778

10.6.2	Test applicability	778

10.6.3	Minimum conformance requirements	778

10.6.3.1	Measurements on the secondary component carrier	778

10.6.3.2	Measurements on both primary component carrier and secondary component carrier	778

10.6.3.3	Measurements on different secondary component carriers	779

10.6.4	Test description	779

10.6.4.1	Initial conditions	779

10.6.4.2	Test procedure	784

10.6.4.3	Message contents	785

10.6.5	Test Requirements	791

10.7	FDD RSTD Measurement Accuracy for 3DL Carrier Aggregation	794

10.7.1	Test purpose	794

10.7.2	Test applicability	794

10.7.3	Minimum conformance requirements	794

10.7.4	Test description	795

10.7.4.1	Initial conditions	795

10.7.4.2	Test procedure	798

10.7.4.3	Message contents	799

10.7.5	Test requirement	804

10.8	TDD RSTD Measurement Accuracy for 3DL Carrier Aggregation	805

10.8.1	Test purpose	805

10.8.2	Test applicability	805

10.8.3	Minimum conformance requirements	805

10.8.4	Test description	805

10.8.4.1	Initial conditions	805

10.8.4.2	Test procedure	809

10.8.4.3	Message contents	810

10.8.5	Test requirement	815

11	E-UTRA and NR MBS measurement requirements	816

11.0	General	816

11.1	MBS Measurement Reporting Delay (Release 13 only)	816

11.1.1	Test purpose	816

11.1.2	Test applicability	816

11.1.3	Minimum conformance requirements	816

11.1.4	Test description	816

11.1.4.1	Initial conditions	816

11.1.4.2	Test procedure	817

11.1.4.3	Message contents	817

11.1.5	Test requirement	819

11.1A	MBS Measurement Reporting Delay (Release 14 Onwards)	820

11.1A.1	Test purpose	820

11.1A.2	Test applicability	820

11.1A.3	Minimum conformance requirements	820

11.1A.4	Test description	820

11.1A.4.1	Initial conditions	820

11.1A.4.2	Test procedure	821

11.1A.4.3	Message contents	821

11.1A.5	Test requirement	822

11.1B	MBS Measurement Reporting Delay (NR)	823

11.1B.1	Test purpose	823

11.1B.2	Test applicability	823

11.1B.3	Minimum conformance requirements	823

11.1B.4	Test description	823

11.1B.4.1	Initial conditions	823

11.1B.4.2	Test procedure	823

11.1B.4.3	Message contents	823

11.1B.5	Test requirement	823

11.2	MBS Sensitivity Measurement Accuracy (Release 13 only)	823

11.2.1	Test purpose	823

11.2.2	Test applicability	823

11.2.3	Minimum conformance requirements	823

11.2.4	Test description	824

11.2.4.1	Initial conditions	824

11.2.4.2	Test procedure	824

11.2.4.3	Message contents	825

11.2.5	Test requirement	827

11.2A	MBS Sensitivity Measurement Accuracy (Release 14 Onwards)	828

11.2A.1	Test purpose	828

11.2A.2	Test applicability	828

11.2A.3	Minimum conformance requirements	829

11.2A.4	Test description	829

11.2A.4.1	Initial conditions	829

11.2A.4.2	Test procedure	829

11.2A.4.3	Message contents	829

11.2A.5	Test requirement	831

11.2B	MBS Sensitivity Measurement Accuracy (NR)	831

11.2B.1	Test purpose	831

11.2B.2	Test applicability	831

11.2B.3	Minimum conformance requirements	831

11.2B.4	Test description	831

11.2B.4.1	Initial conditions	831

11.2B.4.2	Test procedure	831

11.2B.4.3	Message contents	831

11.2B.5	Test requirement	831

11.3	MBS Nominal Measurement Accuracy (Release 13 only)	832

11.3.1	Test purpose	832

11.3.2	Test applicability	832

11.3.3	Minimum conformance requirements	832

11.3.4	Test description	832

11.3.4.1	Initial conditions	832

11.3.4.2	Test procedure	832

11.3.4.3	Message contents	833

11.3.5	Test requirement	835

11.3A	MBS Nominal Measurement Accuracy (Release 14 Onwards)	836

11.3A.1	Test purpose	836

11.3A.2	Test applicability	836

11.3A.3	Minimum conformance requirements	837

11.3A.4	Test description	837

11.3A.4.1	Initial conditions	837

11.3A.4.2	Test procedure	837

11.3A.4.3	Message contents	837

11.3A.5	Test requirement	839

11.3B	MBS Nominal Measurement Accuracy (NR)	839

11.3B.1	Test purpose	839

11.3B.2	Test applicability	839

11.3B.3	Minimum conformance requirements	839

11.3B.4	Test description	839

11.3B.4.1	Initial conditions	839

11.3B.4.2	Test procedure	839

11.3B.4.3	Message contents	839

11.3B.5	Test requirement	839

11.4	MBS Dynamic Range Measurement Accuracy (Release 13 only)	840

11.4.1	Test purpose	840

11.4.2	Test applicability	840

11.4.3	Minimum conformance requirements	840

11.4.4	Test description	840

11.4.4.1	Initial conditions	840

11.4.4.2	Test procedure	840

11.4.4.3	Message contents	841

11.4.5	Test requirement	845

11.4A	MBS Dynamic Range Measurement Accuracy (Release 14 Onwards)	846

11.4A.1	Test purpose	846

11.4A.2	Test applicability	846

11.4A.3	Minimum conformance requirements	846

11.4A.4	Test description	846

11.4A.4.1	Initial conditions	846

11.4A.4.2	Test procedure	846

11.4A.4.3	Message contents	846

11.4A.5	Test requirement	849

11.4B	MBS Dynamic Range Measurement Accuracy (NR)	849

11.4B.1	Test purpose	849

11.4B.2	Test applicability	849

11.4B.3	Minimum conformance requirements	849

11.4B.4	Test description	850

11.4B.4.1	Initial conditions	850

11.4B.4.2	Test procedure	850

11.4B.4.3	Message contents	850

11.4B.5	Test requirement	850

11.5	MBS Measurement Accuracy in Multipath (Release 13 only)	850

11.5.1	Test purpose	850

11.5.2	Test applicability	850

11.5.3	Minimum conformance requirements	850

11.5.4	Test description	851

11.5.4.1	Initial conditions	851

11.5.4.2	Test procedure	851

11.5.4.3	Message contents	851

11.5.5	Test requirement	853

11.5A	MBS Measurement Accuracy in Multipath (Release 14 Onwards)	854

11.5A.1	Test purpose	854

11.5A.2	Test applicability	854

11.5A.3	Minimum conformance requirements	855

11.5A.4	Test description	855

11.5A.4.1	Initial conditions	855

11.5A.4.2	Test procedure	855

11.5A.4.3	Message contents	855

11.5A.5	Test requirement	857

11.5B	MBS Measurement Accuracy in Multipath (NR)	857

11.5B.1	Test purpose	857

11.5B.2	Test applicability	857

11.5B.3	Minimum conformance requirements	857

11.5B.4	Test description	857

11.5B.4.1	Initial conditions	857

11.5B.4.2	Test procedure	857

11.5B.4.3	Message contents	857

11.5B.5	Test requirement	857

12	E-UTRA WLAN and BLE measurement requirements	857

12.0	General	857

12.1	WLAN Access Point Identification and Reporting Delay	858

12.1.1	WLAN AP Identification and reporting delay under nominal conditions	858

12.1.1.1	Test purpose	858

12.1.1.2	Test applicability	858

12.1.1.3	Minimum conformance requirements	858

12.1.1.4	Test description	859

12.1.1.4.1	Initial conditions	859

12.1.1.4.2	Test procedure	859

12.1.1.4.3	Message contents	860

12.1.1.5	Test requirement	861

12.1.2	WLAN AP Identification and reporting delay under dynamic range conditions	863

12.1.2.1	Test purpose	863

12.1.2.2	Test applicability	864

12.1.2.3	Minimum conformance requirements	864

12.1.2.4	Test description	864

12.1.2.4.1	Initial conditions	865

12.1.2.4.2	Test procedure	865

12.1.2.4.3	Message contents	865

12.1.2.5	Test requirement	865

12.2	BLE Identification and Reporting Delay	867

12.2.1	Bluetooth identification	867

12.2.1.1	Test purpose	867

12.2.1.2	Test applicability	867

12.2.1.3	Minimum conformance requirements	867

12.2.1.4	Test description	867

12.2.1.4.1	Initial conditions	867

12.2.1.4.2	Test procedure	868

12.2.1.4.3	Message contents	868

12.2.1.5	Test requirement	869

13	NR A-GNSS minimum performance requirements	871

13.1	General	871

13.2	Sensitivity	872

13.2.1	Sensitivity Coarse time assistance	872

13.2.1.1	Sub-tests	872

13.2.1.2	Test purpose	872

13.2.1.3	Test applicability	872

13.2.1.4	Minimum conformance requirements	872

13.2.1.5	Test description	874

13.2.1.5.1	Initial conditions	874

13.2.1.5.2	Test procedure	874

13.2.1.5.3	Message contents	875

13.2.1.6	Test requirement	876

13.2.2	Sensitivity Fine time assistance	877

13.2.2.1	Sub-tests	877

13.2.2.2	Test purpose	878

13.2.2.3	Test applicability	878

13.2.2.4	Minimum conformance requirements	878

13.2.2.5	Test description	879

13.2.2.5.1	Initial conditions	879

13.2.2.5.2	Test procedure	880

13.2.2.5.3	Message contents	880

13.2.2.6	Test requirement	881

13.3	Nominal Accuracy	882

13.3.1	Sub-tests	882

13.3.2	Test purpose	883

13.3.3	Test applicability	883

13.3.4	Minimum conformance requirements	883

13.3.5	Test description	884

13.3.5.1	Initial conditions	884

13.3.5.2	Test procedure	885

13.3.5.3	Message contents	885

13.3.6	Test requirement	886

13.4	Dynamic Range	887

13.4.1	Sub-tests	887

13.4.2	Test purpose	888

13.4.3	Test applicability	888

13.4.4	Minimum conformance requirements	888

13.4.5	Test description	889

13.4.5.1	Initial conditions	889

13.4.5.2	Test procedure	890

13.4.5.3	Message contents	891

13.4.6	Test requirement	891

13.5	Multi-Path scenario	893

13.5.1	Sub-tests	893

13.5.2	Test purpose	893

13.5.3	Test applicability	893

13.5.4	Minimum conformance requirements	893

13.5.5	Test description	894

13.5.5.1	Initial conditions	894

13.5.5.2	Test procedure	895

13.5.5.3	Message contents	896

13.5.6	Test requirement	896

13.6	Moving scenario and periodic update (Rel-9 to Rel-13)	898

13.6.1	Sub-tests	898

13.6.2	Test purpose	898

13.6.3	Test applicability	898

13.6.4	Minimum conformance requirements	898

13.6.5	Test description	899

13.6.5.1	Initial conditions	899

13.6.5.2	Test procedure	900

13.6.5.3	Message contents	901

13.6.6	Test requirement	902

13.7	Moving scenario and periodic update (Rel-14 onwards)	903

13.7.1	Sub-tests	903

13.7.2	Test purpose	904

13.7.3	Test applicability	904

13.7.4	Minimum conformance requirements	904

13.7.5	Test description	904

13.7.6	Test requirement	904

14	NR RSTD measurement requirements	904

14.1	General	904

14.2	NR RSTD measurement period test cases in RRC_CONNECTED	904

14.2.1	NR RSTD measurement period test case for single positioning frequency layer in FR1 SA	904

14.2.1.1	Test purpose	904

14.2.1.2	Test applicability	904

14.2.1.3	Minimum conformance requirements	904

14.2.1.4	Test description	906

14.2.1.4.1	Initial conditions	906

14.2.1.4.2	Test procedure	906

14.2.1.4.3	Message contents	907

14.2.1.5	Test requirement	917

14.2.2	NR RSTD measurement period test case for dual positioning frequency layer in FR1 SA	923

14.2.2.1	Test purpose	923

14.2.2.2	Test applicability	923

14.2.2.3	Minimum conformance requirements	923

14.2.2.4	Test description	923

14.2.2.4.1	Initial conditions	923

14.2.2.4.2	Test procedure	924

14.2.2.4.3	Message contents	924

14.2.2.5	Test requirement	927

14.2.3	NR RSTD measurement period test case for single positioning frequency layer in FR2 SA	933

14.2.3.1	Test purpose	933

14.2.3.2	Test applicability	933

14.2.3.3	Minimum conformance requirements	933

14.2.3.4	Test description	933

14.2.3.4.1	Initial conditions	933

14.2.3.4.2	Test procedure	934

14.2.3.4.3	Message contents	934

14.2.3.5	Test requirement	945

14.2.4	NR RSTD measurement period test case for dual positioning frequency layers in FR2 SA	952

14.2.4.1	Test purpose	952

14.2.4.2	Test applicability	952

14.2.4.3	Minimum conformance requirements	953

14.2.4.4	Test description	953

14.2.4.4.1	Initial conditions	953

14.2.4.4.2	Test procedure	953

14.2.4.4.3	Message contents	953

14.2.4.5	Test requirement	963

14.2.5	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	970

14.2.5.1	Test purpose	970

14.2.5.2	Test applicability	970

14.2.5.3	Minimum conformance requirements	970

14.2.5.4	Test description	973

14.2.5.4.1	Initial conditions	973

14.2.5.4.2	Test procedure	973

14.2.5.4.3	Message contents	974

14.2.5.5	Test requirement	984

14.2.6	NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA	990

14.2.6.1	Test purpose	990

14.2.6.2	Test applicability	990

14.2.6.3	Minimum conformance requirements	990

14.2.6.4	Test description	992

14.2.6.4.1	Initial conditions	992

14.2.6.4.2	Test procedure	993

14.2.6.4.3	Message contents	994

14.2.6.5	Test requirement	1006

14.2.7	NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR1 SA	1012

14.2.7.1	Test purpose	1012

14.2.7.2	Test applicability	1012

14.2.7.3	Minimum conformance requirements	1012

14.2.7.4	Test description	1012

14.2.7.4.1	Initial conditions	1012

14.2.7.4.2	Test procedure	1012

14.2.7.4.3	Message contents	1013

14.2.7.5	Test requirement	1024

14.2.8	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	1029

14.2.8.1	Test purpose	1030

14.2.8.2	Test applicability	1030

14.2.8.3	Minimum conformance requirements	1030

14.2.8.4	Test description	1032

14.2.8.4.1	Initial conditions	1032

14.2.8.4.2	Test procedure	1033

14.2.8.4.3	Message contents	1034

14.2.8.5	Test requirement	1044

14.2.9	NR RSTD measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA	1049

14.2.9.1	Test purpose	1050

14.2.9.2	Test applicability	1050

14.2.9.3	Minimum conformance requirements	1050

14.2.9.4	Test description	1052

14.2.9.4.1	Initial conditions	1052

14.2.9.4.2	Test procedure	1053

14.2.9.4.3	Message contents	1054

14.2.9.5	Test requirement	1066

14.2.10	NR RSTD measurement reporting delay test case for single positioning frequency layer with Rx TEG in FR2 SA	1070

14.2.10.1	Test purpose	1071

14.2.10.2	Test applicability	1071

14.2.10.3	Minimum conformance requirements	1071

14.2.10.4	Test description	1071

14.2.10.4.1	Initial conditions	1071

14.2.10.4.2	Test procedure	1071

14.2.10.4.3	Message contents	1071

14.2.10.5	Test requirement	1083

14.2.11	NR RSTD measurement reporting delay test case for PRS aggregation in FR1	1087

14.2.11.1	Test purpose	1087

14.2.11.2	Test applicability	1088

14.2.11.4	Test description	1091

14.2.11.4.1	Initial conditions	1091

14.2.11.4.2	Test procedure	1092

14.2.11.4.3	Message contents	1093

14.2.11.5	Test requirement	1104

14.2.12	NR RSTD measurement reporting delay test case for PRS aggregation in FR2	1110

14.2.12.1	Test purpose	1110

14.2.12.4	Test description	1110

14.2.12.4.1	Initial conditions	1110

14.2.12.4.2	Test procedure	1111

14.2.12.4.3	Message contents	1111

14.2.12.5	Test requirement	1117

14.2.13	NR RSTD measurement reporting delay test case in FR1 for Redcap UE without Rx FH	1124

14.2.13.1	Test purpose	1124

14.2.13.2	Test applicability	1124

14.2.13.3	Minimum conformance requirements	1124

14.2.13.4	Test description	1131

14.2.13.4.1	Initial conditions	1131

14.2.13.4.2	Test procedure	1132

14.2.13.4.3	Message contents	1132

14.2.13.5	Test requirement	1132

14.2.14	RSTD measurement reporting delay test case in FR2 for Redcap UE without Rx FH	1136

14.2.14.1	Test purpose	1136

14.2.14.2	Test applicability	1136

14.2.14.3	Minimum conformance requirements	1137

14.2.14.4	Test description	1137

14.2.14.4.1	Initial conditions	1137

14.2.14.4.2	Test procedure	1137

14.2.14.4.3	Message contents	1137

14.2.14.5	Test requirement	1137

14.2.15	NR RSTD measurement reporting delay test case in FR1 for Redcap UE with Rx FH	1144

14.2.15.1	Test purpose	1144

14.2.15.2	Test applicability	1144

14.2.15.3	Minimum conformance requirements	1144

14.2.15.4	Test description	1147

14.2.15.4.1	Initial conditions	1147

14.2.15.4.2	Test procedure	1147

14.2.15.4.3	Message contents	1147

14.2.15.5	Test requirement	1147

14.2.16	NR RSTD measurement reporting delay test case in FR2 for Redcap UE with Rx FH	1151

14.2.16.1	Test purpose	1152

14.2.16.2	Test applicability	1152

14.2.16.3	Minimum conformance requirements	1152

14.2.16.4	Test description	1152

14.2.16.4.1	Initial conditions	1152

14.2.16.4.2	Test procedure	1152

14.2.16.4.3	Message contents	1152

14.2.16.5	Test requirement	1152

14.3	NR RSTD measurement accuracy test case in RRC_CONNECTED	1157

14.3.1	NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA	1157

14.3.1.1	Test purpose	1157

14.3.1.2	Test applicability	1157

14.3.1.3	Minimum conformance requirements	1157

14.3.1.4	Test description	1159

14.3.1.4.1	Initial conditions	1159

14.3.1.4.2	Test procedure	1160

14.3.1.4.3	Message contents	1160

14.3.1.5	Test requirement	1170

14.3.2	NR RSTD measurement accuracy test case for dual positioning frequency layer in FR1 SA	1172

14.3.2.1	Test purpose	1172

14.3.2.2	Test applicability	1172

14.3.2.3	Minimum conformance requirements	1172

14.3.2.4	Test description	1172

14.3.2.4.1	Initial conditions	1172

14.3.2.4.2	Test procedure	1173

14.3.2.4.3	Message contents	1173

14.3.2.5	Test requirement	1176

14.3.3	NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA	1178

14.3.3.1	Test purpose	1178

14.3.3.2	Test applicability	1178

14.3.3.3	Minimum conformance requirements	1178

14.3.3.4	Test description	1178

14.3.3.4.1	Initial conditions	1178

14.3.3.4.2	Test procedure	1179

14.3.3.4.3	Message contents	1180

14.3.3.5	Test requirement	1188

14.3.4	NR RSTD measurement accuracy test case for dual positioning frequency layer in FR2 SA	1190

14.3.4.1	Test purpose	1191

14.3.4.2	Test applicability	1191

14.3.4.3	Minimum conformance requirements	1191

14.3.4.4	Test description	1191

14.3.4.4.1	Initial conditions	1191

14.3.4.4.2	Test procedure	1191

14.3.4.4.3	Message contents	1192

14.3.4.5	Test requirement	1202

14.3.5	NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA	1205

14.3.5.1	Test purpose	1205

14.3.5.2	Test applicability	1205

14.3.5.3	Minimum conformance requirements	1205

14.3.5.4	Test description	1207

14.3.5.4.1	Initial conditions	1207

14.3.5.4.2	Test procedure	1208

14.3.5.4.3	Message contents	1209

14.3.5.5	Test requirement	1220

14.3.6	NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR1 SA	1222

14.3.6.1	Test purpose	1222

14.3.6.2	Test applicability	1222

14.3.6.3	Minimum conformance requirements	1222

14.3.6.4	Test description	1222

14.3.6.4.1	Initial conditions	1222

14.3.6.4.2	Test procedure	1223

14.3.6.4.3	Message contents	1224

14.3.6.5	Test requirement	1235

14.3.7	NR RSTD measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA	1237

14.3.7.1	Test purpose	1237

14.3.7.2	Test applicability	1237

14.3.7.3	Minimum conformance requirements	1238

14.3.7.4	Test description	1239

14.3.7.4.1	Initial conditions	1239

14.3.7.4.2	Test procedure	1240

14.3.7.4.3	Message contents	1240

14.3.7.5	Test requirement	1249

14.3.8	NR RSTD measurement accuracy test case for single positioning frequency layer with Rx TEG in FR2 SA	1251

14.3.8.1	Test purpose	1251

14.3.8.2	Test applicability	1251

14.3.8.3	Minimum conformance requirements	1251

14.3.8.4	Test description	1251

14.3.8.4.1	Initial conditions	1251

14.3.8.4.2	Test procedure	1252

14.3.8.4.3	Message contents	1253

14.3.8.5	Test requirement	1262

14.3.9	NR RSTD measurement accuracy test case for PRS aggregation in FR1 SA	1264

14.3.9.1	Test purpose	1265

14.3.9.2	Test applicability	1265

14.3.9.3	Minimum conformance requirements	1265

14.3.9.4	Test description	1272

14.3.9.4.1	Initial conditions	1272

14.3.9.4.2	Test procedure	1273

14.3.9.4.3	Message contents	1273

14.3.9.5	Test requirement	1284

14.3.10	NR RSTD measurement accuracy test case for PRS aggregation in FR2 SA	1287

14.3.10.1	Test purpose	1287

14.3.10.2	Test applicability	1287

14.3.10.3	Minimum conformance requirements	1287

14.3.10.4	Test description	1287

14.3.10.4.1	Initial conditions	1287

14.3.10.4.2	Test procedure	1288

14.3.10.4.3	Message contents	1288

14.3.10.5	Test requirements	1293

14.3.11	NR RSTD measurement accuracy test case in FR1 for Redcap UE without Rx FH	1294

14.3.11.1	Test purpose	1294

14.3.11.2	Test applicability	1294

14.3.11.3	Minimum conformance requirements	1295

14.3.11.4	Test description	1295

14.3.11.4.1	Initial conditions	1295

14.3.11.4.2	Test procedure	1295

14.3.11.4.3	Message contents	1295

14.3.11.5	Test requirement	1295

14.3.12	NR RSTD measurement accuracy test case in FR2 for Redcap UE without Rx FH	1297

14.3.12.1	Test purpose	1298

14.3.12.2	Test applicability	1298

14.3.12.3	Minimum conformance requirements	1298

14.3.12.4	Test description	1298

14.3.12.4.1	Initial conditions	1298

14.3.12.4.2	Test procedure	1298

14.3.12.4.3	Message contents	1299

14.3.12.5	Test requirement	1299

14.3.13	NR RSTD measurement accuracy test case in FR1 for Redcap UE with Rx FH	1301

14.3.13.1	Test purpose	1301

14.3.13.2	Test applicability	1301

14.3.13.3	Minimum conformance requirements	1302

14.3.13.4	Test description	1302

14.3.13.4.1	Initial conditions	1302

14.3.13.4.2	Test procedure	1302

14.3.13.4.3	Message contents	1302

14.3.13.5	Test requirement	1302

14.3.14	NR RSTD measurement accuracy test case in FR2 for Redcap UE with Rx FH	1305

14.3.14.1	Test purpose	1305

14.3.14.2	Test applicability	1305

14.3.14.3	Minimum conformance requirements	1306

14.3.14.4	Test description	1306

14.3.14.4.1	Initial conditions	1306

14.3.14.4.2	Test procedure	1306

14.3.14.4.3	Message contents	1306

14.3.14.5	Test requirement	1306

14.4	NR RSTD measurement period test cases in RRC_INACTIVE	1308

14.4.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1308

14.4.1.1	Test purpose	1308

14.4.1.2	Test applicability	1308

14.4.1.3	Minimum conformance requirements	1308

14.4.1.4	Test description	1311

14.4.1.4.1	Initial conditions	1311

14.4.1.4.2	Test procedure	1311

14.4.1.4.3	Message contents	1312

14.4.1.5	Test requirement	1323

14.4.2	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	1328

14.4.2.1	Test purpose	1328

14.4.2.2	Test applicability	1328

14.4.2.3	Minimum conformance requirements	1328

14.4.2.4	Test description	1328

14.4.2.4.1	Initial conditions	1328

14.4.2.4.2	Test procedure	1329

14.4.2.4.3	Message contents	1329

14.4.2.5	Test requirement	1333

14.4.3	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1338

14.4.3.1	Test purpose	1338

14.4.3.2	Test applicability	1338

14.4.3.3	Minimum conformance requirements	1338

14.4.3.4	Test description	1340

14.4.3.4.1	Initial conditions	1340

14.4.3.4.2	Test procedure	1341

14.4.3.4.3	Message contents	1342

14.4.3.5	Test requirement	1355

14.4.4	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	1359

14.4.4.1	Test purpose	1359

14.4.4.2	Test applicability	1359

14.4.4.3	Minimum conformance requirements	1360

14.4.4.4	Test description	1360

14.4.4.4.1	Initial conditions	1360

14.4.4.4.2	Test procedure	1360

14.4.4.4.3	Message contents	1360

14.4.4.5	Test requirement	1365

14.5	NR RSTD measurement accuracy test case in RRC_INACTIVE	1369

14.5.1	NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA	1369

14.5.1.1	Test purpose	1369

14.5.1.2	Test applicability	1370

14.5.1.3	Minimum conformance requirements	1370

14.5.1.4	Test description	1371

14.5.1.4.1	Initial conditions	1372

14.5.1.4.2	Test procedure	1372

14.5.1.4.3	Message contents	1373

14.5.1.5	Test requirement	1379

14.5.2	NR RSTD measurement accuracy for single positioning frequency layer with reduced number of samples in FR1 SA	1381

14.5.2.1	Test purpose	1381

14.5.2.2	Test applicability	1381

14.5.2.3	Minimum conformance requirements	1381

14.5.2.4	Test description	1384

14.5.2.4.1	Initial conditions	1384

14.5.2.4.2	Test procedure	1384

14.5.2.4.3	Message contents	1384

14.5.2.5	Test requirement	1390

14.5.3	NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA	1392

14.5.3.1	Test purpose	1392

14.5.3.2	Test applicability	1392

14.5.3.3	Minimum conformance requirements	1392

14.5.3.4	Test description	1394

14.5.3.4.1	Initial conditions	1394

14.5.3.4.2	Test procedure	1394

14.5.3.4.3	Message contents	1395

14.5.3.5	Test requirement	1400

14.5.4	NR RSTD measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA	1402

14.5.4.1	Test purpose	1402

14.5.4.2	Test applicability	1402

14.5.4.3	Minimum conformance requirements	1402

14.5.4.4	Test description	1403

14.5.4.4.1	Initial conditions	1404

14.5.4.4.2	Test procedure	1404

14.5.4.4.3	Message contents	1404

14.5.4.5	Test requirement	1409

15	UE Rx-Tx time difference measurement requirements	1412

15.1	General	1412

15.2	UE Rx-Tx time difference measurement period test cases in RRC_CONNECTED	1412

15.2.1	UE Rx-Tx time difference measurement period for single positioning frequency layer in FR1 SA	1412

15.2.1.1	Test purpose	1412

15.2.1.2	Test applicability	1412

15.2.1.3	Minimum conformance requirements	1412

15.2.1.4	Test description	1414

15.2.1.4.1	Initial conditions	1414

15.2.1.4.2	Test procedure	1414

15.2.1.4.3	Message contents	1415

15.2.1.5	Test requirement	1427

15.2.2	UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR1 SA	1430

15.2.2.1	Test purpose	1430

15.2.2.2	Test applicability	1430

15.2.2.3	Minimum conformance requirements	1430

15.2.2.4	Test description	1430

15.2.2.4.1	Initial conditions	1430

15.2.2.4.2	Test procedure	1431

15.2.2.4.3	Message contents	1431

15.2.2.5	Test requirement	1444

15.2.3	UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA	1447

15.2.3.1	Test purpose	1447

15.2.3.2	Test applicability	1447

15.2.3.3	Minimum conformance requirements	1447

15.2.3.4	Test description	1447

15.2.3.4.1	Initial conditions	1447

15.2.3.4.2	Test procedure	1448

15.2.3.4.3	Message contents	1448

15.2.3.5	Test requirement	1459

15.2.4	UE Rx-Tx time difference measurement period for dual positioning frequency layer in FR2 SA	1463

15.2.4.1	Test purpose	1463

15.2.4.2	Test applicability	1463

15.2.4.3	Minimum conformance requirements	1463

15.2.4.4	Test description	1463

15.2.4.4.1	Initial conditions	1464

15.2.4.4.2	Test procedure	1464

15.2.4.4.3	Message contents	1464

15.2.4.5	Test requirement	1476

15.2.5	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	1480

15.2.5.1	Test purpose	1480

15.2.5.2	Test applicability	1480

15.2.5.3	Minimum conformance requirements	1480

15.2.5.4	Test description	1482

15.2.5.4.1	Initial conditions	1482

15.2.5.4.2	Test procedure	1483

15.2.5.4.3	Message contents	1484

15.2.5.5	Test requirement	1497

15.2.6	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA	1501

15.2.6.1	Test purpose	1501

15.2.6.2	Test applicability	1501

15.2.6.3	Minimum conformance requirements	1501

15.2.6.4	Test description	1503

15.2.6.4.1	Initial conditions	1503

15.2.6.4.2	Test procedure	1504

15.2.6.4.3	Message contents	1505

15.2.6.5	Test requirement	1519

15.2.7	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with RxTx TEG in FR1 SA	1523

15.2.7.1	Test purpose	1523

15.2.7.2	Test applicability	1524

15.2.7.3	Minimum conformance requirements	1524

15.2.7.4	Test description	1524

15.2.7.4.1	Initial conditions	1524

15.2.7.4.2	Test procedure	1524

15.2.7.4.3	Message contents	1524

15.2.7.5	Test requirement	1538

15.2.8	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	1542

15.2.8.1	Test purpose	1543

15.2.8.2	Test applicability	1543

15.2.8.3	Minimum conformance requirements	1543

15.2.8.4	Test description	1545

15.2.8.4.1	Initial conditions	1545

15.2.8.4.2	Test procedure	1545

15.2.8.4.3	Message contents	1546

15.2.8.5	Test requirement	1559

15.2.9	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA	1562

15.2.9.1	Test purpose	1562

15.2.9.2	Test applicability	1562

15.2.9.3	Minimum conformance requirements	1562

15.2.9.4	Test description	1564

15.2.9.4.1	Initial conditions	1564

15.2.9.4.2	Test procedure	1565

15.2.9.4.3	Message contents	1566

15.2.9.5	Test requirement	1580

15.2.10	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with RxTx TEG in FR2 SA	1583

15.2.10.1	Test purpose	1583

15.2.10.2	Test applicability	1583

15.2.10.3	Minimum conformance requirements	1583

15.2.10.4	Test description	1583

15.2.10.4.1	Initial conditions	1583

15.2.10.4.2	Test procedure	1584

15.2.10.4.3	Message contents	1584

15.2.10.5	Test requirement	1596

15.2.11	UE Rx-Tx time difference measurement reporting delay test case for PRS aggregation in FR1 SA	1599

15.2.11.1	Test purpose	1600

15.2.11.2	Test applicability	1600

15.2.11.3	Minimum conformance requirements	1600

15.2.11.4	Test description	1603

15.2.11.4.1	Initial conditions	1603

15.2.11.4.2	Test procedure	1603

15.2.11.4.3	Message contents	1604

15.2.11.5	Test requirement	1619

15.2.12	UE Rx-Tx time difference measurement reporting delay test case for PRS aggregation in FR2 SA	1619

15.2.12.1	Test purpose	1619

15.2.12.2	Test applicability	1619

15.2.12.3	Minimum conformance requirements	1619

15.2.12.4	Test description	1619

15.2.12.4.1	Initial conditions	1619

15.2.12.4.2	Test procedure	1619

15.2.12.4.3	Message contents	1620

15.2.12.5	Test requirement	1628

15.2.13	UE Rx-Tx time difference measurement reporting delay test case in FR1 for Redcap UE without Rx FH	1628

15.2.13.1	Test purpose	1628

15.2.13.2	Test applicability	1628

15.2.13.3	Minimum conformance requirements	1628

15.2.13.4	Test description	1628

15.2.13.4.1	Initial conditions	1628

15.2.13.4.2	Test procedure	1629

15.2.13.4.3	Message contents	1629

15.2.13.5	Test requirement	1633

15.2.14	UE Rx-Tx time difference measurement reporting delay test case in FR2 for Redcap UE without Rx FH	1633

15.2.14.1	Test purpose	1633

15.2.14.2	Test applicability	1633

15.2.14.3	Minimum conformance requirements	1633

15.2.14.4	Test description	1633

15.2.14.4.1	Initial conditions	1634

15.2.14.4.2	Test procedure	1634

15.2.14.4.3	Message contents	1634

15.2.14.5	Test requirement	1645

15.2.15	UE Rx-Tx time difference measurement reporting delay test case in FR1 for Redcap UE with Rx FH	1645

15.2.15.1	Test purpose	1645

15.2.15.2	Test applicability	1645

15.2.15.3	Minimum conformance requirements	1645

15.2.15.4	Test description	1647

15.2.15.4.1	Initial conditions	1647

15.2.15.4.2	Test procedure	1647

15.2.15.4.3	Message contents	1647

15.2.15.5	Test requirement	1657

15.2.16	UE Rx-Tx time difference measurement reporting delay test case in FR2 for Redcap UE with Rx FH	1657

15.2.16.1	Test purpose	1657

15.2.16.2	Test applicability	1657

15.2.16.3	Minimum conformance requirements	1658

15.2.16.4	Test description	1658

15.2.16.4.1	Initial conditions	1658

15.2.16.4.2	Test procedure	1658

15.2.16.4.3	Message contents	1658

15.2.16.5	Test requirement	1668

15.3	UE Rx-Tx time difference measurement accuracy test cases in RRC_CONNECTED	1668

15.3.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1668

15.3.1.1	Test purpose	1668

15.3.1.2	Test applicability	1668

15.3.1.3	Minimum conformance requirements	1668

15.3.1.4	Test description	1671

15.3.1.4.1	Initial conditions	1671

15.3.1.4.2	Test procedure	1672

15.3.1.4.3	Message contents	1673

15.3.1.5	Test requirement	1688

15.3.2	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	1692

15.3.2.1	Test purpose	1692

15.3.2.2	Test applicability	1692

15.3.2.3	Minimum conformance requirements	1692

15.3.2.4	Test description	1692

15.3.2.4.1	Initial conditions	1692

15.3.2.4.2	Test procedure	1692

15.3.2.4.3	Message contents	1692

15.3.2.5	Test requirement	1704

15.3.3	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR1 SA	1707

15.3.3.1	Test purpose	1707

15.3.3.2	Test applicability	1708

15.3.3.3	Minimum conformance requirements	1708

15.3.3.4	Test description	1710

15.3.3.4.1	Initial conditions	1710

15.3.3.4.2	Test procedure	1710

15.3.3.4.3	Message contents	1711

15.3.3.5	Test requirement	1724

15.3.4	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR1 SA	1726

15.3.4.1	Test purpose	1726

15.3.4.2	Test applicability	1726

15.3.4.3	Minimum conformance requirements	1726

15.3.4.4	Test description	1728

15.3.4.4.1	Initial conditions	1728

15.3.4.4.2	Test procedure	1728

15.3.4.4.3	Message contents	1729

15.3.4.5	Test requirement	1743

15.3.5	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with reduced number of samples in FR2 SA	1747

15.3.5.1	Test purpose	1747

15.3.5.2	Test applicability	1747

15.3.5.3	Minimum conformance requirements	1747

15.3.5.4	Test description	1748

15.3.5.4.1	Initial conditions	1748

15.3.5.4.2	Test procedure	1749

15.3.5.4.3	Message contents	1750

15.3.5.5	Test requirement	1762

15.3.6	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR2 SA	1765

15.3.6.1	Test purpose	1765

15.3.6.2	Test applicability	1765

15.3.6.3	Minimum conformance requirements	1766

15.3.6.4	Test description	1767

15.3.6.4.1	Initial conditions	1767

15.3.6.4.2	Test procedure	1767

15.3.6.4.3	Message contents	1768

15.3.6.5	Test requirement	1780

15.3.7	UE Rx-Tx time difference measurement accuracy test case for PRS aggregation in FR1 SA	1783

15.3.7.1	Test purpose	1784

15.3.7.2	Test applicability	1784

15.3.7.3	Minimum conformance requirements	1784

15.3.7.4	Test description	1788

15.3.7.4.1	Initial conditions	1788

15.3.7.4.2	Test procedure	1788

15.3.7.4.3	Message contents	1790

15.3.7.5	Test requirement	1804

15.3.8	UE Rx-Tx time difference measurement accuracy test case for PRS aggregation in FR2 SA	1804

15.3.8.1	Test purpose	1804

15.3.8.2	Test applicability	1804

15.3.8.3	Minimum conformance requirements	1804

15.3.8.4	Test description	1804

15.3.8.4.1	Initial conditions	1804

15.3.8.4.2	Test procedure	1804

15.3.8.4.3	Message contents	1805

15.3.8.5	Test requirement	1809

15.3.9	UE Rx-Tx time difference measurement accuracy test case in FR1 for Redcap UE without Rx FH	1809

15.3.9.1	Test purpose	1809

15.3.9.2	Test applicability	1809

15.3.9.3	Minimum conformance requirements	1809

15.3.9.4	Test description	1813

15.3.9.4.1	Initial conditions	1813

15.3.9.4.2	Test procedure	1814

15.3.9.4.3	Message contents	1814

15.3.9.5	Test requirement	1820

15.3.10	UE Rx-Tx time difference measurement accuracy test case in FR2 for Redcap UE without Rx FH	1820

15.3.10.1	Test purpose	1820

15.3.10.2	Test applicability	1820

15.3.10.3	Minimum conformance requirements	1820

15.3.10.4	Test description	1820

15.3.10.4.1	Initial conditions	1821

15.3.10.4.2	Test procedure	1821

15.3.10.4.3	Message contents	1821

15.3.10.5	Test requirement	1828

15.3.11	UE Rx-Tx time difference measurement accuracy test case in FR1 for Redcap UE with Rx FH	1828

15.3.11.1	Test purpose	1828

15.3.11.2	Test applicability	1828

15.3.11.3	Minimum conformance requirements	1828

15.3.11.4	Test description	1834

15.3.11.4.1	Initial conditions	1834

15.3.11.4.2	Test procedure	1834

15.3.11.4.3	Message contents	1834

15.3.11.5	Test requirement	1841

15.3.12	UE Rx-Tx time difference measurement accuracy test case in FR2 for Redcap UE with Rx FH	1841

15.3.12.1	Test purpose	1841

15.3.12.2	Test applicability	1841

15.3.12.3	Minimum conformance requirements	1841

15.3.12.4	Test description	1841

15.3.12.4.1	Initial conditions	1841

15.3.12.4.2	Test procedure	1842

15.3.12.4.3	Message contents	1842

15.3.12.5	Test requirement	1848

15.4	UE Rx-Tx time difference measurement period test cases in RRC_INACTIVE	1849

15.4.1	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR1 SA	1849

15.4.1.1	Test purpose	1849

15.4.1.2	Test applicability	1849

15.4.1.3	Minimum conformance requirements	1849

15.4.1.4	Test description	1851

15.4.1.4.1	Initial conditions	1851

15.4.1.4.2	Test procedure	1851

15.4.1.4.3	Message contents	1853

15.4.1.5	Test requirement	1867

15.4.2	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	1870

15.4.2.1	Test purpose	1870

15.4.2.2	Test applicability	1870

15.4.2.3	Minimum conformance requirements	1870

15.4.2.4	Test description	1870

15.4.2.4.1	Initial conditions	1870

15.4.2.4.2	Test procedure	1871

15.4.2.4.3	Message contents	1871

15.4.2.5	Test requirement	1874

15.4.3	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer in FR2 SA	1877

15.4.3.1	Test purpose	1877

15.4.3.2	Test applicability	1877

15.4.3.3	Minimum conformance requirements	1877

15.4.3.4	Test description	1877

15.4.3.4.1	Initial conditions	1877

15.4.3.4.2	Test procedure	1878

15.4.3.4.3	Message contents	1878

15.4.3.5	Test requirement	1882

15.4.4	UE Rx-Tx time difference measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	1885

15.4.4.1	Test purpose	1885

15.4.4.2	Test applicability	1885

15.4.4.3	Minimum conformance requirements	1885

15.4.4.4	Test description	1885

15.4.4.4.1	Initial conditions	1885

15.4.4.4.2	Test procedure	1886

15.4.4.4.3	Message contents	1886

15.4.4.5	Test requirement	1893

15.5	UE Rx-Tx time difference measurement accuracy test cases in RRC_INACTIVE	1896

15.5.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1896

15.5.1.1	Test purpose	1896

15.5.1.2	Test applicability	1896

15.5.1.3	Minimum conformance requirements	1896

15.5.1.4	Test description	1899

15.5.1.4.1	Initial conditions	1899

15.5.1.4.2	Test procedure	1899

15.5.1.4.3	Message contents	1900

15.5.1.5	Test requirement	1906

15.5.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	1907

15.5.3	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	1919

15.5.3.1	Test purpose	1919

15.5.3.2	Test applicability	1919

15.5.3.3	Minimum conformance requirements	1919

15.5.3.4	Test description	1921

15.5.3.4.1	Initial conditions	1921

15.5.3.4.2	Test procedure	1922

15.5.3.4.3	Message contents	1923

15.5.3.5	Test requirement	1928

16	NR PRS-RSRP measurement requirements	1941

16.1	General	1941

16.2	NR PRS-RSRP measurement period test cases in RRC_CONNECTED	1941

16.2.1	PRS-RSRP measurement period test case for single positioning frequency layer in FR1 SA	1941

16.2.1.1	Test purpose	1941

16.2.1.2	Test applicability	1941

16.2.1.3	Minimum conformance requirements	1942

16.2.1.4	Test description	1944

16.2.1.4.1	Initial conditions	1944

16.2.1.4.2	Test procedure	1945

16.2.1.4.3	Message contents	1945

16.2.1.5	Test requirement	1957

16.2.2	PRS-RSRP measurement period test case for dual positioning frequency layer in FR1 SA	1960

16.2.2.1	Test purpose	1960

16.2.2.2	Test applicability	1960

16.2.2.3	Minimum conformance requirements	1961

16.2.2.4	Test description	1961

16.2.2.4.1	Initial conditions	1961

16.2.2.4.2	Test procedure	1961

16.2.2.4.3	Message contents	1961

16.2.2.5	Test requirement	1972

16.2.3	PRS-RSRP measurement period test case for single positioning frequency layer in FR2 SA	1975

16.2.3.1	Test purpose	1975

16.2.3.2	Test applicability	1975

16.2.3.3	Minimum conformance requirements	1976

16.2.3.4	Test description	1977

16.2.3.4.1	Initial conditions	1977

16.2.3.4.2	Test procedure	1977

16.2.3.4.3	Message contents	1978

16.2.3.5	Test requirement	1987

16.2.4	PRS-RSRP measurement period test case for dual positioning frequency layer in FR2 SA	1991

16.2.4.1	Test purpose	1991

16.2.4.2	Test applicability	1991

16.2.4.3	Minimum conformance requirements	1991

16.2.4.4	Test description	1991

16.2.4.4.1	Initial conditions	1992

16.2.4.4.2	Test procedure	1992

16.2.4.4.3	Message contents	1992

16.2.4.5	Test requirement	2002

16.2.5	PRS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR1 SA	2006

16.2.5.1	Test purpose	2006

16.2.5.2	Test applicability	2006

16.2.5.3	Minimum conformance requirements	2006

16.2.5.4	Test description	2008

16.2.5.4.1	Initial conditions	2008

16.2.5.4.2	Test procedure	2009

16.2.5.4.3	Message contents	2009

16.2.5.5	Test requirement	2019

16.2.6	PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA	2023

16.2.6.1	Test purpose	2023

16.2.6.2	Test applicability	2023

16.2.6.3	Minimum conformance requirements	2024

16.2.6.4	Test description	2025

16.2.6.4.1	Initial conditions	2025

16.2.6.4.2	Test procedure	2026

16.2.6.4.3	Message contents	2027

16.2.6.5	Test requirement	2038

16.2.7	PRS-RSRP measurement reporting delay test case for single positioning frequency with reduced number of samples in FR2 SA	2043

16.2.7.1	Test purpose	2043

16.2.7.2	Test applicability	2043

16.2.7.3	Minimum conformance requirements	2043

16.2.7.4	Test description	2045

16.2.7.4.1	Initial conditions	2045

16.2.7.4.2	Test procedure	2045

16.2.7.4.3	Message contents	2046

16.2.7.5	Test requirement	2055

16.2.8	PRS-RSRP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA	2060

16.2.8.1	Test purpose	2060

16.2.8.2	Test applicability	2060

16.2.8.3	Minimum conformance requirements	2060

16.2.8.4	Test description	2062

16.2.8.4.1	Initial conditions	2062

16.2.8.4.2	Test procedure	2063

16.2.8.4.3	Message contents	2064

16.2.8.5	Test requirement	2074

16.3	NR PRS-RSRP measurement accuracy test case in RRC_CONNECTED	2079

16.3.1	PRS-RSRP measurement accuracy with PRS in FR1	2079

16.3.1.1	Test purpose	2079

16.3.1.2	Test applicability	2079

16.3.1.3	Minimum conformance requirements	2079

16.3.1.4	Test description	2081

16.3.1.4.1	Initial conditions	2081

16.3.1.4.2	Test procedure	2082

16.3.1.4.3	Message contents	2083

16.3.1.5	Test requirement	2094

16.3.2	PRS-RSRP measurement accuracy with PRS in FR2	2098

16.3.2.1	Test purpose	2098

16.3.2.2	Test applicability	2098

16.3.2.3	Minimum conformance requirements	2098

16.3.2.4	Test description	2100

16.3.2.4.1	Initial conditions	2100

16.3.2.4.2	Test procedure	2100

16.3.2.4.3	Message contents	2101

16.3.2.5	Test requirement	2110

16.3.3	PRS-RSRP measurement accuracy with PRS in FR1 with reduced sample number	2113

16.3.3.1	Test purpose	2113

16.3.3.2	Test applicability	2113

16.3.3.3	Minimum conformance requirements	2113

16.3.3.4	Test description	2115

16.3.3.4.1	Initial conditions	2115

16.3.3.4.2	Test procedure	2115

16.3.3.4.3	Message contents	2116

16.3.3.5	Test requirement	2126

16.3.4	PRS-RSRP measurement accuracy with PRS in FR2 with reduced sample number	2129

16.3.4.1	Test purpose	2129

16.3.4.2	Test applicability	2129

16.3.4.3	Minimum conformance requirements	2129

16.3.4.4	Test description	2130

16.3.4.4.1	Initial conditions	2130

16.3.4.4.2	Test procedure	2130

16.3.4.4.3	Message contents	2131

16.3.4.5	Test requirement	2140

16.4	NR PRS-RSRP measurement period test cases in RRC_INACTIVE	2143

16.4.1	PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR1 SA	2143

16.4.1.1	Test purpose	2143

16.4.1.2	Test applicability	2143

16.4.1.3	Minimum conformance requirements	2143

16.4.1.4	Test description	2144

16.4.1.4.1	Initial conditions	2144

16.4.1.4.2	Test procedure	2145

16.4.1.4.3	Message contents	2146

16.4.1.5	Test requirement	2158

16.4.2	PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2161

16.4.2.1	Test purpose	2161

16.4.2.2	Test applicability	2161

16.4.2.3	Minimum conformance requirements	2161

16.4.2.4	Test description	2161

16.4.2.4.1	Initial conditions	2161

16.4.2.4.2	Test procedure	2162

16.4.2.4.3	Message contents	2162

16.4.2.5	Test requirement	2168

16.4.3	PRS-RSRP measurement reporting delay test case for single positioning frequency layer in FR2 SA	2171

16.4.3.1	Test purpose	2171

16.4.3.2	Test applicability	2171

16.4.3.3	Minimum conformance requirements	2171

16.4.3.4	Test description	2173

16.4.3.4.1	Initial conditions	2173

16.4.3.4.2	Test procedure	2173

16.4.3.4.3	Message contents	2175

16.4.3.5	Test requirement	2185

16.4.4	PRS-RSRP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	2189

16.4.4.1	Test purpose	2189

16.4.4.2	Test applicability	2189

16.4.4.3	Minimum conformance requirements	2189

16.4.4.4	Test description	2190

16.4.4.4.1	Initial conditions	2190

16.4.4.4.2	Test procedure	2190

16.4.4.4.3	Message contents	2190

16.4.4.5	Test requirement	2194

16.5	NR PRS-RSRP measurement accuracy test case in RRC_INACTIVE	2199

16.5.1	PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR1 SA	2199

16.5.1.1	Test purpose	2199

16.5.1.2	Test applicability	2199

16.5.1.3	Minimum conformance requirements	2199

16.5.1.4	Test description	2202

16.5.1.4.1	Initial conditions	2202

16.5.1.4.2	Test procedure	2202

16.5.1.4.3	Message contents	2203

16.5.1.5	Test requirement	2209

16.5.2	PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR1 with reduced sample number	2212

16.5.2.1	Test purpose	2212

16.5.2.2	Test applicability	2212

16.5.2.3	Minimum conformance requirements	2212

16.5.2.4	Test description	2214

16.5.2.4.1	Initial conditions	2214

16.5.2.4.2	Test procedure	2215

16.5.2.4.3	Message contents	2215

16.5.2.5	Test requirement	2221

16.5.3	PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 SA	2223

16.5.3.1	Test purpose	2223

16.5.3.2	Test applicability	2223

16.5.3.3	Minimum conformance requirements	2223

16.5.3.4	Test description	2223

16.5.3.4.1	Initial conditions	2223

16.5.3.4.2	Test procedure	2224

16.5.3.4.3	Message contents	2225

16.5.3.5	Test requirement	2228

16.5.4	PRS-RSRP measurement accuracy test case for single positioning frequency layer with PRS in FR2 with reduced sample number	2230

16.5.4.1	Test purpose	2230

16.5.4.2	Test applicability	2230

16.5.4.3	Minimum conformance requirements	2230

16.5.4.4	Test description	2230

16.5.4.4.1	Initial conditions	2230

16.5.4.4.2	Test procedure	2230

16.5.4.4.3	Message contents	2231

16.5.4.5	Test requirement	2236

17	NR PRS-RSRPP measurement requirements	2239

17.1	General	2239

17.2	NR PRS-RSRPP measurement period test cases in RRC_CONNECTED	2239

17.2.1	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA	2239

17.2.1.1	Test purpose	2239

17.2.1.2	Test applicability	2239

17.2.1.3	Minimum conformance requirements	2239

17.2.1.4	Test description	2241

17.2.1.4.1	Initial conditions	2241

17.2.1.4.2	Test procedure	2241

17.2.1.4.3	Message contents	2242

17.2.1.5	Test requirement	2252

17.2.2	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2255

17.2.2.1	Test purpose	2255

17.2.2.2	Test applicability	2255

17.2.2.3	Minimum conformance requirements	2255

17.2.2.4	Test description	2255

17.2.2.4.1	Initial conditions	2255

17.2.2.4.2	Test procedure	2256

17.2.2.4.3	Message contents	2256

17.2.2.5	Test requirement	2265

17.2.3	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR1 SA	2268

17.2.3.1	Test purpose	2268

17.2.3.2	Test applicability	2268

17.2.3.3	Minimum conformance requirements	2268

17.2.3.4	Test description	2268

17.2.3.4.1	Initial conditions	2268

17.2.3.4.2	Test procedure	2269

17.2.3.4.3	Message contents	2270

17.2.3.5	Test requirement	2280

17.2.4	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR2 SA	2284

17.2.4.1	Test purpose	2284

17.2.4.2	Test applicability	2284

17.2.4.3	Minimum conformance requirements	2284

17.2.4.4	Test description	2284

17.2.4.4.1	Initial conditions	2284

17.2.4.4.2	Test procedure	2285

17.2.4.4.3	Message contents	2285

17.2.4.5	Test requirement	2294

17.2.5	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	2299

17.2.5.1	Test purpose	2299

17.2.5.2	Test applicability	2300

17.2.5.3	Minimum conformance requirements	2300

17.2.5.4	Test description	2300

17.2.5.4.1	Initial conditions	2300

17.2.5.4.2	Test procedure	2300

17.2.5.4.3	Message contents	2300

17.2.5.5	Test requirement	2308

17.2.6	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer without measurement gap in FR2 SA	2313

17.2.6.1	Test purpose	2313

17.2.6.2	Test applicability	2313

17.2.6.3	Minimum conformance requirements	2314

17.2.6.4	Test description	2314

17.2.6.4.1	Initial conditions	2314

17.2.6.4.2	Test procedure	2314

17.2.6.4.3	Message contents	2314

17.2.6.5	Test requirement	2323

17.3	NR PRS-RSRPP measurement accuracy test case in RRC_CONNECTED	2328

17.3.1	PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR1 SA	2328

17.3.1.1	Test purpose	2328

17.3.1.2	Test applicability	2329

17.3.1.3	Minimum conformance requirements	2329

17.3.1.4	Test description	2330

17.3.1.4.1	Initial conditions	2330

17.3.1.4.2	Test procedure	2331

17.3.1.4.3	Message contents	2332

17.3.1.5	Test requirement	2342

17.3.2	PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA	2345

17.3.2.1	Test purpose	2345

17.3.2.2	Test applicability	2345

17.3.2.3	Minimum conformance requirements	2345

17.3.2.4	Test description	2346

17.3.2.4.1	Initial conditions	2346

17.3.2.4.2	Test procedure	2347

17.3.2.4.3	Message contents	2347

17.3.2.5	Test requirement	2352

17.3.3	PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR2 SA	2354

17.3.3.1	Test purpose	2354

17.3.3.2	Test applicability	2354

17.3.3.3	Minimum conformance requirements	2354

17.3.3.4	Test description	2355

17.3.3.4.1	Initial conditions	2355

17.3.3.4.2	Test procedure	2356

17.3.3.4.3	Message contents	2356

17.3.3.5	Test requirement	2358

17.3.4	PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA	2360

17.3.4.1	Test purpose	2361

17.3.4.2	Test applicability	2361

17.3.4.3	Minimum conformance requirements	2361

17.3.4.4	Test description	2362

17.3.4.4.1	Initial conditions	2362

17.3.4.4.2	Test procedure	2362

17.3.4.4.3	Message contents	2362

17.3.4.5	Test requirement	2367

17.4	NR PRS-RSRPP measurement period test cases in RRC_INACTIVE	2370

17.4.1	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR1 SA	2370

17.4.1.1	Test purpose	2370

17.4.1.2	Test applicability	2370

17.4.1.3	Minimum conformance requirements	2370

17.4.1.4	Test description	2370

17.4.1.4.1	Initial conditions	2370

17.4.1.4.2	Test procedure	2370

17.4.1.4.3	Message contents	2372

17.4.1.5	Test requirement	2383

17.4.2	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2386

17.4.2.1	Test purpose	2386

17.4.2.2	Test applicability	2386

17.4.2.3	Minimum conformance requirements	2386

17.4.2.4	Test description	2386

17.4.2.4.1	Initial conditions	2386

17.4.2.4.2	Test procedure	2387

17.4.2.4.3	Message contents	2387

17.4.2.5	Test requirement	2392

17.4.3	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer in FR2 SA	2395

17.4.3.1	Test purpose	2395

17.4.3.2	Test applicability	2395

17.4.3.3	Minimum conformance requirements	2395

17.4.3.4	Test description	2395

17.4.3.4.1	Initial conditions	2395

17.4.3.4.2	Test procedure	2396

17.4.3.4.3	Message contents	2396

17.4.3.5	Test requirement	2398

17.4.4	PRS-RSRPP measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	2403

17.4.4.1	Test purpose	2403

17.4.4.2	Test applicability	2403

17.4.4.3	Minimum conformance requirements	2404

17.4.4.4	Test description	2404

17.4.4.4.1	Initial conditions	2404

17.4.4.4.2	Test procedure	2404

17.4.4.4.3	Message contents	2404

17.4.4.5	Test requirement	2410

17.5	NR PRS-RSRPP measurement accuracy test case in RRC_INACTIVE	2413

17.5.1	PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR1 SA	2413

17.5.1.1	Test purpose	2413

17.5.1.2	Test applicability	2413

17.5.1.3	Minimum conformance requirements	2413

17.5.1.4	Test description	2414

17.5.1.4.1	Initial conditions	2414

17.5.1.4.2	Test procedure	2414

17.5.1.4.3	Message contents	2415

17.5.1.5	Test requirement	2419

17.5.2	PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR1 SA	2421

17.5.2.1	Test purpose	2421

17.5.2.2	Test applicability	2421

17.5.2.3	Minimum conformance requirements	2421

17.5.2.4	Test description	2421

17.5.2.4.1	Initial conditions	2421

17.5.2.4.2	Test procedure	2422

17.5.2.4.3	Message contents	2422

17.5.2.5	Test requirement	2427

17.5.3	PRS-RSRPP measurement accuracy test case for single positioning frequency layer in FR2 SA	2430

17.5.3.1	Test purpose	2430

17.5.3.2	Test applicability	2430

17.5.3.3	Minimum conformance requirements	2430

17.5.3.4	Test description	2430

17.5.3.4.1	Initial conditions	2430

17.5.3.4.2	Test procedure	2431

17.5.3.4.3	Message contents	2431

17.5.3.5	Test requirement	2433

17.5.4	PRS-RSRPP measurement accuracy test case for single positioning frequency layer with reduced number of samples in FR2 SA	2435

17.5.4.1	Test purpose	2436

17.5.4.2	Test applicability	2436

17.5.4.3	Minimum conformance requirements	2436

17.5.4.4	Test description	2436

17.5.4.4.1	Initial conditions	2436

17.5.4.4.2	Test procedure	2436

17.5.4.4.3	Message contents	2436

17.5.4.5	Test requirement	2442

Annex A (informative): Connection Diagrams	2445

Annex B (normative): Converting A-GNSS UE-assisted measurement reports into position estimates	2456

B.1	Introduction	2456

B.2	UTRAN UE measurement reports for A-GPS L1 C/A only	2456

B.3	UTRAN UE measurement reports for A-GNSS	2456

B.4	E-UTRAN and NR UE measurement reports	2458

B.5	WLS position solution	2458

Annex C (normative): General test conditions and declarations	2461

C.1	Acceptable uncertainty of Test System	2461

C.1.1	Measurement of test environments	2461

C.1.2	A-GNSS Minimum Performance requirements	2462

C.1.3	ECID and OTDOA Measurement requirements	2463

C.1.4	MBS Minimum Performance requirements	2474

C.1.5	WLAN and BLE measurement requirements	2474

C.1.6	NR PRS-based measurement requirements	2475

C.2	Test Parameter Relaxations (This clause is informative)	2483

C.2.1	A-GNSS Minimum Performance requirements	2484

C.2.2	ECID and OTDOA Measurement requirements	2485

C.2.3	MBS Minimum Performance requirements	2492

C.2.4	WLAN and BLE measurement requirements	2492

C.2.5	NR PRS-based Measurement requirements	2493

C.3	Interpretation of measurement results	2500

C.4	Derivation of Test Requirements (This clause is informative)	2501

Annex D (normative): Rules for statistical testing	2535

D.1	Test Method	2535

D.2	Error Ratio (ER)	2535

D.3	Test Design	2535

D.3.1	Confidence level	2535

D.3.2	Introduction: Supplier Risk versus Customer Risk	2535

D.3.3	Supplier Risk versus Customer Risk	2536

D.3.4	Introduction: Standard test versus early decision concept	2536

D.3.5	Standard test versus early decision concept	2537

D.3.6	Selectivity	2537

D.3.7	Design of the test	2538

D.4	Pass fail decisions	2540

D.4.1	Numerical definition of the pass fail limits for A-GNSS test cases	2540

D.4.2	Pass fail decision rules for A-GNSS test cases	2540

D.4.3	Numerical definition of the pass fail limits for ECID, OTDOA, MBS, WLAN and BLE test cases	2541

D.4.4	Pass fail decision rules for ECID, OTDOA, MBS, WLAN and BLE test cases	2542

D.4.5	Background information to the pass fail limits	2542

Annex E (normative): Conditions for ECID and OTDOA requirements	2544

E.1	Conditions for E-CID UE Rx – Tx time difference Measurements	2544

E.1.1	Conditions for E-CID UE Rx – Tx time difference by UE Category M1/M2	2544

E.2	Conditions for OTDOA intra-frequency RSTD Measurements	2545

E.2.1	Conditions for OTDOA intra-frequency RSTD Measurements by UE Category M1 and M2	2545

E.3	Conditions for OTDOA inter-frequency RSTD Measurements	2546

E.3.1	Conditions for OTDOA inter-frequency RSTD Measurements by UE Category M1 and M2	2546

E.4	Conditions for UE Rx-Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	2547

E.5	Conditions for NB-IOT OTDOA intra-frequency RSTD Measurements	2547

E.6	Conditions for NB-IOT OTDOA inter-frequency RSTD Measurements	2547

Annex F (normative): UTRAN Generic procedures	2549

F.1	General	2549

F.2	UTRAN connection set up	2549

F.2.1	Initial conditions	2549

F.2.2	Procedures	2549

F.2.3	Specific message contents	2550

F.3	UTRAN connection release	2550

F.3.1	Procedure	2550

F.3.2	Specific message contents	2550

Annex G (normative): Environmental conditions	2551

G.1	General	2551

G.2	Environmental requirements	2551

G.2.1	Temperature	2551

G.2.2	Voltage	2551

Annex H (informative): MBS Beacon parameters	2552

H.1	General	2552

H.2	Beacon parameters	2552

Annex I (normative): Conditions for NR PRS-based measurements	2553

I.1	General	2553

I.2	General	2553

Annex J (informative): Change history	2555