| 3GPP TS 29.549 V19.3.0 (2025-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	35

1	Scope	37

2	References	37

3	Definitions of terms and abbreviations	39

3.1	Terms	39

3.2	Abbreviations	39

4	Overview	40

5	Services offered by the SEAL servers	40

5.1	Introduction of SEAL services	40

5.2	Location management APIs	47

5.2.1	SS_LocationReporting API	47

5.2.1.1	Service Description	47

5.2.1.1.1	Overview	47

5.2.1.2	Service Operations	47

5.2.1.2.1	Introduction	47

5.2.1.2.2	Create_Trigger_Location_Reporting	47

5.2.1.2.2.1	General	47

5.2.1.2.2.2	 VAL server providing trigger configuration using Create_Trigger_Location_Reporting service operation	47

5.2.1.2.3	Fetch_Location_Report_Trigger	48

5.2.1.2.3.1	General	48

5.2.1.2.3.2	VAL server fetching trigger configuration using Fetch_Location_Report_Trigger service operation	48

5.2.1.2.4	Update_Trigger_Location_Reporting	48

5.2.1.2.4.1	General	48

5.2.1.2.4.2	 VAL server providing trigger configuration using Update_Trigger_Location_Reporting service operation	48

5.2.1.2.5	Cancel_Trigger_Location_Reporting	49

5.2.1.2.5.1	General	49

5.2.1.2.5.2	 VAL server providing trigger configuration using Cancel_Trigger_Location_Reporting service operation	49

5.2.1.2.6	Notify_Trigger_Location_Reporting	49

5.2.1.2.6.1	General	49

5.2.1.2.6.2	LM Server notifies the VAL Server on the location trigger event using Notify_Trigger_Location_Reporting	49

5.2.1.2.7	Notify_Adaptive_Configuration	49

5.2.1.2.7.1	General	49

5.2.1.2.7.2	Adaptive Location Configuration Notification	49

5.2.1.2.7.2.1	General	49

5.2.1.2.7.2.2	Adaptive Location Configuration Notification	50

5.2.2	SS_LocationInfoEvent API	50

5.2.3	SS_LocationInfoRetrieval API	50

5.2.4	SS_LocationAreaInfoRetrieval API	50

5.2.4.1	Service Description	50

5.2.4.1.1	Overview	50

5.2.4.2	Service Operations	50

5.2.4.2.1	Introduction	50

5.2.4.2.2	Obtain_UEs_Info	51

5.2.4.2.2.1	General	51

5.2.4.2.2.2	 VAL server obtains UE(s) information in an application defined proximity range of a location using Obtain_UEs_Info service operation	51

5.2.5	SS_LocationMonitoring API	51

5.2.6	SS_LocationAreaMonitoring API	52

5.2.7	SS_VALServiceAreaConfiguration API	52

5.2.7.1	Service Description	52

5.2.7.1.1	Overview	52

5.2.7.2	Service Operations	52

5.2.7.2.1	Introduction	52

5.2.7.2.2	Configure_VAL_Service_Area	53

5.2.7.2.2.1	General	53

5.2.7.2.2.2	 VAL Server configures VAL service area(s) using the Configure_VAL_Service_Area service operation	53

5.2.7.2.3	Obtain_VAL_Service_Area	53

5.2.7.2.3.1	General	53

5.2.7.2.3.2	 VAL Server obtains VAL service area(s) using the Obtain_VAL_Service_Area service operation	53

5.2.7.2.4	Update_VAL_Service_Area	54

5.2.7.2.4.1	General	54

5.2.7.2.4.2	 VAL Server updates VAL service area(s) using the Update_VAL_Service_Area service operation	54

5.2.7.2.5	Delete_VAL_Service_Area	54

5.2.7.2.5.1	General	54

5.2.7.2.5.2	 VAL Server deletes service area(s) using the Delete_VAL_Service_Area service operation	54

5.2.7.2.6	Subscribe_VAL_Service_Area_Change_Event	55

5.2.7.2.6.1	General	55

5.2.7.2.6.2	 SEAL Server subscribes for the VAL service area(s) change event(s) reporting using the Subscribe_VAL_Service_Area_Change_Event service operation	55

5.2.7.2.7	Update_Subscription_VAL_Service_Area_Change_Event	55

5.2.7.2.7.1	General	55

5.2.7.2.7.2	 SEAL Server updates the subscription for the VAL service area(s) change event(s) reporting using the Update_Subscription_VAL_Service_Area_Change_Event service operation	55

5.2.7.2.8	Unsubscribe_VAL_Service_Area_Change_Event	56

5.2.7.2.8.1	General	56

5.2.7.2.8.2	SEAL server unsubscribes from the VAL service area(s) change event(s) using Unsubscribe_VAL_Service_Area_Change_Event	56

5.2.7.2.9	Notify_VAL_Service_Area_Change_Event	56

5.2.7.2.9.1	General	56

5.2.7.2.9.2	LM server notifies the SEAL Server on VAL service area(s) change event(s) using Notify_VAL_Service_Area_Change_Event	56

5.2.8	SS_LocationHistoryInfoEvent Service	56

5.2.8.1	Service Description	56

5.2.8.2	Service Operations	57

5.2.8.2.1	Introduction	57

5.2.8.2.2	SS_LocationHistoryInfoEvent_Create	57

5.2.8.2.2.1	General	57

5.2.8.2.2.2	Location Tracing Configuration Creation	57

5.2.8.2.3	SS_LocationHistoryInfoEvent_Update	57

5.2.8.2.3.1	General	57

5.2.8.2.3.2	Location Tracing Configuration Update	58

5.2.8.2.4	SS_LocationHistoryInfoEvent_Delete	58

5.2.8.2.4.1	General	58

5.2.8.2.4.2	Location Tracing Configuration Deletion	58

5.2.8.2.5	SS_LocationHistoryInfoEvent_Query	59

5.2.8.2.5.1	General	59

5.2.8.2.5.2	Location History Report(s) Retrieval	59

5.2.9	SS_ConfirmLocation Service	59

5.2.9.1	Service Description	59

5.2.9.2	Service Operations	59

5.2.9.2.1	Introduction	59

5.2.9.2.2	SS_ConfirmLocation_Subscribe	59

5.2.9.2.2.1	General	59

5.2.9.2.2.2	Location Confirmation Service Subscription Creation	60

5.2.9.2.2.3	Location Confirmation Service Subscription Update	60

5.2.9.2.2.4	Location Confirmation Service Subscription Deletion	60

5.2.9.2.3	SS_ConfirmLocation_Notify	61

5.2.9.2.3.1	General	61

5.2.9.2.3.2	Location Confirmation Service Usage Notification	61

5.2.10	SS_SLPositioningManagement API	61

5.2.10.1	Service Description	61

5.2.10.2	Service Operations	61

5.2.10.2.1	Introduction	61

5.2.10.2.2	SS_SLPositioningManagement_Subscribe	62

5.2.10.2.2.1	General	62

5.2.10.2.2.2	SL Positioning Management Subscription Creation	62

5.2.10.2.3	SL Positioning Management Subscription Update	62

5.2.10.2.2.4	SL Positioning Management Subscription Deletion	63

5.2.10.2.3	SS_SLPositioningManagement_Notify	63

5.2.10.2.3.1	General	63

5.2.10.2.3.2	SL Positioning Management Event Notification	63

5.2.10.2.4	SS_SLPositioningManagement_SRInfoRequest	63

5.2.10.2.4.1	General	63

5.2.10.2.4.2	Short-Range based positioning information request	63

5.3	Group management APIs	64

5.3.1	SS_GroupManagement API	64

5.3.1.1	Service Description	64

5.3.1.1.1	Overview	64

5.3.1.2	Service Operations	64

5.3.1.2.1	Introduction	64

5.3.1.2.2	Query_Group_Info	64

5.3.1.2.2.1	General	64

5.3.1.2.2.2	 VAL server fetching VAL group documents, group membership and configuration information using Query_Group_Info service operation	64

5.3.1.2.3	Update_Group_Info	65

5.3.1.2.3.1	General	65

5.3.1.2.3.2	VAL server modifying group membership and configuration using Update_Group_Info service operation	65

5.3.1.2.4	Create_Group	66

5.3.1.2.4.1	General	66

5.3.1.2.4.2	VAL server creating new group using Create_Group service operation	66

5.3.1.2.5	Delete_Group	66

5.3.1.2.5.1	General	66

5.3.1.2.5.2	VAL server deleting VAL group using Delete_Group service operation	67

5.3.2	SS_GroupManagementEvent API	67

5.4	Configuration management APIs	67

5.4.1	SS_UserProfileRetrieval API	67

5.4.1.1	Service Description	67

5.4.1.1.1	Overview	67

5.4.1.2	Service Operations	67

5.4.1.2.1	Introduction	67

5.4.1.2.2	Obtain_User_Profile	68

5.4.1.2.2.1	General	68

5.4.1.2.2.2	 VAL server retrieving VAL user profile information using Obtain_User_Profile service operation	68

5.4.2	SS_UserProfileEvent API	68

5.4.3	SS_VALServiceData API	68

5.4.3.1	Service Description	68

5.4.3.1.1	Overview	68

5.4.3.2	Service Operations	68

5.4.3.2.1	Introduction	68

5.4.3.2.2	Obtain_VAL_Service_Data	69

5.4.3.2.2.1	General	69

5.4.3.2.2.2	 SEAL Server retrieving VAL service data using the Obtain_VAL_Service_Data service operation	69

5.5	Network resource management APIs	69

5.5.1	SS_NetworkResourceAdaptation API	69

5.5.1.1	Service Description	69

5.5.1.1.1	Overview	69

5.5.1.2	Service Operations	69

5.5.1.2.1	Introduction	69

5.5.1.2.2	Void	71

5.5.1.2.3	Reserve_Network_Resource	71

5.5.1.2.3.1	General	71

5.5.1.2.3.2	 Service consumer requesting for network resource adaptation using Reserve_Network_Resource service operation	71

5.5.1.2.4	Reserve_Network_Resource_Modify	71

5.5.1.2.4.1	General	71

5.5.1.2.4.2	Service consumer requesting for updating the network resource adaptation using Reserve_Network_Resource_Modify service operation	71

5.5.1.2.5	Request_Multicast_Resource	71

5.5.1.2.5.1	General	71

5.5.1.2.5.2	Service consumer requesting for multicast resource using Request_Multicast_Resource service operation	72

5.5.1.2.6	Notify_UP_Delivery_Mode	72

5.5.1.2.6.1	General	72

5.5.1.2.6.2	Notifying user plane events using Notify_UP_Delivery_Mode service operation	72

5.5.1.2.7	Create_TSC_Stream	72

5.5.1.2.7.1	General	72

5.5.1.2.7.2	 Service consumer requesting for create TSC stream using Create_TSC_Stream service operation	72

5.5.1.2.8	Delete_TSC_Stream	73

5.5.1.2.8.1	General	73

5.5.1.2.8.2	Service consumer requesting to delete a TSC stream using Delete_TSC_Stream service operation	73

5.5.1.2.9	Discover_TSC_Stream_Availability	74

5.5.1.2.9.1	General	74

5.5.1.2.9.2	Service consumer discovering TSC stream availability using Discover_TSC_Stream_Availability service operation	74

5.5.1.2.10	Create_MBS_Resource	74

5.5.1.2.10.1	General	74

5.5.1.2.10.2	Service consumer requesting the creation of an MBS Resource using the Create_MBS_Resource service operation	74

5.5.1.2.11	Update_MBS_Resource	75

5.5.1.2.11.1	General	75

5.5.1.2.11.2	Service consumer requesting the update of an existing MBS Resource using the Update_MBS_Resource service operation	75

5.5.1.2.12	Delete_MBS_Resource	75

5.5.1.2.12.1	General	75

5.5.1.2.12.2	Service consumer requesting the deletion of an existing MBS Resource using the Delete_MBS_Resource service operation	75

5.5.1.2.13	Activate_MBS_Resource	76

5.5.1.2.13.1	General	76

5.5.1.2.13.2	Service consumer requesting the activation of an existing MBS Resource using the Activate_MBS_Resource service operation	76

5.5.1.2.14	Deactivate_MBS_Resource	76

5.5.1.2.14.1	General	76

5.5.1.2.14.2	Service consumer requesting the deactivation of an existing MBS Resource using the Deactivate_MBS_Resource service operation	76

5.5.1.2.15	BDT_Configuration_Request	77

5.5.1.2.15.1	General	77

5.5.1.2.15.2	Creation of background data transfer policy configuration using the BDT_Configuration_Request service operation	77

5.5.1.2.16	BDT_Negotiation_Notification	77

5.5.1.2.16.1	General	77

5.5.1.2.16.2	NRM Server notifying the background data transfer policy using the BDT_Negotiation_Notification service operation	77

5.5.1.2.17	BDT_Configuration_Get	77

5.5.1.2.17.1	General	77

5.5.1.2.17.2	Retreivel of an individual background data transfer policy configuration using the BDT_Configuration_Get service operation	78

5.5.1.2.18	BDT_Configuration_Update	78

5.5.1.2.18.1	General	78

5.5.1.2.18.2	Update of an individual background data transfer policy configuration using the BDT_Configuration_Update service operation	78

5.5.1.2.19	BDT_Configuration_Delete	78

5.5.1.2.19.1	General	78

5.5.1.2.19.2	Deletion an individual background data transfer policy configuration using the BDT_Configuration_Delete service operation	79

5.5.1.2.20	Subscribe_Unified_Traffic_Pattern_and_Monitoring_Management	79

5.5.1.2.20.1	General	79

5.5.1.2.20.2	Service consumer subscribing to unified traffic pattern using the Subscribe_Unified_Traffic_Pattern_and_Monitoring_Management service operation	79

5.5.1.2.21	Update_Unified_Traffic_Pattern_and_Monitoring_Management_Subscription	79

5.5.1.2.21.1	General	79

5.5.1.2.21.2	Service consumer updating the subscription to unified traffic pattern using the Update_Unified_Traffic_Pattern_and_Monitoring_Management_Subscription service operation	79

5.5.1.2.22	Unsubscribe_Unified_Traffic_Pattern_and_Monitoring_Management	80

5.5.1.2.22.1	General	80

5.5.1.2.22.2	Service consumer deleting the subscription to unified traffic pattern using the Unsubscribe_Unified_Traffic_Pattern_and_Monitoring_Management service operation	80

5.5.1.2.23	Notify_Unified_Traffic_Pattern_Update	80

5.5.1.2.23.1	General	80

5.5.1.2.23.2	NRM Server notifying on unified traffic pattern monitoring and management related event(s) using the Notify_Unified_Traffic_Pattern_Update service operation	80

5.5.1.2.24	Reliable_Transmission_Request	81

5.5.1.2.24.1	General	81

5.5.1.2.24.2	Reliable Transmission Service Request	81

5.5.2	SS_EventsMonitoring API	81

5.5.3	SS_NetworkResourceMonitoring API	81

5.5.3.1	Service Description	81

5.5.3.1.1	Overview	81

5.5.3.2	Service Operations	81

5.5.3.2.1	Introduction	81

5.5.3.2.2	Subscribe_Unicast_QoS_Monitoring	82

5.5.3.2.2.1	General	82

5.5.3.2.2.2	VAL server subscribes for Unicast QoS Monitoring using Subscribe_Unicast_QoS_Monitoring	82

5.5.3.2.3	Unsubscribe_Unicast_QoS_Monitoring	83

5.5.3.2.3.1	General	83

5.5.3.2.3.2	VAL server unsubscribes for Unicast QoS Monitoring using Unsubscribe_Unicast_QoS_Monitoring	83

5.5.3.2.4	Notify_Unicast_QoS_Monitoring	83

5.5.3.2.4.1	General	83

5.5.3.2.4.2	NRM server notifies for Unicast QoS Monitoring using Notify_Unicast_QoS_Monitoring	83

5.5.3.2.5	Obtain_Unicast_QoS_Monitoring_Data	84

5.5.3.2.5.1	General	84

5.5.3.2.6	Update_Unicast_QoS_Monitoring_Subscription	84

5.5.3.2.6.1	General	84

5.5.3.2.6.2	VAL server modifies for Unicast QoS Monitoring Subscription using Update_Unicast_QoS_Monitoring Subscription	84

5.5.4	SS_SatelliteSFInfoEvent API	85

5.6	Events APIs	85

5.6.1	SS_Events API	85

5.6.1.1	Service Description	85

5.6.1.1.1	Overview	85

5.6.1.2	Service Operations	85

5.6.1.2.1	Introduction	85

5.6.1.2.2	Subscribe_Event	86

5.6.1.2.2.1	General	86

5.6.1.2.2.2	Subscribing to SEAL events using Subscribe_Event service operation	86

5.6.1.2.3	Notify_Event	86

5.6.1.2.3.1	General	86

5.6.1.2.3.2	Notifying SEAL events using Notify_Event service operation	87

5.6.1.2.4	Unsubscribe_Event	87

5.6.1.2.4.1	General	87

5.6.1.2.4.2	Unsubscribing from SEAL events using Unsubscribe_Event service operation	87

5.6.1.2.5	Update_Subscription	87

5.6.1.2.5.1	General	87

5.6.1.2.5.2	Updating the SEAL events subscription using Update_Subscription service operation	87

5.7	Key management APIs	88

5.7.1	SS_KeyInfoRetrieval API	88

5.7.1.1	Service Description	88

5.7.1.1.1	Overview	88

5.7.1.2	Service Operations	88

5.7.1.2.1	Introduction	88

5.7.1.2.2	Obtain_Key_Info	88

5.7.1.2.2.1	General	88

5.7.1.2.2.2	 VAL server obtaining VAL service specific key material using Obtain_Key_Info service operation	88

5.7.2	SS_KMParametersProvisioning API	89

5.7.2.1	Service Description	89

5.7.2.1.1	Overview	89

5.7.2.2	Service Operations	89

5.7.2.2.1	Introduction	89

5.7.2.2.2	Provide_Configuration	89

5.7.2.2.2.1	General	89

5.7.2.2.2.2	VAL server provisioning key parameter information using Provide_Configuration service operation	89

5.8	Network Slice Capability Enablement APIs	89

5.9	Identity Management APIs	90

5.9.1	SS_IdmParameterProvisioning API	90

5.9.1.1	Service Description	90

5.9.1.1.1	Overview	90

5.9.1.2	Service Operations	90

5.9.1.2.1	Introduction	90

5.9.1.2.2	Provide_Configuration	90

5.9.1.2.2.1	General	90

5.9.1.2.2.2	VAL server provisioning VAL service specific information using Provide_Configuration service operation	90

5.9.1.2.3	Get_Configuration	91

5.9.1.2.3.1	General	91

5.9.1.2.3.2	Service consumer obtaining the VAL service specific information provisioned using Get_Configuration service operation	91

5.9.1.2.4	Update_Configuration	91

5.9.1.2.4.1	General	91

5.9.1.2.4.2	Service consumer updating VAL service specific information using Update_Configuration service operation	91

5.9.1.2.5	Delete_Configuration	92

5.9.1.2.5.1	General	92

5.9.1.2.5.2	Service consumer deleting VAL service specific information using Delete_Configuration service operation	92

5.10	Data Delivery APIs	92

5.11	Application data analytics enablement service configuration APIs	92

5.11.1	SS_ADAE_VALPerformanceAnalytics API	92

5.11.1.1	Service Description	92

5.11.1.1.1	Overview	92

5.11.1.2	Service Operations	92

5.11.1.2.1	Introduction	92

5.11.1.2.2	Subscribe_VAL_Performance_Analytics	93

5.11.1.2.2.1	General	93

5.11.1.2.2.2	Subscribing to VAL performance analytics event using Subscribe_VAL_Performance_Analytics service operation	93

5.11.1.2.3	Notify_VAL_Performance_Analytics	93

5.11.1.2.3.1	General	93

5.11.1.2.3.2	Notifying VAL performance analytics event using Notify_VAL_Performance_Analytics service operation	93

5.11.1.2.4	Unsubscribe_VAL_Performance_Analytics	94

5.11.1.2.4.1	General	94

5.11.1.2.4.2	Unsubscribing from VAL performance analytics event using Unsubscribe_VAL_Performance_Analytics service operation	94

5.11.2	SS_ADAE_SlicePerformanceAnalytics API	94

5.11.2.1	Service Description	94

5.11.2.1.1	Overview	94

5.11.2.2	Service Operations	94

5.11.2.2.1	Introduction	94

5.11.2.2.2	Subscribe_Slice_Performance_Analytics	95

5.11.2.2.2.1	General	95

5.11.2.2.2.2	Subscribing to slice-specific application performance analytics event using Subscribe_Slice_Performance_Analytics service operation	95

5.11.2.2.3	Notify_Slice_Performance_Analytics	95

5.11.2.2.3.1	General	95

5.11.2.2.3.2	Notifying slice-specific application performance analytics event using Notify_Slice_Performance_Analytics service operation	95

5.11.2.2.4	Unsubscribe_Slice_Performance_Analytics	96

5.11.2.2.4.1	General	96

5.11.2.2.4.2	Unsubscribing from slice-specific application performance analytics event using Unsubscribe_Slice_Performance_Analytics service operation	96

5.11.3	SS_ADAE_Ue2UePerformanceAnalytics API	96

5.11.3.1	Service Description	96

5.11.3.1.1	Overview	96

5.11.3.2	Service Operations	96

5.11.3.2.1	Introduction	96

5.11.3.2.2	UE-to-UE_Performance_Analytics_Subscribe	97

5.11.3.2.2.1	General	97

5.11.3.2.2.2	Subscribing for UE-to-UE performance analytics using UE-to-UE_Performance_Analytics_Subscribe	97

5.11.3.2.3	UE-to-UE_Performance_Analytics_Notify	97

5.11.3.2.3.1	General	97

5.11.3.2.3.2	Notifying UE-to-UE performance analytics events using UE-to-UE_Performance_Analytics_Notify	97

5.11.3.2.3.2	UE-to-UE_Performance_Analytics_Unsubscribe	98

5.11.3.2.3.2.1	General	98

5.11.3.2.3.2.2	Unsubscribing from the UE-to-UE performance analytics events using UE-to-UE_Performance_Analytics_Unsubscribe	98

5.11.4	SS_ADAE_LocationAccuracyAnalytics API	98

5.11.4.1	Service Description	98

5.11.4.1.1	Overview	98

5.11.4.2	Service Operations	98

5.11.4.2.1	Introduction	98

5.11.4.2.2	Subscribe_Location_Accuracy_Analytics	99

5.11.4.2.2.1	General	99

5.11.4.2.2.2	Subscribing to location accuracy performance analytics event using Subscribe_Location_Accuracy_Analytics service operation	99

5.11.4.2.3	Notify_Location_Accuracy_Analytics	99

5.11.4.2.3.1	General	99

5.11.4.2.3.2	Notifying location accuracy performance analytics event using Notify_Location_Accuracy_Analytics service operation	99

5.11.4.2.4	Unsubscribe_Location_Accuracy_Analytics	99

5.11.4.2.4.1	General	99

5.11.4.2.4.2	Unsubscribing from location accuracy performance analytics event using Unsubscribe_Location_Accuracy_Analytics service operation	99

5.11.5	SS_ADAE_ServiceApiAnalytics API	100

5.11.5.1	Service Description	100

5.11.5.1.1	Overview	100

5.11.5.2	Service Operations	100

5.11.5.2.1	Introduction	100

5.11.5.2.2	Subscribe_Service_API_Analytics	100

5.11.5.2.2.1	General	100

5.11.5.2.2.2	Subscribing to service API analytics event using Subscribe_Service_API_Analytics service operation	100

5.11.5.2.3	Notify_Service_API_Analytics	101

5.11.5.2.3.1	General	101

5.11.5.2.3.2	Notifying service API analytics event using Notify_Service_API_Analytics service operation	101

5.11.5.2.4	Unsubscribe_Service_API_Analytics	101

5.11.5.2.4.1	General	101

5.11.5.2.4.2	Unsubscribing from service API analytics event using Unsubscribe_Service_API_Analytics service operation	101

5.11.6	SS_ADAE_SliceUsagePatternAnalytics API	102

5.11.6.1	Service Description	102

5.11.6.1.1	Overview	102

5.11.6.2	Service Operations	102

5.11.6.2.1	Introduction	102

5.11.6.2.2	Subscribe_Slice_Usage_Pattern_Analytics	102

5.11.6.2.2.1	General	102

5.11.6.2.2.2	Subscribing to slice usage pattern analytics using the Subscribe_Slice_Usage_Pattern_Analytics service operation	102

5.11.6.2.3	Notify_Slice_Usage_Pattern_Analytics	103

5.11.6.2.3.1	General	103

5.11.6.2.3.2	Notifying slice usage pattern analytics using the Notify_Slice_Usage_Pattern_Analytics service operation	103

5.11.6.2.4	Unsubscribe_Slice_Usage_Pattern_Analytics	103

5.11.6.2.4.1	General	103

5.11.6.2.4.2	Unsubscribing from slice usage pattern analytics events using the Unsubscribe_Slice_Usage_Pattern_Analytics service operation	103

5.11.6.2.5	Get_Slice_Usage_Stats	103

5.11.6.2.5.1	General	103

5.11.6.2.5.2	Retrieving slice usage statistics using the Get_Slice_Usage_Stats service operation	103

5.11.7	SS_ADAE_EdgeLoadAnalytics API	104

5.11.7.1	Service Description	104

5.11.7.1.1	Overview	104

5.11.7.2	Service Operations	104

5.11.7.2.1	Introduction	104

5.11.7.2.2	Subscribe_Edge_Load	104

5.11.7.2.2.1	General	104

5.11.7.2.2.2	Subscribing to edge load events using Subscribe_Edge_Load service operation	104

5.11.7.2.3	Notify_Edge_Load	105

5.11.7.2.3.1	General	105

5.11.7.2.3.2	Notifying edge load events using Notify_Edge_Load service operation	105

5.11.7.2.4	Unsubscribe_Edge_Load	105

5.11.7.2.4.1	General	105

5.11.7.2.4.2	Unsubscribing from edge load events using Unsubscribe_Edge_Load service operation	105

5.11.7.2.5	Get_Edge_Load_Data	105

5.11.7.2.5.1	General	105

5.11.7.2.5.2	Retrieving edge load analytics data using the Get_Edge_Load_Data service operation	105

5.11.8	SS_AADRF_DataManagement API	106

5.11.8.1	Service Description	106

5.11.8.1.1	Overview	106

5.11.8.2	Service Operations	106

5.11.8.2.1	Introduction	106

5.11.8.2.2	SS_AADRF_DataManagement_Subscribe service operation	106

5.11.8.2.2.1	General	106

5.11.8.2.2.2	Subscribing to the offline analytics/data using the SS_AADRF_DataManagement_Subscribe service operation	106

5.11.8.2.3	SS_AADRF_DataManagement_Notify service operation	107

5.11.8.2.3.1	General	107

5.11.8.2.3.2	Notifying the offline analytics/data using the SS_AADRF_DataManagement_Notify service operation	107

5.11.8.2.4	SS_AADRF_DataManagement_Unsubscribe	107

5.11.8.2.4.1	General	107

5.11.8.2.5	Data_Storage service operation	107

5.11.8.2.5.1	General	107

5.11.8.2.5.2	Requesting to store data using the Data_Storage service operation	107

5.11.9	SS_ADAE_LocationRelatedUeGroupAnalytics API	108

5.11.9.1	Service Description	108

5.11.9.2	Service Operations	108

5.11.9.2.1	Introduction	108

5.11.9.2.2	Subscribe	108

5.11.9.2.2.1	General	108

5.11.9.2.2.2	Location-Related UE Group Analytics Subscription Creation	108

5.11.9.2.2.3	Location-Related UE Group Analytics Subscription Deletion	109

5.11.9.2.2.4	Location-Related UE Group Analytics Subscription Update	109

5.11.9.2.3	Notify	109

5.11.9.2.3.1	General	109

5.11.9.2.3.2	Location-Related UE Group Analytics Event Notification	110

5.11.10	SS_ADAE_CollisionDetectionAnalytics API	110

5.11.10.1	Service Description	110

5.11.10.2	Service Operations	110

5.11.10.2.1	Introduction	110

5.11.10.2.2	Subscribe	110

5.11.10.2.2.1	General	110

5.11.10.2.2.2	Collision Detection Analytics Subscription Creation	110

5.11.10.2.2.3	Collision Detection Analytics Subscription Deletion	111

5.11.10.2.2.4	Collision Detection Analytics Subscription Update	111

5.11.10.2.3	Notify	111

5.11.10.2.3.1	General	111

5.11.10.2.3.2	Collision Detection Analytics Event Notification	112

5.11.11	SS_ADAE_AIMLMemberCapabilityAnalytics API	112

5.11.11.1	Service Description	112

5.11.11.2	Service Operations	112

5.11.11.2.1	Introduction	112

5.11.11.2.2	SS_ADAE_AIML_MemberCapabilityAnalytics_Subscribe	112

5.11.11.2.2.1	General	112

5.11.11.2.2.2	AIML Member Capability Analytics Subscription Creation	113

5.11.11.2.3	SS_ADAE_AIML_MemberCapabilityAnalytics_Unsubscribe	113

5.11.11.2.3.1	General	113

5.11.11.2.3.2	AIML Member Capability Analytics Subscription Deletion	113

5.11.11.2.4	SS_ADAE_AIML_MemberCapabilityAnalytics_Notify	114

5.11.11.2.4.1	General	114

5.11.11.2.4.2	AIML Member Capability Analytics Notification	114

5.11.11.2.5	SS_ADAE_AIML_MemberCapabilityAnalytics_Update	115

5.11.11.2.5.1	General	115

5.11.11.2.5.2	AIML Member Capability Analytics Subscription Update	115

5.11.12	SS_ADCCF_DataCollection API	116

5.11.12.1	Service Description	116

5.11.12.2	Service Operations	116

5.11.12.2.1	Introduction	116

5.11.12.2.2	Subscribe	117

5.11.12.2.2.1	General	117

5.11.12.2.2.2	Data Collection Subscription Creation	117

5.11.12.2.2.3	Data Collection Subscription Deletion	117

5.11.12.2.3	Notify	118

5.11.12.2.3.1	General	118

5.11.12.2.3.2	Data collection Event Notification	118

5.11.13	SS_ADAE_ServerToServerPerformanceAnalytics API	118

5.11.13.1	Service Description	118

5.11.13.2	Service Operations	118

5.11.13.2.1	Introduction	118

5.11.13.2.2	Subscribe	118

5.11.13.2.2.1	General	118

5.11.13.2.2.2	Server to Server Performance Analytics Subscription Creation	119

5.11.13.2.2.3	Server to Server Performance Analytics Subscription Update	119

5.11.13.2.2.4	Server to Server Performance Analytics Subscription Deletion	119

5.11.13.2.3	Notify	120

5.11.13.2.3.1	General	120

5.11.13.2.3.2	Server to Server Performance Analytics Event Notification	120

5.11.14	SS_ADAE_UeRatConnectivityAnalytics API	120

5.11.14.1	Service Description	120

5.11.14.2	Service Operations	120

5.11.14.2.1	Introduction	120

5.11.14.2.2	SS_ADAE_UeRatConnectivityAnalytics_Subscribe	121

5.11.14.2.2.1	General	121

5.11.14.2.2.2	UE RAT Connectivity Analytics Subscription Creation	121

5.11.14.2.3	SS_ADAE_UeRatConnectivityAnalytics_Subscription_Update	122

5.11.14.2.3.1	General	122

5.11.14.2.3.2	UE RAT Connectivity Analytics Subscription Update	122

5.11.14.2.4	SS_ADAE_UeRatConnectivityAnalytics_Unsubscribe	123

5.11.14.2.4.1	General	123

5.11.14.2.4.2	UE RAT Connectivity Analytics Subscription Deletion	123

5.11.14.2.5	SS_ADAE_UeRatConnectivityAnalytics_Notify	123

5.11.14.2.5.1	General	123

5.11.14.2.5.2	UE RAT Connectivity Analytics Notification	123

5.12	AIML Enablement APIs	124

5.13	Metaverse Enablement APIs	124

6	SEAL Design Aspects Common for All APIs	124

6.1	General	124

6.2	Data Types	124

6.2.1	General	124

6.2.2	Referenced structured data types	125

6.2.3	Referenced Simple data types and enumerations	125

6.3	Usage of HTTP	125

6.4	Content type	125

6.5	URI structure	126

6.6	Notifications	126

6.7	Error Handling	127

6.8	Feature negotiation	127

6.9	HTTP headers	127

6.10	Conventions for Open API specification files	127

7	SEAL API Definitions	127

7.1	Location management APIs	127

7.1.1	SS_LocationReporting API	127

7.1.1.1	API URI	127

7.1.1.2	Resources	127

7.1.1.2.1	Overview	127

7.1.1.2.2	Resource: SEAL Location Reporting Configurations	128

7.1.1.2.2.1	Description	128

7.1.1.2.2.2	Resource Definition	128

7.1.1.2.2.3	Resource Standard Methods	129

7.1.1.2.2.3.1	POST	129

7.1.1.2.2.4	 Resource Custom Operations	129

7.1.1.2.3	Resource: Individual SEAL Location Reporting Configuration	129

7.1.1.2.3.1	Description	129

7.1.1.2.3.2	Resource Definition	129

7.1.1.2.3.3	Resource Standard Methods	130

7.1.1.2.3.3.1	GET	130

7.1.1.2.3.3.2	PUT	131

7.1.1.2.3.3.3	DELETE	132

7.1.1.2.3.3.4	PATCH	133

7.1.1.2.3.4	Resource Custom Operations	134

7.1.1.3	Notifications	134

7.1.1.3.1	General	134

7.1.1.3.2	Location Trigger Event Notification	134

7.1.1.3.2.1	Description	134

7.1.1.3.2.2	Notification definition	134

7.1.1.3.3	Adaptive Location Configuration Notification	135

7.1.1.3.3.1	Description	135

7.1.1.3.3.2	Target URI	135

7.1.1.3.3.3	Standard Methods	136

7.1.1.3.3.3.1	POST	136

7.1.1.4	Data Model	137

7.1.1.4.1	General	137

7.1.1.4.2	Structured data types	139

7.1.1.4.2.1	Introduction	139

7.1.1.4.2.2	Type: LocationReportConfiguration	139

7.1.1.4.2.3	Type: LocationReportConfigurationPatch	140

7.1.1.4.2.4	Type: TriggeringCriteria	140

7.1.1.4.2.5	Type: LocationReport	141

7.1.1.4.2.6	Type: AdaptConfigNotif	141

7.1.1.4.2.7	Type: AdaptConfigNotifResp	141

7.1.1.4.3	Simple data types and enumerations	141

7.1.1.4.3.1	Introduction	141

7.1.1.4.3.2	Simple data types	141

7.1.1.4.3.3	Enumeration: InsideOutsideInd	141

7.1.1.4.3.4	Enumeration: LocChangeCond	142

7.1.1.4.3.5	Enumeration: AdaptiveReporting	142

7.1.1.4.3.6	Enumeration: Confirmation	142

7.1.1.5	Error Handling	143

7.1.1.5.1	General	143

7.1.1.5.2	Protocol Errors	143

7.1.1.5.3	Application Errors	143

7.1.1.6	Feature negotiation	143

7.1.2	SS_LocationAreaInfoRetrieval API	143

7.1.2.1	API URI	143

7.1.2.2	Resources	144

7.1.2.2.1	Overview	144

7.1.2.2.2	Resource: Location Information	144

7.1.2.2.2.1	Description	144

7.1.2.2.2.2	Resource Definition	144

7.1.2.2.2.3	Resource Standard Methods	145

7.1.2.2.2.3.1	GET	145

7.1.2.2.2.4	Resource Custom Operations	146

7.1.2.3	Notifications	146

7.1.2.4	Data Model	146

7.1.2.4.1	General	146

7.1.2.4.2	Structured Data Types	146

7.1.2.4.3	Simple data types and enumerations	146

7.1.2.5	Error Handling	146

7.1.2.5.1	General	146

7.1.2.5.2	Protocol Errors	147

7.1.2.5.3	Application Errors	147

7.1.2.6	Feature Negotiation	147

7.1.3	SS_VALServiceAreaConfiguration API	147

7.1.3.1	API URI	147

7.1.3.1A	Usage of HTTP	147

7.1.3.2	Resources	148

7.1.3.2.1	Overview	148

7.1.3.2.2	Resource: VAL Service Areas	149

7.1.3.2.2.1	Description	149

7.1.3.2.2.2	Resource Definition	149

7.1.3.2.2.3	Resource Standard Methods	149

7.1.3.2.2.3.1	GET	149

7.1.3.2.2.4	Resource Custom Operations	150

7.1.3.2.2.4.1	Overview	150

7.1.3.2.2.4.2	Operation: Configure	151

7.1.3.2.2.4.3	Operation: Update	151

7.1.3.2.2.4.4	Operation: Delete	152

7.1.3.2.3	Resource: VAL Service Area Change Subscriptions	153

7.1.3.2.3.1	Description	153

7.1.3.2.3.2	Resource Definition	153

7.1.3.2.3.3	Resource Standard Methods	153

7.1.3.2.3.3.1	POST	153

7.1.3.2.3.4	Resource Custom Operations	155

7.1.3.2.4	Resource: Individual VAL Service Area Change Subscription	155

7.1.3.2.4.1	Description	155

7.1.3.2.4.2	Resource Definition	155

7.1.3.2.4.3	Resource Standard Methods	155

7.1.3.2.4.3.1	GET	155

7.1.3.2.4.3.2	PUT	156

7.1.3.2.4.3.3	PATCH	157

7.1.3.2.4.3.4	DELETE	158

7.1.3.3.2	Individual Unicast Monitoring Notification	160

7.1.3.3.2.1	Description	160

7.1.3.3.2.2	Notification definition	160

7.1.3.4	Data Model	161

7.1.3.4.1	General	161

7.1.3.4.2	Structured data types	162

7.1.3.4.2.1	Introduction	162

7.1.3.4.2.2	Type: ValServiceArea	162

7.1.3.4.2.3	Type: ValServiceAreaReq	162

7.1.3.4.2.4	Type: ValServiceAreaData	162

7.1.3.4.2.5	Type: ValServiceAreaResp	162

7.1.3.4.2.6	Type: ValServiceAreaSubsc	163

7.1.3.4.2.7	Type: ValServiceAreaEventType	163

7.1.3.4.2.8	Type: ValServiceAreaNotif	163

7.1.3.4.2.9	Type: ValServiceAreaEventInfo	163

7.1.3.4.2.10	Type: ValServiceAreaSubscPatch	164

7.1.3.4.3	Simple data types and enumerations	164

7.1.3.4.3.1	Introduction	164

7.1.3.4.3.2	Simple data types	164

7.1.3.4.3.3	Enumeration: ValServiceAreaEvent	164

7.1.3.5	Error Handling	164

7.1.3.5.1	General	164

7.1.3.5.2	Protocol Errors	164

7.1.3.5.3	Application Errors	164

7.1.3.6	Feature negotiation	165

7.1.4	SS_LocationHistoryInfoEvent API	165

7.1.4.1	Introduction	165

7.1.4.2	Usage of HTTP	165

7.1.4.3	Resources	165

7.1.4.3.1	Overview	165

7.1.4.3.2	Resource: Location Tracing Configurations	166

7.1.4.3.2.1	Description	166

7.1.4.3.2.2	Resource Definition	166

7.1.4.3.2.3	Resource Standard Methods	167

7.1.4.3.2.3.1	POST	167

7.1.4.3.2.4	Resource Custom Operations	167

7.1.4.3.3	Resource: Individual Location Tracing Configuration	167

7.1.4.3.3.1	Description	167

7.1.4.3.3.2	Resource Definition	168

7.1.4.3.3.3	Resource Standard Methods	168

7.1.4.3.3.3.1	GET	168

7.1.4.3.3.3.2	PUT	169

7.1.4.3.3.3.3	PATCH	170

7.1.4.3.3.3.4	DELETE	171

7.1.4.3.3.4	Resource Custom Operations	172

7.1.4.3.4	Resource: Location History Reports	172

7.1.4.3.4.1	Description	172

7.1.4.3.4.2	Resource Definition	172

7.1.4.3.4.3	Resource Standard Methods	173

7.1.4.3.4.3.1	GET	173

7.1.4.3.4.4	Resource Custom Operations	174

7.1.4.4	Custom Operations without associated resources	174

7.1.4.5	Notifications	174

7.1.4.6	Data Model	174

7.1.4.6.1	General	174

7.1.4.6.2	Structured data types	175

7.1.4.6.2.1	Introduction	175

7.1.4.6.2.2	Type: LocTracingConfig	175

7.1.4.6.2.3	Type: LocTracingConfigPatch	176

7.1.4.6.2.4	Type: ReqLocHistory	176

7.1.4.6.2.5	Type: LocHistoryReports	176

7.1.4.6.2.6	Type: LocHistoryReport	177

7.1.4.6.3	Simple data types and enumerations	177

7.1.4.6.3.1	Introduction	177

7.1.4.6.3.2	Simple data types	177

7.1.4.6.4	Data types describing alternative data types or combinations of data types	177

7.1.4.6.5	Binary data	177

7.1.4.6.5.1	Binary Data Types	177

7.1.4.7	Error Handling	177

7.1.4.7.1	General	177

7.1.4.7.2	Protocol Errors	178

7.1.4.7.3	Application Errors	178

7.1.4.8	Feature negotiation	178

7.1.4.9	Security	178

7.1.5	SS_ConfirmLocation Service API	178

7.1.5.1	Introduction	178

7.1.5.2	Usage of HTTP	178

7.1.5.3	Resources	179

7.1.5.3.1	Overview	179

7.1.5.3.2	Resource: Location Confirmation Service Subscriptions	179

7.1.5.3.2.1	Description	179

7.1.5.3.2.2	Resource Definition	179

7.1.5.3.2.3	Resource Standard Methods	180

7.1.5.3.2.3.1	POST	180

7.1.5.3.2.4	Resource Custom Operations	180

7.1.5.3.3	Resource: Individual Location Confirmation Service Subscription	181

7.1.5.3.3.1	Description	181

7.1.5.3.3.2	Resource Definition	181

7.1.5.3.3.3	Resource Standard Methods	181

7.1.5.3.3.3.1	GET	181

7.1.5.3.3.3.2	PUT	182

7.1.5.3.3.3.3	PATCH	183

7.1.5.3.3.3.4	DELETE	184

7.1.5.3.3.4	Resource Custom Operations	185

7.1.5.4	Custom Operations without associated resources	185

7.1.5.5	Notifications	185

7.1.5.5.1	General	185

7.1.5.5.2	Location Confirmation Service Usage Notification	186

7.1.5.5.2.1	Description	186

7.1.5.5.2.2	Target URI	186

7.1.5.5.2.3	Standard Methods	186

7.1.5.5.2.3.1	POST	186

7.1.5.6	Data Model	187

7.1.5.6.1	General	187

7.1.5.6.2	Structured data types	188

7.1.5.6.2.1	Introduction	188

7.1.5.6.2.2	Type: LocConfirmSubsc	188

7.1.5.6.2.3	Type: LocConfirmSubscPatch	189

7.1.5.6.2.4	Type: LocConfirmUsageNotif	189

7.1.5.6.2.5	Type: LocConfirmUsageReport	189

7.1.5.6.3	Simple data types and enumerations	189

7.1.5.6.3.1	Introduction	189

7.1.5.6.3.2	Simple data types	189

7.1.5.6.4	Data types describing alternative data types or combinations of data types	190

7.1.5.6.5	Binary data	190

7.1.5.6.5.1	Binary Data Types	190

7.1.5.7	Error Handling	190

7.1.5.7.1	General	190

7.1.5.7.2	Protocol Errors	190

7.1.5.7.3	Application Errors	190

7.1.5.8	Feature negotiation	190

7.1.5.9	Security	190

7.1.6	SS_SLPositioningManagement Service API	190

7.1.6.1	Introduction	190

7.1.6.2	Usage of HTTP	191

7.1.6.3	Resources	191

7.1.6.3.1	Overview	191

7.1.6.3.2	Resource: SL Positioning Management Subscriptions	192

7.1.6.3.2.1	Description	192

7.1.6.3.2.2	Resource Definition	192

7.1.6.3.2.3	Resource Standard Methods	192

7.1.6.3.2.3.1	POST	192

7.1.6.3.2.4	Resource Custom Operations	193

7.1.6.3.3	Resource: Individual SL Positioning Management Subscription	193

7.1.6.3.3.1	Description	193

7.1.6.3.3.2	Resource Definition	193

7.1.6.3.3.3	Resource Standard Methods	193

7.1.6.3.3.3.1	GET	193

7.1.6.3.3.3.2	PUT	194

7.1.6.3.3.3.3	PATCH	195

7.1.6.3.3.3.4	DELETE	197

7.1.6.3.3.4	Resource Custom Operations	197

7.1.6.4	Custom Operations without associated resources	198

7.1.6.4.1	Overview	198

7.1.6.4.2	Operation: SR Positioning Information Request	198

7.1.6.4.2.1	Description	198

7.1.6.4.2.2	Operation Definition	198

7.1.6.5.2	SL Positioning Management Event Notification	200

7.1.6.5.2.1	Description	200

7.1.6.5.2.2	Target URI	200

7.1.6.5.2.3	Standard Methods	200

7.1.6.6	Data Model	201

7.1.6.6.1	General	201

7.1.6.6.2	Structured data types	202

7.1.6.6.2.1	Introduction	202

7.1.6.6.2.2	Type: SlPositionMgmtSubsc	203

7.1.6.6.2.3	Type: SlPositionMgmtSubscPatch	203

7.1.6.6.2.4	Type: SlPositionMgmtNotif	204

7.1.6.6.2.5	Type: ValUeInfo	204

7.1.6.6.2.6	Type: SrPosInfoReq	204

7.1.6.6.2.7	Type: SrPosInfoResp	205

7.1.6.6.2.8	Type: SrUeList	205

7.1.6.6.2.9	Type: SrPosInfo	205

7.1.6.6.3	Simple data types and enumerations	205

7.1.6.6.3.1	Introduction	205

7.1.6.6.3.2	Simple data types	206

7.1.6.6.3.4	Enumeration: Cause	206

7.1.6.6.3.5	Enumeration: ValUeSel	206

7.1.6.6.3.6	Enumeration: SrPosFilter	206

7.1.6.6.4	Data types describing alternative data types or combinations of data types	206

7.1.6.6.5	Binary data	207

7.1.6.6.5.1	Binary Data Types	207

7.1.6.7	Error Handling	207

7.1.6.7.1	General	207

7.1.6.7.2	Protocol Errors	207

7.1.6.7.3	Application Errors	207

7.1.6.8	Feature negotiation	207

7.1.6.9	Security	207

7.2	Group management APIs	207

7.2.1	SS_GroupManagement API	207

7.2.1.1	API URI	207

7.2.1.2	Resources	208

7.2.1.2.1	Overview	208

7.2.1.2.2	Resource: VAL Group Documents	209

7.2.1.2.2.1	Description	209

7.2.1.2.2.2	Resource Definition	209

7.2.1.2.2.3	Resource Standard Methods	209

7.2.1.2.2.3.1	POST	209

7.2.1.2.2.3.2	GET	210

7.2.1.2.2.4	 Resource Custom Operations	211

7.2.1.2.3	Resource: Individual VAL Group Document	211

7.2.1.2.3.1	Description	211

7.2.1.2.3.2	Resource Definition	211

7.2.1.2.3.3	Resource Standard Methods	212

7.2.1.2.3.3.1	GET	212

7.2.1.2.3.3.2	PUT	213

7.2.1.2.3.3.3	DELETE	214

7.2.1.2.3.3.4	PATCH	215

7.2.1.2.3.4	Resource Custom Operations	216

7.2.1.3	Notifications	216

7.2.1.4	Data Model	216

7.2.1.4.1	General	216

7.2.1.4.2	Structured data types	217

7.2.1.4.2.1	Introduction	217

7.2.1.4.2.2	Type: VALGroupDocument	217

7.2.1.4.2.3	Type: VALGroupDocumentPatch	218

7.2.1.4.3	Simple data types and enumerations	218

7.2.1.5	Error Handling	218

7.2.1.5.1	General	218

7.2.1.5.2	Protocol Errors	218

7.2.1.5.3	Application Errors	218

7.2.1.6	Feature negotiation	219

7.3	Configuration management APIs	219

7.3.1	SS_UserProfileRetrieval API	219

7.3.1.1	API URI	219

7.3.1.2	Resources	219

7.3.1.2.1	Overview	219

7.3.1.2.2	Resource: VAL Services	220

7.3.1.2.2.1	Description	220

7.3.1.2.2.2	Resource Definition	220

7.3.1.2.2.3	Resource Standard Methods	220

7.3.1.2.2.3.1	GET	220

7.3.1.2.2.4	 Resource Custom Operations	221

7.3.1.3	Notifications	221

7.3.1.4	Data Model	221

7.3.1.4.1	General	221

7.3.1.4.2	Structured data types	222

7.3.1.4.2.1	Introduction	222

7.3.1.4.2.2	Type: ProfileDoc	222

7.3.1.4.2.3	Type: ValTargetUe	222

7.3.1.4.2.4	Type: ValTargetUeRm	222

7.3.1.4.3	Simple data types and enumerations	222

7.3.1.5	Error Handling	222

7.3.1.5.1	General	222

7.3.1.5.2	Protocol Errors	223

7.3.1.5.3	Application Errors	223

7.3.1.6	Feature negotiation	223

7.3.2	SS_VALServiceData API	223

7.3.2.1	API URI	223

7.3.2.1A	Usage of HTTP	223

7.3.2.2	Resources	223

7.3.2.2.1	Overview	223

7.3.2.2.2	Resource: VAL Service Data Sets	224

7.3.2.2.2.1	Description	224

7.3.2.2.2.2	Resource Definition	224

7.3.2.2.2.3	Resource Standard Methods	224

7.3.2.2.2.3.1	GET	224

7.3.2.2.2.4	 Resource Custom Operations	225

7.3.2.3	Custom Operations without associated resources	225

7.3.2.4	Notifications	225

7.3.2.5	Data Model	226

7.3.2.5.1	General	226

7.3.2.5.2	Structured data types	226

7.3.2.5.2.1	Introduction	226

7.3.2.5.2.2	Type: ValServDataResp	226

7.3.2.5.2.3	Type: ValServiceData	227

7.3.2.5.3	Simple data types and enumerations	227

7.3.2.5.3.1	Introduction	227

7.3.2.5.3.2	Simple data types	227

7.3.2.5.4	Data types describing alternative data types or combinations of data types	227

7.3.2.5.5	Binary data	227

7.3.2.5.5.1	Binary Data Types	227

7.3.2.6	Error Handling	227

7.3.2.6.1	General	227

7.3.2.6.2	Protocol Errors	227

7.3.2.6.3	Application Errors	228

7.3.2.7	Feature negotiation	228

7.4	Network resource management APIs	228

7.4.1	SS_NetworkResourceAdaptation API	228

7.4.1.1	API URI	228

7.4.1.1A	Usage of HTTP	228

7.4.1.2	Resources	228

7.4.1.2.1	Overview	228

7.4.1.2.2	Resource: Multicast Subscriptions	234

7.4.1.2.2.1	Description	234

7.4.1.2.2.2	Resource Definition	234

7.4.1.2.2.3	Resource Standard Methods	234

7.4.1.2.2.3.1	POST	234

7.4.1.2.2.4	 Resource Custom Operations	235

7.4.1.2.3	Resource: Individual Multicast Subscription	235

7.4.1.2.3.1	Description	235

7.4.1.2.3.2	Resource Definition	235

7.4.1.2.3.3	Resource Standard Methods	235

7.4.1.2.3.3.1	GET	235

7.4.1.2.3.3.2	DELETE	236

7.4.1.2.3.4	 Resource Custom Operations	237

7.4.1.2.4	Resource: Unicast Subscriptions	237

7.4.1.2.4.1	Description	237

7.4.1.2.4.2	Resource Definition	237

7.4.1.2.4.3	Resource Standard Methods	238

7.4.1.2.4.3.1	POST	238

7.4.1.2.4.4	 Resource Custom Operations	238

7.4.1.2.5	Resource: Individual Unicast Subscription	238

7.4.1.2.5.1	Description	238

7.4.1.2.5.2	Resource Definition	238

7.4.1.2.5.3	Resource Standard Methods	239

7.4.1.2.5.3.1	GET	239

7.4.1.2.5.3.2	DELETE	240

7.4.1.2.5.3.3	PUT	240

7.4.1.2.5.3.4	PATCH	241

7.4.1.2.5.4	Resource Custom Operations	242

7.4.1.2.6	Resource: TSC Stream Availability	243

7.4.1.2.6.1	Description	243

7.4.1.2.6.2	Resource Definition	243

7.4.1.2.6.3	Resource Standard Methods	243

7.4.1.2.6.3.1	GET	243

7.4.1.2.6.4	 Resource Custom Operations	244

7.4.1.2.7	Resource: TSC streams	244

7.4.1.2.7.1	Description	244

7.4.1.2.7.2	Resource Definition	244

7.4.1.2.7.3	Resource Standard Methods	245

7.4.1.2.7.3.1	GET	245

7.4.1.2.7.4	 Resource Custom Operations	245

7.4.1.2.8	Resource: Individual TSC Stream	246

7.4.1.2.8.1	Description	246

7.4.1.2.8.2	Resource Definition	246

7.4.1.2.8.3	Resource Standard Methods	246

7.4.1.2.8.3.1	GET	246

7.4.1.2.8.3.2	PUT	247

7.4.1.2.8.3.3	DELETE	247

7.4.1.2.9	Resource: MBS Resources	248

7.4.1.2.9.1	Description	248

7.4.1.2.9.2	Resource Definition	248

7.4.1.2.9.3	Resource Standard Methods	249

7.4.1.2.9.3.1	POST	249

7.4.1.2.9.4	 Resource Custom Operations	249

7.4.1.2.10	Resource: Individual MBS Resource	250

7.4.1.2.10.1	Description	250

7.4.1.2.10.2	Resource Definition	250

7.4.1.2.10.3	Resource Standard Methods	250

7.4.1.2.10.3.1	GET	250

7.4.1.2.10.3.2	PUT	251

7.4.1.2.10.3.3	PATCH	252

7.4.1.2.10.3.4	DELETE	253

7.4.1.2.10.4	Resource Custom Operations	255

7.4.1.2.10.4.1	Overview	255

7.4.1.2.10.4.2	Operation: Activate	255

7.4.1.2.10.4.3	Operation: Deactivate	256

7.4.1.2.11	Resource: BDT Policy Configurations	257

7.4.1.2.11.1	Description	257

7.4.1.2.11.2	Resource definition	257

7.4.1.2.11.3	Resource methods	257

7.4.1.2.11.3.1	POST	257

7.4.1.2.12	Resource: Individual BDT Policy Configuration	258

7.4.1.2.12.1	Description	258

7.4.1.2.12.2	Resource definition	258

7.4.1.2.12.3	Resource methods	258

7.4.1.2.12.3.1	GET	258

7.4.1.2.12.3.2	PUT	259

7.4.1.2.12.3.3	PATCH	260

7.4.1.2.12.3.4	DELETE	261

7.4.1.2.12.4	 Resource Custom Operations	262

7.4.1.2.13	Resource: Unified Traffic Pattern Subscriptions	262

7.4.1.2.13.1	Description	262

7.4.1.2.13.2	Resource definition	262

7.4.1.2.13.3	Resource methods	263

7.4.1.2.13.3.1	POST	263

7.4.1.2.14	Resource: Individual Unified Traffic Pattern Subscription	263

7.4.1.2.14.1	Description	263

7.4.1.2.14.2	Resource definition	263

7.4.1.2.14.3	Resource methods	264

7.4.1.2.14.3.1	GET	264

7.4.1.2.14.3.2	PUT	265

7.4.1.2.14.3.3	PATCH	266

7.4.1.2.14.3.4	DELETE	267

7.4.1.2.14.4	 Resource Custom Operations	268

7.4.1.2A	Custom Operations without associated resources	269

7.4.1.2A.1	Overview	269

7.4.1.2A.2	Operation: RelTransRequest	269

7.4.1.2A.2.1	Description	269

7.4.1.2A.2.2	Operation Definition	269

7.4.1.3	Notifications	270

7.4.1.3.1	General	270

7.4.1.3.2	Notify_UP_Delivery_Mode	271

7.4.1.3.2.1	Description	271

7.4.1.3.2.2	Notification definition	271

7.4.1.3.3	BDT_Negotiation_Notification	272

7.4.1.3.3.1	Description	272

7.4.1.3.3.2	Notification definition	272

7.4.1.3.4	Unified_Traffic_Pattern_Notification	273

7.4.1.3.4.1	Description	273

7.4.1.3.4.2	Notification definition	273

7.4.1.4	Data Model	274

7.4.1.4.1	General	274

7.4.1.4.2	Structured data types	277

7.4.1.4.2.1	Introduction	277

7.4.1.4.2.2	Type: MulticastSubscription	277

7.4.1.4.2.3	Type: UnicastSubscription	278

7.4.1.4.2.4	Type: UserPlaneNotification	278

7.4.1.4.2.5	Type: NrmEventNotification	279

7.4.1.4.2.6	Type: TscStreamData	280

7.4.1.4.2.7	Type: TrafficSpecInformation	280

7.4.1.4.2.8	Type: TscStreamAvailability	280

7.4.1.4.2.9	Type: StreamSpecification	280

7.4.1.4.2.10	Type: TrafficSpecification	281

7.4.1.4.2.11	Type: MBSResourceReq	281

7.4.1.4.2.12	Type: MBSResource	282

7.4.1.4.2.13	Type: MBSResourceRespInfo	283

7.4.1.4.2.14	Type: MBSResourceResp	283

7.4.1.4.2.15	Type: MBSResourcePatch	284

7.4.1.4.2.16	Type: MbsResAct	284

7.4.1.4.2.17	Type: MbsResDeact	284

7.4.1.4.2.18	Type: BdtPolConfig	285

7.4.1.4.2.19	Type: GeoArea	285

7.4.1.4.2.20	Type: BdtNotification	286

7.4.1.4.2.21	Type: BdtPolConfigPatch	286

7.4.1.4.2.22	Type: PolicyGuidance	286

7.4.1.4.2.23	Type: UnfTrafficSubc	287

7.4.1.4.2.24	Type: UnfTrafficSubcPatch	287

7.4.1.4.2.25	Type: ManagementSubc	287

7.4.1.4.2.26	Type: TrafficPatternConfig	288

7.4.1.4.2.27	Type: UnfTrafficUpdNotif	288

7.4.1.4.2.28	Type: RelTransReq	289

7.4.1.4.2.29	Type: AppTraffDesc	289

7.4.1.4.2.30	Type: ValUeAddrInfo	289

7.4.1.4.2.31	Type: UnicastSubscriptionPatch	290

7.4.1.4.3	Simple data types and enumerations	290

7.4.1.4.3.0	Introduction	290

7.4.1.4.3.0A	Simple data types	290

7.4.1.4.3.1	Enumeration: ServiceAnnoucementMode	290

7.4.1.4.3.2	Enumeration: DeliveryMode	290

7.4.1.4.3.4	Enumeration: NetSysIndicator	291

7.4.1.4.3.5	Enumeration: NrmEvent	291

7.4.1.4.3.6	Enumeration: PolicyType	291

7.4.1.4.3.7	Enumeration: TrafficPatternIndication	291

7.4.1.5	Error Handling	291

7.4.1.5.1	General	291

7.4.1.5.2	Protocol Errors	291

7.4.1.5.3	Application Errors	291

7.4.1.6	Feature negotiation	292

7.4.2	SS_NetworkResourceMonitoring API	292

7.4.2.1	API URI	292

7.4.2.2	Resources	293

7.4.2.2.1	Overview	293

7.4.2.2.2	Resource: Unicast Monitoring Subscriptions	294

7.4.2.2.2.1	Description	294

7.4.2.2.2.2	Resource Definition	294

7.4.2.2.2.3	Resource Standard Methods	294

7.4.2.2.2.3.1	POST	294

7.4.2.2.2.4	Resource Custom Operations	295

7.4.2.2.3	Resource: Individual Unicast Monitoring Subscription	295

7.4.2.2.3.1	Description	295

7.4.2.2.3.2	Resource Definition	295

7.4.2.2.3.3	Resource Standard Methods	295

7.4.2.2.3.3.1	DELETE	295

7.4.2.2.3.3.2	GET	296

7.4.2.2.3.3.3	PUT	297

7.4.2.2.3.3.4	PATCH	298

7.4.2.3.2	Individual Unicast Monitoring Notification	299

7.4.2.3.2.1	Description	299

7.4.2.3.2.2	Notification definition	299

7.4.2.4	Data Model	300

7.4.2.4.1	General	300

7.4.2.4.2	Structured data types	302

7.4.2.4.2.1	Introduction	302

7.4.2.4.2.2	Type: MonitoringReport	302

7.4.2.4.2.3	Type: MeasurementData	302

7.4.2.4.2.4	Type: MeasurementPeriod	303

7.4.2.4.2.5	Type: ReportingRequirements	304

7.4.2.4.2.6	Type: MeasurementRequirements	305

7.4.2.4.2.7	Type: MonitoringSubscription	306

7.4.2.4.2.8	Void	307

7.4.2.4.2.9	Type: FailureReport	307

7.4.2.4.2.10	Type: ReportingThreshold	307

7.4.2.4.2.11	Type: MonitoringSubscriptionPatch	307

7.4.2.4.3	Simple data types and enumerations	307

7.4.2.4.3.0A	Introduction	307

7.4.2.4.3.0B	Simple data types	308

7.4.2.4.3.1	Enumeration: MeasurementDataType	308

7.4.2.4.3.2	Enumeration: TerminationMode	308

7.4.2.4.3.3	Enumeration: FailureReason	308

7.4.2.4.3.4	Enumeration: ThresholdHandlingMode	309

7.4.2.5	Error Handling	309

7.4.2.5.1	General	309

7.4.2.5.2	Protocol Errors	309

7.4.2.5.3	Application Errors	309

7.4.2.6	Feature negotiation	309

7.5	Event APIs	310

7.5.1	SS_Events API	310

7.5.1.1	API URI	310

7.5.1.2	Resources	310

7.5.1.2.1	Overview	310

7.5.1.2.2	Resource: SEAL Events Subscriptions	311

7.5.1.2.2.1	Description	311

7.5.1.2.2.2	Resource Definition	311

7.5.1.2.2.3	Resource Standard Methods	311

7.5.1.2.2.3.1	POST	311

7.5.1.2.2.4	Resource Custom Operations	311

7.5.1.2.3	Resource: Individual SEAL Events Subscription	312

7.5.1.2.3.1	Description	312

7.5.1.2.3.2	Resource Definition	312

7.5.1.2.3.3	Resource Standard Methods	312

7.5.1.2.3.3.1	DELETE	312

7.5.1.2.3.3.2	PATCH	313

7.5.1.2.3.3.3	PUT	314

7.5.1.2.3.4	Resource Custom Operations	315

7.5.1.3	Notifications	315

7.5.1.3.1	General	315

7.5.1.3.2	SEAL Event Notification	315

7.5.1.3.2.1	Description	315

7.5.1.3.2.2	Notification definition	315

7.5.1.4	Data Model	316

7.5.1.4.1	General	316

7.5.1.4.2	Structured data types	323

7.5.1.4.2.1	Introduction	323

7.5.1.4.2.2	SEALEventSubscription	323

7.5.1.4.2.3	SEALEventNotification	323

7.5.1.4.2.4	EventSubscription	324

7.5.1.4.2.5	SEALEventDetail	326

7.5.1.4.2.6	VALGroupFilter	327

7.5.1.4.2.7	IdentityFilter	328

7.5.1.4.2.8	LMInformation	330

7.5.1.4.2.9	MessageFilter	330

7.5.1.4.2.10	MonitorFilter	331

7.5.1.4.2.11	MonitorEvents	331

7.5.1.4.2.12	MonitorEventsReport	331

7.5.1.4.2.13	ValidityConditions	331

7.5.1.4.2.14	MonitorLocationInterestFilter	332

7.5.1.4.2.15	LocationDevMonReport	332

7.5.1.4.2.16	TempGroupInfo	332

7.5.1.4.2.17	MonLocAreaInterestFltr	333

7.5.1.4.2.18	LocationInfoCriteria	333

7.5.1.4.2.19	ReferenceUEDetail	333

7.5.1.4.2.20	LocationAreaMonReport	334

7.5.1.4.2.21	MoveInOutUEDetails	335

7.5.1.4.2.22	SEALEventSubscriptionPatch	335

7.5.1.4.2.23	PartialEventSubscFailRep	336

7.5.1.4.2.24	DataStatisticsReq	336

7.5.1.4.2.25	TimeReqs	336

7.5.1.4.2.26	GeofencingConditions	337

7.5.1.4.2.27	MovTimingInfo	337

7.5.1.4.2.28	SatSfInfo	338

7.5.1.4.2.29	RefUeCap	338

7.5.1.4.3	Simple data types and enumerations	338

7.5.1.4.3.1	Introduction	338

7.5.1.4.3.2	Simple data types	338

7.5.1.4.3.3	Enumeration: SEALEvent	339

7.5.1.4.3.4	Enumeration: LocDevNotification	339

7.5.1.4.3.5	Enumeration: MonLocTriggerEvent	340

7.5.1.4.3.6	Enumeration: SatSfTrigger	340

7.5.1.5	Error Handling	340

7.5.1.5.1	General	340

7.5.1.5.2	Protocol Errors	340

7.5.1.5.3	Application Errors	340

7.5.1.6	Feature Negotiation	341

7.6	Key management APIs	344

7.6.1	SS_KeyInfoRetrieval API	344

7.6.1.1	API URI	344

7.6.1.2	Resources	344

7.6.1.2.1	Overview	344

7.6.1.2.2	Resource: Key Records	345

7.6.1.2.2.1	Description	345

7.6.1.2.2.2	Resource Definition	345

7.6.1.2.2.3	Resource Standard Methods	345

7.6.1.2.2.3.1	GET	345

7.6.1.2.2.4	Resource Custom Operations	346

7.6.1.3	Notifications	346

7.6.1.4	Data Model	346

7.6.1.4.1	General	346

7.6.1.4.2	Structured Data Types	347

7.6.1.4.2.1	Introduction	347

7.6.1.4.2.2	ValKeyInfo	347

7.6.1.4.3	Simple data types and enumerations	347

7.6.1.5	Error Handling	347

7.6.1.5.1	General	347

7.6.1.5.2	Protocol Errors	347

7.6.1.5.3	Application Errors	348

7.6.1.6	Feature Negotiation	348

7.6.2	SS_KMParametersProvisioning API	348

7.6.2.1	Introduction	348

7.6.2.3	Usage of HTTP	348

7.6.2.3	Resources	348

7.6.2.4	Custom operations without associated resources	348

7.6.2.4.1	Overview	348

7.6.2.4.2	Operation: Request	349

7.6.2.4.2.1	Description	349

7.6.2.4.2.2	Operation Definition	349

7.6.2.5	Notifications	350

7.6.2.6	Data Model	350

7.6.2.6.1	General	350

7.6.2.6.2	Structured data types	351

7.6.2.6.2.1	Introduction	351

7.6.2.6.2.2	Type: VALKeyPpReq	351

7.6.2.6.2.2	Type: VALKeyPpResp	352

7.6.2.6.3	Simple data types and enumerations	352

7.6.2.6.3.1	Introduction	352

7.6.2.6.3.2	Simple data types	352

7.6.2.6.4	Data types describing alternative data types or combinations of data types	352

7.6.2.6.5	Binary data	352

7.6.2.6.5.1	Binary Data Types	352

7.6.2.7	Error Handling	353

7.6.2.7.1	General	353

7.6.2.7.2	Protocol Errors	353

7.6.2.7.3	Application Errors	353

7.6.2.8	Feature negotiation	353

7.7	Network Slice Capability Enablement APIs	353

7.8	Identity management APIs	353

7.8.1	SS_IdmParameterProvisioning API	353

7.8.1.1	API URI	353

7.8.1.2	Resources	354

7.8.1.2.1	Overview	354

7.8.1.2.2	Resource: VAL Services Configurations	354

7.8.1.2.2.1	Description	354

7.8.1.2.2.2	Resource Definition	354

7.8.1.2.2.3	Resource Standard Methods	355

7.8.1.2.2.3.1	POST	355

7.8.1.2.2.3.2	GET	355

7.8.1.2.2.4	 Resource Custom Operations	357

7.8.1.2.3	Resource: Individual VAL Services Configuration	357

7.8.1.2.3.1	Description	357

7.8.1.2.3.2	Resource Definition	357

7.8.1.2.3.3	Resource Standard Methods	357

7.8.1.2.3.3.1	GET	357

7.8.1.2.3.3.2	PUT	358

7.8.1.2.3.3.3	PATCH	359

7.8.1.2.3.3.4	DELETE	360

7.8.1.2.3.4	Resource Custom Operations	361

7.8.1.3	Custom operations without associated resources	361

7.8.1.4	Notifications	361

7.8.1.5	Data Model	362

7.8.1.5.1	General	362

7.8.1.5.2	Structured data types	362

7.8.1.5.2.1	Introduction	362

7.8.1.5.2.2	Type: VALServicesConfig	362

7.8.1.5.2.3	Type: VALServiceParams	362

7.8.1.5.2.4	Type: VALServicesConfigPatch	363

7.8.1.5.3	Simple data types and enumerations	363

7.8.1.5.3.1	Introduction	363

7.8.1.5.3.2	Simple data types	363

7.8.1.5.4	Data types describing alternative data types or combinations of data types	363

7.8.1.5.5	Binary data	363

7.8.1.5.5.1	Binary Data Types	363

7.8.1.6	Error Handling	363

7.8.1.6.1	General	363

7.8.1.6.2	Protocol Errors	363

7.8.1.6.3	Application Errors	363

7.8.1.7	Feature negotiation	364

7.9	Data Delivery APIs	364

7.10	Application data analytics enablement service configuration APIs	364

7.10.1	SS_ADAE_VALPerformanceAnalytics API	364

7.10.1.1	API URI	364

7.10.1.2	Resources	364

7.10.1.2.1	Overview	364

7.10.1.2.2	Resource: Application performance event subscription	365

7.10.1.2.2.1	Description	365

7.10.1.2.2.2	Resource Definition	365

7.10.1.2.2.3	Resource Standard Methods	365

7.10.1.2.2.3.1	POST	365

7.10.1.2.2.4	Resource Custom Operations	366

7.10.1.3	Notifications	368

7.10.1.3.2	Application performance event notification	368

7.10.1.3.2.1	Description	368

7.10.1.3.2.2	Notification definition	369

7.10.1.4	Data Model	370

7.10.1.4.1	General	370

7.10.1.4.2	Structured data types	371

7.10.1.4.2.1	Introduction	371

7.10.1.4.2.2	Type: AppPerfSub	371

7.10.1.4.2.3	Type: AppPerfNotif	372

7.10.1.4.2.4	Type: ProdProfileInfo	373

7.10.1.4.2.5	Type: ProducerCap	373

7.10.1.4.2.7	Type: PerfData	374

7.10.1.4.2.8	Type: PerfThreshold	374

7.10.1.4.2.9	Type: ClientApplicationData	374

7.10.1.4.2.10	Type: AnalyticsOutput	375

7.10.1.4.3	Simple data types and enumerations	375

7.10.1.4.3.1	Introduction	375

7.10.1.4.3.2	Simple data types	375

7.10.1.4.3.3	Enumeration: AnalyticsCategory	375

7.10.1.4.3.4	Enumeration: ProducerType	375

7.10.1.4.3.5	Enumeration: ProducerData	376

7.10.1.4.3.6	Enumeration: ProducerRole	376

7.10.1.4.3.7	Void	376

7.10.1.4.3.8	Enumeration: AnalyticsIdVpa	376

7.10.1.5	Error Handling	376

7.10.1.5.1	General	376

7.10.1.5.2	Protocol Errors	376

7.10.1.5.3	Application Errors	376

7.10.1.6	Feature Negotiation	377

7.10.2	SS_ADAE_SlicePerformanceAnalytics API	377

7.10.2.1	API URI	377

7.10.2.2	Resources	377

7.10.2.2.1	Overview	377

7.10.2.2.2	Resource: Slice-specific application performance event subscription	378

7.10.2.2.2.1	Description	378

7.10.2.2.2.2	Resource Definition	378

7.10.2.2.2.3	Resource Standard Methods	378

7.10.2.2.2.3.1	POST	378

7.10.2.2.2.4	Resource Custom Operations	379

7.10.2.3	Notifications	381

7.10.2.3.2	Slice-specific application performance event notification	381

7.10.2.3.2.1	Description	381

7.10.2.3.2.2	Notification definition	382

7.10.2.4	Data Model	383

7.10.2.4.1	General	383

7.10.2.4.2	Structured data types	383

7.10.2.4.2.1	Introduction	383

7.10.2.4.2.2	Type: SliceAppPerfSub	384

7.10.2.4.2.3	Type: SliceAppPerfNotif	384

7.10.2.4.2.4	Type: SliceAppPerfAnalytics	385

7.10.2.5	Error Handling	385

7.10.2.5.1	General	385

7.10.2.5.2	Protocol Errors	385

7.10.2.5.3	Application Errors	385

7.10.2.6	Feature Negotiation	386

7.10.3	SS_ADAE_Ue2UePerformanceAnalytics API	386

7.10.3.1	API URI	386

7.10.3.2	Resources	386

7.10.3.2.1	Overview	386

7.10.3.2.2	Resource: UE-to-UE session performance event subscription	387

7.10.3.2.2.1	Description	387

7.10.3.2.2.2	Resource Definition	387

7.10.3.2.2.3	Resource Standard Methods	387

7.10.3.2.2.3.1	POST	387

7.10.3.2.2.4	Resource Custom Operations	388

7.10.3.2.3	Resource: Individual UE-to-UE Session Performance Event Subscription	388

7.10.3.2.3.1	Description	388

7.10.3.2.3.2	Resource Definition	388

7.10.3.2.3.3	Resource Standard Methods	388

7.10.3.2.3.3.1	GET	388

7.10.3.2.3.3.2	DELETE	389

7.10.3.3	Notifications	390

7.10.3.3.2	UE-to-UE session performance event notification	390

7.10.3.3.2.1	Description	390

7.10.3.3.2.2	Notification definition	391

7.10.3.4	Data Model	392

7.10.3.4.1	General	392

7.10.3.4.2	Structured data types	393

7.10.3.4.2.1	Introduction	393

7.10.3.4.2.2	Type: U2UPerfSub	393

7.10.3.4.2.3	Type: U2UPerfNotif	393

7.10.3.4.2.4	Void	394

7.10.3.4.2.5	Type: U2UThreshold	394

7.10.3.4.2.6	Type: U2UAnalyticsData	394

7.10.3.4.2.7	Type: U2UPair	394

7.10.3.4.3	Simple data types and enumerations	395

7.10.3.4.3.1	Enumeration: U2UAnalytics	395

7.10.3.4.3.2	Enumeration: U2UReportingGranularity	395

7.10.3.5	Error Handling	395

7.10.3.5.1	General	395

7.10.3.5.2	Protocol Errors	395

7.10.3.5.3	Application Errors	395

7.10.3.6	Feature Negotiation	395

7.10.4	SS_ADAE_LocationAccuracyAnalytics API	396

7.10.4.1	API URI	396

7.10.4.2	Resources	396

7.10.4.2.1	Overview	396

7.10.4.2.2	Resource: Location accuracy event subscription	397

7.10.4.2.2.1	Description	397

7.10.4.2.2.2	Resource Definition	397

7.10.4.2.2.3	Resource Standard Methods	397

7.10.4.2.2.3.1	POST	397

7.10.4.2.2.4	Resource Custom Operations	397

7.10.4.3	Notifications	400

7.10.4.3.2	Location accuracy event notification	400

7.10.4.3.2.1	Description	400

7.10.4.3.2.2	Notification definition	400

7.10.4.4	Data Model	401

7.10.4.4.1	General	401

7.10.4.4.2	Structured data types	402

7.10.4.4.2.1	Introduction	402

7.10.4.4.2.2	Type: LocAccurSub	403

7.10.4.4.2.3	Type: LocAccurNotif	403

7.10.4.4.2.4	Type: LocAccurAnalyticsOutput	404

7.10.4.4.2.5	Type: LocAccurPrediction	404

7.10.4.4.2.6	Type: LocAccurSustain	404

7.10.4.4.3	Simple data types and enumerations	404

7.10.4.4.3.1	Introduction	404

7.10.4.4.3.2	Simple data types	405

7.10.4.4.3.3	Enumeration: LocAccurAnalytics	405

7.10.4.4.3.4	Enumeration: LocAccurChange	405

7.10.4.5	Error Handling	405

7.10.4.5.1	General	405

7.10.4.5.2	Protocol Errors	405

7.10.4.5.3	Application Errors	405

7.10.4.6	Feature Negotiation	406

7.10.5	SS_ADAE_ServiceApiAnalytics API	406

7.10.5.1	API URI	406

7.10.5.2	Resources	406

7.10.5.2.1	Overview	406

7.10.5.2.2	Resource: Service API event subscription	407

7.10.5.2.2.1	Description	407

7.10.5.2.2.2	Resource Definition	407

7.10.5.2.2.3	Resource Standard Methods	407

7.10.5.2.2.3.1	POST	407

7.10.5.2.2.4	Resource Custom Operations	408

7.10.5.3	Notifications	410

7.10.5.3.2	Service API event notification	410

7.10.5.3.2.1	Description	410

7.10.5.3.2.2	Notification definition	410

7.10.5.4	Data Model	412

7.10.5.4.1	General	412

7.10.5.4.2	Structured data types	412

7.10.5.4.2.1	Introduction	412

7.10.5.4.2.2	Type: SrvApiSub	413

7.10.5.4.2.3	Type: SrvApiNotif	413

7.10.5.4.3	Simple data types and enumerations	413

7.10.5.4.3.1	Introduction	413

7.10.5.4.3.2	Simple data types	413

7.10.5.5	Error Handling	413

7.10.5.5.1	General	413

7.10.5.5.2	Protocol Errors	414

7.10.5.5.3	Application Errors	414

7.10.5.6	Feature Negotiation	414

7.10.6	SS_ADAE_SliceUsagePatternAnalytics API	414

7.10.6.1	API URI	414

7.10.6.2	Resources	414

7.10.6.2.1	Overview	414

7.10.6.2.2	Resource: Slice usage pattern event subscriptions	415

7.10.6.2.2.1	Description	415

7.10.6.2.2.2	Resource Definition	415

7.10.6.2.2.3	Resource Standard Methods	415

7.10.6.2.2.3.1	POST	415

7.10.6.2.2.4	Resource Custom Operations	416

7.10.6.3	Notifications	418

7.10.6.3.2	Slice usage pattern event notification	419

7.10.6.3.2.1	Description	419

7.10.6.3.2.2	Notification definition	419

7.10.6.4	Data Model	420

7.10.6.4.1	General	420

7.10.6.4.2	Structured data types	420

7.10.6.4.2.1	Introduction	420

7.10.6.4.2.2	Type: SUPSub	421

7.10.6.4.2.3	Type: SUPNotif	422

7.10.6.4.2.4	Void	422

7.10.6.4.2.5	Void	422

7.10.6.4.3	Simple data types and enumerations	422

7.10.6.4.3.1	Introduction	422

7.10.6.4.3.2	Simple data types	422

7.10.6.4.3.3	Void	422

7.10.6.5	Error Handling	422

7.10.6.5.1	General	422

7.10.6.5.2	Protocol Errors	422

7.10.6.5.3	Application Errors	422

7.10.6.6	Feature Negotiation	423

7.10.7	SS_ADAE_EdgeLoadAnalytics API	423

7.10.7.1	API URI	423

7.10.7.2	Resources	423

7.10.7.2.1	Overview	423

7.10.7.2.2	Resource: Edge Load Event Subscription	424

7.10.7.2.2.1	Description	424

7.10.7.2.2.2	Resource Definition	424

7.10.7.2.2.3	Resource Standard Methods	424

7.10.7.2.2.3.1	POST	424

7.10.7.2.2.4	Resource Custom Operations	425

7.10.7.2.3	Resource: Individual Edge Load Event Subscription	425

7.10.7.2.3.1	Description	425

7.10.7.2.3.2	Resource Definition	425

7.10.7.2.3.3	Resource Standard Methods	425

7.10.7.2.3.3.1	GET	425

7.10.7.2.3.3.2	DELETE	426

7.10.7.2.3.4	Resource Custom Operations	427

7.10.7.3	Notifications	427

7.10.7.3.2	Edge load event notification	427

7.10.7.3.2.1	Description	427

7.10.7.3.2.2	Notification definition	428

7.10.7.4	Data Model	429

7.10.7.4.1	General	429

7.10.7.4.2	Structured data types	430

7.10.7.4.2.1	Introduction	430

7.10.7.4.2.2	Type: EdgeSub	431

7.10.7.4.2.3	Type: EdgeNotif	432

7.10.7.4.2.4	Void	432

7.10.7.4.2.5	Void	432

7.10.7.4.2.6	Void	432

7.10.7.4.2.7	Type: AnalyticsOutputEl	432

7.10.7.4.2.8	Type: EdgePlatformAnalytics	433

7.10.7.4.2.9	Type: EesAnalytics	433

7.10.7.4.2.10	Type: EasAnalytics	434

7.10.7.4.3	Simple data types and enumerations	434

7.10.7.4.3.1	Introduction	434

7.10.7.4.3.2	Simple data types	434

7.10.7.4.3.3	Enumeration: AnalyticsIdEl	434

7.10.7.5	Error Handling	434

7.10.7.5.1	General	434

7.10.7.5.2	Protocol Errors	434

7.10.7.5.3	Application Errors	435

7.10.7.6	Feature Negotiation	435

7.10.8	SS_AADRF_DataManagement API	435

7.10.8.1	API URI	435

7.10.8.2	Resources	435

7.10.8.2.1	Overview	435

7.10.8.2.2	Resource: A-ADRF Data Management Subscriptions	436

7.10.8.2.2.1	Description	436

7.10.8.2.2.2	Resource Definition	436

7.10.8.2.2.3	Resource Standard Methods	436

7.10.8.2.2.3.1	POST	436

7.10.8.2.2.4	Resource Custom Operations	437

7.10.8.2.3	Resource: Individual A-ADRF Data Management Subscription	437

7.10.8.2.3.1	Description	437

7.10.8.2.3.2	Resource definition	437

7.10.8.2.3.3	Resource Standard Methods	437

7.10.8.2.3.3.1	DELETE	437

7.10.8.2.3.4	Resource Custom Operations	438

7.10.8.3	Custom Operations without associated resources	438

7.10.8.4	Notifications	440

7.10.8.4.1	General	440

7.10.8.4.2	Event Notification	440

7.10.8.4.2.1	Description	440

7.10.8.4.2.2	Notification definition	440

7.10.8.5	Data Model	441

7.10.8.5.1	General	441

7.10.8.5.2	Structured data types	443

7.10.8.5.2.1	Introduction	443

7.10.8.5.2.2	Type: DataManageSub	443

7.10.8.5.2.3	Type: EventSubscription	444

7.10.8.5.2.4	Type: EdgeReq	445

7.10.8.5.2.5	Type: ApiLogReq	445

7.10.8.5.2.6	Type: DataManageNotification	446

7.10.8.5.2.7	Type: ApiLogInfo	447

7.10.8.5.2.8	Type: EdgeInfo	447

7.10.8.5.3	Simple data types and enumerations	450

7.10.8.5.3.1	Enumeration: AadrfEvent	450

7.10.8.5.3.2	Enumeration: ExposureLevel	451

7.10.8.6	Error Handling	451

7.10.8.6.1	General	451

7.10.8.6.2	Protocol Errors	451

7.10.8.6.3	Application Errors	451

7.10.8.7	Feature negotiation	451

7.10.9	SS_ADAE_LocationRelatedUeGroupAnalytics	452

7.10.9.1	Introduction	452

7.10.9.2	Usage of HTTP	452

7.10.9.3	Resources	452

7.10.9.3.1	Overview	452

7.10.9.3.2	Resource: Location-Related UE Group Analytics Subscriptions	453

7.10.9.3.2.1	Description	453

7.10.9.3.2.2	Resource Definition	453

7.10.9.3.2.3	Resource Standard Methods	454

7.10.9.3.2.3.1	POST	454

7.10.9.3.2.4	Resource Custom Operations	454

7.10.9.3.3	Resource: Individual Location-Related UE Group Analytics Subscription	454

7.10.9.3.3.1	Description	454

7.10.9.3.3.2	Resource Definition	454

7.10.9.3.3.3	Resource Standard Methods	455

7.10.9.3.3.3.1	GET	455

7.10.9.3.3.3.2	DELETE	455

7.10.9.3.3.3.3	PUT	456

7.10.9.3.3.3.4	PATCH	457

7.10.9.4	Custom Operations without associated resources	459

7.10.9.5	Notifications	459

7.10.9.5.2	Location-Related UE Group Analytics Notification	459

7.10.9.5.2.1	Description	459

7.10.9.5.2.2	Notification definition	459

7.10.9.6	Data Model	460

7.10.9.6.1	General	460

7.10.9.6.2	Structured data types	461

7.10.9.6.2.1	Introduction	461

7.10.9.6.2.2	Type: LocRelUeGroupSub	462

7.10.9.6.2.3	Type: LocRelUeGroupNotif	462

7.10.9.6.2.4	LocRelUeGroupFilter	463

7.10.9.6.2.5	Type: LocDeviationCriterion	463

7.10.9.6.2.6	Type: LocDeviationTypeData	463

7.10.9.6.2.7	Type: LocDeviationCriteria	463

7.10.9.6.2.8	Type: LocRelUeGroupSubPatch	464

7.10.9.6.2.9	Type: LocRelUeGroupAnalytics	464

7.10.9.6.2.10	Type: UeGroupRouteAnalytics	464

7.10.9.6.2.11	Type: UeGroupMemberDevAnalytics	464

7.10.9.6.2.12	Type: Deviation	465

7.10.9.6.3	Simple data types and enumerations	465

7.10.9.6.3.1	Introduction	465

7.10.9.6.3.2	Simple data types	465

7.10.9.6.3.3	Enumeration: AnalyticsIdLocGrp	465

7.10.9.6.4	Data types describing alternative data types or combinations of data types	465

7.10.9.6.5	Binary data	465

7.10.9.6.5.1	Binary Data Types	465

7.10.9.7	Error Handling	466

7.10.9.7.1	General	466

7.10.9.7.2	Protocol Errors	466

7.10.9.7.3	Application Errors	466

7.10.9.8	Feature Negotiation	466

7.10.9.9	Security	466

7.10.10	SS_ADAE_CollisionDetectionAnalytics	466

7.10.10.1	Introduction	466

7.10.10.2	Usage of HTTP	467

7.10.10.3	Resources	467

7.10.10.3.1	Overview	467

7.10.10.3.2	Resource: Collision Detection Analytics Subscriptions	468

7.10.10.3.2.1	Description	468

7.10.10.3.2.2	Resource Definition	468

7.10.10.3.2.3	Resource Standard Methods	468

7.10.10.3.2.3.1	POST	468

7.10.10.3.2.4	Resource Custom Operations	468

7.10.10.3.3	Resource: Individual Collision Detection Analytics Subscription	469

7.10.10.3.3.1	Description	469

7.10.10.3.3.2	Resource Definition	469

7.10.10.3.3.3	Resource Standard Methods	469

7.10.10.3.3.3.1	GET	469

7.10.10.3.3.3.2	DELETE	470

7.10.10.3.3.3.3	PUT	471

7.10.10.3.3.3.4	PATCH	472

7.10.10.4	Custom Operations without associated resources	473

7.10.10.5	Notifications	473

7.10.10.5.2	Collision detection analytics Notification	474

7.10.10.5.2.1	Description	474

7.10.10.5.2.2	Notification definition	474

7.10.10.6	Data Model	475

7.10.10.6.1	General	475

7.10.10.6.2	Structured data types	476

7.10.10.6.2.1	Introduction	476

7.10.10.6.2.2	Type: CollisionDetectionSub	477

7.10.10.6.2.3	Type: CollisionDetectionNotif	477

7.10.10.6.2.4	CollisionDetectionFilter	478

7.10.10.6.2.5	Type: TargetOtherUesFilter	478

7.10.10.6.2.6	Type: CollisionDetectionCriteria	478

7.10.10.6.2.7	Type: TargetToAnyUeLocFilter	478

7.10.10.6.2.8	Type: TargetOtherUesAnalytics	479

7.10.10.6.2.9	Type: TargetToAnyUeLocAnalytics	479

7.10.10.6.2.10	Type: Direction	479

7.10.10.6.2.11	Type: CollisionDetectionSubPatch	479

7.10.10.6.3	Simple data types and enumerations	480

7.10.10.6.3.1	Introduction	480

7.10.10.6.3.2	Simple data types	480

7.10.10.6.3.3	Enumeration: AnalyticsIdCda	480

7.10.10.6.4	Data types describing alternative data types or combinations of data types	480

7.10.10.6.5	Binary data	480

7.10.10.6.5.1	Binary Data Types	480

7.10.10.7	Error Handling	480

7.10.10.7.1	General	480

7.10.10.7.2	Protocol Errors	480

7.10.10.7.3	Application Errors	481

7.10.10.8	Feature Negotiation	481

7.10.10.9	Security	481

7.10.11	SS_ADAE_AIMLMemberCapabilityAnalytics API	481

7.10.11.1	Introduction	481

7.10.11.2	Usage of HTTP	481

7.10.11.3	Resources	482

7.10.11.3.1	Overview	482

7.10.11.3.2	Resource: AIML Member Capability Analytics Subscriptions	482

7.10.11.3.2.1	Description	482

7.10.11.3.2.2	Resource Definition	483

7.10.11.3.2.3	Resource Standard Methods	483

7.10.11.3.2.3.1	POST	483

7.10.11.3.2.4	Resource Custom Operations	483

7.10.11.3.3	Resource: Individual AIML Member Capability Analytics Subscription	483

7.10.11.3.3.1	Description	483

7.10.11.3.3.2	Resource Definition	484

7.10.11.3.3.3	Resource Standard Methods	484

7.10.11.3.3.3.1	GET	484

7.10.11.3.3.3.2	DELETE	485

7.10.11.3.3.3.3	PUT	485

7.10.11.3.3.3.4	PATCH	486

7.10.11.3.3.4	Resource Custom Operations	487

7.10.11.3A	Custom Operations without associated resources	488

7.10.11.4	Notifications	488

7.10.11.4.1	General	488

7.10.11.4.2	AIML Member Capability Analytics Notification	488

7.10.11.4.2.1	Description	488

7.10.11.4.2.2	Notification definition	488

7.10.11.5	Data Model	489

7.10.11.5.1	General	489

7.10.11.5.2	Structured data types	490

7.10.11.5.2.1	Introduction	490

7.10.11.5.2.2	Type: AppAimlMemCapSub	491

7.10.11.5.2.3	Type: Type: AppAimlMemCapNotif	491

7.10.11.5.2.4	Type: AppAimlMemCapSubPatch	491

7.10.11.5.2.5	Type: AppAimlMemCapAnalyticsSub	492

7.10.11.5.2.6	Type: AppAimlMemCapAnalyticsOutput	492

7.10.11.5.2.7	Type: AnalyticsOutput	493

7.10.11.5.2.8	Type: AimlMemCapability	493

7.10.11.5.3	Simple data types and enumerations	493

7.10.11.5.3.1	Introduction	493

7.10.11.5.3.2	Simple data types	493

7.10.11.5.3.3	Enumeration: AppAimlMemCapAnalytics	493

7.10.11.5.3.4	Enumeration: AppAimlMemCapAttributes	494

7.10.11.6	Error Handling	494

7.10.11.6.1	General	494

7.10.11.6.2	Protocol Errors	494

7.10.11.6.3	Application Errors	494

7.10.11.7	Feature Negotiation	494

7.10.11.8	Security	494

7.10.12	SS_ADAE_UeRatConnectivityAnalytics API	495

7.10.12.1	Introduction	495

7.10.12.2	Usage of HTTP	495

7.10.12.3	Resources	495

7.10.12.3.1	Overview	495

7.10.12.3.2	Resource: UE RAT Connectivity Analytics Subscriptions	496

7.10.12.3.2.1	Description	496

7.10.12.3.2.2	Resource Definition	496

7.10.12.3.2.3	Resource Standard Methods	496

7.10.12.3.2.3.1	POST	496

7.10.12.3.2.4	Resource Custom Operations	497

7.10.12.3.3	Resource: Individual UE RAT Connectivity Analytics Subscription	497

7.10.12.3.3.1	Description	497

7.10.12.3.3.2	Resource Definition	497

7.10.12.3.3.3	Resource Standard Methods	497

7.10.12.3.3.3.1	GET	497

7.10.12.3.3.3.2	PUT	498

7.10.12.3.3.3.3	PATCH	500

7.10.12.3.3.3.4	DELETE	501

7.10.12.3.3.4	Resource Custom Operations	502

7.10.12.4	Custom Operations without associated resources	502

7.10.12.5	Notifications	502

7.10.12.5.2	UE RAT Connectivity Analytics Notification	502

7.10.12.5.2.1	Description	502

7.10.12.5.2.2	Target URI	502

7.10.12.5.2.3	Standard Methods	502

7.10.12.5.2.3.1	POST	502

7.10.12.6	Data Model	503

7.10.12.6.1	General	503

7.10.12.6.2	Structured data types	504

7.10.12.6.2.1	Introduction	504

7.10.12.6.2.2	Type: UeRatConnectivitySub	505

7.10.12.6.2.3	Type: UeRatConnectivitySubPatch	505

7.10.12.6.2.4	Type: UeRatConnectivityNotif	505

7.10.12.6.2.5	Type: AreaPrediction	506

7.10.12.6.2.6	Type: TimeWindowPrediction	506

7.10.12.6.2.7	Type: UeRatAnalyticsSub	507

7.10.12.6.2.8	Type: UeRatAnalyticsOutput	507

7.10.12.6.3	Simple data types and enumerations	507

7.10.12.6.3.1	Introduction	507

7.10.12.6.3.2	Simple data types	508

7.10.12.6.3.3	Enumeration: AnalyticsUeIdRat	508

7.10.12.6.4	Data types describing alternative data types or combinations of data types	508

7.10.12.6.5	Binary data	508

7.10.12.7	Error Handling	508

7.10.12.7.1	General	508

7.10.12.7.2	Protocol Errors	508

7.10.12.7.3	Application Errors	508

7.10.12.8	Feature Negotiation	509

7.10.12.9	Security	509

7.10.13	SS_ADCCF_DataCollection	509

7.10.13.1	Introduction	509

7.10.13.2	Usage of HTTP	509

7.10.13.3	Resources	509

7.10.13.3.1	Overview	509

7.10.13.3.2	Resource: Data Collection Subscriptions	510

7.10.13.3.2.1	Description	510

7.10.13.3.2.2	Resource Definition	510

7.10.13.3.2.3	Resource Standard Methods	510

7.10.13.3.2.3.1	POST	510

7.10.13.3.2.4	Resource Custom Operations	511

7.10.13.3.3	Resource: Individual Data Collection Subscription	511

7.10.13.3.3.1	Description	511

7.10.13.3.3.2	Resource Definition	511

7.10.13.3.3.3	Resource Standard Methods	511

7.10.13.3.3.3.1	GET	511

7.10.13.3.3.3.2	DELETE	512

7.10.13.4	Custom Operations without associated resources	513

7.10.13.5	Notifications	513

7.10.13.5.2	Data Collection Notification	514

7.10.13.5.2.1	Description	514

7.10.13.5.2.2	Notification definition	514

7.10.13.6	Data Model	515

7.10.13.6.1	General	515

7.10.13.6.2	Structured data types	516

7.10.13.6.2.1	Introduction	516

7.10.13.6.2.2	Type: DataCollectionSub	517

7.10.13.6.2.3	Type: DataCollectionNotif	517

7.10.13.6.2.4	DataCollectionReq	517

7.10.13.6.2.5	Type: ProcessingReq	518

7.10.13.6.2.6	Type: AdccfDataType	518

7.10.13.6.2.7	Type: StorageRequirements	518

7.10.13.6.2.8	Type: DataOutput	518

7.10.13.6.3	Simple data types and enumerations	518

7.10.13.6.3.1	Introduction	518

7.10.13.6.3.2	Simple data types	518

7.10.13.6.3.3	Enumeration: DataType	519

7.10.13.6.3.4	Enumeration: GranularityType	519

7.10.13.6.4	Data types describing alternative data types or combinations of data types	519

7.10.13.6.5	Binary data	519

7.10.13.6.5.1	Binary Data Types	519

7.10.13.7	Error Handling	519

7.10.13.7.1	General	519

7.10.13.7.2	Protocol Errors	519

7.10.13.7.3	Application Errors	519

7.10.13.8	Feature Negotiation	520

7.10.13.9	Security	520

7.10.14	SS_ADAE_ServerToServerPerformanceAnalytics API	520

7.10.14.1	Introduction	520

7.10.14.2	Usage of HTTP	520

7.10.14.3	Resources	520

7.10.14.3.1	Overview	520

7.10.14.3.2	Resource: Server to Server Performance Analytics Subscriptions	521

7.10.14.3.2.1	Description	521

7.10.14.3.2.2	Resource Definition	521

7.10.14.3.2.3	Resource Standard Methods	522

7.10.14.3.2.3.1	POST	522

7.10.14.3.2.4	Resource Custom Operations	522

7.10.14.3.3	Resource: Individual Server to Server Performance Analytics Subscription	522

7.10.14.3.3.1	Description	522

7.10.14.3.3.2	Resource Definition	522

7.10.14.3.3.3	Resource Standard Methods	523

7.10.14.3.3.3.1	GET	523

7.10.14.3.3.3.2	PUT	524

7.10.14.3.3.3.3	PATCH	525

7.10.14.3.3.3.4	DELETE	526

7.10.14.4	Custom Operations without associated resources	527

7.10.14.5	Notifications	528

7.10.14.5.2	Server to Server Performance analytics Notification	528

7.10.14.5.2.1	Description	528

7.10.14.5.2.2	Target URI	528

7.10.14.5.2.3	Standard Methods	528

7.10.14.6	Data Model	529

7.10.14.6.1	General	529

7.10.14.6.2	Structured data types	530

7.10.14.6.2.1	Introduction	530

7.10.14.6.2.2	Type: S2SPerfSubsc	531

7.10.14.6.2.3	Type: S2SPerfNotif	531

7.10.14.6.2.4	Type: S2SPerfSubscPatch	532

7.10.14.6.2.5	S2SPerfAnalyticsData	532

7.10.14.6.2.6	Type: ServerInfo	533

7.10.14.6.3	Simple data types and enumerations	533

7.10.14.6.3.1	Introduction	533

7.10.14.6.3.2	Simple data types	533

7.10.14.6.3.3	Enumeration: AnalyticsIdSspa	533

7.10.14.6.4	Data types describing alternative data types or combinations of data types	533

7.10.14.6.5	Binary data	533

7.10.14.6.5.1	Binary Data Types	533

7.10.14.7	Error Handling	534

7.10.14.7.1	General	534

7.10.14.7.2	Protocol Errors	534

7.10.14.7.3	Application Errors	534

7.10.14.8	Feature Negotiation	534

7.10.14.9	Security	534

7.11	AIML Enablement APIs	534

7.12	Metaverse Enablement APIs	534

8	Using Common API Framework	534

8.1	General	534

8.2	Security	535

9	Security	535

9.1	General	535

9.2	SEAL-S security	535

Annex A (normative): OpenAPI specification	537

A.1	General	537

A.2	SS_LocationReporting API	537

A.3	SS_GroupManagement API	544

A.4	SS_UserProfileRetrieval API	550

A.5	SS_NetworkResourceAdaptation API	552

A.6	SS_Events API	582

A.7	SS_KeyInfoRetrieval API	595

A.8	SS_LocationAreaInfoRetrieval API	597

A.9	Void	598

A.10	SS_NetworkResourceMonitoring API	598

A.11	SS_VALServiceData API	607

A.12	SS_VALServiceAreaConfiguration API	609

A.13	SS_IdmParameterProvisioning API	618

A.14	SS_KMParametersProvisioning API	623

A.15	SS_ADAE_VALPerformanceAnalytics API	624

A.16	SS_ADAE_SlicePerformanceAnalytics API	631

A.17	SS_ADAE_Ue2UePerformanceAnalytics API	635

A.18	SS_ADAE_LocationAccuracyAnalytics API	640

A.19	SS_ADAE_ServiceApiAnalytics API	645

A.20	SS_ADAE_SliceUsagePatternAnalytics API	648

A.21	SS_ADAE_EdgeLoadAnalytics API	652

A.22	SS_AADRF_DataManagement API	656

A.23	SS_ADAE_LocationRelatedUeGroupAnalytics API	665

A.24	SS_ADAE_CollisionDetectionAnalytics API	672

A.25	SS_LocationHistoryInfoEvent API	679

A.26	SS_ConfirmLocation API	685

A.27	SS_SLPositioningManagement API	690

A.28	SS_ADAE_AIMLMemberCapabilityAnalytics API	698

A.29	SS_ADCCF_DataCollection API	705

A.30	SS_ADAE_ServerToServerPerformanceAnalytics API	709

A.31	SS_ADAE_UeRatConnectivityAnalytics API	716

Annex B (normative): SEAL NRM server support integration with TSN	723

Annex C (informative): Change history	724

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present specification describes the APIs for the Service Enabler Architecture Layer for Verticals (SEAL). The related stage 2 architecture, functional requirements and information flows are specified in 3GPP TS 23.434 [2] and 3GPP TS 23.433 [34] (for SEAL Data Delivery).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[4]	IETF RFC 6455: "The Websocket Protocol".

[5]	IETF RFC 9112: "HTTP/1.1".

[6]	IETF RFC 9110: "HTTP Semantics".

[7]	Void.

[8]	Void.

[9]	IETF RFC 9111: "HTTP Caching".

[10]	Void.

[11]	Void.

[12]	IETF RFC 9113: "HTTP/2".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[15]	Open API: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[16]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[17]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[18]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[19]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[20]	3GPP TS 29.523: "5G System; Policy Control Event Exposure Service; Stage 3".

[21]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[22]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[23]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 reference point; Stage 3".

[24]	3GPP TR 21.900: "Technical Specification Group working methods".

[25]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[26]	3GPP TS 33.434: "Service Enabler Architecture Layer for Verticals (SEAL); Security Aspects".

[27]	3GPP TS 29.486: "V2X Application Enabler (VAE) Services; Stage 3".

[28]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[29]	IEEE 802.1Qcc-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[30]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[31]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[32]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[33]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[34]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[35]	3GPP TS 29.548: "Service Enabler Architecture Layer for Verticals (SEAL); SEAL Data Delivery (SEALDD) Server Services; Stage 3".

[36]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[37]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[38]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[39]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[40]	IETF RFC 9562: "Universally Unique IDentifier (UUIDs)".

[41]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[42]	3GPP TS 29.435: "Service Enabler Architecture Layer for Verticals (SEAL); Network Slice Capability Exposure (NSCE) Server Services; Stage 3".

[43]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

[44]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[45]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial map and Spatial anchors".

[46]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital asset".

[47]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[48]	3GPP TS 29.482: "Service Enabler Architecture Layer for Verticals (SEAL); Artificial Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3".

[49]	3GPP TS 29.437: "Service Enabler Architecture Layer for Verticals (SEAL); Metverse Enablement Services; Stage 3".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

VAL service: A generic name for any service offered by the VAL service provider to their VAL users.

SEAL service: A generic name for a common service (e.g. group management, configuration management, location management) that can be utilized by multiple vertical applications.

SEAL provider: Provider of SEAL service(s).

VAL server: A generic name for the server application function of a specific VAL service.

SEAL server: An entity that provides the server side functionalities corresponding to the specific SEAL service.

VAL system: The collection of applications, services, and enabling capabilities required to support a VAL service.

VAL group: A defined set of VAL UEs or VAL users configured for specific purpose in a VAL service.

NOTE:	The set could be of either VAL UEs or VAL users depending on the specific VAL service.

VAL group home system: The VAL system where the VAL group is defined.

VAL group member: A VAL service user, whose VAL user ID is listed in a particular VAL group.

Vertical application: An application catering to a specific vertical.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

ADAE	Application Data Analytics Enablement

A-ADRF	Application layer - Analytical Data Repository Function

A-DCCF	Application layer - Data Collection and Coordination Function

ADAES	ADAE Server

AEF	API Exposing Function

API	Application Programming Interface

BDT	Background Data Transfer

DA	Digital Asset

DS-TT	Device-Side TSN Translator

JSON	JavaScript Object Notation

NDS	Network Domain Security

NDS/IP	NDS for IP based protocols

NRM	Network Resource Management

NSCE	Network Slice Capability Enablement

PLMN	Public Land Mobile Network

REST	Representational State Transfer

SAn	Spatial Anchors

SCEF	Service Capability Exposure Function

SCS	Service Capability Server

SR	Short Range

SEAL	Service Enabler Architecture Layer for Verticals

SEALDD	SEAL Data Delivery

SM	Spatial Map

TMGI	Temporary Mobile Group Identity

TSC	Time Sensitive Communication

TSN	Time Sensitive Networking

UE	User Equipment

VAL	Vertical Application Layer

UASS	UAS Application Specific Server

# 4	Overview

3GPP has considered in 3GPP TS 23.434 [2], 3GPP TS 23.433 [34] (for SEALDD), 3GPP TS 23.435 [41] (for NSCE), 3GPP TS 23.436 [38] (for ADAE), 3GPP TS 23.437 [45] (for SM and SAn), 3GPP TS 23.438 [46] (for DA management) and 3GPP TS 23.482 [47] (for AIMLE) the development of Service enabler architecture layer for verticals (SEAL) over 3GPP networks to support vertical applications (e.g. V2X applications). It specifies the functional architecture for SEAL and the procedures, information flows and APIs for each service within SEAL in order to support vertical applications over the 3GPP systems. To ensure efficient use and deployment of vertical applications over 3GPP systems, SEAL services include, group management, configuration management, location management, identity management, key management, network resource management, network slice capability enablement (NSCE), application data analytics enablement (ADAE), data delivery management (SEALDD), spatial anchor management, spatial map management, digital asset management and AIML enablement (AIMLE).

Clause 6 of 3GPP TS 23.434 [2] specifies the functional entities and domains of the functional model, reference points descriptions and SEAL APIs for SEAL services.

The present document specifies the APIs needed to support SEAL. 3GPP TS 29.548 [35] specifies the APIs needed for SEALDD. 3GPP TS 29.435 [42] specifies the APIs needed for NSCE. 3GPP TS 29.482 [48] specifies the APIs needed for AIML Enablement. 3GPP TS 29.437 [49] specifies the APIs needed for Spatial Anchor and Spatial Map services.

# 5	Services offered by the SEAL servers

## 5.1	Introduction of SEAL services

The table 5.1-1 lists the SEAL server APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 5.1-1: List of SEAL Service APIs

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting | Request/ Response | VAL server |
|  | Fetch_Location_Report_Trigger | Request/Response | VAL server |
|  | Update_Trigger_Location_Reporting | Request/ Response | VAL server |
|  | Cancel_Trigger_Location_Reporting | Request/ Response | VAL server |
|  | Notify_Trigger_Location_Reporting | Notify | VAL server |
|  | Notify_Adaptive_Configuration | Notify | VAL server |
| SS_LocationInfoEvent | Subscribe_Location_Info | Subscribe/Notify | VAL server |
|  | Update_Location_Info_Subscription |  | VAL server |
|  | Unsubscribe_Location_Info |  | VAL server |
|  | Notify_Location_Info |  | VAL server |
| SS_LocationInfoRetrieval | Obtain_Location_Info | Request/ Response | VAL server |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | Request/ Response | VAL server |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | Subscribe/Notify | VAL server |
|  | Update_Location_Monitoring_Subscription |  |  |
|  | Unsubscribe_Location_Monitoring |  |  |
|  | Notify_Location_Monitoring_Events |  |  |
| SS_LocationAreaMonitoring | Subscribe_Location_Area_Monitoring | Subscribe/Notify | VAL server |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | Request/Response | VAL server |
|  | Obtain_VAL_Service_Area | Request/Response | VAL server |
|  | Update_VAL_Service_Area | Request/Response | VAL server |
|  | Delete_VAL_Service_Area | Request/Response | VAL server |
|  | Subscribe_VAL_Service_Area_Change_Event | Subscribe/Notify | SEAL server |
|  | Update_Subscription_VAL_Service_Area_Change_Event |  |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event |  |  |
|  | Notify_VAL_Service_Area_Change_Event |  |  |
| SS_LocationHistoryInfoEvent | Create | Request/Response | e.g., VAL Server |
|  | Update |  | e.g., VAL Server |
|  | Delete |  | e.g., VAL Server |
|  | Query |  | e.g., VAL Server |
| SS_ConfirmLocation | Subscribe | Subscribe/Notify | e.g., VAL Server |
|  | Notify |  | LM Server |
| SS_SLPositioningManagement | Subscribe | Subscribe/Notify | VAL server |
|  | Notify |  | VAL Server |
|  | SRInfoRequest | Request/Response | VAL Server |
| SS_GroupManagement | Query_Group_Info | Request/ Response | VAL server |
|  | Update_Group_Info | Request/ Response | VAL server |
|  | Create_Group | Request/ Response | VAL server |
|  | Delete_Group | Request/Response | VAL server |
| SS_GroupManagementEvent | Subscribe_Group_Info_Modification | Subscribe/Notify | VAL server |
|  | Notify_Group_Info_Modification |  | VAL server |
|  | Notify_Group_Creation |  | VAL server |
| SS_UserProfileRetrieval | Obtain_User_Profile | Request/ Response | VAL server |
| SS_VALServiceData | Obtain_VAL_Service_Data | Request/Response | SEAL server |
| SS_UserProfileEvent | Subscribe_User_Profile_Update | Subscribe/Notify | VAL server |
|  | Notify_User_Profile_Update |  | VAL server |
| SS_NetworkResourceAdaptation (NOTE 3) |  |  |  |
|  | Reserve_Network_Resource | Request/Response | e.g., VAL server, SEALDD Server |
|  | Reserve_Network_Resource_Modify | Request/Response | e.g., VAL server, SEALDD Server |
|  | Request_Multicast_Resource | Request/Response | e.g., VAL server |
|  | Notify_UP_Delivery_Mode | Subscribe/Notify | e.g., VAL server |
|  | Discover_TSC_Stream_Availability | Request/Response | e.g., VAL server |
|  | Create_TSC_Stream | Request/Response | e.g., VAL server |
|  | Delete_TSC_Stream | Request/Response | e.g., VAL server |
|  | Create_MBS_Resource | Request/Response | e.g., VAL server |
|  | Update_MBS_Resource | Request/Response | e.g., VAL server |
|  | Delete_MBS_Resource | Request/Response | e.g., VAL server |
|  | Activate_MBS_Resource | Request/Response | e.g., VAL server |
|  | Deactivate_MBS_Resource | Request/Response | e.g., VAL server |
|  | Reliable_Transmission_Request | Request/Response | e.g., SEALDD Server, VAL Server |
|  | BDT_Configuration_Request | Request/Response | e.g., VAL server, SEALDD Server |
|  | BDT_Negotiation_Notification | Subscribe/Notify | e.g., VAL server, SEALDD Server |
|  | BDT_Configuration_Get | Request/Response | e.g., VAL server, SEALDD Server |
|  | BDT_Configuration_Update | Request/Response | e.g., VAL server, SEALDD Server |
|  | BDT_Configuration_Delete | Request/Response | e.g., VAL server, SEALDD Server |
|  | Subscribe_Unified_Traffic_Pattern_and_Monitoring_Management | Subscribe/Notify | e.g., VAL server |
|  | Update_Unified_Traffic_Pattern_and_Monitoring_Management_Subscription |  |  |
|  | Unsubscribe_Unified_Traffic_Pattern_and_Monitoring_Management |  |  |
|  | Notify_Unified_Traffic_Pattern_Update |  |  |
| SS_EventsMonitoring | Subscribe_Monitoring_Events | Subscribe/Notify | VAL server |
|  | Notify_Monitoring_Events |  |  |
| SS_Events | Subscribe_Event | Subscribe/Notify | VAL server |
|  | Notify_Event |  | VAL server |
|  | Unsubscribe_Event |  | VAL server |
|  | Update_Subscription |  | VAL server |
| SS_KeyInfoRetrieval | Obtain_Key_Info | Request/Response | VAL server |
| SS_KMParametersProvisioning | Request | Request/Response | VAL server |
| SS_NetworkResourceMonitoring | Subscribe_Unicast_QoS_Monitoring_Data | Subscribe/Notify | VAL server |
|  | Unsubscribe_Unicast_QoS_Monitoring_Data |  | VAL server |
|  | Notify_Unicast_QoS_Monitoring_Data |  | VAL server |
|  | Obtain_Unicast_QoS_Monitoring_Data | Request/Response | VAL server |
|  | Update_Unicast_QoS_Monitoring_Subscription |  | VAL server |
| SS_IdmParameterProvisioning | Provide_Configuration | Request/Response | VAL server |
|  | Get_Configuration |  |  |
|  | Update_Configuration |  |  |
|  | Delete_Configuration |  |  |
| SS_ADAE_VALPerformanceAnalytics | Subscribe_VAL_Performance_Analytics | Subscribe/Notify | VAL server |
|  | Notify_VAL_Performance_Analytics |  |  |
|  | Unsubscribe_VAL_Performance_Analytics |  |  |
| SS_ADAE_SlicePerformanceAnalytics API | Subscribe_Slice_Performance_Analytics | Subscribe/Notify | VAL server |
|  | Notify_Slice_Performance_Analytics |  |  |
|  | Unsubscribe_Slice_Performance_Analytics |  |  |
| SS_ADAE_Ue2UePerformanceAnalytics | UE-to-UE_Performance_Analytics_Subscribe | Subscribe/Notify | VAL server |
|  | UE-to-UE_Performance_Analytics_Notify |  |  |
|  | UE-to-UE_Performance_Analytics_Unsubscribe |  |  |
| SS_ADAE_LocationAccuracyAnalytics | Subscribe_Location_Accuracy_Analytics | Subscribe/Notify | VAL server |
|  | Notify_Location_Accuracy_Analytics |  |  |
|  | Unsubscribe_Location_Accuracy_Analytics |  |  |
| SS_ADAE_ServiceApiAnalytics | Subscribe_Service_API_Analytics | Subscribe/Notify | VAL server |
|  | Notify_Service_API_Analytics |  |  |
|  | Unsubscribe_Service_API_Analytics |  |  |
| SS_ADAE_SliceUsagePatternAnalytics | Subscribe_Slice_Usage_Pattern_Analytics | Subscribe/Notify | VAL server, NSCE server |
|  | Notify_Slice_Usage_Pattern_Analytics |  |  |
|  | Unsubscribe_Slice_Usage_Pattern_Analytics |  |  |
|  | Get_Slice_Usage_Stats | Request/Response | VAL server, NSCE server |
| SS_ADAE_EdgeLoadAnalytics | Subscribe_Edge_Load | Subscribe/Notify | VAL server, EAS, EES |
|  | Notify_Edge_Load |  |  |
|  | Unsubscribe_Edge_Load |  |  |
|  | Get_Edge_Load_Data | Request/Response | VAL server, EAS, EES |
| SS_ADAE_LocationRelatedUeGroupAnalytics | Subscribe | Subscribe/Notify | e.g. LM Server |
|  | Notify |  |  |
| SS_ADAE_CollisionDetectionAnalytics | Subscribe | Subscribe/Notify | e.g., VAL Server, LM Server, UAE Server, UASS |
|  | Notify |  |  |
| SS_ADAE_UeRatConnectivityAnalytics | Subscribe | Subscribe/Notify | e.g., VAL Server |
|  | Subscription_Update |  |  |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| SS_ADAE_ServerToServerPerformanceAnalytics | Subscribe | Subscribe/Notify | e.g., VAL Server, EES |
|  | Notify |  |  |
| SS_AADRF_DataManagement | Data_Storage | Request/Response | ADAE server, A-DCCF |
|  | SS_AADRF_DataManagement_Subscribe | Subscribe/Notify | ADAE server |
|  | SS_AADRF_DataManagement_Unsubscribe |  |  |
|  | SS_AADRF_DataManagement_Notify |  |  |
| SS_ADCCF_DataCollection | Subscribe | Subscribe/Notify | e.g., ADAE server |
|  | Notify |  |  |
|  | Request | Request/Response | e.g., ADAE server |
| NOTE 1: The service operations of SS_Events API are reused by the SS_LocationInfoEvent, SS_LocationMonitoring, SS_LocationAreaMonitoring, SS_GroupManagementEvent, SS_UserProfileEvent, SS_EventsMonitoring and SS_SatelliteSFInfoEvent for events related services. NOTE 2: The service APIs exposed by the SEALDD Server and the corresponding service operations, operation semantics and service consumers are specified in clause 5 of 3GPP TS 29.548 [35]. NOTE 3: The "Create_MBS_Resource", "Update_MBS_Resource", "Delete_MBS_Resource", "Activate_MBS_Resource" and "Deactivate_MBS_Resource" service operations correspond to the stage 2 "Request_Multicast/Broadcast_Resource", "Update_Multicast/Broadcast_Resource", "Delete_Multicast/Broadcast_Resource", "Activate_Multicast_Resource" and "Deactivate_Multicast_Resource" service operations defined in clause 14.4.2 of 3GPP TS 23.434 [2]. NOTE 4: The service APIs exposed by the NSCE Server and the corresponding service operations, operation semantics and service consumers are specified in clause 5 of 3GPP TS 29.435 [42]. NOTE 5: The service APIs exposed by the SAn Server and SM Server, and the corresponding service operations, operation semantics and service consumers are specified in 3GPP TS 29.437 [49]. NOTE 6: The SS_SLPositioningManagement API is reused by the SS_SRPositioningInformation API defined in clause 9.4.11 of 3GPP TS 23.434 [2]. |  |  |  |

Table 5.1-2 summarizes the corresponding APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| SS_LocationReporting | 7.1.1 | Report Location Information Service. | TS29549_SS_LocationReporting.yaml | ss-lr | A.2 |
| SS_GroupManagement | 7.2.1 | Group Management Service | TS29549_SS_GroupManagement.yaml | ss-gm | A.3 |
| SS_UserProfileRetrieval | 7.3.1 | User Profile Retrieval Service | TS29549_SS_UserProfileRetrieval.yaml | ss-upr | A.4 |
| SS_NetworkResourceAdaptation | 7.4.1 | Network Resource Adaptation Service | TS29549_SS_NetworkResourceAdaptation.yaml | ss-nra | A.5 |
| SS_Events | 7.5.1 | Events Notify Service | TS29549_SS_Events.yaml | ss-events | A.6 |
| SS_KeyInfoRetrieval (NOTE 2) | 7.6.1 | Key Information Retrieval Service | TS29549_SS_KeyInfoRetrieval.yaml | ss-kir | A.7 |
| SS_LocationAreaInfoRetrieval | 7.1.2 | Location Area Info Retrieval Service | TS29549_SS_LocationAreaInfoRetrieval.yaml | ss-lair | A.8 |
| SS_NetworkResourceMonitoring | 7.4.2 | Network Resource Monitoring | TS29549_SS_NetworkResourceMonitoring.yaml | ss-nrm | A.10 |
| SS_VALServiceData | 7.3.2 | VAL Service Data Service | TS29549_SS_VALServiceData.yaml | ss-vsd | A.11 |
| SS_VALServiceAreaConfiguration | 7.1.3 | VAL Service Area Configuration Service | TS29549_SS_VALServiceAreaConfiguration.yaml | ss-vsac | A.12 |
| SS_KMParametersProvisioning (NOTE 3) | 7.6.2 | Key Management Parameters Provisioning Service | TS29549_SS_KMParametersProvisioning.yaml | ss-kpp | A.14 |
| SS_ADAE_VALPerformanceAnalytics | 7.10.1 | ADAE VAL performance analytics service | TS29549_SS_ADAE_VALPerformanceAnalytics.yaml | ss-adae-pa | A.15 |
| SS_ADAE_SlicePerformanceAnalytics | 7.10.2 | ADAE slice specific application performance analytics service | TS29549_SS_ADAE_SlicePerformanceAnalytics.yaml | ss-adae-sspa | A.16 |
| SS_ADAE_Ue2UePerformanceAnalytics | 7.10.3 | ADAE UE-to-UE PerformanceAnalytics Service | TS29549_SS_ADAE_Ue2UePerformanceAnalytics.yaml | ss-adae-uupa | A.17 |
| SS_ADAE_LocationAccuracyAnalytics | 7.10.4 | ADAE location accuracy performance analytics service | TS29549_SS_ADAE_LocationAccuracyAnalytics.yaml | ss-adae-laa | A.18 |
| SS_ADAE_ServiceApiAnalytics | 7.10.5 | ADAE service API analytics service | TS29549_SS_ADAE_ServiceApiAnalytics.yaml | ss-adae-sa | A.19 |
| SS_ADAE_SliceUsagePatternAnalytics | 7.10.6 | ADAE slice usage pattern analytics service | TS29549_SS_ADAE_SliceUsagePatternAnalytics.yaml | ss-adae-sup | A.20 |
| SS_ADAE_EdgeLoadAnalytics | 7.10.7 | Edge load analytics service | TS29549_SS_ADAE_EdgeLoadAnalytics.yaml | ss-adae-el | A.21 |
| SS_AADRF_DataManagement | 7.10.8 | A-ADRF Data Management Service | TS29549_SS_AADRF_DataManagement.yaml | ss-aadrf-datamanagement | A.22 |
| SS_ADAE_LocationRelatedUeGroupAnalytics | 7.10.9 | ADAE Location-Related UE Group Analytics service | TS29549_SS_ADAE_LocationRelatedUeGroupAnalytics.yaml | ss-adae-lruga | A.23 |
| SS_ADAE_CollisionDetectionAnalytics | 7.10.10 | ADAE Collision Detection Analytics service | TS29549_SS_ADAE_CollisionDetectionAnalytics.yaml | ss-adae-cda | A.24 |
| SS_LocationHistoryInfoEvent | 7.1.4 | Location Tracing Configuration Management | TS29549_SS_LocationHistoryInfoEvent.yaml | ss-lhie | A.25 |
| SS_ConfirmLocation | 7.1.5 | Location Confirmation Service Management | TS29549_SS_ConfirmLocation.yaml | ss-cl | A.26 |
| SS_SLPositioningManagement | 7.1.6 | SL Positioning Management service | TS29549_SS_SLPositioningManagement.yaml | ss-slpm | A.27 |
| SS_ADCCF_DataCollection | 7.10.13 | A-DCCF Data Collection service | TS29549_SS_ADCCF_DataCollection.yaml | ss-adccf-dc | A.29 |
| SS_ADAE_ServerToServerPerformanceAnalytics | 7.10.14 | ADAE Server-to-Server Performance Analytics service | TS29549_SS_ADAE_ServerToServerPerformanceAnalytics.yaml | ss-adae-sspa | A.30 |
| SS_ADAE_UeRatConnectivityAnalytics | 7.10.12 | ADAE UE RAT Connectivity Analytics service | TS29549_SS_ADAE_UeRatConnectivityAnalytics.yaml | ss-adae-uerat | A.31 |
| NOTE 1: The APIs exposed by the SEALDD Server are specified in clause 5 of 3GPP TS 29.548 [35]. NOTE 2: The stage 2 requirements for this API are defined in clause 5.3 of 3GPP TS 33.434 [26]. NOTE 3: The stage 2 requirements for this API are defined in clause 5.8 of 3GPP TS 33.434 [26]. NOTE 4: The APIs exposed by the NSCE Server are specified in clause 5 of 3GPP TS 29.435 [42]. NOTE 5: The APIs exposed by the SAn Server and SM Server are specified in 3GPP TS 29.437 [49]. |  |  |  |  |  |

## 5.2	Location management APIs

### 5.2.1	SS_LocationReporting API

#### 5.2.1.1	Service Description

##### 5.2.1.1.1	Overview

The SS_LocationReporting API, as defined 3GPP TS 23.434 [2], allows VAL server via LM-S reference point to configure reporting trigger of location information to the location management server.

#### 5.2.1.2	Service Operations

##### 5.2.1.2.1	Introduction

The service operation defined for SS_LocationReporting API is shown in the table 5.2.1.2.1-1.

Table 5.2.1.2.1-1: Operations of the SS_LocationReporting API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Create_Trigger_Location_Reporting | This service operation is used by VAL server to create the trigger to report location information. | VAL server |
| Fetch_Location_Report_Trigger | This service operation is used by VAL server to retrieve the location reporting trigger information. | VAL server |
| Update_Trigger_Location_Reporting | This service operation is used by VAL server to update the trigger to report location information. | VAL server |
| Cancel_Trigger_Location_Reporting | This service operation is used by VAL server to cancel the trigger to report location information. | VAL server |
| Notify_Trigger_Location_Reporting | This service operation is used by LM Server to send the notifications to the VAL Server. | LM Server |
| Notify_Adaptive_Configuration | This service operation is used by the LM Server to notify of the suggested adaptive location configuration and request feedback from the service consumer. | LM Server |

##### 5.2.1.2.2	Create_Trigger_Location_Reporting

###### 5.2.1.2.2.1	General

This service operation is used by a VAL server to create the trigger to report location information.

###### 5.2.1.2.2.2		VAL server providing trigger configuration using Create_Trigger_Location_Reporting service operation

To create the reporting trigger configuration, the VAL server shall send HTTP POST request message to location management server. The body of the HTTP POST message shall include the LocationReportConfiguration data type, as specified in the clause 7.1.1.2.2.3.1.

Upon receiving the HTTP POST message as described above, the location management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to provide the trigger;

2.	if the VAL server is authorized to provide the triggers, the location management server shall;

a.	create a new resource for Individual SEAL Location Reporting Configuration as specified in clause 7.1.1.2.1; and

b.	return within the response message body the created resource representation within the LocationReportConfiguration data structure, and within an HTTP Location header the created resource URI in the response message. and

3.	if errors occur when processing the request, the LM Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.1.1.5.

##### 5.2.1.2.3	Fetch_Location_Report_Trigger

###### 5.2.1.2.3.1	General

This service operation is used by VAL server to retrieve an individual location reporting configuration information.

###### 5.2.1.2.3.2	VAL server fetching trigger configuration using Fetch_Location_Report_Trigger service operation

To fetch the location report trigger configuration, the VAL server shall send HTTP GET request message to location management server on the resource URI representing the individual SEAL location reporting configuration, as specified in 7.1.1.2.3.3.1.

Upon receiving the HTTP GET message as described above, the location management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to fetch the trigger information;

2.	if the VAL server is authorized to fetch the trigger information, the location management server shall;

a.	return the location report trigger configuration in LocationReportConfiguration data type, as specified in clause 7.1.1.2.1.

##### 5.2.1.2.4	Update_Trigger_Location_Reporting

###### 5.2.1.2.4.1	General

This service operation is used by a VAL server to update the trigger to report location information.

###### 5.2.1.2.4.2		VAL server providing trigger configuration using Update_Trigger_Location_Reporting service operation

To modify the reporting trigger configuration, the VAL server shall send HTTP PUT message to the location management server to the Resource URI identifying the individual SEAL location reporting configuration resource representation, as specified in the clause 7.1.1.2.3.3.2. If the "PatchUpdate" feature defined in clause 7.1.1.6 is supported, the VAL server may send an HTTP PATCH request message to the Individual SEAL Location Reporting Configuration resource URI as specified in clause 7.1.1.2.3.3.4. The body of the HTTP PATCH request message shall include the requested modifications as specified in clause 7.1.1.2.3.3.4.

When HTTP PUT is used, the "valServerId" attribute within the LocationReportConfiguration data structure shall not be updated.

Upon receiving the HTTP PUT or PATCH request message, the location management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to modify the configuration information;

2.	if the VAL server is authorized to modify the information, then the location management server shall;

a.	if the configuration information in the request is valid, update/modify the resource identified by the Resource URI of the configuration received in the request;

b.	return a 200 OK status code with the updated location reporting configuration information in the response or a 204 No Content status code.

##### 5.2.1.2.5	Cancel_Trigger_Location_Reporting

###### 5.2.1.2.5.1	General

This service operation is used by a VAL server to cancel the trigger to report location information.

###### 5.2.1.2.5.2		VAL server providing trigger configuration using Cancel_Trigger_Location_Reporting service operation

To delete the reporting trigger configuration, the VAL server shall send an HTTP DELETE message to the resource representing Individual SEAL Location Reporting Configuration as specified in clause 7.1.1.2.3.3.3.

Upon receiving the HTTP DELETE message, the location management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to delete the configuration information; and

2.	if the VAL server is authorized to delete the configuration information, the location management server shall delete the resource pointed by the Resource URI for Individual SEAL Location Reporting Configuration.

##### 5.2.1.2.6	Notify_Trigger_Location_Reporting

###### 5.2.1.2.6.1	General

This service operation is used by a LM Server to notify of the location trigger event.

###### 5.2.1.2.6.2	LM Server notifies the VAL Server on the location trigger event using Notify_Trigger_Location_Reporting

In order to notify the VAL Server about location event, the LM Server shall send an HTTP POST request message to the VAL Server targeting the notification URI provided during location trigger configuration (see clauses 5.2.1.2.2.2 and 5.2.1.2.4.2).

Upon receiving the HTTP POST request message, the VAL Server shall:

1.	process the location trigger event notification;

2.	upon success, respond to the LM Server with a "204 No Content" status code; and

3.	if errors occur when processing the request, the VAL Server shall respond to the LM Server with an appropriate error response as specified in clause 7.1.1.5.

##### 5.2.1.2.7	Notify_Adaptive_Configuration

###### 5.2.1.2.7.1	General

This service operation is used by the LM Server to notify the suggested adaptive location configuration and request feedback.

###### 5.2.1.2.7.2	Adaptive Location Configuration Notification

###### 5.2.1.2.7.2.1	General

This service operation is used by the LM Server to notify of the suggested adaptive location configuration and request feedback from the service consumer.

The following procedures are supported by the "Notify_Adaptive_Configuration" service operation:

-	Adaptive Location Configuration Notification.

###### 5.2.1.2.7.2.2	Adaptive Location Configuration Notification

To notify of the suggested adaptive location configuration and request feedback from the service consumer, the LM Server shall send an HTTP POST request to the service consumer targeting the notification URI received during the corresponding SEAL Location Reporting Configuration creation/update defined in clauses 5.2.1.2.2 and 5.2.1.2.4, with the request body containing the AdaptConfigNotif data structure as defined in clause 7.1.1.3.3.3.

Upon reception of the HTTP POST request message, the service consumer shall process the suggested adaptive location configuration, and in case of success, respond with an HTTP "200 OK" status code with the response body containing the AdaptConfigNotifResp data structure.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the response body, as specified in clause 7.1.1.5.

### 5.2.2	SS_LocationInfoEvent API

The SS_LocationInfoEvent API, as defined 3GPP TS 23.434 [2], allows a VAL server via LM-S reference point to subscribe for and receive notifications of location information from the location management server. The SS_LocationInfoEvent API supports this via the event "LM_LOCATION_INFO_CHANGE" of the SS_Events API as specified in clause 7.5. If the event subscription includes an indication for supplementary location information, then the location management server obtains the UE location information from the 3GPP core network and/or a 3rd party location management server.

NOTE:	How the location management server obtains the UE location from a 3rd party location management server is out of scope of this specification.

### 5.2.3	SS_LocationInfoRetrieval API

The SS_LocationInfoRetrieval API, as defined 3GPP TS 23.434 [2], enables the VAL server via LM-S reference point to obtain location information from the location management server. The SS_LocationInfoRetrieval API supports this via the event "LM_LOCATION_INFO_CHANGE" of the SS_Events API by setting the "immRep" attribute to true and setting the "notifMethod" attribute to "ONE_TIME" within the "eventReq" attribute, as specified in clause 7.5.

Upon receipt of the immediate reporting indication in the "immRep" attribute within the "eventReq" attribute sets to true in the HTTP POST request, the location management server shall ignore the "notificationDestination" attribute within the SEALEventSubscription data type and include the event details in the "eventDetails" attribute, if available, in the HTTP POST response.

### 5.2.4	SS_LocationAreaInfoRetrieval API

#### 5.2.4.1	Service Description

##### 5.2.4.1.1	Overview

The SS_LocationAreaInfoRetrieval API, as defined 3GPP TS 23.434 [2], enables the VAL server via LM-S reference point to obtain UE(s) information in an application defined proximity range of a location.

#### 5.2.4.2	Service Operations

##### 5.2.4.2.1	Introduction

The service operation defined for SS_LocationAreaInfoRetrieval API is shown in the table 5.2.4.2.1-1.

Table 5.2.4.2.1-1: Operations of the SS_LocationAreaInfoRetrieval API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Obtain_UEs_Info | This service operation is used by VAL server to obtain UE(s) information in an application defined proximity range of a location. | VAL server |

##### 5.2.4.2.2	Obtain_UEs_Info

###### 5.2.4.2.2.1	General

This service operation is used by a VAL server to obtain UE(s) information in an application defined proximity range of a location.

###### 5.2.4.2.2.2		VAL server obtains UE(s) information in an application defined proximity range of a location using Obtain_UEs_Info service operation

To obtain the UE(s) information in an application defined proximity range of a location, the VAL server shall send HTTP GET message to the location management server, on location information collection resource representation URI as specified in the clause 7.1.2.2.2.3.1. The GET message shall include the query parameters: Location information or VAL service area identifier and proximity range.

Upon receiving the HTTP GET message as described above, the location management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to fetch the UE(s) information;

2.	if the VAL server is authorized to fetch the UE(s) information, the location management server shall;

a.	determine the VAL UE(s) information that are in the proximity range of the location as per the query parameters in the request message from the VAL server;

b.	return HTTP "200 OK" status code with the determined VAL UE(s) information in the LMInformation data type to the VAL server.

### 5.2.5	SS_LocationMonitoring API

The SS_LocationMonitoring API, as defined 3GPP TS 23.434 [2], allows a VAL server via the LM-S reference point to monitor the VAL UE(s) in relation to a given area of interest. The VAL server subscribes to the LM server to receive notifications of deviation of VAL UE(s) / User(s) location from a given location information. The SS_LocationMonitoring API supports this via the event "LM_LOCATION_DEVIATION_MONITOR" of the SS_Events API as specified in clause 7.5.

Upon the receipt of "LM_LOCATION_DEVIATION_MONITOR" event subscription request from the VAL server, in order to notify the location deviation events to the VAL server, the location management server shall:

1.	verify the VAL user(s) / VAL UE(s) identifier(s) provided in the "tgtUes" attribute within the MonitorLocationInterestFilter structure as specified in steps 2a and 2b of clause 5.6.1.2.2.2;

2.	periodically obtain the VAL UE location information using the SEAL location information procedures as per the SS_LocationReporting and SS_LocationAreaInfoRetrieval APIs as specified in clauses 5.2.1 and 5.2.4;

3.	using the MonitoringEvent API as specified in 3GPP TS 29.122 [3] and 3GPP TS 29.522 [28], periodically obtain the VAL UE location information from the 3GPP core network using the relevant location related monitoring event(s) and subscribe to the service for area of interest monitoring via the "AREA_OF_INTEREST" monitoring event, and may optionally obtain VAL UE location information from the 3rd party location management server;

NOTE:	How the location management server obtains the UE location from a 3rd party location management server is out of scope of this specification.

4.	process the location information received in steps 2 and 3 above and continue as follows:

a.	if the location information from the location management client, the 3GPP core network and from the 3rd party location management server (if available) do not match, or when the result of the presence status in the area of interest based on the location information collected from the location management client and the result of the presence status in the area of interest reported by the NEF do not match, then notify the VAL server with the "NOTIFY_MISMATCH_LOCATION" value in the event report;

b.	if the location information from the location management client, the 3GPP core network and from the 3rd party location management server (if available) match and is not within the area of interest of the VAL server, and such result is also aligned with the NEF reported area of interest monitoring result, then notify the VAL server with the "NOTIFY_ABSENCE" value in the event report;

c.	if the location information from the location management client, the 3GPP core network and from the 3rd party location management server (if available) match and is within the area of interest of the VAL server, and such result is also aligned with the NEF reported area of interest monitoring result, then notify to the VAL server with the "NOTIFY_PRESENCE" value in the event report, based on the notification interval parameter in VAL server's event subscription; or

d.	if the "LM_LocationDeviation_Ext1" feature is supported and the LM Server is not able to determine whether or not the current location of VAL UE is within the dynamic area of interest information provided within the "geofCond" attribute of the MonitorLocationInterestFilter data structure (as specified in steps 2a and 2b of clause 5.6.1.2.2.2), then notify the VAL Server with the "NOTIFY_UNKNOWN" value in the event report;

and

5.	if the SEAL location management server is unable to satisfy the request, the SEAL location management server shall respond to the VAL server with an appropriate error status code as defined in clause 7.5.1.5.

### 5.2.6	SS_LocationAreaMonitoring API

The SS_LocationAreaMonitoring API, as defined 3GPP TS 23.434 [2], allows a VAL server via the LM-S reference point to subscribe for and receive notifications of list of UE(s) moving in or out of a given area of interest from the location management server. The SS_LocationAreaMonitoring API supports this via the event "LM_LOCATION_AREA_MONITOR" of the SS_Events API as specified in clause 7.5. The VAL server may indicate the periodic time interval in which the LM server needs to notify the VAL UE's location information in the Reporting Requirements, during the Subscribe_Event service operation of SS_Events API.

### 5.2.7	SS_VALServiceAreaConfiguration API

#### 5.2.7.1	Service Description

##### 5.2.7.1.1	Overview

The SS_VALServiceAreaConfiguration API, as defined 3GPP TS 23.434 [2], enables a VAL Server to configure and manage VAL service area(s) via the LM-S reference point.

#### 5.2.7.2	Service Operations

##### 5.2.7.2.1	Introduction

The service operations defined for the SS_VALServiceAreaConfiguration API are shown in the table 5.2.7.2.1-1.

Table 5.2.7.2.1-1: Service operations of the SS_VALServiceAreaConfiguration API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Configure_VAL_Service_Area | This service operation is used by a VAL Server to configure VAL service area(s). | VAL Server |
| Obtain_VAL_Service_Area | This service operation is used by a VAL Server to obtain VAL service area(s) related information. | VAL Server |
| Update_VAL_Service_Area | This service operation is used by a VAL Server to request the update of VAL service area(s). | VAL Server |
| Delete_VAL_Service_Area | This service operation is used by a VAL Server to request the deletion of VAL service area(s). | VAL Server |
| Subscribe_VAL_Service_Area_Change_Event | This service operation is used by a SEAL Server to subscribe to the VAL service area(s) change event(s) reporting. | SEAL Server |
| Update_Subscription_VAL_Service_Area_Change_Event | This service operation is used by a SEAL Server to update the VAL service area(s) change event(s) subscription. | SEAL Server |
| Unsubscribe_VAL_Service_Area_Change_Event | This service operation is used by a SEAL Server to unsubscribe from the VAL service area(s) change event(s) reporting. | SEAL Server |
| Notify_VAL_Service_Area_Change_Event | This service operation is used by a LM Server to notify for the VAL service area(s) change event(s). | SEAL Server |

##### 5.2.7.2.2	Configure_VAL_Service_Area

###### 5.2.7.2.2.1	General

This service operation is used by a VAL Server to configure VAL service area(s) with VAL service area ID(s).

###### 5.2.7.2.2.2		VAL Server configures VAL service area(s) using the Configure_VAL_Service_Area service operation

In order to configure VAL service area(s), the VAL Server shall send an HTTP POST request message (i.e., custom operation "Configure") to the LM Server as specified in clause 7.1.3.2.2.4.2. The request body shall include the ValServiceAreaReq data structure defined in clause 7.1.3.4.2.3.

Upon reception of the HTTP POST request message, the LM Server shall:

1.	verify the identity of the VAL Server and whether the VAL Server is authorized to configure the VAL service area(s) at the LM Server;

2.	if the VAL Server is authorized, the LM Server shall configure the requested VAL service area(s) and respond to the VAL Server with an HTTP "200 OK" status code with the response body including the ValServiceAreaResp data structure defined in clause 7.1.3.4.2.5; and

3.	if the LM server is unable to satisfy the request, the LM Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.1.3.5.

##### 5.2.7.2.3	Obtain_VAL_Service_Area

###### 5.2.7.2.3.1	General

This service operation is used by a VAL Server to obtain the VAL service area(s) based on the corresponding VAL service area ID(s).

###### 5.2.7.2.3.2		VAL Server obtains VAL service area(s) using the Obtain_VAL_Service_Area service operation

In order to obtain VAL service area(s) and the corresponding information, the VAL Server shall send an HTTP GET request message to the LM Server targeting the URI of the "VAL Service Areas" resource as specified in clause 7.1.3.2.2.3.1. The request URI may include the requested VAL service identifier(s) as defined in clause 7.1.3.2.2.3.1.

Upon reception of the HTTP GET request message, the LM Server shall:

1.	verify the identity of the VAL Server and whether the VAL Server is authorized to obtain the requested VAL service area(s) information from the LM Server;

2.	if the VAL Server is authorized, the LM server shall respond to the VAL Server with an HTTP "200 OK" status code with the response body including the ValServiceAreaData data structure defined in clause 7.1.3.2.2.3.1; and

3.	if the LM Server is unable to satisfy the request, the LM Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.1.3.5.

##### 5.2.7.2.4	Update_VAL_Service_Area

###### 5.2.7.2.4.1	General

This service operation is used by a VAL Server to update a VAL service area(s) with the corresponding VAL service area ID(s).

###### 5.2.7.2.4.2		VAL Server updates VAL service area(s) using the Update_VAL_Service_Area service operation

In order to update existing VAL service area(s) related information, the VAL Server shall send an HTTP POST request message (i.e., custom operation "Update") to the LM Server as specified in clause 7.1.3.2.2.4.3. The request body shall include the ValServiceAreaReq data structure specified in clause 7.1.3.2.2.4.3.

Upon reception of the HTTP POST request message, the LM Server shall:

1.	verify the identity of the VAL Server and whether the VAL Server is authorized to update the requested VAL service area(s) related information at the LM Server;

2.	if the VAL Server is authorized, the LM server shall check whether the VAL service area(s) exist and then update the VAL service area(s) information of each VAL service area ID(s), and respond to the VAL Server with an HTTP "200 OK" status code with the response body including the ValServiceAreaResp data structure defined in clause 7.1.3.4.2.5; and

3.	if the LM Server is unable to satisfy the request, the LM Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.1.3.5.

##### 5.2.7.2.5	Delete_VAL_Service_Area

###### 5.2.7.2.5.1	General

This service operation is used by a VAL Server to delete VAL service area(s) with the given VAL service area ID(s).

###### 5.2.7.2.5.2		VAL Server deletes service area(s) using the Delete_VAL_Service_Area service operation

In order to delete existing VAL service area(s) and the related information, the VAL Server shall send an HTTP POST request message (i.e., custom operation "Delete") to the LM Server as specified in clause 7.1.3.2.2.4.4.

Upon reception of the HTTP POST request message, the LM Server shall:

1.	verify the identity of the VAL Server and whether the VAL Server is authorized to delete the requested VAL service area(s) related information at the LM Server;

2.	if the VAL Server is authorized, the LM server shall check whether the target VAL service area(s) exist(s), then delete the corresponding VAL service area(s) information, and respond to the VAL Server with an HTTP "200 OK" status code with the response body including the ValServiceAreaResp data structure defined in clause 7.1.3.4.2.5; and

3.	if the LM Server is unable to satisfy the request, the LM Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.1.3.5.

##### 5.2.7.2.6	Subscribe_VAL_Service_Area_Change_Event

###### 5.2.7.2.6.1	General

This service operation is used by a SEAL Server to subscribe to the VAL service area(s) change event(s) reporting.

###### 5.2.7.2.6.2		SEAL Server subscribes for the VAL service area(s) change event(s) reporting using the Subscribe_VAL_Service_Area_Change_Event service operation

In order to subscribe to the VAL service area(s) change event(s) reporting, the SEAL Server shall send an HTTP POST request message targeting the URI of the "VAL Service Area Change Subscriptions" resource as specified in clause 7.1.3.2.3.3.1. The request body shall include the ValServiceAreaSubsc data structure defined in clause 7.1.3.4.2.6.

Upon reception of the HTTP POST request message, the LM Server shall:

1.	verify the identity of the SEAL Server and whether the SEAL Server is authorized to subscribe to the VAL service area(s) change event(s) reporting at the LM Server;

2.	if the SEAL Server is authorized, the LM Server shall create a new "Individual VAL Service Area Change Subscription" resource and respond to the VAL server with an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Individual VAL Service Area Change Subscription" resource and the response body including the ValServiceAreaSubsc data structure containing a representation of the created resource as defined in clause 7.1.3.4.2.6; and

3.	if errors occur when processing the request, the LM Server shall respond to the SEAL Server with an appropriate error response as specified in clause 7.1.3.5.

##### 5.2.7.2.7	Update_Subscription_VAL_Service_Area_Change_Event

###### 5.2.7.2.7.1	General

This service operation is used by a SEAL Server to update the subscription for the VAL service area(s) change event(s) reporting.

###### 5.2.7.2.7.2		SEAL Server updates the subscription for the VAL service area(s) change event(s) reporting using the Update_Subscription_VAL_Service_Area_Change_Event service operation

In order to update an existing individual VAL service area change event subscription reporting, the SEAL Server shall send an HTTP PUT request message including the ValServiceAreaSubsc data structure to the LM Server, targeting the corresponding "Individual VAL Service Area Change Subscription" resource URI as specified in clause 7.1.3.2.4.3.2.

In order to modify an existing individual VAL service area change event subscription reporting, the SEAL Server shall send an HTTP PATCH request message including the ValServiceAreaSubscPatch data structure targeting the "Individual VAL Service Area Change Subscription" resource URI as specified in clause 7.1.3.2.4.3.3.

Upon receiving the HTTP PUT or HTTP PATCH request message, the LM Server shall:

1.	verify the identity of the SEAL Server and check if the SEAL Server is authorised to update or modify the "Individual VAL Service Area Change Subscription" resource;

2.	if the SEAL Server is authorized, then the LM Server shall:

a.	update/modify the resource identified by the Resource URI of the configuration received in the request; and

b.	respond to the SEAL server with an HTTP "200 OK" with the response body containing the updated "Individual VAL Service Area Change Subscription" resource within the ValServiceAreaSubsc data structure or "204 No Content" status code;

and

3.	if errors occur when processing the request, the LM Server shall respond to the SEAL Server with an appropriate error response as specified in clause 7.1.3.5.

##### 5.2.7.2.8	Unsubscribe_VAL_Service_Area_Change_Event

###### 5.2.7.2.8.1	General

This service operation is used by a SEAL Server to unsubscribe from the VAL service area(s) change event(s) reporting.

###### 5.2.7.2.8.2	SEAL server unsubscribes from the VAL service area(s) change event(s) using Unsubscribe_VAL_Service_Area_Change_Event

In order to terminate an individual VAL service area change event subscription reporting, the SEAL server shall send an HTTP DELETE request message to the LM Server, on the corresponding "Individual VAL Service Area Change Subscription" resource URI as specified in clause 7.1.3.2.4.3.4.

Upon reception of the HTTP DELETE request message, the LM server shall:

1.	verify the identity of the SEAL Server and check if the SEAL Server is authorised to terminate the targeted "Individual VAL Service Area Change Subscription" associated with the resource URI;

2.	if the SEAL Server is authorized to unsubscribe from VAL service area(s) change event(s), the LM Server shall delete the related "Individual VAL Service Area Change Subscription" subscription resource at the LM Server;

3.	upon success, respond to the SEAL server with a "204 No Content" status code; and

4. if errors occur when processing the request, the LM Server shall respond to the SEAL Server with an appropriate error response as specified in clause 7.1.3.5.

##### 5.2.7.2.9	Notify_VAL_Service_Area_Change_Event

###### 5.2.7.2.9.1	General

This service operation is used by a LM Server to notify for the VAL service area(s) change event(s).

###### 5.2.7.2.9.2	LM server notifies the SEAL Server on VAL service area(s) change event(s) using Notify_VAL_Service_Area_Change_Event

In order to notify the SEAL server about VAL service area(s) change event(s), the LM Server shall send an HTTP POST request message to the SEAL Server targeting the notification URI provided during subscription creation as specified in clause 5.2.7.2.6.

Upon receiving the HTTP POST request message, the SEAL Server shall:

1.	process the VAL service area change event(s) notification; and

2.	upon success, respond to the LM Server with a "204 No Content" status code; and

3.	if errors occur when processing the request, the SEAL Server shall respond to the LM Server with an appropriate error response as specified in clause 7.1.3.5.

### 5.2.8	SS_LocationHistoryInfoEvent Service

#### 5.2.8.1	Service Description

The SS_LocationHistoryInfoEvent service exposed by the LM Server enables a service consumer to:

-	create/update/delete a Location Tracing Configuration; and

-	request Location History Report(s).

#### 5.2.8.2	Service Operations

##### 5.2.8.2.1	Introduction

The service operations defined for the SS_LocationHistoryInfoEvent service are shown in table 5.2.8.2.1-1.

Table 5.2.8.2.1-1: SS_LocationHistoryInfoEvent Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| SS_LocationHistoryInfoEvent_Create | This service operation enables a service consumer to create a Location Tracing Configuration. | e.g., VAL Server |
| SS_LocationHistoryInfoEvent_Update | This service operation enables a service consumer to update a Location Tracing Configuration. | e.g., VAL Server |
| SS_LocationHistoryInfoEvent_Delete | This service operation enables a service consumer to delete a Location Tracing Configuration. | e.g., VAL Server |
| SS_LocationHistoryInfoEvent_Query | This service operation enables a service consumer to request Location History Report(s). | e.g., VAL Server |

##### 5.2.8.2.2	SS_LocationHistoryInfoEvent_Create

###### 5.2.8.2.2.1	General

This service operation is used by a service consumer to request the creation of a Location Tracing Configuration at the LM Server.

The following procedures are supported by the "SS_LocationHistoryInfoEvent_Create" service operation:

-	Location Tracing Configuration Creation.

###### 5.2.8.2.2.2	Location Tracing Configuration Creation

This procedure is used by a service consumer to request the LM Server to create a Location Tracing Configuration (see also clause 9.3.21 of 3GPP°TS°23.434°[2]).

1.	In order to request the creation of a Location Tracing Configuration, the service consumer shall send an HTTP POST request to the LM Server targeting the "Location Tracing Configurations" collection resource, with the request body including the LocTracingConfig data structure defined in clause 7.1.4.6.2.2.

2a.	Upon success, the LM Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Location Tracing Configuration" resource within the LocTracingConfig data structure defined in clause 7.1.4.6.2.2.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.1.4.7.

##### 5.2.8.2.3	SS_LocationHistoryInfoEvent_Update

###### 5.2.8.2.3.1	General

This service operation is used by a service consumer to request the update of an existing Location Tracing Configuration at the LM Server.

The following procedures are supported by the "SS_LocationHistoryInfoEvent_Update" service operation:

-	Location Tracing Configuration Update.

###### 5.2.8.2.3.2	Location Tracing Configuration Update

This procedure is used by a service consumer to request the LM Server to update an existing Location Tracing Configuration (see also clause 9.3.21 of 3GPP°TS°23.434°[2]).

1.	In order to request the update/modification of an existing Location Tracing Configuration, the service consumer shall send an HTTP PUT/PATCH request to the LM Server targeting the corresponding "Individual Location Tracing Configuration" resource, with the request body including either:

-	the updated representation of the resource within the LocTracingConfig data structure defined in clause 7.1.4.6.2.2, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the LocTracingConfigPatch data structure defined in clause 7.1.4.6.2.3, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

If the LM Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI of the resource located in an alternative LM Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [3].

2a.	Upon success, the LM Server shall respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated/modified "Individual Location Tracing Configuration" resource within the LocTracingConfig data structure defined in clause 7.1.4.6.2.2; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 7.1.4.7.

##### 5.2.8.2.4	SS_LocationHistoryInfoEvent_Delete

###### 5.2.8.2.4.1	General

This service operation is used by a service consumer to request the deletion of an existing Location Tracing Configuration at the LM Server.

The following procedures are supported by the "SS_LocationHistoryInfoEvent_Delete" service operation:

-	Location Tracing Configuration Deletion.

###### 5.2.8.2.4.2	Location Tracing Configuration Deletion

This procedure is used by a service consumer to request the LM Server to delete an existing Location Tracing Configuration (see also clause 9.3.21 of 3GPP°TS°23.434°[2]).

1.	In order to request the deletion of an existing Location Tracing Configuration, the service consumer shall send an HTTP DELETE request to the LM Server targeting the corresponding "Individual Location Tracing Configuration" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation/update of the targeted resource) can initiate this request.

2a.	Upon success, the LM Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 7.1.4.7.

##### 5.2.8.2.5	SS_LocationHistoryInfoEvent_Query

###### 5.2.8.2.5.1	General

This service operation is used by a service consumer to retrieve Location History Report(s) from the LM Server.

The following procedures are supported by the "SS_LocationHistoryInfoEvent_Query" service operation:

-	Location History Report(s) Retrieval.

###### 5.2.8.2.5.2	Location History Report(s) Retrieval

This procedure is used by a service consumer to request to retrieve Location History Report(s) from the LM Server (see also clause 9.3.21 of 3GPP°TS°23.434°[2]).

1.	In order to retrieve Location History Report(s), the service consumer shall send an HTTP GET request to the LM Server targeting the URI of the "Location History Reports" collection resource, with the request URI containing query parameter(s) to filter the responses from the LM Server.

2a.	Upon success, the LM Server shall respond with an HTTP "200 OK" status code with the response body containing the requested Location History Report(s) within the LocHistoryReports data structure defined in clause 7.1.4.6.2.5.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 7.1.4.7.

### 5.2.9	SS_ConfirmLocation Service

#### 5.2.9.1	Service Description

The SS_ConfirmLocation service exposed by the LM Server enables a service consumer to:

-	create/update/delete a Location Confirmation Service Subscription; and

-	receive location confirmation service usage related event(s) reporting.

#### 5.2.9.2	Service Operations

##### 5.2.9.2.1	Introduction

The service operations defined for the SS_ConfirmLocation service are shown in table 5.2.9.2.1-1.

Table 5.2.9.2.1-1: SS_ConfirmLocation Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| SS_ConfirmLocation_Subscribe | This service operation enables a service consumer to create/update/delete a Location Confirmation Service Subscription. | e.g., VAL Server |
| SS_ConfirmLocation_Notify | This service operation enables the LM Server to notify on location confirmation service usage related event(s). | LM Server |

##### 5.2.9.2.2	SS_ConfirmLocation_Subscribe

###### 5.2.9.2.2.1	General

This service operation is used by a service consumer to request the creation of a Location Confirmation Service Subscription at the LM Server.

The following procedures are supported by the "SS_ConfirmLocation_Subscribe" service operation:

-	Location Confirmation Service Subscription Creation.

-	Location Confirmation Service Subscription Update.

-	Location Confirmation Service Subscription Deletion.

###### 5.2.9.2.2.2	Location Confirmation Service Subscription Creation

This procedure is used by a service consumer to request the LM Server to create a Location Confirmation Service Subscription (see also clause 9.3.23 of 3GPP°TS°23.434°[2]).

1.	In order to request the creation of a Location Confirmation Service Subscription, the service consumer shall send an HTTP POST request to the LM Server targeting the "Location Confirmation Service Subscriptions" collection resource, with the request body including the LocConfirmSubsc data structure.

2a.	Upon success, the LM Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Location Confirmation Service Subscription" resource within the LocConfirmSubsc data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.1.5.7.

###### 5.2.9.2.2.3	Location Confirmation Service Subscription Update

This procedure is used by a service consumer to request the LM Server to update an existing Location Confirmation Service Subscription (see also clause 9.3.23 of 3GPP°TS°23.434°[2]).

1.	In order to request the update/modification of an existing Location Confirmation Service Subscription, the service consumer shall send an HTTP PUT/PATCH request to the LM Server targeting the corresponding "Individual Location Confirmation Service Subscription" resource, with the request body including either:

-	the updated representation of the resource within the LocConfirmSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the LocConfirmSubscPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

If the LM Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI of the resource located in an alternative LM Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [3].

2a.	Upon success, the LM Server shall respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated/modified "Individual Location Confirmation Service Subscription" resource within the LocConfirmSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 7.1.5.7.

###### 5.2.9.2.2.4	Location Confirmation Service Subscription Deletion

This procedure is used by a service consumer to request the LM Server to delete an existing Location Confirmation Service Subscription (see also clause 9.3.23 of 3GPP°TS°23.434°[2]).

1.	In order to request the deletion of an existing Location Confirmation Service Subscription, the service consumer shall send an HTTP DELETE request to the LM Server targeting the corresponding "Individual Location Confirmation Service Subscription" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation/update of the targeted resource) can initiate this request.

2a.	Upon success, the LM Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 7.1.5.7.

##### 5.2.9.2.3	SS_ConfirmLocation_Notify

###### 5.2.9.2.3.1	General

This service operation is used by the LM Server to notify on location confirmation service usage related event(s).

The following procedures are supported by the "SS_ConfirmLocation_Notify" service operation:

-	Location Confirmation Service Usage Notification.

###### 5.2.9.2.3.2	Location Confirmation Service Usage Notification

This procedure is used by the LM Server to notify on location confirmation service usage related event(s) (see also clause 9.3.23 of 3GPP°TS°23.434°[2]).

1.	In order to notify a previously subscribed service consumer on location confirmation service usage related event(s), the LM Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation/update of the corresponding Location Confirmation Service Subscription using the procedures defined in clauses 5.2.9.2.2, and the request body including the LocConfirmUsageNotif data structure.

2a.	Upon success, the service consumer shall respond to the LM Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.1.5.7.

### 5.2.10	SS_SLPositioningManagement API

#### 5.2.10.1	Service Description

The SS_SLPositioningManagement API, as defined 3GPP TS 23.434 [2], enables a service consumer to:

-	create/update/delete a SL Positioning Management Subscription;

-	receive SL Positioning Management related event(s) notification; and

-	request SR based positioning information.

#### 5.2.10.2	Service Operations

##### 5.2.10.2.1	Introduction

The service operations defined for the SS_SLPositioningManagement API are shown in the table 5.2.10.2.1-1.

Table 5.2.10.2.1-1: Service operations of the SS_SLPositioningManagement API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| SS_SLPositioningManagement_Subscribe | This service operation is used by a service consumer to to create/update/delete a SL Positioning Management Subscription. | VAL Server |
| SS_SLPositioningManagement_Notify | This service operation is used by the LM Server to notify on the SL Positioning Management event(s). | SEAL Server |
| SS_SLPositioningManagement_SRInfoRequest | This service operation is used by a service consumer to request SR based positioning information. | VAL Server |

##### 5.2.10.2.2	SS_SLPositioningManagement_Subscribe

###### 5.2.10.2.2.1	General

This service operation is used by a service consumer to request the creation of a SL Positioning Management Subscription at the LM Server.

The following procedures are supported by the "SS_SLPositioningManagement_Subscribe" service operation:

-	SL Positioning Management Subscription Creation.

-	SL Positioning Management Subscription Update.

-	SL Positioning Management Subscription Deletion.

###### 5.2.10.2.2.2	SL Positioning Management Subscription Creation

This procedure is used by a service consumer to request the LM Server to create a SL Positioning Management Subscription.

1.	In order to request the creation of a SL Positioning Management Subscription, the service consumer shall send an HTTP POST request to the LM Server targeting the "SL Positioning Management Subscriptions" collection resource, with the request body including the SlPositionMgmtSubsc data structure.

2a.	Upon success, the LM Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual SL Positioning Management Subscription" resource within the SlPositionMgmtSubsc data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.1.6.7.

##### 5.2.10.2.3	SL Positioning Management Subscription Update

This procedure is used by a service consumer to request the LM Server to update an existing SL Positioning Management Subscription Subscription.

1.	In order to request the update/modify an existing SL Positioning Management Subscription, the service consumer shall send an HTTP PUT/PATCH request to the LM Server targeting the corresponding "Individual SL Positioning Management Subscription" resource, with the request body including either:

-	the updated representation of the resource within the SlPositionMgmtSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the SlPositionMgmtSubscPatch data structure, in case the HTTP PATCH method is used.

If the LM Server is not able to handle the request, it may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI of the resource located in an alternative LM Server, as defined in clause 5.2.10 of 3GPP TS 29.122 [3].

2a.	Upon success, the LM Server shall respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated/modified "Individual SL Positioning Management Subscription" resource within the SlPositionMgmtSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 7.1.6.7.

###### 5.2.10.2.2.4	SL Positioning Management Subscription Deletion

This procedure is used by a service consumer to request the LM Server to delete an existing SL Positioning Management Subscription.

1.	In order to request the deletion of an existing SL Positioning Management Subscription, the service consumer shall send an HTTP DELETE request to the LM Server targeting the corresponding "Individual SL Positioning Management Subscription" resource.

2a.	Upon success, the LM Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 7.1.6.7.

##### 5.2.10.2.3	SS_SLPositioningManagement_Notify

###### 5.2.10.2.3.1	General

This service operation is used by the LM Server to notify on SL Positioning Management related event(s).

The following procedures are supported by the "SS_SLPositioningManagement_Notify" service operation:

-	SL Positioning Management Event Notification.

###### 5.2.10.2.3.2	SL Positioning Management Event Notification

This procedure is used by the LM Server to notify on SL Positioning Management related event(s).

1.	In order to notify a previously subscribed service consumer on SL Positioning related event(s), the LM Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation/update of the corresponding SL Positioning Management Subscription using the procedures defined in clauses 5.2.10.2.2, and the request body including the SlPositionMgmtNotif data structure.

2a.	Upon success, the service consumer shall respond to the LM Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.1.6.7.

##### 5.2.10.2.4	SS_SLPositioningManagement_SRInfoRequest

###### 5.2.10.2.4.1	General

This service operation is used by a service consumer to request SR based positioning information to the LM Server.

###### 5.2.10.2.4.2	Short-Range based positioning information request

This procedure is used by a service consumer to request Short-Range based positioning information.

1.	In order to request the Short-Range based positioning information, the service consumer shall send an HTTP POST request to the LM Server with the request body including the SrPosInfoReq data structure specified in clause 7.1.6.4.

2a.	Upon success, the LM Server shall respond with an HTTP "200 OK" status code with the response body containing a representation of the Short-Range based positioning information within the SrPosInfoResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.1.6.4.

## 5.3	Group management APIs

### 5.3.1	SS_GroupManagement API

#### 5.3.1.1	Service Description

##### 5.3.1.1.1	Overview

The SS_GroupManagement API, as defined 3GPP TS 23.434 [2], allows VAL server via GM-S reference point to create, fetch, update and delete VAL group membership and configuration information.

#### 5.3.1.2	Service Operations

##### 5.3.1.2.1	Introduction

The service operation defined for SS_GroupManagement API is shown in the table 5.3.1.2.1-1.

Table 5.3.1.2.1-1: Operations of the SS_GroupManagement API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Query_Group_Info | This service operation is used by VAL server to query for VAL group documents, group membership list and configuration information. | VAL Server |
| Update_Group_Info | This service operation is used by VAL server to modify group membership and configuration information. | VAL server |
| Create_Group | This service operation is used by VAL server to configure new VAL group. | VAL server |
| Delete_Group | This service operation is used by the VAL server to delete the VAL group. | VAL server |

##### 5.3.1.2.2	Query_Group_Info

###### 5.3.1.2.2.1	General

This service operation is used by a VAL server to obtain VAL group documents, group membership and configuration information.

###### 5.3.1.2.2.2		VAL server fetching VAL group documents, group membership and configuration information using Query_Group_Info service operation

To obtain membership, configuration information of a VAL group, the VAL server shall send a HTTP GET message to the group management server, on VAL group document's resource representation URI as specified in clause 7.2.1.2.3.3.1. The GET message may include the following query parameters: membership list, group configuration. To obtain VAL groups information, the VAL server shall send a HTTP GET message to the group management server, on VAL group documents collection resource representation URI as specified in clause 7.2.1.2.2.3.2. The GET message may include the following query parameters: VAL Group ID, VAL Service ID.

Upon receiving the HTTP GET message as described above, the group management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to fetch the VAL group information;

2.	if the VAL server is authorized to obtain the group information, the group management server shall;

a.	if the request to VAL group document's resource representation URI includes query parameters, then, return in the response message with VAL group information which includes, group membership list information if the request includes membership list query, group configuration information if the request includes group configuration query and VAL group identifier;

b.	if the request to VAL group document's resource representation URI does not include query parameter, then, return the VAL group document resource in the response message;

c.	in the request to VAL group documents collection resource representation URI, return the VAL group documents matching the query parameters in the response message;

and

3.	if the group management server is unable to satisfy the request, the group management server shall respond to the VAL server with an appropriate error status code as specified in clause 7.2.1.5.

##### 5.3.1.2.3	Update_Group_Info

###### 5.3.1.2.3.1	General

This service operation is used by a VAL server to modify group membership and configuration information.

###### 5.3.1.2.3.2	VAL server modifying group membership and configuration using Update_Group_Info service operation

To modify group information of a VAL group, the VAL server shall send HTTP PUT message to the group management server to the Resource URI identifying the VAL group document resource representation, as specified in the clause 7.2.1.2.3.3.2. This request shall not replace valGroupId property in the existing resource. If the "PatchUpdate" feature defined in clause 7.2.1.6 is supported, then the VAL server may send an HTTP PATCH request message to the Individual VAL Group Document resource URI as specified in clause 7.2.1.2.3.3.4, to partially update the VAL group document. The body of the HTTP PATCH request message shall include the requested modifications as specified in clause 7.2.1.2.3.3.4. Upon receiving the HTTP PUT message, the group management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to modify VAL group information;

2.	for the HTTP PUT request message, verify that valGroupId in the request is same as valGroupId of the VAL group document resource;

3.	if the VAL server is authorized to modify/update the group information and the valGroupId matches, then the group management server shall;

a.	if the group configuration information in the request is valid, update/modify the resource identified by the Resource URI of the group document with group members list, group configuration information, description, VAL service identifiers, external group identifier and location information received in the request;

b.	if the group document information in the request includes 5G LAN-Type communication, invoke the 5GLANParameterProvision API towards the NEF via an HTTP PUT/PATCH message as defined in clause 4.4.15.3 of 3GPP TS 29.522 [28];

c.	return a 200 OK status code with the updated VAL group document in the response or a 204 No Content status code;

and

4.	if the group management server is unable to satisfy the request, the group management server shall respond to the VAL server with an appropriate error status code as specified in clause 7.2.1.5.

NOTE:	The group management server maintains a mapping between DNN and S-NSSAI of the 5GVN group and the VAL server requester identity based on operator policy. How such mapping is configured is implementation specific and out of the scope of this specification.

##### 5.3.1.2.4	Create_Group

###### 5.3.1.2.4.1	General

This service operation is used by a VAL server to create VAL group.

###### 5.3.1.2.4.2	VAL server creating new group using Create_Group service operation

To create a VAL group, the VAL server shall send a HTTP POST message to the group management server. The body of the POST message shall include VAL group document information as specified in clause 7.2.1.2.2.3.1. The VAL server shall use this service operation to create the location-based VAL group as specified for Create_LocationBasedGroup_Info service operation of SS_GroupManagement API, in 3GPP TS 23.434 [2].

Upon receiving HTTP POST message, the group management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to create VAL group document;

2.	if the VAL group document information in the request includes location criteria, then shall obtain the list of VAL users or VAL UEs within the requested location criteria information from the Location Management server and include them in VAL group members of the new VAL group;

a.	if the "valServiceIds" attribute within VALGroupDocument structure was present in the received VAL group document information and the received response from the location management server does not include the identical list of the VAL service IDs as defined in the "valServiceIds" attribute within VALGroupDocument structure, the group management server shall retrieve the VAL service data from the configuration management server for the missed VAL service IDs and include the relevant VAL users or VAL UEs in the VAL group members of the new VAL group;

3.	if the VAL group document information in the request includes 5G LAN-Type communication, invoke the 5GLANParameterProvision API towards the NEF via an HTTP POST message as defined in clause 4.4.15.2 of 3GPP TS 29.522 [28];

4.	if the VAL server is authorized to create VAL group document, shall create a new resource as defined in 7.2.1.2.2.3.1 and return the VAL group document and its Resource URI in the response message; and

5.	if the group management server is unable to satisfy the request, the group management server shall respond to the VAL server with an appropriate error status code as defined in clause 7.2.1.5.

NOTE:	The group management server maintains a mapping between DNN and S-NSSAI of the 5GVN group and the VAL server requester identity based on operator policy. How such mapping is configured is implementation specific and out of the scope of this specification.

##### 5.3.1.2.5	Delete_Group

###### 5.3.1.2.5.1	General

This service operation is used by a VAL server to delete a VAL group.

###### 5.3.1.2.5.2	VAL server deleting VAL group using Delete_Group service operation

To delete a VAL group, the VAL server shall send a HTTP DELETE message to the Group Management server to its resource representation in the Group Management server as specified in clause 7.2.1.2.3.3.3. Upon receiving HTTP DELETE message, the Group Management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to delete the VAL group document;

2.	if the VAL server is authorized to delete the VAL group document, the Group Management server shall:

a.	 if the group communication type is 5GLAN communication, invoke the 5GLANParameterProvision API towards the NEF via an HTTP DELETE message as defined in clause 4.4.15.4 of 3GPP TS 29.522 [28];

b.	delete the resource representation pointed by the group document resource identifier;

and

3.	if the group management server is unable to satisfy the request, the group management server shall respond to the VAL server with an appropriate error status code as specified in clause 7.2.1.5.

### 5.3.2	SS_GroupManagementEvent API

The SS_GroupManagementEvent API, as defined 3GPP TS 23.434 [2], allows a VAL server via GM-S reference point to subscribe for and receive notifications from Group Management server on new VAL group creations, on modifications to VAL Group membership and configuration information, on temporary VAL group formation and on VAL group deletion. The SS_GroupManagementEvent API supports this via the "GM_GROUP_CREATE", "GM_GROUP_INFO_CHANGE", "GM_TEMP_GROUP_FORMATION" and "GM_GROUP_DELETION" events of the SS_Events API as specified in clause 7.5. In order to authorize the VAL servers that have to be notified of a "GM_GROUP_CREATE" event, the Group Management server shall identify the VAL services (VAL Service IDs) allowed for the VAL server by the "subscriberId" attribute and shall notify the VAL server if the VAL services enabled for the created VAL group are allowed for the VAL server.

Upon the receipt of the VAL group document from the group management server during Create_Group service operation, if the VAL server is interested in receiving the notifications about newly registered or de-registered VAL UE IDs to the VAL group, then the VAL server may subscribe to "GM_GROUP_INFO_CHANGE" event using the SS_Events API as specified in clause 7.5.1, to receive any VAL group membership update notifications.

Upon the receipt of the message filters information in the "GM_GROUP_INFO_CHANGE" event notification from the group management server, the VAL server shall consider the message filters in VAL specific communication.

## 5.4	Configuration management APIs

### 5.4.1	SS_UserProfileRetrieval API

#### 5.4.1.1	Service Description

##### 5.4.1.1.1	Overview

The SS_UserProfileRetrieval API, as defined in 3GPP TS 23.434 [2], allows VAL server via CM-S reference point to obtain user profile from the configuration management server.

#### 5.4.1.2	Service Operations

##### 5.4.1.2.1	Introduction

The service operation defined for SS_UserProfileRetrieval API is shown in the table 5.4.1.2.1-1.

Table 5.4.1.2.1-1: Operations of the SS_UserProfileRetrieval API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Obtain_User_Profile | This service operation is used by VAL server to obtain user profile. | VAL server |

##### 5.4.1.2.2	Obtain_User_Profile

###### 5.4.1.2.2.1	General

This service operation is used by a VAL server to obtain VAL user profile information.

###### 5.4.1.2.2.2		VAL server retrieving VAL user profile information using Obtain_User_Profile service operation

To obtain a VAL user's profile, the VAL server shall send HTTP GET request message to configuration management server, on VAL service's resource representation URI, with query parameters VAL user ID or VAL UE ID and optionally VAL service ID, as specified in 7.3.1.2.2.3.1.

Upon receiving the HTTP GET message as described above, the configuration management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to fetch the VAL user profile information;

2.	if the VAL server is authorized to obtain the requested VAL user profile information, the configuration management server shall;

a.	return in the response message with profile information corresponding to the query parameters that was sent in the request message.

### 5.4.2	SS_UserProfileEvent API

The SS_UserProfileEvent API, as defined in 3GPP TS 23.434 [2], allows a VAL server via CM-S reference point to subscribe for and receive notifications from the Configuration Management server on profile updates to VAL User or VAL UE. The SS_UserProfileEvent API supports this via the "CM_USER_PROFILE_CHANGE" event in SS_Events API as specified in clause 7.5.

### 5.4.3	SS_VALServiceData API

#### 5.4.3.1	Service Description

##### 5.4.3.1.1	Overview

The SS_VALServiceData API, as defined in 3GPP TS 23.434 [2], allows SEAL Server (e.g., GM Server via SEAL-X3 reference point) to obtain the VAL service data from the CM Server.

#### 5.4.3.2	Service Operations

##### 5.4.3.2.1	Introduction

The service operation defined for SS_VALServiceData API is shown in the table 5.4.3.2.1-1.

Table 5.4.3.2.1-1: Operations of the SS_VALServiceData API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Obtain_VAL_Service_Data | This service operation is used by SEAL Server to obtain VAL service data. | SEAL Server (e.g., GM Server) |

##### 5.4.3.2.2	Obtain_VAL_Service_Data

###### 5.4.3.2.2.1	General

This service operation is used by a SEAL Server to retrieve VAL service data.

###### 5.4.3.2.2.2		SEAL Server retrieving VAL service data using the Obtain_VAL_Service_Data service operation

To obtain a VAL service data, the SEAL Server shall send an HTTP GET request message to CM Server targeting the URI of the "VAL Service Data Sets" collection resource, including optionally one or several of the query parameter(s) defined in clause 7.3.2.2.2.3.1.

Upon reception of the HTTP GET request message, the CM Server shall:

1.	verify the identity of the SEAL Server and check whether it is authorized to obtain the VAL service data;

2.	if the SEAL Server is authorized to obtain the requested VAL service data and upon successful processing of the request, the CM Server shall respond with an HTTP "200 OK" status code with the response body including the ValServDataResp data structure containing the requested VAL service data set(s) corresponding to the received query parameters. If the request message includes both the "val-tgt-ues" and "val-service-ids" query parameters, the response message shall include the VAL service data set(s) for only the VAL service(s) (the identifiers of which are provided within the "val-service-ids" query parameter) that are common to the VAL user(s)/VAL UE(s) provided within the "val-tgt-ues" query parameter; and

3.	if errors occur when processing the request, the CM Server shall take proper error handling actions, as specified in clause 7.3.2.6, and respond with an appropriate error status code.

## 5.5	Network resource management APIs

### 5.5.1	SS_NetworkResourceAdaptation API

#### 5.5.1.1	Service Description

##### 5.5.1.1.1	Overview

The SS_NetworkResourceAdaptation API, as defined 3GPP TS 23.434 [2], allows a service consumer to communicate with the network resource management server for network resource adaptation including reserving network resource, requesting and subscribing for unicast and multicast/broadcast resources.

#### 5.5.1.2	Service Operations

##### 5.5.1.2.1	Introduction

The service operation defined for SS_NetworkResourceAdaptation API is shown in the table 5.5.1.2.1-1.

Table 5.5.1.2.1-1: Operations of the SS_NetworkResourceAdaptation API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Reserve_Network_Resource | Requesting network resource adaptation. | e.g., VAL server, SEALDD Server |
| Reserve_Network_Resource_Modify | Updating network resource adaptation | e.g., VAL server, SEALDD Server |
| Request_Multicast_Resource | Requesting multicast resource | e.g., VAL server |
| Notify_UP_Delivery_Mode | Notifying the user plane delivery mode | NRM server |
| Discover_TSC_Stream_Availability | Requesting the NRM server to discover the connectivity and available QoS characteristics between the source and the destination DS-TT ports. | e.g., VAL server |
| Create_TSC_Stream | Requesting the NRM server to create a TSC stream. | e.g., VAL server |
| Delete_TSC_Stream | Requesting the NRM server to delete a TSC stream. | e.g., VAL server |
| Create_MBS_Resource | Request the creation of a new MBS Resource. | e.g., VAL server |
| Update_MBS_Resource | Request the update of an existing "Individual MBS Resource" resource. | e.g., VAL server |
| Delete_MBS_Resource | Request the deletion of an existing "Individual MBS Resource" resource. | e.g., VAL server |
| Activate_MBS_Resource | Request the activation of an existing MBS Resource. | e.g., VAL server |
| Deactivate_MBS_Resource | Request the deactivation of an existing MBS Resource. | e.g., VAL server |
| BDT_Configuration_Request | Request the creation of a BDT policy configuration. | e.g., VAL server, SEALDD Server |
| BDT_Negotiation_Notification | Notify about negotiated BDT policy configuration related event(s). | NRM server |
| BDT_Configuration_Get | Request to retreive an existing "Individual BDT Policy Configuration" resource. | e.g., VAL server, SEALDD Server |
| BDT_Configuration_Update | Request to update an existing "Individual BDT Policy Configuration" resource. | e.g., VAL server, SEALDD Server |
| BDT_Configuration_Delete | Request to delete an existing "Individual BDT Policy Configuration" resource. | e.g., VAL server, SEALDD Server |
| Subscribe_Unified_Traffic_Pattern_and_Monitoring_Management | Request UE unified traffc pattern and monitoring management subscription. | e.g., VAL server |
| Update_Unified_Traffic_Pattern_and_Monitoring_Management_Subscription | Request the update of an existing UE unified traffc pattern and monitoring management subscription. | e.g., VAL server |
| Unsubscribe_Unified_Traffic_Pattern_and_Monitoring_Management | Request the removal of an existing UE unified traffc pattern and monitoring management subscription. | e.g., VAL server |
| Notify_Unified_Traffic_Pattern_Update | Notify about the update of UE unified traffic patterns. | NRM server |
| Reliable_Transmission_Request | Request reliable transmission service. | e.g., SEALDD Server, VAL Server |

##### 5.5.1.2.2	Void

##### 5.5.1.2.3	Reserve_Network_Resource

###### 5.5.1.2.3.1	General

This service operation is used by a service consumer to request for network resource.

###### 5.5.1.2.3.2		Service consumer requesting for network resource adaptation using Reserve_Network_Resource service operation

The service consumer shall send a HTTP POST message to the NRM server. The body of the POST message shall include VAL user or UE information and the VAL service requirement. Upon receiving HTTP POST message, the NRM server shall

1.	verify the identity of the service consumer and check if the service consumer is authorized to request for unicast resource;

2.	if the service consumer is authorized, the NRM server evaluates the need for network resources and use of resource sharing;

3.	for the VAL user or UE, the NRM server initiates interaction via SIP core;

4.	the NRM server creates a unicast subscription as specified in clause 7.4.1.2.4.3.1; and

5.	the NRM server provides result in the response message.

##### 5.5.1.2.4	Reserve_Network_Resource_Modify

###### 5.5.1.2.4.1	General

This service operation is used by a service consumer to request for updating the network resource adaptation used by the VAL user or UE.

###### 5.5.1.2.4.2	Service consumer requesting for updating the network resource adaptation using Reserve_Network_Resource_Modify service operation

The service consumer shall send a HTTP PUT or PATCH message to the NRM server. The body of the message shall include VAL user or UE information and the VAL service requirement. Upon receiving HTTP PUT or PATCH message, the NRM server shall

1.	verify the identity of the service consumer and check if the service consumer is authorized to request for updating the unicast resource;

2.	if the service consumer is authorized, the NRM server decides the need to update the unicast resource. If NRM server decides that no update is required for the unicast resource, then the NRM server sends a failure indication in the response message;

3.	if NRM server decides to update the unicast resource, then the NRM server initiates interaction via SIP core; and

4.	the NRM server provides result in the response message.

##### 5.5.1.2.5	Request_Multicast_Resource

###### 5.5.1.2.5.1	General

This service operation is used by a service consumer to request for multicast resource.

###### 5.5.1.2.5.2	Service consumer requesting for multicast resource using Request_Multicast_Resource service operation

The service consumer shall send a HTTP POST message to the NRM server. The body of the POST message shall include VAL group information, service announcement mode, QoS information, Broadcast area, the local MBMS information or the local MBMS activation indication and service consumer notification endpoint address information. Upon receiving HTTP POST message, the NRM server shall

1.	verify the identity of the service consumer and check if the service consumer is authorized to request for multicast resource;

2.	if the service consumer is authorized, the NRM server decides to establish an MBMS bearer in EPS using the procedures defined in 3GPP TS 29.468 [23];

3.	the NRM server creates a multicast subscription as specified in clause 7.4.1.2.2.3.1;

4.	the NRM server provides the result in the response message.

##### 5.5.1.2.6	Notify_UP_Delivery_Mode

###### 5.5.1.2.6.1	General

This service operation is used by the NRM server to send user plane notifications to the service consumer.

###### 5.5.1.2.6.2	Notifying user plane events using Notify_UP_Delivery_Mode service operation

To notify the user plane events, the NRM server shall send an HTTP POST message using the Notification Destination URI received in the multicast resource request defined in clause 5.5.1.2.5, or the MBS Resource creation/update request defined in clauses 5.5.1.2.10 and 5.5.1.2.11. The body of the HTTP POST message shall include an UserPlaneNotification as specified in clause 7.4.1.3.2.

Upon receiving the HTTP POST message, the service consumer shall process the Event Notification.

##### 5.5.1.2.7	Create_TSC_Stream

###### 5.5.1.2.7.1	General

This service operation is used by a service consumer to request the NRM server to create TSC stream resources.

###### 5.5.1.2.7.2		Service consumer requesting for create TSC stream using Create_TSC_Stream service operation

In order to create a TSC stream resource, the service consumer shall send an HTTP PUT message to the NRM server with {valStreamId} in the request URI path to identify the TSC stream to be created. The request body with the "TscStreamData" data structure shall include stream specification and Traffic Specification Information which includes MaxFrameInterval, MaxFrameSize, MaxIntervalFrames, MaxLatency.

Upon reception of the HTTP PUT message, the NRM server shall:

1.	verify the requestor identity of the service consumer, check whether the service consumer is authorized to request the NRM server to create a TSC stream with the VAL Stream ID as the TSC stream resource identifier;

NOTE: It's up to the service consumer to secure the uniqueness of the VAL Stream ID.

2.	if the service consumer is authorized, the NRM server shall calculate the schedule for the VAL Stream ID based on the information collected earlier from the 5GS. It provides per-stream filtering and policy parameters (e.g as defined in IEEE 802.1Qcc [29]) used to derive the TSC QoS information and related flow information. The NRM server also provides the forwarding rule (e.g.as defined in IEEE 802.1Qcc [29]) used to identify the DS-TT MAC address of the corresponding PDU session. Based on the 5GS bridge delay information it determines the TSC QoS information and TSC Assistance information for the stream;

3.	determine whether time synchronization needs to be activated for the TSC stream on the DS-TTs. If the DS-TTs are time synchronized, then the NRM shall not activate the time synchronization for the corresponding DS-TTs;

4.	for each VAL UE, the trusted NRM server within the PLMN operator domain acting as a TSCTSF shall initiate the PCC procedures by triggering the Npcf_policy_Authorization_Create service operation as described in 3GPP TS 29.514 [30] for the TSC stream for both uplink QoS flow (sender UE to UPF/bridge) and downlink QoS flow (UPF/bridge to receiver UE). The creation request includes the DS-TT port MAC address, TSC QoS information, TSC Assistance Information, flow bit rate, priority, Service Data Flow Filter containing flow description including Ethernet Packet Filters. The QoS flow will be assigned for the PDU session with the source MAC address for the uplink direction and with the destination MAC address for the downlink direction. This information is delivered to the DS-TT by the 5GS;

5.	if the time synchronization for the TSC stream on the DS-TTs was determined as required in step 3, the NRM server shall use the procedures described in 3GPP TS 29.514 [30] to activate the time synchronization for the corresponding DS-TTs; and

6.	after the NRM server receiving a successful response from the PCF, the NRM server shall create an "Individual TSC Stream" resource which represents the created TSC stream, addressed by a URI that contains the {valStreamId} as the VAL Stream ID identifier the TSC Stream, and shall respond to the service consumer with a 201 Created status code, including a Location header field containing the URI for the created resource. If the NRM server receives an error response from the PCF, the NRM server shall not create the resource and shall respond to the service consumer with a proper error status code.

##### 5.5.1.2.8	Delete_TSC_Stream

###### 5.5.1.2.8.1	General

This service operation is used by a service consumer to delete a TSC stream.

###### 5.5.1.2.8.2	Service consumer requesting to delete a TSC stream using Delete_TSC_Stream service operation

In order to delete a TSC stream, the service consumer shall send an HTTP DELETE message to the NRM server, with "{apiRoot}/ss-nra/<apiVersion>/tsc-streams/{valStreamId}" as the Resource URI representing the TSC stream identified by the VAL sStream ID to be deleted.

Upon reception of the HTTP DELETE message, the NRM server shall:

1.	identify the MAC addresses of the DS-TTs involved in the stream based on the stored information for the VAL Stream ID;

2.	determine the actions related to the time synchronization deactivation for the DS-TTs:

-	if none of the streams require to keep the time synchronization activated, deactivate the time synchronization for the involved DS-TTs in step 3; and

-	otherwise, keep the time synchronization activated for the involved DS-TTs for which time synchronization was previously activated;

3.	for each VAL UE, as the trusted NRM server within the PLMN operator domain acting as a TSCTSF shall:

-	deactivate the time synchronization for the DS-TTs as per step 2 above by invoking the Npcf_PolicyAuthorization_Update service operation as defined in 3GPP TS 29.514 [30]; and

-	initiate the PCC procedures by triggering the Npcf_policy_Authorization_Delete service operation to delete the QoS flows as defined in 3GPP TS 29.514 [30] with all the MAC addresses referred by the VAL Stream ID; and

4.	if the NRM server receive a successful response from the PCF, the NRM server shall delete the existing TSC stream in the "Individual TSC Stream" resource. Then the NRM server shall send an HTTP DELETE response message with "204 No Content" status code to the service consumer as a successful result of TSC stream deletion for the VAL Stream ID. If the NRM server receive an error code from the PCF, the NRM server shall take proper error handling action and shall respond to the service consumer with a proper error status code as unsuccessful result.

##### 5.5.1.2.9	Discover_TSC_Stream_Availability

###### 5.5.1.2.9.1	General

This service operation is used by a service consumer to request the NRM server to retrieve the connectivity information between the source and destination DS-TT ports and the related available QoS characteristics, prior to creating the stream.

###### 5.5.1.2.9.2	Service consumer discovering TSC stream availability using Discover_TSC_Stream_Availability service operation

In order to discover the connectivity between the source and destination DS-TT ports and the related available QoS characteristics, the service consumer shall send an HTTP GET request message to the NRM server, with the query parameters containing the targeted stream specifications.

Upon reception of the HTTP GET request message, the NRM server shall:

1.	verify if the service consumer is authorized to discover the TSC stream availability;

2.	if the service consumer is authorized, the NRM server shall check the connectivity between the DS-TTs ports indicated in the requested TSC stream definition based on the collected 5GS TSC bridge management and port management information, the traffic classes supported by the DS-TTs and the end-to-end latency (including the UE-DS-TT residence times, UPF residence time, and propagation delays) per traffic class;

The necessary information to process the TSC stream availability discovery request, i.e. 5GS TSC bridge management and port management information, the traffic classes supported by the DS-TTs and the end-to-end latency (including the UE-DS-TT residence times, UPF residence time, and propagation delays) per traffic class, is collected by the NRM server and shall be available at the NRM server prior to the reception of the request from the service consumer.

3.	if the discovery result is successful upon the connectivity between the DS-TT ports is discovered, the NRM server shall return an HTTP GET response message to the service consumer with an HTTP "200 OK" status code with the TscStreamAvailability data structure as the response body which shall include the stream specification matching the received query parameters and the corresponding list of traffic specifications;

4.	if there is no stream specification matching the query parameters or no TSC stream availability information was discovered between the DS-TT ports, then the NRM server shall responsde with a 204 No Content status code without response body;

5.	Otherwise if error case occur (e.g. the necessary information to process the request is not available at the NRM server), the NRM server shall respond to the service consumer with a proper error status code.

##### 5.5.1.2.10	Create_MBS_Resource

###### 5.5.1.2.10.1	General

This service operation is used by a service consumer to request the creation of a new MBS Resource at the NRM Server (see also clause 14.3.4A of 3GPP°TS°23.434°[2]).

###### 5.5.1.2.10.2	Service consumer requesting the creation of an MBS Resource using the Create_MBS_Resource service operation

In order to request the creation of a new MBS Resource, the service consumer shall send an HTTP POST request message to the NRM Server targeting the "MBS Resources" collection resource, with the request body containing the MBSResourceReq data structure.

Upon reception of the HTTP POST request message, the NRM Server shall:

-	verify the identity of the service consumer and whether the service consumer is authorized to initiate such request;

-	if the service consumer is authorized and upon successful processing of the request, the NRM Server shall create a new "Individual MBS Resource" resource and respond to the service consumer with an HTTP "201 Created" status code with the response body containing a representation of the created resource and potentially additional information (e.g. the user plane addressing information of the NRM Server for downlink data delivery) within the MBSResourceResp data structure; and

-	if errors occur when processing the request, the NRM Server shall take proper error handling actions, as specified in clause 7.4.1.5, and respond to the service consumer with an appropriate error status code.

##### 5.5.1.2.11	Update_MBS_Resource

###### 5.5.1.2.11.1	General

This service operation is used by a service consumer to request the update of an existing MBS Resource at the NRM Server (see also clause 14.3.4A of 3GPP°TS°23.434°[2]).

###### 5.5.1.2.11.2	Service consumer requesting the update of an existing MBS Resource using the Update_MBS_Resource service operation

In order to request the update of an existing MBS Resource, the service consumer shall send an HTTP PUT/PATCH request message to the NRM Server targeting the corresponding "Individual MBS Resource" resource, with the request body containing either:

-	the updated representation of the resource within the MBSResource data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the MBSResourcePatch data structure, in case the HTTP PATCH method is used.

Upon reception of the HTTP PUT/PATCH request message, the NRM Server shall:

-	verify whether the service consumer is authorized to initiate such request;

-	if the service consumer is authorized and upon successful processing of the request, the NRM Server shall update the targeted "Individual MBS Resource" resource and respond to the service consumer with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated resource and potentially additional information (e.g., the updated user plane addressing information of the NRM Server for downlink data delivery) within the MBSResource data structure; or

-	an HTTP "204 No Content" status code;

and

-	if errors occur when processing the request, the NRM Server shall take proper error handling actions, as specified in clause 7.4.1.5, and respond to the service consumer with an appropriate error status code.

##### 5.5.1.2.12	Delete_MBS_Resource

###### 5.5.1.2.12.1	General

This service operation is used by a service consumer to request the deletion of an existing MBS Resource at the NRM Server (see also clause 14.3.4A of 3GPP°TS°23.434°[2]).

###### 5.5.1.2.12.2	Service consumer requesting the deletion of an existing MBS Resource using the Delete_MBS_Resource service operation

In order to request the deletion of an existing MBS Resource, the service consumer shall send an HTTP DELETE request message to the NRM Server targeting the corresponding "Individual MBS Resource" resource.

Upon reception of the HTTP DELETE request message, the NRM Server shall:

-	verify whether the service consumer is authorized to initiate such request;

-	if the service consumer is authorized and upon successful processing of the request, the NRM Server shall delete the targeted "Individual MBS Resource" resource and respond to the service consumer with an HTTP "204 No Content" status code; and

-	if errors occur when processing the request, the NRM Server shall take proper error handling actions, as specified in clause 7.4.1.5, and respond to the service consumer with an appropriate error status code.

##### 5.5.1.2.13	Activate_MBS_Resource

###### 5.5.1.2.13.1	General

This service operation is used by a service consumer to request the activation of an existing MBS Resource at the NRM Server (see also clause 14.3.4A of 3GPP°TS°23.434°[2]).

###### 5.5.1.2.13.2	Service consumer requesting the activation of an existing MBS Resource using the Activate_MBS_Resource service operation

In order to request the activation of an existing MBS Resource, the service consumer shall invoke the "Activate" resource custom operation by sending an HTTP POST request message to the NRM Server targeting the URI of the corresponding "Individual MBS Resource" resource custom operation, i.e. "{apiRoot}/ss-nra/<apiVersion>/mbs-resources/{mbsResId}/activate", with the request body including the MbsResAct data structure.

Upon reception of the HTTP POST request message, the NRM Server shall:

-	verify whether the service consumer is authorized to initiate such request;

-	if the service consumer is authorized and upon successful processing of the request, the NRM Server shall respond to the service consumer with an HTTP "200 OK" status code with the response body including the the MbsResAct data structure; and

-	if errors occur when processing the request, the NRM Server shall take proper error handling actions, as specified in clause 7.4.1.5, and respond to the service consumer with an appropriate error status code.

##### 5.5.1.2.14	Deactivate_MBS_Resource

###### 5.5.1.2.14.1	General

This service operation is used by a service consumer to request the deactivation of an existing MBS Resource at the NRM Server (see also clause 14.3.4A of 3GPP°TS°23.434°[2]).

###### 5.5.1.2.14.2	Service consumer requesting the deactivation of an existing MBS Resource using the Deactivate_MBS_Resource service operation

In order to request the deactivation of an existing MBS Resource, the service consumer shall invoke the "Deactivate" resource custom operation by sending an HTTP POST request message to the NRM Server targeting the URI of the corresponding "Individual MBS Resource" resource custom operation, i.e. "{apiRoot}/ss-nra/<apiVersion>/mbs-resources/{mbsResId}/deactivate", with the request body including the MbsResDeact data structure.

Upon reception of the HTTP POST request message, the NRM Server shall:

-	verify whether the service consumer is authorized to initiate such request;

-	if the service consumer is authorized and upon successful processing of the request, the NRM Server shall respond to the service consumer with an HTTP "200 OK" status code with the response body including the the MbsResDeact data structure; and

-	if errors occur when processing the request, the NRM Server shall take proper error handling actions, as specified in clause 7.4.1.5, and respond to the service consumer with an appropriate error status code.

##### 5.5.1.2.15	BDT_Configuration_Request

###### 5.5.1.2.15.1	General

This service operation is used by a service consumer to request the creation of a background data transfer policy configuration at the NRM Server.

###### 5.5.1.2.15.2	Creation of background data transfer policy configuration using the BDT_Configuration_Request service operation

In order to create a background data transfer policy configuration, the service consumer shall send an HTTP POST request message to the NRM server targeting the URI of the "BDT Policy Configurations" collection resource, with the request body including the BdtPolConfig data structure.

Upon reception of the HTTP POST request message, the NRM Server shall:

-	verify whether the service consumer is authorized to initiate such request;

-	if the service consumer is authorized, when the NRM server decides to negotiate BDT policy with the 3GPP network, it triggers the background data transfer procedures defined in the clause 4.16.7.2 of 3GPP TS 23.502 [39];

-	upon success, the NRM Server shall respond to the service consumer with an HTTP "201 Created" status code with the response body including the representation of the created "Individual BDT Policy Configuration" resource within the BdtPolConfig data structure; and

-	if errors occur when processing the request or the NRM Server receives an error response from 5GC, the NRM Server shall take proper error handling actions, as specified in clause 7.4.1.5, and respond to the service consumer with an appropriate error status code.

##### 5.5.1.2.16	BDT_Negotiation_Notification

###### 5.5.1.2.16.1	General

This service operation is used by the NRM Server to notify about update in the negotiated background data transfer policy.

###### 5.5.1.2.16.2	NRM Server notifying the background data transfer policy using the BDT_Negotiation_Notification service operation

To notify on the updates in the negotiated BDT policies received from the 3GPP network, the NRM server shall send an HTTP POST request message with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation/update of the corresponding BDT policy configuration as defined in clauses 5.5.1.2.15 and 5.5.1.2.18, and the request body including the BdtNotification data structure.

Upon successful processing of the request, the service consumer shall:

-	respond with an HTTP "204 No Content" status code.

On failure, the service consumer shall take proper error handling actions, as specified in clause 7.4.1.5, and respond to the NRM server with an appropriate error status code.

##### 5.5.1.2.17	BDT_Configuration_Get

###### 5.5.1.2.17.1	General

This service operation is used by a service consumer to request the retrieval of an existing "Individual BDT Policy Configuration" resource at the NRM Server.

###### 5.5.1.2.17.2	Retreivel of an individual background data transfer policy configuration using the BDT_Configuration_Get service operation

In order to retrieve an individual background data transfer policy configuration resource, the service consumer shall send an HTTP GET request message targeting the URI of the corresponding "Individual BDT Policy Configuration" resource.

Upon reception of the HTTP GET request message, the NRM server shall:

-	verify whether the service consumer is authorized to retrieve the BDT policy configuration information;

-	if the service consumer is authorized to fetch the BDT policy configuration information, the NRM server shall;

-	responds to the service consumer with an HTTP "200 OK" status code with the response body including the requested BDT policy configuration within the BdtPolConfig data structure.

##### 5.5.1.2.18	BDT_Configuration_Update

###### 5.5.1.2.18.1	General

This service operation is used by a service consumer to request the update of an existing "Individual BDT Policy Configuration" resource at the NRM server.

###### 5.5.1.2.18.2	Update of an individual background data transfer policy configuration using the BDT_Configuration_Update service operation

In order to update an existing background data transfer policy configuration, the service consumer shall send an HTTP PUT/PATCH request message to the NRM server targeting the corresponding "Individual BDT Policy Configuration" resource with the request body including either:

-	the updated representation of the resource within the BdtPolConfig data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the BdtPolConfigPatch data structure, in case the HTTP PATCH method is used.

Upon receiving the HTTP PUT/PATCH request message from the service consumer, the NRM server shall:

-	check whether the service consumer is authorized to request the update of the targeted existing "Individual BDT Policy Configuration" resource;

-	if the service consumer is authorized, then the NRM server shall update/modify the targeted resource accordingly and respond to the service consumer with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual BDT Policy Configuration" resource within the BdtPolConfig data structure; or

-	an HTTP "204 No Content" status code;

and

-	if errors occur when processing the request or the NRM Server receives an error response from 5GC, the NRM Server shall take proper error handling actions, as specified in clause 7.4.1.5, and respond to the service consumer with an appropriate error status code.

##### 5.5.1.2.19	BDT_Configuration_Delete

###### 5.5.1.2.19.1	General

This service operation is used by a service consumer to request the deletion of an existing "Individual BDT Policy Configuration" resource at the NRM server.

###### 5.5.1.2.19.2	Deletion an individual background data transfer policy configuration using the BDT_Configuration_Delete service operation

In order to delete an existing BDT policy configuration, the service consumer shall send an HTTP DELETE request message to the NRM server targeting the corresponding "Individual BDT Policy Configuration" resource.

Upon receiving the HTTP DELETE request message:

-	check whether the service consumer is authorized to request the deletion of the targeted existing "Individual BDT Policy Configuration" resource;

-	if the service consumer is authorized, the NRM server shall delete the resource and respond to the service consumer with an HTTP "204 No Content" status code; and

-	if errors occur when processing the request, the NRM Server shall respond to the service consumer with an appropriate error response as specified in clause 7.4.1.5.

##### 5.5.1.2.20	Subscribe_Unified_Traffic_Pattern_and_Monitoring_Management

###### 5.5.1.2.20.1	General

This service operation is used by a service consumer to subscribe to UE unified traffic pattern and monitoring management at the NRM Server.

###### 5.5.1.2.20.2	Service consumer subscribing to unified traffic pattern using the Subscribe_Unified_Traffic_Pattern_and_Monitoring_Management service operation

In order to create subscribe to UE unified traffic pattern monitoring and management, the service consumer shall send an HTTP POST request message to the NRM server targeting the URI of the "Unified Traffic Pattern Subscriptions" resource, with the request body including the UnfTrafficSubc data structure.

Upon reception of the HTTP POST request message, the NRM Server shall:

-	verify whether the service consumer is authorized to initiate such request;

-	if the service consumer is authorized, then the NRM Server executes one or more management and 5GC exposure procedures (per UE) as specified in clause 14.3.12.2 of 3GPP TS 23.434 [2];

-	upon success, the NRM shall create the requested "Individual Unified Traffic Pattern Subscription" resource and respond to the service consumer with an HTTP "201 Created" status code with the response body including the the representation of the created resource within the UnfTrafficSubc data structure; and

-	if errors occur when processing the request, the NRM Server shall take proper error handling actions, as specified in clause 7.4.1.5, and respond to the service consumer with an appropriate error status code.

##### 5.5.1.2.21	Update_Unified_Traffic_Pattern_and_Monitoring_Management_Subscription

###### 5.5.1.2.21.1	General

This service operation is used by a service consumer to update an existing UE unified traffic pattern and monitoring management subscription at the NRM Server.

###### 5.5.1.2.21.2	Service consumer updating the subscription to unified traffic pattern using the Update_Unified_Traffic_Pattern_and_Monitoring_Management_Subscription service operation

To update an existing UE unified traffic pattern monitoring and management subscription, the service consumer shall send to the NRM server targeting the URI of the "Individual Unified Traffic Pattern Subscription" resource either,

-	an HTTP PUT request message with the request body including the UnfTrafficSubc data structure, or

-	an HTTP PATCH request message with the request body including the UnfTrafficSubcPatch data structure

Upon receiving the HTTP PUT/PATCH request message, if the service consumer is authorized to modify/update the UE unified traffic pattern and monitoring management subscription, then the NRM server shall:

1.	update/modify the targeted resource accordingly;

2.	respond to the service consumer with either:

-	an HTTP "200 OK" with the response body containing the updated representation of the "Individual Unified Traffic Pattern Subscription" resource within UnfTrafficSubc data structure; or

-	an HTTP "204 No Content" status code;

and

3.	if errors occur when processing the request, the NRM Server shall respond to the service consumer with an appropriate error response as specified in clause 7.1.3.5.

##### 5.5.1.2.22	Unsubscribe_Unified_Traffic_Pattern_and_Monitoring_Management

###### 5.5.1.2.22.1	General

This service operation is used by a service consumer to delete an existing UE unified traffic pattern and monitoring management subscription at the NRM Server.

###### 5.5.1.2.22.2	Service consumer deleting the subscription to unified traffic pattern using the Unsubscribe_Unified_Traffic_Pattern_and_Monitoring_Management service operation

To delete an existing UE unified traffic pattern subscription, the service consumer shall send a HTTP DELETE request message to the NRM server targeting the corresponding "Individual Unified Traffic Pattern Subscription" resource.

Upon reception of the HTTP DELETE request message, if the service consumer is authorized to delete the UE unified traffic pattern subscription, then the NRM server shall:

1.	delete the targeted resource and respond to the service consumer with an HTTP "204 No Content" status code;and

2.	if errors occur when processing the request, the NRM Server shall respond to the service consumer with an appropriate error response as specified in clause 7.1.3.5.

##### 5.5.1.2.23	Notify_Unified_Traffic_Pattern_Update

###### 5.5.1.2.23.1	General

This service operation is used by the NRM Server to notify the service consumer on UE unified traffic pattern monitoring and management related event(s).

###### 5.5.1.2.23.2	NRM Server notifying on unified traffic pattern monitoring and management related event(s) using the Notify_Unified_Traffic_Pattern_Update service operation

To notify on UE unified traffic pattern monitoring and management related event(s) as specified in clause 14.3.12.3 of 3GPP TS 23.434 [2], the NRM server shall send an HTTP POST request message to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation/update of the corresponding unified traffic pattern subscription as defined in clauses 5.5.1.2.20 and 5.5.1.2.21 and the request body including the UpdTrafficUpdNotif data structure.

Upon successful processing of the request, the service consumer shall respond with an HTTP "204 No Content" status code.

On failure, the service consumer shall take proper error handling actions, as specified in clause 7.4.1.5, and respond to the NRM server with an appropriate error status code.

##### 5.5.1.2.24	Reliable_Transmission_Request

###### 5.5.1.2.24.1	General

This service operation is used by a service consumer to request reliable transmission service to the NRM Server (see also clause 14.3.10.2 of 3GPP°TS°23.434°[2]).

The following procedures are supported by the "Reliable_Transmission_Request" service operation:

-	Reliable Transmission Service Request.

###### 5.5.1.2.24.2	Reliable Transmission Service Request

1.	In order to request reliable transmission service, the service consumer shall send an HTTP POST request to the NRM Server targeting the URI of the corresponding custom operation (i.e., "RelTransRequest"), with the request body including the RelTransReq data structure.

2.	Upon reception of the HTTP POST request message, the NRM Server shall:

-	check whether the service consumer is authorized to initiate such request;

-	if the service consumer is authorized and upon successful processing of the request, the NRM Server shall respond with an HTTP "204 No Content" status code; and

-	on failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.4.1.5.

### 5.5.2	SS_EventsMonitoring API

The SS_EventsMonitoring API, as defined in 3GPP TS 23.434 [2], allows a VAL server via NRM-S reference point to subscribe for and receive notifications from the Network Resource Management server about events related to VAL UE(s). The SS_EventsMonitoring API supports this via the "NRM_MONITOR_UE_USER_EVENTS" event in SS_Events API as specified in clause 7.5. Based on the events of interest information related to the VAL UE(s), the NRM server shall subscribe to UE monitoring types and analytics events as specified in clause 4.4.2 and clause 4.4.14 of 3GPP TS 29.522 [28].

### 5.5.3	SS_NetworkResourceMonitoring API

#### 5.5.3.1	Service Description

##### 5.5.3.1.1	Overview

The SS_NetworkResourceMonitoring API, as defined in 3GPP TS 23.434 [2], allows VAL server via NRM-S reference point to communicate with the network resource management server for network resource monitoring including requesting unicast QoS monitoring data and managing unicast QoS monitoring subscription.

#### 5.5.3.2	Service Operations

##### 5.5.3.2.1	Introduction

The service operations defined for the SS_NetworkResourceMonitoring API are shown in the table 5.5.3.2.1-1.

Table 5.5.3.2.1-1: Operations of the SS_NetworkResourceMonitoring API

| Service operation name |  | Description |  | Initiated by |  |
| --- | --- | --- | --- | --- | --- |
| Subscribe_Unicast_QoS_Monitoring |  | This service operation is used by VAL server to subscribe to unicast QoS monitoring events from SEAL servers. |  | VAL server |  |
| Unsubscribe_Unicast_QoS_Monitoring |  | This service operation is used by VAL server to unsubscribe from unicast QoS monitoring events from SEAL servers. |  | VAL server |  |
| Notify_Unicast_QoS_Monitoring |  | This service operation is used by SEAL server to send the notifications to the VAL server. |  | NRM server |  |
| Obtain_Unicast_QoS_Monitoring |  | This service operation is used by VAL server to obtain unicast QoS monitoring data. |  | VAL server |  |
| Update_Unicast_QoS_Monitoring_Subscription |  | This service operation is used by VAL server to update or modify the individual unicast QoS monitoring subscription. |  | VAL server |  |

##### 5.5.3.2.2	Subscribe_Unicast_QoS_Monitoring

###### 5.5.3.2.2.1	General

This service operation is used by a VAL server to create a unicast QoS monitoring subscription to the NRM server.

###### 5.5.3.2.2.2	VAL server subscribes for Unicast QoS Monitoring using Subscribe_Unicast_QoS_Monitoring

In order to subscribe to unicast QoS monitoring, the VAL server shall send an HTTP POST message to the NRM server targeting the URI of the "Unicast Monitoring Subscriptions" resource as specified in clause 7.4.2.2.2.3.1. The request body shall include the MonitoringSubscription data structure as defined in clause 7.4.2.4.2.8. The VAL server shall indicate within the ReportingRequirement data structure whether one-time reporting and/or immediate reporting is requested, i.e.:

-	the "immRep" attribute set to "true", if immediate reporting of the unicast QoS Monitoring data is requested; and/or

-	the "reportingMode" attribute set to "ONE_TIME" and the "immRep" attribute set to "true", if one-time reporting of the unicast QoS Monitoring data is requested via the Obtain_Unicast_QoS_Monitoring service operation.

Upon reception of the HTTP POST request message, the NRM server shall:

1.	verify the identity of the VAL server and whether the VAL server is authorized to create a unicast QoS monitoring subscription at the NRM server;

2.	if the VAL server is not authorized, the NRM server shall respond to the VAL server with an appropriate error status code;

3.	if the VAL server is authorized:

a.	if immediate reporting and one-time reporting are requested, the NRM server determines if it the requested data is available internally or not and whether to interact with the NEF to retrieve the data using the Nnef_AnalyticsExposure API (UE Communication Analytics Events and DN Performance Analytics) as defined in 3GPP TS 29.522 [28];

b.	otherwise, the NRM server shall interact with the NEF to establish the associated QoS monitoring subscriptions by invoking the Nnef_AnalyticsExposure API (UE Communication Analytics Events and DN Performance Analytics) and AsSessionWithQoS API as defined in 3GPP TS 29.522 [28]. The NRM server determines the relevant NEF subscription procedures and the parameters for these subscriptions based on the inputs received from the VAL server;

4.	upon reception of successful response(s) from the NEF or retrieval of the requested data internally:

a.	if immediate reporting and one-time reporting are requested, an HTTP "200 OK" status code, with the response body including the MonitoringReport data structure containing the available requested Unicast QoS Monitoring data as defined in clause 7.4.2.4.2.2;

b.	otherwise, the NRM server shall create a new "Individual Unicast Monitoring Subscription" resource and respond to the VAL server with:

-	an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Individual Unicast Monitoring Subscription" resource and the response body including the MonitoringSubscription data structure containing a representation of the created resource as defined in clause 7.4.2.4.2.8; and

-	if immediate reporting was requested by the VAL server, the returned MonitoringSubscription data structure shall also contain the requested Unicast QoS Monitoring data within the "monRep" attribute, if the requested data is available, as defined in clause 7.4.2.4.2.2;

c.	in the case of partial failure, i.e. the request fails for only a subset of the targeted VAL UE(s) or VAL Stream ID(s), the NRM server shall include the "failureRep" attribute within the returned MonitoringReport data structure indicating the list of VAL UE(s) or VAL Stream ID(s) for which the NRM server failed to obtain the requested data and the related failure reasons;

and

5.	if the NRM server is unable to satisfy the request, the NRM server shall respond to the VAL server with an appropriate error status code.

##### 5.5.3.2.3	Unsubscribe_Unicast_QoS_Monitoring

###### 5.5.3.2.3.1	General

This service operation is used by a VAL server to terminate a unicast QoS monitoring subscription at the NRM server.

###### 5.5.3.2.3.2	VAL server unsubscribes for Unicast QoS Monitoring using Unsubscribe_Unicast_QoS_Monitoring

In order to terminate a unicast QoS monitoring subscription, the VAL server shall send an HTTP DELETE request message to the NRM server, on the corresponding "Individual Unicast Monitoring Subscription" resource URI as specified in clause 7.4.2.2.3.3.1.

Upon reception of the HTTP DELETE request message, the NRM server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorised to terminate the targeted "Individual Unicast Monitoring Subscription" associated with the resource URI;

2.	if the VAL server is authorized to unsubscribe from Unicast QoS Monitoring, interact with the NEF to terminate the related QoS monitoring subscription and delete the related "Individual Unicast Monitoring Subscription" subscription resource at the NRM server; and

3.	upon success, respond to the VAL server with a "204 No Content" status code.

##### 5.5.3.2.4	Notify_Unicast_QoS_Monitoring

###### 5.5.3.2.4.1	General

This service operation is used by the NRM server to notify the VAL server of unicast QoS monitoring data.

###### 5.5.3.2.4.2	NRM server notifies for Unicast QoS Monitoring using Notify_Unicast_QoS_Monitoring

The NRM server receives unicast QoS monitoring data by means of notifications provided by the NEF. The NRM server coordinates and aggregates the received information from the NEF notifications and determines whether to send a notification to the VAL server based on the VAL server subscription's reporting requirements. For event-triggered reporting, the NRM server notifies the VAL server when any given event is triggered. For a VAL group or a list of VAL UEs, the NRM server aggregates QoS monitoring data for each UE belonging to the group or the list; for a VAL stream, the NRM server aggregates the QoS monitoring data for the stream.

The NRM server stops reporting according to the VAL server subscription's termination of reporting requirements. In the case user-triggered termination of reporting is requested or no termination of reporting requirements are provided, the NRM server terminates the Unicast Monitoring Subscription after receiving an explicit request from the VAL Server as specified in clause 5.5.3.2.3. In the case of time-triggered or event-triggered termination of reporting, the NRM server shall stop the reporting and terminate the subscription when the provided conditions are met. The NRM server may also store internally the QoS monitoring data as needed for later retrieval.

In order to notify the VAL server about Unicast QoS Monitoring information updates, the NRM server shall send an HTTP POST request message to the VAL server targeting the notification URI provided during subscription creation as specified in clause 5.5.3.2.2.2.

Upon receiving the HTTP POST request message, the VAL server shall:

1. process the Unicast QoS Monitoring notification; and

2. upon success, respond to the NRM server with a "204 No Content" status code.

##### 5.5.3.2.5	Obtain_Unicast_QoS_Monitoring_Data

###### 5.5.3.2.5.1	General

This service operation is used by a VAL server to obtain the QoS monitoring data from the NRM server for a time period of interest either in the past or in the present (i.e. current time). This service operation is supported via the Subscribe_Unicast_QoS_Monitoring service operation using the immediate reporting and one-time reporting mechanisms as defined in clause 5.5.3.2.2.2.

##### 5.5.3.2.6	Update_Unicast_QoS_Monitoring_Subscription

###### 5.5.3.2.6.1	General

This service operation is used by a VAL server to update a unicast QoS monitoring subscription at the NRM server.

###### 5.5.3.2.6.2	VAL server modifies for Unicast QoS Monitoring Subscription using Update_Unicast_QoS_Monitoring Subscription

If the "UpdateSupport" feature defined in clause 7.4.2.6 is supported, the VAL server may send an HTTP PUT or an HTTP PATCH request to update or modify an individual unicast QoS monitoring subscription, respectively.

In order to update an existing individual unicast QoS monitoring subscription, the VAL server shall send an HTTP PUT request message to the NRM server, targeting the corresponding "Individual Unicast Monitoring Subscription" resource URI as specified in clause 7.4.2.2.3.3.3.

In order to modify an existing individual unicast QoS monitoring subscription, the VAL server shall send an HTTP PATCH request message targeting the "Individual Unicast Monitoring Subscription" resource URI as specified in clause 7.4.2.2.3.3.4. The body of the HTTP PATCH request message shall include the requested modifications as specified in clause 7.4.2.2.3.3.4.

The updated resource representation information provided via an HTTP PUT method shall not change any target identifier, i.e., the "valUeIds", "valGroupId", and "valStreamIds" attributes within the "Individual Unicast Monitoring Subscription" resource.

Upon receiving the HTTP PUT or HTTP PATCH request message, the NRM server shall:

1.	check if the required features for the received HTTP request are supported as defined in clause 7.4.2.6;

2.	verify the identity of the VAL server and check if the VAL server is authorised to update or modify the "Individual Unicast Monitoring Subscription" resource;

3.	if the required feature for the received HTTP request is supported and the VAL server is authorized, then the NRM server shall:

a.	if the configuration information in the request is valid, interact with the NEF to update, modify, or establish the associated QoS monitoring subscriptions by invoking the Nnef_AnalyticsExposure API (UE Communication Analytics Events and DN Performance Analytics) and AsSessionWithQoS API as defined in 3GPP TS 29.522 [28]. The NRM server determines the relevant NEF subscription procedures and the parameters for these subscriptions based on the inputs received from the VAL server;

4.	upon reception of successful response(s) from the NEF:

a.	update/modify the resource identified by the Resource URI of the configuration received in the request; and

b.	respond to the VAL server with an HTTP "200 OK" status;

and

5.	if the NRM server is unable to satisfy the request, the NRM server shall respond to the VAL server with an appropriate error status code as defined in clause 7.4.2.5.

### 5.5.4	SS_SatelliteSFInfoEvent API

The SS_SatelliteSFInfoEvent API, as defined in 3GPP TS 23.434 [2], allows a VAL Server via the NRM-S reference point to subscribe and receive notifications from the NRM Server on Satellite S&F related event(s). The SS_SatelliteSFInfoEvent API supports this via the "NRM_SAT_SF_INFO" event of the SS_Events API as specified in clause 7.5.

## 5.6	Events APIs

### 5.6.1	SS_Events API

#### 5.6.1.1	Service Description

##### 5.6.1.1.1	Overview

The SS_Events API, allows a VAL server via LM-S, GM-S, CM-S and NRM-S reference points to subscribe and unsubscribe from SEAL events and to receive notifications from the Location Management Server, Group Management Server, Configuration Management Server and Network Resource Management Server respectively.

#### 5.6.1.2	Service Operations

##### 5.6.1.2.1	Introduction

The service operations defined for the SS_Events API are shown in the table 5.6.1.2.1-1.

Table 5.6.1.2.1-1: Operations of the SS_Events API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_Event | This service operation is used by VAL server to subscribe for events from SEAL servers. | VAL Server |
| Unsubscribe_Event | This service operation is used by VAL server to unsubscribe for events from SEAL servers. | VAL Server |
| Notify_Event | This service operation is used by SEAL servers to send the notifications to the VAL server. | SEAL servers (Location Management, Group Management, Configuration Management). |
| Update_Subscription | This service operation is used by VAL server to update its events subscription at SEAL server. | VAL Server |

##### 5.6.1.2.2	Subscribe_Event

###### 5.6.1.2.2.1	General

This service operation is used by a VAL server to subscribe to the SEAL events.

###### 5.6.1.2.2.2	Subscribing to SEAL events using Subscribe_Event service operation

To subscribe to SEAL events, the VAL server shall send an HTTP POST message to the SEAL server. The body of the HTTP POST message shall include VAL Server Identifier, Event Type, Event Filters, Reporting Requirements and a Notification Destination URI as specified in clause 7.5.1.2.2.3.1.

Upon receiving the above described HTTP POST message, the SEAL server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to subscribe to the SEAL events mentioned in the HTTP POST message;

2.	if the VAL server is authorized to subscribe to the SEAL events, the SEAL server shall:

a.	verify the provided identifier(s) in the request;

b.	if the PartialFailureSupport feature is supported and partial failure has occurred (e.g. the SEAL server fails to verify the identifier(s) of only a subset of the provided target identifiers), the SEAL server shall process the request for the identifier(s) for which the verification is successful, create a new resource as specified in clause 7.5.1.2.1 and include in the response to the service consumer (i.e. VAL server) the "partialFailRep" attribute within the EventSubscription data structure indicating the list of the target identifier(s) for which the verification failed;

c.	otherwise, upon successful verification of the provided identifier(s) and successful processing of the request, create a new resource as specified in clause 7.5.1.2.1;

d.	return the created resource representation and the created SEAL Resource URI in the response message ; and

e.	if the SEAL server is unable to satisfy the request, the SEAL server shall respond to the VAL server with an appropriate error status code as defined in clause 7.5.1.5.

##### 5.6.1.2.3	Notify_Event

###### 5.6.1.2.3.1	General

This service operation is used by the SEAL servers to send notifications to the VAL server.

###### 5.6.1.2.3.2	Notifying SEAL events using Notify_Event service operation

To notify the SEAL events, the SEAL server shall send an HTTP POST message using the Notification Destination URI received in the subscription request. The body of the HTTP POST message shall include an Event Notification and SEAL Resource URI.

Upon receiving the HTTP POST message, the VAL server shall process the Event Notification.

##### 5.6.1.2.4	Unsubscribe_Event

###### 5.6.1.2.4.1	General

This service operation is used by a VAL server to un-subscribe from the SEAL events.

###### 5.6.1.2.4.2	Unsubscribing from SEAL events using Unsubscribe_Event service operation

To unsubscribe from SEAL events, the VAL server shall send an HTTP DELETE message to the resource representing the event in the SEAL server as specified in clause 7.5.1.2.3.3.1.

Upon receiving the HTTP DELETE message, the SEAL sever shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to Unsubscribe from the SEAL event associated with the SEAL Resource URI; and

2.	if the VAL server is authorized to unsubscribe from the SEAL events, the SEAL server shall delete the resource pointed by the SEAL Resource URI

##### 5.6.1.2.5	Update_Subscription

###### 5.6.1.2.5.1	General

This service operation is used by a VAL server to update its SEAL events subscription.

###### 5.6.1.2.5.2	Updating the SEAL events subscription using Update_Subscription service operation

If the "SubscUpdate" feature as defined in clause 7.5.1.6 is supported, then to request the update/modification of an existing Individual SEAL Events Subscription, the VAL server shall send a HTTP PATCH request (for partial modification) or PUT request (for fully replacement) message to the SEAL server on resource URI "Individual SEAL Events Subscription" resource as specified in clause 7.5.1.2.3.3.2 for HTTP PATCH message and in clause 7.5.1.2.3.3.3 for HTTP PUT message.

Upon receiving the HTTP PATCH or PUT message from the VAL server, the SEAL server shall:

1.	check whether the VAL server is authorized to update the existing "Individual SEAL Events Subscription" resource or not;

2. 	if the VAL server is authorized to update the SEAL Events Subscription, then:

a.	if the "PartialFailureSupport" feature is supported and partial failure has occurred (e.g., the SEAL server fails to verify the identifier(s) of only a subset of the provided target identifiers), the SEAL server shall process the request for the identifier(s) for which the verification is successful, update the existing "Individual SEAL Events Subscription" resource and respond to the service consumer (i.e., VAL server) with an HTTP "200 OK" status code with the response body including the "partialFailRep" attribute, indicating the list of the target identifier(s) for which the verification failed, within the concerned instance(s) (encoded using the EventSubscription data structure) of the SEALEventSubscription data structure;

b.	otherwise, the SEAL server shall update the existing "Individual SEAL Events Subscription" resource and respond to the VAL server with either an HTTP "204 No Content" status code or an HTTP "200 OK" status code with the response body including the updated representation of the "Individual SEAL Events Subscription" resource within the SEALEventSubscription data structure.

## 5.7	Key management APIs

### 5.7.1	SS_KeyInfoRetrieval API

#### 5.7.1.1	Service Description

##### 5.7.1.1.1	Overview

As specified in 3GPP TS 33.434 [26], the SS_KeyInfoRetrieval API, allows the VAL server via KM-S reference point to obtain the VAL service specific key management information from the key management server.

#### 5.7.1.2	Service Operations

##### 5.7.1.2.1	Introduction

The service operation defined for SS_KeyInfoRetrieval API is shown in the table 5.7.1.2.1-1.

Table 5.7.1.2.1-1: Operations of the SS_ KeyInfoRetrieval API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Obtain_Key_Info | This service operation is used by VAL server to obtain key management information. | VAL server |

##### 5.7.1.2.2	Obtain_Key_Info

###### 5.7.1.2.2.1	General

This service operation is used by the VAL server to obtain VAL service specific key management information.

###### 5.7.1.2.2.2		VAL server obtaining VAL service specific key material using Obtain_Key_Info service operation

To obtain key management information specific to VAL service, the VAL server shall send HTTP GET request message to key management server, on Key records resource collection URI, with query parameters VAL service ID and optionally VAL user ID or VAL UE ID, as specified in 7.6.1.2.2.3.1.

Upon receiving the HTTP GET message as described above, the key management server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to obtain key management information specific to VAL service, VAL user or VAL UE, the URI in the request is of target SEAL KMS and date/time of the request is in recent time window;

2.	if the VAL server is authorized to obtain the requested key management information, the key management server shall;

a.	return in the response message with key management information corresponding to the query parameters that were sent in the request message.

### 5.7.2	SS_KMParametersProvisioning API

#### 5.7.2.1	Service Description

##### 5.7.2.1.1	Overview

As specified in 3GPP TS 23.434 [2] and 3GPP TS 33.434 [26], the SS_KMParametersProvisioning API enables a VAL server to provision the key parameters information specific to VAL service, VAL User or VAL UE at the Key Management (KM) server over the SEAL KM-S reference point.

#### 5.7.2.2	Service Operations

##### 5.7.2.2.1	Introduction

The service operations defined for the SS_KMParametersProvisioning API are shown in the table 5.7.2.2.1-1.

Table 5.7.2.2.1-1: Operations of the SS_KMParametersProvisioning API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Provide_Configuration | This service operation is used by a VAL server to provision the VAL service, VAL User or VAL UE specific key parameter information to the KM server. | VAL server |

##### 5.7.2.2.2	Provide_Configuration

###### 5.7.2.2.2.1	General

This service operation is used by a VAL server to provision key parameter information to the KM server.

###### 5.7.2.2.2.2	VAL server provisioning key parameter information using Provide_Configuration service operation

To provision the VAL service, VAL User or VAL UE specific key parameter information, the VAL server shall send an HTTP POST request message to the KM Server with the request body including the VALKeyConfiguration data structure specified in clause 7.6.2.5.2.2.

Upon receiving HTTP POST message, the key management server shall:

1.	verify the identity of the VAL Server and check if the VAL Server is authorized to provision the key parameter information as specified in 3GPP TS 33.434 [26];

2.	if the VAL Server is authorized to provision the key parameter information, shall create a new resource as defined in 7.6.2.2.2.3.1; and

3.	if the KM Server is unable to satisfy the request, the KM Server shall respond to the VAL Server with an appropriate error status code as defined in clause 7.6.2.5.

## 5.8	Network Slice Capability Enablement APIs

NSCE APIs are defined in 3GPP TS 29.435 [42].

## 5.9	Identity Management APIs

### 5.9.1	SS_IdmParameterProvisioning API

#### 5.9.1.1	Service Description

##### 5.9.1.1.1	Overview

As specified in 3GPP TS 23.434 [2], the SS_IdmParameterProvisioning API enables a VAL server to communicate with the Identity Management (IM) server for the provisioning of the VAL service specific information for a VAL server (i.e. VAL server's VAL service(s) and the related VAL user(s) information) over the SEAL IM-S reference point.

#### 5.9.1.2	Service Operations

##### 5.9.1.2.1	Introduction

The service operations defined for the SS_IdmParameterProvisioning API are shown in the table 5.9.1.2.1-1.

Table 5.9.1.2.1-1: Operations of the SS_IdmParameterProvisioning API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Provide_Configuration | This service operation is used to provision the VAL service specific information to the IM server. | e.g. VAL server |
| Get_Configuration | This service operation is used to retrieve the VAL service specific information provisioned to the IM server. | e.g. VAL server |
| Update_Configuration | This service operation is used to update the VAL service specific information provisioned to the IM server. | e.g. VAL server |
| Delete_Configuration | This service operation is used to delete the VAL service specific information provisioned to the IM server. | e.g. VAL server |

##### 5.9.1.2.2	Provide_Configuration

###### 5.9.1.2.2.1	General

This service operation is used by a VAL server to provision the list of VAL service specific information of the VAL server to the IM server.

###### 5.9.1.2.2.2	VAL server provisioning VAL service specific information using Provide_Configuration service operation

To provision the VAL services specific information, the VAL server shall send an HTTP POST request message to the IM Server with the request body including the VALServicesConfig data structure specified in clause 7.8.1.5.2.2.

Upon receiving HTTP POST message, the identity management server shall:

1.	verify the identity of the VAL Server and check if the VAL Server is authorized to provision the VAL services specific configuration information;

2.	if the VAL Server is authorized to provision the VAL services specific configuration information, shall create a new resource as defined in 7.8.1.2.2.3.1; and

3.	if the IM Server is unable to satisfy the request, the IM Server shall respond to the VAL Server with an appropriate error status code as defined in clause 7.8.1.5.

##### 5.9.1.2.3	Get_Configuration

###### 5.9.1.2.3.1	General

This service operation is used by a service consumer to fetch the provisioned list of VAL services specific information of the VAL server from the IM server.

###### 5.9.1.2.3.2	Service consumer obtaining the VAL service specific information provisioned using Get_Configuration service operation

To obtain the provisioned VAL services configuration information, the service consumer shall send a HTTP GET message to the IM server targeting either,

- 	"Individual VAL Services Configuration" resource representation URI as specified in clause 7.8.1.2.3.3.1; or

-	"VAL Services Configurations" collection resource representation URI as specified in clause 7.8.1.2.2.3.2. In the GET message to "VAL Services Configurations" collection resource, the service consumer may include the VAL Server ID of the requesting VAL server, list of identifiers identifying the "Individual VAL Services Configuration" resources, as query parameters.

Upon receiving the HTTP GET message as described above, if the service consumer is authorized to obtain the VAL Services Configuration information, then the IM server shall:

1.	in the request to "VAL Services Configurations" collection resource representation URI, return the VAL Service Configurations, list of "VALServicesConfig" data type, matching the query parameters in the response message;

2.  in the request to "Individual VAL Services Configuration" resource representation URI, return the VAL Service Configuration resource "VALServicesConfig" data type in the response message.

3.	if errors occur when processing the request, the IM Server shall respond to the service consumer with an appropriate error response as specified in clause 7.1.3.5.

##### 5.9.1.2.4	Update_Configuration

###### 5.9.1.2.4.1	General

This service operation is used by a service consumer to udpate the list of VAL service specific information of the VAL server provisioned to the IM server.

###### 5.9.1.2.4.2	Service consumer updating VAL service specific information using Update_Configuration service operation

To modify the VAL Services Configuration, the service consumer shall send either,

-	HTTP PUT message, including the "VALServicesConfig" data type, to the IM server to the Resource URI identifying the "Individual VAL Services Configuration" resource representation, as specified in the clause 7.8.1.2.3.3.2. This request shall not replace valServerId property in the existing resource, or

-	HTTP PATCH request message, including the "VALServicesConfigPatch" data type, to the "Individual VAL Services Configuration" resource URI as specified in clause 7.8.1.2.3.3.3, to partially update the VAL Service Configuration. The body of the HTTP PATCH request message shall include the requested modifications as specified in clause 7.8.1.2.3.3.3.

Upon receiving the HTTP PUT / PATCH message, if the service consumer is authorized to modify/update the VAL services configuration information, then the IM server shall:

1.	update/modify the resource identified by the Resource URI of the "Individual VAL Services Configuration" with the VAL Services information received in the request;

2.	respond to the service consumer with HTTP "200 OK" with the response body containing the updated "Individual VAL Services Configuration" in "VALServicesConfig" data type, in the response or a 204 No Content status code;

3.	if errors occur when processing the request, the IM Server shall respond to the service consumer with an appropriate error response as specified in clause 7.1.3.5.

##### 5.9.1.2.5	Delete_Configuration

###### 5.9.1.2.5.1	General

This service operation is used by a service consumer to delete the list of VAL service specific information of the VAL server provisioned to the IM server.

###### 5.9.1.2.5.2	Service consumer deleting VAL service specific information using Delete_Configuration service operation

To delete a VAL Services Configuration information, the service consumer shall send a HTTP DELETE message to the IM server to its "Individual VAL Services Configuration" resource representation in the IM server as specified in clause 7.8.1.2.3.3.4. Upon receiving the HTTP DELETE message, if the service consumer is authorized to delete the VAL Services Configuration, then the IM server shall:

1.	delete the resource representation pointed by the Individual VAL Service Configuration resource identifier;

2.	if errors occur when processing the request, the IM Server shall respond to the service consumer with an appropriate error response as specified in clause 7.1.3.5.

## 5.10	Data Delivery APIs

SEALDD APIs are defined in 3GPP TS 29.548 [35].

## 5.11	Application data analytics enablement service configuration APIs

### 5.11.1	SS_ADAE_VALPerformanceAnalytics API

#### 5.11.1.1	Service Description

##### 5.11.1.1.1	Overview

The SS_ADAE_VALPerformanceAnalytics API, as defined 3GPP TS 23.436 [38], allows the VAL server via ADAE-S reference point to subscribe to VAL performance analytics event to the ADAE Server.

#### 5.11.1.2	Service Operations

##### 5.11.1.2.1	Introduction

The service operation defined for SS_ADAE_VALPerformanceAnalytics API is shown in the table 5.11.1.2.1-1.

Table 5.11.1.2.1-1: Operations of the SS_ADAE_VALPerformanceAnalytics API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_VAL_Performance_Analytics | This service operation is used by VAL server to subsribe to the event of the VAL performance analytics. | VAL Server |
| Notify_VAL_Performance_Analytics | This service operation is used by ADAE Server to notify about the VAL performance analytics. | ADAE Server |
| Unsubscribe_VAL_Performance_Analytics | This service operation is used by VAL server to unsubsribe from the event of the VAL performance analytics. | VAL server |

##### 5.11.1.2.2	Subscribe_VAL_Performance_Analytics

###### 5.11.1.2.2.1	General

This service operation is used by the VAL server for VAL performance analytics event subscription to the ADAE Server.

###### 5.11.1.2.2.2	Subscribing to VAL performance analytics event using Subscribe_VAL_Performance_Analytics service operation

To subscribe to VAL performance analytics event, the VAL server shall send an HTTP POST request to the ADAES with a Request-URI set to "{apiRoot}/ss-adae-pa/<apiVersion>/application-performance", and the body containing the AppPerfSub data structure as defined in clause 7.10.1.4.2.2.

Upon receipt of the HTTP POST request, the ADAES shall:

1.	verify the identity of the VAL server and determine if the VAL server is authorized to subscribe to the VAL performance analytics event; and

2.	if the VAL server:

a.	is not authorized, the ADAES shall respond to an appropriate error status code; or

b.	is authorized, the ADAES shall create a new "Individual application performance event subscription" resource and respond to the VAL server with an HTTP "201 Created" status code, including a Location header field containing the URI for the created resource and the response body including the AppPerfSub data structure containing a representation of the created resource.

If errors occur when processing the HTTP POST request, the ADAES shall send an HTTP error response as specified in clause 7.10.1.5.

##### 5.11.1.2.3	Notify_VAL_Performance_Analytics

###### 5.11.1.2.3.1	General

This service operation is used by the ADAES to send a notification about the VAL performance analytics to a previously subscribed VAL Server.

###### 5.11.1.2.3.2	Notifying VAL performance analytics event using Notify_VAL_Performance_Analytics service operation

To notify VAL performance analytics event, the ADAES shall send an HTTP POST request to the VAL server at the notification URI that was provided during the subscription, and the body containing the AppPerfNotif data structure as defined in clause 7.10.1.4.2.3.

Upon receipt of the HTTP POST request, the VAL server shall process the event notification, and in case of success, respond with an HTTP "204 No Content" status code.

If errors occur when processing the HTTP POST request, the VAL server shall send an appropriate HTTP error response as specified in clause 7.10.1.5.

##### 5.11.1.2.4	Unsubscribe_VAL_Performance_Analytics

###### 5.11.1.2.4.1	General

This service operation is used by the VAL server to unsubscribe from the VAL performance analytics events.

###### 5.11.1.2.4.2	Unsubscribing from VAL performance analytics event using Unsubscribe_VAL_Performance_Analytics service operation

To unsubscribe from VAL performance analytics event, the VAL server shall send an HTTP DELETE request to the resource representing the event in the ADAES as specified in clause 7.10.1.2.3.3.2.

Upon receiving the HTTP DELETE request:

1.	the ADAES shall verify the identity of the VAL server and check if the VAL server is authorized to unsubscribe from the VAL performance analytics event associated with the resource URI "{apiRoot}/ss-adae-pa/<apiVersion>/application-performance/{appPerfId}";

2.	if the VAL server is authorized to unsubscribe from the VAL performance analytics event, the ADAES shall delete the resource pointed by the resource URI "{apiRoot}/ss-adae-pa/<apiVersion>/application-performance/{appPerfId}";

3.	if the request is successfully processed, the ADAES shall respond with a "204 No Content" status code; and

If errors occur when processing the request, the ADAES shall send an appropriate HTTP error response as specified in clause 7.10.1.5.

### 5.11.2	SS_ADAE_SlicePerformanceAnalytics API

#### 5.11.2.1	Service Description

##### 5.11.2.1.1	Overview

The SS_ADAE_SlicePerformanceAnalytics API, as defined 3GPP TS 23.436 [38], allows the VAL server via ADAE-S reference point to subscribe to slice specific application performance analytics event to the ADAE Server.

#### 5.11.2.2	Service Operations

##### 5.11.2.2.1	Introduction

The service operation defined for SS_ADAE_SlicePerformanceAnalytics API is shown in the table 5.11.2.2.1-1.

Table 5.11.2.2.1-1: Operations of the SS_ADAE_SlicePerformanceAnalytics API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_Slice_Performance_Analytics | This service operation is used by VAL server to subsribe to the event of the slice-specific application performance analytics. | VAL Server |
| Notify_Slice_Performance_Analytics | This service operation is used by ADAE Server to notify about the slice-specific application performance analytics. | ADAE Server |
| Unsubscribe_Slice_Performance_Analytics | This service operation is used by VAL server to unsubsribe from the event of the slice-specific application performance analytics. | VAL server |

##### 5.11.2.2.2	Subscribe_Slice_Performance_Analytics

###### 5.11.2.2.2.1	General

This service operation is used by the VAL server for slice-specific application performance analytics event subscription to the ADAE Server.

###### 5.11.2.2.2.2	Subscribing to slice-specific application performance analytics event using Subscribe_Slice_Performance_Analytics service operation

To subscribe to slice specific application performance analytics event, the VAL server shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/ss-adae-sspa/<apiVersion>/slice-specific-application-performance" and with a body containing data type SliceAppPerfSub as defined in clause 7.10.2.4.2.2.

Upon receipt of the HTTP POST request, the ADAE Server shall:

1.	verify the identity of the VAL server and determine if the VAL server is authorized to subscribe to the slice-specific application performance analytics event; and

2.	if the VAL server:

a.	is not authorized, the ADAE Server shall respond to the VAL server with an appropriate error status code; or

b.	is authorized, the ADAE Server shall create a new "Individual slice specific application performance event subscription" resource and respond to the VAL server with an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Individual slice specific application performance event subscription" and the response body including the SliceAppPerfSub data structure containing a representation of the created resource as defined in clause 7.10.2.2.

##### 5.11.2.2.3	Notify_Slice_Performance_Analytics

###### 5.11.2.2.3.1	General

This service operation is used by the ADAE Server to send notification to the VAL server with the slice-specific application performance analytics event subscription to the ADAE Server.

###### 5.11.2.2.3.2	Notifying slice-specific application performance analytics event using Notify_Slice_Performance_Analytics service operation

To notify slice-specific application performance analytics event, the ADAE Server shall send an HTTP POST request with a Request-URI according to the pattern "{notiUri}" and with a body containing data type SliceAppPerfNotif as defined in clause 7.10.2.4.2.3.

Upon receipt of the HTTP POST request, the VAL server shall respond to the ADAE Server with:

1.	if the request is successfully processed, a "204 No Content" status code and process the event notification; or

2.	if errors occur when processing the request, an appropriate error response as specified in clause 7.10.2.5.

##### 5.11.2.2.4	Unsubscribe_Slice_Performance_Analytics

###### 5.11.2.2.4.1	General

This service operation is used by the VAL server to unsubscribe from the slice-specific application performance analytics event.

###### 5.11.2.2.4.2	Unsubscribing from slice-specific application performance analytics event using Unsubscribe_Slice_Performance_Analytics service operation

To unsubscribe from slice-specific application performance analytics event, the VAL server shall send an HTTP DELETE request to the resource representing the event in the ADAE Server as specified in clause 7.10.2.2.3.2.

Upon receiving the HTTP DELETE request:

1.	the ADAE Server shall verify the identity of the VAL server and check if the VAL server is authorized to unsubscribe from the slice-specific application performance analytics event associated with the resource URI "{apiRoot}/ss-adae-sspa/<apiVersion>/slice-specific-application-performance/{ssAppPerfId}";

2.	if the VAL server is authorized to unsubscribe from the slice-specific application performance analytics event, the ADAE Server shall delete the resource pointed by the resource URI "{apiRoot}/ss-adae-sspa/<apiVersion>/slice-specific-application-performance/{ssAppPerfId}";

3.	if the request is successfully processed, the ADAE Server shall respond to the VAL server with a "204 No Content" status code; and

4.	if errors occur when processing the request, the ADAE Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.10.2.5.

### 5.11.3	SS_ADAE_Ue2UePerformanceAnalytics API

#### 5.11.3.1	Service Description

##### 5.11.3.1.1	Overview

The SS_ADAE_Ue2UePerformanceAnalytics API, as defined in 3GPP TS 23.436 [38], allows VAL server via ADAE-S reference point to communicate with the ADAE Server for UE-to-UE session performance analytics retrieval including managing UE-to-UE session performance subscriptions.

#### 5.11.3.2	Service Operations

##### 5.11.3.2.1	Introduction

The service operations defined for the SS_ADAE_Ue2UePerformanceAnalytics API are shown in the table 5.11.3.2.1-1.

Table 5.11.3.2.1-1: Operations of the SS_ADAE_Ue2UePerformanceAnalytics API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| UE-to-UE_Performance_Analytics_Subscribe | This service operation is used by the VAL Server to subscribe to the UE-to-UE performance analytics. | VAL Server |
| UE-to-UE_Performance_Analytics_Notify | This service operation is used by the ADAE Server to notify about the UE-to-UE performance analytics. | ADAE Server |
| UE-to-UE_Performance_Analytics_Unsubscribe | This service operation is used by the VAL Server to unsubscribe to the UE-to-UE performance analytics. | VAL Server |

##### 5.11.3.2.2	UE-to-UE_Performance_Analytics_Subscribe

###### 5.11.3.2.2.1	General

This service operation is used by VAL server to subscribe to UE-to-UE performance analytics to the ADAES.

###### 5.11.3.2.2.2	Subscribing for UE-to-UE performance analytics using UE-to-UE_Performance_Analytics_Subscribe

To subscribe to UE-to-UE performance analytics, the VAL server shall send an HTTP POST message to the ADAES with a Request-URI set to "{apiRoot}/ss-adae-uupa/<apiVersion>/ue2ue-session-performance", and the body containing the U2UPerfSub data structure as defined in clause 7.10.3.4.2.2.

Upon reception of the HTTP POST request message, the ADAES shall:

1.	verify the identity of the VAL server and determine if the VAL server is authorized to subscribe to the UE-to-UE session performance analytics event; and

2.	if the VAL server:

a.	is not authorized, the ADAES shall respond to an appropriate error status code; or

b.	is authorized, the ADAES shall create a new "Individual UE-to-UE Session Performance Event Subscription" resource and respond to the VAL server with an HTTP "201 Created" status code, including a Location header field containing the URI for the created resource and the response body including the U2UPerfSub data structure containing a representation of the created resource.

If errors occur when processing the HTTP POST request, the ADAES shall send an appropriate HTTP error response as specified in clause 7.10.3.5.

##### 5.11.3.2.3	UE-to-UE_Performance_Analytics_Notify

###### 5.11.3.2.3.1	General

This service operation is used by the ADAES to send a notification about UE-to-UE performance analytics events to a previously subscribed VAL server.

###### 5.11.3.2.3.2	Notifying UE-to-UE performance analytics events using UE-to-UE_Performance_Analytics_Notify

To notify about UE-to-UE performance analytics events, the ADAES shall send an HTTP POST request to the VAL Server at the notification URI that was provided during subscription, and the body containing the U2UPerfNotif data structure as defined in clause 7.10.3.4.2.3.

Upon receiving the HTTP POST request message, the VAL Server shall process the UE-to-UE performance analytics events notification, and in case of success, respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP POST request, the VAL Server shall send an appropriate HTTP error response as specified in clause 7.10.3.5.

##### 5.11.3.2.3.2	UE-to-UE_Performance_Analytics_Unsubscribe

###### 5.11.3.2.3.2.1	General

This service operation is used by VAL server to unsubscribe from UE-to-UE performance analytics events.

###### 5.11.3.2.3.2.2	Unsubscribing from the UE-to-UE performance analytics events using UE-to-UE_Performance_Analytics_Unsubscribe

To unsubscribe from UE-to-UE session performance analytics events, the VAL server shall send an HTTP DELETE request with "{apiRoot}/ss-adae-uupa/<apiVersion>/ue2ue-session-performance/{u2uPerfId}" as Resource URI, where "{u2uPerfId}" is the identifier of the existing subscription that is to be deleted.

Upon receiving the HTTP DELETE request, if the ADAES authorizes the service consumer and successfully performs the deletion of the indicated subscriptionshall:

1.	verify the identity of the VAL Server and check if the VAL Server is authorised to terminate the targeted "Individual UE-to-UE Session Performance Event Subscription" associated with the resource URI;

2.	if the VAL Server is authorized to unsubscribe from UE-to-UE performance analytics events, the ADAES shall remove the corresponding subscription and respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP DELETE request, the ADAES shall send an appropriate HTTP error response as specified in clause 7.10.3.5.

### 5.11.4	SS_ADAE_LocationAccuracyAnalytics API

#### 5.11.4.1	Service Description

##### 5.11.4.1.1	Overview

The SS_ADAE_LocationAccuracyAnalytics API, as defined 3GPP TS 23.436 [38], allows the VAL server via ADAE-S reference point to subscribe to location accuracy performance analytics event.

#### 5.11.4.2	Service Operations

##### 5.11.4.2.1	Introduction

The service operation defined for SS_ADAE_LocationAccuracyAnalytics API is shown in the table 5.11.4.2.1-1.

Table 5.11.4.2.1-1: Operations of the SS_ADAE_LocationAccuracyAnalytics API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_Location_Accuracy_Analytics | This service operation is used by VAL server to subsribe to the event of the location accuracy performance analytics. | VAL Server |
| Notify_Location_Accuracy_Analytics | This service operation is used by ADAE Server to notify about the location accuracy performance analytics. | ADAE Server |
| Unsubscribe_Location_Accuracy_Analytics | This service operation is used by VAL server to unsubsribe from the event of the location accuracy performance analytics. | VAL server |

##### 5.11.4.2.2	Subscribe_Location_Accuracy_Analytics

###### 5.11.4.2.2.1	General

This service operation is used by the VAL server for location accuracy performance analytics event subscription to the ADAE Server.

###### 5.11.4.2.2.2	Subscribing to location accuracy performance analytics event using Subscribe_Location_Accuracy_Analytics service operation

To subscribe to location accuracy performance analytics event, the VAL server shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/ss-adae-laa/<apiVersion>/location-accuracy" and with a body containing data type LocAccurSub as defined in clause 7.10.4.4.2.2.

Upon receipt of the HTTP POST request, the ADAE Server shall:

1.	verify the identity of the VAL server and determine if the VAL server is authorized to subscribe to the location accuracy performance analytics event; and

2.	if the VAL server:

a.	is not authorized, the ADAE Server shall respond to the VAL server with an appropriate error status code; or

b.	is authorized, the ADAE Server shall create a new "Individual location accuracy event subscription" resource and respond to the VAL server with an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Individual location accuracy event subscription" and the response body including the LocAccurSub data structure containing a representation of the created resource as defined in clause 7.10.4.2.

##### 5.11.4.2.3	Notify_Location_Accuracy_Analytics

###### 5.11.4.2.3.1	General

This service operation is used by the ADAE Server to send notification to the VAL server with the location accuracy performance analytics event subscription to the ADAE Server.

###### 5.11.4.2.3.2	Notifying location accuracy performance analytics event using Notify_Location_Accuracy_Analytics service operation

To notify location accuracy performance analytics event, the ADAE Server shall send an HTTP POST request with a Request-URI according to the pattern "{notiUri}" and with a body containing data type LocAccurNotif as defined in clause 7.10.4.4.2.3.

Upon receipt of the HTTP POST request, the VAL server shall respond to the ADAE Server:

1.	if the request is successfully processed, a "204 No Content" status code and process the event notification; or

2.	if errors occur when processing the request, an appropriate error response as specified in clause 7.10.4.5.

##### 5.11.4.2.4	Unsubscribe_Location_Accuracy_Analytics

###### 5.11.4.2.4.1	General

This service operation is used by the VAL server to unsubscribe from the location accuracy performance analytics event.

###### 5.11.4.2.4.2	Unsubscribing from location accuracy performance analytics event using Unsubscribe_Location_Accuracy_Analytics service operation

To unsubscribe from location accuracy performance analytics event, the VAL server shall send an HTTP DELETE request to the resource representing the event in the ADAE Server as specified in clause 7.10.4.2.3.2.

Upon receiving the HTTP DELETE request:

1.	the ADAE Server shall verify the identity of the VAL server and check if the VAL server is authorized to unsubscribe from the location accuracy performance analytics event associated with the resource URI "{apiRoot}/ss-adae-laa/<apiVersion>/location-accuracy/{locAccId}";

2.	if the VAL server is authorized to unsubscribe from the location accuracy performance analytics event, the ADAE Server shall delete the resource pointed by the resource URI "{apiRoot}/ss-adae-laa/<apiVersion>/location-accuracy/{locAccId}";

3.	if the request is successfully processed, the ADAE Server shall respond to the VAL server with a "204 No Content" status code; and

4.	if errors occur when processing the request, the ADAE Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.10.4.5.

### 5.11.5	SS_ADAE_ServiceApiAnalytics API

#### 5.11.5.1	Service Description

##### 5.11.5.1.1	Overview

The SS_ADAE_ServiceApiAnalytics API, as defined 3GPP TS 23.436 [38], allows the VAL server via ADAE-S reference point to subscribe to service API analytics event to the ADAE Server.

#### 5.11.5.2	Service Operations

##### 5.11.5.2.1	Introduction

The service operation defined for SS_ADAE_ServiceApiAnalytics API is shown in the table 5.11.5.2.1-1.

Table 5.11.5.2.1-1: Operations of the SS_ADAE_ServiceApiAnalytics API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_Service_API_Analytics | This service operation is used by VAL server to subsribe to the event of the service API analytics. | VAL Server |
| Notify_Service_API_Analytics | This service operation is used by ADAE Server to notify about the service API analytics. | ADAE Server |
| Unsubscribe_Service_API_Analytics | This service operation is used by VAL server to unsubsribe from the event of the service API analytics. | VAL Server |

##### 5.11.5.2.2	Subscribe_Service_API_Analytics

###### 5.11.5.2.2.1	General

This service operation is used by the VAL server for service API analytics event subscription to the ADAE Server.

###### 5.11.5.2.2.2	Subscribing to service API analytics event using Subscribe_Service_API_Analytics service operation

To subscribe to service API analytics event, the VAL server shall send an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/ss-adae-sa/<apiVersion>/service-api" and with a body containing data type SrvApiSub as defined in clause 7.10.5.4.2.2.

Upon receipt of the HTTP POST request, the ADAE Server shall:

1.	verify the identity of the VAL server and determine if the VAL server is authorized to subscribe to the service API analytics event; and

2.	if the VAL server:

a.	is not authorized, the ADAE Server shall respond to the VAL server with an appropriate error status code; or

b.	is authorized, the ADAE Server shall create a new "Individual service API event subscription" resource and respond to the VAL server with an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Individual service API event subscription" and the response body including the SrvApiSub data structure containing a representation of the created resource as defined in clause 7.10.5.2.

##### 5.11.5.2.3	Notify_Service_API_Analytics

###### 5.11.5.2.3.1	General

This service operation is used by the ADAE Server to send notification to the VAL server with the service API analytics event subscription to the ADAE Server.

###### 5.11.5.2.3.2	Notifying service API analytics event using Notify_Service_API_Analytics service operation

To notify service API analytics event, the ADAE Server shall send an HTTP POST request with a Request-URI according to the pattern "{notiUri}" and with a body containing data type SrvApiNotif as defined in clause 7.10.5.4.2.3.

Upon receipt of the HTTP POST request, the VAL server shall respond to the ADAE Server with:

1.	if the request is successfully processed, a "204 No Content" status code and process the event notification; or

2.	if errors occur when processing the request, an appropriate error response as specified in clause 7.10.5.5.

##### 5.11.5.2.4	Unsubscribe_Service_API_Analytics

###### 5.11.5.2.4.1	General

This service operation is used by the VAL server to unsubscribe from the service API analytics event.

###### 5.11.5.2.4.2	Unsubscribing from service API analytics event using Unsubscribe_Service_API_Analytics service operation

To unsubscribe from service API analytics event, the VAL server shall send an HTTP DELETE request to the resource representing the event in the ADAE Server as specified in clause 7.10.5.2.3.2.

Upon receiving the HTTP DELETE request:

1.	the ADAE Server shall verify the identity of the VAL server and check if the VAL server is authorized to unsubscribe from the service API analytics event associated with the resource URI "{apiRoot}/ss-adae-sa/<apiVersion>/service-api/{srvApiId}";

2.	if the VAL server is authorized to unsubscribe from the service-api analytics event, the ADAE Server shall delete the resource pointed by the resource URI "{apiRoot}/ss-adae-sa/<apiVersion>/service-api/{srvApiId}";

3.	if the request is successfully processed, the ADAE Server shall respond to the VAL server with a "204 No Content" status code; and

4.	if errors occur when processing the request, the ADAE Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.10.5.5.

### 5.11.6	SS_ADAE_SliceUsagePatternAnalytics API

#### 5.11.6.1	Service Description

##### 5.11.6.1.1	Overview

The SS_ADAE_SliceUsagePatternAnalytics API allows the VAL server and the NSCE server to subscribe to slice usage pattern analytics events and to request slice usage statistics.

#### 5.11.6.2	Service Operations

##### 5.11.6.2.1	Introduction

The service operations defined for the SS_ADAE_SliceUsagePatternAnalytics API is shown in table 5.11.6.2.1-1.

Table 5.11.6.2.1-1: Operations of the SS_ADAE_SliceUsagePatternAnalytics API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_Slice_Usage_Pattern_Analytics | This service operation is used by the VAL Server or the NSCE server to subsribe to the slice usage pattern analytics. | VAL Server, NSCE Server |
| Notify_Slice_Usage_Pattern_Analytics | This service operation is used by the ADAE Server to notify about slice usage pattern analytics. | ADAE Server |
| Unsubscribe_Slice_Usage_Pattern_Analytics | This service operation is used by the VAL Server or the NSCE server to unsubscribe from slice usage pattern analytics. | VAL Server, NSCE Server |
| Get_Slice_Usage_Stats | This service operation is used by the VAL Server or the NSCE server to request slice usage statistics. | VAL Server, NSCE Server |

##### 5.11.6.2.2	Subscribe_Slice_Usage_Pattern_Analytics

###### 5.11.6.2.2.1	General

This service operation is used by the service consumer to subscribe to slice usage pattern analytics to the ADAE server.

###### 5.11.6.2.2.2	Subscribing to slice usage pattern analytics using the Subscribe_Slice_Usage_Pattern_Analytics service operation

To subscribe to slice usage pattern analytics, the service consumer shall send an HTTP POST request to the ADAES with a Request-URI according as "{apiRoot}/ss-adae-sup/<apiVersion>/slice-usage-pattern", with the body containing the data type SUPSub as defined in clause 7.10.6.4.2.2.

Upon receipt of the HTTP POST request, the ADAES shall:

1.	verify the identity of the service consumer and determine if the service consumer is authorized to subscribe to the slice usage pattern analytics event; and

2.	if the service consumer:

a.	is not authorized, the ADAES shall respond with an appropriate error status code; or

b.	is authorized, the ADAES shall create a new "Individual slice usage pattern event subscription" resource and respond to the service consumer with an HTTP "201 Created" status code, including a Location header field containing the URI for the created resource and the response body including the SUPSub data structure containing a representation of the created resource.

If errors occur when processing the HTTP POST request, the ADAES shall send an HTTP error response as specified in clause 7.10.6.5.

##### 5.11.6.2.3	Notify_Slice_Usage_Pattern_Analytics

###### 5.11.6.2.3.1	General

This service operation is used by the ADAE server to send a notification about slice usage pattern analytics to a previously subscribed service consumer.

NOTE:	The steps performed by the ADAE server for data collection, interaction with other entities, and generation of the analytics, which take place between the subscription to slice usage pattern analytics and the sending of the respective notification, are described in 3GPP TS 23.436 [38].

###### 5.11.6.2.3.2	Notifying slice usage pattern analytics using the Notify_Slice_Usage_Pattern_Analytics service operation

To notify about slice usage pattern analytics, the ADAE server shall send an HTTP POST request to the service consumer at the notification URI that was provided during the subscription, with a body containing the data type SUPNotif as defined in clause 7.10.6.4.2.3.

Upon receipt of the HTTP POST request, the service consumer shall process the event notification and in case of success it shall respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP POST request, the service consumer shall send an HTTP error response as specified in clause 7.10.6.5.

##### 5.11.6.2.4	Unsubscribe_Slice_Usage_Pattern_Analytics

###### 5.11.6.2.4.1	General

This service operation is used by the service consumer to unsubscribe from slice usage pattern analytics events.

###### 5.11.6.2.4.2	Unsubscribing from slice usage pattern analytics events using the Unsubscribe_Slice_Usage_Pattern_Analytics service operation

To unsubscribe from slice usage pattern analytics events, the service consumer shall send to the ADAES an HTTP DELETE request with "{apiRoot}/ss-adae-sup/<apiVersion>/slice-usage-pattern/{subscriptionId}" as Resource URI, where "{subscriptionId}" is the identifier of the existing subscription that is to be deleted.

Upon receiving the HTTP DELETE request, if the ADAES authorizes the service consumer and successfully performs the deletion of the indicated subscription, the ADAES shall remove the corresponding subscription and respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP DELETE request, the ADAES shall send an HTTP error response as specified in clause 7.10.6.5.

##### 5.11.6.2.5	Get_Slice_Usage_Stats

###### 5.11.6.2.5.1	General

This service operation is used by the service consumer to retrieve slice usage statistics.

###### 5.11.6.2.5.2	Retrieving slice usage statistics using the Get_Slice_Usage_Stats service operation

To retrieve slice usage statistics, the service consumer shall invoke the Subscribe_Slice_Usage_Pattern_Analytics service operation as described in 5.11.6.2.2.2, requesting immediate and one-time reporting of historical statistics as described in clause 7.10.6.4.2.2.

### 5.11.7	SS_ADAE_EdgeLoadAnalytics API

#### 5.11.7.1	Service Description

##### 5.11.7.1.1	Overview

The SS_ADAE_EdgeLoadAnalytics API corresponding to SS_ ADAE_edge_analytics API defined in 3GPP TS 23.436 [38], allows a VAL server via ADAE-S reference point or EAS to subscribe and unsubscribe from edge load events to receive notifications from the ADAE server for edge load analytics and for getting notified when analytics are derived, or to request and get response from the ADAE server for edge load analytics data.

#### 5.11.7.2	Service Operations

##### 5.11.7.2.1	Introduction

The service operations defined for the SS_ADAE_EdgeLoadAnalytics API are shown in the table 5.11.7.2.1-1.

Table 5.11.7.2.1-1: Operations of the SS_ADAE_EdgeLoadAnalytics API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe_Edge_Load | This service operation is used by analytics consumer (VAL Server, EAS, EES) to subscribe for edge load events from ADAE servers. | VAL Server, EAS, EES |
| Unsubscribe_Edge_Load | This service operation is used by analytics consumer to unsubscribe for edge load events from ADAE servers. | VAL Server, EAS, EES |
| Notify_Edge_Load | This service operation is used by ADAE servers to send the notifications to the analytics consumer. | ADAES |
| Get_Edge_Load_Data | This service operation is used by analytics consumer to retrieve edge load analytics data from ADAE server. | VAL Server, EAS, EES |

##### 5.11.7.2.2	Subscribe_Edge_Load

###### 5.11.7.2.2.1	General

This service operation is used by an analytics consumer (VAL server, EAS, EES) to subscribe to the events for edge load analytics and for getting notified when analytics are derived.

###### 5.11.7.2.2.2	Subscribing to edge load events using Subscribe_Edge_Load service operation

To subscribe to edge load events, the analytics consumer (VAL server, EAS, EES) shall send an HTTP POST message to the ADAE server. The body of the HTTP POST message shall include EdgeSub data type as specified in clause 7.10.7.4.2.2.

Upon receiving the above described HTTP POST message, the ADAE Server shall:

1.	verify the identity of the analytics consumer and check if the Consumer is authorized to subscribe to the events mentioned in the HTTP POST message;

2.	if the analytics consumer is authorized to subscribe to the events, the ADAE server shall:

a.	verify the provided identifier(s) in the request;

b.	upon successful verification of the provided identifier(s) and successful processing of the request, create a new resource as specified in clause 7.10.7.2.1;

c.	return the created resource representation and the created Resource URI in the response message; and

d.	if the ADAE server is unable to satisfy the request, the ADAE server shall respond to the analytics consumer with an appropriate error status code as defined in clause 7.10.7.5.

##### 5.11.7.2.3	Notify_Edge_Load

###### 5.11.7.2.3.1	General

This service operation is used by the ADAE server to send notifications to the analytics consumer.

###### 5.11.7.2.3.2	Notifying edge load events using Notify_Edge_Load service operation

To notify the edge load events, the ADAE server shall send an HTTP POST message using the Notification Destination URI received in the subscription request. The body of the HTTP POST message shall include an Event Notification and Resource URI.

Upon receiving the HTTP POST request message, the VAL Server shall:

1.	process the edge load analytics events notification;

2.	upon success, respond to the ADAE Server with a "204 No Content" status code; and

3.	if errors occur when processing the request, the VAL Server shall respond to the ADAE Server with an appropriate error response as specified in clause 7.10.3.5.

##### 5.11.7.2.4	Unsubscribe_Edge_Load

###### 5.11.7.2.4.1	General

This service operation is used by an analytics consumer to unsubscribe from the edge load events for edge load analytics.

###### 5.11.7.2.4.2	Unsubscribing from edge load events using Unsubscribe_Edge_Load service operation

To unsubscribe for edge load events, the analytics consumer shall send an HTTP DELETE message to the resource representing the event in the ADAE server as specified in clause 7.10.7.2.3.3.2.

Upon receiving the HTTP DELETE message, the ADAE server shall:

1.	verify the identity of the analytics consumer and check if the analytics consumer is authorized to Unsubscribe from the event associated with the Resource URI; and

2.	if the analytics consumer is authorized to unsubscribe from the events, the ADAE server shall delete the resource pointed by the Resource URI.

##### 5.11.7.2.5	Get_Edge_Load_Data

###### 5.11.7.2.5.1	General

This service operation is used by the service consumer to retrieve edge load analytics data.

###### 5.11.7.2.5.2	Retrieving edge load analytics data using the Get_Edge_Load_Data service operation

To retrieve edge load analytics data, the service consumer shall invoke the Subscribe_Edge_Load service operation as described in 5.11.7.2.2.2, requesting immediate and one-time reporting of edge load analytics data as described in clause 7.10.7.4.2.2.

### 5.11.8	SS_AADRF_DataManagement API

#### 5.11.8.1	Service Description

##### 5.11.8.1.1	Overview

The SS_AADRF_DataManagement API corresponding to SS_AADRF_Data_Collection, SS_ AADRF_Historical_serviceAPI_logs, SS_ AADRF_NetworkSlice_data, SS_AADRF_EdgeData_Collection, SS_AADRF_Location_Accuracy, SS_AADRF_ServerToServer_Analytics and SS_AADRF_UE_RAT_Connectivity_Analytics services as defined in 3GPP TS 23.436 [38], is provided by the Application layer - Analytical Data Repository Function (A-ADRF) and allows ADAE server to retrieve the offline analytics/data from the A-ADRF.

#### 5.11.8.2	Service Operations

##### 5.11.8.2.1	Introduction

The following services are specified for the A-ADRF.

Table 5.11.8.2.1-1: Services provided by A-ADRF

| Service Name | Description | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- | --- |
| SS_AADRF_DataManagement (NOTE) | This service enables the NF service consumers to subscribe to/unsubscribe from notifications for offline analytics/data from the A-ADRF and request data strorage. | Data_Storage | Request/Response | ADAE server, A-DCCF |
|  |  | Subscribe | Subscribe / Notify | ADAE server |
|  |  | Unsubscribe |  |  |
|  |  | Notify |  |  |
| NOTE: This service implements the AADRF_Historical_serviceAPI_logs, SS_ AADRF_NetworkSlice_data, SS_AADRF_Location_Accuracy, SS_AADRF_ServerToServer_Analytics and SS_AADRF_UE_RAT_Connectivity_Analytics services as specified in 3GPP TS 23.436 [38] by using immediate and one-time reporting requirement. |  |  |  |  |

##### 5.11.8.2.2	SS_AADRF_DataManagement_Subscribe service operation

###### 5.11.8.2.2.1	General

This service operation is used by an ADAE server to subscribe the offline analytics/data from the A-ADRF.

###### 5.11.8.2.2.2	Subscribing to the offline analytics/data using the SS_AADRF_DataManagement_Subscribe service operation

In order to subscribe to the offline analytics/data from the A-ADRF, the consumer shall send an HTTP POST message to the A-ADRF, with "{apiRoot}/ss-aadrf-datamanagement/<apiVersion>/subscriptions" as Resource URI, to create a subscription for an "Individual A-ADRF Data Management Subscription" according to the DataManageSub data type contained in the message body as defined in clause 7.10.8.5.2.2.

Upon receiving the HTTP POST message as described above, the A-ADRF shall:

1.	verify the identity of the consumer and check if the consumer is authorized to retrieve the requested analytics/data;

2.	if the consumer is authorized, the A-ADRF shall create a new "Individual A-ADRF Data Management Subscription" resource and respond to the consumer with an HTTP "201 Created" status code, including a Location header field containing the URI for the created "Individual A-ADRF Data Management Subscription" resource and the response body including the DataManageSub data structure containing a representation of the created resource as defined in clause 7.10.8.5.2.2;

and

3.	if the A-ADRF is unable to satisfy the request, the A-ADRF shall respond to the consumer with an appropriate error status code as specified in clause 7.10.8.5.

##### 5.11.8.2.3	SS_AADRF_DataManagement_Notify service operation

###### 5.11.8.2.3.1	General

This service operation is used by an A-ADRF to send a notification about the offline analytics/data to a previously subscribed service consumer.

###### 5.11.8.2.3.2	Notifying the offline analytics/data using the SS_AADRF_DataManagement_Notify service operation

To notify about offline analytics/data, the A-ADRF shall send an HTTP POST request to the service consumer at the notification URI that was provided during the subscription, with a body containing the data type DataManageNotification as defined in clause 7.10.8.5.2.6.

Upon receipt of the HTTP POST request, the service consumer shall process the event notification and in case of success it shall respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP POST request, the service consumer shall send an HTTP error response as specified in clause 7.10.8.6.

If the service consumer determines the received HTTP POST request needs to be redirected, the service consumer shall send an HTTP redirect response as defined in clause 7.10.8.4.2.

##### 5.11.8.2.4	SS_AADRF_DataManagement_Unsubscribe

###### 5.11.8.2.4.1	General

This service operation is used by the service consumer to unsubscribe from the offline analytics/data events.

##### 5.11.8.2.5	Data_Storage service operation

###### 5.11.8.2.5.1	General

This service operation is used by a service consumer to store data at A-ADRF.

###### 5.11.8.2.5.2	Requesting to store data using the Data_Storage service operation

In order to store data at A-ADRF, the consumer shall send an HTTP POST request (i.e., custom operation "Store") to the A-ADRF as specified in clause 7.10.8.3.2. The request body shall include the DataStoreReq data structure defined in clause 7.10.8.5.2.9.

Editor's note:	Whether a custom operation is appropriate to implement this procedure is FFS.

Upon receiving the HTTP POST message as described above, the A-ADRF shall:

1.	verify the identity of the consumer and check if the consumer is authorized to store the data;

2.	if the consumer is authorized, the A-ADRF shall process the request:

a.	if "subDataStore" attribute is provided within DataStoreReq data structure as described in clause 8.10.2.1 of 3GPP TS 29.436 [38];

b.	if "dirStorage" attribute is provided within DataStoreReq data structure as described in clause 8.10.2.2 of 3GPP TS 29.436 [38]; and

c.	respond to the consumer with an HTTP "200 OK" status code with DataStoreResp data structure defined in clause 7.10.8.5.2.15.

and

3.	if the A-ADRF is unable to satisfy the request, the A-ADRF shall respond to the consumer with an appropriate error status code as specified in clause 7.10.8.5.

### 5.11.9	SS_ADAE_LocationRelatedUeGroupAnalytics API

#### 5.11.9.1	Service Description

The SS_ADAE_LocationRelatedUeGroupAnalytics API, as defined 3GPP TS 23.434 [2], enables a service consumer to:

-	create/update/delete a Location-Related UE Group Analytics Subscription; and

-	receive Location-Related UE Group Analytics Subscription related event(s) notification.

#### 5.11.9.2	Service Operations

##### 5.11.9.2.1	Introduction

The service operations defined for the SS_ADAE_LocationRelatedUeGroupAnalytics are shown in the table 5.11.9.2.1-1.

Table 5.11.9.2.1-1: Operations of the SS_ADAE_LocationRelatedUeGroupAnalytics

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe | This service operation is used by the VAL Server to create/update/delete Location-Related UE Group Analytics Subscription. | LM Server |
| Notify | This service operation is used by the ADAE Server to notify about the Location-Related UE Group Analytics Subscription event(s). | ADAE Server |

##### 5.11.9.2.2	Subscribe

###### 5.11.9.2.2.1	General

This service operation is used by a service consumer to request the creation of a Location-Related UE Group Analytics Subscription at the ADAE Server.

The following procedures are supported by the "Subscribe" service operation:

-	Location-Related UE Group Analytics Subscription Creation.

-	Location-Related UE Group Analytics Subscription Update.

-	Location-Related UE Group Analytics Subscription Deletion.

###### 5.11.9.2.2.2	Location-Related UE Group Analytics Subscription Creation

To subscribe to location-related UE group analytics, the VAL server shall send an HTTP POST message to the ADAES with a Request-URI set to "{apiRoot}/ss-adae-lruga/<apiVersion>/subscriptions", and the body containing the LocRelUeGroupSub data structure as defined in clause 7.10.9.4.2.2.

Upon reception of the HTTP POST request message, the ADAES shall:

1.	verify the identity of the VAL server and determine if the VAL server is authorized to subscribe to location-related UE group analytics event; and

2.	if the VAL server:

a.	is not authorized, the ADAES shall respond to an appropriate error status code; or

b.	is authorized, the ADAES shall create a new "Individual Location-Related UE Group Analytics Subscription" resource and respond to the VAL server with an HTTP "201 Created" status code, including a Location header field containing the URI for the created resource and the response body including the LocRelUeGroupSub data structure containing a representation of the created resource.

If errors occur when processing the HTTP POST request, the ADAES shall send an appropriate HTTP error response as specified in clause 7.10.3.5.

###### 5.11.9.2.2.3	Location-Related UE Group Analytics Subscription Deletion

To unsubscribe from location-related UE group analytics events, the VAL server shall send an HTTP DELETE request with "{apiRoot}/ss-adae-lruga/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI, where "{subscriptionId}" is the identifier of the existing subscription that is to be deleted.

Upon receiving the HTTP DELETE request, if the ADAES authorizes the service consumer and successfully performs the deletion of the indicated subscriptionshall:

1.	verify the identity of the VAL Server and check if the VAL Server is authorised to terminate the targeted "Individual Location-Related UE Group Analytics Subscription" associated with the resource URI;

2.	if the VAL Server is authorized to unsubscribe from location-related UE group analytics events, the ADAES shall remove the corresponding subscription and respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP DELETE request, the ADAES shall send an appropriate HTTP error response as specified in clause 7.10.3.5.

###### 5.11.9.2.2.4	Location-Related UE Group Analytics Subscription Update

In order to request the update of an existing location-related UE group analytics subscription, the service consumer shall send an HTTP PUT/PATCH request to the ADAE Server, targeting the corresponding "Individual Location-Related UE Group Analytics Subscription" resource, with the request body including either:

-	the updated representation of the resource within the LocRelUeGroupSub data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the LocRelUeGroupSubPatch data structure, in case the HTTP PATCH method is used.

Upon success, the ADAES Server shall update the targeted "Individual Location-Related UE Group Analytics Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Location-Related UE Group Analytics Subscription" resource within the LocRelUeGroupSub data structure; or

-	an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 7.10.9.7.

##### 5.11.9.2.3	Notify

###### 5.11.9.2.3.1	General

This service operation is used by the ADAE Server to notify on Location-Related UE Group Analytics event(s).

The following procedures are supported by the "Notify" service operation:

-	Location-Related UE Group Analytics Event Notification.

###### 5.11.9.2.3.2	Location-Related UE Group Analytics Event Notification

To notify about location-related UE group analytics events, the ADAES shall send an HTTP POST request to the service consumer at the notification URI that was provided during subscription, and the body containing the LocRelUeGroupNotif data structure as defined in clause 7.10.9.4.2.3.

Upon receiving the HTTP POST request message, the service consumer shall process the location-related UE group analytics events notification, and in case of success, respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP POST request, the service consumer shall send an appropriate HTTP error response as specified in clause 7.10.3.5.

### 5.11.10	SS_ADAE_CollisionDetectionAnalytics API

#### 5.11.10.1	Service Description

The SS_ADAE_CollisionDetectionAnalytics API, as defined 3GPP TS 23.434 [2], enables a service consumer to:

-	create/update/delete a Collision Detection Analytics Subscription; and

-	receive Collision Detection Analytics Subscription related event(s) notification.

#### 5.11.10.2	Service Operations

##### 5.11.10.2.1	Introduction

The service operations defined for the SS_ADAE_CollisionDetectionAnalytics are shown in the table 5.11.10.2.1-1.

Table 5.11.10.2.1-1: Operations of the SS_ADAE_CollisionDetectionAnalytics

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Subscribe | This service operation is used by the service consumer to create/update/delete Collision Detection Analytics Subscription. | VAL server, LM Server, UAE server, UASS |
| Notify | This service operation is used by the ADAE Server to notify about the Collision Detection Analytics event(s). | ADAE Server |

##### 5.11.10.2.2	Subscribe

###### 5.11.10.2.2.1	General

This service operation is used by a service consumer to request the creation of a Collision Detection Analytics Subscription at the ADAE Server.

The following procedures are supported by the "Subscribe" service operation:

-	Collision Detection Analytics Subscription Creation.

-	Collision Detection Analytics Subscription Update.

-	Collision Detection Analytics Subscription Deletion.

###### 5.11.10.2.2.2	Collision Detection Analytics Subscription Creation

To subscribe to collision detection analytics, the service consumer shall send an HTTP POST message to the ADAES with a Request-URI set to "{apiRoot}/ss-adae-cda/<apiVersion>/subscriptions", and the body containing the CollisionDetectionSub data structure as defined in clause 7.10.10.4.2.2.

Upon reception of the HTTP POST request message, the ADAES shall:

1.	verify the identity of the service consumer and determine if the service consumer is authorized to subscribe to collision detection analytics event; and

2.	if the service consumer:

a.	is not authorized, the ADAES shall respond to an appropriate error status code; or

b.	is authorized, the ADAES shall create a new "Individual Collision detection analytics Subscription" resource and respond to the service consumer with an HTTP "201 Created" status code, including a Location header field containing the URI for the created resource and the response body including the CollisionDetectionSub data structure containing a representation of the created resource.

If errors occur when processing the HTTP POST request, the ADAES shall send an appropriate HTTP error response as specified in clause 7.10.10.5.

###### 5.11.10.2.2.3	Collision Detection Analytics Subscription Deletion

To unsubscribe from Collision Detection Analytics Event(s), the service consumer shall send an HTTP DELETE request with "{apiRoot}/ss-adae-cda/<apiVersion>/subscriptions/{subscriptionId}" as Resource URI, where "{subscriptionId}" is the identifier of the existing subscription that is to be deleted.

Upon receiving the HTTP DELETE request, if the ADAES authorizes the service consumer and successfully performs the deletion of the indicated subscriptionshall:

1.	verify the identity of the service consumer and check if service consumer is authorised to terminate the targeted "Individual Collision Detection Analytics Subscription" associated with the resource URI;

2.	if the service consumer is authorized to unsubscribe from collision detection analytics events, the ADAES shall remove the corresponding subscription and respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP DELETE request, the ADAES shall send an appropriate HTTP error response as specified in clause 7.10.10.5.

###### 5.11.10.2.2.4	Collision Detection Analytics Subscription Update

In order to request the update of an existing collision detection analytics subscription, the service consumer shall send an HTTP PUT/PATCH request to the ADAE Server, targeting the corresponding "Individual Collision Detection Analytics Subscription" resource, with the request body including either:

-	the updated representation of the resource within the CollisionDetectionSub data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the CollisionDetectionSubPatch data structure, in case the HTTP PATCH method is used.

Upon success, the ADAES Server shall update the targeted "Individual Collision detection analytics Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Collision detection analytics Subscription" resource within the CollisionDetectionSub data structure; or

-	an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 7.10.10.7.

##### 5.11.10.2.3	Notify

###### 5.11.10.2.3.1	General

This service operation is used by the ADAE Server to notify on Collision Detection Analytics event(s).

The following procedures are supported by the "Notify" service operation:

-	Collision Detection Analytics Event Notification.

###### 5.11.10.2.3.2	Collision Detection Analytics Event Notification

To notify about Collision Detection Analytics Event(s), the ADAES shall send an HTTP POST request to the service consumer at the notification URI that was provided during subscription, and the body containing the CollisionDetectionNotif data structure as defined in clause 7.10.10.4.2.3.

Upon receiving the HTTP POST request message, the service consumer shall process the collision detection analytics events notification, and in case of success, respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP POST request, the service consumer shall send an appropriate HTTP error response as specified in clause 7.10.10.5.

### 5.11.11	SS_ADAE_AIMLMemberCapabilityAnalytics API

#### 5.11.11.1	Service Description

The SS_ADAE_AIMLMemberCapabilityAnalytics service exposed by the ADAE Server enables a service consumer to:

-	create/update/delete AIML Member Capability Analytics Subscription; and

-	receive AIML Member Capability Analytics Notifications.

#### 5.11.11.2	Service Operations

##### 5.11.11.2.1	Introduction

The service operations defined for the SS_ADAE_AIMLMemberCapabilityAnalytics service are shown in table 5.11.11.2.1-1.

Table 5.11.11.2.1-1: SS_ADAE_AIMLMemberCapabilityAnalytics Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| SS_ADAE_AIML_MemberCapabilityAnalytics_Subscribe | This service operation enables a service consumer to create a AIML member capability analytics Subscription. | e.g. VAL Server, AIMLE Server |
| SS_ADAE_AIML_MemberCapabilityAnalytics_Update | This service operation enables a service consumer to update an existing AIML member capability analytics Subscription. | e.g. VAL Server, AIMLE Server |
| SS_ADAE_AIML_MemberCapabilityAnalytics_Unsubscribe | This service operation enables a service consumer to delete an existing AIML member capability analytics Subscription. | e.g. VAL Server, AIMLE Server |
| SS_ADAE_AIML_MemberCapabilityAnalytics_Notify | This service operation enables a service consumer to receive AIML member capability analytics notifications. | ADAE Server |

##### 5.11.11.2.2	SS_ADAE_AIML_MemberCapabilityAnalytics_Subscribe

###### 5.11.11.2.2.1	General

This service operation is used by a service consumer to request the creation of AIML Member Capability Analytics Subscription at the ADAE Server.

The following procedures are supported by the "SS_ADAE_AIML_MemberCapabilityAnalytics_Subscribe" service operation:

-	AIML Member Capability Analytics Subscription Creation.

This service operation is used by the service consumer for AIML Member Capability Analytics Subscription to the ADAE Server.

###### 5.11.11.2.2.2	AIML Member Capability Analytics Subscription Creation

Figure 5.11.11.2.2.2-1 depicts a scenario where a service consumer sends a request to the ADAE Server to request the creation of AIML Member Capability Analytics Subscription (see also clause 8.16 of 3GPP°TS°23.436°[38]).

Figure 5.11.11.2.2.2-1: Procedure for AIML Member Capability Analytics Subscription Creation

1.	In order to subscribe to AIML Member Capability Analytics reporting, the service consumer shall send an HTTP POST request to the ADAE Server targeting the URI of the "AIML Member Capability Analytics Subscriptions" collection resource, with the request body including the AppAimlMemCapSub data structure.

2a.	Upon success, the ADAE Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual AIML Member Capability Analytics Subscription" resource within the AppAimlMemCapSub data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.10.11.6.

##### 5.11.11.2.3	SS_ADAE_AIML_MemberCapabilityAnalytics_Unsubscribe

###### 5.11.11.2.3.1	General

This service operation is used by a service consumer to request the deletion of AIML Member Capability Analytics Subscription at the ADAE Server.

The following procedures are supported by the "SS_ADAE_AIML_MemberCapabilityAnalytics_Unsubscribe" service operation:

-	AIML Member Capability Analytics Subscription Deletion.

###### 5.11.11.2.3.2	AIML Member Capability Analytics Subscription Deletion

Figure 5.5.2.3.2-1 depicts a scenario where a service consumer sends a request to the ADAE Server to delete an existing AIML Member Capability Analytics Subscription (see also clause 8.16 of 3GPP°TS°23.436°[38])

Figure 5.5.2.3.2-1: Procedure for AIML Member Capability Analytics Subscription Deletion

1.	In order to request the deletion of an existing AIML Member Capability Analytics Subscription, the service consumer shall send an HTTP DELETE request to the ADAE Server targeting the URI of the corresponding "Individual AIML Member Capability Analytics Subscription" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the ADAE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 7.10.11.6.

##### 5.11.11.2.4	SS_ADAE_AIML_MemberCapabilityAnalytics_Notify

###### 5.11.11.2.4.1	General

This service operation is used by a ADAE Server to notify a previously subscribed service consumer on:

-	AIML Member Capability Analytics report(s).

The following procedures are supported by the "SS_ADAE_AIML_MemberCapabilityAnalytics_Notify" service operation:

-	AIML Member Capability Analytics Notification.

###### 5.11.11.2.4.2	AIML Member Capability Analytics Notification

Figure 5.11.11.2.3.2-1 depicts a scenario where the ADAE Server sends a request to notify a previously subscribed service consumer on AIML Member Capability Analytics report(s) (see also clause 8.16 of 3GPP°TS°23.436°[38])

Figure 5.5.2.5.4-1: Procedure for AIML Member Capability Analytics Notification

1.	In order to notify a previously subscribed service consumer on AIML Member Capability Analytics report(s), the ADAE Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation of the corresponding AIML Member Capability Analytics Subscription using the procedures defined in clauses 5.11.11.2.2, and the request body including the AppAimlMemCapNotif data structure.

2a.	Upon success, the service consumer shall respond to the ADAE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.10.11.6.

##### 5.11.11.2.5	SS_ADAE_AIML_MemberCapabilityAnalytics_Update

###### 5.11.11.2.5.1	General

This service operation is used by a service consumer to request the update of an ADAE_AIML_MemberCapabilityAnalytics Subscription at the ADAE Server.

The following procedures are supported by the "SS_ADAE_AIML_MemberCapabilityAnalytics_Update" service operation:

-	AIML Member Capability Analytics Subscription Update.

###### 5.11.11.2.5.2	AIML Member Capability Analytics Subscription Update

Figure 5.5.2.3.2-1 depicts a scenario where a service consumer sends a request to the ADAE Server to request the update of an existing ADAE_AIML_MemberCapabilityAnalytics Subscription (see also clause 8.16 of 3GPP°TS°23.436°[38]).

Figure 5.11.11.2.5.2-1: Procedure for AIML Member Capability Analytics Subscription Update

1.	In order to request the update of an existing AIML Member Capability Analytics reporting, the service consumer shall send an HTTP PUT/PATCH request to the ADAE Server, targeting the URI of the corresponding "Individual AIML Member Capability Analytics Subscription" resource, with the request body including either:

-	the updated representation of the resource within the AppAimlMemCapSub data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the AppAimlMemCapSubPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the ADAE Server shall update the targeted "Individual AIML Member Capability Analytics Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual AIML Member Capability Analytics Subscription" resource within the AppAimlMemCapSub data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 7.10.11.6.

### 5.11.12	SS_ADCCF_DataCollection API

#### 5.11.12.1	Service Description

The SS_ADCCF_DataCollection API, as defined 3GPP TS 23.434 [2], enables a service consumer to:

-	create a Data Collection Subscription; and

-	receive Data Collection Subscription related event(s) notification.

#### 5.11.12.2	Service Operations

##### 5.11.12.2.1	Introduction

The service operations defined for the SS_ADCCF_DataCollection are shown in the table 5.11.12.2.1-1.

Table 5.11.12.2.1-1: Operations of the SS_ADCCF_DataCollection

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Request | This service operation is used by the service consumer to request data collection. | e.g., ADAE Server |
| Subscribe | This service operation is used by the service consumer to create/delete Data Collection Subscription. | e.g., ADAE Server |
| Notify | This service operation is used by the A-DCCF to notify about the Data Collection Subscription event(s). | A-DCCF |

##### 5.11.12.2.2	Subscribe

###### 5.11.12.2.2.1	General

This service operation is used by a service consumer to request the creation of a Data Collection Subscription at the A-DCCF.

The following procedures are supported by the "Subscribe" service operation:

-	Data Collection Subscription Creation.

-	Data Collection Subscription Deletion.

###### 5.11.12.2.2.2	Data Collection Subscription Creation

To subscribe to data collection, the service consumer shall send an HTTP POST message targeting the "Data Collection Subscriptions" resource, and the body containing the DataCollectionSub data structure as defined in clause 7.10.13.4.2.2.

Editor’s note:	The support of Request service operation in FFS.

Upon reception of the HTTP POST request message, the A-DCCF shall:

1.	verify the identity of the service consumer and determine if the service consumer is authorized to subscribe to data collection event; and

2.	if the service consumer authorized, the A-DCCF shall create a new "Individual Data Collection Subscription" resource and respond to the service consumer with an HTTP "201 Created" status code, including a Location header field containing the URI for the created resource and the response body including the DataCollectionSub data structure containing a representation of the created resource.

If errors occur when processing the HTTP POST request, the A-DCCF shall send an appropriate HTTP error response as specified in clause 7.10.13.5.

###### 5.11.12.2.2.3	Data Collection Subscription Deletion

To unsubscribe from data collection events, the service consumer shall send an HTTP DELETE request targeting the "Individual Data Collection Subscription" resource.

Upon receiving the HTTP DELETE request, if the A-DCCF authorizes the service consumer and successfully performs the deletion of the indicated subscription shall:

1.	verify the identity of the service consumer and check if the service consumer is authorised to terminate the targeted "Individual Data Collection Subscription" associated with the resource URI;

2.	if the service consumer is authorized to unsubscribe from data collection events, the A-DCCF shall remove the corresponding subscription and respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP DELETE request, the A-DCCF shall send an appropriate HTTP error response as specified in clause 7.10.13.5.

##### 5.11.12.2.3	Notify

###### 5.11.12.2.3.1	General

This service operation is used by the A-DCCF to notify on Data collection event(s).

The following procedures are supported by the "Notify" service operation:

-	Data collection Event Notification.

###### 5.11.12.2.3.2	Data collection Event Notification

To notify about data collection events, the A-DCCF shall send an HTTP POST request to the service consumer at the notification URI that was provided during subscription, and the body containing the DataCollectionNotif data structure as defined in clause 7.10.13.6.2.3.

Upon receiving the HTTP POST request message, the service consumer shall process the data collection events notification, and in case of success, respond with the HTTP "204 No Content" status code.

If errors occur when processing the HTTP POST request, the service consumer shall send an appropriate HTTP error response as specified in clause 7.10.13.5.

### 5.11.13	SS_ADAE_ServerToServerPerformanceAnalytics API

#### 5.11.13.1	Service Description

The SS_ADAE_ServerToServerPerformanceAnalytics API enables a service consumer to:

-	create/update/delete a Server to Server Performance Analytics Subscription (see also clause 8.13 of 3GPP TS 23.436 [38]); and

-	receive Server to Server Performance Analytics Subscription related event(s) reporting (see also clause 8.13 of 3GPP TS 23.436 [38]).

#### 5.11.13.2	Service Operations

##### 5.11.13.2.1	Introduction

The service operations defined for the SS_ADAE_ServerToServerPerformanceAnalytics API are shown in the table 5.11.13.2.1-1.

Table 5.11.13.2.1-1: SS_ADAE_ServerToServerPerformanceAnalytics Service Operations

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| SS_ADAE_ServerToServerPerformanceAnalytics_Subscribe | This service operation is used by the service consumer to create/update/delete Server to Server Performance Analytics Subscription. | VAL server, EES |
| SS_ADAE_ServerToServerPerformanceAnalytics_Notify | This service operation is used by the ADAE Server to notify about the subscribed Server to Server Performance Analytics event(s). | ADAE Server |

##### 5.11.13.2.2	Subscribe

###### 5.11.13.2.2.1	General

This service operation is used by a service consumer to request the creation of a Server to Server Performance Analytics Subscription at the ADAE Server.

The following procedures are supported by the "SS_ADAE_ServerToServerPerformanceAnalytics_Subscribe" service operation:

-	Server to Server Performance Analytics Subscription Creation.

-	Server to Server Performance Analytics Subscription Update.

-	Server to Server Performance Analytics Subscription Deletion.

###### 5.11.13.2.2.2	Server to Server Performance Analytics Subscription Creation

To subscribe to Server to Server Performance analytics, the service consumer shall send an HTTP POST message to the ADAES targeting the URI of the "Server to Server Performance analytics Subscriptions" collection resource, with the request body containing the S2SPerfSubsc data structure as defined in clause 7.10.14.6.2.2.

Upon reception of the HTTP POST request message, the ADAE Server shall:

1.	verify the identity of the service consumer and determine if the service consumer is authorized to subscribe to Server to Server Performance Analytics event(s) reporting; and

2.	if the service consumer is authorized and upon successful processing of the request, the ADAES shall create a new "Individual Server to Server Performance analytics Subscription" resource and respond to the service consumer with an HTTP "201 Created" status code, including a Location header field containing the URI for the created resource and the response body including the S2SPerfSubsc data structure containing a representation of the created resource.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.10.14.7.

###### 5.11.13.2.2.3	Server to Server Performance Analytics Subscription Update

In order to request the update of an existing Server to Server Performance analytics subscription, the service consumer shall send an HTTP PUT/PATCH request to the ADAE Server, targeting the corresponding "Individual Server to Server Performance Analytics Subscription" resource, with the request body including either:

-	the updated representation of the resource within the S2SPerfSubsc data structure as defined in clause 7.10.14.6.2.2, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the S2SPerfSubscPatch data structure as defined in clause 7.10.14.6.2.4, in case the HTTP PATCH method is used.

Upon reception of the HTTP PUT/PATCH request message, the ADAE Server shall:

1.	verify the identity of the service consumer and determine if the service consumer is authorized to updated the targeted "Individual Server to Server Performance Analytics Subscription" resource; and

2.	if the service consumer is authorized and upon successful processing of the request, the ADAES Server shall update the targeted "Individual Server to Server Performance analytics Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Server to Server Performance analytics Subscription" resource within the S2SPerfSubsc data structure; or

-	an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 7.10.14.7.

###### 5.11.13.2.2.4	Server to Server Performance Analytics Subscription Deletion

To unsubscribe from Server to Server Performance Analytics event(s) reporting, the service consumer shall send an HTTP DELETE request targeting the URI of the corresponding "Individual Server to Server Performance Analytics Subscription" resource.

Upon reception of the HTTP DELETE request, the ADAE Server shall:

1.	verify the identity of the service consumer and check if service consumer is authorised to terminate the targeted "Individual Server to Server Performance Analytics Subscription" resource; and

2.	if the service consumer is authorized, the ADAE Server shall delete the corresponding subscription and respond with an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 7.10.14.7.

##### 5.11.13.2.3	Notify

###### 5.11.13.2.3.1	General

This service operation is used by the ADAE Server to notify Server to Server Performance Analytics event(s).

The following procedures are supported by the "SS_ADAE_ServerToServerPerformanceAnalytics_Notify" service operation:

-	Server to Server Performance Analytics Event Notification.

###### 5.11.13.2.3.2	Server to Server Performance Analytics Event Notification

To notify on Server to Server Performance Analytics Event(s), the ADAE Server shall send an HTTP POST request to the service consumer targeting the notification URI that was received during the creation/update of the corresponding subscription as defined in clauses 5.11.13.2.2.2 and 5.11.13.2.2.3, with the request body containing the S2SPerfNotif data structure as defined in clause 7.10.14.6.2.3.

Upon reception of the HTTP POST request message, the service consumer shall process the Server to Server Performance Analytics Notification, and in case of success, respond with an HTTP "204 No Content" status code.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.10.14.7.

### 5.11.14	SS_ADAE_UeRatConnectivityAnalytics API

#### 5.11.14.1	Service Description

The SS_ADAE_UeRatConnectivityAnalytics service exposed by the ADAE Server enables a service consumer to:

-	create/update/delete UE RAT connectivity Analytics Subscription; and

-	receive UE RAT Connectivity Analytics Notifications.

#### 5.11.14.2	Service Operations

##### 5.11.14.2.1	Introduction

The service operations defined for the SS_ADAE_UeRatConnnectivityAnalytics service are shown in table 5.11.14.2.1-1.

Table 5.11.14.2.1-1: SS_ADAE_UeRatConnectivityAnalytics Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| SS_ADAE_UeRatConnectivityAnalytics_Subscribe | This service operation enables a service consumer to create a UE RAT connectivity analytics Subscription. | e.g. VAL Server |
| SS_ADAE_UeRatConnectivityAnalytics_Subscription_Update | This service operation enables a service consumer to update an existing UE RAT connectivity analytics Subscription. | e.g. VAL Server |
| SS_ADAE_UeRatConnectivityAnalytics_Unsubscribe | This service operation enables a service consumer to delete an existing UE RAT connectivity analytics Subscription. | e.g. VAL Server |
| SS_ADAE_UeRatConnectivityAnalytics_Notify | This service operation enables a service consumer to receive UE RAT connectivity analytics notifications. | ADAE Server |

##### 5.11.14.2.2	SS_ADAE_UeRatConnectivityAnalytics_Subscribe

###### 5.11.14.2.2.1	General

This service operation is used by a service consumer to request the creation of UE RAT Connectivity Analytics Subscription at the ADAE Server.

The following procedures are supported by the "SS_ADAE_UeRatConnectivityAnalytics_Subscribe" service operation:

-	UE RAT Connectivity Analytics Subscription Creation.

###### 5.11.14.2.2.2	UE RAT Connectivity Analytics Subscription Creation

Figure 5.11.14.2.2.2-1 depicts a scenario where a service consumer sends a request to the ADAE Server to request the creation of UE RAT Connectivity Analytics Subscription (see also clause 10.2.3 of 3GPP°TS°23.436°[38]).

Figure 5.11.14.2.2.2-1: Procedure for UE RAT Connectivity Analytics Subscription Creation

1.	In order to subscribe to UE RAT Connectivity Analytics reporting, the service consumer shall send an HTTP POST request to the ADAE Server targeting the URI of the "UE RAT Connectivity Analytics Subscriptions" collection resource, with the request body including the UeRatConnectivitySub data structure.

2a.	Upon success, the ADAE Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual UE RAT Connectvity Analytics Subscription" resource within the UeRatConnectivitySub data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.10.12.7.

##### 5.11.14.2.3	SS_ADAE_UeRatConnectivityAnalytics_Subscription_Update

###### 5.11.14.2.3.1	General

This service operation is used by a service consumer to request the update of an existing UE RAT Connectivity Analytics Subscription at the ADAE Server.

The following procedures are supported by the "SS_ADAE_UeRatConnectivityAnalytics_Subscription_Update" service operation:

-	UE RAT Connectivity Analytics Subscription Update.

###### 5.11.14.2.3.2	UE RAT Connectivity Analytics Subscription Update

Figure 5.11.14.2.3.2-1 depicts a scenario where a service consumer sends a request to the ADAE Server to request the update of an existing UE RAT Connectivity Analytics Subscription.

Figure 5.11.14.2.3.2-1: Procedure for UE RAT Connectivity Analytics Subscription Update

1.	In order to update an existing UE RAT Connectivity Analytics subscription, the service consumer shall send an HTTP PUT/PATCH request to the ADAE Server, targeting the URI of the corresponding "Individual UE RAT Connectivity Analytics Subscription" resource, with the request body including either:

-	the updated representation of the resource within the UeRatConnectivitySub data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the UeRatConnectivitySubPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the ADAE Server shall update the targeted "Individual UE RAT Connectivity Analytics Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual UE RAT Connectivity Analytics Subscription" resource within the UeRatConnectivitySub data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 7.10.12.7.

##### 5.11.14.2.4	SS_ADAE_UeRatConnectivityAnalytics_Unsubscribe

###### 5.11.14.2.4.1	General

This service operation is used by a service consumer to request the deletion of an existing UE RAT Connectivity Analytics Subscription at the ADAE Server.

The following procedures are supported by the "SS_ADAE_UeRatConnectivityAnalytics_Unsubscribe" service operation:

-	UE RAT Connectivity Analytics Subscription Deletion.

###### 5.11.14.2.4.2	UE RAT Connectivity Analytics Subscription Deletion

Figure 5.11.14.2.4.2-1 depicts a scenario where a service consumer sends a request to the ADAE Server to delete an existing UE RAT Connectivity Analytics Subscription (see also clause 10.2.3 of 3GPP°TS°23.436°[38])

Figure 5.11.14.2.4.2-1: Procedure for UE RAT Connectivity Analytics Subscription Deletion

1.	In order to request the deletion of an existing UE RAT Connectivity Analytics Subscription, the service consumer shall send an HTTP DELETE request to the ADAE Server targeting the URI of the corresponding "Individual UE RAT Connectivity Analytics Subscription" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the ADAE Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 7.10.12.7.

##### 5.11.14.2.5	SS_ADAE_UeRatConnectivityAnalytics_Notify

###### 5.11.14.2.5.1	General

This service operation is used by a ADAE Server to notify a previously subscribed service consumer on:

-	UE RAT Connectivity Analytics report(s).

The following procedures are supported by the "SS_ADAE_UeRatConnectivityAnalytics_Notify" service operation:

-	UE RAT Connectivity Analytics Notification.

###### 5.11.14.2.5.2	UE RAT Connectivity Analytics Notification

Figure 5.11.14.2.5.2-1 depicts a scenario where the ADAE Server sends a request to notify a previously subscribed service consumer on UE RAT Connectivity Analytics report(s) (see also clause 10.2.3 of 3GPP°TS°23.436°[38])

Figure 5.11.14.2.5.2-1: Procedure for UE RAT Connectivity Analytics Notification

1.	In order to notify a previously subscribed service consumer on UE RAT Connectivity Analytics report(s), the ADAE Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation of the corresponding UE RAT Connectivity Analytics Subscription using the procedures defined in clauses 5.11.14.2.2, and the request body including the UeRatConnectivityNotif data structure.

2a.	Upon success, the service consumer shall respond to the ADAE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 7.10.12.7.

## 5.12	AIML Enablement APIs

AIML Enablement APIs are defined in 3GPP TS 29.482 [48].

## 5.13	Metaverse Enablement APIs

Spatial Anchor Server and Spatial Map Server APIs are defined in 3GPP TS 29.437 [49].

# 6	SEAL Design Aspects Common for All APIs

## 6.1	General

SEAL APIs allow secure access to the capabilities provided by SEAL.

This document specifies the procedures triggered at different functional entities as a result of API invocation requests and event notifications. The stage-2 level requirements and signalling flows are defined in 3GPP TS 23.434 [2], 3GPP TS 23.433 [34] (for SEALDD), 3GPP TS 23.435 [41] (for NSCE), 3GPP TS 23.437 [45] (for Spatial Map and Spatial Anchor services) and 3GPP TS 23.438 [46] (for Digital Asset management services).

Several design aspects, as mentioned in the following clauses, are specified in 3GPP TS 29.122 [3] and referenced by this specification.

## 6.2	Data Types

### 6.2.1	General

This clause defines structured data types, simple data types and enumerations that are applicable to several APIs defined in the present specification and can be referenced from data structures defined in the subsequent clauses.

In addition, data types that are defined in OpenAPI Specification [15] can also be referenced from data structures defined in the subsequent clauses.

NOTE:	As a convention, data types names in the present specification are with an upper-case letter in the beginning. Parameters are with a lower-case letter in the beginning. As an exception, data types that are also defined in OpenAPI Specification [15] can use a lower-case case letter in the beginning for consistency.

Table 6.2.1-1 specifies data types re-used by the SEAL from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the SEAL.

Table 6.2.1-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uri | 3GPP TS 29.122 [3] |  |
| TestNotification | 3GPP TS 29.122 [3] | Following clarifications apply: - The SCEF is the SEAL server; and - The SCS/AS is the VAL server. |
| WebsockNotifConfig | 3GPP TS 29.122 [3] | Following clarifications apply: - The SCEF is the SEAL server; and - The SCS/AS is the VAL server. |

### 6.2.2	Referenced structured data types

Table 6.2.2-1 lists structured data types defined in this specification referenced by multiple services:

Table 6.2.2-1: Referenced Structured Data Types

| Data type | Reference | Description |
| --- | --- | --- |
| VALGroupDocument | Clause 7.2.1.4.2.2 | VAL Group document information. |
| ProfileDoc | Clause 7.3.1.4.2.2 | VAL User or VAL UE profile information. |

### 6.2.3	Referenced Simple data types and enumerations

Following simple data types defined in Table 6.2.3-1 are applicable to several APIs in this document:

Table 6.2.3-1: Simple data types applicable to several APIs

| Type name | Reference | Description |
| --- | --- | --- |

## 6.3	Usage of HTTP

For SEAL APIs, support of HTTP/1.1 (IETF RFC 9112 [5], IETF RFC 9110 [6] and IETF RFC 9111 [9]) over TLS is mandatory and support of HTTP/2 (IETF RFC 9113 [12]) over TLS is recommended.

A functional entity desiring to use HTTP/2 shall use the HTTP upgrade mechanism to negotiate applicable HTTP version as described in IETF RFC 9113 [12].

Usage of HTTP over TLS and the TLS profiles shall be as specified in clause 5.1.1.4 of 3GPP TS 33.434 [26].

## 6.4	Content type

The bodies of HTTP request and successful HTTP responses shall be encoded in JSON format (see IETF RFC 8259 [13]).

The MIME media type that shall be used within the related Content-Type header field is "application/json", as defined in IETF RFC 8259 [13].

The JSON objects defined in clause 5.2.3 of 3GPP TS 29.122 [3] for the HTTP PATCH request shall be supported.

NOTE:	This release only supports the content type JSON.

## 6.5	URI structure

### 6.5.1	Resource URI structure

All API URIs of SEAL APIs shall be:

- {apiRoot}/<apiName>/<apiVersion>

"apiRoot" is configured by means outside the scope of the present document. It includes the scheme ("https"), host and optional port, and an optional prefix string. "apiName" and "apiVersion" shall be set dependent on the API, as defined in the corresponding clauses below.

All resource URIs in the clauses below are defined relative to the above root API URI.

NOTE 1:	The "apiVersion" will only be increased if the new API version contains backward incompatible changes. Otherwise, the supported feature mechanism defined in clause 6.8 can be used to negotiate extensions.

NOTE 2:	A different root structure can be used when the resource URI is preconfigured in the API invoking entity.

The root structure may be followed by "apiSpecificSuffixes" that are dependent on the API and are defined separately for each API as resource URI where they apply:

- {apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

### 6.5.2	Custom operations URI structure

The custom operation definition is in Annex C of 3GPP TS 29.501 [14].

The URI of a custom operation which is associated with a resource shall have the following structure:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>/<custOpName>

Custom operations can also be associated with the service instead of a resource. The URI of a custom operation which is not associated with a resource shall have the following structure:

{apiRoot}/<apiName>/<apiVersion>/<custOpName>

In the above URI structures, "apiRoot", "apiName", "apiVersion" and "apiSpecificResourceUriPart" are as defined in clause 6.5.1 and "custOpName" represents the name of the custom operation as defined in clause 5.1.3.2 of 3GPP TS 29.501 [14]

## 6.6	Notifications

The functional entities

-	shall support the delivery of notifications using a separate HTTP connection towards an address (see clause 5.2.5.2 of 3GPP TS 29.122 [3]);

-	may support testing delivery of notifications (see clause 5.2.5.3 of 3GPP TS 29.122 [3]); and

-	may support the delivery of notification using WebSocket protocol (see IETF RFC 6455 [4] and clause 5.2.5.4 of 3GPP TS 29.122 [3]),

as described in 3GPP TS 29.122 [3], with the following clarifications:

-	the SCEF is the SEAL server; and

-	the SCS/AS is the Subscriber.

## 6.7	Error Handling

Response bodies and error handling described in clause 5.2.6 of 3GPP TS 29.122 [3] are applicable to all APIs in the present specification unless specified otherwise, with the following clarifications:

-	the SCEF is the SEAL server; and

-	the SCS/AS is the functional entity invoking an API.

## 6.8	Feature negotiation

The functional entity invoking an API (i.e. the VAL server) and the SEAL server use feature negotiation procedures defined in 3GPP TS 29.122 [3] to negotiate the supported features, with the following clarifications:

-	description of the SCEF applies to the SEAL server; and

-	description of the SCS/AS applies to the functional entity invoking an API.

## 6.9	HTTP headers

The HTTP headers and the HTTP custom headers described in 3GPP TS 29.122 [3] are applicable to all APIs in this document.

## 6.10	Conventions for Open API specification files

The conventions for Open API specification files as specified in clause 5.2.9 of 3GPP TS 29.122 [3] shall be applicable for all APIs in this document.

# 7	SEAL API Definitions

## 7.1	Location management APIs

### 7.1.1	SS_LocationReporting API

#### 7.1.1.1	API URI

The SS_LocationReporting service shall use the SS_LocationReporting API.

The request URIs used in HTTP requests from the VAL server towards the location management server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-lr".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.1.1.2.

#### 7.1.1.2	Resources

##### 7.1.1.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.1.1.2.1-1 depicts the resource URIs structure for the SS_LocationReporting API.

Figure 7.1.1.2.1-1: Resource URI structure of the SS_LocationReporting API

Table 7.1.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.1.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| SEAL Location Reporting Configurations | /trigger-configurations | POST | Creates a new Individual SEAL Location Reporting Configuration information. |
| Individual SEAL Location Reporting Configuration | /trigger-configurations/{configurationId} | GET | Retrieves an Individual SEAL Location Reporting Configuration information identified by {configurationId}. |
|  |  | PUT | Updates an Individual SEAL Location Reporting Configuration information identified by {configurationId}. |
|  |  | PATCH | Partially modifies an Individual SEAL Location Reporting Configuration information identified by {configurationId}. |
|  |  | DELETE | Delete an Individual SEAL Location Reporting Configuration information identified by {configurationId}. |

##### 7.1.1.2.2	Resource: SEAL Location Reporting Configurations

###### 7.1.1.2.2.1	Description

The resource allows the VAL server to request to create a new individual SEAL location reporting configuration information at the location management server.

###### 7.1.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-lr/<apiVersion>/trigger-configurations

This resource shall support the resource URI variables defined in the table 7.1.1.2.2.2-1.

Table 7.1.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.1.1.2.2.3	Resource Standard Methods

7.1.1.2.2.3.1	POST

Table 7.1.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.1.1.2.2.3.1-3.

Table 7.1.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationReportConfiguration | M | 1 | Location reporting configuration information. |

Table 7.1.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationReportConfiguration | M | 1 | 201 Created | Location reporting configuration is created successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.1.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-lr/<apiVersion>/trigger-configurations/{configurationId} |

###### 7.1.1.2.2.4		Resource Custom Operations

None.

##### 7.1.1.2.3	Resource: Individual SEAL Location Reporting Configuration

###### 7.1.1.2.3.1	Description

The resource represents an individual SEAL location reporting configuration that is created at the location management server.

###### 7.1.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-lr/<apiVersion>/trigger-configurations/{configurationId}

This resource shall support the resource URI variables defined in the table 7.1.1.2.3.2-1.

Table 7.1.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| configurationId | string | Represents an individual SEAL location reporting configuration resource. |

###### 7.1.1.2.3.3	Resource Standard Methods

7.1.1.2.3.3.1	GET

This operation retrieves an individual SEAL location reporting configuration information. This method shall support the URI query parameters specified in table 7.1.1.2.3.3.1-1.

Table 7.1.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.1.2.3.3.1-2 and the response data structures and response codes specified in table 7.1.1.2.3.3.1-3.

Table 7.1.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationReportConfiguration | M | 1 | 200 OK | The location reporting configuration information. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative location management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative location management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.1.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

Table 7.1.1.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

7.1.1.2.3.3.2	PUT

This operation updates the individual SEAL location reporting configuration. This method shall support the URI query parameters specified in table 7.1.1.2.3.3.2-1.

Table 7.1.1.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.1.2.3.3.2-2 and the response data structures and response codes specified in table 7.1.1.2.3.3.2-3.

Table 7.1.1.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationReportConfiguration | M | 1 | Updated details of the location reporting configuration. |

Table 7.1.1.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationReportConfiguration | M | 1 | 200 OK | The configuration is updated successfully and the updated configuration information returned in the response. |
| n/a |  |  | 204 No Content | The location reporting configuration updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative location management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative location management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.1.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

Table 7.1.1.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

7.1.1.2.3.3.3	DELETE

This operation deletes the individual SEAL location reporting configuration. This method shall support the URI query parameters specified in table 7.1.1.2.3.3.3-1.

Table 7.1.1.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.1.2.3.3.3-2 and the response data structures and response codes specified in table 7.1.1.2.3.3.3-3.

Table 7.1.1.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.1.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual configuration matching the configurationId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative location management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative location management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.1.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

Table 7.1.1.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

7.1.1.2.3.3.4	PATCH

This method shall support the URI query parameters specified in table 7.1.1.2.3.3.4-1.

Table 7.1.1.2.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.1.2.3.3.4-2 and the response data structures and response codes specified in table 7.1.1.2.3.3.4-3.

Table 7.1.1.2.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationReportConfigurationPatch | M | 1 | Contains the modifications to be applied to the Individual SEAL Location Reporting Configuration resource. |

Table 7.1.1.2.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationReportConfiguration | M | 1 | 200 OK | Individual SEAL Location Reporting Configuration resource is modified successfully and representation of the modified Individual SEAL Location Reporting Configuration resource is returned. |
| n/a |  |  | 204 No Content | The Individual SEAL Location Reporting Configuration resource is modified successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.1.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

Table 7.1.1.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

###### 7.1.1.2.3.4	Resource Custom Operations

None.

#### 7.1.1.3	Notifications

##### 7.1.1.3.1	General

Table 7.1.1.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Location Reporting Notification | {notifUri} | POST | Notify on location event. |
| Adaptive Location Configuration Notification | {notifUri}/configure | configure (POST) | Notify the suggested adaptive location configuration and request feedback. |

##### 7.1.1.3.2	Location Trigger Event Notification

###### 7.1.1.3.2.1	Description

###### 7.1.1.3.2.2	Notification definition

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.1.1.3.2.2-1.

Table 7.1.1.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.1.3.2.2-2 and the response data structures and response codes specified in table 7.1.1.3.2.2-3.

Table 7.1.1.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationReport | M | 1 | Represents the reported location event. |

Table 7.1.1.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The notification is successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative VAL server where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative VAL server where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.1.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contans an alternative URI representing the end point of an alternative VAL server towards which the notification should be redirected. |

Table 7.1.1.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative VAL server towards which the notification should be redirected. |

##### 7.1.1.3.3	Adaptive Location Configuration Notification

###### 7.1.1.3.3.1	Description

The Adaptive Location Configuration Notification is used by the LM Server to notify the suggested adaptive location configuration and request feedback.

###### 7.1.1.3.3.2	Target URI

The Callback URI "{notifUri}/configure" shall be used with the callback URI variables defined in table 7.1.1.3.3.2-1.

Table 6.1.5.4.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | Contains a string formatted as a URI containing the Callback URI. |

###### 7.1.1.3.3.3	Standard Methods

7.1.1.3.3.3.1	POST

This method shall support the request data structures specified in table 7.1.1.3.3.3.1-1 and the response data structures and response codes specified in table 7.1.1.3.3.3.1-2.

Table 7.1.1.3.3.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AdaptConfigNotif | M | 1 | Contains the Adaptive Location Configuration Notification. |

Table 7.1.1.3.3.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AdaptConfigNotifResp | M | 1 | 200 OK | Successful case. The Adaptive Location Configuration Notification is successfully received and Adaptive Location Configuration related information is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.1.3.3.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative VAL Server towards which the notification should be redirected. |

Table 7.1.1.3.3.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative VAL Server towards which the notification should be redirected. |

#### 7.1.1.4	Data Model

##### 7.1.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.1.1.4.1-1 specifies the data types defined specifically for the SS_LocationReporting API service.

Table 7.1.1.4.1-1: SS_LocationReporting API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AdaptConfigNotif | 7.1.1.4.2.6 | Represents the adaptive reporting notification. |  |
| AdaptConfigNotifResp | 7.1.1.4.2.7 | Represents the adaptive reporting notification response. |  |
| AdaptiveReporting | 7.1.1.4.3.5 | Represents a desired adaptive reporting configuration mode. | eLSApp |
| Confirmation | 7.1.1.4.3.6 | Represents a confirmation response. |  |
| InsideOutsideInd | 7.1.1.4.3.3 | Represents a desired condition of the location reporting, e.g., inside or outside the given area. | TriggeringCriteria |
| LocChangeCond | 7.1.1.4.3.4 | Represents a desired condition of the requested location change | TriggeringCriteria |
| LocationReport | 7.1.1.4.2.5 | Represents the location trigger report. | NotifSupport |
| LocationReportConfiguration | 7.1.1.4.2.2 | Represents the location reporting configuration information. |  |
| LocationReportConfigurationPatch | 7.1.1.4.2.3 | Represents the requested modifications to the location reporting configuration information. Used to partially update Individual SEAL Location Reporting Configuration resource. | PatchUpdate |
| TriggeringCriteria | 7.1.1.4.2.4 | Represents the location reporting triggering criteria. | TriggeringCriteria |

Table 7.1.1.4.1-2 specifies data types re-used by the SS_LocationReporting API service.

Table 7.1.1.4.1-2: SS_LocationReporting API Re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Accuracy |  | 3GPP TS 29.122 [3] |  | Used to represent the desired level of accuracy of the requested location information. |  |  |  |
| DateTime |  | 3GPP TS 29.122 [3] |  | Used to represent the date and time. |  |  |  |
| DurationSec |  | 3GPP TS 29.571 [21] |  | Used to represent the time interval between successive location reports. |  |  |  |
| LocationInfo |  | 3GPP TS 29.122 [3] |  | Used to represent the location information. |  | NotifSupport |  |
| ScheduledCommunicationTime |  | 3GPP TS 29.571 [21] |  | Used to represent the scheduled location reporting time interval. |  | TriggeringCriteria |  |
| SupportedFeatures |  | 3GPP TS 29.571 [21] |  | Used to negotiate the applicability of optional features defined in table 7.1.1.6-1. |  |  |  |
| ValTargetUe |  | Clause 7.3.1.4.2.3 |  | Used to indicate either VAL User ID or VAL UE ID, to which location reporting applies. |  |  |  |
| ValSvcAreaId |  | Clause 7.1.3.4.3.2 |  | Used to represent the VAL service area identifier. |  | ValSrvArea |  |
| Uri |  | 3GPP TS 29.122 [3] |  | Represents a URI. |  | NotifSupport |  |

##### 7.1.1.4.2	Structured data types

###### 7.1.1.4.2.1	Introduction

###### 7.1.1.4.2.2	Type: LocationReportConfiguration

Table 7.1.1.4.2.2-1: Definition of type LocationReportConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServerId | string | M | 1 | Represents the VAL server identifier. |  |
| valTgtUe | ValTargetUe | M | 1 | Represents the VAL User ID or VAL UE ID to which the location reporting applies. |  |
| immRep | boolean | O | 0..1 | Indication of immediate reporting. If included, when it is set to true it indicates immediate reporting of the subscribed events, if available. Otherwise, reporting will occur when the event is met. |  |
| monDur | DateTime | O | 0..1 | Represents the time at which the subscription ceases to exist (i.e the reporting trigger becomes invalid). If omitted, there is no time limit. |  |
| repPeriod | DurationSec | O | 0..1 | Indicates the minimum time interval between successive location reports. |  |
| notifUri | Uri | C | 0..1 | Represents the notification URI. This attribute shall be provided when the "NotifSupport" feature is supported. | NotifSupport |
| accuracy | Accuracy | O | 0..1 | Represents the desired level of accuracy of the requested location information. |  |
| triggCriteria | TriggeringCriteria | O | 0..1 | Represents the location report triggering criteria based on the VAL service area information. | TriggeringCriteria |
| adaptRep | AdaptiveReporting | O | 0..1 | Indicates that adaptive location reporting is requested and includes the adaptive location configuration mode. | eLSApp |
| valSvcAreaIds | array(ValSvcAreaId) | O | 1..N | Represents the VAL service area ID(s). | ValSrvArea |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features as defined in clause 7.1.1.6. This attribute shall be provided only if feature negotiation needs to take place. |  |
| report | LocationReport | O | 0..1 | Represents location report. This attribute may be present only in the response to a Location Trigger creation/update request. | NotifSupport |

###### 7.1.1.4.2.3	Type: LocationReportConfigurationPatch

Table 7.1.1.4.2.3-1: Definition of type LocationReportConfigurationPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valTgtUe | ValTargetUe | O | 1 | Represents the VAL User ID or VAL UE ID to which the location reporting applies. |  |
| monDur | DateTime | O | 0..1 | Represents the time at which the subscription ceases to exist (i.e the reporting trigger becomes invalid). If omitted, there is no time limit. |  |
| repPeriod | DurationSec | O | 0..1 | Indicates the minimum time interval between successive location reports. |  |
| notifUri | Uri | O | 0..1 | Represents the notification URI. | NotifSupport |
| accuracy | Accuracy | O | 0..1 | Represents the desired level of accuracy of the requested location information. |  |
| valSvcAreaIds | array(ValSvcAreaId) | O | 1..N | Represents the VAL service area ID(s). | ValSrvArea |
| triggCriteria | TriggeringCriteria | O | 0..1 | Represents the location report triggering criteria. | TriggeringCriteria |

###### 7.1.1.4.2.4	Type: TriggeringCriteria

Table 7.1.1.4.2.4-1: Definition of type TriggeringCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reportingMode | NotificationMethod | M | 1 | The indication of the requested reporting mode: periodic or event-triggered. The "ONE_TIME" value is not applicable for this attribute. |  |
| repPer | DurationSec | C | 0..1 | Indicates the reporting periodicy. This attribute shall be present if the "reportingMode" attribute is set to the "PERIODIC". |  |
| locChgCond | LocChangeCond | C | 0..1 | Identifies the location change condition for the event-triggered reporting. This attribute shall be present if the "reportingMode" attribute is set to the "ON_EVENT_DETECTION". |  |
| ioInd | InsideOutsideInd | O | 0..1 | Indicates the condition when the reporting shall occur. |  |
| repSchedules | array(ScheduledCommunicationTime) | O | 1..N | Indicates the requested reporting schedule, e.g., day(s) of the week and/or time period(s) for the location reporting. |  |

###### 7.1.1.4.2.5	Type: LocationReport

Table 7.1.1.4.2.5-1: Definition of type LocationReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Represents the identifier of the "Individual SEAL Location Reporting Configuration" resource to which the location reporting notification is related. |  |
| valTgtUe | ValTargetUe | M | 1 | VAL User ID or UE ID whose location information is notified. |  |
| locInfo | LocationInfo | M | 1 | The location information associated with the valTgtUe. |  |
| timeStamp | DateTime | O | 0..1 | Timestamp of the location report. |  |

###### 7.1.1.4.2.6	Type: AdaptConfigNotif

Table 7.1.1.4.2.6-1: Definition of type AdaptConfigNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valTgtUe | ValTargetUe | M | 1 | Represents VAL User ID or UE ID. |  |
| suggConfig | TriggeringCriteria | M | 1 | Contains the suggested adaptive location configuration. |  |

###### 7.1.1.4.2.7	Type: AdaptConfigNotifResp

Table 7.1.1.4.2.7-1: Definition of type AdaptConfigNotifResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| confirmation | Confirmation | M | 1 | Represents the confirmation of the Adaptive Location Configuration. |  |

##### 7.1.1.4.3	Simple data types and enumerations

###### 7.1.1.4.3.1	Introduction

This clause defines simple data types and enumerations that are referenced from data structures defined in the previous clauses. In addition, data types and enumerations defined in clause 6.2.1 can be referenced.

###### 7.1.1.4.3.2	Simple data types

The simple data types defined in table 7.1.1.4.3.2-1 shall be supported.

Table 7.1.1.4.3.2-1: Simple data types

| Type name | Description |
| --- | --- |

###### 7.1.1.4.3.3	Enumeration: InsideOutsideInd

The enumeration InsideOutsideInd represents a desired condition of the location reporting, e.g., inside or outside the given area. It shall comply with the provisions defined in table 7.1.1.4.3.3-1.

Table 7.1.1.4.3.3-1: Enumeration InsideOutsideInd

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| INSIDE | Indicates that the reporting shall occur when the UE is inside the given location. |  |
| OUTSIDE | Indicates that the reporting shall occur when the UE is outside the given location. |  |

###### 7.1.1.4.3.4	Enumeration: LocChangeCond

The enumeration LocChangeCond represents a desired condition of the requested location change. It shall comply with the provisions defined in table 7.1.1.4.3.4-1.

Table 7.1.1.4.3.4-1: Enumeration LocChangeCond

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| CELL | The condition is cell change. |  |
| NODEB | The condition is eNodeB or gNodeB change. |  |
| TA_RA | The condition is TA or RA change. |  |
| WLAN_AN | The condition is WLAN access network change (e.g., SSID or BSSID change). |  |
| CIVIC_ADDR | The condition is civic address change. |  |
| GPS | The condition is GPS coordinate change. (NOTE) |  |
| SAI | The condition is SAI change. |  |
| ECGI | The condition is ECGI change. |  |
| RAT | The condition is RAT change. |  |
| VAL_SERVICE_AREA | The condition is VAL service area change. | ValSrvArea |
| NOTE: Care needs to be taken with regards to load and signalling cost when using the "GPS" condition. |  |  |

###### 7.1.1.4.3.5	Enumeration: AdaptiveReporting

The enumeration AdaptiveReporting represents the adaptive location configuration mode. It shall comply with the provisions defined in table 7.1.1.4.3.5-1.

Table 7.1.1.4.3.5-1: Enumeration AdaptiveReporting

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DIRECT_UPDATE | Indicates that the adaptive location configuration mode is to directly update the LM Client once configuration is adjusted without prior validation. |  |
| SUGGESTIVE_UPDATE | Indicates that the adaptive location configuration mode is to suggest and validate the planned updates with the service consumer prior to applying them to the LM Client. |  |

###### 7.1.1.4.3.6	Enumeration: Confirmation

The enumeration Confirmation represents a confirmation response. It shall comply with the provisions defined in table 7.1.1.4.3.6-1.

Table 7.1.1.4.3.6-1: Enumeration Confirmation

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACCEPT | Indicates the acceptance of the proposed action. |  |
| REJECT | Indicates the rejection of the proposed action. |  |

#### 7.1.1.5	Error Handling

##### 7.1.1.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.1.1.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_LocationReporting API.

##### 7.1.1.5.3	Application Errors

The application errors defined for SS_LocationReporting API are listed in table 7.1.1.5.3-1.

Table 7.1.1.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.1.1.6	Feature negotiation

General feature negotiation procedures are defined in clause 6.8.

Table 7.1.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | PatchUpdate | Indicates the support of the PATCH method for updating an Individual SEAL Location Reporting Configuration resource. |
| 2 | ValSrvArea | This feature indicates the support of VAL service area ID functionality as part of phase-3 of the enhancements to the SEAL framework. The following functionalities are supported: - Support the usage of the VAL service area identifier to identify a VAL service area. |
| 3 | TriggeringCriteria | Indicates the support of the triggering criteria for the location reporting trigger as a part of the enhancements to the SEAL framework. The following functionalities are supported: - Support location reporting based on triggering criteria information. |
| 4 | NotifSupport | Indicates the support of the notification service operation. |
| 5 | eLSApp | This feature indicates the support of the enhancement to enable application layer support for location services. The following functionalities are supported: - Support of adaptive location reporting. |

### 7.1.2	SS_LocationAreaInfoRetrieval API

#### 7.1.2.1	API URI

The request URI used in each HTTP request from the VAL server towards the location management server shall have the structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-lair".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.1.2.2.

#### 7.1.2.2	Resources

##### 7.1.2.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.1.2.2.1-1 depicts the resource URIs structure for the SS_LocationAreaInfoRetrieval API.

Figure 7.1.2.2.1-1: Resource URI structure of the SS_LocationAreaInfoRetrieval API

Table 7.1.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.1.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Location Information | /location-retrievals | GET | Obtains the UE(s) information in an application defined proximity range of a location. |

##### 7.1.2.2.2	Resource: Location Information

###### 7.1.2.2.2.1	Description

The Location Information resource represents the collection of UE(s) location information at the location management server.

###### 7.1.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-lair/<apiVersion>/location-retrievals

This resource shall support the resource URI variables defined in the table 7.1.2.2.2.2-1.

Table 7.1.2.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.1.2.2.2.3	Resource Standard Methods

7.1.2.2.2.3.1	GET

This operation obtains the UE(s) information in an application defined proximity range of a location. This method shall support the URI query parameters specified in table 7.1.2.2.2.3.1-1.

Table 7.1.2.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| location-info | LocationInfo | M | 1 | Location information around which the UE(s) information is requested. (NOTE) |  |
| val-svc-area-id | ValSvcAreaId | O | 0..1 | Contains the identifier of the VAL service area around which the UE(s) information is requested. (NOTE) | ValSrvArea |
| range | Float | M | 1 | The range information over which the UE(s) information is required, expressed in meters. Minimum = 0 |  |
| NOTE: If the "ValSrvArea" feature is supported and the "val-svc-area-id" query parameter is provided, then the LM server shall ignore the "location-info" query parameter. |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.2.2.2.3.1-2 and the response data structures and response codes specified in table 7.1.2.2.2.3.1-3.

Table 7.1.2.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.2.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(LMInformation) | O | 1..N | 200 OK | The UE(s) information in an application defined proximity range of a location |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative location management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative location management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.2.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

Table 7.1.2.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

###### 7.1.2.2.2.4	Resource Custom Operations

None.

#### 7.1.2.3	Notifications

None.

#### 7.1.2.4	Data Model

##### 7.1.2.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.1.2.4.1-1 specifies the data types defined specifically for the SS_LocationAreaInfoRetrieval API service.

Table 7.1.2.4.1-1: SS_LocationAreaInfoRetrieval API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |

Table 7.1.2.4.1-2 specifies data types re-used by the SS_LocationAreaInfoRetrieval API service.

Table 7.1.2.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Float | 3GPP TS 29.571 [21] | Used to represent the value of the range. |  |
| LMInformation | 7.5.1.4.2.8 | Used to represent the location information for a VAL User ID or a VAL UE ID. |  |
| LocationInfo | 3GPP TS 29.122 [3] | Used to represent the location information. |  |
| ValSvcAreaId | Clause 7.1.3.4.3.2 | Used to represent the VAL service area identifier. | ValSrvArea |

##### 7.1.2.4.2	Structured Data Types

None.

##### 7.1.2.4.3	Simple data types and enumerations

None.

#### 7.1.2.5	Error Handling

##### 7.1.2.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.1.2.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_LocationAreaInfoRetrieval API.

##### 7.1.2.5.3	Application Errors

The application errors defined for SS_LocationAreaInfoRetrieval API are listed in table 7.1.2.5.3-1.

Table 7.1.2.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.1.2.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8.

Table 7.1.2.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | ValSrvArea | This feature indicates the support of VAL service area ID functionality as part of the enhancements to SEAL. The following functionalities are supported: - Support the usage of the VAL service area identifier to identify a VAL service area. |

### 7.1.3	SS_VALServiceAreaConfiguration API

#### 7.1.3.1	API URI

The SS_VALServiceAreaConfiguration service shall use the SS_VALServiceAreaConfiguration API.

The API URI of the SS_VALServiceAreaConfiguration API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-vsac".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.1.3.2.

#### 7.1.3.1A	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_VALServiceAreaConfiguration API.

#### 7.1.3.2	Resources

##### 7.1.3.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.1.3.2.1-1 depicts the resource URIs structure for the SS_VALServiceAreaConfiguration API.

Figure 7.1.3.2.1-1: Resource URI structure of the SS_VALServiceAreaConfiguration API

Table 7.1.3.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.1.3.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| VAL Service Areas | /areas | GET | Obtain the VAL service area(s) according to the provided filtering criteria. |
|  | /areas/configure | Configure | Configure VAL service area(s). |
|  | /areas/update | Update | Update existing VAL service area(s). |
|  | /areas/delete | Delete | Delete existing VAL service area(s). |
| VAL Service Area Change Subscriptions | /subscriptions | POST | Create a new VAL service area change event(s) subscription. |
| Individual VAL Service Area Change Subscription | /subscriptions/{subscriptionId} | GET | Retrieve the individual VAL service area change event(s) subscription resource according to the subscriptionId. |
|  |  | DELETE | Delete an existing VAL service area change event(s) subscription resource according to the subscriptionId. |

##### 7.1.3.2.2	Resource: VAL Service Areas

###### 7.1.3.2.2.1	Description

###### 7.1.3.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-vsac/<apiVersion>/areas

This resource shall support the resource URI variables defined in table 7.1.3.2.2.2-1.

Table 7.1.3.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.3.1. |

###### 7.1.3.2.2.3	Resource Standard Methods

7.1.3.2.2.3.1	GET

This operation enables to retrieve one or several "Individual VAL Service Area" resources managed by the LM Server. This method shall support the URI query parameters specified in table 7.1.3.2.2.3.1-1.

Table 7.1.3.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| val-svc-area-ids | array(ValSvcAreaId) | O | 1..N | Represents the requested VAL service area(s). |
| supp-feats | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |
| NOTE: At least one of these query parameters shall be present, unless the request targets to retrieve all the VAL service area(s) available for the VAL server at the LM Server. |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.2.2.3.1-2 and the response data structures and response codes specified in table 7.1.3.2.2.3.1-3.

Table 7.1.3.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.3.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ValServiceAreaData | M | 1 | 200 OK | The requested VAL service area(s) information is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.3.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM Server. |

Table 7.1.3.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM Server. |

###### 7.1.3.2.2.4	Resource Custom Operations

7.1.3.2.2.4.1	Overview

Table 7.1.3.2.2.4.1-1: Custom operations

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Configure | /areas/configure | POST | Enables to configure VAL Service Area(s). |
| Update | /areas/update | POST | Enables to update existing VAL Service Area(s). |
| Delete | /areas/delete | POST | Enables to delete existing VAL Service Area(s). |

7.1.3.2.2.4.2	Operation: Configure

This custom operation enables to configure VAL Service Area(s).

This operation shall support the request data structures specified in table 7.1.3.2.2.4.2-2 and the response data structures and response codes specified in table 7.1.3.2.2.4.2-3.

Table 7.1.3.2.2.4.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ValServiceAreaReq | M | 1 | Represents the VAL service area(s) configuration information. |

Table 7.1.3.2.2.4.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ValServiceAreaResp | M | 1 | 200 OK | Indicates the successfully configured VAL service area ID(s). |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.3.2.2.4.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM Server. |

Table 7.1.3.2.2.4.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM Server. |

7.1.3.2.2.4.3	Operation: Update

This custom operation enables to update existing VAL Service Area(s).

This operation shall support the request data structures specified in table 7.1.3.2.2.4.3-2 and the response data structures and response codes specified in table 7.1.3.2.2.4.3-3.

Table 7.1.3.2.2.4.3-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ValServiceAreaReq | M | 1 | Represents the VAL service area(s) information that shall be updated. |

Table 7.1.3.2.2.4.3-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ValServiceAreaResp | M | 1 | 200 OK | Indicates the successfully updated VAL service area ID(s). |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.3.2.2.4.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM Server. |

Table 7.1.3.2.2.4.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM Server. |

7.1.3.2.2.4.4	Operation: Delete

This custom operation enables to delete existing VAL Service Area(s).

This operation shall support the request data structures specified in table 7.1.3.2.2.4.4-2 and the response data structures and response codes specified in table 7.1.3.2.2.4.4-3.

Table 7.1.3.2.2.4.4-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ValServiceAreaReq | M | 1 | Represents the VAL service area(s) information that shall be deleted. |

Table 7.1.3.2.2.4.4-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ValServiceAreaResp | M | 1 | 200 OK | Indicates the successfully deleted VAL service area ID(s). |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.3.2.2.4.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM Server. |

Table 7.1.3.2.2.4.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM Server. |

##### 7.1.3.2.3	Resource: VAL Service Area Change Subscriptions

###### 7.1.3.2.3.1	Description

###### 7.1.3.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-vsac/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 7.1.3.2.3.2-1.

Table 7.1.3.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.3.1. |

###### 7.1.3.2.3.3	Resource Standard Methods

7.1.3.2.3.3.1	POST

This method enables a SEAL Server to request the creation of the individual VAL service area change subscription at the LM Server. This method shall support the URI query parameters specified in table 7.1.3.2.3.3.1-1.

Table 7.1.3.2.3.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.1.3.2.3.3.1-2 and the response data structures and response codes specified in table 7.1.3.2.3.3.1-3.

Table 7.1.3.2.3.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ValServiceAreaSubsc | M | 1 | Represents the requested VAL service area change subscription parameters. |

Table 7.1.3.2.3.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ValServiceAreaSubsc | M | 1 | 201 Created | Successful case. The requested individual VAL service area change event(s) subscription resource is successfully created and a representation of the created resource is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.3.2.3.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-vsac/<apiVersion>/subscriptions{subscriptionId} |

Table 7.1.3.2.3.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM Server. |

Table 7.1.3.2.3.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM Server. |

###### 7.1.3.2.3.4	Resource Custom Operations

None.

##### 7.1.3.2.4	Resource: Individual VAL Service Area Change Subscription

###### 7.1.3.2.4.1	Description

###### 7.1.3.2.4.2	Resource Definition

Resource URI: {apiRoot}/ss-vsac/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 7.1.3.2.4.2-1.

Table 7.1.3.2.4.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.3.1. |
| subscriptionId | string | Represents the identifier of an individual VAL service area change event(s) subscription resource. |

###### 7.1.3.2.4.3	Resource Standard Methods

7.1.3.2.4.3.1	GET

This operation reads the Individual VAL Service Area Change Subscription resource. This method shall support the URI query parameters specified in table 7.1.3.2.4.3.1-1.

Table 7.1.3.2.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.1.3.2.4.3.1-2 and the response data structures and response codes specified in table 7.1.3.2.4.3.1-3.

Table 7.1.3.2.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.3.2.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ValServiceAreaSubsc | M | 1 | 200 OK | The requested individual VAL service area change event(s) subscription is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.3.2.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM server. |

Table 7.1.3.2.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM server. |

7.1.3.2.4.3.2	PUT

This operation updates the Individual VAL Service Area Change Subscription resource. This method shall support the URI query parameters specified in table 7.1.3.2.4.3.2-1.

Table 7.1.3.2.4.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.2.4.3.2-2 and the response data structures and response codes specified in table 7.1.3.2.4.3.2-3.

Table 7.1.3.2.4.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ValServiceAreaSubsc | M | 1 | Represents the updated representation of the VAL service area change event(s) subscription. |

Table 7.1.3.2.4.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ValServiceAreaSubsc | M | 1 | 200 OK | The individual VAL service area change event(s) subscription is updated successfully, and the representation of the updated resource is returned. |
| n/a |  |  | 204 No Content | The individual VAL service area change event(s) subscription is updated successfully and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative VAL server where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative VAL server where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.3.2.4.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.3.2.4.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.3.2.4.3.3	PATCH

This operation modifies the Individual VAL Service Area Change Subscription resource. This method shall support the URI query parameters specified in table 7.1.3.2.4.3.3-1.

Table 7.1.3.2.4.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.2.4.3.3-2 and the response data structures and response codes specified in table 7.1.3.2.4.3.3-3.

Table 7.1.3.2.4.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ValServiceAreaSubscPatch | M | 1 | Represents the requested modifications to the VAL service area change event(s) subscription. |

Table 7.1.3.2.4.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ValServiceAreaSubsc | M | 1 | 200 OK | The individual VAL service area change event(s) subscription is modified successfully, and the representation of the modified resource is returned. |
| n/a |  |  | 204 No Content | The individual VAL service area change event(s) subscription is modified successfully and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.3.2.4.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.3.2.4.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.3.2.4.3.4	DELETE

This operation deletes the Individual VAL Service Area Change Subscription resource. This method shall support the URI query parameters specified in table 7.1.3.2.4.3.4-1.

Table 7.1.3.2.4.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.2.4.3.4-2 and the response data structures and response codes specified in table 7.1.3.2.4.3.4-3.

Table 7.1.3.2.4.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.3.2.4.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual VAL service area change event(s) subscription resource matching the subscriptionId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.3.2.4.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM server. |

Table 7.1.3.2.4.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative LM server. |

#### 7.1.3.3	Notifications

##### 7.1.3.3.1	General

Table 7.1.3.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| VAL Service Area Change Notification | {notifUri} | POST | Notify on changes of the VAL service area(s). |

##### 7.1.3.3.2	Individual Unicast Monitoring Notification

###### 7.1.3.3.2.1	Description

###### 7.1.3.3.2.2	Notification definition

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.1.3.3.2.2-1.

Table 7.1.3.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.3.2.2-2 and the response data structures and response codes specified in table 7.1.3.3.2.2-3.

Table 7.1.3.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ValServiceAreaNotif | M | 1 | Represents the reported VAL service area data. |

Table 7.1.3.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The notification is successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative VAL server where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative VAL server where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.1.3.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative VAL server towards which the notification should be redirected. |

Table 7.1.3.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative VAL server towards which the notification should be redirected. |

#### 7.1.3.4	Data Model

##### 7.1.3.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.1.3.4.1-1 specifies the data types defined specifically for the SS_VALServiceAreaConfiguration API service.

Table 7.1.3.4.1-1: SS_VALServiceAreaConfiguration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValServiceArea | 7.1.3.4.2.2 | Represents the VAL service area. |  |
| ValServiceAreaReq | 7.1.3.4.2.3 | Represents the VAL service area configuration/update/delete request. |  |
| ValServiceAreaData | 7.1.3.4.2.4 | Represents the VAL service area retrieval information. |  |
| ValServiceAreaResp | 7.1.3.4.2.5 | Represents the VAL service area configuration/update/delete response. |  |
| ValServiceAreaSubsc | 7.1.3.4.2.6 | Represents the VAL service area change event(s) subscription. |  |
| ValServiceAreaEventType | 7.1.3.4.2.7 | Represents the VAL service area change event type. |  |
| ValServiceAreaNotif | 7.1.3.4.2.8 | Represents the VAL service area change event(s) notifcation. |  |
| ValServiceAreaEventInfo | 7.1.3.4.2.9 | Represents the VAL service area change event(s) content. |  |
| ValServiceAreaEvent | 7.1.3.4.3.3 | Represents the VAL service area change event. |  |
| ValServiceAreaSubscPatch | 7.1.3.4.2.10 | Represents the VAL service area change event(s) modification request. |  |
| ValSvcAreaId | Clause 7.1.3.4.3.2 | Used to represent the VAL service area identifier. |  |

Table 7.1.3.4.1-2 specifies data types re-used by the SS_VALServiceAreaConfiguration API service.

Table 7.1.3.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DurationSec | 3GPP TS 29.571 [21] | Used to indicate the subscription duration. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Used to indicate the location information. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the applicability of optional features defined in table 7.1.3.6-1. |  |
| Uri | 3GPP TS 29.122 [3] | Used to indicate the notification URI. |  |

##### 7.1.3.4.2	Structured data types

###### 7.1.3.4.2.1	Introduction

###### 7.1.3.4.2.2	Type: ValServiceArea

Table 7.1.3.4.2.2-1: Definition of type ValServiceArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcAreaId | ValSvcAreaId | M | 1 | Represents the VAL service area ID. |  |
| locations | array(LocationArea5G) | M | 1..N | Represents the locations associated with the VAL service area. (NOTE) |  |
| NOTE: The "nwAreaInfo" attribute within the LocationArea5G data type provided within this attriute is not applicable and shall not be present. |  |  |  |  |  |

###### 7.1.3.4.2.3	Type: ValServiceAreaReq

Table 7.1.3.4.2.3-1: Definition of type ValServiceAreaReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcAreas | array(ValServiceArea) | M | 1..N | Represents the VAL service area(s). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

###### 7.1.3.4.2.4	Type: ValServiceAreaData

Table 7.1.3.4.2.4-1: Definition of type ValServiceAreaData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcAreas | array(ValServiceArea) | O | 1..N | Represents the requested VAL service area(s). If this attribute is not present, this means that no VAL service data instance satisfies the provided query parameter(s) in the request. |  |
| suppFeats | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features defined in table 7.1.3.6-1. This attribute shall be present only if feature negotiation needs to take place. |  |

###### 7.1.3.4.2.5	Type: ValServiceAreaResp

Table 7.1.3.4.2.5-1: Definition of type ValServiceAreaResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcAreaIds | array(ValSvcAreaId) | M | 1..N | Represents the successfully handled VAL service area ID(s). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

###### 7.1.3.4.2.6	Type: ValServiceAreaSubsc

Table 7.1.3.4.2.6-1: Definition of type ValServiceAreaSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| events | array(ValServiceAreaEventType) | M | 1..N | Represents the subscribed VAL service area change event(s). |  |
| notifUri | Uri | M | 1 | Indicates the URI towards which the notification should be delivered. |  |
| subscDur | DurationSec | O | 0..1 | Indicates the subscription duration. If omitted, there is no time limit. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

###### 7.1.3.4.2.7	Type: ValServiceAreaEventType

Table 7.1.3.4.2.7-1: Definition of type ValServiceAreaEventType

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | ValServiceAreaEvent | M | 1 | Represents the requested VAL service area change event. |  |
| valSvcAreaIds | array(ValSvcAreaId) | M | 1..N | Represents the VAL service area ID(s) associated with the event. |  |

###### 7.1.3.4.2.8	Type: ValServiceAreaNotif

Table 7.1.3.4.2.8-1: Definition of type ValServiceAreaNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcAreaConts | array(ValServiceAreaEventInfo) | M | 1..N | Represents the VAL service area change event(s) content. |  |

###### 7.1.3.4.2.9	Type: ValServiceAreaEventInfo

Table 7.1.3.4.2.9-1: Definition of type ValServiceAreaEventInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | ValServiceAreaEvent | M | 1 | Represents the VAL service area change event. |  |
| valSvcAreas | array(ValServiceArea) | C | 1..N | Represents the VAL service area(s) associated with the event. This attribute shall be provided if the "event" is "UPDATE". |  |
| valSvcAreaIds | array(ValSvcAreaId) | C | 1..N | Represents the VAL service area ID(s) associated with the event. This attribute shall be provided if the "event" is "DELETE". |  |

###### 7.1.3.4.2.10	Type: ValServiceAreaSubscPatch

Table 7.1.3.4.2.10-1: Definition of type ValServiceAreaSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| events | array(ValServiceAreaEventType) | O | 1..N | Represents the subscribed VAL service area change event(s). |  |
| notifUri | Uri | O | 0..1 | Indicates the URI towards which the notification should be delivered. |  |
| subscDur | DurationSec | O | 0..1 | Indicates the subscription duration. |  |

##### 7.1.3.4.3	Simple data types and enumerations

###### 7.1.3.4.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.1.3.4.3.2	Simple data types

The simple data types defined in table 7.1.3.4.3.2-1 shall be supported.

Table 7.1.3.4.3.2-1: Simple data types

| Type name | Description |
| --- | --- |
| ValSvcAreaId | Represents the VAL Service Area identifier encoded as a string and generated either based on VAL Server ID or using the Universally Unique Identifier (UUID) version 4 as described in IETF RFC 9562 [40]. |

###### 7.1.3.4.3.3	Enumeration: ValServiceAreaEvent

Table 7.1.3.4.3.3-1: Enumeration ValServiceAreaEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UPDATE | Indicates that the VAL service area change event is VAL service area update. |  |
| DELETE | Indicates that the VAL service area change event is VAL service area delete. |  |

#### 7.1.3.5	Error Handling

##### 7.1.3.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.1.3.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_VALServiceAreaConfiguration API.

##### 7.1.3.5.3	Application Errors

The application errors defined for SS_VALServiceAreaConfiguration API are listed in table 7.1.3.5.3-1.

Table 7.1.3.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

#### 7.1.3.6	Feature negotiation

The optional features in table 7.1.3.6-1 are defined for the SS_VALServiceAreaConfiguration API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.1.3.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 7.1.4	SS_LocationHistoryInfoEvent API

#### 7.1.4.1	Introduction

The SS_LocationHistoryInfoEvent service shall use the SS_LocationHistoryInfoEvent API.

The API URI of the SS_LocationHistoryInfoEvent Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-lhie".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5.

NOTE:	When 3GPP TS 29.122 [3] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 7.1.4, the LM Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 7.1.4.2	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_LocationHistoryInfoEvent API.

#### 7.1.4.3	Resources

##### 7.1.4.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.1.4.3.1-1 depicts the resource URIs structure for the SS_LocationHistoryInfoEvent API.

Figure 7.1.4.3.1-1: Resource URIs structure of the SS_LocationHistoryInfoEvent API

Table 7.1.4.3.1-1 provides an overview of the resources and applicable HTTP methods for the SS_LocationHistoryInfoEvent API.

Table 7.1.4.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Location Tracing Configurations | /configurations | POST | Request the creation of a Location Tracing Configuration. |
| Individual Location Tracing Configuration | /configurations/{configId} | GET | Retrieve an existing "Individual Location Tracing Configuration" resource. |
|  |  | PUT | Request the update of an existing "Individual Location Tracing Configuration" resource. |
|  |  | PATCH | Request the modification of an existing "Individual Location Tracing Configuration" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual Location Tracing Configuration" resource. |
| Location History Reports | /reports | GET | Retrieve location history report(s). |

##### 7.1.4.3.2	Resource: Location Tracing Configurations

###### 7.1.4.3.2.1	Description

This resource represents the collection of Location Tracing Configurations managed by the LM Server.

###### 7.1.4.3.2.2	Resource Definition

Resource URI: {apiRoot}/ss-lhie/<apiVersion>/configurations

This resource shall support the resource URI variables defined in table 7.1.4.3.2.2-1.

Table 7.1.4.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.4.1. |

###### 7.1.4.3.2.3	Resource Standard Methods

7.1.4.3.2.3.1	POST

The HTTP POST method allows a service consumer to request the creation of a Location Tracing Configuration at the LM Server.

This method shall support the URI query parameters specified in table 7.1.4.3.2.3.1-1.

Table 7.1.4.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.4.3.2.3.1-2 and the response data structures and response codes specified in table 7.1.4.3.2.3.1-3.

Table 7.1.4.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocTracingConfig | M | 1 | Represents the parameters to request the creation of a Location Tracing Configuration. |

Table 7.1.4.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocTracingConfig | M | 1 | 201 Created | Successful case. The Location Tracing Configuration is successfully created and a representation of the created "Individual Location Tracing Configuration" resource shall be returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.4.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-lhie/<apiVersion>/configurations/{configId} |

###### 7.1.4.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 7.1.4.3.3	Resource: Individual Location Tracing Configuration

###### 7.1.4.3.3.1	Description

This resource represents a Location Tracing Configuration managed by the LM Server.

###### 7.1.4.3.3.2	Resource Definition

Resource URI: {apiRoot}/ss-lhie/<apiVersion>/configurations/{configId}

This resource shall support the resource URI variables defined in table 7.1.4.3.3.2-1.

Table 7.1.4.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.4.1. |
| configId | string | Represents the identifier of the "Individual Location Tracing Configuration" resource. |

###### 7.1.4.3.3.3	Resource Standard Methods

7.1.4.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual Location Tracing Configuration" resource at the LM Server.

This method shall support the URI query parameters specified in table 7.1.4.3.3.3.1-1.

Table 7.1.4.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.4.3.3.3.1-2 and the response data structures and response codes specified in table 7.1.4.3.3.3.1-3.

Table 7.1.4.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.4.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocTracingConfig | M | 1 | 200 OK | Successful case. The requested "Individual Location Tracing Configuration" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.4.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.4.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.4.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual Location Tracing Configuration" resource at the LM Server.

This method shall support the URI query parameters specified in table 7.1.4.3.3.3.2-1.

Table 7.1.4.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.4.3.3.3.2-2 and the response data structures and response codes specified in table 7.1.4.3.3.3.2-3.

Table 7.1.4.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocTracingConfig | M | 1 | Represents the updated representation of the "Individual Location Tracing Configuration" resource. |

Table 7.1.4.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocTracingConfig | M | 1 | 200 OK | Successful case. The "Individual Location Tracing Configuration" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Location Tracing Configuration" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.4.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.4.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.4.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual Location Tracing Configuration" resource at the LM Server.

This method shall support the URI query parameters specified in table 7.1.4.3.3.3.3-1.

Table 7.1.4.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.4.3.3.3.3-2 and the response data structures and response codes specified in table 7.1.4.3.3.3.3-3.

Table 7.1.4.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocTracingConfigPatch | M | 1 | Represents the parameters to request the modification of the "Individual Location Tracing Configuration" resource. |

Table 7.1.4.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocTracingConfig | M | 1 | 200 OK | Successful case. The "Individual Location Tracing Configuration" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Location Tracing Configuration" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.4.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.4.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.4.3.3.3.4	DELETE

The HTTP DELETE method allows a service consumer to request the deletion of an existing "Individual Location Tracing Configuration" resource at the LM Server.

This method shall support the URI query parameters specified in table 7.1.4.3.3.3.4-1.

Table 7.1.4.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.4.3.3.3.4-2 and the response data structures and response codes specified in table 7.1.4.3.3.3.4-3.

Table 7.1.4.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.4.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Location Tracing Configuration" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.4.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.4.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

###### 7.1.4.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 7.1.4.3.4	Resource: Location History Reports

###### 7.1.4.3.4.1	Description

This resource represents the collection of Location History Report(s) managed by the LM Server.

###### 7.1.4.3.4.2	Resource Definition

Resource URI: {apiRoot}/ss-lhie/<apiVersion>/reports

This resource shall support the resource URI variables defined in table 7.1.4.3.4.2-1.

Table 7.1.4.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.4.1. |

###### 7.1.4.3.4.3	Resource Standard Methods

7.1.4.3.4.3.1	GET

The HTTP GET method allows a service consumer to retrieve Location History Report(s) from the LM Server.

This method shall support the URI query parameters specified in table 7.1.4.3.4.3.1-1.

Table 7.1.4.3.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| target-id | ValTargetUe | M | 1 | Contains the identifier of the target VAL UE or VAL user. |  |
| val-service-id | string | O | 0..1 | Contains the identifier of the target VAL service. |  |
| req-loc-history | ReqLocHistory | O | 0..1 | Contains the requested location history filtering information. |  |
| supp-feat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.1.4.7. This query parameter shall be present only when feature negotiation is required. |  |

This method shall support the request data structures specified in table 7.1.4.3.4.3.1-2 and the response data structures and response codes specified in table 7.1.4.3.4.3.1-3.

Table 7.1.4.3.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.4.3.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocHistoryReports | M | 1 | 200 OK | Successful case. The requested Location History Report(s), if any, shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.4.3.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.4.3.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

###### 7.1.4.3.4.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 7.1.4.4	Custom Operations without associated resources

There are no custom Operations without associated resources defined for this resource in this release of the specification.

#### 7.1.4.5	Notifications

There are no notifications defined for this resource in this release of the specification.

#### 7.1.4.6	Data Model

##### 7.1.4.6.1	General

This clause specifies the application data model supported by the API.

Table 7.1.4.6.1-1 specifies the data types defined for the SS_LocationHistoryInfoEvent API.

Table 7.1.4.6.1-1: SS_LocationHistoryInfoEvent API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| LocTracingConfig | 7.1.4.6.2.2 | Represents a Location Tracing Configuration. |  |
| LocTracingConfigPatch | 7.1.4.6.2.3 | Represents the parameters to request the modification of a Location Tracing Configuration. |  |
| ReqLocHistory | 7.1.4.6.2.4 | Represents the requested location history filtering information. |  |
| LocHistoryReport | 7.1.4.6.2.6 | Represents a location history report. |  |
| LocHistoryReports | 7.1.4.6.2.5 | Represents the location history report(s). |  |

Table 7.1.4.6.1-2 specifies data types re-used by the SS_LocationHistoryInfoEvent API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SS_LocationHistoryInfoEvent API.

Table 7.1.4.6.1-2: SS_LocationHistoryInfoEvent API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [3] | Represents a date and a time. |  |
| DurationSec | 3GPP TS 29.122 [3] | Represents a time duration expressed in units of seconds. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents a location area. |  |
| LocationInfo | 3GPP TS 29.122 [3] | Represents user location information. |  |
| LocationQoS | 3GPP TS 29.572 [31] | Represents the location QoS. |  |
| PositioningMethod | 3GPP TS 29.572 [31] | Represents the positioning method. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| TriggeringCriteria | Clause 7.1.1.4.2.4 | Represents the location reporting triggering criteria. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Represents the identifier of a VAL UE or VAL user. |  |

##### 7.1.4.6.2	Structured data types

###### 7.1.4.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

###### 7.1.4.6.2.2	Type: LocTracingConfig

Table 7.1.4.6.2.2-1: Definition of type LocTracingConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| target | ValTargetUe | M | 1 | Contains the identifier of the targeted VAL UE or VAL user. |  |
| valServiceId | string | O | 0..1 | Contains the identifier of the targeted VAL service. |  |
| locArea | LocationArea5G | O | 0..1 | Contains the location area where the location tracing configuration shall apply. |  |
| locQoS | LocationQoS | O | 0..1 | Contains the requested location QoS. |  |
| triggerConds | TriggeringCriteria | O | 0..1 | Contains the triggering conditions of the location tracing configuration. |  |
| duration | DurationSec | O | 0..1 | Contains the time duration within which the location tracing configuration stays valid. |  |
| posMethod | PositioningMethod | O | 0..1 | Contains the requested positioning method. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.1.4.8. This attribute shall be present only when feature negotiation is required. |  |

###### 7.1.4.6.2.3	Type: LocTracingConfigPatch

Table 7.1.4.6.2.3-1: Definition of type LocTracingConfigPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locArea | LocationArea5G | O | 0..1 | Contains the location area where the location tracing configuration shall apply. |  |
| locQoS | LocationQoS | O | 0..1 | Contains the requested location QoS. |  |
| triggerConds | TriggeringCriteria | O | 0..1 | Contains the triggering conditions of the location tracing configuration. |  |
| duration | DurationSec | O | 0..1 | Contains the time duration within which the location tracing configuration stays valid. |  |
| posMethod | PositioningMethod | O | 0..1 | Contains the requested positioning method. |  |

###### 7.1.4.6.2.4	Type: ReqLocHistory

Table 7.1.4.6.2.4-1: Definition of type ReqLocHistory

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locArea | LocationArea5G | C | 0..1 | Contains the location area where the location history report(s) is/are requested. (NOTE) |  |
| time | DateTime | C | 0..1 | Contains the time at which the location history report(s) is/are requested. (NOTE) |  |
| duration | DurationSec | C | 0..1 | Contains the time duration within which the location history report(s) is/are requested. (NOTE) |  |
| NOTE 1: At least one of these attributes shall be present. |  |  |  |  |  |

###### 7.1.4.6.2.5	Type: LocHistoryReports

Table 7.1.4.6.2.5-1: Definition of type LocHistoryReports

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reports | array(LocHistoryReport) | M | 0..N | Contains the Location History Report(s). If there are no Location History Report(s) fulfilling the request, this attribute shall contain an empty array. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.1.4.7. This attribute shall be present only when feature negotiation is required. |  |

###### 7.1.4.6.2.6	Type: LocHistoryReport

Table 7.1.4.6.2.6-1: Definition of type LocHistoryReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| targets | array(ValTargetUe) | C | 1..N | Contains the identifier(s) of the VAL UE/user(s) to which the location history report is related. (NOTE) |  |
| locData | map(LocationInfo) | M | 1..N | Contains the location history information report. The key of the map shall be set to the identifier of the VAL UE/user (among the ones provided within the "targets" attribute) to which the provided location history information provided within the map value is related. |  |
| timestamp | DateTime | O | 0..1 | Contains the timestamp of the location history report. |  |

##### 7.1.4.6.3	Simple data types and enumerations

###### 7.1.4.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.1.4.6.3.2	Simple data types

The simple data types defined in table 7.1.4.6.3.2-1 shall be supported.

Table 7.1.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 7.1.4.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 7.1.4.6.5	Binary data

###### 7.1.4.6.5.1	Binary Data Types

Table 7.1.4.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.1.4.7	Error Handling

##### 7.1.4.7.1	General

For the SS_LocationHistoryInfoEvent API, error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses are applicable for the SS_LocationHistoryInfoEvent API.

##### 7.1.4.7.2	Protocol Errors

No specific protocol errors for the SS_LocationHistoryInfoEvent API are specified.

##### 7.1.4.7.3	Application Errors

The application errors defined for the SS_LocationHistoryInfoEvent API are listed in Table 7.1.4.7.3-1.

Table 7.1.4.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.1.4.8	Feature negotiation

The optional features listed in table 7.1.4.8-1 are defined for the SS_LocationHistoryInfoEvent API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.1.4.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 7.1.4.9	Security

The provisions of clause 9 shall apply for the SS_LocationHistoryInfoEvent API.

### 7.1.5	SS_ConfirmLocation Service API

#### 7.1.5.1	Introduction

The SS_ConfirmLocation service shall use the SS_ConfirmLocation API.

The API URI of the SS_ConfirmLocation Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-cl".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5.

NOTE:	When 3GPP TS 29.122 [3] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 7.1.5, the LM Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 7.1.5.2	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_ConfirmLocation API.

#### 7.1.5.3	Resources

##### 7.1.5.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.1.5.3.1-1 depicts the resource URIs structure for the SS_ConfirmLocation API.

Figure 7.1.5.3.1-1: Resource URIs structure of the SS_ConfirmLocation API

Table 7.1.5.3.1-1 provides an overview of the resources and applicable HTTP methods for the SS_ConfirmLocation API.

Table 7.1.5.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Location Confirmation Service Subscriptions | /subscriptions | POST | Request the creation of a Location Confirmation Service Subscription. |
| Individual Location Confirmation Service Subscription | /subscriptions/{subscId} | GET | Retrieve an existing "Individual Location Confirmation Service Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual Location Confirmation Service Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual Location Confirmation Service Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual Location Confirmation Service Subscription" resource. |

##### 7.1.5.3.2	Resource: Location Confirmation Service Subscriptions

###### 7.1.5.3.2.1	Description

This resource represents the collection of Location Confirmation Service Subscriptions managed by the LM Server.

###### 7.1.5.3.2.2	Resource Definition

Resource URI: {apiRoot}/ss-cl/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 7.1.5.3.2.2-1.

Table 7.1.5.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.5.1. |

###### 7.1.5.3.2.3	Resource Standard Methods

7.1.5.3.2.3.1	POST

The HTTP POST method allows a service consumer to request the creation of a Location Confirmation Service Subscription at the LM Server.

This method shall support the URI query parameters specified in table 7.1.5.3.2.3.1-1.

Table 7.1.5.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.5.3.2.3.1-2 and the response data structures and response codes specified in table 7.1.5.3.2.3.1-3.

Table 7.1.5.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocConfirmSubsc | M | 1 | Represents the parameters to request the creation of a Location Confirmation Service Subscription. |

Table 7.1.5.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocConfirmSubsc | M | 1 | 201 Created | Successful case. The Location Confirmation Service Subscription is successfully created and a representation of the created "Individual Location Confirmation Service Subscription" resource shall be returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.5.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-cl/<apiVersion>/subscriptions/{subscId} |

###### 7.1.5.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 7.1.5.3.3	Resource: Individual Location Confirmation Service Subscription

###### 7.1.5.3.3.1	Description

This resource represents a Location Confirmation Service Subscription managed by the LM Server.

###### 7.1.5.3.3.2	Resource Definition

Resource URI: {apiRoot}/ss-cl/<apiVersion>/subscriptions/{subscId}

This resource shall support the resource URI variables defined in table 7.1.5.3.3.2-1.

Table 7.1.5.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.5.1. |
| subscId | string | Represents the identifier of the "Individual Location Confirmation Service Subscription" resource. |

###### 7.1.5.3.3.3	Resource Standard Methods

7.1.5.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual Location Confirmation Service Subscription" resource at the LM Server.

This method shall support the URI query parameters specified in table 7.1.5.3.3.3.1-1.

Table 7.1.5.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.5.3.3.3.1-2 and the response data structures and response codes specified in table 7.1.5.3.3.3.1-3.

Table 7.1.5.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.5.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocConfirmSubsc | M | 1 | 200 OK | Successful case. The requested "Individual Location Confirmation Service Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.5.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.5.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.5.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual Location Confirmation Service Subscription" resource at the LM Server.

This method shall support the URI query parameters specified in table 7.1.5.3.3.3.2-1.

Table 7.1.5.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.5.3.3.3.2-2 and the response data structures and response codes specified in table 7.1.5.3.3.3.2-3.

Table 7.1.5.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocConfirmSubsc | M | 1 | Represents the updated representation of the "Individual Location Confirmation Service Subscription" resource. |

Table 7.1.5.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocConfirmSubsc | M | 1 | 200 OK | Successful case. The "Individual Location Confirmation Service Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Location Confirmation Service Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.5.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.5.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.5.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual Location Confirmation Service Subscription" resource at the LM Server.

This method shall support the URI query parameters specified in table 7.1.5.3.3.3.3-1.

Table 7.1.5.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.5.3.3.3.3-2 and the response data structures and response codes specified in table 7.1.5.3.3.3.3-3.

Table 7.1.5.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocConfirmSubscPatch | M | 1 | Represents the parameters to request the modification of the "Individual Location Confirmation Service Subscription" resource. |

Table 7.1.5.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocConfirmSubsc | M | 1 | 200 OK | Successful case. The "Individual Location Confirmation Service Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Location Confirmation Service Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.5.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.5.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.5.3.3.3.4	DELETE

The HTTP DELETE method allows a service consumer to request the deletion of an existing "Individual Location Confirmation Service Subscription" resource at the LM Server.

This method shall support the URI query parameters specified in table 7.1.5.3.3.3.4-1.

Table 7.1.5.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.5.3.3.3.4-2 and the response data structures and response codes specified in table 7.1.5.3.3.3.4-3.

Table 7.1.5.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.5.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Location Confirmation Service Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.5.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.5.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

###### 7.1.5.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 7.1.5.4	Custom Operations without associated resources

There are no custom Operations without associated resources defined for this resource in this release of the specification.

#### 7.1.5.5	Notifications

##### 7.1.5.5.1	General

Notifications shall comply to clause 6.6.

Table 7.1.5.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Location Confirmation Service Usage Notification | {notifUri} | POST | This service operation enables an LM Server to notify a previously subscribed service consumer on location confirmation service usage related event(s). |

##### 7.1.5.5.2	Location Confirmation Service Usage Notification

###### 7.1.5.5.2.1	Description

The Location Confirmation Service Usage Notification is used by an LM Server to notify a previously subscribed service consumer on location confirmation service usage related event(s).

###### 7.1.5.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 7.1.5.5.2.2-1.

Table 7.1.5.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | Represents the callback URI encoded as a string formatted as a URI. |

###### 7.1.5.5.2.3	Standard Methods

7.1.5.5.2.3.1	POST

This method shall support the request data structures specified in table 7.1.5.5.2.3.1-1 and the response data structures and response codes specified in table 7.1.5.5.2.3.1-2.

Table 7.1.5.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocConfirmUsageNotif | M | 1 | Represents the Location Confirmation Service Usage Notification. |

Table 7.1.5.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Location Confirmation Service Usage Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.5.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 7.1.5.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

#### 7.1.5.6	Data Model

##### 7.1.5.6.1	General

This clause specifies the application data model supported by the API.

Table 7.1.5.6.1-1 specifies the data types defined for the SS_ConfirmLocation API.

Table 7.1.5.6.1-1: SS_ConfirmLocation API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| LocConfirmSubsc | 7.1.5.6.2.2 | Represents a Location Confirmation Service Subscription. |  |
| LocConfirmSubscPatch | 7.1.5.6.2.3 | Represents the parameters to request the modification of a Location Confirmation Service Subscription. |  |
| LocConfirmUsageNotif | 7.1.5.6.2.4 | Represents a Location Confirmation Service Usage Notification. |  |
| LocConfirmUsageReport | 7.1.5.6.2.5 | Represents a location confirmation service usage report. |  |

Table 7.1.5.6.1-2 specifies data types re-used by the SS_ConfirmLocation API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SS_ConfirmLocation API.

Table 7.1.5.6.1-2: SS_ConfirmLocation API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [3] | Represents a date and a time. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |

##### 7.1.5.6.2	Structured data types

###### 7.1.5.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

###### 7.1.5.6.2.2	Type: LocConfirmSubsc

Table 7.1.5.6.2.2-1: Definition of type LocConfirmSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appIds | array(string) | C | 1..N | Contains the identifier(s) of the application(s) to which the subscription shall apply. (NOTE) |  |
| valServiceIds | array(string) | C | 1..N | Contains the identifier(s) of the VAL service(s) to which the subscription shall apply. (NOTE) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.1.5.8. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: These attributes are mutually exclusive and only one of them shall be present. |  |  |  |  |  |

###### 7.1.5.6.2.3	Type: LocConfirmSubscPatch

Table 7.1.5.6.2.3-1: Definition of type LocConfirmSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appIds | array(string) | O | 1..N | Contains the identifier(s) of the application(s) to which the subscription shall apply. |  |
| valServiceIds | array(string) | O | 1..N | Contains the identifier(s) of the VAL service(s) to which the subscription shall apply. |  |

###### 7.1.5.6.2.4	Type: LocConfirmUsageNotif

Table 7.1.5.6.2.4-1: Definition of type LocConfirmUsageNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reports | array(LocConfirmUsageReport) | M | 0..N | Contains the location confirmation service usage report(s). |  |
| timestamp | DateTime | O | 0..1 | Contains the timestamp of the notification. |  |

###### 7.1.5.6.2.5	Type: LocConfirmUsageReport

Table 7.1.5.6.2.5-1: Definition of type LocConfirmUsageReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appId | string | C | 0..1 | Contains the identifier of the application to which the location confirmation service report is related. |  |
| valServiceId | string | C | 0..1 | Contains the identifier of the VAL service to which the location confirmation service report is related. |  |
| valUeId | string | M | 1 | Contains the identifier of the VAL UE that requested the location confirmation service. |  |
| timestamp | DateTime | O | 0..1 | Contains the time at which the location confirmation service was triggered by the VAL UE identified by the "valUeId" attribute. |  |
| NOTE: These attributes are mutually exclusive and either one of them shall be present. |  |  |  |  |  |

##### 7.1.5.6.3	Simple data types and enumerations

###### 7.1.5.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.1.5.6.3.2	Simple data types

The simple data types defined in table 7.1.5.6.3.2-1 shall be supported.

Table 7.1.5.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 7.1.5.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 7.1.5.6.5	Binary data

###### 7.1.5.6.5.1	Binary Data Types

Table 7.1.5.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.1.5.7	Error Handling

##### 7.1.5.7.1	General

For the SS_ConfirmLocation API, error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses are applicable for the SS_ConfirmLocation API.

##### 7.1.5.7.2	Protocol Errors

No specific protocol errors for the SS_ConfirmLocation API are specified.

##### 7.1.5.7.3	Application Errors

The application errors defined for the SS_ConfirmLocation API are listed in Table 7.1.5.7.3-1.

Table 7.1.5.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.1.5.8	Feature negotiation

The optional features listed in table 7.1.5.8-1 are defined for the SS_ConfirmLocation API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.1.5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 7.1.5.9	Security

The provisions of clause 9 shall apply for the SS_ConfirmLocation API.

### 7.1.6	SS_SLPositioningManagement Service API

#### 7.1.6.1	Introduction

The SS_SLPositioningManagement service shall use the SS_SLPositioningManagement API.

The API URI of the SS_SLPositioningManagement API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-slpm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5.

NOTE:	When 3GPP TS 29.122 [3] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 7.1.6, the LM Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 7.1.6.2	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_SLPositioningManagement API.

#### 7.1.6.3	Resources

##### 7.1.6.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.1.6.3.1-1 depicts the resource URIs structure for the SS_SLPositioningManagement API.

Figure 7.1.6.3.1-1: Resource URI structure of the SS_SLPositioningManagement API

Table 7.1.6.3.1-1 provides an overview of the resources and applicable HTTP methods for the SS_SLPositioningManagement API.

Table 7.1.6.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| SL Positioning Management Subscriptions | /subscriptions | POST | Create a new SL Positioning Management Subscription. |
| Individual SL Positioning Management Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual SL Positioning Management Subscription" resource. |
|  |  | PUT | Update an existing "Individual SL Positioning Management Subscription" resource. |
|  |  | PATCH | Modify an existing "Individual SL Positioning Management Subscription" resource. |
|  |  | DELETE | Delete an existing "Individual SL Positioning Management Subscription" resource. |

##### 7.1.6.3.2	Resource: SL Positioning Management Subscriptions

###### 7.1.6.3.2.1	Description

This resource represents the collection of SL Positioning Management Subscriptions managed by the LM Server.

###### 7.1.6.3.2.2	Resource Definition

Resource URI: {apiRoot}/ss-slpm/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 7.1.6.3.2.2-1.

Table 7.1.6.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.6.1. |

###### 7.1.6.3.2.3	Resource Standard Methods

7.1.6.3.2.3.1	POST

The HTTP POST method enables a service consumer to request the creation of a new SL Positioning Management Subscription at the LM Server.

This method shall support the URI query parameters specified in table 7.1.6.3.2.3.1-1.

Table 7.1.6.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.1.6.3.2.3.1-2 and the response data structures and response codes specified in table 7.1.6.3.2.3.1-3.

Table 7.1.6.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SlPositionMgmtSubsc | M | 1 | Represents the parameters to request the creation of an SL Positioning Management Subscription. |

Table 7.1.6.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SlPositionMgmtSubsc | M | 1 | 201 Created | Successful case. The SL Position Management Subscription is successfully created, and a representation of the created "Individual SL Positioning Management Subscription" resource is returned in the response body. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.6.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-slpm/<apiVersion>/subscriptions{subscriptionId} |

###### 7.1.6.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 7.1.6.3.3	Resource: Individual SL Positioning Management Subscription

###### 7.1.6.3.3.1	Description

###### 7.1.6.3.3.2	Resource Definition

Resource URI: {apiRoot}/ss-slpm/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 7.1.6.3.3.2-1.

Table 7.1.6.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.6.1. |
| subscriptionId | string | Represents the identifier of an "Individual SL Positioning Management Subscription" resource. |

###### 7.1.6.3.3.3	Resource Standard Methods

7.1.6.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual SL Positioning Management Subscription" resource at the LM Server.

This method shall support the URI query parameters specified in table 7.1.6.3.3.3.1-1.

Table 7.1.6.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.6.3.3.3.1-2 and the response data structures and response codes specified in table 7.1.6.3.3.3.1-3.

Table 7.1.6.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.6.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SlPositionMgmtSubsc | M | 1 | 200 OK | Successful case. The requested "Individual SL Positioning Management Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.6.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.6.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.6.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual SL Positioning Management Subscription" resource.

This method shall support the URI query parameters specified in table 7.1.6.3.3.3.2-1.

Table 7.1.6.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.6.3.3.3.2-2 and the response data structures and response codes specified in table 7.1.6.3.3.3.2-3.

Table 7.1.6.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SlPositionMgmtSubsc | M | 1 | Represents the updated representation of the "Individual SL Positioning Management Subscription" resource. |

Table 7.1.6.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SlPositionMgmtSubsc | M | 1 | 200 OK | Successful case. The "Individual SL Positioning Management Subscription" resource is updated successfully, and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual SL Positioning Management Subscription" resource is updated successfully, and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.6.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.6.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.6.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual SL Positioning Management Subscription" resource.

This method shall support the URI query parameters specified in table 7.1.6.3.3.3.3-1.

Table 7.1.6.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.6.3.3.3.3-2 and the response data structures and response codes specified in table 7.1.6.3.3.3.3-3.

Table 7.1.6.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SlPositionMgmtSubscPatch | M | 1 | Represents the requested modifications to the "Individual SL Positioning Management Subscription" resource. |

Table 7.1.6.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SlPositionMgmtSubsc | M | 1 | 200 OK | Successful case. The "Individual SL Positioning Management Subscription" resource is modified successfully, and the representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual SL Positioning Management Subscription" resource is modified successfully, and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.6.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

Table 7.1.6.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM Server. |

7.1.6.3.3.3.4	DELETE

The HTTP PATCH method allows a service consumer to request the deletion of an existing "Individual SL Positioning Management Subscription" resource.

Table 7.1.6.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.6.3.3.3.4-2 and the response data structures and response codes specified in table 7.1.6.3.3.3.4-3.

Table 7.1.6.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.6.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual SL Positioning Management Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.6.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM server. |

Table 7.1.6.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative LM server. |

###### 7.1.6.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 7.1.6.4	Custom Operations without associated resources

##### 7.1.6.4.1	Overview

The structure of the custom operation URIs of the SS_SLPositioningManagement API is shown in Figure 7.1.6.4.1-1.

Figure 7.1.6.4.1-1 depicts the resource URIs structure for the SS_SLPositioningManagement API.

Figure 7.1.6.4.1-1: Custom operation URI structure of the SS_SLPositioningManagement API

Table 7.1.6.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the SS_SLPositioningManagement API.

Table 7.1.6.4.1-1: Custom operations without associated resources

| Custom operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| SR Positioning Information Request | /request-srp | POST | Enables to request SR Positioning Information. |

The custom operations shall support the URI variables defined in table 7.1.6.4.1-2.

Table 7.1.6.4.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.6.2.1. |

##### 7.1.6.4.2	Operation: SR Positioning Information Request

###### 7.1.6.4.2.1	Description

This custom operation enables a service consumer to request SR based positioning information to the LM Server.

###### 7.1.6.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 7.1.6.4.2.2-1 and the response data structures and response codes specified in table 7.1.6.4.2.2-2.

Table 7.1.6.4.2.2-1: Data structures supported by the POST Request Body on this custom operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SrPosInfoReq | M | 1 | Contains the SR Positioning Information Request. |

Table 7.1.6.4.2.2-2: Data structures supported by the POST Response Body on this custom operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SrPosInfoResp |  |  | 200 OK | Successful case. The Positioning Information Request is successfully received and processed, and the requested SR based positioning information is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative LM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.6.4.2.2-3: Headers supported by the 307 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative LM Server. |

Table 7.1.6.4.2.2-4: Headers supported by the 308 Response Code on this custom operation

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative LM Server. |

#### 7.1.6.5	Notifications

##### 7.1.6.5.1	General

Notifications shall comply to clause 6.6.

Table 7.1.6.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| SL Positioning Management Event Notification | {notifUri} | POST | This service operation enables an LM Server to notify a previously subscribed service consumer on SL Positioning related event(s). |

##### 7.1.6.5.2	SL Positioning Management Event Notification

###### 7.1.6.5.2.1	Description

The SL Positioning Management Event Notification is used by the LM Server to notify a previously subscribed service consumer on SL Positioning related event(s).

###### 7.1.6.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 7.1.6.5.2.2-1.

Table 7.1.6.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

###### 7.1.6.5.2.3	Standard Methods

7.1.6.5.2.3.1	POSTThis method shall support the request data structures specified in table 7.1.6.5.2.3-1 and the response data structures and response codes specified in table 7.1.6.5.2.3-2.

Table 7.1.6.5.2.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SlPositionMgmtNotif | M | 1 | Represents the SL Positioning Management Event Notification. |

Table 7.1.6.5.2.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The SL Positioning Management Event Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.1.6.5.2.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 7.1.6.5.2.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

#### 7.1.6.6	Data Model

##### 7.1.6.6.1	General

This clause specifies the application data model supported by the API.

Table 7.1.6.6.1-1 specifies the data types defined for the SS_SLPositioningManagement API.

Table 7.1.6.6.1-1: SS_SLPositioningManagement API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SlPositionMgmtSubsc | 7.1.6.6.2.2 | Represents the SL Positioning Management subscription information. |  |
| SlPositionMgmtSubscPatch | 7.1.6.6.2.3 | Represents the update in the SL Positioning Management subscription. |  |
| SlPositionMgmtNotif | 7.1.6.6.2.4 | Represents the SL Positioning Management notification. |  |
| ValUeInfo | 7.1.6.6.2.5 | Represents the Val UE and its address information. |  |
| SrPosInfoReq | 7.1.6.6.2.6 | Represents the SR Positioning Information Request. |  |
| SrPosFilter | 7.1.6.6.3.6 | Represents the requested SR based positioning information filters. |  |
| SrPosInfoResp | 7.1.6.6.2.7 | Represents the SR Positioning Information Response. |  |
| SrUeList | 7.1.6.6.2.8 | Represents the list of UE identities of SR based positioning. |  |
| SrPosInfo | 7.1.6.6.2.9 | Represents the SR based positioning information. |  |

Table 7.1.6.6.1-2 specifies data types re-used by the SS_SLPositioningManagement API service.

Table 7.1.6.6.1-2: SS_SLPositioningManagement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| 2DRelativeLocation | 3GPP TS 29.572 [31] | Represents 2D local co-ordinates with origin corresponding to another known point. |  |
| 3DRelativeLocation | 3GPP TS 29.572 [31] | Represents 3D local co-ordinates with origin corresponding to another known point. |  |
| DurationSec | 3GPP TS 29.122 [3] | Represents a time duration in units of seconds. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| LocationQoS | 3GPP TS 29.572 [31] | Represents the location QoS. |  |
| RangeDirection | 3GPP TS 29.572 [31] | Represents the distance and direction between two points. |  |
| RelativeVelocityWithUncertainty | 3GPP TS 29.572 [31] | Represents relative velocity with uncertainty. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable for LM Server. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the applicability of optional features defined in table 7.1.4.6-1. |  |
| Uri | 3GPP TS 29.122 [3] | Used to indicate the notification URI. |  |
| ValServiceArea | Clause 7.1.3.4.2.2 | Represents the VAL service area. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to indicate either VAL User ID or VAL UE ID, to which location reporting applies. |  |
| ValUeAddrInfo | Clause 7.4.1.4.2.30 | Represents VAL UE address information. |  |

##### 7.1.6.6.2	Structured data types

###### 7.1.6.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

###### 7.1.6.6.2.2	Type: SlPositionMgmtSubsc

Table 7.1.6.6.2.2-1: Definition of type SlPositionMgmtSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | M | 1 | The VAL service ID of the VAL application for which the SL Positioning change event(s) is subscribed. |  |
| tgtUes | array(ValTargetUe) | C | 1..N | List of VAL User(s) or UE ID(s) for which SL Positioning management monitoring is requested for the given location information. (NOTE) |  |
| valServArea | LocationArea5G | C | 0..1 | Represents the VAL Service Area for which the subscription applies. |  |
| slPosMgmtParams | FFS | M | 0..1 | Represents the SL Positioning parameters. |  |
| valUeSel | array(ValUeSel) | O | 1..N | List of VAL UE(s) selection criteria for SL Positioning Management which when satisfied requires the LM server to send the notification. |  |
| eventReport | ReportingInformation | M | 1 | Represents the reporting requirements of the event subscription. |  |
| notifUri | Uri | M | 1 | Indicates the URI towards which the notification should be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |
| NOTE: At least one of the "tgtUes" attribute and the “valServArea” attribute shall be present. |  |  |  |  |  |

Editor's Note: The content of this data type is FFS.

###### 7.1.6.6.2.3	Type: SlPositionMgmtSubscPatch

Table 7.1.6.6.2.3-1: Definition of type SlPositionMgmtSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tgtUes | array(ValTargetUe) | O | 1..N | List of VAL User(s) or UE ID(s) for which SL Positioning management monitoring is requested for the given location information. (NOTE) |  |
| valServArea | LocationArea5G | O | 0..1 | Represents the VAL Service Area for which the subscription update applies. |  |
| slPosMgmtParams | FFS | O | 0..1 | Represents the SL Positioning parameters that LMS has to configure the identified VAL UE(s). |  |
| valUeSel | array(ValUeSel) | O | 1..N | List of VAL UE(s) selection criteria for SL Positioning Management which when satisfied requires the LM server to send the notification. |  |
| eventReport | ReportingInformation | O | 1 | Represents the reporting requirements of the event subscription. |  |
| notifUri | Uri | O | 1 | Indicates the URI towards which the notification should be delivered. |  |

Editor's Note: The content of this data type is FFS.

###### 7.1.6.6.2.4	Type: SlPositionMgmtNotif

Table 7.1.6.6.2.4-1: Definition of type SlPositionMgmtNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscId | string | M | 0..1 | Subscription Identifier of the SL Position Management subscription. |  |
| refUes | array(ValUeInfo) | C | 1..N | Val UE(s) that are identified as the Reference UE(s). This attribute shall be present if reference UE is subscribed and discovered by the LMS. |  |
| tgtUes | array(ValUeInfo) | C | 1..N | Val UE(s) that are identified as the Target UE(s). This attribute shall be present if target UE is subscribed and discovered by the LMS. |  |
| clientUes | array(ValUeInfo) | C | 1..N | Val UE(s) that are identified as the Client UE(s). This attribute shall be present if client UE is subscribed and discovered by the LMS. |  |
| failCause | array(Cause) | C | 0..1 | Notification from the LM server regarding the criteria that is matched. This attribute shall be present if reference UE and/or target UE and/or client UE are not discovered. |  |

Editor's Note: The content of this data type is FFS.

###### 7.1.6.6.2.5	Type: ValUeInfo

Table 7.1.6.6.2.5-1: Definition of type ValUeInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeId | string | C | 0..1 | Contains the unique identifier of the concerned VAL UE. (NOTE) |  |
| valUeAddr | ValUeAddrInfo | C | 0..1 | Contains the address information of the concerned VAL UE. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 7.1.6.6.2.6	Type: SrPosInfoReq

Table 7.1.6.6.2.6-1: Definition of type SrPosInfoReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | O | 0..1 | The VAL service ID of the VAL application. |  |
| ueList | SrUeList | M | 1 | Contains the list of UE identities. |  |
| srPosFltr | array(SrPosFilter) | M | 1 | Contains the requested SR based positioning information filters. |  |
| locQoS | LocationQoS | O | 0..1 | Contains the requested location QoS. |  |
| expTime | DurationSec | O | 0..1 | The expiration timer for the SR based positioning information request. |  |

Editor's Note: Whether to define the information element "Reporting events" in the Table 9.3.2.65-1 of 3GPP TS 23.434 [2] is FFS.

###### 7.1.6.6.2.7	Type: SrPosInfoResp

Table 7.1.6.6.2.7-1: Definition of type SrPosInfoResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| srPosInfo | array(SrPosInfo) | M | 1..N | Contains the SR based positioning information. |  |

###### 7.1.6.6.2.8	Type: SrUeList

Table 7.1.6.6.2.8-1: Definition of type SrUeList

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientUes | array(string) | M | 1..N | Contains the identity of client UE. |  |
| tgtUes | array(string) | M | 1..N | Contains the identity of target UE. |  |
| refUes | array(string) | M | 1..N | Contains the list of identities of reference UEs. |  |

Editor's Note: This data type may be updated based on the LS reply from SA6.

###### 7.1.6.6.2.9	Type: SrPosInfo

Table 7.1.6.6.2.9-1: Definition of type SrPosInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| refUe | string | M | 1 | Contains the identity of reference UE. |  |
| tgtUe | string | M | 1 | Contains the identity of target UE. |  |
| relativeLocation2D | 2DRelativeLocation | C | 0..1 | Represents 2D local co-ordinates of the reference UE with origin corresponding to target UE. (NOTE) |  |
| relativeLocation3D | 3DRelativeLocation | C | 0..1 | Represents 3D local co-ordinates of the reference UE with origin corresponding to target UE. (NOTE) |  |
| relativeVelocity | RelativeVelocityWithUncertainty | C | 0..1 | Represents 3D relative velocity of the reference UE with origin corresponding to target UE. (NOTE) |  |
| distanceDirection | RangeDirection | C | 0..1 | Represents the distance and direction of the reference UE with origin corresponding to target UE. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

Editor's Note: The content of this data type is FFS.

##### 7.1.6.6.3	Simple data types and enumerations

###### 7.1.6.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.1.6.6.3.2	Simple data types

The simple data types defined in table 7.1.6.6.3.2-1 shall be supported.

Table 7.1.6.6.3.2-1: Simple data types

| Type name | Description |
| --- | --- |

###### 7.1.6.6.3.4	Enumeration: Cause

Table 7.1.6.6.3.4-1: Enumeration Cause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| NOTIFY_NO_REFERENCE_UE_FOUND | This value indicates that the none of the VAL UE matched the criteria for the Reference UE. |  |
| NOTIFY_NO_TARGET_UE_FOUND | This value indicates that the none of the VAL UE matched the criteria for Target UE. |  |
| NOTIFY_NO_CLIENT_UE_FOUND | This value indicates that the none of the VAL UE matched the criteria for Client UE. |  |

###### 7.1.6.6.3.5	Enumeration: ValUeSel

Table 7.1.6.6.3.5-1: Enumeration ValUeSel

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REF_UE | Indicate the selection criteria of UE role is reference UE. |  |
| TARGET_UE | Indicate the selection criteria of UE role is target UE. |  |
| CLIENT_UE | Indicate the selection criteria of UE role is client UE. |  |

###### 7.1.6.6.3.6	Enumeration: SrPosFilter

Table 7.1.6.6.3.6-1: Enumeration SrPosFilter

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| RANGE | Indicates that the range is requested. |  |
| DIRECTION | Indicates that the direction is requested. |  |
| RELATIVE_POS | Indicates that the relative position is requested. |  |
| RELATIVE_VEL | Indicates that the relative velocity is requested. |  |

##### 7.1.6.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 7.1.6.6.5	Binary data

###### 7.1.6.6.5.1	Binary Data Types

Table 7.1.6.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.1.6.7	Error Handling

##### 7.1.6.7.1	General

For the SS_SLPositioningManagement API, error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses are applicable for the SS_SLPositioningManagement API.

##### 7.1.6.7.2	Protocol Errors

No specific protocol errors for the SS_SLPositioningManagement API are specified.

##### 7.1.6.7.3	Application Errors

The application errors defined for SS_SLPositioningManagement API are listed in table 7.1.6.6.3-1.

Table 7.1.6.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

#### 7.1.6.8	Feature negotiation

The optional features in table 7.1.4.8-1 are defined for the SS_SLPositioningManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.1.6.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 7.1.6.9	Security

The provisions of clause 9 shall apply for the SS_SLPositioningManagement API.

## 7.2	Group management APIs

### 7.2.1	SS_GroupManagement API

#### 7.2.1.1	API URI

The SS_GroupManagement service shall use the SS_GroupManagement API.

The request URIs used in HTTP requests from the VAL server towards the Group management server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-gm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.2.1.2

#### 7.2.1.2	Resources

##### 7.2.1.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.2.1.2.1-1 depicts the resource URIs structure for the SS_GroupManagement API.

Figure 7.2.1.2.1-1: Resource URI structure of the SS_GroupManagement API

Table 7.2.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.2.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| VAL Group Documents | /group-documents | POST | Create a new VAL group document. |
|  |  | GET | Retrieve VAL group documents according to the query parameters. If there are no query parameters, do not fetch any VAL group document. |
| Individual VAL Group Document | /group-documents/{groupDocId} | GET | Retrieve an individual VAL group's membership and configuration information according to query parameter on the resource identified by {groupDocId}. If there are no query parameter, fetch the whole VAL group document resource identified by {groupDocId}. |
|  |  | PUT | Update an individual VAL group's membership and configuration information identified by {groupDocId}. |
|  |  | PATCH | Partially update an individual VAL group's membership and configuration information identified by {groupDocId} |
|  |  | DELETE | Deletes an individual VAL group's membership and configuration information identified by {groupDocId}. |

##### 7.2.1.2.2	Resource: VAL Group Documents

###### 7.2.1.2.2.1	Description

The VAL Group Documents resource represents all the VAL group documents that are created at a given group management server.

###### 7.2.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-gm/<apiVersion>/group-documents

This resource shall support the resource URI variables defined in the table 7.2.1.2.2.2-1.

Table 7.2.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.2.1.2.2.3	Resource Standard Methods

7.2.1.2.2.3.1	POST

This method shall support the URI query parameters specified in table 7.2.1.2.2.3.1-1.

Table 7.2.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.2.1.2.2.3.1-3.

Table 7.2.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VALGroupDocument | M | 1 | Details of the VAL group that needs to be created, |

Table 7.2.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALGroupDocument | M | 1 | 201 Created | VAL group created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.2.1.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-gm/<apiVersion>/group-documents/{groupDocId} |

7.2.1.2.2.3.2	GET

This operation retrieves VAL group documents satisfying filter criteria. This method shall support the URI query parameters specified in table 7.2.1.2.2.3.2-1.

Table 7.2.1.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| val-group-id | string | O | 0..1 | String identifying the VAL group. |
| val-service-id | string | O | 0..1 | String identifying the VAL service. |

This method shall support the request data structures specified in table 7.2.1.2.2.3.2-2 and the response data structures and response codes specified in table 7.2.1.2.2.3.2 -3.

Table 7.2.1.2.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.2.1.2.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(VALGroupDocument) | M | 0..N | 200 OK | List of VAL group documents. This response shall include VAL group documents matching the query parameters provided in the request. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative group management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative group management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.2.1.2.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative group management server. |

Table 7.2.1.2.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative group management server. |

###### 7.2.1.2.2.4		Resource Custom Operations

None.

##### 7.2.1.2.3	Resource: Individual VAL Group Document

###### 7.2.1.2.3.1	Description

The Individual VAL Group Document resource represents an individual group document that is created at a given group management server.

###### 7.2.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-gm/<apiVersion>/group-documents/{groupDocId}

This resource shall support the resource URI variables defined in the table 7.2.1.2.3.2-1.

Table 7.2.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| groupDocId | string | Represents an individual group document resource. |

###### 7.2.1.2.3.3	Resource Standard Methods

7.2.1.2.3.3.1	GET

This operation retrieves VAL group information satisfying filter criteria. This method shall support the URI query parameters specified in table 7.2.1.2.3.3.1-1.

Table 7.2.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| group-members | boolean | O | 0..1 | When set to 'true', it indicates the group management server to send the members list information of the VAL group. Set to false or omitted otherwise. |
| group-configuration | boolean | O | 0..1 | When set to 'true', it indicates the group management server to send the configuration information of the VAL group. Set to false or omitted otherwise. |

This method shall support the request data structures specified in table 7.2.1.2.3.3.1-2 and the response data structures and response codes specified in table 7.2.1.2.3.3.1-3.

Table 7.2.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.2.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALGroupDocument | M | 1 | 200 OK | The VAL group information based on the request from the VAL server. This response shall include VAL group members list if group-members flag is set to true in the request, VAL group configuration information if the group-configuration flag is set to true in the request, VAL group identifier, whole VAL group document resource if both group-members and group-configuration flags are omitted/set to false in the request. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative group management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative group management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.2.1.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative group management server. |

Table 7.2.1.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative group management server. |

7.2.1.2.3.3.2	PUT

This operation updates the VAL group document. This method shall support the URI query parameters specified in table 7.2.1.2.3.3.2-1.

Table 7.2.1.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.1.2.3.3.2-2 and the response data structures and response codes specified in table 7.2.1.2.3.3.2-3.

Table 7.2.1.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VALGroupDocument | M | 1 | Updated details of the VAL group document. |

Table 7.2.1.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALGroupDocument | M | 1 | 200 OK | The VAL group document updated successfully and the updated VAL group document returned in the response. |
| n/a |  |  | 204 No Content | The VAL group document updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative group management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative group management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.2.1.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative group management server. |

Table 7.2.1.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative group management server. |

7.2.1.2.3.3.3	DELETE

This operation deletes the VAL group document. This method shall support the URI query parameters specified in table 7.2.1.2.3.3.3-1.

Table 7.2.1.2.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.1.2.3.3.3-2 and the response data structures and response codes specified in table 7.2.1.2.3.3.3-3.

Table 7.2.1.2.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.2.1.2.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual VAL group document matching the groupDocId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative group management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative group management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.2.1.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative group management server. |

Table 7.2.1.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative group management server. |

7.2.1.2.3.3.4	PATCH

This method shall support the URI query parameters specified in table 7.2.1.2.3.3.4-1.

Table 7.2.1.2.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.1.2.3.3.4-2 and the response data structures and response codes specified in table 7.2.1.2.3.3.4-3.

Table 7.2.1.2.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VALGroupDocumentPatch | M | 1 | Contains the modifications to be applied to the Individual VAL Group Document resource. |

Table 7.2.1.2.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALGroupDocument | M | 1 | 200 OK | Individual VAL Group Document resource is modified successfully and representation of the modified VAL Group Document resource is returned. |
| n/a |  |  | 204 No Content | The Individual VAL Group Document resource is updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.2.1.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

Table 7.2.1.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

###### 7.2.1.2.3.4	Resource Custom Operations

None.

#### 7.2.1.3	Notifications

None.

#### 7.2.1.4	Data Model

##### 7.2.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.2.1.4.1-1 specifies the data types defined specifically for the SS_GroupManagement API service.

Table 7.2.1.4.1-1: SS_GroupManagement API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| VALGroupDocument | 7.2.1.4.2.2 | Represents details of the VAL group document information. |  |
| VALGroupDocumentPatch | 7.2.1.4.2.3 | Represent details of the partial update of VAL group document information. | PatchUpdate |

Table 7.2.1.4.1-2 specifies data types re-used by the SS_GroupManagement API service.

Table 7.2.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ExternalGroupId | 3GPP TS 29.122 [3] | Used to represent the the external group identifier related to the member UEs of the group. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| LocationInfo | 3GPP TS 29.122 [3] | The location information related to VAL group. |  |
| PduSessionType | 3GPP TS 29.571 [21] | Identifies PDU Session Type. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the applicability of optional features defined in table 7.2.1.6-1. |  |
| ValSvcAreaId | Clause 7.1.3.4.3.2 | Used to represent the VAL service area identifier. | ValSrvArea |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to indicate either VAL User ID or VAL UE ID, to which location reporting applies. |  |

##### 7.2.1.4.2	Structured data types

###### 7.2.1.4.2.1	Introduction

###### 7.2.1.4.2.2	Type: VALGroupDocument

Table 7.2.1.4.2.2-1: Definition of type VALGroupDocument

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valGroupId | string | M | 1 | This is VAL group identity (VAL group ID) as per TS 23.434 [2], which is a unique identifier within the VAL service that represents a VAL group, set of VAL users or VAL UEs according to the VAL service. |  |
| grpDesc | string | O | 0..1 | Text description of the VAL group. |  |
| members | array(ValTargetUe) | O | 1..N | List of VAL User IDs or VAL UE IDs, which are members of the VAL group. |  |
| valGrpConf | string | O | 0..1 | Configuration data for the VAL group. Shall be present in HTTP POST request message from VAL server to Group Management server. |  |
| valServiceIds | array(string) | O | 1..N | List of VAL services whose communications enabled on the group. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features of the API as described in clause 6.8. This attribute shall be provided only if feature negotiation needs to take place.. |  |
| resUri | Uri | O | 0..1 | The URI for individual VAL group document resource. (NOTE 1) |  |
| locInfo | LocationInfo | O | 0..1 | The location information related to the VAL group. This information is used to determine the members of the group. (NOTE 3) |  |
| addLocInfo | LocationArea5G | O | 0..1 | The additional location information related to the VAL group. This information is used to determing the members of the group. (NOTE 3) |  |
| valSvcAreaId | ValSvcAreaId | O | 0..1 | Identifier of the VAL service area. (NOTE 3) | ValSrvArea |
| extGrpId | ExternalGroupId | O | 0..1 | The external group identifier, identifying the member UEs of the VAL group at the 3GPP core network. |  |
| com5GLanType | PduSessionType | O | 0..1 | Identifies the 5G LAN-Type communication. (NOTE 2) |  |
| valSvcInf | string | O | 0..1 | VAL service specific information that may be present during group membership update and in the notification of the events "GM_GROUP_INFO_CHANGE" and "GM_GROUP_CREATE". |  |
| NOTE 1: The "resUri" attribute is not modifiable by the VAL server. NOTE 2: The enumeration value "UNSTRUCTURED" in data type "PduSessionType" is not applicable. NOTE 3: When the "ValSrvArea" feature is supported, the "locInfo" and "valSvcAreaId" attributes are mutually exclusive; and the “addLocInfo” and "valSvcAreaId" attributes are mutually exclusive. |  |  |  |  |  |

###### 7.2.1.4.2.3	Type: VALGroupDocumentPatch

Table 7.2.1.4.2.3-1: Definition of type VALGroupDocumentPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| grpDesc | string | O | 0..1 | Text description of the VAL group. |  |
| members | array(ValTargetUe) | O | 1..N | List of VAL User IDs or VAL UE IDs, which are members of the VAL group. |  |
| valGrpConf | string | O | 0..1 | Configuration data for the VAL group. |  |
| valServiceIds | array(string) | O | 1..N | List of VAL services whose communications enabled on the group. |  |
| locInfo | LocationInfo | O | 0..1 | The location information related to the VAL group. This information is used to determine the members of the group. (NOTE 2) |  |
| addLocInfo | LocationArea5G | O | 0..1 | The additional location information related to the VAL group. This information is used to determining the members of the group. (NOTE 2) |  |
| valSvcAreaId | ValSvcAreaId | O | 0..1 | Identifier of the VAL service area. (NOTE 2) | ValSrvArea |
| extGrpId | ExternalGroupId | O | 0..1 | The external group identifier, identifying the member UEs of the VAL group at the 3GPP core network. |  |
| com5GLanType | PduSessionType | O | 0..1 | Identifies the 5G LAN-Type communication. (NOTE 1) |  |
| NOTE 1: The enumeration value "UNSTRUCTURED" in data type "PduSessionType" is not applicable. NOTE 2: When the “ValSrvArea" feature is supported, the "locInfo" attribute and "valSvcAreaId" attribute are mutually exclusive within the resource representation, and the "valSvcAreaId" attribute and the “addLocInfo” attribute are also mutually exclusive in the resource representation. If one of them is provided in this data type for the first time as part of an update request, the other shall be removed from the resource representation, if present. |  |  |  |  |  |

##### 7.2.1.4.3	Simple data types and enumerations

None.

#### 7.2.1.5	Error Handling

##### 7.2.1.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.2.1.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_GroupManagement API.

##### 7.2.1.5.3	Application Errors

The application errors defined for SS_GroupManagement API are listed in table 7.2.1.5.3-1.

Table 7.2.1.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.2.1.6	Feature negotiation

General feature negotiation procedures are defined in clause 6.8.

Table 7.2.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | PatchUpdate | Indicates the support of the PATCH method for updating an Individual VAL Group Document resource. |
| 2 | enNB1 | This feature indicates the support of enhancements to this application layer API in Rel-18. |
| 3 | ValSrvArea | This feature indicates the support of VAL service area ID functionality as part of the phase-3 of the enhancements to the SEAL framework. The following functionalities are supported: - Support the usage of the VAL service area identifier to identify a VAL service area. |

## 7.3	Configuration management APIs

### 7.3.1	SS_UserProfileRetrieval API

#### 7.3.1.1	API URI

The SS_UserProfileRetrieval service shall use the SS_UserProfileRetrieval API.

The request URIs used in HTTP requests from the VAL server towards the Configuration management server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-upr".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.3.1.2.

#### 7.3.1.2	Resources

##### 7.3.1.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.3.1.2.1-1 depicts the resource URIs structure for the SS_UserProfileRetrieval API.

Figure 7.3.1.2.1-1: Resource URI structure of the SS_UserProfileRetrieval API

Table 7.3.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.3.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| VAL Services | /val-services | GET | Retrieve VAL User or VAL UE's profile information. |

##### 7.3.1.2.2	Resource: VAL Services

###### 7.3.1.2.2.1	Description

The VAL Services resource represents all the VAL services that are created at a given configuration management server.

###### 7.3.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-upr/<apiVersion>/val-services

This resource shall support the resource URI variables defined in the table 7.3.1.2.2.2-1.

Table 7.3.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.3.1.2.2.3	Resource Standard Methods

7.3.1.2.2.3.1	GET

This operation retrieves VAL User or VAL UE profile information satisfying the filter criteria. This method shall support the URI query parameters specified in table 7.3.1.2.2.3.1-1.

Table 7.3.1.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| val-tgt-ue | ValTargetUe | M | 1 | Identifying a VAL target UE. |
| val-service-id | string | O | 0..1 | String identifying a VAL service. |

This method shall support the request data structures specified in table 7.3.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.3.1.2.2.3.1 -3.

Table 7.3.1.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.3.1.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(ProfileDoc) | M | 0..N | 200 OK | List of VAL User / VAL UE profile documents. This response shall include user profile information matching the query parameters provided in the request. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative configuration management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative configuration management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.3.1.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative configuration management server. |

Table 7.3.1.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative configuration management server. |

###### 7.3.1.2.2.4		Resource Custom Operations

None.

#### 7.3.1.3	Notifications

None.

#### 7.3.1.4	Data Model

##### 7.3.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.3.1.4.1-1 specifies the data types defined specifically for the SS_UserProfileRetrieval API service.

Table 7.3.1.4.1-1: SS_UserProfileRetrieval API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ProfileDoc | 7.3.1.4.2.2 | Represents the profile information associated with VAL user ID or VAL UE ID. |  |
| ValTargetUe | 7.3.1.4.2.3 | Represents the information identifying a VAL user ID or VAL UE ID. |  |
| ValTargetUeRm | 7.3.1.4.2.4 | Represents the same as the ValTargetUe data type but with the OpenAPI "nullable" property set to "true". |  |

Table 7.3.1.4.1-2 specifies data types re-used by the SS_UserProfileRetrieval API service.

Table 7.3.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

##### 7.3.1.4.2	Structured data types

###### 7.3.1.4.2.1	Introduction

###### 7.3.1.4.2.2	Type: ProfileDoc

Table 7.3.1.4.2.2-1: Definition of type ProfileDoc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| profileInformation | string | M | 1 | Profile information associated with valTgtUe. |  |
| valTgtUe | ValTargetUe | M | 1 | Unique identifier of a VAL user or a VAL UE. |  |

###### 7.3.1.4.2.3	Type: ValTargetUe

Table 7.3.1.4.2.3-1: Definition of type ValTargetUe

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUserId | string | O | 0..1 | Unique identifier of a VAL user. |  |
| valUeId | string | O | 0..1 | Unique identifier of a VAL UE. |  |
| NOTE: Either "valUserId" or "valUeId" shall be provided. |  |  |  |  |  |

###### 7.3.1.4.2.4	Type: ValTargetUeRm

This data type is defined in the same way as the "ValTargetUe" data type, but with the OpenAPI "nullable" preperty set to "true".

##### 7.3.1.4.3	Simple data types and enumerations

None.

#### 7.3.1.5	Error Handling

##### 7.3.1.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.3.1.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_UserProfileRetrieval API.

##### 7.3.1.5.3	Application Errors

The application errors defined for SS_UserProfileRetrieval API are listed in table 7.3.1.5.3-1.

Table 7.3.1.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.3.1.6	Feature negotiation

General feature negotiation procedures are defined in clause 6.8.

Table 7.3.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 7.3.2	SS_VALServiceData API

#### 7.3.2.1	API URI

The SS_VALServiceData service shall use the SS_VALServiceData API.

The API URI of the SS_VALServiceData API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-vsd".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.3.1.2.

#### 7.3.2.1A	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_VALServiceData API.

#### 7.3.2.2	Resources

##### 7.3.2.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.3.2.2.1-1 depicts the resource URIs structure for the SS_VALServiceData API.

Figure 7.3.2.2.1-1: Resource URI structure of the SS_VALServiceData API

Table 7.3.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.3.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| VAL Service Data Sets | /val-servdata | GET | Retrieve the VAL service data according to the provided query parameter(s). |

##### 7.3.2.2.2	Resource: VAL Service Data Sets

###### 7.3.2.2.2.1	Description

This resource represents the collection of VAL Service Data resources managed by the CM Server.

###### 7.3.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-vsd/<apiVersion>/val-servdata

This resource shall support the resource URI variables defined in the table 7.3.2.2.2.2-1.

Table 7.3.2.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |

###### 7.3.2.2.2.3	Resource Standard Methods

7.3.2.2.2.3.1	GET

This operation retrieves the VAL service data satisfying the filter criteria. This method shall support the URI query parameters specified in table 7.3.2.2.2.3.1-1.

Table 7.3.2.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| val-tgt-ues | array(string) | O | 1..N | Identifying the list of the target VAL UE(s). (NOTE) |
| val-tgt-users | array(string) | O | 1..N | Identifying the list of the target VAL user(s). (NOTE) |
| val-service-ids | array(string) | O | 1..N | Identifying the list of the target VAL service(s). (NOTE) |
| supp-feats | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |
| NOTE: At least one of these query parameters shall be present, unless the request targets to retrieve all the VAL Service Data Sets managed by the CM Server. |  |  |  |  |

This method shall support the request data structures specified in table 7.3.2.2.2.3.1-2 and the response data structures and response codes specified in table 7.3.2.2.2.3.1-3.

Table 7.3.2.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.3.2.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ValServDataResp | M | 1 | 200 OK | Represents the requested VAL service data. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative CM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.3.2.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CM Server. |

Table 7.3.2.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative CM server. |

###### 7.3.2.2.2.4		Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 7.3.2.3	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

#### 7.3.2.4	Notifications

There are no notifications defined for this API in this release of the specification.

#### 7.3.2.5	Data Model

##### 7.3.2.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.3.2.5.1-1 specifies the data types defined specifically for the SS_VALServiceData API service.

Table 7.3.2.5.1-1: SS_VALServiceData API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValServDataResp | 7.3.2.5.2.2 | Represents the response to a VAL service data retrieval request. |  |
| ValServiceData | 7.3.2.5.2.3 | Represents the VAL service data. |  |

Table 7.3.2.5.1-2 specifies data types re-used by the SS_VALServiceData API service.

Table 7.3.2.5.1-2: SS_VALServiceData re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| ValTargetUe | 7.3.1.4.2.3 | Used to indicate the VAL user ID or VAL UE ID. |  |

##### 7.3.2.5.2	Structured data types

###### 7.3.2.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.3.2.5.2.2	Type: ValServDataResp

Table 7.3.2.5.2.2-1: Definition of type ValServDataResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServData | array(ValServiceData) | M | 0..N | Contains the requested VAL service data. If an empty array is provided within this attribute, this means that no VAL service data set satisfies the provided query parameter(s) in the corresponding request. |  |
| suppFeats | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.3.2.7. This attribute shall be present only when feature negotiation needs to take place. |  |

###### 7.3.2.5.2.3	Type: ValServiceData

Table 7.3.2.5.2.3-1: Definition of type ValServiceData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valTgtUe | ValTargetUe | M | 1 | Contains the unique identifier of a VAL user or a VAL UE. |  |
| valServIds | array(string) | M | 1..N | Contains the list of the VAL service(s) associated with the VAL user or VAL UE provided within the "valTgtUe" attribute. |  |
| valServSpecInfo | string | O | 0..1 | Contains the VAL service specific information. |  |

##### 7.3.2.5.3	Simple data types and enumerations

###### 7.3.2.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.3.2.5.3.2	Simple data types

The simple data types defined in table 7.3.2.5.3.2-1 shall be supported.

Table 7.3.2.5.3.2-1: Simple data types

| Type name | Description |
| --- | --- |

##### 7.3.2.5.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 7.3.2.5.5	Binary data

###### 7.3.2.5.5.1	Binary Data Types

Table 7.3.2.5.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.3.2.6	Error Handling

##### 7.3.2.6.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.3.2.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_VALServiceData API.

##### 7.3.2.6.3	Application Errors

The application errors defined for SS_VALServiceData API are listed in table 7.3.2.6.3-1.

Table 7.3.2.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.3.2.7	Feature negotiation

The optional features in table 7.3.2.7-1 are defined for the SS_VALServiceData API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.3.2.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 7.4	Network resource management APIs

### 7.4.1	SS_NetworkResourceAdaptation API

#### 7.4.1.1	API URI

The SS_NetworkResourceAdaptation service shall use the SS_NetworkResourceAdaptation API.

The API URI of the SS_NetworkResourceAdaptation API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-nra".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.4.1.2

#### 7.4.1.1A	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_NetworkResourceAdaptation API.

#### 7.4.1.2	Resources

##### 7.4.1.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.4.1.2.1-1 depicts the resource URIs structure for the SS_NetworkResourceAdaptation API.

Figure 7.4.1.2.1-1: Resource URI structure of the SS_NetworkResourceAdaptation API

Table 7.4.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.4.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Multicast Subscriptions | /multicast-subscriptions | POST | Create a new Individual Multicast Subscription resource. |
| Individual Multicast Subscription | /multicast-subscriptions/{multiSubId} | GET | Read an Individual Multicast Subscription resource. |
|  |  | DELETE | Remove an Individual Multicast Subscription resource. |
| MBS Resources | /mbs-resources | POST | Request the creation of an MBS resource. |
| Individual MBS Resource | /mbs-resources/{mbsResId} | GET | Request the retrieval of an existing "Individual MBS Resource" resource. |
|  |  | PUT | Request the update of an existing "Individual MBS Resource" resource. |
|  |  | PATCH | Request the modification of an existing "Individual MBS Resource" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual MBS Resource" resource. |
|  |  | Activate | Request the activation of an existing MBS Resource. |
|  |  | Deactivate | Request the deactivation of an existing MBS Resource. |
| Unicast Subscriptions | /unicast-subscriptions | POST | Create a new Individual Unicast Subscription resource. |
| Individual Unicast Subscription | /unicast-subscriptions/{uniSubId} | GET | Read an Individual Unicast Subscription resource. |
|  |  | PUT | Update an Individual Unicast Subscription resource. |
|  |  | PATCH | Modify an Individual Unicast Subscription resource. |
|  |  | DELETE | Remove an Individual Unicast Subscription resource. |
| TSC Stream Availability | /tsc-stream-availability | GET | Retrieve TSC stream availability information. |
| TSC Streams | /tsc-streams | GET | Retrieve TSC stream information. |
| Individual TSC Stream | /tsc-streams/{valStreamId} | GET | Read an Individual TSC stream resource. |
|  |  | PUT | Create a new Individual TSC stream resource. |
|  |  | DELETE | Remove an Individual TSC stream resource. |
|  |  | GET | Read an Individual TSC stream resource. |
| BDT Policy Configurations | /bdt-policy-configs | POST | Create a new background data transfer policy configuration. |
| Individual BDT Policy Configuration | /bdt-policy-configs/{bdtPolConfigId} | GET | Request the retrieval of an existing "Individual BDT Policy Configuration" resource. |
|  |  | PUT | Request the update of an existing "Individual BDT Policy Configuration" resource. |
|  |  | PATCH | Request the modification of an existing "Individual BDT Policy Configuration" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual BDT Policy Configuration" resource. |
| Unified Traffic Pattern Subscriptions | /unif-traff-subscriptions | POST | Create a new Unified Traffic Pattern Subscription. |
| Individual Unified Traffic Pattern Subscription | /unif-traff-subscriptions/{unfTrffSubId} | GET | Retrieve an existing "Individual Unified Traffic Pattern Subscription" resource. |
|  |  | PUT | Request to update of an existing "Individual Unified Traffic Pattern Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual Unified Traffic Pattern Subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual Unified Traffic Pattern Subscription" resource. |

##### 7.4.1.2.2	Resource: Multicast Subscriptions

###### 7.4.1.2.2.1	Description

###### 7.4.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/multicast-subscriptions

This resource shall support the resource URI variables defined in table 7.4.1.2.2.2-1.

Table 7.4.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.4.1.2.2.3	Resource Standard Methods

7.4.1.2.2.3.1	POST

Table 7.4.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.4.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.4.1.2.2.3.1-3.

Table 7.4.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MulticastSubscription | M | 1 |  |

Table 7.4.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MulticastSubscription | M | 1 | 201 Created |  |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.500 [22] shall also apply. |  |  |  |  |

Table 7.4.1.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-nra/<apiVersion>/multicast-subscriptions/{multiSubId} |

###### 7.4.1.2.2.4		Resource Custom Operations

None.

##### 7.4.1.2.3	Resource: Individual Multicast Subscription

###### 7.4.1.2.3.1	Description

###### 7.4.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/multicast-subscriptions/{multiSubId}

This resource shall support the resource URI variables defined in table 7.4.1.2.3.2-1.

Table 7.4.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | 6.5 |
| multiSubId | string | The multicast subscription identifier. |

###### 7.4.1.2.3.3	Resource Standard Methods

7.4.1.2.3.3.1	GET

Table 7.4.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.3.3.1-2 and the response data structures and response codes specified in table 7.4.1.2.3.3.1-3.

Table 7.4.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MulticastSubscription | M | 1 | 200 OK |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

Table 7.4.1.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

7.4.1.2.3.3.2	DELETE

Table 7.4.1.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.3.3.2-2 and the response data structures and response codes specified in table 7.4.1.2.3.3.2-3.

Table 7.4.1.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Individual Multicast Subscription resource was deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

Table 7.4.1.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

###### 7.4.1.2.3.4		Resource Custom Operations

None.

##### 7.4.1.2.4	Resource: Unicast Subscriptions

###### 7.4.1.2.4.1	Description

###### 7.4.1.2.4.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/unicast-subscriptions

This resource shall support the resource URI variables defined in table 7.4.1.2.4.2-1.

Table 7.4.1.2.4.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.4.1.2.4.3	Resource Standard Methods

7.4.1.2.4.3.1	POST

Table 7.4.1.2.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.4.1.2.4.3.1-2 and the response data structures and response codes specified in table 7.4.1.2.4.3.1-3.

Table 7.4.1.2.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UnicastSubscription | M | 1 |  |

Table 7.4.1.2.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UnicastSubscription | M | 1 | 201 Created |  |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.4.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-nra/<apiVersion>/unicast-subscriptions/{uniSubId} |

###### 7.4.1.2.4.4		Resource Custom Operations

None.

##### 7.4.1.2.5	Resource: Individual Unicast Subscription

###### 7.4.1.2.5.1	Description

###### 7.4.1.2.5.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/unicast-subscriptions/{uniSubId}

This resource shall support the resource URI variables defined in table 7.4.1.2.5.2-1.

Table 7.4.1.2.5.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| uniSubId | string | The unicast subscription identifier. |

###### 7.4.1.2.5.3	Resource Standard Methods

7.4.1.2.5.3.1	GET

Table 7.4.1.2.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.5.3.1-2 and the response data structures and response codes specified in table 7.4.1.2.5.3.1-3.

Table 7.4.1.2.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UnicastSubscription | M | 1 | 200 OK |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.5.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

Table 7.4.1.2.5.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

7.4.1.2.5.3.2	DELETE

Table 7.4.1.2.5.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.5.3.2-2 and the response data structures and response codes specified in table 7.4.1.2.5.3.2-3.

Table 7.4.1.2.5.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.5.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Individual Unicast Subscription resource was deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.5.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

Table 7.4.1.2.5.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

7.4.1.2.5.3.3	PUT

Table 7.4.1.2.5.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.5.3.3-2 and the response data structures and response codes specified in table 7.4.1.2.5.3.3-3.

Table 7.4.1.2.5.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UnicastSubscription | M | 1 | Represents the updated representation of the "Individual Unicast Subscription" resource. |

Table 7.4.1.2.5.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UnicastSubscription | M | 1 | 200 OK | Successful case. The "Individual Unicast Subscription" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Unicast Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.5.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

Table 7.4.1.2.5.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

7.4.1.2.5.3.4	PATCH

Table 7.4.1.2.5.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.5.3.4-2 and the response data structures and response codes specified in table 7.4.1.2.5.3.4-3.

Table 7.4.1.2.5.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UnicastSubscriptionPatch | M | 1 | Represents the modified parameters for network resource. |

Table 7.4.1.2.5.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UnicastSubscription | M | 1 | 200 OK | Successful case. The "Individual Unicast Subscription" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Unicast Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.5.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM server. |

Table 7.4.1.2.5.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM server. |

###### 7.4.1.2.5.4	Resource Custom Operations

None.

##### 7.4.1.2.6	Resource: TSC Stream Availability

###### 7.4.1.2.6.1	Description

The TSC stream availability represent for TSC stream availability discovery with the given stream specification.

###### 7.4.1.2.6.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/tsc-stream-availability

This resource shall support the resource URI variables defined in table 7.4.1.2.6.2-1.

Table 7.4.1.2.6.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |

###### 7.4.1.2.6.3	Resource Standard Methods

7.4.1.2.6.3.1	GET

This operation retrieves the TSC stream availability information. This method shall support the URI query parameters specified in table 7.4.1.2.6.3.1-1.

Table 7.4.1.2.6.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| stream-specs | array(StreamSpecification) | M | 1..N | The MAC address(es) of the source DS-TT port(s) and the destination DS-TT port(s). |

This method shall support the request data structures specified in table 7.4.1.2.6.3.1-2 and the response data structure and response codes specified in table 7.4.1.2.6.3.1-3, table 7.4.1.2.6.3.1-4 and table 7.4.1.2.6.3.1-5.

Table 7.4.1.2.6.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.6.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(TscStreamAvailability) | M | 1..N | 200 OK | List of TSC stream availability information, each including the stream specification and list of traffic specifications. This response shall include stream specification matching the query parameters provided in the request. |
| N/A | O | 0..1 | 204 No Content | Indicates no stream specification matching with the query parameters, no TSC stream avaiability information. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.1.2.6.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

Table 7.4.1.2.6.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

###### 7.4.1.2.6.4		Resource Custom Operations

None.

##### 7.4.1.2.7	Resource: TSC streams

###### 7.4.1.2.7.1	Description

The TSC streams represent the resources for TSC communication with the given stream specification.

###### 7.4.1.2.7.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/tsc-streams

This resource shall support the resource URI variables defined in table 7.4.1.2.7.2-1.

Table 7.4.1.2.7.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |

###### 7.4.1.2.7.3	Resource Standard Methods

7.4.1.2.7.3.1	GET

This method shall support the URI query parameters specified in the table 7.4.1.2.7.3.1-1.

Table 7.4.1.2.7.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| val-stream-ids | array(string) | O | 1..N | Retrieval of all the TSC stream resources managed by the NRM server or the TSC Stream resource(s) identified by the VAL Stream ID(s). |

This method shall support the request data structures specified in table 7.4.1.2.7.3.1-2 and the response data structure and response codes specified in table 7.4.1.2.7.3.1-3, table 7.4.1.2.7.3.1-4 and table 7.4.1.2.7.3.1-5.

Table 7.4.1.2.7.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.7.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(TscStreamData) | M | 1..N | 200 OK | Retrieval of TSC stream data information. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.7.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

Table 7.4.1.2.7.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

###### 7.4.1.2.7.4		Resource Custom Operations

None.

##### 7.4.1.2.8	Resource: Individual TSC Stream

###### 7.4.1.2.8.1	Description

###### 7.4.1.2.8.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/tsc-streams/{valStreamId}

This resource shall support the resource URI variables defined in table 7.4.1.2.8.2-1.

Table 7.4.1.2.8.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| valStreamId | string | The VAL Stream ID identifies the TSC stream. |

###### 7.4.1.2.8.3	Resource Standard Methods

7.4.1.2.8.3.1	GET

This operation retrieves an individual TSC stream information. This method shall support the URI query parameters specified in the table 7.4.1.2.8.3.1-1.

Table 7.4.1.2.8.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.8.3.1-2 and the response data structure and response codes specified in table 7.4.1.2.8.3.1-3, table 7.4.1.2.8.3.1-4 and table 7.4.1.2.8.3.1-5.

Table 7.4.1.2.8.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.8.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TscStreamData | M | 1 | 200 OK |  |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.8.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

Table 7.4.1.2.8.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

7.4.1.2.8.3.2	PUT

This operation create an individual TSC stream identified by VAL Stream ID. This method shall support the URI query parameters specified in the table 7.4.1.2.8.3.2-1.

Table 7.4.1.2.8.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.8.3.2-2 and the response data structure and response codes specified in table 7.4.1.2.8.3.2-3 and table 7.4.1.2.8.3.2-4.

Table 7.4.1.2.8.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TscStreamData | M | 1 | Represents the TSC stream creation request. |

Table 7.4.1.2.8.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TscStreamData | M | 1 | 201 Created | Successful case. The TSC stream is successfully created and a representation of the created "Individual TSC Stream" resource is returned in the response body. The URI of the created resource shall also be returned in an HTTP "Location" header. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.8.3.2-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-nra/<apiVersion>/tsc-streams/{valStreamId} |

7.4.1.2.8.3.3	DELETE

This operation deletes the individual TSC stream resource. This method shall support the URI query parameters specified in the table 7.4.1.2.8.3.3-1.

Table 7.4.1.2.8.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.4.1.2.8.3.3-2 and the response data structure and response codes specified in table 7.4.1.2.8.3.3-3, table 7.4.1.2.8.3.3-4 and table 7.4.1.2.8.3.3-5.

Table 7.4.1.2.8.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |

Table 7.4.1.2.8.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Individual TSC Stream resource was deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.8.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

Table 7.4.1.2.8.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative network resource management server. |

##### 7.4.1.2.9	Resource: MBS Resources

###### 7.4.1.2.9.1	Description

This resource represents the collection of MBS Resources managed by the NEF.

###### 7.4.1.2.9.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/mbs-resources

This resource shall support the resource URI variables defined in table 7.4.1.2.9.2-1.

Table 7.4.1.2.9.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |

###### 7.4.1.2.9.3	Resource Standard Methods

7.4.1.2.9.3.1	POST

This method enables to request the creation of a new MBS Resource at the NRM Server.

This method shall support the URI query parameters specified in table 7.4.1.2.9.3.1-1.

Table 7.4.1.2.9.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.4.1.2.9.3.1-2 and the response data structures and response codes specified in table 7.4.1.2.9.3.1-3.

Table 7.4.1.2.9.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsResourceReq | M | 1 | Represents the parameters to request the creation of a new MBS resource. |

Table 7.4.1.2.9.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsResourceResp | M | 1 | 201 Created | Successful case. The requested MBS resource is successfully created and a representation of the created "Individual MBS Resource" resource is returned in the response body. The URI of the created resource shall also be returned in an HTTP "Location" header. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. NOTE 2: Failure cases are described in clause 7.4.1.5. |  |  |  |  |

Table 7.4.1.2.9.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-nra/<apiVersion>/mbs-resources/{mbsResId} |

###### 7.4.1.2.9.4		Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 7.4.1.2.10	Resource: Individual MBS Resource

###### 7.4.1.2.10.1	Description

This resource represents an "Individual MBS Resource" resource managed by the NEF.

###### 7.4.1.2.10.2	Resource Definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/mbs-resources/{mbsResId}

This resource shall support the resource URI variables defined in table 7.4.1.2.10.2-1.

Table 7.4.1.2.10.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| mbsResId | string | Represents the identifier of the "Individual MBS Resource" resource. |

###### 7.4.1.2.10.3	Resource Standard Methods

7.4.1.2.10.3.1	GET

This method enables to retrieve an existing "Individual MBS Resource" resource managed by the NRM Server.

This method shall support the URI query parameters specified in table 7.4.1.2.10.3.1-1.

Table 7.4.1.2.10.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.10.3.1-2 and the response data structures and response codes specified in table 7.4.1.2.10.3.1-3.

Table 7.4.1.2.10.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.10.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSResource | M | 1 | 200 OK | Successful case. A representation of the requested "Individual MBS Resource" resource is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.10.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

Table 7.4.1.2.10.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

7.4.1.2.10.3.2	PUT

This method enables to request the update of an existing "Individual MBS Resource" resource managed by the NRM Server.

This method shall support the URI query parameters specified in table 7.4.1.2.10.3.2-1.

Table 7.4.1.2.10.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.10.3.2-2 and the response data structures and response codes specified in table 7.4.1.2.10.3.2-3.

Table 7.4.1.2.10.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSResource | M | 1 | Represents the updated representation of the "Individual MBS Resource" resource. |

Table 7.4.1.2.10.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSResource | M | 1 | 200 OK | Successful case. The "Individual MBS Resource" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual MBS Resource" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. NOTE 2: Failure cases are described in clause 7.4.1.5. |  |  |  |  |

Table 7.4.1.2.10.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

Table 7.4.1.2.10.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

7.4.1.2.10.3.3	PATCH

This method enables to request the modification of an existing "Individual MBS Resource" resource managed by the NRM Server.

This method shall support the URI query parameters specified in table 7.4.1.2.10.3.3-1.

Table 7.4.1.2.10.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.10.3.3-2 and the response data structures and response codes specified in table 7.4.1.2.10.3.3-3.

Table 7.4.1.2.10.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MBSResourcePatch | M | 1 | Represents the requested modifications to the "Individual MBS Resource" resource. |

Table 7.4.1.2.10.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MBSResource | M | 1 | 200 OK | Successful case. The "Individual MBS Resource" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual MBS Resource" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| ProblemDetails | O | 0..1 | 400 Bad Request | (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. NOTE 2: Failure cases are described in clause 7.4.1.5. |  |  |  |  |

Table 7.4.1.2.10.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

Table 7.4.1.2.10.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

7.4.1.2.10.3.4	DELETE

This method enables to request the deletion of an existing "Individual MBS Resource" resource managed by the NRM Server.

This method shall support the URI query parameters specified in table 7.4.1.2.10.3.4-1.

Table 7.4.1.2.10.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.10.3.4-2 and the response data structures and response codes specified in table 7.4.1.2.10.3.4-3.

Table 7.4.1.2.10.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.10.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual MBS Resource" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.10.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

Table 7.4.1.2.10.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

###### 7.4.1.2.10.4	Resource Custom Operations

7.4.1.2.10.4.1	Overview

Table 7.4.1.2.10.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Activate | /mbs-resources/{mbsResId}/activate | POST | Request the activation of an existing MBS Resource. |
| Deactivate | /mbs-resources/{mbsResId}/deactivate | POST | Request the deactivation of an existing MBS Resource. |

7.4.1.2.10.4.2	Operation: Activate

7.4.1.2.10.4.2.1	Description

This resource custom operation enables to request the activation of an existing MBS Resource at the NRM Server.

7.4.1.2.10.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 7.4.1.2.10.4.2.2-1 and the response data structure and response codes specified in table 7.4.1.2.10.4.2.2-2.

Table 7.4.1.2.10.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsResAct | M | 1 | Contains the parameters to request the activation of the MBS Resource. |

Table 7.4.1.2.10.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsResAct | M | 1 | 200 OK | Successful case. The activation request is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.1.2.10.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

Table 7.4.1.2.10.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

7.4.1.2.10.4.3	Operation: Deactivate

7.4.1.2.10.4.3.1	Description

This resource custom operation enables to request the deactivation of an existing MBS Resource at the NRM Server.

7.4.1.2.10.4.3.2	Operation Definition

This operation shall support the request data structures specified in table 7.4.1.2.10.4.3.2-1 and the response data structure and response codes specified in table 7.4.1.2.10.4.3.2-2.

Table 7.4.1.2.10.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MbsResDeact | M | 1 | Contains the parameters related to the deactivation of the MBS Resource. |

Table 7.4.1.2.10.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MbsResDeact | M | 1 | 200 OK | Successful case. The deactivation request is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.1.2.10.4.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

Table 7.4.1.2.10.4.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

##### 7.4.1.2.11	Resource: BDT Policy Configurations

###### 7.4.1.2.11.1	Description

This resource represents the collection of BDT Policy Configurations managed by the NRM Server.

###### 7.4.1.2.11.2	Resource definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/bdt-policy-configs

This resource shall support the resource URI variables defined in table 7.4.1.2.11.2-1.

Table 7.4.1.2.11.2-1: Resource URI variables for resource "BDT Policy Configurations"

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |

###### 7.4.1.2.11.3	Resource methods

7.4.1.2.11.3.1	POST

This method enables to request the creation of a new "Individual BDT Policy Configuration" resource at the NRM Server

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in the table 7.4.1.2.11.3.1-1, table 7.4.1.2.11.3.1-2 and table 7.4.1.2.11.3.1-3.

Table 7.4.1.2.11.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

Table 7.4.1.2.11.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| BdtPolConfig | M | 1 | Represents the parameters to request the creation of a new "BDT Policy Configurations" resource. |

Table 7.4.1.2.11.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| BdtPolConfig | M | 1 | 201 Created | Successful case. The requested BDT Policy configuration resource is successfully created and a representation of the created "Individual BDT Policy Configuration" resource is returned in the response body. The URI of the created resource shall also be returned in an HTTP "Location" header. |
| n/a |  |  | 204 No Content | The request is successfully received. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.11.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-nra/<apiVersion>/bdt-policy-configs/{bdtPolConfigId} |

##### 7.4.1.2.12	Resource: Individual BDT Policy Configuration

###### 7.4.1.2.12.1	Description

This resource represents an "Individual BDT Policy Configuration" resource managed by the NRM Server.

###### 7.4.1.2.12.2	Resource definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/bdt-policy-configs/{bdtPolConfigId}

This resource shall support the resource URI variables defined in table 7.4.1.2.12.2-1.

Table 7.4.1.2.12.2-1: Resource URI variables for resource "Individual BDT Policy Config"

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| bdtPolConfigId | string | Represents the identifier of the "Individual BDT Policy Configuration" resource. |

###### 7.4.1.2.12.3	Resource methods

7.4.1.2.12.3.1	GET

The GET method allows to read an "Individual BDT Policy Configuration" resource to obtain details of an active resource BDT policy configuration.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in the table 7.4.1.2.12.3.1-1 and table 7.4.1.2.12.3.1-2.

Table 7.4.1.2.12.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.12.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| BdtPolConfig | M | 1 | 200 OK | Successful case. A representation of the requested "Individual BDT Policy Configuration" resource is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative network resource management server. |

Table 7.4.1.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative network resource management server. |

7.4.1.2.12.3.2	PUT

This method enables a service consumer to request the update of an existing "Individual BDT Policy Configuration" resource managed by the NRM Server.

This method shall support the URI query parameters specified in table 7.4.1.2.12.3.2-1.

Table 7.4.1.2.12.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.12.3.2-2 and the response data structures and response codes specified in table 7.4.1.2.12.3.2-3.

Table 7.4.1.2.12.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| BdtPolConfig | M | 1 | Represents the updated representation of the "Individual BDT Policy Configuration" resource. |

Table 7.4.1.2.12.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| BdtPolConfig | M | 1 | 200 OK | Successful case. The targeted "Individual BDT Policy Configuration" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The targeted "Individual BDT Policy Configuration" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.12.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

Table 7.4.1.2.13.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

7.4.1.2.12.3.3	PATCH

This method enables a service consumer to request the modification of an existing "Individual BDT Policy Configuration" resource managed by the NRM Server.

This method shall support the URI query parameters specified in table 7.4.1.2.12.3.3-1.

Table 7.4.1.2.12.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.2.12.3.3-2 and the response data structures and response codes specified in table 7.4.1.2.12.3.3-3.

Table 7.4.1.2.12.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| BdtPolConfigPatch | M | 1 | Represents the requested modifications to the "Individual BDT Policy Configuration" resource. |

Table 7.4.1.2.12.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| BdtPolConfig | M | 1 | 200 OK | Successful case. The targeted "Individual BDT Policy Configuration" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The targeted "Individual BDT Policy Configuration" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM Server. Redirection handling is defined in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.12.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

Table 7.4.1.2.12.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains an alternative URI of the resource located in an alternative NRM Server. |

7.4.1.2.12.3.4	DELETE

The DELETE method deletes the resource related to the BDT policy configuration.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in the table 7.4.1.12.3.4-1 and table 7.4.1.2.12.3.4-2.

Table 7.4.1.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual BDT policy Configuration" resource was deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative network resource management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative network resource management server. |

Table 7.4.1.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative network resource management server. |

###### 7.4.1.2.12.4		Resource Custom Operations

None.

##### 7.4.1.2.13	Resource: Unified Traffic Pattern Subscriptions

###### 7.4.1.2.13.1	Description

This resource represents the collection of Unified Traffic Patterns Subscriptions managed by the NRM Server.

###### 7.4.1.2.13.2	Resource definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/unif-traff-subscriptions

This resource shall support the resource URI variables defined in table 7.4.1.2.13.2-1.

Table 7.4.1.2.13.2-1: Resource URI variables for resource " Unified Traffic Pattern Subscriptions"

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |

###### 7.4.1.2.13.3	Resource methods

7.4.1.2.13.3.1	POST

This method enables to request the creation of a new "Individual Unified Traffic Pattern Subscription" resource at the NRM Server

This method shall support the URI query parameters specified in the table 7.4.1.2.13.3.1-1.

Table 7.4.1.2.13.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

This method shall support the request and response data structures and response codes specified in the table 7.4.1.2.13.3.1-2 and table 7.4.1.2.13.3.1-3.

Table 7.4.1.2.13.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UnfTrafficSubc | M | 1 | Represents the parameters to request the creation of a new "Individual Unified Traffic Pattern Subscription" resource. |

Table 7.4.1.2.13.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UnfTrafficSubc | M | 1 | 201 Created | Successful case. The requested Unified Traffic Pattern Subscription is successfully created and a representation of the created "Individual Unified Traffic Pattern Subscription" resource is returned in the response body. The URI of the created resource shall also be returned in an HTTP "Location" header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.13.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-nra/<apiVersion>/ unif-traff-subscriptions/{unfTrffSubId} |

##### 7.4.1.2.14	Resource: Individual Unified Traffic Pattern Subscription

###### 7.4.1.2.14.1	Description

This resource represents an "Individual Unified Traffic Pattern Subscription" resource managed by the NRM Server.

###### 7.4.1.2.14.2	Resource definition

Resource URI: {apiRoot}/ss-nra/<apiVersion>/unif-traff-subscriptions/{unfTrffSubId}

This resource shall support the resource URI variables defined in table 7.4.1.2.14.2-1.

Table 7.4.1.2.14.2-1: Resource URI variables for resource "Individual BDT Policy Config"

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| unfTrffSubId | string | Represents the identifier of the "Individual Unified Traffic Pattern Subscription" resource. |

###### 7.4.1.2.14.3	Resource methods

7.4.1.2.14.3.1	GET

The GET method allows to retrieve an existing "Individual Unified Traffic Pattern Subscription" resource at the NRM Server.

This method shall support the URI query parameters specified in the table 7.4.1.2.14.3.1-1.

Table 7.4.1.2.14.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

This method shall support the request and response data structures and response codes specified in the table 7.4.1.2.14.3.1-2 and table 7.4.1.2.14.3.1-3.

Table 7.4.1.2.14.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.14.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UnfTrafficSubc | M | 1 | 200 OK | Successful case. The requested "Individual Unified Traffic Pattern Subscription" resource is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.14.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM server. |

Table 7.4.1.2.14.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM server. |

7.4.1.2.14.3.2	PUT

The PUT method allows to update an existing "Individual Unified Traffic Pattern Subscription" resource at the NRM Server.

This method shall support the URI query parameters specified in the table 7.4.1.2.14.3.2-1.

Table 7.4.1.2.14.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

This method shall support the request and response data structures and response codes specified in the table 7.4.1.2.14.3.2-2 and table 7.4.1.2.14.3.2-3.

Table 7.4.1.2.14.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UnfTrafficSubc | M | 1 | Contains the updated representation of the "Individual Unified Traffic Pattern Subscription" resource. |

Table 7.4.1.2.14.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UnfTrafficSubc | M | 1 | 200 OK | Successful case. The "Individual Unified Traffic Pattern Subscription" resource is successfully updated and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Unified Traffic Pattern Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.14.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM server. |

Table 7.4.1.2.14.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM server. |

7.4.1.2.14.3.3	PATCH

The PATCH method allows to modify an existing "Individual Unified Traffic Pattern Subscription" resource at the NRM Server.

This method shall support the URI query parameters specified in the table 7.4.1.2.14.3.3-1.

Table 7.4.1.2.14.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

This method shall support the request and response data structures and response codes specified in the table 7.4.1.2.14.3.3-2 and table 7.4.1.2.14.3.3-3.

Table 7.4.1.2.14.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UnfTrafficSubcPatch | M | 1 | Contains the requested modifications to the "Individual Unified Traffic Pattern Subscription" resource. |

Table 7.4.1.2.14.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UnfTrafficSubc | M | 1 | 200 OK | Successful case. The "Individual Unified Traffic Pattern Subscription" resource is successfully modified and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Unified Traffic Pattern Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.14.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM server. |

Table 7.4.1.2.14.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM server. |

7.4.1.2.14.3.4	DELETE

The DELETE method allows to delete an existing "Individual Unified Traffic Pattern Subscription" resource at the NRM Server.

This method shall support the URI query parameters specified in the table 7.4.1.2.14.3.4-1.

Table 7.4.1.2.14.3.4-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

This method shall support the request and response data structures and response codes specified in the table 7.4.1.2.14.3.4-2 and table 7.4.1.2.14.3.4-3.

Table 7.4.1.2.14.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.1.2.14.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Unified Traffic Pattern Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2.14.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM server. |

Table 7.4.1.2.14.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative NRM server. |

###### 7.4.1.2.14.4		Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 7.4.1.2A	Custom Operations without associated resources

##### 7.4.1.2A.1	Overview

The structure of the custom operation URIs of the SS_NetworkResourceAdaptation API is shown in Figure 7.4.1.2A.1-1.

Figure 7.4.1.2A.1-1: Custom operation URI structure of the SDD_Transmission API

Table 7.4.1.2A.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the SS_NetworkResourceAdaptation API.

Table 7.4.1.2A.1-1: Custom operations without associated resources

| Custom operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| RelTransRequest | /request-rel-trans | POST | Enables a service consumer to request reliable transmission service. |

The custom operations shall support the URI variables defined in table 7.4.1.2A.1-2.

Table 7.4.1.2A.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.4.1.1. |

##### 7.4.1.2A.2	Operation: RelTransRequest

###### 7.4.1.2A.2.1	Description

The custom operation enables a service consumer to request reliable transmission service to the NRM Server.

###### 7.4.1.2A.2.2	Operation Definition

This operation shall support the request data structures specified in table 7.4.1.2A.2.2-1 and the response data structures and response codes specified in table 7.4.1.2A.2.2-2.

Table 7.4.1.2A.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RelTransReq | M | 1 | Contains the parameters to request reliable transmission service. |

Table 7.4.1.2A.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The reliable transmission service request is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NRM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NRM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2] |
| NOTE: The manadatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.1.2A.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative NRM Server. |

Table 7.4.1.2A.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative NRM Server. |

#### 7.4.1.3	Notifications

##### 7.4.1.3.1	General

Table 7.4.1.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Notify_UP_Delivery_Mode | {notifUri} (NOTE) | POST | Report User Plane delivery mode. |
| BDT_Negotiation_Notification | {notifUri} | POST | Report BDT negotiation related event(s). |
| Unified_Traffic_Pattern_Notification | {notifUri} | POST | Report UE unified traffic pattern monitoring and management event(s). |
| NOTE: This notification is used for both multicast resource management for EPS and multicast/broadcast resource management for 5GS. The "notifUri" is hence either provided via the Request_Multicast_Resource service operation for the former case, or the CreateMBSResource/UpdateMBSResource service operation for the latter case. |  |  |  |

##### 7.4.1.3.2	Notify_UP_Delivery_Mode

###### 7.4.1.3.2.1	Description

###### 7.4.1.3.2.2	Notification definition

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.4.1.3.2.2-1.

Table 7.4.1.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.3.2.2-2 and the response data structures and response codes specified in table 7.4.1.3.2.2-3.

Table 7.4.1.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UserPlaneNotification | M | 1 | Represents the content of the user plane delivery mode notification. |

Table 7.4.1.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.1.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.4.1.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

##### 7.4.1.3.3	BDT_Negotiation_Notification

###### 7.4.1.3.3.1	Description

###### 7.4.1.3.3.2	Notification definition

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.4.1.3.3.2-1.

Table 7.4.1.3.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.3.3.2-2 and the response data structures and response codes specified in table 7.4.1.3.3.2-3.

Table 7.4.1.3.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| BdtNotification | M | 1 | Represents the negotiated BDT policy notification. |

Table 7.4.1.3.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.1.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.4.1.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

##### 7.4.1.3.4	Unified_Traffic_Pattern_Notification

###### 7.4.1.3.4.1	Description

###### 7.4.1.3.4.2	Notification definition

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.4.1.3.4.2-1.

Table 7.4.1.3.4.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.1.3.4.2-2 and the response data structures and response codes specified in table 7.4.1.3.4.2-3.

Table 7.4.1.3.4.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UnfTrafficUpdNotif | M | 1 | Represents the Unified Traffic Pattern Notification |

Table 7.4.1.3.4.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.1.3.4.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.4.1.3.4.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.4.1.4	Data Model

##### 7.4.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.4.1.4.1-1 specifies the data types defined specifically for the SS_NetworkResourceAdaptation API service.

Table 7.4.1.4.1-1: SS_NetworkResourceAdaptation API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AppTraffDesc | 7.4.1.4.2.29 | Represents the application traffic descriptor. |  |
| BdtPolConfig | 7.4.1.4.2.18 | Represents the parameters related to the BDT Policy Configuration Resource. |  |
| BdtNotification | 7.4.1.4.2.20 | Represents the negotiated BDT Policy notification. |  |
| DeliveryMode | 7.4.1.4.3.2 | Indicates the delivery mode. |  |
| GeoArea | 7.4.1.4.2.19 | Represents the Geographic Area. |  |
| EpsMbmsInfo | 7.4.1.4.3.0A | Represents the EPS MBMS bearer information. |  |
| ManagementSubc | 7.4.1.4.2.25 | Represents the unified traffic pattern management subscription details. |  |
| MbsResAct | 7.4.1.4.2.16 | Represents the parameters related to the activation of the MBS Resource. |  |
| MbsResDeact | 7.4.1.4.2.17 | Represents the parameters related to the deactivation of the MBS Resource. |  |
| MbsResource | 7.4.1.4.2.12 | Represents an MBS Resource. |  |
| MbsResourcePatch | 7.4.1.4.2.15 | Represents the parameters to request the modification of an MBS Resource. |  |
| MbsResourceReq | 7.4.1.4.2.11 | Represents the parameters to request the creation of an MBS Resource. |  |
| MbsResourceRespInfo | 7.4.1.4.2.13 | Represents NRM Server side information related to the MBS Resource. |  |
| MbsResourceResp | 7.4.1.4.2.14 | Represents a response to an MBS Resource creation/modification request. |  |
| MulticastSubscription | 7.4.1.4.2.2 | Represents a multicast subscription. |  |
| NetSysIndicator | 7.4.1.4.3.4 | Represents the network system indicator, i.e. 5GS, EPS or both. |  |
| NrmEvent | 7.4.1.4.3.3 | Indicates the NRM event. |  |
| NrmEventNotification | 7.4.1.4.2.5 | Represents a notification on an individual User Plane event. |  |
| RelTransReq | 7.4.1.4.2.28 | Represents the parameters to request reliable transmission service. |  |
| ServiceAnnoucementMode | 7.4.1.4.3.1 | Indicates the service announcement mode. |  |
| StreamSpecification | 7.4.1.4.2.9 | Represents a stream specification that includes MAC addresses of the source and destination DS-TT ports. |  |
| TrafficPatternIndication | 7.4.1.4.3.7 | Represents the traffic pattern indication requests. |  |
| TrafficPatternConfig | 7.4.1.4.2.26 | Represents the traffic pattern configuration of the VAL service. |  |
| TrafficSpecification | 7.4.1.4.2.10 | Represents the traffic class supported by the DS-TTs and available end-to-end maximum latency value. |  |
| TrafficSpecInformation | 7.4.1.4.2.7 | Represents the traffic class supported by the DS-TTs and available end-to-end latency value and Priority Code Point (PCP) value. |  |
| TscStreamAvailability | 7.4.1.4.2.8 | Represents the TSC stream availability information including the stream specification and list of traffic specifications. |  |
| TscStreamData | 7.4.1.4.2.6 | Represents the TSC stream data information. |  |
| UnfTrafficSubc | 7.4.1.4.2.23 | Represents the Unified Traffic Pattern Subscription. |  |
| UnfTrafficSubcPatch | 7.4.1.4.2.24 | Represents the Unified Traffic Pattern Subscription Patch. |  |
| UnfTrafficUpdNotif | 7.4.1.4.2.27 | Represents the Unified Traffic Pattern Notification. |  |
| UnicastSubscription | 7.4.1.4.2.3 | Represents a network resource adaptation subscription. |  |
| UnicastSubscriptionPatch | 7.4.1.4.2.31 | Represents the requested modifications to a network resource adaptation subscription. |  |
| UserPlaneNotification | 7.4.1.4.2.4 | Represents a notification on User Plane events. |  |
| ValUeAddrInfo | 7.4.1.4.2.30 | Represents VAL UE address information. |  |

Table 7.4.1.4.1-2 specifies data types re-used by the SS_NetworkResourceAdaptation API service.

Table 7.4.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ConnInfo | 3GPP TS 29.548 [35] | Represents the data transmission connection information. |  |
| DateTime | 3GPP TS 29.122 [3] | Used to represent the subscription duration. |  |
| DurationSec | 3GPP TS 29.122 [3] | Used to represent the duration in seconds. |  |
| ExternalGroupId | 3GPP TS 29.122 [3] | Represents an External Group Identifier. |  |
| ExternalMbsServiceArea | 3GPP TS 29.571 [21] | Represents an MBS Service Area. |  |
| GeographicArea | 3GPP TS 29.572 [31] | Represents Geographical area. |  |
| CivicAddress | 3GPP TS 29.572 [31] | Represents civic address of an area. |  |
| Ipv4Addr | 3GPP TS 29.571 [21] | Used to identify the IPv4 address. |  |
| Ipv6Addr | 3GPP TS 29.571 [21] | Used to identify the IPv6 address. |  |
| LocalMbmsInfo | 3GPP TS 29.486 [27] | Used to represent the local MBMS information. |  |
| MacAddr48 | 3GPP TS 29.571 [21] | Used to identify a MAC address. |  |
| MbmsLocArea | 3GPP TS 29.122 [3] | Used to indicate the requested area of the MBMS bearer. |  |
| MbsServiceInfo | 3GPP TS 29.571 [21] | Represents MBS Service Information. |  |
| MbsSession | 3GPP TS 29.571 [21] | Represents an MBS Session. |  |
| MbsSessionId | 3GPP TS 29.571 [21] | Represents the identifier of an MBS Session. |  |
| Port | 3GPP TS 29.122 [3] | Used to identify the port. |  |
| ProblemDetails | 3GPP TS 29.571 [21] | Represents error related information. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [3] | Used to define the schedules for traffic pattern configurations. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the supported optional features of the API. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Represents a time interval. |  |
| TransportProtocol | 3GPP TS 29.558 [43] | Represents the transport layer protocol. |  |
| UsageThreshold | 3GPP TS 29.122 [3] | Represents the data usage threshold. |  |
| Uint32 | 3GPP TS 29.571 [21] | Represents an unsigned integer. |  |
| Uinteger | 3GPP TS 29.571 [21] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.122 [3] | Used to indicate the notification URI. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to identify either a VAL User ID or a VAL UE ID. |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] | Used to indicate the configuration parameters to set up notification delivery over Websocket protocol. |  |

##### 7.4.1.4.2	Structured data types

###### 7.4.1.4.2.1	Introduction

###### 7.4.1.4.2.2	Type: MulticastSubscription

Table 7.4.1.4.2.2-1: Definition of type MulticastSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valGroupId | string | M | 1 | The identity of the group that the MBMS bearer is requested for. |  |
| anncMode | ServiceAnnoucementMode | M | 1 | Indicates whether the service announcement is sent by NRM Server or by the service consumer. |  |
| multiQosReq | string | M | 1 | The QoS requirement for the multicast. |  |
| locArea | MbmsLocArea | O | 0..1 | Indicate the area where the MBMS bearer is requested for. |  |
| duration | DateTime | O | 0..1 | Identifies the absolute time at which the subscription resource is considered to expire. When omitted in the request, it indicates the resource is requested to be valid forever by the service consumer. When omitted in the response, it indicates the resource is set to valid forever by the service consumer. |  |
| tmgi | Uint32 | O | 0..1 | TMGI. Shall be provided by the NRM server if announcement mode is set to VAL. |  |
| localMbmsInfo | LocalMbmsInfo | O | 0..1 | Contains the local MBMS information. This attribute may be present only within the trust domain. | LocalMBMS |
| localMbmsActInd | boolean | O | 0..1 | Set to true to indicate that the local MBMS is activated. Set to false or omitted otherwise. | LocalMBMS |
| notifUri | Uri | M | 1 | Identifies the notification URI where the NRM notification shall be sent to. |  |
| reqTestNotif | boolean | O | 0..1 | Set to true to request the VAE server to send a test notification as defined in clause 6.6. Set to false or omitted otherwise. | Notification_test_event |
| wsNotifCfg | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 6.6. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features defined in table 7.4.1.6-1. This parameter may be supplied by the service consumer in the POST request that requests the creation of a Multicast Subscription resource and shall be supplied in the corresponding POST response if it was present in the request. |  |
| upIpv4Addr | Ipv4Addr | O | 0..1 | Ipv4address of the user plane. (NOTE) |  |
| upIpv6Addr | Ipv6Addr | O | 0..1 | Ipv6address of the user plane. (NOTE) |  |
| upPortNum | Port | O | 0..1 | UDP port number of the user plane. |  |
| radioFreqs | array(Uint32) | O | 1..N | The radio frequencies which may be provided by the NRM server. |  |
| NOTE: At least one of upIpv4Addr or upIpv6Addr shall be provided by the NRM server. |  |  |  |  |  |

###### 7.4.1.4.2.3	Type: UnicastSubscription

Table 7.4.1.4.2.3-1: Definition of type UnicastSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUes | array(string) | C | 1..N | Represents the list of the VAL UE identifier(s). (NOTE) | enNB |
| valGroupId | string | C | 0..1 | Represents the VAL group identifier. (NOTE) | enNB |
| valTgtUe | ValTargetUe | C | 0..1 | Contains the identity of the VAL user or VAL UE that the bearer is requested for. (NOTE) |  |
| bdtId | string | O | 0..1 | Represents the BDT identifier. | SEALDD_BDT |
| uniQosReq | string | O | 0..1 | Contains the QoS requirement. |  |
| duration | DateTime | O | 0..1 | Identifies the absolute time at which the subscription resource is considered to expire. When omitted in the request, it indicates the resource is requested to be valid forever by the service consumer. When omitted in the response, it indicates the resource is set to valid forever by the service consumer. |  |
| notifUri | Uri | M | 1 | Identifies the notification URI where the NRM notification shall be sent to. |  |
| reqTestNotif | Boolean | O | 0..1 | Set to true to request the VAE server to send a test notification as defined in clause 6.6. Set to false or omitted otherwise. | Notification_test_event |
| wsNotifCfg | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 6.6. | Notification_websocket |
| suppFeat | SupportedFeatures | O | 1 | This parameter may be supplied by service consumer in the POST request that request the creation of a Unicast Subscription resource and may be supplied in the reply of corresponding request. |  |
| NOTE: When the "enNB" feature is supported, these attributes are mutually exclusive and only one of them shall be present. When the "enNB" feature is not supported, only the "valTgtUe" attribute is applicable and shall be present. |  |  |  |  |  |

###### 7.4.1.4.2.4	Type: UserPlaneNotification

Table 7.4.1.4.2.4-1: Definition of type UserPlaneNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifId | Uri | M | 1 | Contains the URI of the resource (e.g., "Individual Multicast Subscription" resource, "Individual MBS Resource" resource) to which the notification is related. |  |
| eventNotifs | array(NrmEventNotification) | M | 1..N | Contains the notifications about Individual Events. The only applicable event within the "event" attribute of the NrmEventNotification data structure is the "UP_DELIVERY_MODE" event. |  |

###### 7.4.1.4.2.5	Type: NrmEventNotification

Table 7.4.1.4.2.5-1: Definition of type NrmEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | NrmEvent | M | 1 | Contains the reported event. |  |
| ts | DateTime | M | 1 | Contains the time at which the event is observed. |  |
| deliveryMode | DeliveryMode | C | 0..1 | Indicates mode of delivery of the user data to the UE(s), i.e, via unicast mode, EPS MBMS multicast mode, 5GS MBS multicast mode or 5GS MBS broadcast mode. This attribute shall be present only if the "event" attribute is set to "UP_DELIVERY_MODE". |  |
| streamIds | array(string) | O | 1..N | Indicates the idenitifier(s) of the unicast/multicast stream(s) currently being used or the unicast/multicast stream(s) to be used to deliver the media data that is currently delivered over multicast/unicast. This attribute may be present only if the "event" attribute is set to "UP_DELIVERY_MODE" and the NRM Server already has the streams available. |  |
| stopUniStreamIds | array(string) | O | 1..N | Contains the unicast streams to be stopped as data delivery is switched to multicast delivery. This attribute may be present only if the "event" attribute is set to "UP_DELIVERY_MODE" and the "deliveryMode" attribute is set to "MULTICAST", "MBS_MULTICAST" or "MBS_BROADCAST". | enNB1 |
| mbsResServiceArea | ExternalMbsServiceArea | O | 0..1 | Contains the Service Area of the MBS Resource where the reported delivery mode is applied. This attribute may be present only if the "deliveryMode" attribute is present and set to either "MBS_MULTICAST" or "MBS_BROADCAST". | SEAL_3 |
| valUeIds | array(string) | O | 1..N | Contains the identifier(s) of the VAL UE(s) for which unicast delivery is applied. This attribute may be present only if the "deliveryMode" attribute is present and set to "UNICAST". | SEAL_3_Ext |

###### 7.4.1.4.2.6	Type: TscStreamData

Table 7.4.1.4.2.6-1: Definition of type TscStreamData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| streamSpec | StreamSpecification | M | 1 | Stream specification includes MAC addresses of the source and destination DS-TT ports. |  |
| trafficSpecInfo | TrafficSpecInformation | M | 1 | Traffic Specification Information includes Priority Code Point (PCP), MaxFrameInterval, MaxFrameSize, MaxIntervalFrames, MaxLatency, etc. (e.g. as described in IEEE 802.1Qcc [29] in clause 46.2). |  |

###### 7.4.1.4.2.7	Type: TrafficSpecInformation

Table 7.4.1.4.2.7-1: Definition of type TrafficSpecInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pcpValue | Uint32 | M | 1 | The Priority Code Point (PCP) value identify the traffic class, with value between 0 to 7. |  |
| maxFramInt | DurationSec | M | 1 | Maximum Frame Interval. |  |
| maxFramSize | Uint32 | M | 1 | Maximum frame size will transmit, excluding the overhead. |  |
| maxIntFrames | Uint32 | M | 1 | Maximum interval frames. |  |
| maxLatency | Uint32 | M | 1 | Indicates the end-to-end latency (including the UE-DS-TT residence times, UPF residence time, and propagation delays) in milliseconds. |  |

###### 7.4.1.4.2.8	Type: TscStreamAvailability

Table 7.4.1.4.2.8-1: Definition of type TscStreamAvailability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| streamSpec | StreamSpecification | M | 1 | Stream specification includes MAC addresses of the source and destination DS-TT ports. |  |
| trafficSpecs | array(TrafficSpecification) | M | 1..N | The traffic classes supported by the DS-TTs and available end-to-end maximum latency values. |  |

###### 7.4.1.4.2.9	Type: StreamSpecification

Table 7.4.1.4.2.9-1: Definition of type StreamSpecification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| srcMacAddr | MacAddr48 | M | 1 | The MAC address of the source DS-TT port. |  |
| dstMacAddr | MacAddr48 | M | 1 | The MAC address of the destination DS-TT port. |  |

###### 7.4.1.4.2.10	Type: TrafficSpecification

Table 7.4.1.4.2.10-1: Definition of type TrafficSpecification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trafficClass | Uint32 | M | 1 | The traffic class supported by the DS-TTs with value between 0 to 7. |  |
| e2eMaxLatency | Uinteger | M | 1 | Indicates the end to end maximum latency (including the UE-DS-TT residence times, UPF residence time, and propagation delays), in the units of milliseconds. |  |

###### 7.4.1.4.2.11	Type: MBSResourceReq

Table 7.4.1.4.2.11-1: Definition of type MBSResourceReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsResource | MBSResource | M | 1 | Contains the parameters to request the creation of the MBS Resource. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.4.1.6. This attribute shall be present only when feature negotiation needs to take place. |  |

###### 7.4.1.4.2.12	Type: MBSResource

Table 7.4.1.4.2.12-1: Definition of type MBSResource

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valGroupId | string | C | 0..1 | Contains the identity of the VAL group to which the MBS Resource is related. (NOTE 1) |  |
| valUeIdsList | array(ValTargetUe) | C | 1..N | Contains the list of the identitier(s) of the VAL UE(s) to which the MBS Resource is related. (NOTE 1) |  |
| servAnmtMode | ServiceAnnoucementMode | M | 1 | Contain the MBS Service Announcement mode to be used, i.e. whether the MBS Service Announcement is delivered by the NRM Server or the service consumer. |  |
| mbsResServInfo | MbsServiceInfo | M | 1 | Contains the MBS Service Information for the MBS Resource. |  |
| mbsResServiceArea | ExternalMbsServiceArea | O | 0..1 | Contains the Service Area of the MBS Resource. |  |
| notifUri | Uri | M | 1 | Contains the URI towards which the user plane delivery mode notifications shall be sent. |  |
| netSysInd | NetSysIndicator | O | 0..1 | Indicates whether the MBS Resource requires multicast/broadcast services from 5GS, EPS or both. |  |
| localMbmsInfo | LocalMbmsInfo | O | 0..1 | Contains the local MBMS information. This attribute may only be provided if the "netSysInd" attribute is set to "EPS" or "5GS_AND_EPS". (NOTE 2) |  |
| localMbmsActInd | boolean | O | 0..1 | Set to true by the service consumer to indicate that the local MBMS is activated. Set to false or omitted otherwise. (NOTE 2) |  |
| mbsResRespInfo | MBSResourceRespInfo | O | 0..1 | Represents the NRM Server side information related to the created MBS Resource. This attribute may only be present in the response to the request to create/update/modify/retrieve the MBS Resource. |  |
| NOTE 1: These attributes are mutually exclusive. Either one of them shall be present. NOTE 2: These attributes are mutually exclusive. Either one of them may be present. |  |  |  |  |  |

###### 7.4.1.4.2.13	Type: MBSResourceRespInfo

Table 7.4.1.4.2.13-1: Definition of type MBSResourceRespInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbs5gSessionId | MbsSessionId | O | 0..1 | Contains the identifier of the MBS session (i.e. TMGI and/or SSM, in addition to the NID in the case an SNPN). This attribute may not be present if the MBS Service Announcement is delivered by the NRM Server (i.e. the "servAnmtMode" attribute in the corresponding request is set to "NRM"). |  |
| mbmsBearerId | MbsSessionId | O | 0..1 | Contains the identifier of the MBMS bearer (i.e. TMGI). This attribute may not be present if the MBS Service Announcement is delivered by the NRM Server (i.e. the "servAnmtMode" attribute in the corresponding request is set to "NRM"). |  |
| upIpv4Addr | Ipv4Addr | C | 0..1 | Contains the user plane IPv4 address of the NRM Server to be used by the service consumer for downlink packets delivery. (NOTE 1) |  |
| upIpv6Addr | Ipv6Addr | C | 0..1 | Contains the user plane IPv6 address of the NRM Server to be used by the service consumer for downlink packets delivery. (NOTE 1) |  |
| upPortNum | Port | O | 0..1 | Contains the user plane UDP port number to be used by service consumer for downlink packets delivery. |  |
| mbs5GInfo | MbsSession | O | 0..1 | Contains MBS related configuration information. This attribute may only be present if the network system used for multicast/broadcast services is 5GS or both 5GS and EPS. |  |
| epsMbmsInfo | EpsMbmsInfo | O | 0..1 | Contains MBMS related configuration information. This attribute may only be present if the network system used for multicast/broadcast services is EPS or both 5GS and EPS. |  |
| NOTE 1: At least one of these attributes shall be present. |  |  |  |  |  |

###### 7.4.1.4.2.14	Type: MBSResourceResp

Table 7.4.1.4.2.14-1: Definition of type MBSResourceResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsResource | MBSResource | M | 1 | Contains the the created MBS Resource. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.4.1.6. This attribute shall be present only when feature negotiation is taking place. |  |

###### 7.4.1.4.2.15	Type: MBSResourcePatch

Table 7.4.1.4.2.15-1: Definition of type MBSResourcePatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsResServInfo | MbsServiceInfo | O | 0..1 | Contains the updated MBS Service Information for the MBS Resource. |  |
| mbsResServiceArea | ExternalMbsServiceArea | O | 0..1 | Contains the updated Service Area of the MBS Resource. |  |
| notifUri | Uri | O | 0..1 | Contains the URI towards which the user plane delivery mode notifications shall be sent. |  |
| localMbmsInfo | LocalMbmsInfo | O | 0..1 | Contains the local MBMS information. This attribute may only be provided if the "netSysInd" attribute is set to "EPS" or "5GS_AND_EPS". (NOTE) |  |
| localMbmsActInd | boolean | O | 0..1 | Set to true by the service consumer to indicate that the local MBMS is activated. Set to false or omitted otherwise. (NOTE) |  |
| NOTE: These attributes are mutually exclusive. Either one of them may be present. |  |  |  |  |  |

###### 7.4.1.4.2.16	Type: MbsResAct

Table 7.4.1.4.2.16-1: Definition of type MbsResAct

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbs5gSessionId | MbsSessionId | M | 1 | Contains the identifier of the MBS session to be activated. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.4.1.6. This attribute shall be present only when feature negotiation needs to take place. |  |

###### 7.4.1.4.2.17	Type: MbsResDeact

Table 7.4.1.4.2.17-1: Definition of type MbsResDeact

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbs5gSessionId | MbsSessionId | M | 1 | Contains the identifier of the MBS session to be deactivated. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.4.1.6. This attribute shall be present only when feature negotiation needs to take place. |  |

###### 7.4.1.4.2.18	Type: BdtPolConfig

Table 7.4.1.4.2.18-1: Definition of type BdtPolConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServId | string | M | 1 | Identifies a VAL Service. |  |
| valGroupId | string | C | 0..1 | Identifies a group of UEs. (NOTE) |  |
| valUeIds | array(string) | C | 1..N | Represents a List of VAL UEs whose BDT policy is provided. (NOTE) |  |
| dataVolUe | UsageThreshold | M | 1 | Represents the Data volume per UE for BDT. |  |
| desiredTimeWindow | TimeWindow | M | 1 | Represents the desired time window for BDT. |  |
| grantTimeWindow | TimeWindow | C | 0..1 | Represents the granted time window by the 3GPP network for BDT. This attribute may be present only in the responses. |  |
| desiredGeoArea | GeoArea | M | 1 | Represents the desired geographical area for BDT. |  |
| expTime | DurationSec | O | 0..1 | Represents the expiration time for the background data transfer request. |  |
| policyGuidance | array(PolicyGuidance) | O | 0..N | Represents a list of policy guidance. If no value is included, NRM server may use local policies. |  |
| bdtPolRefId | string | C | 0..1 | Identifies a BDT policy reference identifier. This attribute may be present only in the responses. |  |
| notifUri | Uri | M | 1 | Identifies the destination notification URI where the NRM notification shall be sent to. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |
| NOTE: Either one of the "valGroupId" or "valUeIds" attribute shall be present. |  |  |  |  |  |

###### 7.4.1.4.2.19	Type: GeoArea

Table 7.4.1.4.2.19-1: Definition of type GeoArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| geographicalAreas | array(GeographicArea) | O | 1..N | Represents a list of Geographical area. |  |
| civicAddress | array(CivicAddress) | O | 1..N | Represents a list of Civic address of an area. |  |

###### 7.4.1.4.2.20	Type: BdtNotification

Table 7.4.1.4.2.20-1: Definition of type BdtNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifId | Uri | M | 1 | Contains the URI of the " Individual BDT Policy Configuration" resource to which this notification is related. |  |
| bdtConfigId | string | M | 0..1 | Identifies the "Individual BDT Policy Configuration" to which the notification is related. This attribute shall be set to the same value as the "bdtPolConfigId" variable (see clause 7.4.1.2.12.2) of the resource URI provided within the "notifId" attribute. |  |
| grantTimeWindow | TimeWindow | C | 0..1 | Represents the updated time window granted by the 3GPP network for BDT. This attribute shall be present only if the "bdtPolicyRemoveInd" attribute value is not present, or preset and set to "false". |  |
| bdtPolicyRemoveInd | boolean | O | 0..1 | Indicates whether the BDT policy at the 3GPP network correponding to the BDT policy reference identifier has been removed. When present, it shall be set as follows: - "true": The BDT policy was removed by the 3GPP network. - "false": the BDT policy was not removed at the 3GPP network. - Default value when omitted is "false". |  |

###### 7.4.1.4.2.21	Type: BdtPolConfigPatch

Table 7.4.1.4.2.21-1: Definition of type BdtPolConfigPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataVolUe | UsageThreshold | O | 0..1 | Represents the Data volume per UE for BDT. |  |
| desiredTimeWindow | TimeWindow | O | 0..1 | Represents the desrired time window for BDT. |  |
| desiredGeoArea | GeoArea | O | 0..1 | Represents the desired geographical area for BDT. |  |
| expTime | DurationSec | O | 0..1 | Represents the expiration time for the background data transfer request. |  |
| policyGuidance | array(PolicyGuidance) | O | 0..N | Represents a list of policy guidance. |  |

###### 7.4.1.4.2.22	Type: PolicyGuidance

Table 7.4.1.4.2.22-1: Definition of type PolicyGuidance

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| policyType | PolicyType | O | 0..1 | Indicates the policy type. |  |
| cost | UInteger | C | 0..1 | Represents the cost associated with the policy type and shall be included when the value of the "policyType" attribute is set to "HIGH_THROUGHPUT_WITH_COST" |  |

###### 7.4.1.4.2.23	Type: UnfTrafficSubc

Table 7.4.1.4.2.21-1: Definition of type UnfTrafficSubc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mgmtSubs | map(ManagementSubc) | M | 1..N | Represent the list of UE unified traffic pattern management subscriptions. The key of the map shall be any unique string encoded value. |  |
| notifUri | Uri | M | 1 | Identifies the destination notification URI where the notifications shall be sent to. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |

###### 7.4.1.4.2.24	Type: UnfTrafficSubcPatch

Table 7.4.1.4.2.22-1: Definition of type UnfTrafficSubcPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mgmtSubs | map(ManagementSubc) | 0 | 1..N | Represent the list of UE unified traffic pattern management subscriptions. The key of the map shall be any unique string encoded value and shall be set to the same value as the one received during creation of the corresponding resource. |  |
| notifUri | Uri | 0 | 1 | Identifies the updated destination notification URI where the notifications shall be sent to. |  |

###### 7.4.1.4.2.25	Type: ManagementSubc

Table 7.4.1.4.2.23-1: Definition of type ManagementSubc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServId | string | M | 1 | Identifies a VAL Service. |  |
| valUeIds | array(string) | M | 1..N | Represents a List of VAL UE(s) for which unified traffic pattern subscription is requested. |  |
| traffPatInd | array(TrafficPatternIndication) | M | 1..N | Represents the traffic pattern indications. |  |
| traffPatConfig | TrafficPatternConfig | O | 0..1 | Contains the traffic pattern configuration of the VAL service for the UE(s) provided within the "valUeIds" attribute. |  |

###### 7.4.1.4.2.26	Type: TrafficPatternConfig

Table 7.4.1.4.2.24-1: Definition of type TrafficPatternConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| scheds | array(ScheduledCommunicationTime) | O | 1..N | Contains the list of schedule(s) applicable to the traffic patterns of the VAL service for the concerned UE(s). |  |
| expTime | DateTime | O | 0..1 | Identifies the absolute time at which the VAL traffic pattern parameter configuration expires. When omitted, it indicates that the VAL traffic pattern parameter configuration never expires. |  |
| stationaryInd | boolean | O | 0..1 | Contains the stationary indication. - Set to "true" to indicate that the UE(s) are expected to be stationary while communicating using this traffic pattern configuration. - Set to "false" or omitted, to indicate that the UE(s) are expected to be mobile while communicating using this traffic pattern configuration. - The Default value is "false" when this attribute is omitted. |  |

###### 7.4.1.4.2.27	Type: UnfTrafficUpdNotif

Table 7.4.1.4.2.25-1: Definition of type UnfTrafficUpdNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeIds | array(string) | M | 1..N | Represents the list of VAL UEs to which the unified traffic pattern update notification is related. |  |
| sched | ScheduledCommunicationTime | O | 0..1 | Contains the schedule applicable to the unified traffic patterns of the UE(s) provided within the "valUeIds" attribute, and indicates the times or durations of UE availability. |  |
| stationInd | boolean | O | 0..1 | Contains the stationary indication. - Set to "true" to indicate that the UE(s) are expected to be stationary while communicating using this traffic pattern configuration. - Set to "false" or omitted, to indicate that the UE(s) are expected to be mobile while communicating using this traffic pattern configuration. - The Default value is "false" when this attribute is omitted. |  |
| cause | string | C | 0..1 | This attribute shall be present when the notification is provided to inform of a parameter configuration applied by the network which is incompatible with the existing Traffic Patterns. The attribute may be present when the notification informs of UE unified traffic pattern updates, providing additional information on the reason for the UE unified traffic pattern update (e.g. monitoring events received). |  |

###### 7.4.1.4.2.28	Type: RelTransReq

Table 7.4.1.4.2.28-1: Definition of type RelTransReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appDescs | array(AppTraffDesc) | O | 1..N | Contains the application traffic descriptors of the service consumer, which are used to establish redundant transmission paths. (NOTE) |  |
| valUeId | string | O | 0..1 | Contains the unique identifier of the concerned VAL UE. |  |
| valAddrInfo | ValUeAddrInfo | O | 0..1 | Contains the address information of the concerned VAL UE. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.4.1.6. This attribute shall be present only when feature negotiation needs to take place. |  |
| NOTE: In this release of this specification, this attribute shall not contain more than two array elements. |  |  |  |  |  |

###### 7.4.1.4.2.29	Type: AppTraffDesc

Table 7.4.1.4.2.29-1: Definition of type AppTraffDesc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| connInfo | ConnInfo | M | 1 | Contains the data transmission connection information, i.e., address/port and/or URI via which application traffic is exchanged. |  |
| transProtoc | TransportProtocol | O | 0..1 | Represents the transport layer protocol via which application traffic is exchanged. |  |

###### 7.4.1.4.2.30	Type: ValUeAddrInfo

Table 7.4.1.4.2.30-1: Definition of type ValUeAddrInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeIpv4Addr | Ipv4Addr | C | 0..1 | Contains the IPv4 address of the targeted VAL UE. (NOTE) |  |
| valUeIpv6Addr | Ipv6Addr | C | 0..1 | Contains the IPv6 address of the targeted VAL UE. (NOTE) |  |
| port | Port | O | 0..1 | Contains the port information of the targeted VAL UE. |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

###### 7.4.1.4.2.31	Type: UnicastSubscriptionPatch

Table 7.4.1.4.2.31-1: Definition of type UnicastSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUes | array(string) | O | 1..N | Represents the list of the VAL UE identifier(s). |  |
| valGroupId | string | O | 0..1 | Represents the VAL group identifier. |  |
| valTgtUe | ValTargetUeRm | O | 0..1 | Contains the identity of the VAL user or VAL UE that the bearer is requested for. |  |
| bdtId | string | O | 0..1 | Represents the BDT identifier. |  |
| uniQosReq | string | O | 0..1 | Contains the QoS requirement. |  |
| duration | DateTime | O | 0..1 | Identifies the absolute time at which the subscription resource is considered to expire. When omitted in the request, it indicates the resource is requested to be valid forever by the service consumer. When omitted in the response, it indicates the resource is set to valid forever by the service consumer. |  |
| notifUri | Uri | O | 0..1 | Identifies the notification URI where the NRM notification shall be sent to. |  |

##### 7.4.1.4.3	Simple data types and enumerations

###### 7.4.1.4.3.0	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.4.1.4.3.0A	Simple data types

The simple data types defined in table 7.4.1.4.3.0A-1 shall be supported.

Table 7.4.1.4.3.0A-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| EpsMbmsInfo | Bytes | Represents the EPS MBMS bearer information encoded as specified in clause 6.4.7 of 3GPP TS 29.468 [23]. |  |

###### 7.4.1.4.3.1	Enumeration: ServiceAnnoucementMode

Table 7.4.1.4.3.1-1: Enumeration ServiceAnnoucementMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| NRM | Indicates that the NRM Server performs the service announcement. |  |
| VAL | Indicates that the service consumer performs the service announcement. |  |

###### 7.4.1.4.3.2	Enumeration: DeliveryMode

Table 7.4.1.4.3.2-1: Enumeration DeliveryMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UNICAST | Indicates Unicast delivery. |  |
| MULTICAST | Indicates EPS MBMS Multicast delivery. |  |
| MBS_MULTICAST | Indicates 5GS MBS Multicast delivery. | SEAL_3 |
| MBS_BROADCAST | Indicates 5GS MBS Broadcast delivery. | SEAL_3 |

###### 7.4.1.4.3.4	Enumeration: NetSysIndicator

Table 7.4.1.4.3.4-1: Enumeration NetSysIndicator

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| 5GS | Indicates that the network system is 5GS. |  |
| EPS | Indicates that the network system is EPS. |  |
| 5GS_AND_EPS | Indicates that the network system is both 5GS and EPS. |  |

###### 7.4.1.4.3.5	Enumeration: NrmEvent

Table 7.4.1.4.3.5-1: Enumeration NrmEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UP_DELIVERY_MODE | User Plane delivery mode. |  |

###### 7.4.1.4.3.6	Enumeration: PolicyType

Table 7.4.1.4.3.6-1: Enumeration PolicyType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LOWEST_COST | Indicates that the policy guidance is to consider lowest cost. |  |
| HIGH_THROUGHPUT_WITH_COST | Indicates that the policy guidance is to consider highest throughput. |  |

###### 7.4.1.4.3.7	Enumeration: TrafficPatternIndication

Table 7.4.1.4.3.6-1: Enumeration TrafficPatternIndication

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| TRAFFIC_PATTERN_MANAGE | Indicate that the management of the UE unified traffic pattern is requested. |  |
| TRAFFIC_PATTERN_MONITOR | Indicate that UE unified traffic pattern monitoring related notifications is requested. |  |
| NETWORK_PARAM_COORDINATION | Indicate that network parameter coordination by the NRM with 5GC is requested. |  |

#### 7.4.1.5	Error Handling

##### 7.4.1.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.4.1.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_NetworkResourceAdaptation API.

##### 7.4.1.5.3	Application Errors

The application errors defined for SS_NetworkResourceAdaptation API are listed in table 7.4.1.5.3-1.

Table 7.4.1.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |
| INVALID_MBS_SERVICE_INFO | 400 Bad Request | The provided MBS Service Information is invalid (e.g. invalid QoS reference), incorrect or insufficient to perform MBS policy authorization. |  |
| FILTER_RESTRICTIONS_NOT_OBSERVED | 400 Bad Request | The MBS IP flow(s) description provided within the MBS Service Information cannot be handled due to the restrictions defined in clause 5.3.8 of 3GPP TS 29.214 [44] not being observed. |  |
| MBS_SERVICE_AREA_NOT_SUPPORTED | 403 Forbidden | The requested MBS Service Area is not supported by the 3GPP Core Network. |  |
| MBS_SERVICE_INFO_NOT_AUTHORIZED | 403 Forbidden | The provided MBS Service Information is rejected. |  |

#### 7.4.1.6	Feature negotiation

Table 7.4.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | The testing of notification connection is supported according to clause 6.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 6.6. This feature requires that the Notification_test_event feature is also supported. |
| 3 | LocalMBMS | Indicate the support of local MBMS transmission. |
| 4 | SEAL_3 | This feature indicates the support of the third set of enhancements to the SEAL Enabler Layer framework. Within this feature, the following enhancements are covered: - Support MBS resources management. |
| 5 | enNB | Indicates the support of enhancements to this application enabler layer API in Rel-19. Within this feature, the following enhancements are covered: - Support to provide the unicast streams to be stopped (i.e., as data delivery is switched to multicast delivery) within the user plane delivery mode notification. - Support of the network resource adaptation service operation consolidation. |
| 6 | SEAL_3_Ext | This feature indicates the support of an extension to the third set of enhancements to the SEAL Enabler Layer framework. Within this feature, the following enhancements are covered: - Support to report the list of VAL UE(s) for which unicast delivery is used as part of the user plane delivery mode reporting. |
| 7 | SEALDD_BDT | Indicates the support of enhancements to this application enabler layer API to support SEALDD services. Within this feature, the following enhancements are covered: - Support to provide the BDT reference in network resource request. |

### 7.4.2	SS_NetworkResourceMonitoring API

#### 7.4.2.1	API URI

The SS_NetworkResourceMonitoring service shall use the SS_NetworkResourceMonitoring API.

The request URIs used in HTTP requests from the VAL server towards the NRM server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-nrm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.4.2.2

#### 7.4.2.2	Resources

##### 7.4.2.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.4.2.2.1-1 depicts the resource URIs structure for the SS_NetworkResourceMonitoring API.

Figure 7.4.2.2.1-1: Resource URI structure of the SS_NetworkResourceMonitoring API

Table 7.4.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.4.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Unicast Monitoring Subscriptions | /subscriptions | POST | Create individual unicast monitoring subscription resource or obtain unicast QoS monitoring data for VAL UEs, VAL Group, or VAL Streams. |
| Individual Unicast Monitoring Subscription | /subscriptions/{subscriptionId} | DELETE | Remove an existing individual unicast monitoring subscription resource according to the subscriptionId. |
|  |  | GET | Read an existing individual unicast monitoring subscription resource according to the subscriptionId. |
|  |  | PUT | Update an individual unicast monitoring subscription identified by the subscriptionId. |
|  |  | PATCH | Modify an individual unicast monitoring subscription identified by the subscriptionId. |

##### 7.4.2.2.2	Resource: Unicast Monitoring Subscriptions

###### 7.4.2.2.2.1	Description

###### 7.4.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-nrm/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 7.4.2.2.2.2-1.

Table 7.4.2.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.4.2.1. |

###### 7.4.2.2.2.3	Resource Standard Methods

7.4.2.2.2.3.1	POST

This method enables a VAL Server to request the creation of a unicast QoS monitoring subscription at the NRM server. This method shall support the URI query parameters specified in table 7.4.2.2.2.3.1-1.

Table 7.4.2.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.4.2.2.2.3.1-2 and the response data structures and response codes specified in table 7.4.2.2.2.3.1-3.

Table 7.4.2.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MonitoringSubscription | M | 1 |  |

Table 7.4.2.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MonitoringSubscription | M | 1 | 201 Created | The requested individual monitoring subscription resource is successfully created and a representation of the created resource is returned in the response body. |
| MonitoringReport | M | 1 | 200 OK | The requested unicast QoS monitoring data is returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.2.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-nrm/<apiVersion>/subscriptions{subscriptionId} |

###### 7.4.2.2.2.4	Resource Custom Operations

None.

##### 7.4.2.2.3	Resource: Individual Unicast Monitoring Subscription

###### 7.4.2.2.3.1	Description

###### 7.4.2.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-nrm/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 7.4.2.2.3.2-1.

Table 7.4.2.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.4.2.1. |
| subscriptionId | string | Represents the identifier of an individual unicast monitoring subscription resource. |

###### 7.4.2.2.3.3	Resource Standard Methods

7.4.2.2.3.3.1	DELETE

This operation deletes the Individual Unicast Monitoring Subscription resource. This method shall support the URI query parameters specified in table 7.4.2.2.3.3.1-1.

Table 7.4.2.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.2.2.3.3.1-2 and the response data structures and response codes specified in table 7.4.2.2.3.3.1-3.

Table 7.4.2.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.2.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The Individual Unicast Monitoring Subscription resource matching the subscriptionId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.2.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NRM server. |

Table 7.4.2.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NRM server. |

7.4.2.2.3.3.2	GET

This operation reads the individual unicast monitoring subscription resource. This method shall support the URI query parameters specified in table 7.4.2.2.3.3.2-1.

Table 7.4.2.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.4.2.2.3.3.2-2 and the response data structures and response codes specified in table 7.4.2.2.3.3.2-3.

Table 7.4.2.2.3.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.4.2.2.3.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MonitoringSubscription | M | 1 | 200 OK | The requested individual unicast monitoring subscription is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.4.2.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NRM server. |

Table 7.4.2.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NRM server. |

7.4.2.2.3.3.3	PUT

This operation updates the "Individual Unicast Monitoring Subscription". This method shall support the URI query parameters specified in table 7.4.2.2.3.3.3-1.

Table 7.4.2.2.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.2.2.3.3.3-2 and the response data structures and response codes specified in table 7.4.2.2.3.3.3-3.

Table 7.4.2.2.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MonitoringSubscription | M | 1 | Updated details of the unicast QoS monitoring subscription. The target identifiers, i.e., the "valUeIds", "valGroupId”, and "valStreamIds" attributes shall not be changed in the MonitoringSubscription data structure. |

Table 7.4.2.2.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MonitoringSubscription | M | 1 | 200 OK | The subscription is updated successfully, and the updated subscription information returned in the response. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NRM server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.2.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

Table 7.4.2.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative location management server. |

7.4.2.2.3.3.4	PATCH

This operation modifies the "Individual Unicast Monitoring Subscription". This method shall support the URI query parameters specified in table 7.4.2.2.3.3.4-1.

Table 7.4.2.2.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.2.2.3.3.4-2 and the response data structures and response codes specified in table 7.4.2.2.3.3.4-3.

Table 7.4.2.2.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MonitoringSubscriptionPatch | M | 1 | Contains the modifications to be applied to the individual unicast QoS monitoring subscription resource. |

Table 7.4.2.2.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MonitoringSubscription | M | 1 | 200 OK | Individual unicast QoS monitoring subscription resource is modified successfully and representation of the modified individual unicast QoS monitoring subscription resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.2.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

Table 7.4.2.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

#### 7.4.2.3	Notifications

##### 7.4.2.3.1	General

Table 7.4.2.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Individual Unicast Monitoring Notification | {notifUri} | POST | Notify on updates of the individual monitoring resource accodring to the requested reporting settings. |

##### 7.4.2.3.2	Individual Unicast Monitoring Notification

###### 7.4.2.3.2.1	Description

###### 7.4.2.3.2.2	Notification definition

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.4.2.3.2.2-1.

Table 7.4.2.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.4.2.3.2.2-2 and the response data structures and response codes specified in table 7.4.2.3.2.2-3.

Table 7.4.2.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MonitoringReport | M | 1 | Represents the reported monitoring data. |

Table 7.4.2.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The notification is successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative VAL server where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative VAL server where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.4.2.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative VAL server towards which the notification should be redirected. |

Table 7.4.2.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative VAL server towards which the notification should be redirected. |

#### 7.4.2.4	Data Model

##### 7.4.2.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.4.2.4.1-1 specifies the data types defined specifically for the SS_NetworkResourceMonitoring API service.

Table 7.4.2.4.1-1: SS_NetworkResourceMonitoring API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| FailureReport | 7.4.2.4.2.9 | Represents the failure report indicating the VAL UE(s) or VAL Stream ID(s) for which the NRM server failed to obtain the requested data. |  |
| FailureReason | 7.4.2.4.3.3 | Represents the failure reason. |  |
| MeasurementData | 7.4.2.4.2.3 | Presents the aggregated measurement data. |  |
| MeasurementDataType | 7.4.2.4.3.1 | Indicates the requested measurement data type. |  |
| MeasurementPeriod | 7.4.2.4.2.4 | Indicates the measurement time period. |  |
| MeasurementRequirements | 7.4.2.4.2.6 | Indicates the measurement requirements. |  |
| MonitoringReport | 7.4.2.4.2.2 | Indicates the monitoring report for VAL UEs list, VAL Group, or VAL Stream. |  |
| MonitoringSubscription | 7.4.2.4.2.7 | The monitoring subscription request. |  |
| MonitoringSubscriptionPatch | 7.4.2.4.2.11 | Represents the monitoring subscription modification request. | UpdateSupport |
| ReportingRequirements | 7.4.2.4.2.5 | Indicates the requested requirements of reporting. |  |
| ReportingThreshold | 7.4.2.4.2.10 | Indicates a requested reporting threshold. |  |
| TerminationMode | 7.4.2.4.3.2 | Indicates the termination mode. |  |
| ThresholdHandlingMode | 7.4.2.4.3.4 | Indicates the multi-parameter threshold handling mode. |  |

Table 7.4.2.4.1-2 specifies data types re-used by the SS_NetworkResourceMonitoring API service.

Table 7.4.2.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AverWindow | 3GPP TS 29.571 [21] | Used to represent an aggregation window. |  |
| BitRate | 3GPP TS 29.571 [21] | Used to represent a bit rate measurement value. |  |
| DateTime | 3GPP TS 29.122 [3] | Used to represent a date and time. |  |
| DurationSec | 3GPP TS 29.571 [21] | Used to represent a measurement timestamp and measurement start time. |  |
| MatchingDirection | 3GPP TS 29.520 [33] | Used to indicate a threshold matching direction. |  |
| NotificationMethod | 3GPP TS 29.508 [32] | Used to indicate the reporting mode. |  |
| PacketLossRate | 3GPP TS 29.571 [21] | Used to represent a packet loss rate measurement value. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the supported optional features of the API. |  |
| Uinteger | 3GPP TS 29.571 [21] | Used to represent integer attributes within MeasurementData and ReportingRequirements data structures. |  |
| Uri | 3GPP TS 29.122 [3] | Used to indicate the notification URI. |  |
| ValTargetUe | clause 7.3.1.4.2.3 | Used to identify either a VAL User ID or a VAL UE ID. |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] | Used to indicate the configuration parameters to set up notification delivery over Websocket protocol. |  |

##### 7.4.2.4.2	Structured data types

###### 7.4.2.4.2.1	Introduction

###### 7.4.2.4.2.2	Type: MonitoringReport

Table 7.4.2.4.2.2-1: Definition of type MonitoringReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeIds | array(ValTargetUe) | C | 1..N | List of VAL UEs whose measurement data is provided (NOTE). |  |
| valGroupId | string | C | 0..1 | The group ID used for the VAL group for which measurement data is provided (NOTE). |  |
| valStreamIds | array(string) | C | 1..N | List of VAL stream IDs whose measurement data (NOTE). |  |
| measData | MeasurementData | M | 1 | The aggregated measurement data. |  |
| termCause | TerminationMode | O | 0..1 | Contains the subscription termination cause. The presence of this attribute indicates that the subscription is terminated following this notification. | SEAL_Ext |
| timestamp | DateTime | M | 1 | The timestamp of the measurement. |  |
| failureRep | array(FailureReport) | C | 1..N | The failure report from the NRM server indicating the VAL UE(s) or VAL Stream ID(s) whose measurement data is not obtained successfully and is not provided in the monitoring report. This attribute shall be provided by the NRM server when the requested measurement data is not obtained successfully for all the requested VAL UE(s) or VAL Stream ID(s). |  |
| monProfId | MonProfId | O | 0..1 | Contains the identifier of the monitoring profile to which the notification is related. | SEAL_Ext |
| NOTE: Only one of these attributes shall be provided. |  |  |  |  |  |

###### 7.4.2.4.2.3	Type: MeasurementData

Table 7.4.2.4.2.3-1: Definition of type MeasurementData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dlDelay | Uinteger | O | 0..1 | The downlink packet delay in milliseconds (NOTE). |  |
| ulDelay | Uinteger | O | 0..1 | The uplink packet delay in milliseconds (NOTE). |  |
| rtDelay | Uinteger | O | 0..1 | The round trip packet delay in milliseconds (NOTE). |  |
| avgPlr | PacketLossRate | O | 0..1 | The average packet loss rate (NOTE). |  |
| avgDataRate | BitRate | O | 0..1 | The average data rate (NOTE). |  |
| maxDataRate | BitRate | O | 0..1 | The maximum data rate (NOTE). |  |
| avrDlTrafficVol | Uinteger | O | 0..1 | The average traffic volume for downlink in bytes (NOTE). |  |
| avrUlTrafficVol | Uinteger | O | 0..1 | The average traffic volume for uplink in bytes (NOTE). |  |
| NOTE: At least one of the measurement indexes shall be provided. |  |  |  |  |  |

###### 7.4.2.4.2.4	Type: MeasurementPeriod

Table 7.4.2.4.2.4-1: Definition of type MeasurementPeriod

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measStartTime | DateTime | M | 1 | Indicate the starting time for the measurement. |  |
| measDuration | DurationSec | M | 1 | Indicate the duration for the measurement starting from the measStartTime. |  |

###### 7.4.2.4.2.5	Type: ReportingRequirements

Table 7.4.2.4.2.5-1: Definition of type ReportingRequirements

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reportingMode | NotificationMethod | M | 1 | The indication of the requested reporting option: one-time, periodic or event-triggered (i.e. "ON_EVENT_DETECTION") This attribute may be set to the value "ONE_TIME" only if the "immRep" attribute is provided and set to "true".. |  |
| reportingPeriod | DurationSec | C | 0..1 | Identifies the reporting time interval for the periodic reporting. (NOTE 1). |  |
| reportingThrs | array(ReportingThreshold) | C | 1..N | Identifies reporting threshold corresponding to the measurement index(es). (NOTE 2). |  |
| immRep | boolean | O | 0..1 | It indicates immediate reporting. When included and set to true, it indicates that immediate reporting of the subscribed event(s) is requested. |  |
| repTerminMode | TerminationMode | O | 0..1 | The indication of the requested reporting termination mode: time-triggered, event-triggered (number of reports reached), event-triggered (threshold reached) or user-triggered. If absent, user-triggered reporting is used as the default termination mode. |  |
| expirationTimer | DurationSec | C | 0..1 | Identifies the reporting time interval for the time triggered termination mode. (NOTE 3). |  |
| maxNumRep | Uinteger | C | 0..1 | Indicates the maximum number of reports (number of reports reached). (NOTE 4). |  |
| termThr | MeasurementData | C | 0..1 | Indicates the reporting termination threshold(s) corresponding to the measurement index(ex). (NOTE 5, NOTE 6, NOTE 7). |  |
| termThrMode | ThresholdHandlingMode | C | 0..1 | Indicates the multi-parameter threshold handling for the event-triggered threshold reached reporting termination. (NOTE 8). |  |
| NOTE 1: The "reportingPeriod" attribute shall be present only when the "reportingMode" attribute is set to "PERIODIC". NOTE 2: The "reportingThr" attribute shall be present only when the "reportingMode" attribute is set to "ON_EVENT_DETECTION". NOTE 3: The "expirationTimer" attribute shall be present only if the "repTerminMode" attribute is present and set to "TIME_TRIGGERED". NOTE 4: The "maxNumRep" attribute shall be present only if the "repTerminMode" attribute is present and set to "EVENT_TRIGGERED_NUM_REPORTS_REACHED". NOTE 5: The "termThr" attributes shall be present only when the "repTerminMode" attribute is present and set to "EVENT_TRIGGERED_MEAS_THR_REACHED". NOTE 6: The event-triggered reporting termination condition for the "dlDelay", "ulDelay", "rtDelay" and "avgPlr" attributes within the MeasurementData data structure is met when the measured value is greater than or equal to the given threshold. NOTE 7: The event-triggered reporting termination condition for the "avgDataRate", "maxDataRate", "avrDlTrafficVol" and "avrUlTrafficVol" attributes within the MeasurementData data structure is met when the measured value is less than or equal to the given threshold. NOTE 8: The "termThrMode" attribute shall be present only when the "repTerminMode" attribute is present and set to "EVENT_TRIGGERED_MEAS_THR_REACHED" and the "termThr" contains more than one threshold. |  |  |  |  |  |

###### 7.4.2.4.2.6	Type: MeasurementRequirements

Table 7.4.2.4.2.6-1: Definition of type MeasurementRequirements

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measDataTypes | array(MeasurementDataType) | M | 1..N | Indicates the required types of measurement data. At least one measurement data type shall be present in the array structure. |  |
| measAggrGranWnd | AverWindow | O | 0..1 | It indicates the aggregation granularity window for the measured data (NOTE 1). The aggregation granularity window shall not exceed the duration of the measurement time period defined in the "measPeriod" attribute. |  |
| measPeriod | MeasurementPeriod | O | 0..1 | It indicates the required measurement time period (NOTE 2). |  |
| NOTE 1: If absent, 1 minute shall be used as default setting. NOTE 2: If absent, current time and 5 minutes duration shall be used as default setting. |  |  |  |  |  |

###### 7.4.2.4.2.7	Type: MonitoringSubscription

Table 7.4.2.4.2.7-1: Definition of type MonitoringSubscription

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeIds | array(ValTargetUe) | C | 1..N | List of VAL UEs which measurement data reporting is requested (NOTE 1). |  |
| valGroupId | string | C | 0..1 | The group ID used for the VAL group for which measurement data reporting is requested (NOTE 1). |  |
| valStreamIds | array(string) | C | 1..N | List of VAL streams for which measurement data reporting is requested (NOTE 1) |  |
| valServId | string | O | 0..1 | Contains the VAL service identifier. (NOTE 2, NOTE 3) | SEAL_Ext |
| monProfId | MonProfId | O | 0..1 | Contains the monitoring profile identifier. (NOTE 2, NOTE 3, NOTE 4) | SEAL_Ext |
| measReqs | MeasurementRequirements | O | 0..1 | Contains the measurement requirements (NOTE 2, NOTE 4). |  |
| reportReqs | ReportingRequirements | O | 0..1 | Contains the reporting requirements. (NOTE 3, NOTE 4) |  |
| notifUri | Uri | C | 0..1 | Contains the URI where the notification should be delivered to. This attribute shall be present only for subscriptions without immediate reporting. |  |
| monRep | MonitoringReport | C | 0..1 | Contains the unicast QoS monitoring data reporting. The NRM server shall provide this attribute when immediate reporting is requested and the requested data is available. |  |
| reqTestNotif | boolean | O | 0..1 | Set to true to request the VAL server to send a test notification as defined in clause 6.6. Set to false or omitted otherwise. | Notification_test_event |
| wsNotifCfg | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 6.6. | Notification_websocket |
| suppFeat | SupportedFeatures | C | 0..1 | This parameter shall be supplied by VAL server in the POST request that request the creation of an individual measurement resource and shall be supplied in the reply of corresponding request. |  |
| NOTE 1: Only one of these attributes shall be provided. NOTE 2: If the "measReqs" attribute, and if the "SEAL_Ext" feature is supported, the "monProfId" attribute is/are all absent, then the default monitoring requirements values shall be used or the monitoring requirements values shall be derived based on the VAL Service ID provided within the "valServId" attribute, if received. NOTE 3: If the "reportReqs" attribute, and if the "SEAL_Ext" feature is supported, the "monProfId" attribute is/are all absent, the default event triggered reporting requirements shall be used or the event triggered reporting requirements values shall be derived based on the VAL Service ID provided within the "valServId" attribute, if received. NOTE 4: When the "SEAL_Ext" feature is supported and the "monProfId" attribute is present in the resource creation/update request, then both the "measReqs" and "reportReqs" attributes shall not be present neither in the resource creation/update request nor in the corresponding response. |  |  |  |  |  |

###### 7.4.2.4.2.8	Void

###### 7.4.2.4.2.9	Type: FailureReport

Table 7.4.2.4.2.9-1: Definition of type FailureReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeIds | array(ValTargetUe) | C | 1..N | List of VAL UE(s) whose measurement data is not obtained successfully and is not provided (NOTE). The VAL UE(s) may be member(s) of the VAL group identified by the "valGroupId" attribute in the MonitoringReport data structure. |  |
| valStreamIds | array(string) | C | 1..N | List of VAL stream ID(s) whose measurement data is not obtained successfully and is not provided (NOTE). |  |
| failureReason | FailureReason | M | 1 | Identifies the failure reason. |  |
| measDataType | MeasurementDataType | C | 0..1 | The indication of the measurement data type that is not obtained successfully and is not provided in the monitoring report. This attribute shall be provided if the failure reason does not apply to all the requested measurement data types. |  |
| NOTE: Only one of these attributes shall be provided. |  |  |  |  |  |

###### 7.4.2.4.2.10	Type: ReportingThreshold

Table 7.4.2.4.2.10-1: Definition of type ReportingThreshold

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measThrValues | MeasurementData | M | 1 | Indicates the value(s) for the measurement threshold index(es). |  |
| thrDirection | MatchingDirection | M | 1 | Indicates the threshold matching direction for the measurement threshold index(es) provided in the "measThrValues" attribute. |  |

###### 7.4.2.4.2.11	Type: MonitoringSubscriptionPatch

Table 7.4.2.4.2.11-1: Definition of type MonitoringSubscriptionPatch

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measReqs | MeasurementRequirements | O | 0..1 | Indicates the measurement requirements. |  |
| reportReqs | ReportingRequirements | O | 0..1 | Indicates the reporting requirements. |  |
| notifUri | Uri | O | 0..1 | Indicates the URI where the notification should be delivered to. |  |
| monProfId | MonProfId | O | 0..1 | Contains the monitoring profile identifier. | SEAL_Ext |

##### 7.4.2.4.3	Simple data types and enumerations

###### 7.4.2.4.3.0A	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.4.2.4.3.0B	Simple data types

The simple data types defined in table 7.4.2.4.3.0B-1 shall be supported.

Table 7.4.2.4.3.0B-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| MonProfId | string | Represents the monitoring profile identifier. |  |

###### 7.4.2.4.3.1	Enumeration: MeasurementDataType

Table 7.4.2.4.3.1-1: Enumeration MeasurementDataType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DL_DELAY | The indication for requesting the downlink packet delay data type. |  |
| UL_DELAY | The indication for requesting the uplink packet delay data type. |  |
| RT_DELAY | The indication for requesting the round trip packet delay data type. |  |
| AVG_PLR | The indication for requesting the average packet loss rate data type. |  |
| AVG_DATA_RATE | The indication for requesting the average data rate data type. |  |
| MAX_DATA_RATE | The indication for requesting the maximum data rate data type. |  |
| AVG_DL_TRAFFIC_VOLUME | The indication for requesting the average traffic volume for downlink data type. |  |
| AVG_UL_TRAFFIC_VOLUME | The indication for requesting the average traffic volume for uplink data type. |  |

###### 7.4.2.4.3.2	Enumeration: TerminationMode

Table 7.4.2.4.3.2-1: Enumeration TerminationMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| TIME_TRIGGERED | The time-triggered termination mode. |  |
| EVENT_TRIGGERED_NUM_REPORTS_REACHED | The event-triggered termination number of reports reached mode. |  |
| EVENT_TRIGGERED_MEAS_THR_REACHED | The event-triggered termination measurement index threshold reached mode. |  |
| USER_TRIGGERED | The user-triggered termination mode. |  |

###### 7.4.2.4.3.3	Enumeration: FailureReason

Table 7.4.2.4.3.3-1: Enumeration FailureReason

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| USER_NOT_FOUND | The user is not found. |  |
| STREAM_NOT_FOUND | The stream is not found. |  |
| DATA_NOT_AVAILABLE | The requested data is not available. |  |
| OTHER_REASON | Other reason (unspecified). |  |

###### 7.4.2.4.3.4	Enumeration: ThresholdHandlingMode

Table 7.4.2.4.3.4-1: Enumeration ThresholdHandlingMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ALL_REACHED | The decision criterion is met when all the provided thresholds are reached. |  |
| ANY_REACHED | The decision criterion is met when any of the provided threshold(s) is reached. |  |

#### 7.4.2.5	Error Handling

##### 7.4.2.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.4.2.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_NetworkResourceMonitoring API.

##### 7.4.2.5.3	Application Errors

The application errors defined for SS_NetworkResourceMonitoring API are listed in table 7.4.2.5.3-1.

Table 7.4.2.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.4.2.6	Feature negotiation

General feature negotiation procedures are defined in clause 6.8. Table 7.4.2.6-1 lists the supported features for SS_NetworkResourceMonitoring API.

Table 7.4.2.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | The testing of notification connection is supported according to clause 6.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 6.6. This feature requires that the Notification_test_event feature is also supported. |
| 3 | UpdateSupport | Indicates the support of the Update_Unicast_QoS_Monitoring_Subscription service operation. This feature enables the support of both HTTP PUT and HTTP PATCH methods for the SS_NetworkResourceMonitoring API. |
| 4 | SEAL_Ext | Indicates the support of the SEAL enhancements in Release-19. Within this feature, the following enhancements are covered: - Support of the monitoring profiles. - Support of the subscription termination indication. |

## 7.5	Event APIs

### 7.5.1	SS_Events API

#### 7.5.1.1	API URI

The SS_Events service shall use the SS_Events API.

The request URIs use in HTTP requests from the VAL server towards the SEAL server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-events".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.5.1.2.

#### 7.5.1.2	Resources

##### 7.5.1.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.5.1.2.1-1 depicts the resource URIs structure for the SS_Events API.

Figure 7.5.1.2.1-1: Resource URI structure of the SS_Events API

Table 7.5.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.5.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| SEAL Events Subscriptions | /subscriptions | POST | Creates a new individual SEAL Event Subscription. |
| Individual SEAL Events Subscription | /subscriptions/{subscriptionId} | DELETE | Deletes an individual SEAL Event Subscription identified by the subscriptionId. |
|  |  | PATCH | Modifies an individual SEAL Event subscription identified by the subscriptionId. |
|  |  | PUT | Updates an individual SEAL Event subscription identified by the subscriptionId. |

##### 7.5.1.2.2	Resource: SEAL Events Subscriptions

###### 7.5.1.2.2.1	Description

The SEAL Events Subscriptions represents all event subscriptions on the SEAL server.

###### 7.5.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-events/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 7.5.1.2.2.2-1.

Table 7.5.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.5.1.2.2.3	Resource Standard Methods

7.5.1.2.2.3.1	POST

This method shall support the URI query parameters specified in the table 7.5.1.2.2.3.1-1.

Table 7.5.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.5.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.5.1.2.2.3.1-3.

Table 7.5.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SEALEventSubscription | M | 1 | Create a new individual SEAL Events Subscription resource. |

Table 7.5.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SEALEventSubscription | M | 1 | 201 Created | SEAL Events Subscription resource created successfully. The URI of the created resource shall be returned in the "Location" HTTP header |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.5.1.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-events/<apiVersion>/subscriptions/{subscriptionId} |

###### 7.5.1.2.2.4	Resource Custom Operations

None.

##### 7.5.1.2.3	Resource: Individual SEAL Events Subscription

###### 7.5.1.2.3.1	Description

The Individual SEAL Events Subscription resource represents an individual event subscription of a VAL server.

###### 7.5.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-events/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 7.5.1.2.3.2-1.

Table 7.5.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| subscriptionId | string | Identifies an Individual Events Subscription |

###### 7.5.1.2.3.3	Resource Standard Methods

7.5.1.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 7.5.1.2.3.3.1-1.

Table 7.5.1.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.5.1.2.3.3.1-2 and the response data structures and response codes specified in table 7.5.1.2.3.3.1-3.

Table 7.5.1.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.5.1.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual SEAL Events Subscription matching the subscriptionId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.5.1.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

Table 7.5.1.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

7.5.1.2.3.3.2	PATCH

This method shall support the URI query parameters specified in table 7.5.1.2.3.3.2-1.

Table 7.5.1.2.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.5.1.2.3.3.2-2 and the response data structures and response codes specified in table 7.5.1.2.3.3.2-3.

Table 7.5.1.2.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SEALEventSubscriptionPatch | M | 1 | Contains the modifications to be applied to the SEAL Event subscription resource. |

Table 7.5.1.2.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SEALEventSubscription | M | 1 | 200 OK | SEAL Events Subscription resource is modified successfully and representation of the modified SEAL Event subscription is returned. |
| n/a |  |  | 204 No Content | The SEAL Events Subscription is updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.5.1.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

Table 7.5.1.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

7.5.1.2.3.3.3	PUT

This method requests fully replacement of an existing Individual SEAL Events Subscription at the SEAL server. The request shall not udpate the values of the "subscriberId", "requestTestNotification" (if previously provisioned), "websockNotifConfig" (if previously provisioned) and/or "suppFeat" attributes within the SEALEventSubscription data type. This method shall support the URI query parameters specified in table 7.5.1.2.3.3.3-1.

Table 7.5.1.2.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.5.1.2.3.3.3-2 and the response data structures and response codes specified in table 7.5.1.2.3.3.3-3.

Table 7.5.1.2.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SEALEventSubscription | M | 1 | Contains the SEAL Event subscription to be updated. |

Table 7.5.1.2.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SEALEventSubscription | M | 1 | 200 OK | SEAL Event Subscription resource is updated successfully and representation of the modified SEAL Event subscription is returned. |
| n/a |  |  | 204 No Content | The SEAL Events Subscription is updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEAL server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.5.1.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

Table 7.5.1.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEAL server. |

###### 7.5.1.2.3.4	Resource Custom Operations

None.

#### 7.5.1.3	Notifications

##### 7.5.1.3.1	General

The delivery of notifications shall conform to clause 6.6.

Table 7.5.1.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| SEAL Event Notification | {notificationDestination} | POST | Notifies subscriber of a SEAL Event |

##### 7.5.1.3.2	SEAL Event Notification

###### 7.5.1.3.2.1	Description

SEAL Event Notification is used by the SEAL server notify a VAL server of an Event. The VAL server shall be subscribed to such SEAL Event Notifications via the Individual SEAL Events Subscription Resource.

###### 7.5.1.3.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the VAL server during the subscription to the event.

Callback URI: {notificationDestination}

This method shall support the URI query parameters specified in table 7.5.1.3.2.2-1.

Table 7.5.1.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.5.1.3.2.2-2 and the response data structures and response codes specified in table 7.5.1.3.2.2-3.

Table 7.5.1.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SEALEventNotification | M | 1 | Notification information of a SEAL Event |

Table 7.5.1.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.5.1.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.5.1.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.5.1.4	Data Model

##### 7.5.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.5.1.4.1-1 specifies the data types defined specifically for the SS_Events API service.

Table 7.5.1.4.1-1: SS_Events API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EventSubscription | 7.5.1.4.2.4 | Represents the subscription to a single SEAL event. |  |
| GeofencingConditions | 7.5.1.4.2.26 | Represents the geofencing conditions. | LM_LocationDeviation_Ext1 |
| PartialEventSubscFailRep | 7.5.1.4.2.23 | Represents the partial failure report during the subscription creation. | PartialFailureSupport |
| IdentityFilter | 7.5.1.4.2.7 | Represents a filter of VAL User / UE identities belonging to a VAL service. |  |
| LMInformation | 7.5.1.4.2.8 | The location information for a VAL User ID or a VAL UE ID. |  |
| LocationAreaMonReport | 7.5.1.4.2.20 | Represents the event report to notify the VAL UEs moving in or moving out from a given location. |  |
| LocationDevMonReport | 7.5.1.4.2.15 | Represents the event report to notify the VAL UE/User's location deviation from a given location. |  |
| LocationInfoCriteria | 7.5.1.4.2.18 | Represents the location information to be monitored. It includes the geographic location information or a reference UE along with the application defined proximity range from the reference UE. |  |
| LocDevNotification | 7.5.1.4.3.4 | Enumeration of location deviation notification reports. |  |
| MessageFilter | 7.5.1.4.2.9 | The message filter information applicable to member VAL UEs or Users of the VAL group in the group change notification. |  |
| MonitorEvents | 7.5.1.4.2.11 | Represents the details of the monitoring and analytics events. | NRM_EventMonitor |
| MonitorEventsReport | 7.5.1.4.2.12 | Represents the monitoring and analytics events information related to VAL UE or User. | NRM_EventMonitor |
| MonitorFilter | 7.5.1.4.2.10 | Represents the filter information VAL User or UEs and the related events to be monitored. | NRM_EventMonitor |
| MonitorLocationInterestFilter | 7.5.1.4.2.14 | Filter information to subscribe for monitoring the VAL UE/User location in a given area of interest. |  |
| MonLocAreaInterestFltr | 7.5.1.4.2.17 | Filter information to subscribe for location area monitoring information for a given area of interest. |  |
| MonLocTriggerEvent | 7.5.1.4.3.5 | Identifies the triggering event in the location area monitor filtering. |  |
| MoveInOutUEDetails | 7.5.1.4.2.21 | Represents the list of UEs either moved into the location area or moved out of the location area. |  |
| MovTimingInfo | 7.5.1.4.2.27 | Represents the timing information of a VAL UE's movements. | LM_LocationAreaMonitor_Ext1 |
| ReferenceUEDetail | 7.5.1.4.2.19 | Represents the reference UE details |  |
| RefUeCap | 7.5.1.4.2.29 | Represents the capabilities of the reference UE. |  |
| DataStatisticsReq | 7.5.1.4.2.24 | Represents the required information to calculate the target UE's location. |  |
| SatSfInfo | 7.5.1.4.2.28 | Represents the event reporting of satellite SF information. | NRM_SatSfInfo |
| SEALEvent | 7.5.1.4.3.3 | Represents the type of SEAL events that can be subscribed. |  |
| SEALEventDetail | 7.5.1.4.2.5 | Represents the SEAL event detail. |  |
| SEALEventNotification | 7.5.1.4.2.3 | Represents an individual SEAL Event Subscription Notification. |  |
| SEALEventSubscription | 7.5.1.4.2.2 | Represents an individual SEAL Event Subscription resource. |  |
| SEALEventSubscriptionPatch | 7.5.1.4.2.22 | Represents the parameters to request the modification of a SEAL Event subscription resource. | SubscUpdate |
| TempGroupInfo | 7.5.1.4.2.16 | Represents the created temporary VAL group information. | GM_TempGroup |
| TimeReqs | 7.5.1.4.2.25 | Represents the required time information to calculate the target UE's location. |  |
| VALGroupFilter | 7.5.1.4.2.6 | Represents a filter of VAL group identifiers belonging to a VAL service. |  |
| ValidityConditions | 7.5.1.4.2.13 | Represents the temporal and/or spatial conditions applied for the events to be monitored. | NRM_EventMonitor |

Table 7.5.1.4.1-2 specifies data types re-used by the SS_Events API service:

Table 7.5.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AccessType | 3GPP TS 29.522 [28] | Used to represent the access type. | LM_LocationDeviation_Ext1 |
| AnalyticsEvent | 3GPP TS 29.522 [28] | Analytics event in NWDAF. | NRM_EventMonitor |
| DateTime | 3GPP TS 29.122 [3] | Used to indicate a timestamp. |  |
| DurationSec | 3GPP TS 29.122 [3] | Used to indicate the notification interval in the location monitoring filter. |  |
| Float | 3GPP TS 29.571 [21] | Used to represent the fractional part of the proximity range in the reference UE details. |  |
| GeoArea | Clause 7.4.1.4.2.19 | Represents the geographical area. |  |
| GeographicArea | 3GPP TS 29.572 [31] | Identifies the geographical information of the user(s). |  |
| LocationArea5G | 3GPP TS 29.122 [3] | User location area when the UE is attached to 5G. | NRM_EventMonitor |
| LocationInfo | 3GPP TS 29.122 [3] | Location information |  |
| LocationQoS | 3GPP TS 29.572 [31] | Identifies QoS requested by VAL server. | LM_LocationInfoChange_Extension1 |
| MonitoringType | 3GPP TS 29.122 [3] | Monitoring event type in 3GPP system core network. | NRM_EventMonitor |
| PlmnIdNid | 3GPP TS 29.571 [21] | Used to represent the identifier of the PLMN. | LM_LocationDeviation_Ext1 |
| ProfileDoc | Clause 7.3.1.4.2.2 | Used to send VAL User or VAL UE profile information as part of event detail in the event notification. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable for SEAL: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [3] | Used to define the time frame for message filters. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the applicability of optional features defined in table 7.5.1.6-1. |  |
| TestNotification | 3GPP TS 29.122 [3] | Following differences apply: - The SCEF is the SEAL server; and - The SCS/AS is the subscribing VAL server. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Time window identified by a start time and a stop time. | NRM_EventMonitor |
| UeStrAndFwdStatus | 3GPP TS 29.122 [3] | Identifies the UE status in Store and Forward mode. |  |
| Uinteger | 3GPP TS 29.571 [21] | Used to represent maximum number of messages in MesageFilter data type. |  |
| Uri | 3GPP TS 29.122 [3] | Used to indicate a notification URI. |  |
| VALGroupDocument | Clause 7.2.1.4.2.2 | Used to send VAL group document as part of event detail in the event notification. |  |
| ValSvcAreaId | Clause 7.1.3.4.3.2 | Used to represent the VAL service area identifier. | ValSrvArea |
| ValTargetUe | 7.3.1.4.2.3 | Used to identify a VAL user ID or a VAL UE ID. |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] | Following differences apply: - The SCEF is the CAPIF core function; and - The SCS/AS is the Subscribing functional entity. |  |

##### 7.5.1.4.2	Structured data types

###### 7.5.1.4.2.1	Introduction

###### 7.5.1.4.2.2	SEALEventSubscription

Table 7.5.1.4.2.2-1: Definition of type SEALEventSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriberId | string | M | 1 | String identifying the subscriber of the event. |  |
| eventSubs | array(EventSubscription) | M | 1..N | Subscribed events. |  |
| eventReq | ReportingInformation | M | 1 | Represents the reporting requirements of the event subscription. |  |
| notificationDestination | Uri | M | 1 | URI where the notification should be delivered to. |  |
| requestTestNotification | boolean | O | 0..1 | Set to true by Subscriber to request the SEAL server to send a test notification as defined in clause 6.6. Set to false or omitted otherwise. | Notification_test_event |
| websockNotifConfig | WebsockNotifConfig | O | 0..1 | Configuration parameters to set up notification delivery over Websocket protocol as defined in clause 6.6. | Notification_websocket |
| eventDetails | array(SEALEventDetail) | C | 1..N | Detailed information of individual Events. Shall only be present in the response from the server if the immediate reporting indication in the "immRep" attribute within the "eventReq" attribute is set to true, and the reports are available. |  |
| suppFeat | SupportedFeatures | O | 0..1 | Used to negotiate the supported optional features of the API as described in clause 6.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. |  |

###### 7.5.1.4.2.3	SEALEventNotification

Table 7.5.1.4.2.3-1: Definition of type SEALEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Identifier of the subscription resource to which the notification is related – SEAL resource identifier |  |
| eventDetails | array(SEALEventDetail) | M | 1..N | Detailed notifications of individual Events. |  |

###### 7.5.1.4.2.4	EventSubscription

Table 7.5.1.4.2.4-1: Definition of type EventSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventId | SEALEvent | M | 1 | Subscribed event |  |
| valGroups | array(VALGroupFilter) | C | 1..N | Each element of the array represents the VAL group identifier(s) of a VAL service that the subscriber wants to know in the interested event. This parameter shall be present only if the event subscribed is "GM_GROUP_INFO_CHANGE". | GM_GroupInfoChange |
| identities | array(IdentityFilter) | C | 1..N | Each element of the array represents the VAL User / UE IDs of a VAL service that the event subscriber wants to know in the interested event. This parameter shall be present only if the event subscribed is "CM_USER_PROFILE_CHANGE" "LM_LOCATION_INFO_CHANGE", or if the "NRM_SatSfInfo" fetaure is supported, "NRM_SAT_SF_INFO". (NOTE) | CM_UserProfileChange, LM_LocationInfoChange, NRM_SatSfInfo |
| monFltr | array(MonitorFilter) | C | 1..N | Each element of the array represents the event monitoring request details that the subscriber wishes to monitor the events related to a set of VAL UEs, VAL group and/or VAL service. This parameter shall be present only if the event subscribed is "NRM_MONITOR_UE_USER_EVENTS" | NRM_EventMonitor |
| areaInt | array(MonitorLocationInterestFilter) | C | 1..N | Each element represent the list of VAL User / UE IDs and the area of interest information for which the subscriber wishes to monitor the location deviation of the VAL User / UEs. This parameter shall be present only if the subscribed event is "LM_LOCATION_DEVIATION_MONITOR". | LM_LocationDeviation |
| locAreaMon | array(MonLocAreaInterestFltr) | C | 1..N | Each element represent the location area monitoring details that the subscriber wishes to monitor for the VAL UEs moving in or moving out of the provided location area. This parameter shall be present only if the subscribed event is "LM_LOCATION_AREA_MONITOR". | LM_LocationAreaMonitor |
| partialFailRep | PartialEventSubscFailRep | C | 0..1 | The partial failure report from the SEAL server indicating the target identifier(s) whose event subscription is not created/updated successfully. This attribute shall be provided only if the PartialFailureSupport feature is supported and the event subscription is not created/updated successfully for all requested target identifier(s). | PartialFailureSupport |
| satSfTrigger | array(SatSfTrigger) | C | 1..N | Contains the satellite S&F related event(s) triggering conditions. This attribute shall be present only if the subscribed event within the "eventId" attribute is "NRM_SAT_SF_INFO". | NRM_SatSfInfo |
| NOTE: The "valSvcId" attribute within IdentityFilter is not applicable for the event "LM_LOCATION_INFO_CHANGE". |  |  |  |  |  |

###### 7.5.1.4.2.5	SEALEventDetail

Table 7.5.1.4.2.5-1: Definition of type SEALEventDetail

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventId | SEALEvent | M | 1 | Event that is notified |  |
| lmInfos | array(LMInformation) | C | 1..N | The location information for the interested VAL User IDs or VAL UE IDs. This parameter shall be present only if the event in event notification is "LM_LOCATION_INFO_CHANGE". | LM_LocationInfoChange |
| valGroupDocuments | array(VALGroupDocument) | C | 1..N | Newly created VAL group documents or the VAL groups documents with modified membership and configuration information. This parameter shall be present only if the event in event notification is "GM_GROUP_INFO_CHANGE" or "GM_GROUP_CREATE". | GM_GroupInfoChange, GM_GroupCreate |
| profileDocs | array(ProfileDoc) | C | 1..N | Updated profile information associated with VAL Users or VAL UEs. This parameter shall be present only if the event in event notification is "CM_USER_PROFILE_CHANGE". | CM_UserProfileChange |
| msgFltrs | array(MessageFilter) | C | 1..N | The message filters applicable to various member VAL User or UEs of the VAL group. This parameter may be present only if the event in the even notification is "GM_GROUP_INFO_CHANGE" | GM_MessageFilter |
| monRep | array(MonitorEventsReport) | C | 1..N | The events report with details of the events related to VAL UE(s). This parameter shall be present only if the event in the event notification is "NRM_MONITOR_UE_USER_EVENTS" | NRM_EventMonitor |
| locAdhr | array(LocationDevMonReport) | C | 1..N | The location deviation information for the interested VAL User IDs or VAL UE IDs in a given location. This parameter shall be present only if the event in event notification is "LM_LOCATION_DEVIATION_MONITOR". | LM_LocationDeviation |
| tempGroupInfo | TempGroupInfo | C | 0..1 | Contains the created temporary VAL group information. This attribute shall be present only if the "eventId" attribute is set to the value "GM_TEMP_GROUP_FORMATION". | GM_TempGroup |
| locAreaMonRep | array(LocationAreaMonReport) | C | 1..N | The location area monitoring information of the given area of interest. This parameter shall be present only if the event in event notification is "LM_LOCATION_AREA_MONITOR". | LM_LocationAreaMonitor |
| valGroupIds | array(string) | C | 1..N | Contains the identifier(s) of the delated VAL Group(s). This attribute shall be present only if the reported event within the "eventId" attribute is "GM_GROUP_DELETION". | GM_GroupDeletion |
| satSfInfo | array(SatSfInfo) | C | 1..N | Contains the satellite SF information for the target VAL User ID(s) or VAL UE ID(s). This attribute shall be present only if the reported event within the "eventId" attribute is "NRM_SAT_SF_INFO". | NRM_SatSfInfo |

###### 7.5.1.4.2.6	VALGroupFilter

Table 7.5.1.4.2.6-1: Definition of type VALGroupFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | O | 0..1 | Identity of the VAL Service that the subscriber is interested in. |  |
| valGrpIds | array(string) | M | 1..N | VAL Group identifiers that the event subscriber wants to know in the interested event. |  |

###### 7.5.1.4.2.7	IdentityFilter

Table 7.5.1.4.2.7-1: Definition of type IdentityFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcId | string | O | 0..1 | Identity of the VAL Service that the subscriber is interested in. |  |
| valTgtUes | array(ValTargetUe) | C | 1..N | VAL User IDs or VAL UE IDs that the event subscriber wants to know in the interested event. This parameter shall be present if the event subscribed is "CM_USER_PROFILE_CHANGE" "LM_LOCATION_INFO_CHANGE", or if the "NRM_SatSfInfo" fetaure is supported, "NRM_SAT_SF_INFO". |  |
| suppLoc | boolean | O | 0..1 | Indication to request for supplementary location information of the VAL UE IDs. Set to true by Subscriber to request the SEAL server to send supplementary location information from the 3GPP core network. Set to false or omitted otherwise. This parameter may be present if the event subscribed is "LM_LOCATION_INFO_CHANGE". | LM_SuppLoc |
| locQos | LocationQoS | O | 0..1 | Indicates the expected location QoS requirement for which the location information is requested. This parameter may be present if the event subscribed is "LM_LOCATION_INFO_CHANGE". | LM_LocationInfoChange_Extension1 |
| velocityReq | boolean | C | 0..1 | Indicates whether velocity of the location information is requested. - "true" indicates that the velocity information is requested. - "false" indicates that the velocity information is not requested. - or omitted (default), to indicate that the velocity information is not requested. This attribute may be present only if the subscribed event is "LM_LOCATION_INFO_CHANGE". | LM_LocationInfoChange_Ext2 |
| locDataStatsReq | boolean | C | 0..1 | Indicates whether the location data statistics of the location information is requested. - "true" indicates that the location data statistics of the location information is requested. - "false" indicates that the location data statistics of the location information is not requested. - or omittedd (default), to indicate that the velocity information is not requested. This attribute may be present only if the subscribed event is "LM_LOCATION_INFO_CHANGE". | LM_LocationInfoChange_Ext2 |
| dataStatisticsReq | DataStatisticsReq | O | 0..1 | Contains the required information to support location data statistics of the location information. This attribute may be present only when the "locDataStatsReq" is present and set to "true". | LM_LocationInfoChange_Ext2 |

###### 7.5.1.4.2.8	LMInformation

Table 7.5.1.4.2.8-1: Definition of type LMInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valTgtUe | ValTargetUe | M | 1 | VAL User ID or UE ID that the event subscriber wants to know in the interested event. |  |
| locInfo | LocationInfo | M | 1 | The location information associated with the valTgtUe. |  |
| timeStamp | DateTime | O | 0..1 | Timestamp of the location report |  |
| valSvcId | string | O | 0..1 | The VAL service ID of the VAL application for which the location information is subscribed. |  |

###### 7.5.1.4.2.9	MessageFilter

Table 7.5.1.4.2.9-1: Definition of type MessageFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reqUe | ValTargetUe | M | 1 | Identity of the VAL User ID or UE ID that the message filter information is related to. |  |
| tgtUe | array(ValTargetUe) | O | 1..N | List of VAL USER or UE IDs whose messages will be sent to the VAL User or UE in reqUe attribute. |  |
| maxMsgs | Uinteger | O | 0..1 | Total number of messages allowed to be sent to the VAL User or UE in the given time frame in the filter. |  |
| scheds | array(ScheduledCommunicationTime) | O | 1..N | Time frame associated to the total number of messages in mxMsgs attribute. |  |
| msgTypes | array(string) | O | 1..N | List of message types be sent to VAL UE. |  |

###### 7.5.1.4.2.10	MonitorFilter

Table 7.5.1.4.2.10-1: Definition of type MonitorFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| idnts | array(ValTargetUe) | C | 1..N | Identities of the VAL Users or UEs whose events monitoring is requested. (NOTE 1) |  |
| valSvcId | string | O | 0..1 | Identity of the VAL service. |  |
| valGrpId | string | C | 0..1 | Identity of the VAL group of the target UEs whose events monitoring is requested. (NOTE 1) |  |
| profId | string | C | 0..1 | The monitoring profile ID identifying a list of monitoring and/or analytics events. (NOTE 2) |  |
| valCnds | array(ValidityConditions) | O | 1..N | The temporal and/or spatial conditions applied for the events to be considered as valid. |  |
| evntDets | array(MonitorEvents) | C | 1..N | List of monitoring and/or analytics events that the VAL server is interested in. (NOTE 2) |  |
| NOTE 1: Either VAL users/UEs or a VAL group identifying VAL UEs shall be present. NOTE 2: Either event details or monitoring profile ID shall be present in the subscription request. The monitoring profile ID shall present in the subscription response when event details are provided in the subscription request. |  |  |  |  |  |

###### 7.5.1.4.2.11	MonitorEvents

Table 7.5.1.4.2.11-1: Definition of type MonitorEvents

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cnEvnts | array(MonitoringType) | O | 1..N | List of monitoring events related to VAL UE. |  |
| anlEvnts | array(AnalyticsEvent) | O | 1..N | List of analytics events related to VAL UE. |  |

###### 7.5.1.4.2.12	MonitorEventsReport

Table 7.5.1.4.2.12-1: Definition of type MonitorEventsReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tgtUe | ValTargetUe | M | 1 | VAL UE for which the events are related. |  |
| evnts | array(MonitorEvents) | M | 1..N | List of monitoring and analytics events related to VAL UE. |  |

###### 7.5.1.4.2.13	ValidityConditions

Table 7.5.1.4.2.13-1: Definition of type ValidityConditions

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locArea | LocationArea5G | O | 0..1 | Spatial validity conditions. |  |
| tmWdws | array(TimeWindow) | O | 1..N | Time window validity conditions |  |

###### 7.5.1.4.2.14	MonitorLocationInterestFilter

Table 7.5.1.4.2.14-1: Definition of type MonitorLocationInterestFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tgtUes | array(ValTargetUe) | M | 1..N | List of VAL User(s) or UE ID(s) for which location monitoring is requested for the given location information. |  |
| locInt | LocationInfo | C | 0..1 | Location information where the VAL server wishes to monitor the target VAL UE(s) location deviation. (NOTE) |  |
| valSvcId | ValSvcAreaId | C | 0..1 | Identifier of the VAL service area where the VAL server wishes to monitor the target VAL UE(s) location deviation. (NOTE) | ValSrvArea, LM_LocationDeviation_Ext1 |
| notInt | DurationSec | M | 1 | Periodic time interval in which the LM server needs to notify the VAL UE's location information. |  |
| dynamicLoc | ReferenceUEDetail | C | 0..1 | Contains the dynamic location area information. (NOTE) | LM_LocationDeviation_Ext1 |
| events | array(LocDevNotification) | O | 1..N | Contains the required location monitoring events. | LM_LocationDeviation_Ext1 |
| geofCond | GeofencingConditions | O | 0..1 | Contains the geofencing conditions, | LM_LocationDeviation_Ext1 |
| NOTE: If the "ValSrvArea" feature is supported, then one of "locInt" or "valSvcId" attributes shall be present; otherwise the "locInt" attribute shall be present. If the "LM_LocationDeviation_Ext1" feature is supported, then only one of the "locInt", "valSvcId" or "dynamicLoc" attributes shall be present; otherwise the "locInt" attribute shall be present. |  |  |  |  |  |

###### 7.5.1.4.2.15	LocationDevMonReport

Table 7.5.1.4.2.15-1: Definition of type LocationDevMonReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tgtUes | array(ValTargetUe) | M | 1..N | VAL User ID(s) or UE ID(s) to which the report is related. |  |
| locInfo | LocationInfo | M | 1 | The location information associated with the valTgtUe. |  |
| notifType | LocDevNotification | M | 1 | Notification about the deviation of the VAL UE(s) in "valTgtUe" attribute to the location in "locInfo" attribute. |  |

###### 7.5.1.4.2.16	TempGroupInfo

Table 7.5.1.4.2.16-1: Definition of type TempGroupInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valGrpIds | array(string) | M | 1..N | Contains a list of the identifiers of the VAL groups constituting the created temporary VAL group. |  |
| tempValGrpId | string | M | 1 | Contains the identifier of the created temporary VAL group. |  |
| valServIds | array(string) | O | 1..N | Contains a list of the identifiers of the VAL services for which communications are to be enabled on the created temporary VAL group. |  |

###### 7.5.1.4.2.17	MonLocAreaInterestFltr

Table 7.5.1.4.2.17-1: Definition of type MonLocAreaInterestFltr

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locInfoCri | LocationInfoCriteria | M | 1 | Contains the location area information where the VAL server wishes to monitor the VAL UE(s) moving in or moving out. This attribute applies also for geofencing services when the "LM_LocationAreaMonitor_Ext1" feature is supported. In such case, it contains the geofencing area. |  |
| trigEvnts | array(MonLocTriggerEvent) | O | 1..N | Contains the triggering events when to send the notifications. This attribute applies also for geofencing services when the "LM_LocationAreaMonitor_Ext1" feature is supported. In such case, it shall be present and contain the subscribed geofencing monitoring event(s). |  |
| valServiceId | string | O | 0..1 | Contains the identifier of the VAL service for which location area monitoring is requested. | LM_LocationAreaMonitor_Ext1 |

###### 7.5.1.4.2.18	LocationInfoCriteria

Table 7.5.1.4.2.18-1: Definition of type LocationInfoCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| geoArea | GeographicArea | C | 0..1 | Geographic location information where the VAL server wishes to monitor the VAL UE(s) moving in or moving out. |  |
| valSvcAreaId | ValSvcAreaId | C | 0..1 | Identifier of the VAL service area. | ValSrvArea |
| refUe | ReferenceUEDetail | C | 0..1 | Reference UE details with proximity range where the VAL server wishes to monitor the VAL UE(s) moving in or moving out. |  |
| NOTE: Either "geoArea" or "refUe" shall be provided. |  |  |  |  |  |

###### 7.5.1.4.2.19	ReferenceUEDetail

Table 7.5.1.4.2.19-1: Definition of type ReferenceUEDetail

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valTgtUe | ValTargetUe | M | 1 | VAL User ID or UE ID that the event subscriber wants to know in the interested event. |  |
| proxRange | Uinteger | M | 1 | Proximity range of the area around the VAL target UE in meters. This attribure represents the integer part of the proximity range value. |  |
| proxRangeFrac | Float | O | 0..1 | The fractional part of the proximity range value in meters. The minimum value of this attribute is 0. The maximum value of this attribute is 1. |  |

###### 7.5.1.4.2.20	LocationAreaMonReport

Table 7.5.1.4.2.20-1: Definition of type LocationAreaMonReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| curPreUEs | array(ValTargetUe) | O | 1..N | Contains the list of the identifier(s) of all VAL UE(s) who are currently present in the given location area. (NOTE) |  |
| moveInOutUEs | MoveInOutUEDetails | C | 0..1 | Contains the list of the UE(s) that either moved in to the location area or moved out of the location area. This attribute applies also for geofencing services when the "LM_LocationAreaMonitor_Ext1" feature is supported. In such case, it may also contain the statistics (e.g., number of UEs, timing information) of the UE(s) moving in/out of the geofencing area. (NOTE) |  |
| trigEvnt | MonLocTriggerEvent | C | 0..1 | Contains the event that triggered the sending of the notification. This attribute applies also for geofencing services when the "LM_LocationAreaMonitor_Ext1" feature is supported. In such case, it shall be present and contain the reported geofencing monitoring event. (NOTE) |  |
| NOTE: When the "LM_LocationAreaMonitor_Ext1" feature is not supported, for the first notification report the "curPreUEs" attribute shall be provided, and for the next notification report, either the "curPreUEs" or the "moveInOutUEs" attribute shall be present. When the "LM_LocationAreaMonitor_Ext1" feature is supported, the "curPreUEs" attribute may not be present, and the "moveInOutUEs" and "trigEvnt" attributes shall be present. |  |  |  |  |  |

###### 7.5.1.4.2.21	MoveInOutUEDetails

Table 7.5.1.4.2.21-1: Definition of type MoveInOutUEDetails

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| moveInUEs | array(ValTargetUe) | O | 1..N | Contains the list of the identifier(s) of the VAL UE(s) that moved into the given location area since the previous reporting. This attribute applies also for geofencing services when the "LM_LocationAreaMonitor_Ext1" feature is supported. In such case, it contains the VAL UE(s) that moved into the geofencing area. |  |
| numUEsMovedIn | Uinteger | O | 0..1 | Contains the number of UE(s) that moved into the geofencing area since the previous notification. | LM_LocationAreaMonitor_Ext1 |
| timingUEsMovedIn | map(MovTimingInfo) | O | 1..N | Contains the timing information of the VAL UE(s) that moved into the geofencing area since the previous notification. The key of the map shall be set to the value of the identifier of the VAL UE (among the ones provided within the "moveInUEs" attribute) to which the timing information provided within the map value is related. | LM_LocationAreaMonitor_Ext1 |
| moveOutUEs | array(ValTargetUe) | O | 1..N | Contains the list of the identifier(s) of the VAL UE(s) that moved out of the given location area since the previous reporting. This attribute applies also for geofencing services when the "LM_LocationAreaMonitor_Ext1" feature is supported. In such case, it contains the VAL UE(s) that moved out of the geofencing area. |  |
| numUEsMovedOut | Uinteger | O | 0..1 | Contains the number of UE(s) that moved out of the geofencing area. | LM_LocationAreaMonitor_Ext1 |
| timingUEsMovedOut | map(MovTimingInfo) | O | 1..N | Contains the timing information of the VAL UE(s) that moved out of the geofencing area since the previous notification. The key of the map shall be set to the value of the identifier of the VAL UE (among the ones provided within the "moveOutUEs" attribute) to which the timing information provided within the map value is related. | LM_LocationAreaMonitor_Ext1 |

###### 7.5.1.4.2.22	SEALEventSubscriptionPatch

Table 7.5.1.4.2.22-1: Definition of type SEALEventSubscriptionPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubs | array(EventSubscription) | O | 1..N | Subscribed events. |  |
| eventReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the event subscription. |  |
| notificationDestination | Uri | O | 0..1 | URI where the notification should be delivered to. |  |

###### 7.5.1.4.2.23	PartialEventSubscFailRep

Table 7.5.1.4.2.23-1: Definition of type PartialEventSubscFailRep

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valTgtUes | array(ValTargetUe) | O | 1..N | List of VAL user(s) / VAL UE(s) whose identifier(s) is not found. This attribute may be present only if the subscribed event is "CM_USER_PROFILE_CHANGE", "LM_LOCATION_INFO_CHANGE", "NRM_MONITOR_UE_USER_EVENTS", or "LM_LOCATION_DEVIATION_MONITOR". |  |
| valGrpIds | array(string) | O | 1..N | List of VAL group(s) whose identifier(s) is not found. This attribute may be present only if the subscribed event is "GM_GROUP_INFO_CHANGE". |  |
| NOTE: Either the "valTgtUes" attribute or the "valGrpIds" attribute shall be present. |  |  |  |  |  |

###### 7.5.1.4.2.24	DataStatisticsReq

Table 7.5.1.4.2.24-1: Definition of type DataStatisticsReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timeReqs | TimeReqs | O | 0..1 | Contains the time requirements applicable for location data statistics. |  |
| locReqs | array(GeoArea) | O | 1..N | Contains the location area requirements applicable for location data statistics. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 7.5.1.4.2.25	TimeReqs

Table 7.5.1.4.2.25-1: Definition of type TimeReqs

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timeWindows | array(TimeWindow) | C | 1..N | Contains the applicable time window(s). (NOTE) |  |
| timeSchedule | array(ScheduledCommunicationTime) | C | 1..N | Contains the applicable time schedule. (NOTE) |  |
| NOTE: These attributes are mutually exclusive and either one of them shall be present. |  |  |  |  |  |

###### 7.5.1.4.2.26	GeofencingConditions

Table 7.5.1.4.2.26-1: Definition of type GeofencingConditions

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| plmn | PlmnIdNid | O | 0..1 | Represents the identifier to be used for geofencing. |  |
| roamingInd | boolean | O | 0..1 | Represents the roaming status to be used for geofencing. - "true" indicates that the roaming status is roaming. - "false" indicates that the roaming status is non-roaming. The default value is "false” when this attribute is omitted. |  |
| accessType | AccessType | O | 0..1 | Represents the access type to be used for geofencing. |  |
| locInfo | LocationInfo | O | 0..1 | Represents the spatial conditions to be used for geofencing. |  |
| schedules | array(ScheduledCommunicationTime) | O | 1..N | Represents the requested monitoring schedule. |  |
| refUECap | RefUeCap | C | 0..1 | Contains the capabilities of the reference UE/user. This attribute may be present only when the "dynamicLoc" attribute is present within the MonitorLocationInterestFilter data structure. |  |

###### 7.5.1.4.2.27	MovTimingInfo

Table 7.5.1.4.2.27-1: Definition of type MovTimingInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timestamp | DateTime | C | 0..1 | Contains the time at which the VAL UE moved in/out. (NOTE) |  |
| duration | DurationSec | C | 0..1 | Contains the time duration for the VAL UE to move in/out. For the case of the UE moving in, this attribute contains the time duration that the UE spends out of the area (i.e., since the last time the UE moved out of the area) before moving inside the area. For the case of the UE moving out, this attribute contains the time duration that the UE spends inside the area before moving outside of the area. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 7.5.1.4.2.28	SatSfInfo

Table 7.5.1.4.2.28-1: Definition of type SatSfInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| satSfTrigger | SatSfTrigger | M | 1 | Contains the reported satellite S&F triggering event. |  |
| tgtUe | ValTargetUe | M | 1 | Contains the identifier of the VAL User or VAL UE to which the report is related. |  |
| ueSfMode | UeStrAndFwdStatus | C | 0..1 | Contains the UE status in S&F mode information. This attribute shall be present only if the triggering event within the "satSfTrigger" attribute is "UE_SF_MODE". |  |
| estTime | DurationSec | C | 0..1 | Contains the DL S&F estimated delivery time. This attribute shall be present only if the triggering event within the "satSfTrigger" attribute is "EST_DELIVERY_TIME". |  |
| timeStamp | DateTime | O | 0..1 | Contains the timestamp of the report. |  |

###### 7.5.1.4.2.29	RefUeCap

Table 7.5.1.4.2.29-1: Definition of type RefUeCap

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| refUECap | Float | M | 1 | Contains the battery power threshold (expressed as a percentage) for the reference UE/user, indicating that the UE can serve as reference UE only if its available battery power is above this threshold. Minimum: 0, maximum: 100 |  |

##### 7.5.1.4.3	Simple data types and enumerations

###### 7.5.1.4.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.5.1.4.3.2	Simple data types

None.

###### 7.5.1.4.3.3	Enumeration: SEALEvent

Table 7.5.1.4.3.3-1: Enumeration SEALEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LM_LOCATION_INFO_CHANGE | Events related to the location information of VAL Users or VAL UEs from the Location Management Server. | LM_LocationInfoChange |
| GM_GROUP_INFO_CHANGE | Events related to the modification of VAL group membership and configuration information from the Group Management Server. | GM_GroupInfoChange |
| CM_USER_PROFILE_CHANGE | Events related to update of user profile information from the Configuration Management Server. | CM_UserProfileChange |
| GM_GROUP_CREATE | Events related to creation of new VAL groups from the Group Management Server. | GM_GroupCreate |
| NRM_MONITOR_UE_USER_EVENTS | Monitoring and analytic events related to VAL UEs, users or VAL group from the Network Resource Management Server. | NRM_EventMonitor |
| LM_LOCATION_DEVIATION_MONITOR | Events from Location Management Server, related to the deviation of the VAL User(s) / UE(s) location from an area of interest. | LM_LocationDeviation |
| GM_TEMP_GROUP_FORMATION | Events related to the formation of new temporary VAL groups from the Group Management Server. | GM_TempGroup |
| LM_LOCATION_AREA_MONITOR | Events from Location Management Server, related to the list of UEs moving in or moving out of the specific location. | LM_LocationAreaMonitor |
| GM_GROUP_DELETION | Events related to deletion of existing VAL Group(s) from the GM Server. | GM_GroupDeletion |
| NRM_SAT_SF_INFO | Events related to satellite S&F information. | NRM_SatSfInfo |

###### 7.5.1.4.3.4	Enumeration: LocDevNotification

Table 7.5.1.4.3.4-1: Enumeration LocDevNotification

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| NOTIFY_MISMATCH_LOCATION | This value indicates that the location information of the VAL UE(s) from the SEAL LM client and the core network are not matching. |  |
| NOTIFY_ABSENCE | This value indicates that the current location information of the VAL UE(s) is deviating from the VAL server's area of interest. |  |
| NOTIFY_PRESENCE | This value indicates that the current location information of the VAL UE(s) is within the VAL server's area of interest. |  |
| NOTIFY_UNKNOWN | This value indicates that the LM Server cannot determine whether or not the targeted VAL UE is within the area of interest. | LM_LocationDeviation_Ext1 |

###### 7.5.1.4.3.5	Enumeration: MonLocTriggerEvent

Table 7.5.1.4.3.5-1: Enumeration MonLocTriggerEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DISTANCE_TRAVELLED | Trigger event for the location area monitoring based on the distance travelled by the reference UE. |  |
| MOVING_IN | Indicates that the event is the UE(s) moving in. This event applies only to geofencing services. | LM_LocationAreaMonitor_Ext1 |
| MOVING_OUT | Indicates that the event is the UE(s) moving out. This event applies only to geofencing services. | LM_LocationAreaMonitor_Ext1 |
| NUMBER_UE_MOVING_IN | Indicates that the event is the number of UE(s) moving in. This event applies only to geofencing services. | LM_LocationAreaMonitor_Ext1 |
| NUMBER_UE_MOVING_OUT | Indicates that the event is the number of UE(s) moving out. This event applies only to geofencing services. | LM_LocationAreaMonitor_Ext1 |
| TIME_UE_MOVING_IN | Indicates that the event is the timing information for the UE(s) moving in. This event applies only to geofencing services. | LM_LocationAreaMonitor_Ext1 |
| TIME_UE_MOVING_OUT | Indicates that the event is the timing information for the UE(s) moving out. This event applies only to geofencing services. | LM_LocationAreaMonitor_Ext1 |

###### 7.5.1.4.3.6	Enumeration: SatSfTrigger

Table 7.5.1.4.3.6-1: Enumeration SatSfTrigger

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE_SF_MODE | Indicates that the triggering condition is the UE status in S&F mode. |  |
| EST_DELIVERY_TIME | Indicates that the triggering condition is the DL S&F estimated delivery time. |  |

#### 7.5.1.5	Error Handling

##### 7.5.1.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.5.1.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_Events API.

##### 7.5.1.5.3	Application Errors

The application errors defined for SS_Events API are listed in table 7.5.1.5.3-1.

Table 7.5.1.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.5.1.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8. Table 7.5.1.6-1 lists the supported features for SS_Events API.

Table 7.5.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 6.6. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 6.6. This feature requires that the Notification_test_event feature is also supported. |
| 3 | LM_LocationInfoChange | This feature supports the location information change event. |
| 4 | GM_GroupInfoChange | This feature supports the group information change event. |
| 5 | CM_UserProfileChange | This feature supports the user profile change event. |
| 6 | GM_GroupCreate | This feature supports the group creation event. |
| 7 | GM_MessageFilter | This feature supports the message filter information in group information change event. |
| 8 | NRM_EventMonitor | This feature supports the monitoring of events related to VAL UEs or Users. |
| 9 | LM_LocationDeviation | This feature supports the monitoring of VAL UE / User's deviation from a given area of interest. |
| 10 | GM_TempGroup | This feature supports the functionality of temporary VAL group formation within a VAL system. |
| 11 | LM_LocationAreaMonitor | This feature supports the monitoring of VAL UEs which are moving in or moving out from a given area of interest. |
| 12 | SubscUpdate | Indicates the support for updating an SEAL event subscription resource. |
| 13 | LM_SuppLoc | This feature indicates the support of supplementary location information. This feature requires the support of the LM_LocationInfoChange feature. |
| 14 | enNB1 | This feature indicates the support of enhancements to this application layer API in Rel-18. |
| 15 | PartialFailureSupport | Indicates the support of the partial failure cases during a SEAL event subscription creation/update. |
| 16 | ValSrvArea | This feature indicates the support of VAL service area ID functionality as part of the phase-3 of the enhancements to the SEAL framework. The following functionalities are supported: - Support the usage of the VAL service area identifier to identify a VAL service area. |
| 17 | LM_LocationInfoChange_Extension1 | This feature indicates the support of the enhancement to location information change event as part of the 5G-enabled fused location service capability exposure. The following functionalities are supported: - Support location information requests with expected location QoS requirements. This feature requires the support of the LM_LocationInfoChange feature. |
| 18 | GM_GroupDeletion | This feature indicates the support of group deletion event notifications as part of the enhancements to this application layer API. |
| 19 | LM_LocationInfoChange_Ext2 | This feature indicates the support of the enhancements to enable application layer support for location services (eLSAPP). The following functionalities are supported: - Support velocity and location data statistic requirements. This feature requires the support of the "LM_LocationInfoChange" feature. |
| 20 | LM_LocationDeviation_Ext1 | This feature indicates the support of the enhancement to enable application layer support for location services. The following functionalities are supported: - Support of geofencing conditions provisioning and management. - Support the new "NOTIFY_UNKNOWN" event. This feature requires the support of the LM_LocationDeviation feature. |
| 21 | LM_LocationAreaMonitor_Ext1 | This feature indicates the support of the enhancement to the location area monitoring functionality to enable enhanced application layer support for location services (eLSAPP). The following functionalities are supported: - Support geofencing services. This feature requires the support of the "LM_LocationAreaMonitor" feature. |
| 22 | NRM_SatSfInfo | This feature indicates the support of the subscription and event notification of satellite S&F related event(s) to enable enhanced application layer support for 5G Satellite services. The following functionalities are supported: - Support satellite to subscribe to S&F related event(s) reporting. |

## 7.6	Key management APIs

### 7.6.1	SS_KeyInfoRetrieval API

#### 7.6.1.1	API URI

The request URI used in each HTTP request from the VAL server towards the Key management server shall have the structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-kir".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.6.1.2.

#### 7.6.1.2	Resources

##### 7.6.1.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.6.1.2.1-1 depicts the resource URIs structure for the SS_KeyInfoRetrieval API.

Figure 7.6.1.2.1-1: Resource URI structure of the SS_KeyInfoRetrieval API

Table 7.6.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.6.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Key records | /key-records | GET | Retrieve key management information uniquely applicable to VAL service, VAL user or VAL UE. |

##### 7.6.1.2.2	Resource: Key Records

###### 7.6.1.2.2.1	Description

The Key Records resource represents the key management information of all VAL services that are created at a given key management server.

###### 7.6.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-kir/<apiVersion>/key-records

This resource shall support the resource URI variables defined in the table 7.6.1.2.2.2-1.

Table 7.6.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.6.1.2.2.3	Resource Standard Methods

7.6.1.2.2.3.1	GET

This operation retrieves VAL service key management information satisfying the filter criteria. This method shall support the URI query parameters specified in table 7.6.1.2.2.3.1-1.

Table 7.6.1.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| val-tgt-ue | ValTargetUe | O | 0..1 | Identifying a VAL user or a VAL UE. |
| val-service-id | string | M | 1 | String identifying a VAL service. |

This method shall support the request data structures specified in table 7.6.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.6.1.2.2.3.1 -3.

Table 7.6.1.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.6.1.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ValKeyInfo | M | 1 | 200 OK | Key management information specific to VAL service, VAL user or VAL UE. This response shall include key management information matching the query parameters provided in the request. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative key management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative key management server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.6.1.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative key management server. |

Table 7.6.1.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative key management server. |

###### 7.6.1.2.2.4	Resource Custom Operations

None.

#### 7.6.1.3	Notifications

None.

#### 7.6.1.4	Data Model

##### 7.6.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.6.1.4.1-1 specifies the data types defined specifically for the SS_KeyInfoRetrieval API service.

Table 7.6.1.4.1-1: SS_KeyInfoRetrieval API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ValKeyInfo | 7.6.1.4.2.3 | Key management information associated with VAL server, VAL user or VAL UE. |  |

Table 7.6.1.4.1-2 specifies data types re-used by the SS_KeyInfoRetrieval API service.

Table 7.6.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to identify a VAL User ID or VAL UE ID applicable to key management information. |  |

##### 7.6.1.4.2	Structured Data Types

###### 7.6.1.4.2.1	Introduction

###### 7.6.1.4.2.2	ValKeyInfo

Table 7.6.1.4.2.3-1: Definition of type ValKeyInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| userUri | Uri | M | 1 | URI of the user for which the response is intended. |  |
| skmsId | string | O | 0..1 | String identifying the SEAL key management server, sending the response. |  |
| valService | string | M | 1 | String identifying the VAL service. This attribute shall be same as in the HTTP GET request. |  |
| valTgtUe | ValTargetUe | O | 0..1 | String identifying a VAL user or VAL UE. This value depends on the value that was in the HTTP GET request. |  |
| keyInfo | string | M | 1 | Key management information uniquely applicable to the requested VAL service, VAL user or VAL UE or VAL client. |  |

##### 7.6.1.4.3	Simple data types and enumerations

None.

#### 7.6.1.5	Error Handling

##### 7.6.1.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.6.1.5.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_KeyInfoRetrieval API.

##### 7.6.1.5.3	Application Errors

The application errors defined for SS_KeyInfoRetrieval API are listed in table 7.6.1.5.3-1.

Table 7.6.1.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.6.1.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8.

Table 7.6.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 7.6.2	SS_KMParametersProvisioning API

#### 7.6.2.1	Introduction

The SS_KMParametersProvisioning service shall use the SS_KMParametersProvisioning API.

The API URI of the SS_KMParametersProvisioning API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-kpp".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.6.2.2.

#### 7.6.2.3	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_KMParametersProvisioning API.

#### 7.6.2.3	Resources

There are no resources defined for this resource in this release of the sepcifciation.

#### 7.6.2.4	Custom operations without associated resources

##### 7.6.2.4.1	Overview

The structure of the custom operation URIs of the SS_KMParametersProvisioning API is shown in Figure 7.6.2.4.1-1.

Figure 7.6.2.4.1-1: Custom operation URI structure of the SS_KMParametersProvisioning API

Table 7.6.2.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the SS_KMParametersProvisioning API.

Table 7.6.2.4.1-1: Custom operations without associated resources

| Custom operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Request | /request | POST | Enables a service consumer to request key parameters provisioning to the SEAL KM Server. |

The custom operations shall support the URI variables defined in table 7.6.2.4.1-2.

Table 7.6.2.4.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.6.2.1. |

##### 7.6.2.4.2	Operation: Request

###### 7.6.2.4.2.1	Description

The custom operation enables a service consumer to request key parameters provisioning to the SEAL KM Server.

###### 7.6.2.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 7.6.2.4.2.2-1 and the response data structures and response codes specified in table 7.6.2.4.2.2-2.

Table 7.6.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VALKeyPpReq | M | 1 | Contains the key management parameters to provisioned. |

Table 7.6.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALKeyPpResp |  |  | 200 OK | Successful case. The requested key management parameters are successfully received, processed and provisioned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative SEAL KM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative SEAL KM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3] |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.6.2.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative SEAL KM Server. |

Table 7.6.2.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative SEAL KM Server. |

#### 7.6.2.5	Notifications

There are no notifications defined for this API in this release of the sepcifciation.

#### 7.6.2.6	Data Model

##### 7.6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 7.6.2.6.1-1 specifies the data types defined specifically for the SS_KMParametersProvisioning API service.

Table 7.6.2.6.1-1: SS_KMParametersProvisioning API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| VALKeyPpReq | 7.6.2.5.2.2 | Represents the VAL key management parameters to be provisioned. |  |
| VALKeyPpReq | 7.6.2.5.2.2 | Represents the response to a key management parameters provisioning request. |  |

Table 7.6.2.6.1-2 specifies data types re-used by the SS_KMParametersProvisioning API service.

Table 7.6.2.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Bytes | 3GPP TS 29.571 [21] | Represents a string formatted as a sequence of bytes. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.571 [21] | Represents a URI. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Represents the identifier of the targeted VAL UE or VAL User. |  |

##### 7.6.2.6.2	Structured data types

###### 7.6.2.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

###### 7.6.2.6.2.2	Type: VALKeyPpReq

Table 7.6.2.6.2.2-1: Definition of type VALKeyPpReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| skmcUri | Uri | M | 1 | Contains the URI of the SEAL KM Client (residing in the VAL Server) that is initiating the request. |  |
| valServiceId | string | M | 1 | Contains the identifier of the VAL service to which the provisioned key management parameters are related. |  |
| valTgtUe | ValTargetUe | O | 0..1 | Contains the identifier of the VAL User or VAL UE for which the provisioned key management parameters are related. |  |
| payloadId | string | O | 0..1 | Contains the identifier of the provisioned key management parameters. |  |
| payload | Bytes | C | 0..1 | Contains the provisioned key management parameters information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.6.2.8. This attribute shall be present only when feature negotiation needs to take place. |  |

###### 7.6.2.6.2.2	Type: VALKeyPpResp

Table 7.6.2.6.2.2-1: Definition of type VALKeyPpResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| skmcUri | Uri | M | 1 | Contains the URI of the SEAL Key Management Client (residing in the VAL Server) that initiated the corresponding request. |  |
| valServiceId | string | M | 1 | Contains the identifier of the VAL service to which the provisioned key management parameters are related. |  |
| skmsId | string | O | 0..1 | Contains the identifier of the SEAL KM Server that is sending the response. |  |
| valTgtUe | ValTargetUe | O | 0..1 | Contains the identifier of the VAL User or VAL UE for which the provisioned key management parameters are related. |  |
| payloadId | string | O | 0..1 | Contains the identifier of the provisioned key management parameters. |  |
| suppFeat | SupportedFeatures | O | 0..1 | Contains the list of supported features among the ones defined in clause 7.6.2.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 7.6.2.6.3	Simple data types and enumerations

###### 7.6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.6.2.6.3.2	Simple data types

The simple data types defined in table 6.3.6.3.2-1shall be supported.

Table 7.6.2.6.3.2-1: Simple data types

| Type name | Description |
| --- | --- |

##### 7.6.2.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 7.6.2.6.5	Binary data

###### 7.6.2.6.5.1	Binary Data Types

Table 7.6.2.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.6.2.7	Error Handling

##### 7.6.2.7.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.6.2.7.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_KMParametersProvisioning API.

##### 7.6.2.7.3	Application Errors

The application errors defined for SS_KMParametersProvisioning API are listed in table 7.6.2.7.3-1.

Table 7.6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.6.2.8	Feature negotiation

The optional features listed in table 7.6.2.8-1 are defined for the SS_KMParametersProvisioning API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 7.7	Network Slice Capability Enablement APIs

NSCE APIs are defined in 3GPP TS 29.435 [42].

## 7.8	Identity management APIs

### 7.8.1	SS_IdmParameterProvisioning API

#### 7.8.1.1	API URI

The SS_IdmParameterProvisioning service shall use the SS_IdmParameterProvisioning API.

The API URI of the SS_IdmParameterProvisioning API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-ipp".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.8.1.2.

#### 7.8.1.2	Resources

##### 7.8.1.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.8.1.2.1-1 depicts the resource URIs structure for the SS_IdmParameterProvisioning API.

Figure 7.8.1.2.1-1: Resource URI structure of the SS_IdmParameterProvisioning API

Table 7.8.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.8.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| VAL Services Configurations | /configurations | POST | Request the creation of a VAL Services Configuration. |
|  |  | GET | Retrieve one or several VAL services configuration(s). |
| Individual VAL Services Configuration | /configurations/{confId} | GET | Retrieve an existing "Individual VAL Services Configuration" resource. |
|  |  | PUT | Update an "Individual VAL Services Configuration" resource. |
|  |  | PATCH | Modify an existing "Individual VAL Services Configuration" resource. |
|  |  | DELETE | Delete an existing "Individual VAL Services Configuration" resource. |

##### 7.8.1.2.2	Resource: VAL Services Configurations

###### 7.8.1.2.2.1	Description

This resource represents the collection of VAL Services Configurations managed by the IM Server.

###### 7.8.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-ipp/<apiVersion>/configurations

This resource shall support the resource URI variables defined in the table 7.8.1.2.2.2-1.

Table 7.8.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |

###### 7.8.1.2.2.3	Resource Standard Methods

7.8.1.2.2.3.1	POST

The HTTP POST method enables to request the creation of a VAL Services Configuration to the IM Server.

This method shall support the URI query parameters specified in table 7.8.1.2.2.3.1-1.

Table 7.8.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.8.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.8.1.2.2.3.1-3.

Table 7.8.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VALServicesConfig | M | 1 | Represents the VAL services configurations that needs to be created. |

Table 7.8.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALServicesConfig | M | 1 | 201 Created | Successful case. The requested VAL services configuration is successfully created. The URI of the created "Individual VAL Services Configuration" resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.8.1.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-ipp/<apiVersion>/configurations/{confId} |

7.8.1.2.2.3.2	GET

The HTTP GET method enables to retrieve one or several VAL services configuration(s) satisfying filtering criteria at the IM Server.

This method shall support the URI query parameters specified in table 7.8.1.2.2.3.2-1.

Table 7.8.1.2.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| val-server-id | string | O | 0..1 | Contains the identifier of the VAL Server. |
| config-ids | array(string) | O | 1..N | Contains the list of the identifier(s) of the targeted "Individual VAL Services Configuration" resource(s). |
| supp-feats | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.8.1.7. This query parameter shall be present only when feature negotiation needs to take place. |

This method shall support the request data structures specified in table 7.8.1.2.2.3.2-2 and the response data structures and response codes specified in table 7.8.1.2.2.3.2-3.

Table 7.8.1.2.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.8.1.2.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(VALServicesConfig) | M | 0..N | 200 OK | Successful case. The requested VAL services configuration(s) matching the received query parameter(s) shall be returned. If there are no active VAL services configuration(s) matching the received query parameter(s), an empty array is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative IM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative IM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.8.1.2.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative IM Server. |

Table 7.8.1.2.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative IM Server. |

###### 7.8.1.2.2.4		Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the sepcifciation.

##### 7.8.1.2.3	Resource: Individual VAL Services Configuration

###### 7.8.1.2.3.1	Description

This resource represents an "Individual VAL Services Configuration" resource managed by the IM Server.

###### 7.8.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-ipp/<apiVersion>/configurations/{confId}

This resource shall support the resource URI variables defined in the table 7.8.1.2.3.2-1.

Table 7.8.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| confId | string | Represents an "Individual VAL Services Configuration" resource. |

###### 7.8.1.2.3.3	Resource Standard Methods

7.8.1.2.3.3.1	GET

The HTTP GET method enables to retrieve an "Individual VAL Services Configuration" resource at the IM Server.

This method shall support the URI query parameters specified in table 7.8.1.2.3.3.1-1.

Table 7.8.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.8.1.2.3.3.1-2 and the response data structures and response codes specified in table 7.8.1.2.3.3.1-3.

Table 7.8.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.8.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALServicesConfig | M | 1 | 200 OK | Successful case. The requested "Individual VAL Services Configuration" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative IM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative IM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.8.1.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative IM Server. |

Table 7.8.1.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative IM Server. |

7.8.1.2.3.3.2	PUT

The HTTP PUT method enables to update an existing "Individual VAL Services Configuration" resource at the IM Server.

This method shall support the URI query parameters specified in table 7.8.1.2.3.3.2-1.

Table 7.8.1.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.8.1.2.3.3.2-2 and the response data structures and response codes specified in table 7.8.1.2.3.3.2-3.

Table 7.8.1.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VALServicesConfig | M | 1 | Represents the updated representation of the "Individual VAL Services Configuration" resource. |

Table 7.8.1.2.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALServicesConfig | M | 1 | 200 OK | Successful case. The VAL services configuration is successfully updated and a representation of the updated "Individual VAL Services Configuration" resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The VAL services configuration is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative IM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative IM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.8.1.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative IM Server. |

Table 7.8.1.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative IM Server. |

7.8.1.2.3.3.3	PATCH

The HTTP PATCH method enables to modify an existing "Individual VAL Services Configuration" resource at the IM Server.

This method shall support the URI query parameters specified in table 7.8.1.2.3.3.3-1.

Table 7.8.1.2.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.8.1.2.3.3.3-2 and the response data structures and response codes specified in table 7.8.1.2.3.3.3-3.

Table 7.8.1.2.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| VALServicesConfigPatch | M | 1 | Represents the requested modifications to be applied to the "Individual VAL Services Configuration" resource. |

Table 7.8.1.2.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| VALServicesConfig | M | 1 | 200 OK | Successful case. The VAL services configuration is successfully modified and a representation of the updated "Individual VAL Services Configuration" resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The VAL services configuration is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative IM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative IM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.8.1.2.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative IM Server. |

Table 7.8.1.2.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative IM Server. |

7.8.1.2.3.3.4	DELETE

The HTTP DELETE method enables to delete an existing "Individual VAL Services Configuration" resource at the IM Server.

This method shall support the URI query parameters specified in table 7.8.1.2.3.3.4-1.

Table 7.2.1.2.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.8.1.2.3.3.4-2 and the response data structures and response codes specified in table 7.8.1.2.3.3.4-3.

Table 7.8.1.2.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.8.1.2.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual VAL Services Configuration" is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative IM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative IM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.8.1.2.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative IM Server. |

Table 7.8.1.2.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative IM Server. |

###### 7.8.1.2.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the sepcifciation.

#### 7.8.1.3	Custom operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the sepcifciation.

#### 7.8.1.4	Notifications

There are no notifications defined for this API in this release of the sepcifciation.

#### 7.8.1.5	Data Model

##### 7.8.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.8.1.5.1-1 specifies the data types defined specifically for the SS_IdmParameterProvisioning API service.

Table 7.8.1.5.1-1: SS_IdmParameterProvisioning API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| VALServicesConfig | 7.8.1.5.2.2 | Represents a VAL services configuration. |  |
| VALServicesParams | 7.8.1.5.2.3 | Represents VAL services configuration information. |  |
| VALServicesConfigPatch | 7.8.1.5.2.4 | Represents of the requested modifications to a VAL services configuration. |  |

Table 7.8.1.5.1-2 specifies data types re-used by the SS_IdmParameterProvisioning API service.

Table 7.8.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Represents the identifier of a VAL User or a VAL UE. |  |

##### 7.8.1.5.2	Structured data types

###### 7.8.1.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.8.1.5.2.2	Type: VALServicesConfig

Table 7.8.1.5.2.2-1: Definition of type VALServicesConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcConf | array(VALServiceParams) | M | 1..N | Contains the list of VAL services configuration information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.8.1.7. This attribute shall be present only when feature negotiation needs to take place. |  |

###### 7.8.1.5.2.3	Type: VALServiceParams

Table 7.8.1.5.2.3-1: Definition of type VALServiceParams

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServiceId | string | M | 1 | Contains the identifier of the VAL service to which the provisioned VAL service configuration, information is provisioned. |  |
| idList | array(ValTargetUe) | M | 1..N | Contains the list of VAL User ID(s) or VAL UE ID(s) that are provisioned for the VAL service identified by the "valServiceId" attribute. |  |

###### 7.8.1.5.2.4	Type: VALServicesConfigPatch

Table 7.8.1.5.2.4-1: Definition of type VALServicesConfigPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valSvcConf | array(VALServiceParams) | O | 1..N | Contains the updated list of VAL services configuration information. |  |

##### 7.8.1.5.3	Simple data types and enumerations

###### 7.8.1.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.8.1.5.3.2	Simple data types

The simple data types defined in table 7.8.1.5.3.2-1 shall be supported.

Table 7.8.1.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 7.8.1.5.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 7.8.1.5.5	Binary data

###### 7.8.1.5.5.1	Binary Data Types

Table 7.8.1.5.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.8.1.6	Error Handling

##### 7.8.1.6.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.8.1.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_IDMParameterProvisioning API.

##### 7.8.1.6.3	Application Errors

The application errors defined for SS_IDMParameterProvisioning API are listed in table 7.8.1.6.3-1.

Table 7.8.1.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.8.1.7	Feature negotiation

The optional features in table 7.8.1.7-1 are defined for the SS_IDMParameterProvisioning API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.8.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 7.9	Data Delivery APIs

SEALDD APIs are defined in 3GPP TS 29.548 [35].

## 7.10	Application data analytics enablement service configuration APIs

### 7.10.1	SS_ADAE_VALPerformanceAnalytics API

#### 7.10.1.1	API URI

The SS_ADAE_VALPerformanceAnalytics service shall use the SS_ADAE_VALPerformanceAnalytics API.

The request URIs used in HTTP requests from the VAL server towards the ADAE server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-adae-pa".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.10.1.2.

#### 7.10.1.2	Resources

##### 7.10.1.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.1.2.1-1 depicts the resource URIs structure for the SS_ADAE_VALPerformanceAnalytics API.

Figure 7.10.1.2.1-1: Resource URI structure of the SS_ADAE_VALPerformanceAnalytics API

Table 7.10.1.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Application performance event subscription | /application-performance | POST | Subscription to VAL performance analytics. |
| Individual application performance event subscription | /application-performance/{appPerfId} | GET | Request the retrieval of an existing "Individual application performance event subscription" resource identified by "appPerfId". |
|  |  | DELETE | Removal of VAL performance analytics subscription identified by "appPerfId". |

##### 7.10.1.2.2	Resource: Application performance event subscription

###### 7.10.1.2.2.1	Description

The subscription to the event of the VAL (application) performance analytics.

###### 7.10.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-pa/<apiVersion>/application-performance

This resource shall support the resource URI variables defined in the table 7.10.1.2.2.2-1.

Table 7.10.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.10.1.2.2.3	Resource Standard Methods

7.10.1.2.2.3.1	POST

This method to subscribe to the event of the VAL performance analytics and shall support the URI query parameters specified in table 7.10.1.2.2.3.1-1.

Table 7.10.1.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.1.2.2.3.1-2 and the response data structures and response codes specified in table 7.10.1.2.2.3.1-3.

Table 7.10.1.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppPerfSub | M | 1 | Subscription to the event of VAL performance analytics. |

Table 7.10.1.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppPerfSub | M | 1 | 201 Created | The subscription to the VAL performance analytics is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.1.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-pa/<apiVersion>/application-performance/{appPerfId} |

###### 7.10.1.2.2.4	Resource Custom Operations

None.

##### 7.10.1.2.3	Resource: Individual application performance event subscription

###### 7.10.1.2.3.1	Description

The Individual application performance event subscription resource represents an individual event subscription of a service consumer to the application performance analytics of the ADAE server.

###### 7.10.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-pa/<apiVersion>/application-performance/{appPerfId}

This resource shall support the resource URI variables defined in the table 7.10.1.2.3.2-1.

Table 7.10.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| appPerfId | string | Identifies an Individual Events Subscription |

###### 7.10.1.2.3.3	Resource Standard Methods

7.10.1.2.3.3.1	GET

This operation retrieves an individual application performance event subscription resource. This method shall support the URI query parameters specified in table 7.10.1.2.3.3.1-1.

Table 7.10.1.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.1.2.3.3.1-2 and the response data structures and response codes specified in table 7.10.1.2.3.3.1-3.

Table 7.10.1.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.1.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppPerfSub | M | 1 | 200 OK | The requested individual application performance event subscription is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.1.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.1.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

7.10.1.2.3.3.2	DELETE

This method shall support the URI query parameters specified in table 7.10.1.2.3.3.2-1.

Table 7.10.1.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.1.2.3.3.2-2 and the response data structures and response codes specified in table 7.10.1.2.3.3.2-3.

Table 7.10.1.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.1.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual subscription is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.1.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.1.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

###### 7.10.1.2.3.4	Resource Custom Operations

None.

#### 7.10.1.3	Notifications

##### 7.10.1.3.1	General

Table 7.10.1.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Application performance event notification | {notifUri} | POST | Notification on VAL performance analytics. |

##### 7.10.1.3.2	Application performance event notification

###### 7.10.1.3.2.1	Description

Application performance event notification is to notify on the event of the VAL performance analytics.

###### 7.10.1.3.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the service consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.1.3.2.2-1.

Table 7.10.1.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.1.3.2.2-2 and the response data structures and response codes specified in table 7.10.1.3.2.2-3.

Table 7.10.1.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppPerfNotif | M | 1 | Notification on VAL performance analytics. |

Table 7.10.1.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The notification for the VAL performance analytics event is accepted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.1.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.1.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.1.4	Data Model

##### 7.10.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.1.4.1-1 specifies the data types defined specifically for the SS_ADAE_VALPerformanceAnalytics API service.

Table 7.10.1.4.1-1: SS_ADAE_VALPerformanceAnalytics API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AnalyticsIdVpa | 7.10.1.4.3.8 | Represents the analytics identifier for VAL performance analytics. | ADAE_Ext1 |
| AnalyticsOutput | 7.10.1.4.2.10 | Represents the VAL performance analytics output. | ADAE_Ext1 |
| AnalyticsCategory | 7.10.1.4.3.3 | Represents the category of analytics. |  |
| AnalyticsType | 7.10.1.4.2.6 | Represents the type of analytics. |  |
| AppPerfNotif | 7.10.1.4.2.3 | Notification information of the application performance analytics. |  |
| AppPerfSub | 7.10.1.4.2.2 | Subscription to the VAL application performance analytics |  |
| ClientApplicationData | 7.10.1.4.2.9 | Represents the ADAE client application data. | ADAE_Ext1 |
| PerfData | 7.10.1.4.2.7 | Represents the performance data. | ADAE_Ext1 |
| PerfThreshold | 7.10.1.4.2.8 | Represents the performance threshold. | ADAE_Ext1 |
| ProdProfileInfo | 7.10.1.4.2.4 | Information about the data producer's support data collection and its access to the produced data |  |
| ProducerCap | 7.10.1.4.2.5 | Indicates data producer capabilities for this data type. |  |
| ProducerData | 7.10.1.4.3.5 | Type of the data produced by the data producer. |  |
| ProducerRole | 7.10.1.4.3.6 | The role of the data producer. |  |
| ProducerType | 7.10.1.4.3.4 | Type of the data producer. |  |

Table 7.10.1.4.1-2 specifies data types re-used by the SS_ADAE_VALPerformanceAnalytics API service:

Table 7.10.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| BitRate | 3GPP TS 29.571 [21] | Represents a bit rate measurement value. |  |
| DurationSec | 3GPP TS 29.122 [3] | Represents a period of time in units of seconds. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable for SEAL: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [3] | Used to define the time frame for message filters. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the supported features. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [21] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.122 [3] | Represents a URI. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to indicate either VAL User ID or VAL UE ID. |  |

##### 7.10.1.4.2	Structured data types

###### 7.10.1.4.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.10.1.4.2.2	Type: AppPerfSub

Table 7.10.1.4.2.2-1: Definition of type AppPerfSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| analyticsType | AnalyticsType | M | 1 | Identifies the type of the VAL application performance analytics. |  |
| analyticsId | AnalyticsIdVpa | C | 0..1 | Represents the requested analytics identifier. This attribute shall be present when "ADAE_Ext1" feature is supported. | ADAE_Ext1 |
| valServiceId | string | M | 1 | The identifier of the VAL service, to which the performance analytics subscription is applied. |  |
| valUeIds | array(ValTargetUe) | O | 1..N | A list of identities of one or more VAL UEs, whose performance analytics are subscribed to. |  |
| valServerId | string | O | 0..1 | If the consumer is different from the VAL server, this identifier represents the VAL server to which the VAL performance analytics subscription is applied. |  |
| dataProdProfile | ProdProfileInfo | O | 0..1 | Characteristics of the data producer to be used. |  |
| clientAppData | ClientApplicationData | O | 0..1 | Represents the ADAE client application data. | ADAE_Ext1 |
| confLevel | Uinteger | O | 0..1 | Indicates the preferred confidence level of the prediction. Minimum = 0. Maximum = 100. |  |
| area | LocationArea5G | O | 0..1 | The geographical or service area to which the VAL performance analytics subscription is applied. |  |
| timeValidity | TimeWindow | O | 0..1 | The time validity of the subscription. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirement of the VAL performance analytics subscription. |  |
| repThrs | array(PerfThreshold) | C | 0..1 | Represents the reporting thresholds. This attribute shall be present when the "notifMethod" attribute within ReportingInformation data structure in the "repReq" attribute is set to "ON_EVENT_DETECTION". | ADAE_Ext1 |
| expReq | ReportingInformation | O | 0..1 | Represents the ADAE client exposure requirements requirement of the VAL performance analytics subscription. | ADAE_Ext1 |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported features of the API as defined in clause 7.10.1.6. This attribute shall be provided in the HTTP POST response of successful subscription creation if it was provided in the request. |  |

###### 7.10.1.4.2.3	Type: AppPerfNotif

Table 7.10.1.4.2.3-1: Definition of type AppPerfNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsOutput | AnalyticsOutput | C | 0..1 | Represents the analytics output. (NOTE 2) | ADAE_Ext1 |
| output | string | C | 0..1 | Predicted (or computed, for statistics) change or sustainability of the VAL performance for a VAL server or a VAL session. (NOTE 2) |  |
| valServerId | string | M | 1 | Identity of the VAL server, the data collection is related to, in the case of the notification is on the VAL performance data collection. |  |
| valUeIds | array(ValTargetUe) | O | 1..N | A list of identities of one or more VAL UEs, the data collection is related to, in the case of the notification is on the VAL performance data collection. |  |
| area | LocationArea5G | C | 0..1 | Represents the applicable area. This attribute shall be present when "ADAE_Ext1" feature is supported. | ADAE_Ext1 |
| confLevel | Uinteger | C | 0..1 | Indicates the achieved confidence level in the case of prediction. Minimum = 0. Maximum = 100. (NOTE 1) |  |
| timeHorizon | TimeWindow | C | 0..1 | The time window to which the predictive analytics apply. (NOTE 1) |  |
| NOTE 1: This attribute shall be provided if the "analyticsType" attribute in the subscription was set to "PREDICTIVE" and it may not be provided otherwise. |  |  |  |  |  |
| NOTE 2: When the “ADAE_Ext1” feature is supported, these attributes are mutually exclusive and only one of them shall be present. |  |  |  |  |  |

###### 7.10.1.4.2.4	Type: ProdProfileInfo

Table 7.10.1.4.2.4-1: Definition of type ProdProfileInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| prodId | string | M | 1 | Identity of the data producer |  |
| prodType | ProducerType | M | 1 | Type of the data producer. |  |
| dataType | ProducerData | M | 1 | Type of information that can be provided by the data producer. |  |
| prodRole | ProducerRole | O | 0..1 | Role of the data producer. |  |
| origProdIds | array(string) | O | 1..N | Identifies the identity of the original data producer if the prod-role is not set to the value "ORIGINAL_PRODUCER"or "GENERATING_ENTITY". If the type of the data producer is that value of “A_DCCF”, this attribute is a list of identities of data producers. |  |
| dataFresh | integer | O | 0..1 | It is set to the duration of the time (in seconds) elapsed time after the data generated if the producer-role does not have the value "ORIGINAL_PRODUCER"or "GENERATING_ENTITY". |  |
| producerCap | ProducerCap | O | 0..1 | Represents data producer capability. |  |

###### 7.10.1.4.2.5	Type: ProducerCap

Table 7.10.1.4.2.5-1: Definition of type ProducerCap

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| durationTime | DurationSec | O | 0..1 | Duration time that the data can be stored |  |
| anonymization | boolean | O | 0..1 | The value "true" means that anonymization is supported, while the value "false" means that anonymization is not supported. The default value is "false" if omitted. |  |
| dataRate | BitRate | O | 0..1 | Rate of data generation. |  |
| schedule | ScheduledCommunicationTime | O | 0..1 | Represents scheduling. |  |

###### 7.10.1.4.2.6	Type: AnalyticsType

Table 7.10.1.4.2.6-1: Definition of type AnalyticsType

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| category | AnalyticsCategory | M | 1 | Represents the analytics category. |  |

###### 7.10.1.4.2.7	Type: PerfData

Table 7.10.1.4.2.7-1: Definition of type PerfData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rtt | Uinteger | O | 0..1 | Contains the RTT in milliseconds. |  |
| avgDataRate | BitRate | O | 0..1 | Contains the average data rate. |  |
| maxDataRate | BitRate | O | 0..1 | Contains the maximum data rate. |  |
| jitter | Uint32 | O | 0..1 | Contains the jitter in nanoseconds. |  |
| qoeMos | Float | O | 0..1 | Contains the MOS. Minimum = 1. Maximum = 5. |  |
| qoeStalEvent | Uinteger | O | 0..1 | Contains the number of stalling events. |  |
| qoeStalRatio | Float | O | 0..1 | Contains the stalling ratio. Minimum = 0. Maximum = 1. |  |
| qosProfileLoad | integer | O | 0..1 | Contains the load level in percentage of the QoS profile. Minimum = 0. Maximum = 100. |  |
| valTrafLoad | integer | O | 0..1 | Contains the load level in percentage of the VAL application traffic. Minimum = 0. Maximum = 100. |  |
| valSesPerfChange | integer | O | 0..1 | Contains the VAL server load and VAL server predicted or expected performance change in percentage. Minimum = -100. Maximum = 100. |  |
| NOTE: At least one of the attributes shall be present. |  |  |  |  |  |

###### 7.10.1.4.2.8	Type: PerfThreshold

Table 7.10.1.4.2.8-1: Definition of type PerfThreshold

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| value | PerfData | M | 1 | Represents the value. |  |
| thrDirect | MatchingDirection | M | 1 | Indicates the threshold matching direction for the analytics threshold provided in the "value" attribute. |  |

###### 7.10.1.4.2.9	Type: ClientApplicationData

Table 7.10.1.4.2.9-1: Definition of type ClientApplicationData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueRoute | UasRoute | C | 0..1 | Represents the expected route of the client. |  |
| sesSched | array(ScheduledCommunicationTime) | C | 1..N | Represents the VAL application traffic schedule. |  |
| NOTE: At least one of the attributes shall be provided. |  |  |  |  |  |

###### 7.10.1.4.2.10	Type: AnalyticsOutput

Table 7.10.1.4.2.10-1: Definition of type AnalyticsOutput

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsId | AnalyticsIdVpa | M | 1 | Represents the requested analytics identifier. |  |
| valServ | PerfData | C | 0..1 | Represents the VAL server performance analytics output. This attribute shall be present only if the "analyticsId" is set to "VAL_SERVER_PERF". |  |
| valSess | PerfData | C | 0..1 | Represents the VAL session performance analytics output. This attribute shall be present only if the "analyticsId" is set to "VAL_SESSION_PERF". |  |

##### 7.10.1.4.3	Simple data types and enumerations

###### 7.10.1.4.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.1.4.3.2	Simple data types

None.

###### 7.10.1.4.3.3	Enumeration: AnalyticsCategory

Table 7.10.1.4.3.3-1: Enumeration AnalyticsCategory

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PREDICTIVE | The event for the VAL application performance analytics is for predictive analytics. |  |
| STATISTICS | The event for the VAL application performance analytics is for statistics analytics. |  |

###### 7.10.1.4.3.4	Enumeration: ProducerType

Table 7.10.1.4.3.4-1: Enumeration ProducerType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ADAE_CLIENT | The data producer is ADAE client. |  |
| A_DCCF | The data producer is A-DDCF. |  |
| VAL_SERVER | The data producer is VAL server. |  |
| SEAL_SERVER | The data producer is SEAL server. |  |
| SEAL_CLIENT | The data producer is SEAL client. |  |
| EES | The data producer is EES. |  |
| EAS | The data producer is EAS. |  |

###### 7.10.1.4.3.5	Enumeration: ProducerData

Table 7.10.1.4.3.5-1: Enumeration ProducerData

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PERFORMANCE_INDICATOR | The data type of the data producer is performance indictor. |  |
| REPRODUCER_USAGE_DATA | The data type of the data producer is reproducer usage data. |  |
| SERVER_LOAD_DATA | The data type of the data producer is server load data. |  |
| APPLICATION_PERFORMANCE | The data type of the data producer is application performance. |  |
| EDGE_LOAD | The data type of the data producer is edge load. |  |

###### 7.10.1.4.3.6	Enumeration: ProducerRole

Table 7.10.1.4.3.6-1: Enumeration ProducerRole

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| GENERATING_ENTITY | The role of the data producer is generating entity. |  |
| ORIGINAL_PRODUCER | The role of the data producer is origina producer. |  |
| RESPOSITORY | The role of the data producer is repository. |  |

###### 7.10.1.4.3.7	Void

###### 7.10.1.4.3.8	Enumeration: AnalyticsIdVpa

Table 7.10.1.4.3.8-1: Enumeration AnalyticsIdVpa

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| VAL_SERVER_PERF | Indicates that requested analytics is VAL server performance analytics. |  |
| VAL_SESSION_PERF | Indicates that requested analytics is VAL session performance analytics. |  |

#### 7.10.1.5	Error Handling

##### 7.10.1.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.10.1.5.2	Protocol Errors

In this release of the specification, there are no additional protocol errors applicable for the SS_ADAE_VALPerformanceAnalytics API.

##### 7.10.1.5.3	Application Errors

The application errors defined for SS_ADAE_VALPerformanceAnalytics API are listed in table 7.10.1.5.3-1.

Table 7.10.1.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.1.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8. Table 7.10.1.6-1 lists the supported features for SS_ADAE_VALPerformanceAnalytics API.

Table 7.10.1.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | ADAE_Ext1 | Indicates the support of the ADAE enhancements in Release-19. Within this feature, the following enhancements are covered: - Support of the detailed analytics output. |

### 7.10.2	SS_ADAE_SlicePerformanceAnalytics API

#### 7.10.2.1	API URI

The SS_ADAE_SlicePerformanceAnalytics service shall use the SS_ADAE_SlicePerformanceAnalytics API.

The request URIs used in HTTP requests from the VAL server towards the ADAE server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-adae-sspa".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.10.2.2.

#### 7.10.2.2	Resources

##### 7.10.2.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.2.2.1-1 depicts the resource URIs structure for the SS_ADAE_SlicePerformanceAnalytics API.

Figure 7.10.2.2.1-1: Resource URI structure of the SS_ADAE_SlicePerformanceAnalytics API

Table 7.10.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Slice specific application performance event subscription | /slice-specific-application-performance | POST | Subscription to the event of the slice-specific application performance analytics. |
| Individual slice specific application performance event subscription | /slice-specific-application-performance/{ssAppPerfId} | GET | Request the retrieval of an existing "Individual slice specific application performance event subscription" resource identified by {ssAppPerfId}. |
|  |  | DELETE | Request the deletion of an existing subscription to slice-specific application performance analytics identified by {ssAppPerfId}. |

##### 7.10.2.2.2	Resource: Slice-specific application performance event subscription

###### 7.10.2.2.2.1	Description

The subscription to the event of the slice-specific application performance analytics.

###### 7.10.2.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-sspa/<apiVersion>/slice-specific-application-performance

This resource shall support the resource URI variables defined in the table 7.10.2.2.2.2-1.

Table 7.10.2.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.10.2.2.2.3	Resource Standard Methods

7.10.2.2.2.3.1	POST

This method to subscribe to the event of the slice-specific application performance analytics and shall support the URI query parameters specified in table 7.10.2.2.2.3.1-1.

Table 7.10.2.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.2.2.2.3.1-2 and the response data structures and response codes specified in table 7.10.2.2.2.3.1-3.

Table 7.10.2.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SliceAppPerfSub | M | 1 | Subscription to the slice-specific application performance analytics event. |

Table 7.10.2.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SliceAppPerfSub | M | 1 | 201 Created | Subscription to the slice-specific application performance analytics is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.2.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-sspa/<apiVersion>/slice-specific-application-performance/{ssAppPerfId} |

###### 7.10.2.2.2.4	Resource Custom Operations

None.

##### 7.10.2.2.3	Resource: Individual slice-specific application performance event subscription

###### 7.10.2.2.3.1	Description

The Individual slice-specific application performance event subscription resource represents an individual event subscription of a service consumer to the slice-specific application performance analytics of the ADAE server.

###### 7.10.2.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-pa/<apiVersion>/slice-specific-application-performance/{ssAppPerfId}

This resource shall support the resource URI variables defined in the table 7.10.2.2.3.2-1.

Table 7.10.2.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| ssAppPerfId | string | Identifies an Individual Events Subscription |

###### 7.10.2.2.3.3	Resource Standard Methods

7.10.2.2.3.3.1	GET

This operation retrieves an individual slice-specific application performance event subscription resource. This method shall support the URI query parameters specified in table 7.10.2.2.3.3.1-1.

Table 7.10.2.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.2.2.3.3.1-2 and the response data structures and response codes specified in table 7.10.2.2.3.3.1-3.

Table 7.10.2.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.2.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SliceAppPerfSub | M | 1 | 200 OK | The requested individual slice-specific application performance event subscription is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.2.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.2.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

7.10.2.2.3.3.2	DELETE

This method shall support the URI query parameters specified in table 7.10.2.2.3.3.2-1.

Table 7.10.2.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.2.2.3.3.2-2 and the response data structures and response codes specified in table 7.10.2.2.3.3.2-3.

Table 7.10.2.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.2.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual subscription is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.2.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.2.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

###### 7.10.2.2.3.4	Resource Custom Operations

None.

#### 7.10.2.3	Notifications

##### 7.10.2.3.1	General

Table 7.10.2.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Slice-specific application performance event notification | {notifUri} | POST | Notification on the slice-specific application performance analytics |

##### 7.10.2.3.2	Slice-specific application performance event notification

###### 7.10.2.3.2.1	Description

Slice-specific application performance event notification is to notify on the event of the slice-specific application performance analytics.

###### 7.10.2.3.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.2.3.2.2-1.

Table 7.10.2.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

If the notification is on the slice-specific application performance analytics, this method shall support the request data structures specified in table 7.10.2.3.2.2-2 and the response data structures and response codes specified in table 7.10.2.3.2.2-3.

Table 7.10.2.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SliceAppPerfNotif | M | 1 | Notification information of the slice-specific application performance analytics. |

Table 7.10.2.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification for the slice-specific application performance analytics event is accepted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.2.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.2.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.2.4	Data Model

##### 7.10.2.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.2.4.1-1 specifies the data types defined specifically for the SS_ADAE_SlicePerformanceAnalytics API service.

Table 7.10.2.4.1-1: SS_ADAE_SlicePerformanceAnalytics API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SliceAppPerfNotif | 7.10.2.4.2.3 | Notification information of the slice specific application performance analytics. |  |
| SliceAppPerfSub | 7.10.2.4.2.2 | Subscription to the slice-specific application performance analytics |  |
| SliceAppPerfAnalytics | 7.10.2.4.2.4 | Represents the analytics output for slice performance analytics. | ADAE_Ext1 |

Table 7.10.2.4.1-2 specifies data types re-used by the SS_ADAE_SlicePerformanceAnalytics API service:

Table 7.10.2.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.2.6 | Represents the type of analytics. |  |
| Dnn | 3GPP TS 29.571 [21] | Used to Identify a DNN. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable for SEAL: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| Snssai | 3GPP TS 29.571 [21] | Used to Identify the S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the supported features. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [21] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.122 [3] | Represents a URI. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to indicate either VAL User ID or VAL UE ID. |  |

##### 7.10.2.4.2	Structured data types

###### 7.10.2.4.2.1	Introduction

This clause defines the data structures to be used in resource representations of this service.

###### 7.10.2.4.2.2	Type: SliceAppPerfSub

Table 7.10.2.4.2.2-1: Definition of type SliceAppPerfSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| analyticsType | AnalyticsType | M | 1 | Identifies the type of the slice-specific application performance analytics. |  |
| sliceId | Snssai | M | 1 | The identifier of the slice or slice instance to which the performance analytics subscription is applied. |  |
| dnn | Dnn | O | 0..1 | Associated DNN. |  |
| valUeIds | array(ValTargetUe) | O | 1..N | A list of identities of one or more VAL UEs whose slice-specific performance analytics are subscribed to. |  |
| valServerId | string | O | 0..1 | If the consumer is different from the VAL server, this identifier represents the VAL server to which the slice-specific performance analytics subscription is applied. |  |
| valServiceId | string | O | 0..1 | Represents the target VAL service identifier. | ADAE_Ext1 |
| confLevel | Uinteger | O | 0..1 | Indicates the preferred confidence level of the prediction. Minimum = 0. Maximum = 100. |  |
| area | LocationArea5G | O | 0..1 | The geographical or service area to which the slice specific application performance analytics subscription is applied. |  |
| timeValidity | TimeWindow | O | 0..1 | The time validity of the subscription in seconds. |  |
| timeHorizon | TimeWindow | O | 0..1 | The time horizon for predictive analytics. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirement of the slice-specific performance analytics subscription. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported features of the API as defined in clause 7.10.2.6. This attribute shall be provided in the HTTP POST response of successful subscription creation if it was provided in the request. |  |

###### 7.10.2.4.2.3	Type: SliceAppPerfNotif

Table 7.10.2.4.2.3-1: Definition of type SliceAppPerfNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsOutput | array(SliceAppPerfAnalytics) | C | 0..1 | Represents the analytics output. (NOTE 2) | ADAE_Ext1 |
| output | string | C | 0..1 | Predicted or expected performance change or sustainability of the slice-specific performance for a VAL server or a VAL session. (NOTE 2) |  |
| confLevel | Uinteger | C | 0..1 | Indicates the achieved confidence level in the case of prediction. Minimum = 0. Maximum = 100.(NOTE 1) |  |
| timeHorizon | TimeWindow | C | 0..1 | The time window to which the predictive analytics apply. (NOTE 1) |  |
| NOTE 1: This attribute shall be provided if the "analyticsType" attribute in the subscription was set to "PREDICTIVE" and it may not be provided otherwise. NOTE 2: When the “ADAE_Ext1” feature is supported, these attributes are mutually exclusive and only one of them shall be present. |  |  |  |  |  |

###### 7.10.2.4.2.4	Type: SliceAppPerfAnalytics

Table 7.10.2.4.2.4-1: Definition of type SliceAppPerfAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServerId | string | C | 0..1 | Represents the VAL server identity. (NOTE 1) |  |
| valServiceId | string | C | 0..1 | Represents the VAL service identity. (NOTE 1) |  |
| sliceId | Snssai | C | 1..N | Represents the network slice. (NOTE 1) |  |
| area | LocationArea5G | O | 0..1 | Represents the area of interest. |  |
| minRtt | Uinteger | O | 0..1 | Represents the minimum RTT in milliseconds. (NOTE 2) |  |
| avgRtt | Uinteger | O | 0..1 | Represents the average RTT in milliseconds. (NOTE 2) |  |
| maxRtt | Uinteger | O | 0..1 | Represents the minimum RTT in milliseconds. (NOTE 2) |  |
| minLat | Uinteger | O | 0..1 | Represents the minimum end-to-end latency in milliseconds. (NOTE 2) |  |
| avgLat | Uinteger | O | 0..1 | Represents the average end-to-end latency in milliseconds. (NOTE 2) |  |
| maxLat | Uinteger | O | 0..1 | Represents the maximum end-to-end latency in milliseconds. (NOTE 2) |  |
| NOTE 1: At least one of these attributes shall be present. NOTE 2: At least one of these attributes shall be present. |  |  |  |  |  |

#### 7.10.2.5	Error Handling

##### 7.10.2.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.10.2.5.2	Protocol Errors

In this release of the specification, there are no additional protocol errors applicable for the SS_ADAE_SlicePerformanceAnalytics API.

##### 7.10.2.5.3	Application Errors

The application errors defined for SS_ADAE_SlicePerformanceAnalytics API are listed in table 7.10.2.5.3-1.

Table 7.10.2.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.2.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8. Table 7.10.2.6-1 lists the supported features for SS_ADAE_SlicePerformanceAnalytics API.

Table 7.10.2.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | ADAE_Ext1 | Indicates the support of the ADAE enhancements in Release-19. Within this feature, the following enhancements are covered: - Support of the detailed analytics output. |

### 7.10.3	SS_ADAE_Ue2UePerformanceAnalytics API

#### 7.10.3.1	API URI

The SS_ADAE_Ue2UePerformanceAnalytics service shall use the SS_ADAE_Ue2UePerformanceAnalytics API.

The request URIs used in HTTP requests from the VAL server towards the ADAE server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-adae-uupa".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.10.3.2.

#### 7.10.3.2	Resources

##### 7.10.3.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.3.2.1-1 depicts the resource URIs structure for the SS_ADAE_Ue2UePerformanceAnalytics API.

Figure 7.10.3.2.1-1: Resource URI structure of the SS_ADAE_Ue2UePerformanceAnalytics API

Table 7.10.3.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.3.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| UE-to-UE Session Performance Event Subscription | /ue2ue-session-performance | POST | Create an individual UE-to-UE session performance analytics event subscription. |
| Individual UE-to-UE Session Performance Event Subscription | /ue2ue-session-performance/{u2uPerfId} | GET | Read the individual UE-to-UE session performance analytics event subscription. |
|  |  | DELETE | Remove the individual UE-to-UE session performance analytics event subscription. |

##### 7.10.3.2.2	Resource: UE-to-UE session performance event subscription

###### 7.10.3.2.2.1	Description

The UE-to-UE session performance event subscription to the event of the UE-to-UE session performance analytics.

###### 7.10.3.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-uupa/<apiVersion>/ue2ue-session-performance

This resource shall support the resource URI variables defined in the table 7.10.3.2.2.2-1.

Table 7.10.3.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.10.3.2.2.3	Resource Standard Methods

7.10.3.2.2.3.1	POST

This method to subscribe to the event of the UE-to-UE session performance analytics and shall support the URI query parameters specified in table 7.10.3.2.2.3.1-1.

Table 7.10.3.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.3.2.2.3.1-2 and the response data structures and response codes specified in table 7.10.3.2.2.3.1-3.

Table 7.10.3.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| U2UPerfSub | M | 1 | Subscription to the UE-to-UE session performance analytics event. |

Table 7.10.3.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| U2UPerfSub | M | 1 | 201 Created | Subscription to the UE-to-UE session performance analytics is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.3.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-uupa/<apiVersion>/ue2ue-session-performance/{u2uPerfId} |

###### 7.10.3.2.2.4	Resource Custom Operations

None.

##### 7.10.3.2.3	Resource: Individual UE-to-UE Session Performance Event Subscription

###### 7.10.3.2.3.1	Description

###### 7.10.3.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-uupa/<apiVersion>/ue2ue-session-performance/{u2uPerfId}

This resource shall support the resource URI variables defined in table 7.10.3.2.3.2-1.

Table 7.10.3.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| u2uPerfId | string | Represents the identifier of an individual UE-to-UE session performance event subscription. |

###### 7.10.3.2.3.3	Resource Standard Methods

7.10.3.2.3.3.1	GET

This operation reads the individual unicast monitoring subscription resource. This method shall support the URI query parameters specified in table 7.10.3.2.3.3.1-1.

Table 7.10.3.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.10.3.2.3.3.1-2 and the response data structures and response codes specified in table 7.10.3.2.3.3.1-3.

Table 7.10.3.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.3.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| U2UPerfSub | M | 1 | 200 OK | The requested individual UE-to-UE session performance event subscription is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.3.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

Table 7.10.3.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

7.10.3.2.3.3.2	DELETE

This operation deletes the Individual UE-to-UE Session Performance Event Subscription resource. This method shall support the URI query parameters specified in table 7.10.3.2.3.3.2-1.

Table 7.10.3.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.3.2.3.3.2-2 and the response data structures and response codes specified in table 7.10.3.2.3.3.2-3.

Table 7.10.3.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.3.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual UE-to-UE session performance event subscription matching the u2uPerfId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.3.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.3.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

###### 7.10.3.2.3.4	Resource Custom Operations

None.

#### 7.10.3.3	Notifications

##### 7.10.3.3.1	General

Table 7.10.3.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| UE-to-UE session performance event notification | {notifUri} | POST | Notification on the UE-to-UE session performance analytics |

##### 7.10.3.3.2	UE-to-UE session performance event notification

###### 7.10.3.3.2.1	Description

UE-to-UE session performance event notification is to notify on the event of the UE-to-UE session performance analytics.

###### 7.10.3.3.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.3.3.2.2-1.

Table 7.10.3.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.3.3.2.2-2 and the response data structures and response codes specified in table 7.10.3.3.2.2-3.

Table 7.10.3.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| U2UPerfNotif | M | 1 | Notification information of the UE-to-UE session performance analytics. |

Table 7.10.3.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification for the UE-to-UE session performance analytics event is accepted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.3.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.3.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.3.4	Data Model

##### 7.10.3.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.3.4.1-1 specifies the data types defined specifically for the SS_ADAE_Ue2UePerformanceAnalytics API service.

Table 7.10.3.4.1-1: SS_ADAE_Ue2UePerformanceAnalytics API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| U2UAnalytics | 7.10.3.4.3.1 | Represents the UE-to-UE analytics types. |  |
| U2UAnalyticsData | 7.10.3.4.2.6 | Represents the UE-to-UE analytics data. |  |
| U2UPair | 7.10.3.4.2.7 | Represents the UE-to-UE pair. |  |
| U2UPerfNotif | 7.10.3.4.2.3 | Represents the UE-to-UE session performance analytics notification. |  |
| U2UPerfSub | 7.10.3.4.2.2 | Represents the UE-to-UE session performance analytics subscription. |  |
| U2UReportingGranularity | 7.10.3.4.3.2 | Represents the UE-to-UE reporting granularity. |  |
| U2UThreshold | 7.10.3.4.2.5 | Represents the threshold for UE-to-UE session performance analytics. |  |

Table 7.10.3.4.1-2 specifies data types re-used by the SS_ADAE_Ue2UePerformanceAnalytics API service:

Table 7.10.3.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.2.6 | Type of analytics for the event of the VAL application performance analytics. |  |
| DurationSec | 3GPP TS 29.122 [3] | Represents a period of time in units of seconds. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| MatchingDirection | 3GPP TS 29.520 [33] | Used to indicate a threshold matching direction. |  |
| NotificationMethod | 3GPP TS 29.508 [32] | Used to indicate the reporting mode. |  |
| PacketErrRate | 3GPP TS 29.571 [21] | Used to represent packet error rate. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable for SEAL: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the supported optional features of the API. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Used to indicate the time window. |  |
| Uinteger | 3GPP TS 29.571 [21] | Used to represent integer attributes. |  |
| Uri | 3GPP TS 29.122 [3] | Used to indicate the notification URI. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to indicate either VAL User ID or VAL UE ID. |  |

##### 7.10.3.4.2	Structured data types

###### 7.10.3.4.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.10.3.4.2.2	Type: U2UPerfSub

Table 7.10.3.4.2.2-1: Definition of type U2UPerfSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsType | AnalyticsType | M | 1 | Represents the type of the UE-to-UE session performance analytics. |  |
| valUeIds | array(ValTargetUe) | M | 1..N | Represent the list of VAL UEs, whose UE-to-UE session analytics are subscribed to. |  |
| valServiceId | string | O | 0..1 | Represents the VAL service for which the subscription applies. |  |
| confLevel | Uinteger | O | 0..1 | Indicates the preferred accuracy level for the UE-to-UE session prediction. Minimum = 0. Maximum = 100. |  |
| area | LocationArea5G | O | 0..1 | Represents the geographical or service area, to which the UE-to-UE session performance analytics subscription is applied. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the subscription. |  |
| reportingInds | array(U2UAnalytics) | O | 1..N | Indicates the list of the requested analytics. |  |
| reportingThrs | array(U2UThreshold) | C | 1..N | Identifies reporting threshold corresponding to the application QoS index(es). (NOTE) |  |
| reportingGrn | U2UReportingGranularity | O | 0..1 | Indicates the reporting granularity. |  |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| timeInterval | TimeWindow | O | 0..1 | The time interval as the start time and end time, to which the UE-to-UE session performance analytics subscription is applied. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |
| NOTE: This attribute may be provided if the "notifMethod" attribute within the ReportingInformation data type provided in the "repReq" attribute is set to "ON_EVENT_DETECTION". |  |  |  |  |  |

###### 7.10.3.4.2.3	Type: U2UPerfNotif

Table 7.10.3.4.2.3-1: Definition of type U2UPerfNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsOutput | array(U2UAnalyticsData) | M | 1..N | UE-to-UE session performance analytics for prediction or statistics depending on the type. |  |
| confLevel | Uinteger | C | 0..1 | Indicates the confidence of the prediction. This attribute shall be provided if the "analyticsType" attribute in the request is set to "PREDICTIVE". Minimum = 0. Maximum = 100. |  |

###### 7.10.3.4.2.4	Void

###### 7.10.3.4.2.5	Type: U2UThreshold

Table 7.10.3.4.2.5-1: Definition of type U2UThreshold

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| value | U2UAnalyticsData | M | 1 | Indicates the value for the analytics threshold. |  |
| thrDirect | MatchingDirection | M | 1 | Indicates the threshold matching direction for the analytics threshold provided in the "value" attribute. |  |

###### 7.10.3.4.2.6	Type: U2UAnalyticsData

Table 7.10.3.4.2.6-1: Definition of type U2UAnalyticsData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUes | U2UPair | O | 0..1 | Represent the pair of VAL UEs, whose UE-to-UE session analytics are observed. This attribute shall represent a sub-set of the VAL UEs defined in the "valUeIds" attribute within the U2UPerfSub data type. If omitted, the analytics data is applied for all active VAL UE sessions. |  |
| avgLatency | Uinteger | C | 0..1 | The average latency in milliseconds. (NOTE) |  |
| avgPer | PacketErrRate | C | 0..1 | The average packet error rate. (NOTE) |  |
| avgDataRate | BitRate | C | 0..1 | The average data rate. (NOTE) |  |
| jitter | Float | C | 0..1 | The jitter. (NOTE) |  |
| NOTE: At least one of these attributes shall be provided. |  |  |  |  |  |

###### 7.10.3.4.2.7	Type: U2UPair

Table 7.10.3.4.2.7-1: Definition of type U2UPair

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeA | ValTargetUe | M | 1 | Represent the first VAL UE in the pair. |  |
| valUeB | ValTargetUe | M | 1 | Represent the second VAL UE in the pair. |  |

##### 7.10.3.4.3	Simple data types and enumerations

###### 7.10.3.4.3.1	Enumeration: U2UAnalytics

Table 7.10.3.4.3.1-1: Enumeration U2UAnalytics

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| AVG_LATENCY | The indication for requesting the average latency analytics. |  |
| AVG_PER | The indication for requesting the average packet error rate analytics. |  |
| AVG_DATA_RATE | The indication for requesting the average data rate analytics. |  |
| JITTER | The indication for requesting the jitter analytics. |  |

###### 7.10.3.4.3.2	Enumeration: U2UReportingGranularity

Table 7.10.3.4.3.2-1: Enumeration U2UReportingGranularity

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| GROUP | The indication for requesting the analytics for all VAL UE-to-UE application sessions. |  |
| INDIVIDUAL | The indication for requesting the analytics for individual VAL UE-to-UE application sessions. |  |

#### 7.10.3.5	Error Handling

##### 7.10.3.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.10.3.5.2	Protocol Errors

In this release of the specification, there are no additional protocol errors applicable for the SS_ADAE_Ue2UePerformanceAnalytics API.

##### 7.10.3.5.3	Application Errors

The application errors defined for SS_ADAE_Ue2UePerformanceAnalytics API are listed in table 7.10.3.5.3-1.

Table 7.10.3.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.3.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8. Table 7.10.3.6-1 lists the supported features for SS_ADAE_Ue2UePerformanceAnalytics API.

Table 7.10.3.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 7.10.4	SS_ADAE_LocationAccuracyAnalytics API

#### 7.10.4.1	API URI

The SS_ADAE_LocationAccuracyAnalytics service shall use the SS_ADAE_LocationAccuracyAnalytics API.

The request URIs used in HTTP requests from the VAL server towards the ADAE server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-adae-laa".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.10.4.2.

#### 7.10.4.2	Resources

##### 7.10.4.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.4.2.1-1 depicts the resource URIs structure for the SS_ADAE_LocationAccuracyAnalytics API.

Figure 7.10.4.2.1-1: Resource URI structure of the SS_ADAE_LocationAccuracyAnalytics API

Table 7.10.4.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.4.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Location accuracy event subscription | /location-accuracy | POST | Subscription to the event of the location accuracy performance analytics. |
| Individual location accuracy event subscription | /location-accuracy/{locAccId} | GET | Request the retrieval of an existing "Individual location accuracy event subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual location accuracy event subscription" resource. |

##### 7.10.4.2.2	Resource: Location accuracy event subscription

###### 7.10.4.2.2.1	Description

Subscription to the event of the location accuracy analytics.

###### 7.10.4.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-laa/<apiVersion>/location-accuracy

This resource shall support the resource URI variables defined in the table 7.10.4.2.2.2-1.

Table 7.10.4.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.10.4.2.2.3	Resource Standard Methods

7.10.4.2.2.3.1	POST

This method to subscribe to the event of the location accuracy analytics and shall support the URI query parameters specified in table 7.10.4.2.2.3.1-1.

Table 7.10.4.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.4.2.2.3.1-2 and the response data structures and response codes specified in table 7.10.4.2.2.3.1-3.

Table 7.10.4.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocAccurSub | M | 1 | Subscription to the location accuracy analytics event. |

Table 7.10.4.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocAccurSub | M | 1 | 201 Created | Subscription to the location accuracy analytics is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.4.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-laa/<apiVersion>/location-accuracy/{locAccId} |

###### 7.10.4.2.2.4	Resource Custom Operations

None.

##### 7.10.4.2.3	Resource: Individual location accuracy event subscription

###### 7.10.4.2.3.1	Description

The Individual location accuracy event subscription resource represents an individual event subscription of a service consumer to the location accuracy analytics of the ADAE server.

###### 7.10.4.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-laa/<apiVersion>/location-accuracy/{locAccId}

This resource shall support the resource URI variables defined in the table 7.10.4.2.3.2-1.

Table 7.10.4.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| locAccId | string | Identifies an Individual Events Subscription |

###### 7.10.4.2.3.3	Resource Standard Methods

7.10.4.2.3.3.1	GET

This operation retrieves an individual location accuracy event subscription resource. This method shall support the URI query parameters specified in table 7.10.4.2.3.3.1-1.

Table 7.10.4.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.4.2.3.3.1-2 and the response data structures and response codes specified in table 7.10.4.2.3.3.1-3.

Table 7.10.4.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.4.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocAccurSub | M | 1 | 200 OK | The requested individual location accuracy event subscription is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.4.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.4.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

7.10.4.2.3.3.2	DELETE

This method shall support the URI query parameters specified in table 7.10.4.2.3.3.2-1.

Table 7.10.4.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.4.2.3.3.2-2 and the response data structures and response codes specified in table 7.10.4.2.3.3.2-3.

Table 7.10.4.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.4.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual subscription is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.4.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.4.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

###### 7.10.4.2.3.4	Resource Custom Operations

None.

#### 7.10.4.3	Notifications

##### 7.10.4.3.1	General

Table 7.10.4.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Location accuracy event notification | {notifUri} | POST | Notification on the location accuracy analytics |

##### 7.10.4.3.2	Location accuracy event notification

###### 7.10.4.3.2.1	Description

Location accuracy event notification is to notify on the event of the location accuracy analytics.

###### 7.10.4.3.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.4.3.2.2-1.

Table 7.10.4.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

If the notification is on the location accuracy analytics, this method shall support the request data structures specified in table 7.10.4.3.2.2-2 and the response data structures and response codes specified in table 7.10.4.3.2.2-3.

Table 7.10.4.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocAccurNotif | M | 1 | Notification information of the location accuracy analytics. |

Table 7.10.4.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification for the location accuracy analytics event is accepted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.4.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.4.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.4.4	Data Model

##### 7.10.4.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.4.4.1-1 specifies the data types defined specifically for the SS_ADAE_LocationAccuracyAnalytics API service.

Table 7.10.4.4.1-1: SS_ADAE_LocationAccuracyAnalytics API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| LocAccurAnalytics | 7.10.4.4.3.3 | Represents the analytics identifier for location accuracy analytics. | ADAE_Ext1 |
| LocAccurAnalyticsOutput | 7.10.4.4.2.4 | Represents the location accuracy analytics output. | ADAE_Ext1 |
| LocAccurChange | 7.10.4.4.3.4 | Rerpresents the type of location accuracy change. | ADAE_Ext1 |
| LocAccurPrediction | 7.10.4.4.2.5 | Represents the location accuracy prediction analytics output. | ADAE_Ext1 |
| LocAccurSustain | 7.10.4.4.2.6 | Represents the location accuracy sustainability analytics output. | ADAE_Ext1 |
| LocAccurNotif | 7.10.4.4.2.3 | Notification information of the location accuracy analytics event. |  |
| LocAccurSub | 7.10.4.4.2.2 | Subscription to the location accuracy analytics event. |  |

Table 7.10.4.4.1-2 specifies data types re-used by the SS_ADAE_LocationAccuracyAnalytics API service:

Table 7.10.4.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Accuracy | 3GPP TS 29.122 [3] | Represent the desired level of accuracy of the requested location information. |  |
| AnalyticsType | Clause 7.10.1.4.2.6 | Represents the type of analytics. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable for SEAL: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the supported features. |  |
| UeMobility | 3GPP TS 29.520 [33] | Represents UE Mobility and route information. |  |
| Uinteger | 3GPP TS 29.571 [21] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.122 [3] | Represents a URI. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Indicate either VAL User ID or VAL UE ID. |  |

##### 7.10.4.4.2	Structured data types

###### 7.10.4.4.2.1	Introduction

This clause defines the data structures to be used in resource representations of this service.

###### 7.10.4.4.2.2	Type: LocAccurSub

Table 7.10.4.4.2.2-1: Definition of type LocAccurSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| analyticsId | LocAccurAnalytics | C | 0..1 | Represents the analytics identifier. This attribute shall be present when "ADAE_Ext1" feature is supported. | ADAE_Ext1 |
| analyticsType | AnalyticsType | M | 1 | Identifies the type of the location accuracy analytics. |  |
| valUeIds | array(ValTargetUe) | M | 1..N | A list of identities of one or more VAL UEs whose location accuracy analytics are subscribed to. |  |
| accuracy | Accuracy | M | 1 | Represents the desired level of accuracy of the requested location information. |  |
| valServiceId | string | O | 0..1 | The identifier of the VAL service for which location accuracy analytics is requested. |  |
| confLevel | Uinteger | O | 0..1 | Indicates the preferred confidence level of the prediction. Minimum = 0. Maximum = 100. |  |
| area | LocationArea5G | O | 0..1 | The geographical or service area to which the location accuracy analytics subscription is applied. |  |
| timeValidity | TimeWindow | O | 0..1 | The time validity of the location accuracy analytics subscription. |  |
| ueMobs | array(UeMobility) | O | 1..N | Mobility and route information of the one or more target VAL UE. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirement of the location accuracy analytics subscription. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported features of the API as defined in clause 7.10.4.6. This attribute shall be provided in the HTTP POST response of successful subscription creation if it was provided in the request. |  |

###### 7.10.4.4.2.3	Type: LocAccurNotif

Table 7.10.4.4.2.3-1: Definition of type LocAccurNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsOutput | LocAccurAnalyticsOutput | C | 0..1 | Contains location accuracy analytics output. (NOTE) | ADAE_Ext1 |
| output | string | C | 0..1 | Location accuracy analytics output. (NOTE) |  |
| confLevel | Uinteger | C | 0..1 | Indicates the achieved confidence level in the case of prediction. This attribute shall be provided if the "analyticsType" attribute in the subscription is set to "PREDICTIVE". Minimum = 0. Maximum = 100. When the "ADAE_Ext1" feature is supported, this attribute shall be only present when the "output" attribute is present. |  |
| NOTE: When the "ADAE_Ext1" feature is supported, these attributes are mutually exclusive and only one of them shall be present. |  |  |  |  |  |

###### 7.10.4.4.2.4	Type: LocAccurAnalyticsOutput

Table 7.10.4.4.2.4-1: Definition of type LocAccurAnalyticsOutput

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsId | LocAccAnalytics | M | 1 | Represents the analytics identifier. |  |
| valUeIds | array(string) | O | 1..N | Contains a list of identities of one or more VAL UEs whose location accuracy analytics are subscribed to. |  |
| valServId | string | O | 0..1 | Represents the identifier of VAL service. |  |
| predictions | array(LocAccurPredictions) | C | 1..N | Represents the location accuracy prediction. This attribute shall be present only if the "analyticsId" is set to "LOCATION_ACCURACY_PREDICTION". |  |
| sustainAnalytics | LocAccurSustain | C | 0..1 | Represents the location accuracy prediction. This attribute shall be present only if the "analyticsId" is set to "LOCATION_ACCURACY_SUSTAINABILITY". |  |

###### 7.10.4.4.2.5	Type: LocAccurPrediction

Table 7.10.4.4.2.5-1: Definition of type LocAccurPrediction

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| accurChange | LocAccurChange | M | 1 | Contains the predicted or expected location accuracy change. |  |
| areas | array(LocationArea5G) | O | 0..1 | Contains the applicable areas. |  |
| timeWnd | TimeWindow | O | 0..1 | Contains the time period that the analytics applies to. |  |
| confLevel | integer | O | 0..1 | Contains the achieved confidence level in the case of prediction. Minimum = 0. Maximum = 100. |  |

###### 7.10.4.4.2.6	Type: LocAccurSustain

Table 7.10.4.4.2.6-1: Definition of type LocAccurSustain

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| areas | array(LocationArea5G) | O | 0..1 | Represents the applicable areas. |  |
| timeWnd | TimeWindow | O | 0..1 | Represents the time period that the analytics applies to. |  |
| thrs | FFS | O | 1..N | Represents the met reporting thresholds |  |
| confLevel | integer | O | 0..1 | Indicates the achieved confidence level in the case of prediction. Minimum = 0. Maximum = 100. |  |

Editor’s note:	The data type for the "thrs" attribute is FFS.

##### 7.10.4.4.3	Simple data types and enumerations

###### 7.10.4.4.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.4.4.3.2	Simple data types

The simple data types defined in table 7.10.4.4.3.2-1 shall be supported.

Table 7.10.4.4.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

###### 7.10.4.4.3.3	Enumeration: LocAccurAnalytics

The enumeration LocAccurAnalytics represents the analytic IDs for location accuracy analytics. It shall comply with the provisions defined in table 7.10.4.4.3.3-1.

Table 7.10.4.4.3.3-1: Enumeration LocAccurAnalytics

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LOCATION_ACCURACY_PREDICTION | Indicates that the requested analytics is location accuracy prediction. |  |
| LOCATION_ACCURACY_SUSTAINABILITY | Indicates that the requested analytics is location accuracy sustainability. |  |

###### 7.10.4.4.3.4	Enumeration: LocAccurChange

The enumeration LocAccurChange represents the change . It shall comply with the provisions defined in table 7.10.4.4.3.4-1.

Table 7.10.4.4.3.4-1: Enumeration LocAccurChange

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DOWNGRADE | Indicates the downgrade of the location accuracy. |  |
| UPGRADE | Indicates the upgrade of the location accuracy. |  |

#### 7.10.4.5	Error Handling

##### 7.10.4.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.10.4.5.2	Protocol Errors

In this release of the specification, there are no additional protocol errors applicable for the SS_ADAE_LocationAccuracyAnalytics API.

##### 7.10.4.5.3	Application Errors

The application errors defined for SS_ADAE_LocationAccuracyAnalytics API are listed in table 7.10.4.5.3-1.

Table 7.10.4.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.4.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8. Table 7.10.4.6-1 lists the supported features for SS_ADAE_LocationAccuracyAnalytics API.

Table 7.10.4.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | ADAE_Ext1 | Indicates the support of the ADAE enhancements in Release-19. Within this feature, the following enhancements are covered: - Support of the detailed analytics output. |

### 7.10.5	SS_ADAE_ServiceApiAnalytics API

#### 7.10.5.1	API URI

The SS_ADAE_ServiceApiAnalytics service shall use the SS_ADAE_ServiceApiAnalytics API.

The request URIs used in HTTP requests from the VAL server towards the ADAE server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-adae-sa".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.10.5.2.

#### 7.10.5.2	Resources

##### 7.10.5.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.5.2.1-1 depicts the resource URIs structure for the SS_ADAE_ServiceApiAnalytics API.

Figure 7.10.5.2.1-1: Resource URI structure of the SS_ADAE_ServiceApiAnalytics API

Table 7.10.5.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.5.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Service API event subscription | /service-api | POST | Subscription to the event of the service API analytics. |
| Individual service API event subscription | /service-api/{srvApiId} | GET | Request the retrieval of an existing "Individual service API event subscription" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual service API event subscription" resource. |

##### 7.10.5.2.2	Resource: Service API event subscription

###### 7.10.5.2.2.1	Description

Subscription to the event of the service API analytics.

###### 7.10.5.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-sa/<apiVersion>/service-api

This resource shall support the resource URI variables defined in the table 7.10.5.2.2.2-1.

Table 7.10.5.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.10.5.2.2.3	Resource Standard Methods

7.10.5.2.2.3.1	POST

This method to subscribe to the event of the service API analytics and shall support the URI query parameters specified in table 7.10.5.2.2.3.1-1.

Table 7.10.5.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.5.2.2.3.1-2 and the response data structures and response codes specified in table 7.10.5.2.2.3.1-3.

Table 7.10.5.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SrvApiSub | M | 1 | Subscription to the service API analytics event. |

Table 7.10.5.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SrvApiSub | M | 1 | 201 Created | Subscription to the service API analytics is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.5.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-sa/<apiVersion>/service-api/{srvApiId} |

###### 7.10.5.2.2.4	Resource Custom Operations

None.

##### 7.10.5.2.3	Resource: Individual Service API event subscription

###### 7.10.5.2.3.1	Description

The Individual Service API event subscription resource represents an individual event subscription of a service consumer to the Service API analytics of the ADAE server.

###### 7.10.5.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-laa/<apiVersion>/service-api/{srvApiId}

This resource shall support the resource URI variables defined in the table 7.10.5.2.3.2-1.

Table 7.10.5.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| srvApiId | string | Identifies an Individual Events Subscription |

###### 7.10.5.2.3.3	Resource Standard Methods

7.10.5.2.3.3.1	GET

This operation retrieves an individual service API event subscription resource. This method shall support the URI query parameters specified in table 7.10.5.2.3.3.1-1.

Table 7.10.5.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.5.2.3.3.1-2 and the response data structures and response codes specified in table 7.10.5.2.3.3.1-3.

Table 7.10.5.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.5.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SrvApiSub | M | 1 | 200 OK | The requested individual service API event subscription is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.5.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.5.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

7.10.5.2.3.3.2	DELETE

This method shall support the URI query parameters specified in table 7.10.5.2.3.3.2-1.

Table 7.10.5.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.5.2.3.3.2-2 and the response data structures and response codes specified in table 7.10.5.2.3.3.2-3.

Table 7.10.5.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.5.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual subscription is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.5.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.5.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

###### 7.10.5.2.3.4	Resource Custom Operations

None.

#### 7.10.5.3	Notifications

##### 7.10.5.3.1	General

Table 7.10.5.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Service API event notification | {notifUri} | POST | Notification on the service API analytics |

##### 7.10.5.3.2	Service API event notification

###### 7.10.5.3.2.1	Description

Service API event notification is to notify on the event of the service API analytics.

###### 7.10.5.3.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.5.3.2.2-1.

Table 7.10.5.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

If the notification is on the service API analytics, this method shall support the request data structures specified in table 7.10.5.3.2.2-2 and the response data structures and response codes specified in table 7.10.5.3.2.2-3.

Table 7.10.5.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SrvApiNotif | M | 1 | Notification information of the service API analytics. |

Table 7.10.5.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification for the service API analytics event is accepted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.5.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.5.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.5.4	Data Model

##### 7.10.5.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.5.4.1-1 specifies the data types defined specifically for the SS_ADAE_ServiceApiAnalytics API service.

Table 7.10.5.4.1-1: SS_ADAE_ServiceApiAnalytics API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SrvApiNotif | 7.10.5.4.2.3 | Notification information of the service API analytics event. |  |
| SrvApiSub | 7.10.5.4.2.2 | Subscription to the service API analytics event. |  |

Table 7.10.5.4.1-2 specifies data types re-used by the SS_ADAE_ServiceApiAnalytics API service:

Table 7.10.5.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.2.6 | Type of analytics for the event of the VAL application performance analytics. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable for SEAL: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the supported features. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Represents a time window. |  |
| Uinteger | 3GPP TS 29.571 [21] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.122 [3] | Represents a URI. |  |

##### 7.10.5.4.2	Structured data types

###### 7.10.5.4.2.1	Introduction

This clause defines the data structures to be used in resource representations of this service.

###### 7.10.5.4.2.2	Type: SrvApiSub

Table 7.10.5.4.2.2-1: Definition of type SrvApiSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| analyticsType | AnalyticsType | M | 1 | Identifies the type of the service API analytics. |  |
| eventCriteria | string | M | 1 | Criteria matching the service API analytics event. |  |
| serviceApiName | string | C | 0..1 | The Service API name. (NOTE) |  |
| serviceApiType | string | C | 0..1 | The Service API type. (NOTE) |  |
| area | LocationArea5G | O | 0..1 | The geographical or service area to which the service API analytics subscription is applied. |  |
| timeValidity | TimeWindow | O | 0..1 | The time validity of the service API analytics subscription. |  |
| timeHorizon | TimeWindow | O | 0..1 | Time horizon for the predictions. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirement of the service API analytics subscription. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported features of the API as defined in clause 7.10.5.6. This attribute shall be provided in the HTTP POST response of successful subscription creation if it was provided in the request. |  |
| NOTE: Only one of these attributes shall be provided. |  |  |  |  |  |

###### 7.10.5.4.2.3	Type: SrvApiNotif

Table 7.10.5.4.2.3-1: Definition of type SrvApiNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | The identifier of the requestor of these analytics. |  |
| output | string | M | 1 | Service API analytics for prediction or statistics depending on the type. (NOTE) |  |
| area | LocationArea5G | O | 0..1 | The geographical or service area to which the service API analytics subscription is applied. |  |
| confLevel | Uinteger | C | 0..1 | Indicates the achieved confidence level in the case of prediction. This attribute shall be provided if the "analyticsType" is set to "PREDICTIVE". Minimum = 0. Maximum = 100. |  |
| NOTE: The content of "output" attribute is not defined in this Release of the specification. |  |  |  |  |  |

##### 7.10.5.4.3	Simple data types and enumerations

###### 7.10.5.4.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.5.4.3.2	Simple data types

None.

#### 7.10.5.5	Error Handling

##### 7.10.5.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.10.5.5.2	Protocol Errors

In this release of the specification, there are no additional protocol errors applicable for the SS_ADAE_ServiceApiAnalytics API.

##### 7.10.5.5.3	Application Errors

The application errors defined for SS_ADAE_ServiceApiAnalytics API are listed in table 7.10.5.5.3-1.

Table 7.10.5.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.5.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8. Table 7.10.5.6-1 lists the supported features for SS_ADAE_ServiceApiAnalytics API.

Table 7.10.5.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 7.10.6	SS_ADAE_SliceUsagePatternAnalytics API

#### 7.10.6.1	API URI

The SS_ADAE_SliceUsagePatternAnalytics service shall use the SS_ADAE_SliceUsagePatternAnalytics API.

The request URIs used in HTTP requests from the service consumer towards the ADAE server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-adae-sup".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.10.6.2.

#### 7.10.6.2	Resources

##### 7.10.6.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.6.2.1-1 depicts the resource URIs structure for the SS_ADAE_SliceUsagePatternAnalytics API.

Figure 7.10.6.2.1-1: Resource URI structure of the SS_ADAE_SliceUsagePatternAnalytics API

Table 7.10.6.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.6.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Slice usage pattern event subscription | /slice-usage-pattern | POST | Subscriptions to the event of the slice usage pattern analytics |
| Individual slice usage pattern event subscription | /slice-usage-pattern/{subscriptionId} | GET | Request the retrieval of an existing "Individual slice usage pattern event subscription" resource identified by {subscriptionId}. |
|  |  | DELETE | Request the deletion of an existing "Individual slice usage pattern event subscription" resource identified by {subscriptionId}. |

##### 7.10.6.2.2	Resource: Slice usage pattern event subscriptions

###### 7.10.6.2.2.1	Description

Slice usage pattern event subscriptions to the event of the slice usage pattern analytics.

###### 7.10.6.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-sup/<apiVersion>/slice-usage-pattern

This resource shall support the resource URI variables defined in the table 7.10.6.2.2.2-1.

Table 7.10.6.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.10.6.2.2.3	Resource Standard Methods

7.10.6.2.2.3.1	POST

This method is used to subscribe to the event of the slice usage pattern analytics and shall support the URI query parameters specified in table 7.10.6.2.2.3.1-1.

Table 7.10.6.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.6.2.2.3.1-2 and the response data structures and response codes specified in table 7.10.6.2.2.3.1-3.

Table 7.10.6.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SUPSub | M | 1 | Subscription to the slice usage pattern analytics event. |

Table 7.10.6.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SUPSub | M | 1 | 201 Created | Subscription to the slice usage pattern analytics is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.6.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-sup/<apiVersion>/slice-usage-pattern/{subscriptionId} |

###### 7.10.6.2.2.4	Resource Custom Operations

None.

##### 7.10.6.2.3	Resource: Individual slice usage pattern event subscription

###### 7.10.6.2.3.1	Description

The Individual Slice usage pattern event subscription resource represents an individual event subscription of a service consumer to the slice usage pattern analytics of the ADAE server.

###### 7.10.6.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-sup/<apiVersion>/slice-usage-pattern/{subscriptionId}

This resource shall support the resource URI variables defined in the table 7.10.6.2.3.2-1.

Table 7.10.6.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| subscriptionId | string | Identifies an Individual Events Subscription |

###### 7.10.6.2.3.3	Resource Standard Methods

7.10.6.2.3.3.1	GET

This operation retrieves an individual slice usage pattern event subscription resource. This method shall support the URI query parameters specified in table 7.10.6.2.3.3.1-1.

Table 7.10.6.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.6.2.3.3.1-2 and the response data structures and response codes specified in table 7.10.6.2.3.3.1-3.

Table 7.10.6.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.6.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SUPSub | M | 1 | 200 OK | The requested individual slice usage pattern subscription is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.6.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.6.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

7.10.6.2.3.3.2	DELETE

This method shall support the URI query parameters specified in table 7.10.6.2.3.3.2-1.

Table 7.10.6.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.6.2.3.3.2-2 and the response data structures and response codes specified in table 7.10.6.2.3.3.2-3.

Table 7.10.6.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.6.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual subscription is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.6.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.6.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

###### 7.10.6.2.3.4	Resource Custom Operations

None.

#### 7.10.6.3	Notifications

##### 7.10.6.3.1	General

Table 7.10.6.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Slice usage pattern event notification | {notifUri} | POST | Notification on the slice usage pattern analytics |

##### 7.10.6.3.2	Slice usage pattern event notification

###### 7.10.6.3.2.1	Description

Slice usage pattern event notification is to notify on the event of the slice usage pattern analytics.

###### 7.10.6.3.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.6.3.2.2-1.

Table 7.10.6.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.6.3.2.2-2 and the response data structures and response codes specified in table 7.10.6.3.2.2-3.

Table 7.10.6.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| SUPNotif | M | 1 | Notification information of the slice usage pattern analytics |

Table 7.10.6.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification for the slice usage pattern analytics event is accepted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.6.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.6.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.6.4	Data Model

##### 7.10.6.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.6.4.1-1 specifies the data types defined specifically for the SS_ADAE_SliceUsagePatternAnalytics API service.

Table 7.10.6.4.1-1: SS_ADAE_SliceUsagePatternAnalytics API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SUPNotif | 7.10.6.4.2.3 | Notification information of the slice usage pattern analytics event. |  |
| SUPSub | 7.10.6.4.2.2 | Subscription to the slice usage pattern analytics event. |  |

Table 7.10.6.4.1-2 specifies data types re-used by the SS_ADAE_SliceUsagePatternAnalytics API service:

Table 7.10.6.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.2.6 | Represents the type of analytics. |  |
| Dnn | 3GPP TS 29.571 [21] | Identifies a DNN. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable for SEAL: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| Snssai | 3GPP TS 29.571 [21] | Identifies the S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Supported features. |  |
| TimeWindow | 3GPP TS 29.122 [3] | A time window. |  |
| Uinteger | 3GPP TS 29.571 [21] | Unsigned integer. |  |
| Uri | 3GPP TS 29.122 [3] | Represents a URI. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Indicate either VAL User ID or VAL UE ID. |  |

##### 7.10.6.4.2	Structured data types

###### 7.10.6.4.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.10.6.4.2.2	Type: SUPSub

Table 7.10.6.4.2.2-1: Definition of type SUPSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| analyticsType | AnalyticsType | M | 1 | Identifies the type of the slice usage pattern analytics. Only the "mode" attribute is applicable. |  |
| sliceId | Snssai | M | 1 | Identity of the network slice. |  |
| dnn | Dnn | O | 0..1 | Associated target DNN. |  |
| valUeIds | array(ValTargetUe) | O | 1..N | A list of identities of one or more VAL UEs, whose slice usage patterns are subscribed to. |  |
| valServerId | string | O | 0..1 | If the consumer is different from the VAL server, this identifier represents the VAL server, to which the slice usage pattern analytics subscription is applied. |  |
| confLevel | Uinteger | O | 0..1 | Indicates the preferred confidence level of the prediction. Minimum = 0. Maximum = 100. |  |
| area | LocationArea5G | O | 0..1 | The geographical or service area, to which the slice usage pattern analytics subscription is applied. |  |
| timeValidity | TimeWindow | O | 0..1 | The time validity of the subscription. |  |
| historicTimeInt | TimeWindow | C | 0..1 | The historic time interval as the start and the end time, to which the slice usage pattern analytics subscription is applied. This attribute shall be provided in case immediate reporting of statistics is requested. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirement of the slice usage pattern analytics subscription. |  |
| immNotif | SUPNotif | C | 0..1 | Immediate notifications. It shall only be provided in the subscription request, if immediate reporting is requested and the outputs are available. |  |
| dataId | string | C | 0..1 | Identity of the slice usage statistics data which is to be collected. It shall be provided if immediate reporting of statistics is requested and may not be provided otherwise. |  |
| valServiceId | string | C | 0..1 | The identifier of the VAL service, for which the request applies. It shall be provided if immediate reporting of statistics is requested and may not be provided otherwise. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported features of the API as defined in clause 7.10.6.6. This attribute shall be provided in the HTTP POST response of successful subscription creation if it was provided in the request. |  |

###### 7.10.6.4.2.3	Type: SUPNotif

Table 7.10.6.4.2.3-1: Definition of type SUPNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| output | string | M | 1 | Slice usage pattern analytics. The format is up to the implementation. |  |
| confLevel | Uinteger | C | 0..1 | Indicates the achieved confidence level in the case of prediction. This attribute shall be provided if the "analyticsType" attribute in the subscription is set to "PREDICTIVE". Minimum = 0. Maximum = 100. |  |

###### 7.10.6.4.2.4	Void

###### 7.10.6.4.2.5	Void

##### 7.10.6.4.3	Simple data types and enumerations

###### 7.10.6.4.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.6.4.3.2	Simple data types

None.

###### 7.10.6.4.3.3	Void

#### 7.10.6.5	Error Handling

##### 7.10.6.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.10.6.5.2	Protocol Errors

In this release of the specification, there are no additional protocol errors applicable for the SS_ADAE_SliceUsagePatternAnalytics API.

##### 7.10.6.5.3	Application Errors

The application errors defined for SS_ADAE_SliceUsagePatternAnalytics API are listed in table 7.10.6.5.3-1.

Table 7.10.6.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.6.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8. Table 7.10.6.6-1 lists the supported features for SS_ADAE_SliceUsagePatternAnalytics API.

Table 7.10.6.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 7.10.7	SS_ADAE_EdgeLoadAnalytics API

#### 7.10.7.1	API URI

The SS_ADAE_EdgeLoadAnalytics service shall use the SS_ADAE_EdgeLoadAnalytics API.

The request URIs used in HTTP requests from the VAL server towards the ADAE server shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-adae-el".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.10.7.2.

#### 7.10.7.2	Resources

##### 7.10.7.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.7.2.1-1 depicts the resource URIs structure for the SS_ADAE_EdgeLoadAnalytics API.

Figure 7.10.7.2.1-1: Resource URI structure of the SS_ADAE_EdgeLoadAnalytics API

Table 7.10.7.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.7.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Edge Load Event Subscription | /edge-load | POST | Creates a new individual Event Subscription for edge load analytics. |
| Individual Edge Load Event Subscription | /edge-load/{edgeLdId} | GET | Retrieve an individual edge load analytics according to query parameter on the resource identified by {edgeLdId}. If there are no query parameter, fetch the whole edge load resource identified by {edgeLdId}. |
|  |  | DELETE | Deletes an individual Event Subscription identified by the {edgeLdId}. |

##### 7.10.7.2.2	Resource: Edge Load Event Subscription

###### 7.10.7.2.2.1	Description

The edge load event subscription to the event of the edge load analytics.

###### 7.10.7.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-el/<apiVersion>/edge-load

This resource shall support the resource URI variables defined in the table 7.10.7.2.2.2-1.

Table 7.10.7.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |

###### 7.10.7.2.2.3	Resource Standard Methods

7.10.7.2.2.3.1	POST

This method to subscribe to the event of the edge load analytics and shall support the URI query parameters specified in table 7.10.7.2.2.3.1-1.

Table 7.10.7.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.7.2.2.3.1-2 and the response data structures and response codes specified in table 7.10.7.2.2.3.1-3.

Table 7.10.7.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EdgeSub | M | 1 | Create a new individual Event Subscription to the event of edge load analytics. |

Table 7.10.7.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EdgeSub | M | 1 | 201 Created | Edge load Event Subscription resource created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.7.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-el/<apiVersion>/edge-load/{edgeLdId} |

###### 7.10.7.2.2.4	Resource Custom Operations

None.

##### 7.10.7.2.3	Resource: Individual Edge Load Event Subscription

###### 7.10.7.2.3.1	Description

The Individual Edge Load Event Subscription resource represents an individual event subscription of the edge load analytics.

###### 7.10.7.2.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-el/<apiVersion>/edge-load/{edgeLdId}

This resource shall support the resource URI variables defined in the table 7.10.7.2.3.2-1.

Table 7.10.7.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| edgeLdId | string | Identifies an Individual Edge Load Events Subscription. |

###### 7.10.7.2.3.3	Resource Standard Methods

7.10.7.2.3.3.1	GET

This operation reads the individual edge load subscription resource. This method shall support the URI query parameters specified in table 7.10.7.2.3.3.1-1.

Table 7.10.7.2.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.10.7.2.3.3.1-2 and the response data structures and response codes specified in table 7.10.7.2.3.3.1-3.

Table 7.10.7.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.7.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EdgeSub | M | 1 | 200 OK | The requested individual edge load subscription is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.7.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

Table 7.10.7.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

7.10.7.2.3.3.2	DELETE

This method shall support the URI query parameters specified in table 7.10.7.2.3.3.2-1.

Table 7.10.7.2.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.7.2.3.3.2-2 and the response data structures and response codes specified in table 7.10.7.2.3.3.2-3.

Table 7.10.7.2.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.7.2.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual Edge Load Events Subscription matching the edgeLdId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.7.2.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

Table 7.10.7.2.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

###### 7.10.7.2.3.4	Resource Custom Operations

None.

#### 7.10.7.3	Notifications

##### 7.10.7.3.1	General

Table 7.10.7.3.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Edge load event notification | {notifUri} | POST | Notification on the event of the edge load analytics or the edge load analytics data. |

##### 7.10.7.3.2	Edge load event notification

###### 7.10.7.3.2.1	Description

Edge load event notification is to notify on the event of the edge load analytics or the edge load analytics data.

###### 7.10.7.3.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.7.3.2.2-1.

Table 7.10.7.3.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

If the notification is on the edge load analytics and the edge load analytics data, this method shall support the request data structures specified in table 7.10.7.3.2.2-2 and the response data structures and response codes specified in table 7.10.7.3.2.2-3.

Table 7.10.7.3.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EdgeNotif | M | 1 | Notification information on edge load analytics or edge load analytics data. |

Table 7.10.7.3.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.7.3.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.7.3.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.7.4	Data Model

##### 7.10.7.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.7.4.1-1 specifies the data types defined specifically for the SS_ADAE_EdgeLoadAnalytics API service.

Table 7.10.7.4.1-1: SS_ADAE_EdgeLoadAnalytics API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AnalyticsIdEl | 7.10.7.4.3.3 | Represents the analytics identifier for the edge load analytics. | ADAE_Ext1 |
| AnalyticsOutputEl | 7.10.7.4.2.7 | Represents the edge load analytics output. | ADAE_Ext1 |
| EasAnalytics | 7.10.7.4.2.10 | Represents the EAS analytics data. | ADAE_Ext1 |
| EesAnalytics | 7.10.7.4.2.9 | Represents the EES analytics data. | ADAE_Ext1 |
| EdgePlatformAnalytics | 7.10.7.4.2.8 | Represents the edge platform analytics data. | ADAE_Ext1 |
| EdgeNotif | 7.10.7.4.2.3 | Notification information of the edge load analytics event. |  |
| EdgeSub | 7.10.7.4.2.2 | Represents the subscription to the edge load event. |  |

Table 7.10.7.4.1-2 specifies data types re-used by the SS_ADAE_EdgeLoadAnalytics API service:

Table 7.10.7.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.2.6 | Type of analytics for the event of the edge load analytics. |  |
| Dnai | 3GPP TS 29.571 [21] | Identifies a user plane access to one or more DN(s). |  |
| Dnn | 3GPP TS 29.571 [21] | Identifies a DNN. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| NotificationMethod | 3GPP TS 29.508 [32] | Used to indicate the reporting mode. |  |
| ProdProfileInfo | Clause 7.10.1.4.2.4 | Information about the data producer's support data collection and its access to the produced data. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable for SEAL: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the supported optional features of the API. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Represents a period of time with start time and end time. |  |
| Uinteger | 3GPP TS 29.571 [21] | Used to represent integer attributes. |  |
| Uri | 3GPP TS 29.122 [3] | Used to indicate the notification URI. |  |

##### 7.10.7.4.2	Structured data types

###### 7.10.7.4.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.10.7.4.2.2	Type: EdgeSub

Table 7.10.7.4.2.2-1: Definition of type EdgeSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsId | AnalyticsIdEl | C | 0..1 | Represents the analytics identifier. This attribute shall be present when the "ADAE_Ext1" feature is supported. | ADAE_Ext1 |
| analyticsType | AnalyticsType | M | 1 | Represents the anaytics type of the edge load analytics. |  |
| destinationEasInfo | string | O | 0..1 | Represents the information for the destination EAS, including destination EAS ID and detination EAS address. (NOTE 1) |  |
| destinationEesId | string | O | 0..1 | The identifier of the destination EES. (NOTE 1) |  |
| dnai | Dnai | O | 0..1 | Represents the Data Network Access Identifier of user plane access to DN(s) which the subscription applies. (NOTE 1) |  |
| dnn | Dnn | O | 0..1 | Represents the target DNN which the subscription applies. (NOTE 1) |  |
| dataProdProfile | ProdProfileInfo | O | 0..1 | Represents the characteristics of the data producer to be used. |  |
| confLevel | Uinteger | O | 0..1 | Represents the accuracy level for the edge load analytics in case of prediction. Minimum = 0. Maximum = 100. |  |
| area | LocationArea5G | O | 0..1 | Represents the geographical or service area, to which the edge load analytics subscription is applied. |  |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| report | EdgeNotif | C | 0..1 | Indicates the reporting data for one-time immediate report. It shall be provided if one-time immediate reporting is requested by indicated in the "reportReq” attribute, and the outputs are available. |  |
| reportReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the subscription. (NOTE 2) |  |
| timeInterval | TimeWindow | O | 0..1 | The time interval as the start and the end time, to which the edge load analytics subscription applies. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |
| NOTE 1: At least one of the attributes, "destinationEasInfo”, "destinationEesId", and either "dnai" or "dnn", shall be present. |  |  |  |  |  |
| NOTE 2: If absent, the applied reporting parameters are based on local configuration. |  |  |  |  |  |

###### 7.10.7.4.2.3	Type: EdgeNotif

Table 7.10.7.4.2.3-1: Definition of type EdgeNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| output | AnalyticsOutputEl | C | 0..1 | Represents the analytics output. (NOTE) | ADAE_Ext1 |
| analyticsOutput | string | C | 0..1 | Represents the edge load analytics. (NOTE) |  |
| confLevel | Uinteger | C | 0..1 | Represents the accuracy level of the edge load analytics in case of prediction. Minimum = 0. Maximum = 100. This attribute shall be provided if the analyticsType in the subscription request is set to "PREDICTIVE". When the "ADAE_Ext1" feature is supported, this attribute shall be only present when the "analyticsOutput" attribute is present. |  |
| NOTE: When the "ADAE_Ext1" feature is supported, these attributes are mutually exclusive and only one of them shall be present. |  |  |  |  |  |

###### 7.10.7.4.2.4	Void

###### 7.10.7.4.2.5	Void

###### 7.10.7.4.2.6	Void

###### 7.10.7.4.2.7	Type: AnalyticsOutputEl

Table 7.10.7.4.2.7-1: Definition of type AnalyticsOutputEl

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsId | AnalyticsIdEl | M | 1 | Represents the analytics identifier. |  |
| platAnalytics | EdgePlatformAnalytics | C | 0..1 | Represents the edge platform analytics output. This attribute shall be present only if the "analyticsId" is set to "EDGE_PLATFORM". |  |
| eesAnalytics | EesAnalytics | C | 0..1 | Represents the EES analytics output. This attribute shall be present only if the "analyticsId" is set to "EES". |  |
| easAnalytics | EasAnalytics | C | 0..1 | Represents the EAS analytics output. This attribute shall be present only if the "analyticsId" is set to "EAS". |  |

###### 7.10.7.4.2.8	Type: EdgePlatformAnalytics

Table 7.10.7.4.2.8-1: Definition of type EdgePlatformAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easConn | Uinteger | C | 0..1 | Represents the number of EAS connections. (NOTE) |  |
| eesConn | Uinteger | C | 0..1 | Represents the number of EES connections. (NOTE) |  |
| confLevel | integer | C | 0..1 | Represents the accuracy level in case of prediction. Minimum = 0. Maximum = 100. This attribute shall be present if the analyticsType in the subscription request is set to "PREDICTIVE". |  |
| timeWnd | TimeWindow | O | 0..1 | Represents the time period that the analytics applies to. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 7.10.7.4.2.9	Type: EesAnalytics

Table 7.10.7.4.2.9-1: Definition of type EesAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eesUnavail | array(ScheduledCommunicationTime) | C | 1..N | Represents the EES unavailability information. (NOTE) |  |
| eesUnavailPer | integer | C | 0..1 | Represents the percentage of EES unavailability. Minimum = 0. Maximum = 100. (NOTE) |  |
| confLevel | integer | C | 0..1 | Represents the accuracy level in case of prediction. Minimum = 0. Maximum = 100. This attribute shall be present if the analyticsType in the subscription request is set to "PREDICTIVE". |  |
| timeWnd | TimeWindow | O | 0..1 | Represents the time period that the analytics applies to. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 7.10.7.4.2.10	Type: EasAnalytics

Table 7.10.7.4.2.10-1: Definition of type EasAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easUnavail | array(ScheduledCommunicationTime) | C | 1..N | Represents the EAS unavailability information. (NOTE) |  |
| easUnavailPer | integer | C | 0..1 | Represents the percentage of EAS unavailability. Minimum = 0. Maximum = 100. (NOTE) |  |
| confLevel | integer | C | 0..1 | Represents the accuracy level in case of prediction. Minimum = 0. Maximum = 100. This attribute shall be present if the analyticsType in the subscription request is set to "PREDICTIVE". |  |
| timeWnd | TimeWindow | O | 0..1 | Represents the time period that the analytics applies to. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

##### 7.10.7.4.3	Simple data types and enumerations

###### 7.10.7.4.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.7.4.3.2	Simple data types

None.

###### 7.10.7.4.3.3	Enumeration: AnalyticsIdEl

Table 7.10.7.4.3.3-1: Enumeration AnalyticsIdEl

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| EDGE_PLATFORM | Indicates that the analytics identifier is edge platform analytics. |  |
| EES | Indicates that the analytics identifier is EES analytics. |  |
| EAS | Indicates that the analytics identifier is EAS analytics. |  |

#### 7.10.7.5	Error Handling

##### 7.10.7.5.1	General

HTTP error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply.

##### 7.10.7.5.2	Protocol Errors

In this release of the specification, there are no additional protocol errors applicable for the SS_ADAE_EdgeLoadAnalytics API.

##### 7.10.7.5.3	Application Errors

The application errors defined for SS_ADAE_EdgeLoadAnalytics API are listed in table 7.10.7.5.3-1.

Table 7.10.7.5.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.7.6	Feature Negotiation

General feature negotiation procedures are defined in clause 6.8. Table 7.10.7.6-1 lists the supported features for SS_ADAE_EdgeLoadPatternAnalytics API.

Table 7.10.7.6-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 7.10.8	SS_AADRF_DataManagement API

#### 7.10.8.1	API URI

The SS_AADRF_DataManagement service shall use the SS_AADRF_DataManagement API.

The request URIs used in HTTP requests from the ADAE server towards the A-ADRF shall have the Resource URI structure as defined in clause 6.5 with the following clarifications:

-	The <apiName> shall be "ss-aadrf-datamanagement".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 7.10.8.2

#### 7.10.8.2	Resources

##### 7.10.8.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.8.2.1-1 depicts the resource URIs structure for the SS_AADRF_DataManagement API.

Figure 7.10.8.2.1-1: Resource URI structure of the SS_AADRF_DataManagement API

Table 7.10.8.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.8.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| A-ADRF Data Management Subscriptions | /subscriptions | POST | Creates a new Individual A-ADRF Data Management Subscription resource. |
| Individual A-ADRF Data Management Subscription | /subscriptions/{subscriptionId} | DELETE | Deletes an Individual A-ADRF Data Management Subscription identified by {subscriptionId}. |

##### 7.10.8.2.2	Resource: A-ADRF Data Management Subscriptions

###### 7.10.8.2.2.1	Description

The A-ADRF Data Management Subscriptions resource represents all the subscriptions stored at A-ADRF. The resource allows an NF service consumer to create a new Individual A-ADRF Data Management Subscription resource.

###### 7.10.8.2.2.2	Resource Definition

Resource URI: {apiRoot}/ss-aadrf-datamanagement/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 7.10.8.2.2.2-1.

Table 7.10.8.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |

###### 7.10.8.2.2.3	Resource Standard Methods

7.10.8.2.2.3.1	POST

This method shall support the URI query parameters specified in table 7.10.8.2.2.3.1-1.

Table 7.10.8.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.8.2.2.3.1-2 and the response data structures and response codes specified in table 7.10.8.2.2.3.1-3.

Table 7.10.8.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataManageSub | M | 1 | Create a new Individual A-ADRF Data Management Subscription resource. |

Table 7.10.8.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataManageSub | M | 1 | 201 Created | The creation of an Individual A-ADRF Data Management Subscription resource is confirmed and a representation of that resource is returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.8.2.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-aadrf-datamanagement/<apiVersion>/subscriptions/{subscriptionId} |

###### 7.10.8.2.2.4	Resource Custom Operations

None in this release of the specification.

##### 7.10.8.2.3	Resource: Individual A-ADRF Data Management Subscription

###### 7.10.8.2.3.1	Description

The Individual A-ADRF Data Management Subscription resource represents a single subscription to the SS_AADRF_DataManagement Service at a given A-ADRF.

###### 7.10.8.2.3.2	Resource definition

Resource URI: {apiRoot}/ss-aadrf-datamanagement/<apiVersion>/subscriptions/{subscriptionId}

The <apiVersion> shall be set as described in clause 7.10.8.2.3.2-1.

This resource shall support the resource URI variables defined in table 7.10.8.2.3.2-1.

Table 7.10.8.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| subscriptionId | string | Identifies a subscription to the SS_AADRF_DataManagement Service. |

###### 7.10.8.2.3.3	Resource Standard Methods

7.10.8.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 7.10.8.2.3.3.1-1.

Table 7.10.8.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.8.2.3.3.1-2 and the response data structures and response codes specified in table 7.10.8.2.3.3.1-3.

Table 7.10.8.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.8.2.3.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Individual A-ADRF Data Management Subscription resource matching the subscriptionId was deleted. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative A-ADRF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative A-ADRF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.8.2.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative A-ADRF. |

Table 7.10.8.2.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server-ADRF. |

###### 7.10.8.2.3.4	Resource Custom Operations

None in this release of the specification.

#### 7.10.8.3	Custom Operations without associated resources

##### 7.10.8.3.1	Overview

The structure of the custom operation URIs of the SS_AADRF_DataManagement API is shown in Figure 7.10.8.3.1-1.

Figure 7.10.8.3.1-1: Custom operation URI structure of the SDD_Transmission API

Table 7.10.8.3.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the SS_AADRF_DataManagement API.

Table 7.10.8.3.1-1: Custom operations without associated resources

| Custom operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Store | /store | POST | Enables a service consumer to request data storage. |

The custom operations shall support the URI variables defined in table 7.10.8.3.1-2.

Table 7.10.8.3.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.10.8.1. |

##### 7.10.8.3.2	Operation: Store

###### 7.10.8.3.2.1	Description

The custom operation enables a service consumer to request the data storage at A-ADRF.

###### 7.10.8.3.2.2	Operation Definition

This operation shall support the request data structures specified in table 7.10.8.3.2.2-1 and the response data structures and response codes specified in table 7.10.8.3.2.2-2.

Table 7.10.8.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataStoreReq | M | 1 | Contains the parameters to request data storage. |

Table 7.10.8.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataStoreResp | 1 | M | 200 OK | Successful case. The data storage request is successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative A-ADRF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative A-ADRF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2] |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.8.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative A-ADRF. |

Table 7.10.8.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative A-ADRF. |

#### 7.10.8.4	Notifications

##### 7.10.8.4.1	General

Table 7.10.8.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Event Notification | {notifUri} | POST | Report the subscribed data. |

##### 7.10.8.4.2	Event Notification

###### 7.10.8.4.2.1	Description

The Event Notification is used by the A-ADRF to report the subscribed data to an NF service consumer that has subscribed to such Notifications.

###### 7.10.8.4.2.2	Notification definition

Callback URI: {notifUri}

The operation shall support the callback URI variables defined in Table 7.10.8.4.2.2-1, the request data structures specified in table 7.10.8.4.2.2-2 and the response data structure and response codes specified in Table 7.10.8.4.2.2-3.

Table 7.10.8.4.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | The Notification Uri is assigned within the Individual A-ADRF Data Management Subscription Resource and described within the DataManageSub type. |

Table 7.10.8.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataManageNotification | M | 1 | Provides information about the requested data. |

Table 7.10.8.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |  |

Table 7.10.8.4.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative notification destination towards which the notification is redirected. |

Table 7.10.8.4.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative notification destination instance towards which the notification is redirected. |

#### 7.10.8.5	Data Model

##### 7.10.8.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API

Table 7.10.8.5.1-1 specifies the data types defined specifically for the SS_AADRF_DataManagement API service.

Table 7.10.8.5.1-1: SS_AADRF_DataManagement API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AnalyticsMode | 7.10.8.5.2.16 | Represents the analytics mode. |  |
| AadrfEvent | 7.10.8.5.3.1 | Indicates the subscribed events. |  |
| ApiLogInfo | 7.10.8.5.2.7 | Represents the API log data. |  |
| ApiLogReq | 7.10.8.5.2.5 | Represents the API log request requirement. |  |
| Data | 7.10.8.5.2.11 | Represent the data for storage at A-ADRF. |  |
| DataManageNotification | 7.10.8.5.2.6 | Represents the notification to the consumer. |  |
| DataManageSub | 7.10.8.5.2.2 | Represents the events subscription. |  |
| DataProducerProfile | 7.10.8.5.2.12 | Represents the data producer profile. |  |
| DataSourceInfo | 7.10.8.5.2.13 | Represents the data source information. |  |
| DataStoreReq | 7.10.8.5.2.9 | Represents the data storage request. |  |
| DataStoreResp | 7.10.8.5.2.15 | Represents the data storage response. |  |
| DataStoreSub | 7.10.8.5.2.10 | Represents the requirements for subscription based data storage. |  |
| DataType | 7.10.8.5.3.3 | Represents the type of data. |  |
| EdgeInfo | 7.10.8.5.2.8 | Represents the EDGE related data. |  |
| EdgeReq | 7.10.8.5.2.4 | Represents the EDGE data request requirement. |  |
| EventSubscription | 7.10.8.5.2.3 | Represents the event subscription. |  |
| ExposureLevel | 7.10.8.5.3.2 | Indicates the exposure level. |  |
| Mode | 7.10.8.5.3.4 | Represents the supported modes of analytics. |  |
| S2SAnalyticsInfo | 7.10.8.5.2.18 | Represents the Server to Server Analytics related data. | EdgeApp_3 |
| S2SAnalyticsReq | 7.10.8.5.2.17 | Represents the Server to Server Analytics request requirements. | EdgeApp_3 |
| StorageHandlingInfo | 7.10.8.5.2.14 | Represent the storage handing information. |  |
| UERatAnalyticsInfo | 7.10.8.5.2.20 | Represents UE RAT Connectivity Analytics data. | 5GSatApp_1 |
| UERatAnalyticsReq | 7.10.8.5.2.19 | Represents UE RAT Connectivity Analytics requirements. | 5GSatApp_1 |

Table 7.10.8.5.1-2 specifies data types re-used by the SS_AADRF_DataManagement API service.

Table 7.10.8.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DataTime | 3GPP TS 29.122 [3] | Represents data and time. |  |
| Dnai | 3GPP TS 29.571 [21] | Identifies a user plane access to one or more DN(s). |  |
| Dnn | 3GPP TS 29.571 [21] | Used to Identify a DNN. |  |
| DurationSec | 3GPP TS 29.122 [3] | Represents duration in seconds. |  |
| Float | 3GPP TS 29.571 [21] | Used to represent the fractional part of the proximity range in the reference UE details. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents the location information. |  |
| S2SPerfAnalyticsData | Clause 7.10.14.6.2.5 | Contains the Server to Server Analytics data. | EdgeApp_3 |
| ServerInfo | Clause 7.10.14.6.2.6 | Represents the Server Information. | EdgeApp_3 |
| Snssai | 3GPP TS 29.571 [21] | Used to Identify the S-NSSAI. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the applicability of optional features. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Indicates the time window. | EdgeApp_3 |
| UeRatAnalyticsOutput | Clause 7.10.12.6.2.8 | Contains the UE RAT connectivity analytics output. |  |
| Uinteger | 3GPP TS 29.571 [21] | Used to represent integer attributes within MeasurementData and ReportingRequirements data structures. |  |
| Uri | 3GPP TS 29.571 [21] | Used to indicate the notification URI. |  |
| ValTargetUe | Clause 7.3.1.4.2.3 | Used to indicate either VAL User ID or VAL UE ID. |  |
| ValidityConditions | Clause 7.5.1.4.2.13 | Represents the temporal and/or spatial conditions applied for the events to be monitored. |  |
| ValUeAddrInfo | Clause 7.4.1.4.2.30 | Represents VAL UE address information. |  |

##### 7.10.8.5.2	Structured data types

###### 7.10.8.5.2.1	Introduction

###### 7.10.8.5.2.2	Type: DataManageSub

Table 7.10.8.5.2.2-1: Definition of type DataManageSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubscriptions | array(EventSubscription) | M | 1..N | Subscribed events. |  |
| notifUri | Uri | M | 1 | Identifies the recipient of Notifications sent by the A-ADRF. |  |
| notifCorrId | string | O | 0..1 | Notification correlation identifier. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | List of Supported features used as described in clause 7.10.8.7. This parameter shall be supplied by NF service consumer in the POST request that request the creation of an A-ADRF Event Subscriptions resource and shall be supplied by the A-ADRF in the reply of corresponding request. |  |

###### 7.10.8.5.2.3	Type: EventSubscription

Table 7.10.8.5.2.3-1: Definition of type EventSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AadrfEvent | M | 1 | The event that is subscribed. |  |
| dataCollectReq | string | O | 0..1 | The requirements for data collection. (NOTE) |  |
| dataProducerIds | array(string) | O | 1..N | The list of Data Producer IDs. |  |
| valUes | array(ValTargetUe) | O | 1..N | The target VAL UE(s) identifiers. |  |
| valServerId | string | O | 0..1 | Identifies the target VAL server for which the data collection subscription applies. |  |
| valServiceId | string | O | 0..1 | The VAL service ID of the VAL application. |  |
| profileCriteria | string | O | 0..1 | The characteristics of the data producers to be used. (NOTE) |  |
| validConds | ValidityConditions | O | 0..1 | Represents the temporal and/or spatial conditions applied for the request. |  |
| edgeReq | EdgeReq | O | 0..1 | The EDGE data collection requirement. This attribute may be present if the subscribed event is "EDGE_DATA". |  |
| snssais | array(Snssai) | C | 1..N | Identification(s) of network slice(s) to which the subscription applies. This attribute shall be present if the subscribed event is "NETWORK_SLICE". |  |
| apiLogReq | ApiLogReq | O | 0..1 | The historical service API logs requirement. This attribute may be present if the subscribed event is "HISTORICAL_SERVICEAPI". |  |
| s2sAnaReq | S2SAnalyticsReq | O | 0..1 | Contains the Server to Server Analytics request requirement. This attribute may be present if the subscribed event is "SERVER_TO_SERVER_ANALYTICS". | EdgeApp_3 |
| ueRatAnaReq | UERatAnalyticsReq | O | 0..1 | Contains the UE RAT Connectivity analytics requirements. This attribute may be present if the subscribed event is "UE_RAT_ANALYTICS". | 5GSatApp_1 |
| NOTE: The format of this attribute is not specified in this release of the specification and is up to implementation. |  |  |  |  |  |

###### 7.10.8.5.2.4	Type: EdgeReq

Table 7.10.8.5.2.4-1: Definition of type EdgeReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnais | array(Dnai) | C | 1..N | Identifiers the DN Access Identifier representing location of the service flow. (NOTE) |  |
| destEasId | string | C | 0..1 | Identifier for the destination EAS, for which the edge load analytics subscription or the edge load data collection subscription, is requested. (NOTE) |  |
| destEesId | string | C | 0..1 | Identifier for the destination EES, for which the edge load analytics subscription or the edge load data collection subscription, is requested. (NOTE) |  |
| dnns | array(Dnn) | C | 1..N | Identifiers of the DNNs. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |  |

###### 7.10.8.5.2.5	Type: ApiLogReq

Table 7.10.8.5.2.5-1: Definition of type ApiLogReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiRequestorInfo | string | M | 1 | Identifier of the originated application querying service API log request. |  |
| apiId | string | O | 0..1 | String identifying the API invoked. |  |
| apiName | string | O | 0..1 | Name of the API which was invoked, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [3]. |  |
| apiVersion | string | O | 0..1 | Version of the API which was invoked. |  |
| inputParameters | string | O | 0..1 | List of input parameters. (NOTE) |  |
| result | string | O | 0..1 | For HTTP protocol, it contains HTTP status code of the invocation. |  |
| apiInvokerId | string | O | 0..1 | Identity of the API invoker which invoked the service API. |  |
| exposureLvl | ExposureLevel | O | 0..1 | The level of exposure requirement for the logs to be exposed. |  |
| apiAggreInfo | string | O | 0..1 | The aggregation or abstraction/filtering requirement needs to be applied. (NOTE) |  |
| NOTE: The content of "apiAggreInfo" and "inputParameters" attributes is not defined in this Release of the specification. |  |  |  |  |  |

###### 7.10.8.5.2.6	Type: DataManageNotification

Table 7.10.1.4.2.6-1: Definition of type DataManageNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | AadrfEvent | M | 1 | The event that is subscribed. |  |
| notifCorrId | string | O | 0..1 | Notification correlation identifier. |  |
| valServerId | string | C | 0..1 | Identifies the target VAL server for which the data collection subscription applies. This attribute shall be present if the subscribed event is "HISTORICAL_SERVICEAPI". |  |
| valServiceId | string | O | 0..1 | The VAL service ID of the VAL application. |  |
| valUeIds | array(ValTargetUe) | O | 1..N | The target VAL UE(s) identifiers. |  |
| locAccuracy | string | O | 0..1 | The requested location analytics/data. (NOTE) |  |
| validConds | ValidityConditions | O | 0..1 | Represents the temporal and/or spatial conditions applied for the request. |  |
| apiLogs | array(ApiLogInfo) | O | 1..N | The API logs information. This attribute may be present if the subscribed event is "HISTORICAL_SERVICEAPI". |  |
| rttDeviation | Float | O | 0..1 | Indicates the deviation value of RTT. This attribute may be present if the subscribed event is "NETWORK_SLICE". |  |
| snssais | array(Snssai) | O | 1..N | Identification(s) of network slice(s) to which the subscription applies. This attribute may be present if the subscribed event is "NETWORK_SLICE". |  |
| edgeInfo | EdgeInfo | O | 0..1 | The EDGE related information. This attribute may be present if the subscribed event is "EDGE_DATA". |  |
| s2SAnaResp | S2SAnalyticsInfo | O | 0..1 | Contains the Server to Server Analytics. This attribute may be present if the reported event within the "event" attribute is "SERVER_TO_SERVER_ANALYTICS". | EdgeApp_3 |
| ueRatAnaInfo | UERatAnalyticsInfo | O | 0..1 | Contains UE RAT Connectivity Analytics information. This attribute may be present for the event "UE_RAT_ANALYTICS". | 5GSatApp_1 |
| NOTE: The format of this attribute is not specified in this release of the specification and it is up to the implementation. |  |  |  |  |  |

###### 7.10.8.5.2.7	Type: ApiLogInfo

Table 7.10.1.4.2.7-1: Definition of type ApiLogInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiId | string | M | 1 | String identifying the API invoked. |  |
| apiName | string | O | 0..1 | Name of the API which was invoked, it is set as {apiName} part of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [3]. |  |
| failNum | Uinteger | O | 0..1 | Indicates the number of failure API invocations. |  |
| apiAvailableInd | boolean | O | 0..1 | Indicates the API availability. Set to "true" to indicate that the API is available. Default value is "false" if omitted. |  |
| apiVerChgNum | Uinteger | O | 0..1 | Indicates the number of API version changes. |  |
| apiLocChg | boolean | O | 0..1 | Indicates the API location changes for the target API. Set to "true" to indicate that the API location has changed. Default value is "false" if omitted. |  |
| apiThrottlingEvents | string | O | 0..1 | Indicates the API throttling events. (NOTE) |  |
| invokeNum | Uinteger | O | 0..1 | Indicates the number of API invocations. |  |
| apiVersion | string | O | 0..1 | Version of the API which was invoked. |  |
| NOTE: The format of this attribute is not specified in this release of the specification and it is up to the implementation. |  |  |  |  |  |

###### 7.10.8.5.2.8	Type: EdgeInfo

Table 7.10.8.5.2.8-1: Definition of type EdgeInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| edgeReq | EdgeReq | O | 0..1 | Indicates the DNAI, EAS and EES information which the data applies. |  |
| output | string | O | 0..1 | The reported data. (NOTE) |  |
| NOTE: The content of "output" attribute is not defined in this Release of the specification. |  |  |  |  |  |

###### 7.10.8.5.2.9	Type: DataStoreReq

Table 7.10.8.5.2.9-1: Definition of type DataStoreReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subDataStore | DataStoreSub | C | 0..1 | Represents the requirements for subscription based data storage. (NOTE) |  |
| dirStorage | Data | C | 0..1 | Represent the data for direct storage at A-ADRF. (NOTE) |  |
| storeHandl | StorageHandlingInfo | O | 0..1 | Represents the storage handling information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported features of the API as defined in clause 7.10.8.6. This attribute shall only be present when feature negotiation is need. |  |
| NOTE: Only one of these attributes shall be present. |  |  |  |  |  |

###### 7.10.8.5.2.10	Type: DataStoreSub

Table 7.10.8.5.2.10-1: Definition of type DataStoreSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataProdId | string | O | 0..1 | Represents the data producer identifier (e.g. ADAE server, A-DCCF). |  |
| analyticsId | string | C | 0..1 | Represents the analytics identifier. This attribute shall be provided if "dataProdId" is provided. |  |
| analyticsType | AnalyticsType | C | 0..1 | Represents the analytics type. This attribute shall be provided if "dataProdId" is provided. |  |
| analyticsMode | AnalyticsMode | C | 0..1 | Represents the analytics mode. This attribute shall be provided if "dataProdId" is provided. |  |
| profiles | array(DataProducerProfile) | C | 1..N | Represent the data producer profiles. This attribute shall be provided if "dataProdId" is provided. |  |
| area | LocationArea5G | O | 0..1 | Represents the location information for the data producers subscription requests. |  |
| duration | DurationSec | O | 0..1 | Represents the time validity for the data producers subscription requests. |  |

###### 7.10.8.5.2.11	Type: Data

Table 7.10.8.5.2.11-1: Definition of type Data

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataType | DataType | M | 1 | Represents the type of data. |  |
| content | Bytes | M | 1 | Represents the data that shall be stored. |  |
| timestamp | DateTime | M | 1 | Represents the timestamp |  |
| analyticsId | string | M | 1 | Represents the analytics identifier. |  |
| analyticsType | AnalyticsType | M | 1 | Represents the analytics type. |  |
| dataSource | DataSourceInfo | M | 1 | Represent the data source informaion. |  |

Editor's note:	the encoding of analyticsId attribute is FFS.

###### 7.10.8.5.2.12	Type: DataProducerProfile

Table 7.10.8.5.2.12-1: Definition of type DataProducerProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| profile | string | C | 0..1 | Represents the data profile information. (NOTE) |  |
| NOTE: Only one of these attributes shall be present. |  |  |  |  |  |

###### 7.10.8.5.2.13	Type: DataSourceInfo

Table 7.10.8.5.2.13-1: Definition of type DataSourceInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| source | string | C | 0..1 | Represents the data source information. (NOTE) |  |
| NOTE: Only one of these attributes shall be present. |  |  |  |  |  |

###### 7.10.8.5.2.14	Type: StorageHandlingInfo

Table 7.10.8.5.2.14-1: Definition of type StorageHandlingInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lifetime | DurationSec | C | 0..1 | Indicates the lifetime of the provided data or analytics as a duration in seconds. |  |
| delNotifUri | Uri | C | 0..1 | Represents notification URI for receiving deletion alerts. Its presence indicates that the consumer wants to receive deletion event notifications to the URI present in this attribute. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 7.10.8.5.2.15	Type: DataStoreResp

Table 7.10.8.5.2.15-1: Definition of type DataStoreResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| storeHandl | StorageHandlingInfo | M | 1 | Represents the storage handling information approved by the A-ADRF. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported features of the API as defined in clause 7.10.8.6. This attribute shall only be present when feature negotiation is need. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 7.10.8.5.2.16	Type: AnalyticsMode

Table 7.10.8.5.2.16-1: Definition of type AnalyticsMode

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mode | Mode | M | 1 | Represents the mode of analytics. |  |
| isMlEnab | boolean | C | 0..1 | Indicates the support of ML-enabled analytics. - Set to "true" to indicate the ML-enabled analytics. - Set to "false" otherwise. - Default: "false". |  |

###### 7.10.8.5.2.17	Type: S2SAnalyticsReq

Table 7.10.8.5.2.17-1: Definition of type S2SAnalyticsReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serverInfoList | array(ServerInfo) | M | 1..N | Contains the list of the identifier(s) of the server(s) for which the analytics subscription applies. |  |
| timeVal | TimeWindow | O | 0..1 | Contains the time interval within which the request applies. |  |

Editor's Note: FFS on contents of the S2SAnalyticsReq data type.

###### 7.10.8.5.2.18	Type: S2SAnalyticsInfo

Table 7.10.8.5.2.18-1: Definition of type S2SAnalyticsInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataOutput | S2SPerfAnalyticsData | M | 1 | Contains the Server to Server Performance Analytics data. |  |
| serverInfoList | array(ServerInfo) | M | 1..N | Contains the list of the identifier(s) of the server(s) for which the analytics report applies. |  |

###### 7.10.8.5.2.19	Type: UERatAnalyticsReq

Table 7.10.8.5.2.19-1: Definition of type UERatAnalyticsReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| area | LocationArea5G | O | 0..1 | Contains the geographical or service area for which the subscription request applies. |  |
| timeVal | TimeWindow | O | 0..1 | Contains the time validity of the request. |  |
| valUeAddrInfo | array(ValUeAddrInfo) | M | 0..N | Contains VAL UE address information. |  |

###### 7.10.8.5.2.20	Type: UERatAnalyticsInfo

Table 7.10.8.5.2.20-1: Definition of type UERatAnalyticsInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsOut | array(UeRatAnalyticsOutput) | M | 1..N | Contains the reported analytics for the UE RAT connectivity, which can be in form of offline stats/historical data for a specific VAL service or for particular UE(s) or group of UEs. |  |

##### 7.10.8.5.3	Simple data types and enumerations

###### 7.10.8.5.3.1	Enumeration: AadrfEvent

Table 7.10.8.5.3.1-1: Enumeration AadrfEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| HISTORICAL_SERVICEAPI | The event for historical service API logs. |  |
| NETWORK_SLICE | The event for the network slice data. |  |
| EDGE_DATA | The event for the EDGE related data. |  |
| LOCATION_ACCURACY | The event for the location accuracy data. |  |
| SERVER_TO_SERVER_ANALYTICS | The event for the Server to Server analytics data. | EdgeApp_3 |
| UE_RAT_ANALYTICS | The event for the UE RAT Connecitivity Analytics. |  |

NOTE:	The A-ADRF is not required to support all the events simultaneously. The supported events negotiation between the consumer and A-ADRF depends on SLA.

###### 7.10.8.5.3.2	Enumeration: ExposureLevel

Table 7.10.8.5.3.2-1: Enumeration ExposureLevel

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| READ | The exposure level for the logs to be exposed is read. |  |
| WRITE | The exposure level for the logs to be exposed is write. |  |
| DELETE | The exposure level for the logs to be exposed is delete. |  |

###### 7.10.8.5.3.3	Enumeration: DataType

Table 7.10.8.5.3.3-1: Enumeration DataType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DATA | Indicates the data. |  |
| ANALYTICS | Indicates the analytics. |  |

###### 7.10.8.5.3.4	Enumeration: Mode

Table 7.10.8.5.3.4-1: Enumeration Mode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ONLINE | Indicates the online analytics mode. |  |
| OFFLINE | Indicates the offline analytics mode. |  |

#### 7.10.8.6	Error Handling

##### 7.10.8.6.1	General

HTTP error handling shall be supported as specified in clause 6.7.

##### 7.10.8.6.2	Protocol Errors

In this Release of the specification, there are no additional protocol errors applicable for the SS_AADRF_DataManagement API.

##### 7.10.8.6.3	Application Errors

The application errors defined for SS_AADRF_DataManagement API are listed in table 7.10.8.6.3-1.

Table 7.10.8.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.8.7	Feature negotiation

General feature negotiation procedures are defined in clause 6.8.

Table 7.10.8.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | EdgeApp_3 | This feature indicates the support of the second set of enhancements to the Edge Applications functionalities. The following sub-functionalities are supported: - Support Server to Server Analytics data management. |
| 2 | 5GSatApp_1 | This feature indicates the support of the enablements to the SEAL ADAE layer to support satellite applications. Within this feature, the following enhancements are supported: - UE RAT Connectivity analytics data management. |

### 7.10.9	SS_ADAE_LocationRelatedUeGroupAnalytics

#### 7.10.9.1	Introduction

The ADAE Location-Related UE Group Analytics service shall use the SS_ADAE_LocationRelatedUeGroupAnalytics.

The API URI of the SS_ADAE_LocationRelatedUeGroupAnalytics API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-adae-lruga".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5.

NOTE:	When 3GPP TS 29.122 [3] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.5, the ADAE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 7.10.9.2	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_ADAE_LocationRelatedUeGroupAnalytics API.

#### 7.10.9.3	Resources

##### 7.10.9.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.9.3.1-1 depicts the resource URIs structure for the SS_ADAE_LocationRelatedUeGroupAnalytics.

Figure 7.10.9.3.1-1: Resource URI structure of the SS_ADAE_LocationRelatedUeGroupAnalytics

Table 7.10.9.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.9.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Location-Related UE Group Analytics Subscriptions | /subscriptions | POST | Create an "Individual Location-Related UE Group Analytics Subscription" resource. |
| Individual Location-Related UE Group Analytics Subscription | /subscriptions/{subscriptionId} | GET | Read the "Individual Location-Related UE Group Analytics Subscription" resource. |
|  |  | PUT | Update the "Individual Location-Related UE Group Analytics Subscription" resource. |
|  |  | PATCH | Modify the "Individual Location-Related UE Group Analytics Subscription" resource. |
|  |  | DELETE | Remove the "Individual Location-Related UE Group Analytics Subscription" resource. |

##### 7.10.9.3.2	Resource: Location-Related UE Group Analytics Subscriptions

###### 7.10.9.3.2.1	Description

The Location-Related UE Group Analytics Subscriptions to the event of the location-related UE group analytics.

###### 7.10.9.3.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-lruga/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 7.10.9.3.2.2-1.

Table 7.10.9.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.10.9.3.2.3	Resource Standard Methods

7.10.9.3.2.3.1	POST

This method to subscribe to the event of the location-related UE group analytics and shall support the URI query parameters specified in table 7.10.9.3.2.3.1-1.

Table 7.10.9.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.9.3.2.3.1-2 and the response data structures and response codes specified in table 7.10.9.3.2.3.1-3.

Table 7.10.9.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocRelUeGroupSub | M | 1 | Subscription to the location-related UE group analytics. |

Table 7.10.9.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocRelUeGroupSub | M | 1 | 201 Created | The "Individual Location-Related UE Group Analytics Subscription" resource is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.9.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-lruga/<apiVersion>/subscriptions/{subscriptionId} |

###### 7.10.9.3.2.4	Resource Custom Operations

None.

##### 7.10.9.3.3	Resource: Individual Location-Related UE Group Analytics Subscription

###### 7.10.9.3.3.1	Description

###### 7.10.9.3.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-lruga/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 7.10.9.3.3.2-1.

Table 7.10.9.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| subscriptionId | string | Represents the identifier of an "Individual Location-Related UE Group Analytics Subscription" resource. |

###### 7.10.9.3.3.3	Resource Standard Methods

7.10.9.3.3.3.1	GET

This operation reads the "Individual Location-Related UE Group Analytics Subscription" resource. This method shall support the URI query parameters specified in table 7.10.9.3.3.3.1-1.

Table 7.10.9.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.10.9.3.3.3.1-2 and the response data structures and response codes specified in table 7.10.9.3.3.3.1-3.

Table 7.10.9.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.9.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocRelUeGroupSub | M | 1 | 200 OK | The requested "Individual Location-Related UE Group Analytics Subscription" resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.9.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

Table 7.10.9.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

7.10.9.3.3.3.2	DELETE

This operation deletes "Individual Location-Related UE Group Analytics Subscription" resource. This method shall support the URI query parameters specified in table 7.10.9.3.3.3.2-1.

Table 7.10.9.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.9.3.3.3.2-2 and the response data structures and response codes specified in table 7.10.9.3.3.3.2-3.

Table 7.10.9.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.9.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The "Individual Location-Related UE Group Analytics Subscription" resource matching the subscriptionId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.9.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.9.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

7.10.9.3.3.3.3	PUT

This method enables to update the "Individual Location-Related UE Group Analytics Subscription" resource.

This method shall support the URI query parameters specified in table 7.10.9.3.3.3.3-1.

Table 7.10.9.3.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.9.3.3.3.3-2 and the response data structures and response codes specified in table 7.10.9.3.3.3.3-3.

Table 7.10.9.3.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocRelUeGroupSub | M | 1 | Contains the updated representation of the "Individual Location-Related UE Group Analytics Subscription" resource. |

Table 7.10.9.3.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocRelUeGroupSub | M | 1 | 200 OK | Successful case. The "Individual Location-Related UE Group Analytics Subscription" resource is successfully updated, and the representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Location-Related UE Group Analytics Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.9.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.9.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

7.10.9.3.3.3.4	PATCH

This method modifies the "Individual Location-Related UE Group Analytics Subscription" resource.

This method shall support the URI query parameters specified in table 7.10.9.3.3.3.4-1.

Table 7.10.9.3.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.9.3.3.3.4-2 and the response data structures and response codes specified in table 7.10.9.3.3.3.4-3.

Table 7.10.9.3.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocRelUeGroupSubPatch | M | 1 | Contains the modifications to be applied to the Individual Location-Related UE Group Analytics Subscription resource. |

Table 7.10.9.3.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocRelUeGroupSub | M | 1 | 200 OK | Successful case. The "Individual Location-Related UE Group Analytics Subscription" resource is successfully modified, and a representation of the updated resource is returned in the response. |
| n/a |  |  | 204 No Content | Successful case. The “Individual Location-Related UE Group Analytics Subscription” resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAES server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.9.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE server. |

Table 7.10.9.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE server. |

###### 7.10.9.3.3.4	Resource Custom Operations

None.

#### 7.10.9.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

#### 7.10.9.5	Notifications

##### 7.10.9.5.1	General

Notifications shall comply to clause 6.6.

Table 7.10.9.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Location-Related UE Group Analytics Notification | {notifUri} | POST | Notification on location-related UE group analytics. |

##### 7.10.9.5.2	Location-Related UE Group Analytics Notification

###### 7.10.9.5.2.1	Description

Location-related UE group analytics notification is to notify on the event of location-related UE group analytics.

###### 7.10.9.5.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.9.5.2.2-1.

Table 7.10.9.5.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.9.5.2.2-2 and the response data structures and response codes specified in table 7.10.9.5.2.2-3.

Table 7.10.9.5.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocRelUeGroupNotif | M | 1 | Notification information of location-related UE group analytics. |

Table 7.10.9.5.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification for the location-related UE group analytics is accepted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.9.5.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.9.5.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.9.6	Data Model

##### 7.10.9.6.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.9.6.1-1 specifies the data types defined specifically for the SS_ADAE_LocationRelatedUeGroupAnalytics service.

Table 7.10.9.6.1-1: SS_ADAE_LocationRelatedUeGroupAnalytics specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AnalyticsIdLocGrp | 7.10.9.6.3.3 | Represents the analytic IDs for location related UE group analytics |  |
| Deviation | 7.10.9.6.2.12 | Represents the deviation in VAL UE movement. |  |
| LocDeviationTypeData | 7.10.9.6.2.6 | Represents the location-related UE group analytics deviation data. |  |
| LocDeviationCriteria | 7.10.9.6.2.7 | Represents the location-related UE group analytics deviation criteria. |  |
| LocRelUeGroupAnalytics | 7.10.9.6.2.9 | Represents the location-related UE group analytics output. |  |
| LocRelUeGroupFilter | 7.10.9.6.2.4 | Represent the the location-related UE group analytics filtering information. |  |
| LocRelUeGroupNotif | 7.10.9.6.2.3 | Represents the location-related UE group analytics notification. |  |
| LocRelUeGroupSub | 7.10.9.6.2.2 | Represents the location-related UE group analytics subscription. |  |
| LocRelUeGroupSubPatch | 7.10.9.6.2.8 | Represents the requested modifications to a the location-related UE group analytics subscription. |  |
| LocDeviationCriterion | 7.10.9.6.2.5 | Represents the threshold for the location-related UE group analytics deviation. |  |
| LocRelUeGroupAnalytics | 7.10.9.6.2.10 | Represents the UE group route analytics output. |  |
| UeGroupMemberDevAnalytics | 7.10.9.6.2.11 | Represents the group member deviation analytics output. |  |

Table 7.10.9.6.1-2 specifies data types re-used by the SS_ADAE_LocationRelatedUeGroupAnalytics service:

Table 7.10.9.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.2.6 | Represents the type of analytics. |  |
| AppGrpProfile | 3GPP TS 29.558 [A] | Represents the application group profile. |  |
| DurationSec | 3GPP TS 29.122 [3] | Represents a period of time in units of seconds. |  |
| Float | 3GPP TS 29.571 [21] | Represents float attributes. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| MatchingDirection | 3GPP TS 29.520 [33] | Indicates a threshold matching direction. |  |
| Direction | Clause 7.10.10.6.2.10 | Represents the direction. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Indicates the reporting requirements. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the supported optional features of the API. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Indicates the time window. |  |
| Uri | 3GPP TS 29.122 [3] | Indicate the notification URI. |  |

##### 7.10.9.6.2	Structured data types

###### 7.10.9.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.10.9.6.2.2	Type: LocRelUeGroupSub

Table 7.10.9.6.2.2-1: Definition of type LocRelUeGroupSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsType | AnalyticsType | M | 1 | Represents the type of the location-related UE group analytics. |  |
| analyticsId | AnalyticsIdLocGrp | O | 0..1 | Represents the analytics ID. |  |
| analyticsFilt | LocRelUeGroupFilter | M | 1 | Represents the analytics filter information. |  |
| confLevel | integer | C | 0..1 | Indicates the preferred accuracy level prediction. This attribute shall be provided if the "analyticsType" attribute in the request is set to "PREDICTIVE". Minimum = 0. Maximum = 100. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the subscription. |  |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| timeInterval | TimeWindow | O | 0..1 | The time interval as the start time and end time, to which the subscription is applied. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |

###### 7.10.9.6.2.3	Type: LocRelUeGroupNotif

Table 7.10.9.6.2.3-1: Definition of type LocRelUeGroupNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsOutput | LocRelUeGroupAnalytics | M | 1 | Represents the analytics output. |  |
| timeValid | DurationSec | O | 0..1 | Represents the valid time duration of the analytics. |  |
| confLevel | integer | C | 0..1 | Indicates the achieved confidence of the prediction. This attribute shall be provided if the "analyticsType" attribute in the request is set to "PREDICTIVE". Minimum = 0. Maximum = 100. |  |

###### 7.10.9.6.2.4	LocRelUeGroupFilter

Table 7.10.9.6.2.4-1: Definition of type LocRelUeGroupFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| groupId | string | C | 0..1 | Contains the identifier of the application group. (NOTE) |  |
| valTrgUes | array(string) | C | 1..N | Represents the list of VAL UEs, whose analytics are subscribed to. (NOTE) |  |
| area | LocationArea5G | O | 0..1 | Represents the geographical or service area, to which the subscription is applied. |  |
| profiles | array(AppGrpProfile) | O | 1..N | Represents the application group profiles. |  |
| deviation | array(LocDeviationCriteria) | O | 1..N | Represents the deviation information. |  |
| NOTE: Only one of these attributes shall be provided. |  |  |  |  |  |

###### 7.10.9.6.2.5	Type: LocDeviationCriterion

Table 7.10.9.6.2.5-1: Definition of type LocDeviationCriterion

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| value | LocDeviationTypeData | M | 1 | Indicates the value for the analytics threshold. |  |
| thrDirect | MatchingDirection | M | 1 | Indicates the threshold matching direction for the analytics threshold provided in the "value" attribute. |  |

###### 7.10.9.6.2.6	Type: LocDeviationTypeData

Table 7.10.9.6.2.6-1: Definition of type LocDeviationTypeData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| speed | Float | C | 0..1 | The speed in unit of meters per second. (NOTE) |  |
| distance | Float | C | 0..1 | The distance in unit of meters. (NOTE) |  |
| direction | Direction | C | 0..1 | The direction deviation. (NOTE) |  |
| NOTE: Only one of these attributes shall be provided. |  |  |  |  |  |

###### 7.10.9.6.2.7	Type: LocDeviationCriteria

Table 7.10.9.6.2.7-1: Definition of type LocDeviationCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| target | string | M | 1 | Represent the target VAL UE. |  |
| criteria | array(LocDeviationCriterion) | M | 1..N | Represent the deviation criteria. |  |

###### 7.10.9.6.2.8	Type: LocRelUeGroupSubPatch

Table 7.10.9.6.2.8-1: Definition of type LocRelUeGroupSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsFilt | LocRelUeGroupFilter | O | 0..1 | Represents the analytics filter information. |  |
| confLevel | integer | O | 0..1 | Indicates the preferred accuracy level prediction. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the subscription. |  |
| notifUri | Uri | O | 0..1 | Represents the notification URI. |  |
| timeInterval | TimeWindow | O | 0..1 | The time interval as the start time and end time, to which the subscription is applied. |  |

###### 7.10.9.6.2.9	Type: LocRelUeGroupAnalytics

Table 7.10.9.6.2.9-1: Definition of type LocRelUeGroupAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsId | AnalyticsIdLocGrp | M | 1 | Represents the analytics ID. |  |
| ueGroupRoute | array(UeGroupRouteAnalytics) | C | 1..N | Represents the UE group route analytics. This attribute shall be present only if the "analyticsId" attribute is set to "GROUP_ROUTE". |  |
| ueGrMemDev | array(UeGroupMemberDevAnalytics) | C | 1..N | Represents the group member deviation analytics. This attribute shall be present only provided if the "analyticsId" attribute is set to "GROUP_MEMBER_DEVIATION". |  |

###### 7.10.9.6.2.10	Type: UeGroupRouteAnalytics

Table 7.10.9.6.2.10-1: Definition of type UeGroupRouteAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easIds | array(string) | O | 1,,N | Represents the list of EAS identifier(s). |  |
| routeId | string | M | 1 | Represents identifier of the common route for the UE(s) to EAS(s). |  |
| valUes | array(string) | M | 1..N | Represents the list of VAL UE ID(s) with the common route to the EAS(s). |  |

###### 7.10.9.6.2.11	Type: UeGroupMemberDevAnalytics

Table 7.10.9.6.2.11-1: Definition of type UeGroupMemberDevAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| groupId | string | M | 1 | Contains the identifier of the application group. |  |
| refUe | string | O | 0..1 | Indicates the reference VAL UE that defines the correct UE behaviour. |  |
| devUes | array(Deviation) | M | 1..N | Represents the list of VAL UEs with different behaviour compared to majority group members in "groupId" attribute or the reference VAL UE. |  |

###### 7.10.9.6.2.12	Type: Deviation

Table 7.10.9.6.2.12-1: Definition of type Deviation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| target | string | M | 1 | Represent the deviating VAL UE. |  |
| criteria | array(LocDeviationCriterion) | M | 1..N | Represent the deviation criteria. |  |
| duration | DurationSec | O | 0..1 | Represent the deviation duration. |  |

##### 7.10.9.6.3	Simple data types and enumerations

###### 7.10.9.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.9.6.3.2	Simple data types

The simple data types defined in table 7.10.9.6.3.2-1 shall be supported.

Table 7.10.9.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

###### 7.10.9.6.3.3	Enumeration: AnalyticsIdLocGrp

The enumeration AnalyticsIdLocGrp represents the analytic IDs for location related UE group analytics. It shall comply with the provisions defined in table 7.10.9.6.3.3-1.

Table 7.10.9.6.3.3-1: Enumeration AnalyticsIdLocGrp

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| GROUP_ROUTE | Indicates that the requested analytics is UE group route analytics. |  |
| GROUP_MEMBER_DEVIATION | Indicates that the requested analytics is UE group member deviation analytics. |  |

##### 7.10.9.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 7.10.9.6.5	Binary data

###### 7.10.9.6.5.1	Binary Data Types

Table 7.10.9.6.5.1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.10.9.7	Error Handling

##### 7.10.9.7.1	General

For the SS_ADAE_LocationRelatedUeGroupAnalytics API, error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses are applicable for the SS_ADAE_LocationRelatedUeGroupAnalytics API.

##### 7.10.9.7.2	Protocol Errors

No specific protocol errors for the SS_ADAE_LocationRelatedUeGroupAnalytics API are specified.

##### 7.10.9.7.3	Application Errors

The application errors defined for SS_ADAE_LocationRelatedUeGroupAnalytics are listed in table 7.10.9.7.3-1.

Table 7.10.9.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.9.8	Feature Negotiation

The optional features in table 7.10.9.8-1 are defined for the SS_ADAE_LocationRelatedUeGroupAnalytics API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.10.9.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 7.10.9.9	Security

The provisions of clause 9 shall apply for the SS_ADAE_LocationRelatedUeGroupAnalytics API.

### 7.10.10	SS_ADAE_CollisionDetectionAnalytics

#### 7.10.10.1	Introduction

The ADAE Collision Detection Analytics service shall use the SS_ADAE_CollisionDetectionAnalytics.

The API URI of the SS_ADAE_CollisionDetectionAnalytics API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-adae-cda".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5.

NOTE:	When 3GPP TS 29.122 [3] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.5, the ADAE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 7.10.10.2	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_ADAE_CollisionDetectionAnalytics API.

#### 7.10.10.3	Resources

##### 7.10.10.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.10.3.1-1 depicts the resource URIs structure for the SS_ADAE_CollisionDetectionAnalytics.

Figure 7.10.10.3.1-1: Resource URI structure of the SS_ADAE_CollisionDetectionAnalytics

Table 7.10.10.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.10.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Collision Detection Analytics Subscriptions | /subscriptions | POST | Create an "Individual Collision Detection Analytics Subscription" resource. |
| Individual Collision Detection Analytics Subscription | /subscriptions/{subscriptionId} | GET | Read the "Individual Collision Detection Analytics Subscription" resource. |
|  |  | PUT | Update the "Individual Collision Detection Analytics Subscription" resource. |
|  |  | PATCH | Modify the "Individual Collision Detection Analytics Subscription" resource. |
|  |  | DELETE | Remove the "Individual Collision Detection Analytics Subscription" resource. |

##### 7.10.10.3.2	Resource: Collision Detection Analytics Subscriptions

###### 7.10.10.3.2.1	Description

The "Collision Detection Analytics Subscriptions" resource collection represents the collision detection analytics subscriptions.

###### 7.10.10.3.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-cda/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 7.10.10.3.2.2-1.

Table 7.10.10.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.10.10.3.2.3	Resource Standard Methods

7.10.10.3.2.3.1	POST

This method to subscribe to the event of the UE-to-UE session performance analytics and shall support the URI query parameters specified in table 7.10.10.3.2.3.1-1.

Table 7.10.10.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.10.3.2.3.1-2 and the response data structures and response codes specified in table 7.10.10.3.2.3.1-3.

Table 7.10.10.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CollisionDetectionSub | M | 1 | Subscription to the collision detection analytics. |

Table 7.10.10.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CollisionDetectionSub | M | 1 | 201 Created | Subscription to the collision detection analytics is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.10.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-cda/<apiVersion>/subscriptions/{subscriptionId} |

###### 7.10.10.3.2.4	Resource Custom Operations

None.

##### 7.10.10.3.3	Resource: Individual Collision Detection Analytics Subscription

###### 7.10.10.3.3.1	Description

###### 7.10.10.3.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-cda/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 7.10.10.3.3.2-1.

Table 7.10.10.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| subscriptionId | string | Represents the identifier of an Individual Collision Detection Analytics Subscription. |

###### 7.10.10.3.3.3	Resource Standard Methods

7.10.10.3.3.3.1	GET

This operation reads the "Individual Collision Detection Analytics Subscription" resource. This method shall support the URI query parameters specified in table 7.10.10.3.3.3.1-1.

Table 7.10.10.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.10.10.3.3.3.1-2 and the response data structures and response codes specified in table 7.10.10.3.3.3.1-3.

Table 7.10.10.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.10.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CollisionDetectionSub | M | 1 | 200 OK | The requested "Individual Collision Detection Analytics Subscription" resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.10.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

Table 7.10.10.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

7.10.10.3.3.3.2	DELETE

This operation deletes the "Individual Collision Detection Analytics Subscription" resource. This method shall support the URI query parameters specified in table 7.10.10.3.3.3.2-1.

Table 7.10.10.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.10.3.3.3.2-2 and the response data structures and response codes specified in table 7.10.10.3.3.3.2-3.

Table 7.10.10.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.10.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The "Individual Collision Detection Analytics Subscription" resource matching the subscriptionId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.10.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.10.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative ADAE server. |

7.10.10.3.3.3.3	PUT

This method enables to update the "Individual Collision Detection Analytics Subscription" resource.

This method shall support the URI query parameters specified in table 7.10.10.3.3.3.3-1.

Table 7.10.10.3.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.10.3.3.3.3-2 and the response data structures and response codes specified in table 7.10.10.3.3.3.3-3.

Table 7.10.10.3.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CollisionDetectionSub | M | 1 | Contains the updated representation of the "Individual Collision Detection Analytics Subscription" resource. |

Table 7.10.10.3.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CollisionDetectionSub | M | 1 | 200 OK | Successful case. The "Individual Collision Detection Analytics Subscription" resource is successfully updated, and the representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Collision Detection Analytics Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.10.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.10.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

7.10.10.3.3.3.4	PATCH

This method modifies the "Individual Collision Detection Analytics Subscription" resource.

This method shall support the URI query parameters specified in table 7.10.10.3.3.3.4-1.

Table 7.10.10.3.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.10.3.3.3.4-2 and the response data structures and response codes specified in table 7.10.10.3.3.3.4-3.

Table 7.10.10.3.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CollisionDetectionSubPatch | M | 1 | Contains the modifications to the "Individual Collision Detection Analytics Subscription" resource. |

Table 7.10.10.3.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CollisionDetectionSub | M | 1 | 200 OK | Successful case. The "Individual Collision Detection Analytics Subscription" resource is successfully modified, and a representation of the updated resource is returned in the response. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Collision Detection Analytics Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAES server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.10.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE server. |

Table 7.10.10.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE server. |

###### 7.10.10.3.3.4	Resource Custom Operations

None.

#### 7.10.10.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

#### 7.10.10.5	Notifications

##### 7.10.10.5.1	General

Notifications shall comply to clause 6.6.

Table 7.10.10.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Collision Detection Analytics Notification | {notifUri} | POST | Notification on collision detection analytics. |

##### 7.10.10.5.2	Collision detection analytics Notification

###### 7.10.10.5.2.1	Description

Collision detection analytics notification is to notify on the event of collision detection analytics.

###### 7.10.10.5.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.10.5.2.2-1.

Table 7.10.10.5.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.10.5.2.2-2 and the response data structures and response codes specified in table 7.10.10.5.2.2-3.

Table 7.10.10.5.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CollisionDetectionNotif | M | 1 | Notification information of collision detection analytics. |

Table 7.10.10.5.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification for the collision detection analytics is accepted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.10.5.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.10.5.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.10.6	Data Model

##### 7.10.10.6.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.10.6.1-1 specifies the data types defined specifically for the SS_ADAE_CollisionDetectionAnalytics service.

Table 7.10.10.6.1-1: SS_ADAE_CollisionDetectionAnalytics specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| Direction | 7.10.10.6.2.10 | Represents the direction. |  |
| TargetToAnyUeLocAnalytics | 7.10.10.6.2.9 | Represents the output for collision detection analytics between any VAL UEs within given location. |  |
| TargetToAnyUeLocFilter | 7.10.10.6.2.7 | Represent the analytics filter between any VAL UEs within the given location. |  |
| CollisionDetectionFilter | 7.10.10.6.2.4 | Represent the the collision detection filtering information. |  |
| CollisionDetectionNotif | 7.10.10.6.2.3 | Represents the collision detection analytics notification. |  |
| CollisionDetectionSub | 7.10.10.6.2.2 | Represents the collision detection analytics subscription. |  |
| CollisionDetectionSubPatch | 7.10.10.6.2.11 | Represents the requested modifications to a collision detection analytics subscription. |  |
| CollisionDetectionCriteria | 7.10.10.6.2.6 | Represents the collision detection criteria. |  |
| TargetOtherUesAnalytics | 7.10.10.6.2.8 | Represents the output for collision detection analytics between the sets of target VAL UEs and other VAL UEs. |  |
| TargetOtherUesFilter | 7.10.10.6.2.5 | Represent the analytics filter between sets of target and other UEs within a location. |  |

Table 7.10.10.6.1-2 specifies data types re-used by the SS_ADAE_CollisionDetectionAnalytics service:

Table 7.10.10.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Angle | 3GPP TS 29.572 [31] | Represents the angle. |  |
| AnalyticsType | Clause 7.10.1.4.2.6 | Represents the type of analytics. |  |
| DurationSec | 3GPP TS 29.122 [3] | Represents a period of time in units of seconds. |  |
| Float | 3GPP TS 29.571 [21] | Represents float attributes. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| MatchingDirection | 3GPP TS 29.520 [33] | Represents threshold matching direction. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Indicates the reporting requirements. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the supported optional features of the API. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Indicate the time window. |  |
| Uri | 3GPP TS 29.122 [3] | Indicates the notification URI. |  |

##### 7.10.10.6.2	Structured data types

###### 7.10.10.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.10.10.6.2.2	Type: CollisionDetectionSub

Table 7.10.10.6.2.2-1: Definition of type CollisionDetectionSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsType | AnalyticsType | M | 1 | Represents the type of the collision detection analytics. |  |
| analyticsId | AnalyticsIdCda | O | 0..1 | Represents the requested analytics ID. |  |
| analyticsFilt | CollisionDetectionFilter | M | 1 | Represents the analytics filter information. |  |
| colDetCriteria | CollisionDetectionCriteria | O | 0..1 | Represents the collision detection criteria. |  |
| confLevel | integer | O | 0..1 | Indicates the preferred accuracy level prediction. This attribute shall be provided if the "analyticsType" attribute in the request is set to "PREDICTIVE". Minimum = 0. Maximum = 100. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the subscription. |  |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| timeInterval | TimeWindow | O | 0..1 | The time interval as the start time and end time, to which the subscription is applied. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |

###### 7.10.10.6.2.3	Type: CollisionDetectionNotif

Table 7.10.10.6.2.3-1: Definition of type CollisionDetectionNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsId | AnalyticsIdCda | M | 1 | Represents the analytics ID. |  |
| targetOther | array(TargetOtherUesAnalytics) | C | 1..N | Represents the output for collision detection analytics between the sets of target VAL UEs and other VAL UEs. This attribute shall be present only if the "analyticsId" attribute is set to "TARGET_WITH_OTHER_UES". |  |
| anyInLoc | array(TargetToAnyUeLocAnalytics) | C | 1..N | Represents the output for collision detection analytics between any VAL UEs within given location. This attribute shall be present only if the "analyticsId" attribute is set to "ANY_UES_IN_LOC". |  |

###### 7.10.10.6.2.4	CollisionDetectionFilter

Table 7.10.10.6.2.4-1: Definition of type CollisionDetectionFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trgToOther | TargetOtherUesFilter | C | 0..1 | Represents the filter information for the collision detection between two sets of VAL UEs within the given area. |  |
| targetToAnyUeLoc | TargetToAnyUeLocFilter | C | 0..1 | Represents the filter information for the collision detection between any VAL UEs within the given location. |  |
| NOTE: Only one of these attributes shall be present. |  |  |  |  |  |

###### 7.10.10.6.2.5	Type: TargetOtherUesFilter

Table 7.10.10.6.2.5-1: Definition of type TargetOtherUesFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trgValServerId | string | O | 0..1 | Represents the target VAL Server ID for which the analytics subscription applies. |  |
| targetUes | array(string) | M | 1..N | Represent the list of VAL UEs, whose analytics are subscribed to. |  |
| otherUes | array(string) | O | 1..N | Represent the list of any VAL UEs collision with that shall be analysed. |  |
| area | LocationArea5G | O | 0..1 | Represents the geographical or service area, to which the subscription is applied. |  |

###### 7.10.10.6.2.6	Type: CollisionDetectionCriteria

Table 7.10.10.6.2.6-1: Definition of type CollisionDetectionCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| distance | Float | C | 0..1 | The distance in unit of meters. The collision is detected when the distance is less than given value within this attribute. (NOTE) |  |
| NOTE: At least one of these attributes shall be provided. |  |  |  |  |  |

###### 7.10.10.6.2.7	Type: TargetToAnyUeLocFilter

Table 7.10.10.6.2.7-1: Definition of type TargetToAnyUeLocFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trgValServerId | string | O | 0..1 | Represents the target VAL Server ID for which the analytics subscription applies. |  |
| area | LocationArea5G | M | 1 | Represents the geographical or service area, where the collision between any VAL UEs is requirested to be detected. |  |

###### 7.10.10.6.2.8	Type: TargetOtherUesAnalytics

Table 7.10.10.6.2.8-1: Definition of type TargetOtherUesAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| targetUe | string | M | 1 | Represents the target VAL UE. |  |
| otherUe | string | M | 1 | Represents the other VAL UE. |  |
| colTime | DateTime | M | 1 | Represents the time of potential collision. |  |
| location | LocationArea5G | M | 1 | Represents the location of potential collision. |  |
| targetUeVel | Float | O | 0..1 | Represents the velocity of the target UE in units of meters per second. |  |
| otherUeVel | Float | O | 0..1 | Represents the velocity of the other UE in units of meters per second. |  |
| targetUeDir | Direction | O | 0..1 | Represents the direction of the target UE. |  |
| otherUeDir | Direction | O | 0..1 | Represents the direction of the other UE. |  |
| confLevel | integer | O | 0..1 | Indicates the achieved confidence of the prediction. Minimum = 0. Maximum = 100. |  |

###### 7.10.10.6.2.9	Type: TargetToAnyUeLocAnalytics

Table 7.10.10.6.2.9-1: Definition of type TargetToAnyUeLocAnalytics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ues | array(string) | M | 2..2 | Represents the colliding VAL UEs. |  |
| colTime | DateTime | M | 1 | Represents the time of potential collision between VAL UEs. |  |
| location | LocationArea5G | M | 1 | Represents the location of potential collision between VAL UEs. |  |
| confLevel | integer | O | 0..1 | Indicates the achieved confidence of the prediction. Minimum = 0. Maximum = 100. |  |

###### 7.10.10.6.2.10	Type: Direction

Table 7.10.10.6.2.10-1: Definition of type Direction

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| azimuthDirection | Angle | M | 0..1 | Indentifies the azimuth direction. |  |
| elevationDirection | Angle | M | 0..1 | Indentifies the elevation direction. |  |

###### 7.10.10.6.2.11	Type: CollisionDetectionSubPatch

Table 7.10.10.6.2.11-1: Definition of type CollisionDetectionSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsFilt | CollisionDetectionFilter | O | 0..1 | Represents the analytics filter information. |  |
| colDetCriteria | CollisionDetectionCriteria | O | 0..1 | Represents the collision detection criteria. |  |
| confLevel | integer | O | 0..1 | Indicates the preferred accuracy level prediction. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the subscription. |  |
| notifUri | Uri | O | 0..1 | Represents the notification URI. |  |
| timeInterval | TimeWindow | O | 0..1 | The time interval as the start time and end time, to which the subscription is applied. |  |

##### 7.10.10.6.3	Simple data types and enumerations

###### 7.10.10.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.10.6.3.2	Simple data types

The simple data types defined in table 7.10.10.6.3.2-1 shall be supported.

Table 7.10.10.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

###### 7.10.10.6.3.3	Enumeration: AnalyticsIdCda

The enumeration AnalyticsIdCda represents the analytic IDs for Collision Detection Analytics. It shall comply with the provisions defined in table 7.10.10.6.3.3-1.

Table 7.10.10.6.3.3-1: Enumeration AnalyticsIdCda

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| TARGET_WITH_OTHER_UES | Indicates that the requested analytics is collision detection between the sets of target and other VAL UEs within a location. |  |
| ANY_UES_IN_LOC | Indicates that the requested analytics is collision detection between any VAL UEs within given location. |  |

##### 7.10.10.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 7.10.10.6.5	Binary data

###### 7.10.10.6.5.1	Binary Data Types

Table 7.10.10.6.5.1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.10.10.7	Error Handling

##### 7.10.10.7.1	General

For the SS_ADAE_CollisionDetectionAnalytics API, error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses are applicable for the SS_ADAE_CollisionDetectionAnalytics API.

##### 7.10.10.7.2	Protocol Errors

No specific protocol errors for the SS_ADAE_CollisionDetectionAnalytics API are specified.

##### 7.10.10.7.3	Application Errors

The application errors defined for SS_ADAE_CollisionDetectionAnalytics are listed in table 7.10.10.5.3-1.

Table 7.10.10.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.10.8	Feature Negotiation

The optional features in table 7.10.10.8-1 are defined for the SS_ADAE_CollisionDetectionAnalytics API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.10.10.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 7.10.10.9	Security

The provisions of clause 9 shall apply for the SS_ADAE_CollisionDetectionAnalytics API.

### 7.10.11	SS_ADAE_AIMLMemberCapabilityAnalytics API

#### 7.10.11.1	Introduction

The SS_ADAE_AIMLMemberCapabilityAnalytics service shall use the SS_ADAE_AIMLMemberCapabilityAnalytics API.

The API URI of the SS_ADAE_AIMLMemberCapabilityAnalytics Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-adae-aimlmca".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clauses 7.10.11.3 and 7.10.11.3A.

NOTE:	When 3GPP TS 29.122 [3] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 7.10.11, the ADAE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 7.10.11.2	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_ADAE_AIMLMemberCapabilityAnalytics API.

#### 7.10.11.3	Resources

##### 7.10.11.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.11.3.1-1 depicts the resource URIs structure for the SS_ADAE_AIMLMemberCapabilityAnalytics API.

Figure 7.10.11.3.1-1: Resource URI structure of the SS_ADAE_AIMLMemberCapabilityAnalytics API

Table 7.10.11.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.11.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| AIML Member Capability Analytics Subscriptions | /application-aimlmcas | POST | Request the creation of AIML Member Capability Analytics Subscription. |
| Individual AIML Member Capability Analytics Subscription | /application-aimlmcas/{appAimlmcaId} | GET | Retrieve an existing "Individual AIML Member Capability Analytics Subscription" resource. |
|  |  | PUT | Request the update of an existing "Individual AIML Member Capability Analytics Subscription" resource. |
|  |  | PATCH | Request the modification of an existing "Individual AIML Member Capability Analytics Subscription" resource. |
|  |  | DELETE | Delete an existing "Individual AIML Member Capability Analytics Subscription" resource. |

##### 7.10.11.3.2	Resource: AIML Member Capability Analytics Subscriptions

###### 7.10.11.3.2.1	Description

The "AIML Member Capability Analytics Subscriptions" resource collection represents the AIML Member Capability Analytics Subscriptions.

###### 7.10.11.3.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-aimlmca/<apiVersion>/application-aimlmcas

This resource shall support the resource URI variables defined in the table 7.10.11.3.2.2-1.

Table 7.10.11.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.10.11.3.2.3	Resource Standard Methods

7.10.11.3.2.3.1	POST

This method shall support the URI query parameters specified in table 7.10.11.3.2.3.1-1.

Table 7.10.11.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.11.3.2.3.1-2 and the response data structures and response codes specified in table 7.10.11.3.2.3.1-3.

Table 7.10.11.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppAimlMemCapSub | M | 1 | Subscription to the AIML member capability analytics. |

Table 7.10.11.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppAimlMemCapSub | M | 1 | 201 Created | The subscription to the AIML member capability analytics is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.11.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-aimlmca/<apiVersion>/application-aimlmcas/{appAimlmcaId} |

###### 7.10.11.3.2.4	Resource Custom Operations

None.

##### 7.10.11.3.3	Resource: Individual AIML Member Capability Analytics Subscription

###### 7.10.11.3.3.1	Description

The Individual AIML member capability analytics subscription resource represents an individual event subscription of a service consumer to the AIML member capability analytics analytics of the ADAE server.

###### 7.10.11.3.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-aimlmca/<apiVersion>/application-aimlmcas/{appAimlmcaId}

This resource shall support the resource URI variables defined in the table 7.10.11.3.3.2-1.

Table 7.10.11.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| appAimlmcaId | string | Identifies an Individual Events Subscription |

###### 7.10.11.3.3.3	Resource Standard Methods

7.10.11.3.3.3.1	GET

This method shall support the URI query parameters specified in table 7.10.11.3.3.3.1-1.

Table 7.10.11.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.11.3.3.3.1-2 and the response data structures and response codes specified in table 7.10.11.3.3.3.1-3.

Table 7.10.11.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.11.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppAimlMemCapSub | M | 1 | 200 OK | The requested "Individual AIML Member Capability Analytics Subscription" resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.11.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.11.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

7.10.11.3.3.3.2	DELETE

This method shall support the URI query parameters specified in table 7.10.11.3.3.3.2-1.

Table 7.10.11.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.11.3.3.3.2-2 and the response data structures and response codes specified in table 7.10.11.3.3.3.2-3.

Table 7.10.11.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.11.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The "Individual AIML Member Capability Analytics Subscription" resource matching the appAimlmcaId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.11.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

Table 7.10.11.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative server. |

7.10.11.3.3.3.3	PUT

This method shall support the URI query parameters specified in table 7.10.11.3.3.3.3-1.

Table 7.10.11.3.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.11.3.3.3.3-2 and the response data structures and response codes specified in table 7.10.11.3.3.3.3-3.

Table 7.10.11.3.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppAimlMemCapSub | M | 1 | Represents the updated representation of "Individual AIML Member Capability Analytics Subscription" resource. |

Table 7.10.11.3.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppAimlMemCapSub | M | 1 | 200 OK | Successful case. The "Individual AIML Member Capability Analytics Subscription" resource is updated successfully, and a representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIML Member Capability Analytics Subscription" resource is updated successfully, and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.11.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.11.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

7.10.11.3.3.3.4	PATCH

This method shall support the URI query parameters specified in table 7.10.11.3.3.3.4-1.

Table 7.10.11.3.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.11.3.3.3.4-2 and the response data structures and response codes specified in table 7.10.11.3.3.3.4-3.

Table 7.10.11.3.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppAimlMemCapSubPatch | M | 1 | Represents the requested modifications to the "Individual AIML Member Capability Analytics Subscription" resource. |

Table 7.10.11.3.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppAimlMemCapSub | M | 1 | 200 OK | Successful case. The "Individual AIML Member Capability Analytics Subscription" resource is modified successfully, and a representation of the modified resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual AIML Member Capability Analytics Subscription" resource is modified successfully, and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.11.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.11.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

###### 7.10.11.3.3.4	Resource Custom Operations

None.

#### 7.10.11.3A	Custom Operations without associated resources

There are no custom operations without associated resources in the present release of the document.

#### 7.10.11.4	Notifications

##### 7.10.11.4.1	General

Notifications shall comply to clause 6.6.

Table 7.10.11.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| AIML Member Capability Analytics Notification | {notifUri} | POST | Notification on AIML member capability analytics. |

##### 7.10.11.4.2	AIML Member Capability Analytics Notification

###### 7.10.11.4.2.1	Description

AIML member capability analytics notification is to notify on the event of the AIML member capability analytics.

###### 7.10.11.4.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the service consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.11.4.2.2-1.

Table 7.10.11.4.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.11.4.2.2-2 and the response data structures and response codes specified in table 7.10.11.4.2.2-3.

Table 7.10.11.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppAimlMemCapNotif | M | 1 | Notification information on AIML member capability analytics. |

Table 7.10.11.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The notification for the AIML member capability analytics event is accepted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.11.4.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.11.4.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.11.5	Data Model

##### 7.10.11.5.1	General

This clause specifies the application data model supported by the API.

Table 7.10.11.5.1-1 specifies the data types defined specifically for the SS_ADAE_AIMLMemberCapabilityAnalytics API service.

Table 7.10.11.5.1-1: SS_ADAE_AIMLMemberCapabilityAnalytics API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AimlMemCapability | 7.10.11.5.2.8 | Represents AIML member capability information in the notification. |  |
| AnalyticsOutput | 7.10.11.5.2.7 | Represents analytics output parameters in the notification. |  |
| AppAimlMemCapAnalyticsOutput | 7.10.11.5.2.6 | Represents analytics output information in the notification. |  |
| AppAimlMemCapAnalyticsSub | 7.10.11.5.2.5 | Represents the information related to the analytics subscription. |  |
| AppAimlMemCapNotif | 7.10.11.5.2.3 | Represent the AIML member capability analytics notification. |  |
| AppAimlMemCapSub | 7.10.11.5.2.2 | Represents the AIML member capability analytics subscription details. |  |
| AppAimlMemCapSubPatch | 7.10.11.5.2.4 | Represents the AIML member capability analytics analytics subscription patch details. |  |

Table 7.10.11.5.1-2 specifies data types re-used by the SS_ADAE_AIMLMemberCapabilityAnalytics API service:

Table 7.10.11.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.2.6 | Represents the type of analytics. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Used to indicate the reporting requirement, only the following information are applicable: - immRep - notifMethod - maxReportNbr - monDur - repPeriod |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [3] | Used to define the time frame. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the supported features. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Represents a time window. |  |
| Uri | 3GPP TS 29.122 [3] | Represents a URI. |  |

##### 7.10.11.5.2	Structured data types

###### 7.10.11.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.10.11.5.2.2	Type: AppAimlMemCapSub

Table 7.10.11.5.2.2-1: Definition of type AppAimlMemCapSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| analyticsId | AppAimlMemCapAnalytics | M | 1 | Represents the analytics ID. |  |
| analyticsType | AnalyticsType | M | 1 | Represents the type of the AIML member capability analytics. |  |
| analyticsSub | AppAimlMemCapAnalyticsSub | M | 1 | Contains the requested analytics details. |  |
| confLevel | Integer | C | 0..1 | Indicates the preferred confidence level of the prediction. This attribute shall be provided if the "analyticsType" attribute in the request is set to "PREDICTIVE". Minimum = 0. Maximum = 100. |  |
| timeValidity | TimeWindow | O | 0..1 | Contains the time interval as the start and the end time, to which the AIML member capability analytics subscription is applied.. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirement of the AIML member capability analytics subscription. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains supported features information, used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |

###### 7.10.11.5.2.3	Type: Type: AppAimlMemCapNotif

Table 7.10.11.5.2.3-1: Definition of type AppAimlMemCapNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsOutput | AppAimlMemCapAnalyticsOutput | M | 1 | Represents the analytics output |  |
| confLevel | integer | C | 0..1 | Indicates the achieved confidence level in the case of prediction. This attribute shall be provided if the "analyticsType" attribute in the request is set to "PREDICTIVE". Minimum = 0. Maximum = 100. |  |

###### 7.10.11.5.2.4	Type: AppAimlMemCapSubPatch

Table 7.10.11.5.2.4-1: Definition of type AppAimlMemCapSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsSub | AppAimlMemCapAnalyticsSub | O | 0..1 | Contains the requested analytics information to be modified. |  |
| notifUri | Uri | O | 0..1 | Contains the notification URI. |  |
| confLevel | integer | O | 0..1 | Indicates the preferred confidence level of the prediction. |  |
| timeValidity | TimeWindow | O | 0..1 | Contains the time interval as the start and the end time, to which the AIML member capability analytics subscription is applied. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirement of the AIML member capability analytics subscription. |  |

###### 7.10.11.5.2.5	Type: AppAimlMemCapAnalyticsSub

Table 7.10.11.5.2.5-1: Definition of type AppAimlMemCapAnalyticsSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueIds | array(string) | C | 1..N | Contains a list of identities of one or more VAL UEs whose AIML member capability analytics are subscribed to. (NOTE) |  |
| aimlMemberIds | array(AimlMemberId) | C | 1..N | Contains a list of identities of one or more AI/ML members whose AIML member capability analytics are subscribed to. (NOTE) |  |
| aimlMemCapAttribute | AppAimlMemCapAttributes | M | 1 | Contains AIML member capability attributes to be analyzed at the ADAE client. |  |
| area | LocationArea5G | O | 0..1 | Contains the geographical or service area to which the AIML member capability analytics subscription is applied. |  |
| NOTE: Either "ueIds" or "aimlMemberIds" shall be provided. |  |  |  |  |  |

###### 7.10.11.5.2.6	Type: AppAimlMemCapAnalyticsOutput

Table 7.10.11.5.2.6-1: Definition of type AppAimlMemCapAnalyticsOutput

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsId | AppAimlMemCapAnalytics | M | 1 | Identity of the Analytics consumer, the data collection is related to, in the case of the notification is on the AIML member capability data collection. |  |
| analyticsOutput | array(AnalyticsOutput) | C | 1..N | A list of identities of one or more VAL UEs or AIML members, the data collection is related to, in the case of the notification is on the AIML member capability data collection. This attribute shall be present if the "analticsId" attribute is set to "APP_MEMBER_CAPABILITY_ANALYTICS". |  |

###### 7.10.11.5.2.7	Type: AnalyticsOutput

Table 7.10.11.5.2.7-1: Definition of type AnalyticsOutput

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueId | string | C | 0..1 | Represents the VAL ID for which the analytics output is applied. |  |
| aimlMemberId | AimlMemberId | C | 0..1 | Represents the AIML member ID for which the analytics output is applied. |  |
| timeValidity | TimeWindow | O | 0..1 | Contains the time window to which the analytics is applied to. |  |
| aimlMemAvails | array(ScheduledCommunicationTime) | M | 1..N | Contains the statistical or the predicted time window during which the AI/ML member is expected to be operational or down. |  |
| aimlMemCap | AimlMemCapability | C | 0..1 | Contains the statistical or the predicted time window during which the AI/ML member is expected to be operational or down. This attribute shall be present only if the "AppAimlMemCapAttributes" attribute is set to "COMMUNICATION_CAPABILITY". |  |
| NOTE: Either "ueId" or "aimlMemberId" shall be provided. |  |  |  |  |  |

###### 7.10.11.5.2.8	Type: AimlMemCapability

Table 7.10.11.5.2.8-1: Definition of type AimlMemCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| maxAimlMemConn | Uinteger | C | 0..1 | Contains the statistical or the predicted maximum number of supported active connections. |  |
| minAimlMemConn | Uinteger | C | 0..1 | Contains the statistical or the predicted minimum number of supported active connections. |  |

##### 7.10.11.5.3	Simple data types and enumerations

###### 7.10.11.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.11.5.3.2	Simple data types

The simple data types defined in table 7.10.11.5.3.2-1 shall be supported.

Table 7.10.11.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| AimlMemberId | string | Represents unique identifier of a AIMLE Member. |  |

###### 7.10.11.5.3.3	Enumeration: AppAimlMemCapAnalytics

The enumeration AppAimlMemCapAnalytics represents a analytics event. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 7.10.11.5.3.3-1: Enumeration AppAimlMemCapAnalytics

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| APP_MEMBER_CAPABILITY_ANALYTICS | Indicates that the identifier of the Application layer AIML member capability analytics event. |  |

###### 7.10.11.5.3.4	Enumeration: AppAimlMemCapAttributes

The enumeration AppAimlMemCapParams represents the AIML member capability attributes to be analyzed at the ADAE client. It shall comply with the provisions defined in table 7.10.11.5.3.4-1.

Table 7.10.11.5.3.4-1: Enumeration AppAimlMemCapAttributes

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| COMMUNICATION_CAPABILITY | Indicates that the requested attribute to be analyzed is communication capability. |  |

#### 7.10.11.6	Error Handling

##### 7.10.11.6.1	General

For the SS_ADAE_UeRatConnectivityAnalytics API, error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses shall apply for the SS_ADAE_AIMLMemberCapabilityAnalytics API.

##### 7.10.11.6.2	Protocol Errors

No specific protocol errors for the SS_ADAE_AIMLMemberCapabilityAnalytics API are specified.

##### 7.10.11.6.3	Application Errors

The application errors defined for SS_ADAE_AIMLMemberCapabilityAnalytics API are listed in table 7.10.11.6.3-1.

Table 7.10.11.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.11.7	Feature Negotiation

The optional features in Table 7.10.11.7-1 are defined for the SS_ADAE_AIMLMemberCapabilityAnalytics API. They shall be negotiated using the extensibility mechanism defined in clause 6.8

Table 7.10.11.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 7.10.11.8	Security

The provisions of clause 9 shall apply for the SS_ADAE_AIMLMemberCapabilityAnalytics API.

### 7.10.12	SS_ADAE_UeRatConnectivityAnalytics API

#### 7.10.12.1	Introduction

The SS_ADAE_UeRatConnectivityAnalytics service shall use the SS_ADAE_UeRatConnectivityAnalytics API.

The API URI of the SS_ADAE_UeRatConnectivityAnalytics API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-adae-uerat ".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clauses 7.10.12.3 and 7.10.12.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 7.10.12, the SEAL ADAE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 7.10.12.2	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_ADAE_UeRatConnectivityAnalytics API.

#### 7.10.12.3	Resources

##### 7.10.12.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.12.3.1-1 depicts the resource URIs structure for the SS_ADAE_UeRatConnectivityAnalytics API.

Figure 7.10.12.3.1-1: Resource URI structure of the SS_ADAE_UeRatConnectivityAnalytics API

Table 7.10.12.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.12.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| UE RAT Connectivity Analytics Subscriptions | /subscriptions | POST | Creates a new UE RAT Connectivity Analytics Subscription. |
| Individual UE RAT Connectivity Analytics Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual UE RAT Connectivity Analytics Subscription" resource. |
|  |  | PUT | Update an existing "Individual UE RAT Connectivity Analytics Subscription" resource. |
|  |  | PATCH | Modify an existing "Individual UE RAT Connectivity Analytics Subscription" resource. |
|  |  | DELETE | Delete an existing "Individual UE RAT Connectivity Analytics Subscription" resource. |

##### 7.10.12.3.2	Resource: UE RAT Connectivity Analytics Subscriptions

###### 7.10.12.3.2.1	Description

This resource represents the collection of UE RAT Connectivity Analytics Subscriptions managed by the ADAE Server.

###### 7.10.12.3.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-uerat/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 7.10.12.3.2.2-1.

Table 7.10.12.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.10.12.1. |

###### 7.10.12.3.2.3	Resource Standard Methods

7.10.12.3.2.3.1	POST

The HTTP POST method enables a service consumer to request the creation of a new UE RAT Connectivity Analytics Subscription at the ADAE Server.

This method shall support the URI query parameters specified in table 7.10.12.3.2.3.1-1.

Table 7.10.12.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.12.3.2.3.1-2 and the response data structures and response codes specified in table 7.10.12.3.2.3.1-3.

Table 7.10.12.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeRatConnectivitySub | M | 1 | Represents the parameters to request the creation of a UE RAT Connectivity Analytics Subscription. |

Table 7.10.12.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeRatConnectivitySub | M | 1 | 201 Created | Successful case. The UE RAT Connectivity Analytics Subscription is successfully created and a representation of the created "Individual UE RAT Connectivity Analytics Subscription" resource shall be returned in the response body. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.12.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-uerat/<apiVersion>/subscriptions/{subscriptionId} |

###### 7.10.12.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 7.10.12.3.3	Resource: Individual UE RAT Connectivity Analytics Subscription

###### 7.10.12.3.3.1	Description

This resource represents an "Individual UE RAT Connectivity Analytics Subscription" resource managed by the ADAE Server.

###### 7.10.12.3.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-uerat/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 7.10.12.3.3.2-1.

Table 7.10.12.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.10.12.1. |
| subscriptionId | string | Represents the identifier of the "Individual UE RAT Connectivity Analytics Subscription" resource. |

###### 7.10.12.3.3.3	Resource Standard Methods

7.10.12.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual UE RAT Connectivity Analytics Subscription" resource at the ADAE Server.

This method shall support the URI query parameters specified in table 7.10.12.3.3.3.1-1.

Table 7.10.12.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.10.12.3.3.3.1-2 and the response data structures and response codes specified in table 7.10.12.3.3.3.1-3.

Table 7.10.12.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.12.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeRatConnectivitySub | M | 1 | 200 OK | Successful case. The requested "Individual UE RAT Connectivity Analytics Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.12.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.12.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

7.10.12.3.3.3.2	PUT

The HTTP PUT method allows a service consumer to request the update of an existing "Individual UE RAT Connectivity Analytics Subscription" resource at the ADAE Server.

This method shall support the URI query parameters specified in table 7.10.12.3.3.3.2-1.

Table 7.10.12.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.12.3.3.3.2-2 and the response data structures and response codes specified in table 7.10.12.3.3.3.2-3.

Table 7.10.12.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeRatConnectivitySub | M | 1 | Contains the updated representation of the "Individual UE RAT Connectivity Analytics Subscription" resource. |

Table 7.10.12.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeRatConnectivitySub | M | 1 | 200 OK |  | Successful case. The "Individual UE RAT Connectivity Analytics Subscription" resource is successfully updated, and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content |  | Successful case. The "Individual UE RAT Connectivity Analytics Subscription" resource is successfully updated, and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect |  | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |  |
| n/a |  |  | 308 Permanent Redirect |  | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.12.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string |  | M |  | 1 |  | Contains an alternative target URI of the resource located in an alternative ADAE Server. |  |

Table 7.10.12.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string |  | M |  | 1 |  | Contains an alternative target URI of the resource located in an alternative ADAE Server. |  |

7.10.12.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer to request the modification of an existing "Individual UE RAT Connectivity Analytics Subscription" resource at the ADAE Server.

This method shall support the URI query parameters specified in table 7.10.12.3.3.3.3-1.

Table 7.10.12.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.12.3.3.3.3-2 and the response data structures and response codes specified in table 7.10.12.3.3.3.3-3.

Table 7.10.12.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeRatConnectivitySubPatch | M | 1 | Contains the requested modifications to the "Individual UE RAT Connectivity Analytics Subscription" resource. |

Table 7.10.12.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeRatConnectivitySub | M | 1 | 200 OK |  | Successful case. The "Individual UE RAT Connectivity Analytics Subscription" resource is successfully modified and a representation of the updated resource is returned in the response body. |  |
| n/a |  |  | 204 No Content |  | Successful case. The "Individual UE RAT Connectivity Analytics Subscription" resource is successfully modified and no content is returned in the response body. |  |
| n/a |  |  | 307 Temporary Redirect |  | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |  |
| n/a |  |  | 308 Permanent Redirect |  | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.12.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String |  | M |  | 1 |  | Contains an alternative target URI of the resource located in an alternative ADAE Server. |  |

Table 7.10.12.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string |  | M |  | 1 |  | Contains an alternative target URI of the resource located in an alternative ADAE Server. |  |

7.10.12.3.3.3.4	DELETE

The HTTP DELETE method allows a service consumer to request the deletion of an existing "Individual UE RAT Connectivity Analytics Subscription" resource at the ADAE Server.

This method shall support the URI query parameters specified in table 7.10.12.3.3.3.4-1.

Table 7.10.12.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.12.3.3.3.4-2 and the response data structures and response codes specified in table 7.10.12.3.3.3.4-3.

Table 7.10.12.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |  |  |  |

Table 7.10.12.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual UE RAT Connectivity Analytics Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.12.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string |  | M |  | 1 |  | Contains an alternative target URI of the resource located in an alternative ADAE Server. |  |

Table 7.10.12.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative ADAE Server. |

###### 7.10.12.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 7.10.12.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

#### 7.10.12.5	Notifications

##### 7.10.12.5.1	General

Notifications shall comply to clause 6.6.

Table 7.10.12.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| UE RAT Connectivity Analytics Notification | {notifUri} | POST | Enables the ADAE Server to notify a previously subscribed service consumer on UE RAT connectivity analytics related event(s). |

##### 7.10.12.5.2	UE RAT Connectivity Analytics Notification

###### 7.10.12.5.2.1	Description

The UE RAT Connectivity Analytics Notification is used by the ADAE Server to to notify a previously subscribed service consumer on UE RAT connectivity analytics related event(s).

###### 7.10.12.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 7.10.12.5.2.2-1.

Table 7.10.12.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | Represents the callback URI encoded as a string formatted as a URI. |

###### 7.10.12.5.2.3	Standard Methods

7.10.12.5.2.3.1	POST

This method shall support the URI query parameters specified in table 7.10.12.5.2.3.1-1.

Table 7.10.12.5.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.12.5.2.3.1-2 and the response data structures and response codes specified in table 7.10.12.5.2.3.1-3.

Table 7.10.12.5.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeRatConnectivityNotif | M |  | 1 |  | Contains the UE RAT Connectivity Analytics Notification. |  |

Table 7.10.12.5.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  | 204 No Content |  | Successful case. The UE RAT Connectivity Analytics Notification is successfully received and acknowledged. |  |
| n/a |  |  |  |  | 307 Temporary Redirect |  | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |  |
| n/a |  |  |  |  | 308 Permanent Redirect |  | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |  |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.12.5.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string |  | M |  | 1 |  | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |  |

Table 7.10.12.5.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string |  | M |  | 1 |  | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |  |

#### 7.10.12.6	Data Model

##### 7.10.12.6.1	General

This clause specifies the application data model supported by the API.

Table 7.10.12.6.1-1 specifies the data types defined for the SS_ADAE_UeRatConnectivityAnalytics API.

Table 7.10.12.6.1-1: SS_ADAE_UeRatConnectivityAnalytics API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AnalyticsIdUeRat | 7.10.12.6.3.3 | Represents the UE RAT connectivity analytics identifier. |  |
| AreaPrediction | 7.10.12.6.2.5 | Represents the RAT type changes for a given way point. |  |
| TimeWindowPrediction | 7.10.12.6.2.6 | Represents the time periods for predictions. |  |
| UeRatAnalyticsSub | 7.10.12.6.2.7 | Represents the UE RAT Connectivity Analytics Subscription related information. |  |
| UeRatAnalyticsOutput | 7.10.12.6.2.8 | Represents the UE RAT Connectivity Analytics output information. |  |
| UeRatConnectivityNotif | 7.10.12.6.2.4 | Represents the UE RAT Connectivity Analytics Notification. |  |
| UeRatConnectivitySub | 7.10.12.6.2.2 | Represents the UE RAT Connectivity Analytics Subscription. |  |
| UeRatConnectivitySubPatch | 7.10.12.6.2.3 | Represents the requested modifications to a UE RAT Connectivity Analytics Subscription. |  |

Table 7.10.12.6.1-2 specifies data types re-used by the SS_ADAE_UeRatConnectivityAnalytics API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SS_ADAE_UeRatConnectivityAnalytics API.

Table 7.10.12.6.1-2: SS_ADAE_UeRatConnectivityAnalytics API Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.2.6 | Represents the type of analytics. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents the user location information. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Represents the reporting requirements information. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Represents a time window. |  |
| UeMobility | 3GPP TS 29.520 [33] | Represents the UE Mobility and route information. |  |
| Uri | 3GPP TS 29.122 [3] | Represent a URI. |  |

##### 7.10.12.6.2	Structured data types

###### 7.10.12.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.10.12.6.2.2	Type: UeRatConnectivitySub

Table 7.10.12.6.2.2-1: Definition of type UeRatConnectivitySub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsSubs | array(UeRatAnalyticsSub) | M | 1..N | Contains the requested analytics details. |  |
| notifUri | Uri | M | 1 | Contains the notification URI. |  |
| report | UeRatConnectivityNotif | C | 0..1 | Contains the reporting data for immediate reporting. This attribute shall be present only if immediate reporting is requested via the "reportReq" attribute, and the report(s) are available. |  |
| reportReq | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| timeInterval | TimeWindow | O | 0..1 | Contains the time interval within which the UE RAT Connectivity Analytics Subscription applies. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 7.10.12.8. This attribute shall be present only if feature negotiation is required. |  |

###### 7.10.12.6.2.3	Type: UeRatConnectivitySubPatch

Table 7.10.12.6.2.3-1: Definition of type UeRatConnectivitySubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsSubs | array(UeRatAnalyticsSub) | O | 1..N | Contains the requested analytics information to be modified. |  |
| notifUri | Uri | O | 0..1 | Contains the updated notification URI. |  |
| reportReq | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| timeInterval | TimeWindow | O | 0..1 | Contains the updated time interval within which the UE RAT Connectivity Analytics Subscription applies. |  |

###### 7.10.12.6.2.4	Type: UeRatConnectivityNotif

Table 7.10.12.6.2.4-1: Definition of type UeRatConnectivityNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subId | string | M | 1 | Contains the identifier of the UE RAT Connectivity Analytics Subscription to which the notification is related. |  |
| analyticsOut | array(UeRatAnalyticsOutput) | M | 1..N | Contains the analytics output report(s). |  |

###### 7.10.12.6.2.5	Type: AreaPrediction

Table 7.10.12.6.2.5-1: Definition of type AreaPrediction

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| area | LocationArea5G | M | 0..1 | Contains the set of location co-ordinates or waypoints for which the analytics output is applicable. |  |
| timePer | TimeWindow | M | 0..1 | Contains the time period within which the analytics apply. |  |
| confLevel | Uinteger | C | 0..1 | Contains the confidence level of the prediction. Minimum = 0. Maximum = 100. This attribute shall be present only if the "analyticsType" attribute within the corresponding UE RAT Connectivity Analytics Subscription is set to "PREDICTIVE". |  |

###### 7.10.12.6.2.6	Type: TimeWindowPrediction

Table 7.10.12.6.2.6-1: Definition of type TimeWindowPrediction

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timePer | TimeWindow | M | 0..1 | Contains the time period within which the analytics apply. |  |
| confLevel | Uinteger | C | 0..1 | Contains the confidence level of the prediction. Minimum = 0. Maximum = 100. This attribute shall be present only if the "analyticsType" attribute within the corresponding UE RAT Connectivity Analytics Subscription is set to "PREDICTIVE". |  |

###### 7.10.12.6.2.7	Type: UeRatAnalyticsSub

Table 7.10.12.6.2.7-1: Definition of type UeRatAnalyticsSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsIds | array(AnalyticsIdUeRat) | M | 1..N | Contains the requested analytics ID(s). |  |
| valUeIds | array(string) | C | 1..N | Contains the list of the identitifier(s) of the VAL UE(s) to which the subscription is related. (NOTE) |  |
| valGroupId | string | C | 0..1 | Contains the identifier of the VAL group to which the subscription is related. (NOTE) |  |
| ueMobs | map(UeMobility) | O | 1..N | Contains the mobility and route information of the target VAL UE(s). The key of the map shall be set to the value of the identifier of the VAL UE (among either the ones provided within the "valUeIds" attribute or the members of VAL group identified by the "valGroupId" attribute) to which the mobility and route information provided within the map value is related. |  |
| valSvcId | string | O | 0..1 | Contains the identifier of the VAL service for which the subscription is related. |  |
| confLevel | integer | O | 0..1 | Contains the preferred accuracy level for the UE RAT connectivity analytics in case of prediction. Minimum = 0. Maximum = 100. |  |
| area | LocationArea5G | O | 0..1 | Contains the area within which the subscription applies. |  |
| NOTE: These attributes are mutually exclusive and only one of them shall be present. |  |  |  |  |  |

###### 7.10.12.6.2.8	Type: UeRatAnalyticsOutput

Table 7.10.12.6.2.8-1: Definition of type UeRatAnalyticsOutput

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsId | array(AnalyticsIdUeRat) | M | 1..N | Contains the reported analytics ID(s). |  |
| areaApp | AreaPrediction | O | 0..1 | Contains a predicted or expected RAT type changes for the given waypoints. |  |
| timeApp | TimeWindowPrediction | O | 0..1 | Contains a predicted or expected RAT type changes for the given time period. |  |
| valUeIds | array(string) | M | 1..N | Contains a list of the identitifiers of the VAL UE(s) to which the reported anaylytics output is related. |  |
| valSvcId | string | O | 0..1 | Contains the identifier of the VAL service for which the the reported anaylytics output is related. |  |

##### 7.10.12.6.3	Simple data types and enumerations

###### 7.10.12.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.12.6.3.2	Simple data types

The simple data types defined in table 7.10.12.6.3.2-1 shall be supported.

Table 7.10.12.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

###### 7.10.12.6.3.3	Enumeration: AnalyticsUeIdRat

The enumeration AnalyticsIdUeRat represents the analytic IDs for Collision Detection Analytics. It shall comply with the provisions defined in table 7.10.12.6.3.3-1.

Table 7.10.12.6.3.3-1: Enumeration AnalyticsIdUeRat

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE_RAT_CONNECTIVITY_ANALYTICS | Indicates that the requested analytics is UE RAT connectivity analytics event. |  |

##### 7.10.12.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types and combinations of data types in this release of the specification.

##### 7.10.12.6.5	Binary data

7.10.12.6.5.1	Binary Data Types

Table 7.10.12.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.10.12.7	Error Handling

##### 7.10.12.7.1	General

For the SS_ADAE_UeRatConnectivityAnalytics API, error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses are applicable for the SS_ADAE_UeRatConnectivityAnalytics API.

##### 7.10.12.7.2	Protocol Errors

No specific protocol errors for the SS_ADAE_UeRatConnectivityAnalytics API are specified.

##### 7.10.12.7.3	Application Errors

The application errors defined for SS_ADAE_UeRatConnectivityAnalytics API are listed in table 7.10.12.7.3-1.

Table 7.10.12.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.12.8	Feature Negotiation

The optional features in table 7.10.12.8-1 are defined for the SS_ADAE_UeRatConnectivityAnalytics API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.10.12.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 7.10.12.9	Security

The provisions of clause 9 shall apply for the SS_ADAE_UeRatConnectivityAnalytics API.

### 7.10.13	SS_ADCCF_DataCollection

#### 7.10.13.1	Introduction

The A-DCCF Data Collection service shall use the SS_ADCCF_DataCollection API.

The API URI of the SS_ADCCF_DataCollection API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-adccf-dc".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5.

NOTE:	When 3GPP TS 29.122 [3] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.5, the A-DCCF takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 7.10.13.2	Usage of HTTP

The provisions of clause 6.3 shall apply for the SS_ADCCF_DataCollection API.

#### 7.10.13.3	Resources

##### 7.10.13.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.13.3.1-1 depicts the resource URIs structure for the SS_ADCCF_DataCollection.

Figure 7.10.13.3.1-1: Resource URI structure of the SS_ADCCF_DataCollection

Table 7.10.13.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.13.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Data Collection Subscriptions | /subscriptions | POST | Create an "Individual Data Collection Subscription" resource. |
| Data Collection Subscription | /subscriptions/{subscriptionId} | GET | Read the "Individual Data Collection Subscription" resource. |
|  |  | DELETE | Remove the "Individual Data Collection Subscription" resource. |

Editor's note:	PUT and PATCH methods are FFS.

##### 7.10.13.3.2	Resource: Data Collection Subscriptions

###### 7.10.13.3.2.1	Description

The Data Collection Subscriptions to the event of the location-related UE group analytics.

###### 7.10.13.3.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adccf-dc/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 7.10.13.3.2.2-1.

Table 7.10.13.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |

###### 7.10.13.3.2.3	Resource Standard Methods

7.10.13.3.2.3.1	POST

This method to subscribe to the event of the location-related UE group analytics and shall support the URI query parameters specified in table 7.10.13.3.2.3.1-1.

Table 7.10.13.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.13.3.2.3.1-2 and the response data structures and response codes specified in table 7.10.13.3.2.3.1-3.

Table 7.10.13.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataCollectionSub | M | 1 | Subscription to the location-related UE group analytics. |

Table 7.10.13.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataCollectionSub | M | 1 | 201 Created | The "Individual Data Collection Subscription" resource is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.13.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adccf-dc/<apiVersion>/subscriptions/{subscriptionId} |

###### 7.10.13.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this API in this release of the specification.

##### 7.10.13.3.3	Resource: Individual Data Collection Subscription

###### 7.10.13.3.3.1	Description

###### 7.10.13.3.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adccf-dc/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 7.10.13.3.3.2-1.

Table 7.10.13.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5. |
| subscriptionId | string | Represents the identifier of an "Individual Data Collection Subscription" resource. |

###### 7.10.13.3.3.3	Resource Standard Methods

7.10.13.3.3.3.1	GET

This operation reads the "Individual Data Collection Subscription" resource. This method shall support the URI query parameters specified in table 7.10.13.3.3.3.1-1.

Table 7.10.13.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.10.13.3.3.3.1-2 and the response data structures and response codes specified in table 7.10.13.3.3.3.1-3.

Table 7.10.13.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.13.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataCollectionSub | M | 1 | 200 OK | The requested "Individual Data Collection Subscription" resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative A-DCCF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative A-DCCF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.13.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative A-DCCF. |

Table 7.10.13.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative A-DCCF. |

7.10.13.3.3.3.2	DELETE

This operation deletes "Individual Data Collection Subscription" resource. This method shall support the URI query parameters specified in table 7.10.13.3.3.3.2-1.

Table 7.10.13.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.13.3.3.3.2-2 and the response data structures and response codes specified in table 7.10.13.3.3.3.2-3.

Table 7.10.13.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.13.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The "Individual Data Collection Subscription" resource matching the subscriptionId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative A-DCCF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative A-DCCF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.10.13.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative A-DCCF. |

Table 7.10.13.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative A-DCCF. |

###### 7.10.13.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this API in this release of the specification.

#### 7.10.13.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

#### 7.10.13.5	Notifications

##### 7.10.13.5.1	General

##### Notifications shall comply to clause 5.2.5 of 3GPP TS 29.122 [2].

Table 7.10.13.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Data Collection Notification | {notifUri} | POST | Notification on location-related UE group analytics. |

##### 7.10.13.5.2	Data Collection Notification

###### 7.10.13.5.2.1	Description

Data Collection Notification is to notify on the event of location-related UE group analytics.

###### 7.10.13.5.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notifUri}

This method shall support the URI query parameters specified in table 7.10.13.5.2.2-1.

Table 7.10.13.5.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.13.5.2.2-2 and the response data structures and response codes specified in table 7.10.13.5.2.2-3.

Table 7.10.13.5.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataCollectionNotif | M | 1 | Notification information of data collection events. |

Table 7.10.13.5.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Notification for the data collection event is accepted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during notification. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.13.5.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

Table 7.10.13.5.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative notification destination towards which the notification should be redirected. |

#### 7.10.13.6	Data Model

##### 7.10.13.6.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.13.6.1-1 specifies the data types defined specifically for the SS_ADCCF_DataCollection service.

Table 7.10.13.6.1-1: SS_ADCCF_DataCollection specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AdccfDataType | 7.10.13.6.2.6 | Represents type of A-DCCF data. |  |
| DataCollectionNotif | 7.10.13.6.2.3 | Represents the Data Collection Notification. |  |
| DataCollectionReq | 7.10.13.6.2.4 | Represents the data collection requirements. |  |
| DataCollectionSub | 7.10.13.6.2.2 | Represents the Data Collection Subscription. |  |
| DataOutput | 7.10.13.6.2.8 | Represents the A-DCCF data output. |  |
| DataType | 7.10.13.6.3.3 | Represents the type of data. |  |
| GranularityType | 7.10.13.6.3.4 | Represents the granularity type. |  |
| ProcessingReq | 7.10.13.6.2.5 | Represents the data processing requirements. |  |
| StorageRequirements | 7.10.13.6.2.7 | Represents the data storage requirements. |  |

Table 7.10.13.6.1-2 specifies data types re-used by the SS_ADCCF_DataCollection service:

Table 7.10.13.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.2.6 | Represents the type of analytics. |  |
| DataProducerProfile | Clause 7.10.8.5.2.12 | Represents the data producer profile. |  |
| DurationSec | 3GPP TS 29.122 [3] | Represents a period of time in units of seconds. |  |
| dataFormat | 3GPP TS 29.222 [16] | Represents the data format. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Used to negotiate the supported optional features of the API. |  |
| Uri | 3GPP TS 29.122 [3] | Indicate the notification URI. |  |

##### 7.10.13.6.2	Structured data types

###### 7.10.13.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.10.13.6.2.2	Type: DataCollectionSub

Table 7.10.13.6.2.2-1: Definition of type DataCollectionSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataColEvnId | string | M | 1 | Represents data collection event identifier. |  |
| dataColReq | DataCollectionReq | M | 1 | Represents data collection requirements. |  |
| repReq | ReportingInformation | O | 0..1 | Represents the reporting requirements of the subscription. |  |
| analyticsId | string | O | 0..1 | Represents the analytics ID. |  |
| dataProdIds | array(string) | O | 0..1 | Represents list of data producer identifier(s). |  |
| profiles | array(DataProducerProfile) | C | 1..N | Represent the data producer profile criteria. This attribute shall be provided if "dataProdId" is provided. |  |
| area | LocationArea5G | O | 0..1 | Represents the location information for the data producers subscription requests. |  |
| duration | DurationSec | O | 0..1 | Represents the time validity for the data producers subscription requests. |  |
| procReq | ProcessingReq | O | 0..1 | Represents the processing requirements. |  |
| storageReqs | StorageRequirements | O | 1..N | Represents the strorage requirements. |  |
| notifUri | Uri | M | 1 | Represents the notification URI. |  |
| report | DataCollectionNotif | C | 0..1 | Represents the immediate report. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation needs to take place. |  |

Editor's note:	How to support one-time immediate report is FFS.

###### 7.10.13.6.2.3	Type: DataCollectionNotif

Table 7.10.13.6.2.3-1: Definition of type DataCollectionNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataColEvnId | string | M | 1 | Represents data collection event identifier. |  |
| analyticsId | string | O | 0..1 | Represents the analytics ID. |  |
| analyticsType | AnalyticsType | O | 0..1 | Represents the analytics type. |  |
| dataType | AdccfDataType | O | 0..1 | Represents the type of data |  |
| dataOutput | DataOutput | M | 1 | Represents the data output. |  |
| procReq | ProcessingReq | O | 0..1 | Represents the processing requirements. |  |
| storageReqs | array(string) | O | 1..N | Represents the list of A-ADRF ID(s) for data storage. |  |

###### 7.10.13.6.2.4	DataCollectionReq

Table 7.10.13.6.2.4-1: Definition of type DataCollectionReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataFormat | DataFormat | M | 1 | Represents the data format. |  |
| repFreq | DurationSec | M | 1 | Represents the reporting frequency. |  |
| abstraction | integer | M | 1 | Represents the level of abstraction. Minimum = 0. Maximum = 100. |  |
| accuracy | integer | M | 1 | Represents the level of accuracy. Minimum = 0. Maximum = 100. |  |

###### 7.10.13.6.2.5	Type: ProcessingReq

Table 7.10.13.6.2.5-1: Definition of type ProcessingReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| req | string | C | 0..1 | Indicates the data processing requirements. (NOTE) |  |
| NOTE: At least one of these attributes shall be provided. |  |  |  |  |  |

###### 7.10.13.6.2.6	Type: AdccfDataType

Table 7.10.13.6.2.6-1: Definition of type AdccfDataType

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| type | DataType | C | 0..1 | Represents the type of data. (NOTE) |  |
| gran | GranularityType | C | 0..1 | Represents the data granularity type. (NOTE) |  |
| NOTE: These attributes are mutually exclusive and only one of them shall be present. |  |  |  |  |  |

###### 7.10.13.6.2.7	Type: StorageRequirements

Table 7.10.13.6.2.7-1: Definition of type StorageRequirements

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| storageReqs | array(string) | M | 1..N | Represents the list of A-ADRF ID(s) for data storage. |  |

###### 7.10.13.6.2.8	Type: DataOutput

Table 7.10.13.6.2.8-1: Definition of type DataOutput

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataOutput | string | M | 1 | Represents the data output. |  |

##### 7.10.13.6.3	Simple data types and enumerations

###### 7.10.13.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.13.6.3.2	Simple data types

The simple data types defined in table 7.10.13.6.3.2-1 shall be supported.

Table 7.10.13.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

###### 7.10.13.6.3.3	Enumeration: DataType

The enumeration DataType represents the types of data samples. It shall comply with the provisions defined in table 7.10.13.6.3.3-1.

Table 7.10.13.6.3.3-1: Enumeration DataType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| UE_DATA | Indicates that the data sample related to the UE data. |  |
| NETWORK_DATA | Indicates that the data sample related to the network data. |  |
| APP_DATA | Indicates that the data sample related to the application data. |  |
| EDGE_DATA | Indicates that the data sample related to the edge data. |  |

###### 7.10.13.6.3.4	Enumeration: GranularityType

The enumeration GranularityType represents the types of data granularity. It shall comply with the provisions defined in table 7.10.13.6.3.4-1.

Table 7.10.13.6.3.4-1: Enumeration GranularityType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REAL_TIME | Indicates that the data granularity is real time. |  |
| NON_REAL_TIME | Indicates that the data granularity is non-real time. |  |

##### 7.10.13.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 7.10.13.6.5	Binary data

###### 7.10.13.6.5.1	Binary Data Types

Table 7.10.13.6.5.1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.10.13.7	Error Handling

##### 7.10.13.7.1	General

For the SS_ADCCF_DataCollection API, error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses are applicable for the SS_ADCCF_DataCollection API.

##### 7.10.13.7.2	Protocol Errors

No specific protocol errors for the SS_ADCCF_DataCollection API are specified.

##### 7.10.13.7.3	Application Errors

The application errors defined for SS_ADCCF_DataCollection are listed in table 7.10.13.7.3-1.

Table 7.10.13.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.13.8	Feature Negotiation

The optional features in table 7.10.13.8-1 are defined for the SS_ADCCF_DataCollection API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.10.13.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 7.10.13.9	Security

The provisions of clause 9 shall apply for the SS_ADCCF_DataCollection API.

### 7.10.14	SS_ADAE_ServerToServerPerformanceAnalytics API

#### 7.10.14.1	Introduction

The ADAE Server to Server Performance Analytics service shall use the SS_ADAE_ServerToServerPerformanceAnalytics API.

The API URI of the SS_ADAE_ServerToServerPerformanceAnalytics API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5, i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5.

-	The <apiName> shall be "ss-adae-sspa".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5.

NOTE:	When 3GPP TS 29.122 [3] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.5, the ADAE Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 7.10.14.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [15] shall apply for the SS_ADAE_ServerToServerPerformanceAnalytics API.

#### 7.10.14.3	Resources

##### 7.10.14.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.10.14.3.1-1 depicts the resource URIs structure for the SS_ADAE_ServerToServerPerformanceAnalytics.

Figure 7.10.14.3.1-1: Resource URI structure of the SS_ADAE_ServerToServerPerformanceAnalytics

Table 7.10.14.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.10.14.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Server to Server Performance Analytics Subscriptions | /subscriptions | POST | Create an "Individual Server to Server Performance Analytics Subscription" resource. |
| Individual Server to Server Performance Analytics Subscription | /subscriptions/{subscriptionId} | GET | Retrieve the "Individual Server to Server Performance Analytics Subscription" resource. |
|  |  | PUT | Update the "Individual Server to Server Performance Analytics Subscription" resource. |
|  |  | PATCH | Modify the "Individual Server to Server Performance Analytics Subscription" resource. |
|  |  | DELETE | Delete the "Individual Server to Server Performance Analytics Subscription" resource. |

##### 7.10.14.3.2	Resource: Server to Server Performance Analytics Subscriptions

###### 7.10.14.3.2.1	Description

The "Server to Server Performance Analytics Subscriptions" resource collection represents Server to Server Performance analytics subscriptions.

###### 7.10.14.3.2.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-sspa/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 7.10.14.3.2.2-1.

Table 7.10.14.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.10.14.1. |

###### 7.10.14.3.2.3	Resource Standard Methods

7.10.14.3.2.3.1	POST

The HTTP POST method allows a service consumer to request the creation of a Server to Server Performance Analytics Subscription at the ADAE Server.

This method shall support the URI query parameters specified in table 7.10.14.3.2.3.1-1.

Table 7.10.14.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.14.3.2.3.1-2 and the response data structure and response codes specified in table 7.10.14.3.2.3.1-3.

Table 7.10.14.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| S2SPerfSubsc | M | 1 | Contains the parameters to request the creation of a Server to Server Performance Analytics Subscription. |

Table 7.10.14.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| S2SPerfSubsc | M | 1 | 201 Created | Successful case. The Server to Server Performance Analytics Subscription is successfully created and a representation of the created "Individual Server to Server Performance Analytics Subscription" resource shall be returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.14.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-adae-sspa/<apiVersion>/subscriptions/{subscriptionId} |

###### 7.10.14.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

##### 7.10.14.3.3	Resource: Individual Server to Server Performance Analytics Subscription

###### 7.10.14.3.3.1	Description

###### 7.10.14.3.3.2	Resource Definition

Resource URI: {apiRoot}/ss-adae-sspa/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 7.10.14.3.3.2-1.

Table 7.10.14.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.10.14.1. |
| subscriptionId | string | Represents the identifier of the "Individual Server to Server Performance Analytics Subscription" resource. |

###### 7.10.14.3.3.3	Resource Standard Methods

7.10.14.3.3.3.1	GET

The HTTP GET method allows a service consumer to retrieve an existing "Individual Server to Server Performance Analytics Subscription" resource at the ADAE Server.

This method shall support the URI query parameters specified in table 7.10.14.3.3.3.1-1.

Table 7.10.14.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 7.10.14.3.3.3.1-2 and the response data structures and response codes specified in table 7.10.14.3.3.3.1-3.

Table 7.10.14.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.14.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| S2SPerfSubsc | M | 1 | 200 OK | Successful case. The requested "Individual Server to Server Performance Analytics Subscription" resource is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.14.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE server. |

Table 7.10.14.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE server. |

7.10.14.3.3.3.2	PUT

This method enables to update an existing "Individual Server to Server Performance Analytics Subscription" resource at the ADAE Server.

This method shall support the URI query parameters specified in table 7.10.14.3.3.3.2-1.

Table 7.10.14.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.14.3.3.3.2-2 and the response data structures and response codes specified in table 7.10.14.3.3.3.2-3.

Table 7.10.14.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| S2SPerfSubsc | M | 1 | Contains the updated representation of the "Individual Server to Server Performance Analytics Subscription" resource. |

Table 7.10.14.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| S2SPerfSubsc | M | 1 | 200 OK | Successful case. The "Individual Server to Server Performance Analytics Subscription" resource is successfully updated, and the representation of the updated resource is returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Server to Server Performance Analytics Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.14.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.14.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

7.10.14.3.3.3.3	PATCH

This method modifies an existing "Individual Server to Server Performance Analytics Subscription" resource at the ADAE Server.

This method shall support the URI query parameters specified in table 7.10.14.3.3.3.3-1.

Table 7.10.14.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.14.3.3.3.3-2 and the response data structures and response codes specified in table 7.10.14.3.3.3.3-3.

Table 7.10.14.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| S2SPerfSubscPatch | M | 1 | Contains the requested modifications to the "Individual Server to Server Performance Analytics Subscription" resource. |

Table 7.10.14.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| S2SPerfSubsc | M | 1 | 200 OK | Successful case. The "Individual Server to Server Performance Analytics Subscription" resource is successfully modified, and a representation of the updated resource is returned in the response. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Server to Server Performance Analytics Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAES Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.14.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.14.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

7.10.14.3.3.3.4	DELETE

This method enables to delete an existing "Individual Server to Server Performance Analytics Subscription" resource at the ADAE Server.

This method shall support the URI query parameters specified in table 7.10.14.3.3.3.4-1.

Table 7.10.14.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.14.3.3.3.4-2 and the response data structures and response codes specified in table 7.10.14.3.3.3.4-3.

Table 7.10.14.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.10.14.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Server to Server Performance Analytics Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative ADAE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.14.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

Table 7.10.14.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative ADAE Server. |

###### 7.10.14.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 7.10.14.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

#### 7.10.14.5	Notifications

##### 7.10.14.5.1	General

Table 7.10.14.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Server to Server Performance Analytics Notification | {notifUri} | POST | Enables the ADAE Server to notify a previously subscribed service consumer on Server to Server Performance Analytics related event(s). |

##### 7.10.14.5.2	Server to Server Performance analytics Notification

###### 7.10.14.5.2.1	Description

Server to Server Performance Analytics Notification is used by the ADAE Server to notify a previously subscribed service consumer on Server to Server Performance Analytics related event(s).

###### 7.10.14.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 7.10.14.5.2.2-1.

Table 7.10.14.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | Represents the callback URI encoded as a string formatted as a URI. |

###### 7.10.14.5.2.3	Standard Methods

The HTTP POST method shall be used for delivering the Server to Server Performance Analytics Notification.

This method shall support the URI query parameters specified in table 7.10.14.5.2.3-1.

Table 7.10.14.5.2.3-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.10.14.5.2.3-2 and the response data structures and response codes specified in table 7.10.14.5.2.3-3.

Table 7.10.14.5.2.3-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| S2SPerfNotif | M | 1 | Contains the Server to Server Performance Analytics Notification. |

Table 7.10.14.5.2.3-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Server to Server Performance Analytics Notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative notification destination towards which the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.7.1-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 7.10.14.5.2.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 7.10.14.5.2.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

#### 7.10.14.6	Data Model

##### 7.10.14.6.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 6.2 apply to this API.

Table 7.10.14.6.1-1 specifies the data types defined specifically for the SS_ADAE_Server2ServerPerformanceAnalytics service.

Table 7.10.14.6.1-1: SS_ADAE_Server2ServerPerformanceAnalytics specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| S2SPerfAnalyticsData | 7.10.14.6.2.5 | Represents the Server to Server Performance Analytics data. |  |
| S2SPerfNotif | 7.10.14.6.2.3 | Represents the Server to Server Performance Analytics Notification. |  |
| S2SPerfSubsc | 7.10.14.6.2.2 | Represents the Server to Server Performance Analytics Subscription. |  |
| S2SPerfSubscPatch | 7.10.14.6.2.4 | Represents the requested modifications to a Server to Server Performance Analytics Subscription. |  |
| AnalyticsIdSspa | 7.10.14.6.3.3 | Represents the Analytics ID for Server to Server Performance Analytics. |  |
| ServerInfo | 7.10.14.6.2.6 | Represents the Server Information. |  |

Table 7.10.14.6.1-2 specifies data types re-used by the SS_ADAE_Server2ServerPerformanceAnalytics service:

Table 7.10.14.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AnalyticsType | Clause 7.10.1.4.2.6 | Represents the type of the analytics. |  |
| IpAddr | 3GPP TS 29.571 [8] | Represents an IP Address. |  |
| LocationArea5G | 3GPP TS 29.122 [3] | Represents location information. |  |
| ReportingInformation | 3GPP TS 29.523 [20] | Indicates the reporting requirements. |  |
| SupportedFeatures | 3GPP TS 29.571 [21] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| TimeWindow | 3GPP TS 29.122 [3] | Indicate the time window. |  |
| U2UAnalyticsData | 7.10.3.4.2.6 | Represents the UE-to-UE analytics data. |  |
| Uri | 3GPP TS 29.122 [3] | Indicates a URI. |  |

##### 7.10.14.6.2	Structured data types

###### 7.10.14.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 7.10.14.6.2.2	Type: S2SPerfSubsc

Table 7.10.14.6.2.2-1: Definition of type S2SPerfSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsIds | array(AnalyticsIdSspa) | O | 1..N | Contains the requested analytics ID(s). |  |
| analyticsType | AnalyticsType | M | 1 | Contains the type of the Server to Server Performance analytics. |  |
| serverInfolist | array(ServerInfo) | M | 1..N | Contains the list of the identifier(s) of the server for which the analytics subscription applies. |  |
| valServId | string | O | 0..1 | Contains the identifier of the VAL service for which the subscription applies. |  |
| repReq | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| area | LocationArea5G | O | 0..1 | Contains the area within which the subscription applies. |  |
| confLevel | integer | O | 0..1 | Contains the preferred confidence level of the prediction. This attribute may be provided only if the "analyticsType" attribute in the request is set to "PREDICTIVE". Minimum = 0. Maximum = 100. |  |
| notifUri | Uri | M | 1 | Contains the notification URI. |  |
| timeVal | TimeWindow | O | 0..1 | Contains the time validity within which the subscription applies. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 7.10.14.8.This attribute shall be present only when feature negotiation needs to take place. |  |

###### 7.10.14.6.2.3	Type: S2SPerfNotif

Table 7.10.14.6.2.3-1: Definition of type S2SPerfNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticsIds | array(AnalyticsIdSspa) | O | 1..N | Contains the analytics ID(s). |  |
| analyticsOutput | S2SPerfAnalyticsData) | C | 0..1 | Contains the Server-to-Server Performance Analytics data. This attribute shall be present only if one of the reported analytics IDs within the "analyticsId" attribute is "SERVER_TO_SERVER_PERFORMANCE". |  |
| confLevel | integer | O | 0..1 | Indicates the achieved confidence level for prediction. This attribute may be provided only if the "analyticsType" attribute in the request is set to "PREDICTIVE". Minimum = 0. Maximum = 100. |  |

###### 7.10.14.6.2.4	Type: S2SPerfSubscPatch

Table 7.10.14.6.2.4-1: Definition of type S2SPerfSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serverInfolist | array(ServerInfo) | O | 1..N | Contains the list of the identifier(s) of the server for which the analytics subscription applies. |  |
| valServId | string | O | 0..1 | Contains the identifier of the VAL service for which the subscription applies. |  |
| repReq | ReportingInformation | O | 0..1 | Contains the reporting requirements of the subscription. |  |
| area | LocationArea5G | O | 0..1 | Contains the area within which the subscription applies. |  |
| confLevel | integer | O | 0..1 | Contains the preferred confidence level of the prediction. |  |
| notifUri | Uri | O | 0..1 | Contains the notification URI. |  |
| timeVal | TimeWindow | O | 0..1 | Contains the time interval within which the subscription applies. |  |

###### 7.10.14.6.2.5	S2SPerfAnalyticsData

Table 7.10.14.6.2.5-1: Definition of type S2SPerfAnalyticsData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| performChgInd | boolean | C | 0..1 | Indicates whether a server-to-server session performance has changed. When present, it shall be set as follows: - "true": the performance has changed. - "false": the performance is not changed. - Default value when omitted is "false". |  |
| timHorizOverInd | boolean | C | 0..1 | Indicates whether a server-to-server session performance sustainability has over a given time horizon. When present, it shall be set as follows: - "true": the performance sustainability has over the given time horizon. - "false": the performance sustainability has not over the given time horizon. - Default value when omitted is "false". |  |
| areaOverInd | boolean | C | 0..1 | Indicates whether a server-to-server session performance sustainability has over in a given area. When present, it shall be set as follows: - "true": the performance sustainability has over the given area. - "false": the performance sustainability has not over the given area. - Default value when omitted is "false". |  |
| qoSMeasureRslt | U2UAnalyticsData | C | 0..1 | Represents the QoS measurement result for the Server to Server Performance between any VAL UEs within the given area. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

Editor's Note: The contents of the S2SPerfAnalyticsData data type is FFS.

###### 7.10.14.6.2.6	Type: ServerInfo

Table 7.10.14.6.2.6-1: Definition of type ServerInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serverId | string | C | 0..1 | Represents the lserver identifier for which the analytics subscription applies. (NOTE) |  |
| serverIpAddList | array(IpAddr) | C | 1..N | Represents the list of server IP Address(es) for which the analytics subscription applies. (NOTE) |  |
| NOTE: At least one of the attributes shall be included. |  |  |  |  |  |

##### 7.10.14.6.3	Simple data types and enumerations

###### 7.10.14.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

###### 7.10.14.6.3.2	Simple data types

The simple data types defined in table 7.10.14.6.3.2-1 shall be supported.

Table 7.10.14.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

###### 7.10.14.6.3.3	Enumeration: AnalyticsIdSspa

The enumeration AnalyticsIdSspa represents the analytic ID for Server-to-Server Performance Analytics. It shall comply with the provisions defined in table 7.10.14.6.3.3-1.

Table 7.10.14.6.3.3-1: Enumeration AnalyticsIdSspa

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SERVER_TO_SERVER_PERFORMANCE | Indicates that the requested analytics is Server-to-Server Performance Analytics. |  |

##### 7.10.14.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 7.10.14.6.5	Binary data

###### 7.10.14.6.5.1	Binary Data Types

Table 7.10.14.6.5.1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

#### 7.10.14.7	Error Handling

##### 7.10.14.7.1	General

For the SS_ADAE_Server2ServerPerformanceAnalytics API, error handling shall be supported as specified in clause 6.7.

In addition, the requirements in the following clauses are applicable for the SS_ADAE_Server2ServerPerformanceAnalytics API.

##### 7.10.14.7.2	Protocol Errors

No specific protocol errors for the SS_ADAE_Server2ServerPerformanceAnalytics API are specified.

##### 7.10.14.7.3	Application Errors

The application errors defined for SS_ADAE_Server2ServerPerformanceAnalytics are listed in table 7.10.14.7.3-1.

Table 7.10.14.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

#### 7.10.14.8	Feature Negotiation

The optional features in table 7.10.14.8-1 are defined for the SS_ADAE_Server2ServerPerformanceAnalytics API. They shall be negotiated using the extensibility mechanism defined in clause 6.8.

Table 7.10.14.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

#### 7.10.14.9	Security

The provisions of clause 9 shall apply for the SS_ADAE_Server2ServerPerformanceAnalytics API.

## 7.11	AIML Enablement APIs

AIML Enablement APIs are defined in 3GPP TS 29.482 [48].

## 7.12	Metaverse Enablement APIs

Spatial Anchor Server and Spatial Map Server APIs are defined in 3GPP TS 29.437 [49].

# 8	Using Common API Framework

## 8.1	General

When CAPIF is used with a SEAL service, the SEAL server shall support the following as defined in 3GPP TS 29.222 [16]:

-	the API exposing function and related APIs over CAPIF-2/2e and CAPIF-3/3e reference points;

-	the API publishing function and related APIs over CAPIF-4/4e reference point;

-	the API management function and related APIs over CAPIF-5/5e reference point; and

-	at least one of the security methods for authentication and authorization, and related security mechanisms.

In a centralized deployment as defined in 3GPP TS 23.222 [17], where the CAPIF core function and API provider domain functions are co-located, the interactions between the CAPIF core function and API provider domain functions may be independent of CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e reference points.

When CAPIF is used with a SEAL service, the SEAL server shall register all the features for northbound APIs in the CAPIF Core Function.

## 8.2	Security

When CAPIF is used for external exposure, before invoking the API exposed by the SEAL server, the VAL server as API invoker shall negotiate the security method (PKI, TLS-PSK or OAUTH2) with CAPIF core function and ensure the SEAL server has enough credential to authenticate the VAL server (see 3GPP TS 29.222 [16], clause 5.6.2.2 and clause 6.2.2.2).

If PKI or TLS-PSK is used as the selected security method between the VAL server and the SEAL server, upon API invocation, the SEAL server shall retrieve the authorization information from the CAPIF core function as described in 3GPP TS 29.222 [16], clause 5.6.2.4.

As indicated in 3GPP TS 33.122 [18], the access to the SEAL APIs may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [19]), using the "Client Credentials" authorization grant, where the CAPIF core function (see 3GPP TS 29.222 [16]) plays the role of the authorization server.

NOTE 1:	In this release, only "Client Credentials" authorization grant is supported.

If OAuth2 is used as the selected security method between the VAL server and the SEAL server, the VAL server, prior to consuming services offered by the SEAL APIs, shall obtain a "token" from the authorization server, by invoking the Obtain_Authorization service, as described in 3GPP TS 29.222 [16], clause 5.6.2.3.2.

The SEAL APIs do not define any scopes for OAuth2 authorization. It is the SEAL server responsibility to check whether the VAL server is authorized to use an API based on the "token". Once the SEAL server verifies the "token", it shall check whether the SEAL server identifier in the "token" matches its own published identifier, and whether the API name in the "token" matches its own published API name. If those checks are passed, the VAL server has full authority to access any resource or operation for the invoked API

NOTE 2:	For aforementioned security methods, the SEAL server needs to apply admission control according to access control policies after performing the authorization checks.

# 9	Security

## 9.1	General

The security aspects of SEAL reference points are specified in 3GPP TS 33.434 [26].

## 9.2	SEAL-S security

As specified in clause 5.1.1.8 of 3GPP TS 33.434 [26], the protection of SEAL-S reference point shall be supported according to NDS/IP as specified in 3GPP TS 33.210 [25].

When CAPIF is not used, then TLS and OAuth 2.0 shall be supported as described in clause 5.1.1.8 of 3GPP TS 33.434 [26]. When TLS is used, mutual authentication based on client and server certificates shall be performed between the SEAL server and VAL server using TLS. After the authentication, the SEAL server determines whether the VAL server is authorized to send requests to the SEAL server. The SEAL server shall authorize the requests from VAL server using OAuth-based authorization mechanism.

When CAPIF is used, the security mechanisms described in clause 8.2 shall be applied.

Annex A (normative):
OpenAPI specification

# A.1	General

This annex is based on the OpenAPI Specification [15] and provides corresponding representations of all APIs defined in the present specification in YAML format.

This Annex shall take precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API.

NOTE:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification file contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5B of the 3GPP TR 21.900 [24] and clause 5.3.1 of the 3GPP TS 29.501 [14] for further information).

# A.2	SS_LocationReporting API

openapi: 3.0.0

info:

title: SS_LocationReporting

description: |

API for SEAL Location Reporting Configuration.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.3.0-alpha.3"

externalDocs:

description: >

3GPP TS 29.549 V19.2.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-lr/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/trigger-configurations:

post:

description: Creates a new location reporting configuration.

operationId: CreateLocReportingConfig

tags:

- SEAL Location Reporting Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationReportConfiguration'

responses:

'201':

description: Location reporting configuration resource is created sucessfully.

content:

application/json:

schema:

$ref: '#/components/schemas/LocationReportConfiguration'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

NotifyLocationTriggerEvent:

'{$request.body#/notifUri}':

post:

summary: Notify on location event.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationReport'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

AdaptiveLocationConfig:

'{$request.body#/notifUri}/configure':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AdaptConfigNotif'

responses:

'200':

description: >

OK. The Adaptive Location Configuration

Notification is successfully received and Adaptive Location

Configuration related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AdaptConfigNotifResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/trigger-configurations/{configurationId}:

get:

description: Retrieves an individual SEAL location reporting configuration information.

operationId: RetrieveLocReportingConfig

tags:

- Individual SEAL Location Reporting Configuration (Document)

parameters:

- name: configurationId

in: path

description: String identifying an individual configuration resource.

required: true

schema:

type: string

responses:

'200':

description: The location reporting configuration information.

content:

application/json:

schema:

$ref: '#/components/schemas/LocationReportConfiguration'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Updates an individual SEAL location reporting configuration.

operationId: UpdateLocReportingConfig

tags:

- Individual SEAL Location Reporting Configuration (Document)

parameters:

- name: configurationId

in: path

description: String identifying an individual configuration resource.

required: true

schema:

type: string

requestBody:

description: Configuration information to be updated in location management server.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationReportConfiguration'

responses:

'200':

description: The configuration is updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/LocationReportConfiguration'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual SEAL location reporting configuration.

operationId: DeleteLocReportingConfig

tags:

- Individual SEAL Location Reporting Configuration (Document)

parameters:

- name: configurationId

in: path

description: String identifying an individual configuration resource.

required: true

schema:

type: string

responses:

'204':

description: The individual configuration matching configurationId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modify an existing SEAL Location Reporting Configuration.

operationId: ModifyLocReportingConfig

tags:

- Individual SEAL Location Reporting Configuration (Document)

parameters:

- name: configurationId

in: path

description: Identifier of an individual SEAL location reporting configuration.

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/LocationReportConfigurationPatch'

responses:

'200':

description: >

The individual SEAL location reporting configuration is modified successfully and

a representation of the updated SEAL location reporting configuration is returned

in the request body.

content:

application/json:

schema:

$ref: '#/components/schemas/LocationReportConfiguration'

'204':

description: >

No Content. The individual SEAL location reporting configuration is

modified successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

LocationReportConfiguration:

description: Represents the location reporting configuration information.

type: object

properties:

valServerId:

type: string

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

immRep:

type: boolean

monDur:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

repPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

accuracy:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/Accuracy'

valSvcAreaIds:

type: array

minItems: 1

items:

$ref: 'TS29549_SS_VALServiceAreaConfiguration.yaml#/components/schemas/ValSvcAreaId'

description: >

Represents the VAL service area ID(s).

triggCriteria:

$ref: '#/components/schemas/TriggeringCriteria'

adaptRep:

$ref: '#/components/schemas/AdaptiveReporting'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

report:

$ref: '#/components/schemas/LocationReport'

required:

- valServerId

- valTgtUe

LocationReportConfigurationPatch:

description: Represents the location reporting configuration information patch.

type: object

properties:

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

monDur:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

repPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

accuracy:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/Accuracy'

valSvcAreaIds:

type: array

minItems: 1

items:

$ref: 'TS29549_SS_VALServiceAreaConfiguration.yaml#/components/schemas/ValSvcAreaId'

description: >

Represents the VAL service area ID(s).

triggCriteria:

$ref: '#/components/schemas/TriggeringCriteria'

TriggeringCriteria:

description: Represents the location reporting triggering criteria.

type: object

properties:

reportingMode:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/NotificationMethod'

repPer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

locChgCond:

$ref: '#/components/schemas/LocChangeCond'

ioInd:

$ref: '#/components/schemas/InsideOutsideInd'

repSchedules:

type: array

minItems: 1

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

description: >

Indicates the requested reporting schedule, e.g., day(s) of the week

and/or time period(s) for the location reporting.

required:

- reportingMode

LocationReport:

description: Represents the location trigger report.

type: object

properties:

subscriptionId:

type: string

description: >

Represents the identifier of the subscription to which the location

reporting notification is related.

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

timeStamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- subscriptionId

- valTgtUe

- locInfo

AdaptConfigNotif:

description: Represents the adaptive reporting notification.

type: object

properties:

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

suggConfig:

$ref: '#/components/schemas/TriggeringCriteria'

required:

- valTgtUe

- suggConfig

AdaptConfigNotifResp:

description: Represents the adaptive reporting notification response.

type: object

properties:

confirmation:

$ref: '#/components/schemas/Confirmation'

required:

- confirmation

# Simple data types and Enumerations

InsideOutsideInd:

anyOf:

- type: string

enum:

- INSIDE

- OUTSIDE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents a desired condition of the location reporting,

e.g., inside or outside the given area.

Possible values are:

- INSIDE: Indicates that the reporting shall occur when the UE is inside the given location.

- OUTSIDE: Indicates that the reporting shall occur when the UE is outside the given

location.

LocChangeCond:

anyOf:

- type: string

enum:

- CELL

- NODEB

- TA_RA

- WLAN_AN

- CIVIC_ADDR

- GPS

- SAI

- ECGI

- RAT

- VAL_SERVICE_AREA

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents a desired condition of the requested location change.

Possible values are:

- CELL: The condition is cell change.

- NODEB: The condition is eNodeB or gNodeB change.

- TA_RA: The condition is TA or RA change.

- WLAN_AN: The condition is WLAN access network change (e.g., SSID or BSSID change).

- CIVIC_ADDR: The condition is civic address change.

- GPS: The condition is GPS coordinate change.

- SAI: The condition is SAI change..

- ECGI: The condition is ECGI change.

- RAT: The condition is RAT change.

- VAL_SERVICE_AREA: The condition is VAL service area change.

AdaptiveReporting:

anyOf:

- type: string

enum:

- DIRECT_UPDATE

- SUGGESTIVE_UPDATE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents a desired adaptive reporting configuration mode.

Possible values are:

- DIRECT_UPDATE: Indicates the SEAL LMS to directly update the SEAL

LM client once configuration is adjusted.

- SUGGESTIVE_UPDATE: Indicates that the VAL Server subscribes to receive

the suggested configurations from the SEAL LMS. The related notifications

shall be sent by SEAL LMS before updating to the SEAL LM client.

Confirmation:

anyOf:

- type: string

enum:

- ACCEPT

- REJECT

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents a confirmation response.

Possible values are:

- ACCEPT: Indicates the acceptance of the proposed action.

- REJECT: Indicates the rejection of the proposed action.

# A.3	SS_GroupManagement API

openapi: 3.0.0

info:

title: SS_GroupManagement

description: |

API for SEAL Group management.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.3.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.549 V19.2.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-gm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/group-documents:

post:

description: Creates a new VAL group document.

operationId: CreateValGroupDoc

tags:

- VAL Group Documents (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

responses:

'201':

description: VAL group created sucessfully.

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

description: Retrieves VAL group documents satisfying filter criteria

operationId: RetrieveValGroupDocs

tags:

- VAL Group Documents (Collection)

parameters:

- name: val-group-id

in: query

description: String identifying the VAL group.

schema:

type: string

- name: val-service-id

in: query

description: String identifying the Val service.

schema:

type: string

responses:

'200':

description: List of VAL group documents matching the query parameters in the request.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/VALGroupDocument'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/group-documents/{groupDocId}:

get:

description: Retrieves VAL group information satisfying filter criteria.

operationId: RetrieveIndValGroupDoc

tags:

- Individual VAL Group Document (Document)

parameters:

- name: groupDocId

in: path

description: String identifying an individual VAL group document resource.

required: true

schema:

type: string

- name: group-members

in: query

description: >

When set to true indicates the group management server to send the members

list information of the VAL group.

schema:

type: boolean

- name: group-configuration

in: query

description: >

When set to true indicates the group management server to send the group

configuration information of the VAL group.

schema:

type: boolean

responses:

'200':

description: >

The VAL group information based on the request from the VAL server. Includes

VAL group members list if group-members flag is set to true in the request,

VAL group configuration information if the group-configuration flag is set to

true in the request, VAL group identifier, whole VAL group document resource

if both group-members and group-configuration flags are omitted/set to false

in the request.

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Updates an individual VAL group document.

operationId: UpdateIndValGroupDoc

tags:

- Individual VAL Group Document (Document)

parameters:

- name: groupDocId

in: path

description: String identifying an individual VAL group document resource

required: true

schema:

type: string

requestBody:

description: VAL group document to be updated in Group management server.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

responses:

'200':

description: VAL group document updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes a VAL Group.

operationId: DeleteIndValGroupDoc

tags:

- Individual VAL Group Document (Document)

parameters:

- name: groupDocId

in: path

description: String identifying an individual VAL group document resource.

required: true

schema:

type: string

responses:

'204':

description: The individual VAL group matching groupDocId was deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modify an existing VAL Group document.

operationId: ModifyIndValGroupDoc

tags:

- Individual VAL Group Document (Document)

parameters:

- name: groupDocId

in: path

description: Identifier of an individual VAL group document.

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/VALGroupDocumentPatch'

responses:

'200':

description: >

The individual VAL Group document is modified successfully and a

representation of the updated VAL Group document is returned in the request body.

content:

application/json:

schema:

$ref: '#/components/schemas/VALGroupDocument'

'204':

description: No Content. The individual VAL group document is modified successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

VALGroupDocument:

description: Represents details of the VAL group document information.

type: object

properties:

valGroupId:

type: string

description: The VAL group idenitity.

grpDesc:

type: string

description: The text description of the VAL group.

members:

type: array

description: The list of VAL User IDs or VAL UE IDs, which are members of the VAL group.

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

valGrpConf:

type: string

description: Configuration data for the VAL group.

valServiceIds:

type: array

description: The list of VAL services enabled on the group.

items:

type: string

minItems: 1

valSvcInf:

type: string

description: VAL service specific information.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

resUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

addLocInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

valSvcAreaId:

$ref: 'TS29549_SS_VALServiceAreaConfiguration.yaml#/components/schemas/ValSvcAreaId'

extGrpId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

com5GLanType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

required:

- valGroupId

VALGroupDocumentPatch:

description: Represents details of the partial update of VAL group document information.

type: object

properties:

grpDesc:

type: string

description: The text description of the VAL group.

members:

type: array

description: The list of VAL User IDs or VAL UE IDs, which are members of the VAL group.

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

valGrpConf:

type: string

description: Configuration data for the VAL group.

valServiceIds:

type: array

description: The list of VAL services enabled on the group.

items:

type: string

minItems: 1

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

addLocInfo:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

valSvcAreaId:

$ref: 'TS29549_SS_VALServiceAreaConfiguration.yaml#/components/schemas/ValSvcAreaId'

extGrpId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

com5GLanType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

# A.4	SS_UserProfileRetrieval API

openapi: 3.0.0

info:

title: SS_UserProfileRetrieval

description: |

API for SEAL User Profile Retrieval.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.3.0-alpha.2"

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-upr/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/val-services:

get:

description: Retrieve VAL User or VAL UE profile information.

operationId: RetrieveValUserProfile

tags:

- VAL Services

parameters:

- name: val-service-id

in: query

description: String identifying an individual VAL service

required: false

schema:

type: string

- name: val-tgt-ue

in: query

description: Identifying a VAL target UE.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ValTargetUe'

responses:

'200':

description: The Profile information of the VAL User or VAL UE.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ProfileDoc'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ProfileDoc:

description: Represents the profile information associated with a VAL user ID or a VAL UE ID.

type: object

properties:

profileInformation:

type: string

description: Profile information associated with the valUserId or valUeId.

valTgtUe:

$ref: '#/components/schemas/ValTargetUe'

required:

- profileInformation

- valTgtUe

ValTargetUe:

description: Represents the information identifying a VAL user ID or a VAL UE ID.

type: object

properties:

valUserId:

type: string

description: Unique identifier of a VAL user.

valUeId:

type: string

description: Unique identifier of a VAL UE.

oneOf:

- required: [valUserId]

- required: [valUeId]

ValTargetUeRm:

description: >

Represents the same as the ValTargetUe data type but with the OpenAPI

"nullable" property set to "true".

type: object

properties:

valUserId:

type: string

valUeId:

type: string

oneOf:

- required: [valUserId]

- required: [valUeId]

nullable: true

# A.5	SS_NetworkResourceAdaptation API

openapi: 3.0.0

info:

version: 1.3.0-alpha.4

title: SS_NetworkResourceAdaptation

description: |

SS Network Resource Adaptation Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.3.0; Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-nra/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

paths:

/multicast-subscriptions:

post:

summary: Creates a new Individual Multicast Subscription resource

operationId: CreateMulticastSubscription

tags:

- Multicast Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MulticastSubscription'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/MulticastSubscription'

headers:

Location:

description: >

Contains the URI of the created individual multicast subscription resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

UserPlaneNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UserPlaneNotification'

responses:

'204':

description: No Content, Notification was succesfull

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/multicast-subscriptions/{multiSubId}:

get:

summary: "Reads an existing Individual Multicast Subscription"

operationId: GetMulticastSubscription

tags:

- Individual Multicast Subscription (Document)

parameters:

- name: multiSubId

in: path

description: Multicast Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/MulticastSubscription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: "Delete an existing Individual Multicast Subscription"

operationId: DeleteMulticastSubscription

tags:

- Individual Multicast Subscription (Document)

parameters:

- name: multiSubId

in: path

description: Multicast Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-resources:

post:

summary: Request the creation of a new MBS Resource.

operationId: CreateMBSResource

tags:

- MBS Resources (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MBSResourceReq'

responses:

'201':

description: >

Created. Successfull case. The requested MBS resource is successfully created and a

representation of the created Individual MBS Resource resource is returned in the

response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSResourceResp'

headers:

Location:

description: >

Contains the URI of the created Individual MBS Resource resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

UserPlaneNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UserPlaneNotification'

responses:

'204':

description: No Content. The notification was succesfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-resources/{mbsResId}:

parameters:

- name: mbsResId

in: path

description: Represents the identifier of the Individual MBS Resource resource.

required: true

schema:

type: string

get:

summary: Request the retrieval of an existing Individual MBS Resource.

operationId: GetIndivMBSResource

tags:

- Individual MBS Resource (Document)

responses:

'200':

description: >

OK. Successful case. A representation of the requested Individual MBS Resource resource

is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSResource'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual MBS Resource.

operationId: UpdateIndivMBSResource

tags:

- Individual MBS Resource (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MBSResource'

responses:

'200':

description: >

OK. Successful case. The targeted Individual MBS Resource resource is successfully

updated and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSResource'

'204':

description: >

No Content. Successful case. The targeted Individual MBS Resource resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual MBS Resource.

operationId: ModifyIndivMBSResource

tags:

- Individual MBS Resource (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MBSResourcePatch'

responses:

'200':

description: >

OK. Successful case. The targeted Individual MBS Resource resource is successfully

modified and a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MBSResource'

'204':

description: >

No Content. Successful case. The targeted Individual MBS Resource resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual MBS Resource.

operationId: DeleteIndivMBSResource

tags:

- Individual MBS Resource (Document)

responses:

'204':

description: >

No Content. Successful case. The targeted Individual MBS Resource resource is

successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-resources/{mbsResId}/activate:

parameters:

- name: mbsResId

in: path

description: Represents the identifier of the Individual MBS Resource resource.

required: true

schema:

type: string

post:

summary: Request the activation of an existing MBS Resource.

operationId: ActivateMBSResource

tags:

- Individual MBS Resource (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsResAct'

responses:

'200':

description: >

OK. The activation request is successfully received and processed.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsResAct'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/mbs-resources/{mbsResId}/deactivate:

parameters:

- name: mbsResId

in: path

description: Represents the identifier of the Individual MBS Resource resource.

required: true

schema:

type: string

post:

summary: Request the deactivation of an existing MBS Resource.

operationId: DeactivateMBSResource

tags:

- Individual MBS Resource (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsResDeact'

responses:

'200':

description: >

OK. The deactivation request is successfully received and processed.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsResDeact'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/unicast-subscriptions:

post:

summary: Creates a new Individual Unicast Subscription resource

operationId: CreateUnicastSubscription

tags:

- Unicast Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UnicastSubscription'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/UnicastSubscription'

headers:

Location:

description: >

Contains the URI of the created individual unicast subscription resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

UserPlaneNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UserPlaneNotification'

responses:

'204':

description: No Content, Notification was succesfull

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/unicast-subscriptions/{uniSubId}:

get:

summary: "Reads an existing Individual Unicast Subscription"

operationId: GetUnicastSubscription

tags:

- Individual Unicast Subscription (Document)

parameters:

- name: uniSubId

in: path

description: Unicast Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/UnicastSubscription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update Individual Unicast Subscription resource.

operationId: UpdateUnicastSubscription

tags:

- Individual Unicast Subscription (Document)

parameters:

- name: uniSubId

in: path

description: Unicast Subscription ID

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UnicastSubscription'

responses:

'200':

description: >

Successful case. The "Individual Unicast Subscription" resource is

successfully updated and a representation of the updated resource is

returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UnicastSubscription'

'204':

description: >

Successful case. The "Individual Unicast Subscription" resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify Individual Unicast Subscription resource.

operationId: ModifyUnicastSubscription

tags:

- Individual Unicast Subscription (Document)

parameters:

- name: uniSubId

in: path

description: Unicast Subscription ID

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UnicastSubscriptionPatch'

responses:

'200':

description: >

Successful case. The "Individual Unicast Subscription" resource is

successfully modified and a representation of the updated resource is returned

in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UnicastSubscription'

'204':

description: >

Successful case. The "Individual Unicast Subscription" resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: "Delete an existing Individual Unicast Subscription"

operationId: DeleteUnicastSubscription

tags:

- Individual Unicast Subscription (Document)

parameters:

- name: uniSubId

in: path

description: Unicast Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/tsc-stream-availability:

get:

summary: Discover the TSC stream availability information.

operationId: GetTscStreamAvailability

tags:

- TSC stream availability discovery

parameters:

- name: stream-specs

in: query

description: >

The MAC address(es) of the source DS-TT port(s) and the destination DS-TT port(s).

required: true

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/StreamSpecification'

minItems: 1

responses:

'200':

description: OK.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/TscStreamAvailability'

minItems: 0

'204':

description: No Content.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/tsc-streams:

get:

summary: Retrieval of TSC stream data.

operationId: GetTscStream

tags:

- TSC stream retrieval

parameters:

- name: val-stream-ids

in: query

description: Retrieval of TSC Stream data, identified by the VAL Stream ID(s).

required: false

schema:

type: array

items:

type: string

minItems: 1

responses:

'200':

description: OK (successful query of TSC stream resource)

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/TscStreamData'

minItems: 1

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/tsc-streams/{valStreamId}:

get:

summary: "Reads an existing Individual TSC stream data information"

operationId: GetTscStreamData

tags:

- Individual TSC Stream Retrieval

parameters:

- name: valStreamId

in: path

description: The VAL Stream ID identifies the TSC stream.

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/TscStreamData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Create a TSC stream identified by a VAL stream identifier.

operationId: PutTscStream

tags:

- TSC stream creation

description: Create an individual TSC stream identified by VAL Stream ID.

parameters:

- name: valStreamId

in: path

description: VAL stream identifier

required: true

schema:

type: string

requestBody:

description: TSC stream creation request data from the service consumer to the NRM server.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TscStreamData'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/TscStreamData'

headers:

Location:

description: Contains the URI of the created individual TSC stream resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: "Delete an existing Individual TSC stream"

operationId: DeleteTscStream

tags:

- Individual TSC Stream Deletion

parameters:

- name: valStreamId

in: path

description: The VAL Stream ID identifies the TSC stream.

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/bdt-policy-configs:

post:

summary: Creates a new Individual BDT Policy Configuration

operationId: CreateBDTPolicyConfig

tags:

- BDT Policy Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/BdtPolConfig'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/BdtPolConfig'

headers:

Location:

description: >

Contains the URI of the created individual BDT Policy configuration resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

BdtNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/BdtNotification'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/bdt-policy-configs/{bdtPolConfigId}:

parameters:

- name: bdtPolConfigId

in: path

description: Represents the identifier of the Individual BDT Policy Configuration.

required: true

schema:

type: string

get:

summary: Reads an existing Individual BDT Policy Configuration

operationId: GetBDTPolicyConfig

tags:

- Individual BDT Policy Configuration(Document)

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/BdtPolConfig'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual BDT Policy Configuration.

operationId: UpdateBDTPolicyConfig

tags:

- Individual BDT Policy Configuration(Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/BdtPolConfig'

responses:

'200':

description: >

OK. Successful case. The targeted Individual BDT Policy Configuration resource

is successfully updated and a representation of the updated resource is returned

in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/BdtPolConfig'

'204':

description: >

No Content. Successful case. The targeted Individual BDT Policy Configuration

resource is successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual BDT Policy Configuration.

operationId: ModifyBDTPolicyConfig

tags:

- Individual BDT Policy Configuration(Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/BdtPolConfigPatch'

responses:

'200':

description: >

OK. Successful case. The targeted Individual BDT Policy Configuration resource

is successfully modified and a representation of the updated resource is returned

in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/BdtPolConfig'

'204':

description: >

No Content. Successful case. The targeted Individual BDT Policy Configuration

resource is successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual BDT policy Configuration

operationId: DeleteBDTPolicyConfig

tags:

- Individual BDT Policy Configuration (Document)

responses:

'204':

description: No Content. Resource was successfully deleted

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/request-rel-trans:

post:

summary: Enables a service consumer to request reliable transmission service.

operationId: RelTransRequest

tags:

- Reliable Transmission Service Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/RelTransReq'

responses:

'204':

description: >

No Content. The reliable transmission service request is successfully received and

processed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/unif-traff-subscriptions:

post:

summary: Creates a new Individual Unified Traffic Pattern Subscription.

operationId: CreateUnifiedTrafficPatternSubsc

tags:

- Unified Traffic Pattern Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UnfTrafficSubc'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/UnfTrafficSubc'

headers:

Location:

description: >

Contains the URI of the created individual unified traffic pattern subscription

resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

UnfTrafficUpdNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UnfTrafficUpdNotification'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/unif-traff-subscriptions/{unfTrffSubId}:

parameters:

- name: unfTrffSubId

in: path

description: >

Represents the identifier of the Individual Unified Traffic Pattern Subscription resource.

required: true

schema:

type: string

get:

summary: Reads an existing Individual Unified Traffic Pattern Subscription.

operationId: GetUnfTrffSubc

tags:

- Individual Unified Traffic Pattern Subscription (Document)

responses:

'200':

description: OK. Resource representation is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UnfTrafficSubc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual Unified Traffic Pattern Subscription.

operationId: UpdateUnfTrffSubc

tags:

- Individual Unified Traffic Pattern Subscription (Document)

requestBody:

description: Updated details of the Unified Traffic Pattern subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UnfTrafficSubc'

responses:

'200':

description: >

The subscription is updated successfully, and the updated subscription

information returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/UnfTrafficSubc'

'204':

description: No Content. The subscription is updated successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual Unified Traffic Pattern Subscription.

operationId: ModifyUnfTrffSubc

tags:

- Individual Unified Traffic Pattern Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/UnfTrafficSubcPatch'

responses:

'200':

description: >

The individual Unified Traffic Pattern subscription is

modified successfully, and the representation of the modified resource

is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UnfTrafficSubcPatch'

'204':

description: No Content. The subscription is modified successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual Unified Traffic Pattern Subscription

operationId: DeleteUnfTrffSubc

tags:

- Individual Unified Traffic Pattern Subscription (Document)

responses:

'204':

description: No Content. Resource was successfully deleted

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

MulticastSubscription:

description: Represents a multicast subscription.

type: object

properties:

valGroupId:

type: string

anncMode:

$ref: '#/components/schemas/ServiceAnnoucementMode'

multiQosReq:

type: string

locArea:

$ref: 'TS29122_GMDviaMBMSbyMB2.yaml#/components/schemas/MbmsLocArea'

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

tmgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

localMbmsInfo:

$ref: 'TS29486_VAE_FileDistribution.yaml#/components/schemas/LocalMbmsInfo'

localMbmsActInd:

type: boolean

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

reqTestNotif:

type: boolean

wsNotifCfg:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

upIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

upIpv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

upPortNum:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

radioFreqs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

minItems: 1

required:

- valGroupId

- anncMode

- multiQosReq

- notifUri

UnicastSubscription:

description: Represents a network resource adaptation subscription.

type: object

properties:

valUes:

type: array

items:

type: string

minItems: 1

valGroupId:

type: string

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

bdtId:

type: string

uniQosReq:

type: string

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

reqTestNotif:

type: boolean

wsNotifCfg:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

oneOf:

- required: [valUes]

- required: [valGroupId]

- required: [valTgtUe]

UnicastSubscriptionPatch:

description: >

Represents the requested modifications to a network resource adaptation subscription.

type: object

properties:

valUes:

type: array

items:

type: string

minItems: 1

nullable: true

valGroupId:

type: string

nullable: true

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUeRm'

bdtId:

type: string

uniQosReq:

type: string

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

UserPlaneNotification:

description: Represents a notification on User Plane events.

type: object

properties:

notifId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

eventNotifs:

type: array

items:

$ref: '#/components/schemas/NrmEventNotification'

minItems: 1

required:

- notifId

- eventNotifs

NrmEventNotification:

description: Represents a notification on an individual User Plane event.

type: object

properties:

event:

$ref: '#/components/schemas/NrmEvent'

ts:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

deliveryMode:

$ref: '#/components/schemas/DeliveryMode'

streamIds:

type: array

items:

type: string

minItems: 1

stopUniStreamIds:

type: array

items:

type: string

minItems: 1

mbsResServiceArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalMbsServiceArea'

valUeIds:

type: array

items:

type: string

minItems: 1

required:

- event

- ts

TscStreamAvailability:

description: >

TSC stream availability information includes the stream specification and list of traffic

specifications. This response shall include stream specification matching one of the query

parameters provided in the request.

type: object

properties:

streamSpec:

$ref: '#/components/schemas/StreamSpecification'

trafficSpecs:

type: array

items:

$ref: '#/components/schemas/TrafficSpecification'

minItems: 1

required:

- streamSpec

- trafficSpecs

StreamSpecification:

description: >

Stream specification includes MAC addresses of the source and destination DS-TT ports.

type: object

properties:

srcMacAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

dstMacAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

required:

- srcMacAddr

- dstMacAddr

TrafficSpecification:

description: >

The traffic classe supported by the DS-TTs and available end-to-end maximum latency value.

type: object

properties:

trafficClass:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

e2eMaxLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- trafficClass

- e2eMaxLatency

TscStreamData:

description: TSC stream data information.

type: object

properties:

streamSpec:

$ref: '#/components/schemas/StreamSpecification'

trafficSpecInfo:

$ref: '#/components/schemas/TrafficSpecInformation'

required:

- streamSpec

- trafficSpecInfo

TrafficSpecInformation:

description: >

The traffic classe supported by the DS-TTs and available end-to-end latency

value and Priority Code Point (PCP) value.

type: object

properties:

pcpValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxFramInt:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

maxFramSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxIntFrames:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

required:

- pcpValue

- maxFramInt

- maxFramSize

- maxIntFrames

- maxLatency

MBSResourceReq:

description: Represents the parameters to request the creation of an MBS Resource.

type: object

properties:

mbsResource:

$ref: '#/components/schemas/MBSResource'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsResource

MBSResource:

description: Represents an MBS Resource.

type: object

properties:

valGroupId:

type: string

valUeIdsList:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

servAnmtMode:

$ref: '#/components/schemas/ServiceAnnoucementMode'

mbsResServInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceInfo'

mbsResServiceArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalMbsServiceArea'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

netSysInd:

$ref: '#/components/schemas/NetSysIndicator'

localMbmsInfo:

$ref: 'TS29486_VAE_FileDistribution.yaml#/components/schemas/LocalMbmsInfo'

localMbmsActInd:

type: boolean

mbsResRespInfo:

$ref: '#/components/schemas/MBSResourceRespInfo'

oneOf:

- required: [valGroupId]

- required: [valUeIdsList]

not:

required: [localMbmsInfo, localMbmsActInd]

required:

- servAnmtMode

- mbsResServInfo

- notifUri

MBSResourceRespInfo:

description: Represents NRM Server side information related to the MBS Resource.

type: object

properties:

mbs5gSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

mbmsBearerId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

upIpv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

upIpv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

upPortNum:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

mbs5GInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSession'

epsMbmsInfo:

$ref: '#/components/schemas/EpsMbmsInfo'

anyOf:

- required: [upIpv4Addr]

- required: [upIpv6Addr]

MBSResourceResp:

description: Represents a response to an MBS Resource creation/modification request.

type: object

properties:

mbsResource:

$ref: '#/components/schemas/MBSResource'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsResource

MBSResourcePatch:

description: Represents the parameters to request the modification of an MBS Resource.

type: object

properties:

mbsResServInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceInfo'

mbsResServiceArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalMbsServiceArea'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

localMbmsInfo:

$ref: 'TS29486_VAE_FileDistribution.yaml#/components/schemas/LocalMbmsInfo'

localMbmsActInd:

type: boolean

mbsResRespInfo:

$ref: '#/components/schemas/MBSResourceRespInfo'

anyOf:

- required: [mbsResServInfo]

- required: [mbsResServiceArea]

- required: [localMbmsInfo]

not:

required: [localMbmsInfo, localMbmsActInd]

MbsResAct:

description: Represents the parameters related to the activation of the MBS Resource.

type: object

properties:

mbs5gSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbs5gSessionId

MbsResDeact:

description: Represents the parameters related to the deactivation of the MBS Resource.

type: object

properties:

mbs5gSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsSessionId'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbs5gSessionId

BdtPolConfig:

description: Represents the parameters related to the BDT Policy configuration.

type: object

properties:

valServId:

type: string

valGroupId:

type: string

valUeIds:

type: array

items:

type: string

minItems: 1

dataVolUe:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

desiredTimeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

grantTimeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

desiredGeoArea:

$ref: '#/components/schemas/GeoArea'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

policyGuidance:

$ref: '#/components/schemas/PolicyGuidance'

bdtPolRefId:

type: string

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

oneOf:

- required: [valGroupId]

- required: [valUeIds]

required:

- valServId

- notifUri

- dataVolUe

- desiredTimeWindow

- desiredGeoArea

BdtPolConfigPatch:

description: >

Represents the parameters to request modification of the BDT Policy configuration.

type: object

properties:

dataVolUe:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

desiredTimeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

desiredGeoArea:

$ref: '#/components/schemas/GeoArea'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

PolicyGuidance:

description: >

Represents the list of Policy Guidance.

type: object

properties:

policyType:

$ref: '#/components/schemas/PolicyType'

cost:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

GeoArea:

description: Represents a Geographical area.

type: object

properties:

geographicAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

description: Represents a list of Geographical area.

civicAddresses:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

minItems: 1

description: Represents a list of Civic address of an area.

BdtNotification:

description: Represents a notification on update related to BDT Policy configuration resource.

type: object

properties:

notifId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

bdtConfigId:

type: string

grantTimeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

bdtPolicyRemoveInd:

type: boolean

default: false

required:

- notifId

- bdtConfigId

UnfTrafficSubc:

description: Represents the request parameters for Unified Traffic Pattern subscription.

type: object

properties:

mgmtSubs:

type: object

additionalProperties:

$ref: '#/components/schemas/ManagementSubc'

minProperties: 1

description: >

Contains contains the unified traffic pattern subscription(s).

The key of the map is any unique string encoded value.

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mgmtSubs

- notifUri

UnfTrafficSubcPatch:

description: Represents the request parameters to update Unified Traffic Pattern subscription.

type: object

properties:

mgmtSubs:

type: object

additionalProperties:

$ref: '#/components/schemas/ManagementSubc'

minProperties: 1

description: >

Contains contains the unified traffic pattern subscription(s).

The key of the map is any unique string encoded value.

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

ManagementSubc:

description: Represents the Unified Traffic Pattern subscription details.

type: object

properties:

valServId:

type: string

valUeIds:

type: array

items:

type: string

minItems: 1

traffPatInd:

type: array

items:

$ref: '#/components/schemas/TrafficPatternIndication'

minItems: 1

traffPatConfig:

$ref: '#/components/schemas/TrafficPatternConfig'

required:

- valServId

- valUeIds

- traffPatInd

TrafficPatternConfig:

description: Represents the traffic pattern configuration parameters.

type: object

properties:

scheds:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

stationInd:

type: boolean

default: false

UnfTrafficUpdNotification:

description: Represents a notification on update related to unified traffic pattern.

type: object

properties:

valUeIds:

type: array

items:

type: string

minItems: 1

sched:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

stationInd:

type: boolean

default: false

cause:

type: string

required:

- valUeIds

RelTransReq:

description: Represents the parameters to request reliable transmission service.

type: object

properties:

requestorId:

type: string

appDescs:

type: array

items:

$ref: '#/components/schemas/AppTraffDesc'

minItems: 2

valUeId:

type: string

valAddrInfo:

$ref: '#/components/schemas/ValUeAddrInfo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- requestorId

AppTraffDesc:

description: Represents the application traffic descriptor.

type: object

properties:

connInfo:

$ref: 'TS29548_SDD_Transmission.yaml#/components/schemas/ConnInfo'

transProtoc:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/TransportProtocol'

required:

- connInfo

ValUeAddrInfo:

description: Represents VAL UE address information.

type: object

properties:

valUeIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

valUeIpv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

port:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

oneOf:

- required: [valUeIpv4Addr]

- required: [valUeIpv6Addr]

# Simple data types and Enumerations

EpsMbmsInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

ServiceAnnoucementMode:

anyOf:

- type: string

enum:

- NRM

- VAL

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates the service announcement mode.

Possible values are:

- NRM: Indicates that the NRM server performs the service announcement.

- VAL: Indicates that the service consumer performs the service announcement.

DeliveryMode:

anyOf:

- type: string

enum:

- UNICAST

- MULTICAST

- MBS_MULTICAST

- MBS_BROADCAST

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Indicates the user plane delivery mode.

Possible values are:

- UNICAST: Indicates Unicast delivery.

- MULTICAST: Indicates EPS MBMS Multicast delivery.

- MBS_MULTICAST: Indicates 5GS MBS Multicast delivery.

- MBS_BROADCAST: Indicates 5GS MBS Broadcast delivery.

NrmEvent:

anyOf:

- type: string

enum:

- UP_DELIVERY_MODE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates the NRM event.

Possible values are:

- UP_DELIVERY_MODE: User Plane delivery mode.

NetSysIndicator:

anyOf:

- type: string

enum:

- 5GS

- EPS

- 5GS_AND_EPS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents the network system indicator, i.e. 5GS, EPS or both.

Possible values are:

- 5GS: Indicates that the network system is 5GS.

- EPS: Indicates that the network system is EPS.

- 5GS_AND_EPS: Indicates that the network system is 5GS and EPS.

PolicyType:

anyOf:

- type: string

enum:

- LOWEST_COST

- HIGH_THROUGHPUT_WITH_COST

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates the Policy types.

Possible values are:

- LOWEST_COST: Indicates that the policy guidance is to consider lowest cost.

- HIGH_THROUGHPUT_WITH_COST: Indicates that the policy guidance is to consider highest

throughput.

TrafficPatternIndication:

anyOf:

- type: string

enum:

- TRAFFIC_PATTERN_MANAGE

- TRAFFIC_PATTERN_MONITOR

- NETWORK_PARAM_COORDINATION

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents the unified traffic pattern indication requests.

Possible values are:

- TRAFFIC_PATTERN_MANAGE: Indicates management of the UE unified traffic pattern request.

- TRAFFIC_PATTERN_MONITOR: Indicates monitoring of the UE unified traffic pattern.

- NETWORK_PARAM_COORDINATION: Indicates network parameter coordination by NRM with 5GC.

# A.6	SS_Events API

openapi: 3.0.0

info:

title: SS_Events

description: |

API for SEAL Events management.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.3.0-alpha.4"

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-events/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

description: Creates a new individual SEAL Event Subscription.

operationId: CreateSealEventSubsc

tags:

- SEAL Events Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SEALEventSubscription'

callbacks:

notificationDestination:

'{$request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SEALEventNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: SEAL Events subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/SEALEventSubscription'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

delete:

description: Deletes an individual SEAL Event Subscription.

operationId: DeleteIndSealEventSubsc

tags:

- Individual SEAL Events Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifier of an individual Events Subscription

required: true

schema:

type: string

responses:

'204':

description: >

The individual SEAL Events Subscription matching the subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modify an existing SEAL Event Subscription.

operationId: ModifyIndSealEventSubsc

tags:

- Individual SEAL Events Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifier of an individual Events Subscription

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SEALEventSubscriptionPatch'

responses:

'200':

description: >

The definition SEAL event subscription is modified successfully and

a representation of the updated service API is returned in the request body.

content:

application/json:

schema:

$ref: '#/components/schemas/SEALEventSubscription'

'204':

description: No Content. The SEAL Event Subscription is modified successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Replace an existing SEAl event subscription.

operationId: UpdateIndSealEventSubsc

tags:

- Individual SEAL Events Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifier of an individual Events Subscription

required: true

schema:

type: string

requestBody:

description: Individual SEAL events subscription to be replaced.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SEALEventSubscription'

responses:

'200':

description: SEAL Event subscription updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/SEALEventSubscription'

'204':

description: No Content. Individual SEAL event subscription was updated successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SEALEventSubscription:

description: Represents an individual SEAL Event Subscription resource.

type: object

properties:

subscriberId:

type: string

description: String identifying the subscriber of the event.

eventSubs:

type: array

items:

$ref: '#/components/schemas/EventSubscription'

minItems: 1

description: Subscribed events.

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true by Subscriber to request the SEAL server to send a test notification.

Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

eventDetails:

type: array

items:

$ref: '#/components/schemas/SEALEventDetail'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- subscriberId

- eventSubs

- eventReq

- notificationDestination

SEALEventSubscriptionPatch:

description: Represents the partial update of individual SEAL Event Subscription resource.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/EventSubscription'

minItems: 1

description: Subscribed events.

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

SEALEventNotification:

description: Represents notification information of a SEAL Event.

type: object

properties:

subscriptionId:

type: string

description: Identifier of the subscription resource.

eventDetails:

type: array

items:

$ref: '#/components/schemas/SEALEventDetail'

minItems: 1

description: Detailed notifications of individual events.

required:

- subscriptionId

- eventDetails

EventSubscription:

description: Represents the subscription to a single SEAL event.

type: object

properties:

eventId:

$ref: '#/components/schemas/SEALEvent'

valGroups:

type: array

items:

$ref: '#/components/schemas/VALGroupFilter'

minItems: 1

description: >

Each element of the array represents the VAL group identifier(s) of a VAL service

that the subscriber wants to know in the interested event.

identities:

type: array

items:

$ref: '#/components/schemas/IdentityFilter'

minItems: 1

description: >

Each element of the array represents the VAL User / UE IDs of a VAL service

that the event subscriber wants to know in the interested event.

monFltr:

type: array

items:

$ref: '#/components/schemas/MonitorFilter'

minItems: 1

description: >

List of event monitoring details that the subscriber wishes to mmonitor the VAL UEs,

VAL group and/or VAL service.

areaInt:

type: array

items:

$ref: '#/components/schemas/MonitorLocationInterestFilter'

minItems: 1

description: >

Represents the list of VAL User / UE IDs and the area of interest information

which the subscriber wishes to monitor the location deviation of VAL User / UEs.

locAreaMon:

type: array

items:

$ref: '#/components/schemas/MonLocAreaInterestFltr'

minItems: 1

description: >

Each element represents the location area monitoring details to monitor the

VA UEs moving in and out of the provided location area.

partialFailRep:

$ref: '#/components/schemas/PartialEventSubscFailRep'

satSfTrigger:

type: array

items:

$ref: '#/components/schemas/SatSfTrigger'

minItems: 1

required:

- eventId

SEALEventDetail:

description: Represents the SEAL event details.

type: object

properties:

eventId:

$ref: '#/components/schemas/SEALEvent'

lmInfos:

type: array

items:

$ref: '#/components/schemas/LMInformation'

minItems: 1

valGroupDocuments:

type: array

items:

$ref: 'TS29549_SS_GroupManagement.yaml#/components/schemas/VALGroupDocument'

minItems: 1

description: >

The VAL groups documents with modified membership and configuration information.

profileDocs:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ProfileDoc'

minItems: 1

description: Updated profile information associated with VAL Users or VAL UEs.

msgFltrs:

type: array

items:

$ref: '#/components/schemas/MessageFilter'

minItems: 1

description: >

The message filter information for various member VAL User or UEs of the VAL group.

monRep:

type: array

items:

$ref: '#/components/schemas/MonitorEventsReport'

minItems: 1

description: The events reports with details of the events related to the VAL UE(s).

locAdhr:

type: array

items:

$ref: '#/components/schemas/LocationDevMonReport'

minItems: 1

description: >

The location deviation information for the interested VAL User ID or UE IDs

in a given location.

tempGroupInfo:

$ref: '#/components/schemas/TempGroupInfo'

locAreaMonRep:

type: array

items:

$ref: '#/components/schemas/LocationAreaMonReport'

minItems: 1

description: The location area monitoring of the given area of interest.

valGroupIds:

type: array

items:

type: string

minItems: 1

description: Contains the identifier(s) of the deleted VAL Group(s).

satSfInfo:

type: array

items:

$ref: '#/components/schemas/SatSfInfo'

minItems: 1

required:

- eventId

VALGroupFilter:

description: Represents a filter of VAL group identifiers belonging to a VAL service.

type: object

properties:

valSvcId:

type: string

description: Identity of the VAL service

valGrpIds:

type: array

items:

type: string

minItems: 1

description: >

VAL group identifiers that event subscriber wants to know in the interested event.

required:

- valGrpIds

IdentityFilter:

description: Represents a filter of VAL User / UE identities belonging to a VAL service.

type: object

properties:

valSvcId:

type: string

description: Identity of the VAL service

valTgtUes:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: >

VAL User IDs or VAL UE IDs that the event subscriber wants to know

in the interested event.

suppLoc:

type: boolean

description: Set to true by Subscriber to request the supplementary location information.

locQoS:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

velocityReq:

type: boolean

description: Indicates whether velocity of the location information is requested.

locDataStatsReq:

type: boolean

description: Indicates whether the location data statistics is requested.

dataStatisticsReq:

$ref: '#/components/schemas/DataStatisticsReq'

LMInformation:

description: Represents the location information for a VAL User ID or a VAL UE ID.

type: object

properties:

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

timeStamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

valSvcId:

type: string

description: Identity of the VAL service

required:

- locInfo

- valTgtUe

MessageFilter:

description: Represents the message filters applicable to a VAL User ID or VAL UE ID.

type: object

properties:

reqUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

tgtUe:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: List of VAL User or UE IDs whose message to be sent.

maxMsgs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

scheds:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

description: Time frame associated with total number of messages.

msgTypes:

type: array

items:

type: string

minItems: 1

description: List of message types to be sent to VAL UE.

required:

- reqUe

MonitorFilter:

description: Represents the event monitoring filters applicable to a VAL User ID or VAL UE ID.

type: object

properties:

idnts:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: List of VAL User or UE IDs whose events monitoring is requested.

valSvcId:

type: string

description: Identity of the VAL service.

valGrpId:

type: string

description: Identity of the group of the target UEs.

profId:

type: string

description: The monitoring profile ID identifying a list of monitoring, analytics events.

valCnds:

type: array

items:

$ref: '#/components/schemas/ValidityConditions'

minItems: 1

description: The temporal,spatial conditions for the events to be considered valid.

evntDets:

type: array

items:

$ref: '#/components/schemas/MonitorEvents'

minItems: 1

description: List of monitoring, analytics events to be monitored.

MonitorEvents:

description: List of event types to be monitored in the context of events monitoring service.

type: object

properties:

cnEvnts:

type: array

items:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/MonitoringType'

minItems: 1

description: List of monitoring events related to VAL UE.

anlEvnts:

type: array

items:

$ref: 'TS29522_AnalyticsExposure.yaml#/components/schemas/AnalyticsEvent'

minItems: 1

description: List of analytics events related to VAL UE.

MonitorEventsReport:

description: List of monitoring and/or analytics events related to VAL UE.

type: object

properties:

tgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

evnts:

type: array

items:

$ref: '#/components/schemas/MonitorEvents'

minItems: 1

description: List of monitoring and analytics events related to VAL UE.

required:

- tgtUe

- evnts

ValidityConditions:

description: List of monitoring and/or analytics events related to VAL UE.

type: object

properties:

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

tmWdws:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

description: Time window validity conditions.

MonitorLocationInterestFilter:

description: Represents the location monitoring filter information.

type: object

properties:

tgtUes:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: List of VAL Users or UE IDs for which location monitoring is requested.

locInt:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

valSvcId:

$ref: 'TS29549_SS_VALServiceAreaConfiguration.yaml#/components/schemas/ValSvcAreaId'

notInt:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

dynamicLoc:

$ref: '#/components/schemas/ReferenceUEDetail'

events:

type: array

items:

$ref: '#/components/schemas/LocDevNotification'

minItems: 1

geofCond:

$ref: '#/components/schemas/GeofencingConditions'

required:

- tgtUes

- notInt

oneOf:

- required: [locInt]

- required: [valSvcId]

- required: [dynamicLoc]

LocationDevMonReport:

description: Location deviation monitoring report.

type: object

properties:

tgtUes:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: List of VAL Users or UE IDs for which report is related to.

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

notifType:

$ref: '#/components/schemas/LocDevNotification'

required:

- tgtUes

- locInfo

- notifType

TempGroupInfo:

description: Represents the created temporary VAL group information.

type: object

properties:

valGrpIds:

type: array

items:

type: string

minItems: 1

tempValGrpId:

type: string

valServIds:

type: array

items:

type: string

minItems: 1

required:

- valGrpIds

- tempValGrpId

MonLocAreaInterestFltr:

description: >

Filter information indicate the area of interest and triggering/geofencing events.

type: object

properties:

locInfoCri:

$ref: '#/components/schemas/LocationInfoCriteria'

trigEvnts:

type: array

items:

$ref: '#/components/schemas/MonLocTriggerEvent'

minItems: 1

valServiceId:

type: string

required:

- locInfoCri

LocationInfoCriteria:

description: >

Geographic location and reference UE details, where the UEs moving in and out

to be monitored.

type: object

properties:

geoArea:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

refUe:

$ref: '#/components/schemas/ReferenceUEDetail'

valSvcAreaId:

$ref: 'TS29549_SS_VALServiceAreaConfiguration.yaml#/components/schemas/ValSvcAreaId'

oneOf:

- required: [geoArea]

- required: [refUe]

ReferenceUEDetail:

description: Reference UE details, where the UEs moving in and out to be monitored.

type: object

properties:

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

proxRange:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

proxRangeFrac:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

required:

- valTgtUe

- proxRange

LocationAreaMonReport:

description: Event report to notify the VAL UEs moving in or out from a given location.

type: object

properties:

curPreUEs:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: List of identities of all VAL UEs present in the given location area.

moveInOutUEs:

$ref: '#/components/schemas/MoveInOutUEDetails'

trigEvnt:

$ref: '#/components/schemas/MonLocTriggerEvent'

MoveInOutUEDetails:

description: List of UEs moved in and out.

type: object

properties:

moveInUEs:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: >

List of identities of VAL UEs who moved in to given location area

since previous notification.

numUEsMovedIn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timingUEsMovedIn:

type: object

additionalProperties:

$ref: '#/components/schemas/MovTimingInfo'

minProperties: 1

description: >

Contains the timing information of the VAL UE(s) that moved into the geofencing area

since the previous notification.

The key of the map shall be set to the value of the identifier of the VAL UE (among the

ones provided within the moveInUEs attribute) to which the timing information provided

within the map value is related.

moveOutUEs:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: >

List of identities of VAL UEs who moved out of the given location area

since previous notification.

numUEsMovedOut:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timingUEsMovedOut:

type: object

additionalProperties:

$ref: '#/components/schemas/MovTimingInfo'

minProperties: 1

description: >

Contains the timing information of the VAL UE(s) that moved out of the geofencing area

since the previous notification.

The key of the map shall be set to the value of the identifier of the VAL UE (among the

ones provided within the "moveOutUEs" attribute) to which the timing information

provided within the map value is related.

PartialEventSubscFailRep:

description: Represents the partial failure report during the subscription creation or update.

type: object

properties:

valTgtUes:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: >

List of VAL user(s) / VAL UE(s) whose identifier(s) is not found.

valGrpIds:

type: array

items:

type: string

minItems: 1

description: >

List of VAL group(s) whose identifier(s) is not found.

oneOf:

- required: [valTgtUes]

- required: [valGrpIds]

DataStatisticsReq:

description: >

Represents the required information when the target UE's location data is calculated.

type: object

properties:

timeReqs:

$ref: '#/components/schemas/TimeReqs'

locReqs:

type: array

items:

$ref: 'TS29549_SS_NetworkResourceAdaptation.yaml#/components/schemas/GeoArea'

minItems: 1

anyOf:

- required: [timeReqs]

- required: [locReqs]

TimeReqs:

description: >

Represents the time information when the target UE's location data is calculated.

type: object

properties:

timeWindows:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

timeSchedule:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

oneOf:

- required: [timeWindows]

- required: [timeSchedule]

GeofencingConditions:

description: Represents the geofencing conditions.

type: object

properties:

plmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

roamingInd:

type: boolean

description: Represents the roaming status.

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

locInfo:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

schedules:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

refUECap:

$ref: '#/components/schemas/RefUeCap'

MovTimingInfo:

description: Represents the timing information of a VAL UE's movements.

type: object

properties:

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

anyOf:

- required: [timestamp]

- required: [duration]

SatSfInfo:

description: Represents the event report of satellite SF information.

type: object

properties:

satSfTrigger:

$ref: '#/components/schemas/SatSfTrigger'

tgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

ueSfMode:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/UeStrAndFwdStatus'

estTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

timeStamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- satSfTrigger

- tgtUe

RefUeCap:

description: >

Represents the capabilities of the reference UE.

type: object

properties:

refUECap:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

required:

- refUECap

# Simple data types and Enumerations

SEALEvent:

anyOf:

- type: string

enum:

- LM_LOCATION_INFO_CHANGE

- GM_GROUP_INFO_CHANGE

- CM_USER_PROFILE_CHANGE

- GM_GROUP_CREATE

- NRM_MONITOR_UE_USER_EVENTS

- LM_LOCATION_DEVIATION_MONITOR

- GM_TEMP_GROUP_FORMATION

- LM_LOCATION_AREA_MONITOR

- GM_GROUP_DELETION

- NRM_SAT_SF_INFO

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the type of SEAL events that can be subscribed.

Possible values are:

- LM_LOCATION_INFO_CHANGE: Events related to the location information of VAL Users or

VAL UEs from the Location Management Server.

- GM_GROUP_INFO_CHANGE: Events related to the modification of VAL group membership

and configuration information from the Group Management Server.

- CM_USER_PROFILE_CHANGE: Events related to update of user profile information from

the Configuration Management Server.

- GM_GROUP_CREATE: Events related to creation of new VAL groups from the Group

Mananagement Server.

- NRM_MONITOR_UE_USER_EVENTS: Monitoring and analytic events related to VAL UEs,

users or VAL group, from the Network Resource Management Server.

- LM_LOCATION_DEVIATION_MONITOR: Events from Location Management server,

related to the deviation of the VAL User(s) / UE(s) location from an area of interest.

- GM_TEMP_GROUP_FORMATION: Events related to the formation of new temporary VAL groups

from the Group Management Server.

- LM_LOCATION_AREA_MONITOR: Events from Location Management server, related to the list

of UEs moving in or moving out of the specific location.

- GM_GROUP_DELETION: Events related to deletion of existing VAL Group(s) from the GM Server.

- NRM_SAT_SF_INFO: Events related to satellite S&F information.

LocDevNotification:

anyOf:

- type: string

enum:

- NOTIFY_MISMATCH_LOCATION

- NOTIFY_ABSENCE

- NOTIFY_PRESENCE

- NOTIFY_UNKNOWN

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

Enumeration of location deviation notification reports.

- NOTIFY_MISMATCH_LOCATION: This value indicates that the location information of

the VAL UE(s) from the SEAL LM client and the core network are not matching.

- NOTIFY_ABSENCE: This value indicates that the current location information of

the VAL UE(s)is deviating from the VAL server's area of interest.

- NOTIFY_PRESENCE: This value indicates that the current location information of

the VAL UE(s) is within the VAL server's area of interest.

- NOTIFY_UNKNOWN: This value indicates that the LM server cannot determine

whether the target VAL UE current location is within the VAL server's

dynamic area of interest information or not.

MonLocTriggerEvent:

anyOf:

- type: string

enum:

- DISTANCE_TRAVELLED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Identifies the triggering event in the location area monitor filtering.

Possible values are:

- DISTANCE_TRAVELLED: This value indicates the trigger event for the location area

monitoring based on the distance travelled by the reference UE.

SatSfTrigger:

anyOf:

- type: string

enum:

- UE_SF_MODE

- EST_DELIVERY_TIME

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Identifies the triggering event in the location area monitor filtering.

Possible values are:

- UE_SF_MODE: Indicates that the triggering condition is the UE status in S&F mode.

- EST_DELIVERY_TIME: Indicates that the triggering condition is the DL S&F estimated

delivery time.

# A.7	SS_KeyInfoRetrieval API

openapi: 3.0.0

info:

title: SS_KeyInfoRetrieval

description: |

API for SEAL Key Information Retrieval.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.3.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.549 V19.1.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-kir/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/key-records:

get:

description: Retrieve key management information specific to VAL service.

operationId: RetrieveKeyMgmtInfo

tags:

- Key Records (Collection)

parameters:

- name: val-service-id

in: query

description: String identifying an individual VAL service

required: true

schema:

type: string

- name: val-tgt-ue

in: query

description: Identifying a VAL target.

required: false

content:

application/json:

schema:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

responses:

'200':

description: The key management information of the VAL service, VAL User or VAL UE.

content:

application/json:

schema:

$ref: '#/components/schemas/ValKeyInfo'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ValKeyInfo:

description: >

Represents key management information associated with VAL server, VAL user or VAL UE.

type: object

properties:

userUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

skmsId:

type: string

description: String identifying the key management server.

valService:

type: string

description: Unique identifier of a VAL Service.

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

keyInfo:

type: string

description: Key management information specific to VAL service, VAL User or VAL UE.

required:

- userUri

- valService

- keyInfo

# A.8	SS_LocationAreaInfoRetrieval API

openapi: 3.0.0

info:

title: SS_LocationAreaInfoRetrieval

description: |

API for SEAL Location Area Info Retrieval.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0-alpha.2"

externalDocs:

description: >

3GPP TS 29.549 V19.2.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-lair/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/location-retrievals:

get:

description: >

Retrieve the UE(s) information in an application defined proximity range of a location.

operationId: RetrieveUeLocInfo

tags:

- Location Information (Collection)

parameters:

- name: location-info

in: query

description: Location information around which the UE(s) information is requested.

required: true

content:

application/json:

schema:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

- name: val-svc-area-id

in: query

description: >

The val service area identifier around which which the UE(s) information is requested.

schema:

$ref: 'TS29549_SS_VALServiceAreaConfiguration.yaml#/components/schemas/ValSvcAreaId'

- name: range

in: query

description: >

The range information over which the UE(s) information is required,

expressed in meters.

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

responses:

'200':

description: >

The UE(s) information in an application defined proximity range of a location.

content:

application/json:

schema:

type: array

items:

$ref: 'TS29549_SS_Events.yaml#/components/schemas/LMInformation'

minItems: 0

description: >

The UE(s) information in an application defined proximity range of a location.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

# A.9	Void

# A.10	SS_NetworkResourceMonitoring API

openapi: 3.0.0

info:

title: SS_NetworkResourceMonitoring

description: |

API for SEAL Network Resource Monitoring.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0-alpha.2"

externalDocs:

description: >

3GPP TS 29.549 V19.1.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-nrm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

summary: Create individual unicast monitoring subscription resource or obtain unicast QoS monitoring data for VAL UEs, VAL Group, or VAL Streams.

operationId: SubscribeUnicastMonitoring

tags:

- Unicast Monitoring Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringSubscription'

responses:

'201':

description: >

The requested individual monitoring subscription resource is successfully created

and a representation of the created resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringSubscription'

headers:

Location:

description: Contains the URI of the newly created individual monitoring resource.

required: true

schema:

type: string

'200':

description: The requested unicast QoS monitoring data is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringReport'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

NotifyUnicastMonitoringData:

'{$request.body#/notifUri}':

post:

summary: Notify on updates of the individual monitoring resorce accoring the requested reporting settings.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringReport'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

delete:

summary: Remove an existing individual unicast monitoring subscription resource according to the subscriptionId.

operationId: UnsubscribeUnicastMonitoring

tags:

- Individual Unicast Monitoring Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual unicast monitoring subscription resource.

required: true

schema:

type: string

responses:

'204':

description: >

The Individual Unicast Monitoring Subscription resource matching the

subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

summary: Read an existing individual unicast monitoring subscription resource according to the subscriptionId.

operationId: ReadUnicastMonitoringSubscription

tags:

- Individual Unicast Monitoring Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual unicast monitoring subscription resource.

required: true

schema:

type: string

responses:

'200':

description: The requested individual unicast monitoring subscription returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringSubscription'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: >

Update an individual unicast monitoring subscription identified by the subscriptionId.

operationId: UpdateUnicastMonitoring

tags:

- Individual Unicast Monitoring Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual unicast monitoring subscription resource.

required: true

schema:

type: string

requestBody:

description: Updated details of the unicast QoS monitoring subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringSubscription'

responses:

'200':

description: >

The subscription is updated successfully, and the updated subscription

information returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringSubscription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: >

Modify an individual unicast monitoring subscription identified

by the subscriptionId.

operationId: ModifyUnicastMonitoring

tags:

- Individual Unicast Monitoring Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual unicast monitoring subscription resource.

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MonitoringSubscriptionPatch'

responses:

'200':

description: >

Individual individual unicast QoS monitoring subscription resource is modified

successfully and representation of the modified individual unicast QoS monitoring

subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MonitoringSubscription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

MonitoringReport:

description: Indicates the monitoring information for VAL UEs list, VAL Group, or VAL Stream.

type: object

properties:

valUeIds:

type: array

minItems: 1

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

description: List of VAL UEs whose QoS monitoring data is requested.

valGroupId:

type: string

description: The VAL Group Id which QoS monitoring data is requested.

valStreamIds:

type: array

minItems: 1

items:

type: string

description: List of VAL streams for which QoS monitoring data is requested.

measData:

$ref: '#/components/schemas/MeasurementData'

termCause:

$ref: '#/components/schemas/TerminationMode'

failureRep:

type: array

items:

$ref: '#/components/schemas/FailureReport'

description: >

The failure report indicating the VAL UE(s) or VAL Stream ID(s) whose measurement

data is not obtained successfully.

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

monProfId:

$ref: '#/components/schemas/MonProfId'

required:

- measData

- timestamp

oneOf:

- required: [valUeIds]

- required: [valGroupId]

- required: [valStreamIds]

MeasurementData:

description: Presents the aggregated measurement data.

type: object

properties:

dlDelay:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

ulDelay:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

rtDelay:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgPlr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

avgDataRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxDataRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

avrDlTrafficVol:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avrUlTrafficVol:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

anyOf:

- required: [dlDelay]

- required: [ulDelay]

- required: [rtDelay]

- required: [avgPlr]

- required: [avgDataRate]

- required: [maxDataRate]

- required: [avrDlTrafficVol]

- required: [avrUlTrafficVol]

MeasurementPeriod:

description: >

Indicates the measurement time period.

type: object

properties:

measStartTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

measDuration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

required:

- measStartTime

- measDuration

ReportingRequirements:

description: Indicates the requested frequency of reporting.

type: object

properties:

reportingMode:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/NotificationMethod'

reportingPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

reportingThrs:

type: array

items:

$ref: '#/components/schemas/ReportingThreshold'

minItems: 1

immRep:

type: boolean

repTerminMode:

$ref: '#/components/schemas/TerminationMode'

expirationTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

maxNumRep:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

termThr:

$ref: '#/components/schemas/MeasurementData'

termThrMode:

$ref: '#/components/schemas/ThresholdHandlingMode'

required:

- reportingMode

FailureReport:

description: >

Represents the failure report indicating the VAL UE(s) or VAL Stream ID(s)

for which the NRM server failed to obtain the requested data.

type: object

properties:

valUeIds:

type: array

minItems: 1

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

description: >

List of VAL UE(s) whose measurement data is not obtained successfully.

valStreamIds:

type: array

minItems: 1

items:

type: string

description: >

List of VAL stream ID(s) whose measurement data is not obtained successfully.

failureReason:

$ref: '#/components/schemas/FailureReason'

measDataType:

$ref: '#/components/schemas/MeasurementDataType'

required:

- measDataType

MeasurementRequirements:

description: Indicates the measurement requirements.

type: object

properties:

measDataTypes:

type: array

items:

$ref: '#/components/schemas/MeasurementDataType'

minItems: 1

description: Indicates the required the QoS measurement data types.

measAggrGranWnd:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AverWindow'

measPeriod:

$ref: '#/components/schemas/MeasurementPeriod'

required:

- measDataTypes

MonitoringSubscription:

description: The unicast monitoring subscription request.

type: object

properties:

valUeIds:

description: List of VAL UEs whose QoS monitoring data is requested.

type: array

minItems: 1

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

valGroupId:

type: string

description: The VAL Group Id which QoS monitoring data is requested.

valStreamIds:

type: array

minItems: 1

items:

type: string

description: List of VAL streams for which QoS monitoring data is requested.

valServId:

type: string

description: Contains the VAL service identifier.

monProfId:

$ref: '#/components/schemas/MonProfId'

measReqs:

$ref: '#/components/schemas/MeasurementRequirements'

monRep:

$ref: '#/components/schemas/MonitoringReport'

reportReqs:

$ref: '#/components/schemas/ReportingRequirements'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

reqTestNotif:

type: boolean

wsNotifCfg:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

oneOf:

- required: [valUeIds]

- required: [valGroupId]

- required: [valStreamIds]

ReportingThreshold:

description: >

Indicates the requested reporting termination threshold for the measurement index(es).

type: object

properties:

measThrValues:

$ref: '#/components/schemas/MeasurementData'

thrDirection:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- measThrValues

- thrDirection

MonitoringSubscriptionPatch:

description: Represents the monitoring subscription modification request.

type: object

properties:

measReqs:

$ref: '#/components/schemas/MeasurementRequirements'

reportReqs:

$ref: '#/components/schemas/ReportingRequirements'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

monProfId:

$ref: '#/components/schemas/MonProfId'

# Simple data types and Enumerations

MonProfId:

description: Represents the monitoring profile identifier.

type: string

MeasurementDataType:

anyOf:

- type: string

enum:

- DL_DELAY

- UL_DELAY

- RT_DELAY

- AVG_PLR

- AVG_DATA_RATE

- MAX_DATA_RATE

- AVG_DL_TRAFFIC_VOLUME

- AVG_UL_TRAFFIC_VOLUME

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Indicates the requested measurement data type.

Possible values are:

- DL_DELAY: Downlink packet delay.

- UL_DELAY: Uplink packet delay.

- RT_DELAY: Round trip packet delay.

- AVG_PLR: Average packet loss rate.

- AVG_DATA_RATE: Average data rate.

- MAX_DATA_RATE: Maximum data rate.

- AVG_DL_TRAFFIC_VOLUME: Average downlink traffic volume.

- AVG_UL_TRAFFIC_VOLUME: Average uplink traffic volume.

TerminationMode:

anyOf:

- type: string

enum:

- TIME_TRIGGERED

- EVENT_TRIGGERED_NUM_REPORTS_REACHED

- EVENT_TRIGGERED_MEAS_THR_REACHED

- USER_TRIGGERED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Indicates the termination mode.

Possible values are:

- TIME_TRIGGERED: Time-triggered termination mode.

- EVENT_TRIGGERED_NUM_REPORTS_REACHED: Event-triggered termination number of reports

reached mode.

- EVENT_TRIGGERED_MEAS_THR_REACHED: The event-triggered termination measurement index

threshold reached mode.

- USER_TRIGGERED: User-triggered termination mode.

FailureReason:

anyOf:

- type: string

enum:

- USER_NOT_FOUND

- STREAM_NOT_FOUND

- DATA_NOT_AVAILABLE

- OTHER_REASON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the failure reason.

Possible values are:

- USER_NOT_FOUND: The user is not found.

- STREAM_NOT_FOUND: The stream is not found.

- DATA_NOT_AVAILABLE: The requested data is not available.

- OTHER_REASON: Other reason (unspecified).

ThresholdHandlingMode:

anyOf:

- type: string

enum:

- ALL_REACHED

- ANY_REACHED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Indicates the multi-parameter threshold handling mode.

Possible values are:

- ALL_REACHED: The decision criterion is met when all the provided thresholds are reached.

- ANY_REACHED: The decision criterion is met when any of the provided threshold(s)

is reached.

# A.11	SS_VALServiceData API

openapi: 3.0.0

info:

title: SS_VALServiceData

version: 1.0.0

description: |

API for VAL Service Data.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V18.6.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-vsd/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/val-servdata:

get:

summary: Retrieve VAL service data.

operationId: RetrieveValServData

tags:

- VAL Service Data Sets (Collection)

parameters:

- name: val-tgt-ues

in: query

description: Identifying the list of the target VAL UE(s).

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: val-tgt-users

in: query

description: Identifying the list of the target VAL user(s).

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: val-service-ids

in: query

description: Identifies the list of the target VAL service(s).

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: supp-feats

in: query

description: >

Contains the list of supported features among the ones defined in clause 7.3.2.7..

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK. The requested VAL Service Data shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ValServDataResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ValServDataResp:

description: Represents the response to a VAL service data retrieval request.

type: object

properties:

valServData:

type: array

items:

$ref: '#/components/schemas/ValServiceData'

minItems: 0

suppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- valServData

ValServiceData:

description: >

Represents the VAL service data.

type: object

properties:

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

valServIds:

type: array

items:

type: string

minItems: 1

description: >

List of the VAL services associated with the VAL user or a VAL UE defined

in the "valTgtUe" attribute.

valServSpecInfo:

type: string

required:

- valTgtUe

- valServIds

# A.12	SS_VALServiceAreaConfiguration API

openapi: 3.0.0

info:

title: SS_VALServiceAreaConfiguration

description: |

API for SEAL VAL Service Area Configuration Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0-alpha.2"

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-vsac/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/areas:

get:

summary: Obtain the VAL service area(s) according to the provided filtering criteria.

operationId: ObtainValServiceAreas

tags:

- VAL Service Areas (Collection)

parameters:

- name: val-svc-area-ids

in: query

description: Represents the requested VAL service areas.

required: false

schema:

type: array

items:

$ref: '#/components/schemas/ValSvcAreaId'

minItems: 1

- name: supp-feats

in: query

description: To filter irrelevant responses related to unsupported features.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: The requested VAL service areas information is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaData'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/areas/configure:

post:

summary: Configure VAL service area(s).

operationId: ConfigureValServiceAreas

tags:

- Configure

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaReq'

responses:

'200':

description: >

Successful case. The identifier(s) of the configured VAL service area(s) information are

Returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaResp'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/areas/update:

post:

summary: Update existing VAL service area(s).

operationId: UpdateValServiceAreas

tags:

- Update

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaReq'

responses:

'200':

description: >

Successful case. The identifier(s) of the updated VAL service area(s) information are

returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaResp'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/areas/delete:

post:

summary: Delete existing VAL service area(s).

operationId: DeleteValServiceAreas

tags:

- Delete

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaReq'

responses:

'200':

description: >

Successful case. The identifier(s) of the deleted VAL service area(s) information are

returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaResp'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Create individual VAL service area change event(s) subscription.

operationId: SubscribeChangeEvents

tags:

- VAL Service Area Change Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaSubsc'

responses:

'201':

description: >

The requested individual VAL service area change event(s) subscription

resource is successfully created and a representation of the created

resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaSubsc'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

NotifyValServiceAreaChange:

'{$request.body#/notifUri}':

post:

summary: Notify on changes of the VAL service area(s) accodring to the requested reporting settings.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual VAL service area change event(s)

subscription resource.

required: true

schema:

type: string

get:

summary: Read an existing individual unicast monitoring subscription resource according to the subscriptionId.

operationId: ReadValServiceAreaChange

tags:

- Individual VAL Service Area Change Subscription (Document)

responses:

'200':

description: >

The requested individual VAL service area change event(s) subscription is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaSubsc'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update individual VAL service area change event(s) subscription.

operationId: UpdateIndValServAreaChangeSubsc

tags:

- Individual VAL Service Area Change Subscription (Document)

requestBody:

description: Updated details of the unicast VAL service area change event(s) subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaSubsc'

responses:

'200':

description: >

The subscription is updated successfully, and the updated subscription

information returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaSubsc'

'204':

description: No Content. The subscription is updated successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify the individual VAL service area change event(s) subscription.

operationId: ModifyIndValServAreaChangeSubsc

tags:

- Individual VAL Service Area Change Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ValServiceAreaSubscPatch'

responses:

'200':

description: >

The individual VAL service area change event(s) subscription is

modified successfully, and the representation of the modified resource

is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ValServiceAreaSubsc'

'204':

description: No Content. The subscription is modified successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the existing individual VAL service area change event(s) subscription resource according to the subscriptionId.

operationId: UnsubscribeValServiceAreaChange

tags:

- Individual VAL Service Area Change Subscription (Document)

responses:

'204':

description: >

The individual VAL service area change event(s) subscription resource

matching the subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ValServiceArea:

description: Represents the VAL service area.

type: object

properties:

valSvcAreaId:

$ref: '#/components/schemas/ValSvcAreaId'

locations:

type: array

description: Represents the locations associated with the VAL service area.

minItems: 1

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

required:

- valSvcAreaId

- locations

ValServiceAreaReq:

description: Represents the VAL service area configuration/update/delete request.

type: object

properties:

valSvcAreas:

type: array

description: Represents the VAL service area(s).

minItems: 1

items:

$ref: '#/components/schemas/ValServiceArea'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- valSvcAreas

ValServiceAreaData:

description: Represents the VAL service area retrieval information.

type: object

properties:

valSvcAreas:

type: array

description: Represents the requested VAL service area(s).

minItems: 1

items:

$ref: '#/components/schemas/ValServiceArea'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ValServiceAreaResp:

description: Represents the VAL service area configuration/update/delete response.

type: object

properties:

valSvcAreaIds:

type: array

description: Represents the identifier(s) of the successfully handled VAL service area(s).

minItems: 1

items:

$ref: '#/components/schemas/ValSvcAreaId'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- valSvcAreaIds

ValServiceAreaSubscPatch:

description: Represents the VAL service area change event(s) modification request.

type: object

properties:

events:

type: array

description: Represents the subscribed VAL service area change event(s).

items:

$ref: '#/components/schemas/ValServiceAreaEventType'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

subscDur:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

ValSvcAreaId:

type: string

description: >

Represents the VAL Service Area identifier encoded as a string and generated

either based on VAL Server ID or using the Universally Unique Identifier (UUID)

version 4 as described in IETF RFC 9562.

ValServiceAreaSubsc:

description: Represents the VAL service area change event(s) subscription.

type: object

properties:

events:

type: array

description: Represents the subscribed VAL service area change event(s).

minItems: 1

items:

$ref: '#/components/schemas/ValServiceAreaEventType'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

subscDur:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- events

- notifUri

ValServiceAreaEventType:

description: Represents the VAL service area change event type.

type: object

properties:

event:

$ref: '#/components/schemas/ValServiceAreaEvent'

valSvcAreaIds:

type: array

description: Represents the VAL service area ID(s) associated with the event.

minItems: 1

items:

$ref: '#/components/schemas/ValSvcAreaId'

required:

- event

- valSvcAreaIds

ValServiceAreaNotif:

description: Represents the VAL service area change event(s) notifcation.

type: object

properties:

valSvcAreaConts:

description: Represents the VAL service area change event(s) content.

minItems: 1

items:

$ref: '#/components/schemas/ValServiceAreaEventInfo'

required:

- valSvcAreaConts

ValServiceAreaEventInfo:

description: Represents the VAL service area change event(s) content.

type: object

properties:

event:

$ref: '#/components/schemas/ValServiceAreaEvent'

valSvcAreas:

type: array

description: Represents the VAL service area(s) associated with the event.

minItems: 1

items:

$ref: '#/components/schemas/ValServiceArea'

valSvcAreaIds:

type: array

description: Represents the VAL service area(s) associated with the event.

minItems: 1

items:

$ref: '#/components/schemas/ValSvcAreaId'

required:

- event

# Simple data types and Enumerations

ValServiceAreaEvent:

anyOf:

- type: string

enum:

- UPDATE

- DELETE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the VAL service area change event.

Possible values are:

- UPDATE: Indicates that the VAL service area change event is VAL service area update.

- DELETE: Indicates that the VAL service area change event is VAL service area delete.

# A.13	SS_IdmParameterProvisioning API

openapi: 3.0.0

info:

title: SS_IdmParameterProvisioning

version: 1.0.0

description: |

API for SEAL Identity management.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V18.6.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-ipp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/configurations:

post:

description: Provisions VAL Services configuration information.

operationId: CreateValServiceConf

tags:

- VAL Services Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VALServicesConfig'

responses:

'201':

description: VAL services configuration created sucessfully.

content:

application/json:

schema:

$ref: '#/components/schemas/VALServicesConfig'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

description: Retrieves VAL services configuration satisfying filter criteria

operationId: RetrieveValServiceConf

tags:

- VAL Services Configurations (Collection)

parameters:

- name: val-server-id

in: query

description: String identifying the VAL server.

required: false

schema:

type: string

- name: config-ids

in: query

description: Identifying the list of the val services configuration resources.

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: supp-feats

in: query

description: >

Contains the list of supported features among the ones defined in clause 7.8.1.7.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The requested VAL services configuration(s) matching the received query parameter(s)

shall be returned.

If there are no active VAL services configuration(s) matching the received query

parameter(s), an empty array is returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/VALServicesConfig'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/configurations/{confId}:

get:

description: Retrieves Individual VAL services configuration information.

operationId: RetrieveIndValServicesConf

tags:

- Individual VAL Services Configuration (Document)

parameters:

- name: confId

in: path

description: String identifying an individual VAL services configuration resource.

required: true

schema:

type: string

responses:

'200':

description: The whole Individual VAL services configuration resource.

content:

application/json:

schema:

$ref: '#/components/schemas/VALServicesConfig'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Updates an individual VAL services configuration resource.

operationId: UpdateIndValServicesConf

tags:

- Individual VAL Services Configuration (Document)

parameters:

- name: confId

in: path

description: String identifying an individual VAL services configuration resource

required: true

schema:

type: string

requestBody:

description: VAL services configuration to be updated in IM server.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VALServicesConfig'

responses:

'200':

description: VAL services configuration updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/VALServicesConfig'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modify an existing VAL services configuration.

operationId: ModifyIndValServicesConf

tags:

- Individual VAL Services Configuration (Document)

parameters:

- name: confId

in: path

description: Identifier of an individual VAL services configuration resource.

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/VALServicesConfigPatch'

responses:

'200':

description: >

The individual VAL services configuration resource is modified successfully and a

representation of the updated VAL services configuration resource is returned in

the request body.

content:

application/json:

schema:

$ref: '#/components/schemas/VALServicesConfig'

'204':

description: No Content. The individual VAL services configuration is modified successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes a VAL Services Configuration.

operationId: DeleteIndValServicesConf

tags:

- Individual VAL Services Configuration (Document)

parameters:

- name: confId

in: path

description: String identifying an individual VAL Service Configuration resource.

required: true

schema:

type: string

responses:

'204':

description: The individual VAL services configuration matching confId was deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

VALServicesConfig:

description: Represents a VAL services configuration.

type: object

properties:

valSvcConf:

type: array

items:

$ref: '#/components/schemas/VALServiceParams'

minItems: 1

description: The list of the provisioned VAL services configuration parameters.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- valSvcConf

VALServiceParams:

description: Represents VAL services configuration information.

type: object

properties:

valServiceId:

type: string

description: The identifier of the VAL service.

idList:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: The list of VAL User IDs or VAL UE IDs.

required:

- valServiceId

- idList

VALServicesConfigPatch:

description: >

Represents of the requested modifications to a VAL services configuration.

type: object

properties:

valSvcConf:

type: array

description: The list of the provisioned VAL services configuration parameters.

items:

$ref: '#/components/schemas/VALServiceParams'

minItems: 1

# A.14	SS_KMParametersProvisioning API

openapi: 3.0.0

info:

title: SEAL KM Server Key Management Parameters Provisioning Service

version: "1.0.0"

description: |

SEAL KM Server Key Management Parameters Provisioning Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V18.6.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-kpp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/request:

post:

summary: Enables a service consumer to request key parameters provisioning to the SEAL KM Server..

operationId: Request

tags:

- Key Management Parameters Provisioning Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/VALKeyPpReq'

responses:

'200':

description: >

OK. The requested key management parameters are successfully received, processed and

provisioned.

content:

application/json:

schema:

$ref: '#/components/schemas/VALKeyPpResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

VALKeyPpReq:

description: Represents the key management parameters to be provisioned.

type: object

properties:

skmcUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

valServiceId:

type: string

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

payloadId:

type: string

payload:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- skmcUri

- valServiceId

VALKeyPpResp:

description: Represents the response to a key management parameters provisioning request.

type: object

properties:

skmcUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

valServiceId:

type: string

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

payloadId:

type: string

skmsId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- skmcUri

- valServiceId

# A.15	SS_ADAE_VALPerformanceAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_VALPerformanceAnalytics

description: |

API for ADAE VAL performance analytics service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0-alpha.2"

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-pa/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/application-performance:

post:

description: Creates a new individual VAL performance event subscription.

operationId: CreateIndValPerfEventSubsc

tags:

- VAL performance event subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AppPerfSub'

callbacks:

paNotification:

'{$request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AppPerfNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: VAL performance event subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/AppPerfSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/application-performance/{appPerfId}:

get:

description: Retrieves an individual VAL performance event subscription.

operationId: ReadIndValPerfEventSubsc

tags:

- Individual VAL performance event subscription (Document)

parameters:

- name: appPerfId

in: path

description: Identifier of an individual VAL performance event subscription.

required: true

schema:

type: string

responses:

'200':

description: >

The individual VAL performance subscription matching the appPerfId is retrieved.

content:

application/json:

schema:

$ref: '#/components/schemas/AppPerfSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual VAL performance event subscription.

operationId: DeleteIndValPerfEventSubsc

tags:

- Individual VAL performance event subscription (Document)

parameters:

- name: appPerfId

in: path

description: Identifier of an individual VAL performance event subscription.

required: true

schema:

type: string

responses:

'204':

description: >

The individual VAL performance subscription matching the appPerfId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AppPerfSub:

description: Represents an individual VAL performance event subscription resource.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

analyticsType:

$ref: '#/components/schemas/AnalyticsType'

analyticsId:

$ref: '#/components/schemas/AnalyticsIdVpa'

valServiceId:

type: string

valUeIds:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: >

One or more VAL UE IDs whose analytics or data collection are subject to subcription.

valServerId:

type: string

description: Identity of the VAL server

dataProdProfile:

$ref: '#/components/schemas/ProdProfileInfo'

clientAppData:

$ref: '#/components/schemas/ClientApplicationData'

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

repThrs:

type: array

items:

$ref: '#/components/schemas/PerfThreshold'

minItems: 1

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

expReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- analyticsType

- valServiceId

AppPerfNotif:

description: Represents notification of the VAL performance event.

type: object

properties:

analyticsOutput:

$ref: '#/components/schemas/AnalyticsOutput'

output:

type: string

description: Provided output for analytics.

valServerId:

type: string

description: VAL Server identifier.

valUeIds:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: List of VAL UE identifiers.

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeHorizon:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

required:

- valServerId

oneOf:

- required: [output]

- required: [analyticsOutput]

ProdProfileInfo:

description: Represents capability of the data producer.

type: object

properties:

prodId:

type: string

description: Identity of the data producer.

prodType:

$ref: '#/components/schemas/ProducerType'

dataType:

$ref: '#/components/schemas/ProducerData'

prodRole:

$ref: '#/components/schemas/ProducerRole'

origProdIds:

type: array

items:

type: string

minItems: 1

description: >

Identifies the identity of the original data producers.

dataFresh:

type: integer

description: Duration of elapsed time in seconds.

producerCap:

$ref: '#/components/schemas/ProducerCap'

required:

- prodId

- prodType

- dataType

ProducerCap:

description: Represents capability of the data producer.

type: object

properties:

durationTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

anonymization:

type: boolean

description: >

Set to true if anonymization is supported. Set to false or omitted otherwise.

dataRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

schedule:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

AnalyticsType:

description: Represents type of requested analytics.

type: object

properties:

category:

$ref: '#/components/schemas/AnalyticsCategory'

required:

- category

PerfData:

description: Represents the performance data.

type: object

properties:

rtt:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgDataRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxDataRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

jitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

qoeMos:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

qoeStalEvent:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

qoeStalRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

qosProfileLoad:

type: integer

minimum: -100

maximum: 100

valTrafLoad:

type: integer

minimum: -100

maximum: 100

valSesPerfChange:

type: integer

minimum: -100

maximum: 100

anyOf:

- required: [rtt]

- required: [avgDataRate]

- required: [maxDataRate]

- required: [jitter]

- required: [qoeMos]

- required: [qoeStalEvent]

- required: [qoeStalRatio]

- required: [qosProfileLoad]

- required: [valTrafLoad]

- required: [valSesPerfChange]

PerfThreshold:

description: >

Represents the performance threshold.

type: object

properties:

value:

$ref: '#/components/schemas/PerfData'

thrDirect:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- value

- thrDirect

ClientApplicationData:

description: >

Represents the ADAE client application data.

type: object

properties:

ueRoute:

$ref: 'TS29257_UAE_ChangeUSSManagement.yaml#/components/schemas/UasRoute'

sesSched:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

anyOf:

- required: [ueRoute]

- required: [sesSched]

AnalyticsOutput:

description: >

Represents the VAL performance analytics output.

type: object

properties:

analyticsId:

$ref: '#/components/schemas/AnalyticsIdVpa'

valServ:

$ref: '#/components/schemas/PerfData'

valSess:

$ref: '#/components/schemas/PerfData'

required:

- analyticsId

# Simple data types and Enumerations

AnalyticsCategory:

anyOf:

- type: string

enum:

- PREDICTIVE

- STATISTICS

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the type of the analytics.

Possible values are:

- PREDICTIVE: The event for the analytics is for predictive analytics.

- STATISTICS: The event for analytics is for statistics analytics.

ProducerType:

anyOf:

- type: string

enum:

- ADAE_CLIENT

- A_DCCF

- VAL_SERVER

- SEAL_SERVER

- SEAL_CLIENT

- EES

- EAS

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the type of the data producer.

Possible values are:

- ADAE_CLIENT: The data producer is ADAE client.

- A_DCCF: The data producer is A-DCCF.

- VAL_SERVER: The data producer is VAL server.

- SEAL_SERVER: The data producer is SEAL server.

- SEAL_CLIENT: The data producer is SEAL client.

- EES: The data producer is EES.

- EAS: The data producer is EAS.

ProducerData:

anyOf:

- type: string

enum:

- PERFORMANCE_INDICATOR

- REPRODUCER_USAGE_DATA

- SERVER_LOAD_DATA

- APPLICATION_PERFORMANCE

- EDGE_LOAD

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the data type of the data producer.

Possible values are:

- PERFORMANCE_INDICATOR: The data type of the data producer is

performance indictor.

- REPRODUCER_USAGE_DATA: The data type of the data producer is

reproducer usage data.

- SERVER_LOAD_DATA: The data type of the data producer is

server load data.

- APPLICATION_PERFORMANCE: The data type of the data producer is

application performance.

- EDGE_LOAD: The data type of the data producer is

edge load.

ProducerRole:

anyOf:

- type: string

enum:

- GENERATING_ENTITY

- ORIGINAL_PRODUCER

- RESPOSITORY

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the role of the data producer.

Possible values are:

- GENERATING_ENTITY: The role of the data producer is generating entity.

- ORIGINAL_PRODUCER: The role of the data producer is origina producer.

- RESPOSITORY: The role of the data producer is repository.

AnalyticsIdVpa:

anyOf:

- type: string

enum:

- VAL_SERVER_PERF

- VAL_SESSION_PERF

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the analytics identifier for VAL performance analytics.

Possible values are:

- VAL_SERVER_PERF: Indicates that requested analytics is VAL server performance analytics.

- VAL_SESSION_PERF: Indicates that requested analytics is VAL session performance analytics.

# A.16	SS_ADAE_SlicePerformanceAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_SlicePerformanceAnalytics

description: |

API for ADAE slice-specific application performance analytics service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-sspa/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/slice-specific-application-performance:

post:

description: >

Creates a new individual slice-specific application performance event subscription.

operationId: SliceAppPerfSubs

tags:

- Slice-specific application performance event subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SliceAppPerfSub'

callbacks:

sspaNotification:

'{$request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SliceAppPerfNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Slice-specific performance event subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/SliceAppPerfSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/slice-specific-application-performance/{ssAppPerfId}:

get:

description: >

Retrieves an individual slice-specific application performance event subscription.

operationId: ReadIndSliceAppPerfEventSubsc

tags:

- Individual slice-specific application performance event subscription (Document)

parameters:

- name: ssAppPerfId

in: path

description: >

Identifier of an individual slice-specific application performance event subscription.

required: true

schema:

type: string

responses:

'200':

description: >

The individual slice-specific application performance

subscription matching the ssAppPerfId is retrieved.

content:

application/json:

schema:

$ref: '#/components/schemas/SliceAppPerfSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual slice-specific application performance event subscription.

operationId: DeleteIndSliceAppPerfEventSubsc

tags:

- Individual slice-specific application performance event subscription (Document)

parameters:

- name: ssAppPerfId

in: path

description: >

Identifier of an individual slice-specific application performance event subscription.

required: true

schema:

type: string

responses:

'204':

description: >

The individual slice-specific application performance subscription matching

the ssAppPerfId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SliceAppPerfSub:

description: >

Represents an individual slice-specific application performance event subscription resource.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

sliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

valUeIds:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: >

One or more VAL UE IDs whose analytics are subject to subcription.

valServerId:

type: string

description: >

Identity of the VAL server if the consumer is different than the VAL server of interest.

valServiceId:

type: string

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

timeHorizon:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyticsType

- notifUri

- sliceId

SliceAppPerfNotif:

description: Represents notification of the slice-specific application performance event.

type: object

properties:

analyticsOutput:

type: array

items:

$ref: '#/components/schemas/SliceAppPerfAnalytics'

minItems: 1

output:

type: string

description: Output data.

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeHorizon:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

oneOf:

- required: [output]

- required: [analyticsOutput]

SliceAppPerfAnalytics:

description: >

Represents the analytics output for slice performance analytics.

type: object

properties:

valServerId:

type: string

valServiceId:

type: string

sliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

minRtt:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgRtt:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxRtt:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minLat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgLat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxLat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

allOf:

- anyOf:

- required: [valServerId]

- required: [valServiceId]

- required: [sliceId]

- anyOf:

- required: [minRtt]

- required: [avgRtt]

- required: [maxRtt]

- required: [minLat]

- required: [avgLat]

- required: [maxLat]

# A.17	SS_ADAE_Ue2UePerformanceAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_Ue2UePerformanceAnalytics

description: |

API for ADAE UE-to-UE Performance Analytics Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0-alpha.2"

externalDocs:

description: >

3GPP TS 29.549 V19.1.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-uupa/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/ue2ue-session-performance:

post:

summary: Create an individual UE-to-UE session performance analytics event subscription.

operationId: SubscribeUe2UePerfAnalytics

tags:

- UE-to-UE Performance Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/U2UPerfSub'

responses:

'201':

description: >

Subscription to the UE-to-UE session performance analytics is created.

content:

application/json:

schema:

$ref: '#/components/schemas/U2UPerfSub'

headers:

Location:

description: Contains the URI of the newly created individual resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

NotifyU2USessionEvent:

'{$request.body#/notifUri}':

post:

summary: Notify the UE-to-UE session performance analytics.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/U2UPerfNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/ue2ue-session-performance/{u2uPerfId}:

get:

summary: Read the individual UE-to-UE session performance analytics event subscription.

operationId: ReadU2UPerfSubscription

tags:

- Individual UE-to-UE Performance Subscription (Document)

parameters:

- name: u2uPerfId

in: path

description: >

Represents the identifier of an individual UE-to-UE session performance

analytics subscription.

required: true

schema:

type: string

responses:

'200':

description: >

The requested individual UE-to-UE session performance event subscription is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/U2UPerfSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the individual UE-to-UE session performance analytics event subscription.

operationId: RemoveU2UPerfSubscription

tags:

- Individual UE-to-UE Performance Subscription (Document)

parameters:

- name: u2uPerfId

in: path

description: >

Represents the identifier of an individual UE-to-UE session performance

analytics subscription.

required: true

schema:

type: string

responses:

'204':

description: >

The individual UE-to-UE session performance event subscription matching

the u2uPerfId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

U2UPerfSub:

description: >

Represents the UE-to-UE session performance analytics subscription.

type: object

properties:

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

valUeIds:

type: array

description: >

Represent the list of VAL UEs, whose UE-to-UE session analytics are subscribed to.

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

valServiceId:

description: Represents the VAL service for which the subscription applies.

type: string

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

reportingInds:

type: array

description: Indicates the list of the requested analytics.

items:

$ref: '#/components/schemas/U2UAnalytics'

minItems: 1

reportingThrs:

type: array

description: >

Identifies reporting threshold corresponding to the application QoS index(es).

items:

$ref: '#/components/schemas/U2UThreshold'

minItems: 1

reportingGrn:

$ref: '#/components/schemas/U2UReportingGranularity'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

timeInterval:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyticsType

- valUeIds

- notifUri

U2UPerfNotif:

description: >

Represents the threshold for UE-to-UE session performance analytics.

type: object

properties:

analyticsOutput:

type: array

description: >

Represents the exposure level requirements for the analytics to be exposed.

items:

$ref: '#/components/schemas/U2UAnalyticsData'

minItems: 1

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- analyticsOutput

U2UThreshold:

description: >

Represents the threshold for UE-to-UE session performance analytics.

type: object

properties:

value:

$ref: '#/components/schemas/U2UAnalyticsData'

thrDirect:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- value

- thrDirect

U2UAnalyticsData:

description: >

Represents the UE-to-UE analytics data.

type: object

properties:

valUes:

$ref: '#/components/schemas/U2UPair'

avgLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgPer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

avgDataRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

jitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

U2UPair:

description: >

Represents the UE-to-UE pair.

type: object

properties:

valUeA:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

valUeB:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

required:

- valUeA

- valUeB

# Simple data types and Enumerations

U2UAnalytics:

anyOf:

- type: string

enum:

- AVG_LATENCY

- AVG_PER

- AVG_DATA_RATE

- JITTER

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the UE-to-UE analytics types.

Possible values are:

- AVG_LATENCY: The indication for requesting the average latency analytics.

- AVG_PER: The indication for requesting the average packet error rate analytics.

- AVG_DATA_RATE: The indication for requesting the average data rate analytics.

- JITTER: The indication for requesting the jitter analytics.

U2UReportingGranularity:

anyOf:

- type: string

enum:

- GROUP

- INDIVIDUAL

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the UE-to-UE reporting granularity.

Possible values are:

- GROUP: The indication for requesting the analytics for all

VAL UE-to-UE application sessions.

- INDIVIDUAL: The indication for requesting the analytics for individual

VAL UE-to-UE application sessions.

# A.18	SS_ADAE_LocationAccuracyAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_LocationAccuracyAnalytics

description: |

API for ADAE location accuracy performance analytics service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-laa/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/location-accuracy:

post:

description: >

Creates a new individual location accuracy performance event subscription.

operationId: LocAccurSubs

tags:

- Location accuracy performance event subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocAccurSub'

callbacks:

laaNotification:

'{$request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocAccurNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Location accuracy subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/LocAccurSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/location-accuracy/{locAccId}:

get:

description: Retrieves an individual location accuracy performance event subscription.

operationId: ReadIndLocAccurPerfEventSubsc

tags:

- Individual location accuracy performance event subscription (Document)

parameters:

- name: locAccId

in: path

description: >

Identifier of an individual location accuracy performance event subscription.

required: true

schema:

type: string

responses:

'200':

description: >

The individual location accuracy subscription matching the locAccId is retrieved.

content:

application/json:

schema:

$ref: '#/components/schemas/LocAccurSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual location accuracy performance event subscription.

operationId: DeleteIndLocAccurPerfEventSubsc

tags:

- Individual location accuracy performance event subscription (Document)

parameters:

- name: locAccId

in: path

description: >

Identifier of an individual location accuracy performance event subscription.

required: true

schema:

type: string

responses:

'204':

description: >

The individual location accuracy performance subscription matching the locAccId

is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

LocAccurSub:

description: >

Represents an individual location accuracy performance event subscription resource.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

analyticsId:

$ref: '#/components/schemas/LocAccurAnalytics'

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

valUeIds:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: >

One or more VAL UE IDs whose analytics are subject to subcription.

accuracy:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/Accuracy'

valServiceId:

type: string

description: >

Identifier of the VAL service for which location accuracy analytics is requested.

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

ueMobs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/UeMobility'

minItems: 1

description: Mobility and route information of the target VAL UEs.

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyticsType

- valUeIds

- accuracy

- notifUri

LocAccurNotif:

description: Represents notification of the location accuracy performance event.

type: object

properties:

analyticsOutput:

$ref: '#/components/schemas/LocAccurAnalyticsOutput'

output:

type: string

description: Event output data.

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

oneOf:

- required: [analyticsOutput]

- required: [output]

LocAccurAnalyticsOutput:

description: >

Represents the location accuracy analytics output.

type: object

properties:

analyticsId:

$ref: '#/components/schemas/LocAccurAnalytics'

valUeIds:

type: array

items:

type: string

minItems: 1

valServId:

type: string

predictions:

type: array

items:

$ref: '#/components/schemas/LocAccurPrediction'

minItems: 1

sustainAnalytics:

$ref: '#/components/schemas/LocAccurSustain'

required:

- analyticsId

LocAccurPrediction:

description: >

Represents the location accuracy prediction analytics output.

type: object

properties:

accurChange:

$ref: '#/components/schemas/LocAccurChange'

areas:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

minItems: 1

timeWnd:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

confLevel:

type: integer

minimum: 0

maximum: 100

required:

- accurChange

LocAccurSustain:

description: >

Represents the location accuracy sustainability analytics output.

type: object

properties:

areas:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

minItems: 1

timeWnd:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

confLevel:

type: integer

minimum: 0

maximum: 100

# Simple data types and Enumerations

LocAccurAnalytics:

anyOf:

- type: string

enum:

- LOCATION_ACCURACY_PREDICTION

- LOCATION_ACCURACY_SUSTAINABILITY

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the analytics identifier for location accuracy analytics.

Possible values are:

- LOCATION_ACCURACY_PREDICTION: Indicates that the requested analytics is location

accuracy prediction.

- LOCATION_ACCURACY_SUSTAINABILITY: Indicates that the requested analytics is

location accuracy sustainability.

LocAccurChange:

anyOf:

- type: string

enum:

- DOWNGRADE

- UPGRADE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Rerpresents the type of location accuracy change.

Possible values are:

- DOWNGRADE: Indicates the downgrade of the location accuracy.

- UPGRADE: Indicates the upgrade of the location accuracy.

# A.19	SS_ADAE_ServiceApiAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_ServiceApiAnalytics

description: |

API for ADAE service API analytics service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.1"

externalDocs:

description: >

3GPP TS 29.549 V18.7.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-sa/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/service-api:

post:

description: >

Creates a new individual service API event subscription.

operationId: SrvApiSubs

tags:

- Service API event subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SrvApiSub'

callbacks:

saaNotification:

'{$request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SrvApiNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Service API event subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/SrvApiSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/service-api/{srvApiId}:

get:

description: Retrieves an individual service API event subscription.

operationId: ReadIndSrvApiEventSubsc

tags:

- Individual service API event subscription (Document)

parameters:

- name: srvApiId

in: path

description: >

Identifier of an individual service API event subscription.

required: true

schema:

type: string

responses:

'200':

description: >

The individual service API event subscription matching the srvApiId is retrieved.

content:

application/json:

schema:

$ref: '#/components/schemas/SrvApiSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual service API event subscription.

operationId: DeleteIndSrvApiEventSubsc

tags:

- Individual service API event subscription (Document)

parameters:

- name: srvApiId

in: path

description: >

Identifier of an individual service API event subscription.

required: true

schema:

type: string

responses:

'204':

description: >

The individual service API subscription matching the srvApiId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SrvApiSub:

description: Represents an individual service API event subscription.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

eventCriteria:

type: string

description: Criteria matching service API analytics event.

serviceApiName:

type: string

description: Service API name.

serviceApiType:

type: string

description: Service API type.

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

timeHorizon:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyticsType

- notifUri

- eventCriteria

oneOf:

- required: [serviceApiName]

- required: [serviceApiType]

SrvApiNotif:

description: Represents notification of the service API event.

type: object

properties:

requestorId:

type: string

description: Identifier of the requestor of the analytics.

output:

type: string

description: Output for analytics.

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- requestorId

- output

# A.20	SS_ADAE_SliceUsagePatternAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_SliceUsagePatternAnalytics

description: |

API for ADAE Slice Usage Pattern Analytics.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0"

externalDocs:

description: >

3GPP TS 29.549 V18.6.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-sup/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/slice-usage-pattern:

post:

description: Creates a new Individual slice usage pattern subscription.

operationId: CreateSliceUsageSub

tags:

- Slice Usage Pattern Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SUPSub'

responses:

'201':

description: Slice Usage Pattern subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/SUPSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

SUPNotification:

'{$request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SUPNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/slice-usage-pattern/{subscriptionId}:

get:

description: Retrieves an individual Slice Usage Pattern Subscription.

operationId: ReadSUPSub

tags:

- Individual slice usage pattern subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifier of an individual Events Subscription

required: true

schema:

type: string

responses:

'200':

description: The individual slice usage pattern subscription.

content:

application/json:

schema:

$ref: '#/components/schemas/SUPSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual Slice Usage Pattern Subscription.

operationId: DeleteSUPSub

tags:

- Individual slice usage pattern subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifier of an individual Events Subscription

required: true

schema:

type: string

responses:

'204':

description: >

The individual Slice Usage Pattern Subscription matching the subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SUPSub:

description: Represents an individual Slice Usage Pattern Subscription.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

sliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

valUeIds:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: List of identities of one or more VAL UEs.

valServerId:

type: string

description: VAL server identifier.

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

historicTimeInt:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

immNotif:

$ref: '#/components/schemas/SUPNotif'

dataId:

type: string

description: Identity of the slice usage statistics data which is to be collected.

valServiceId:

type: string

description: VAL service identifier.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- analyticsType

- sliceId

SUPNotif:

description: Represents notification information of a Slice Usage Pattern Event.

type: object

properties:

output:

type: string

description: Implementation-specific string representing the slice usage analytics.

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- output

# A.21	SS_ADAE_EdgeLoadAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_EdgeLoadAnalytics

description: |

API for ADAE Edge Load Analytics.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-el/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/edge-load:

post:

description: Creates a new individual Edge Load Event Subscription.

operationId: EdgeSub

tags:

- Edge Load Events Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EdgeSub'

callbacks:

notifUri:

'{$request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EdgeNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Edge Load Events subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/EdgeSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/edge-load/{edgeLdId}:

get:

description: Retrieves an individual Edge Load Event Subscription.

operationId: RetrieveIndEdgeLdEventSubsc

tags:

- Individual Edge Load Events Subscription (Document)

parameters:

- name: edgeLdId

in: path

description: Identifier of an individual Events Subscription.

required: true

schema:

type: string

responses:

'200':

description: The individual edge load subscription.

content:

application/json:

schema:

$ref: '#/components/schemas/EdgeSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual Edge Load Event Subscription.

operationId: DeleteIndEdgeLdEventSubsc

tags:

- Individual Edge Load Events Subscription (Document)

parameters:

- name: edgeLdId

in: path

description: Identifier of an individual Events Subscription

required: true

schema:

type: string

responses:

'204':

description: >

The individual Edge Load Events Subscription matching the edgeLdId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

EdgeSub:

description: Represents an individual Edge Load Subscription resource.

type: object

properties:

analyticsId:

$ref: '#/components/schemas/AnalyticsIdEl'

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

destinationEasInfo:

type: string

description: String representing the information for the destination EAS.

destinationEesId:

type: string

description: String identifying the the destination EES.

dnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

dataProdProfile:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/ProdProfileInfo'

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

report:

$ref: '#/components/schemas/EdgeNotif'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

timeInterval:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyticsType

- notifUri

anyOf:

- required: [destinationEasInfo]

- required: [destinationEesId]

- oneOf:

- required: [dnai]

- required: [dnn]

EdgeNotif:

description: Represents notification information of an Edge Load Event.

type: object

properties:

output:

$ref: '#/components/schemas/AnalyticsOutputEl'

analyticsOutput:

type: string

description: String representing the edge load analytics.

confLevel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

oneOf:

- required: [output]

- required: [analyticsOutput]

AnalyticsOutputEl:

description: Represents the edge load analytics output.

type: object

properties:

analyticsId:

$ref: '#/components/schemas/AnalyticsIdEl'

platAnalytics:

$ref: '#/components/schemas/EdgePlatformAnalytics'

eesAnalytics:

$ref: '#/components/schemas/EesAnalytics'

easAnalytics:

$ref: '#/components/schemas/EasAnalytics'

required:

- analyticsId

EdgePlatformAnalytics:

description: Represents the edge platform analytics data.

type: object

properties:

easConn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

eesConn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

confLevel:

type: integer

minimum: 0

maximum: 100

timeWnd:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

anyOf:

- required: [easConn]

- required: [eesConn]

EesAnalytics:

description: Represents the EES analytics data.

type: object

properties:

eesUnavail:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

eesUnavailPer:

type: integer

minimum: 0

maximum: 100

confLevel:

type: integer

minimum: 0

maximum: 100

timeWnd:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

anyOf:

- required: [eesUnavail]

- required: [eesUnavailPer]

EasAnalytics:

description: Represents the EAS analytics data.

type: object

properties:

easUnavail:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

easUnavailPer:

type: integer

minimum: 0

maximum: 100

confLevel:

type: integer

minimum: 0

maximum: 100

timeWnd:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

anyOf:

- required: [easUnavail]

- required: [easUnavailPer]

# Simple data types and Enumerations

AnalyticsIdEl:

anyOf:

- type: string

enum:

- EDGE_PLATFORM

- EES

- EAS

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Represents the analytics identifier for the edge load analytics.

Possible values are:

- EDGE_PLATFORM: Indicates that the analytics identifier is edge platform analytics.

- EES: Indicates that the analytics identifier is EES analytics.

- EAS: Indicates that the analytics identifier is EAS analytics.

# A.22	SS_AADRF_DataManagement API

openapi: 3.0.0

info:

title: SS_AADRF_DataManagement

description: |

API for A-ADRF Data Management Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-aadrf-datamanagement/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

summary: Create Individual A-ADRF Data Management Subscription.

operationId: CreateAADRFEventSubscription

tags:

- A-ADRF Data Management Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataManageSub'

responses:

'201':

description: >

The requested Individual A-ADRF Data Management Subscription is successfully created and

a representation of the created resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/DataManageSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

summary: Notify on the requested data.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataManageNotification'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the Individual A-ADRF Data Management Subscription resource.

required: true

schema:

type: string

delete:

summary: Remove the Individual A-ADRF Data Management Subscription.

operationId: UnsubscribeAADRFEventSubscription

tags:

- Individual A-ADRF Data Management Subscription (Document).

responses:

'204':

description: >

The individual A-ADRF Data Management Subscription resource matching the subscriptionId

is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/store:

post:

summary: Enables a service consumer to request data storage.

operationId: StoreData

tags:

- Data Storage Service

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/DataStoreReq'

required: true

responses:

'200':

description: >

Successful case. The data storage request is successfully received and processed.

content:

application/json:

schema:

$ref: '#/components/schemas/DataStoreResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

DataManageSub:

description: Represents the event subscriptions.

type: object

properties:

eventSubscriptions:

type: array

items:

$ref: '#/components/schemas/EventSubscription'

minItems: 1

description: Subscribed events.

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

notifCorrId:

type: string

description: Notification correlation identifier.

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eventSubscriptions

- notifUri

EventSubscription:

description: Represents the event subscription.

type: object

properties:

event:

$ref: '#/components/schemas/AadrfEvent'

dataCollectReq:

type: string

dataProducerIds:

type: array

items:

type: string

minItems: 1

description: The list of Data Producer IDs.

valUes:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: The target VAL UE(s) identifiers.

valServerId:

type: string

description: The target VAL server ID.

valServiceId:

type: string

description: The VAL service ID.

profileCriteria:

type: string

validConds:

$ref: 'TS29549_SS_Events.yaml#/components/schemas/ValidityConditions'

snssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

description: Identification(s) of network slice(s) to which the subscription applies.

edgeReq:

$ref: '#/components/schemas/EdgeReq'

apiLogReq:

$ref: '#/components/schemas/ApiLogReq'

s2sAnaReq:

$ref: '#/components/schemas/S2SAnalyticsReq'

ueRatAnaReq:

$ref: '#/components/schemas/UERatAnalyticsReq'

required:

- event

EdgeReq:

description: Represents the EDGE data request requirement.

type: object

properties:

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

description: Identifiers the DN Access Identifier.

dnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

description: Identifiers the DN Access Identifier.

destEasId:

type: string

description: Identifier for the destination EAS.

destEesId:

type: string

description: Identifier for the destination EES.

anyOf:

- required: [dnais]

- required: [dnns]

- required: [destEasId]

- required: [destEesId]

ApiLogReq:

description: Represents the API log request requirement.

type: object

properties:

apiRequestorInfo:

type: string

apiId:

type: string

description: String identifying the API invoked.

apiName:

type: string

description: Name of the API which was invoked.

apiVersion:

type: string

description: Version of the API which was invoked.

inputParameters:

type: string

description: List of input parameters.

result:

type: string

description: For HTTP protocol, it contains HTTP status code of the invocation.

apiInvokerId:

type: string

description: Identity of the API invoker which invoked the service API.

exposureLvl:

$ref: '#/components/schemas/ExposureLevel'

apiAggreInfo:

type: string

required:

- apiRequestorInfo

S2SAnalyticsReq:

description: >

Represents the Server to Server analytics data request requirement.

type: object

properties:

serverInfoList:

type: array

items:

$ref: 'TS29549_SS_ADAE_ServerToServerPerformanceAnalytics.yaml#/components/schemas/ServerInfo'

minItems: 1

timeVal:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

required:

- serverInfoList

UERatAnalyticsReq:

description: Represents the UE RAT connectivity analytics requirements.

type: object

properties:

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

timeVal:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

valUeAddrInfo:

type: array

items:

$ref: 'TS29549_SS_NetworkResourceAdaptation.yaml#/components/schemas/ValUeAddrInfo'

minItems: 1

required:

- valUeAddrInfo

DataManageNotification:

description: Represents the notification to the consumer.

type: object

properties:

event:

$ref: '#/components/schemas/AadrfEvent'

notifCorrId:

type: string

description: Notification correlation identifier.

valServerId:

type: string

description: The target VAL server ID.

valServiceId:

type: string

description: The VAL service ID.

valUeIds:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: The target VAL UE(s) identifiers.

locAccuracy:

type: string

validConds:

$ref: 'TS29549_SS_Events.yaml#/components/schemas/ValidityConditions'

apiLogs:

type: array

items:

$ref: '#/components/schemas/ApiLogInfo'

minItems: 1

description: The API logs information.

rttDeviation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

snssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

description: Identification(s) of network slice(s) to which the subscription applies.

edgeInfo:

$ref: '#/components/schemas/EdgeInfo'

s2SAnaResp:

$ref: '#/components/schemas/S2SAnalyticsInfo'

ueRatAnaInfo:

$ref: '#/components/schemas/UERatAnalyticsInfo'

required:

- event

ApiLogInfo:

description: Represents the API log data.

type: object

properties:

apiId:

type: string

description: String identifying the API invoked.

apiName:

type: string

description: Name of the API which was invoked.

failNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

apiAvailableInd:

type: boolean

description: Indicates the API availability.

apiVerChgNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

apiLocChg:

type: boolean

description: Indicates the API location changes for the target API.

apiThrottlingEvents:

type: string

invokeNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

apiVersion:

type: string

description: Version of the API which was invoked.

required:

- apiId

EdgeInfo:

description: Represents the EDGE related data.

type: object

properties:

edgeReq:

$ref: '#/components/schemas/EdgeReq'

output:

type: string

S2SAnalyticsInfo:

description: >

Represents the Server to Server analytics data information.

type: object

properties:

dataOutput:

$ref: 'TS29549_SS_ADAE_ServerToServerPerformanceAnalytics.yaml#/components/schemas/S2SPerfAnalyticsData'

serverInfoList:

type: array

items:

$ref: 'TS29549_SS_ADAE_ServerToServerPerformanceAnalytics.yaml#/components/schemas/ServerInfo'

minItems: 1

required:

- dataOutput

- serverInfoList

UERatAnalyticsInfo:

description: Represents the UE RAT Connectivity Analytics data information.

type: object

properties:

analyticsOut:

type: array

items:

$ref: 'TS29549_SS_ADAE_UeRatConnectivityAnalytics.yaml#/components/schemas/UeRatAnalyticsOutput'

minItems: 1

required:

- analyticsOut

DataStoreReq:

description: Represents the data storage request.

type: object

properties:

subDataStore:

$ref: '#/components/schemas/DataStoreSub'

dirStorage:

$ref: '#/components/schemas/Data'

storeHandl:

$ref: '#/components/schemas/StorageHandlingInfo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

oneOf:

- required: [subDataStore]

- required: [dirStorage]

DataStoreSub:

description: Represents the requirements for subscription based data storage.

type: object

properties:

dataProdId:

type: string

analyticsId:

type: string

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

analyticsMode:

$ref: '#/components/schemas/AnalyticsMode'

profiles:

type: array

items:

$ref: '#/components/schemas/DataProducerProfile'

minItems: 1

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

Data:

description: Represent the data for storage at A-ADRF.

type: object

properties:

dataType:

$ref: '#/components/schemas/DataType'

content:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

analyticsId:

type: string

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

dataSource:

$ref: '#/components/schemas/DataSourceInfo'

required:

- dataType

- content

- timestamp

- analyticsId

- analyticsType

- dataSource

DataProducerProfile:

description: Represents the data producer profile.

type: object

properties:

profile:

type: string

oneOf:

- required: [profile]

DataSourceInfo:

description: Represents the data source information.

type: object

properties:

source:

type: string

oneOf:

- required: [source]

StorageHandlingInfo:

description: Represent the storage handing information.

type: object

properties:

lifetime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

delNotifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

anyOf:

- required: [lifetime]

- required: [delNotifUri]

DataStoreResp:

description: Represents the data storage response.

type: object

properties:

storeHandl:

$ref: '#/components/schemas/StorageHandlingInfo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- storeHandl

AnalyticsMode:

description: Represents the analytics mode.

type: object

properties:

mode:

$ref: '#/components/schemas/Mode'

isMlEnab:

type: boolean

required:

- mode

# Simple data types and Enumerations

AadrfEvent:

anyOf:

- type: string

enum:

- HISTORICAL_SERVICEAPI

- NETWORK_SLICE

- EDGE_DATA

- LOCATION_ACCURACY

- SERVER_TO_SERVER_ANALYTICS

- UE_RAT_ANALYTICS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Indicates the subscribed events.

Possible values are:

- HISTORICAL_SERVICEAPI: The event for historical service API logs.

- NETWORK_SLICE: The event for the network slice data.

- EDGE_DATA: The event for the EDGE related data.

- LOCATION_ACCURACY: The event for the location accuracy data.

- SERVER_TO_SERVER_ANALYTICS: The event for the Server to Server analytics data.

- UE_RAT_ANALYTICS: The event for the UE RAT Connectivity analytics data.

ExposureLevel:

anyOf:

- type: string

enum:

- READ

- WRITE

- DELETE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Indicates the exposure level.

Possible values are:

- READ: The exposure level for the logs to be exposed is read.

- WRITE: The exposure level for the logs to be exposed is write.

- DELETE: The exposure level for the logs to be exposed is delete.

DataType:

anyOf:

- type: string

enum:

- DATA

- ANALYTICS

- DELETE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the type of data.

Mode:

anyOf:

- type: string

enum:

- ONLINE

- OFFLINE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the supported modes of analytics.

# A.23	SS_ADAE_LocationRelatedUeGroupAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_LocationRelatedUeGroupAnalytics

version: 1.0.0-alpha.2

description: |

API for ADAE Location-Related UE Group Analytics service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.2.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-lruga/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

summary: Create an "Individual Location-Related UE Group Analytics Subscription" resource.

operationId: SubscribeLocRelUeAnalytics

tags:

- Location Related UE Group Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocRelUeGroupSub'

responses:

'201':

description: >

The "Individual Location-Related UE Group Analytics Subscription" resource is created.

content:

application/json:

schema:

$ref: '#/components/schemas/LocRelUeGroupSub'

headers:

Location:

description: Contains the URI of the newly created individual resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

NotifyLocRelUeEvent:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocRelUeGroupNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an Individual Location-Related UE

Group Analytics Subscription.

required: true

schema:

type: string

get:

summary: Read the "Individual Location-Related UE Group Analytics Subscription" resource.

operationId: ReadLocRelUeGroupSubscription

tags:

- Individual Location-Related UE Group Analytics Subscription (Document)

responses:

'200':

description: >

The requested "Individual Location-Related UE Group Analytics Subscription"

resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/LocRelUeGroupSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: >

Update the "Individual Location-Related UE Group Analytics Subscription" resource.

operationId: UpdateLocRelUeGroupSubscription

tags:

- Individual Location-Related UE Group Analytics Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocRelUeGroupSub'

responses:

'200':

description: >

OK. The Individual Location-Related UE Group Analytics Subscription resource

is successfully updated, and representation of updated resource is

returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/LocRelUeGroupSub'

'204':

description: >

No Content. The Individual Location-Related UE Group Analytics Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: >

Modify the "Individual Location-Related UE Group Analytics Subscription" resource.

operationId: ModifyLocRelUeGroupSubscription

tags:

- Individual Location-Related UE Group Analytics Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/LocRelUeGroupSubPatch'

responses:

'200':

description: >

OK. The Individual Location-Related UE Group Analytics Subscription resource is

successfully modified, and representation of the modified resource is returned

in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/LocRelUeGroupSub'

'204':

description: >

No Content. The Individual Location-Related UE Group Analytics Subscription resource is

Successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the "Individual Location-Related UE Group Analytics Subscription" resource.

operationId: RemoveLocRelUeGroupSubscription

tags:

- Individual Location-Related UE Group Analytics Subscription (Document)

responses:

'204':

description: >

The "Individual Location-Related UE Group Analytics Subscription" resource

matching the subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

LocRelUeGroupSub:

description: >

Represents the location-related UE group analytics subscription.

type: object

properties:

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

analyticsId:

$ref: '#/components/schemas/AnalyticsIdLocGrp'

analyticsFilt:

$ref: '#/components/schemas/LocRelUeGroupFilter'

confLevel:

type: integer

minimum: 0

maximum: 100

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

timeInterval:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyticsType

- notifUri

LocRelUeGroupSubPatch:

description: >

Represents the location-related UE group analytics subscription modification request.

type: object

properties:

analyticsFilt:

$ref: '#/components/schemas/LocRelUeGroupFilter'

confLevel:

type: integer

minimum: 0

maximum: 100

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

timeInterval:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

LocRelUeGroupNotif:

description: >

Represents the location-related UE group analytics notification.

type: object

properties:

analyticsOutput:

$ref: '#/components/schemas/LocRelUeGroupAnalytics'

timeValid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

confLevel:

type: integer

minimum: 0

maximum: 100

required:

- analyticsOutput

LocRelUeGroupFilter:

description: >

Represent the the location-related UE group analytics filtering information.

type: object

properties:

groupId:

type: string

valTrgUes:

type: array

items:

type: string

minItems: 1

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

profiles:

type: array

items:

$ref: 'TS29558_Eecs_ECSServiceProvisioning.yaml#/components/schemas/AppGrpProfile'

minItems: 1

deviation:

type: array

items:

$ref: '#/components/schemas/LocDeviationCriteria'

minItems: 1

LocDeviationCriterion:

description: >

Represents the threshold for the location-related UE group analytics deviation.

type: object

properties:

value:

$ref: '#/components/schemas/LocDeviationTypeData'

thrDirect:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- value

- thrDirect

LocDeviationTypeData:

description: >

Represents the location-related UE group analytics deviation data.

type: object

properties:

speed:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

distance:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

direction:

$ref: 'TS29549_SS_ADAE_CollisionDetectionAnalytics.yaml#/components/schemas/Direction'

oneOf:

- required: [speed]

- required: [distance]

- required: [direction]

LocDeviationCriteria:

description: >

Represents the location-related UE group analytics deviation criteria.

type: object

properties:

target:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

criteria:

type: array

items:

$ref: '#/components/schemas/LocDeviationCriterion'

minItems: 1

required:

- criteria

- target

LocRelUeGroupAnalytics:

description: >

Represents the location-related UE group analytics output.

type: object

properties:

analyticsId:

$ref: '#/components/schemas/AnalyticsIdLocGrp'

ueGroupRoute:

type: array

items:

$ref: '#/components/schemas/UeGroupRouteAnalytics'

minItems: 1

ueGrMemDev:

type: array

items:

$ref: '#/components/schemas/UeGroupMemberDevAnalytics'

minItems: 1

required:

- analyticsId

UeGroupRouteAnalytics:

description: >

Represents the location-related UE group analytics output.

type: object

properties:

easIds:

type: array

items:

type: string

minItems: 1

routeId:

type: string

valUes:

type: array

items:

type: string

minItems: 1

required:

- routeId

- valUes

UeGroupMemberDevAnalytics:

description: >

Represents the group member deviation analytics output.

type: object

properties:

groupId:

type: string

refUe:

type: string

devUes:

type: array

items:

$ref: '#/components/schemas/Deviation'

minItems: 1

required:

- groupId

- devUes

Deviation:

description: >

Represents the deviation in VAL UE movement.

type: object

properties:

target:

type: string

criteria:

type: array

items:

$ref: '#/components/schemas/LocDeviationCriterion'

minItems: 1

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

required:

- criteria

- target

# Simple data types and Enumerations

AnalyticsIdLocGrp:

anyOf:

- type: string

enum:

- GROUP_ROUTE

- GROUP_MEMBER_DEVIATION

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the analytic IDs for location related UE group analytics.

Possible values are:

- GROUP_ROUTE: Indicates that the requested analytics is UE group route analytics.

- GROUP_MEMBER_DEVIATION: Indicates that the requested analytics

is UE group member deviation analytics.

# A.24	SS_ADAE_CollisionDetectionAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_CollisionDetectionAnalytics

version: 1.0.0-alpha.2

description: |

API for ADAE Collision Detection Analytics service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.2.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-cda/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

summary: Create an "Individual Collision Detection Analytics Subscription" resource.

operationId: SubscribeCollisionDetectionAnalytics

tags:

- Location Related UE Group Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CollisionDetectionSub'

responses:

'201':

description: >

Subscription to the collision detection analytics is created.

content:

application/json:

schema:

$ref: '#/components/schemas/CollisionDetectionSub'

headers:

Location:

description: Contains the URI of the newly created individual resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

NotifyCollisionDetectionEvent:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CollisionDetectionNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an Individual Collision Detection Analytics Subscription.

required: true

schema:

type: string

get:

summary: Read the "Individual Collision Detection Analytics Subscription" resource.

operationId: ReadCollisionDetectionSubscription

tags:

- Individual Collision Detection Analytics Subscription (Document)

responses:

'200':

description: >

The requested "Individual Collision Detection Analytics Subscription"

resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/CollisionDetectionSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Update the "Individual Collision Detection Analytics Subscription" resource.

operationId: UpdateCollisionDetectionSubscription

tags:

- Individual Collision Detection Analytics Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CollisionDetectionSub'

responses:

'200':

description: >

OK. The Individual Collision Detection Analytics Subscription resource

is successfully updated, and representation of updated resource is

returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/CollisionDetectionSub'

'204':

description: >

No Content. The Individual Collision Detection Analytics Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modify the "Individual Collision Detection Analytics Subscription" resource.

operationId: ModifyCollisionDetectionSubscription

tags:

- Individual Collision Detection Analytics Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/CollisionDetectionSubPatch'

responses:

'200':

description: >

OK. The Individual Collision Detection Analytics Subscription resource is

successfully modified, and representation of the modified resource is returned

in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/CollisionDetectionSub'

'204':

description: >

No Content. The Individual Collision Detection Analytics Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the "Individual Collision Detection Analytics Subscription" resource.

operationId: RemoveCollisionDetectionSubscription

tags:

- Individual Collision Detection Analytics Subscription (Document)

responses:

'204':

description: >

The "Individual Collision Detection Analytics Subscription" resource

matching the subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

CollisionDetectionSub:

description: >

Represents the collision detection analytics subscription.

type: object

properties:

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

analyticsId:

$ref: '#/components/schemas/AnalyticsIdCda'

analyticsFilt:

$ref: '#/components/schemas/CollisionDetectionFilter'

colDetCriteria:

$ref: '#/components/schemas/CollisionDetectionCriteria'

confLevel:

type: integer

minimum: 0

maximum: 100

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

timeInterval:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyticsType

- notifUri

CollisionDetectionSubPatch:

description: >

Represents the collision detection analytics subscription modification request.

type: object

properties:

analyticsFilt:

$ref: '#/components/schemas/CollisionDetectionFilter'

colDetCriteria:

$ref: '#/components/schemas/CollisionDetectionCriteria'

confLevel:

type: integer

minimum: 0

maximum: 100

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

timeInterval:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

CollisionDetectionNotif:

description: >

Represents the collision detection analytics notification.

type: object

properties:

analyticsId:

$ref: '#/components/schemas/AnalyticsIdCda'

targetOther:

type: array

items:

$ref: '#/components/schemas/TargetOtherUesAnalytics'

minItems: 1

anyInLoc:

type: array

items:

$ref: '#/components/schemas/TargetToAnyUeLocAnalytics'

minItems: 1

required:

- analyticsId

CollisionDetectionFilter:

description: >

Represent the the collision detection analytics filtering information.

type: object

properties:

trgToOther:

$ref: '#/components/schemas/TargetOtherUesFilter'

targetToAnyUeLoc:

$ref: '#/components/schemas/TargetToAnyUeLocFilter'

oneOf:

- required: [trgToOther]

- required: [targetToAnyUeLoc]

CollisionDetectionCriteria:

description: >

Represents the collision detection criteria.

type: object

properties:

distance:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

oneOf:

- required: [distance]

TargetOtherUesFilter:

description: >

Represent the analytics filter between any VAL UEs within the given location.

type: object

properties:

trgValServerId:

type: string

targetUes:

type: array

items:

type: string

minItems: 1

otherUes:

type: array

items:

type: string

minItems: 1

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

required:

- targetUes

TargetToAnyUeLocFilter:

description: >

Represent the analytics filter between sets of target and other UEs within a location.

type: object

properties:

trgValServerId:

type: string

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

required:

- area

TargetOtherUesAnalytics:

description: >

Represents the output for collision detection analytics between the sets

of target and other VAL UEs.

type: object

properties:

targetUe:

type: string

otherUe:

type: string

colTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

location:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

targetUeVel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

otherUeVel:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

targetUeDir:

$ref: '#/components/schemas/Direction'

otherUeDir:

$ref: '#/components/schemas/Direction'

confLevel:

type: integer

minimum: 0

maximum: 100

required:

- targetUe

- otherUe

- colTime

- location

TargetToAnyUeLocAnalytics:

description: >

Represents the output for collision detection analytics between

any VAL UEs within given location.

type: object

properties:

ues:

type: array

items:

type: string

minItems: 2

maxItems: 2

colTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

location:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

confLevel:

type: integer

minimum: 0

maximum: 100

required:

- ues

- colTime

- location

Direction:

description: Represents the direction.

type: object

properties:

azimuthDirection:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Angle'

elevationDirection:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Angle'

required:

- azimuthDirection

- elevationDirection

# Simple data types and Enumerations

AnalyticsIdCda:

anyOf:

- type: string

enum:

- TARGET_WITH_OTHER_UES

- ANY_UES_IN_LOC

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the analytic IDs for Collision Detection Analytics.

Possible values are:

- TARGET_WITH_OTHER_UES: Indicates that the requested analytics is collision

detection between the sets of target and other VAL UEs within a location.

- ANY_UES_IN_LOC: Indicates that the requested analytics is collision detection

between any VAL UEs within given location.

# A.25	SS_LocationHistoryInfoEvent API

openapi: 3.0.0

info:

title: LM Server Location Tracing Configuration Management Service

version: 1.0.0-alpha.2

description: |

LM Server Location Tracing Configuration Management Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.2.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/ss-lhie/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/configurations:

post:

summary: Request the creation of a Location Tracing Configuration.

operationId: CreateLocTracingConfig

tags:

- Location Tracing Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocTracingConfig'

responses:

'201':

description: >

Created. The Location Tracing Configuration is successfully created and

a representation of the created Individual Location Tracing Configuration

resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/LocTracingConfig'

headers:

Location:

description: >

Contains the URI of the created Individual Location Tracing Configuration resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/configurations/{configId}:

parameters:

- name: configId

in: path

description: >

Represents the identifier of the Individual Location Tracing Configuration resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Location Tracing Configuration resource.

operationId: GetIndLocTracingConfig

tags:

- Individual Location Tracing Configuration (Document)

responses:

'200':

description: >

OK. The requested Individual Location Tracing Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/LocTracingConfig'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Location Tracing Configuration resource.

operationId: UpdateIndLocTracingConfig

tags:

- Individual Location Tracing Configuration (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocTracingConfig'

responses:

'200':

description: >

OK. The Individual Location Tracing Configuration resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/LocTracingConfig'

'204':

description: >

No Content. The Individual Location Tracing Configuration resource is successfully

updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Location Tracing Configuration resource.

operationId: ModifyIndLocTracingConfig

tags:

- Individual Location Tracing Configuration (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/LocTracingConfigPatch'

responses:

'200':

description: >

OK. The Individual Location Tracing Configuration resource is successfully modified and

a representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/LocTracingConfig'

'204':

description: >

No Content. The Individual Location Tracing Configuration resource is successfully

modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Location Tracing Configuration resource.

operationId: DeleteIndLocTracingConfig

tags:

- Individual Location Tracing Configuration (Document)

responses:

'204':

description: >

No Content. The Individual Location Tracing Configuration resource is successfully

deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/reports:

get:

summary: Retrieve Location History Report(s).

operationId: GetHistTransQualMeasReports

tags:

- Location History Reports (Collection)

parameters:

- name: target-id

in: query

required: true

content:

application/json:

schema:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

- name: val-service-id

in: query

required: false

schema:

type: string

- name: req-loc-history

in: query

required: false

content:

application/json:

schema:

$ref: '#/components/schemas/ReqLocHistory'

- name: supp-feat

in: query

required: false

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The requested Location History Report(s), if any, shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/LocHistoryReports'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

LocTracingConfig:

description: >

Represents a Location Tracing Configuration.

type: object

properties:

target:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

valServiceId:

type: string

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

locQoS:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

triggerConds:

$ref: 'TS29549_SS_LocationReporting.yaml#/components/schemas/TriggeringCriteria'

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

posMethod:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/PositioningMethod'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- target

LocTracingConfigPatch:

description: >

Represents the parameters to request the modification of a Location Tracing Configuration.

type: object

properties:

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

locQoS:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

triggerConds:

$ref: 'TS29549_SS_LocationReporting.yaml#/components/schemas/TriggeringCriteria'

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

posMethod:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/PositioningMethod'

ReqLocHistory:

description: >

Represents the requested location history filtering information.

type: object

properties:

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

time:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

anyOf:

- required: [locArea]

- required: [time]

- required: [duration]

LocHistoryReports:

description: >

Represents Location History Report(s).

type: object

properties:

reports:

type: array

items:

$ref: '#/components/schemas/LocHistoryReport'

minItems: 0

description: >

Contains the Location History Report(s).

If there are no Location History Report(s) fulfilling the request, this attribute shall

contain an empty array.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- reports

LocHistoryReport:

description: >

Represents a Location History Report.

type: object

properties:

targets:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

locData:

type: object

additionalProperties:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

minProperties: 1

description: >

Contains the location history information report.

The key of the map shall be set to the identifier of the VAL UE/user (among the ones

provided within the targets attribute) to which the provided location history

information provided within the map value is related.

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- locData

oneOf:

- required: [valUEs]

- required: [valUsers]

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# Data types describing alternative data types or combinations of data types

#

# A.26	SS_ConfirmLocation API

openapi: 3.0.0

info:

title: LM Server Location Confirmation Service Management Service

version: 1.0.0-alpha.1

description: |

LM Server Location Confirmation Service Management Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.1.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/ss-cl/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Request the creation of a Location Confirmation Service Subscription.

operationId: CreateLocConfirmSubsc

tags:

- Location Confirmation Service Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocConfirmSubsc'

responses:

'201':

description: >

Created. The Location Confirmation Service Subscription is successfully created and a

representation of the created Individual Location Confirmation Service Subscription

resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/LocConfirmSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual Location Confirmation Service

Subscription resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

LocConfirmUsageNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocConfirmUsageNotif'

responses:

'204':

description: >

No Content. The Location Confirmation Service Usage Notification is successfully

received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscription/{subscId}:

parameters:

- name: subscId

in: path

description: >

Represents the identifier of the Individual Location Confirmation Service Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Location Confirmation Service Subscription resource.

operationId: GetIndLocConfirmSubsc

tags:

- Individual Location Confirmation Service Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Location Confirmation Service Subscription resource shall

be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/LocConfirmSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Location Confirmation Service Subscription resource.

operationId: UpdateIndLocConfirmSubsc

tags:

- Individual Location Confirmation Service Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocConfirmSubsc'

responses:

'200':

description: >

OK. The Individual Location Confirmation Service Subscription resource is successfully

updated and a representation of the updated resource shall be returned in the response

body.

content:

application/json:

schema:

$ref: '#/components/schemas/LocConfirmSubsc'

'204':

description: >

No Content. The Individual Location Confirmation Service Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Location Confirmation Service Subscription resource.

operationId: ModifyIndLocConfirmSubsc

tags:

- Individual Location Confirmation Service Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/LocConfirmSubscPatch'

responses:

'200':

description: >

OK. The Individual Location Confirmation Service Subscription resource is successfully

modified and a representation of the updated resource shall be returned in the response

body.

content:

application/json:

schema:

$ref: '#/components/schemas/LocConfirmSubsc'

'204':

description: >

No Content. The Individual Location Confirmation Service Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Location Confirmation Service Subscription resource.

operationId: DeleteIndLocConfirmSubsc

tags:

- Individual Location Confirmation Service Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Location Confirmation Service Subscription resource is

successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

LocConfirmSubsc:

description: >

Represents a Location Confirmation Service Subscription.

type: object

properties:

appIds:

type: array

items:

type: string

minItems: 1

valServiceIds:

type: array

items:

type: string

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

oneOf:

- required: [appIds]

- required: [valServiceIds]

LocConfirmSubscPatch:

description: >

Represents the parameters to request the modification of a Location Confirmation Service

Subscription.

type: object

properties:

appIds:

type: array

items:

type: string

minItems: 1

valServiceIds:

type: array

items:

type: string

minItems: 1

LocConfirmUsageNotif:

description: >

Represents a Location Confirmation Service Usage Notification.

type: object

properties:

reports:

type: array

items:

$ref: '#/components/schemas/LocConfirmUsageReport'

minItems: 0

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- reports

LocConfirmUsageReport:

description: >

Represents a Location Confirmation Service Usage Notification.

type: object

properties:

appId:

type: string

valServiceId:

type: string

valUeId:

type: string

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- valUeId

oneOf:

- required: [appId]

- required: [valServiceId]

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# Data types describing alternative data types or combinations of data types

#

# A.27	SS_SLPositioningManagement API

openapi: 3.0.0

info:

title: SS_SLPositioningManagement

version: 1.0.0-alpha.2

description: |

API for SEAL SL Positioning management Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/ss-slpm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Create individual SL Positioning Management subscription.

operationId: SubscribeSlPositionMgmt

tags:

- SL Positioning Management Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SlPositionMgmtSubsc'

responses:

'201':

description: >

The requested individual SL Positioning MAnagement subscription

resource is successfully created and a representation of the created

resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SlPositionMgmtSubsc'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

SlPositionMgmtNotif:

'{$request.body#/notifUri}':

post:

summary: Notify on changes of the SL Positioning Management according to the

requested reporting settings.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SlPositionMgmtNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Read an existing individual SL Positioning Management subscription resource

according to the subscriptionId.

operationId: ReadSlPositioningMgmtSubscription

tags:

- Individual SL Positioning Management Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual SL Positioning Management

subscription resource.

required: true

schema:

type: string

responses:

'200':

description: The requested individual SL Positioning Management subscription returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SlPositionMgmtSubsc'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: >

Update an individual SL Positioning Management subscription identified by the

subscriptionId.

operationId: UpdateSlPositionMgmt

tags:

- Individual Unicast SL Positioning Management Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual unicast monitoring subscription resource.

required: true

schema:

type: string

requestBody:

description: Updated details of the unicast QoS monitoring subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SlPositionMgmtSubsc'

responses:

'200':

description: >

The subscription is updated successfully, and the updated subscription

information returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/SlPositionMgmtSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: >

Modify an individual SL Positioning Management subscription identified

by the subscriptionId.

operationId: ModifySlPositionMgmt

tags:

- Individual SL Positioning Management Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual SL Positioning Management

subscription resource.

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SlPositionMgmtSubscPatch'

responses:

'200':

description: >

Individual individual SL Position Management resource is modified

successfully and representation of the modified individual SL Positioning Management

resource

subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SlPositionMgmtSubscPatch'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove an existing individual SL Positioning Management resource according to

the subscriptionId.

operationId: UnsubscribeSlPositionMgmt

tags:

- Individual SL Positioning Management Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an individual SL Positioning Management

subscription resource.

required: true

schema:

type: string

responses:

'204':

description: >

The Individual SL Positioning Management Subscription resource matching the

subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/request-srp:

post:

summary: Enables a service consumer to request SR positioning information.

operationId: RequestSRPosInfo

tags:

- SR Positioning Information Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SrPosInfoReq'

responses:

'200':

description: >

OK. The Positioning Information Request is successfully received and processed, and the

requested SR based positioning information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SrPosInfoResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

SlPositionMgmtSubsc:

description: Indicate the SL Positioning Management subscription.

type: object

properties:

valSvcId:

type: string

description: Identity of the VAL service

tgtUes:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: List of VAL Users or UE IDs for which SL location management is requested.

valServArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

valUeSel:

type: array

items:

$ref: '#/components/schemas/ValUeSel'

minItems: 1

eventReport:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- eventReport

- valSvcId

SlPositionMgmtSubscPatch:

description: Indicate the SL Positioning Management subscription update.

type: object

properties:

valSvcId:

type: string

description: Identity of the VAL service

tgtUes:

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

description: List of VAL Users or UE IDs for which SL location management is requested.

valServArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

valUeSel:

type: array

items:

$ref: '#/components/schemas/ValUeSel'

minItems: 1

eventReport:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

SlPositionMgmtNotif:

description: SL Positioning Management notification.

type: object

properties:

subscId:

type: string

description: Subscription Identifier

refUes:

type: array

items:

$ref: '#/components/schemas/ValUeInfo'

minItems: 1

description: List of VAL UE(s) identified as Reference UE(s).

tgtUes:

type: array

items:

$ref: '#/components/schemas/ValUeInfo'

minItems: 1

description: List of VAL UE(s) identified as Target UE(s).

clientUes:

type: array

items:

$ref: '#/components/schemas/ValUeInfo'

minItems: 1

description: List of VAL UE(s) identified as Client UE(s).

failCause:

type: array

items:

$ref: '#/components/schemas/Cause'

minItems: 1

required:

- subscId

- refUes

- tgtUes

ValUeInfo:

description: Indicate the Val UE Information.

type: object

properties:

valUeId:

type: string

description: Identity of the VAL UE ID.

valUeAddr:

$ref: 'TS29549_SS_NetworkResourceAdaptation.yaml#/components/schemas/ValUeAddrInfo'

SrPosInfoReq:

description: >

Represents the SR Positioning Information Request.

type: object

properties:

valSvcId:

type: string

ueList:

$ref: '#/components/schemas/SrUeList'

srPosFltr:

type: array

items:

$ref: '#/components/schemas/SrPosFilter'

minItems: 1

locQoS:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

required:

- ueList

- srPosFltr

SrPosInfoResp:

description: >

Represents the SR Positioning Information Response.

type: object

properties:

srPosInfo:

type: array

items:

$ref: '#/components/schemas/SrPosInfo'

minItems: 1

required:

- srPosInfo

SrUeList:

description: >

Contains the list of UE identities of Short-Range based positioning.

type: object

properties:

clientUes:

type: array

items:

type: string

minItems: 1

tgtUes:

type: array

items:

type: string

minItems: 1

refUes:

type: array

items:

type: string

minItems: 1

required:

- clientUes

- tgtUes

- refUes

SrPosInfo:

description: >

Contains the Short-Range based positioning information.

type: object

properties:

refUe:

type: string

tgtUe:

type: string

relativeLocation2D:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/2DRelativeLocation'

relativeLocation3D:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/3DRelativeLocation'

relativeVelocity:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/RelativeVelocityWithUncertainty'

distanceDirection:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/RangeDirection'

required:

- refUe

- tgtUe

anyOf:

- required: [ relativeLocation2D ]

- required: [ relativeLocation3D ]

- required: [ relativeVelocity ]

- required: [ distanceDirection ]

# Simple data types and Enumerations

Cause:

anyOf:

- type: string

enum:

- NOTIFY_NO_REFERENCE_UE_FOUND

- NOTIFY_NO_TARGET_UE_FOUND

- NOTIFY_NO_CLIENT_UE_FOUND

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

Enumeration of SL Positioning management failure cause.

- NOTIFY_NO_REFERENCE_UE_FOUND: This value indicates that the none of the VAL UE matches

the criteria for the Reference UE.

- NOTIFY_NO_TARGET_UE_FOUND: This value indicates that the none of the VAL UE matches

the criteria for the Target UE.

- NOTIFY_NO_CLIENT_UE_FOUND: This value indicates that the none of the VAL UE matches

the criteria for the Client UE.

ValUeSel:

anyOf:

- type: string

enum:

- REF_UE

- TARGET_UE

- CLIENT_UE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

Enumeration of VAL UE(s) selection criteria for SL Positioning management.

- REF_UE: Indicates the selection criteria of UE role is reference UE.

- TARGET_UE: Indicates the selection criteria of UE role is target UE.

- CLIENT_UE: Indicates the selection criteria of UE role is client UE.

SrPosFilter:

anyOf:

- type: string

enum:

- RANGE

- DIRECTION

- RELATIVE_POS

- RELATIVE_VEL

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Possible values are:

Enumeration of requested SR based positioning information filters.

- RANGE: Indicates that the range is requested.

- DIRECTION: Indicates that the direction is requested.

- RELATIVE_POS: Indicates that the relative position is requested.

- RELATIVE_VEL: Indicates that the relative velocity is requested.

# A.28	SS_ADAE_AIMLMemberCapabilityAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_AIMLMemberCapabilityAnalytics

description: |

API for ADAE AIML member capability analytics analytics service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.2"

externalDocs:

description: >

3GPP TS 29.549 v19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-aimlmca/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/application-aimlmcas:

post:

summary: Request the creation of AIML Member Capability Analytics Subscription.

operationId: CreateIndAimlmcaSubsc

tags:

- AIML Member Capability Analytics Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapSub'

callbacks:

paNotification:

'{$request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: The AIML Member Capability Analytics Subscription is successfully

created and a representation of the created Individual AIML Member Capability Analytics

Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/application-aimlmcas/{appAimlmcaId}:

parameters:

- name: appAimlmcaId

in: path

description: >

Represents the identifier of an Individual AIML Member Capability Analytics Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieves an existing Individual AIML Member Capability Analytics Subscription.

operationId: ReadIndAimlmcaSubsc

tags:

- Individual AIML Member Capability Analytics Subscription (Document)

responses:

'200':

description: >

The requested Individual AIML Member Capability Analytics subscription matching the appAimlmcaId shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual AIML Member Capability Analytics Subscription resource.

operationId: UpdateIndAimlmcaSubsc

tags:

- Individual AIML Member Capability Analytics Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapSub'

responses:

'200':

description: >

OK. The Individual AIML Member Capability Analytics Subscription resource is

successfully updated and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapSub'

'204':

description: >

No Content. The Individual AIML Member Capability Analytics Subscription resource

is successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual AIML Member Capability Analytics Subscription resource.

operationId: ModifyIndAimlmcaSubsc

tags:

- Individual AIML Member Capability Analytics Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/AppAimlMemCapSubPatch'

responses:

'200':

description: >

OK. The Individual AIML Member Capability Analytics Subscription resource is

successfully modified and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/AppAimlMemCapSub'

'204':

description: >

No Content. The Individual AIML Member Capability Analytics Subscription resource

is successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual AIML Member Capability Analytics Subscription resource.

operationId: DeleteIndAimlmcaSubsc

tags:

- Individual AIML Member Capability Analytics Subscription (Document)

parameters:

- name: appAimlmcaId

in: path

description: Identifier of an Individual AIML member Capability Analytics Subscription.

required: true

schema:

type: string

responses:

'204':

description: >

The Individual AIML Member Capability Analytics Subscription matching the appAimlmcaId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

AppAimlMemCapSub:

description: Represents an individual AIML member capability analytics event subscription resource.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

analyticsId:

$ref: '#/components/schemas/AppAimlMemCapAnalytics'

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

analyticsSub:

$ref: '#/components/schemas/AppAimlMemCapAnalyticsSub'

confLevel:

type: integer

minimum: 0

maximum: 100

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- notifUri

- analyticsId

- analyticsType

- analyticsSub

AppAimlMemCapNotif:

description: Represents notification of the AIML member capability analytics event.

type: object

properties:

analyticsOutput:

$ref: '#/components/schemas/AppAimlMemCapAnalyticsOutput'

confLevel:

type: integer

minimum: 0

maximum: 100

required:

- analyticsOutput

AppAimlMemCapSubPatch:

description: Represents the requested modification to a Individual AIML member capability analytics subscription resource.

type: object

properties:

analyticsSub:

$ref: '#/components/schemas/AppAimlMemCapAnalyticsSub'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

confLevel:

type: integer

minimum: 0

maximum: 100

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

AppAimlMemCapAnalyticsSub:

description: Represents the information related to the analytics subscription.

type: object

properties:

ueIds:

type: array

items:

type: string

minItems: 1

description: >

One or more VAL UE IDs whose analytics or data collection are subject to subcription.

aimlMemberIds:

type: array

items:

$ref: '#/components/schemas/AimlMemberId'

minItems: 1

description: >

One or more AIML members whose analytics or data collection are subject to subcription.

aimlMemCapAttribute:

$ref: '#/components/schemas/AppAimlMemCapAttributes'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

oneOf:

- required: [ueIds]

- required: [aimlMemberIds]

AppAimlMemCapAnalyticsOutput:

description: Represents analytics output information in the notification.

type: object

properties:

analyticsId:

$ref: '#/components/schemas/AppAimlMemCapAnalytics'

analyticsOutput:

type: array

items:

$ref: '#/components/schemas/AnalyticsOutput'

minItems: 1

description: >

Contains a list of identities of one or more VAL UEs or AIML members, the data

collection related to, in the case of the notification is on the AIML member capability

data collection.

required:

- analyticsId

AnalyticsOutput:

description: Represents analytics output information in the notification.

type: object

properties:

ueId:

type: string

aimlMemberId:

items:

$ref: '#/components/schemas/AimlMemberId'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

aimlMemAvails:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

aimlMemCaps:

$ref: '#/components/schemas/AimlMemCapability'

required:

- aimlMemAvails

oneOf:

- required: [ueId]

- required: [aimlMemberId]

AimlMemCapability:

description: Represents AIML member capability information in the notification.

type: object

properties:

maxAimlMemConn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minAimlMemConn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

# SIMPLE DATA TYPES

#

AimlMemberId:

description: Represents unique identifier of a AIMLE Member.

type: string

#

# ENUMERATIONS

#

AppAimlMemCapAnalytics:

anyOf:

- type: string

enum:

- APP_MEMBER_CAPABILITY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents Connection Status Events.

Possible values are:

- APP_MEMBER_CAPABILITY: Indicates that the identifier of the analytics consumer.

AppAimlMemCapAttributes:

anyOf:

- type: string

enum:

- COMMUNICATION_CAPABILITY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents Connection Status Events.

Possible values are:

- COMMUNICATION_CAPABILITY: Indicates that the requested attribute to be analyzed is

communication capability.

# A.29	SS_ADCCF_DataCollection API

openapi: 3.0.0

info:

title: SS_ADCCF_DataCollection API

version: 1.0.0-alpha.1

description: |

API for A-DCCF Data Collection service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adccf-dc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

summary: Create an "Individual Data Collection Subscription" resource.

operationId: SubscribeDataCollection

tags:

- Data Collection Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataCollectionSub'

responses:

'201':

description: >

The "Individual Data Collection Subscription" resource is created.

content:

application/json:

schema:

$ref: '#/components/schemas/DataCollectionSub'

headers:

Location:

description: Contains the URI of the newly created individual resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

DataCollectionNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DataCollectionNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an Individual Location-Related UE

Group Analytics Subscription.

required: true

schema:

type: string

get:

summary: Read the "Individual Data Collection Subscription" resource.

operationId: ReadDataCollectionSubscription

tags:

- Individual Data Collection Subscription (Document)

responses:

'200':

description: >

The requested "Individual Data Collection Subscription"

resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/DataCollectionSub'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Remove the "Individual Data Collection Subscription" resource.

operationId: RemoveDataCollectionSubscription

tags:

- Individual Data Collection Subscription (Document)

responses:

'204':

description: >

The "Individual Data Collection Subscription" resource

matching the subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

DataCollectionSub:

description: >

Represents the Data Collection Subscription.

type: object

properties:

dataColEvnId:

type: string

dataColReq:

$ref: '#/components/schemas/DataCollectionReq'

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

analyticsId:

type: string

dataProdIds:

type: array

items:

type: string

minItems: 1

profiles:

type: array

items:

$ref: 'TS29549_SS_AADRF_DataManagement.yaml#/components/schemas/DataProducerProfile'

minItems: 1

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

procReq:

$ref: '#/components/schemas/ProcessingReq'

storageReqs:

$ref: '#/components/schemas/StorageRequirements'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

report:

$ref: '#/components/schemas/DataCollectionNotif'

required:

- dataColEvnId

- dataColReq

- notifUri

DataCollectionNotif:

description: >

Represents the Data Collection Notification.

type: object

properties:

dataColEvnId:

type: string

analyticsId:

type: string

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

dataType:

$ref: '#/components/schemas/AdccfDataType'

dataOutput:

type: string

procReq:

$ref: '#/components/schemas/ProcessingReq'

storageReqs:

type: array

items:

type: string

minItems: 1

required:

- dataColEvnId

- dataOutput

DataCollectionReq:

description: >

Represents the data collection requirements.

type: object

properties:

dataFormat:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

repFreq:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

abstraction:

type: integer

minimum: 0

maximum: 100

accuracy:

type: integer

minimum: 0

maximum: 100

required:

- dataFormat

- repFreq

- abstraction

- accuracy

ProcessingReq:

description: >

Represents the data processing requirements.

type: object

properties:

req:

type: string

anyOf:

- required: [req]

AdccfDataType:

description: >

Represents A-DCCD data type.

type: object

properties:

type:

$ref: '#/components/schemas/DataType'

gran:

$ref: '#/components/schemas/GranularityType'

oneOf:

- required: [type]

- required: [gran]

DataOutput:

description: >

Represents the A-DCCF data output.

type: object

properties:

dataOutput:

type: string

required:

- dataOutput

StorageRequirements:

description: >

Represents the data storage requirements.

type: object

properties:

storageReqs:

type: array

items:

type: string

minItems: 1

required:

- storageReqs

# Simple data types and Enumerations

DataType:

anyOf:

- type: string

enum:

- UE_DATA

- NETWORK_DATA

- APP_DATA

- EDGE_DATA

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the data type.

Possible values are:

- UE_DATA: Indicates that the data sample related to the UE data.

- NETWORK_DATA: Indicates that the data sample related to the network data.

- APP_DATA: Indicates that the data sample related to the application data.

- EDGE_DATA: Indicates that the data sample related to the edge data.

GranularityType:

anyOf:

- type: string

enum:

- REAL_TIME

- NON_REAL_TIME

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the granularity type.

Possible values are:

- REAL_TIME: Indicates that the data granularity is real time.

- NON_REAL_TIME: Indicates that the data granularity is non-real time.

# A.30	SS_ADAE_ServerToServerPerformanceAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_ServerToServerPerformanceAnalytics

version: 1.0.0-alpha.1

description: |

API for ADAE Server to Server Performance Analytics service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-sspa/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

summary: Create an "Individual Server to Server Performance Analytics Subscription" resource.

operationId: SubscribeServer2ServerPerformanceAnalytics

tags:

- Server to Server Performance Analytics Subscription (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/S2SPerfSubsc'

responses:

'201':

description: >

Subscription to the Server to Server Performance analytics is created.

content:

application/json:

schema:

$ref: '#/components/schemas/S2SPerfSubsc'

headers:

Location:

description: Contains the URI of the newly created individual resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

NotifyServerToServerPerformance:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/S2SPerfNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of an Individual Server to Server Performance Analytics

Subscription.

required: true

schema:

type: string

get:

summary: Read the "Individual Server to Server Performance Analytics Subscription" resource.

operationId: ReadIndServer2ServerPerformanceSubscription

tags:

- Individual Server to Server Performance Analytics Subscription (Document)

responses:

'200':

description: >

The requested "Individual Server to Server Performance Analytics Subscription"

resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/S2SPerfSubsc'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: >

Update the "Individual Server to Server Performance Analytics Subscription" resource.

operationId: UpdateIndServer2ServerPerformanceSubscription

tags:

- Individual Server to Server Performance Analytics Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/S2SPerfSubsc'

responses:

'200':

description: >

OK. The Individual Server to Server Performance Analytics Subscription resource

is successfully updated, and representation of updated resource is returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/S2SPerfSubsc'

'204':

description: >

No Content. The Individual Server to Server Performance Analytics Subscription resource

is successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: >

Modify the "Individual Server to Server Performance Analytics Subscription" resource.

operationId: ModifyIndServer2ServerPerformanceSubscription

tags:

- Individual Server to Server Performance Analytics Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/S2SPerfSubscPatch'

responses:

'200':

description: >

OK. The Individual Server to Server Performance Analytics Subscription resource is

successfully modified, and representation of the modified resource is returned

in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/S2SPerfSubsc'

'204':

description: >

No Content. The Individual Server to Server Performance Analytics Subscription resource

is successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete the "Individual Server to Server Performance Analytics Subscription" resource.

operationId: DeleteIndServer2ServerPerformanceSubscription

tags:

- Individual Server to Server Performance Analytics Subscription (Document)

responses:

'204':

description: >

The "Individual Server to Server Performance Analytics Subscription" resource

matching the subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

S2SPerfSubsc:

description: >

Represents the Server to Server Performance analytics subscription.

type: object

properties:

analyticsIds:

type: array

items:

$ref: '#/components/schemas/AnalyticsIdSspa'

minItems: 1

analyticsType:

$ref: 'TS29549_SS_ADAE_VALPerformanceAnalytics.yaml#/components/schemas/AnalyticsType'

serverInfoList:

type: array

items:

$ref: '#/components/schemas/ServerInfo'

minItems: 1

valServId:

type: string

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

confLevel:

type: integer

minimum: 0

maximum: 100

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

serverIdList:

type: array

items:

type: string

minItems: 1

serverIpAddList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

minItems: 1

timeVal:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyticsType

- notifUri

- serverInfoList

S2SPerfSubscPatch:

description: >

Represents the Server to Server Performance analytics subscription modification request.

type: object

properties:

serverInfoList:

type: array

items:

$ref: '#/components/schemas/ServerInfo'

minItems: 1

valServId:

type: string

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

confLevel:

type: integer

minimum: 0

maximum: 100

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

timeVal:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

S2SPerfNotif:

description: >

Represents the Server to Server Performance analytics notification.

type: object

properties:

analyticsIds:

type: array

items:

$ref: '#/components/schemas/AnalyticsIdSspa'

minItems: 1

analyticsOutput:

type: array

items:

$ref: '#/components/schemas/S2SPerfAnalyticsData'

minItems: 1

confLevel:

type: integer

minimum: 0

maximum: 100

required:

- analyticsOutput

S2SPerfAnalyticsData:

description: >

Represent the Server to Server Performance analytics data.

type: object

properties:

performChgInd:

type: boolean

description: >

Indicates whether a server-to-server session performance has changed.When present, it

shall be set as follows:

- true: the performance has changed.

- false: the performance is not changed.

- Default value when omitted is "false".

timHorizOverInd:

type: boolean

description: >

Indicates whether a server-to-server session performance sustainability has over a given

time horizon.When present, it shall be set as follows:

- true: the performance sustainability has over the given time horizon.

- false: the performance sustainability has not over the given time horizon.

- Default value when omitted is "false".

areaOverInd:

type: boolean

description: >

Indicates whether a server-to-server session performance sustainability has over a given

area.When present, it shall be set as follows:

- true: the performance sustainability has over the given area.

- false: the performance sustainability has not over the given area.

- Default value when omitted is "false".

qosMeasureRslt:

description: >

Represents the QoS measurement result for the Server to Server Performance between any

VAL UEs within the given area.

$ref: 'TS29549_SS_Ue2UePerformanceAnalytics.yaml#/components/schemas/U2UAnalyticsData'

anyOf:

- required: [performChgInd]

- required: [timHorizOverInd]

- required: [areaOverInd]

- required: [qosMeasureRslt]

ServerInfo:

description: >

Represents the Server Information.

type: object

properties:

serverId:

type: string

serverIpAddList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

minItems: 1

anyOf:

- required: [serverId]

- required: [serverIpAddList]

# Simple data types and Enumerations

AnalyticsIdSspa:

anyOf:

- type: string

enum:

- SERVER_TO_SERVER_PERFORMANCE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the analytic IDs for Server to Server Performance Analytics.

Possible values are:

- SERVER_TO_SERVER_PERFORMANCE: Indicates that the requested analytics is Server to Server

Performance analytics.

# A.31	SS_ADAE_UeRatConnectivityAnalytics API

openapi: 3.0.0

info:

title: SS_ADAE_UeRatConnectivityAnalytics

version: 1.0.0-alpha.1

description: |

API for ADAE UE RAT Connectivity Analytics service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.3.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-adae-uerat/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/subscriptions:

post:

summary: Create a new UE RAT Connectivity Analytics Subscription.

operationId: CreateUeRatConnAnalyticsSubsc

tags:

- UE RAT Connectivity Analytics Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UeRatConnectivitySub'

responses:

'201':

description: >

Created. The UE RAT Connectivity Analytics Subscription is successfully created and a

representation of the created Individual UE RAT Connectivity Analytics Subscription

resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UeRatConnectivitySub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

UeRatConnAnalyticsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UeRatConnectivityNotif'

responses:

'204':

description: >

No Content. The UE RAT Connectivity Analytics Notification is successfully

received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the UE RAT Connectivity Analytics Subscription.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual UE RAT Connectivity Analytics Subscription resource.

operationId: GetIndUeRatConnAnalyticsSubsc

tags:

- Individual UE RAT Connectivity Analytics Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual UE RAT Connectivity Analytics Subscription

resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/UeRatConnectivitySub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual UE RAT Connectivity Analytics Subscription resource.

operationId: UpdateIndUeRatConnAnalyticsSubsc

tags:

- Individual UE RAT Connectivity Analytics Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UeRatConnectivitySub'

responses:

'200':

description: >

OK. The Individual UE RAT Connectivity Analytics Subscription resource is successfully

updated and a representation of the updated resource shall be returned in the response

body.

content:

application/json:

schema:

$ref: '#/components/schemas/UeRatConnectivitySub'

'204':

description: >

No Content. The Individual UE RAT Connectivity Analytics Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual UE RAT Connectivity Analytics Subscription resource.

operationId: ModifyIndUeRatConnAnalyticsSubsc

tags:

- Individual UE RAT Connectivity Analytics Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/UeRatConnectivitySubPatch'

responses:

'200':

description: >

OK. The Individual UE RAT Connectivity Analytics Subscription resource is successfully

modified and a representation of the updated resource shall be returned in the response

body.

content:

application/json:

schema:

$ref: '#/components/schemas/UeRatConnectivitySub'

'204':

description: >

No Content. The Individual UE RAT Connectivity Analytics Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual UE RAT Connectivity Analytics Subscription resource.

operationId: RemoveUeRatConnectivityAnalytics

tags:

- Individual UE RAT Connectivity Analytics Subscription (Document)

responses:

'204':

description: >

The Individual UE RAT Connectivity Analytics Subscription resource

is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured Data Types

UeRatConnectivitySub:

description: >

Represents the UE RAT Connectivity Analytics subscription.

type: object

properties:

analyticsSubs:

type: array

items:

$ref: '#/components/schemas/UeRatAnalyticsSub'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

report:

$ref: '#/components/schemas/UeRatConnectivityNotif'

repReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

timeInterval:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- analyticsSubs

- notifUri

UeRatConnectivitySubPatch:

description: >

Represents the requested modifications to a UE RAT Connectivity Analytics subscription.

type: object

properties:

analyticsSubs:

type: array

items:

$ref: '#/components/schemas/UeRatAnalyticsSub'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

timeInterval:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

UeRatConnectivityNotif:

description: >

Represents the UE RAT Connectivity AnalyticsUE RAT connectivity analytics

Notification.

type: object

properties:

subId:

type: string

analyticsOut:

type: array

items:

$ref: '#/components/schemas/UeRatAnalyticsOutput'

minItems: 1

required:

- subId

- analyticsOut

UeRatAnalyticsOutput:

description: >

Represents the UE RAT Connectivity Analytics output information.

type: object

properties:

analyticsIds:

type: array

items:

$ref: '#/components/schemas/AnalyticsIdUeRat'

minItems: 1

areaApp:

$ref: '#/components/schemas/AreaPrediction'

timeApp:

$ref: '#/components/schemas/TimeWindowPrediction'

valUeIds:

type: array

items:

type: string

minItems: 1

valSvcId:

type: string

required:

- analyticsIds

- valUeIds

UeRatAnalyticsSub:

description: >

Represents the UE RAT Connectivity Analytics Subscription related information.

type: object

properties:

analyticsIds:

type: array

items:

$ref: '#/components/schemas/AnalyticsIdUeRat'

minItems: 1

valUeIds:

type: array

items:

type: string

minItems: 1

valGroupId:

type: string

ueMobs:

type: object

additionalProperties:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/UeMobility'

minProperties: 1

description: >

Contains the mobility and route information of the target VAL UEs.

The key of the map shall be set to the value of the identifier of the VAL UE

(among the ones provided within the valUeIds attribute or the members of the VAL group

identified by the valGroupId attribute) to which the mobility and route information

provided within the map value is related.

valSvcId:

type: string

description: Identity of the VAL service

confLevel:

type: integer

minimum: 0

maximum: 100

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

required:

- analyticsIds

oneOf:

- required: [valUeIds]

- required: [valGroupId]

AreaPrediction:

description: >

Represents the RAT type changes for a given way point.

type: object

properties:

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

timePer:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

confLevel:

type: integer

minimum: 0

maximum: 100

required:

- area

- timePer

TimeWindowPrediction:

description: >

Represents the time periods for predictions.

type: object

properties:

timePer:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

confLevel:

type: integer

minimum: 0

maximum: 100

required:

- timePer

# Simple data types and Enumerations

AnalyticsIdUeRat:

anyOf:

- type: string

enum:

- UE_RAT_CONNECTIVITY_ANALYTICS

- type: string

description: >

Indicates that the requested analytics is UE RAT connectivity analytics event.

description: |

Represents the UE RAT connectivity analytics identifier.

- UE_RAT_CONNECTIVITY_ANALYTICS: Indicates that the requested analytics is UE RAT

Connectivity Analytics.

Annex B (normative):
SEAL NRM server support integration with TSN

When the SEAL Network Resource Management (NRM) server act as a TSN AF, the NRM server shall support integration with TSN including 5GS Bridge information reporting as defined in clause 14.3.8.2 of 3GPP TS 23.434 [2] and 5GS Bridge configuration as defined in clause 14.3.8.3 of 3GPP TS 23.434 [2].

The 5GS integration with TSN only support fully-centralized model as defined in IEEE Std 802.1Qcc-2018 [29], the NRM server acts as a TSN AF as defined in clause 14.2.2.2 of 3GPP TS 23.434 [2], shall support the TSN bridge information report as defined in clause 14.3.2.29 of 3GPP TS 23.434 [2], TSN bridge information confirmation as defined in clause 14.3.2.30 of 3GPP TS 23.434 [2], TSN bridge configuration request as defined in clause 14.3.2.31 of 3GPP TS 23.434 [2] and TSN bridge configuration response as defined in clause 14.3.2.32 of 3GPP TS 23.434 [2]. TSN CNC (as defined in IEEE 802.1Qcc [29]) via the NRM-S reference point configures the TSN flows in the 5GS. As a TSN AF, the SEAL NRM server shall interact with the 5GS PCF over the N5 reference point to configure the 5G QoS and TSCAI parameters in 5GS as defined in clause 14.2.2.24 of 3GPP TS 29.514 [30].

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | CT3#106 |  |  |  |  | TS skeleton for Services Enabler Architecture Layer for Verticals Application Programming Interface specification. | 0.0.0 |
| 2019-10 | CT3#106 | C3-194418 |  |  |  | Inclusion of TS skeleton document with clauses reflecting SEAL service APIs, agreed in the meeting CT3#106: C3-194418 | 0.1.0 |
| 2019-10 | CT3#106 | C3-194314 |  |  |  | Inclusion of documents agreed in CT3#106: C3-194297, C3-194298, C3-194299, C3-194300 | 0.2.0 |
| 2019-11 | CT3#107 | C3-195307 |  |  |  | Inclusion of documents agreed in CT3#107: C3-195157, C3-195260, C3-195441, C3-195262, C3-195263, C3-195264, C3-195185 | 0.3.0 |
| 2019-12 | CT#86 | CP-193176 |  |  |  | Sent to plenary for Information | 1.0.0 |
| 2020-03 | CT3#108e |  |  |  |  | Inclusion of documents agreed in CT3#108-e meeting: C3-201346, C3-201347, C3-201348, C3-201349, C3-201350, C3-201456, C3-201457, C3-201351, C3-201352, C3-201271 | 1.1.0 |
| 2020-04 | CT3#109e | C3-202444 |  |  |  | Inclusion of documents agreed in CT3#109e meeting: C3-202241, C3-202275, C3-202334, C3-202335, C3-202336, C3-202337, C3-202338, C3-202339, C3-202340, C3-202341, C3-202342, C3-202343, C3-202481 | 1.2.0 |
| 2020-06 | CT3#110e | C3-203459 |  |  |  | Inclusion of documents agreed in CT3#110e meeting: C3-203233, C3-203317, C3-203409, C3-203411, C3-203412, C3-203413, C3-203414, C3-203415, C3-203416, C3-203417, C3-203418, C3-203419, C3-203530, C3-203587, C3-203634 | 1.3.0 |
| 2020-06 | CT#88e | CP-201209 |  |  |  | TS sent to plenary for approval | 2.0.0 |
| 2020-06 | CT#88e | CP-201334 |  |  |  | Implementation errors fixed. TS sent to plenary for approval | 2.0.1 |
| 2020-06 | CT#88e | CP-201334 |  |  |  | TS approved by plenary | 16.0.0 |
| 2020-09 | CT#89e | CP-202074 | 0001 |  | F | Correct apiVersion notation | 16.1.0 |
| 2020-09 | CT#89e | CP-202074 | 0002 | 1 | F | Corrections to API and Event names | 16.1.0 |
| 2020-09 | CT#89e | CP-202074 | 0003 |  | F | Correct Identity filter in Events API | 16.1.0 |
| 2020-09 | CT#89e | CP-202087 | 0004 | 1 | F | SS_KeyInfoRetrieval API correction | 16.1.0 |
| 2020-09 | CT#89e | CP-202074 | 0005 |  | F | Key Management API description | 16.1.0 |
| 2020-09 | CT#89e | CP-202074 | 0006 | 1 | F | UnicastSubscription attribute presence correction | 16.1.0 |
| 2020-09 | CT#89e | CP-202084 | 0009 |  | F | Update of OpenAPI version and TS version in externalDocs field | 16.1.0 |
| 2020-12 | CT#90e | CP-203139 | 0010 | 1 | F | Essential corrections and alignments | 16.2.0 |
| 2020-12 | CT#90e | CP-203142 | 0011 | 1 | F | Immediate reporting | 16.2.0 |
| 2020-12 | CT#90e | CP-203139 | 0012 | 1 | F | Storage of YAML files in 3GPP Forge | 16.2.0 |
| 2020-12 | CT#90e | CP-203142 | 0013 | 1 | F | SEAL Group configuration corrections | 16.2.0 |
| 2020-12 | CT#90e | CP-203152 | 0014 |  | F | Update of OpenAPI version and TS version in externalDocs field | 16.2.0 |
| 2021-03 | CT#91e | CP-210221 | 0015 | 1 | F | Adding some missing description fields to data type definitions in OpenAPI specification files | 17.0.0 |
| 2021-03 | CT#91e | CP-210221 | 0016 | 1 | F | Corrections to HTTP custom headers handling for Northbound APIs | 17.0.0 |
| 2021-03 | CT#91e | CP-210220 | 0017 |  | F | OpenAPI reference | 17.0.0 |
| 2021-03 | CT#91e | CP-210240 | 0018 |  | F | Update of OpenAPI version and TS version in externalDocs field | 17.0.0 |
| 2021-06 | CT#92e | CP-211238 | 0019 | 1 | F | Correction of invalid characters in OpenAPI specification files | 17.1.0 |
| 2021-06 | CT#92e | CP-211241 | 0022 | 1 | F | 204 No Content during modification procedure on SS_GroupManagement API | 17.1.0 |
| 2021-06 | CT#92e | CP-211241 | 0023 | 1 | F | Support redirection for SEAL APIs | 17.1.0 |
| 2021-06 | CT#92e | CP-211235 | 0025 |  | A | Notifcation URI | 17.1.0 |
| 2021-06 | CT#92e | CP-211240 | 0026 |  | F | 204 No Conent for resource modification in SS_LocationReporting API | 17.1.0 |
| 2021-06 | CT#92e | CP-211265 | 0027 |  | F | Update of OpenAPI version and TS version in externalDocs field | 17.1.0 |
| 2021-09 | CT#93e | CP-212214 | 0028 |  | F | Resource URI correction on SEAL APIs | 17.2.0 |
| 2021-09 | CT#93e | CP-212207 | 0029 | 1 | B | Support 5G CN external group information for SEAL groups | 17.2.0 |
| 2021-09 | CT#93e | CP-212207 | 0030 | 1 | B | Message filters for SEAL groups | 17.2.0 |
| 2021-09 | CT#93e | CP-212223 | 0031 |  | F | Update of OpenAPI version and TS version in externalDocs field | 17.2.0 |
| 2021-12 | CT#94e | CP-213232 | 0032 | 1 | B | Support local MBMS | 17.3.0 |
| 2021-12 | CT#94e | CP-213232 | 0033 | 1 | B | Support Tracking UE and obtaining dynamic UE information | 17.3.0 |
| 2021-12 | CT#94e | CP-213231 | 0034 | 1 | B | Group Management enhancement for 5G-VN groups | 17.3.0 |
| 2021-12 | CT#94e | CP-213250 | 0035 | 4 | B | eSEAL Events Monitoring service | 17.3.0 |
| 2021-12 | CT#94e | CP-213220 | 0036 |  | B | Alignment with SA3 supported TLS profiles | 17.3.0 |
| 2021-12 | CT#94e | CP-213254 | 0037 | 2 | B | Network slice capability management API for SEAL | 17.3.0 |
| 2021-12 | CT#94e | CP-213231 | 0038 | 1 | B | eSEAL location deviation service | 17.3.0 |
| 2021-12 | CT#94e | CP-213231 | 0039 | 1 | B | Introduce TSC related service operations | 17.3.0 |
| 2021-12 | CT#94e | CP-213231 | 0041 | 1 | B | Support Create_TSC_Stream service operation | 17.3.0 |
| 2021-12 | CT#94e | CP-213231 | 0042 | 1 | B | Support Delete_TSC_Stream service operation | 17.3.0 |
| 2021-12 | CT#94e | CP-213231 | 0045 | 2 | B | Create_TSC_Stream data model and OpenAPI | 17.3.0 |
| 2021-12 | CT#94e | CP-213231 | 0046 |  | B | Delete_TSC_Stream OpenAPI definition | 17.3.0 |
| 2021-12 | CT#94e | CP-213236 | 0052 | 1 | D | Editorial corrections for tables, figures, clauses, headers and refernces | 17.3.0 |
| 2021-12 | CT#94e | CP-213246 | 0054 |  | F | Update of OpenAPI version and TS version in externalDocs field | 17.3.0 |
| 2022-03 | CT#95e | CP-220205 | 0040 | 4 | B | Support Discover_TSC_Stream_Availability service operation | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0043 | 4 | B | Resource structure to support TSC related service operations | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0044 | 4 | B | Discover_TSC_Stream_Availability data model and OpenAPI | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0055 | 2 | B | Obtain service operation in SS_NetworkResourceMonitoring | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0056 | 2 | B | SS_NetworkResourceMonitoring API support | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0057 | 2 | B | SS_NetworkResourceMonitoring OpenAPI implementation | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0058 | 1 | B | Support integration with TSN | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0059 |  | F | Terminology replacement of NSCM with NSCE | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0060 | 1 | B | Location deviation service and Open API | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0061 | 2 | B | SS_LocationAreaMonitoring API | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0062 |  | B | Location report timestamp support | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0063 |  | B | Add VAL service specific information | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0065 | 1 | B | SS_NetworkResourceMonitoring API definition and Subscribe/Unsubscribe/Notify service operations | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0066 |  | B | Supporting temporary group formation within a VAL system | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0067 | 1 | B | Subscription update for SS_Events API | 17.4.0 |
| 2022-03 | CT#95e | CP-220205 | 0068 | 1 | B | Clarification on location based group for SS_GroupManagement API | 17.4.0 |
| 2022-03 | CT#95e | CP-220204 | 0070 | 1 | B | Support PATCH for update of Individual SEAL Location Reporting Configuration resource | 17.4.0 |
| 2022-03 | CT#95e | CP-220204 | 0071 |  | B | Support PATCH for update of Individual VAL group document resource | 17.4.0 |
| 2022-03 | CT#95e | CP-220194 | 0072 |  | F | Update of info and externalDocs fields | 17.4.0 |
| 2022-06 | CT#96 | CP-221140 | 0073 | 1 | F | Tags and OperationId support in the SS_NetworkResourceMonitoring API | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0074 | 1 | F | Dimension of QoS parameters in the SS_NetworkResourceMonitoring API | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0075 | 2 | F | Clarifications of reporting, reporting termination, and subscription termination in SS_NetworkResourceMonitoring API | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0076 | 1 | F | The "Requestor identity" attributes removal in Release 17 APIs | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0077 | 1 | F | Resolving EN for the Subscribe service operation in the SS_NetworkResourceMonitoring API | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0078 |  | F | The corrections of naming in the SS_NetworkResourceMonitoring API | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0079 | 1 | F | NO-REF_SIBLINGS error correction | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0080 | 2 | F | SEAL-S security update for Release-17 | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0082 | 3 | F | Partial success support in the SS_NetworkResourceMonitoring API | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0083 | 1 | B | Supplementary location information to verticals | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0084 |  | F | Resolution of the Editor’s note for Network slice capability Enablement API. | 17.5.0 |
| 2022-06 | CT#96 | CP-221140 | 0085 | 1 | F | Correction of arrays cardinality in the SS_NetworkResourceMonitoring OpenAPI file | 17.5.0 |
| 2022-06 | CT#96 | CP-221156 | 0087 |  | A | SEAL-S security update | 17.5.0 |
| 2022-06 | CT#96 | CP-221148 | 0088 | 1 | F | Resource URI overview and apiVersion placeholder | 17.5.0 |
| 2022-06 | CT#96 | CP-221148 | 0089 | 1 | F | OpenAPI long descriptions | 17.5.0 |
| 2022-06 | CT#96 | CP-221156 | 0091 |  | A | Correcting the ValTargetUe data type name in two occurrences | 17.5.0 |
| 2022-06 | CT#96 | CP-221260 | 0092 | 2 | F | Organizing and correcting the data model of the SS_NetworkResourceMonitoring API | 17.5.0 |
| 2022-06 | CT#96 | CP-221151 | 0093 |  | F | Update of info and externalDocs fields | 17.5.0 |
| 2022-09 | CT#97e | CP-222108 | 0094 | 2 | F | Modification of data type for Network slice capability Enablement API | 17.6.0 |
| 2022-09 | CT#97e | CP-222181 | 0095 | 1 | F | Termination of reporting related updates | 17.6.0 |
| 2022-09 | CT#97e | CP-222118 | 0096 | 1 | F | Tags and OperationId support in the SS_Events API | 17.6.0 |
| 2022-09 | CT#97e | CP-222118 | 0097 | 1 | F | Tags and OperationId support in the SS_GroupManagement API | 17.6.0 |
| 2022-09 | CT#97e | CP-222118 | 0098 | 1 | F | Tags and OperationId support in the SS_KeyInfoRetrieval API | 17.6.0 |
| 2022-09 | CT#97e | CP-222118 | 0099 | 1 | F | Tags and OperationId support in the SS_LocationAreaInfoRetrieval API | 17.6.0 |
| 2022-09 | CT#97e | CP-222117 | 0100 |  | F | Tags and OperationId support in the SS_NetworkSliceAdaptation API | 17.6.0 |
| 2022-09 | CT#97e | CP-222117 | 0101 |  | F | Tags and OperationId support in the SS_UserProfileRetrieval API | 17.6.0 |
| 2022-09 | CT#97e | CP-222118 | 0102 | 1 | F | Tags and OperationId support in the SS_LocationReporting API | 17.6.0 |
| 2022-09 | CT#97e | CP-222108 | 0103 | 1 | F | Path correction in the SS_LocationAreaInfoRetrieval OpenAPI file | 17.6.0 |
| 2022-09 | CT#97e | CP-222182 | 0104 | 1 | F | "Error handling" clause: alignment with other NBI and 5GS APIs | 17.6.0 |
| 2022-09 | CT#97e | CP-222108 | 0105 |  | F | Correction of the SS_NetworkResourceAdaptation API naming | 17.6.0 |
| 2022-09 | CT#97e | CP-222108 | 0106 | 1 | F | Correction in error cases in the SS_NetworkResourceMonitoring API | 17.6.0 |
| 2022-09 | CT#97e | CP-222117 | 0107 |  | F | Correction of the LocationInfo data type spelling in the SS_LocationAreaInfoRetrieval API | 17.6.0 |
| 2022-09 | CT#97e | CP-222117 | 0108 |  | F | Correction of the "SubscriptionId" resource URI variable name | 17.6.0 |
| 2022-09 | CT#97e | CP-222108 | 0109 |  | F | Correction of the note for LocationInfoCriteria data type in SS_Events API | 17.6.0 |
| 2022-09 | CT#97e | CP-222108 | 0110 | 1 | F | Correction of the "proxRange" attribute within the ReferenceUEDetail structure in SS_Events API | 17.6.0 |
| 2022-09 | CT#97e | CP-222108 | 0111 | 1 | F | Correction in the validation criteria for the Update_Trigger_Location_Reporting service operation in the SS_LocationReporting API | 17.6.0 |
| 2022-09 | CT#97e | CP-222117 | 0112 |  | F | Definitions of HTTP "406 Not Acceptable" response | 17.6.0 |
| 2022-09 | CT#97e | CP-222125 | 0113 |  | F | SS_GroupManagement API: delete operation | 17.6.0 |
| 2022-09 | CT#97e | CP-222121 | 0114 |  | F | Update of info and externalDocs fields | 17.6.0 |
| 2022-12 | CT#98e | CP-223193 | 0119 | 1 | A | Correction of the presence and cardinality of the “suppFeat” attribute within the MulticastSubscription data structure in the SS_NetworkResourceAdaptation API | 17.7.0 |
| 2022-12 | CT#98e | CP-223185 | 0116 | 1 | F | Correction of the tables for the re-used and API-specific data structures in the SEAL APIs | 18.0.0 |
| 2022-12 | CT#98e | CP-223185 | 0117 | 1 | F | Correction the enumerations in the SS_NetworkResourceMonitoring API | 18.0.0 |
| 2022-12 | CT#98e | CP-223185 | 0118 |  | F | Correction of the descriptions in the OpenAPI files of the SEAL APIs | 18.0.0 |
| 2022-12 | CT#98e | CP-223194 | 0120 | 1 | B | Update_Unicast_QoS_Monitoring service operation for the SS_NetworkResourceMonitoring API | 18.0.0 |
| 2022-12 | CT#98e | CP-223194 | 0121 | 1 | B | PATCH and PUT methods for the SS_NetworkResourceMonitoring AP | 18.0.0 |
| 2022-12 | CT#98e | CP-223194 | 0122 | 1 | B | OpenAPI implementation for the Update_Unicast_QoS_Monitoring_Subscription service operation in the SS_NetworkResourceMonitoring API | 18.0.0 |
| 2022-12 | CT#98e | CP-223194 | 0123 | 1 | B | Interaction with CM server in the Create_Group service operation of the SS_GroupManagement API | 18.0.0 |
| 2022-12 | CT#98e | CP-223185 | 0124 |  | F | Cardinality for data types of SS_Events API | 18.0.0 |
| 2022-12 | CT#98e | CP-223189 | 0127 |  | F | Update of info and externalDocs fields | 18.0.0 |
| 2023-03 | CT#99 | CP-230157 | 0128 | 1 | B | Updates on location reporting | 18.1.0 |
| 2023-03 | CT#99 | CP-230156 | 0129 | 1 | F | Correction of the description fields in enumerations | 18.1.0 |
| 2023-03 | CT#99 | CP-230157 | 0132 | 1 | F | Essential correction of the Create_Group service operation in the SS_GroupManagement API | 18.1.0 |
| 2023-03 | CT#99 | CP-230157 | 0134 | 1 | B | Partial failure support in the SS_Events API | 18.1.0 |
| 2023-03 | CT#99 | CP-230157 | 0137 | 1 | F | Correction of the websocket procedures in the SS_Events API | 18.1.0 |
| 2023-03 | CT#99 | CP-230156 | 0138 |  | D | Correction of the references for the WebsockNotifConfig and reqTestNotif | 18.1.0 |
| 2023-03 | CT#99 | CP-230168 | 0139 | 1 | B | VAL Service area – Location Area Monitoring | 18.1.0 |
| 2023-03 | CT#99 | CP-230168 | 0140 | 1 | B | VAL Service area – Location based group | 18.1.0 |
| 2023-03 | CT#99 | CP-230168 | 0142 | 1 | B | SS_IdmParameterProvisioning API description | 18.1.0 |
| 2023-03 | CT#99 | CP-230161 | 0143 |  | F | Update of info and externalDocs fields | 18.1.0 |
| 2023-06 | CT#100 | CP-231146 | 0141 | 2 | B | SS_VALServiceAreaConfiguration service API | 18.2.0 |
| 2023-06 | CT#100 | CP-231175 | 0144 | 2 | B | Introduce a new attribute to support location QoS | 18.2.0 |
| 2023-06 | CT#100 | CP-231185 | 0145 |  | B | Definition of SEALDD related functionalities | 18.2.0 |
| 2023-06 | CT#100 | CP-231146 | 0146 | 1 | B | Definition of the service description part of the MBS resource management service operations | 18.2.0 |
| 2023-06 | CT#100 | CP-231146 | 0147 | 1 | B | Definition of the API resources part of the MBS resource management service operations | 18.2.0 |
| 2023-06 | CT#100 | CP-231146 | 0148 |  | B | Definition of the API notifications part of the MBS resource management service operations | 18.2.0 |
| 2023-06 | CT#100 | CP-231329 | 0149 | 4 | B | Definition of the API data model part of the MBS resource management service operations | 18.2.0 |
| 2023-06 | CT#100 | CP-231330 | 0150 | 4 | B | Definition of the OpenAPI description part of the MBS resource management service operations | 18.2.0 |
| 2023-06 | CT#100 | CP-231139 | 0151 | 1 | F | Correction of the error handling in the SS_GroupManagement API | 18.2.0 |
| 2023-06 | CT#100 | CP-231146 | 0152 | 1 | B | Definition of the SS_VALServiceData API | 18.2.0 |
| 2023-06 | CT#100 | CP-231146 | 0153 | 1 | B | Implementation of the SS_VALServiceData API | 18.2.0 |
| 2023-06 | CT#100 | CP-231146 | 0154 | 1 | B | OpenAPI file for the SS_VALServiceData API | 18.2.0 |
| 2023-06 | CT#100 | CP-231139 | 0156 | 1 | F | Support of Network slice capability management | 18.2.0 |
| 2023-06 | CT#100 | CP-231203 | 0157 | 2 | B | VAL service area support in the SS_LocationReporting API | 18.2.0 |
| 2023-06 | CT#100 | CP-231188 | 0158 | 1 | B | Support of ADAES | 18.2.0 |
| 2023-06 | CT#100 | CP-231146 | 0159 | 1 | B | SS_VALServiceAreaConfiguration service API implementation | 18.2.0 |
| 2023-06 | CT#100 | CP-231204 | 0160 | 2 | B | SS_VALServiceAreaConfiguration service API implementation of the OpenAPI file | 18.2.0 |
| 2023-06 | CT#100 | CP-231175 | 0161 | 1 | B | Improvement of the Subscribe_Location_Monitoring service operation in the SS_LocationMonitoring API | 18.2.0 |
| 2023-06 | CT#100 | CP-231146 | 0162 | 1 | B | SS_IdmParameterProvisioning API service description and data model | 18.2.0 |
| 2023-06 | CT#100 | CP-231175 | 0163 | 1 | B | Support of obtaining location information from a 3rd party LMS | 18.2.0 |
| 2023-06 | CT#100 | CP-231141 | 0164 |  | F | Update of info and externalDocs fields | 18.2.0 |
| 2023-09 | CT#101 | CP-232100 | 0165 | 1 | B | Defining the Activate_MBS_Resource service operation | 18.3.0 |
| 2023-09 | CT#101 | CP-232100 | 0166 | 1 | B | Defining the Deactivate_MBS_Resource service operation | 18.3.0 |
| 2023-09 | CT#101 | CP-232125 | 0167 |  | F | Complete the list of SEAL service APIs | 18.3.0 |
| 2023-09 | CT#101 | CP-232116 | 0169 | 1 | A | Corrections of the attribute name and missed clause | 18.3.0 |
| 2023-09 | CT#101 | CP-232100 | 0171 | 1 | B | Improvement of the Create_TSC_Stream service operation for the time synchronization capabilities | 18.3.0 |
| 2023-09 | CT#101 | CP-232100 | 0172 | 1 | F | IDM Parameter provisioning correction | 18.3.0 |
| 2023-09 | CT#101 | CP-232100 | 0173 | 1 | B | LocationAreaInfoRetreival - VAL service area identifer | 18.3.0 |
| 2023-09 | CT#101 | CP-232100 | 0174 | 1 | B | SS_LocationMonitoring - VAL service area identifer | 18.3.0 |
| 2023-09 | CT#101 | CP-232085 | 0175 |  | F | Update of info and externalDocs fields | 18.3.0 |
| 2023-12 | CT#102 | CP-233231 | 0176 |  | F | Further progressing the definition of MBS resources management | 18.4.0 |
| 2023-12 | CT#102 | CP-233205 | 0177 | 3 | B | Subscribe/Notify service operation implementation in the SS_VALServiceAreaConfiguration API | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0178 | 1 | B | Subscribe/Notify service operation OpenAPI implementation in the SS_VALServiceAreaConfiguration API | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0179 | 1 | B | Subscribe/Notify service operation procedures in the SS_VALServiceAreaConfiguration API | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0180 | 1 | B | Triggering criteria in the SS_LocationReporting API | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0181 | 1 | B | Improvement of the Delete_TSC_Stream service operation for the time synchronization capabilities | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0182 | 1 | B | VAL service area feature name update | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0184 | 1 | B | VAL application performance API | 18.4.0 |
| 2023-12 | CT#102 | CP-233259 | 0186 | 1 | B | Slice-specific application performance API | 18.4.0 |
| 2023-12 | CT#102 | CP-233259 | 0187 | 1 | B | UE-to-UE session performance API | 18.4.0 |
| 2023-12 | CT#102 | CP-233259 | 0188 | 1 | B | Location accuracy API | 18.4.0 |
| 2023-12 | CT#102 | CP-233259 | 0189 | 1 | B | Service API API | 18.4.0 |
| 2023-12 | CT#102 | CP-233259 | 0190 | 1 | B | Slice Usage Pattern API | 18.4.0 |
| 2023-12 | CT#102 | CP-233259 | 0191 | 1 | B | Edge Load API | 18.4.0 |
| 2023-12 | CT#102 | CP-233259 | 0192 | 1 | B | Updates to the MBS resources related service descriptions | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0194 |  | B | BDT_Configuration_request API support with description update | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0196 | 1 | B | BDT_Configuration_request API support with resources and data model update | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0197 | 1 | B | BDT_Negotiation_notification support with description update | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0198 | 1 | B | BDT_Negotiation_Notification support with Notification message and data model update | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0199 | 1 | B | BDT_Configuration_request API support with open API update | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0200 | 1 | B | Rename the feature name for 5GFLS and remove ENs | 18.4.0 |
| 2023-12 | CT#102 | CP-233285 | 0201 | 1 | F | EN resolution for the VAL service area location representation | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0202 | 1 | F | EN resolution for the VAL service area ID representation | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0203 | 1 | B | Update subscription service operation implementation in the SS_VALServiceAreaConfiguration API | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0205 | 1 | B | Update subscription service operation implementation in the SS_VALServiceAreaConfiguration OpenAPI file | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0206 | 1 | B | Update subscription service operation in the SS_VALServiceAreaConfiguration API | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0207 | 1 | B | Notification service operation in the SS_LocationReporting API | 18.4.0 |
| 2023-12 | CT#102 | CP-233138 | 0208 | 2 | B | SS_IdmParameterProvisioning API Other CRUD operations | 18.4.0 |
| 2023-12 | CT#102 | CP-233243 | 0209 | 1 | B | Further progressing the definition of MBS resources management | 18.4.0 |
| 2023-12 | CT#102 | CP-233237 | 0210 |  | F | Update of info and externalDocs fields | 18.4.0 |
| 2024-03 | CT#103 | CP-240169 | 0183 | 3 | B | SEAL Key management parameter provisioning | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0212 | 2 | B | VAL performance analytics | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0213 | 1 | B | SS_ADAE_VALPerformanceAnalytics API | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0214 | 3 | B | Slice-specific application performance analytics | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0215 | 1 | B | SS_ADAE_SlicePerformanceAnalytics API | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0216 | 2 | B | Location accuracy performance analytics | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0217 | 2 | B | SS_ADAE_LocationAccuracyAnalytics API | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0218 | 3 | B | Service API analytics | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0219 | 1 | B | SS_ADAE_ServiceApiAnalytics API | 18.5.0 |
| 2024-03 | CT#103 | CP-240193 | 0220 | 1 | B | Transferring the NSCE_NetworkSliceAdaptation API to TS 29.435 | 18.5.0 |
| 2024-03 | CT#103 | CP-240165 | 0221 | 3 | B | Updates to API Definition for SS_ADAE_EdgeLoadAnalytics API and Solve ENs | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0222 | 1 | B | Service Operation Description for SS_ADAE_EdgeLoadAnalytics API | 18.5.0 |
| 2024-03 | CT#103 | CP-240165 | 0223 | 3 | B | OpenAPI for SS_ADAE_EdgeLoadAnalytics API | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0224 | 1 | B | Define API of SS_AADRF_DataManagement API | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0225 | 1 | B | Define OpenAPI of SS_AADRF_DataManagement API | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0226 | 1 | B | Define service descriptions of SS_AADRF_DataManagement API | 18.5.0 |
| 2024-03 | CT#103 | CP-240165 | 0227 | 2 | B | Implementation of the OpenAPI file for the SS_ADAE_Ue2UePerformanceAnalytics API | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0228 | 1 | B | The procedures of the SS_ADAE_Ue2UePerformanceAnalytics API | 18.5.0 |
| 2024-03 | CT#103 | CP-240165 | 0229 | 3 | B | Implementation of the SS_ADAE_Ue2UePerformanceAnalytics API | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0230 | 1 | B | Slice usage pattern analytics operations | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0231 | 1 | B | Slice usage pattern analytics data model completion | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0232 | 1 | B | Slice usage pattern analytics OpenAPI | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0233 | 1 | B | VAL performance analytics data model updates | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0234 | 1 | B | Slice-specific performance analytics data model updates | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0235 | 1 | B | Location accuracy analytics data model updates | 18.5.0 |
| 2024-03 | CT#103 | CP-240164 | 0236 | 1 | B | Service API analytics data model updates | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0237 | 1 | B | Support BDT_Configuration Get, Update and Delete service operations | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0238 | 1 | B | Update open API for BDT_Configuration | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0239 | 1 | B | Update resources and data model clauses fro BDT_Configuration | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0240 | 1 | F | Supported features presence condition correction | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0241 |  | F | SS_IdmParameterProvisioning correction | 18.5.0 |
| 2024-03 | CT#103 | CP-240171 | 0242 |  | F | HTTP RFCs obsoleted by IETF RFC 9112, 9110, 9111 and 9113 | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0243 | 1 | B | Update service operations for location management services | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0244 | 1 | B | Alignment of VAL service area identifier alignment with new data type | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0245 | 1 | B | Unified traffic pattern and monitoring management | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0246 | 1 | F | Corrections to partial failure report | 18.5.0 |
| 2024-03 | CT#103 | CP-240171 | 0247 | 1 | B | Define the missing Group Deletion Notification | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0248 | 1 | B | Define the NRM reliable transmission service operation | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0249 |  | B | Support 5MBS related error handling | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0250 | 1 | F | Various corrections and updates | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0253 | 1 | B | EN resolution for the VAL service area ID in the SS_GroupManagement API | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0254 | 1 | B | EN resolution for the LocationReport data type in the SS_LocationReporting API | 18.5.0 |
| 2024-03 | CT#103 | CP-240169 | 0255 |  | F | OpenAPI file correction of the the SS_VALServiceAreaConfiguration API | 18.5.0 |
| 2024-03 | CT#103 | CP-240166 | 0259 |  | F | Update of info and externalDocs fields | 18.5.0 |
| 2024-06 | CT#104 | CP-241116 | 0258 | 2 | F | EN resolution for VAL UE(s) and User(s) identifiers in the SS_VALServiceData API | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0260 | 1 | F | Clarification for Exposure level requirements | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0262 | 2 | F | Add reporting requirements to SliceAppPerfSub | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0264 | 1 | F | Add reporting requirements to LocAccurSub | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0266 | 1 | F | Add new attributes to SrvApiSub | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0267 | 1 | F | Add reporting requirements to SUPSub | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0268 |  | F | Edge Load Analytics corrections | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0269 |  | F | A-ADRF Data Management API corrections | 18.6.0 |
| 2024-06 | CT#104 | CP-241096 | 0270 | 1 | F | Various SEAL API corrections | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0271 | 1 | F | Removal of the Editor’s Note for the analytics type in the notification | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0272 | 1 | F | Resolve the Editor's Notes for the analytics and data output | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0273 | 1 | F | Update the analytics type in the subscription | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0274 |  | F | 307 and 308 responses support in ADAES APIs | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0276 | 1 | F | Alignment of the SS_ADAE_Ue2UePerformanceAnalytics API | 18.6.0 |
| 2024-06 | CT#104 | CP-241116 | 0277 | 2 | B | EN resolution for application error in SEAL APIs | 18.6.0 |
| 2024-06 | CT#104 | CP-241116 | 0278 |  | F | Correction of OpenAPI syntax errors | 18.6.0 |
| 2024-06 | CT#104 | CP-241083 | 0279 | 1 | F | Various updates and corrections about APIs | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0280 | 1 | B | Updates to the API Definition for SS_ADAE_EdgeLoadAnalytics API | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0281 | 1 | B | Updates to the OpenAPI for SS_ADAE_EdgeLoadAnalytics API | 18.6.0 |
| 2024-06 | CT#104 | CP-241116 | 0282 |  | F | Various corrections to the MBS resources definition | 18.6.0 |
| 2024-06 | CT#104 | CP-241084 | 0287 | 1 | F | Correct the reference to HTTP error status codes and NBI usages | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0289 | 1 | F | UE to UE Performance Analytics API corrections | 18.6.0 |
| 2024-06 | CT#104 | CP-241116 | 0292 | 1 | F | Essential corrections to the SS_IdmParameterProvisioning API | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0294 |  | F | Corrections on SS_ADAE_EdgeLoadAnalytics service | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0295 | 1 | F | Corrections on SS_ADAE_LocationAccuracyAnalytics service | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0296 |  | F | Corrections on SS_ADAE_ServiceApiAnalytics service | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0297 | 1 | F | Corrections on SS_ADAE_SlicePerformanceAnalytics service | 18.6.0 |
| 2024-06 | CT#104 | CP-241100 | 0298 | 1 | F | Corrections on SS_ADAE_SliceUsagePatternAnalytics service | 18.6.0 |
| 2024-06 | CT#104 | CP-241099 | 0299 |  | F | Corrections on SS_ADAE_VALPerformanceAnalytics service | 18.6.0 |
| 2024-06 | CT#104 | CP-241086 | 0300 |  | F | Update of info and externalDocs fields | 18.6.0 |
| 2024-09 | CT#105 | CP-242123 | 0302 | 1 | F | Corrections to SS_ADAE_EdgeLoadAnalytics API | 18.7.0 |
| 2024-09 | CT#105 | CP-242123 | 0306 | 1 | F | Correction on data types of SS_AADRF_DataManagement API | 18.7.0 |
| 2024-09 | CT#105 | CP-242162 | 0307 |  | F | EN resolution in the SS_VALServiceAreaConfiguration API | 18.7.0 |
| 2024-09 | CT#105 | CP-242123 | 0308 | 1 | F | Service names clarifications | 18.7.0 |
| 2024-09 | CT#105 | CP-242123 | 0309 | 1 | F | A-ADRF reporting requirements | 18.7.0 |
| 2024-09 | CT#105 | CP-242123 | 0310 | 1 | F | A-ADRF data type contents | 18.7.0 |
| 2024-09 | CT#105 | CP-242123 | 0311 |  | F | Corrections on event Criteria | 18.7.0 |
| 2024-09 | CT#105 | CP-242123 | 0312 |  | F | Corrections on SS_ADAE_Ue2UePerformanceAnalytics API Uri | 18.7.0 |
| 2024-09 | CT#105 | CP-242123 | 0313 | 1 | F | Corrections on the pair of VAL UEs | 18.7.0 |
| 2024-09 | CT#105 | CP-242123 | 0314 |  | F | Corrections on the SS_ADAE_ServiceApiAnalytics API | 18.7.0 |
| 2024-09 | CT#105 | CP-242123 | 0315 |  | F | Essential corrections on analytics output | 18.7.0 |
| 2024-09 | CT#105 | CP-242123 | 0319 | 1 | F | Resolve the Editor's Note and corrections on the ADAES API | 18.7.0 |
| 2024-09 | CT#105 | CP-242120 | 0320 |  | F | Update of info and externalDocs fields | 18.7.0 |
| 2024-09 | CT#105 | CP-242114 | 0301 | 1 | F | Corrections to Location Reporting Notification | 19.0.0 |
| 2024-09 | CT#105 | CP-242121 | 0303 | 1 | B | Monitoring profiles in the SS_NetworkResourceMonitoring API | 19.0.0 |
| 2024-09 | CT#105 | CP-242121 | 0304 | 1 | B | Termination indication in the SS_NetworkResourceMonitoring API | 19.0.0 |
| 2024-09 | CT#105 | CP-242114 | 0305 | 1 | F | The correction of common data types references from 29.571 to 29.122. | 19.0.0 |
| 2024-09 | CT#105 | CP-242114 | 0316 |  | F | Corrections on SS_ADAE_VALPerformanceAnalytics API | 19.0.0 |
| 2024-09 | CT#105 | CP-242114 | 0317 | 1 | F | Corrections on the several ADAE APIs | 19.0.0 |
| 2024-09 | CT#105 | CP-242114 | 0318 | 1 | F | Corrections on the SS_ADAE_Ue2UePerformanceAnalytics API | 19.0.0 |
| 2024-09 | CT#105 | CP-242113 | 0321 |  | F | Update of info and externalDocs fields | 19.0.0 |
| 2024-12 | CT#106 | CP-243092 | 0322 | 1 | B | Support of value-added UE location information exposure | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0323 | 1 | B | Adaptive reporting in the SS_LocationReporting API | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0324 | 1 | B | Dynamic geofencing support in Location Management Service | 19.1.0 |
| 2024-12 | CT#106 | CP-243141 | 0325 | 1 | B | Metaverse introduction. | 19.1.0 |
| 2024-12 | CT#106 | CP-243094 | 0329 |  | B | Updates to the SS_NetworkResourceAdaptation API to support the SEALDD Server as a service consumer | 19.1.0 |
| 2024-12 | CT#106 | CP-243082 | 0330 | 1 | F | Corrections to the functionalities of the SEAL_Ext feature | 19.1.0 |
| 2024-12 | CT#106 | CP-243143 | 0332 | 1 | B | Introduction of AIMLE service in SEAL | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0333 | 1 | B | SS_ADAE_collision_detection_analytics API definition | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0334 | 1 | B | SS_ADAE_location-related_UE_group_analytics API definition | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0335 | 1 | B | Updates to the SS_LocationAreaMonitoring API to support geofencing | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0336 |  | B | Define the service description clauses of the SS_ConfirmLocation API | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0337 | 1 | B | Define the API definition clauses of the SS_ConfirmLocation API | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0338 | 1 | B | Define the OpenAPI description of the SS_ConfirmLocation API | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0339 |  | B | Define the service description clauses of the SS_LocationHistoryInfoEvent API | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0340 | 1 | B | Define the API definition clauses of the SS_LocationHistoryInfoEvent API | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0341 | 1 | B | Define the OpenAPI description of the SS_LocationHistoryInfoEvent API | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0342 | 1 | B | Define SS_SLPositioningManagement API procedures | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0343 | 1 | B | Define SS_SLPositioningManagement open API | 19.1.0 |
| 2024-12 | CT#106 | CP-243086 | 0344 | 1 | F | SEAL – Correcting structured data types in query parameters | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0345 | 1 | B | Define SS_SLPositioningManagement API resources and data model | 19.1.0 |
| 2024-12 | CT#106 | CP-243086 | 0347 | 1 | F | Introduction of ADAE service in SEAL | 19.1.0 |
| 2024-12 | CT#106 | CP-243086 | 0348 |  | F | Correction of the SS_ADAE_Ue2UePerformanceAnalytics API description | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0349 | 1 | B | SS_ADAE_collision_detection_analytics OpenAPI file | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0350 | 1 | B | SS_ADAE_collision_detection_analytics API service operations | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0351 | 1 | B | SS_ADAE_location-related_UE_group_analytics OpenAPI file | 19.1.0 |
| 2024-12 | CT#106 | CP-243092 | 0352 | 1 | B | SS_ADAE_location-related_UE_group_analytics API service operations | 19.1.0 |
| 2024-12 | CT#106 | CP-243147 | 0353 |  | F | Update of info and externalDocs fields | 19.1.0 |
| 2025-03 | CT#107 | CP-250121 | 0354 |  | F | Digital asset management update | 19.2.0 |
| 2025-03 | CT#107 | CP-250103 | 0355 | 1 | B | Introduction of AIMLE specification | 19.2.0 |
| 2025-03 | CT#107 | CP-250087 | 0356 | 1 | B | Update service operation definition in the SS_ADAE_CollisionDetectionAnalytics API | 19.2.0 |
| 2025-03 | CT#107 | CP-250087 | 0357 | 1 | B | Update service operation definition in the SS_ADAE_LocationRelatedUeGroupAnalytics API | 19.2.0 |
| 2025-03 | CT#107 | CP-250196 | 0358 | 2 | B | Analytics output for the SS_ADAE_CollisionDetectionAnalytics API | 19.2.0 |
| 2025-03 | CT#107 | CP-250087 | 0359 | 1 | B | PUT and PATCH methods definition in the SS_ADAE_CollisionDetectionAnalytics API | 19.2.0 |
| 2025-03 | CT#107 | CP-250087 | 0360 | 1 | B | PUT and PATCH methods definition in the SS_ADAE_LocationRelatedUeGroupAnalytics API | 19.2.0 |
| 2025-03 | CT#107 | CP-250087 | 0361 | 1 | B | Adaptive Location Configuration Notification procedure definition | 19.2.0 |
| 2025-03 | CT#107 | CP-250197 | 0362 | 2 | B | Location deviation criteria definition in the SS_ADAE_LocationRelatedUeGroupAnalytics API | 19.2.0 |
| 2025-03 | CT#107 | CP-250087 | 0363 | 1 | B | Analytics output for the SS_ADAE_LocationRelatedUeGroupAnalytics API | 19.2.0 |
| 2025-03 | CT#107 | CP-250085 | 0364 |  | F | PATCH method description correction | 19.2.0 |
| 2025-03 | CT#107 | CP-250103 | 0365 | 1 | B | Procedure of SS_ADAE_AIMLMemberCapabilityAnalytics API | 19.2.0 |
| 2025-03 | CT#107 | CP-250103 | 0366 | 1 | B | Data model of SS_ADAE_AIMLMemberCapabilityAnalytics API | 19.2.0 |
| 2025-03 | CT#107 | CP-250258 | 0367 | 3 | B | Definition of SS_ADAE_AIMLMemberCapabilityAnalytics API | 19.2.0 |
| 2025-03 | CT#107 | CP-250087 | 0368 |  | F | Corrected incorrect data type name | 19.2.0 |
| 2025-03 | CT#107 | CP-250087 | 0369 | 1 | F | Corrected incorrect API name | 19.2.0 |
| 2025-03 | CT#107 | CP-250087 | 0370 |  | B | Complete the updates to the SS_LocationAreaMonitoring API to support geofencing | 19.2.0 |
| 2025-03 | CT#107 | CP-250087 | 0371 |  | F | Corrections to the definition of the Adaptive Location Configuration Notification | 19.2.0 |
| 2025-03 | CT#107 | CP-250080 | 0372 |  | B | Necessary updates to the notifications of the SS_NetworkResourceAdaptation API | 19.2.0 |
| 2025-03 | CT#107 | CP-250114 | 0374 | 2 | A | Updates to complete the support of MBS Service Area related handling | 19.2.0 |
| 2025-03 | CT#107 | CP-250121 | 0375 | 1 | B | Update with new TS for metaverse enablement services | 19.2.0 |
| 2025-03 | CT#107 | CP-250115 | 0376 | 1 | B | SS_ADAE_UE_RAT_Connectivity_Analytics API – API data model | 19.2.0 |
| 2025-03 | CT#107 | CP-250129 | 0378 |  | F | Update of info and externalDocs fields | 19.2.0 |
| 2025-06 | CT#108 | CP-251096 | 0379 |  | F | Updates and corrections to the definition of the SS_ADAE_UE_RAT_Connectivity_Analytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251082 | 0380 |  | B | Further updates to complete the support of MBS Service Area related handling | 19.3.0 |
| 2025-06 | CT#108 | CP-251096 | 0383 | 1 | B | Support of Satellite SF events information | 19.3.0 |
| 2025-06 | CT#108 | CP-251108 | 0384 | 1 | F | Resolve the ENs in SS_SLPositioningManagement Service API | 19.3.0 |
| 2025-06 | CT#108 | CP-251108 | 0385 | 2 | B | Support of Short-Range based positioning information procedure | 19.3.0 |
| 2025-06 | CT#108 | CP-251082 | 0386 | 1 | B | SS_ADCCF_DataCollection API definition | 19.3.0 |
| 2025-06 | CT#108 | CP-251082 | 0387 | 1 | B | SS_ADCCF_DataCollection API service operations | 19.3.0 |
| 2025-06 | CT#108 | CP-251082 | 0388 | 1 | B | SS_AADRF_Data_Storage API definition | 19.3.0 |
| 2025-06 | CT#108 | CP-251082 | 0390 | 1 | B | SS_AADRF_Data_Storage API service operations | 19.3.0 |
| 2025-06 | CT#108 | CP-251112 | 0391 | 1 | B | Analytics output for SS_ADAE_EdgeLoadAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251112 | 0392 | 2 | B | Analytics output for SS_ADAE_LocationAccuracyAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251112 | 0393 | 2 | B | Analytics output for SS_ADAE_SlicePerformanceAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251112 | 0394 | 1 | B | Analytics output for SS_ADAE_VALPerformanceAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251082 | 0395 | 1 | B | BDT support and related corrections in SS_NetworkResourceAdaptation API | 19.3.0 |
| 2025-06 | CT#108 | CP-251110 | 0396 | 1 | B | Update to the procedure of SS_ADAE_AIMLMemberCapabilityAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251110 | 0397 | 1 | B | Update to the Data model of SS_ADAE_AIMLMemberCapabilityAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251110 | 0398 | 1 | B | Update to the definition of SS_ADAE_AIMLMemberCapabilityAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251084 | 0399 | 2 | F | Updates to obsoleted IETF RFC | 19.3.0 |
| 2025-06 | CT#108 | CP-251100 | 0400 | 1 | B | Service Descriptions for SS_ADAE_ServerToServerPerformanceAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251100 | 0401 | 1 | B | API definitions for SS_ADAE_ServerToServerPerformanceAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251100 | 0402 | 1 | B | OpenAPI file definitions for SS_ADAE_Server2ServerPerformanceAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251100 | 0403 | 1 | B | Server To Server Analytics service descriptions | 19.3.0 |
| 2025-06 | CT#108 | CP-251100 | 0404 | 1 | B | API definitions updates for Server To Server Analytics | 19.3.0 |
| 2025-06 | CT#108 | CP-251100 | 0405 | 2 | B | OpenAPI definitions updates for Server To Server Analytics | 19.3.0 |
| 2025-06 | CT#108 | CP-251096 | 0406 | 2 | B | SS_ADAE_UeRatConnectivityAnalytics OpenAPI file | 19.3.0 |
| 2025-06 | CT#108 | CP-251096 | 0407 | 1 | B | SS_ADAE_UeRatConnectivityAnalytics service operations description | 19.3.0 |
| 2025-06 | CT#108 | CP-251096 | 0408 | 1 | B | ADRF UE RAT connectivity Analytics - API Definition | 19.3.0 |
| 2025-06 | CT#108 | CP-251096 | 0409 | 1 | B | ADRF UE RAT Connectivity Analytics - Service Operations | 19.3.0 |
| 2025-06 | CT#108 | CP-251288 | 0410 | 3 | B | ADRF UE RAT Connectivity Analytics - OpenAPI | 19.3.0 |
| 2025-06 | CT#108 | CP-251096 | 0411 |  | F | Correcting references for the SS_ADAE_UeRatConnectivityAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251096 | 0412 |  | F | Updates and corrections to the SS_ADAE_UeRatConnectivityAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251108 | 0413 |  | F | Correcting references for the SS_ADAE_CollisionDetectionAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251108 | 0414 |  | F | Correcting references for the SS_ADAE_LocationRelatedUeGroupAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251110 | 0415 | 2 | F | Correcting references for the SS_ADAE_AIMLMemberCapabilityAnalytics API | 19.3.0 |
| 2025-06 | CT#108 | CP-251111 | 0419 |  | A | Correcting the "identities" attribute name | 19.3.0 |
| 2025-06 | CT#108 | CP-251108 | 0420 | 1 | B | Update geofencing conditions | 19.3.0 |
| 2025-06 | CT#108 | CP-251121 | 0423 |  | A | Incorrect data structure name | 19.3.0 |
| 2025-06 | CT#108 | CP-251084 | 0424 |  | F | Incorrect HTTP method reference for mandatory HTTP error status code | 19.3.0 |
| 2025-06 | CT#108 | CP-251232 | 0426 |  | F | Update of info and externalDocs fields | 19.3.0 |