3GPP TS 29.522 V19.5.0 (2025-10)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Network Exposure Function Northbound APIs;

Stage 3

(Release 19)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	40

1	Scope	41

2	References	41

3	Definitions and abbreviations	44

3.1	Definitions	44

3.2	Abbreviations	44

4	NEF Northbound Interface	45

4.1	Overview	45

4.2	Reference model	49

4.3	Functional elements	49

4.3.1	NEF	49

4.3.2	AF	50

4.4	Procedures over NEF Northbound Interface	50

4.4.1	Introduction	50

4.4.2	Procedures for Monitoring	50

4.4.3	Procedures for Device Triggering	61

4.4.4	Procedures for resource management of Background Data Transfer	61

4.4.5	Procedures for CP Parameters Provisioning	62

4.4.6	Procedures for PFD Management	63

4.4.7	Procedures for Traffic Influence	64

4.4.7.1	General	64

4.4.7.2	AF request identified by UE address	65

4.4.7.3	AF request not identified by UE address	65

4.4.7.4	Handling of Event Notifications	66

4.4.7.4.1	Handling of UP path management event notification	66

4.4.7.4.2	Handling of Traffic routing requirements enforcement outcome event notification	67

4.4.7.5	Processing AF requests to influence traffic routing for HR-SBO session	67

4.4.7.6	Handling of payload headers event notification	68

4.4.8	Procedures for changing the chargeable party at session set up or during the session	68

4.4.9	Procedures for AF required QoS	69

4.4.9.1	General	69

4.4.9.2	Procedures for AF setting up an AF session with required QoS for target UE identified by UE address or for target list of UEs identified by list of UE addresses	69

4.4.9.2.0	General	69

4.4.9.2.1	Procedure for QoS handling and QoS monitoring for list of UEs	71

4.4.9.2.2	Subscriptions to QoS handling and QoS Monitoring	72

4.4.9.2.3	Provisioning of TSCAI input Information and QoS related data	75

4.4.9.2.4	Subscription to the access type change and plmn change	77

4.4.9.2.5	Provisioning of ToS traffic class	77

4.4.9.2.6	Provisioning of multi-modal services	77

4.4.9.2.7	Provisioning of Round-Trip latency requirements	79

4.4.9.2.8	Subscription to the report of L4S support information	79

4.4.9.2.9	Provisioning of PDU Set handling	79

4.4.9.2.10	Provisioning of traffic information for UE power saving management	80

4.4.9.2.11	Provisioning of the QoS timing information	80

4.4.9.2.12	Provisioning of Dynamic ChangingTraffic Characteristics	80

4.4.9.2.13	Provisioning of the Multiplexed Media Identification Information	81

4.4.9.2.14 Provisioning of Media Related Information deliver	81

4.4.9.2.15	Subscription to the Data Rate Limitation information reporting	81

4.4.9.3	Procedures for AF requested QoS for a target UE or group of UE(s) not identified by UE address(es)	81

4.4.10	Procedures for MSISDN-less Mobile Originated SMS	82

4.4.11	Procedures for Network Configuration Parameters Provisioning	82

4.4.12	Procedures for Non-IP data delivery	84

4.4.12.1	General	84

4.4.12.2	NIDD configuration Triggered by the NEF	84

4.4.12.3	NIDD configuration triggered by the AF and NIDD delivery	84

4.4.13	Procedures for RACS Parameter Provisioning	84

4.4.14	Procedures for analytics information exposure	84

4.4.14.1	Subscription/unsubscription to notification of analytics information	84

4.4.14.2	Fetch analytics information	87

4.4.15	Procedures for 5G LAN Parameter Provisioning	87

4.4.15.1	General	87

4.4.15.2	Creation of a new subscription for 5G LAN parameter provisioning	87

4.4.15.3	Modification of an existing subscription for 5G LAN parameter provisioning	88

4.4.15.4	Deletion of an existing subscription for 5G LAN parameter provisioning	88

4.4.15.5	5G LAN parameter provisioning event notification	88

4.4.16	Procedures for applying BDT policy	88

4.4.17	Procedures for Enhanced Coverage Restriction Control	89

4.4.18	Procedures for IPTV Configuration	89

4.4.20	Procedures for service specific parameter provisioning	91

4.4.21	Procedures for ACS configuration parameter provisioning	95

4.4.22	Procedures for Mobile Originated Location Request	95

4.4.22.1	General	95

4.4.22.2	Location Update Notification triggered by UE	95

4.4.23	Procedures for AKMA	96

4.4.23.1	General	96

4.4.23.2	AKMA Application Key Request	96

4.4.23.3	AKMA Service Disablement Notification	96

4.4.24	Procedures for Time Synchronization Exposure	96

4.4.24.0	General	96

4.4.24.1	Subscription and unsubscription to notification of Time Synchronization Capabilites	97

4.4.24.2	Time Synchronization Exposure Configuration	98

4.4.24.3	Management of 5G access stratum time distribution	99

4.4.25	Procedures for ECS address Provisioning	101

4.4.26	Procedures for AM Policy Authorization	101

4.4.26.1	General	101

4.4.26.2	Creation of a new Individual Application AM Context	101

4.4.26.3	Modification of an existing individual Application AM Context	102

4.4.26.4	Deletion of an existing individual Application AM Context	102

4.4.26.5	Create or modify subscription to notification of AM policy event	103

4.4.26.6	Unsubscription to notification of AM policy event	103

4.4.26.7	Notification of AM policy event	103

4.4.27	Procedures for AF triggered Access and Mobility Influence	104

4.4.27.1	General	104

4.4.27.2	Create the AM Influence Subscription	104

4.4.27.3	Modifiy the AM Influence Subscription	104

4.4.27.4	Delete the AM Influence Subscription	104

4.4.27.5	Notification of AM influence events	105

4.4.28	Procedures for Northbound EAS Deployment Information management	105

4.4.28.1	General	105

4.4.28.2	Creation of a new Individual EAS Deployment information resource	105

4.4.28.3	Modification of an existing individual EAS Deployment Information resource	106

4.4.28.4	Deletion of an existing individual EAS Deployment Information resource	106

4.4.28.5	Deletion of EAS Deplyoment Information based on given criteria	107

4.4.29	Procedures for MBS Management	107

4.4.29.1	General	107

4.4.29.2	Procedures for MBS TMGI management	107

4.4.29.2.1	General	107

4.4.29.2.2	Procedure for MBS TMGI(s) allocation or MBS TMGI(s) expiry time refresh	107

4.4.29.2.3	Procedure for MBS TMGI(s) deallocation	108

4.4.29.2.4	Procedure for MBS TMGI(s) timer expiry notification	109

4.4.29.3	Procedures for MBS session management	109

4.4.29.3.1	General	109

4.4.29.3.2	Procedure for MBS session creation	109

4.4.29.3.3	Procedure for MBS session update	112

4.4.29.3.4	Procedure for MBS session deletion	112

4.4.29.3.5	Procedure for MBS session status subscription	113

4.4.29.3.6	Procedure for MBS session status unsubscription	113

4.4.29.3.7	Procedure for MBS session status notification	113

4.4.29.4	Procedures for MBS Parameters Provisioning	114

4.4.29.4.1	General	114

4.4.29.4.2	Procedure for multicast MBS Session Authorization information provisioning	114

4.4.29.4.3	Procedure for multicast MBS Session Assistance information provisioning	115

4.4.29.5	Procedures for MBS User Service management	116

4.4.29.5.1	General	116

4.4.29.5.2	Procedure for MBS User Service creation	116

4.4.29.5.3	Procedure for MBS User Service retrieval	117

4.4.29.5.4	Procedure for MBS User Service update/modification	117

4.4.29.5.5	Procedure for MBS User Service deletion	117

4.4.29.6	Procedures for MBS User Data Ingest Session management	118

4.4.29.6.1	General	118

4.4.29.6.2	Procedure for MBS User Data Ingest Session creation	118

4.4.29.6.3	Procedure for MBS User Data Ingest Session retrieval	118

4.4.29.6.4	Procedure for MBS User Data Ingest Session update/modification	119

4.4.29.6.5	Procedure for MBS User Data Ingest Session deletion	119

4.4.29.6.6	Procedure for MBS User Data Ingest Session Status Subscription	119

4.4.29.6.7	Procedure for MBS User Data Ingest Session Status update/modification	120

4.4.29.6.8	Procedure for MBS User Data Ingest Session Status Unsubscription	120

4.4.29.6.9	Procedure for MBS User Data Ingest Session Status Notification	121

4.4.29.7	Procedures for MBS Group Message Delivery Management	121

4.4.29.7.1	General	121

4.4.29.7.2	Procedure for MBS Group Message Delivery Creation	121

4.4.29.7.3	Procedure for MBS Group Message Delivery Update	122

4.4.29.7.4	Procedure for MBS Group Message Delivery Deletion	122

4.4.29.7.5	Procedure for MBS Group Message Delivery Status Notification	123

4.4.30	Procedures for Data Reporting	123

4.4.30.1	General	123

4.4.30.2	Procedure for Data Reporting Session Management	123

4.4.30.3	Procedure for Data Report	124

4.4.31	Procedures for Data Reporting Provisioning	124

4.4.31.1	General	124

4.4.31.2	Procedure for Data Reporting Provisioning Session Management	124

4.4.31.3	Procedure for Data Reporting Configuration management	124

4.4.32	Procedures for UE ID retrieval	125

4.4.32.1	General	125

4.4.32.2	Retrieve AF specific UE ID service operation	126

4.4.32.3	Retrieve UE ID in the form of MSISDN	127

4.4.32.4	UE ID Mapping Information Provisioning	128

4.4.32.5	Verify UE ID in the form of MSISDN	129

4.4.33	Procedures for Media Streaming Event Exposure	129

4.4.33.1	General	129

4.4.33.2	Procedure for Media Streaming Event Exposure Subscription Creation	129

4.4.33.3	Procedure for Media Streaming Event Exposure Subscription Update	130

4.4.33.4	Procedure for Media Streaming Event Exposure Unsubscription	130

4.4.33.5	Procedure for Media Streaming Event Exposure Notification	131

4.4.34	Procedures for DNAI Mapping	131

4.4.34.1	General	131

4.4.34.2	Creation of a new subscription for DNAI Mapping	131

4.4.34.3	Deletion of an existing individual DNAI Mapping subscription	132

4.4.34.4	Notification for updated DNAI Mapping information	132

4.4.35	Procedures for negotiation of planned data transfer with QoS requirements	132

4.4.36	Procedures for Member UE Selection Assistance	133

4.4.36.1	General	133

4.4.36.2	Procedure for Member UE Selection Assistance Subscription Creation	133

4.4.36.3	Procedure for Member UE Selection Assistance Subscription Update	133

4.4.36.4	Procedure for Member UE Selection Assistance Subscription Unsubscription	134

4.4.36.5	Procedure for Member UE Selection Assistance Notification	134

4.4.37	Procedures for Group Parameters Provisioning	134

4.4.37.1	General	134

4.4.37.2	Procedures for DNN and S-NSSAI specific Group Parameters Provisioning	134

4.4.38	Procedures for Network Slice Parameters Provisioning	135

4.4.38.1	General	135

4.4.38.2	Procedures for Network Slice Usage Control Parameters Provisioning	137

4.4.39	Procedures for UE Address Information Management	137

4.4.39.1	General	137

4.4.39.2	Procedures for UE Address(es) Information Retrieval	137

4.4.39.3	UE Address Subscription Creation	138

4.4.39.4	UE Address Subscription Deletion	138

4.4.39.5	Notification for UE Address information	138

4.4.40	Procedures for ECS Address Configuration Information provisioning in roaming	139

4.4.40.1	General	139

4.4.40.2	Creation of new ECS Address Configuration Information Set	139

4.4.40.3	Modification of existing ECS Address Configuration Information Set	139

4.4.40.4	Deletion of existing Individual ECS Address Configuration Information Set	140

4.4.40.5	Deletion of ECS Address Configuration Information based on given criteria	140

4.4.41.1	General	140

4.4.41.2	Procedures for RSLPPI Parameters Provisioning	141

4.4.42.1	General	142

4.4.42.2	Procedures for Static IP Addressing Parameters Provisioning	142

4.4.43.1	General	143

4.4.43.2	Procedures for UAV Flight Assistance Configuration Management	143

4.4.43.2.1	General	143

4.4.43.2.2	Procedure for UAV Flight Assistance Configuration Creation	144

4.4.43.2.3	Procedure for UAV Flight Assistance Configuration Update	144

4.4.43.2.4	Procedure for UAV Flight Assistance Configuration Deletion	144

4.4.43.2.5	Procedure for UAV Flight Assistance Notification	144

4.4.43.3	Procedures for UAV Flight information Retrieval	145

4.4.43.3.1	General	145

4.4.43.3.2	Procedures for UAV Flight information Retrieval	145

4.4.44	Procedures for CAG Information Parameters Provisioning	145

4.4.44.1	General	145

4.4.45	Void	146

4.4.46	Procedures for IMS Session Management	146

4.4.46.1	General	146

4.4.46.2	Procedure for IMS session Creation	147

4.4.46.3	Procedure for IMS session Update	147

4.4.46.4	Procedure for IMS session Deletion	148

4.4.46.5	Procedure for IMS Session Notification	148

4.4.47	Procedures for IMS Event Exposure Management	148

4.4.47.1	General	148

4.4.47.2	Procedure for IMS Event Exposure Subscription Creation	149

4.4.47.3	Procedure for IMS Event Exposure Subscription Update	149

4.4.47.4	Procedure for IMS Event Exposure Subscription Deletion	150

4.4.47.5	Procedure forIMS Event Exposure Notification	150

4.4.48	Procedures for IMS Parameters Provision Service	151

4.4.48.1	General	151

4.4.48.2	Procedures for IMS Parameters Provisioning	151

4.4.49	Procedures for AIoT Services Management	152

4.4.49.1	General	152

4.4.49.2	Procedures for AIoT Inventory Management	152

4.4.49.3	Procedures for AIoT Command Management	153

4.4.49.4	Procedures for AIoT Operations Notification	153

4.4.52	Procedures for VFL NF discovery	156

4.4.52.1	General	156

4.4.52.2	Procedures for NWDAF discovery	156

4.4.52.3	Procedures for NWDAF release	157

5	NEF Northbound APIs	157

5.1	Introduction	157

5.2	Information applicable to several APIs	160

5.3	Reused APIs	160

5.4	TrafficInfluence API	161

5.4.0	Introduction	161

5.4.1	Resources	162

5.4.1.1	Overview	162

5.4.1.2	Resource: Traffic Influence Subscription	162

5.4.1.2.1	Introduction	162

5.4.1.2.2	Resource Definition	162

5.4.1.2.3	Resource Methods	163

5.4.1.2.3.1	General	163

5.4.1.2.3.2	GET	163

5.4.1.2.3.3	POST	164

5.4.1.3	Resource: Individual Traffic Influence Subscription	164

5.4.1.3.1	Introduction	164

5.4.1.3.2	Resource Definition	164

5.4.1.3.3	Resource Methods	165

5.4.1.3.3.1	General	165

5.4.1.3.3.2	GET	165

5.4.1.3.3.3	PUT	166

5.4.1.3.3.4	PATCH	166

5.4.1.3.3.5	DELETE	167

5.4.1A	Custom Operations without associated resources	168

5.4.2	Notifications	168

5.4.2.1	Introduction	168

5.4.2.2	Event Notification	169

5.4.2.2.1	Description	169

5.4.2.2.2	Target URI	169

5.4.2.2.3	Operation Definition	169

5.4.2.3	Acknowledgement of event notification	170

5.4.2.3.1	Description	170

5.4.2.3.2	Target URI	170

5.4.2.3.3	Operation Definition	171

5.4.2.3.3.1	Notification via HTTP POST	171

5.4.2.4	Event Notification of Payload Headers	171

5.4.2.4.1	Description	171

5.4.2.4.2	Target URI	171

5.4.2.4.3	Operation Definition	172

5.4.3	Data Model	173

5.4.3.1	General	173

5.4.3.2	Reused data types	173

5.4.3.3	Structured data types	174

5.4.3.3.1	Introduction	174

5.4.3.3.2	Type: TrafficInfluSub	174

5.4.3.3.3	Type: TrafficInfluSubPatch	181

5.4.3.3.4	Type: EventNotification	186

5.4.3.3.5	Type: AfResultInfo	189

5.4.3.3.6	Type AfAckInfo	190

5.4.3.3.7	Type TrafficDataSet	190

5.4.3.3.8	Type TrafficDataSetRm	190

5.4.3.3.9	Type: HeaderHandlingReport	191

5.4.3.4	Simple data types and enumerations	191

5.4.3.4.1	Introduction	191

5.4.3.4.2	Simple data types	191

5.4.3.4.3	Enumeration: SubscribedEvent	191

5.4.3.4.4	Enumeration: AfResultStatus	191

5.4.4	Used Features	192

5.4.5	Error handling	193

5.4.5.1	General	193

5.4.5.2	Protocol Errors	193

5.4.5.3	Application Errors	193

5.5	NiddConfigurationTrigger API	193

5.5.0	Introduction	193

5.5.1	Resources	193

5.5.1A	Custom Operations without associated resources	193

5.5.2	Notifications	193

5.5.2.1	Introduction	193

5.5.2.2	Event Notification	194

5.5.2.3	Operation Definition	194

5.5.2.3.1	Notification via HTTP POST	194

5.5.2.3.2	Notification via Websocket	195

5.5.3	Data Model	195

5.5.3.1	General	195

5.5.3.2	Reused data types	195

5.5.3.3	Structured data types	195

5.5.3.3.1	Introduction	195

5.5.3.3.2	Type: NiddConfigurationTrigger	195

5.5.3.3.3	Type: NiddConfigurationTriggerReply	195

5.5.3.4	Simple data types and enumerations	196

5.5.3.4.1	Introduction	196

5.5.3.4.2	Simple data types	196

5.5.4	Used Features	196

5.5.5	Error handling	196

5.5.5.1	General	196

5.5.5.2	Protocol Errors	196

5.5.5.3	Application Errors	196

5.6	AnalyticsExposure API	197

5.6.0	Introduction	197

5.6.1	Resources	197

5.6.1.1	Overview	197

5.6.1.2	Resource: Analytics Exposure Subscriptions	198

5.6.1.2.1	Introduction	198

5.6.1.2.2	Resource Definition	198

5.6.1.2.3	Resource Methods	198

5.6.1.2.3.1	General	198

5.6.1.2.3.2	GET	198

5.6.1.2.3.3	POST	199

5.6.1.3	Resource: Individual Analytics Exposure Subscription	200

5.6.1.3.1	Introduction	200

5.6.1.3.2	Resource Definition	200

5.6.1.3.3	Resource Methods	200

5.6.1.3.3.1	General	200

5.6.1.3.3.2	GET	200

5.6.1.3.3.3	PUT	201

5.6.1.3.3.4	DELETE	202

5.6.1A	Custom Operations without associated resources	203

5.6.1A.1	Overview	203

5.6.1A.2	Operation: fetch	204

5.6.1A.2.1	Description	204

5.6.1A.2.2	Operation Definition	204

5.6.2	Notifications	205

5.6.2.1	Introduction	205

5.6.2.2	Event Notification	205

5.6.2.3	Operation Definition	205

5.6.2.3.1	Notification via HTTP POST	205

5.6.2.3.2	Notification via Websocket	206

5.6.3	Data Model	206

5.6.3.1	General	206

5.6.3.2	Reused data types	207

5.6.3.3	Structured data types	212

5.6.3.3.1	Introduction	212

5.6.3.3.2	Type: AnalyticsExposureSubsc	212

5.6.3.3.3	Type: AnalyticsEventNotification	214

5.6.3.3.4	Type: AnalyticsEventNotif	215

5.6.3.3.5	Type: AnalyticsEventSubsc	219

5.6.3.3.6	Type: AnalyticsEventFilterSubsc	220

5.6.3.3.7	Type TargetUeId	227

5.6.3.3.8	Void	228

5.6.3.3.9	Type UeMobilityExposure	228

5.6.3.3.10	Type UeLocationInfo	229

5.6.3.3.11	Void	229

5.6.3.3.12	Type: AnalyticsRequest	229

5.6.3.3.13	Type AnalyticsEventFilter	230

5.6.3.3.14	Type AnalyticsData	236

5.6.3.3.15	Type AbnormalExposure	239

5.6.3.3.16	Type CongestInfo	240

5.6.3.3.17	Type CongestionAnalytics	240

5.6.3.3.18	Type QosSustainabilityExposure	241

5.6.3.3.19	Type NetworkPerfExposure	242

5.6.3.3.20	Type AnalyticsFailureEventInfo	242

5.6.3.3.21	Type WlanPerformInfo	243

5.6.3.4	Simple data types and enumerations	243

5.6.3.4.1	Introduction	243

5.6.3.4.2	Simple data types	243

5.6.3.4.3	Enumeration: AnalyticsEvent	243

5.6.3.4.4	Enumeration: AnalyticsFailureCode	244

5.6.4	Used Features	245

5.6.5	Error handling	249

5.6.5.1	General	249

5.6.5.2	Protocol Errors	249

5.6.5.3	Application Errors	249

5.7	5GLANParameterProvision API	250

5.7.0	Introduction	250

5.7.1	Resources	251

5.7.1.1	Overview	251

5.7.1.2	Resource: 5GLAN Parameters Provision Subscriptions	251

5.7.1.2.1	Introduction	251

5.7.1.2.2	Resource Definition	251

5.7.1.2.3	Resource Methods	252

5.7.1.2.3.1	General	252

5.7.1.2.3.2	GET	252

5.7.1.2.3.3	POST	253

5.7.1.3	Resource: Individual 5GLAN Parameters Provision Subscription	253

5.7.1.3.1	Introduction	253

5.7.1.3.2	Resource Definition	253

5.7.1.3.3	Resource Methods	254

5.7.1.3.3.1	General	254

5.7.1.3.3.2	GET	254

5.7.1.3.3.3	PUT	255

5.7.1.3.3.4	DELETE	255

5.7.1.3.3.5	PATCH	256

5.7.1A	Custom Operations without associated resources	257

5.7.1B	Notifications	257

5.7.1B.1	General	257

5.7.1B.2	5G LAN Parameter Provisioning Event Notification	258

5.7.1B.2.1	Description	258

5.7.1B.2.2	Target URI	258

5.7.1B.2.3	Operation Definition	258

5.7.1B.2.3.1	Notification via HTTP POST	258

5.7.1B.2.3.2	Void	259

5.7.2	Data Model	259

5.7.2.1	General	259

5.7.2.2	Reused data types	259

5.7.2.3	Structured data types	260

5.7.2.3.1	Introduction	260

5.7.2.3.2	Type: 5GLanParametersProvision	261

5.7.2.3.3	Type: 5GLanParameters	261

5.7.2.3.4	Type: AppDescriptor	264

5.7.2.3.5	Type: 5GLanParametersProvisionPatch	265

5.7.2.3.6	Type: 5GLanParametersPatch	265

5.7.2.3.7	Type: AppDescriptorRm	266

5.7.2.3.8	Void	266

5.7.2.3.9	Type: CpParams	266

5.7.2.3.10	Type: NpConfigParams	267

5.7.2.3.11	Type: LpiParams	267

5.7.2.3.12	Type: AcsParams	267

5.7.2.3.13	Type: ECSAddrParams	267

5.7.2.3.14	Type: DnnSnssaiParams	268

5.7.2.3.15	Type: 5GLanParamProvNotif	268

5.7.2.3.16	Type: NpConfigNotif	268

5.7.2.3.17	Type: MaxGrpDataRateInfo	268

5.7.2.4	Simple data types and enumerations	269

5.7.2.4.1	Introduction	269

5.7.2.4.2	Simple data types	269

5.7.2.4.3	Enumeration: AaaUsage	269

5.7.3	Used Features	269

5.7.4	Error handling	270

5.7.4.1	General	270

5.7.4.2	Protocol Errors	270

5.7.4.3	Application Errors	270

5.8	ApplyingBdtPolicy API	270

5.8.0	Introduction	270

5.8.1	Resources	270

5.8.1.1	Overview	270

5.8.1.2	Resource: Applied BDT Policy Subscriptions	271

5.8.1.2.1	Introduction	271

5.8.1.2.2	Resource Definition	271

5.8.1.2.3	Resource Methods	272

5.8.1.2.3.1	General	272

5.8.1.2.3.2	GET	272

5.8.1.2.3.3	POST	273

5.8.1.3	Resource: Individual Applied BDT Policy Subscription	273

5.8.1.3.1	Introduction	273

5.8.1.3.2	Resource Definition	273

5.8.1.3.3	Resource Methods	273

5.8.1.3.3.1	General	273

5.8.1.3.3.2	GET	274

5.8.1.3.3.3	PATCH	274

5.8.1.3.3.4	DELETE	275

5.8.1A	Custom Operations without associated resources	276

5.8.2	Notifications	276

5.8.3	Data Model	276

5.8.3.1	General	276

5.8.3.2	Reused data types	277

5.8.3.3	Structured data types	277

5.8.3.3.1	Introduction	277

5.8.3.3.2	Type: AppliedBdtPolicy	277

5.8.3.3.3	Type: AppliedBdtPolicyPatch	277

5.8.3.4	Simple data types and enumerations	278

5.8.3.4.1	Introduction	278

5.8.3.4.2	Simple data types	278

5.8.4	Used Features	278

5.8.5	Error handling	278

5.8.5.1	General	278

5.8.5.2	Protocol Errors	278

5.8.5.3	Application Errors	278

5.9	IPTVConfiguration API	279

5.9.0	Introduction	279

5.9.1	Resources	279

5.9.1.1	Overview	279

5.9.1.2	Resource: IPTV Configurations	280

5.9.1.2.1	Introduction	280

5.9.1.2.2	Resource Definition	280

5.9.1.2.3	Resource Methods	280

5.9.1.2.3.1	General	280

5.9.1.2.3.2	GET	280

5.9.1.2.3.3	POST	281

5.9.1.3	Resource: Individual IPTV Configuration	282

5.9.1.3.1	Introduction	282

5.9.1.3.2	Resource Definition	282

5.9.1.3.3	Resource Methods	282

5.9.1.3.3.1	General	282

5.9.1.3.3.2	GET	282

5.9.1.3.3.3	PUT	283

5.9.1.3.3.4	DELETE	284

5.9.1.3.3.5	PATCH	285

5.9.1A	Custom Operations without associated resources	286

5.9.1B	Notifications	286

5.9.2	Data Model	286

5.9.2.1	General	286

5.9.2.2	Reused data types	286

5.9.2.3	Structured data types	286

5.9.2.3.1	Introduction	286

5.9.2.3.2	Type: IptvConfigData	287

5.9.2.3.3	Type: MulticastAccessControl	287

5.9.2.3.4	Type: IptvConfigDataPatch	288

5.9.2.4	Simple data types and enumerations	288

5.9.2.4.1	Introduction	288

5.9.2.4.2	Simple data types	288

5.9.2.4.3	Enumeration: AccessRightStatus	288

5.9.3	Used Features	288

5.9.4	Error handling	288

5.9.4.1	General	288

5.9.4.2	Protocol Errors	289

5.9.4.3	Application Errors	289

5.10	LpiParameterProvision API	289

5.10.0	Introduction	289

5.10.1	Resources	289

5.10.1.1	Overview	289

5.10.1.2	Resource: LPI Parameters Provisionings	290

5.10.1.2.1	Introduction	290

5.10.1.2.2	Resource Definition	290

5.10.1.2.3	Resource Methods	290

5.10.1.2.3.1	General	290

5.10.1.2.3.2	GET	290

5.10.1.2.3.3	POST	291

5.10.1.3	Resource: Individual LPI Parameters Provisioning	292

5.10.1.3.1	Introduction	292

5.10.1.3.2	Resource Definition	292

5.10.1.3.3	Resource Methods	292

5.10.1.3.3.1	General	292

5.10.1.3.3.2	GET	292

5.10.1.3.3.3	PUT	293

5.10.1.3.3.3A	PATCH	294

5.10.1.3.3.4	DELETE	295

5.10.1A	Custom Operations without associated resources	296

5.10.1B	Notifications	296

5.10.2	Data Model	296

5.10.2.1	General	296

5.10.2.2	Reused data types	296

5.10.2.3	Structured data types	297

5.10.2.3.1	Introduction	297

5.10.2.3.2	Type: LpiParametersProvision	297

5.10.2.3.3	Type: LpiParametersProvisionPatch	298

5.10.2.4	Simple data types and enumerations	298

5.10.2.4.1	Introduction	298

5.10.2.4.2	Simple data types	298

5.10.3	Used Features	298

5.10.4	Error handling	298

5.10.4.1	General	298

5.10.4.2	Protocol Errors	298

5.10.4.3	Application Errors	299

5.11	ServiceParameter API	299

5.11.0	Introduction	299

5.11.1	Resources	299

5.11.1.1	Overview	299

5.11.1.2	Resource: Service Parameter Subscriptions	300

5.11.1.2.1	Introduction	300

5.11.1.2.2	Resource Definition	300

5.11.1.2.3	Resource Methods	300

5.11.1.2.3.1	General	300

5.11.1.2.3.2	GET	300

5.11.1.2.3.3	POST	302

5.11.1.3	Resource: Individual Service Parameter Subscription	302

5.11.1.3.1	Introduction	302

5.11.1.3.2	Resource Definition	302

5.11.1.3.3	Resource Methods	302

5.11.1.3.3.1	General	302

5.11.1.3.3.2	GET	303

5.11.1.3.3.3	PUT	303

5.11.1.3.3.4	DELETE	304

5.11.1.3.3.5	PATCH	305

5.11.1A	Notifications	306

5.11.1A.1	Introduction	306

5.11.1A.2	AF Notifications	306

5.11.1A.2.1	Description	306

5.11.1A.2.2	Target URI	306

5.11.1A.3	Operation Definition	307

5.11.1A.3.1	Notification via HTTP POST	307

5.11.1A.3.2	Notification via Websocket	308

5.11.1B	Custom Operations without associated resources	308

5.11.2	Data Model	308

5.11.2.1	General	308

5.11.2.2	Reused data types	312

5.11.2.3	Structured data types	313

5.11.2.3.1	Introduction	313

5.11.2.3.2	Type: ServiceParameterData	314

5.11.2.3.3	Type: ServiceParameterDataPatch	318

5.11.2.3.4	Type: UrspRuleRequest	320

5.11.2.3.5	Type: RouteSelectionParameterSet	321

5.11.2.3.6	Type: AfNotification	321

5.11.2.3.7	Type: EventInfo	322

5.11.2.3.8	Type: TrafficDescriptorComponents	323

5.11.2.3.9	Type: NetworkDescription	325

5.11.2.3.10	Void	325

5.11.2.3.11	Void	325

5.11.2.3.12	Type: Non3gppDeviceInformation	325

5.11.2.4	Simple data types and enumerations	326

5.11.2.4.1	Introduction	326

5.11.2.4.2	Simple data types	326

5.11.2.4.3	Enumeration: Event	329

5.11.2.4.4	Enumeration: AuthorizationResult	330

5.11.2.4.5	Enumeration: Failure	330

5.11.2.4.6	Enumeration: ConnectionCapabilities	330

5.11.2.4.7	Enumeration: UePolPartType	331

5.11.3	Used Features	332

5.11.4	Error handling	335

5.11.4.1	General	335

5.11.4.2	Protocol Errors	335

5.11.4.3	Application Errors	335

5.12	ACSParameterProvision API	335

5.12.0	Introduction	335

5.12.1	Resources	335

5.12.1.1	Overview	335

5.12.1.2	Resource: ACS Configuration Subscriptions	336

5.12.1.2.1	Introduction	336

5.12.1.2.2	Resource Definition	336

5.12.1.2.3	Resource Methods	337

5.12.1.2.3.1	General	337

5.12.1.2.3.2	GET	337

5.12.1.2.3.3	POST	338

5.12.1.3	Resource: Individual ACS Configuration Subscription	338

5.12.1.3.1	Introduction	338

5.12.1.3.2	Resource Definition	338

5.12.1.3.3	Resource Methods	338

5.12.1.3.3.1	General	338

5.12.1.3.3.2	GET	339

5.12.1.3.3.3	PUT	339

5.12.1.3.3.3A	PATCH	340

5.12.1.3.3.4	DELETE	341

5.12.1A	Custom Operations without associated resources	342

5.12.1B	Notifications	342

5.12.2	Data Model	342

5.12.2.1	General	342

5.12.2.2	Reused data types	343

5.12.2.3	Structured data types	343

5.12.2.3.1	Introduction	343

5.12.2.3.2	Type: AcsConfigurationData	343

5.12.2.3.3	Type: AcsConfigurationDataPatch	343

5.12.2.4	Simple data types and enumerations	344

5.12.2.4.1	Introduction	344

5.12.2.4.2	Simple data types	344

5.12.3	Used Features	344

5.12.4	Error handling	344

5.12.4.1	General	344

5.12.4.2	Protocol Errors	344

5.12.4.3	Application Errors	344

5.13	MoLcsNotify API	345

5.13.0	Introduction	345

5.13.1	Resources	345

5.13.1A	Custom Operations without associated resources	345

5.13.2	Notifications	345

5.13.2.1	Introduction	345

5.13.2.2	Event Notification	345

5.13.2.3	Operation Definition	345

5.13.2.3.1	Notification via HTTP POST	345

5.13.3	Data Model	346

5.13.3.1	General	346

5.13.3.2	Reused data types	346

5.13.3.3	Structured data types	346

5.13.3.3.1	Introduction	346

5.13.3.3.2	Type: LocUpdateData	347

5.13.3.3.3	Type: LocUpdateDataReply	347

5.13.3.4	Simple data types and enumerations	347

5.13.3.4.1	Introduction	347

5.13.3.4.2	Simple data types	347

5.13.4	Used Features	347

5.13.5	Error handling	348

5.13.5.1	General	348

5.13.5.2	Protocol Errors	348

5.13.5.3	Application Errors	348

5.14	AKMA API	348

5.14.1	Introduction	348

5.14.2	Resources	348

5.14.3	Custom Operations without associated resources	348

5.14.3.1	Overview	348

5.14.3.2	Operation: Retrieve	349

5.14.3.2.1	Description	349

5.14.3.2.2	Operation Definition	349

5.14.4	Notifications	350

5.14.4.1	General	350

5.14.4.2	AKMA Service Disablement Notification	350

5.14.4.2.1	Description	350

5.14.4.2.2	Target URI	350

5.14.4.2.3	Operation Definition	350

5.14.5	Data Model	351

5.14.5.1	General	351

5.14.5.2	Reused data types	352

5.14.5.3	Structured data types	352

5.14.5.3.1	Introduction	352

5.14.5.3.2	Type: AkmaAfKeyRequest	352

5.14.5.3.3	Type: AkmaAfKeyData	353

5.14.5.3.4	Type: ServiceDisableNotif	353

5.14.5.4	Simple data types and enumerations	353

5.14.5.4.1	Introduction	353

5.14.5.4.2	Simple data types	353

5.14.6	Used Features	354

5.14.7	Error handling	354

5.14.7.1	General	354

5.14.7.2	Protocol Errors	354

5.14.7.3	Application Errors	354

5.15	TimeSyncExposure API	355

5.15.0	Introduction	355

5.15.1	Resources	355

5.15.1.1	Overview	355

5.15.1.2	Resource: Time Synchronization Exposure Subscriptions	356

5.15.1.2.1	Introduction	356

5.15.1.2.2	Resource Definition	357

5.15.1.2.3	Resource Methods	357

5.15.1.2.3.1	General	357

5.15.1.2.3.2	GET	357

5.15.1.2.3.3	POST	358

5.15.1.3	Resource: Individual Time Synchronization Exposure Subscription	359

5.15.1.3.1	Introduction	359

5.15.1.3.2	Resource Definition	359

5.15.1.3.3	Resource Methods	359

5.15.1.3.3.1	General	359

5.15.1.3.3.2	GET	359

5.15.1.3.3.3	PUT	360

5.15.1.3.3.4	DELETE	361

5.15.1.4	Resource: Time Synchronization Exposure Configurations	362

5.15.1.4.1	Introduction	362

5.15.1.4.2	Resource Definition	362

5.15.1.4.3	Resource Methods	363

5.15.1.4.3.1	General	363

5.15.1.4.3.2	GET	363

5.15.1.4.3.3	POST	364

5.15.1.5	Resource: Individual Time Synchronization Exposure Configuration	365

5.15.1.5.1	Introduction	365

5.15.1.5.2	Resource Definition	365

5.15.1.5.3	Resource Methods	365

5.15.1.5.3.1	General	365

5.15.1.5.3.2	GET	365

5.15.1.5.3.3	PUT	366

5.15.1.5.3.4	DELETE	367

5.15.2	Custom Operations without associated resources	368

5.15.3	Notifications	368

5.15.3.1	Introduction	368

5.15.3.2	Time Synchronization Capability Notification	369

5.15.3.2.1	Description	369

5.15.3.2.2	Callback URI	369

5.15.3.2.3	Operation Definition	369

5.15.3.2.3.1	Notification via HTTP POST	369

5.15.3.2.3.2	Notification via Websocket	370

5.15.3.3	Time Synchronization Configuration Notification	370

5.15.3.3.1	Description	370

5.15.3.3.2	Callback URI	370

5.15.3.3.3	Operation Definition	370

5.15.3.3.3.1	Notification via HTTP POST	370

5.15.3.3.3.2	Void	371

5.15.4	Data Model	371

5.15.4.1	General	371

5.15.4.2	Reused data types	372

5.15.4.3	Structured data types	373

5.15.4.3.1	Introduction	373

5.15.4.3.2	Type: TimeSyncExposureSubsc	374

5.15.4.3.3	Type: TimeSyncCapability	376

5.15.4.3.4	Void	377

5.15.4.3.5	Void	377

5.15.4.3.6	Type: TimeSyncExposureConfig	377

5.15.4.3.7	Type: TimeSyncExposureSubsNotif	379

5.15.4.3.8	Type SubsEventNotification	379

5.15.4.3.9	Type: TimeSyncExposureConfigNotif	379

5.15.4.3.10	Type: EventFilter	379

5.15.4.3.11	Type: PtpCapabilitiesPerUe	380

5.15.4.3.12	Type: PtpInstance	380

5.15.4.3.13	Void	381

5.15.4.3.14	Void	381

5.15.4.3.15	Void	381

5.15.4.3.16	Void	381

5.15.4.3.17	Type: StateOfConfiguration	381

5.15.4.3.18	Type: ConfigForPort	382

5.15.4.3.19	Type: StateOfDstt	384

5.15.4.3.20	Void	385

5.15.4.3.21	Void	385

5.15.4.3.22	Type: AvailReport	385

5.15.4.4	Simple data types and enumerations	385

5.15.4.4.1	Introduction	385

5.15.4.4.2	Simple data types	385

5.15.4.4.3	Void	385

5.15.4.4.4	Enumeration: Protocol	385

5.15.4.4.5	Enumeration: GmCapable	385

5.15.4.4.6	Enumeration: SubscribedEvent	386

5.15.4.4.7	Enumeration: InstanceType	386

5.15.4.4.8	Enumeration: AsTimeResource	386

5.15.4.4.9	Enumeration: AcceptanceCriteriaResultIndication	386

5.15.4.4.10	Enumeration: AvailStatus	387

5.15.4.5	Data types describing alternative data types or combinations of data types	387

5.15.4.5.1	Type: ReportedCabability	387

5.15.5	Used Features	387

5.15.6	Error handling	387

5.15.6.1	General	387

5.15.6.2	Protocol Errors	387

5.15.6.3	Application Errors	388

5.16	EcsAddressProvision API	388

5.16.0	Introduction	388

5.16.1	Resources	388

5.16.1.1	Overview	388

5.16.1.2	Resource: ECS Address Provision Configurations	389

5.16.1.2.1	Introduction	389

5.16.1.2.2	Resource Definition	389

5.16.1.2.3	Resource Methods	389

5.16.1.2.3.1	General	389

5.16.1.2.3.2	GET	389

5.16.1.2.3.3	POST	390

5.16.1.3	Resource: Individual ECS Address Provision Configuration	391

5.16.1.3.1	Introduction	391

5.16.1.3.2	Resource Definition	391

5.16.1.3.3	Resource Methods	391

5.16.1.3.3.1	General	391

5.16.1.3.3.2	GET	391

5.16.1.3.3.3	PUT	392

5.16.1.3.3.4	DELETE	393

5.16.1A	Custom Operations without associated resources	394

5.16.1B	Notifications	394

5.16.2	Data Model	394

5.16.2.1	General	394

5.16.2.2	Reused data types	394

5.16.2.3	Structured data types	395

5.16.2.3.1	Introduction	395

5.16.2.3.2	Type: EcsAddressProvision	395

5.16.2.4	Simple data types and enumerations	395

5.16.2.4.1	Introduction	395

5.16.2.4.2	Simple data types	396

5.16.3	Used Features	396

5.16.4	Error handling	396

5.16.4.1	General	396

5.16.4.2	Protocol Errors	396

5.16.4.3	Application Errors	396

5.17	AMPolicyAuthorization API	396

5.17.0	Introduction	396

5.17.1	Resources	397

5.17.1.1	Overview	397

5.17.1.2	Resource: Application AM Contexts	398

5.17.1.2.1	Introduction	398

5.17.1.2.2	Resource Definition	398

5.17.1.2.3	Resource Methods	398

5.17.1.2.3.1	General	398

5.17.1.2.3.2	POST	398

5.17.1.3	Resource: Individual Application AM Context	399

5.17.1.3.1	Introduction	399

5.17.1.3.2	Resource Definition	399

5.17.1.3.3	Resource Methods	399

5.17.1.3.3.1	General	399

5.17.1.3.3.2	GET	399

5.17.1.3.3.3	PATCH	400

5.17.1.3.3.4	DELETE	402

5.17.1.4	Resource: AM Policy Events Subscription	403

5.17.1.4.1	Introduction	403

5.17.1.4.2	Resource Definition	403

5.17.1.4.3	Resource Methods	403

5.17.1.4.3.1	General	403

5.17.1.4.3.2	PUT	403

5.17.1.4.3.3	DELETE	405

5.17.1A	Custom Operations without associated resources	405

5.17.2	Notifications	406

5.17.2.1	Introduction	406

5.17.2.2	AM Event Notification	406

5.17.2.2.1	Description	406

5.17.2.2.2	Callback URI	406

5.17.2.2.3	Operation Definition	406

5.17.2.2.3.1	Notification via HTTP POST	406

5.17.2.2.3.2	Notification via Websocket	407

5.17.3	Data Model	407

5.17.3.1	General	407

5.17.3.2	Reused data types	408

5.17.3.3	Structured data types	409

5.17.3.3.1	Introduction	409

5.17.3.3.2	Type: AppAmContextExpData	409

5.17.3.3.3	Type: AppAmContextExpUpdateData	410

5.17.3.3.4	Type: GeographicalArea	410

5.17.3.3.5	Type: SliceReplReqInfo	410

5.17.3.3.6	Void	411

5.17.3.3.7	Type: SliceReplReq	411

5.17.3.4	Simple data types and enumerations	411

5.17.3.4.1	Introduction	411

5.17.3.4.2	Simple data types	411

5.17.3.4.3	Enumeration: SliceReplType	411

5.17.3.5	Data types describing alternative data types or combinations of data types	412

5.17.3.5.1	Type: AppAmContextExpRespData	412

5.17.4	Used Features	412

5.17.5	Error handling	412

5.17.5.1	General	412

5.17.5.2	Protocol Errors	412

5.17.5.3	Application Errors	412

5.18	AMInfluence API	413

5.18.0	Introduction	413

5.18.1	Resources	413

5.18.1.1	Overview	413

5.18.1.2	Resource: AM Influence Subscription	414

5.18.1.2.1	Introduction	414

5.18.1.2.2	Resource Definition	414

5.18.1.2.3	Resource Methods	414

5.18.1.2.3.1	General	414

5.18.1.2.3.2	GET	414

5.18.1.2.3.3	POST	415

5.18.1.3	Resource: Individual AM Influence Subscription	416

5.18.1.3.1	Introduction	416

5.18.1.3.2	Resource Definition	416

5.18.1.3.3	Resource Methods	416

5.18.1.3.3.1	General	416

5.18.1.3.3.2	GET	416

5.18.1.3.3.3	PUT	417

5.18.1.3.3.4	PATCH	418

5.18.1.3.3.5	DELETE	419

5.18.1A	Custom Operations without associated resources	420

5.18.2	Notifications	420

5.18.2.1	Introduction	420

5.18.2.2	Event Notification	420

5.18.2.2.1	Description	420

5.18.2.2.2	Target URI	420

5.18.2.2.3	Operation Definition	420

5.18.3	Data Model	421

5.18.3.1	General	421

5.18.3.2	Reused data types	422

5.18.3.3	Structured data types	422

5.18.3.3.1	Introduction	422

5.18.3.3.2	Type: AmInfluSub	422

5.18.3.3.3	Type: AmInfluSubPatch	425

5.18.3.3.4	Type: AmInfluEventNotif	427

5.18.3.3.5	Type: DnnSnssaiInformation	427

5.18.3.3.6	Type: SliceReplReqInfoRm	427

5.18.3.4	Simple data types and enumerations	427

5.18.3.4.1	Introduction	427

5.18.3.4.2	Simple data types	427

5.18.3.4.3	Enumeration: AmInfluEvent	428

5.18.4	Used Features	428

5.18.5	Error handling	428

5.18.5.1	General	428

5.18.5.2	Protocol Errors	428

5.18.5.3	Application Errors	429

5.19	MBSTMGI API	429

5.19.1	Introduction	429

5.19.2	Resources	429

5.19.3	Custom Operations without associated resources	429

5.19.3.1	Overview	429

5.19.3.2	Operation: Allocate	430

5.19.3.2.1	Description	430

5.19.3.2.2	Operation Definition	430

5.19.3.3	Operation: Deallocate	431

5.19.3.3.1	Description	431

5.19.3.3.2	Operation Definition	431

5.19.4	Notifications	432

5.19.4.1	General	432

5.19.4.2	Notification of Allocated MBS TMGI(s) Timer Expiry	432

5.19.4.2.1	Description	432

5.19.4.2.2	Target URI	432

5.19.4.2.3	Operation Definition	432

5.19.4.2.3.1	Notification via HTTP POST	432

5.19.4.2.3.2	Notification via Websocket	433

5.19.5	Data Model	433

5.19.5.1	General	433

5.19.5.2	Structured data types	434

5.19.5.2.1	Introduction	434

5.19.5.2.2	Type: TmgiAllocRequest	435

5.19.5.2.3	Type: TmgiAllocResponse	436

5.19.5.2.4	Type: TmgiDeallocRequest	436

5.19.5.2.5	Type: ExpiryNotif	436

5.19.5.2.6	Type: ReducedMbsServArea	437

5.19.5.3	Simple data types and enumerations	437

5.19.5.3.1	Introduction	437

5.19.5.3.2	Simple data types	437

5.19.5.4	Data types describing alternative data types or combinations of data types	438

5.19.5.4.1	Type: ProblemDetailsTmgiAlloc	438

5.19.6	Used Features	438

5.19.7	Error handling	438

5.19.7.1	General	438

5.19.7.2	Protocol Errors	438

5.19.7.3	Application Errors	438

5.20	MBSSession API	439

5.20.1	Introduction	439

5.20.2	Resources	439

5.20.2.1	Overview	439

5.20.2.2	Resource: MBS sessions	441

5.20.2.2.1	Introduction	441

5.20.2.2.2	Resource Definition	441

5.20.2.2.3	Resource Methods	442

5.20.2.2.3.1	POST	442

5.20.2.2.4	Resource Custom Operations	442

5.20.2.3	Resource: Individual MBS Session	442

5.20.2.3.1	Introduction	442

5.20.2.3.2	Resource Definition	442

5.20.2.3.3	Resource Standard Methods	443

5.20.2.3.3.1	PATCH	443

5.20.2.3.3.3	DELETE	444

5.20.2.3.4	Resource Custom Operations	445

5.20.2.4	Resource: MBS Session Subscriptions	445

5.20.2.4.1	Introduction	445

5.20.2.4.2	Resource Definition	445

5.20.2.4.3	Resource Methods	445

5.20.2.4.3.1	GET	445

5.20.2.4.3.2	POST	446

5.20.2.4.4	Resource Custom Operations	447

5.20.2.5	Resource: Individual MBS Session Subscription	447

5.20.2.5.1	Introduction	447

5.20.2.5.2	Resource Definition	447

5.20.2.5.3	Resource Methods	447

5.20.2.5.3.1	GET	447

5.20.2.5.3.2	DELETE	448

5.20.2.5.4	Resource Custom Operations	449

5.20.2.6	Resource: MBS Parameters Provisionings	449

5.20.2.6.1	Introduction	449

5.20.2.6.2	Resource Definition	449

5.20.2.6.3	Resource Methods	450

5.20.2.6.3.1	GET	450

5.20.2.6.3.2	POST	451

5.20.2.6.4	Resource Custom Operations	451

5.20.2.7	Resource: Individual MBS Parameters Provisioning	451

5.20.2.7.1	Introduction	451

5.20.2.7.2	Resource Definition	452

5.20.2.7.3	Resource Methods	452

5.20.2.7.3.1	GET	452

5.20.2.7.3.2	PUT	453

5.20.2.7.3.3	PATCH	454

5.20.2.7.3.4	DELETE	455

5.20.2.7.4	Resource Custom Operations	456

5.20.3	Custom Operations without associated resources	456

5.20.4	Notifications	456

5.20.4.1	General	456

5.20.4.2	MBS Session Status Notification	456

5.20.4.2.1	Description	456

5.20.4.2.2	Target URI	456

5.20.4.2.3	Operation Definition	457

5.20.4.2.3.1	Notification via HTTP POST	457

5.20.5	Data Model	457

5.20.5.1	General	457

5.20.5.2	Structured data types	458

5.20.5.2.1	Introduction	458

5.20.5.2.2	Type: MbsSessionCreateReq	459

5.20.5.2.3	Type: MbsSessionCreateRsp	459

5.20.5.2.4	Type: MbsSessionSubsc	459

5.20.5.2.5	Type: MbsSessionStatusNotif	459

5.20.5.2.6	Type: MbsPpData	460

5.20.5.2.7	Type: MbsSessAuthData	460

5.20.5.2.8	Type: MbsPpDataPatch	461

5.20.5.2.9	Type: MbsSessAssistInfo	461

5.20.5.2.10	Type: MbsSessionUpdateResp	461

5.20.5.3	Simple data types and enumerations	461

5.20.5.3.1	Introduction	461

5.20.5.3.2	Simple data types	462

5.20.6	Used Features	462

5.20.7	Error handling	462

5.20.7.1	General	462

5.20.7.2	Protocol Errors	462

5.20.7.3	Application Errors	462

5.21	EASDeployment API	463

5.21.0	Introduction	463

5.21.1	Resources	463

5.21.1.1	Overview	463

5.21.1.2	Resource: EAS Deployment Information	464

5.21.1.2.1	Introduction	464

5.21.1.2.2	Resource Definition	464

5.21.1.2.3	Resource Methods	464

5.21.1.2.3.1	General	464

5.21.1.2.3.2	GET	465

5.21.1.2.3.3	POST	465

5.21.1.3	Resource: Individual EAS Deployment Information	466

5.21.1.3.1	Introduction	466

5.21.1.3.2	Resource Definition	466

5.21.1.3.3	Resource Methods	466

5.21.1.3.3.1	General	466

5.21.1.3.3.2	GET	466

5.21.1.3.3.3	PUT	467

5.21.1.3.3.4	DELETE	468

5.21.2	Custom Operations without associated resources	469

5.21.2.1	Overview	469

5.21.2.2	Operation: remove-edis	470

5.21.2.2.1	Description	470

5.21.2.2.2	Operation Definition	470

5.21.3	Notifications	470

5.21.4	Data Model	470

5.21.4.1	General	470

5.21.4.2	Reused data types	471

5.21.4.3	Structured data types	471

5.21.4.3.1	Introduction	471

5.21.4.3.2	Type: EasDeployInfo	472

5.21.4.3.3	Type: DnaiInformation	472

5.21.4.3.4	Type: DnsServerIdentifier	473

5.21.4.3.5	Type: EdiDeleteCriteria	473

5.21.4.4	Simple data types and enumerations	474

5.21.4.4.1	Introduction	474

5.21.4.4.2	Simple data types	474

5.21.5	Used Features	474

5.21.6	Error handling	474

5.21.6.1	General	474

5.21.6.2	Protocol Errors	474

5.21.6.3	Application Errors	474

5.22	ASTI API	475

5.22.0	Introduction	475

5.22.1	Resources	475

5.22.1.1	Overview	475

5.22.1.2	Resource: ASTI Configurations	476

5.22.1.2.1	Introduction	476

5.22.1.2.2	Resource Definition	476

5.22.1.2.3	Resource Methods	476

5.22.1.2.3.1	General	476

5.22.1.2.3.2	GET	476

5.22.1.2.3.3	POST	477

5.22.1.2.4	Resource Custom Operations	478

5.22.1.2.4.1	Overview	478

5.22.1.2.4.2	Operation: retrieve	478

5.22.1.2.4.2.1	Description	478

5.22.1.2.4.2.2	Operation Definition	478

5.22.1.3	Resource: Individual ASTI Configuration	479

5.22.1.3.1	Introduction	479

5.22.1.3.2	Resource Definition	479

5.22.1.3.3	Resource Methods	479

5.22.1.3.3.1	General	479

5.22.1.3.3.2	GET	479

5.22.1.3.3.3	PUT	480

5.22.1.3.3.4	DELETE	481

5.22.2	Custom Operations without associated resources	482

5.22.3	Notifications	482

5.22.3.1	General	482

5.22.3.2	ASTI Notification	482

5.22.3.2.1	Description	482

5.22.3.2.2	Target URI	482

5.22.3.2.3	Operation Definition	482

5.22.3.2.3.1	Notification via HTTP POST	482

5.22.3.2.3.2	Void	483

5.22.4	Data Model	483

5.22.4.1	General	483

5.22.4.2	Reused data types	484

5.22.4.3	Structured data types	484

5.22.4.3.1	Introduction	484

5.22.4.3.2	Type: AccessTimeDistributionData	485

5.22.4.3.3	Type: StatusRequestData	485

5.22.4.3.4	Type: StatusResponseData	486

5.22.4.3.5	Type: ActiveUe	486

5.22.4.3.6	Type AstiConfigNotification	486

5.22.4.3.7	Type AstiConfigStateNotification	486

5.22.4.4	Simple data types and enumerations	486

5.22.4.4.1	Introduction	486

5.22.4.4.2	Simple data types	487

5.22.5	Used Features	487

5.22.6	Error handling	487

5.22.6.1	General	487

5.22.6.2	Protocol Errors	487

5.22.6.3	Application Errors	487

5.23	DataReporting API	488

5.23.1	Introduction	488

5.23.2	Resources	488

5.23.2.1	Overview	488

5.23.2.2	Resource: Data Reporting Sessions	489

5.23.2.2.1	Introduction	489

5.23.2.2.2	Resource definition	489

5.23.2.2.3	Resource Methods	489

5.23.2.2.3.1	POST	489

5.23.2.3	Resource: Individual Data Reporting Session	490

5.23.2.3.1	Introduction	490

5.23.2.3.2	Resource Definition	490

5.23.2.3.3	Resource standard methods	490

5.23.2.3.3.1	GET	490

5.23.2.3.3.2	PUT	491

5.23.2.3.3.3	DELETE	493

5.23.2.3.4	Resource custom operations	494

5.23.3	Custom Operations without associated resources	495

5.23.4	Notifications	495

5.23.5	Data Model	495

5.23.3.1	General	495

5.23.6	Used Features	495

5.23.7	Error handling	495

5.23.7.1	General	495

5.23.7.2	Protocol Errors	495

5.23.7.3	Application Errors	495

5.24	DataReportingProvisioning API	496

5.24.1	Introduction	496

5.24.2	Resources	496

5.24.2.1	Overview	496

5.24.2.2	Resource: Data Reporting Provisioning Sessions	498

5.24.2.2.1	Introduction	498

5.24.2.2.2	Resource definition	499

5.24.2.2.3	Resource Methods	499

5.24.2.2.3.1	POST	499

5.24.2.3	Resource: Individual Data Reporting Provisioning Session	500

5.24.2.3.1	Introduction	500

5.24.2.3.2	Resource Definition	500

5.24.2.3.3	Resource standard methods	500

5.24.2.3.3.1	GET	500

5.24.2.3.3.2	Void	501

5.24.2.3.3.3	DELETE	501

5.24.2.4	Resource: Data Reporting Configurations	502

5.24.2.4.1	Introduction	502

5.24.2.4.2	Resource definition	503

5.24.2.4.3	Resource Methods	503

5.24.2.4.3.1	POST	503

5.24.2.5	Resource: Individual Data Reporting Configuration	505

5.24.2.5.1	Introduction	505

5.24.2.5.2	Resource Definition	505

5.24.2.5.3	Resource standard methods	505

5.24.2.5.3.2	GET	505

5.24.2.5.3.3	PUT	506

5.24.2.5.3.3A	PATCH	508

5.24.2.5.3.4	DELETE	509

5.24.3	Custom Operations without associated resources	509

5.24.4	Notifications	510

5.24.5	Data Model	510

5.24.5.1	General	510

5.24.6	Used Features	510

5.24.7	Error handling	510

5.24.7.1	General	510

5.24.7.2	Protocol Errors	510

5.24.7.3	Application Errors	511

5.25	UEId API	511

5.25.1	Introduction	511

5.25.2	Resources	511

5.25.2.1	Overview	511

5.25.2.2	Resource: UE ID Mapping Information Provisionings	512

5.25.2.2.1	Introduction	512

5.25.2.2.2	Resource Definition	512

5.25.2.2.3	Resource Methods	512

5.25.2.2.3.1	GET	512

5.25.2.2.3.2	POST	513

5.25.2.2.4	Resource Custom Operations	514

5.25.2.3	Resource: Individual UE ID Mapping Provisioning	514

5.25.2.3.1	Introduction	514

5.25.2.3.2	Resource Definition	514

5.25.2.3.3	Resource Methods	515

5.25.2.3.3.1	GET	515

5.25.2.3.3.2	PUT	516

5.25.2.3.3.3	PATCH	517

5.25.2.3.3.4	DELETE	518

5.25.3	Custom Operations without associated resources	519

5.25.3.1	Overview	519

5.25.3.2	Operation: Retrieve	520

5.25.3.2.1	Description	520

5.25.3.2.2	Operation Definition	520

5.25.3.2.3	Void	521

5.25.3.3	Operation: GetMsisdn	521

5.25.3.3.1	Description	521

5.25.3.3.2	Operation Definition	521

5.25.3.4	Operation: VerifyMsisdn	522

5.25.3.4.1	Description	522

5.25.3.4.2	Operation Definition	522

5.25.4	Notifications	523

5.25.5	Data Model	523

5.25.5.1	General	523

5.25.5.2	Structured data types	524

5.25.5.2.1	Introduction	524

5.25.5.2.2	Type: UeIdReq	525

5.25.5.2.3	Type: UeIdInfo	525

5.25.5.2.4	Type: MsisdnReq	526

5.25.5.2.5	Type: MsisdnInfo	526

5.25.5.2.6	Type: UeIdMappingInfo	527

5.25.5.2.7	Type: RangSlUeIdMappInfo	527

5.25.5.2.8	Type: UeIdMappingInfoPatch	527

5.25.5.2.9	Type: MsisdnVerifReq	528

5.25.5.2.10	Type: MsisdnVerifResp	528

5.25.5.3	Simple data types and enumerations	529

5.25.5.3.1	Introduction	529

5.25.5.3.2	Simple data types	529

5.25.6	Used Features	529

5.25.7	Error handling	529

5.25.7.1	General	529

5.25.7.2	Protocol Errors	529

5.25.7.3	Application Errors	529

5.25.8	Security	530

5.26	MBSUserService API	530

5.26.1	Introduction	530

5.26.2	Resources	530

5.26.2.1	Overview	530

5.26.2.2	Resource: MBS User Services	531

5.26.2.2.1	Introduction	531

5.26.2.2.2	Resource Definition	531

5.26.2.2.3	Resource Standard Methods	531

5.26.2.2.3.1	GET	531

5.26.2.2.3.2	POST	532

5.26.2.2.4	Resource Custom Operations	533

5.26.2.3	Resource: Individual MBS User Service	533

5.26.2.3.1	Introduction	533

5.26.2.3.2	Resource Definition	533

5.26.2.3.3	Resource Standard Methods	533

5.26.2.3.3.1	GET	533

5.26.2.3.3.2	PUT	534

5.26.2.3.3.3	PATCH	535

5.26.2.3.3.4	DELETE	536

5.26.2.3.4	Resource Custom Operations	537

5.26.3	Custom Operations without associated resources	537

5.26.4	Notifications	537

5.26.5	Data Model	537

5.26.5.1	General	537

5.26.5.2	Structured data types	538

5.26.5.2.1	Introduction	538

5.26.5.3	Simple data types and enumerations	538

5.26.5.3.1	Introduction	538

5.26.5.3.2	Simple data types	538

5.26.6	Used Features	538

5.26.7	Error handling	539

5.26.7.1	General	539

5.26.7.2	Protocol Errors	539

5.26.7.3	Application Errors	539

5.27	MBSUserDataIngestSession API	539

5.27.1	Introduction	539

5.27.2	Resources	539

5.27.2.1	Overview	539

5.27.2.2	Resource: MBS User Data Ingest Sessions	541

5.27.2.2.1	Introduction	541

5.27.2.2.2	Resource Definition	541

5.27.2.2.3	Resource Standard Methods	541

5.27.2.2.3.1	GET	541

5.27.2.2.3.2	POST	542

5.27.2.2.4	Resource Custom Operations	543

5.27.2.3	Resource: Individual MBS User Data Ingest Session	543

5.27.2.3.1	Introduction	543

5.27.2.3.2	Resource Definition	543

5.27.2.3.3	Resource Standard Methods	543

5.27.2.3.3.1	GET	543

5.27.2.3.3.2	PUT	544

5.27.2.3.3.3	PATCH	545

5.27.2.3.3.4	DELETE	547

5.27.2.3.4	Resource Custom Operations	547

5.27.2.4	Resource: MBS User Data Ingest Session Status Subscriptions	548

5.27.2.4.1	Introduction	548

5.27.2.4.2	Resource Definition	548

5.27.2.4.3	Resource Standard Methods	548

5.27.2.4.3.1	GET	548

5.27.2.4.3.2	POST	549

5.27.2.4.4	Resource Custom Operations	550

5.27.2.5	Resource: Individual MBS User Data Ingest Session Status Subscription	550

5.27.2.5.1	Introduction	550

5.27.2.5.2	Resource Definition	550

5.27.2.5.3	Resource Standard Methods	550

5.27.2.5.3.1	GET	550

5.27.2.5.3.2	PUT	551

5.27.2.5.3.3	PATCH	552

5.27.2.5.3.4	DELETE	553

5.27.2.5.4	Resource Custom Operations	554

5.27.3	Custom Operations without associated resources	554

5.27.4	Notifications	554

5.27.4.1	General	554

5.27.4.2	MBS User Data Ingest Session Status Change Notification	555

5.27.4.2.1	Description	555

5.27.4.2.2	Target URI	555

5.27.4.2.3	Operation Definition	555

5.27.4.2.3.1	Notification via HTTP POST	555

5.27.4.2.3.2	Void	556

5.27.5	Data Model	556

5.27.5.1	General	556

5.27.5.2	Structured data types	557

5.27.5.2.1	Introduction	557

5.27.5.3	Simple data types and enumerations	557

5.27.5.3.1	Introduction	557

5.27.5.3.2	Simple data types	557

5.27.6	Used Features	557

5.27.7	Error handling	558

5.27.7.1	General	558

5.27.7.2	Protocol Errors	558

5.27.7.3	Application Errors	558

5.28	MSEventExposure API	559

5.28.1	Introduction	559

5.28.2	Resources	560

5.28.2.1	Overview	560

5.28.2.2	Resource: Media Streaming Event Exposure Subscriptions	560

5.28.2.2.1	Introduction	560

5.28.2.2.2	Resource Definition	561

5.28.2.2.3	Resource Standard Methods	561

5.28.2.2.3.1	GET	561

5.28.2.2.3.2	POST	562

5.28.2.2.4	Resource Custom Operations	562

5.28.2.3	Resource: Individual Media Streaming Event Exposure Subscription	562

5.28.2.3.1	Introduction	562

5.28.2.3.2	Resource Definition	563

5.28.2.3.3	Resource Standard Methods	563

5.28.2.3.3.1	GET	563

5.28.2.3.3.2	PUT	564

5.28.2.3.3.3	DELETE	565

5.28.2.3.4	Resource Custom Operations	566

5.28.3	Custom Operations without associated resources	566

5.28.4	Notifications	566

5.28.4.1	General	566

5.28.4.2	Media Streaming Event Exposure Notification	566

5.28.4.2.1	Description	566

5.28.4.2.2	Target URI	566

5.28.4.2.3	Operation Definition	566

5.28.4.2.3.1	Notification via HTTP POST	566

5.28.4.2.3.2	Void	567

5.28.5	Data Model	567

5.28.5.1	General	567

5.28.5.2	Structured data types	568

5.28.5.2.1	Introduction	568

5.28.5.3	Simple data types and enumerations	568

5.28.5.3.1	Introduction	568

5.28.5.3.2	Simple data types	568

5.28.6	Used Features	568

5.28.7	Error handling	568

5.28.7.1	General	568

5.28.7.2	Protocol Errors	569

5.28.7.3	Application Errors	569

5.29	MBSGroupMsgDelivery API	569

5.29.1	Introduction	569

5.29.2	Resources	569

5.29.2.1	Overview	569

5.29.2.2	Resource: MBS Group Message Deliveries	570

5.29.2.2.1	Introduction	570

5.29.2.2.2	Resource Definition	570

5.29.2.2.3	Resource Standard Methods	571

5.29.2.2.3.1	GET	571

5.29.2.2.3.2	POST	572

5.29.2.2.4	Resource Custom Operations	572

5.29.2.3	Resource: Individual MBS Group Message Delivery	572

5.29.2.3.1	Introduction	572

5.29.2.3.2	Resource Definition	572

5.29.2.3.3	Resource Standard Methods	573

5.29.2.3.3.1	GET	573

5.29.2.3.3.2	PATCH	574

5.29.2.3.3.3	DELETE	575

5.29.2.3.4	Resource Custom Operations	576

5.29.3	Custom Operations without associated resources	576

5.29.4	Notifications	576

5.29.4.1	General	576

5.29.4.2	MBS Group Message Delivery Notification	576

5.29.4.2.1	Description	576

5.29.4.2.2	Target URI	576

5.29.4.2.3	Operation Definition	577

5.29.5	Data Model	577

5.29.5.1	General	577

5.29.5.2	Structured data types	578

5.29.5.2.1	Introduction	578

5.29.5.2.2	Void	579

5.29.5.2.3	Type: MbsGroupMsgDel	579

5.29.5.2.4	Void	580

5.29.5.2.5	Type: MbsGroupMsgDelPatch	580

5.29.5.2.6	Type: MbsGroupMsgDelStatusNotif	580

5.29.5.3	Simple data types and enumerations	580

5.29.5.3.1	Introduction	580

5.29.5.3.2	Simple data types	580

5.29.5.4	Data types describing alternative data types or combinations of data types	581

5.29.5.4.1	Type: MbsServArea	581

5.29.6	Used Features	581

5.29.7	Error handling	581

5.29.7.1	General	581

5.29.7.2	Protocol Errors	581

5.29.7.3	Application Errors	581

5.30	DNAIMapping API	581

5.30.2	Resources	582

5.30.2.1	Overview	582

5.30.2.2	Resource: DNAI Mapping Subscriptions	583

5.30.2.2.1	Introduction	583

5.30.2.2.2	Resource Definition	583

5.30.2.2.3	Resource Standard Methods	583

5.30.2.2.3.1	Void	583

5.30.2.2.3.2	GET	583

5.30.2.2.3.3	POST	584

5.30.2.2.4	Resource Custom Operations	585

5.30.2.3	Resource: Individual DNAI Mapping Subscription	585

5.30.2.3.1	Introduction	585

5.30.2.3.2	Resource Definition	585

5.30.2.3.3	Resource Standard Methods	585

5.30.2.3.3.1	Void	585

5.30.2.3.3.2	GET	585

5.30.2.3.3.3	DELETE	586

5.30.2.3.4	Resource Custom Operations	587

5.30.4	Notifications	588

5.30.4.1	General	588

5.30.4.2	DNAI Mapping Information Update Notification	588

5.30.4.2.1	Description	588

5.30.4.2.2	Target URI	588

5.30.4.2.3	Operation Definition	588

5.30.4.2.3.1	Notification via HTTP POST	588

5.30.4.2.3.2	Void	589

5.30.5	Data Model	589

5.30.5.1	General	589

5.30.5.2	Structured data types	590

5.30.5.2.1	Introduction	590

5.30.5.2.2	Type: DnaiMapSub	591

5.30.5.2.3	Type: DnaiMapUpdateNotif	592

5.30.5.2.4	Void	592

5.30.5.3	Simple data types and enumerations	592

5.30.5.3.1	Introduction	592

5.30.5.3.2	Simple data types	592

5.30.5.4	Data types describing alternative data types or combinations of data types	592

5.30.6	Used Features	592

5.30.7	Error handling	593

5.30.7.1	General	593

5.30.7.2	Protocol Errors	593

5.30.7.3	Application Errors	593

5.31	PdtqPolicyNegotiation API	593

5.31.0	Introduction	593

5.31.1	Resources	593

5.31.1.1	Overview	593

5.31.1.2	Resource: PDTQ Policy Subscriptions	594

5.31.1.2.1	Introduction	594

5.31.1.2.2	Resource Definition	594

5.31.1.2.3	Resource Standard Methods	595

5.31.1.2.3.1	Void	595

5.31.1.2.3.2	GET	595

5.31.1.2.3.3	POST	596

5.31.1.2.4	Resource Custom Operations	596

5.31.1.3	Resource: Individual PDTQ Policy Subscription	596

5.31.1.3.1	Introduction	596

5.31.1.3.2	Resource Definition	597

5.31.1.3.3	Resource Standard Methods	597

5.31.1.3.3.1	Void	597

5.31.1.3.3.2	GET	597

5.31.1.3.3.3	PATCH	598

5.31.1.3.3.4	DELETE	599

5.31.1.3.4	Resource Custom Operations	600

5.31.1A	Custom Operations without associated resources	600

5.31.2	Notifications	600

5.31.2.1	General	600

5.31.2.2	PDTQ Warning Notification	601

5.31.2.2.1	Description	601

5.31.2.2.2	Target URI	601

5.31.2.2.3	Operation Definition	601

5.31.2.2.3.1	Notification via HTTP POST	601

5.31.3	Data Model	602

5.31.3.1	General	602

5.31.3.2	Void	602

5.31.3.3	Structured data types	602

5.31.3.3.1	Introduction	602

5.31.3.3.2	Type: Pdtq	603

5.31.3.3.3	Type: PdtqPatch	605

5.31.3.3.4	Type: PdtqNotification	605

5.31.3.4	Simple data types and enumerations	605

5.31.3.4.1	Introduction	605

5.31.3.4.2	Simple data types	605

5.31.3.4	Data types describing alternative data types or combinations of data types	606

5.31.4	Used Features	606

5.31.5	Error handling	606

5.31.5.1	General	606

5.31.5.2	Protocol Errors	606

5.31.5.3	Application Errors	606

5.32	MemberUESelectionAssistance API	606

5.32.1	Introduction	606

5.32.2	Resources	607

5.32.2.1	Overview	607

5.32.2.2	Resource: Member UE Selection Assistance Subscriptions	608

5.32.2.2.1	Introduction	608

5.32.2.2.2	Resource definition	608

5.32.2.2.3	Resource Standard Methods	609

5.32.2.2.3.1	POST	609

5.32.2.2.3.2	GET	609

5.32.2.2.4	Resource Custom Operations	610

5.32.2.3	Resource: Individual Member UE Selection Assistance Subscription	610

5.32.2.3.1	Introduction	610

5.32.2.3.2	Resource Definition	611

5.32.2.3.3	Resource Standard Methods	611

5.32.2.3.3.1	GET	611

5.32.2.3.3.2	PUT	612

5.32.2.3.3.3	PATCH	613

5.32.2.3.3.4	DELETE	614

5.32.2.3.4	Resource Custom Operations	615

5.32.3	Custom Operations without associated resources	615

5.32.4	Notifications	615

5.32.5	Data Model	617

5.32.5.1	General	617

5.32.5.2	Structured data types	619

5.32.5.2.1	Introduction	619

5.32.5.2.2	Type: MemUeSelectAssistSubsc	620

5.32.5.2.3	Type: MemUeSeletAssistNotif	622

5.32.5.2.4	Type: QoSFilterCriteria	623

5.32.5.2.5	Type: AccessRatTypeFilterCriteria	623

5.32.5.2.6	Type: E2ETransTimeFilterCriteria	624

5.32.5.2.7	Type: UeLocFilterCriteria	624

5.32.5.2.8	Type: UeHisLocFilterCriteria	625

5.32.5.2.9	Type: UeDirectionFilterCriteria	625

5.32.5.2.10	Type: UeDistanceFilterCriteria	625

5.32.5.2.11	Type: ServiceExpFilterCriteria	626

5.32.5.2.12	Type: DnnFilterCriteria	626

5.32.5.2.13	Type: CandiUeListInfo	627

5.32.5.2.14	Type: MemUeSeletReport	627

5.32.5.2.15	Type: MemUeSelectAssistSubscPatch	628

5.32.5.2.16	Type: CandUeInfo	629

5.32.5.3	Simple data types and enumerations	629

5.32.5.3.1	Introduction	629

5.32.5.3.2	Simple data types	629

5.32.5.3.3	Enumeration: FilterCriterionType	630

5.32.5.4	Data types describing alternative data types or combinations of data types	630

5.32.6	Used Features	630

5.32.7	Error handling	630

5.32.7.1	General	630

5.32.7.2	Protocol Errors	630

5.32.7.3	Application Errors	630

5.33	GroupParametersProvisioning API	631

5.33.1	Introduction	631

5.33.2	Resources	631

5.33.2.1	Overview	631

5.33.2.2	Resource: Group Parameters Provisionings	632

5.33.2.2.1	Introduction	632

5.33.2.2.2	Resource Definition	632

5.33.2.2.3	Resource Methods	632

5.33.2.2.3.1	GET	632

5.33.2.2.3.2	POST	633

5.33.2.2.4	Resource Custom Operations	634

5.33.2.3	Resource: Individual Group Parameters Provisioning	634

5.33.2.3.1	Introduction	634

5.33.2.3.2	Resource Definition	634

5.33.2.3.3	Resource Methods	635

5.33.2.3.3.1	GET	635

5.33.2.3.3.2	PUT	636

5.33.2.3.3.3	PATCH	637

5.33.2.3.3.4	DELETE	638

5.33.2.3.4	Resource Custom Operations	639

5.33.3	Custom Operations without associated resources	639

5.33.4	Notifications	639

5.33.5	Data Model	639

5.33.5.1	General	639

5.33.5.2	Structured data types	640

5.33.5.2.1	Introduction	640

5.33.5.2.2	Type: GrpPpData	640

5.33.5.2.3	Type: GrpPpDataPatch	640

5.33.5.2.4	Type: DnnSnssaiGrpData	641

5.33.5.2.5	Type: AfReqDefaultQoS	641

5.33.5.2.6	Type: LadnServArea	641

5.33.5.3	Simple data types and enumerations	641

5.33.5.3.1	Introduction	641

5.33.5.3.2	Simple data types	642

5.33.5.4	Data types describing alternative data types or combinations of data types	642

5.33.6	Used Features	642

5.33.7	Error handling	642

5.33.7.1	General	642

5.33.7.2	Protocol Errors	642

5.33.7.3	Application Errors	642

5.34	SliceParamProvision API	642

5.34.1	Introduction	642

5.34.2	Resources	643

5.34.2.1	Overview	643

5.34.2.2	Resource: Slice Parameters Provisionings	644

5.34.2.2.1	Introduction	644

5.34.2.2.2	Resource Definition	644

5.34.2.2.3	Resource Methods	644

5.34.2.2.3.1	GET	644

5.34.2.2.3.2	POST	645

5.34.2.2.4	Resource Custom Operations	646

5.34.2.3	Resource: Individual Slice Parameters Provisioning	646

5.34.2.3.1	Introduction	646

5.34.2.3.2	Resource Definition	646

5.34.2.3.3	Resource Methods	647

5.34.2.3.3.1	GET	647

5.34.2.3.3.2	PUT	648

5.34.2.3.3.3	PATCH	649

5.34.2.3.3.4	DELETE	650

5.34.2.3.4	Resource Custom Operations	651

5.34.3	Custom Operations without associated resources	651

5.34.4	Notifications	651

5.34.5	Data Model	651

5.34.5.1	General	651

5.34.5.2	Structured data types	652

5.34.5.2.1	Introduction	652

5.34.5.2.2	Type: SlicePpData	652

5.34.5.2.3	Type: SlicePpDataPatch	652

5.34.5.3	Simple data types and enumerations	652

5.34.5.3.1	Introduction	652

5.34.5.3.2	Simple data types	652

5.34.6	Used Features	653

5.34.7	Error handling	653

5.34.7.1	General	653

5.34.7.2	Protocol Errors	653

5.34.7.3	Application Errors	653

5.35	UeAddress API	653

5.35.1	Introduction	653

5.35.2	Resources	654

5.35.2.1	Overview	654

5.35.2.2	Resource: UE Address Subscriptions	654

5.35.2.2.1	Introduction	654

5.35.2.2.2	Resource Definition	654

5.35.2.2.3	Resource Standard Methods	655

5.35.2.2.3.1	Void	655

5.35.2.2.3.2	GET	655

5.35.2.2.3.3	POST	656

5.35.2.2.4	Resource Custom Operations	657

5.35.2.3	Resource: Individual UE Address Subscription	657

5.35.2.3.1	Introduction	657

5.35.2.3.2	Resource Definition	657

5.35.2.3.3	Resource Standard Methods	657

5.35.2.3.3.1	Void	657

5.35.2.3.3.2	GET	657

5.35.2.3.3.3	DELETE	658

5.35.2.3.4	Resource Custom Operations	659

5.35.3	Custom Operations without associated resources	660

5.35.3.1	Overview	660

5.35.3.2	Operation: Retrieve	660

5.35.3.2.1	Description	660

5.35.3.2.2	Operation Definition	660

5.35.4	Notifications	661

5.35.4.1	General	661

5.35.4.2	UE Address Notification	662

5.35.4.2.1	Description	662

5.35.4.2.2	Target URI	662

5.35.4.2.3	Standard Methods	662

5.35.4.2.3.1	POST	662

5.35.5	Data Model	663

5.35.5.1	General	663

5.35.5.2	Structured data types	663

5.35.5.2.1	Introduction	663

5.35.5.2.2	Type: UeAddressReq	664

5.35.5.2.3	Type: UeAddressInfo	664

5.35.5.2.4	Type: UeAddrSubsc	665

5.35.5.2.5	Type: UeAddrInfo	666

5.35.5.2.6	Type: UeAddrNotif	666

5.35.5.2.7	Void	666

5.35.5.3	Simple data types and enumerations	666

5.35.5.3.1	Introduction	666

5.35.5.3.2	Simple data types	666

5.35.5.4	Data types describing alternative data types or combinations of data types	667

5.35.6	Used Features	667

5.35.7	Error handling	667

5.35.7.1	General	667

5.35.7.2	Protocol Errors	667

5.35.7.3	Application Errors	667

5.36	ECSAddress API	667

5.36.1	Introduction	667

5.36.2	Resources	668

5.36.2.1	Overview	668

5.36.2.2	Resource: ECS Address Configuration Information Sets	669

5.36.2.2.1	Introduction	669

5.36.2.2.2	Resource Definition	669

5.36.2.2.3	Resource Standard Methods	669

5.36.2.2.3.1	Void	669

5.36.2.2.3.2	GET	669

5.36.2.2.3.3	POST	670

5.36.2.2.4	Resource Custom Operations	671

5.36.2.3	Resource: Individual ECS Address Configuration Information Set	671

5.36.2.3.1	Introduction	671

5.36.2.3.2	Resource Definition	671

5.36.2.3.3	Resource Standard Methods	672

5.36.2.3.3.1	Void	672

5.36.2.3.3.2	GET	672

5.36.2.3.3.3	PUT	673

5.36.2.3.3.3A	PATCH	674

5.36.2.3.3.4	DELETE	675

5.36.2.3.4	Resource Custom Operations	676

5.36.3	Custom Operations without associated resources	676

5.36.3.1	Overview	676

5.36.3.2	Operation: RemoveEcsAddr	677

5.36.3.2.1	Description	677

5.36.3.2.2	Operation Definition	677

5.36.4	Notifications	678

5.36.5	Data Model	678

5.36.5.1	General	678

5.36.5.2	Void	679

5.36.5.3	Structured data types	679

5.36.5.3.1	Introduction	679

5.36.5.3.2	Type: EcsAddrInfo	679

5.36.5.3.3	Type: EcsAddrDeleteCriteria	680

5.36.5.3.4	Type: EcsAddrInfoPatch	680

5.36.5.4	Simple data types and enumerations	680

5.36.5.4.1	Introduction	680

5.36.5.4.2	Simple data types	681

5.36.5.4	Data types describing alternative data types or combinations of data types	681

5.36.6	Used Features	681

5.36.7	Error handling	681

5.36.7.1	General	681

5.36.7.2	Protocol Errors	681

5.36.7.3	Application Errors	681

5.37	RSLPPIParametersProvisioning API	681

5.37.1	Introduction	681

5.37.2	Resources	682

5.37.2.1	Overview	682

5.37.2.2	Resource: RSLPPI Parameters Provisionings	683

5.37.2.2.1	Introduction	683

5.37.2.2.2	Resource Definition	683

5.37.2.2.3	Resource Methods	683

5.37.2.2.3.1	GET	683

5.37.2.2.3.2	POST	684

5.37.2.2.4	Resource Custom Operations	685

5.37.2.3	Resource: Individual RSLPPI Parameters Provisioning	685

5.37.2.3.1	Introduction	685

5.37.2.3.2	Resource Definition	685

5.37.2.3.3	Resource Methods	686

5.37.2.3.3.1	GET	686

5.37.2.3.3.2	PUT	687

5.37.2.3.3.3	PATCH	688

5.37.2.3.3.4	DELETE	689

5.37.2.3.4	Resource Custom Operations	690

5.37.3	Custom Operations without associated resources	690

5.37.4	Notifications	690

5.37.5	Data Model	690

5.37.5.1	General	690

5.37.5.2	Structured data types	691

5.37.5.2.1	Introduction	691

5.37.5.2.2	Type: RslppiData	691

5.37.5.2.3	Type: RslppiPpData	691

5.37.5.2.4	Type: RslppiPpDataPatch	691

5.37.5.3	Simple data types and enumerations	691

5.37.5.3.1	Introduction	691

5.37.5.3.2	Simple data types	692

5.37.5.4	Data types describing alternative data types or combinations of data types	692

5.37.6	Used Features	692

5.37.7	Error handling	692

5.37.7.1	General	692

5.37.7.2	Protocol Errors	692

5.37.7.3	Application Errors	692

5.38	AddressingParamProvision API	692

5.38.1	Introduction	692

5.38.2	Resources	693

5.38.2.1	Overview	693

5.38.2.2	Resource: Addressing Parameters Provisionings	694

5.38.2.2.1	Introduction	694

5.38.2.2.2	Resource Definition	694

5.38.2.2.3	Resource Standard Methods	694

5.38.2.2.3.1	GET	694

5.38.2.2.3.2	POST	695

5.38.2.2.4	Resource Custom Operations	696

5.38.2.3	Resource: Individual Addressing Parameters Provisioning	696

5.38.2.3.1	Introduction	696

5.38.2.3.2	Resource Definition	696

5.38.2.3.3	Resource Standard Methods	697

5.38.2.3.3.1	GET	697

5.38.2.3.3.2	PUT	698

5.38.2.3.3.3	PATCH	699

5.38.2.3.3.4	DELETE	700

5.38.2.3.4	Resource Custom Operations	701

5.38.3	Custom Operations without associated resources	701

5.38.4	Notifications	701

5.38.5	Data Model	702

5.38.5.1	General	702

5.38.5.2	Structured data types	702

5.38.5.2.1	Introduction	702

5.38.5.2.2	Type: AddrPpData	702

5.38.5.2.3	Type: StaticIpAddrParams	703

5.38.5.2.4	Type: AddrPpDataPatch	703

5.38.5.3	Simple data types and enumerations	703

5.38.5.3.1	Introduction	703

5.38.5.3.2	Simple data types	703

5.38.5.4	Data types describing alternative data types or combinations of data types	703

5.38.5.5	Binary data	704

5.38.5.5.1	Binary Data Types	704

5.38.6	Used Features	704

5.38.7	Error handling	704

5.38.7.1	General	704

5.38.7.2	Protocol Errors	704

5.38.7.3	Application Errors	704

5.39	UAVFlightAssistance API	704

5.39.1	Introduction	704

5.39.2	Resources	705

5.39.2.1	Overview	705

5.39.2.2	Resource: UAV Flight Assistance Configurations	706

5.39.2.2.1	Introduction	706

5.39.2.2.2	Resource definition	706

5.39.2.2.3	Resource Standard Methods	706

5.39.2.2.3.0	GET	706

5.39.2.2.3.1	POST	708

5.39.2.2.4	Resource Custom Operations	708

5.39.2.3	Resource: Individual UAV Flight Assistance Configuration	708

5.39.2.3.1	Introduction	708

5.39.2.3.2	Resource Definition	709

5.39.2.3.3	Resource Standard Methods	709

5.39.2.3.3.0	GET	709

5.39.2.3.3.1	PUT	710

5.39.2.3.3.1A	PATCH	711

5.39.2.3.3.2	DELETE	712

5.39.2.3.4	Resource Custom Operations	713

5.39.3	Custom Operations without associated resources	713

5.39.3.1	Overview	713

5.39.3.2	Operation: Retrieve	714

5.39.3.2.1	Description	714

5.39.3.2.2	Operation Definition	714

5.39.4	Notifications	715

5.39.4.1	General	715

5.39.5	Data Model	717

5.39.5.1	General	717

5.39.5.2	Structured data types	718

5.39.5.2.1	Introduction	718

5.39.5.2.2	Type: UavFlightAssistance	719

5.39.5.2.3	Type: FlightPath	721

5.39.5.2.4	Type: FlightPathSegInfo	722

5.39.5.2.5	Type: UAVFlightAssistNotif	723

5.39.5.2.6	Type: UAVFlightAssistNotifResp	725

5.39.5.2.7	Type: UavFlightAssistancePatch	725

5.39.5.2.8	Type: AltitudeRepReqs	726

5.39.5.2.9	Type: AltitudeReport	726

5.39.5.2.10	Type: AltThresholds	726

5.39.5.2.11	Type: RangSlUavData	727

5.39.5.2.12	Type: UAVFlightInfoReq	727

5.39.5.2.13	CandidateFlightPathInfo	727

5.39.5.2.14	Type: UAVFlightInfoResp	728

5.39.5.3	Simple data types and enumerations	728

5.39.5.3.1	Introduction	728

5.39.5.3.2	Simple data types	728

5.39.5.3.3	Enumeration: UavStatus	728

5.39.5.3.4	Enumeration: RequestPurpose	728

5.39.5.3.5	Enumeration: AltitudeRepEvent	729

5.39.5.3.6	Enumeration: MonitoringMode	729

5.39.5.3.7	Enumeration: RetrievePurpose	729

5.39.6	Used Features	729

5.39.7	Error handling	730

5.39.7.1	General	730

5.39.7.2	Protocol Errors	730

5.39.7.3	Application Errors	730

5.40	CagInfoParamProvision API	730

5.40.1	Introduction	730

5.40.2	Resources	731

5.40.2.1	Overview	731

5.40.2.2	Resource: CAG Information Parameters Provisionings	732

5.40.2.2.1	Introduction	732

5.40.2.2.2	Resource Definition	732

5.40.2.2.3	Resource Standard Methods	732

5.40.2.2.3.1	GET	732

5.40.2.2.3.2	POST	733

5.40.2.2.4	Resource Custom Operations	734

5.40.2.3	Resource: Individual CAG Information Parameters Provisioning	734

5.40.2.3.1	Introduction	734

5.40.2.3.2	Resource Definition	734

5.40.2.3.3	Resource Standard Methods	734

5.40.2.3.3.1	GET	734

5.40.2.3.3.2	PUT	735

5.40.2.3.3.3	PATCH	736

5.40.2.3.3.4	DELETE	737

5.40.2.3.4	Resource Custom Operations	738

5.40.3	Custom Operations without associated resources	738

5.40.4	Notifications	739

5.40.5	Data Model	739

5.40.5.1	General	739

5.40.5.2	Structured data types	739

5.40.5.2.1	Introduction	739

5.40.5.2.2	Type: CagInfoPpData	739

5.40.5.2.3	Type: CagInfoPpDataPatch	740

5.40.5.2.4	Type: CagInformation	740

5.40.5.3	Simple data types and enumerations	740

5.40.5.3.1	Introduction	740

5.40.5.3.2	Simple data types	740

5.40.5.5	Binary data	740

5.40.5.5.1	Binary Data Types	740

5.40.6	Used Features	740

5.40.7	Error handling	741

5.40.7.1	General	741

5.40.7.2	Protocol Errors	741

5.40.7.3	Application Errors	741

5.41	Void	741

5.42	ImsSessionManagement API	741

5.42.1	Introduction	741

5.42.2	Resources	741

5.42.2.1	Overview	741

5.42.2.2	Resource: IMS Sessions	742

5.42.2.2.1	Description	742

5.42.2.2.2	Resource Definition	742

5.42.2.2.3	Resource Standard Methods	743

5.42.2.2.3.1	POST	743

5.42.2.2.3.2	GET	743

5.42.2.2.4	Resource Custom Operations	744

5.42.2.3	Resource: Individual IMS Session	744

5.42.2.3.1	Description	744

5.42.2.3.2	Resource Definition	744

5.42.2.3.3	Resource Standard Methods	745

5.42.2.3.3.0A	GET	745

5.42.2.3.3.0B	PUT	746

5.42.2.3.3.1	PATCH	747

5.42.2.3.3.2	DELETE	748

5.42.2.3.4	Resource Custom Operations	749

5.42.3	Custom Operations without associated resources	749

5.42.4	Notifications	749

5.42.5	Data Model	751

5.42.5.1	General	751

5.42.5.2	Structured data types	751

5.42.5.2.1	Introduction	751

5.42.5.2.2	Type: ImsSession	752

5.42.5.3	Simple data types and enumerations	752

5.42.5.3.1	Introduction	752

5.42.5.3.2	Simple data types	752

5.42.5.4	Data types describing alternative data types or combinations of data types	752

5.42.5.5	Binary data	752

5.42.5.5.1	Binary Data Types	752

5.42.6	Used Features	752

5.42.7	Error Handling	753

5.42.7.1	General	753

5.42.7.2	Protocol Errors	753

5.42.7.3	Application Errors	753

5.43	ImsEventExposure API	753

5.43.1	Introduction	753

5.43.2	Resources	754

5.43.2.1	Overview	754

5.43.2.2	Resource: IMS EE Subscriptions	754

5.43.2.2.1	Introduction	754

5.43.2.2.2	Resource definition	755

5.43.2.2.3	Resource Standard Methods	755

5.43.2.2.3.1	POST	755

5.43.2.2.3.2	GET	756

5.43.2.2.4	Resource Custom Operations	757

5.43.2.3	Resource: Individual IMS EE Subscription	757

5.43.2.3.1	Introduction	757

5.43.2.3.2	Resource Definition	757

5.43.2.3.3	Resource Standard Methods	757

5.43.2.3.3.1	GET	757

5.43.2.3.3.2	PUT	758

5.43.2.3.3.3	PATCH	759

5.43.2.3.3.4	DELETE	760

5.43.2.3.4	Resource Custom Operations	761

5.43.3	Custom Operations without associated resources	761

5.43.4	Notifications	761

5.43.5	Data Model	763

5.43.5.1	General	763

5.43.5.2	Structured data types	763

5.43.5.2.1	Introduction	763

5.43.5.2.2	Type: ImsEESubsc	764

5.43.5.2.3	Type: ImsEENotif	764

5.43.5.2.5	Type: ImsEventRequirement	765

5.43.5.2.6	Type: ImsEventReport	765

5.43.5.3	Simple data types and enumerations	765

5.43.5.3.1	Introduction	765

5.43.5.3.2	Simple data types	765

5.43.5.3.3	Enumeration: ImsEventType	765

5.43.5.4	Data types describing alternative data types or combinations of data types	766

5.43.5.5	Binary data	766

5.43.5.5.1	Binary Data Types	766

5.43.6	Used Features	766

5.43.7	Error handling	766

5.43.7.1	General	766

5.43.7.2	Protocol Errors	766

5.43.7.3	Application Errors	766

5.44	ImsParamProvision API	767

5.44.1	Introduction	767

5.44.2	Resources	767

5.44.2.1	Overview	767

5.44.2.2	Resource: IMS Parameters Provisionings	768

5.44.2.2.1	Introduction	768

5.44.2.2.2	Resource Definition	768

5.44.2.2.3	Resource Methods	768

5.44.2.2.3.1	GET	768

5.44.2.2.3.2	POST	769

5.44.2.2.4	Resource Custom Operations	770

5.44.2.3	Resource: Individual IMS Parameters Provisioning	770

5.44.2.3.1	Introduction	770

5.44.2.3.2	Resource Definition	770

5.44.2.3.3	Resource Methods	770

5.44.2.3.3.1	GET	770

5.44.2.3.3.2	PUT	771

5.44.2.3.3.3	Void	773

5.44.2.3.3.4	DELETE	773

5.44.2.3.4	Resource Custom Operations	774

5.44.3	Custom Operations without associated resources	774

5.44.4	Notifications	774

5.44.5	Data Model	774

5.44.5.1	General	774

5.44.5.2	Structured data types	774

5.44.5.2.1	Introduction	774

5.44.5.2.2	Type: ImsData	774

5.44.5.2.3	Type: ImsPpData	775

5.44.5.2.4	Void	775

5.44.5.3	Simple data types and enumerations	775

5.44.5.3.1	Introduction	775

5.44.5.3.2	Simple data types	775

5.44.5.4	Data types describing alternative data types or combinations of data types	775

5.44.5.5	Binary data	775

5.44.5.5.1	Binary Data Types	775

5.44.6	Used Features	775

5.44.7	Error handling	776

5.44.7.1	General	776

5.44.7.2	Protocol Errors	776

5.44.7.3	Application Errors	776

5.45	AIoT API	776

5.45.1	Introduction	776

5.45.2	Resources	776

5.45.3	Custom Operations without associated resources	777

5.45.3.1	Overview	777

5.45.3.2	Operation: InventoryRequest	777

5.45.3.2.1	Description	777

5.45.3.2.2	Operation Definition	777

5.45.3.3	Operation: CommandRequest	778

5.45.3.3.1	Description	778

5.45.3.3.2	Operation Definition	778

5.45.4	Notifications	779

5.45.4.1	General	779

5.45.4.2	AIoT Operations Notification	780

5.45.4.2.1	Description	780

5.45.4.2.2	Target URI	780

5.45.4.2.3	Standard Methods	780

5.45.5	Data Model	781

5.45.5.1	General	781

5.45.5.2	Structured data types	781

5.45.5.2.1	Introduction	781

5.45.5.2.2	Type: InventoryReq	782

5.45.5.2.3	Type: InventoryResp	782

5.45.5.2.4	Type: CommandReq	783

5.45.5.2.5	Type: CommandResp	785

5.45.5.2.6	Type: AIoTNotif	785

5.45.5.2.7	Type: ExtAIoTArea	786

5.45.5.3	Simple data types and enumerations	786

5.45.5.3.1	Introduction	786

5.45.5.3.2	Simple data types	786

5.45.5.3.3	Enumeration: CommandType	786

5.45.5.4	Data types describing alternative data types or combinations of data types	786

5.45.5.5	Binary data	787

5.45.5.1	Binary Data Types	787

5.45.6	Used Features	787

5.45.7	Error handling	787

5.45.7.1	General	787

5.45.7.2	Protocol Errors	787

5.45.7.3	Application Errors	787

5.47	VFLInference API	800

5.47.1	Introduction	800

5.47.2	Resources	800

5.47.2.1	Overview	800

5.47.2.2	Resource: VFL Inference subscriptions	801

5.47.2.2.1	Description	801

5.47.2.2.2	Resource Definition	801

5.47.2.2.3	Resource Standard Methods	801

5.47.2.2.3.1	POST	802

5.42.2.2.4	Resource Custom Operations	802

5.47.2.3	Resource: Individual VFL Inference subscription	802

5.47.2.3.1	Description	802

5.47.2.3.2	Resource Definition	803

5.47.2.3.3	Resource Standard Methods	803

5.47.2.3.3.1	PUT	803

5.47.2.3.3.2	PATCH	804

5.47.2.3.3.3	DELETE	806

6.1.3.2.4	Resource Custom Operations	807

5.47.3	Custom Operations without associated resources	807

5.47.4	Notifications	807

5.47.4.1	General	807

5.47.4.2	VFL Inference Event Notification	807

5.47.4.2.1	Description	807

5.47.4.2.2	Target URI	807

5.47.4.2.3	Operation Definition	807

5.47.5	Data Model	808

5.47.5.1	General	808

5.47.5.2	Structured data types	809

5.47.5.2.1	Introduction	809

5.47.5.2.2	Type VflInferSub	809

5.47.5.2.3	Type VflInferSubPatch	810

5.47.5.3	Simple data types and enumerations	810

5.47.5.3.1	Introduction	810

5.47.5.3.2	Simple data types	810

5.47.6	Used Features	810

5.47.7	Error handling	810

5.47.7.1	General	810

5.47.7.2	Protocol Errors	810

5.47.7.3	Application Errors	811

5.48	VFLNFDiscovery API	811

5.48.1	Introduction	811

5.48.2	Resources	811

5.48.3	Custom Operations without associated resources	811

5.48.3.1	Overview	811

5.48.3.2	Operation: DiscoverNwdaf	812

5.48.3.2.1	Description	812

5.48.3.2.2	Operation Definition	812

5.48.3.3	Operation: ReleaseNwdaf	813

5.48.3.3.1	Description	813

5.48.3.3.2	Operation Definition	813

5.48.4	Notifications	814

5.48.5	Data Model	814

5.48.5.1	General	814

5.48.5.2	Structured data types	815

5.48.5.2.1	Introduction	815

5.48.5.2.2	Type: NwdafDiscoveryRequest	815

5.48.5.2.3	Type: NwdafDiscoveryResponse	815

5.48.5.2.4	Type: NwdafReleaseRequest	815

5.48.5.3	Simple data types and enumerations	815

5.48.5.3.1	Introduction	815

5.48.5.3.2	Simple data types	816

5.48.5.3.3	VFLCapabilityType	816

5.48.6	Used Features	816

5.48.7	Error handling	816

5.48.7.1	General	816

5.48.7.2	Protocol Errors	816

5.48.7.3	Application Errors	816

6	Security	817

7	Using Common API Framework	817

7.1	General	817

7.2	Security	817

Annex A (normative):	OpenAPI representation for NEF Northbound APIs	819

A.1	General	819

A.2	TrafficInfluence API	819

A.3	NiddConfigurationTrigger API	831

A.4	AnalyticsExposure API	833

A.5	5GLANParameterProvision API	851

A.6	ApplyingBdtPolicy API	860

A.7	IPTVConfiguration API	864

A.8	LpiParameterProvision API	870

A.9	ServiceParameter API	875

A.10	ACSParameterProvision API	890

A.11	MoLcsNotify API	895

A.12	AKMA API	897

A.13	TimeSyncExposure API	899

A.14	EcsAddressProvision API	914

A.15	AMPolicyAuthorization API	918

A.16	AMInfluence API	926

A.17	MBSTMGI API	932

A.18	MBSSession API	936

A.19	EASDeployment API	947

A.20	ASTI API	953

A.21	DataReporting API	959

A.22	DataReportingProvisioning API	963

A.23	UEId API	968

A.24	MBSUserService API	976

A.25	MBSUserDataIngestSession API	980

A.26	MSEventExposure API	990

A.27	MBSGroupMsgDelivery API	994

A.28	DNAIMapping API	999

A.29	PDTQPolicyNegotiation API	1003

A.30	MemberUESelectionAssistance API	1009

A.31	GroupParametersProvisioning API	1020

A.32	SliceParamProvision API	1025

A.33	UEAddress API	1030

A.34	ECSAddress API	1035

A.35	RSLPPIParametersProvisioning API	1041

A.36	AddressingParamProvision API	1046

A.37	UAVFlightAssistance API	1051

A.38	CagInfoParamProvision API	1061

A.39	Void	1066

A.40	ImsSessionManagement API	1066

A.41	ImsEventExposure API	1071

A.42	ImsParamProvision API	1077

A.43	AIoT API	1081

A.45	VFLInference API	1091

A.46	VFLNFDiscovery API	1096

Annex B (informative):	Change history	1099