3GPP TSG CT WG4 Meeting #131 	C4-254340

# Sophia-Antipolis , FR, 13th – 17th October 2025			was C4-254020

# Source:	China Telecom

# Title:	New SID on 6G Resilience and Restoration

# Document for:	Approval

# Agenda Item:	20.2.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on 6G Resilience and Restoration

Acronym:	FS_6G_RERE_CT

Unique identifier:	xxxxxx

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_6G_REQ | SA1 | 1050110 | Study on 6G Use Cases and Service Requirements |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1080057 | Study on Architecture for 6G System | SA2 6G study; TR 23.801-01 |

3	Justification

Over the past ten years, 5G technologies have significantly enriched network access methods and network services, led to a sharp increase in the user number, and enabled different kinds of network deployments (e.g. MEC, NPN). But not everything is going well, prosperity of 5G may have other meanings for operators:

-	Flexible access methods may bring complicated access management.

-	Enriched network services may bring new NFs, interfaces and control flows.

-	The surge in users (especially millions of IoT UEs) has increased the risk of network overload and signaling storms.

-	The other risks brought by big changes in network architecture (e.g. Heterogeneous network), protocol (e.g. from MAP to HTTP), services (e.g. AI, sensing) and so on.

Compared to the big changes mentioned above, mechanism for disaster prevention and restoration almost stay in 3G/4G, only some patch-works are done in 5G. The robustness and resilience of the 5G network do not catch up with the quick development of 5G architecture, protocols and services. From the 2019 to 2023 public information, the reported significant outage number of each year is “3,4,7,15,15”, and the 10 million users level outage number of each year is “2,2,2,5,6”, we can see an obvious increase in outage number after 5G is deployed. Based on the situation, it is hard to believe that 5G network meet the required accident rate, i.e.,0.0001%, which is the target accident rate designed for 5G.

Now we are at the early stage for 6G design, it is a good chance for us to take robustness and resilience into consideration and make some innovations, to make 6G disaster prevention and restoration more effective and efficient. So new study on 6G Disaster Prevention and Restoration is proposed.

Generally, the following directions are considered:

-	6G builds up a collaboration mechanism across the whole network, for monitoring, disaster prevention, disaster control, and restoration. In 5G and before, one disaster prevention or restoration mechanism is typically designed only for one particular NF (e.g. UDM, PCF, AMF), and do not consider the relations and influences between NFs network-wide. But failures of different NFs are often correlated, one abnormal NF may result in failures of other NFs. For example, UDM overload may lead to a signal storm that may affect almost all the NFs in the network, and during the UDM failure recovery, misoperation (e.g. Let UEs reconnect too fast) can also easily cause overload and failures in other NFs (e.g. AMF, SMF). Another problem is that NFs are usually lack of other NF statues. In 5G, the detection of NF status largely relies on subscription and notification mechanism of NRF. However, this indirect method has latency and inaccuracy issues.

-	6G protocol helps to improve disaster prevention and restoration. In Rel-19 we already studied protocol enhancements for disaster prevention and restoration, for example in TR 29.866, we studied to extent OCI HTTP header to mitigate HSS/UDM congestion. In 6G, when we design new protocols or enhance legacy protocols, disaster prevention and restoration should be taken into account, because protocol may be the most direct and efficient approach for network robustness and resilience.

-	6G supports seamless resilience. 6G network resilience requires to minimize service disruption during NF failures or data loss. The current situation is, when NFs (especially the database NFs, e.g., UDM, PCF, UDR) are unavailable, all the UEs are interrupted. This is not friendly to the UEs on service, how to make these UEs feel seamless resilience is valuable for operators. In order to realize seamless resilience, some alternative methods can be considered, for example NF bypass, enhanced authentication, seamless UE migration, and so on. Terminals can also use local-configured or cached context to help seamless resilience.

-	6G supports failure isolation. As NF failure may have impacts on other NFs, and one service failure may also impact other services, failure isolation can help reduce the impacts. For example, in current cases, when IMS voice fails, usually the UE's data service will be affected as well, such as triggering PDU release. Another case is network sharing, how to isolate failure needs to be studied. After failure, how to restoration in isolation cases should also be considered.

-	6G supports AI+ disaster prevention and restoration. In Rel-19, SA2 have already studied NWDAF-assisted Network Abnormal Behaviour Mitigation and Prevention in TR 23700-84. In 6G, the capabilities of AI become stronger, network AI can be applicated to more scenarios and cases.

4	Objective

The main objective is to study resilience and restoration aspects for the 6G System, based on the service requirements developed by SA1, architecture requirements developed by SA2. Besides that, security requirements developed by SA3 and the new 6G protocols developed by CT1/3/4 should be taken into consideration.

More specifically, in the early phase of CT4 study, prior to SA2 interim agreements and conclusions being available, the CT4 work shall rely on the Architectural Assumptions and Requirements specified in Clause 4 of TR 23.801-1 and the CT4 study should not start on items dependent on network architecture until related work has reached progress.

The expected work tasks include:

WT#1:	Study and identify network failure scenarios and limitations with current restoration procedures, and their impacts as a basis to provide resilience and restoration framework for 6G services.

WT#2:	Study the 6G resiliency and restoration requirements, considering the 6G architecture (SA2 dependence), new 6G services (SA1 dependence) and new 6G protocols (CT1/3/4 dependence), including:

WT#2.1:	Map the 6G service requirements (e.g. Clause 5.6 of 3GPP TS 22.870) to 6G resilience and restoration requirements (e.g. requirement on framework, requirement on mechanisms, requirement on procedures, and etc.).

WT#2.2:	Study on resilience and restoration requirements (e.g. requirement on framework, requirement on mechanisms, requirement on procedures, and etc.) based on the identified network failure scenarios and limitations in WT#1.

WT#2.3:	Follow the studies for 6G protocols in CT1/3/4, identify new resiliency and restoration requirements (e.g. framework, mechanism, procedure, and etc.) brought by the new protocols.

WT#2.4:	Follow the studies for AI in SA2, identify possible ways for AI to help resilience and restoration (e.g. Self-healing), and also identify new challenges/requirements brought by AI (e.g. AI may participate/assist in service logic computation and provide networking policies, and bring uncertainty to the network).

WT#3:	Study on 6G resilience and restoration framework based on the requirements and 6G architecture, both control plane and user plane should be considered, the expected work includes:

WT#3.1:	Identify the resilience and restoration related network entities, define their resilience-related functions and the interfaces between them.

WT#3.2:	Study on systematic collaboration for resilience and restoration, including collaboration between network entities, and collaboration between 6GS and 5GS.

WT#3.3:	Identify the framework for user plane resilience and restoration.

WT#4:	Study on 6G resilience and restoration mechanisms, based on the framework, including:

WT#4.1:	Study on network failure prevention, including:

WT#4.1.1: Eliminating single points of failure (e.g. through adequate redundancy).

WT#4.1.2:	 Enhance failure perception, assessment, and the timely supervision.

WT#4.1.3: Enhance NF overload control.

WT#4.2:	Study on network failure control, including:

WT#4.2.1:	 Failure isolation between networks/operators (e.g. IMS and core network, core network and edge network, network sharing operators).

WT#4.2.2:	 Service maintenance for existing users in case of network failure.

WT#4.2.3:	 Stability of services provided by the edge network in case of abnormal transmission occurring between the core network and the edge network.

WT#4.2.4: User migration in case of network failure.

WT#4.2.5:	 Prevention of signal storm (e.g. Seamless Data Centre switch, as described in Clause 6.5 of 3GPP TS 22.870).

WT#4.3:	Study on network failure recovery, including:

WT#4.3.1: Zero-outage network to fulfill different service reliability requirement (e.g. Quick recovery for high priority services).

WT#4.3.2: Rapid network recovery and continuity during emergency situations (e.g. Natural disasters or large-scale outages).

WT#4.3.3: Interoperability and coordination among different network operators in case of network outage.

WT#5:	Study on 6G resilience and restoration procedures, based on the framework and mechanisms, including:

WT#5.1:	Study to build up data flows for resilience and restoration mechanisms.

WT#5.2:	Study to build up data flows for resilience and restoration collaborations.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 29.xxx | 6G Disaster Prevention and Restoration | TSG CT#114 (December 2026) | TSG CT#115 (March 2027) |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| N/A |  |  |  |

6	Work item Rapporteur(s)

7	Work item leadership

CT4

8	Aspects that involve other WGs

N/A.

9	Supporting Individual Members

Supporting IM name

China Telecom

Orange

CATT